Digital Key Telephone System GDK-162/100/FPII Programming Manual Feature Description

CONTENTS

1 . CO LINE FEATURES	1-1

AUTO CALL NUMBER REDIAL	1-1

AUTOMATIC HOLD	1-1

AUTOMATIC PAUSE INSERTION	1-1

CALL PARK	1-2

CALL TRANSFER	1-2

CALLING LINE IDENTIFICATION PRESENTATION (CLIP)	1-2

CO CALL DROP TIMER	1-3

CO CALL TRANSFER TO CO LINE	1-4

CO FLASH	1-4

CO LINE ACCESS	1-5

CO LINE GROUPS	1-5

CO LINE NAME DISPLAY	1-5

CO LINE QUEUING	1-6

CO LINE RING ASSIGNMENT	1-6

CO LINE RELEASE GUARD TIME	1-6

CONNECTED LINE IDENTIFICATION PRESENTATION (COLP)	1-7

CONNECTED LINE IDENTIFICATION RESTRICTION (COLR)	1-7

CO RING DETECT – Analog CO only	1-7

CO STEP CALL – Analog CO only	1-8

DIAL MEMO	1-8

DIAL PULSE SENDING – Analog CO only	1-8

DIAL PULSE TO TONE SWITCH OVER	1-9

DISPLAY SECURITY	1-9

DISTINCT CO RING	1-9

DTMF SIGNAL SENDING	1-9

EXCLUSIVE HOLD RECALL TIMER	1-9

EXECUTIVE/SECRETARY TRANSFER	1-10

HOLD PREFERENCE	1-10

KEYPAD FACILITY SERVICE	1-11

LAST NUMBER REDIAL (LNR)	1-11

MUSIC ON HOLD (MOH)	1-12

PAUSE INSERTION IN SPEED DIAL	1-12

PREFERRED LINE ANSWER	1-12

PRIVATE LINE	1-12

SAVE NUMBER REDIAL	1-13

STATION SPEED DIAL	1-13

SYSTEM HOLD RECALL TIMER	1-14

SYSTEM SPEED DIAL	1-14

SYSTEM SPEED ZONE GROUP	1-15

WORLD TIME DISPLAY	1-15

2 . INTERCOM FEATURES	2-1

BACKGROUND MUSIC	2-1

BUSY LAMP FIELD (BLF)	2-1

CALCULATOR	2-1

CALENDAR DISPLAY	2-2

CALL FORWARD-BUSY/NO ANSWER	2-2

CALL FORWARD BUTTON PROGRAM	2-2

CALL FORWARD-FOLLOW ME	2-3

CALL FORWARD-INCOMING CO LINE OFF-NET	2-3

CALL FORWARD-PRESET	2-4

CALL FORWARD-STATION OFF-NET	2-4

CALL FORWARD - UNCONDITIONAL	2-4

CALL WAITING/CAMP-ON	2-5

DIRECTED CALL PICK UP	2-5

DIRECT CALL PICK UP IN ICM BOX	2-6

DO NOT DISTURB (DND)	2-6

EXTERNAL PAGING FOR QSIG/DID/DISA	2-6

FLASH ON ICM CALL	2-7

GROUP CALL PICK UP	2-7

HEADSET/SPEAKERPHONE MODE	2-7

HOT LINE/WARM LINE	2-7

INTERCOM BOX	2-8

INTERCOM CALL	2-8

INTERCOM CALL ON HOLD	2-9

INTERCOM SIGNAL MODE	2-9

INTERCOM STEP CALL	2-9

INTERCOM TENANCY GROUP	2-10

LINKED STATION PAIR	2-10

LOCK-OUT FOR ONLY INTERCOM	2-10

MEET ME PAGE	2-11

MSN/SUB-ADDRESSING	2-11

OFF-HOOK RINGING	2-11

ONE-TIME DND	2-11

ON-HOOK DIALING	2-12

PAGING	2-12

PAGE ACCESS RESTRICTION	2-13

PAGING CONFERENCE GROUP	2-13

SERIAL CALLING	2-13

2B PHONE	2-13

3 . SYSTEM FEATURES	3-1

ACCOUNT CODE	3-1

ADVICE OF CHARGE	3-1

ALARM	3-2

AUTHORIZATION CODE (PASSWORD)	3-2

AUTO CALL RELEASE	3-3

AUTOMATIC CALL DISTRIBUTION	3-3

AUTOMATIC PRIVACY	3-4

AUTO RELEASE OF [MON] BUTTON	3-4

CALL COST DISPLAY	3-4

CANNED TOLL RESTRICTION	3-4

CCR (CUSTOMER CALL ROUTING)	3-5

CHANGE KEYSET RING CADENCE	3-6

CLASS OF SERVICE (COS)	3-6

CO MESSAGE WAITING	3-7

CONFERENCE	3-7

DATA LINE SECURITY	3-8

DELAYED CO LINE RING	3-8

DIAL BY NAME	3-8

DID (DIRECT INWARDS DIALING)	3-9

DIFFERENTIAL RING	3-9

DISA(DIRECT INWARD SYSTEM ACCESS)	3-10

DUMMY DIAL TONE	3-10

DURATION TIMER	3-10

EMERGENCY CALL	3-10

ENHANCED DIAL BY NAME	3-11

FLEXIBLE DID ON QSIG FOR AUTO REROUTING	3-11

FUNCTION BUTTON ASSIGNMENT	3-12

LCD MESSAGES (CUSTOM: 00, 11~20)	3-12

LCD MESSAGES-PRESELECTED	3-12

LEAST COST ROUTING (LCR)	3-13

LOUD BELL CONTROL (LBC)	3-15

MESSAGE WAITING / CALLBACK	3-15

MEMORY FULL WARNING OF VM SYSTEM	3-16

MOH CHANNEL VIA SLT PORT	3-16

MUTE	3-17

RETRIEVING PARKED CALL WITH QSIG	3-17

SMDR	3-17

SMDR - LOST CALL PRINTING	3-18

SPEAKERPHONE	3-18

STATION GROUP:          CIRCULAR/TERMINAL/UCD/RING/VM	3-19

TEMPORARY STATION COS CHANGE	3-20

TRAFFIC ANALYSIS	3-21

UNIVERSAL NIGHT ANSWER (UNA)	3-22

VOICE ANNOUNCEMENT	3-22

VOICE OVER	3-25

WAKE-UP	3-25

WALKING COS	3-26

4 . ATTENDANT FEATURES	4-1

ATTENDANT	4-1

ATTENDANT CALL/QUEUING	4-1

ATTENDANT CLOCK SET	4-1

ATTENDANT DISABLE OUTGOING ACCESS	4-2

ATTENDANT INTRUSION	4-2

ATTENDANT OVERRIDE	4-2

ATTENDANT RECALL	4-3

DAY/ON-DEMAND/NIGHT MODE	4-3

DSS/DLS CONSOLES	4-4

FLEXIBLE ATTENDANT	4-4

INTERCOM BOX MUSIC SELECT	4-4

RESTORING/BLOCKING DID CALL	4-5

STATION FEATURE CANCEL	4-5

5 . SLT FEATURES	5-1

ABILITY FOR SLT TO PROGRAM NAME	5-1

BROKERS CALL	5-1

CALL FORWARD	5-1

CALL TRANSFER	5-2

CALL WAITING / CAMP-ON	5-2

CO LINE QUEUING	5-2

DO NOT DISTURB	5-3

HOWLING TONE	5-3

MESSAGE WAIT/CALL BACK	5-3

MESSAGE WAITING INDICATION	5-3

PICK-UP (DIRECT/GROUP)	5-4

SIGNALING	5-4

SPEED DIAL	5-4

6 .  INDEX	6-1

# . CO LINE FEATURES

## AUTO CALL NUMBER REDIAL

Description

A user can direct the system to continually redial a number which returned busy or when no answer was obtained. The system will retry the number based on programming with appropriate pauses in between dialing. (default: 3 times)

Operation

To use ACNR while receiving busy/no answer on a CO line,

- Press the [REDIAL] button.

- Replace the handset or go on-hook.

- The system will automatically retry the call at programmed intervals.

To cancel,

Press the flashing [REDIAL] button.

Admin Programming

- ACNR Pause Timer (PGM 43-BTN 14)

- ACNR Delay Timer (PGM 43-BTN 15)

- ACNR Tone Detect Timer (PGM 43-BTN 16) – applicable to analog CO line only

- ACNR No Answer Timer (PGM 43-BTN 17)

- ACNR Retry Counter (PGM 43-BTN 18)

- ACNR Tone Cadence (PGM 68)

- for normal LCD keyset

Condition

- A keyset which doesn't have [REDIAL] button, should be programmed with a [REDIAL] Flexible Button to use ACNR.

[TRANS/PGM] + Flex. BTN + [TRANS/PGM] +  8 8 + [HOLD/SAVE]

- The analog CO lines in the system should be equipped with Call Progress Tone detection Units (CPTU).

## AUTOMATIC HOLD

Description

When a keyset user places an intercom call using a DSS key during connecting with an active CO line call, it is automatically placed on holding state. In addition, the station can be programmed to support CO line to CO line automatic hold. In this case, when the user of a keyset selects a CO line while on a CO line, the new CO line is accessed and the previous CO line is placed on the preferred hold selection.

Operation

To use Automatic Hold while on a CO line call,

- Press a {CO LINE} button.

- When the CO line is connected, previous selected CO line is on preferred hold state.

Admin Programming

- Automatic Hold (PGM 11-BTN 4)

- Hold Preference (PGM 40 - BTN 1)

Condition

- The number of automatically hold CO line is not limited.

## AUTOMATIC PAUSE INSERTION

### Description

The system will automatically insert a timed pause in a speed dial or redial number, if a flash or PABX access code is included in the number.

Admin Programming

- Pause Timer (PGM 43-BTN 7)

## CALL PARK

Description

A user can park a call in a parking location and retrieve the call by dialing the location number. Also, the user may make a page announcement for the desired user to pick-up the call. If the call remains unanswered for the Call Park Recall time, the station which parking the call receives recall. If the call is still unanswered, the attendant will also receive recall. If unanswered in the Attendant Recall time, the CO line call will be disconnected and returned to idle.

Operation

To park a call while on a CO line call,

- Press the [TRANS/PGM] button.

- Dial the parking location.

GDK-162:  601~619

GDK-100:  601~610

GDK-FPII:  601~605

- Replace the handset or go on-hook

- If desired, page the party to retrieve the call.

To retrieve the parked call from a keyset,

- Lift the Handset or press the [MON] button.

- Dial the parking location.

Admin Programming

- I-Recall Timer (PGM 43-BTN 4)

- Attendant Recall Timer (PGM 43-BTN 5)

- Call Park Recall Timer (PGM 43-BTN 11)

## CALL TRANSFER

Description

A user can transfer calls on CO lines to other stations in the system. If the receiving station does not answer the call in the Transfer Hold Recall time, the transferring station will receive a recall. If the call remains unanswered, the attendant will also receive a recall for the Attendant Recall time. After that the CO line will be disconnected and returned to idle.

Operation

To make an unannounced call transfer,

- Press the [TRANS/PGM] button.

- Intercom dial tone is heard and the CO line call is placed on exclusive hold.

- Dial the station number to receive the transfer.

- Replace the handset or go on-hook.

To make an announced call transfer,

- Press the [TRANS/PGM] button.

- Intercom dial tone is heard and the CO line call is placed on exclusive hold.

- Dial the station number to receive the transfer.

- When the station answers, announce transfer.

- Replace the handset or go on-hook.

Admin Programming

- No Touch Answer (PGM 12-BTN 4)

- Transfer Recall Timer (PGM 43-BTN 3)

- I-Hold Recall Timer (PGM 43-BTN 4)

- Attendant Recall Timer (PGM 43-BTN 5)

## CALLING LINE IDENTIFICATION PRESENTATION (CLIP)

### Description

When an ISDN CO ring with CLI  (Calling Line Identification) is received, the CLI information will be printed to the RS-232C port and displayed on the LCD of ringing station.

Operation

When a DID CO ring is received,

- A CO line ring is received with ISDN.

- While ringing, the CLI (Calling Line Identification) or the name is displayed if the calling number and name is stored on a speed dial bin.

When the external party hangs up before it is answered,

- The CLI will be stored into the original party.

To see the stored CLI Message,

- Press [CALLBK] button.

To answer the CLI message,

- Press [HOLD/SAVE] button after checking the message by pressing [CALLBK] button.

To erase the CLI messages,

- Press [CONF] button after checking the message by pressing [CALLBK] button.

To erase all CLI messages at a station,

- Press the [CALLBK] button.

- Press the [SPEED] button twice.

To erase all CLI messages of other stations at a system attendant,

- Press [TRANS/PGM] button.

- Dial 5 9.

- Enter the station range.

- Press [HOLD/SAVE] button.

### Admin Programming

- CLIP (to RS232 Port) - System Base (PGM 41-BTN 6)

- CLIP (on the LCD of keyset) - Station Base (PGM 23-BTN 7)

- CLI Name Display (on the LCD of keyset) (PGM 23-BTN 11)

- Prefix Code Insertion (PGM 08-BTN 1)

- CO MSG Wait – Station Base (PGM 23 – BTN 6)

Condition

- The MEMU should be installed in the system.

- Max. 15 digits will be displayed as a CLI number.

- When the power of system is off, the stored CLI messages will not be erased.

- If DVU message is stored together with CLI message, all CLI messages will not be deleted.

- Only the CLI message in idle stations may be deleted by system attendant.

## CO CALL DROP TIMER

Description

This feature is used to restrict an outgoing CO call time. Setting the CO Call Drop Timer, the call will be disconnected automatically when the timer is expired. The caller and called parties will  hear warning tone 15sec before the CO call is disconnected.

Admin Programming

- CO Call Drop Timer Enable (PGM 21-BTN 7)

- CO Call Drop Timer

(PGM 43-BTN 33: GDK-162

BTN 28: GDK-100/FPII)

- 3-min Warning Tone (PGM11-BTN 2)

Condition

- This feature can be assigned on a station basis.

- It is applied to just outgoing CO call.

- If the CO Call Drop timer enabled in a station, the timer is still applied even a call is transferred to another station.

- On the add-on conference, the CO Call Drop timer enabled station will be restricted to the outgoing CO call time.

- The CO Call Drop timer is not released even the caller holds, transfers, or flashes the CO line.

## CO CALL TRANSFER TO CO LINE

Description

A station may transfer a CO line call to another CO line. If an external party on ISDN does not answer the transferred call within Transfer Hold Recall time, the transferring station will receive recall ring. If the call remains unanswered, the attendant will also receive recall ring for Attendant Recall time. After that, the CO line will be disconnected and returned to idle state.

Operation

Unscreened CO Line Call Transfer;

- Press the [TRANS/PGM] button.

- Intercom dial tone is heard and the CO line is placed on exclusive hold state.

- Seize a CO line and dial the number of external party to be received the call.

- Replace the handset and go on-hook.

Screened CO Line Call Transfer;

- Press the [TRANS/PGM] button.

- Intercom dial tone is heard and the CO line is placed on exclusive hold state.

- Seize a CO line and dial the number of external party to be received the call.

- When the called party answers, announce the call transfer.

- Replace the handset and go on-hook.

Admin Programming

- System Timer -I: Transfer Hold Recall Timer (PGM 43-BTN 3)

- System Timer-I: I-Recall Timer (PGM 43-BTN 4)

- System Timer-I: Attendant Recall Timer (PGM 43-BTN 5)

- Open Loop Detect Timer (PGM 34)

Condition

- For this feature, CO lines (transferred CO line and transferring CO line) must be able to detect loop lost or disconnection condition.

- If the transferred line is analog CO, recalling will not be presented when it is not answered. And, the call will be disconnected after conference timer (PGM 43-BTN 13).

- If the transferred line is ISDN CO, recall ring will be presented to the CO line after Transfer Hold Recall time while the line is released.

## CO FLASH

Description

While on a CO line, a user can request new dial-tone to place a new call or, when behind a PABX, activate a PABX feature. The system allows to each CO line to be assigned the type of flash and a flash duration.

Operation

To generate a flash while on a CO line;

Press the [FLASH] button.

To generate a flash while on a CO line from a SLT;

Press the hook-switch slightly and dial 5 1.

Admin Programming

- Flash Type (PGM 31-BTN 4)

- CO Flash Timer (PGM 33)

- Hook-switch Flash Timer (PGM 44-BTN 2)

- Hook-switch Bounce Timer (PGM 44-BTN 3)

Condition

- The Flash command is not activated in ISDN CO line.

## CO LINE ACCESS

Description

Stations are permitted to access to outgoing CO lines based on CO Line Group Access programming. Keysets may use flexible buttons assigned to access a CO line or a CO line Group (Pool Flexible button) and loop flexible buttons. If allowed, stations without individual CO line button can access individual CO lines using dial codes.

Operation

To access a CO line from a keyset,

- Lift the Handset or press the [MON] button.

- Press the desired CO line, Pool, or Loop button.

- Or, dial the individual CO line access code, CO group access code, or the first CO line access code from the accessible group.

To access a CO line from a SLT,

- Lift the handset.

- Dial the individual CO line, CO group access code, or the first CO line access code from the accessible group.

Admin Programming

- CO line Access Mode in CO Group

(PGM 42-BTN 2)

- Inter-digit Timer (PGM 44-BTN 9)

- CO Group Access (PGM 15)

- CO Line Group Assign (PGM 30-BTN 1)

- Individual CO Line Dial-up Access (PGM 12-BTN 3)

- CO Group Dial-up Access (PGM 24-BTN 4)

- First CO Override

(PGM 43-BTN 27: GDK-100/FPII

BTN 30: GDK-162)

- Station COS (PGM 14)

Condition

- A keyset should have an idle appearance (CO line/Pool button/Loop button) to access an outgoing CO line/incoming CO line.

- To assign the {LOOP} button,

[TRANS/PGM] + a Flex. BTN + [TRANS/PGM] + 8 4 + [HOLD/SAVE]

## CO LINE GROUPS

Description

The system provides grouping of similar types of CO lines. The groups are used for common access by stations assigned to access the groups. CO lines are selected from the group based on the assigned method, round robin or the last line, in order to minimize call collision (glare).

Operation

The number of CO line group in each system is as follows;

GDK-162: 00~49

GDK-100: 00~25

GDK-FPII: 00~10

The first CO line group in each system (00) is directed line group (it can be used with CO button) and the last CO line group (49, 25, 10) is unassigned line group.

Admin Programming

- CO Line Group (PGM 30-BTN 1)

- CO Line Group Access (PGM 15)

- CO line Access Mode in CO Group

(PGM 42-BTN 2)

## CO LINE NAME DISPLAY

Description

While a CO line call is received, the CO line number is displayed on the LCD. When the CO line name is assigned and the CO line is assigned to CO line Name Selection, the CO line name is displayed instead of the CO line number.

Admin Programming

- CO line Name Assignment (PGM 74)

- CO Line Name Selection -

CO line Base (PGM 30-BTN 6)

Station Base (PGM 24-BTN 1)

Condition

- Max. 10 CO line names can be assigned in the system and each CO line name has max. 12 characters.

- If both station base and CO line base are selected, the CO line name is displayed according to the station base programming (PGM 24).

## CO LINE QUEUING

Description

When a user encounters a busy signal during accessing a CO line, the user can request a call back (queue) when the CO line is available. The station will receive a call back when the busy CO line or a CO line in the CO line group becomes available.

Operation

To activate queuing while receiving busy tone,

Press the [CALLBK] button.

Admin Programming

- CO Line Queuing (PGM 11-BTN 1)

## CO LINE RING ASSIGNMENT

Description

Each station in the system can be programmed to provide an audible signal when the system detects an incoming call on assigned CO lines. Separate ring assignments are made for day, night and on-demand mode operation. Also, the audible signal at the station can be delayed by 1 to 9 ring cycles allowing other stations to answer the call first.

Operation

The LED of [DND/FOR] button and  LCD on the attendants’ keyset indicate current ring type;

Day mode (LED off, LCD shows station number and ICM answer mode.) -> On-demand mode (LED is flashing, LCD shows ‘ON DEMAND RING SERVICE’.) -> Night mode (LED on, LCD shows ‘NIGHT RING SERVICE’)

To change Night mode,

Press the [DND/FOR] button at system attendant.

To change On-demand mode,

- Press the [DND/FOR] button at system attendant in day mode.

- If there is any station assigned to ring in on-demand mode at the system, the [DND/FOR] button is flashing and it is changed to on-demand mode.

Admin Programming

- CO Line Ring Assignment (PGM 32)

Condition

- The MEMU is need for On-demand Mode.

## CO LINE RELEASE GUARD TIME

Description

When a station is released from a CO line, the CO line is held busy to all stations to allow the Central Office equipment time to return the CO line to an idle condition. During the guard time, the system sends an idle condition to the CO and busy status is provided to system users.

Admin Programming

- CO Line Release Guard Timer

(PGM 43-BTN 8)

Condition

- The CO line is considered to be busy in CO line Release Guard Time.

- The CO line Release Guard time is not applied to analog DID line.

## CONNECTED LINE IDENTIFICATION PRESENTATION (COLP)

Description

If COLR/CLIR is not activated, the system will send COLP information to PX when a called party answers according to the type of calling party number.

Admin Programming

- My Area Code (PGM 08-TN 2)

- Type of Calling Party Number (PGM 08-BTN 6)

- COLR - Station base (PGM 24-BTN 2)

- CLIP or Outgoing COLP (PGM 37-BTN 1)

- ISDN DID Table (PGM 09)

- Flexible DID Table (PGM 05) – This is only applicable when the DID conversion type is Type 2.

- Prefix Code Insertion for Outgoing call (PGM 08-BTN 8)

Condition

- The MEMU should be installed in the system.

- It is applied when the CO line type is ISDN.

(BRI, PRI, STIB)

- To use outgoing CLI (PGM 23-BTN 10) or COLP (PGM 37-BTN 1), the DID digits should not be changed in PGM 55.

## CONNECTED LINE IDENTIFICATION RESTRICTION (COLR)

Description

The COLP (Connected Line Identification Presentation) may be restricted with station base. When making a call, your phone number will not be presented to the other party.

Operation

To assign {COLR} BTN at a flexible button,

- Press [TRANS/PGM] button.

- Press a flexible button.

- Press [TRANS/PGM] button.

- Dial 5 8.

- Press [HOLD/SAVE] button.

To activate COLR,

- Press assigned {COLR} button. Or,

- Change the Admin programming (PGM 24-BTN 2)

Admin Programming

- COLR (PGM 24-BTN 2)

## CO RING DETECT – Analog CO only

Description

To detect ring cadence from a CO line, there are two timers; Ring Detect and Ring Stop allow for flexible ring detection on an incoming CO line call.

Admin Programming

- CO Line Ring Detect (PGM 43-BTN 6)

- CO Line Ring Stop (PGM 44-BTN 12)

Condition

- The CO line ring should make ring over CO Ring Detect time and should be detected continuously in CO line Ring Stop time.

- The CO line ring detect time or CO line ring stop time cannot be assigned to each CO line. (System Base)

## CO STEP CALL – Analog CO only

Description

When a station receives a busy tone after placing a CO line call, the user can dial a CO line number which has the same first digits as the called busy CO line without dialing the full number.

Operation

To use CO Step Call when receiving busy tone,

If the next CO line to be called has the same first digits as the called busy CO line in the system, press the [SPEED] button and the last digit of the next CO line.

## DIAL MEMO

Description

A user can dial and store a number while on a CO line or intercom call. The number can then be accessed later without dialing the number manually.

Operation

To store a Dial Memo number while on a call from a keyset;

- Press the {DIAL MEMO} flexible button.

- Dial the number to be stored.

- Press the {DIAL MEMO} flexible button.

To dial a stored Memo Dial number from a keyset;

- Lift the Handset or press the [MON] button.

- Press the [SPEED] button.

- Dial  #.

Condition

- To assign the {DIAL MEMO} button at a Flex. BTN.

[TRANS/PGM] + Flex. BTN + [TRANS/PGM] + 8 0 + [HOLD/SAVE]

- It is not available at a SLT.

- Dial Memo and Save Number Redial (SNR) use the same memory buffer. If a dial memo is stored, the number of SNR is deleted automatically.

## DIAL PULSE SENDING – Analog CO only

Description

Each CO line is programmed to provide the appropriate signaling to the Central Office. A CO line can be programmed to send pulse signal at 10 or 20pps with a break/make ratio of 60/40 or 66/33. Digits dialed by the user are converted to the proper signal for the CO line and sent over the CO line.

Operation

Dialing a phone number, it is changed to Dial Pulse signal and the phone number is dialed.

Admin Programming

- Pulse Dial Speed Ratio (PGM 57)

- Pause Timer (PGM 43-BTN 7)

- CO Line Signal Type (PGM 31-BTN 2)

Condition

- Pause time will be inserted automatically before sending the dial pulse.

## DIAL PULSE TO TONE SWITCH OVER

Description

On a pulse dial CO line, the user can request that the system switch the signaling mode from pulses to DTMF signals.

On an ISDN CO line, it works the same way except that the DTMF sending is delayed until the called party answers the call.

It allows the user to access outside facilities that require incoming DTMF signals such as banking services, etc.

Operation

To switch from pulse to DTMF while on a pulse CO line;

- Dial ＊.

- Dial digits as required.

Admin Programming

- CO Line Type (PGM 31-BTN 2)

Condition

- This command can be used in the CO line of pulse signal mode. After this command, pause is inserted and '＊' is not dialed in CO line.

## DISPLAY SECURITY

Description

The dialed phone number may not be displayed in the LCD while on a speed dial.

Operation

Pressing ＊ at the first digit in a speed dial, it is operated with display security.

## DISTINCT CO RING

Description

The system provides different ring type in each CO line. Four ring types are available allowing users to distinguish which CO line is ringing.

Admin Programming

- CO Line Attribute-4: CO Distinct Ring Frequency (PGM 38-BTN 1)

- Distinct Ring Frequency (PGM 76)

## DTMF SIGNAL SENDING

Description

When a CO line is assigned as DTMF signal in the Admin. Programming, the system may send DTMF signal to the CO line.

Operation

When placing a CO line call at a station, it is dialed with DTMF signal mode.

Admin Programming

- Inter-digit Timer (PGM 44-BTN 9)

- CO Line Signal Type (PGM 31-BTN 2)

## EXCLUSIVE HOLD RECALL TIMER

Description

CO lines can be placed in a waiting state such that other stations in the system will be unable to access the CO line. Only the station placing the CO line on exclusive hold can access the CO line.

When placed on exclusive hold, the Exclusive Hold Recall Timer is initiated. If the CO line is on hold after the timer is expired, the station which placed the call on hold will receive a recall. The station will receive the recall for the I-Hold Recall duration after which, the attendant will also receive a recall. If the call remains unanswered for the Attendant Recall Time, the CO line will be returned to an idle state and recalls are terminated.

Operation

To place a CO line on Exclusive Hold from a keyset,

Press the [HOLD/SAVE] button twice.

To access a CO line on Exclusive Hold from the holding keyset,

- Lift the handset.

- Press the CO line button.

- Or dial 8 # and the CO line number.

Admin Programming

- Hold Preference (PGM 40-BTN 1)

- Exclusive Hold Recall Timer (PGM 43-BTN 1)

- System Hold Recall Timer (PGM 43-BTN 2)

- Transfer Recall Timer (PGM 43-BTN 3)

- I-Hold Recall Timer (PGM 43-BTN 4)

- ATD Recall Timer (PGM 43-BTN 5)

## EXECUTIVE/SECRETARY TRANSFER

Description

Stations in the system can be assigned as Executive/Secretary pairs. Intercom and transferred calls to the executive station will be transferred to the secretary station if the executive is in DND state.

The max. number of Executive/Secretary pairs are as follows;

GDK-162: 24

GDK-100: 12

GDK-FPII: 6

Operation

To activate Executive/Secretary Transfer from the Executive's keyset,

Press the [DND/FOR] button.

Admin Programming

- Do Not Disturb (PGM 10-BTN 1)

- Executive/Secretary Transfer (PGM 49)

Condition

- When both Executive and Secretary are busy, camp-on / transferred calls / messages is remained at the Secretary station.

- Secretary can pass a call to Executive when in DND state.

## HOLD PREFERENCE

Description

A user can place a CO line call in a waiting state so that others may or may not access the held CO line, System and Exclusive Hold respectively. When the CO line is placed on hold using [HOLD /SAVE] button, the first depression of the button places the CO line in the assigned preferred holding state. A subsequent depression of [HOLD /SAVE] button will change the CO line to the non-preferred holding state.

Operation

- When the Exclusive Hold is assigned,

Other station in the group cannot seize the CO line.

- When the System Hold is assigned,

Other station in the group can seize the CO line and answer.

Admin Programming

- Hold Preference (PGM 40-BTN 1)

## KEYPAD FACILITY SERVICE

Description

The system enables users to send Keypad Facility IE (Information Element) instead of called party IE for sending information on ISDN interface to the network when pressing the programmed button as {Keypad Facility}. Some specific features to be supported by network can be operated by dialing each code at the station.

Operation

- When pressing the programmed {Keypad Facility} button after seizing a CO line (ISDN), the system will send the dialed digits as Keypad Facility IE instead of Called Party Number IE in INFORMATION message to the network.

- When a ISDN terminal user sends "#" using Keypad Facility IE with subsequent digits after seizing a CO line (ISDN), the system will send all dialed digits as Keypad Facility IE to network even if the codes are matched with internal supplementary service code (Ex. "*#" to hold CO line).

When the user releases the CO line (ISDN), or pressing the programmed {Keypad Facility} button during dialing digits, Keypad Facility IE Sending will be stopped automatically.

Admin Programming

- Keypad Facility Access Authority (PGM 24-BTN 5)

Condition

- To assign {Keypad Facility} flexible button,

[TRANS/PGM] + Flex. BTN + [TRANS/PGM] + 6 2 + [HOLD/SAVE]

- The Keypad Facility Access Authority (PGM 24-BTN 5) should be ON to use this feature.

- This feature can be activated by pressing the {Keypad Facility} button after seizing a CO line (ISDN).

- When this feature is activated, the system starts to send Keypad Facility IE instead of Called Party Number IE regardless of CONNECT message from the network unless pressing {Keypad Facility} button to deactivate it. So the system will continue to send Keypad Facility IE instead of DTMF tone even after CONNECT message is received from the network.

- If the Speed Dial is stored with [FLASH] as the first digit for ISDN line and the station has the Keypad Facility Access authority, the digits after [FLASH] command will be sent as Keypad Facility and the feature will be deactivated after sending all digits.

- When any of the following features are encountered after the Keypad Facility is activated, the feature will be deactivated automatically;

- Pressing function keys as [REDIAL], [TRANS/PGM], [FLASH], [HOLD/SAVE]

- Pressing CO keys as CO, CO Group, Loop

- Pressing Flex. Buttons assigned some features as {SPEED}, {DSS}, {Call Park}, {Keypad Facility}

- Making On-hook

- When DISCONNECT message is received from the network

- This feature may not be operated depending on the services that require to handle two call references at the same time after B channel connection is made as Hold & Retrieving, Conference, etc.

- When the system receives a Called Party Number IE during sending Keypad Facility IE, the system will stop sending the Keypad Facility IE.

## LAST NUMBER REDIAL (LNR)

Description

The last dialed number with CO line can be stored (up to 48 digits) in the station's Last Number Redial buffer. The user may request the system redial the last number without redialing the full number.

Operation

To use Last Number Redial,

- Lift the handset or press the [MON] button.

- Press the [REDIAL] button.

- Or, press the [SPEED] button and dial *.

Condition

- When the used CO line is busy, an idle CO line in the group is seized and the last dialed number is dialed.

## MUSIC ON HOLD (MOH)

Description

When a CO line call is placed in the hold state (system, exclusive, transfer, conference, etc) the external party will hear music. In this way, the CO line party can be notified that the connection is still established.

If provided, each music source can be assigned to each CO line base.

Admin Programming

- CO Line MOH (PGN 38-BTN 2)

- MOH Channel (PGM 41-BTN 2)

- Music Source on Channel 1 (PGM 41-BTN 4)

- Assign MOH via SLT (PGM#1-BTN 2)

Condition

- In GDK-162/100, the internal music source is in MPB and the external music source is in MISB.

## PAUSE INSERTION IN SPEED DIAL

Description

While dialing a speed number, a timed pause between digits sending can be placed in the number. The length of this pause can be programmed by the Admin. Programming.

Admin Programming

- Pause Timer (PGM 43-BTN 7)

## PREFERRED LINE ANSWER

Description

By lifting the handset or pressing the [MON] button, an incoming call is automatically answered and connected to the station.

Admin Programming

- Preferred Line Answer (PGM 10-BTN 6)

Condition

- CO line call is preferred to intercom to answer.

- The preference of CO line call:

CO line recalling -> CO line transfer -> Incoming CO line ring -> Queued CO line ring

- The CO line is preferred to Hot Line/Warm Line to answer.

## PRIVATE LINE

Description

CO lines in the system can be assigned for exclusive use by one or more keyset users. Private lines are assigned to CO line Group 00 and an appearance (Flexible CO button) is required at the keyset. (Loop or Pool kyes cannot be used.)

Operation

A private line will operate as a normal CO line except access is limited to assigned stations.

Admin Programming

- CO Line Group Assign (PGM 30-BTN 1)

## SAVE NUMBER REDIAL

Description

The current number while a call can be stored in a temporary buffer for future redial. The number is saved in memory until the user stores a new number or the Dial Memo feature is used. The number in the Save Number Redial is not erased when a new number is dialed.

Operation

To save a number in the Save Number Redial buffer from a keyset,

- Press the [SPEED] button twice.

- Replace the handset and go on-hook.

To dial a number from the Save Number Redial buffer from a keyset,

- Lift the handset or press the [MON] button.

- Press the [SPEED] button.

- Dial # button.

Condition

- When the used CO line is busy, an idle CO line in the group is seized and the saved number is dialed.

- The stored save number is not deleted at when the system power is off.

- Dial memo and Save Number Redial use the same memory buffer. If dial memo is stored, the previous stored save number is deleted.

## STATION SPEED DIAL

Description

Each station user can program up to 20 individual speed dial numbers in the station. The numbers are entered and stored by the user and can be recalled by the user at any time.

Space is allocated in the system's memory for up to 200 (GDK-FP II)/500 (GDK-100)/1000(GDK-162) Station Speed Dial numbers with each bin containing up to 24 digits.

Operation

To place a call using Station Speed Dial from a keyset,

- Lift the handset or press the [MON] button.

- Press the [SPEED] button.

- Dial the station speed dial bin (00~19).

To store Station Speed Dial numbers from a keyset,

- Press the [TRANS/PGM] button.

- Press the [SPEED] button.

- Dial the station speed dial bin (00~19).

- If desired, press the CO line or group button.

- Dial the desired telephone number (up to 24 digits).

- Press the [HOLD/SAVE] button.

- If desired, enter the name (Max. 12 characters) by dialing 2-digit for each character.

- Press the [HOLD/SAVE] button.

*. To store continuously, repeat the above procedure from 3.

Admin Programming

- Speed Dial Access (PGM 10-BTN 2)

- Dial Tone Detect (PGM 42-BTN 5)

Condition

- CPTU should be installed to dial tone detect.

- The Station Speed Dial is secured in data protect mode when the power is off.

- There can be max. 24 digits in a system speed dial number including some digits and special function codes.

Special function codes

- [CALLBK] button – Pause

- ‘*’ – If it is stored as the first digit, its function is Display Security. Otherwise, its function is Pulse to DTMF Switchover.

- [DND/FOR] button – If PGM 42-BTN 5 is ON and it is stored as the first or second digit and the seized CO line is behind PBX mode, its function is Dial Tone Detect. Otherwise, its function is ‘*’.

- [FLASH] button – If seized CO line is analog line, its function is Flash to PX (or PBX). If seized CO line is ISDN line and it is stored as the first digit, it makes remaining digits sent with enveloped not in calling party number IE but in keypad facility IE.

## SYSTEM HOLD RECALL TIMER

Description

CO lines can be placed in a waiting state such that other stations in the system can access the CO line. The station attempting to seize the held CO line should have allowance to access the CO line.

When placed on System Hold, the System Hold Recall Timer is initiated. If the CO line is on hold after expiration of the timer, the station which placed the call on hold will receive a recall. The station will receive the recall for the I-Hold recall duration after which the attendant will also receive a recall signal. If the call remains unanswered for the Attendant Recall Time, the CO line will be returned to an idle state and recall is terminated.

Operation

To place a CO line on System Hold,

Press the [HOLD/SAVE] button.

To access a CO line on System Hold from a keyset,

- Lift handset or press the [MON] button.

- Press the CO line button. (or Loop/Pool key)

- Or, dial 8 # and the CO line number.

To access a CO line on System Hold from a SLT,

- Lift the handset.

- Dial 8 # and the CO line number.

Admin Programming

- Hold Preference (PGM 40-BTN 1)

- Exclusive Hold Recall Timer (PGM 43-BTN 1)

- System Hold Recall Timer (PGM 43-BTN 2)

- Transfer Recall Timer (PGM 43-BTN 3)

- I-Hold Recall Timer (PGM 43-BTN 4)

- ATD Recall Timer (PGM 43-BTN 5)

## SYSTEM SPEED DIAL

Description

The system speed dial bins are programmed by attendant. These numbers are available for easy access by all stations allowed in the system.

GDK-162: 200~999

GDK-100: 200~399

GDK-FPII: 200~399

Operation

To store a number in a System Speed Dial from the system attendant,

- Press the [TRANS/PGM] button.

- Press the [SPEED] button.

- Dial the system speed dial bin.

- If desired, press the CO line or Group button.

- Dial the desired phone number (max. 24 digits)

- Press the [HOLD/SAVE] button.

- If desired, enter the name (Max. 12 characters) by dialing 2-digit for each character.

- Press the [HOLD/SAVE] button.

*. To store continuously, repeat the above procedure from 3.

To place a call using System Speed Dial from a keyset,

- Lift the Handset or press the [MON] button.

- Press the [SPEED] button.

- Dial the System Speed Dial bin.

Admin Programming

- Speed Dial Access (PGM 10-BTN 2)

- System Speed Zone Access (PGM 67)

- Dial Tone Detect (PGM 42-BTN 5)

Condition

- The CPTU should be installed for dial tone detect.

- System Speed Dial is restricted by Station COS.

- There can be max. 24 digits in a system speed dial number including  some digits and special function codes.

Special function codes

- [CALLBK] button – Pause

- ‘*’ – If it is stored as the first digit, its function is Display Security. Otherwise, its function is Pulse to DTMF Switchover.

- [DND/FOR] button – If PGM 42-BTN 5 is ON and it is stored as the first or second digit and the seized CO is behind PBX mode, its function is Dial Tone Detect. Otherwise, its function is ‘*’.

- [FLASH] button – If seized CO line is analog line, its function is Flash to PX (or PBX). If seized CO line is ISDN line and it is stored as the first digit, it makes remaining digits sent with enveloped not in calling party number IE but in keypad facility IE.

## SYSTEM SPEED ZONE GROUP

Description

Up to 10 speed number zones can be defined. Speed bins & stations ca be allocated to these zones. Toll checks based on COS can be applied to zones. Only stations allocated to zones can access these bins. Speed bins not allocated to zones can be accessed by all stations and no toll checks are applied.

GDK-162: 400~999

GDK-100: 300~399

GDK-FPII: 300~399

Admin Programming

- Speed Dial Access (PGM 10-BTN 2)

- System Speed Zone Access (PGM 67)

- Dial Tone Detect (PGM 42-BTN 5)

## WORLD TIME DISPLAY

Description

System will display the local time of dialed area instead of the line number when a user tries to an international call. System allows max. 20 nation's local time and temperature.

Admin Programming

- World Time Attribute (PGM 71)

- World Time (PGM 72)

# . INTERCOM FEATURES

## BACKGROUND MUSIC

Description

A keyset can receive audio, generally music, from an internal or external source while idle. Music from the source is received over the keyset speaker and will be shut-off during ringing, paging announcement or when the station is off-hook.

Operation

To assign background music at a keyset,

- Press the [TRANS/PGM] button.

- Dial 7 3 and the music is received.

- Pressing the [TRANS/PGM] button and dial  7 3, the selected music is changed by turns.

Channel 0 (Background music is not used.)

Channel 1

Channel 2

:

Channel 9

Admin Programming

- No. of Background Music Channel (PGM 41-BTN 1)

- MOH Channel (PGM 41-BTN 2)

- Music Source on Channel 1 (PGM 41-BTN 4)

- ASSIGN SLT MOH (PGM #1-BTN 2)

Condition

- When an external music source is assigned, the source should be connected to the proper input on the MPB (GDK-FPll) or MISB (GDK-162/100).

- The same music source can be used with MOH.

- Press the [VOLUME] button to adjust volume while the background music is heard.

- Channel 3 is not used for BGM.

## BUSY LAMP FIELD (BLF)

Description

When the user assigns a flexible button on the keyset as a Direct Station Selection (DSS) button, the associated LED with the button also provides an indication of the status of the assigned station. Such status indicators are known as a Busy Lamp Field that is; a group of lamps which indicate the busy status of stations.

## CALCULATOR

Description

A keyset with LCD has calculator function. (Addition/Subtraction/Multiplication/Division)

Operation

To activate calculator,

- Press the [TRANS/PGM] button.

- Press the [FLASH] button.

- Confirmation tone is heard and calculator is activated.

- Press [HOLD/SAVE] button to see the calculated result.

Condition

The first column of flexible button is Addition (+).

The second column of flexible button is Subtraction (－).

The third column of flexible button is Multiplication (×).

The forth column of flexible button is Division (÷).

## CALENDAR DISPLAY

Description

The calendar is displayed on the large LCD of a digital keyset.

Operation

Press the [MENU] button in standby mode.

Press the [MENU] button again to return to normal menu display.

## CALL FORWARD-BUSY/NO ANSWER

Description

A user can direct the system to re-route call to other station or group when the user's station is busy and/or does not answer in a predefined 'No Answer' time. Incoming CO line, transferred CO line and ringing Intercom calls are forwarded.

Operation

To activate Call Forward Busy/No Answer,

- Lift the handset or press the [MON] button.

- Press the [DND/FOR] button.

- Dial Call Forward Code; (2-busy only, 3-no answer only, 4-busy/no answer)

- Dial station or group pilot number to receive the calls.

- Replace the handset and go on-hook.

To deactivate Call Forward Busy/No answer,

- In idle state;

Press the [DND/FOR] button.

- In off-hook state,

Press the [DND/FOR] button and dial #.

Admin Programming

- Call Forward (PGM 10-BTN 4)

- Call FWS No Answer Timer (PGM 44-BTN 7)

Condition

- Only Unconditional Forward and CO Off-net Call Forward are available in Attendant station.

## CALL FORWARD BUTTON PROGRAM

Description

A flexible button can be programmed as {CALL FORWARD} button.

Operation

To assign {CALL FORWARD} button at a flexible button,

- Press the [TRANS/PGM] button.

- Press a flexible button.

- Dial Call Forward Code 5 4.

- Assign Call Forward Type (1~5, 7~9).

- Dial the station number to forward.

- Press the [HOLD/SAVE] button.

To activate Call Forward,

- Press the assigned flexible button.

- The LED of [DND/FOR] button is flashing and the function assigned to the flexible button is activated.

Condition

Call Forward Type

1: Call Forward, Unconditional

2: Call Forward, Busy

3: Call Forward, No Answer

4: Call Forward, Busy/No Answer

5: Station Off-net Call Forward

* : No Answer Off-net Call Forward

7: No Answer DVU Forward -1 (forwarded after 4sec)

8: No Answer DVU Forward -2 (forwarded after predefined time: PGM 44-BTN 24)

9: Busy DVU Forward

## CALL FORWARD-FOLLOW ME

Description

A user can direct the system to immediately re-route(forward) calls to another station. Call Forward, Follow-Me can be activated at the user's station or from any other station in the system allowed access to Call Forward. It must be programmed from the station that you will be forwarded to and a user password must be entered at the user's station first.

Operation

To activate Follow-me Call Forward from any station,

- Lift the Handset or press the [MON] button.

- Press the [DND/FOR] button.

- Dial Call Forward Code ‘0’.

- Dial forwarding station number.

- Dial forwarding station authorization code.

- Replace the Handset and go on-hook.

To deactivate Follow-me Call Forward from the station, (only the user's own station. not at any other station)

1.  In idle state;

Press the [DND/FOR] button.

2.  In off-hook state,

Press the [DND/FOR] button and dial #.

Admin Programming

- Call Forward Access (PGM 10-BTN 4)

- Authorization Code (PGM 54)

Condition

- Authorization code should be registered to use Follow-me Call Forward.

## CALL FORWARD-INCOMING CO LINE OFF-NET

Description

The system attendant can direct the system to re-route(forward) incoming CO line calls to a directory (telephone) number outside the system. When a call is received, the system will access an outgoing CO line and dial the telephone number assigned by the attendant. Note that the system will automatically disconnect the call after the Conference timer has expired.

Operation

To activate CO Line Off-Net Call Forward from the attendant;

- Lift the handset or press the [MON] button.

- Press the [DND/FOR] button.

- Dial Call Forward code ‘6’.

- Dial CO line/group access code (9, 801-8xx, 8801-88xx, or 8* for all CO lines)or press {CO line} button.

- Dial the Speed Dial bin number with the desired telephone number.

- Replace the Handset and go on-hook.

To deactivate CO line Off-Net Call Forward from the attendant;

- Lift the Handset or press the [MON] button.

- Press the [DND/FOR] button.

- Dial Call Forward code ‘6’.

- Dial CO line/group access code (9, 801-8xx, 8801-88xx, or 8*) or press {CO line} button.

- Dial #.

Admin Programming

- Call Forward Access (PGM 10-BTN 4)

- Unsupervised Conference Timer

(PGM 43-BTN 13)

Condition

- If there is no idle CO line, Off-net Call Forward is not activated.

- This feature is not applicable for DID line.

## CALL FORWARD-PRESET

Description

The system can be configured so that incoming CO lines, which are programmed to ring to station, can be forwarded to an assigned station if the call is not answered in a predetermined time, Preset Forward Timer.

Admin Programming

- Preset Call Forward Timer (PGM 43-BTN 2)

- Preset Call Forward (PGM 18)

Condition

- In Preset Call Forward, a busy station will not receive a CO line ring and the next assigned station will receive the CO line ring.

- The loop of Preset Call Forward is not available.

## CALL FORWARD-STATION OFF-NET

Description

Stations allowed call forward access can forward intercom and transferred CO line calls to a directory number (telephone number) outside of the system. When a call is received, the system will access an outgoing CO line and dial the telephone number entered by the user.

If a station assigned off-net call forward received a call from internal/external caller, the call is forwarded to off-net after no answer forward timer.

Operation

To activate Off-Net Call Forward,

- Lift the handset or press the [MON] button.

- Press the [DND/FOR] button.

- Dial Call Forward Code 5.

- Seize a CO line, if required.

- Dial the Speed Dial bin number with the desired phone number.

- Replace the handset and go on-hook.

To activate no answer off-net call forward,

- Lift handset or press [MON] button.

- Press [DND/FOR] button.

- Dial ＊.

- Seize a CO line, if required.

- Dial a speed bin number with the desired phone number.

- Replace the handset and go on-hook.

To deactivate Off-Net Call Forward,

1.  In idle state;

Press the [DND/FOR] button. Or,

2.  In off-hook state,

Press the [DND/FOR] button and dial #.

Admin Programming

- Call Forward (PGM 10-BTN 4)

- Call Forward No Answer Timer (PGM 44-BTN 7)

Condition

- If the announcement for call forward is recorded at DVU Announcement 20, it is provided to the caller before off-net call forward.

- To assign {No Answer Off-net Call Forward} button at a flexible button,

[TRANS/PGM] + {Flexible Button} + 5 4＊ + Speed bin no. + [HOLD/SAVE]

- No answer off-net call forward will take place with Call Forward no answer timer.

## CALL FORWARD - UNCONDITIONAL

Description

A user forward all calls immediately to another station or station group.

Operation

To activate Unconditional Call Forward,

- Lift the Handset or press the [MON] button.

- Press the [DND/FOR] button.

- Dial Call Forward Code 1.

- Dial station or group pilot number to receive the calls.

- Replace the handset and go on-hook.

To deactivate Unconditional Call Forward,

- In idle state;

Press [DND/FOR] button.

- In off-hook state,

Press [DND/FOR] button and dial #.

Admin Programming

- Call Forward (PGM 10-BTN 4)

Condition

- Call Forward cannot be assigned to a station which is in DND state.

- Assigning Call Forward while CO line queue/ Message Wait/DND/Preselected Message is activated, the previous feature will be canceled and Call Forward will be assigned.

- The Intercom Answer is 'Tone' mode in Call Forward.

## CALL WAITING/CAMP-ON

Description

When a user calls a busy station, busy tone is heard and the user can give a signal to the busy station. The busy station (off-hook or on speakerphone) is notified of the call waiting by a camp-on tone and flashing LED of the [HOLD/SAVE] button.

Operation

To activate Call Wait while receiving intercom busy tone,

Dial ＊, the last digit of the busy station, press the busy DSS flexible button, or flexible button assigned as {CALL WAIT}.

To answer a Call Wait while receiving intercom busy tone,

Press the [HOLD/SAVE] button, the active CO line is placed on exclusive hold and the call waiting is connected.

Condition

- To assign the {CALL WAIT} button at a flexible button,

[TRANS/PGM] + Flex. BTN + [TRANS/ PGM] + 8 5 + [HOLD/SAVE]

- During a conference or paging, Call Waiting is not activated.

## DIRECTED CALL PICK UP

Description

A station can pick up a call ringing other station by pressing the pick up code. Tone ringing intercom calls, incoming CO lines, recalling CO lines and transferred CO lines can be answered by a station instead of the ringing station.

Operation

To answer a call ringing at another station,

- Lift the handset or press the [MON] button.

- Dial Direct Call Pick up code 7.

- Dial the intercom number of the ringing station.

Condition

- Private line calls are not subject to Directed Call Pick-up.

- The preference of Call Pick-up

Before X.2 version:

Intercom ring ->Transferred CO ring -> CO ring -> CO recall ring

X.2 version or later:

CO recall ring -> Transferred CO ring -> CO ring -> Intercom ring

- When the same type of CO lines are queued, the first queued CO line is picked up.

- The queued CO line ring cannot be picked up.

## DIRECT CALL PICK UP IN ICM BOX

Description

A station can answer an intercom call placed to an ICM box using Directed Call Pick-up.

Operation

To answer a call ringing at another station,

- Lift the handset or press the [MON] button.

- Dial Directed Call Pick-up code 7.

- Dial the intercom number of the ICM box.

## DO NOT DISTURB (DND)

Description

A station can be programmed to activate Do-Not-Disturb (DND). While in DND, the station will not receive ring for CO lines, transferred calls, intercom calls and will not receive paging announcements. Only the attendant can call the DND station by pressing '＊' or the last digit of the station.

Operation

To activate DND from a keyset,

Press the [DND/FOR] button.

To deactivate DND from a keyset,

Press the [DND/FOR] button again.

Admin Programming

- Do-Not-Disturb (PGM 10)

Condition

- Pressing the [DND/FOR] button during call forward or preselected message, DND is not activated and call forward or preselected message is released.

- When a station, which is assigned to preset call forward, is in DND, an incoming call is received to the next station by ring assignment.

## EXTERNAL PAGING FOR QSIG/DID/DISA

Description

External user (QSIG/DID/DISA) which is assigned to have CO Page privilege, can be allowed to access page facilities when an incoming call is received to the system via DID or DISA. If GDK system is connected to another GDK system with QSIG configuration, a user in a GDK system can be allowed to access page facilities of the other system via QSIG.

Admin Programming

- CO Page Enable/Disable (PGM 38-BTN 8)

Condition

- Any station receiving paging can use Meet-me page feature as internal paging.

- If the Forced Page Index (PGM 8-BTN 9) is not set to zero, the paging will be conference paging.

- If an external caller cannot make a CO paging, the CO line will be treated as an error.

- This feature may be used for only digital CO line, not for analog ones.

- To use this feature for QSIG, the paging codes of two systems should be differentiated and programmed at QSIG routing table of each system.

- All DID types (0,1,2) are supported.

- This feature can be used for CCR.

- Other rules are the same as internal paging.

## FLASH ON ICM CALL

Description

This feature enables station users to utilize the [FLASH] button to terminate pages and intercom calls. While paging or intercom call pressing the [FLASH] button will terminate the call and return intercom dial tone.

## GROUP CALL PICK UP

Description

A station in the same pick-up group can pick up a call ringing other station in the group. Tone ringing intercom calls, incoming CO lines, recalling CO lines and transferred CO lines can be answered by a station instead of the ringing station if both are members of the same station pick-up group.

Operation

To answer a call ringing at a station in your Pick-up group,

- Lift the handset or press the [MON] button.

- Dial Group Pick-up code 6 6.

Admin Programming

- Pick-up Group (PGM 47)

- Station Group Attribute (PGM 48)

## HEADSET/SPEAKERPHONE MODE

Description

An industry standard headset can be connected to a keyset instead of handset. The station is programmed for headset operation in place of speakerphone operation.

A user can use headset instead of handset to make or answer a call.

Operation

To activate a headset call from a keyset,

Press the [MON] button.

(The LED of [MON] button lights.)

To terminate a headset call from a keyset,

Press the lighting [MON] button.

Admin Programming

- Speakerphone/Headset (PGM 11-BTN 7)

Condition

- The intercom signaling mode (HF/TN/PV) can be set in headset mode keyset as the Speakerphone mode keyset.

- To assign the {HEADSET/SPEAKERPHONE MODE TOGGLE} button at a flex button,

[TRANS/PGM] + Flex. BTN + [TRANS/PGM] +  5 7 + [HOLD/SAVE]

## HOT LINE/WARM LINE

Description

A station can be programmed to perform a pre-assigned function when the station goes to an off-hook state. The function can be activated immediately upon an off-hook state(Hot Line) or after a delay, if no other action is taken within a predefined time (Warm Line).

The following items are can be assigned as Hot Line/Warm Line function;

- Assigned function at a Flex. Button

- CO line or CO Group access

- Intercom call to a predefined station

Operation

To activate Hot Line,

- Lift the handset at a station, which is assigned to Hot Line.

- The assigned hot line feature is activated immediately.

To activate Warm Line,

- Lift the handset at a station, which is assigned to Warm Line.

- If no dialing for assigned warm line timer, the assigned warm line feature is activated.

Admin Programming

- Warm Line Timer (PGM 43-BTN 19)

- Warm Line (PGM 10-BTN 7)

- Hot Line/Warm Line (PGM 19)

Condition

- 1.  If there is no flexible button in a station, the number is operated as speed dial number.

2.  The setting value of Warm Line timer should be less than that of Dial Tone timer.

## INTERCOM BOX

Description

A convenient ICM box can be connected to the system. The ICM box can receive page announcements, intercom calls. And the ICM box can signal assigned stations in the system.

There are 2 types of ICM box; for Digital/for Analog.

Operation

To call an ICM box,

- Lift the handset or press the [MON] button.

- Dial ICM box intercom number or press the DSS flexible button for the ICM box.

- After warning tone, announce the call.

To place a call from an ICM box,

Press the [CALL] button and assigned station will ring.

To answer an ICM call at a station assigned for ICM box signals,

Press DSS flexible button for ICM box.

To place ICM box to DND mode,

Press the [DND] button.

Admin Programming

- ICM Box Signals (PGM 12-BTN 2)

- ICM Box Assignment (PGM 13-BTN 3)

- ICM Box Music Channel (PGM 41-BTN 3)

- ICM Box Call Ring Timer (PGM 44-BTN 5)

Condition

- While hearing music at ICM box, press the [DND] button to stop the music.

- ICM Box cannot attend a conference.

- The CO ring is not received at ICM box.

## INTERCOM CALL

Description

The system incorporates a non-blocking internal communications network(intercom) which can be accessed by all stations connected to the system. A station is allowed to call another station in the system or to access page zones by dialing the applicable number from the numbering plan.

Operation

To place an intercom call,

- Lift the handset or press the [MON] button.

- Dial the desired station number.

- If the called station is in the intercom ring mode, ring back tone is received.

If the called station is in the privacy or HF answer mode, intercom call announcement is received and the calling party can announce the call.

Admin Programming

- ICM Dial Tone Timer (PGM 44-BTN 8)

- Inter-digit Timer (PGM 44-BTN 9)

## INTERCOM CALL ON HOLD

Description

A digital keyset station user can place an intercom call in a waiting state (on hold) using the [HOLD/SAVE] button or the [ICM] button. The call can be retrieved only at the station that placed the intercom call on hold.

Operation

To place an ICM call on hold,

Press the [HOLD/SAVE] button or the {ICM HOLD} button.

To retrieve a held ICM call,

Press the {ICM HOLD} button, or if assigned, the DSS button associated with the held station.

Condition

To hold intercom call, the station should have {ICM HOLD} button.

To assign the {ICM HOLD} button at a flexible button.

[TRANS/PGM] + Flex. BTN + [TRANS/PGM] + 8 3 + [HOLD/SAVE]

## INTERCOM SIGNAL MODE

Description

Each keyset can select the signaling mode for incoming intercom calls. Keysets equipped with the speakerphone can select one from the 3 signaling modes as below;

HF - Call announcing with handsfree answer-back

User can answer a call automatically without lifting handset or pressing the [MON] button.

TN - Tone ringing

User should lift the handset or press [MON] button to answer.

PV - Call announcing with privacy

User can answer a call automatically without lifting handset or pressing [MON] button. But, the calling party cannot hear the voice of the called party. To talk, the called party should lift the handset.

Operation

To assign the Intercom Signal mode from a keyset,

- Press the [TRANS/PGM] button.

- Dial 4 9 and confirmation tone is heard.

- Dial the signal mode type (1 = HF/2 =TN/3 = PV).

- Press the [HOLD/SAVE] button and confirmation tone is heard.

Condition

- To assign the Intercom Signal mode from an electronic keyset, adjust the Intercom Signal Mode switch, HF, TN or PV.

- In Message Wait/ICM Queuing/Call Transfer/ Attendant Override, the ring is received with T mode regardless of the assigned ICM Signal mode.

## INTERCOM STEP CALL

Description

When a station receives a busy signal after placing an intercom call, the user can request a call to a different station by dialing only the last digit of the new station number.

Operation

To use Step Call when receiving intercom busy tone,

If the next station to be called has the same first digit as the called busy station, dial the last digit of the next station.

## INTERCOM TENANCY GROUP

Description

A station can be assigned to one of the system Intercom Tenancy Groups. Each Intercom Tenancy Group can be operated independently and the stations in the group can be assigned an individual CO group to use..

Each group can be assigned with  attendant and can be programmed to allow or deny calls to other groups (allowed access groups). Stations in a group are allowed access to other stations based on the allowed access of the calling group.

Admin Programming

- Intercom Tenancy Group (PGM 16)

Condition

- When a call to other Intercom Tenancy Group is denied, pick-up cannot be activated.

## LINKED STATION PAIR

Description

Two stations can be linked with each other by programming. Linking with a Digital Keyset and a Wireless terminal, the user can use them alternatively. When two stations are linked, the following functions are supported;

- If one of two linked stations receives intercom calls/CO incoming calls (DISA/ DID), then the other linked station will receive ring together.

- If any one station in a linked pair go to DND state or pre-selected message display state, then the other goes to the same state automatically. Also, if any one station in a linked pair comes out of this state, then the other comes out also.

- If one of two linked stations is busy, the LCD of the other station will display “IN USE AT LINK STA”.

- When a linked station is busy, the other idle linked station will not receive ring for CO lines, transferred ring or ICM calls.

Admin Programming

- Linked Station Pairs (PGM 28)

Condition

- Linked station pair is not available at Attendant.

- ICM box or DSS/DLS cannot be linked with a station.

- It is operated with Tone mode in linked station pair regardless of ICM Answer mode.

- A normal station can be linked to a WTU (Wireless Terminal) but, two WTUs cannot be linked pair stations.

- If a linked station assigned forward, the linked pair is released.

## LOCK-OUT FOR ONLY INTERCOM

Description

When a station goes off-hook and does not initiate dialing during the Dial-tone Time or, if the duration between dialed digits exceeds the Inter-digit Timer, the station will receive error tone and the call attempt will be abandoned. In order to complete the call, the user should return to an on-hook state and re-start the call attempt.

Admin Programming

- ICM Dial -tone Timer (PGM 44-BTN 8)

- Inter-digit Timer (PGM 44-BTN 9)

## MEET ME PAGE

Description

A user can respond to a page from any station and connect to the paging party for a private conversation. The user should respond to the page in the Page Time-out duration to connect the paging party.

Operation

To respond a Meet-me Page,

- Lift the handset or press the [MON] button.

- Dial Meet-me Page code 4 4 or press the [HOLD/SAVE] button.

- The call with paging party is established and the zone returns to idle.

Condition

- To assign {MEET-ME PAGE} at a flexible button,

[TRANS/PGM] +Flex. BTN + 4 4 + [HOLD/SAVE]

## MSN/SUB-ADDRESSING

Description

When several ISDN devices are connected to a single ISDN port, the numbering method of each station is as follows;

MSN (Multiple Subscriber Number)

MSN provides the possibility for assigning multiple ISDN numbers to a single interface. It allows dialing from a line connected to a public network directly to terminals connected to a basic access which has subscribed to MSN number.

Sub-Addressing

The sub-addressing allows the called user to expand his addressing capacity beyond the one given by the ISDN number.

Admin Programming

- CO Line Sub-addressing/MSN Table (PGM 03)

- Register Sub-address supporting table (PGM 23-BTN 1)

- Register MSN/Sub-address No. (PGM 23-BTN 3)

Condition

- MSN is Called Party Number in ISDN message.

- Sub-address is Called Party Sub-address in ISDN message.

- To use this feature, the MEMU must be installed in the MPB.

## OFF-HOOK RINGING

Description

A station, which is busy, will receive audible ring signals for incoming calls on CO lines assigned to ring the station. These off-hook signals can be programmed as a muted ring signal or as a single burst of normal ring. Both signals are provided over the speaker of the keyset while using the handset. While using the speaker phone, a camp-on tone is provided over the speaker.

Admin Programming

- Off-hook Ring Signal Type (PGM 40-BTN 5)

- CO Line Ring Assignment (PGM 32-BTN 1,2)

Condition

- The muted ring as an off-hook signal cannot be adjusted.

## ONE-TIME DND

Description

A station assigned to DND can activate DND not to be interrupted by other calls while on a call for the duration of the off-hook period (One-time), eliminating off-hook ring signals. When activated, the phone will be in the DND mode and calls to the keyset will follow normal DND processing. When the station returns to an idle state, the DND mode is automatically deactivated.

Operation

To activate One-time DND,

Press the [DND/FOR] button during a call.

To deactivate One-time DND,

Replace the handset or go on-hook.

Admin Programming

- Do-Not-Disturb (PGM 10-BTN 1)

Condition

- It is not activated in Intercom Box.

## ON-HOOK DIALING

Description

A station can place calls without lifting the Handset by activating the speakerphone or monitor mode. If the speakerphone is disabled (headset mode is enabled) or the keyset is not equipped with speakerphone, the user should lift handset to converse.

Operation

When Auto Speaker Selection is ON,

Assign CO line/Intercom/Speed Dial to a flexible button and press the flexible button. Then it is activated immediately.

When Auto Speaker Selection is OFF,

Assign CO line/Intercom/Speed Dial to a flexible button and press the flexible button. Then 'LIFT HANDSET' is displayed on the LCD. Lift handset or press the [MON] button to activate.

Admin Programming

- Auto Speaker Selection (PGM 10-BTN 6)

## PAGING

Description

A station, which is allowed to access page facilities, can connect and transmit voice announcements to any or all of the systems external/internal page zones. Stations to receive pages for a given zone are assigned to the zone. A page warning tone, if assigned, will be provided to the page zone(s) prior to the audio connection. The user is allowed to continue the page for a specified period. After the time is expired, the user is disconnected and the page zone(s) is returned to idle.

The paging codes are as follows;

|  | Internal Zone | Internal All Call | External Zone | External All Call | All Call | Meet-me Page |
| --- | --- | --- | --- | --- | --- | --- |
| 162 | 401~ 420 | 4 3 | 46~47 | 4 8 | 4 9 | 4 4 |
| 100 | 401~ 415 | 4 3 | 46~47 | 4 8 | 4 9 | 4 4 |
| FPII | 401~ 405 | 4 3 | 46~47 | 4 8 | 4 9 | 4 4 |

Operation

To make a page,

- Lift the handset or press the [MON] button.

- Dial the desired paging code.

- If assigned, after page warning tone, make announcement.

- Replace the handset and go on-hook.

Admin Programming

- Page Timer (PGM 44-BTN 11)

- Page Warning Tone (PGM 40-BTN 4)

- Page Access (PGM 10-BTN 3)

- Internal Page Zone Access (PGM 17)

Condition

- When external paging is required, appropriate external equipment should be attached to the proper external page connections on the MPB.

- A station which is in DND or busy, it cannot receive a page.

- A station which is not allowed to page cannot make a page.

## PAGE ACCESS RESTRICTION

Description

Stations can be programmed such that they are denied to make a page. When programmed, the station can not connect to or make a page announcement.

Admin Programming

- Page Access (PGM 10-BTN 3)

## PAGING CONFERENCE GROUP

Description

It is possible for the second originator to page with the first originator during the paging announcement. Among paging groups, paging group with number larger than [Paging Group Index] can be Paging Conference Group.

It is possible for any paged station user to be the second page originator by off-hook and pressing the [CONF] button. At that time there is another second page originator, then the previous second originator goes to idle status and the new second originator can make paging with the first originator.

Admin Programming

- Paging Warning Tone (PGM 40-BTN 4)

- Paging Access (PGM 10-BTN 3)

- Internal Page Zone Access (PGM 17)

- Forced Page Index (PGM 8-BTN 9)

Condition

- Page Timer is not applied to Paging Conference Group.

- If there is the second page originator, it is impossible to “Meet Me Page”.

## SERIAL CALLING

Description

With DSS flexible buttons, users can place consecutive ICM calls without the need for regaining intercom dial tone (no need to hang-up) between calls. The user can simply press the appropriate DSS flexible button.

Operation

To use serial calling,

Press the appropriate DSS flexible button, the old call is disconnected and the new call will be established.

## 2B PHONE

Description

This feature allows connecting a secondary phone or PC with a primary keyset. There are 2 kinds of 2B module that can be used with some LKD keysets. One type allows the addition of a second keyset, the other allows the addition of a SLT devices (phone, fax, answer mic or modem). To extend a phone, the additional device is connected directly to a primary keyset in a 2B module, and wiring to the MDF is not required.

When CTI functions are used, a PC can be connected to a primary keyset with a CTIU, the 3rd 2B module type.

Operation

A digital keyset, SLT or PC may be connected to the primary keyset with 2B module as a secondary extension.

Admin Programming

- CTI Station Attribute (PGM 22)

Condition

- This feature is available with LKD keysets (8/30/44 BTN) and SLIU, DTIU and CTIU 2B option modules are needed.

- An external CTIU is available for use with KD/E series keysets.

- The max. intercom capacity of the system is the same even if the 2B phone is used.

- In GDK-FPII, one extension is designed as two voice channels. If a secondary extension is connected, the intercom number will be the intercom number of primary plus 38. It can be changed by Admin. Programming.

- In GDK-162/100, one extension is designed as one voice channel. To use 2B feature, the primary keyset should be connected to an odd port.

- In GDK-162, 2B module can be used on a keyset connected to a DTIB2E.

- The secondary phone is operated as the same with a normal phone. But, the secondary SLT is not available for the MSG wait lamp.

# . SYSTEM FEATURES

## ACCOUNT CODE

Description

While on an outside call, users can enter a series of digits that will be included in the SMDR call record. The series of digits, ‘Account Code’ can be used to allocate cost or time for the call.

Operation

To enter an account code during a conversation with an external party;

In a Keyset:

- Press the {ACCOUNT CODE} flexible button.

- Dial the account code (max. 12 digits).

- Press the {ACCOUNT CODE} flexible button, the user is reconnected to the external party.

In a SLT:

- Hook flash and intercom dial tone is heard.

- Dial feature code 5 0.

- Dial the account code (max. 12 digits).

- Hook flash and it is reconnected to the external party.

Condition

- For keysets, you should assign a flexible button as {ACCOUNT CODE} to access the account code feature.

[TRANS/PGM] + Flex. BTN + [TRANS/PGM] +  8 1 + [HOLD/SAVE]

- While entering account code, it is impossible to talk to the other party.

## ADVICE OF CHARGE

Description

The AOC service provides of outgoing call charging to the user in public ISDN. According to the admin programming, the saved and printed information is provided to the PC or user's LCD. This function is applicable to the most European countries.

Operation

After release a CO line call, when network sends release complete message to user, the following information is sent to facility IE.

* CHARGE NOT AVAILABLE

al 08 02 01 ** 02 01 23 05 00

* FREE OF CHARGE

al 0a 02 01 ** 02 01 23 30 02 81 00

* NORMAL CHARGING

al 20 02 01 ** 02 01 23 30 18 30 16 al 05

81 03 @@ @@ @@ a2 07 81 05 %% %% %% %% %%

82 01 01 82 01 01

[**] : The rule is changed in each call.

[@@ @@ @@] : It means charging current unit. (IA5)

[%% %% %% %% %%] : It means real current value displayed with

You can see it with SMDR print after the call.

Admin Programming

- AOC Type (PGM 08-BTN 7)

- SMDR Attribute - I (PGM 62)

- SMDR Attribute - II (PGM 63)

Condition

- It is displayed on the LCD instead of call timer and changed based on the receipt of each call charging information.

- The information of AOC includes the Unit or Currency.

## ALARM

Description

The system can be programmed to recognize the status of an external contact (normally open or closed) from a relay or manually activated switch. When activated, the system will signal programmed stations with a single tone repeated per 1-min. interval or a continuous tone. This capability is commonly employed to provide remote alarm signals or as a door bell. When used as an alarm, the assigned stations receive the programmed signal. To stop the signal, the alarm should be deactivated (reset) from a station assigned to receive the alarm signal. When used as a door bell, the assigned stations receive a single tone each time when the contact is activated and no reset is required.

Operation

At detecting the Alarm/Door Bell contact, the system sends the appropriate alarm signal to assigned stations.

To terminate the alarm signal while idle status;

Dial 6 5 and confirmation tone is heard. The alarm signal is terminated at all assigned stations.

To re-arm alarm;

Clear the alarm condition and reset (terminate) the alarm signal. The proceeding steps can be done in any order to rearm.

Admin Programming

- Alarm Enable (PGM 50-BTN 1)

- Alarm Mode (PGM 50-BTN 2)

- Alarm Contact Type (PGM 50-BTN 3)

- Alarm Signal Mode (PGM 50-BTN 4)

- Alarm/Door Bell Signal (PGM 12-BTN 1)

Condition

- An external contact should be connected to the alarm input on the MISB.

## AUTHORIZATION CODE (PASSWORD)

Description

This feature provides a means for limiting access to specified CO lines by stations and DISA users. To access a CO line, a user should input a valid authorization code matching an authorization code stored in the system. Authorization code may be assigned by station or Admin. Programming.

Operation

To access a CO line when authorization codes are required,

- Attempt a CO line access, and DND warning tone is heard.

- Enter the authorization code (5 digits).

- If valid, the CO line is connected and if not valid, you will hear error tone.

Admin Programming

- Authorization Code (PGM 54)

- DISA Account Code (PGM 30-BTN 5)

- CO Line Group Account (PGM 30-BTN 3)

Condition

- The total number of authorization code in each system is as follows; (x.4 version or later)

GDK-162: 600

GDK-100: 164

GDK-FPII: 110

- The same authorization code cannot be assigned.

- To register the authorization code,

[TRANS/PGM] + 4 4 + current authorization code + new authorization code + [HOLD/SAVE]

- To change the authorization code,

[TRANS/PGM] + 4 5 + current authorization code + new authorization code + [HOLD/SAVE]

## AUTO CALL RELEASE

Description

When a station does not initiate dialing on an outgoing CO line or receive no answer on an intercom call, the system will return the CO line or the called station to idle based on the assigned Auto Release Timer. If in the speaker phone/monitor mode, the station will return to idle, otherwise, the station will receive error tone if using the handset.

Admin Programming

- Station Auto Release Timer (PGM 44-BTN 21)

- Automatic Release Timer (PGM 44-BTN 10)

Condition

- Intercom call in H/P mode is regarded that it is answered and Station Auto Release is not activated.

- When the Automatic Release time is assigned to 0, Auto Call Release is not activated.

## AUTOMATIC CALL DISTRIBUTION

Description

A separate or common supervisor can be assigned in UCD group. And, the supervisor can monitor the status of the group. When a call is queued to a group for longer than a predefined time or when a predefined number of calls are queued, the supervisor's LCD will indicate the number of calls in queue, and the queued time for the longest queue. The supervisor can change overflow destination and timing. The system will provide traffic and on line status reports, based on the supervisor's request for the UCD group including the following group statistics:

- Total calls

- Number of unanswered calls

- Average and the longest queued calls

- Number and total time when all agents are busy

- Average ringing time before answer

- Average service time after answer

Operation

To monitor an agent's conversation at ACD supervisor,

- Call the busy agent and receive busy tone.

- Press {ACD} flexible button.

- The supervisor can monitor the agent, but will not send audio to the agent.

To report ACD status,

- Press the {ACD} flexible button.

- Dial ACD Status Feature code.

- Status / 2-Database / 3-Duty / #-Print)

In the Status mode, the supervisor can initialize the group statistics by pressing the [MUTE] button.

Admin Programming

- Station Groups (PGM 47)

- Group Attribute (PGM 48)

- ACD Print Time (PGM 44-BTN 18)

- ACD Clear (PGM 42-BTN 6)

- ACD Printing Base Timer (PGM 42-BTN 7)

Condition

- The user should assign a flexible button for {ACD}.

[TRANS/PGM] + Flex. BTN + [TRANS/PGM] +   8＊ + [HOLD/SAVE]

- The user can see the group status by ACD supervisor or by printing periodically to RS-232C print.

- To print ACD statistics periodically, program as follows;

In Hour Base,

PGM 42-BTN 7 is OFF and enter the time period in PGM 44-BTN 18 with hour base.

In 10sec Base,

PGM 42-BTN 7 is ON and enter the time period in PGM 44-BTN 18 with 10-second base.

- The agents can also print and view the ACD statistics as the supervisor (99A Upgrade)

## AUTOMATIC PRIVACY

Description

As default, the system provides privacy on all communications. If desired, the customer may set to disable the Privacy feature, which will allow stations to join in on existing CO line conversations without invitation.

Operation

If privacy is enabled from a keyset,

Press a busy CO line button and busy tone will be heard.

If privacy is disabled from a keyset,

Press a busy CO line button and the station is connected to the conversation.

Admin Programming

- Automatic Privacy (PGM 40-BTN 2)

- Privacy Warning Tone (PGM 30-BTN 3)

- Override Privilege (PGM 11-BTN 3)

Condition

- A station cannot override the other station during a conference though the privacy of conference station is disabled.

## AUTO RELEASE OF [MON] BUTTON

Description

After activating a feature; Call Fwd, etc., if the keyset is in the Monitor or Speakerphone mode, the keyset will automatically return to idle and the LED of the [MON] button will be extinguished.

## CALL COST DISPLAY

Description

With installed the CMU(Call Metering Unit), the system can be programmed to display an estimated cost for outside calls on a fixed charge per call metering pulse, as assigned in the system. The cost is shown in the LCD instead of the call timer and is updated based on receipt of each call metering pulse.

Admin Programming

- Metering Type (PGM 31-BTN 5)

- SMDR Cost per Unit Pulse (PGM 63-BTN 1)

- SMDR Fraction (PGM 63-BTN 2)

Condition

- For analog CO line, the Call Metering Unit (CMU) should be installed to display call cost.

- SMDR records is changed by the followings;

- The MEMU is not installed: 100

- The MEMU is installed: 10,000 (GDK-162)

2,000 (GDK-100/FPII)

- In GDK-FPII (x.4 or later version), the MEMU should be installed to use SMDR.

## CANNED TOLL RESTRICTION

Description

This feature may be assigned to apply exception items in CO line Allow/Deny table. It may be applied to only stations which are assigned to COS 5 or 6.

Admin Programming

- Canned Toll Exception Table (PGM 64)

## CCR (CUSTOMER CALL ROUTING)

Description

When a call comes into the system through DISA/DID line, the external user can route the system by pressing only one digit. A user may access the desired station by dialing the station number or group number through DISA.

Admin Programming

- Flexible DID Table (PGM 05)

- DID Conversion Type (PGM 8-BTN 4)

- Flexible DID Timer (PGM 43-BTN 16)

- DISA Retry Counter (PGM 43-BTN 22)

- Inter-digit Time (PGM 44-BTN 9)

- DID/DISA Destination (PGM 56)

- CCR (PGM 79)

Operation

- When a call is answered by a system programmed with CCR, the caller is presented with a DVU announcement, which gives a choice of destination.

- The caller may select a destination with dial pad.

- The call can be routed to a phone system (extension, hunt group, or system greeting).

Condition

- If a caller dials full destination number, the call will be routed to the destination by system numbering plan.

- If a caller dials one digit and wait for Flex. DID Timeout, the GDK system will compare the digit with CCR table. If the digit is matched with CCR table, the call will route to a destination (extension, hunt group, system greeting, speed dial), otherwise the call will be routed to recall destination (by PGM 32: Ring Assignment or Attendant) or call progress tone will be provided.

- If the DVU announcement is presented to the caller, the caller can retry for CCR 3 times. So system greetings can have the depth by the control of system.

- To assign a DVU system announcement for CCR or Flexible DID table, press [DND/FOR] button before pressing the DVU system announcement number.

- To assign a speed bin for CCR or Flexible DID table, press [MUTE] button before pressing the speed number.

- The CCR feature is supported only for DID type 2 (S/W version 5.5 or later) or DISA.

- DVU system announcement 1~9 may be used for CCR.

- Call routing will be operated with current DVU announcement. So the maximum depth is 9.

- The external user can dial during the DVU announcement or inter-digit time (5sec) after the announcement.

- If the caller does not dial any digit within inter-digit time (5sec) after DVU announcement, the call will be routed to recall destination (by PGM 32: Ring Assignment or Attendant) or disconnected after error tone.

- If there is no MEMU in the MPB, the CCR feature is not supported to DID.

- If there is no MEMU in the MPB, the CCR feature is the same as previous version for DISA.

- If the status of MEMU is changed, the CCR table should be programmed again.

- Incoming CCR call will be served or disconnected after programmed DVU announcement according to CCR program (PGM 79).

- Speed bin (station or system) will be supported in X.5Ca or later version.

- If a call is routed by 'station speed dial' in CCR (PGM 79) or Flexible DID table (PGM 05), the call will be charged to the programmed station (same as Station Off-net Call Forward).

- If a call is routed by 'system speed dial' in CCR, of Flexible DID table, the call will be charged to the programmed station (same as Station Off-net Call Forward).

- Both station speed bin and system speed bin cannot be assigned in the same bin number in PGM 05 as mixed entry for each day and night destination.

- To use CCR feature for DID, DVU should be assigned for Flexible DID Destination (PGM 05).

- If a user presses # button while CCR is operated, the first system greeting will be heard again. Press＊button to listen to the previous system greeting. (available in S/W version 5.5 or later)

## CHANGE KEYSET RING CADENCE

Description

The keyset ring cadence for intercom or CO line ring may be reversed.

Admin Programming

- System Attribute - III (PGM 42-BTN 4)

Condition

In OFF mode,

SLT = ICM - 1sec ON, 4sec OFF

CO - 0.4 ON, 0.2 OFF, 0.4 ON, 4 OFF

Keyset = ICM - 0.8 ON, 2.4 OFF

CO - 0.4 ON, 0.4 OFF, 0.4 ON, 2 OFF

In ON mode,

SLT = ICM -0.4 ON, 0.2 OFF, 0.4 ON, 4 OFF

CO - 1sec ON, 4sec OFF

Keyset = ICM - 0.4 ON, 0.4 OFF, 0.4 ON, 2 OFF

CO - 0.8 ON, 2.4 OFF

## CLASS OF SERVICE (COS)

Description

Each station and CO line may be assigned to have different class to allow or restrict call service. Separate COS assignments are programmed to each station and CO line. Applied dialing restrictions are the result of the interaction of COS assignments as shown below.

Station COS

| COS 1 | There is no restriction to dial. |
| --- | --- |
| COS 2 | Monitored by Allow/Deny Table A |
| COS 3 | Monitored by Allow/Deny Table B |
| COS 4 | Monitored by Allow/Deny Table A & B |
| COS 5 | Long distance call is not allowed. |
| COS 6 | Long distance call is not allowed. Max. 8 digits may be dialed. |
| COS 7 | Only intercom, paging and emergency call are allowed. |

CO COS

| COS 1 | There is no restriction. Monitored by STA COS. |
| --- | --- |
| COS 2 | Monitored by Allow/Deny Table STA COS 2/4. |
| COS 3 | Monitored by Allow/Deny Table B & STA COS 2/4. |
| COS 4 | Long distance call is not allowed for all STA COS. Max. 8 digits may be dialed. |
| COS 5 | Overrides STA COS 2,3,4,5,6. (No COS restriction) |

Admin Programming

- Station COS (PGM 14)

- CO line COS (PGM 30-BTN 2)

- CO COS of Connection Public Switching through Tie Line/DISA/DID (PGM 43-BTN 23)

- Toll Restriction (PGM 70)

- Canned Toll Exception Tables (PGM 64)

Condition

- Rule of COS

- In STA COS 7, no dialing is allowed to CO line.

- In CO COS 5, STA COS 1~6 is ignored and there is no restriction to access CO line.

- In CO COS 4, STA COS 1~6 is ignored. The long distance call is not allowed and max. 8 digits may be dialed.

- In CO COS 1, it is restricted by STA COS.

- In CO COS 2 and STA COS 2/4, it is restricted by Exception Table A. There is no restriction in STA COS 1/3. In STA COS 5, long distance call is not allowed and max. 8 digits can be dialed.

- Rule of CO line Allow/Deny Restriction

- If there is no entries, no restriction is provided by the table.

- If there are entries in the Deny table, then restriction is on a 'Deny Only' basis.

- If there are entries in the Allow table, then restriction is on a 'Allow Only' basis.

- If there are entries in both the Allow and Deny Tables, the Allow Table is searched, if the dialed number matches an entry in the Allow Table, the call is allowed. If a match is not found, the Deny Table is searched and, if a match is found in the Deny Table, the number is restricted otherwise, the number is allowed.

## CO MESSAGE WAITING

Description

When a DISA/DID call through ISDN DID line is received, the caller's number is displayed on the LCD of the called party. If the caller hangs up the call before answered by the called party or attendant, the CLI (Calling Line Identification) will be left into the original called party.

Operation

When a CO line ring is received at ISDN CO line (BRI, PRI, STIB),

- The phone number is displayed on the LCD of ringing station.

- If it is not answered, the CLI is displayed on the LCD.

To answer the message,

- Press the [CALLBK] button to confirm the message.

- Press the [HOLD/SAVE] button and the number is dialed.

- To delete the CLI without dialing, press the [CONF] button.

Admin Programming

- CO MSG Wait – Station Base (PGM 23 – BTN 6)

Condition

- The MEMU should be installed.

- Total number of CO message wait of each system is 1000 (system base.)

## CONFERENCE

Description

A station user can establish a call with internal stations and CO lines. In a conference, the supervisor may exit the conference though the conference is still connected.

Operation

To establish a conference,

- With established the first call, press the [CONF] button. The existing call is on hold and intercom dial tone is heard.

- Place a call to internal party.

- When answered, press the [CONF] button.

- When all parties have been called, press the [CONF] button again. And all parties can converse.

To make Unsupervised Conference,

- During a conference, press the [CONF] button in supervisor.

- The conference is still connected and the LED of [CONF] button of supervisor is flashing.

- To re-enter the conference, lift handset and flashing [CONF] button in supervisor.

Admin Programming

- Unsupervised Conference Timer (PGM 43-BTN 13)

Condition

- In unsupervised conference, it is restricted to Unsupervised Conference timer if there is no internal station in the conference.

(Default: 10min.)

- Max. 6 parties (internal/External) can enter a conference.

- In case the supervisor of a conference is SLT station, only 3 parties are in the conference.

## DATA LINE SECURITY

Description

A station can be assigned with ‘Data Security', which will block attempts by other stations and the attendant from overriding, or camp-on to the station. In this way, transmitted data from the station is protected from accidental corruption from a camp-on or override tone. Modems and fax extensions should usually be given this status.

Admin Programming

- Data Line Security (PGM 11-BTN 5)

Condition

- If an analog extension (Data, Fax, Modem), which is assigned to data line security, makes an external call via ISDN line, 3.1KHz Setup message will be sent to ISDN CO line instead of speech Setup message.

- If a busy extension, which is assigned to data line security, receives an incoming CO call, the call is disconnected with cause busy regardless of the DID/DISA Destination setting (PGM 56).

## DELAYED CO LINE RING

Description

The ring signal can be sent to the assigned station(s) immediately upon detection or can be sent after an assigned delay (number of ring cycles).

Admin Programming

- CO Line Ring Assignment (PGM 32)

Condition

- When the delay ring counter is assigned to 0, the ring signal can be sent to the assigned station immediately when it is detected.

## DIAL BY NAME

Description

Each station and speed dial (system and station) number may be assigned a name of up to 16 characters. When names are programmed in the digital display keyset, user may select a station or speed dial number (system or station) by the stored name. The user selects a directory (station, station speed dial or system speed dial) and enters alphanumeric data with 2 digits for each character. The system searches and displays the first match based on the entered characters. The user may scroll at any point using []/[] buttons through the names in the directory and select a name to call.

Operation

To access Dial by Name,

- Press the [SPEED] button twice.

- Dial the desired directory (1, 2, or 3) and confirmation tone is heard.

1 = Intercom / 2 = Station Speed Dial /

3 = System Speed Dial

- Press the []/[] button or dial digits. The LCD displays 2 names, one on each LCD line. If user dials a character (2 digits), the LCD shows the first match. Pressing the []/[] button will scroll 2 lines up or down in the directory.

- When the desired name is shown;

a.  If on line 1, press the [HOLD/SAVE] button and Dial by Name is activated.

b. If on line 2, dial 2 and press the [HOLD/SAVE] button to activate Dial by Name.

Condition

- Dial by name is only available to a digital keyset with LCD for programmed names.

- The name should be registered to use 'Dial By Name'.

## DID (DIRECT INWARDS DIALING)

Description

This feature allows to access to specific stations on specific DID lines from the public telephone network, without going through an attendant answering position.

When a DID is received at a busy station, the call is waited in the station for DID/DISA Answer timer.

The digits entered in DID may be converted by DID Conversion Type (PGM 08-BTN 4)

Admin Programming

- CO Line Group Assignment (PGM 30-BTN 1)

- DID Conversion Type (PGM 8-BTN 4)

- DID-RN Assignment (PGM 37-BTN 2)

- Flexible DID Table (PGM 5)

- DID/DISA No Answer Time (PGM 44-BTN 4)

- DID Digit Conversion (PGM 55)

- DID/DISA Destination (PGM 56)

- DID Ring (PGM 21-BTN 1)

- DID Call Waiting (PGM 11-BTN 9)

Condition

- To receive DID ring on a station, the station should have {CO line} button or {LOOP} button.

- DID call waiting feature gives another DID ring on a station. To receive another ring (call waiting) in a busy station, the station should have more than two Loop keys or the associated {CO line} buttons.

## DIFFERENTIAL RING

Description

There are four types of ring type allowing users to distinguish which keyset in a given area is ringing A user of digital keyset can program the receiving audible signal for incoming ring. The setting will affect all incoming ring signals including DID/DISA CO line and intercom.

Operation

To set the ring signal from a digital keyset only,

- Press the [TRANS/PGM] button.

- Dial 5 0 and the Ring Type code (1~4).

- Press the [HOLD/SAVE] button.

Admin Programming

- Ring Frequency (PGM 75) - incoming ring

- Distinct Ring Frequency (PGM 76) - outgoing ring

- Differential Ring (PGM 21-BTN 6)

Condition

- Ring type selection is only available to digital keysets.

## DISA(DIRECT INWARD SYSTEM ACCESS)

Description

DISA allows to users to access the system's resources by calling specified CO lines connected to the system. Upon answering the outside call, the system will provide the user intercom dial tone or an announcement and the user may then access the desired resource using dial codes.

Operation

To use the DISA from an external caller,

- Call the DISA CO line of the system.

- At the tone or announcement, dial the desired station number or CO line access code. The station receives CO line ring for the DISA CO line.

Admin Programming

- DISA Line Assignment (PGM 30-BTN 4)

- DISA Account Code (PGM 30-BTN 5)

- DISA Retry Counter (PGM 43-BTN 22)

Condition

- If the CO line is assigned to Authorization Code, DND warning tone (or DVU announcement if DVU(EDVU) is fitted and the associated MSG is recorded) is heard and the authorization code should be entered. And then, dial tone will be heard.

## DUMMY DIAL TONE

Description

If the PX does not provide dial tone, dummy dial tone will be provided to the caller by the system until the first digit is dialed.

Operation

- Access a CO line to be provided dummy dial tone.

- Dummy dial tone is heard.

- The call is connected.

Admin Programming

- Dummy Dial Tone (PGM 38-BTN 3)

## DURATION TIMER

Description

A stopwatch may be activated in a digital keyset.

Operation

To use stopwatch,

- Press the {STOP WATCH} flexible button from a keyset.

- Stopwatch is activated.

To release stopwatch,

Press the illuminated {STOP WATCH} button.

Condition

- To program the [STOP WATCH] button at a flexible button,

[TRANS/PGM] + Flex. BTN + [TRANS/PGM] +  8 9 + [HOLD/SAVE]

- When a call is received while the stopwatch is activated, the LCD is not changed.

## EMERGENCY CALL

Description

The user can dial the emergency service code regardless of lower station COS.

Admin Programming

- Emergency Call (PGM 06)

Condition

- If the dialed number is duplicated with system numbering plan, emergency call is not activated. Change the system numbering plan or enter other emergency call number.

- An emergency call can be dialed by pressing CO line in UK, Finland and Italy though the station which is assigned to COS 7.

## ENHANCED DIAL BY NAME

Description

There are 2 changes to give easier interface to users accessing "Dial by Name" feature.

- The easier interface when entering alphabets at keyset which has 24 flexible buttons.

- Dial-by-Name feature is programmable on a flexible button.

Operation

To enter alphabets for Dial-by-Name at keyset which has 24 flexible buttons,

- Dial the digit to enter the character (Ex. 21=A)

- 'A' is displayed.

- Press flexible button 2.

- 'B' is displayed.

- The 1st button; A, 2nd button: B, 24th button: X, [REDIAL]: Y, [TRANS/PGM]: Z.

To program Dial-by-Name feature in a flexible button,

- Press the [TRANS/PGM] button.

- Press a flexible button to assign.

- Press the [SPEED] button.

- Dial the parameter:

00~19: speed dial number

21: User dial by name

22: Station speed dial by name

23: System speed dial by name

- Press the [HOLD/SAVE] button.

Condition

- The keyset which has less than 24 flexible buttons can be used, but full alphabets are not supported. (Ex. 12 Flexible buttons: A~L and Y,Z are possible)

- In the keyset which has more than 24 flexible buttons, only 24 flexible buttons are available to enter alphabets.

- The current method using dial-pad is still available to enter a name.

## FLEXIBLE DID ON QSIG FOR AUTO REROUTING

Description

Incoming DID calls can be rerouted to the destination of the other system automatically with Flexible DID Conversion table through QSIG line when QSIG connection is made between GDK systems.

Operation

If a station number of slave system is programmed to Flexible Conversion table and QSIG Routing table in master system, incoming DID call for the station to master system will be rerouted automatically to slave system with QSIG call.

Admin Programming

- DID Conversion Table (PGM 05): enter the station number of slave system to day/night destination.

- QSIG Routing Table (PGM *2): enter the station range of slave system.

## FUNCTION BUTTON ASSIGNMENT

Description

The button arrangement for 4/8 BTN Keyset is limited so you may re-assign the button for use.

Operation

To assign a button in a 4/8 BTN keyset,

- Press the [TRANS/PGM] button.

- Press a Flex. Button to assign.

- Press the [TRANS/PGM] button and dial the appropriate code.

| BTN | CODE |
| --- | --- |
| SPEED | 9 0 |
| CONF | 9 1 |
| CALLBK | 9 2 |
| DND/FOR | 9 3 |
| FLASH | 9 4 |
| MUTE | 9 5 |
| MON | 9 6 |

- Press the [HOLD/SAVE] button.

## LCD MESSAGES (CUSTOM: 00, 11~20)

Description

This feature permits the system attendant to program Custom Messages which users can employ in addition to the pre-selected messages (11~20). In addition, individual users may program message 00 as a custom message. When selected by a station user, caller can see their LCD display a chosen message.

Operation

To program Custom Message 00 from a station,

- Press the [TRANS/PGM] button.

- Dial 4 8.

- Enter the message (up to 23 characters).

- Press the [HOLD/SAVE] button and confirmation tone is heard.

| Q - 11 Z - 12 . - 13 1 - 10 | A - 21 B - 22 C - 23 2 - 20 | D - 31 E - 32 F - 33 3 - 30 |
| --- | --- | --- |
| G - 41 H - 42 I - 43 4 - 40 | J - 51 K - 52 L - 53 5 - 50 | M - 61 N - 62 O - 63 6 - 60 |
| P - 71 R - 72 S - 73 Q - 7* 7 - 70 | T - 81 U - 82 V - 83 8 - 80 | W - 91 X - 92 Y - 93 Z - 9# 9 - 90 |
| *1 - Blank *2 - : *3 - , | 0-00 | # |

< English Character Set >

To program Custom Message 11~20 from the system attendant,

- Press the [TRANS/PGM] button.

- Dial ＊7 and the message number (11~20).

- Enter the message (up to 23 characters).

- Press the [HOLD/SAVE] button and confirmation tone is heard.

To activate Custom Messages,

- Press the [TRANS/PGM] button.

- Dial 7 7.

- Dial the 2-digit message code 00 or 11~20.

- Press the [HOLD/SAVE] button.

To cancel Custom Message,

Press the flashing [DND/FOR] button or,

- Press the [TRANS/PGM] button.

- Dial 7 7 and #.

- Press the [HOLD/SAVE] button.

## LCD MESSAGES-PRESELECTED

Description

A user can select pre-assigned messages from 1 to 10 to be shown on the LCD of the calling party. Message 00 can be programmed at each station and, message 11~20 can be programmed by attendant (Custom Message).

The Preselected Messages are as follows:

Message 01 LUNCH, RETURN HH:MM

Message 02 ON VACATION / RETURN AT MM:DD

Message 03 OUT OF OFFICE / RETURN TIME HH:MM

Message 04 OUT OF OFFICE / RETURN AT MM:DD

Message 05 OUT OF OFFICE/RETURN UNKNOWN

Message 06 CALL: (Telephone No: Up to 17 digits)

Message 07 IN OFFICE: STA xxxx

Message 08 IN A MEETING/RETURN TIME HH:MM

Message 09 AT HOME

Message 10 AT BRANCH OFFICE

Operation

To activate a preselected message,

- Press the [TRANS/PGM] button and dial 7 7.

- Dial the 2-digit message code (01~10).

- Dial auxiliary data (time/date/etc).

- Press the [HOLD/SAVE] button.

To cancel a preselected message,

Press the illuminated [DND/FOR] button. Or,

- Press the [TRANS/PGM] button.

- Dial 7 7 #.

- Press the [HOLD/SAVE] button.

Condition

- If a station while activating preselected message assigns call forward, the preseleted message is automatically canceled.

## LEAST COST ROUTING (LCR)

Description

LCR is a system programmable feature that is automatically selected the least available expensive route when an outgoing CO call is made. This programming eliminates the necessity for the user to dial the access code of the least expensive carrier. There are three types of operation to activate LCR;

Internal LCR - It can be activated during internal dial tone. If the dialed digits are matched with internal LCR code, the system will seize a CO line from the programmed CO group and send the modified digits according to LCR programming.

Loop LCR - It can be activated by dialing the first accessible CO group code ('9' or '0') or by pressing Loop button. If the dialed digits are matched with COL LCR code, the system will seize a CO line from the programmed CO group and send the modified digits according to LCR programming.

Direct CO LCR - It can be activated during CO dial tone. After dialing CO line or CO group code, or pressing CO line or CO group button, it can be activated. If the dialed digits are matched with COL LCR code, the system will seize a CO line from the programmed CO group and send the modified digits according to LCR programming.

Operation

Internal LCR

- Dial internal LCR code after lifting handset or pressing [MON] button. On-hook dialing can also activate LCR call.

- It is an internal LCR code if the code is programmed with INT or BOTH in Leading Digit Table.

Loop LCR

- Dial COL LCR code after dialing the first accessible CO line or CO group access code ('0' or '9') or press the Loop button.

- It is a COL LCR code if the code is programmed with COL or BOTH in Leading Digit Table.

Direct CO LCR

- Dial COL LCR code after dialing CO or CO group access code or pressing CO or CO group button.

- It is a COL LCR code if the code is programmed with COL or BOTH in Leading Digit Table.

Admin Programming

- LCR (PGM 07)

- Flexible Button Assignment (PGM 29)

- CO Line Access (PGM 12-BTN 3)

- CO Group Access (PGM 24-BTN 4)

Condition

- There are 6 LCR modes. The mode is determined by PGM 07-BTN 1.

- LCR Access Mode 00 (M00): LCR call is disabled. ("DIS" mode in the previous version: prior to X.5Da)

- LCR Access Mode 01 (M01): Only Loop LCR is activated. ("OVR" in the previous version)

- LCR Access Mode 02 (M02): Internal LCR and Loop LCR are activated. ("ALL" in the previous version)

- LCR Access Mode 11 (M11): Loop LCR and CO LCR are activated.

- LCR Access Mode 12 (M12): All types of LCR are activated. (When dial ‘9’ or press Loop key, the system will wait next digits to compare LCR table without seizing CO line.)

- LCR Access Mode 13 (M13): All types of LCR are activated. (When dial ‘9’ or press Loop key, the system will seize first available CO line and wait next digits to compare LCR table.)

- In X.5Da or later version, leading digits can be duplicated. BTN 2 and DMT index make each entry unique.

- Leading digit table is sorted by leading digits, BTN 2 in LDT (INT, COL, BOTH) and DMT index.

- Internal LCR is applied if the dialed digits are matched with one of leading digits and BTN 2 is INT or BOTH.

- Loop LCR is applied if the dialed digits are matched with one of leading digits and BTN 2 is COL or BOTH.

- Direct CO LCR is applied if the dialed digits are matched with one of leading digits and BTN 2 is COL or BOTH and the seized CO line belongs to the programmed CO group in DMT.

- To work Loop LCR and Direct CO LCR differently with the same leading digits, there should be the leading digit entry for loop LCR prior to the leading digits for direct CO LCR. It is possible if the DMT index for loop LCR is smaller than the DMT index for direct CO LCR.

- While direct CO LCR is applied to ISDN CO, an ISDN Information message with called party IE, which includes only numbering plan and numbering type, is sent to the network when a user dials a digit. It is for the network not to disconnect the line.

- For direct CO LCR, leading digits should be programmed in consideration for the dial tone time provided by network.

- Direct CO LCR does not use an alternative DMT index because a CO line is already seized.

- LCR always has the higher precedence than flexible numbering plan table (PGM 01).

- LCR can be applied to the following cases;

- Dialing after seizing CO line by dialing CO line access code (0 or 9) only

- Dialing after seizing CO line by {LOOP} button

- Dialing without seizing CO line

- Speed Dial

- Off-net Call Forward

- Redial (if the previous call is LCR applied)

- ACNR (If the call is LCR applied when activating ACNR)

- LCR can not be applied to the following cases;

-	Dialing after seizing CO line by CO group code (801, 802...).

-	Dialing after seizing CO line by individual CO line code (8801, 8802).

-	Dialing after seizing CO line by CO button.

- Any leading digit string at the LDT table can be sub-string another leading digit string as ‘012’ and ‘0123’.

- Capacity for LCR Table

-	3 Day Zones

-	3 Time Zones

-	Number of ‘Dialed Code Bins’: 25 bins (250 bins if MEMU installed)

-	Number of ‘Modification Code Bins’: 25 bins (100 bins if MEMU installed)

-	Maximum number of ‘Dialed digits’: 12 digits

-	Maximum number of ‘Added digits’: 20 digits

-	Alternative DMT index: 1EA

## LOUD BELL CONTROL (LBC)

Description

The LBC contacts are activated when the assigned station receives ringing from an incoming CO line (if assigned ring), transferred CO line, or intercom call.

Admin Programming

- Universal Night Answer (PGM 31-BTN 3)

- External Night Ring (PGM 42-BTN 1)

- External Control Contacts (PGM 51)

Condition

- The LBC 1 can be programmed to be operated as an external night ring contact as well as a LBC contact.

- In the night mode, LBC 1 will follow UNA ring assignment and will ignore the station ring.

- An external ringing device should be attached to the contacts.

## MESSAGE WAITING / CALLBACK

Description

Station which is in DND or in idle state may receive a message wait from up to 5 other stations in the system. And, a busy station may have call back. The station receiving the message can call the parties that left the message wait.

A station with a message waiting can receive a periodic audible reminder of the message waiting. This tone is sent to keysets only while idle and is presented over the speaker.

Operation

To leave a message wait to an idle station that does not answer,

- Press the [CALLBK] button and receive confirmation tone.

- The LED of the [CALLBK] button at the receiving station will flash.

To leave a message wait to a station in DND,

- Press the [CALLBK] button and receive confirmation tone.

- The LED of the [CALLBK] button at the receiving station will flash.

To answer a message wait,

- Press the flashing [CALLBK] button.

- The station leaving the message will receive intercom ring.

To leave a call back to a busy station,

- Press the [CALLBK] button and receive confirmation tone.

- Hang up.

Admin Programming

- Message Reminder Tone (PGM 11-BTN 2)

- Message Reminder Tone Timer (PGM 44-BTN 1)

Condition

- Message wait reminder tone is not heard at a busy station.

## MEMORY FULL WARNING OF VM SYSTEM

Description

The GDK system may provide memory full warning message to attendant or a station when the memory of VM system is almost exhausted.

Operation

- SMDI format for warning tone to attendant

When 90% or 95% of memory is exhausted, VM system should send warning;

TYPE 3: cr lf MD ggg mmmm a sp yyyy sp cr If ^Y

cr: carriage return

lf: line feed

ggg: message desk no. (001)

mmmm: message desk terminal (0001~9999)

a: action code (F)

sp: space

yyyy: amount of remained memory

(0090 means 90% of memory is used)

^Y: Control Y

2.  SMDI format for warning to each station when a user presses the {Two-way Record} button:

TYPE 3: cr lf MD ggg mmmm a sp yyyy sp cr If ^Y

cr: carriage return

lf: line feed

ggg: message desk no. (001)

mmmm: message desk terminal (0001~9999)

a: action mode (M)

sp: space

yyyy: calling station number

^Y: Control Y

Condition

- When this message is received, the system will provide VM memory full warning (alarm signal) to the system attendant and “VM MEMORY FULL WARNING, xx% FULL” is displayed on the LCD.

- The warning will be deleted by dialing alarm reset code (6 5) at system attendant.

- If this message is received during two-way recording, current two-way recording is stopped and the warning message is displayed on the station, “VM MEMORY FULL WARNING, 100% FULL”. The current message will be left on the station.

- In other recording, if the system receives this message, the current call will be dropped and the station will receive error tone and “VM MEMORY FULL WARNING, 100% FULL” will be displayed.

## MOH CHANNEL VIA SLT PORT

Description

Using SLT ports connected with MOHU (Music On Hold Unit), MOH channels can be provided.

The assigned SLT port will be used as MOH channel on System Hold, MOH channel based on CO line, Music channel on UCD group or Ring group.

Operation

- Assign desired SLT port with MOH channel.

- Connect MOHU to the SLT port.

Admin Programming

- SLT MOH Channel Assignment (PGM #1-BTN 1,2)

- No. of Background Music Channel (PGM 41-BTN 1)

- MOH Channel for System Hold (PGM 41-BTN 2)

- MOH Channel for ICM Box Music (PGM 41-BTN 3)

- MOH Channel for CO Hold (PGM 38-BTN 2)

- Music Channel for UCD Group (PGM 48-BTN 15)

- Music Channel for Ring Group (PGM 48-BTN 3)

- Gain Control for MOH SLT Channel (PGM 66-BTN 9)

Condition

- Up to 5 SLT ports can be used as MOH channel from MOH 5~9.

- MOHU can be used to convert an SLT port into a MOH port.

- A SLT port in using cannot be assigned as MOH channel. At first assign desired SLT port with MOH channel and then connect MOHU to the SLT port.

- The MOHU supports 2 MOH channels.

- Call to SLT, which is assigned to MOH channel is not supported.

## MUTE

Description

The transmitting audio from a keyset can be blocked by the user so that the voice at the keyset is not sent to the connected party thus providing privacy for local discussions.

Operation

To mute the transmitting audio,

Press the [MUTE] button and LED of the [MUTE] button is illuminated. The connected party cannot hear the voice of muted station.

To restore transmission,

Press the illuminated [MUTE] button. LED of [MUTE] button is extinguished and transmission is restored.

## RETRIEVING PARKED CALL WITH QSIG

Description

If you properly adjust parking location number by flexible numbering on QSIG systems, you can retrieve remotely parked call with QSIG.

Operation

If a call is parked to a parking location in a system connected with QSIG, a user can retrieve the call by dialing the parking location number in the other system.

Admin Programming

- QSIG Routing Table (PGM * 2): enter the range of remote parking location same as the range of remote station number.

Condition

- If the remote parking location is empty, the call is routed by DID error destination.

- A call can be parked in the parking location in the same system. (A call cannot be parked in a parking location of the other system.)

## SMDR

Description

Station Message Detail Recording (SMDR) will provide details on both incoming and outgoing calls.

Number of SMDR Records;

With MEMU: 10.000 (GDK-162)

2.000 (GDK-100/FPII)

Without MEMU: 100

Operation

To print SMDR,

- Activate PC utility on PC.

- Connect serial port of MPB to the serial port of PC with the cable of RS-232C.

- Enter the SMDR attribute by Admin. Programming.

- After finishing the call, the SMDR is printed to PC.

Admin Programming

- CO Line Attribute -Ⅱ(PGM 31-BTN 5)

- SMDR Start Time (PGM 44-BTN 15)

- SMDR Attribute -Ⅰ(PGM 62)

- SMDR Attribute -Ⅱ(PGM 63)

## SMDR - LOST CALL PRINTING

Description

Lost call means that the caller gives up and terminates the call before the call is answered. The format of the individual call record is illustrated below, and the contents are focused on each case about the types of lost call.

NO  STA  CO TIME    START          DIALED

-----  ----  ---  ------  -----------------  -----------------------

#### 0001	EXT	01 	00:10  24/05/99 11:55 	R RING 00:05

Normal incoming call is received at an assigned stations of CO 01 during 5 sec.

0002 	101	08 	01:20  25/05/99 16:23 	R RNG 00:09

DID call is disconnected during it is being forwarded to ATD STA 101, because the dialed station does not exist.

0003	100	02 	00:20  25/05/99 18:11 	R100 RING 00:04

DID call is received at STA 100 during 4sec and disconnected.

0004  102	10 	01:20  26/05/99 18:37 	R103 RING 00:04

DID call is received at STA 102 via unconditional call forward to STA 103 during 4 sec and disconnected.

0005  621	08 	00:20  26/05/99 13:02 	G620 RING 00:06

DID call is received at Ring Group 621 during 6sec and disconnected.

0006  100	01	00:04  06/05/99 16:04	H100 RING 00:02

DID call is disconnected while STA 100 is being held it.

0007  102	01	00:07  06/05/99 17:04	H100 RING 00:02

DID call is disconnected while it is being transferred from STA 100 to STA 102.

G: Incoming call to hunt group, but the caller hangs up before answer

H: 1. Answered incoming call was transferred to another extension, but the caller hangs up before answer.

2. Incoming call placed on hold state and cleared down in hold state.

R: Direct call(DID) to an extension, but the call was disconnected before the extension answers. Or direct call to extension (A), but extension (A) does not answer and the call was forwarded to extension (B). The call was disconnected before extension (B) answers.

Operation

To print the lost call count of record,

- Dial ＊0 at attendant.

- Dial #.

- Press the [HOLD/SAVE] button.

- SMDR is printed in the PC connected to the system.

To clear the lost call count of record,

- Dial ＊2 at attendant.

- Dial #.

- Press the [HOLD/SAVE] button.

- All lost calls in the system are deleted.

Admin Programming

- SMDR (PGM 62)

Condition

- This SMDR record is sent to RS-232C automatically as soon as the event takes place.

- The SMDR record about lost call is not saved. Only the records are counted.

- To activate this SMDR record, the SMDR field must be set in Admin. Programming.

## SPEAKERPHONE

Description

Keysets are equipped with circuitry enabling the telephone to be used without lifting the handset and hands-free in two-way conversation.

Operation

To activate a speakerphone call,

Press the [MON] button and LED of the [MON] button is lighted.

To terminate a speakerphone call,

Press the [MON] button and LED of the [MON] button is extinguished.

Admin Programming

- Speakerphone/Headset (PGM 11-BTN 7)

Condition

- LKD-2NS keyset has no microphone and doesn't function as handsfree unit.

## STATION GROUP:          CIRCULAR/TERMINAL/UCD/RING/VM

Description

Stations can be grouped so that incoming calls will search for an idle station in the group and the call will be answered.

The number of station group and the number of station in a group as follows;

| System | STA Group | STA in a Group |
| --- | --- | --- |
| GDK-162 | 30 (620~649) | 64 |
| GDK-100 | 15 (620~634) | 32 |
| GDK-FPII | 8 (620~627) | 32 |

Station group may be assigned as one of the following 5 types;

- Circular Station Group

In Circular Hunt, calls to a station in the group or a pilot number will go to the station or an idle station in the group. If unavailable or unanswered in the hunt no answer time, the call is directed to the next station in the group. The call will continue to route until each station in the group has been tried. The call will remain at the last station in the group or will be passed to the assigned overflow station/group.

- Terminal Station Group

Calls to a station in the group or a pilot number will go to the station or an idle station in the group. If unanswered or unavailable, the call proceeds to the next listed station in the group. The call will continue to be rerouted until reaching the last station in the group where the call will remain or can be sent to an overflow station/group.

- UCD Station Group

Calls are sent to the group by dialing the pilot number (Station Group Number) or assigning CO lines to directly terminate to the group. Calls are directed to the station in the group that has been idle for the longest time. If all stations in the group are busy when a call is received for the group, the call may be routed to an alternate location, or may continue to wait (queue) for an available station in the group. If queued, the call may be sent to a UCD announcement when the queue period exceeds the queue to the 1st announcement timer. The second announcement can be provided if the call continues to wait beyond the queue to the 2nd announcement timer. The second message can be repeated until the call is answered or is disconnected. Based on programming, the queued call may be taken out of the group and directed to an overflow station.

The member of UCD group can assign DND. The station assigned UCD-DND will not receive ring.

Note: Do not make a loop with UCD group alternative destination.

- Ring Station Group

All the stations in the group receive ring simultaneously for a call of station group until one of stations received ring answers the call. If call is not answered until overflow timer, it will be sent to an overflow destination, if assigned.

- Voice Mail Station Group

This group is assigned for voice mail and only SLT can be assigned as a member of station group. Incoming, transferred CO lines and intercom calls can be routed to a Voice Mail group. When a call is routed to a VM group, the system will search an idle member in the calling VM group with Terminal type or Circular type.

The SMDI (Simplified Message Desk Interface) method uses RS-232C port instead of DTMF tone to sent data between Key Telephone system and Voice Mail system.

To make SMDI mode,

1) Set the following DIP Switch to OFF (left position).

- GDK-162/100: The 5th DIP switch

- GDK-FPII: The 3rd DIP switch

2) Connect the RS-232C cable between MPB and serial port on the PC of Voice Mail.

Admin Programming

- Station Group (PGM 47)

- Group Attribute (PGM 48)

- Voice Mail Dialing Table (PGM 73)

Condition

- A station can be a member of only one station group.

- A station in DND/Forward state will not receive group call.

- Transferred ring to station group is not recalled.

- A circular/terminal station group can be assigned with a pilot number (the Station Group Number) so that only calls to the pilot number will hunt.

- When a call is received in a ring group, the call will be in the ring process before receiving the announcement for Ring Group Announcement time. If no Ring Group announcement is assigned, the timer is ignored. If the timer is set to 0, the call will receive the announcement prior to the ring process.

- If all stations in the group are busy when a call is received, the call continues to wait for an available station in the group. If queued, the call may be sent to MOH until the call is answered or disconnected.

- If there is no available member in a group because every member sets DND, UCD DND, or call forward, all new calls to the group and all queued calls in the group are rerouted to another destination. One of the following is determined as the destination in the listed order;

- Overflow destination if assigned.

- Alternative destination if the group is UCD group and it is assigned.

- DVU/EDVU is required for announcement.

- When the number of queued incoming calls are over the pre-defined count (PGM 48-BTN 16) in UCD group, the incoming CO calls will be dropped. If alternative destination is set and all members are busy, incoming calls are not queued. All calls are routed to alternative destination. If overflow destination is programmed, the queued call is re-routed to overflow destination after overflow timer is expired.

## TEMPORARY STATION COS CHANGE

Description

A station user or the attendant can change the station's class of service to COS 7 for preventing unauthorized toll dialing from the station.

Operation

To activate temporary COS,

- Press the [TRANS/PGM] button.

- Dial 4 6.

- Press the [HOLD/SAVE] button.

To restore the station COS,

- Press the [TRANS/PGM] button.

- Dial 4 7.

- Dial the authorization code.

- Press the [HOLD/SAVE] button.

To activate temporary COS,

- Press the [TRANS/PGM] button.

- Dial # 2.

- Dial the desired station range.

- Press the [HOLD/SAVE] button.

To restore the station COS from the attendant,

- Press the [TRANS/PGM] button.

- Dial # 3.

- Dial the desired station range.

- Press the [HOLD/SAVE] button.

Admin Programming

- Authorization Code (PGM 54)

Condition

- At first, authorization code should be entered to change STA COS temporarily.

[TRANS/PGM] + 4 4 + Authorization Code    (5 digits) + [HOLD/SAVE]

## TRAFFIC ANALYSIS

Description

The system can monitor and print various system activities based on the system attendant's request. The information can be used to determine if adequate CO lines and other circuits are available in the system.

The traffic-monitoring program is activated only when enabled and will continue accumulating statistics until disabled. The traffic data is output to the RS-232C port upon manual request. Two types of traffic data, Peg Counts and Usage Times are provided. Peg counts give the actual number of times that a particular operation has occurred and are given in absolute values from 00000 to 99999. The usage duration provides the duration that a station, CO line, DTMF receiver, etc. were busy (in-use). Usage times are given in seconds from 00000 to 99999.  All traffic data are cumulative from the point in time when enabled. To restart the traffic data accumulation from a zero point, the traffic-monitoring program is enabled again.

Peg Counts (in absolute values)

- Internal Attempts - This counter is incremented by one whenever an idle station goes off-hook or activates the speakerphone.

- External CO Attempts - This counter is incremented by one whenever an idle trunk is seized for an outgoing call.

- Incoming CO Attempts - This counter is incremented by one whenever a CO line is seized with an incoming call.

- Calls Complete - This counter is incremented by one whenever an intercom call is made to an idle station and is connected or ring back tone is received.

- Station Busy - This counter is incremented by one whenever an intercom call is made to a busy station and busy tone is returned to the calling party.

- DTMF Receiver Attempts - This counter is incremented by one each time a DTMF receiver is used.

- DTMF Congestion - This counter is incremented by one each time all DTMF receivers are busy and an attempt is made to use a DTMF Receiver.

- CO Group Attempts - This counter is incremented by one each time an attempt is made to seize a CO line from a group.

- CO Group Congestion - This counter is incremented by one each time all CO lines in a group are busy and an attempt is made to seize a CO line from a group.

- Preset Forward- This counter is incremented by one each time a call is presented to a station in a preset forward condition and the call is forwarded to the destination.

- DND - This counter is incremented by one each time a station is placed in the DND mode.

Usage Times (in seconds)

- CO Usage - This gives the total time each CO line was used for outgoing or incoming calls.

- DTMF Receiver Usage - This gives the total time that DTMF receivers were used for call processing.

- Attendant Usage - This gives the total time that the attendant station was active in call processing.

Operation

To start the traffic analysis at system attendant,

- Press the [TRANS/PGM] button.

- Dial # 7 1.

- Press the [HOLD/SAVE] button.

To disable the traffic analysis at system attendant,

- Press the [TRANS/PGM] button.

- Dial # 7 2.

- Press the [HOLD/SAVE] button.

To print results of the traffic analysis at system attendant,

- Press the [TRANS/PGM] button.

- Dial # 7 3.

- Press the [HOLD/SAVE] button.

To abort printing at system attendant,

- Press the [TRANS/PGM] button.

- Dial # 7 4.

- Press the [HOLD/SAVE] button.

Condition

- To program {TRAFFIC ANALYSIS} Flexible Button,

[TRANS/PGM] + Flex. BTN + [TRANS/PGM] +  # 7 + [HOLD/SAVE]

- This feature is available at system attendant.

## UNIVERSAL NIGHT ANSWER (UNA)

Description

When CO lines are programmed for UNA and the system is placed into night mode, users can answer incoming CO line calls employing dial codes regardless of the pick-up group.

Operation

To pick-up a call in night mode,

- Lift the handset or press the [MON] button and receive intercom dial tone.

- Dial 6 9 (UNA code).

- The call is connected.

Admin Programming

- UNA Operation (PGM 31-BTN 3)

- External Night Ring (PGM 42-BTN 1)

## VOICE ANNOUNCEMENT

Description

This feature provides the station an audible system prompt as time, date, station number, call status, etc. Also, user can record his greeting so that the DVU answers an internal/DISA caller when a call is not answered within predefined time or when a call is received during busy state.

Operation

General Voice Announcement

The DVU incorporates several voice announce-ments, which are employed to indicate call progress and general announcements;

- Present Time : Dial 671

- Present Date : Dial 672

- Station Number : Dial 673

- Station Status : Dial 677 and includes,

- station number

- ICM signaling mode

- Number of message waiting

- Wake-up time

- DND state

- Forwarded to other station

- Forwarded to speed bin

- Temporary COS change

- Station COS

- Record Page Announcement : Dial 678

- Retrieve Page Announcement : Dial 679

System Greetings

Space is allocated in the DVU memory for 31 announcements for use with incoming DISA and station group calls.

- 01~09: Day DISA/Station Group Message

(Generally, 01 is for Day DISA/Station Group Message, 02 is for Night DISA/Station Group Message)

- 10: For Invalid Message

- 11: For Timeout Message

- 12: For Try Again Message

- 13: For Transfer Attendant Message

- 14: For Leave Message

- 15: For Record Message

- 16: For Enter Authorization Code Message

- 17: For MOH Message

- 18: For Busy Message

- 20: For Station Off-net Message

- 21~31: Not available

To record a system greeting from the system attendant,

- Lift the handset and intercom dial tone will be heard.

- Press the [TRANS/PGM] button.

- Dial # 4.

- Dial the announcement number (01~31).

- Record the announcement.

(In GDK-162, you should press # key to start recording.)

- Press [MON] button or [HOLD/SAVE] button to save it.

- Announcement playback is automatic as programmed.

To record a system greeting with external music source (at system attendant),

- Lift the handset and intercom dial tone will be heard.

- Press the [TRANS/PGM] button.

- Dial # 4.

- Dial the announcement number (01~31).

- Press * key.

- Record the announcement.

(In GDK-162, you should press # key to start recording.)

- Press [MON] button or [HOLD/SAVE] button to save it.

- Announcement playback is automatic as programmed.

Remote Control

An outside caller through DISA/DID can access DVU feature after calling a station which has a user greeting and preset no answer timer is expired. At this time, press the following codes;

- # : To leave a memo

- ＊+ authorization code + code no.

[74]: Play greeting

[24]: Change greeting

[34]: Delete greeting

[76]: Play message

[23]: All done (Exit & line disconnect)

User Greeting & Short Memo

When allowed access to the DVU function, the user may record a greeting. When recorded, calls to the user's station are forwarded to the greeting  if the call is unanswered. In addition, a caller receiving the greeting can record a short memo for the called station. Each greeting and memo can be recorded up to 20 seconds (programmable).

For No Answer Forwarding;

To record a user greeting for no answer forwarding,

- Lift the handset and intercom dial tone will be heard.

- Dial 674.

- Record the user greeting.

- Replace the handset and go on-hook. Call forward to the DVU is activated and the greeting is stored for 48 hours.

- Or, press the [HOLD/SAVE] button and call forward to the DVU is activated and the greeting is stored permanently until erased.

To remove the greeting and call forward to DVU,

Dial 675.

To retrieve the recorded greeting message,

Dial 676.

To activate No answer forwarding,

- Press [MON] button and [DND/FOR] button.

- Dial 7 (for activating after 4 seconds) or 8 (for activating after DVU Forward Answer Timer).

For Busy Forwarding;

To record a user greeting for busy forwarding,

- Lift the handset and intercom dial tone will be heard.

- Dial 6 7 #.

- You can hear the prompt message to record a busy message and record your own greeting message.

- Replace the handset and go on-hook.

To remove the user greeting message for busy forwarding,

Dial 6 7＊.

To retrieve the user greeting for busy forwarding,

Dial 670.

To activate the busy forwarding,

- Press [MON] button and [DND/FOR] button.

- Dial 9.

To retrieve/erase/transfer message while the message is played,

- Press the [HOLD/SAVE] button to save the current message and listen to the next message.

- Press the [CALLBK] button to listen to the message from the first.

- Press the [CONF] button to listen to the next message after deleting the current message.

- Press the associated DSS button or dial the station number to transfer the current message.

Admin Programming

- DVU Access (PGM 12-BTN 5)

- DVU Forward Answer Timer (PGM 43- BTN 24)

Condition

- To program {Call Forward} flexible button,

[TRANS/PGM] + Flex. BTN + [TRANS/PGM] +    5 4 + 7 (for No Answer forwarding after 4sec), 8 (for No Answer forwarding after DVU Forward Timer) or 9 (for Busy DVU forwarding)  + [HOLD/SAVE]

- The user should be assigned for DVU access to record a greeting or receive memo.

- If a user has recorded his greeting, the [DND/FOR] button will be flashing. To cancel, press the flashing [DND/FOR] button or dial the code 675.

- When a station which has a user greeting uses paging feature, the user greeting is announced instead of his voice.

- If a CO line is set to Special mode (PGM 30-BTN 4), the external caller will hear the System Greeting 02 and the line is disconnected automatically.

- After hearing message from a terminal which doesn't have [CONF] button (SLT or 2/4/8 BTN keyset), it is automatically erased. (Even though you have a flexible key as {CONF} button in 2/4/8 BTN keyset, the message is automatically erased.)

- DVU announcement for attendant call queuing will be applied only for attendant call from external CO line in case the previous destination is busy, error, no answer or time out call (Not available for manual/preset forwarded, attendant recall, transfer to attendant or direct attendant call).

- The user greeting for busy and no answer are recorded to different message pool. So, if both greetings are recorded together, user can use these messages independently.

- When an intercom call is received in a busy station which is forwarded to DVU for busy, the caller will hear busy user greeting. DVU busy forwarding has more priority than Camp-on or Callback feature in intercom call.

- If an incoming call through CO line is directly called or routed to a station which is forwarded to DVU for busy, the call is not routed to attendant or not followed by ring assignment. The call is directly forwarded to DVU.

- When a station is assigned to DID/DISA Call Waiting and DVU Forward for busy, the DVU forward takes precedence.

- If a CO line as multiple ring assignment, forward will not be activated.

- When unscreened transfer is made to a busy station forward to DVU, the station or CO line party placed on hold will still be hold state with hearing hold tone or music.

## VOICE OVER

Description

This feature provides voice announcements to a busy station without interrupting the existing conversation. The announcement is received over the existing conversation so that only the busy station hears both incoming parties. The user can talk back to other party and alternate conversation between the parties.

Operation

To alternate between parties from receiving keyset,

Press the [HOLD/SAVE] button.

Admin Programming

- Voice Over (PGM 12-BTN 6)

## WAKE-UP

Description

A station or attendant can register a "wake-up" or alarm time. When the time is reached, the system will signal the station with an audible and visual signal.

Operation

To register a wake-up time from the attendant station,

- Press the [TRANS/PGM] button.

- Dial 4 1.

- Dial the station range to be alerted, if a single station is to receive alarm, enter '＊' instead of the second station.

- Dial 2-digit hour and minute for alerting.

- Dial '#' for wake-up alarm until canceled, if not it is canceled automatically after the first wake-up alarm ringing.

- Press the [HOLD/SAVE] button.

To cancel a wake-up alarm from the attendant,

- Press the [TRANS/PGM] button.

- Dial 4 2.

- Dial the station range to be alerted, if a single station is to receive alarm, enter '＊' instead of the second station.

- Press the [HOLD/SAVE] button.

To register wake-up time from a station,

- Press the [TRANS/PGM] button.

- Dial 4 1.

- Dial 2-digit hour and minute to for alerting.

- Dial '#' for wake-up alarm until canceled, if not it is canceled automatically after the first wake-up alarm ringing.

- Press the [HOLD/SAVE] button.

To cancel a wake-up timer from a station,

- Press the [TRANS/PGM] button.

- Dial 4 2.

- Press the [HOLD/SAVE] button.

Admin Programming

- Erase Wake-up Fail Ring Timer

(PGM 44-BTN 24)

Condition

- If the wake-up alarm is registered at a station, '＊' is displayed in front of the present time on the LCD.

- If the DVU is installed, voice message for the wake-up time is heard 3 times and MOH is heard

- If the wake-up ring is not answered for 3 times, it is canceled.

- If [Erase Wake-up Fail Ring Timer] is 0, wake-up fail ring is not erased automatically.

- If [Erase Wake-up Fail Ring Timer] is 99, the fail ring is not given to attendant station.

- If [Erase Wake-up Fail Ring Timer] is 1~98, after the Erase Wake-up Fail Ring Timer expires, the wake-up fail ring will be erased automatically and it will be printed through RS-232C.

## WALKING COS

Description

This feature allows to temporarily override toll restriction and makes a toll call from toll restricted phone. It is activated by entering a feature code or pressing programmed [COS Override] button and the password before accessing a CO line. The password can be used as a verified account code for SMDR.

Operation

To activate this feature from a keyset,

- Press the [TRANS/PGM] button.

- Dial 1 8.

- Confirmation tone will be heard and enter the password.

- If the password is accorded, ICM dial tone will be heard. (At this time, the used extension COS is changed to the station’s COS that is accord to the entered password temporarily.)

- You can place a CO line call only one time.

To activate this feature from a SLT,

- Lift handset and dial 3 and 1 8.

- Confirmation tone will be heard and enter the password.

- If the password is vaild, ICM dial tone will be heard. (At this time, the used extension COS is changed to the station’s COS that is accorded to the entered password temporarily.)

- You can place a CO line call without toll restriction only one time.

Condition

- To program {COS OVERRIDE} Flexible Button,

[TRANS/PGM] + Flex. BTN + [TRANS/PGM +   1 8 + [HOLD/SAVE]

- This feature can be used at keyset and SLT.

- This feature is available for only one time. While you are activating this feature, if you hang up or press the [TRANS/PGM] button to hold the call and seize other line, then the COS of extension used is recovered to original COS.

- When you dialed wrong number, press the [FLASH] button to dial again without changing CO line.

- The fee for a call with Walking COS will be charged the station which is accorded to the password, not the actual using station.

# . ATTENDANT FEATURES

## ATTENDANT

Description

The system can be programmed with three types of attendants with max. 5 attendant stations in total. The type and maximum assignments for each type are;

- System Attendant - one system attendant is defined as the first main attendant

- Main Attendant - max. 5 stations can be defined as main attendants. The fist main attendant is the system attendant and it can be changed but, not removed.

- Intercom Group Attendant (only for Intercom Group) - each intercom tenancy group can be assigned to an attendant. The attendant controls the stations belonging to the intercom tenancy group.

As default, the first main attendant, and thus the system attendant, is assigned as STA 101, while other attendants are not assigned.

Admin Programming

- Intercom Tenancy Group (PGM 16)

- Main Attendant Stations (PGM 46)

## ATTENDANT CALL/QUEUING

Description

A station may place an internal call to an attendant using the abbreviated dial code ‘0’. If the attendant is busy, the station will automatically be queued to the attendant.

Operation

To call an attendant,

- Lift handset.

- Dial 0 or the intercom number of the attendant station.

Admin Programming

- Intercom Tenancy Group (PGM 16-BTN 2)

- Main Attendant Stations (PGM 46)

- Attendant Call Queuing Tone Assignment

(PGM 40-BTN 8)

Condition

- If the attendant is busy, busy tone or MOH is heard.

## ATTENDANT CLOCK SET

Description

The attendant can set the date and time of the system.

Operation

To set the date/time of the system,

- Press the [TRANS/PGM] button.

- Dial # 1.

- Dial 6-digit date (2-digit each for month/day/ year).

- Dial 4-digit time (military format).

- Press the [HOLD/SAVE] button.

- If the date or time is not to be changed, dial ＊.

To change the date display format, (toggle)

- Press the [TRANS/PGM] button.

- Dial ＊5 (Confirmation tone is heard).

- If the format is set as Day/Month/Year (DD/MM/YY), it is changed to Month/Day/ Year (MM/DD/YY).

To change the time display format, (toggle)

- Press the [TRANS/PGM] button.

- Dial ＊6 (Confirmation tone is heard).

- If the format is set as 24-hour mode, it is changed to 12-hour mode.

Admin Programming

- System Date/Time Setting (PGM 59)

- LCD Time Display Format (PGM 60-BTN 1)

- LCD Date Display Format (PGM 60-BTN 2)

## ATTENDANT DISABLE OUTGOING ACCESS

Description

The attendant may place CO lines in or out of service to remove a faulty CO line form on a temporary basis.

Operation

To place a CO line in/out-of-service from the attendant,

- Press the [TRANS/PGM] button.

- Dial ＊8.

- Press the desired CO line button. Confirmation tone is heard every time when the status for the selected line is changed.

- Press the [MON] button to return to idle.

Condition

- The LED of CO line out of service is flashing in Attendant and lighting in other stations.

- To release, press the flashing {CO LINE} button in Attendant station.

## ATTENDANT INTRUSION

Description

The attendant can intrude upon a station conversing with a CO line. And the attendant can converse with the station and the CO line.

Operation

To intrude on a call from the attendant station while receiving ICM busy tone,

- Press the programmed {ATD INTRUSION} flexible button.

- After intrusion tone, speak to the party.

Admin Programming

- Privacy (PGM 40-BTN 2)

- Privacy Warning Tone (PGM 40-BTN 3)

- Override Privilege (PGM 11-BTN 3)

Condition

- To assign {ATD INTRUSION} flexible button,

[TRANS/PGM] + Flex. BTN + [TRANS/PGM] +   8 6 + [HOLD/SAVE]

## ATTENDANT OVERRIDE

Description

A station in the DND mode can not generally receive an incoming call. The attendant however, can override the DND state at a station to signal the station of an awaiting call.

Operation

To override DND at a station from the attendant while receiving DND tone,

- Dial ＊or the last digit of the dialed station  number.

- Or, press the programmed {OVERRIDE} flexible button.

Condition

- To assign {OVERRIDE} flexible button,

[TRANS/PGM] + Flex. BTN + [TRANS/PGM] +  8 6 + [HOLD/SAVE]

## ATTENDANT RECALL

Description

Unanswered or abandoned CO line calls will recall the called station. If the recall is unanswered, the call will also be directed to the appropriate attendant station. The attendant will receive the recall signal for a period equal to the Attendant Recall Timer after which the system will return the CO line to idle.

Admin Programming

- Attendant Recall Timer (PGM 43-BTN 5)

- I-Hold Recall Timer (PGM 43-BTN 4)

Condition

- If the attendant of intercom tenancy group is not assigned, the CO ring is recalled to the system attendant.

## DAY/ON-DEMAND/NIGHT MODE

Description

The system can be placed in Day/On-demand/Night mode operation manually by the attendant or automatically, if programmed by the attendant station. Ring and COS assignments will follow the Day/On-demand/Night operation.

It is operated on On-demand mode when a CO line call is received at a station which is assigned to on-demand mode and other cases, it is operated to Day mode.

Operation

To change Day/On-demand/Night mode manually,

- Day mode;

Press the [DND/FOR] button at system attendant.

- On-demand mode;

- Press the [DND/FOR] button at system attendant.

- If the station is assigned to ring on-demand mode, it is changed to On-demand mode and if not, it is changed to Night mode.

- In On-demand mode, the LED of [DND/FOR] button will be flashing in the system attendant and the LCD on attendant’s keyset shows ‘ON DEMAND RING SERVICE’ instead of station number.

- Night mode;

- Press the flashing or off state [DND/FOR] button at system attendant.

- Pressing the lighting [DND/FOR] button to extinguish, the system will be Day mode.

- In Night mode, the LED of [DND/FOR] button is on and the LCD on attendant’s keyset shows ‘NIGHT RING SERVICE’ instead of station number.

To program automatic Day/Night mode from an attendant,

- Press the [TRANS/PGM] button.

- Dial # 9.

- Dial 4-digit for the Day mode start time (military time format).

- Dial 4-digit for the Night mode state time (military time format).

- Press the [HOLD/SAVE] button.

- If only one time is to be changed, you can dial ＊instead of the digits for the time to remain the same.

To deactivate automatic Day/Night mode from an attendant,

- Press the [TRANS/PGM] button.

- Dial # 9.

- Press the [HOLD/SAVE] button.

Admin Programming

- External Contact Assignment (PGM 51)

- External Night Ring (PGM 42-BTN 1)

- CO Line Ring Assignment (PGM 32)

Condition

- The MEMU should be installed in the system for the On-demand ring mode.

- If there is not any station assigned to ring in on-demand mode at the system, the system only operates day and night, 2 mode i.e. system attendant’s [DND/FOR] button is not flashing when pressing [DND/FOR] button continuously.

## DSS/DLS CONSOLES

Description

Attendants and other keyset stations may be equipped with DSS/DLS consoles which provide additional buttons for more convenient operation. The number of DSS/DLS console installed in the system is not restricted, but the number of DSS/DLS console operated with a station is as follows;

GDK-162: 5

GDK-100: 3

GDK-FPII: 2

The DSS/DLS consoles each require a separate line connection to the KSU, and take up a station number,

Admin Programming

- Station ID Assignment (PGM 13)

Condition

The default value for DSS/DLS is as follows;

- DSS Map 1

1:  [ATD INTRUSION]

2:  [ALL CALL PAGE]

3:  [CALL PARK 01]

4:  [INTERCOM GROUP 1]

5:  [CAMP-ON]

6:  [INT.ALL CALL PAGE]

7:  [CALL PARK 02]

8:  [INTERCOM GROUP 2]

9:  [RELEASE]

10: [EXT. ALL CALL PAGE]

11: [CALL PARK 03]

12: [INTERCOM GROUP 03]

13~48: Station Port 00~35

- DSS MAP 2 (GDK-100/162)

Flex. BTN 1~36: Station Port 36~72

Flex. BTN 37~48: not assigned as default

- DSS MAP 3 (GDK-100/162)

Flex. BTN 37~48: not assigned as default

4.  DSS MAP 4 (GDK-162)

Flex. BTN 1~48: not assigned as default

- DLS MAP 1

Flex. BTN 1~48: CO line 01~48

- DLS MAP 2 (GDK-162)

Flex. BTN 1~48: not assigned as default

## FLEXIBLE ATTENDANT

Description

The system attendant can forward (Unconditional Forward) a call to the other station and the forwarded station can be activated as the attendant temporarily for forwarded period. The forwarding attendant station cannot activate as system attendant and activate as other stations for the call forwarding time.

## INTERCOM BOX MUSIC SELECT

Description

The attendant can select the music channel source to be provided to the Intercom Box(es).

Operation

To select the music source from the attendant,

- Press the [TRANS/PGM] button.

- Dial ＊0.

- Dial the music source (0~9).

Channel 0 (Music is not used.)

Channel 1

Channel 2

:

###### Channel 9

- Press the [HOLD/SAVE] button.

Admin Programming

- ICM Box Music Channel (PGM 41-BTN 3)

- Music Source on Channel 1 (PGM 41-BTN 4)

## RESTORING/BLOCKING DID CALL

Description

Attendant can change DID call receiving mode of station in the system.

Operation

To block DID call,

- Press the [TRANS/PGM] button.

- Dial 5 5.

- Enter the station range.

- Press the [HOLD/SAVE] button.

To restore DID call,

- Press the [TRANS/PGM] button.

- Dial 5 6.

- Enter the station range.

- Press the [HOLD/SAVE] button.

Admin Programming

- DIDR (PGM 21-BTN 1)

Condition

- DIDR field allows the station call when DID line is presented at the station.

- If a call is received when DID calls are restricted by the attendant, the ring is routed by ring assignment (PGM 32). If ring assignment is not registered, the call is routed to the attendant.

## STATION FEATURE CANCEL

Description

The attendants can cancel features such as; DND, Call Forward and preselected messages assigned at other stations.

Operation

To disable active features at a station from the attendant,

- Press the [TRANS/PGM] button.

- Dial ＊9.

- Dial the desired station range.

- Press the [HOLD/SAVE] button.

Description

- Do-Not-Disturb (PGM 10-BTN 1)

- Call Forward (PGM 10-BTN 4)

- Intercom Tenancy Group (PGM 16)

- Main Attendant Stations (PGM 46)

# . SLT FEATURES

## ABILITY FOR SLT TO PROGRAM NAME

Description

Entering the name in a SLT, it is displayed instead of the station number while dialing by name or calling a keyset with LCD.

Operation

To register the name in a SLT,

- Lift the handset in a SLT.

- Dial 3 (Confirmation tone is heard.)

- Dial 3 0 (Name Register code).

- Enter the name. (Up to 7 letters; Refer to the below English Character Set)

- Press the [FLASH] button to save.

| Q - 11 Z – 12 . - 13 1 - 10 | A - 21 B - 22 C - 23 2 - 20 | D - 31 E - 32 F - 33 3 - 30 |
| --- | --- | --- |
| G - 41 H - 42 I - 43 4 - 40 | J - 51 K - 52 L - 53 5 - 50 | M - 61 N - 62 O - 63 6 - 60 |
| P - 71 R - 72 S - 73 Q - 7* 7 - 70 | T - 81 U - 82 V - 83 8 - 80 | W - 91 X - 92 Y - 93 Z - 9# 9 - 90 |
| *1 - Blank *2 - : *3 - , | 0-00 | # |

< English Character Set >

## BROKERS CALL

Description

A user of single line telephone can alternate between two calls maintaining private conversations with both parties. The parties may be either internal (stations connected to the system) or external CO line calls, and may be incoming or outgoing.

Operation

To alternate between two calls,

Press and release the hook-switch.

## CALL FORWARD

Description

A user of single line telephone can forward a call to other stations in the system.

Operation

To activate call forward from a SLT,

- Lift the handset.

- Dial call forward code 5 4.

- Dial call forward type (0~5).

- Dial station to receive calls.

- Replace the handset and go on-hook.

To deactivate call forward from a SLT,

- Lift the handset.

- Dial call forward code 5 4 and #.

- Confirmation tone is heard and replace the handset.

Admin Programming

- Call Forward Access (PGM 10-BTN 4)

- Call Forward No Answer Timer (PGM 44-BTN 7)

Condition

- Call Forward type is as follows;

- 1: Unconditional

- 2: Busy

- 3: No answer

- 4: Busy/No answer

- 5: Off-net

- 7: Forward to DVU (After 4 sec)

- 8: Forward to DVU (After predefined time)

- Call forward is maintained until it is released.

- If you try to call forward in DND state, the DND is canceled and the call is not forwarded.

## CALL TRANSFER

Description

A user of single line telephone may transfer calls on CO lines to other stations in the system. If the receiving station does not answer the call in the Transfer Hold Recall Time, the transferring station will receive recall. If the call remains unanswered, the attendant will also receive a recall for the Attendant Recall Time after which the CO line will be disconnected and returned to idle.

Operation

To make an unscreened transfer,

- Press and release the hook-switch and intercom dial tone is heard.

- The CO line is placed on exclusive hold and the Recall timer is activated.

- Dial the station number to be transferred.

- Replace the handset and go on-hook.

To make a screened transfer,

- Press and release the hook-switch and intercom dial tone is heard.

- The CO line is placed on exclusive hold and the Recall timer is activated.

- Dial the station number to be transferred.

- When the station answers, announce transfer.

- Replace the handset and go on-hook.

Admin Programming

- Transfer Hold Recall Timer (PGM 43-BTN 3)

- I-Recall Timer (PGM 43-BTN 4)

- Attendant Recall Timer (PGM 43-BTN 5)

- No Touch Answer (PGM 12-BTN 4)

## CALL WAITING / CAMP-ON

Description

A user of single line telephone may notify a busy station that an outside CO line or internal call is waiting. The busy station is notified of the call by a camp-on tone.

Operation

To activate call waiting while receiving intercom busy tone,

- Dial ＊ or the last digit of the dialed station number.

- Await answer to announce the call.

- Or, replace the handset and go on-hook.

## CO LINE QUEUING

Description

When a user of single line telephone receives a busy signal during an attempt to access a CO line, the user may request a call back (queue) when the CO line is available. The station will receive a call back when the busy CO line or a CO line in the CO line group becomes available.

Operation

To activate queuing while receiving busy tone,

- Press and release hook-switch.

- Dial 5 6.

- Confirmation tone is heard and replace the handset.

- When the CO line becomes idle, callback ring is received at the SLT.

Admin Programming

- CO Line Queuing (PGM 11-BTN 1)

## DO NOT DISTURB

Description

A user of single line telephone, allowed in the system database, can place a call in DND, thus blocking incoming intercom and CO ring with the exception of recalls.

Operation

To activate DND from a SLT,

- Lift the handset.

- Dial 5 3.

- Replace the handset and go on-hook.

To deactivate DND from a SLT,

- Lift the handset.

- Dial 5 3.

- Replace the handset and go on-hook.

Admin Programming

- DND (PGM 10-BTN 1)

## HOWLING TONE

Description

When a station goes off-hook and does not initiate dialing in the Dial-tone Time-out duration or, if the duration between dialed digits exceeds the Inter-digit Timer, the station will receive error tone for about 30sec and howler tone as an error indication. And, the call attempt will be abandoned. In order to complete the call, the user should return to on on-hook state and restart the call.

Admin Programming

- Howling Tone to SLT (PGM 11-BTN 8)

## MESSAGE WAIT/CALL BACK

Description

When calling a SLT which is busy, does not answer or in DND, call back or message waiting can be activated. A SLT may receive a message wait or call back from up to 5 other stations. The SLT receiving the message can call the parties that left the message wait.

Operation

To leave a message wait,

- Press hook-switch.

- Dial 5 6.

- Confirmation tone is heard and replace the handset.

To answer the message wait,

- Lift the handset.

- Dial 5 7.

- The station leaving the message will receive intercom ring.

To leave a call back to a busy SLT,

- Dial 5 6 while busy tone is heard.

- Confirmation tone is heard and replace the handset.

- When the SLT becomes idle state, intercom ring is received.

## MESSAGE WAITING INDICATION

Description

Industry standard message waiting telephones may be connected to the system and, under software control, the system will provide appropriate signals to flash the Message Waiting lamp.

Admin Programming

- Station ID Assignment (PGM 13)

## PICK-UP (DIRECT/GROUP)

Description

A SLT station can pick-up a call ringing other station by pressing pick-up code. Tone ringing intercom calls, incoming CO lines, recalling CO line and transferred CO lines can be answered by a SLT station instead of the ringing station.

Operation

To answer a call ringing at any other station,

- Lift the handset.

- Dial Directed Call Pick-up code 7.

- Dial the intercom number of ringing station.

To answer a call ringing at a station in your pick-up group,

- Lift the handset.

- Dial Group Pick-up code 6 6.

Admin Programming

- Station Group (PGM 47)

- Group Attribute (PGM 48)

Condition

- Private line calls are not subject to Directed Call Pick-up.

## SIGNALING

Description

The ring cadence for SLT can be changed by changing ring signal.

Admin Programming

- SLT Ring Phase Time (PGM 44-BTN 19)

## SPEED DIAL

Description

You can use Speed Dial at SLT by dialing the code.

Operation

To store Station Speed Dial numbers from a SLT,

- Lift the handset.

- Dial the Speed Dial program code 5 5.

- Dial the Speed Dial bin number (00~19).

- Dial the desired phone number (max.24 digits).

- Press and release the hook-switch.

To place a call using Station Speed Dial from a SLT,

- Lift the handset.

- Dial the Speed Dial access code 5 8.

- Dial the Station Speed dial bin (00~19).

To place a call using System Speed Dial from a SLT,

- Lift the handset.

- Dial the Speed Dial access code 5 8.

- Dial the System Speed dial bin.

Admin Programming

- Speed Dial Access (PGM 10-BTN 2)

- System Speed Zone (PGM 67)

# .  INDEX

2B Phone, 2-13

Account Code, 3-1

Advice Of Charge, 3-1

Alarm, 3-2

Attendant, 4-1

Flexible Attendant, 4-4

Attendant Call/Queuing, 4-1

Attendant Cancel Feature, 4-5

Attendant Clock Set, 4-1

Attendant Disable Outgoing Access, 4-2

Attendant Intrusion, 4-2

Attendant Override, 4-2

Attendant Recall, 4-3

Authorization Code, 3-2

Auto Call Number Redial, 1-1

Auto Call Release, 3-3

Auto Release of [MON] Button, 3-4

Automatic Call Distribution, 3-3

Automatic Hold, 1-1

Automatic Privacy, 3-4

Background Music, 2-1

Brokers Call, 5-1

Busy Lamp Field, 2-1

Calculator, 2-1

Calendar Display, 2-2

Call Cost Display, 3-4

Call Forward

Busy/No Answer, 2-2

Call Forward Button Program, 2-2

Follow Me, 2-3

In SLT, 5-1

Incoming CO Line Off-net Call Forward, 2-3

Preset, 2-4

Station Off-Net, 2-4

Unconditional, 2-4

Call Park, 1-2

Retrieving Parked Call with QSIG, 3-17

Call Pick-up

Direct Call Pick-up, 2-5

Direct Call Pick-up in ICM Box, 2-6

Group Call Pick-up, 2-7

In SLT, 5-4

Call Transfer, 1-2

CO Call Transfer to CO Line, 1-4

In SLT, 5-2

Call Waiting/Camp-on, 2-5

In SLT, 5-2

Calling Line Identification Presentation, 1-2

CLI, 1-2

Canned Toll Restriction, 3-4

Change Keyset Ring Cadence, 3-6

Class Of Service, 3-6

Temporary Station COS Change, 3-20

Walking COS, 3-26

CO Call Drop, 1-3

CO Line Access, 1-5

In SLT, 1-5

CO Line Groups, 1-5

CO Line Queuing, 1-6

In SLT, 5-2

CO Line Release Guard Time, 1-6

CO Line Ring Assignment, 1-6

CO Ring Detect, 1-7

Conference, 3-7

Connected Line Identification Presentation, 1-7

Connected Line Identification Restriction, 1-7

Custom Messages, 3-12

Customer Call Routing, 3-5

Data Line Security, 3-8

Day/On-demand/Night Mode, 4-3

Delayed CO Line Ring, 3-8

Dial By Name, 3-8

Dial By Name

Enhanced Dial By Name, 3-11

Dial Memo, 1-8

Dial Pulse Sending, 1-8

Dial Pulse to Tone Switch Over, 1-9

Differential Ring, 3-9

Direct Inward System Access, 3-10

Direct Inwards Dialing, 3-9

Display Security, 1-9

Distinct CO Ring, 1-9

Do Not Disturb, 2-6

In SLT, 5-3

One-time DND, 2-11

DSS/DLS Consoles, 4-4

DTMF Signal Sending, 1-9

Dummy Dial Tone, 3-10

Duration Timer, 3-10

Emergency Call, 3-10

Executive/Secretary Transfer, 1-10

Flash

CO Flash, 1-4

Flash on ICM Call, 2-7

Flexible DID on QSIG, 3-11

Function Button Assignment, 3-12

Headset/Speakerphone Mode, 2-7

Hold Preference, 1-10

Exclusive Hold Recall Timer, 1-9

System Hold Recall Timer, 1-14

Hot Line/Warm Line, 2-7

Howling Tone, 5-3

Intercom Box, 2-8

Intercom Box Music Select, 4-4

Intercom Call, 2-8

Intercom Call On Hold, 2-9

Intercom Signal Mode, 2-9

Intercom Tenancy Group, 2-10

Keypad Facility Service, 1-11

Last Number Redial, 1-11

Least Cost Routing, 3-13

Linked Station Pair, 2-10

Lock-out for Only Intercom, 2-10

Loud Bell Control, 3-15

Memory Full Warning of VM System, 3-16

Message Waiting

CO Message Waiting, 3-7

Message Waiting/Callback, 3-15

Message Waiting/Callback in SLT, 5-3

Message Waiting Indication in SLT, 5-3

MSN/Sub-Addressing, 2-11

CPN (Called Party Number, 2-11

Music On Hold (MOH), 1-12

MOH Channel via SLT Port, 3-16

Mute, 3-17

Name Display

Ability for SLT To Program Name, 5-1

CO Line Name Display, 1-5

Off-hook Ringing, 2-11

On-hook Dialing, 2-12

Paging, 2-12

Meet Me Page, 2-11

Page Access Restriction, 2-6, 2-13

Paging Conference Group, 2-13

Pause Insertion

Automatic Pause Insertion, 1-1

Pause Insertion in Speed Dial, 1-12

Preferred Line Answer, 1-12

Preselected Messages, 3-12

Private Line, 1-12

Restoring/Blocking DID Call, 4-5

Save Number Redial, 1-13

Serial Calling, 2-13

Signaling, 5-4

SMDR

Lost Call Printing, 3-18

SMDR, 3-17

Speakerphone, 3-18

Speed Dial

In SLT, 5-4

Station Speed Dial, 1-13

System Speed Dial, 1-14

System Speed Zone Group, 1-15

Station Group

Circular Station Group, 3-19

Ring Station Group, 3-19

Terminal Station Group, 3-19

UCD Station Group, 3-19

Voice Mail Station Group, 3-19

Step Call

CO Step Call, 1-8

Intercom Step Call, 2-9

Traffic Analysis, 3-21

Universal Night Answer, 3-22

Voice Announcement, 3-22

Voice Over, 3-25

Wake-up, 3-25

World Time Display, 1-15