SECTION 5. CUSTOMER DATABASE PROGRAMMING

### INTRODUCTION

The GDK Key Telephone System can be programmed to meet each customer's individual need. All programming is done at station 100 (station port # 00) using KD-36D digital key telephone. (You cannot program with Large LCD keyset.)

Upon entering the program mode, the key telephone at station 100 cannot operate as a normal telephone but as a programming instrument with all of the buttons redefined. The keys of the dial pad are used to enter the various data fields and to enter numerical information. The 24 buttons located at the top of the phone (Flexible Buttons) are used to indicate the specific data field and to enter information. Sometimes the [SPEED] button and ‘*’ of the dial pad is used to delete the data or to indicate end of data input.

See TABLE 5.5.1 - 5.5.9 for default data. If this pre-programming suits the customer, additional admin program is not necessary. To change admin data, the user enters the admin. programming mode and select program code. During admin programming other keysets operate normally.

When Admin programming, LCD and LEDs indicate the current programmed data and status. If the programmer enters correct data, then LCD and LEDs show the entered data and the data is stored in the temporary buffer area. Real system databases is not changed and has no effect on telephone operation unless permanent-updating procedure is executed. Pressing [HOLD/SAVE] button, all data in the temporary buffer (same as LCD and LEDs show their status) is saved into permanent memory. Tones are provided to let the programmer know data entry is correct (confirmation tone) or not (error tone).

### TO ENTER THE PROGRAMMING MODE

- Lift handset or press the [MON] button on the admin station, and hear ICM dial tone (optional).

- Press the [TRANS/PGM] button and dial ＊# (Confirmation tone is heard).

- Enter Admin password if the password has been set. This places the station into the admin programming mode (confirmation tone heard).

Each program is accessed by pressing the [TRANS/PGM] button and dialing the two-digit program number. If an error is made while entering data, the [] or the [TRANS/PGM] button can go the previous status. When the [TRANS/PGM] button is pressed, the LCD will display;

ENTER PROGRAM CODE

### PERMANENT UPDATE PROCEDURE

When the data has been entered, the [HOLD/SAVE] button is used to store the data permanently. If all data was entered correctly, confirmation tone is heard when pressing the [HOLD/SAVE] button. If there were any errors in the entry, then an error tone is presented and data is not stored in the permanent memory.

### NUMBERING PLAN

The following numbering plan can be changed by Admin Programming 01 depending on the user's needs.

| NUMBER |  | ITEM | REMARK |
| --- | --- | --- | --- |
| 162 | 100–291 | Station Number |  |
| 100 | 100–235 |  |  |
| FPII | 100–177 |  |  |
| 162 | 620–649 | Station Group Number |  |
| 100 | 620–634 |  |  |
| FPII | 620-627 |  |  |
| 3 |  | PGM Enter Code | SLT |
| 162 | 401-420 | Internal Page Zones |  |
| 100 | 401-415 |  |  |
| FPII | 401-405 |  |  |
| 43 |  | Internal All Call Paging |  |
| 44 |  | Meet Me Page |  |
| 46 |  | External Page Zone -1 |  |
| 47 |  | External Page Zone -2 |  |
| 48 |  | External All Call Paging |  |
| 49 |  | All Call Paging |  |
| 50 |  | Enter SMDR Account Code | 4/8 BTN Keyset, SLT |
| 51 |  | Flash Command to CO Line |  |
| 52 |  | Last Number Redial (LNR) |  |
| 53 |  | Do Not Disturb |  |
| 54 |  | Call Forward |  |
| 55 |  | Station Speed Dial |  |
| 56 |  | Message Wait Enable |  |
| 57 |  | Message Wait Answer |  |
| 58 |  | Speed Dial Access |  |
| 59 |  | Cancel DND/Call Forward/Pre-selected MSG |  |
| 162 | 601-619 | Call Park Location |  |
| 100 | 601-610 |  |  |
| FPII | 601-605 |  |  |
| 65 |  | Alarm Reset |  |
| 66 |  | Group Call Pick-up |  |

| NUMBER |  | ITEM | REMARK |
| --- | --- | --- | --- |
| 670 |  | DVU – Record User Greeting |  |
| 671 |  | DVU – Time Announcement |  |
| 672 |  | DVU – Date Announcement |  |
| 673 |  | DVU – Station Number Announcement |  |
| 674 |  | DVU – Record User Greeting |  |
| 675 |  | DVU – Delete User Greeting |  |
| 676 |  | DVU – Play User Greeting |  |
| 677 |  | DVU – Station Statue Announcement |  |
| 678 |  | DVU – Record Page Announcement |  |
| 679 |  | DVU – Retrieve Page Announcement |  |
| 69 |  | Universal Night Answer (UNA) |  |
| 7XXX |  | Direct Call Pick-up (XXX = Station Number) |  |
| 162 | 801-848 | CO Group Access |  |
| 100 | 801-824 |  |  |
| FPII | 801-809 |  |  |
| 162 | 8801-8896 | CO Line Access |  |
| 100 | 8801-8848 |  |  |
| FPII | 8801-8834 |  |  |
| 89 |  | Tie Routing Access |  |
| 8## |  | Retrieve Holding CO Group | Keyset |
| 162 | 8#01-8#96 | Retrieve Holding CO Line | Keyset |
| 100 | 8#01-8#48 |  |  |
| FPII | 8#01-8#34 |  |  |
| 9 |  | Access the first CO line |  |
| 0 |  | Attendant Call |  |
| *0 |  | Print SMDR (Station Base) | Attendant |
| *1 |  | Print SMDR (Account Group Base) | Attendant |
| *2 |  | Delete SMDR (Station Base) | Attendant |
| *3 |  | Delete SMDR (Account Group Base) | Attendant |
| *4 |  | Display Call Charge on LCD | Attendant |
| ** |  | Abort SMDR Printing | Attendant |
| #*1 |  | Door Open 1 |  |
| #*2 |  | Door Open 2 |  |
| #*3 |  | Door Open 3 |  |
| #*4 |  | Door Open 4 |  |
| *8 |  | VM Message Wait Enable |  |
| *9 |  | VM Message Wait Cancel |  |
| [SPEED] + * |  | Last Number Redial |  |
| [SPEED] + # |  | Save Number Redial | Keyset |
| [SPEED] + 00-19 |  | Station Speed Dial Access | Keyset |
| [SPEED] + XXX |  | System Speed Dial Access | XXX (Bin No. of System Speed Dial) 162: 200-999 100/FPII: 200-399 |

- To enter user programming mode, press the [TRANS/PGM] button in a keyset or dial 3 (Programming enter code) in a SLT.

- The following numbering plan is fixed, cannot be changed by Admin. Programming.

| NUMBER | ITEM |  | REMARK |
| --- | --- | --- | --- |
| 18 | Walking COS |  | Keyset |
| 30 | Station User Name Registration |  | 4/8 BTN Keyset, SLT |
| 25 | Receive Station Number from KSU |  | WHTU only |
| 29 | Erase Subscription Data |  | ATD only |
| 2* | Display Date / Time |  | WHTU only |
| 40 | MPB Version Display |  | Keyset |
| 41# | Wake-up Time Registration (Continuous) |  |  |
| 41 | Wake-up Time Registration (One-time) |  |  |
| 42 | Wake-up Time Cancel |  |  |
| 43 | SMEMU Serial Number Display |  | Keyset with LCD |
| 44 | Authorization Code Registration |  |  |
| 45 | Authorization Code Change |  |  |
| 46 | Station COS Change |  |  |
| 47 | Station COS Restore |  |  |
| 48 | Custom Message Display (Message 00) |  |  |
| 49 | Intercom Answer Mode (1 HF / 2 TONE / 3 PV) |  | Digital Keyset |
| 50 | Differential Ring |  |  |
| 51 | LCD Display Mode (English/Korean) |  | Keyset |
| 52 | Set World Time |  | “ |
| 54 | [RECORD] BTN Assignment – With Voice Mail |  | Keyset |
| 55 | Restrict DID Ring |  | System Attendant |
| 56 | Restore DID Ring |  | System Attendant |
| 57 | Headset/Speakerphone Mode |  | Keyset |
| 58 | [COLR] BTN Assignment |  | “ |
| 59 | Erase All CO Message Wait |  | “ |
| 61 | Enable/Disable DID Call Wait |  | “ |
| 62 | Enable/Disable Send Keypad Facility IE |  | “ |
| 73 | Background Music |  | " |
| 77XX | Custom Message Display (XX = 00-20) |  | " |
| 80 | [Dial Memo] BTN Assignment |  | " |
| 81 | [Account Code] BTN Assignment |  | " |
| 83 | [ICM Hold] BTN Assignment |  | " |
| 84 | [LOOP] BTN Assignment |  | " |
| 85 | [Camp-on] BTN Assignment |  | " |
| 86 | [ATD Override] BTN Assignment |  | " |
| 87 | [UCD DND] BTN Assignment |  | " |
| 88 | [ACNR] BTN Assignment |  | " |
| 89 | [Stop Watch] BTN Assignment |  | " |
| 8* | [ACD] BTN Assignment |  | Digital Keyset |
| 8# | [Data Module] BTN Assignment |  | Digital Keyset |
| NUMBER | ITEM |  | REMARK |
| 90 | [SPEED] BTN Assignment |  | 4/8 BTN Keyset |
| 91 | [CONF] BTN Assignment |  | " |
| 92 | [CALLBK] BTN Assignment |  | " |
| 93 | [DND/FWD] BTN Assignment |  | " |
| 94 | [FLASH] BTN Assignment |  | " |
| 95 | [MUTE] BTN Assignment |  | " |
| 96 | [MON] BTN Assignment |  | " |
| 97 | [CLIR] BTN Assignment |  | Keyset |
| 41# | Wake-up Time Registration (Continuous) |  | Attendant |
| 41 | Wake-up Time Registration (One-time) |  | " |
| 42 | Wake-up Time Cancel |  | " |
| *4 | Authorization Code Cancel |  | System Attendant |
| *5 | LCD Date Mode Change |  | " |
| *6 | LCD Time Mode Change |  | " |
| *7 | Custom Display Message Program (11-20) |  | " |
| *8 | CO Line Access |  | Attendant |
| *9 | DND/Call Forward/Pre-selected MSG Cancel |  | " |
| *0 | Door Phone Music Selection |  | " |
| #1 | System Date/Time Setting |  | " |
| #2 | Station COS Change (COS 7) |  | " |
| #3 | Station COS Restore |  | " |
| #4 | DVU - Record System Greeting | System Attendant |  |
| #5 | External Page Music -1 Assignment/Cancel | Attendant |  |
| #6 | External Page Music -2 Assignment/Cancel | Attendant |  |
| #71 | Start Traffic Analysis | System Attendant |  |
| #72 | Disable Traffic Analysis | " |  |
| #73 | Print Traffic Analysis | " |  |
| #74 | Abort Traffic Analysis | " |  |
| #8 | Dial By Name | " |  |
| #9 | Automatic Day/Night Mode Program | Attendant |  |
| ## | Subscribe WHTU |  | ATD only |
| *# | Admin Programming Code |  |  |

### ADMIN PROGRAMMING INDEX

| INITIALIZATION | PGM 00 | Database Initialization |
| --- | --- | --- |
| FLEXIBLE NUMBERING PLAN | PGM 01 | Flexible Numbering Plan |
| SLOT ASSIGNMENT | PGM 02 | Slot Assignment |
| MSN/SUB ADDRESS ASSIGNMENT | PGM 03 PGM 03 PGM 04 | MSN/Sub-address Assignment (GDK-162) PTRP Gain Table Program (GDK-100/FPII) PTRP Gain Table Program (GDK-162) |
| FLEXIBLE DID TABLE | PGM 05 | Flexible DID Table |
| EMERGENCY SERVICE CALL TABLE | PGM 06 | Emergency Call |
| LCR(LEAST COST ROUTING) | PGM 07 | LCR |
| SYSTEM ATTRIBUTE – IV | PGM 08 | System Attribute - IV |
| ISDN COLP TABLE | PGM 09 | ISDN COLP TABLE |
| STATION BASE PROGRAM | PGM 10 | Station Attribute - I |
|  | PGM 11 | Station Attribute - II |
|  | PGM 12 | Station Attribute - III |
|  | PGM 13 | Station ID Assignment |
|  | PGM 14 | Station Class-Of-Service |
|  | PGM 15 | CO Line Group Access |
|  | PGM 16 | ICM Tenancy Group |
|  | PGM 17 | Internal Page Zone |
|  | PGM 18 | Preset Call Forward |
|  | PGM 19 | Hot Line/Warm Line |
|  | PGM 20 | SMDR Account Group |
|  | PGM 21 | Station Attribute - IV |
|  | PGM 22 | CTI Attribute |
|  | PGM 23 | ISDN Station Attributes - V |
|  | PGM 24 | Station Attribute - VI |
|  | PGM 28 | Linked Pairs |
|  | PGM 29 | Flex Buttons Assignment |
| CO LINE BASE PROGRAM | PGM 30 | CO Line Attribute – I |
|  | PGM 31 | CO Line Attribute - II |
|  | PGM 32 | CO Line Ring Assignment |
|  | PGM 33 | CO Flash Timer |
|  | PGM 34 | Open Loop Detect Timer |
|  | PGM 35 | TIE Line Assignment |
|  | PGM 36 | DID Line Assignment |
|  | PGM 37 | CO Line Attribute – III |
|  | PGM 38 | CO Line Attribute – IV |
|  | PGM 39 | R2 DCOB CO Line Attribute |

| SYSTEM BASE PROGRAM | PGM 40 | System Attribute - I |
| --- | --- | --- |
|  | PGM 41 | System Attribute - II |
|  | PGM 42 | System Attribute - III |
|  | PGM 43 | System Timer - I |
|  | PGM 44 | System Timer - II |
|  | PGM 45 | Admin Password |
|  | PGM 46 | Main Attendant Assignment |
|  | PGM 47 | Station Group |
|  | PGM 48 | Station Group Attribute |
|  | PGM 49 | Executive/Secretary Pairs |
|  | PGM 50 | Alarm Attribute |
|  | PGM 51 | External Control Contact |
|  | PGM 52 | PBX Access Code |
|  | PGM 53 | SMDR Long Distance Codes |
|  | PGM 54 | Authorization Code |
|  | PGM 55 | DID Digit Conversion |
|  | PGM 56 | DID/DISA Destination |
|  | PGM 57 | Pulse Dial Speed Ratio |
|  | PGM 58 | Modem Assignment |
|  | PGM 59 | Setting System Date/Time |
|  | PGM 60 | LCD Date/Time Display Mode |
|  | PGM 61 | S/W Version Display |
|  | PGM 62 | SMDR Attribute - I |
|  | PGM 63 | SMDR Attribute - II |
|  | PGM 65 | TIE Routing Tables |
|  | PGM 66 | System Gain Control |
|  | PGM 67 | System Speed Zone Programming |
|  | PGM 68 | ACNR Tone Cadence |
|  | PGM 69 | System Tone Frequency |
|  | PGM 71 | World Time Attribute |
|  | PGM 72 | Display World Time |
|  | PGM 73 | Voice Mail Dialing Codes |
|  | PGM 74 | CO Line Name Assignment |
|  | PGM 75 | Ring Frequency (Called Station base) |
|  | PGM 76 | Ring Frequency (Calling Station base) |
|  | PGM 77 | DCO TX Gain Control |
|  | PGM 78 | Digit Insert Table |
|  | PGM 79 | CCR Table |
|  | PGM 92 | Nation Specific Attribute - I (For Australia) |
|  | PGM #1 | System Attributes - V |
| TOLL TABLE | PGM 70 | Exception Table |
|  | PGM 64 | Canned Toll Exception Table |
| PRINT DATABASE | PGM 80 | Database Print |
| QSIG TABLE | PGM *1 | QSIG Basic Attribute |
|  | PGM *2 | QSIG Routing Table |
|  | PGM *3 | QSIG MSN/Sub-Address |
|  | PGM *4 | DDI for ISDN Service |
|  | PGM *5 | Line Identification |

### DEFAULT VALUES

※ The default values may be different from country adaptation.

### TABLE 5.6.1	INITIALIZATION

| PGM | BTN | ITEM | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 00 | 1 | Station Data Initialization | - |  |
|  | 2 | CO Line Data Initialization | - |  |
|  | 3 | System Data Initialization | - |  |
|  | 4 | Flex. Buttons Data Initialization | - |  |
|  | 5 | Toll Table Initialization | - |  |
|  | 6 | Flexible Numbering Plan Initialization | - |  |
|  | 7 | Slot Assignment Initialization | - |  |
|  | 8 | All Data Initialization | - |  |
|  | 9 | Nation / Extend Initialization | KOREA/BASIC |  |
|  | 10 | System S/W Reset | - |  |

### TABLE 5.6.2	FLEXIBLE NUMBERING PLAN – GDK-162

| PGM | BTN | ITEM | BASIC |  | EXTEND |  | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 01 |  | First 24 Flexible Number |  |  |  |  |  |
|  | 1 | Station Number | 100-291 |  | 100-699 |  |  |
|  | 2 | Station Group Number | 620-649 |  | *620-*649 |  |  |
|  | 3 | Station PGM Enter Code | 3 |  | *3 |  | SLT |
|  | 4 | Internal Page Zones | 401-420 |  | *401-*420 |  |  |
|  | 5 | Internal All Call Page | 43 |  | *43 |  |  |
|  | 6 | Meet Me Page | 44 |  | *44 |  |  |
|  | 7 | External Page Zone - 1 | 46 |  | *46 |  |  |
|  | 8 | External Page Zone - 2 | 47 |  | *47 |  |  |
|  | 9 | External All Call Page | 48 |  | *48 |  |  |
|  | 10 | All Call Page (EXT/INT) | 49 |  | *49 |  |  |
|  | 11 | Enter SMDR Account Code | 50 |  | *50 |  |  |
|  | 12 | Flash Command to CO line | 51 |  | *51 |  |  |
|  | 13 | Last Number Redial (LNR) | 52 |  | *52 |  |  |
|  | 14 | Do-Not-Disturb | 53 |  | *53 |  |  |
|  | 15 | Call Forward | 54 |  | *54 |  |  |
|  | 16 | Speed Dial Programming | 55 |  | *55 |  |  |
|  | 17 | MSG Wait/Call-Back Enable | 56 |  | *56 |  |  |
|  | 18 | MSG Wait/Call-Back Answer | 57 |  | *57 |  |  |
|  | 19 | Speed Dial Access | 58 |  | *58 |  |  |
|  | 20 | Cancel DND/CFW / Pre-selected MSG | 59 |  | *59 |  |  |
|  | 21 | Call Park Locations | 601- 619 |  | *601-*619 |  |  |

| PGM | BTN | ITEM | ###### BASIC |  | EXTEND |  | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 01 |  | First 24 Flexible Number |  |  |  |  |  |
|  | 22 | Alarm Reset | 65 |  | *65 |  |  |
|  | 23 | Group Call Pick-Up | 66 |  | *66 |  |  |
|  | 24 | Station DVU Access | 67 |  | *67 |  |  |
| 01 |  | Second 24 Flexible Number |  |  |  |  |  |
|  | 1 | Night Answer | 69 |  | *69 |  |  |
|  | 2 | Direct Call Pick-Up | 7 |  | *7 |  |  |
|  | 3 | CO Group Access | 801-848 |  | 801-848 |  |  |
|  | 4 | CO Line Access | 8801-8896 |  | 8801-8896 |  |  |
|  | 5 | Tie Routing Access | 89 |  | 89 |  |  |
|  | 6 | Retrieve Held CO Group | 8## |  | 8## |  |  |
|  | 7 | Retrieve Held CO Line | 8#01-8#96 |  | 8#01-8#96 |  |  |
|  | 8 | Access to CO Line in the 1st available CO Group | 9 |  | 9 |  |  |
|  | 9 | Attendant Call | 0 |  | 0 |  |  |
|  | 10 | Print SMDR (Station Base) | *0 |  | **0 |  |  |
|  | 11 | Print SMDR (Account Group Base) | *1 |  | **1 |  |  |
|  | 12 | Delete SMDR (Station Base) | *2 |  | **2 |  |  |
|  | 13 | Delete SMDR (Account Group Base) | *3 |  | **3 |  |  |
|  | 14 | Display Call Charge On LCD | *4 |  | **4 |  |  |
|  | 15 | Abort SMDR Printing | ** |  | *** |  |  |
|  | 16 | Door Open – 1 | #*1 |  | #*1 |  |  |
|  | 17 | Door Open – 2 | #*2 |  | #*2 |  |  |
|  | 18 | Door Open – 3 | #*3 |  | #*3 |  |  |
|  | 19 | Door Open – 4 | #*4 |  | #*4 |  |  |
|  | 20 | VM MSG Wait Enable | *8 |  | *8 |  |  |
|  | 21 | VM MSG Wait Cancel | *9 |  | *9 |  |  |
|  | 22 | UCD DND |  |  |  |  |  |
|  | 23 | Station ISDN Flash |  |  |  |  |  |

### TABLE 5.6.3	FLEXIBLE NUMBERING PLAN – GDK-100

| PGM | BTN | ITEM | NUM SET 2 | NUM SET 3 | NUM SET 4 | NUM SET 5 | NUM SET 6 | NUM SET 7 | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 |  | First 24 Flexible Number |  |  |  |  |  |  |  |
|  | 1 | Station Number | 100-235 | 100-235 | 100-235 | 700-835 | 200-335 | 10-79 |  |
|  | 2 | Station Group Number | 620-634 | *620-*634 | 620-634 | 9700-9714 | 620-634 | *620-*634 |  |
|  | 3 | Station PGM Enter Code | 3 | *3 | *3 | *3 | 1 | *3 | SLT |
|  | 4 | Internal Page Zones | 401-415 | *401-*415 | #01-#15 | #01-#15 | 401-415 | *401-*415 |  |
|  | 5 | Internal All Call Page | 43 | *43 | #5 | #3 | 43 | *43 |  |
|  | 6 | Meet Me Page | 44 | *44 | ## | ## | 44 | *44 |  |
|  | 7 | External Page Zone - 1 | 46 | *46 | #6 | #41 | 46 | *46 |  |
|  | 8 | External Page Zone - 2 | 47 | *47 | #7 | #42 | 47 | *47 |  |
|  | 9 | External All Call Page | 48 | *48 | #8 | #5 | 48 | *48 |  |
|  | 10 | All Call Page (EXT/INT) | 49 | *49 | #00 | #6 | 49 | *49 |  |
|  | 11 | Enter SMDR Account Code | 50 | *50 | 50 | 91 | 50 | *50 |  |
|  | 12 | Flash Command to CO line | 51 | *51 | 51 | 51 | 51 | *51 |  |
|  | 13 | Last Number Redial (LNR) | 52 | *52 | 52 | 52 | 52 | *52 |  |
|  | 14 | Do-Not-Disturb | 53 | *53 | 53 | 53 | 53 | *53 |  |
|  | 15 | Call Forward | 54 | *54 | 54 | 49 | 54 | *54 |  |
|  | 16 | Speed Dial Programming | 55 | *55 | 55 | *40 | 55 | *55 |  |
|  | 17 | MSG Wait/Call-Back Enable | 56 | *56 | 56 | *66 | 56 | *56 |  |
|  | 18 | MSG Wait/Call-Back Answer | 57 | *57 | 57 | *67 | 57 | *57 |  |
|  | 19 | Speed Dial Access | 58 | *58 | 58 | *9 | 58 | *58 |  |
|  | 20 | Cancel DND/CFW / Pre-selected MSG | 59 | *59 | 59 | 59 | 59 | *59 |  |
|  | 21 | Call Park Locations | 601-610 | *601-*610 | 601-610 | 950-959 | 601-610 | *601-*610 |  |
|  | 22 | Alarm Reset | 65 | *65 | 65 | *65 | 65 | *65 |  |
|  | 23 | Group Call Pick-Up | 66 | *66 | ** | *1 | 66 | *66 |  |
|  | 24 | Station DVU Access | 67 | *67 | 40 | *22 | 67 | *67 |  |
| 01 |  | Second 24 Flexible Number |  |  |  |  |  |  |  |
|  | 1 | Night Answer | 69 | *69 | 77 | 2 | 69 | *69 |  |
|  | 2 | Direct Call Pick-Up | 7 | *7 | *7 | *42 | 7 | *7 |  |
|  | 3 | CO Group Access | 801-824 | 801-824 | 801-824 | 401-424 | 801-824 | 801-824 | SLT |
|  | 4 | CO Line Access | 8801-8848 | 8801-8848 | 8801-8848 | 4801-4848 | 8801-8848 | 8801-8848 |  |
|  | 5 | Tie Routing Access | 89 | 89 | 89 | 47 | 89 | 89 |  |
|  | 6 | Retrieve Held CO Group | 8## | 8## | 8## | 4* | 8## | 8## |  |

| PGM | BTN | ITEM | NUM SET 2 | NUM SET 3 | NUM SET 4 | NUM SET 5 | NUM SET 6 | NUM SET 7 | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | 7 | Retrieve Held CO Line | 8#01-8#48 | 8#01-8#48 | 8#01-8#48 | 4#01-4#48 | 8#01-8#48 | 8#01-8#48 |  |
|  | 8 | Access to CO Line in the 1st available CO Group | 9 | 9 | 9 | 1 | 0 | 9 |  |
|  | 9 | Attendant Call | 0 | 0 | 0 | 0 | 9 | 0 |  |
|  | 10 | Print SMDR (Station Base) | *0 | **0 | 661 | 671 | *0 | **0 |  |
|  | 11 | Print SMDR (Account Group Base) | *1 | **1 | 662 | 672 | *1 | **1 |  |
|  | 12 | Delete SMDR (Station Base) | *2 | **2 | 663 | 673 | *2 | **2 |  |
|  | 13 | Delete SMDR (Account Group Base) | *3 | **3 | 664 | 674 | *3 | **3 |  |
|  | 14 | Display Call Charge On LCD | *4 | **4 | 665 | **4 | *4 | **4 |  |
|  | 15 | Abort SMDR Printing | ** | *** | 666 | *** | ** | *** |  |
|  | 16 | Door Open – 1 | #*1 | #*1 | #*1 | #*1 | #*1 | #*1 |  |
|  | 17 | Door Open – 2 | #*2 | #*2 | #*2 | #*2 | #*2 | #*2 |  |
|  | 18 | Door Open – 3 | #*3 | #*3 | #*3 | #*3 | #*3 | #*3 |  |
|  | 19 | Door Open – 4 | #*4 | #*4 | #*4 | #*4 | #*4 | #*4 |  |
|  | 20 | VM MSG Wait Enable | *8 | *8 | *8 | *8 | *8 | *8 |  |
|  | 21 | VM MSG Wait Cancel | *9 | *9 | *9 | **9 | *9 | *9 |  |
|  | 22 | UCD DND | 68 | *68 | 68 | 68 | 68 | *68 |  |
|  | 23 | Station ISDN Flash | *# | *# | *# | *# | *# | *# |  |

### TABLE 5.6.4	FLEXIBLE NUMBERING PLAN – GDK-FPII

| PGM | BTN | ITEM | NUM SET 2 | NUM SET 3 | NUM SET 4 | NUM SET 5 | NUM SET 6 | NUM SET 7 | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 |  | First 24 Flexible Number |  |  |  |  |  |  |  |
|  | 1 | Station Number | 100-177 | 100-177 | 100-177 | 700-777 | 200-277 | 10-79 |  |
|  | 2 | Station Group Number | 620-627 | *620-*627 | 620-627 | 9700-9707 | 620-627 | *620-*627 |  |
|  | 3 | Station PGM Enter Code | 3 | *3 | *3 | *3 | 1 | *3 | SLT |
|  | 4 | Internal Page Zones | 401-405 | *401-*405 | #01-#05 | #01-#05 | 401-405 | *401-*405 |  |
|  | 5 | Internal All Call Page | 43 | *43 | #5 | #3 | 43 | *43 |  |
|  | 6 | Meet Me Page | 44 | *44 | ## | ## | 44 | *44 |  |
|  | 7 | External Page Zone - 1 | 46 | *46 | #6 | #41 | 46 | *46 |  |
|  | 8 | External Page Zone - 2 |  |  |  |  |  |  |  |
|  | 9 | External All Call Page | 48 | *48 | #8 | #5 | 48 | *48 |  |
|  | 10 | All Call Page (EXT/INT) | 49 | *49 | #00 | #6 | 49 | *49 |  |
|  | 11 | Enter SMDR Account Code | 50 | *50 | 50 | 91 | 50 | *50 |  |
|  | 12 | Flash Command to CO line | 51 | *51 | 51 | 51 | 51 | *51 |  |
| 01 | 13 | Last Number Redial (LNR) | 52 | *52 | 52 | 52 | 52 | *52 |  |
|  | 14 | Do-Not-Disturb | 53 | *53 | 53 | 53 | 53 | *53 |  |
|  | 15 | Call Forward | 54 | *54 | 54 | 49 | 54 | *54 |  |
|  | 16 | Speed Dial Programming | 55 | *55 | 55 | *40 | 55 | *55 |  |
|  | 17 | MSG Wait/Call-Back Enable | 56 | *56 | 56 | *66 | 56 | *56 |  |
|  | 18 | MSG Wait/Call-Back Answer | 57 | *57 | 57 | *67 | 57 | *57 |  |
|  | 19 | Speed Dial Access | 58 | *58 | 58 | *9 | 58 | *58 |  |
|  | 20 | Cancel DND/CFW / Pre-selected MSG | 59 | *59 | 59 | 59 | 59 | *59 |  |
|  | 21 | Call Park Locations | 601-605 | *601-*605 | 601-605 | 950-954 | 601-605 | *601-*605 |  |
|  | 22 | Alarm Reset | 65 | *65 | 65 | *65 | 65 | *65 |  |
|  | 23 | Group Call Pick-Up | 66 | *66 | ** | *1 | 66 | *66 |  |
|  | 24 | Station DVU Access | 67 | *67 | 40 | *22 | 67 | *67 |  |
| 01 |  | Second 24 Flexible Number |  |  |  |  |  |  |  |
|  | 1 | Night Answer | 69 | *69 | 77 | 2 | 69 | *69 |  |
|  | 2 | Direct Call Pick-Up | 7 | *7 | *7 | *42 | 7 | *7 |  |
|  | 3 | CO Group Access | 801-809 | 801-809 | 801-809 | 401-409 | 801-809 | 801-809 | SLT |
|  | 4 | CO Line Access | 8801-8834 | 8801-8834 | 8801-8834 | 4801-4834 | 8801-8834 | 8801-8834 |  |
|  | 5 | Tie Routing Access | 89 | 89 | 89 | 47 | 89 | 89 |  |
|  | 6 | Retrieve Held CO Group | 8## | 8## | 8## | 4* | 8## | 8## |  |
|  | 7 | Retrieve Held CO Line | 8#01-8#34 | 8#01-8#34 | 8#01-8#34 | 4#01-4#34 | 8#01-8#34 | 8#01-8#34 |  |
|  | 8 | Access to CO Line in the 1st available CO Group | 9 | 9 | 9 | 1 | 0 | 9 |  |
|  | 9 | Attendant Call | 0 | 0 | 0 | 0 | 9 | 0 |  |
|  | 10 | Print SMDR (Station Base) | *0 | **0 | 661 | 671 | *0 | **0 |  |
|  | 11 | Print SMDR (Account Group Base) | *1 | **1 | 662 | 672 | *1 | **1 |  |
|  | 12 | Delete SMDR (Station Base) | *2 | **2 | 663 | 673 | *2 | **2 |  |
|  | 13 | Delete SMDR (Account Group Base) | *3 | **3 | 664 | 674 | *3 | **3 |  |
|  | 14 | Display Call Charge On LCD | *4 | **4 | 665 | **4 | *4 | **4 |  |
|  | 15 | Abort SMDR Printing | ** | *** | 666 | *** | ** | *** |  |
|  | 16 | Door Open – 1 | #*1 | #*1 | #*1 | #*1 | #*1 | #*1 |  |
|  | 17 | Door Open – 2 | #*2 | #*2 | #*2 | #*2 | #*2 | #*2 |  |
|  | 18 | Door Open – 3 |  |  |  |  |  |  |  |
|  | 19 | Door Open – 4 |  |  |  |  |  |  |  |
|  | 20 | VM MSG Wait Enable | *8 | *8 | *8 | *8 | *8 | *8 |  |
|  | 21 | VM MSG Wait Cancel | *9 | *9 | *9 | **9 | *9 | *9 |  |
|  | 22 | UCD DND | 68 | *68 | 68 | 68 | 68 | *68 |  |
|  | 23 | Station ISDN Flash | *# | *# | *# | *# | *# | *# |  |

### TABLE 5.6.5	SLOT ASSIGNMENT

| PGM | SYS | BTN | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 02 | GDK-FP II | 1 | 03 | CO Line Slot Assignment Max. 34 CO lines or Max. 6 slots |
|  |  | 2 | 02 | Station Slot Assignment Max. 78 stations or Max. 7 slots |
|  |  | 3 | 08 | WTIB Port Number |
|  | GDK-100 | 1 | 08 07 06 12 11 | CO Line Slot Assignment Max. 48 CO lines or Max. 8 slots |
|  |  | 2 | 01 02 03 04 05 09 10 11 | Station Slot Assignment Max. 136 stations |
|  |  | 3 | 08 | WTIB Port Number |
|  | GDK-162 | 1 | 06 07 08 14 15 16 17 | CO Line Slot Assignment Max. 96 CO lines |
|  |  | 2 | 02 03 04 05 09 10 11 12 13 | Station Slot Assignment Max. 186 stations |
|  |  | 3 | 08 | WTIB Port Number |

### TABLE 5.6.6	MSN/SUB-ADDRESS ASSIGNMENT

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 03 | 1 | CO Line Number | 01-96/48/34 |  | PGM 03: 162 |
| (04) | 2 | Flex DID Table(PGM 05) Index |  |  | PGM 04: 100/FPII |
|  | 3 | MSN/Sub-address Number | 0-9 |  |  |
|  | 4 | Telephone Number | 20 digits |  |  |

### TABLE 5.6.7	FLEXIBLE DID TABLE PROGRAM

| PGM | BTN | SUB-BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
| 05 | 1 | 1 | Bin Number | 000-999 |  |  |
|  |  | 2 | CO Name (DID Name) | 11 digits |  |  |
|  |  | 3 | Day Destination Bin Number | STA/STA Grp/DVU |  |  |
|  |  | 4 | Night Destination Bin Number | STA/STA Grp/DVU |  |  |
|  | 2 |  | Restore default |  |  |  |
|  | 3 |  | Clear table |  |  |  |

### TABLE 5.6.8	EMERGENCY SERVICE CALL

| ###### PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 06 | 1~10 | Emergency Service Call Table | 5 digits |  |  |

### TABLE 5.6.9	LCR TABLE ASSIGNMENT

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 07 | 1 | LCR Access Mode | DIS/OVR/ALL | DIS |  |
|  | 2 | Set the Day of week zone | 1~7 | 1234567 |  |
|  | 3 | Set the Time of week zone | 00~24 | 0024 |  |
|  | 4 | LCR Leading Digit Table |  |  |  |
|  | 5 | Digit Modification Table |  |  |  |
|  | 6 | Initialization of LCR Database |  |  |  |

### TABLE 5.6.10	SYSTEM ATTRIBUTE - IV

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 08 | 1 | In zero insertion | 1/0 | 0 (OFF) |  |
|  | 2 | My area code | 5 digits |  |  |
|  | 3 | CO ATD Code | 2 digits |  |  |
|  | 4 | DID Conversion Table | 0/1/2 | 0 |  |
|  | 5 | Calling Sub Address | 1/0 | 0 (OFF) |  |
|  | 6 | Type of Calling Number | 0~4 | 2 (National No.) |  |
|  | 7 | Advice of Charge | 0~5 | 0 |  |
|  | 8 | Out Zero Insertion | 1/0 | 1 (ON) |  |
|  | 9 | Paging Conference Index | Internal Page Group |  |  |
|  | 10 | U-A Law Line Installed | 1/0 | 0 (OFF) |  |
|  | 11 | ISDN Sending Mode | 1/0 | 0 (Overlap) | For Israel For Israel |
|  | 12 | Enblock Inter-digit Timer |  |  |  |
|  | 13 | Prefix Code | 2 digits | 0 |  |
|  | 14 | International Access Code | 4 digits | - |  |

### TABLE 5.6.11	ISDN COLP TABLE ASSIGNMENT

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 09 | 1-10 | ISDN COLP Table | 10 digits |  |  |

### TABLE 5.6.12	STATION BASE PROGRAM (1/2)

| PGM | BTN | ITEM |  | RANGE |  | DEFAULT |  |  |  |  |  |  | REMARK |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | FP II |  |  |  | 100 |  | 162 |  |  |
| 10 | 1 | DND (Do-Not-Disturb) |  | YES / NO |  | NO |  |  |  |  |  |  |  |  |
|  | 2 | Speed Dial Access |  | YES / NO |  | YES |  |  |  |  |  |  |  |  |
|  | 3 | Page Access |  | YES / NO |  | NO |  |  |  |  |  |  |  |  |
|  | 4 | Call Forward Access |  | YES / NO |  | NO |  |  |  |  |  |  |  |  |
|  | 5 | PLA (Preferred Line Answer) |  | YES / NO |  | YES |  |  |  |  |  |  |  |  |
|  | 6 | Auto Speaker Select |  | YES / NO |  | YES |  |  |  |  |  |  |  |  |
|  | 7 | Warm Line |  | YES / NO |  | NO |  |  |  |  |  |  |  |  |
|  | 8 | SMDR Dial Digit Hidden Display |  | YES / NO |  | NO |  |  |  |  |  |  |  |  |
| 11 | 1 | CO Line Queuing |  | YES / NO |  | YES |  |  |  |  |  |  |  |  |
|  | 2 | 3-Minute Warning Tone |  | YES / NO |  | NO |  |  |  |  |  |  |  |  |
|  | 3 | Override Privilege |  | YES / NO |  | NO |  |  |  |  |  |  |  |  |
|  | 4 | Automatic CO Hold YES-For Attendant, NO-For Others |  | YES / NO |  |  |  |  |  |  |  |  |  |  |
|  | 5 | Data Line Security |  | YES / NO |  | NO |  |  |  |  |  |  |  |  |
|  | 6 | Group Listening |  | YES / NO |  | NO |  |  |  |  |  |  |  |  |
|  | 7 | Speaker-Phone/Headset |  | SP / HEAD |  | Speakerphone |  |  |  |  |  |  |  |  |
|  | 8 | Howling Tone To SLT |  | YES / NO |  | YES |  |  |  |  |  |  |  |  |
|  | 9 | DID Call Waiting on busy station |  | YES / NO |  | NO |  |  |  |  |  |  |  |  |
| 12 | 1 | Alarm/Door Bell Signaling |  | YES / NO |  | YES |  |  |  |  |  |  |  |  |
|  | 2 | ICM Box Signaling |  | YES / NO |  | YES |  |  |  |  |  |  |  |  |
|  | 3 | CO Line Access with dialing |  | YES / NO |  | YES |  |  |  |  |  |  |  |  |
|  | 4 | No Touch Answer |  | YES / NO |  | NO |  |  |  |  |  |  |  |  |
|  | 5 | DVU Access |  | YES / NO |  | NO |  |  |  |  |  |  |  |  |
|  | 6 | Voice Over |  | YES / NO |  | NO |  |  |  |  |  |  |  |  |
|  | 7 | Suppress RX Data connected DM |  | YES / NO |  | NO |  |  |  |  |  |  |  |  |
|  | 8 | Prepaid Call |  | YES / NO |  | NO |  |  |  |  |  |  |  |  |
| 13 |  | Station ID Assignment |  | 1-12 |  | Keyset |  |  |  |  |  |  |  |  |
|  | 1 | Keyset |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | DSS MAP 1 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 | DSS MAP 2 |  |  | N/A |  |  |  | - |  | - |  |  |  |
|  | 4 | DSS MAP 3 |  |  | N/A |  |  |  | - |  | - |  |  |  |
|  | 5 | DSS MAP 4 |  |  | N/A |  |  |  | N/A |  | - |  |  |  |
|  | 6 | DLS MAP 1 |  |  |  | - |  |  |  |  |  |  |  |  |
|  | 7 | DLS MAP 2 |  |  | N/A |  |  |  | N/A |  | - |  |  |  |
|  | 8 | ICM Box |  |  |  | - |  |  |  |  |  |  |  |  |
|  | 9 | SLT (DTMF) |  |  |  | - |  |  |  |  |  |  |  |  |
|  | 10 | SLT (Pulse) |  |  |  | - |  |  |  |  |  |  |  |  |
|  | 11 | SLT with Msg Wait (DTMF) |  |  |  | - |  |  |  |  |  |  |  |  |
|  | 12 | SLT with Msg Wait (Pulse) |  |  |  | - |  |  |  |  |  |  |  |  |
|  | 13 | Station ID Display |  |  |  | - |  |  |  |  |  |  |  |  |
| PGM | BTN | ITEM |  | RANGE |  | DEFAULT |  |  |  |  |  |  | REMARK |  |
|  |  |  |  |  |  | FP II |  |  |  | 100 |  | 162 |  |  |
| 14 | 1 | Station COS : Day |  | 1 - 7 |  | 1 |  |  |  |  |  |  |  |  |
|  | 2 | Station COS : Night |  | 1 - 7 |  | 1 |  |  |  |  |  |  |  |  |
| 15 |  | CO Group Access |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1- 24 | CO Line Group 01~24 |  |  | 01-09 |  |  |  | 01-24 |  | 01-24 |  |  |  |
|  |  | CO Line Group 25~48 |  |  | N/A |  |  |  | N/A |  | 25-48 |  |  |  |
| 16 | 1 | ICM Tenancy Group Assign |  | 01 - 05/10 |  |  | 01 |  |  |  |  |  |  |  |
|  | 2 | ICM Tenancy Group Attendant |  | STA No. |  |  | - |  |  |  |  |  |  |  |
|  | 3 | ICM Tenancy Access Group |  | 01 - 05/10 |  |  | 01 |  |  |  |  |  |  |  |
| 17 | 1-xx | Internal Page Zone Access |  | 01 - 05/20 | GRP 01 |  |  |  |  |  |  |  |  |  |
| 18 |  | Preset Call Forward |  |  |  |  | - |  |  |  |  |  |  |  |
| 19 |  | Hot Line / Warm Line |  |  |  |  | - |  |  |  |  |  |  |  |
|  | 1 | Flex Button |  | 01 - 24 |  |  | - |  |  |  |  |  |  |  |
|  | 2 | CO Line |  | 01-34/48/96 | - |  |  |  |  |  |  |  |  |  |
|  | 3 | CO Group |  | 01-09/24/48 | - |  |  |  |  |  |  |  |  |  |
|  | 4 | Station |  | STA No. |  |  | - |  |  |  |  |  |  |  |
| 20 |  | SMDR Account Group Assign |  | 01 - 24/99 | - |  |  |  |  |  |  |  |  |  |
| 21 | 1 | DID Ring |  | YES / NO |  |  | YES |  |  |  |  |  |  |  |
|  | 2 | ACD Warning Tone |  | YES / NO |  |  | YES |  |  |  |  |  |  |  |
|  | 3 | CO Button Program Enable |  | YES / NO |  |  | NO |  |  |  |  |  |  |  |
|  | 4 | ADMIN Program Enable |  | YES / NO |  |  | NO |  |  |  |  |  |  |  |
|  | 5 | UCD Service |  | YES / NO |  |  | NO |  |  |  |  |  |  |  |
|  | 6 | Differential Ring |  | 0 - 4 |  |  | 0 |  |  |  |  |  |  |  |
|  | 7 | CO Call Drop Timer Enable |  | YES / NO |  |  | NO |  |  |  |  |  |  |  |
| 22 | 1 | CTI Mode |  | 1 - 3 |  |  | 2 |  |  |  |  |  |  |  |
|  | 2 | CTI Baud Rate |  | 1 - 3 |  |  | 1 |  |  |  |  |  |  |  |
| 23 | 1 | ISDN Sub Address | YES / NO |  |  |  |  | NO |  |  |  |  |  |  |
|  | 2 | ISDN Long | YES / NO |  |  |  |  | NO |  |  |  |  |  |  |
|  | 3 | MSN | 1/0 |  |  |  |  | 0 (NO) |  |  |  |  |  |  |
|  | 4 | EXT or CO ATD | 1/0 |  |  |  |  | 1 (EXT) |  |  |  |  |  |  |
|  | 5 | Progress IND | 1/0 |  |  |  |  | 0 (NO) |  |  |  |  |  |  |
|  | 6 | CO Message Wait | 1/0 |  |  |  |  | 0 (NO) |  |  |  |  |  |  |
|  | 7 | Station Base CLIP | 1/0 |  |  |  |  | 0 (NO) |  |  |  |  |  |  |
|  | 8 | Station Base COLP | 1/0 |  |  |  |  | 0 (NO) |  |  |  |  |  |  |
|  | 9 | Station Base CPN | 1/0 |  |  |  |  | 0 (NO) |  |  |  |  |  |  |
|  | 10 | Station Base CLIR | 1/0 |  |  |  |  | 0 (NO) |  |  |  |  |  |  |
|  | 11 | STA CLI NAME DISP |  | 1/0 |  |  | 1(ON) |  |  |  |  |  |  |  |
| 24 | 1 | CO Name Bin |  | 00 - 10 |  |  | 00 |  |  |  |  |  |  |  |
|  | 2 | COLR |  | YES / NO |  |  | NO |  |  |  |  |  |  |  |
|  | 3 | HSR (Headset Ring) |  | ON / OFF |  |  | OFF |  |  |  |  |  | UK: ON |  |
|  | 4 | CO Group Access with dialing |  | YES/NO |  |  | YES |  |  |  |  |  |  |  |
|  | 5 | Keypad Facility |  | YES/NO |  |  | NO |  |  |  |  |  |  |  |

### TABLE 5.6.13	STATION LINK PROGRAM

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 28 | 1 | Linked Station Pairs View | - | - |  |

### TABLE 5.6.14	FLEXIBLE BUTTONS PROGRAM

| PGM | BTN | ITEM | RANGE |  |  | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FP II | 100 | 162 |  |  |
| 29 | 01-48 | Flex. Buttons Assignment | BTN 01-48 |  |  |  |  |
|  |  | 1: User Button | - |  |  |  |  |
|  |  | 2: {COxx} Button | 01 -34 | 01 -48 | 01-96 |  |  |
|  |  | 3: {CO Group xx} Button | 01 -09 | 01 -24 | 01-48 |  |  |
|  |  | 4: {LOOP} Button | - |  |  |  |  |
|  |  | 5: {STA xxx} Button | STA No. |  |  |  |  |
|  |  | 6: STA PGM Button | - |  |  |  |  |
|  |  | 7: {SPD xxx} Button | SPD Bin No. |  |  |  |  |

### TABLE 5.6.15	CO LINE BASE PROGRAM

| PGM | BTN | ITEM | RANGE |  |  |  | DEFAULT | REMARK |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30 | 1 | CO Line Group Assign | 00 - 10/25/49 |  |  |  | 01 |  |  |  |  |
|  | 2 | CO Line COS | 1 – 5 |  |  |  | 1 |  |  |  |  |
|  | 3 | CO Line Group Account | YES / NO |  |  |  | NO |  |  |  |  |
|  | 4 | DISA-xx | U/N/B/S/D, 0 - 9 |  |  |  | U0 |  |  |  |  |
|  | 5 | DISA Account Code | YES / NO |  |  |  | YES |  |  |  |  |
|  | 6 | CO Line Name | 00 - 10 |  |  |  | 00 |  |  |  |  |
| 31 | 1 | CO Line Type | CO / PBX |  |  |  | CO |  |  |  |  |
|  | 2 | CO Line Signal Type | DTMF / Pulse |  |  |  | DTMF |  |  |  |  |
|  | 3 | Universal Night Answer (UNA) | YES / NO |  |  |  | NO |  |  |  |  |
|  | 4 | Flash Type | Ground / Loop |  |  |  | Loop |  |  |  |  |
|  | 5 | Metering Type | 0 – 6 |  |  |  | 0 |  |  |  |  |
|  | 6 | Voice/Data For CEPT | VOICE / DATA |  |  |  | VOICE |  |  |  |  |
|  | 7 | Line Drop using CPT | YES / NO |  |  |  | NO |  |  |  |  |
|  | 8 | Night DVU | YES / NO |  |  |  | NO |  |  |  |  |
| 32 |  | CO Line Ring Assignment |  |  |  |  |  |  |  |  |  |
|  | 1 | Ring Assign to Stations - Ring Mode - Delay Ring Counter | U/D/N/B/O 0 - 9 | U0 |  |  |  |  | U : Unassigned (0) D : Day Only (1) N : Night Only (2) B : Day/Night (3) O :On-demand (4) |  |  |
|  | 2 | Ring Assign to Hunt Group | 0 -4 (Ring Mode) |  |  |  |  |  |  |  |  |
| PGM | BTN | ITEM | RANGE |  |  |  | DEFAULT | REMARK |  |  |  |
| 33 |  | CO Flash Timer | 00 -30 |  |  | 05 |  |  |  |  | (100msec base) |
| 34 |  | Open Loop Detect Timer | 00 -20 |  |  | 00 |  |  |  |  | (100msec base) |
| 35 |  | TIE Line Assignment | 1-7 |  | 1 |  |  |  |  |  |  |
|  | 1 2 3 4 5 6 7 | Normal Ring Ring/Down Loop/Dial Ear/Mouth Continuous Polarity Reverse Ear/Mouth Discontinuous DID Line For Digital CO Line |  |  |  |  |  |  |  |  |  |
| 36 |  | DID Line Assignment | 1-4 |  | 1 |  |  |  |  |  |  |
|  | 1 2 3 | Immediate Start Wink Start Delayed Dial Start |  |  |  |  |  |  |  |  |  |
| 37 |  | ISDN CO Line Attributes |  |  |  |  |  |  |  |  |  |
|  | 1 2 | COLP DID RN | 00-11 00-99 |  | 00 00 |  |  |  |  |  |  |
| 38 |  | CO Line Attributes -IV |  |  |  |  |  |  |  |  |  |
|  | 1 2 3 4 5 6 7 | CO Distinct Ring CO Line MOH CO Line Dial Tone Ring Back Cause Handling Error Cause Handling Busy Cause Handling Announce Cause Handling | 0-4 0-4 YES/NO 1/0 (ON/OFF) 1/0 (ON/OFF) 1/0 (ON/OFF) 1/0 (ON/OFF) |  | 0 0 YES 0 (OFF) 0 (OFF) 0 (OFF) 0 (OFF) |  |  |  |  |  |  |
| 39 |  | R2 DCOB CO Line Attribute |  |  |  |  |  |  |  | For GDK-162 |  |
|  | 1 2 3 4 | Incoming Signal Outgoing Signal B-Signal Grp II Signal | Pulse(0)/DTMF(1)/REC(2) Pulse(0)/DTMF(1)/REC(2) 1-9 1-9 |  | DTMF (1) DTMF (1) CHARGE (6) Grp II-1 (1) |  |  |  |  |  |  |

### TABLE 5.6.16	SYSTEM BASE PROGRAM

| PGM | BTN | ITEM |  | RANGE |  |  | DEFAULT |  | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40 | 1 | Hold Preference |  | SYS / EXC |  |  | SYS |  |  |
|  | 2 | Privacy |  | YES / NO |  |  | YES |  |  |
|  | 3 | Privacy Warning Tone |  | YES / NO |  |  | YES |  |  |
|  | 4 | Page Warning Tone |  | YES / NO |  |  | YES |  |  |
|  | 5 | Off-Hook Ring Signal Type |  | Mute Ring /One Burst Ring |  |  | Mute Ring |  |  |
|  | 6 | Multi-Line Conference |  | YES / NO |  |  | YES |  |  |
|  | 7 | Common Speed Toll Check |  | YES / NO |  |  | YES |  | For GDK-162 |
|  | 8 | Attendant Call Queuing |  | Ringback/MOH |  |  | MOH |  |  |
| PGM | BTN | ITEM |  | RANGE |  |  | DEFAULT |  | REMARK |
| 41 | 1 | Background Music Channels |  | 0 - 9 |  |  | 1 |  |  |
|  | 2 | MOH Channel |  | 0 - 9 |  |  | 1 |  |  |
|  | 3 | ICM BOX Music Channel |  | 0 - 9 |  |  | 1 |  |  |
|  | 4 | Music source on Channel 1 |  | Internal / External |  |  | Internal |  | For GDK-162/100 |
|  | 5 | LCD Display Language |  | 0 - 24 |  |  | 1 (KOREAN) |  | 3 : Invalid |
|  | 6 | Print Caller-ID |  | YES / NO |  |  | NO |  |  |
|  | 7 | CAMP, MOH/RBT |  | YES / NO |  |  | NO |  |  |
|  | 8 | SMDR Dial Digit Hidden Display |  | 0 - 4 |  |  | 0 |  | For GDK-100/FPII |
| 42 | 1 | External Night Ring |  | YES / NO |  |  | NO |  |  |
|  | 2 | CO Line Access mode in CO Grp |  | Round Robin /Last Choice |  |  | Last Choice |  |  |
|  | 3 | Continuous Dial-Tone |  | YES / NO |  |  | YES |  |  |
|  | 4 | Ring Exchange on SLT |  | YES / NO |  |  | NO |  |  |
|  | 5 | Dial-Tone Detect |  | YES / NO |  |  | NO |  |  |
|  | 6 | Clear ACD Database after printing |  | YES / NO |  |  | NO |  |  |
|  | 7 | ACD Print Duration |  | 10SEC / 1Hour |  |  | 1 Hour |  |  |
| 43 | 1 | Exclusive Hold Recall Timer |  | 000 - 300 |  |  | 060 |  | 1 sec base |
|  | 2 | System Hold Recall Timer |  | 000 - 300 |  |  | 030 |  | 1 sec base |
|  | 3 | Transfer Recall Timer |  | 000 - 300 |  |  | 030 |  | 1 sec base |
|  | 4 | I-Hold Recall Timer |  | 000 - 300 |  |  | 030 |  | 1 sec base |
|  | 5 | ATD Recall Timer |  | 00 - 60 |  |  | 01 |  | 1 min base |
|  | 6 | CO Ring Detect Timer |  | 1 - 9 |  |  | 2 |  | 100 msec base |
|  | 7 | Pause Timer |  | 3 - 9 |  |  | 3 |  | 1 sec base |
|  | 8 | CO Release Guard Timer |  | 010 - 150 |  |  | 020 |  | 100 msec base |
|  | 9 | CO Warning Tone Timer |  | 060 - 900 |  |  | 180 |  | 1 sec base |
|  | 10 | CO Dial Delay Timer |  | 00 - 99 |  |  | 01 |  | 1 sec base |
|  | 11 | Call Park Timer |  | 000 - 600 |  |  | 120 |  | 1 sec base |
|  | 12 | Preset Call Forward Timer |  | 00 - 99 |  |  | 10 |  | 1 sec base |
|  | 13 | Unsupervised Conference Timer |  | 00 - 99 |  |  | 10 |  | 1 min base |
|  | 14 | ACNR Retrial Pause Timer |  | 030 - 300 |  |  | 030 |  | 1 sec base |
|  | 15 | ACNR Retrial Delay Timer |  | 000 - 300 |  |  | 030 |  | 1 sec base |
|  | 16 | Flexible DID Time-out Timer |  | 000 - 300 |  |  | 030 |  | 100 msec base |
|  | 17 | ACNR No Answer Timer |  | 10 - 50 |  |  | 30 |  | 1 sec base |
|  | 18 | ACNR Retry Counter |  | 1 - 3 |  |  | 3 |  |  |
|  | 19 | Warm Line Timer |  | 01 - 20 |  |  | 05 |  | 1 sec base |
|  | 20 | PBX Code Print |  | 1 / 0 |  |  | 0 |  |  |
|  | 21 | Prepaid Call Warning Tone Timer |  | 00 - 99 |  |  | 15 |  | 1 sec base |
|  | 22 | DISA Retry Counter |  | 1 - 13 |  |  | 3 |  |  |
|  | 23 | COS when DISA seize CO |  | 1 - 6 |  |  | 6 (UK: 1) |  |  |
|  | 24 | DVU Forward Answer Timer |  | 04 - 40 |  |  | 04 |  |  |
|  | 25 | R2 Out Manage Timer |  | 01 - 50 |  |  | 14 |  | For GDK-162 |
|  |  | ACNR Retry Time Count |  | 1 - 9 |  |  | 3 |  | For GDK-100/FPII |
|  | 26 | R2 Pulse Timer |  | 01 - 30 |  |  | 7 |  | For GDK-162 |
|  |  | Switch Pause Timer |  | 00 -40 |  |  | 00 |  | For GDK-100/FPII |
| PGM | BTN | ITEM |  | RANGE |  |  | DEFAULT |  | REMARK |
| 43 | 27 | DT Delay Timer |  | 01 - 30 |  |  | 20 |  | For GDK-162 |
|  |  | First CO Group OVR |  | 0 / 1 |  |  | 1 |  | For GDK-100/FPII |
|  | 28 | ACNR Retry Time Count |  | 1 - 9 |  |  | 3 |  | For GDK-162 |
|  |  | CO Call Drop Timer |  | 01 - 99 |  |  | 10 |  | For GDK-100/FPII |
|  | 29 | Switch Pause Timer |  | 00 - 40 |  |  | 00 |  | For GDK-162 |
|  | 30 | First CO group OVR |  | 0 / 1 |  |  | 1 (UK : 0) |  | For GDK-162 |
|  | 31 | R2 Incoming Manage Timer |  | 01 - 50 |  |  | 14 |  | N/A in FPII/100 |
|  | 32 | R2 Disappear Timer |  | 01 - 50 |  |  | 14 |  | N/A in FPII/100 |
|  | 33 | CO Call Drop Timer |  | 01 - 99 |  |  | 10 |  | 1 min base |
| 44 | 1 | Message Reminder Tone Timer |  |  | 00 - 60 |  | 00 |  | 1 min base |
|  | 2 | Hook Switch Flash Timer |  |  | 01-25 |  | 05 |  | 100msec base |
|  | 3 | Hook Switch Debounce Timer |  |  | 01 - 25 |  | 01 |  | 100msec base |
|  | 4 | DID/DISA No Answer Timer |  |  | 00 - 99 |  | 20 |  | 1 sec base |
|  | 5 | ICM Box Timer |  |  | 00 - 60 |  | 30 |  | 1 sec base |
|  | 6 | Door Open Timer |  |  | 05 - 99 |  | 20 |  | 100msec base |
|  | 7 | Call Forward No Answer Timer |  |  | 000 - 600 |  | 015 |  | 1 sec base |
|  | 8 | ICM Dial-Tone Timer |  |  | 01 - 20 |  | 10 |  | 1 sec base |
|  | 9 | Inter-Digit Timer |  |  | 01 - 20 |  | 05 |  | 1 sec base |
|  | 10 | Automatic Release Timer |  |  | 000 - 300 |  | 030 |  | 1 sec base |
|  | 11 | Page Timer |  |  | 00 - 60 |  | 15 |  | 1 sec base |
|  | 12 | Ring Stop Detection Timer |  |  | 010 - 150 |  | 060 |  | 100 msec base |
|  | 13 | Wink Timer |  |  | 010 - 200 |  | 010 |  | 10 msec base |
|  | 14 | Min Hook-flash Timer |  |  | 000 - 250 |  | 020 |  | 10 msec base |
|  | 15 | SMDR Start Timer |  |  | 000 - 250 |  | 000 |  | 1 sec base |
|  | 16 | DVU Recording Time |  |  | 10 - 90 |  | 20 |  | 1 sec base |
|  | 17 | CO Digit Analysis Counter |  |  | 0 - 9 |  | 3 |  |  |
|  | 18 | ACD Print Timer |  |  | 000 - 255 |  | 000 |  |  |
|  | 19 | SLT Ring Phase Time |  |  | 2 - 5 |  | 5 |  | 1 sec base |
|  | 20 | Camp-On Recall Timer |  |  | 000 - 200 |  | 050 |  | 1 sec base |
|  | 21 | Station Auto Release Timer |  |  | 000 - 200 |  | 060 |  | 1 sec base |
|  | 22 | CEPT CO Type |  |  | 0 – 2 |  | 2 |  | 0:Sweden/Cyprus 1:Italy 2:Korea/Australia |
|  | 23 | Preparation Signal Timer |  |  | 000 - 025 |  | 007 |  | For GDK-162 |
|  |  | Erase Wake Up Fail Ring Timer |  |  | 00 - 99 |  | 20 |  | For GDK-100/FPII |
|  | 24 | Erase Wake Up Fail Ring Timer |  |  | 00 - 99 |  | 20 |  |  |
| 45 |  | ADMIN Password | 4 Digits |  |  | - |  |  |  |
| 46 | 1-5 | Main Attendant Assign | STA No. |  |  | 1 : 101 |  |  |  |
| 47 | 1-6 | Station Groups |  |  |  |  |  |  |  |
|  | 1 | Circular Group |  |  |  |  |  |  |  |
|  | 2 | Terminal Group |  |  |  |  |  |  |  |
|  | 3 | UCD Group |  |  |  |  |  |  |  |
|  | 4 | Voice Mail Group |  |  |  |  |  |  |  |
|  | 5 | Ring Group |  |  |  |  |  |  |  |
|  | 6 | Pick-Up Group |  |  |  |  |  |  |  |

| PGM | BTN | ITEM | RANGE |  |  | DEFAULT |  | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 48 |  | Circular Group Attribute |  |  |  |  |  |  |
|  | 1 | No Answer Timer |  | 00 – 99 | 15 |  | 1 sec base |  |
|  | 2 | Ring Timer |  | 000 – 999 | 999 |  | 1 sec base |  |
|  | 3 | Wrap-Up Timer |  | 002 – 999 | 002 |  | 1 sec base |  |
|  | 4 | Overflow Timer |  | 000 – 600 | 000 |  | 1 sec base |  |
|  | 5 | Overflow Destination |  | STA No. /STA GRP No. | - |  |  |  |
|  | 6 | Announce Location |  | DVU 1-9 | - |  |  |  |
|  | 7 | Group Call By Pilot Number |  | 1/0 | 0 |  |  |  |
|  |  | Terminal Group Attribute |  |  |  |  |  |  |
|  | 1 | No Answer Timer |  | 00 – 99 | 15 |  | 1 sec base |  |
|  | 2 | Ring Timer |  | 000 – 999 | 999 |  | 1 sec base |  |
|  | 3 | Wrap-Up Timer |  | 002 – 999 | 002 |  | 1 sec base |  |
|  | 4 | Overflow Timer |  | 000 – 600 | 000 |  | 1 sec base |  |
|  | 5 | Overflow Destination |  | STA No./ STA GRP No. | - |  |  |  |
|  | 6 | Announce Location |  | DVU 1-9 |  |  |  |  |
|  | 7 | Group Call By Pilot Number |  | 1/0 | 0 |  |  |  |
|  |  | UCD Group Attribute |  |  |  |  |  |  |
|  | 1 | Queuing To DVU Announce 1 Timer |  | 000 – 999 | 015 |  | 1 sec base |  |
|  | 2 | Queuing To DVU Announce 2 Timer |  | 000 – 999 | 000 |  | 1 sec base |  |
|  | 3 | Supervisor Timer |  | 000 – 999 | 030 |  | 1 sec base |  |
|  | 4 | Supervisor Call |  | 00 – 99 | 00 |  |  |  |
|  | 5 | Overflow Timer |  | 000 – 600 | 000 |  | 1 sec base |  |
|  | 6 | Warp-Up Timer |  | 002 – 999 | 002 |  | 1 sec base |  |
|  | 7 | Queuing To DVU Announce Timer |  | 000 – 999 | 000 |  | 1 sec base |  |
|  | 8 | Overflow Destination |  | STA No. / STA GRP No. | - |  |  |  |
|  | 9 | DVU Announce Location 1 |  | DVU 1 - 9 | - |  |  |  |
|  | 10 | DVU Announce Location 2 |  | DVU 1 - 9 | - |  |  |  |
|  | 11 | DVU Announce 2 Repeating |  | 1 / 0 | 0 |  |  |  |
|  | 12 | UCD Station’s Priority |  | 0 – 9 | 0 |  |  |  |
|  | 13 | Supervisor |  | STA No. | - |  |  |  |
|  | 14 | Alternate Destination |  | STA No. / STA GRP No. | - |  |  |  |
|  | 15 | Music Source |  | 0 – 9 | 0 |  |  |  |

| PGM | BTN | ITEM |  | RANGE |  |  | DEFAULT |  |  | REMARK |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | VM Group Attribute |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |
|  | 2 | Ring Timer |  |  | 000 – 999 | 999 |  |  | 1 sec base |  |  |
|  | 3 |  |  |  |  |  |  |  |  |  |  |
|  | 4 | Overflow Timer |  |  | 000 – 600 | 000 |  |  | 1 sec base |  |  |
|  | 5 | Overflow Destination |  |  | STA No./ STA GRP No. | - |  |  |  |  |  |
|  | 6 |  |  |  |  |  |  |  |  |  |  |
|  | 7 | Put Mail Index |  |  | 1 – 4 | 1 |  |  |  |  |  |
|  | 8 | Get Mail Index |  |  | 1 – 4 | 2 |  |  |  |  |  |
|  | 9 | VM Group Hunt Type |  |  |  |  |  |  |  |  |  |
|  |  | Ring Group Attribute |  |  |  |  |  |  |  |  |  |
|  | 1 | Overflow Timer |  |  | 000 – 600 | 600 |  |  | 1 sec base |  |  |
|  | 2 | Overflow Destination |  |  | STA No./ STA GRP No. | - |  |  |  |  |  |
|  | 3 | Music Source |  |  | 0 – 2 | 0 |  |  |  |  |  |
|  | 4 | Ring Group Annc Timer |  |  |  |  |  |  |  |  |  |
|  | 5 | Ring Group Annc Location |  |  |  |  |  |  |  |  |  |
| 48 |  | Pick-Up Group Attribute |  |  |  |  |  |  |  |  |  |
|  | 1 | Auto Pick-Up |  |  | YES / NO | NO |  |  |  |  |  |
|  | 2 | All Ringing |  |  | YES / NO | NO |  |  |  |  |  |
| 49 | 1-xx | Executive/Secretary Pairs |  |  | STA No. | - |  |  |  |  |  |
| 50 | 1 | Alarm Enable |  |  | YES / NO | NO |  |  |  |  |  |
|  | 2 | Alarm Mode |  |  | Alarm / Door-Bell | Alarm |  |  |  |  |  |
|  | 3 | Alarm Contact Type |  |  | Close / Open | Close |  |  |  |  |  |
|  | 4 | Alarm Signal Mode |  |  | Repeat / Once | Repeat |  |  |  |  |  |
| 51 | 1 - x | External Contact Assign |  |  | 1 - 2/4/6 | - |  |  |  |  |  |
| 52 | 1 - 5 | PBX Dialing Codes |  |  | Max 2 digits | - |  |  |  |  |  |
| 53 | 1 - 5 | SMDR Long Distance Codes |  |  | Max 2 digits | 1 : "0" |  |  |  |  |  |
| 54 |  | Authorization Codes |  |  | 5 digits | - |  |  |  |  |  |
| 55 | 1 | DID Received Digit from PX |  |  | 2 - 4 | 3 |  |  |  |  |  |
|  | 2 | DID First Two Digits Conversion |  |  | 4 digits | #*** |  |  | 0 - 9 # : Ignore Digit * : using received digit |  |  |
|  | 3 | DID Second Digit Conversion |  |  | 10 digits | 1234567890 |  |  |  |  |  |
| 56 |  | DID/DISA Destination |  |  |  |  |  |  |  |  |  |
|  | 1 | Busy Destination | Tone / ATD |  |  |  |  | Tone |  |  |  |
|  | 2 | Error Destination | Tone / ATD |  |  |  |  | Tone |  |  |  |

| PGM | BTN | ITEM | RANGE |  | DEFAULT |  | REMARK |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 57 | 1 - 4 | Pulse Dial Speed Ratio |  |  |  | 2 |  | 1: 10 PPS 60/40 2: 10 PPS 66/33 3: 20 PPS 60/40 4: 20 PPS 66/33 |
| 58 | 1 | Modem Mode |  | Modem / Bypass |  | Modem |  |  |
|  | 2 | Modem Baud Rate |  | 1 - 8 |  | 4 |  |  |
|  | 3 | Modem Associated Device |  | CO No.(0)/ STA No.(1) |  | STA 167/171/291 |  |  |
| 59 |  | System DATE/TIME Setting |  |  |  |  |  |  |
| 60 | 1 | LCD Time Display Mode |  | 12/24 Hour |  | 12 Hour |  |  |
|  | 2 | LCD Date Display Mode |  | MMDDYY / DDMMYY |  | DDMMYY |  |  |
| 61 |  | S/W Version Display |  |  |  |  |  |  |
| 62 | 1 | Save SMDR |  | YES / NO |  | NO |  |  |
|  | 2 | Print SMDR |  | YES / NO |  | NO |  |  |
|  | 3 | SMDR Recording Call Type |  | LD / All |  | Long Distance Only |  |  |
|  | 4 | Print Incoming Call |  | YES / NO |  | NO |  |  |
|  | 5 | Record Detailed SMDR |  | YES / NO |  | YES |  |  |
|  | 6 | SMDR Print Baud Rate (RS-232) |  | 1 - 8 |  | 6 |  | 1: Baud 300 2: Baud 600 3: Baud1200 4: Baud 2400 5: Baud 4800 6: Baud 9600 7: Baud 19200 8: Baud 38400 |
|  | 7 | SMDR Currency |  | 3 English Chars |  | - |  |  |
| 63 | 1 | SMDR Cost Per Unit Pulse |  | 6 digits |  | - |  |  |
|  | 2 | SMDR Fraction |  | 0 - 5 |  | 0 |  |  |
| 65 | 1 - 6 | TIE Routing Table (01 - 30) |  | CO 01-34/48/96 |  | - |  |  |
| 66 | 1 -12 | System Gain Control |  | 0 - 63 |  |  |  | 1 dB base |
| 67 | 1 - 3 | System Speed Zone |  | 01 - 10 |  |  |  |  |
| 68 | 1 - 4 | ACNR Tone Cadence |  | 0 - 255 |  |  |  | 20 msec base |
| 69 | 1 - 4 | System Tone Frequency |  | 4 digits |  |  |  |  |
| 71 | 1 - 4 | World Time Attribute |  | 1: Display World Time 2: Celsius/Fahrenheit 3: ISD Code length 4: ISD Code |  | - |  |  |

| PGM | BTN | ITEM | RANGE |  | DEFAULT |  | REMARK |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 72 | 1 - 4 | World Time |  | 1: Nation Code 2: Area Code 3: Time 4: Temperature |  | - |  |  |
| 73 | 1 - 9 | VM dialing Table |  | Max. 12 digits |  |  |  |  |
| 74 |  | CO Line Name (01-10) |  | Max. 12 ENG. chars |  | - |  |  |
| 75 | 1 - 4 | Ring Frequency |  | 0000 - 9999 |  |  |  |  |
| 76 | 1 - 4 | Distinct Ring Frequency |  | 0000 - 9999 |  |  |  |  |
| 77 | 1 - 7 | DCO TX Gain |  | 00 - 63 |  |  |  | 1 dB base |
| 78 | 1 - 3 | Digit Insert Table |  |  |  |  |  |  |

### TABLE 5.6.17	TOLL TABLE

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 70 |  | Exception Tables |  |  |  |
|  | 1 | Allowed Table A | 01 - 20 | - |  |
|  | 2 | Denied Table A | 01 - 10 | - |  |
|  | 3 | Allowed Table B | 01 - 20 | - |  |
|  | 4 | Denied Table B | 01 - 10 | - |  |
| 64 |  | Canned Toll Exception Table |  |  |  |
|  | 1 | Allowed Table | 01 - 10 |  |  |
|  | 2 | Denied Table | 01 - 10 |  |  |
| 79 |  | CCR Tables | 01 - 09 |  |  |

### TABLE 5.6.18	PRINT DATABASE

| PGM | BTN | ITEM | REMARK |
| --- | --- | --- | --- |
| 80 | 1 | Print Flexible Numbering Plan |  |
|  | 2 | Print Slot Assignment |  |
|  | 3 | Print Station Database |  |
|  | 4 | Print Flex Buttons Assignment |  |
|  | 5 | Print CO Line Database |  |
|  | 6 | Print System Database |  |
|  | 7 | Print Toll Tables |  |
|  | 8 | Print System Speed Dial Bins |  |
|  | 9 | Print All Database |  |
|  | 10 | Print Specific Nation’s Database |  |