### Accessories For The Purpose Of Test.

These boards are useful to test GDK-100 system and functional boards for the purpose of investigation. These are not required for an end customer.

Extension Board

These are dummy boards so that the required function board may be positioned at outside of the system cabinet. They are directly installed on the card slot, and then the function cards are connected to the other side of them.

These boards are used when an engineer or a service man wants to investigate the function boards in state of operation.

| Board Name | Function Board Name | Remark |
| --- | --- | --- |
| EXT3(MPB) of GDK-100 | MPB | 2923NP2166A |
| EXT4(PP) of GDK-100 | All GDK-100 cards except MPB. | 2923NP2167A |

Modular connector board

These are the boards for line connection, which contain a RJ21 type connector and RJ-11 type jacks. A RJ21 type connector of the board is connected to the opposite type of RJ21 type connector on the function boards or the system, and RJ-11 type jacks of the board are connected to CO lines or terminals through line cord. It is easy to connect CO line and terminals for the purpose of testing system and terminals.

These boards are used when an engineer or a service man wants to investigate the operation of the system and terminals.

| Board Name | Function Board Name | Remark |
| --- | --- | --- |
| Modular Block of GDK-162/100 | ETIB | 2923NP2168A |
| Modular Block(Station) of GDK-100 | DTIB12, SLIB48, SLIB48, DSIB, SLIBII | 2923NP2169A |
| Modular Block(DTIB) of GDK-162 | DTIB24 (24 ports) | 2923NP2172A |
| Modular Block(CO) of GDK-162 | LCOB, AC15, BWDIDB | 2923NP2171A |

Figure 2.4.1	KD Digital Keysets

KD-36EXE

KD-33LD

Figure 2.4.2	KD/E Digital Keysets

KD/E-8BTN

KD/E-36EXE

KD/E-36LD

LKD-2NS

LKD-8DS

LKD-30LD

Figure 2.4.3	LKD Digital Keysets

LKD-30DS

GSX-8BTN

GSX-33 EXE

Note:

For the connection to any digital keyset behind LKD-30DS, 2B-DTIU30 should be inserted in the bottom side of LKD-30DS.

In case of using 2B-DTIU30 and Secondary digital Keyset, It is recommended to use optional mains adapter, which confirms to the appropriate national or local electrical adapter.

2B-DTIU30 Connection

Primary Station

( LKD-30DS)

Secondary Station

(Digital Keyset)

GDK-100 system

DTIB12/DTIB24/DSIB

2 wire

2 wire

Optional

Mains Adapter

(AC/DC)

Figure 2.4.52B-Modules

Note:

For the connection to SLT device (SLT, Fax, Modem, MOHU) behind LKD-30DS, 2B-SLIU30 should be inserted in the bottom side of LKD-30DS.

2B-SLIU30 Connection

Primary Station

( LKD-30DS)

Secondary Station

(SLT)

GDK-100 system

DTIB12/DTIB24/DSIB

2 wire

2 wire

Optional

Mains Adapter

(AC/DC)

Note:

For this type of the connection using the external CTI module (GDK-PC PHONE unit), KD/C or KD/EC keyset should be required.

All the line cords and RS232C cable are provided with GDK-PC PHONE unit in the same package.

The connection method of the external CTI module

GKD-PC PHONE

GDK-PC PHONE

(CTI Module)

KD/C,KD/EC Keyset

(CTI Keyset)

PC

-PC application

-Window 3.1 or 95

environment

2 wire line cord

6 wire

line cord

RS-232C

(9 pin)

GDK-100 system

-DTIB12/DTIB24/DSIB

-MP Call Software

(99A or later version)

Note:

For this type of connection using the internal CTI module (CTIU30/CTIU8), LKD Keyset should be required.

RS232C cable is provided with CTIU30/CTIU8 in the same package.

The connection method of the internal CTI module

Figure 2.4.6CTI modules

PC

-PC application

-Window 3.1 or 95

environment

LKD Keyset

with CTI module

-LKD-30DS and CTIU30

-LKD-8DS/2NS and CTIU8

2 wire line cord

GDK-100 system

-DTIB12/DTIB24/DSIB

-MP Call Software

(99A or later version)

The cable with RS-232C(9 pin) and RJ11 type plug

Note:

A 4-wire and a RS-232C cables are provided with GDK-TRC1 unit in the same package.

Caution: This tool should be only used by LG Information & Communications, Ltd. and its agent for the purpose of the maintenance of GDK-100 system, but not by a end user.

Figure 2.4.7The connection method of the GDK-TRC1

GKD-TRC1

GDK-TRC1

(External trace Tool)

PC

4 wire cable

RS-232C

(9 pin)

GDK-100 system

-BRIB/STIB/PRIB/E1IB/WTIB