SECTION 2. GENERAL DESCRIPTION

### INTRODUCTION

The GDK-100 Digital Key Telephone System is a fully digital hybrid Key Telephone System, designed to meet the telecommunication needs of medium sized business offices.

The GDK-100 System incorporates state of the art digital technology for command processing and voice switching, utilizing a Pulse Code Modulation/Time Division Multiplexing (PCM/TDM, “A” law) distributed switching matrix.

The GDK-100 achieves a high level of flexibility by 1) employing a Universal Card Slot architecture with a Small Basic, Basic and Expansion cabinet to house plug-in Printed Circuit Boards, and 2) providing support for different types of instrumentation.

The GDK-100 Small Basic, Basic and Expansion KSU are wall mounted cabinets that house the back plane and contain card slots for the system power supplies, CO line /Key Station /SLT /ISDN /DECT interface boards, and other optional PCBs. Station, CO Line, SLT, ISDN, DECT and optional boards are installed in any of the system card slots up to the system’s maximum configuration. MISB can be installed only slot 8 of Basic KSU, Digital Line Boards (E1IB, PRIB, BRIB, STIB) and WTIB can be installed from slot 1 to slot 7 of Basic KSU, and DVIB(Digital Voice Interface Board) can be installed from slot 1 to slot 7 of Basic KSU.  Thus, the system capacity is established by the number of cabinets and the types of interface boards installed and is not limited by an artificially fixed Station to CO Line ratio.

The system architecture has been designed to allow a high level of software control over the system's hardware. The software incorporates a vast array of features and capabilities including PC Database Administration, Auto Route Select, ACD, etc.

The GDK-100 system supports a combination of Digital Keysets (KD, KD/E, KD/S, KD/C, LKD series), Electronic Keysets(GSX, GSX/E series), various kinds of ISDN terminals, and wireless terminals as well as analogue single line devices. With the keysets, commonly used features are activated by direct button selection. Additionally, many functions may be accessed by dialing specific codes or optionally, by assigning these dial codes to Flexible Buttons on the keyset. In addition to key telephones, an array of optional terminals is available including DSS/DLS Console, Intercom/Door Box.

With the flexibility of the GDK-100 extensive feature content, and the capability to use an array of instruments, the GDK-100 can be tailored to meet the short and long term needs of the most demanding customer requirements.

### SYSTEM CAPACITY

The following Table and Chart provide system capacities and displays the configuration flexibility of the system.

### System Capacity

Table 2.2.1	System Capacities

| Description | Basic & Expansion | Total |
| --- | --- | --- |
| Time Slots |  | 144 |
| CO Line Ports |  | Max. 32 (with Analog CO) 1) Max. 48 (with ISDN Lines ) 1) |
| Station Ports (Digital/Electronic/SLT/DECT) |  | Max. 72 (without GAP-DECT) 1) Max. 136 (with GAP-DECT) 1) |
| DSS/DLS Consoles |  | 4/Keyset |
| Attendant Position | 1 / System, 5 / Main, 5 / Group Attendants |  |
| Intercom Links |  | Non-blocking |
| Paging All Call External Internal |  | 1 zone 2 zones 15 zones |
| Station Speed Dial | 20/station, 24 digits each | 500 |
| System Speed Dial | 24 digits each | 200 |
| Last Number Redial |  | 48 digits |
| Save Number Redial |  | 48 digits |
| Memo Dial |  | 48 digits |
| SMDR | 100 (without MEME/SMEMU) 2,000 (with MEMU/SMEMU) | 2,000 |
| Music Source Inputs | 1 internal / MPB, 2 externals / MISB 1 external / DVIB 2 external / MOHU | 9 |
| External Control Contacts | flexibly assigned | 4/MISB |
| Alarm/Door Bell Input |  | 1/MISB |
| Power Fail Circuits | 6 Lines / PFTU | 30 |
| RS 232C Ports | 1 / MPB, 2 / SIU | 3 |
| DTMF Receivers | 2 / DTRU, 4 / DTRUII 2) | 22 |
| CO Line Groups |  | 24 |
| Intercom Groups |  | 5 |
| Hunt/UCD Groups |  | 15 |
| DISA Lines |  | All CO ports |
| Conference | 3-party 4-party 5-party | no limit 13 5 |

- Note :

- Not all the maximum capacities may be achieved simultaneously.

For the maximum number of cards in a system, see APPENDIX A.

- There are 2 kinds of DTMF receiver boards, DTRU and DTRU II.

- A DTRU contains 2 DTMF receivers, and it can be installed on LCOB, SLIB24/48, DSIB, EMIB and/or BWDIDB as a add-on board.

- A DTRU II contains 4 DTMF receivers, it can be installed on SLIB II as a add-on board.

### System Configuration Chart

### Overview

The maximum capacity of the system configured only with the wired extensions will be limited by the physical condition of the system, e.g. the quantity of slots and cards limitation.

Maximum CO lines:		32 only with analog CO lines

48 with ISDN CO lines including analog CO lines

Maximum extensions: 	72 only with wired extensions.

136 including wireless terminals

- The system capacity without WTIB (wireless terminals)

The system capacity without GAP expanded is limited by the maximum 72 ports of extensions and the maximum 48 ports of CO lines, regardless of the maximum time slots.

b) The system capacity with WTIB (wireless terminals)

The total time slots of GDK-100 system are 144, but the maximum time slots for user communication channels are limited by 127 because of the use of system resource and so on.

So the capacity of the system including wireless terminals will be limited by the RMWT for wireless terminals and the maximum time slots 127.

The time slots for wireless terminals will be determined by RMWT. RMWT (default 8) can be set by multiple of 8 in the range of 8 to 64, and the time slots for wireless terminals will be increased up to 32 according to RMWT. So in case that RMWT is above 32, the maximum time slots for wireless terminals will be 32 even though maximum 64 wireless terminals can be registered to the GDK-100 system.

The following clauses 2.2.2.2 to 2.2.2.4 show examples of system configuration chart.

### System Configuration Chart with Analog CO

- Analog CO without WTIB

32

CO

28

24

20

16

12

08

04

00

00   12   24   36   48    60   72

Stations

Figure 2.2.2.2.bConfiguration with Analog CO and 24 port Extension Board

BKSU only

BKSU and EKSU

32

CO

28

24

20

16

12

08

04

00

00   12   24   36   48    60   72

Stations

Figure 2.2.2.2.aConfiguration with Analog CO and 12 port Extension Board

BKSU and EKSU

BKSU only

- Note

- If a customer uses 6 port extension boards, above configuration will be a little different shape.

- Even though a customer uses 2B modules in GDK-100, the maximum capacity will be the same.

2B module can be connected to the every even-numbered port (0,2,4,…) of the boards like DTIB12/DTIB24/DSIB, then any DKTU can not be connected to the consecutive port(1,3,5,…) in this case.

- Analog CO with WTIB (wireless terminal)

00   12   24    88   100   112  124  136

Stations

32

CO

28

24

20

16

12

08

04

00

00   12     76   88  100   112  124  136

Stations

Figure 2.2.2.2.c Configuration with Analog CO and 12 port Extension Board

BKSU only

BKSU and EKSU

1)

32

CO

28

24

20

16

12

08

04

00

Figure 2.2.2.2.dConfiguration with Analog CO and 24 port Extension Board

BKSU only

BKSU

and

EKSU

1)

- Note

- This is only the configuration chart when a customer uses the maximum wireless terminals with RMWT 64. If RMWT is set by another value, above will be different configuration chart according to the RMWT.

RMWT can set by multiple of 8 in the range of 8 to 64. A customer can program this RMWT (default 8) in Admin field 02, Flexible button 3.

- RMWT: Registrable Maximum number of Wireless Terminal. For more details, refer to Installation and Maintenance Manual of GDK-162/100/FPII WOTS (Wireless Office Telephone System)

- See the description on clause 2.2.2.2.a - Note 1) and 2), in relation to the configuration with 6 port extension boards and 2B module.

### System Configuration Chart with BRI Line

- BRIB/STIB(T-interface) without WTIB

CO

48

40

32

24

16

08

00

00   12   24   36   48    60   72

Stations

Figure 2.2.2.3.a Configuration with BRI line and 12 port Extension Board

BKSU only

BKSU and EKSU

CO

48

40

32

24

16

08

00

00   12   24   36   48    60   72

Stations

Figure 2.2.2.3.bConfiguration with BRI line and 24 port Extension Board

BKSU only

BKSU and EKSU

- Note

- CO capacity in above chart means the quantity of B-channels.

- Above configuration can be a little different according to the interface mode (T or S) of STIB.

- See the description on clause 2.2.2.2.a - Note 1) and 2), in relation to the configuration with 6 port extension boards and 2B module.

- BRIB/STIB(T-interface) with WTIB (wireless terminal)

CO

48

40

32

24

16

08

00

00   12     76   88  100   112  124  136

Stations

Figure 2.2.2.3.cConfiguration with BRI line and 12 port Extension Board

BKSU only

BKSU and EKSU

1)

CO

48

40

32

24

16

08

00

00   12   24    88   100   112  124  136

Stations

Figure 2.2.2.3.dConfiguration with BRI line and 24 port Extension Board

BKSU only

BKSU and EKSU

1)

- Note

- This is only the configuration chart when a customer uses the maximum wireless terminals with RMWT 64. If RMWT is set by another value, above will be different configuration chart according to the RMWT.

RMWT can set by multiple of 8 in the range of 8 to 64. A customer can program this RMWT (default 8) in Admin field 02, Flexible button 3.

- RMWT: Registrable Maximum number of Wireless Terminal. For more details, refer to Installation and Maintenance Manual of GDK-162/100/FPII WOTS (Wireless Office Telephone System)

- CO capacity in above chart means the quantity of B-channels.

- Above configuration can be a little different according to the interface mode (T or S) of STIB.

- See the description on clause 2.2.2.2.a- Note 1) and 2), in relation to the configuration with 6 port extension boards and 2B module.

### System Configuration Chart with PRI Line

- PRIB (including BRIB/STIB ) without WTIB

00   12   24   36   48    60   72

Stations

Figure 2.2.2.4.b Configuration with PRI / BRI line and 24 port Extension Board

CO

46

38

30

00

00   12   24   36   48    60   72

Stations

Figure 2.2.2.4.aConfiguration with PRI /BRI line and 12 port Extension Board

BKSU only

CO

46

38

30

00

BKSU

BKSU and EKSU

- Note

- CO capacity in above chart means the quantity of B-channels.

- Above configuration can be a little different according to the interface mode (T or S) of STIB.

- See the description on clause 2.2.2.2.a - Note 1) and 2), in relation to the configuration with 6 port extension boards and 2B module.

- PRIB(including BRIB/STIB) with WTIB (wireless terminal)

CO

46

38

30

00

00   12     76   88  100   112  124  136

Stations

Figure 2.2.2.4.cConfiguration with PRI /BRI line and 12 port Extension Board

BKSU

1)

CO

46

38

30

00

00   12   24    88   100   112  124  136

Stations

Figure 2.2.2.4.dConfiguration with PRI / BRI line and 24 and 12 port Extension Boards

BKSU

1)

- Note

- This is only the configuration chart when a customer uses the maximum wireless terminals with RMWT 64. If RMWT is set by another value, above will be different configuration chart according to the RMWT.

RMWT can set by multiple of 8 in the range of 8 to 64. A customer can program this RMWT (default 8) in Admin field 02, Flexible button 3.

- RMWT: Registrable Maximum number of Wireless Terminal. For more details, refer to Installation and Maintenance Manual of GDK-162/100/FPII WOTS (Wireless Office Telephone System)

- CO capacity in above chart means the quantity of B-channels.

- Above configuration can be a little different according to the interface mode (T or S) of STIB.

- See the description on clause 2.2.2.2.a - Note 1) and 2), in relation to the configuration with 6 port extension boards and 2B module.

### SYSTEM SPECIFICATIONS

The following Tables provide general system specifications.

Table 2.3.1	Dimensions & Weights

| Item | Height | Width | Depth | Weight |
| --- | --- | --- | --- | --- |
| Basic Key Service Unit | 420mm 16.5in | 401.4mm 15.7in | 187mm 7.4in | 15 Kg 33.3 lbs |
| Small Basic Key Service Unit | 420mm 16.5in | 330mm 13.0in | 187mm 7.4in | 12 Kg 26.7 lbs |
| Expansion Key Service Unit | 420mm 16.5in | 162mm 6.4in | 187mm 7.4in | 4.6Kg 10.2 lbs |
| Digital Key Set | 236mm 9.3in | 192mm 7.6in | 84mm 3.3in | 1.5 Kg 3.3 lbs |
| Electronic Key Set | 220mm 8.6in | 220mm 8.6in | 70mm 2.8in | 1.6 Kg 3.51bs |
| Digital DSS/DLS Console | 236mm 9.3in | 125mm 4.9in | 62mm 2.4in | 0.9 Kg 2.0 1bs |
| Electronic DSS/DLS Console | 236mm 9.3in | 125mm 4.9in | 62mm 2.4in | 0.9 Kg 2.0 1bs |
| Digital ICM/Door Box | 45mm 1.8in | 140mm 5.5in | 100mm 3.9in | 0.5 Kg 1.1 1bs |
| Electronic ICM/Door Box | 45mm 1.8in | 140mm 5.5in | 100mm 3.9in | 0.5 Kg 1.1 1bs |

Table 2.3.2	Environmental Specifications

|  | degrees C | degrees F |
| --- | --- | --- |
| Operating Temperature | 0-50 | 32-122 |
| Optimum Operating Temperature | 20-26 | 68-78 |
| Storage Temperature | 0-70 | 32-158 |
| Relative Humidity | 0-85% non-condensing |  |

Table 2.3.3	Electrical Specifications

| Power Supply : AC Voltage Input AC Power Heat Dissipation AC Input Fuse DC Output Voltages | 110/220 - 240 10% Volts AC @ 48-63Hz. PSU1:100W, PSU3:250W 615 BTUs maximum 4.0 amp @ 220-240Volt / 6.0 amp @ 110 Volts +5, -5, +28Volts |
| --- | --- |
| Back-up Battery : PSU Input Voltage PSU Battery Fuse Charging Current | 24 Volt DC 10 amp @ 250 Volt Max 500mA |
| Ext. Relay Contacts | 1 amp @ 24 Volt DC |
| Music Source Input | Max. 0 dBm @ 600 ohms |
| External Page Port | Max. 0 dBm @ 600 ohms 1 milli-watt max. input |

- Note:

The production of GDK-100 PSU1 has been officially stopped since March 1999.

Table 2.3.4	Electrical Specifications

|  | 22 AWG Wire | 24 AWG Wire |
| --- | --- | --- |
| Digital Keyset 2-wire loop | 500 m (1.6 Kft) | 330 m (1 Kft) |
| 2B modules( with Power Adapter, 24Vdc/200 mA) System – Primary Keyset Primary Keyset – Secondary Keyset Primary Keyset – Secondary SLT |  | 300 m 300 m 1000 m |
| 2B modules( without Power Adapter) System – Secondary Keyset System – Secondary SLT |  | 10 m 100 m |
| Electronic Keyset 4-wire loop | 500 m (1.6 Kft) | 330 m (1 Kft) |
| Single Line Telephone With SLIB24 or DSIB With SLIB48 With SLIB II | 2,500 m (8.2 Kft) 5,000 m (16.5 Kft) 2,500 m (8.2 Kft) | 1,600 m (5.2 Kft: 490 ohm) 3,300 m (10 Kft: 790 ohm) 1,600 m (5.2 Kft: 490 ohm) |

- Note: The distance between KSU and SLT may differ from above in some countries, because of the relevant special national specification.

Table 2.3.5	CO Loop Specifications

| Ring Detect Sensitivity | 16-30 Hz, 40 Vrms 30-60 Hz, 30 Vrms |
| --- | --- |
| DTMF Dialing Frequency Deviation Signal Rise time Tone Duration, on-time Interdigit time | less than 1% 3 msec, maximum 75 mesc, minimum 75 mesc, minimum |
| Pulse Dialing Pulse Rate Break/Make Ratio | 10 pps or 20 pps 60/40% or 67/33% |

- Note: Ring Detection sensitivity and Pulse Ratio may differ from above in some countries, because of the relevant special national specification.

Table 2.3.6	Miscellaneous Specifications

| Main System Memory | Memory size |
| --- | --- |
| Read-Only-Memory, EPROM Random Access Memory, RAM | basic = 2 Megabytes basic = 512 Kbytes expansion = 512 Kbytes (MEMU / SMEMU) |
| Account Codes | Number |
| Number of codes Digits per code | no limit 12 |
| Authorization of codes | Number |
| Number of codes Digits per code | 164 5 |

### SYSTEM COMPONENTS

### BKSU/ SBKSU (Basic Key Service Unit/Small Basic Key Service Unit)

The BKSU and SBKSU are metal frame cabinets designed for wall mounting. The BKSU/SBKSU contain a back plane with 10/7 card edge connectors and space is provided for a internal Ring Generator Unit. The backplane also have two connectors. One is for Ring Generator Unit and the other is for PFTU. The left most slot houses the Power Supply Unit and the right most slot is assigned for Main Processor Board. The remaining card slots, numbered slot 1-8 for BKSU, slot 1-4 and 8 for SBKSU, house the various system plug-in boards.

The MISB is operated on only slot #8. Digital Trunk Board (E1IB, PRIB, BRIB), WTIB and DVIB can be installed only on slots #1 through #7. The other cards can be installed on any slots. Cable ties are located on the front edge of the lower card rail to allow connecting cables for the station boards, CO interface boards, and MISB to be neatly installed. The connecting cables for the stations and CO Lines exit through the outlet in the bottom of BKSU/SBKSU and can be connected to the MDF or to a user installed termination point.

- Note: GDK-100 SBKSU is not supplied any more.

### EKSU (Expansion Key Service Unit)

The Expansion KSU is a metal housing designed for wall mount installation. The EKSU contains a back plane with 4 card edge connectors. The card slots numbered slots 9 through 12, house the various system plug-in boards except Digital Trunk Boards, WTIB, DVIB and MISB. On the front edge of the lower card rail are cable ties for the station and CO interface cables. The connecting cables for the stations and CO lines exit through the outlet in the bottom of EKSU and can be connected to the MDF or to a user installed termination point. The EKSU is intended to be directly connected to the right side of BKSU/SBKSU.

### PSU (Power Supply Unit)

A Power Supply Unit, PSU, is required in the Key Service Units. The PSU converts commercial AC power (110 –127 / 220 - 240 VAC @ 50-60 Hz) to DC voltages, regulates the voltages, and provides the appropriate DC voltages to the back plane for distribution to other system components. Three DC outputs are provided : ±5 VDC, +28 VDC. LEDs in the PSU front panel indicate valid outputs as well as the presence of AC input power.

The PSU includes circuitry to float charge externally connected 24 volt batteries and control operation of the battery back-up circuits. And the PSU will provide system operating voltages from the batteries if commercial AC power fails. In addition, battery back-up control circuitry is incorporated in the PSU to disconnect the batteries prior to a deep discharge or over-charged.

There are two kinds of PSUs; PSU1 and PSU2. See the clause; table 2.3.3 and the clause 3.3.5.

### RGU (Ring Generator Unit)

The Ring Generator Unit provides the Ring voltage to the SLIB circuits to ring the SLT. Also the RGU provides the input to the Message Wait source on the SLIB cards. The output of the RGU is 90VAC, 20 Hz or 25 Hz. Two RGU versions are available; one is internal type, the other is external type. The internal RGU can support simultaneous ringing for 5SLTs and the External RGU for 10SLTs.

There are 7 kinds of RGUs as followings,

-  RGU1: 20Hz step sine wave, external

-  RGU2: 25Hz step sine wave, external

-  RGU3: 20Hz step sine wave, internal

-  RGU4: 25Hz step sine wave, internal

-  RGU5: 25Hz pure sine wave, external

-  RGU6: 25Hz pure sine wave, internal

-  RGU7: 20Hz pure sine wave, internal

One of them can be selected and installed according to the desired capacity of ring and the requirements of local regulations.

The internal RGU is fitted on the lower right side panel inside the basic cabinet, the external RGU is mounted suitable place outside the cabinet. And all RGUs are electrically connected to the back plane, regardless of RGU type.

### PFTU (Power Failure Transfer Unit)

The PFTU provides the relay contacts for transfer of 6 CO Lines to 6 SLTs in the event of a power or processor failure. The PFTU is installed outside of the KSU and up to 5 PFTU modules can be connected to the KSU with the cascade connection of control wire. The PFTU is equipped with a manual switch that activates the Power Failure Transfer mode for testing purpose.

A 2-pin connector with screws is fitted for the control signal. And a RJ21 type male connector is fitted for the connection between CO line, CO interface circuit, SLT interface circuit and SLT.

### MPB (Main Processor Board)

The MPB controls and manages the communication between peripheral interface, supervises all resources in the system, controls the gain adjustment of PCM signal, generates system tone and manages call processing of the system.

The MPB, incorporates the system's RAM, master clock, MOH 1 port, RS-232C 1 port and watchdog circuitry as well as the system's PCM voice processing and main micro-processor. The micro-processor is a 16 bit high speed CPU which receives and transmits signaling information from/to other PCBs, controls feature activation, and PCM time-slot interchange. The MPB contains 512 Kbytes of RAM and can be expanded to 1Megabyte with the MEMU or SMEMU. RAM associated with the system database and the real-time-clock are protected from power loss by a long life high energy lithium battery. On board ROM contains PCM tone, gain table etc. needed for the digital voice processing.

The 2 Megabytes of ROM on MPB contains basic system operating software.

MPB should be installed in the MPB slot in the right most side of BKSU/SBKSU.

MPB contains 1 internal MOH source, 1 RS-232C.

Add-on boards: MEMU, SMEMU, PLLU, MODU

### MISB (Miscellaneous Board)

The MISB incorporates the circuitry and interfaces for common optional features including:

 External Paging			2 ports

 External Control Contacts		4 contacts

 External BGM & MOH		2 ports

 Alarm/Door Detection		1 input

 RS-232C(optional SIU)		2 ports

A RJ21 type female connector is mounted on the front edge of the MISB for the connection to above circuits except RS232C serial interfaces.

One MSIB board can be installed only in the slot No. 8.

Add-on board: SIU

### DVIB (Digitized Voice Interface Board)

The Digitized Voice Interface Board (DVIB) provides announcements for the system's ACD/UCD features as well as the System Voice Prompts and Recorded User Greetings. Following table shows the capacity of memory, ADPCM channel of DVIB, and its optional board.

Table 2.4.8	The capacity of DVIB

| Item | Basic | Option(for expansion) | Maximum Capacity |  |
| --- | --- | --- | --- | --- |
|  |  |  | a board | a system |
| ADPCM channel | 4 channels | ADPU 4channels | 8 channels | 16 channels |
| System Voice Prompt | 2 EPROMs 1 Mbytes 240 seconds | none | 1 Mbyte 4 min. | 1 Mbyte 4 min. |
| ACD/UCD Announcement | 6 SRAMs 768 Kbytes 180 seconds | none | 768 Kbytes 180 sec. (3 min) | 768 Kbytes 180 sec. (3 min) |
| User Greeting | 8 DRAMs 4 MBytes 1024seconds | DMEU:2 Mbytes(512 seconds) or DMEU4:4 Mbytes(1024 seconds) | 8Mbytes 2048 sec. (34 min.) | 16 Mbytes 4096 sec. (68 min.) |

- DVIB allows a DMEU or a DMEU4 to be expanded.

- GDK-100 allows up to 2 DVIB board to be installed.

- ACD/UCD Announcements are protected from power loss by a long life lithium battery, but User Greetings are not.

- The capacity of ACD/UCD announcements will not be expanded even though 2 boards are installed in a system.

DVIB employs ADPCM channel to store and play System Voice Prompts, ACD/UCD announcements, and Recorded User Greetings. All voice data will be stored and reproduced by using ADPCM technique that compresses the 64Kbps to the 32Kbps in record mode and expands the 32Kbps to the 64Kbps in play mode.

Note that each channel can operate as either record or play mode at a time.

The System Voice Prompts for the standard version of the DVIB are recorded in English. However, customized recordings can be made available for local market requirements.

There are three option board, one is ADPU for ADPCM channel expansion and the others are DMEU and DMEU4 for DRAM expansion.

DVIB can be installed in any of slot No. 1-7.

And maximum two boards can be installed in a system.

Add-on boards: ADPU, DMEU, DMEU4

### Extension Boards

The various types of GDK-100 Extension Boards have capable of supporting various types of terminals as followings,

| Board Name | Function | Remark |
| --- | --- | --- |
| DTIB12 | Provides 12 Digital Terminal interfaces | 2 wire |
| DTIB24 | Provides 24 Digital Terminal interfaces | 2 wire |
| ETIB | Provides 12 Electronic Terminal interfaces | 4 wire |
| SLIB24 | Provides 6 SLT interfaces with a line voltage of 24V. | 2 wire |
| SLIB48 | Provides 6 SLT interfaces with a line voltage of -48V. | 2 wire |
| SLIBII | Provides 12 SLT interfaces with a line voltage of -28V. | 2 wire |
| DSIB | Provides 6 Digital Terminal and 6 SLT with a line voltage of 24V. | 2 wire |
| WTIB/WTIU | Be capable of accommodating up to 8 Base Stations. (up to 64 wireless terminals) | 4 wire |
| STIB | Be capable of accommodating up to 4 ISDB BRI S interfaces, 8 B channels. (Each interface is T/S switchable) | 4 wire |

### DTIB12/DTIB24 (Digital Terminal Interface Board 12/24)

DTIB allows the LGE proprietary digital terminals to be connected. DTIB provides 2-wire interfaces for terminal connection and three versions according to the capacity are available;

| Board Name | DKT interface circuits | SLT interface circuits | Remarks |
| --- | --- | --- | --- |
| DTIB12 | 12 cct | None |  |
| DTIB24 | 24 cct | None |  |
| DSIB | 6 cct | 6 cct | See clause 2.4.9.5 |

- Note:

There are KD, KD/E, KD/C, LKD series DKTU, KD and KD/E DSS, Digital Phone box, and external CTI module as LG digital terminals. KD, KD/(E)C, external CTI module and Data Module are not available now due to the production stop.

See clause 2.4.13.4 2B-module and clause 2.4.13.5 GDK-PC phone on section 2 for more details regarding 2B-module and CTI module.

Digital Interface Board provides digital voice and data communications to/from digital terminals. The board allows for either 1 or 2 bearer channels from a single hardware interface under the control of system software.

A industry standard RJ21 type female connector is mounted on the front edge of the PCB for connection to the station interfaces.

In addition, one LED is mounted on the PCB to indicate the state of the connected terminals. It will turn on when one or more ports are busy.

DTIB12/DTIB24 can be installed in any of slot No. 1-12.

The maximum extension boards except Wireless Terminals are limited by 72 extensions in a system.

### ETIB (Electronic Terminal Interface Board)

ETIB allows the LG proprietary electronic terminals to be connected. There are GK /GSX EKTU series, GSX DSS and KD-ICM box as electronic terminals. ETIB provides 4-wire interfaces for terminal connection; 2-wire for voice communication and 2-wire for signaling data. The board provides for 12 interface circuits, incorporates circuits for A/D and D/A conversion of voice signals, and interprets and sends signaling data to/from EKTU.

ETIB has an industry standard RJ21 type female connector mounted on the front edge for connection to the station MDF.

In addition, an LED is mounted on the PCB to indicate the status of the connected terminals. It will turn on when one or more ports are busy.

The ETIB can be installed in any of slot No. 1-12.

The maximum extension boards except Wireless Terminals are limited by 72 extensions in a system.

### SLIB24/SLIB48 (Single Line Interface Board with 6 ports)

SLIB allows single line analog devices to be connected.

Four kinds of SLIB boards are available in GDK-100 system;

| Board Name | Interface circuit | Feed voltage | Remark |
| --- | --- | --- | --- |
| SLIB24 | 6 cct | 24VDC battery feed |  |
| SLIB48 | 6 cct | -48VDC battery feed |  |
| SLIBII | 12 cct | -28VDC battery feed | See 2.4.9.4 |
| DSIB | 6 cct | 24 VDC battery feed | See 2.4.9.5 |

- Note

- DSIB provides 6 digital terminal interfaces and 6 single line analogue interfaces.

For SLIB II and DSIB, see the parts of SLIBII and DSIB in this section.

SLIB24/SLIB48 are capable of accommodating up to 6 single line analog devices. The boards are configured with hybrid circuit and COMBO; converts A/D and D/A signal. SLIB24/SLIB48 allow the SLT access to CO Lines connected to the system, other stations, and most features of the system through the use of dial codes. Connections to the SLIB are made through RJ21 type female connector.

The SLIB24/SLIB48 can be equipped with a Message Wait source (optional MSGU for SLIB24 or MSGU48 for SLIB48), which activate a lamp in the SLT, if so equipped. The Message Wait source sends 90 VDC signal to the SLT illuminating the MW lamp. A Ring Generator Unit is required in the cabinet where a SLIB is installed. An optional DTMF receiver unit (DTRU) can be installed on the SLIB24/SLIB48.

The SLIB24 allows a Single Line Telephone to be connected up to 2.5 Kilometers (8,200 feet, loop resistance: 490 ohm) from the system using 22 AWG wire. The SLIB48 allows a SLT to be connected up to 5 Kilometers (16,500 feet, loop resistance: 790 ohm) from the system using 22 AWG wire. See the Table 2.4.3.

In addition, an LED is mounted on the PCB to indicate the status of ports connected to SLIB. It will turn on when one or more ports are busy.

The SLIB24/SLIB48 can be installed in any of slot No. 1-12.

The maximum extension boards except Wireless Terminals are limited by 72 extensions in a system.

Add-on boards: DTRU, MSGU (for SLIB24), MSGU48 (for SLIB48)

### SLIB II (Single Line Interface Board with 12 ports)

SLIB II provides the capability to connect 12 single line analogue devices to the system with appropriate A/D and D/A conversions. SLIB II is -28 VDC Battery feed. The SLIB II allows the SLT access to CO Lines connected to the system, other stations, and most features of the system through the use of dial codes. Connections to the SLIB II are made through RJ21 type female connector.

The SLIB II is equipped with a Message Wait source, optional MSGU II, which activate a lamp in the SLT, if so equipped. The Message Wait source sends 90 VDC signal to the SLT illuminating the MW lamp. A Ring Generator Unit is required in cabinet where a SLIB II is installed.

The SLIB II allows a Single Line Telephone to be connected up to 2.5 Kilometers (8,200 feet, loop resistance: 490 ohm) from the system using 22 AWG wire. See the Table 2.4.3.

In addition, an LED is mounted on the PCB to indicate the status of ports connected to SLIB II. It will turn on when one or more ports are busy.

SLIB II can be installed in any of slot No. 1-12.

The maximum extension boards except Wireless Terminals are limited by 72 extensions in a system.

Add-on boards: DTRU II, MSGU II

### DSIB (DKTU/SLT Interface Board with 12 ports)

DSIB provides 6 SLT and 6 DKT interfaces. Connections to the DSIB are made through RJ21 type female connector. An LED is mounted on the PCB to indicate the status of ports connected to SLIB. It will turn on when one or more ports are busy.

6 DKT interface circuits on this board have the same characteristics that those on DTIB12/24 do. See the part of DTIB12/DTIB24 in this section.

6 SLT interface circuits on this board also have the same characteristics that those on SLIB24 do as followings. DSIB is 24 VDC Battery feed. The DSIB is equipped with a Message Wait source, optional MSGU, which activate a lamp in the SLT, if so equipped. The Message Wait source sends 90 VDC signal to the SLT illuminating the MW lamp. A Ring Generator Unit is required in cabinet where a DSIB is installed.

The DSIB allows a Single Line Telephone to be connected up to 2.5 Kilometers (8,200 feet, loop resistance: 490 ohm) from the system using 22 AWG wire. See the Table 2.4.3. The DSIB also allows a DKTU to be connected up to 0.5 Kilometers (1600 feet) from the system using 22 AWG wire.

The DSIB can be installed in any of slot No. 1-12.

The maximum extension boards except Wireless Terminals are limited by 72 extensions in a system.

Add-on boards: DTRU, MSGU

### WTIB /WTIU (Wireless Terminal Interface Board / Unit)

The WTIB provides standard interface between the GDK-100 digital Key Telephone system and DECT (Digital Enhanced Cordless Telecommunications) Network. The system can accommodate one WTIB, which is capable of supporting up to 4 Base Stations. The WTIB can be optionally equipped with WTIU to expand Base Stations, which can support up to additional 4 base stations. So the system allows maximum 8 Base Stations to be connected. Following table shows the capacity of WTIB and its expansion board.

| Items | WTIB | Max. Capacity with WTIU(expansion) |
| --- | --- | --- |
| Maximum Cell Number (Base Station Number) | 4 | 8 |
| Voice channels / Cell | 5 | 5 |
| Registrable Maximum Terminal Number | 8 – 64 by step 8 | 8 – 64 by step 8 |
| Max. Simultaneous Wireless calls | 16 | 32 |

- Note

- Registrable Maximum Wireless Terminal numbers are programmable in Admin field 02.

- WTIB/WTIU contains RJ11 type connectors for the connection to Base Stations.

A WTIB should be installed in any of slot No. 1-7.

MEMU( or SMEMU ) should be installed on the MPB for proper operation.

For more details, see INSTALLATION and MAINTENANCE MANUAL (GAP) of GDK-162/100/FII WOTS(Wireless Office Telephone System).

### Analog CO Line Boards

GDK-100 Analog CO line Boards have capable of supporting various types of analog CO line as followings,

| Board | Function | connection |
| --- | --- | --- |
| LCOB | Provides Loop Start CO interfaces, 4 interfaces | 2 wire |
| AC15 | Provides AC15a signaling interfaces, 3 interfaces; UK | 4 wire |
| EMIB | Provides 2-wire E&M interfaces, 4 interfaces; Korea | 4 wire |
| DIDB | Provides Direct Inward Dialing interfaces, 4 interfaces; Korea | 2 wire |
| BWDIDB | Provides Both-way DID interfaces, 4 interfaces; New Zealand | 2 wire |

- Note

Analog CO line Boards except LCOB are designed to comply with special national specification and application, so it may not suitable for the other national specification and/or application.

### LCOB (Loop Start CO Line Interface Board)

LCOB provides 4 CO/PBX Loop Start CO Lines which support pulse /DTMF signal. Each Interface contains ring and loop current detection circuits, A/D and D/A conversions, and pulse and ground flash signaling circuit. LCOB contains 4 LEDs to indicate the status of each CO line.

RJ21 type male connector is mounted on the front edge of the LCOB for the connection to lines.

LCOB can be optionally equipped with up to 4 Call Metering Units (CMU) to monitor call charge, a Call Progress Tone Unit (CPTU) to monitor call progress tone and a DTMF Receiver Unit (DTRU) to detect DTMF.

A CMU supports call metering function and can be installed on each line. And the appropriate type of CMU should be installed on the line according to the national specification.

Each CPTU contains two tone detection circuits that are time shared under software control as system resources. These detectors are used in support of several features such as ACNR. If such features are to be used, it is recommended that at least one CPTU be provided for every LCOB. According to sensitivity of tone detector, two types of CPTU (CPTU/A, CPTU/B) are available. CPTU/A has wide tone detection range(305Hz~640Hz) and CPTU/B has narrow tone detection range (350Hz, 620Hz, 440Hz, 480Hz±1%).

LCOB can be installed in any of slot No. 1-12.

Analog CO interface boards are limited by maximum 32 analog lines in a system.

Add-on boards: DTRU, CPTU/A, CPTU/B, CMUPR/CMU50/CMU12/CMU16/CMU12PR/CMU50PR

### AC15 Board (AC15 Private Line Interface Board) - UK

AC15 board provides an analog interface between outside tone controlled private lines and the system. An AC15 board has 3 interface circuits that support AC15a signaling and DTMF or Pulse dialing. Each AC15 private line interface circuit provides two-pair (transmitting and receiving) audio interface circuit and signaling to and from the system and private line.

The AC15 board contains three LEDs to indicate the status of each line.

A RJ21 type male connector is mounted on the front edge of the AC15 Board for the connection to lines.

It is designed to comply with the UK specification and application, so it may not suitable for the other national specification and/or application.

AC15 can be installed in any of slot No. 1-12.

The maximum analog CO interface boards are limited by 32 analog lines in a system.

Add-on board: DTRU

### EMIB (E&M Tie Line Board) -Korea

EMIB provides 4 interface circuits that support 2-wire E&M respectively. Each line supports E&M type V of E&M line.

The EMIB board contains four LEDs to indicate the status of each line. 4-pair male connectors are mounted on the front edge of the EMIB and 4-pair male connectors with screw are provided for the connection to lines.

It is designed to comply with the Korean specification and application, so it may not suitable for the other national specification and/or application.

EMIB can be installed in any of slot No. 1-12.

The maximum analog CO interface boards are limited by 32 analog lines in a system.

Add-on board: DTRU

### DIDB (Direct Inward Dialing Board) -Korea

The DIDB provides 4 analog DID interfaces circuits, which support DTMF or Pulse dialing, but not R2 MFC dialing. It only supports Incoming call.

Each of line circuits contains A/D and D/A conversion circuitry, control circuitry, and incoming dial detection circuitry. DID supports immediate, wink or delay start supervision and can detect incoming dial pulse or DTMF dialed digits.

The board contains 4 LEDs on the front edge of the board, to indicate the status of each line.

4-pair male connectors are mounted on the front edge of the EMIB and 4-pair male connectors with screw are provided for the connection to lines.

It is designed to comply with the Korean specification and application, so it may not suitable for the other national specification and/or application.

DIDB can be installed in any of slot No. 1-12.

The maximum analog CO boards are limited by 32 analog lines in a system.

Add-on board: DTRU

### BWDIDB (Both-way Direct Inward Dialing Board) - New Zealand

BWDIDB provides 4 both-way DID interfaces, which support DTMF or Pulse dialing. It supports both Incoming and Outgoing calls with pulse or DTMF.

Each interface consists of polarity reversal, pulse dialing, call direction control, open loop control, loop detection and polarity detection circuit.

The BWDIDB contains four LEDs to indicate the status of each line. A RJ21 type male connector is mounted on the front edge of the board for the connection to lines.

DTMF Receiver Unit (DTRU), Call Progress Tone detection units (CPTU/A) can be installed optionally on the BWDIDB.

It is designed to comply with the New Zealand specification and application, so it may not suitable for the other national specification and/or application.

BWDIDB can be installed in any of slot No. 1-12.

The maximum analog CO boards are limited by 32 analog lines in a system.

Add-on boards: DTRU, CPTU

### ISDN Boards

GDK-100 ISDN Boards are capable of supporting various types of ISDN as followings,

| Board | Function | connection |
| --- | --- | --- |
| PRIB | Provides ISDN Primary Rate interfaces, 1 interface /30 B channels | 4 wire |
| BRIB | Provides ISDN Basic Rate interfaces(T), 4 interfaces /8 B channels | 4 wire |
| STIB | Provides ISDN Basic Rate interfaces(T/S switchable), 4 interfaces /8 B channels | 4 wire |
| E1IB | Provides Digital Trunk(E1) interfaces, 1 interface /30 B channels | 4 wire |

### ISDN PRIB (Primary Rate Interface Board)

PRIB (Primary Rate Interface Board) has capable of connecting to 1 PRI line, which has an interface circuit for 30 time slots with 2.048Mbps data rate speed. PRIB will be positioned at reference point Ia on ETS 300 001, that is TE slave without power feeding.  And, Data & bit clock are transmitted by the NT master and extracted by TE salve.

PRIB contains reset and watchdog circuit, address decoder, external RAM, program memory (ROM), the circuitry to control PLL device and DBID, and the board line’s specific circuitry. The line interface circuit consists of LXT304A which provides receive jitter attenuation, MT8979 which provides a complete interface between a 2.048Mbps primary rate and MITEL’s Serial Telecomm Bus (ST-BUS), and MT8920 which is provides a simple interface between ST-BUS and parallel system environments.

PRIB should be installed in any of slot No. 1-7.

A PLLU MUST be installed for proper operation.

One PRIB can be installed in a system.

QSIG function can be supported through PRIB (software version 99A), and SMEMU MUST be used for this function.

### ISDN BRIB (Basic Rate Interface Board: T interface only)

BRIB has specified to support four interface circuits that allow interfacing the ISDN BRI. Four interface circuits are used for the transmission of digital signals. Data & clock are transmitted by the NT master and extracted by TE salve. GDK-100 BRIB will be positioned at reference point Ia (T) on ETS 300 012, that is TE slave without power feeding.

BRIB contains reset and watchdog circuit, address decoder, external RAM, program memory(ROM), the circuitry to control the highway interface(DBID), and the board line’s specific circuitry. The board line’s specific circuitry is PEB2084 for Physical layer and PEB2075 for data link layer.

BRIB should be installed in any of slot No. 1-7.

A PLLU MUST be installed for proper operation.

The maximum 6 BRIBs can be installed in a system, but total CO lines including analog CO of the system should not be more than 48 lines.

QSIG function can be supported through BRIB (software version 99A), and SMEMU MUST be used for this function.

### ISDN STIB (Basic Rate Interface Board: switchable S/T interface)

The S or T interfaces are based on the existing interface described in ETSI 300. 012, which is based on ITU_T Recommendations I.430 and provides modification and further requirements.

This is applied at the S or T reference points for the basic interface structure defined in ITU_T I.412. Layer 1 of the interface requires a balanced metallic transmission medium, for each direction of transmission capable of supporting 192Kbps(2B+D). For interface circuits, one per transmission direction, are used for the transmission of digital signals. Data & bit clock are transmitted by the NT master and extracted by TE slave. GDK system can be positioned at reference point Ia (T) or Ib (S) on ETS 300 012 that is TE-slave without power feeding or NT-master with power feeding. By selecting the jumper position on each line interface, STIB supports S-interface (Line card function)or T-interface (Trunk function).

SCC1 of MC68302 will serve the 64Kbps Monitor channel access which is use to convey message oriented local function such as software programming or S/Q channel information or access to internal register of line interface devices. SCC2 of CPU is assigned as 512Kbps HDLC communication to main processor on MPB. SCC3 of CPU is assigned as 38400bps RS232C channel. This RS232 channel will be used only in development stage for software and hardware debugging.

For physical layer interface function, SIEMENS’s PEB2084, Quadruple Transceiver for S/T interface which is implements four wire S/T interface used to link voice/data digital transmission to ISDN BRI is applied. It can handle up to 4 S/T interfaces simultaneously according to ITU-T I.430, ETSI 300.012, and ANSI T1.605 standards.

For data link layer interface function, SIEMENS’s PEB2075 ISDN D-channel exchange controller which is a serial HDLC data communication circuit with four independent channels is used. Each channel is fully independent and programmable by its own register set.

For power feeding control, PEB2025(Power feeding device from SIEMENS) is occupied. It can handle up to 4-channel with current limiting and short circuit protection.

STIB should be installed in any slot of slot No. 1-7.

A PLLU MUST be installed for proper operation.

The maximum 7 STIB boards can be installed in a system, but total CO lines including analog CO should not be more than 48 lines(channels), neither total extensions except WTIB should be more than 72 port(channels). .

QSIG function can be supported through STIB T-interface only (software version 99A), and SMEMU MUST be used for this function.

### E1IB (E1(CEPT) Interface Board)

The E1 line interface is based on the existing interface described in G.703, 704, 823 which is based upon  CCITT Recommendations I.431 and provides modifications and further requirements. It is also affected by CCITT Recommendation, G.703, G.823 for what belongs to the electrical level. It covers data rate at 2048kbit/sec which is used in Europe. Two interfaces circuit, one per transmission direction, are used for the transmission of digital signals. Data & bit clock are transmitted by the NT master and extracted by TE slave.

The PLL devices extract the reference clock from the bit stream and generate the system main clock 32.768MHZ and data clock 2.048MHZ, 4.096MHZ which are synchronized. The Frame has a duration of 125usec and contains 32 time slots (TSs) of 8 bits. TS 0 is allocated to frame alignment sync. and TS 16  is allocated to signaling channel; the other TSs are available for allocation to the B channel. MITEL’s CEPT PCM30 interface device, which provides a complete interface between a 2.048Mbit/s digital stream and MITEL ST-BUS, is applied.

E1IB should be installed in any of slot No. 1-7.

A PLLU MUST be installed for proper operation.

One E1IB can be installed in a system.

Add-on board: CPTU

- Note:

- The existing E1IB supports only Decadic Pulse and DTMF signaling, but not MFC R2 signaling.

- It is designed to comply with the special national specification and application, so it may not suitable for the other national specification and/or application.

### Add-on boards

This chapter describes add-on boards, which can be installed on various types of boards to support the proper function

| Board Name | Function | position |
| --- | --- | --- |
| MEMU | Expands SRAM size. | MPB |
| SMEMU | Expands SRAM size and provides QSIG,CTI, and PCATTN. | MPB |
| MODU | Provides a 1200/2400 baud modem for local access | MPB |
| PLLU | Provides the synchronized system clock when ISDN installed. | MPB |
| SIU | Provides additional two RS232 serial interfaces. | MISB |
| DTRU/DTRUII | Provides further 2/4 DTMF receivers | 1) |
| Board Name | Function | position |
| MSGU/MSGU48 | Provides Message Waiting signals to 6 SLT ports | 2) |
| MSGUII | Provides Message Waiting signals to 12 SLT ports | SLIBII |
| CPTU/A,CPTU/B | Provides Call progress detection function | 3) |
| CMU | Provides Call Metering signal detection | 4) |
| DMEU/DMEU4 | Expands DRAM size for User Greetings(2M/4M) | DVIB |
| ADPU | Expands 4 ADPCM channels | DVIB |

- Note:

- DTRU can be installed on SLIB24, SLIB48, DSIB, LCOB, AC15, BWDIDB, EMIB, and DIDB. And DTRUII can be only installed on SLIBII.

- MSGU can be installed on SLIB24 and DSIB, MSGU48 on SLIB48, and MSGUII on SLIBII.

- CPTU/A, CPTU/B can be installed on LCOB, BWDIDB and E1IB.

- Each type of CMU can be installed on the LCOB according to the local market condition and regulatory requirements.

### MEMU (Memory Expansion Module)

The MPB contains the basic system RAM memory. The RAM provides the memory for the system database, speed dial numbers, SMDR buffer, etc. The basic RAM is 512 Kbytes and can be expanded up to 1Mbyte with the MEMU.

MEMU is required for following function.

- Increase the SMDR record buffer from 100 to 2,000

- On-demand Ring

- ISDN CLIP and CO Message Wait: up to 1000

- ISDN Flexible DID table: 000~999 (* User programmable bins: 000~299)

- MSN

- ACD Statistics

- Expanded LCR Table

- WTIB (DECT GAP)

- Enhanced CCR  1)

- Note:

- Above features are supported by the software 98FEB or later version. But the Enhanced CCR is only supported by the software 99A or later version.

MEMU is optionally installed on the MPB.

MEMU (or SMEMU) MUST be installed on MPB if the WTIB is installed in the system.

### SMEMU (Security Memory Expansion Unit)

The SMEMU is an enhanced version of existing MEMU. So it provides all the function of MEMU. In addition, the SMEMU allows QSIG, TAPI and/or PC-ATTENDANT to be operated in GDK-100 system. See the description of MEMU. The memory size is 512 Kbytes.

SMEMU is required for following features.

- Increase the SMDR record buffer from 100 to 2,000

- On-demand Ring

- ISDN CLIP and CO Message Wait: up to 1000

- ISDN Flexible DID table: 000~999 (* User programmable bins: 000~299)

- MSN

- ACD Statistics

- Expanded LCR Table

- WTIB (DECT GAP)

- Enhanced CCR			1)

- QSIG				1), 2)

- PC-Attendant			1), 2)

- GDKSP based on TAPI 2.1		1), 2)

- Note:

- Above features are supported by the software 98FEB or later version. But the features marked 1) are only supported by the software 99A or later version.

- SMEMU should be required for the features marked 2).

The SMEMU can be optionally installed on the position of MEMU of MPB.

SMEMU MUST be installed on the MPB for proper operation of QSIG, TAPI and/or PC-ATTENDANT in the system.

SMEMU or MEMU MUST be installed for the proper operation of the WTIB.

### MODU (Modem Unit)

The Modem Unit provides an asynchronous modem for access to the system database and fault reporting features from a remote site. The MODU incorporates a 1200/2400 baud modem. The modem may be connected to a pre-selected CO Line through the system-switching matrix. The MODU port is independent of the SIU standard RS 232 port, allowing system database access, etc. without the need to interrupt the SMDR output.

MODU is optionally installed on the MPB

### PLLU/PLLU2 (Phase Locked Loop Unit )

PLLU (PLL Unit) is an option board which generates 32.768MHz clock synchronized to Digital Trunk interface board. The 32.768MHz clock is provided to GSXD on MPB. It consists PLL circuit and clock (from Digital Trunk interface board) monitoring circuit.

For the system synchronization, the clock should be extracted from BRIB, STIB with T mode, PRIB or E1IB. But, if clock is not generated from BRIB, STIB with T mode, PRIB or E1IB(PLL circuit is unstable or the clock from ISDN card is absent), this unit generates master clock(32.768MHz) using internal clock.

The PLLU is installed on MPB.

There are three kinds of PLLU; PLLU, PLLU III and PLLU2 (issue3). Only PLLU2(issue 3) has been available since the end of November, 1998, but neither PLLU nor PLLU3 any longer. For reference, PLLU2 issue 2 or earlier should be not used in GDK-100 system.

PLLU should be installed on MPB when any ISDN card including E1IB is installed in the system.

### SIU (Serial Interface Unit)

SIU provides additional two RS232C interface circuits. Two connectors are 9 pin RS232C.

Serial Interfaces on MPB and SIU The SIU is useful for system maintenance, PC based admin., SMDR print out and SMDI for PC based voice mail system.

Followings are the list of the system’s output;

- Administration database

- Off-line SMDR (on-demand)

- On-line SMDR

- Statistical Information

- SMDI (voice mail)

- System trace data

The SIU can be optionally installed on MISB.

### DTRU/DTRU II (DTMF Receiver Unit)

A DTRU contains 2 DTMF receiver circuits, and a DTRU II has 4 DTMF receiver circuits. Each receiver of DTRU /DTRU II is time-shared under the control of the system software, as a system resource regardless its position.

The DTRU can be optionally installed on SLIB24, SLIB48, DSIB, LCOB, AC15, BWDIDB, EMIB and DIDB.

The DTRU II can be optionally installed on SLIB II.

Maximum DTMF receivers are 22 in a system.

### MSGU/MSGU48/MSGU II (Message Wait Unit)

The Message Wait Unit sends 90VDC signal to the SLT illuminating the Message Wait lamp. The MSGU and MSGU48 contain 6 Circuits to support 6 SLTs. The MSGUII contains 12 circuits to support 12 SLTs.

MSGU can be optionally installed on SLIB24 and DSIB.

MSGU48 can be optionally installed on SLIB48.

MSGU II can be optionally installed on SLIB II.

### CPTU (Call Progress Tone Detection Unit; CPTU/A, CPTU/B)

The Call Progress Tone Detection Unit is used to detect specific call progress tones which are supplied from the CO or PABX. Tone detection is employed to support features such as ACNR(Automatic Called Number Redial).

CPTU provides 2 tone detection circuits that are time-shared under the control of the system software, as a system resource regardless its position.

CPTU have two versions; CPTU/A and CPTU/B. CPTU/A has wide tone detection range (305Hz - 640Hz), and CPTU/B has narrow tone detection range (350Hz, 620Hz, 440Hz, 480Hz  1% ).

Any Kind of CPTU can be optionally installed on the LCOB, BWDIDB and E1IB.

It is important to check the local specification regarding call progress tone to use CPTU/B.

### CMU (Call Metering Unit)

The Call Metering Unit detects call metering signals from the CO or PABX to monitor call duration/charges. There are six kinds of CMUs as followings,

- 12KHz

- 12KHz and Polarity Reversal

- 16KHz

- 50Hz Longitudinal

- 50Hz Longitudinal and Polarity Reversal

- Polarity Reversal

CMU can be optionally installed on the LCOB and each CMU supports one CO Line.

Each type of CMU can be installed on the LCOB according to the local market condition and national regulatory requirements.

### DMEU/DMEU4 (DRAM Memory Expansion Unit)

The DMEU or DMEU4 allows for expansion of the DRAM memory on the DVIB. The DMEU expands to 512 seconds for Recorded User Greetings time. And the DMEU4 expands to 1024 seconds for Recorded User Greetings time.

DMEU or DMEU4 can be optionally installed on the DVIB.

See clause 2.4.8 DVIB.

### ADPU (ADPCM Unit)

The ADPU allows for expansion of ADPCM channels on DVIB. The ADPU expands to 4 channels for ADPCM.

ADPU can be optionally installed on DVIB,

### Keyset & Terminals

In addition to supporting analogue Single Line devices, by selecting appropriate interface boards, the system will support either or both the LG Digital Keysets /Terminals or the LG Electronic Keysets /Terminals.

### Digital Keysets and Terminals

Various types of digital terminals are used with GDK-100 DTIB12/DTIB24/DSIB as below:

| Model | Description | Model | Description |
| --- | --- | --- | --- |
| KD-36EXE | 24 Flexible Button Display | KD/E-36EXE | 24 Flexible Button Display |
| KD-36ENH | 24 Flexible Button Normal | KD/E-36ENH | 24 Flexible Button Normal |
| KD-24EXE | 12 Flexible Button Display | KD/E-24EXE | 12 Flexible Button Display |
| KD-24ENH | 12 Flexible Button Normal | KD/E-24ENH | 12 Flexible Button Normal |
| KD-33LD | 8 Flexible Button Large Display | KD/E-8BTN | 8 Flexible Button Normal |
| KD-DSS | 48 Button DSS/DLS Console | KD/E-36LD | 24 Flexible Button Large Display |
|  |  | KD/E DSS | 48 Button DSS/DLS Console |
| Model | Description | Model | Description |
| LKD-30DS | 30 Flexible Button Display | KD- Digital Phone Box | Digital Intercom Box |
| LKD-8DS | 8 Flexible Button Display | CTI module 1) | CTI module |
| LKD-2NS | 2 Flexible Button Normal |  |  |
| LKD-30LD | 30 Flexible Button Large Display |  |  |
| LKD-48DSS | 48 Button DSS/DLS Console |  |  |

- Note:

- This type of terminal and KD/C and KD/EC series Keysets will not be supplied any more, but CTIU8/CTIU30 which are inserted on the bottom side of LKD Keysets, are supplied for CTI operation. See GDK-PC PHONE (CTI) in this section.

- Minimum software requirements for LKD operation: 3.2 or later version (except 3.4 version)

Digital Keysets

General Description

Each Digital Keyset has a standard 12 button dial-pad, color coordinated handset, an integral Wall Mount kit, a slide-out Directory Tray, an array of  “Flexible Buttons”, and 12 Fixed Feature Access keys. All of the Flexible and Fixed Buttons, except Volume, incorporate a long-life, super-bright LED to indicate the feature or circuit status. The number of “Flexible Buttons” differ from mdes in above table . The fixed feature access keys are the same for each keyset 8/2 button Keyset. Each Digital Keyset includes a RJ-11 type jack for connection to the system. See Figures 2.4.1 to 2.4.3.

Speaker Phone

Each Digital Keyset except LDK-2NS is equipped with circuitry that enables the telephone to be used hands-free in two way conversations. This circuitry provides voice-switched speaker phone operation which gives channel control to the party with the highest energy level. The speaker phone circuit is controlled by the user with the MON button at KD, KD/E and the ON/OFF button at LKD Keyset.

Volume Control

The volume control is a rocker arm button. Pressing the right side increases volume, the left decreases volume. The volume button controls the volume level of voice and ring signals received at the speaker in the keyset. Also, in the Dial-By-Name (Directory Dial) mode, the volume button scrolls the name display up and down, scrolling display information to find a telephone number in the directory.

LCD Display

The Display Keysets with EXE suffix in model name incorporate a 2-line, 48-character Liquid Crystal Display (LCD). The LCD provides alpha-numeric display of various information to assist the user in operation of features. In the idle mode, the display will show the station name or number on the top line and the time and date on the second line.

And The Large Display Keysets with LD suffix also incorporate a 7-line, 112-character large Liquid Crystal Display (LCD)

Digital DSS

The Digital Direct Station Selection/Direct Line Selection (DSS/DLS) Consoles can be connected to any port of a DTIB in place of a Keyset. Up to 6 DSS/DLS Consoles can be associated with a Keyset in the system database. Each console incorporates a 4 by 12 matrix of “Flexible Buttons”, a total of 48 Buttons, each with an associated LED for status indications. The DSS/DLS Console includes a RJ-11 type jack for connection to the system.

Digital Phone Box

The Digital Phone Box provides hands-free intercom conversation from any location that generally does not require a fully functional Key Telephone. The Digital Phone Box is connected to any port of a DTIB in place of a Keyset. The Digital Phone Box includes a volume control slide switch and has 2 feature buttons with LEDs; one to call preprogrammed stations CALL and the second to block incoming calls DND.

### Electronic Keysets and Terminals

Electronic terminal means the Keysets that can be connected to the LG Analog Key Telephone System such as GHX system.

Various types of electronic terminals are used with GDK-100 ETIB as below:

| Model | Description | Model | Description |
| --- | --- | --- | --- |
| GSX 8 | 8 Button Keyset | GSX/E 8 | 8 Button Keyset |
| GSX 21 ENH | 21 Button Enhanced Keyset | GSX/E 21 ENH | 21 Button Enhanced Keyset |
| GSX 21 EXE | 21 Button Executive Keyset | GSX/E 21 EXE | 21 Button Executive Keyset |
| GSX 33 ENH | 33 Button Enhanced Keyset | GSX/E 33 ENH | 33 Button Enhanced Keyset |
| GSX 33 EXE | 33 Button Executive Keyset | GSX/E 33 EXE | 33 Button Executive Keyset |
| GSX 48DSS | 48 Button DSS/DLS Console |  |  |
| KD-ICMB | Electronic Intercom/Door Box 1) |  |  |

- Note:

All the types of electronic terminals can be connected to the GDK-100 ETIB. Almost every terminal can be also connected to the LG analog KTS, but KD-ICMB can be connected only to the LG Digital KTS, not to the LG analog KTS. On the contrary, GSX Phone Box can be connected only to the LG Analog KTS, not to the LG Digital KTS

Following table shows three types of Phone Boxes and the system to which they can be connected.

| Model | Digital KTS |  | Analog KTS |
| --- | --- | --- | --- |
|  | The DTIB of Digital KTS | The ETIB of Digital KTS |  |
| KD-Digital Phone Box | Yes | X | X |
| KD-ICM Box | X | Yes | X |
| GSX Phone Box | X | X | Yes |

Electronic Keysets

General Description

Each Electronic Keyset has a standard 12 button dial-pad, color coordinated handset, an integral Wall Mount kit, a slide-out Directory Tray, volume control slide switches, an ICM control switch, an array of “Flexible Buttons”, and 5 or 11 Fixed Feature Access keys. All of the Flexible and Fixed Buttons, incorporate a long-life LED, indicating the feature or circuit status. Each Electronic Keyset includes a RJ-11 type jack for connection to the system. See Figure 2.4.4.

Speaker Phone

Each Electronic Keyset is equipped with circuitry that enables the telephone to be used hands-free in two way conversations. This circuitry provides voice-switched speaker phone operation which gives channel control to the party with the highest energy level. The speaker phone circuit is controlled by the user with the MON button.

Volume Control

The Electronic Keysets have a volume control slide switch RING VOL, which controls the volume of all tone signals sent to the speaker of the Keyset including muted ring. In addition, each 21 and 33 Button Electronic Keyset includes a second volume control slid switch SPKR VOL to control the level of all voice signals sent to the speaker of the Keyset. Moving the control to the right increase volume, to the left decrease volume.

H-T-P switch

The Electronic Keysets are equipped with a three position slide switch which determines the signaling mode for incoming intercom calls.

| Position | Function |
| --- | --- |
| “HF” position | Intercom call announce w/hands free reply |
| “TN” position | Intercom tone ring |
| “PV” position | Intercom call announce w/Privacy |

LCD Display

The 21 33 Button Executive Keysets incorporate a 2-line, 48-character Liquid Crystal Display (LCD). The LCD provides alpha-numeric display of various information to assist the user in operation of features. In the idle mode, the display will show the station name or number on the top line and the time and date on the second line.

Electronic DSS

The Electronic Direct Station Selection/Direct Line Selection(DSS/DLS) Consoles can be connected to any port of an ETIB in place of a Keyset. Up to 6 DSS/DLS Consoles can be associated with a Keyset in the system database. Each console incorporates a 4 by 12 matrix of “Flexible Buttons”, a total of 48 Buttons, each with an associated LED for status indications. The DSS/DLS Console includes a RJ-11 type jack for connection to the system.

Electronic Phone Box

The Intercom/Door Box, KD-ICM Box, provides hands-free intercom conversation from any location that generally does not require a fully functional Key Telephone. The ICM/Door Box is also used to monitor areas such as a door or entry-way. The ICM/Door Box is connected to any port of an ETIB in place of a Keyset. The ICM/Door Box incorporates a volume control slide switch and has 2 feature buttons with LEDs; one for calling preprogrammed stations CALL and The second to block incoming calls DND.

### MOHU (Music On Hold Unit)

The MOHU provides External Music Source to the system through any kind of Single Line Interface including 2B-SLIU30 in GDK-100 System. The external music source provided through single line interface is used as music source of the system, that is, the music source through MOHU offers the same function that the other external music source of the system do. A MOHU has two interface circuits to connect 2 music sources with 2 single line interface. The system allows the maximum 5 interface circuits (restricted by the program) through the maximum 3 MOHUs to be connected. The single line interfaces connected to MOHU should be selected by Admin programming.( Field #1 and 2nd button)

The characteristics of MOHU interface are followings,

Impedance: 600 ohm

Transmission loss: 1 dB +/- 1dB at 1020Hz

Maximum music source input: Max. 0 dBm @ 600 ohms

The MOHU can be connected to any kind of single line interface including 2B-SLIU30.

This module may be available to support special local market conditions.

### 2B-module

The 2B-module allows another terminal, DKTU (Digital Key Telephone Unit) or SLT (Single line Telephone) behind LKD-30DS to be connected. There are 2 kinds of 2B-modules; 2B-DTIU30 for the connection to another DKTU behind LKD-30DS, and 2B-SLIU30 for the connection to another SLT behind LKD-30DS. These 2B-modules can be only installed in LKD-30DS. See Figure 2.4.5 2B modules.

In GDK-100 system, LDK-30DS with 2B-module can be connected to every even-numbered port (0,2,4,…) of DTIB12/DTIB24/DSIB. In that case, any digital terminal can not be connected to the consecutive port (1,3,5,…). So the total capacity of DTIB12/DTIB24/DSIB is not changed by the connection of 2B-module.

The advantage of 2B module in GDK-100 system;

- Simple wiring with two terminals.

- DTIB is capable of connecting to single line device through 2B-SLIU30. It is useful for the customer who wants more single line devices even though DTIB is installed in the system.

- DTIB is capable of connecting to single line device such as Fax/Modem without any SLIB board.

2B modules are supported in the following condition

- Software: 3.2 or later version (except 3.4 version)

- Hardware: Digital Terminal Interface only, LKD-30DS, 2B-DTIU30 or 2B-SLIU30

- Line length. See Table 2.3.4 Electrical Specifications

- Note:

- In case of using 2B-DTIU30 and the secondary digital keyset, it is recommended to use the separate mains adapter, which confirms to the appropriate national or local electrical adapter for the secondary DKTU.

- The separate mains adapter can be get in local market, the recommended specification is;

- Output capacity: 24 to 30Vdc, more than 200 mA

- Size of output plug: the inside diameter - 2.1mm, the outside diameter: 5.5mm

The following table shows LKD series which the 2B module can be installed.

| DKTU | 2B-DTIU30 | 2B-DTIU30 |
| --- | --- | --- |
| LKD 30DS | Yes | Yes |
| LKD 30LDS | X | X |
| LKD 8DS | X | X |
| LKD 2NS | X | X |

For more details, see “ Installation manual for 2B-DTIU30 and 2B-SLIU30”.

### GDK-PC PHONE (CTI)

GDK-PC PHONE is based on Microsoft TAPI (Telephony Application Programming Interface) Standard Specification and Windows 3.1 or Windows 95 environment. With Computer Telephony, a customer can use computer to take an advantage of a wide range of sophisticated communication features and services over a telephone line. GDK-PC PHONE includes the various value added features to integrate Computer and Telephone, to manage all the details associated with telephone business.

There are 2 kinds of hardware configurations, one is an external CTI module with KD/C or KD/EC series DKTU, the other is an internal CTI module with LKD series DKTU. See Figure 2.4.6 CTI module.

External CTI module package

An external CTI module contains three connectors, 2 RJ-11 type jacks and a RS-232C. One of RJ-11 type jack is used for the connection to Digital Terminal Interface port of the system through 2-wire telephone line, and the other is for the connection to KD/C or KD/EC series through 6-wire telephone line. And RS-232C, comprised of 9-pin connector, is for the connection to a computer. All the line cords and RS-232 cable are provided in the same package.

External CTI module allows only KD/C or KD/EC series to be connected, but not another type of DKTU.

Internal CTI module package

An internal CTI module can be installed on the bottom side of LKD series DKTU. And the cable with RS-232C 9-pin connector and RJ11 type plug is provided for the connection to a computer in the package.

There are two kinds of internal CTI modules, CTIU30 and CTIU8. CTIU30 can be installed in LKD-30DS, and CTIU8 in LKD-8DS and LKD-2NS, but not possible in LKD-30LDS with Large Display.

| DKTU | CTIU30 | CTIU8 |
| --- | --- | --- |
| LKD 30DS | Yes | X |
| LKD 30LDS | X | X |
| LKD 8DS | X | Yes |
| LKD 2NS | X | Yes |

For more details, see “ Installation And User’s Guide for GDK-PC PHONE”.

### Cables (5MC1, 5MC2)

There are two kinds of line connection cables, one (5MC1: 6851NF0023A) is for the connection to analog CO lines or PFTU, and the other (5MC2: 6851NF0028A) is for the connection to the terminals or miscellaneous devices. 5MC1 consists of the 25-pair cable with 5 meter line length and a RJ21 type female connector, and 5MC2 consists of the 25-pair cable with 5 meter line length and a RJ21 type male connector. One end of the cable is connected to a RJ21 type connector, and the other is open.

So the RJ21 type connector of the cable can be connected to another RJ21 type connector on the function boards according to the type of connector described above. And open wires of the cable can be connected to a user connection point, e.g. the MDF which a supplier provides.

MDF, which is in wide use in the countries, can be locally used according to the requirements of local regulation.

If necessary, above cables can be supplied from a supplier on the order.

### BRI Line Terminating Board (GDK-162/100 TERM: 2923NP1656A)

GDK-162/100 TERM unit provides four pairs of 100 ohm terminating resistors. The unit is capable of accommodating 4 BRA lines with RJ45 type jacks. It has metal case.

This can be connected at outside of the system. If a customer use Basic Rate (S or T) interface but the 100 ohm terminating resistors are not introduced on the interface circuit, this board will be useful.

See the parts of BRIB Installation, STIB Installation, and Terminating Resistors on Basic Rate Interfaces in section 3.

### Trace Tool (GDK-TRC1)

GDK-TRC1, Figure 2.4.7, provides a RS-232 Serial Interface for the maintenance of special cards of GDK-100 system. It provides 4-wire interface to connect to GDK-100 boards and RS-232 interface to connect to the serial port of PC. All two cables are offered in the GDK-TRC1 package.

GDK-TRC1 can be connected to the following board for access..

| System Name | Board Name |
| --- | --- |
| GDK-100 | PRIB issue 2B or later, all BRIB, STIB, WTIB, DVIB, E1IB version |

Caution: This tool should be only used by LG Information & Communications, Ltd. and its agent for the purpose of the maintenance of GDK-100 system, but not by an end user.