##### SECTION 3. INSTALLATION

### INTRODUCTION

As with any sophisticated communications device, installation of the GDK-100 System requires the care and fore-thought of a competent technician. Installation proceeds in 5 major steps;

- Site Preparation

- KSU and PSU Installation

- PCB Installation

- System Wiring

- System Programming and Verification

By utilizing the instructions that follow, the installation is quick and efficient. Directions for System Programming are briefly given in SECTION 5. CUSTOMER DATABASE PROGRAMMING

### SITE PREPARATION

### General Site Consideration

The first step is to locate an acceptable site for the common equipment.(KSUs, boards, etc.) When locating a mounting site for the KSUs, the following points must be considered;

a) The KSUs are designed for wall-mounting and should not be mounted directly to a masonry or plaster-board wall. It is recommended that a  inch plywood back-board be firmly mounted to the wall, and the KSU and the MDF, if other than the MDF, be mounted to the back-board.

b) The location must have access to a dedicated 110 Volt AC or 220 Volt AC, 50-60 Hz circuit with a circuit breaker or fuse rated at 15 amps. A 3-wire parallel blade grounded outlet should be within approximately 2 meters(6 feet) of the lower left rear of the BKSU mounting.

c) The location must have access to a good earth ground, such as a metallic cold water pipe with no non-metallic joints. The ground source should be located as close as possible to the system.

d) The system should be located in an area which is well ventilated with a recommended temperature of 20-26 degrees C (68-78.8 degrees F) and a relative humidity of 5-90 % (non-condensing).

e) The system should be located within 8 meters (25 feet) of the telephone company's termination point. Also, the location should be within the prescribed station loop lengths for all keysets and terminals. If existing cabling is to be employed, the location of existing cabling and conduits should be considered.

f) The location should have adequate accessibility, space and lighting for future servicing and should consider the need for future expansion.

g) The site should be away from radio transmitting equipment, arc-welding devices, copy machines, and other electrical equipment capable of generating electrical interference. The system should be protected from flooding and heavy machinery as well as excessive dust and vibration.

### Back-Board Installation

A wooden back-board is recommended for all installations and must be installed when the location has masonry or plaster-board walls. A 0.5 inch plywood material is sufficient for most installations. The back-board should be mounted at a convenient height, about 1 meter above the floor, and be bolted in a number of places to distribute the weight of the system.

Space should be available on the bottom side of the back-board for the MDF and/or other MDF and for optional equipment such as a music source and PFTU, etc.

### Verify On-Site Equipment

Once the equipment installation site has been identified and a dedicated AC outlet, earth ground, and lighting and ventilation are available, verify that all equipment required is on-site and has not been damaged during shipment. Unpack the KSUs assure there is no shipping damage. Note that a mounting template is packed with KSU and this template will be required later in the installation. Check that the type and quantity of boards received is correct and optional equipment and a Power Line Surge Protector are on-site. Note that the individual boards should not be unpacked at this time.

If any equipment is damaged or missing, notify appropriate personnel to correct the situation.

### KSU INSTALLATION

GDK-100 system consists of Basic KSU, Small Basic KSU and Expansion KSU. The basic exterior of GDK-100 is shown in Figure 3.3.a ~ 3.3.b.

Caution:

Installing both Basic and Expansion KSU together on the brick wall can cause the connection between Basic and Expansion KSU to be unstable. So recommend that Basic and Expansion KSU should be installed on the flat wall.

### Mounting BKSU

The Basic KSU and Small Basic KSU are metal frame cabinet designed for wall mounting. Employing the KSU mounting template provided with the BKSU or SBKSU, mark the location of the two screws to mount the BKSU or SBKSU. Again, the KSU must not be mounted on a masonry or dry-wall surface, in this case a wooden Back-Board is required, reference paragraph Back-Board Installation. The distance between mounting holes and the position of mounting holes is shown in Figure 3.3.c ~ 3.3.d.

The BKSU or SBKSU is mounted with four #10 or larger, 1.5 inch or longer screws.

First, drill pilot holes in the two locations marked, insert the screws and tighten leaving about 0.5 inch exposed. Mount the BKSU or SBKSU on the screws and tighten the screws securely.

Second, remove the front cover by turning the two cover screws 0.5 turn counterclockwise, tilt and lift the cover to remove. And, insert the screws to the mounting holes of BKSU or SBKSU and tighten the screws as Figure 3.3c ~ 3.3d.

- Note: GDK-100 SBKSU is not supplied any more.

### Mounting EKSU

The Expansion KSU is a metal housing designed for wall mount installation. If required, an EKSU is mounted on the right of the BKSU or SBKSU.

Before mounting the EKSU, remove the “KSU Interconnection Cover” on the right of the BKSU or SBKSU. The EKSU is mounted in a manner similar to the BKSU or SBKSU, refer to Figure 3.3.c ~ 3.3.d. Select the positions of two holes to fix EKSU on the wall so that the proper connection between BKSU and EKSU will be possible. Mount the EKSU on the screws and tighten the screws securely. And fasten the 2 side brackets [fastener] with 8 screws between the Basic or Small Basic KSU and Expansion KSU, refer to Figure 3.3.e ~ 3.3.f.

Insert Figure 3.1a in the existing Manual here

Figure 3.3.a	Basic KSU Exterior

Insert Figure 3.1b in the existing Manual here

Figure 3.3.b	Small Basic KSU Exterior

Insert Figure 3.1c in the existing Manual here

Figure 3.3.c	BKSU & EKSU Mounting Holes & Installation Layout

Insert Figure 3.1d in the existing Manual here

Figure 3.3.d	SBKSU & EKSU Mounting Holes & Installation Layout

Insert Figure 3.1e in the existing Manual here

Figure 3.3.e	Connection between BKSU and BKSU

Insert Figure 3.1f in the existing Manual here

Figure 3.3.f	Connection between SBKSU and EKSU

### RGU Installation

The Ring Generator Unit is needed in the BKSU or SBKSU if SLIB24, SLIB48,SLIBII, and/or DSIB are to be installed, to provide Ring voltage and Message Wait Source. According to the installation site, two types of RGUs are available; External, Internal.

The external RGU is fixed with the two screws provided, on the wall outside the KSU and is connected to CN12 (marking on PCB) connector on the system back-plane of Basic KSU or CN9 of Small Basic KSU. The internal RGU is fixed on the bottom side panel with the two screws provided, inside the BKSU or SBKSU and is connected to CN12 (marking on PCB) connector on the system back-plane of Basic KSU or CN9 of Small Basic KSU.

See figure 3.3.3.

External

RGU

Backplane

PSU

Slot

Internal

RGU

BKSU or SBKSU

EKSU

CN12 in BKSU

CN9 in SBKSU

Figure 3.3.3 The installation of GDK-100 RGU

A corresponding cable for connecting between external /internal RGU and BKSU/SBKSU is provided with RGU in the same package.

See Figure 3.3.5 for connection earth cable.

### PFTU Installation

The PFTU provides the relay contacts for the direct connection of 6 Exchange Lines (CO Lines) to 6 Extension Lines (SLT Lines) in event of a power or processor failure. The PFTU can be optionally installed outside the BKSU on the wall with two screws provided.

2 control wires are connected to the CN13 (marking on PCB) on back-plane of the system as figure 3.3.4, regardless of the polarity of control wires. The connector for CN13 is provided but control wires should be got locally. The specification of control wires is AWG #22, #24 or #26 regardless of their length.

A RJ21 type connector, which consists of 25 pairs, is connected CO line, LCOB, SLIB and SLT as section 3.5 PFTU Wiring.

An earth terminal is provided, and the grounding method of the PFTU is shown in figure 3.3.5 when the PFTU is installed outside BKSU.

And there is a test switch on the surface of PFTU; TEST position of the switch is used for testing PFTU function, and NORMAL position of the switch for normal operation.

In case of SBKSU, a PFTU can be installed on the frame of BKSU in the system, its position is beside MPB.

See Figure 3.3.5 for connection earth cable.

Earthing point

RJ21 type connector

Figure 3.3.4	The exterior of PFTU

To CN13

on backplane.

Normal

Test

### KSU Grounding

Before connection to the mains supply, the GDK system must be earthed in conformance with the appropriate IEE requirements. Earthing is required for user safety and also to minimize EMC interference.

To ensure proper system operation and for safety purposes, a good earth ground is required. A metallic COLD water pipe usually provides a reliable ground. Carefully check that the pipe does not contain insulated joints that could isolate the ground path.  If insulated joints exist, another earth ground source must be used or, if allowed, the joints may be bridged.

A separate conductor should be used between the ground source and the KSU, which confirms to the appropriate national or local electrical codes. AGW #14 or larger copper wire is recommended. The wire should be kept as short as possible, it is recommended that the wire be no longer than 8 meters (about 25 feet).

Earth Point

BKSU or SBKSU

EKSU

Fastener

to fix EKSU to BKSU

PFTU

External

RGU

Backplane

pPSU

AWG#14 or larger

AWG#18 or larger

PSU

earthing Point

of the system

Connecting Wire: 1. UL 1015 AWG#18 or larger than this.

2. Length ≤ 1 meter  * The shorter the length of wire is, the better it is.

3. Color : Green / Yellow

Note : In case of SBKSU(Small Basic KSU), PFTU can be installed SBKSU inside.

Figure 3.3.5 The Connection Method of Earth Cable between BKSU(SBKSU), EKSU, External RGU and PFTU.

### PSU Installation

The Power Supply Unit (Figure 3.3.6.a) provides power for the system boards and telephones, converting the AC Voltage input to appropriate DC voltages.

Before installation, assure that the AC plug is not plugged into the AC outlet. Place the PSU in the left most slot in the BKSU or SBKSU, aligning the card guides with the PSU PCB and PSU frame flanges. Slide the PSU into place using the injectors to seat the PSU fully in the KSU. And then fix the PSU to the KSU with two screws as figure 3.3.6.b.

The PSU can operate from either 110/115 or 220/230 Volts AC based on the setting of the VOLTAGE SELECTOR Switch on the lower front of the PSU. Generally the PSU is shipped with this switch set for 220/230VAC.

If local AC is 110 volts, move the switch to the upper to display “110/115V”. If local AC is 230 volts, move the switch to the lower to display “220/230V”.

Default setup of the products: 220/230VAC.

The range of mains input voltage and the fuse ratings of PSU3 according to the position of VOLTAGE SELECTOR Switch are as follows;

| Position of VOLTAGE SELECTOR | The Range of Mains Input Voltage | Fuse Ratings |
| --- | --- | --- |
| AC 115V | 93VAC ~ 132VAC | 6A /125V |
| AC 230V | 187VAC ~ 264 VAC | 4A /250V |

GDK-100 system has two kinds of PSU.  Each capacity is as follows;

Power Capacity of GDK-100 PSU1 and PSU3                Unit: mA

|  | +5 Vdc | -5 Vdc | +30 Vdc | -48 Vdc |
| --- | --- | --- | --- | --- |
| GDK-100 PSU1 (Linear) 1) | 1500 | 500 | 2500 | None |
| GDK-100 PSU3 (SMPS) | 5000 | 1000 | 8500 | None |

Unit: mA

* Note: 1) The production of PSU1 has been officially stopped since March 1999.

Insert Figure 3.2a in the existing Manual here

Figure 3.3.6.a	Power Supply Unit

Insert Figure 3.2b in the existing Manual here

Figure 3.3.6.b	Installation of PSU

### MDF Installation

When the external MDF is in wide use in local countries according to the local market conditions or regulatory requirements, the connection method of lines between KSU and MDF is as following figure 3.3.7

Backplane

BKSU or SBKSU

EKSU

PSU

External MDF

- Note

- The external MDF being in wide use in local country can be used according to the requirements of local regulation.

- In this case, the cables with RJ21 type connector(25-pair Amphenol) will be required. Those can be locally got, otherwise supplied by LGIC, see 5MC1 and 5MC2.

- See Figure 3.3.e and 3.3.f for the shape of cable arrangement.

Figure 3.3.7	MDF installation with Local MDF

### PCB INSTALLATION

### PCB Handling & General Installation

All boards should not be installed or removed with power applied.

Power must be turned off prior to insertion or removal of the PCB.

The system PCBs contain digital circuitry which, while extremely reliable, can be damaged by exposure to excessive static electricity. When handling PCBs, a grounded wrist strap should be used to protect the boards from static discharges. Also, use common sense when handling the PCBs. For example, do not place a PCB in locations where heavy objects might fall on the PCB and damage components.

To insert a PCB, hold the PCB by the injectors and, with components facing right, align the top and bottom edge of the PCB in the card guides. Slide the card into the system and use the injectors to seat the PCB firmly into the back plane connectors. To remove a PCB, reverse the procedure. Installation method of PCB is shown in Figure 3.4.1.

Insert Figure 3.3 in the existing Manual here

Figure 3.4.1	 PCB Installation

### MPB Installation

The MPB is installed in the right most PCB card slot (slot MPB) of the BKSU or SBKSU. The MPB may be equipped with 4 add-on boards; a MEMU for memory expansion, a SMEMU for memory expansion, QSIG, TAPI and/or PC-ATTENDANT, a MODU for modem access to the system, and a PLLU for system synchronization. See Figure 3.4.2.

Lithium Battery

The MPB contains a lithium dry-cell to maintain memory and real-time clock functions. The battery is soldered on the MPB, and connected circuitry by on-off DIP switch. So the switch is set to ON position before the MPB installation.

“ CAUTION : Danger of explosion if battery is incorrectly replaced. Replace only with the same or

equivalent type recommended by the manufacturer. Dispose of used batteries according to the manufacturers instruction.”

Function of SW2

The MPB also has an eight-position DIP switch, SW2. The function of each switch position is as followings:

Default setting: All ON position

| Switch | Function | OFF | ON |
| --- | --- | --- | --- |
| 1 | Administration Programming Access | Disable | Enable |
| 2 | CTS (Send to Clear) of RS232 | Enable | Disable |
| 3 | Command/Event Trace (The purpose of testing software) | Enable | Disable |
| 4 | Not used | - | - |
| 5 | SMDI (Simplified Message Desk Control –Voice Mail ) | SMDI ON | SMDI OFF |
| 6 | Not used | - | - |
| 7 | Power up status trace of board | Enable | Disable |
| 8 | Database default on power up | Disable | Enable |

Table 3.4.2 	The function of each pole of SW2

Before programming the system, switch 8 should be placed in the on position and power cycled off and on to initialize the system database to default. Once the database has been initialized, switch 8 should be placed in the off position so as to protect the database.

After the lithium batteries(SW3 ON) and Add-on boards installed, insert the MPB in slot MPB of BKSU or SBKSU.

Function of various switches and connectors

| Switch/Connector | Functions | Remark |
| --- | --- | --- |
| SW1 | CPU Reset |  |
| SW3 | Lithium Battery ON/OFF switch |  |
| CONN5 and CONN6 | PLLU connection | ISDN 1) |
| CONN7 and CONN8 | MEMU or SMEMU connection |  |
| CONN9 and CONN10 | MODU Connection |  |
| RS1 | RS232C Serial Interface | System Access |

- Note

- When ISDN board is installed, PLLU should be installed. Otherwise there is no necessity for installing PLLU. If PLLU is not installed, the shorting trap (short pin) should be set to INT position of CONN5 contacts 12 and 13.

The shorting trap (short pin) is installed on INT position when the board delivered.

Insert Figure 3.4 in the existing Manual here

Figure 3.4.2	MODU, MEMU & PLLU Installation

Note:

- MPB should be installed in MPB slot, the right most slot in BKSU or SBKSU.

- The DIP switch, SW3 should be turned ON position before the MPB installation to protect system data in the case of a power fail.

- The 8th pole (switch 8) of SW2 should be OFF to protect the variety features by Admin programming after the system power up and initialization. See Table 3.4.2

When the system power is on, the default slots will be automatically redefined from the most left slot if the 8th pole of SW2 is set to ON position. So It is recommended that a board among DTIB12, DTIB24 and DSIB is installed in the most left slot and a Digital Keyset with display is connected to the first port the board when the first initialization of the system, since the FIRST STATION of the system can only access the system database without PC Administration.

- The pin #12 and #13 of the connector CONN5 in the MPB should be shorted with a shorting trap (short pin) for proper operation when the PLLU is NOT installed.  Refer to the clause 3.4.2.3 PLLU Installation.

- Add-on boards: MEMU, SMEMU, PLLU, MODU.

### MEMU Installation

The MPB may be equipped with MEMU for memory expansion. Before installing the MPB, install MEMU as required, refer to Figure 3.4.2. The MEMU has a lithium dry-cell and SW1 must be set to ON position to protect the database in the memory, before installing it into the MPB.

- Note:

- If MEMU is newly installed during operation, then the SMDR recorded in the basic memory of MPB before MEMU installation will be lost.

- If MEMU is replaced with SMEMU during operation, the database will be lost. So it is recommended that the database is downloaded to PC before replacement, and unloaded after replacement.

The MEMU should be installed on MPB through the dual line connectors, CONN7 and CONN8 of MPB, so recommend visual inspection of the connecting state between MPB and MEMU after installing it on MPB.

### SMEMU Installation

The MPB may be equipped with SMEMU for memory expansion and the QSIG, CTI, PC-ATTN features. Before installing the MPB, install SMEMU if required, refer to Figure 3.4.2. The SMEMU has a lithium dry-cell and SW1 must be set to ON position to protect the database in the memory, before installing it into the MPB.

The SMEMU should be installed on the position of MEMU on MPB.

- Note:

- If SMEMU is newly installed during operation, then the SMDR recorded in the basic memory of MPB before SMEMU installation will be lost.

- If MEMU is replaced with SMEMU during operation, some database will be lost. So it is recommended that the database is downloaded to PC before replacement and unloaded after replacement.

- If you want to use the QSIG, CTI, PC-ATTN features, the software package 99A version should be required. .(See the SMEMU in section 2)

The SMEMU should be installed on MPB through dual line connectors, so recommend visual inspection of the connecting state between MPB and SMEMU after installing it on MPB.

### PLLU Installation

The PLLU board should be installed on the MPB to use the ISDN/Digital Trunk interface boards such as BRIB, PRIB, STIB and/or E1IB. If PLLU is not installed, the pin #12 and #13 of the connector CONN5 on the MPB should be short with a short pin for proper operation.

There are 3 kinds of PLL units, PLLU, PLLU III and PLLU2 (issue 3) for GDK-100 system.

PLLU2 (Issue 3) has been only supplied since the end of November, 1998.

### MODU Installation

The MODU is used for remote access the system for system maintenance and PC based administration function. MODU can be installed on MPB through CONN9 and CONN10 of MPB.

### MISB Installation

The MISB contains 2 MOH inputs, 2 external paging ports and four dry contacts, and an alarm detection.

Function of various connectors

| Connectors/Switches | Function | Remark |
| --- | --- | --- |
| CONN1 | DRU Connection |  |
| CHAMP1 | RJ21 type female connector (25-pair Amphenol) for External Paging, External MOH, Relay contacts, and Alarm Input | See the clause 3.5 |

One MISB can be installed only in the slot No. 8.

SIU can be installed on MISB.

Insert Figure 3.5 in the existing Manual here

Figure 3.4.3	SIU Installation

### SIU Installation

The SIU board is used for system maintenance, PC based Admin, SMDR print and SMDI. The system supports up to 3 Serial Interface circuits, 1 circuit on MPB and additional 2 circuits on SIU board. The SIU should be installed if more than 1 serial interface devices are required.

Followings are the list of the system’s output;

- Administration database

- Off-line SMDR (on-demand)

- On-line SMDR

- Statistical Information

- SMDI (voice mail)

- System trace data

Note:

- A SIU board can be installed only on MISB.

- SIU should be fixed on MISB with two plastic supporters and two screws provided.

### Extension Board Installation

The system can be equipped with any combination of the extension boards; DTIB12, DTIB24, ETIB, SLIB24, SLIB48, SLIBII, DSIB, and WTIB. The station interface boards can be installed in any card slots in the Basic and Expansion KSU except in MPB slot of the Basic KSU. But WTIB can not be installed in slot 8 of BKSU and in any slot of EKSU.

It is recommended to install extension boards, e.g. DTIB from the left most slot (slot No.1). Also, since the first port of SLOT # 1 will be the database access port, it is recommended that a DTIB be installed in this slot and a Digital Keyset with display be connected to this port.

### DTIB12/DTIB24 Installation

DTIB12/DTIB24/DSIB can be installed in any card slot in the Basic and Expansion KSU except in MPB slot of the Basic KSU.

Since the first port of SLOT 1 will be the database access port, it is recommended that a DTIB12, DTIB24 or DSIB be installed in this slot and a digital keyset with display be connected to this port.

Function of LEDs

| LED | Function | Remark |
| --- | --- | --- |
| LED1 | The status of ports. (ON: in use, OFF: idle) |  |

Function of connector

| Connector | Function | Remark |
| --- | --- | --- |
| CHAMP1 | RJ21 type female connector for terminal connection. | See clause 3.4.5 |

- Note:

- The distance between Digital Terminal Interface Board and Digital Terminal: see Table 2.3.4

In GDK-100 system, LDK-30DS with 2B-module and secondary terminal can be connected to only every even-numbered port (0,2,4,…) of DTIB12/DTIB24/DSIB, then any digital terminal can not be connected to the consecutive odd port(1,3,5,…) in that case. See clause 2.4.13.4 2B-module and clause 2.4.13.5 GDK-PC PHONE in section 2 for more details regarding 2B-module and CTI module.

DTIB12/DTIB24 can be installed in any of slot No. 1-12.

The maximum extension boards except Wireless Terminals are limited by 72 extensions in a system.

So It is recommended that a board among DTIB12, DTIB24 and DSIB is installed in the most left slot and a Digital Keyset with display is connected to the first port the board when the first initialization of the system, since the FIRST STATION of the system at first can only access the system database without PC Administration. See clause 3.4.2 MPB Installation.

### ETIB Installation

The ETIB can be installed in any card slot in the Basic and Expansion KSU except in MPB slot of the Basic KSU.

Function of LED

| LED | Function | Remark |
| --- | --- | --- |
| LED1 | The status of ports. (ON: in use, OFF: idle) |  |

Function connector

| Connector | Function | Remark |
| --- | --- | --- |
| CHAMP1 | RJ21 type connector (female) for terminal connection. | See clause 3.4.5 |

- Note:

- The distance between ETIB and electronic terminal: see Table 2.3.4

ETIB can be installed in any of slot No. 1-12.

The maximum extension boards except Wireless Terminals are limited by 72 extensions in a system.

### SLIB24/SLIB48 Installation

SLIB24/SLIB48 can be installed in any universal slot and allows 6 single line analogue devices to be connected. A MSGU/MSGU48 with 6 circuits can be optionally installed on SLIB24/SLIB48 respectively to light a message waiting lamp in the SLT connected to SLIB24/SLIB48, and a DTRU with 2 receivers can be optionally installed on SLIB24/SLIB48 to receive DTMF signals.

Function of LED

| LED | Function | Remark |
| --- | --- | --- |
| LED1 | The status of ports. (ON: in use, OFF: idle) |  |

Function of various connectors

| Connector | Function | Remark |
| --- | --- | --- |
| CONN1 and CONN2 | DTRU connection |  |
| CONN3 and CONN4 | MSGU and MSGU48 connection | 1) |
| CHAMP1 | RJ21 type connector (female) for terminal connection | See clause 3.4.5 |

- Note:

- If you want Message Waiting function on SLIB24 or SLIB48, MSGU or MSGU48 should be installed on SLIB24 or SLIB48 respectively. MSGU can be installed on SLIB24, and MSGU48 can be installed on SLIB48.

- The distance between SLIB and SLT: see Table 2.3.4

SLIB24/SLIB48 can be installed in any of slot No. 1-12.

The maximum extension boards except Wireless Terminals are limited by 72 extensions in a system.

Add-on boards: DTRU, MSGU(on SLIB24), MSGU48(on SLIB48)

CHAMP1

DTRU

MSGU

or

MSGU48

Figure 3.4.4.3	MSGU/MSGU48 and DTRU Installation

### SLIBII Installation

SLIBII can be installed in any universal slot and allows 12 single line analogue devices to be connected. A MSGU with 12 circuits can be optionally installed on SLIB II to light a message waiting lamp in the SLT, and a DTRU II with 4 receivers can be optionally installed on SLIB II to receive DTMF signals.

Function of LED

| LED | Function | Remark |
| --- | --- | --- |
| LD1 | The status of ports. (ON: in use, OFF: idle) |  |

Function of various connectors

| Connector | Function | Remark |
| --- | --- | --- |
| CN2 and CN4 | DTRUII connection |  |
| CN3 and CN5 | MSGUII connection | 1) |
| CN1 | RJ21 type connector (female) for terminal connection | See clause 3.4.5 |

- Note:

- If you want Message Waiting function on SLIBII, MSGUII should be installed on SLIBII.

- The distance between SLIBII and SLT: see Table 2.3.4

SLIBII can be installed in any of slot No. 1-12.

The maximum extension boards except Wireless Terminals are limited by 72 extensions in a system.

Add-on boards: DTRU II, MSGU II

CN1

MSGU II

DTRU II

Figure 3.4.4.4	MSGU II and DTRU II Installation

### DSIB Installation

DSIB can be installed in any universal slot and allows 6 Digital terminals and 6 single line analogue devices to be connected. A MSGU with 6 circuits can be optionally installed on DSIB to light a message waiting lamp in the SLT connected to DSIB, and a DTRU with 2 receivers can be optionally installed on DSIB to receive DTMF signals using a system resource.

Function of LED

| LED | Function | Remark |
| --- | --- | --- |
| LED1 | The status of ports. (ON: in use, OFF: idle) |  |

Function of various connectors

| Connector | Function | Remark |
| --- | --- | --- |
| CN1 and CN2 | DTRU connection |  |
| CN3 and CN4 | MSGU connection | 1) |
| CN5 | RJ21 type connector (female) for terminal connection | See clause 3.4.5 |

- Note:

- If you want Message Waiting function with SLT, MSGU should be installed on DSIB.

- The distance between DSIB and Digital terminal/SLT: see Table 2.3.4

DSIB can be installed in any of slot No. 1-12.

The maximum extension boards except Wireless Terminals are limited by 72 extensions in a system.

Add-on boards: DTRU, MSGU

CN5

MSGU

DTRU

Figure 3.4.4.5	MSGU and DTRU Installation

### DTRU/DTRUII Installation

A DTRU contains 2 DTMF receiver circuits and a DTRU II has 4 DTMF receiver circuits. Each receiver of DTRU /DTRU II is time-shared under the control of the system software, as a system resource regardless its position.

| BOARD | The position of Installation | The quantity of receivers | Marking on PCB |
| --- | --- | --- | --- |
| DTRU | SLIB24, SLIB48, DSIB, LCOB, AC15, BWDIDB, EMIB, DIDB | 2 DTMF receivers | DTRU of GDK-34/36 |
| DTRU II | SLIB II | 4 DTMF receivers | GDK-100 DTRU II |

The System is capable of supporting DTMF receivers up to 22.

At least a DTRU or DTRU II should be installed to receive DTMF signals.

Take a cautious attitude in installing DTRU on the boards because DTRU contains the connectors without the connection guide, otherwise DTRU may be damaged by wrong connection.

### MSGU/MSGU48/MSGUII Installation

A message waiting unit can be optionally installed on a SLT interface board to light a message waiting lamp in the SLT connected. There are 3 kinds of message waiting unit as followings according to the line voltage.

| BOARD | The position of Installation | The quantity of circuits | The line voltage |
| --- | --- | --- | --- |
| MSGU | SLIB24, DSIB | 6 circuits | +24Vdc |
| MSGU48 | SLIB48 | 6 circuits | -48Vdc |
| DTRU II | SLIB II | 12 circuits | -28Vdc |

- Signal voltage: 90~100Vdc

### Analog CO Line Board Installation

There are several types of analog CO line boards available. These boards include the LCOB and DIDB. Additional CO interface boards such as AC15, EMIB, DIDB and BWDIDB may be available for support of local market conditions and regulatory requirements.

Every CO line board contains several types of connectors for the line connection on the front edge of the boards themselves.

### LCOB Installation

LCOB provides 4 Loop Start CO line interfaces. The board can be optionally equipped with Call Metering Unit (CMU) boards to monitor call duration and a Call Progress Tone Unit (CPTU) daughter board to monitor call progress signals. A CMU must be provided for each Loop Start CO Line which requires call metering. In addition, the appropriate type of CMU must be employed. See CMU in section 2.

If required, install the CMUs and CPTU on the CO interface board as shown in Figure 3.4.5.1 and insert the boards in the selected card slot.

Function of LED

| LED | Function | Remark |
| --- | --- | --- |
| LED1 | The status of line 1, ON: in use, Green |  |
| LED2 | The status of line 2, ON: in use, Green |  |
| LED3 | The status of line 3, ON: in use, Green |  |
| LED4 | The status of line 4, ON: in use, Green |  |

Function of various connectors

| Connector | Function | Remark |
| --- | --- | --- |
| CN1 and CN2 | DTRU connection |  |
| CN3 and CN4 | CPTU/A or CPTU/B connection |  |
| CN100 and CN104 | The connectors for a CMU on line 1 | See Note |
| CN101 and CN105 | The connectors for a CMU on line 2 | See Note |
| CN102 and CN106 | The connectors for a CMU on line 3 | See Note |
| CN103 and CN107 | The connectors for a CMU on line 4 | See Note |
| CN108 | RJ21 type male connector for CO line connection. | Clause 3.5 |

- Note:

- The pin #6 and #7 of CN104, CN105, CN106 and/or CN107 should be shorted by shorting traps (short pin) when CMU12(12 KHz meter pulse) including CMU12PR or CMU16(16 KHz meter pulse) is not installed on the interface circuit(s). Otherwise the receiving voice path of the related line will be open.

- The 0 ohm resistors are usually mounted on R168-R171 of LCOB for the countries using only CMU PR(Polarity Reversal) and/or 50L(50 Hz longitudinal). If these CMUPR, CMU50L or CMU50PR are not installed on any CO interface circuit neither R168-R171 are mounted on LCOB, the pin #6 and #7 of CN104, CN105, CN106 and/or CN107 should be also shorted by shorting traps (short pin) on the related interface circuit.

- CN104/R168 are on the 1st interface circuit, CN105/R169 on the 2nd interface circuit, CN106/R170 on the 3rd interface circuit and CN107/R171 on 4th interface circuit.

LCOB can be installed in any one of slot No. 1-12.

Analog CO line boards are limited by maximum 32 analog lines in a system.

Add-on boards: DTRU, CPTU/A, CPTU/B, CMU

Insert Figure 3.6a in the existing Manual here

Figure 3.4.5.1	CMU & CPTU, DTRU Installation

### AC15 Board Installation - UK

The AC15 board provides 3 private interface circuits, each line supports AC15a signaling.

It is designed to comply with the UK specification and application, so it may not suitable for the other national specification and/or application.

Function of LED

| LED | Function | Remark |
| --- | --- | --- |
| LED1 | The status of line 1, ON: in use |  |
| LED2 | The status of line 2, ON: in use |  |
| LED3 | The status of line 3, ON: in use |  |

Function of connectors

| Connector | Function | Remark |
| --- | --- | --- |
| CN1 and CN2 | DTRU connection |  |
| CN3 | RJ21 type male connector for CO line connection. | Clause 3.5 |

The AC15 can be installed in any of slot No. 1-12.

Analog CO line boards are limited by maximum 32 analog lines in a system.

Add-on board: DTRU

CN3

Figure 3.4.5.2Board Layout and DTRU Installation

DTRU

LED1

LED2

LED3

### EMIB Installation - Korea

The EMIB provides four analog 2-wire E&M interfaces. It can support type I and V of 2-wire E&M.

It is designed to comply with the Korean specification and application, so it may not suitable for the other national specification and/or application.

Function of LED

| LED | Function | Remark |
| --- | --- | --- |
| LED1 | The status of line 1, ON: in use |  |
| LED2 | The status of line 2, ON: in use |  |
| LED3 | The status of line 3, ON: in use |  |
| LED4 | The status of line 4, ON: in use |  |

Function of various connectors

| Connector | Function | Remark |
| --- | --- | --- |
| CN1 and CN2 | DTRU connection |  |
| CN3 | The connector with screws for line 1 |  |
| CN4 | The connector with screws for line 2 |  |
| CN5 | The connector with screws for line 3 |  |
| CN6 | The connector with screws for line 4 |  |

Line mode selection

| No | Type |  | Remark |
| --- | --- | --- | --- |
|  | Short | Open |  |
| PO1 - PO4 | Type I | Type V | Note |

- Note:

EMIB mainly supports 2-wire E&M type V. It is recommended to set 2 wire E&M Type V.

It is recommended to remove shorting wires when installation.

EMIB can be installed in any of slot No. 1-12.

Analog CO line boards are limited by maximum 32 analog lines in a system.

When installing E&M line, It is important to check the type of E&M line and jumper position.

Add-on board: DTRU

Figure 3.4.5.3Board Layout and DTRU installation

DTRU

LED1

LED2

LED3

LED4

1M

1E

1R

1T

CN3

2M

2E

2R

2T

3M

3E

3R

3T

4M

4E

4R

4T

CN4

CN5

CN6

### DIDB Installation - Korea

The DIDB provides 4 analog DID interfaces. Each line supports DTMF and Pulse signaling but not R2 MFC.

It is designed to comply with the Korean specification and application, so it may not suitable for the other national specification and/or application.

Function of LED

| LED | Function | Remark |
| --- | --- | --- |
| LED1 | The status of line 1, ON: in use |  |
| LED2 | The status of line 2, ON: in use |  |
| LED3 | The status of line 3, ON: in use |  |
| LED4 | The status of line 4, ON: in use |  |

Function of connector

| Connector | Function | Remark |
| --- | --- | --- |
| CN1 and CN2 | DTRU connection |  |
| CN3 | The connector with screws for line 1 and 2 | Figure 3.4.5.4 |
| CN4 | The connector with screws for line 3 and 4 | Figure 3.4.5.4 |

The DIDB can be installed in any of slot No. 1-12.

Analog CO line boards are limited by maximum 32 analog lines in a system.

Add-on board: DTRU

Figure 3.4.5.4Board Layout and DTRU installation

DTRU

LED1

LED2

LED3

LED4

2R

2T

1R

1T

CN3

4R

4T

3R

3T

CN4

### BWDIDB Installation – New Zealand

The BWDIDB provides 4 analog both-way DID interfaces. Each line supports Outgoing/Incoming call.

It is designed to comply with New Zealand specification and application, so it may not suitable for the other national specification and/or application.

Function of LED

| LED | Function | Remark |
| --- | --- | --- |
| LD1 | The status of line 1, ON: in use |  |
| LD2 | The status of line 2, ON: in use |  |
| LD3 | The status of line 3, ON: in use |  |
| LD4 | The status of line 4, ON: in use |  |

Function of various connectors

| Connector/Switch | Function | Remark |
| --- | --- | --- |
| CN1 and CN2 | DTRU connection |  |
| CN3 and CN4 | CPTU/A or CPTU/B connection |  |
| CN3 | The connector with screws for line 1 and 2 | Figure 3.4.5.5 |
| CN4 | The connector with screws for line 3 and 4 | Figure 3.4.5.5 |

BWDIDB can be installed in any of slot No. 1-12.

Analog CO line boards are limited by maximum 32 analog lines in a system.

Add-on boards: DTRU, CPTU/A, CPTU/B

Figure 3.4.5.5Board Layout and DTRU installation

LD1

LD2

LD3

LD4

DTRU

CPTU

CN5

### ISDN Board Installation

There are 3 kinds of ISDN boards such as PRIB, BRIB, and STIB are available. And additional E1 Trunk Card, E1IB may be available to support local market condition and regulatory requirements.

Each ISDN board contains RJ45 type jack on boards themselves.

### ISDN BRIB (Basic Rate T interface) Installation

The BRIB card provides 4 interface circuits of BRI (Basic Rate Interface) format digital CO interface. For using this board, the PLLU or PLLU2 (issue 3 or later) board should be installed on the MPB.

If more than one ISDN board is required, some special cable should be used for clock signal control. CON3 and CON4 are used to control the clock signal when multi-ISDN boards are installed. Refer to [Clock Control Cable Wiring] of section 3.5.

Function of LED

| LED | Function | Status | Remark |
| --- | --- | --- | --- |
| LED1 | The status of Line 1 | Red: Error, Green: In-use, OFF: Idle |  |
| LED2 | The status of Line 2 | Red: Error, Green: In-use, OFF: Idle |  |
| LED3 | The status of Line 3 | Red: Error, Green: In-use, OFF: Idle |  |
| LED4 | The status of Line 4 | Red: Error, Green: In-use, OFF: Idle |  |

Function of SW1; default -all OFF when delivering the board

| Pole | Function | ON | OFF | Remark |
| --- | --- | --- | --- | --- |
| 1 | Not used | - | - |  |
| 2 | TEI | Fixed TEI | Automatic TEI | Note 1, 2 |
| 3 | Layer 2 trace | Enable | Disable | Note 3 |
| 4 | Layer 3 trace | Enable | Disable | Note 3 |

- Note:

- The function of the 2nd pole has been implemented in firmware version 3.1 or later.

- This is set to OFF position as default, therefore it is important to note that the 2nd pole is set to the proper position after checking the type of TEI in your country.

- These two poles are for the purpose of developing and maintaining BRIB.

Function of various connectors

| Connector | Function | Remark |
| --- | --- | --- |
| CON1 | Serial interface connector | Note1 |
| CON2 | The purpose of development. It is not fixed on the products. |  |
| CON3 | Clock control IN | Note 2 |
| CON4 | Clock control OUT | Note 2 |

- Note

- CON1: Trace for the purpose of development and maintenance.

The external module for RS 232C serial interface should be required for trace.

See GDK-TRC1 in section 2.

- CON3, CON4: They are used to control the reference clock signal when multi-ISDN boards are installed.

See [Clock Control Cable Wiring] in section 3.5.

The line connector and terminating resistors

| Line No | RJ45 type jack | Terminating Resistors |  |  |
| --- | --- | --- | --- | --- |
|  |  | Switch | All ON | All OFF |
| Line 1 | CON8 | SW5 | terminating | open |
| Line 2 | CON7 | SW4 | terminating | open |
| Line 3 | CON6 | SW3 | terminating | open |
| Line 4 | CON5 | SW2 | terminating | open |

- Note

- Two poles of SW2, SW3, SW4 and SW5 should be set to the ON position to terminate two terminating resistors on a line. (default ON)

- Terminating resistors and selectable switches are introduced only on PCB issue 3 or later, but not on PCB issue 1 and 2. So if you use BRIB issue 1 and 2, the external terminating resistors are required. For the external termination, you can get the terminating unit locally or use GDK-162/100 TERM, which is the terminating unit with 4 P-to-P connections provided by LGIC.

RJ45

CON5

CON3(IN)

SW1

1 2 3 4

GDK-100 BRIB

LED1

LED2

LED3

LED4

CON4(OUT)

CON1

U11

(ROM1)

U10

(ROM2)

RJ45

CON6

RJ45

CON7

CON2

RJ45

CON8

1 2 3 4

SW5

1 2

ON

OFF

SW4

1 2

ON

OFF

SW3

1 2

ON

OFF

SW2

1 2

ON

OFF

Figure 3.4.6.1	BRIB Layout

Check Points

- BRIB boards should be installed in any of the slot No. 1-7 except slot No.8 and EKSU.

- The PLLU or PLLU2(issue 3 or later) MUST be installed on MPB.

- The 2nd pole of SW1 should be set to the proper position according to the type of TEI.

- The RX and TX pairs of line should be correctly connected to the TX and RX pairs of NT1.

- The clock control cable should be connected to the proper position when multi-ISDN cards are installed in a system. See section 3.5.

- Terminating resistors on the line.

- SMEMU MUST be required on MPB for QSIG operation. See SMEMU in section 2.

### ISDN STIB ( Basic Rate S/T Interface) Installation

STIB card provides 4 interface circuits of Basic Rate format digital CO and/or Extension interface. For using this board, the PLLU or PLLU2(issue 3 or later) board should be installed on the MPB. If more than one ISDN board is required, some special cable should be used for clock signal control. CON3 and CON4 are used to control the clock signal when multi-ISDN boards are installed. Refer to [Clock Control Cable Wiring] of section 3.5.

Function of LED

| No | Function | Status | Remark |
| --- | --- | --- | --- |
| LED1 | The status of Line 1 | Red: Error, Green: In-use, OFF: Idle |  |
| LED2 | The status of Line 2 | Red: Error, Green: In-use, OFF: Idle |  |
| LED3 | The status of Line 3 | Red: Error, Green: In-use, OFF: Idle |  |
| LED4 | The status of Line 4 | Red: Error, Green: In-use, OFF: Idle |  |

Function of SW1: default - all OFF when delivering the board

| Pole | Function | ON | OFF | Remark |
| --- | --- | --- | --- | --- |
| 1 | Not used | - | - |  |
| 2 | TEI | Fixed TEI | Automatic TEI | Note 1, 2 |
| 3 | Layer 2 trace | Enable | Disable | Note 3 |
| 4 | Layer 3 trace | Enable | Disable | Note 3 |

- Note:

- The function of 2nd pole has been implemented in firmware version 3.1 or later.

- This is set to OFF position as default, therefore it is important to note that 2nd pole is set to the proper position after checking the type of TEI in your country.

- These two poles are for the purpose of developing and maintaining STIB.

Function of various connectors

| Connector | Function | Remark |
| --- | --- | --- |
| CON1 | Serial interface connector | Note1 |
| CON2 | The purpose of development. It is not fixed on the products. |  |
| CON3 | Clock control IN | Note 2 |
| CON4 | Clock control OUT | Note 2 |

- Note

- CON1: Trace for the purpose of development and maintenance.

The external module for RS 232C serial interface should be required for trace.

See GDK-TRC1 in section 2.

- CON3, CON4: They are used to control the reference clock signal when multi-ISDN boards are installed.  See [Clock Control Cable Wiring] on section 3.5.

The line connector and termination switches

| Line No | RJ45 type Jack | S/T selection |  |  | Terminating Resistors |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Switch | all to “T “ | All to “S” | Switch | all to ON | all to OFF |
| Line 1 | CON8 | SW4 | T | S | SW9 | terminating | open |
| Line 2 | CON7 | SW3 | T | S | SW8 | terminating | open |
| Line 3 | CON6 | SW2 | T | S | SW7 | terminating | open |
| Line 4 | CON5 | SW5 | T | S | SW6 | terminating | open |

- Note:

- The setting method of S/T selection switches: default -all T position when delivering the board.

Each line interface can be used for T interface or S interface with different operation interface. If necessary, the S/T selection switches of SW2, SW3, SW4 and/or SW5 can be respectively set to the T or S position by using shorting traps (short pins) supplied on the boards.

- Two poles of SW6, SW7, SW8 and/or SW9 should set to the ON position to terminate two terminating resistors on a line. (default ON)

- Terminating resistors and selectable switches are introduced only on PCB issue 2 or later, but not on PCB issue 1. So if you use STIB issue 1, the external terminating resistors are required. For the external termination, you can get the terminating unit locally or use GDK-162/100 TERM, which is the terminating unit with 4 P-to-P connection provided by LGIC.

Power does not feeding

Power feeding

POWER

T

S

SW2/SW3/SW4/SW5

Shorting trap

(Short pin)

- Note:

- Each line can be selected to T or S interface according to the needs.

2. After setting all lines respectively, the slot(s) of STIB must be assigned as CO slot(s) and STA slot(s) together in Admin field 02 for the proper operation of STIB. See the clause 4.2 and the Programming Manual.

Figure 3.4.6.2.a	Configuration of S/T selection connectors

Check Points

- STIB can be installed in any of the slot No. 1-7 except slot No.8 and EKSU.

- PLLU or PLLU2(issue 3 or later) MUST be installed on MPB.

- Each interface should be set to the correct type, T or S with S/T selection connectors, according to the needs of user. And the RX and TX pairs of line should be correctly connected to the TX and RX pairs of NT1 or TE according to the interface type of each line.

- Terminating resistors on the line.

- The 2nd pole of SW1 should be set to the proper position according to the type of TEI.

- The clock control cable should be connected to the proper position when multi-ISDN cards are installed in a system. See section 3.5.

- SMEMU MUST be required on MPB for QSIG operation. See SMEMU in section 2.

RJ45

CON5

CON3(IN)

SW1

1 2 3 4

GDK-100 STIB

LED1

LED2

LED3

LED4

CON4(OUT)

CON1

U10

(ROM2)

U11

(ROM1)

RJ45

CON6

RJ45

CON7

CON2

RJ45

CON8

1 2 3 4

SW9

1 2

ON

OFF

SW8

1 2

ON

OFF

SW7

1 2

ON

OFF

SW6

1 2

ON

OFF

CON13

T

S

SW4

T

S

SW3

T

S

SW2

T

S

SW5

Figure 3.4.6.2.b	STIB Layout

### ISDN PRIB ( Primary Rate Interface) Installation

The PRIB card provides one interface circuit of PRI (Primary Rate Interface) format digital CO interface. For using this board, the PLLU or PLLU2 (issue 3 or later) should be installed on MPB.

If more than one ISDN board is required, some special cables should be used for clock signal control. CON6 and CON7 are used to control the clock signal when multi-ISDN boards are installed. Refer to [Clock Control Cable Wiring] of section 3.5.

Function of LED

| LED | Function | Status |  | Remark |
| --- | --- | --- | --- | --- |
|  |  | ON | OFF |  |
| LED1 | PLL Synchronization | Sync. Error | Normal |  |
| LED2 | Loss Of Signal from the Line | LOS | Normal |  |
| LED3 | AIS | Alarm error | Normal |  |
| LED4 | Frame Alignment Error | Error | Normal |  |
| LED5 | Multi-frame Error | Error | Normal |  |
| LED6 | Remote Alarm Error | Error | Normal |  |
| LED7 | CRC Error | Error | Normal |  |
| LED8 | The status of the line | In use | All channel idle |  |

- Note

- All LEDs will be the status of OFF when normal operation.

- When PRI line is plugged out, LED 2,4 and 5 will be ON.

Function of SW1: default - all OFF when delivering the board

| Pole | Function | ON | OFF | Remark |
| --- | --- | --- | --- | --- |
| 1 | CRC | Disable | Enable |  |
| 2 | Loop back | Enable | Disable | Note 1 |
| 3 | Not used | - | - |  |
| 4 | Trace | Enable | Disable | Note 2 |

- Note

- This pole is for the purpose of testing PRIB

- This pole are for the purpose of developing and maintaining PRIB.

Function of various connectors

| Connector/Switch | Function | Remark |
| --- | --- | --- |
| CON1 | Serial interface connector( 4pin) | Note 1 |
| CON2 | The purpose of the software development. |  |
| CON3 | NT/TE mode selection for QSIG. ( Default: open - TE) | Note 2 |
| CON5 | Serial interface connector(9-pin RS232C, male) | Note 1 |
| CON6 | Clock control IN | Note 3 |
| CON7 | Clock control OUT | Note 3 |
| CON8 | RJ45 type jack for PRI Line connection | Clause 3.5 |

- Note

- Access to the board for the purpose of development and maintenance.

Board issue 2 or earlier version:

CON5 is RS 232C, 9 pin male to access the board for the purpose of development and maintenance.. This connector can be directly connected with PC for trace ( PRIB issue 2 or earlier version).

Board issue 2A or later version:

CON5 and application circuits on PRIB are removed on PRIB issue 2A to unify the method of tracing peripheral cards such as PRIB, BRIB, STIB, DVIB and WTIB in all GDK systems.

CON1 is mounted to access the board for the purpose of development and maintenance. The external module(GDK-TRC1) for RS 232C serial interface should be required to access the board through this connector.

See GDK-TRC1 in section 2.

- CON3: NT/TE mode selection

Open circuit between 2 pins: TE mode (normal operation)

Short circuit between 2 pins: NT mode (Special operation, e.g. NT mode in QSIG operation)

- CON6, CON7: They are used to control the clock signal when multi-ISDN boards are installed.

See [Clock Control Cable Wiring] on section 3.5.

Figure 3.4.6.3PRIB Layout

GDK-100 PRIB

RJ45

CON8

LED8

CON6(IN)

SW1

U200

(ROM2)

1 2 3 4

1   4

LED1

LED2

LED3

LED4

LED5

LED6

LED7

CON7(OUT)

CON3(TEST)

CON2(JT)

CON1

CON5

U201

(ROM1)

Check Points

- One PRIB should be installed on one of the slot No. 1-7 except the slot No.8 and EKSU. It is recommended that the board is installed in the right slot of BKSU.

- PLLU or PLLU2(issue 3 or later) MUST be installed on MPB.

- For QSIG operation, check the mode setting method and the contact assignments of RJ45 type connector according to the mode of line , TE or NT. See the clause 3.5.

- For QSIG operation, SMEMU MUST be required on MPB for QSIG operation. See SMEMU in section 2.

- The clock control cable should be connected to the proper position when multi-ISDN cards are installed in a system. See section 3.5.

### E1IB Installation

E1IB provides an E1 (2.048Mbps) line interface with Pulse and DTMF signaling, but not R2 MFC signaling. It can be optionally equipped with 3 CPTUs to monitor call progress signals. For using this board, the PLLU or PLLU2 (issue 3) board should be installed on the MPB. If required, install the CPTU, on the E1IB as shown in Figure 3.4.6.4.

It is designed to comply with the special national specification and application, so it may not suitable for the other national specification and/or application.

Function of LED

| No | Function | Status |  | Remark |
| --- | --- | --- | --- | --- |
|  |  | ON | OFF |  |
| LED1 | PLL Synchronization | Sync. Error | Normal |  |
| LED2 | Loss Of Signal from the Line | LOS | Normal |  |
| LED3 | AIS | Alarm error | Normal |  |
| LED4 | Frame Alignment Error | FA error | Normal |  |
| LED5 | Multi-frame Error | MF error | Normal |  |
| LED6 | Board Sync Error | Error | Normal |  |
| LED7 | The status of the line | In use | All channel idle |  |

- Note

- All LEDs will be the status of OFF when normal operation.

- When E1 line is plugged out, LED 2,4 and 5 will be ON.

Function of various connectors

| Connector | Function | Remark |
| --- | --- | --- |
| CON1 | Serial interface connector( 4pin) | Note 1 |
| CON2 | RJ45 modular type connector for E1 Line connection | Clause 3.5 |
| CON3 | BNC type 75 ohm connector for TX | Note 2 |
| CON4 | BNC type 75 ohm connector for RX | Note 2 |
| CON5 | Clock control IN for clock daisy chain | Note 3 |
| CON6 | Clock control OUT for clock daisy chain | Note 3 |

- Note

- Access to the board for the purpose of development and maintenance.

The external module for RS 232C serial interface should be required for trace using this connector.

See GDK-TRC1 in section 2.

- CON3, CON4: BNC type 75 ohm connectors for E1 line connection.

These connectors are usually not assembled except for special countries.

- CON5, CON6: They are used to control the clock signal when multi-ISDN boards are installed.

See [Clock Control Cable Wiring] in section 3.5.

Check Points

- One E1 board should be installed on one of the slot No. 1-7 except slot No.8 and EKSU.

- PLLU or PLLU2(issue 3 or later) MUST be installed on MPB.

- Check the contact assignments of RJ45 or BNC.

GDK-100 E1IB

Figure 3.4.6.4E1IB Layout and CPTU installation

RJ45

CON2

LED7

CON6(IN)

U2

LED1

LED2

LED3

LED4

LED5

LED6

CON7(OUT)

CON1

CPTU

CPTU

CPTU

CON3

CON4

### DVIB (Digitized Voice Interface Board) Installation

DVIB provides system announcement, ACD/UCD announcement, and User Greeting. If necessary, the DMEU(DVU Memory Expansion Unit) or DMEU4 can be installed on DVIB for the expansion of User Greetings. And, if necessary, the ADPU(ADPCM Unit) can be installed on DVIB for the expansion of user access channels.

Function of LED

| No | Description | Status |  | Remark |
| --- | --- | --- | --- | --- |
|  |  | ON | OFF |  |
| LED1 | The status of channel 1 | In use | Idle |  |
| LED2 | The status of channel 2 | In use | Idle |  |
| LED3 | The status of channel 3 | In use | Idle |  |
| LED4 | The status of channel 4 | In use | Idle |  |
| LED5 | The status of channel 5 | In use | Idle |  |
| LED6 | The status of channel 6 | In use | Idle |  |
| LED7 | The status of channel 7 | In use | Idle |  |
| LED8 | The status of channel 8 | In use | Idle |  |

Function of various connectors

| Connector | Function | Remark |
| --- | --- | --- |
| CONN1 | Serial interface connector( 4pin) for trace | Note 1 |
| CONN3 | RCA jack for Audio Input, DVU MOH input |  |
| CONN4 | RCA jack for Audio Output. | Note 2 |
| CONN5 | For the purpose of development | Note 2 |
| CONN6 | For the purpose of development | Note 2 |
| CONN7 | For the purpose of development | Note 2 |
| CONN8, CONN9 | DMEU or DMEU4 connection |  |
| CONN10, CONN11 | ADPU connection |  |

- Note

- CONN1: Trace for the purpose of development and maintenance.

The external module for RS 232C serial interface should be required for trace using this connector. See GDK-TRC1 in section 2.

- CONN4,5,6,and 7: It is impossible to use them for user access.

These connectors are usually used for the purpose of development and production test. Especially, all the pins of CONN5, 6 and 7 should be open for the normal operation.

Function of various switches

| Switch | Function | Remark |
| --- | --- | --- |
| SW1 | Normal/Service switch | Normal |
| SW2 | SRAM back-up when power interruption or failure | ON |
| SW3 | CPU reset | Not Used |
| SW4 | Not used | - |
| JP8 | A-law(OFF) /u-law(ON) selection | OFF |

- Note

- SW1: It should be Normal position during normal operation.

This is not used.

- SW2: This is for the purpose of SRAM back-up for ACD/UCD announcement.

It should be turned ON during installing the DVIB, to protect announcements in the case of a power fail.

“ Caution :

Danger of explosion if battery is incorrectly replaced. Replace only with the same or equivalent type recommended by the manufacturer. Dispose of used batteries according to the manufacturers instruction.”

- JP8: It should be OFF (A-law) for audio input.

Check Points

- JP8 and IC on U36.

- CONN5, CONN6 and CONN7 should be open.

- Add-on boards: DMEU, DMEU4, ADPU

Figure 3.4.7Board Layout and ADPU, DMEU/DMEU4 installation

U1

CONN1

ADPU

CONN3

LED7

LED8

CONN4

U2

JP8

1

ON

OFF

CONN7

CONN6

CONN5

DMEU

or

DMEU4

SW3

LED1

LED2

LED3

LED4

LED5

LED6

1

ON

OFF

SW2