# APPENDIX A.  The Maximum Number of Boards in a Cabinet

The maximum number of boards is described in this section. Some boards can be limited by the capacity of system power supply.

A.1 The Maximum Number of Cards in a system..

The maximum capacity of boards in a system is given in table A.1. Not all maximum capacities may be achieved simultaneously. Some extension cards or their complex configuration can be limited by the system power supply. Especially if you want to make the complex configuration with various extension boards, consult it with its supplier or authorized agent.

Table A.1	The maximum number of cards in a system

| Board Name | The Maximum Number of Boards in a system | Slot Position | Remark |
| --- | --- | --- | --- |
| AC15(UK) | 8 | 1~12 |  |
| BRIB | 6 | 1~7 |  |
| BWDIDB(NZ) | 8 | 1~12 |  |
| DIDB(KOREA) | 8 | 1~12 |  |
| DSIB | 6 | 1~12 |  |
| DTIB12 | 6 | 1~12 |  |
| DTIB24 | 3 | 1~12 |  |
| DVIB | 2 | 1~7 |  |
| E1IB | 1 | 1~7 |  |
| EMIB(KOREA) | 8 | 1~12 |  |
| ETIB | 6 | 1~12 |  |
| LCOB | 8 | 1~12 |  |
| MISB | 1 | 8 |  |
| MPB | 1 | MPB |  |
| PRIB | 1 | 1~7 |  |
| SLIB24 | 12 | 1~12 |  |
| SLIB48 | 9 | 1~12 | 1) |
| SLIBII | 6 | 1~12 |  |
| STIB | 7 | 1~7 |  |
| WTIB | 1 | 1~7 | 2) |
| WTIU | 1 | on WTIB |  |

- Note;

- The maximum cards can be limited by the power capacity.

- These boards have their add-on boards, and the quantity of these boards indicates when their add-on boards are installed.

A.2 The Capacity of GDK-100 Power Supply

The capacity of GDK-100 Power Supplies, PSU1 and PSU3, is given in table A.2. The capacity of +30Vdc in table A.2 includes the capacity of +5Vdc,-5Vdc. -48Vdc is locally generated on the card that it is required.

Table A.2 	Power Capacity of GDK-100 PSU1 and PSU3

|  | +5Vdc | -5Vdc | +30Vdc | -48Vdc |
| --- | --- | --- | --- | --- |
| GDK-100 PSU1(Linear) 1) | 1500 | 500 | 2500 | None |
| GDK-100 PSU3(SMPS) | 5000 | 1000 | 8500 | None |

- Note:

- Manufacturing GDK-100 PSU1 has been officially stopped since March 1999.

# APPENDIX B.  Power Consumption

| Board Name | The Power Consumption/a Card |  |  |  |
| --- | --- | --- | --- | --- |
| (all boards) | +5V | -5V | +30V | -48V |
| AC15(UK) | 114 | 70 | 0 | 0 |
| BRIB | 190 | 0 | 0 | 0 |
| BWDIDB(NZ) | 124 | 60 | 490 | 0 |
| DIDB(KOREA) | 110 | 65 | 415 | 0 |
| DSIB | 120 | 45 | 595 | 0 |
| DTIB12 | 100 | 0 | 780 | 0 |
| DTIB24 | 165 | 0 | 1560 | 0 |
| DVIB | 545 | 7 | 0 | 0 |
| E1IB | 340 | 30 | 0 | 0 |
| EMIB(KOREA) | 95 | 55 | 230 | 0 |
| ETIB | 135 | 110 | 890 | 0 |
| LCOB | 120 | 80 | 80 | 0 |
| MISB | 55 | 40 | 40 | 0 |
| MPB | 400 | 7 | 0 | 0 |
| PRIB | 280 | 0 | 0 | 0 |
| SLIB24 | 100 | 70 | 215 | 0 |
| SLIB48 | 115 | 75 | 750 | 0 |
| SLIBII | 240 | 160 | 490 | 0 |
| STIB | 200 | 0 | 150 | 0 |
| WTIB | 700 | 0 | 320 | 0 |
| WTIU | 480 | 0 | 320 | 0 |
| MEMU | 4 | 0 | 0 | 0 |
| SMEMU | 7 | 0 | 0 | 0 |
| PLLU | 70 | 0 | 0 | 0 |
| PLLU2(issue 3) | 30 | 0 | 0 | 0 |
| MODU | 15 | 7 | 0 | 0 |
| SIU(UK) | 180 | 0 | 0 | 0 |
| SIU(ST) | 120 | 0 | 0 | 0 |
| CMU | 8 | 3 | 0 | 0 |
| PFTU | 0 | 0 | 55 | 0 |
| RGU1,2,5(External) | 0 | 0 | 120 | 0 |
| RGU3,4,6,7(Internal) | 0 | 0 | 85 | 0 |
| Charging Battery | 0 | 0 | 500 | 0 |