### BRI Line Terminating Unit (GDK-162/100 TERM: 2923NP1656A)

GDK-162/100 TERM unit provides four pairs of 100 ohm terminating resistors. The unit is capable of accommodating 4 BRA lines with RJ45 type jacks. It has metal case.

This can be connected at outside of the system. If a customer use Basic Rate (S or T) interface but the 100 ohm terminating resistors are not introduced on the interface circuit, this board will be useful.

See the parts of BRIB Installation, STIB Installation, and Terminating Resistors on Basic Rate Interfaces in section 3.

### Trace Tool (GDK-TRC1)

GDK-TRC1, Figure 2.4.12, provides a RS-232 Serial Interface for the maintenance of special cards of GDK-162 system. It provides 4-wire interface to connect to GDK-162 boards and RS-232 interface to connect to the serial port of PC. All two cables are offered in the GDK-TRC1 package.

GDK-TRC1 can be connected to the following board for access.

| System Name | Board Name |
| --- | --- |
| GDK-162/186 | PRIB issue 3 or later, BRIB issue 2 or later, all STIB version, all WTIB, DCOB version, |

Caution: This tool should be only used by LG Information & Communications, Ltd. and its agent for the purpose of the maintenance of GDK-162 system, but not by an end user.

### Accessories For The Purpose Of Test.

These boards are useful to test GDK-162 system and functional boards for the purpose of investigation. These are not required for an end customer.

Extension Board

These are dummy boards so that the required function board may be positioned at outside of the system cabinet. They are directly installed on the card slot, and then the function cards are connected to the other side of them.

These boards are used when an engineer or a service man wants to investigate the function boards in state of operation.

| Board Name | Function Board Name | Remark |
| --- | --- | --- |
| EXT1(MPB) of GDK-162 | MPB | 2923NP2164A |
| EXT2(PP) of GDK-162 | All GDK-162 boards except MPB. | 2923NP2165A |

Modular connector board

These are the boards for line connection, which contain a RJ21 type connector and RJ-11 type jacks. A RJ21 type connector of the board is connected to the opposite type of RJ21 type connector on the function boards or the system, and RJ-11 type jacks of the board are connected to CO lines or terminals through line cord. It is easy to connect CO line and terminals for the purpose of testing system and terminals.

These boards are used when an engineer or a service man wants to investigate the operation of the system and terminals.

| Board Name | Function Board Name | Remark |
| --- | --- | --- |
| Modular Block of GDK-162/100 | DTIB, ETIB, SLIB24/48/2E/4E | 2923NP2168A |
| Modular Block(DTIB) of GDK-162 | DTIB2 and DTIBE2 (with 24 ports) | 2923NP2172A |
| Modular Block(CO) of GDK-162 | Analog CO lines: P1, P2 | 2923NP2171A |

Figure 2.4.1	Basic Key Service Unit (BKSU)

Figure 2.4.2	Expansion Key Service Unit (EKSU)

Insert Figure 2.4.1 in the existing Manual here

Insert Figure 2.4.2 in the existing Manual here

Figure 2.4.3Power Supply Unit (PSU)

Figure 2.4.4Program Module Unit (PMU)

Insert Figure 2.4.3 in the existing Manual here

Insert Figure 2.4.5 in the existing Manual here

Figure 2.4.5	Main Distribution Frame (MDF)

Note:

- This is only supplied to Korean Market.

2) 	External MDF, which will be in wide use in his country, can be locally used according to the requirements of local regulation for connection method in overseas countries.

Insert Figure 2.4.13 in the existing Manual here

KD-36EXE

KD-33LD

Figure 2.4.6KD Digital Keysets

KD/E-8BTN

KD/E-36EXE

KD/E-36LD

Figure 2.4.7KD/E Digital Keysets

LKD-2NS

LKD-8DS

Figure 2.4.8	LKD Digital Keysets

LKD-30DS

LKD-30LD

GSX-8BTN

GSX-33 EXE

Figure 2.4.9	GSX Electronic Keysets

Note:

For the connection to any digital keyset behind LKD-30DS, 2B-DTIU30 should be inserted in the bottom side of LKD-30DS.

In case of using 2B-DTIU30 and Secondary digital Keyset, It is recommended to use optional mains adapter, which confirms to the appropriate national or local electrical adapter.

2B-DTIU30 Connection

Primary Station

( LKD-30DS)

Secondary Station

(Digital Keyset)

GDK-162 system

DTIB/DTIB2/DTIBE2

2 wire

2 wire

Optional

Mains Adapter

(AC/DC)

Figure 2.4.102B-Modules

Note:

For the connection to SLT device (SLT, Fax, Modem, MOHU) behind LKD-30DS, 2B-SLIU30 should be inserted in the bottom side of LKD-30DS.

2B-SLIU30 Connection

Primary Station

( LKD-30DS)

Secondary Station

(SLT)

GDK-162 system

DTIB/DTIB2/DTIBE2

2 wire

2 wire

Optional

Mains Adapter

(AC/DC)

Note:

For this type of the connection using the external CTI module (GDK-PC PHONE unit), KD/C or KD/EC keyset should be required.

All the line cords and RS232C cable are provided with GDK-PC PHONE unit in the same package.

The connection method of the external CTI module

GKD-PC PHONE

GDK-PC PHONE

(CTI Module)

KD/C,KD/EC Keyset

(CTI Keyset)

PC

-PC application

-Window 3.1 or 95

environment

2 wire line cord

6 wire

line cord

RS-232C

(9 pin)

GDK-162 system

-DTIB/DTIB2/DTIBE2

-MP Call Software

(99A or later version)

Figure 2.4.11CTI modules

Note:

For this type of connection using the internal CTI module (CTIU30/CTIU8), LKD Keyset should be required.

RS232C cable is provided with CTIU30/CTIU8 in the same package.

The connection method of the internal CTI module

PC

-PC application

-Window 3.1 or 95

environment

LKD Keyset

with CTI module

-LKD-30DS and CTIU30

-LKD-8DS/2NS and CTIU8

2 wire line cord

GDK-162 system

-DTIB/DTIB2/DTIBE2

-MP Call Software

(99A or later version)

The cable with RS-232C(9 pin) and RJ11 type plug

Note:

A 4-wire and a RS232C cables are provided with GDK-TRC1 unit in the same package.

Caution: This tool should be only used by LG Information & Communications, Ltd. and its agent for the purpose of the maintenance of GDK-162 system, but not by a end user.

Figure 2.4.12The connection method of the GDK-TRC1

GKD-TRC1

GDK-TRC1

(External trace Tool)

PC

4 wire cable

RS-232C

(9 pin)

GDK-162 system

-BRIB/STIB/PRIB/DCOB/WTIB