# APPENDIX A. The Maximum Number of Boards in a Cabinet

The maximum number of boards is described in this section. Some boards, especially extension cards can be limited by the capacity of system power supply.

A.1 The Maximum Number of Cards a cabinet.

The maximum capacity of boards per a cabinet is given in table A.1. Not all maximum capacities may be achieved simultaneously. Some extension cards or their complex configuration can be limited by the system power supply. Especially if you want to make the complex configuration with various extension boards, consult it with its supplier or authorized agent.

Table A.1	The maximum number of cards per a cabinet

| Board Name | The Maximum Number of Boards per a cabinet | Slot Position | Remark |
| --- | --- | --- | --- |
| AC15(UK) | 8 | 1~8,10~17 |  |
| BRIB | 6 | 1~8 | 2) |
| BWDIDB(NZ) | 5 | 1~8,10~17 | 1) |
| CTIB(KOREA) | 7 | 1~8,10~17 | 1) |
| DCOB with R2 | 1 | 1~8 |  |
| DCOB without R2 | 1 | 1~8 |  |
| DIDB(KOREA) | 6 | 1~8,10~17 |  |
| DTIB(old) | 8(9) | 1~17 |  |
| DTIB2 | 5 | 1~17 | 2) |
| DVU | 3 | 1~17 |  |
| EDVU | 3 | 1~17 |  |
| ETIB | 8 | 1~17 |  |
| LCOB | 8 | 1~8,10~17 |  |
| LCOBE | 8 | 1~8,10~17 |  |
| MISB | 1 | 1~17 |  |
| MPB(Issue 3) | 1 | 0 |  |
| MPB(Issue 4) | 1 | 0 |  |
| OPXB | 2 | 1~17 | 1) |
| PRIB | 1 | 1~8 | 2) |
| SLIB24(Hybrid feed) | 8(9) | 1~17 | 3) |
| SLIB24(Coil feed) | 8(9) | 1~17 | 3) |
| SLIB2E | 8(9) | 1~17 | 3) |
| SLIB48(Hybrid feed) | 3 | 1~17 | 1) |
| SLIB48(Coil feed) | 2 | 1~17 | 1) |
| SLIB4E | 3 | 1~17 | 1) |
| STIB | 8 | 1~8 | 2) |
| TLIB(KOREA) | 7 | 1~8,10~17 | 1) |
| WTIB(ASIC) | 1 | 1~8 |  |
| WTIB(EPLD) | 1 | 1~8 |  |
| DTIBE2 | 5 | on DTIB2 |  |
| BRIU | 6 | on BRIB |  |
| PRIU | 1 | on PRIB |  |
| STIU | 8 | on STIB |  |
| WTIU(ASIC) | 1 | on WTIB(ASIC) |  |
| WTIU(EPLD) | 1 | on WTIB(EPLD) |  |

- Note;

- The maximum cards are limited by the power capacity.

- These boards have their add-on boards, and the quantity of these boards indicates when their add-on boards are installed.

- The quantity in the parentheses indicates the number of cards permitted in EKSU.

A.2 The Capacity of GDK-162/186 Power Supply

The capacity of GDK-162/186 Power Supply is given in table A.2. The capacity of +30Vdc in table A.2 includes the capacity of +5Vdc, -5Vdc, and -48Vdc.

Table A.2		Power Capacity of GDK-162/70 PSU

|  | +5Vdc | -5Vdc | +30Vdc | -48Vdc |
| --- | --- | --- | --- | --- |
| GDK-162 PSU | 8000 | 1500 | 10000 | 1500 |
| GDK-70 PSU 1) | 2700 | 800 | 6000 | None |

- Note:

1) Manufacturing GDK-70 system and its PSU has been officially stopped since June 1998.

# APPENDIX B. Power Consumption

| Board Name (all boards) | The Power Consumption/Board |  |  |  |
| --- | --- | --- | --- | --- |
|  | +5V | -5V | +30V | -48V |
| AC15(UK) | 320 | 70 | 0 | 0 |
| BRIB | 235 | 0 | 0 | 0 |
| BWDIDB(NZ) | 240 | 75 | 115 | 270 |
| CTIB(KOREA) | 230 | 85 | 165 | 200 |
| DCOB with R2 | 630 | 45 | 0 | 0 |
| DCOB without R2 | 490 | 60 | 0 | 0 |
| DIDB(KOREA) | 215 | 75 | 115 | 240 |
| DTIB(old) | 325 | 0 | 840 | 0 |
| DTIB2 | 165 | 0 | 840 | 0 |
| DVU | 305 | 0 | 0 | 0 |
| EDVU | 580 | 5 | 0 | 0 |
| ETIB | 390 | 135 | 880 | 0 |
| LCOB | 240 | 90 | 115 | 0 |
| LCOBE | 240 | 45 | 65 | 0 |
| MISB | 220 | 65 | 65 | 0 |
| MPB(Issue 3) | 410 | 0 | 0 | 0 |
| MPB(Issue 4) | 340 | 0 | 0 | 0 |
| OPXB | 335 | 110 | 115 | 660 |
| PRIB | 310 | 0 | 0 | 0 |
| SLIB24(Hybrid feed) | 225 | 110 | 470 | 0 |
| SLIB24(Coil feed) | 335 | 110 | 535 | 0 |
| SLIB2E | 295 | 95 | 470 | 0 |
| SLIB48(Hybrid feed) | 245 | 132 | 0 | 410 |
| SLIB48(Coil feed) | 335 | 110 | 0 | 605 |
| SLIB4E | 295 | 95 | 90 | 430 |
| STIB | 270 | 0 | 150 | 0 |
| TLIB(KOREA) | 230 | 80 | 85 | 205 |
| WTIB(ASIC) | 650 | 0 | 240 | 0 |
| WTIB(EPLD) | 1000 | 0 | 240 | 0 |
| DTIBE2 | 165 | 0 | 840 | 0 |
| BRIU | 55 | 0 | 0 | 0 |
| PRIU | 200 | 0 | 0 | 0 |
| STIU | 60 | 0 | 150 | 0 |
| WTIU(ASIC) | 450 | 0 | 240 | 0 |
| WTIU(EPLD) | 800 | 0 | 240 | 0 |
| MEMU | 5 | 0 | 0 | 0 |
| SMEMU | 10 | 0 | 0 | 0 |
| MODU(UK) | 65 | 5 | 0 | 0 |
| MODU(ST) | 135 | 5 | 0 | 0 |
| DTRU | 120 | 65 | 0 | 0 |
| CMU | 8 | 3 | 0 | 0 |
| PFTU | 0 | 0 | 65 | 0 |
| RGU | 0 | 0 | 120 | 0 |
| Charging Battery | 0 | 0 | 500 | 0 |