### SYSTEM WIRING

### Battery Back-Up Wiring

As indicated in paragraph 3.3.3, the system can be equipped to provide operation from external batteries if commercial AC power fails. The Back-Up batteries are connected to the strip connector on the front of the BBU PSU, as shown in Figure 3.5.1. When an Expansion KSU is installed, the Battery Back-Up PSU in the Basic KSU is wired to the Expansion KSU PSU, again shown in Figure 3.5.1.

Figure 3.5.1Battery Back-up wiring

GDK-162

PSU

FUSE

20A

GDK-162 PSU

Front Panel

BAT+

BAT -

+ MARKER

- MARKER

To Expansion KSU

Basic KSU

The external batteries must provide 24 Volts DC. This is generally accomplished by connecting two 12 volt batteries in a series arrangement. Operation on batteries is controlled by the PSU. This BBU PSU will provide charging current to the batteries during normal AC power operation at a maximum of about 1 amp. During battery operation, the PSUs will discontinue battery operation if the AC power is re-applied or the battery voltage is too low to maintain proper system operation. If a low-battery cut-off occurs and a new battery is installed, it is necessary to reset the Battery Back-Up circuit in the PSU manually by momentarily depressing the orange colored BATT RESTART switch located on the face-plate of each PSU. Note that the Battery Back-Up circuit in both PSUs of a two cabinet system must be reset. First, press the BATT RESTART switch on the PSU of EKSU, and secondly press that on the PSU of BKSU.

The length of time the system will operate on the batteries is dependent on several elements including; battery charge state, condition of the batteries, capacity of the batteries, and the size of the system (number of station ports). The chart below gives the approximate back-up time for several system sizes and different battery capacities in ampere-hours.

| Battery Capacity | 36 Ports | 82 ports | 132 ports |
| --- | --- | --- | --- |
| 20AH | 4 hours | 2 hours | 1 hours |
| 40AH | 8 hours | 4 hours | 2 hours |

### RS 232C WIRING on MPB and MODU

The MPB has one standard RS-232 ports and an optional port as part of the MODU daughter board. The RS-232 ports are connected by 9-pin “D” connectors as shown in Figure 3.5.2.a. The RS-232 port wiring must include the Clear to Send (CTS) lead if it is enabled by Switch position 2 of the MPB, refer to paragraph 4.4.2. Note the configuration is 8 bits, no parity and 1 stop bit.

Cable for the connection with Terminal (DTE) is not supplied by LGIC.

2

3

4

6

7

8

5

2

3

4

6

7

8

5

TD

RD

DSR

DTR

CTS

RTS

SG

RD

TD

DTR

DSR

RTS

CTS

SG

DB-9 connector

for RS-232C

on MPB and MODU

DB-9 connector

for RS-232C

on Terminal

2

3

4

6

7

8

5

3

3

20

6

4

5

7

TD

RD

DSR

DTR

CTS

RTS

SG

RD

TD

DTR

DSR

RTS

CTS

SG

DB-9 connector

for RS-232C

on MPB and MODU

DB-25

for RS-232C

on Terminal

Name	Function

TD	Transmitted Data

RD	Received Data

RTS	Request To Send

CTS	Clear To Send

DTR	Data Terminal Ready

DSR	Data Set Ready

SG	Signal Ground

Figure 3.5.2.a	Pin assignments and wiring of DB25 and DB9for RS-232C

- Note:

- Modem for remote control or RS232 Serial Interface of MODU is selectable by Admin. Programming, Field 58 button 1. (default: Modem)

- Protective Ground (Frame Ground) may be used between DTE and DCE(GDK system) for more stable communication. And it is recommended.

In DTEs and DCEs, protective ground (frame ground) is a point which is electrically bonded to the equipment frame. Protective ground (frame ground) is not an interchange circuit in RS232 standard.(EIA/TIA-232-F) If bonding of the equipment frames of the DCE and the DTE is necessary, a separate conductor should be used which confirms to the appropriate national or local electrical codes.

The modem of the MODU may be enabled or disabled through assignments in the system’s database. The database also controls the connections for the modem, by defining the CO Line to be used by the modem, refer to SECTION 5. CUSTOMER DATABASE PROGAMMING, Modem Assignment, program 58. The modem provides an internal connection from a CO Line which is wired to the system allowing access to the system database and maintenance files. The resulting connection is shown in Figure 3.5.2.b.

CO and Extension Line

MPB

LCOB

SLIB

ISDN

MODU

Figure 3.5.2.bModem operation of MODU

### MISB wiring

The MISB has connections for 2 music sources, 2 external page ports, 6 relay contact controls, and an alarm/door-bell input monitor. 2 music sources are connected to the external music source through RCA type jacks respectively, and the rest are connected to the proper devices respectively.

Table 3.5.3 shows the contact assignments of RJ21type female connector on MISB.

The cable with RJ21 type male connector is required to connect with terminal or external devices. It is possible to get the cable locally, or use GDK-162 5MC2 supplied by LGIC on the order if necessary.

External Music Source wiring

The two music sources are connected to the MISB by way of RCA type jacks, refer to Figure 4.5.3a. These sources can be any customer provided source such as a tuner, tape deck, etc.

JACK1: music source 1

JACK2: music source 2

The level can be adjusted by the administration programming (Program 66) in 0.5 dB steps to ±20 dB of the input. Note, if the output of the source is too high or low, it will be necessary to lower or raise the output level control at the source.

Above External Music sources including an Internal Music Source on MPB are selectable by Admin Programming.

GDK-162 MISB

Figure 3.5.3.aan Example of external music source wiring

JACK1

JACK2

External Music source 1

External Music source 2

The remaining MISB connections are made through RJ21 type connector on the front edge of the MISB. Table 3.5.3 gives the wiring connections for RJ21 type connector.

Figure 3.5.3.b displays typical wiring for the remaining MISB connections.

Insert the figure 4.5.3.b in the existing manual here

Figure 3.5.3.b	Connection method of RJ21 type connector

External Paging wiring

GDK-162 MISB can supports up to 2 External Paging ports through 2 pairs of RJ21 type connector, CHAMP1 on the board. These ports are often wired to a paging amplifier which is then connected to over-head speakers.

MOH1: Pin No. 13 and Pin No. 38

MOH1: Pin No. 15 and Pin No. 40

Above External Music sources are selectable by Admin Programming.

External DC

Power Supply

External

Amplifier

Ext. paging

output

MDF

GDK-162 MISB

RJ21 type connector, CHAMP1

1

26

13

38

Figure 3.5.3.c	an Example of External paging wiring

Relay contacts for general purpose

GDK-162 MISB supports up to 6 Relay contacts, which can be used for the functions of Loud Control, Door Open and so on under the software control. These relay contacts are connected with external equipment through 4 pairs of RJ21 type connector, CHAMP1 on the board.

RLY1: Pin No. 1 and Pin No. 26

RLY2: Pin No. 3 and Pin No. 28

RLY3: Pin No. 5 and Pin No. 30

RLY4: Pin No. 7 and Pin No. 32

RLY5: Pin No. 9 and Pin No. 34

RLY6: Pin No. 11 and Pin No. 36

External DC

Power Supply

MDF

GDK-162 MISB

RJ21 type connector, CHAMP1

3

28

Figure 3.5.3.dan Example of Loud Bell wiring

Alarm Detection wiring

GDK-162 MISB provides an external alarm detection input, which can be used to notify to extensions when the external switch is on/off. This alarm detection input is provided through a pair of RJ21 type connector, CHAMP1 on the board.

ALARM: Pin No. 24 and Pin No. 49

Close or open detection is programmable by Admin. Programming.

MDF

GDK-162 MISB

RJ21 type connector, CHAMP1

24

49

Figure 3.5.3.ean Example of External switch wiring

External Switch

EMI Suppression

It is recommended that 2 ferrite cores supplied with MISB should be installed on the cable of external music source and the cable with RJ25 respectively, for EMI suppression as Figure 3.5.3.f. The cable ties can be used to prevent the cores slipping.

Figure 3.5.3.fInstallation method of EMI core on the MISB cable

GDK-162 MISB

within 30 Cm

JACK1

JACK2

Ferrite core

Table 3.5.3 shows the contact assignments of RJ21 type connector on MISB.

| Punch-Down | MISB connector |  |  | MISB |  |
| --- | --- | --- | --- | --- | --- |
| Block Pin | Pair | Pin | Color-Code | Designation |  |
| 1 | 1 | 26 | WH/BL | RLY1-T |  |
| 2 |  | 1 | BL/WH | RLY1-R |  |
| 3 | 2 | 27 | WH/OR | - |  |
| 4 |  | 2 | OR/WH | - |  |
| 5 | 3 | 28 | WH/GN | RLY2-T |  |
| 6 |  | 3 | GN/WH | RLY2-R |  |
| 7 | 4 | 29 | WH/BN | - |  |
| 8 |  | 4 | BN/WH | - |  |
| 9 | 5 | 30 | WH/SL | RLY3-T |  |
| 10 |  | 5 | SL/WH | RLY3-R |  |
| 11 | 6 | 31 | RD/BL | - |  |
| 12 |  | 6 | BL/RD | - |  |
| 13 | 7 | 32 | RD/OR | RLY4-T |  |
| 14 |  | 7 | OR/RD | RLY4-R |  |
| 15 | 8 | 33 | RD/GN | - |  |
| 16 |  | 8 | GN/RD | - |  |
| 17 | 9 | 34 | RD/BN | RLY5-T |  |
| 18 |  | 9 | BN/RD | RLY5-R |  |
| 19 | 10 | 35 | RD/SL | - |  |
| 20 |  | 10 | SL/RD | - |  |
| 21 | 11 | 36 | BK/BL | RLY6-T |  |
| 22 |  | 11 | BL/BK | RLY6-R |  |
| 23 | 12 | 37 | BK/OR | - |  |
| 24 |  | 12 | OR/BK | - |  |
| 25 | 13 | 38 | BK/GN | External Page 1T |  |
| 26 |  | 13 | GN/BK | External Page 1R |  |
| 27 | 14 | 39 | BK/BN | - |  |
| 28 |  | 14 | BN/BK | - |  |
| 29 | 15 | 40 | BK/SL | External Page 2T |  |
| 30 |  | 15 | SL/BK | External Page 2R |  |
| 31 | 16 | 41 | YL/BL | - |  |
| 32 |  | 16 | BL/YL | - |  |
| 33 | 17 | 42 | YL/OR | - |  |
| 34 |  | 17 | OR/YL | - |  |
| 35 | 18 | 43 | YL/GN | - |  |
| 36 |  | 18 | GN/YL | - |  |
| 37 | 19 | 44 | YL/BN | - |  |
| 38 |  | 19 | BN/YL | - |  |
| 39 | 20 | 45 | YL/SL | - |  |
| 40 |  | 20 | SL/YL | - | BL:BLUE |
| 41 | 21 | 46 | VI/BL | - | BK:BLACK |
| 42 |  | 21 | BL/VI | - | BN:BROWN |
| 43 | 22 | 47 | VI/OR | - | OR:ORANGE |
| 44 |  | 22 | OR/VI | - | WH:WHITE |
| 45 | 23 | 48 | VI/GN | - | GN:GREEN |
| 46 |  | 23 | GN/VI | - | SL: SLIVER |
| 47 | 24 | 49 | VI/BN | ALARM-T | VI:VIOLET |
| 48 |  | 24 | BN/VI | ALARM-R | RD:RED |
| 49 | 25 | 50 | VI/SL | - | YL:YELLOW |
| 50 |  | 25 | SL/VI | - |  |

Table 3.5.3	MISB Wiring

### Extension Board wiring

Extension boards such as DTIB, DTIB2, ETIB, SLIB24/SLIB2E, SLIB48/SLIB4E contain a RJ21 type jack (25-pair female Amphenol connector) for station wiring to each interface circuit on each board. But WTIB and WTIU, Wireless Terminal Interface Board/Unit for DECT contain RJ11 type jacks.

The following provides details on the interconnection between each type of Extension Board and the jack on the station.

### Digital Keyset and Terminal wiring

Wiring from digital terminal interface board such as DTIB, DTIB2 and DTIBE2, to station jack requires one pair of wire. The power for a station, PCM data and signaling between board and station are transmitted over this pair.

Contact assignments of RJ11 type jack in Digital Terminals

Figure 3.5.4.1.a shows the contact assignments of the miniature 6-position jack (RJ11 type jack) in LG proprietary digital terminals. Two wires from the digital terminal interface of the system are basically connected to the contact numbers 3 (R) and 4(T).

But two additional contacts, R1 and T1 are provided in the digital terminal as shown in Figure 3.5.4.1. The contact number 3 (R) and 2 (R1) are connected together in the digital terminals, and also the contact number 4 (T) and 5 (T1) are connected together. Because there is a country in which the contact number 2 and 5 are used for the 2-wire connection method according to his local regulation. So LGIC provide two more contacts, R1 and T1 in the digital terminals so that the special customer can use the line cord, which is in wide use in his country.

But only 2-wire connection should be recommended to get better transmission characteristics between digital terminal interface of the system and digital terminals.

Figure 3.5.4.1.acontact assignments of Digital Terminals

T

T1

R

R1

1

2

3

4

5

6

Connection method between the digital interfaces and the digital terminals

The cable with 25-pair male connector is required between Digital Terminal Interface board and MDF. It is possible to get the cable locally, or use GDK-162 5MC2 supplied by LGIC on the order if necessary.

The MDF that is in wide use in the country can be locally used according to the requirements of local regulation.

Note:

Digital terminals operate regardless of line polarity, not with a fixed polarity.

Each area means the followings,

A: GDK system, B: between the GDK system and MDF, C: cable in building,

D: between Terminal connection point, e.g. terminating adapter and Terminal,

E: digital terminals

Figure 3.5.4.1.bBasic connection method

between digital terminal interface of the system and digital terminal

MDF

1

2

3

4

5

6

Digital Terminal Interfaces

of the system

RJ21 type

connector

on the board

1

26

2

27

3

28

4

29

DT1

DR1

DT2

DR2

Area A

Area B

Area C

Area D

Area E

Connection method with special condition between the digital interfaces and the digital terminals

Only two-wire connection between the digital terminal interface and digital terminals is recommended to reduce the line noise exited from useless wires. Because R and R1/T and T1 are connected together in the terminals for various national conditions, the useless wires can cause the environment noise, which may affect the data transmission, to flow in the digital terminal the 4-wire line cord is used.

In this case, cut or remove the useless wires connected the contact number R1 and T1 (or, R and T) of the terminals at the Terminal connection point, e.g. terminating adapter, so that those are not drawn out any more.

Note:

Each area means the followings,

A: GDK system, B: between the GDK system and MDF, C: cable in building,

D: between Terminal connection point, e.g. terminating adapter and Terminal,

E: digital terminals

Only two wires should be connected to digital terminals and Interface Board, so the rest should be removed at MDF or Terminal Connection Point, as X points shown in above figure.

MDF

1

2

3

4

5

6

GDK-162 DTIB/DTIB2

RJ21 type

connector

on the board

1

26

2

27

3

28

4

29

DT1

DR1

DT2

DR2

Area A

Area B

Area C

Area D

Area E

1

2

3

4

5

6

Figure 2.5.4.1.c	Connection method when the 4-wire line cord, or

special connection method with with contact number 2 and 5.

Pin Assignments of RJ21 type jack on DTIB/DTIB2

The digital terminals can be connected to every odd-numbered pair such as 1 and 26, 3 and 28 …of RJ21 type connector on DTIB/DTIB2. In case that DTIBE2 is installed on DTIB2, the terminals can be connected to to every even-numbered pair such as 2 and 27, 4 and 29 …of RJ21 type connector on DTIB2 and they will be 13th to 24th terminals in turn. See the clause 3.5.4.6 contact assignments of RJ21 type connector on extension board.

### Electronic Keyset and Terminal wiring

Contact assignments of RJ11 type jack in Eletronic Terminals

Wiring from electronic terminal interface board to station jack requires two pairs of wires. The first pair is for the audible analog signal including voice, and the second one is for signaling and data communication. The pair for data and signaling is polarity sensitive, so electronic terminal will not properly operate when reversed. However the reversal will not harm terminal or system.

Power is delivered by applying DC voltage to the center tap of the coupling transformers of each of 2 pairs.

Figure 3.5.4.2.a shows contact assignments of RJ11 type jack on LG proprietary electronic terminals.

Figure 3.5.4.2.aContact assignments of Electronic Terminals

VT

DT

VR

DR

1

2

3

4

5

6

Connection method between the ETIB and the Electronic terminals

The cable with 25-pair male connector is required between Electronic Terminal Interface board and MDF. It is possible to get the cable locally, or use GDK-162 5MC2 supplied by LGIC on the order if necessary.

MDF

1

2

3

4

5

6

GDK-162 ETIB

RJ21 type

connector

on the board

1

26

2

27

3

28

4

29

VT1

VR1

DT1

DR1

VT2

VR2

Area A

Area B

Area C

Area D

Area E

DT2

DR2

RJ11 jack

on terminal

The MDF that is in wide use in the country can be locally used according to the requirements of local regulation.

Figure 3.5.4.2.bwiring between board and electronic terminal

Note:

Each area means the followings,

A: GDK system, B: between the GDK system and MDF, C: cable in building,

D: between Terminal connection point, e.g. terminating adapter and Terminal,

E: Electronic terminals

A pair for voice signal can be connected regardless of the polarity of Tip and Ring. But the other for signaling and data transmission should be connected with a fixed polarity as above figure.

ETIB is capable of connecting to up to 12 electronic terminals.

### Single line Telephone wiring

SLT interface boards such as SLIB2E, SLIB4E, SLIB24/SLIB24M, and SLIB48/SLIB48M are wired to SLT or standard analog device with a single pair of wire that provides battery feed, voice and signaling from/to the connected SLT. Typical contact assignments of RJ11 type jack, a miniature 6-position jack as following figure 3.5.4.3.a. But the contact assignments and connector type may be different from following figure in special countries according to the special national condition. Before wiring, it is important to check them.

R

T

Figure 3.5.4.3.acontact assignments of SLT

1

2

3

4

5

6

Registered Jack

specified in FCC, CFR

The cable with 25-pair male connector is required between SLT Interface board and MDF. It is possible to get the cable locally, or use GDK-162 5MC2 supplied by LGIC on the order if necessary.

The MDF that is in wide use in your country can be locally used according to the requirements of local regulation.

Note:

SLT (TE) typically operates with both polarities, since a fixed polarity is not guaranteed according to most of specification.

SLIB24 and SLIB48 are respectively capable of connecting to up to 12 SLT.

SLIB2E and SLIB4E are respectively capable of connecting to up to 12 SLT.

MDF

1

2

3

4

5

6

1

26

2

27

3

28

4

29

T1

R1

T2

R2

SLIB2E, SLIB4E, SLIB24,

SLIB48

RJ21 type connector on the board

RJ11 socket

on SLT

Figure 3.5.4.3.bwiring between board and analog TE including SLT

### Intercom/Door Phone Box installation

The Intercom/Door Phone Box is intended to be wall mounted. There are two kinds of Intercom/Door Phone Boxes in GDK-162 system available. KD-Digital Phone Box is can be connected to the digital terminal interface, and KD-ICM Box is can be connected to the electronic terminal interface(ETIB). But another GK or GSX phone box connected to LG Analog Key Telephone System can not be connected to ETIB.

First, separate the ICM/Door Box from the base plate by inserting a flat thin edged screw driver in the bottom of the product. Press the screw driver inward which will release the retaining tan and twist the screw driver to separate the base plate. Fasten the base plate of the ICM/Door Box to the wall with # 8 screws. Route the cable through the cable entry hole in the base plate and connect the wires to the four screw terminals as shown in the figures 3.5.4.4.a and 3.5.4.4.b. When the wiring is complete, pull any slack wire back through the base plate and push the housing back together.

Connection method between the Digital Interface Board and KD-Digital Phone Box

KD-Digital Phone Box can be only connected to the Digital Interface Board. Wiring from digital terminal interface board to KD-Digital Phone Box requires one pair of wire.

Note:  KD-Digital Phone Box operates regardless of line polarity, not with a fixed polarity

MDF

GDK-162 Digital Interface Board

RJ21 type connector

on the board

1

26

2

27

3

28

4

29

DT1

DR1

DT2

DR2

The soldering

side of

KD-Digital Phone Box

GR

RD

Figure 3.5.4.4.awiring between the board and KD-Digital Phone Box

Connection method between the ETIB and KD-ICM Box

KD-ICM Box can be only connected to ETIB. Wiring from electronic terminal interface board to KD-ICM Box requires two pairs of wires.

Note:

A pair for voice signal can be connected regardless of the polarity of Tip and Ring. But the other for signaling and data transmission should be connected with a fixed polarity as above figure.

The GSX or GK Phone Box that can be connected to the LG Analog Key Telephone System, should not be connected to the GDK-162 ETIB.

Figure 3.5.4.4.bwiring between the board and KD-ICM Box

MDF

GDK-162 ETIB

1

26

2

27

3

28

4

29

VT1

VR1

DT1

DR1

VT2

VR2

DT2

DR2

The soldering

side of

KD-ICM Box

GR

BK

YL

RD

RJ21 type connector

on the board

### MOHU installation

The MOHU (Music On Hold Unit) provides External Music Source to the system by connecting to any type of Single Line Interface including 2B-SLIU30 of the GDK-162 system. The External Music Source provided by Single Line Interface is used as music source of MOH channel for System Hold, Background Music and ICM Box Music.

The MOHU has two connections for 2 external music sources (MOH) and 2 Single Line Interfaces (PORT).

- The ‘PORT 1’ is connected to the ‘MOH 1’

- The ‘PORT 2’ is connected to the ‘MOH 2’

The maximum distance of the installation is 1 Km (3.2 Kft) from the system.

GDK-162 system is capable of accommodating up to 5 external music sources through 5 single line interfaces, which are limited by the main program. This function has been supported since the main program GS86P-5.5a or later version.

To use the external music sources through the desired SLT ports, assign the desired SLT ports in the Admin program field #1 and 2nd button. For more details, see the Programming Manual.

External

Music

Source 1

GDK-162

Single Line

Interface

(SLT port)

External

Music

Source 2

PORT1  PORT2

MOH2    MOH1

GDK-162/100(MOHU)

MUSIC ON HOLD UNIT

Figure 3.5.4.5 Connection method of MOHU

- Note :

It is recommended that the MOHU should not be installed in the humid place and the place that ESD (Electrostatic Discharge) occurs.

### Contact Assignments of RJ21 type Connectors on Extension Boards.

Table 3.5.4.6.a. shows the contact assignments of RJ21 type connector on DTIB/DTIB2.

| Punch-Down |  | DTIB/DTIB2 connector |  |  |  | DTIB/DTIB2 |  | Port |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Block Pin | Pair |  | Pin | Color-Code | Designation |  | No |  |  |  |
| 1 | 1 |  | 26 | WH/BL | DT-1 |  | 0 |  | DTIB/DTIB2 |  |
| 2 |  |  | 1 | BL/WH | DR-1 |  |  |  |  |  |
| 3 | 2 |  | 27 | WH/OR | DT-13 |  | 12 |  | DTIB2 only |  |
| 4 |  |  | 2 | OR/WH | DR-13 |  |  |  | (for DTIBE2) |  |
| 5 | 3 |  | 28 | WH/GN | DT-2 |  | 1 |  | DTIB/DTIB2 |  |
| 6 |  |  | 3 | GN/WH | DR-2 |  |  |  |  |  |
| 7 | 4 |  | 29 | WH/BN | DT-14 |  | 13 |  | DTIB2 only |  |
| 8 |  |  | 4 | BN/WH | DR-14 |  |  |  | (for DTIBE2) |  |
| 9 | 5 |  | 30 | WH/SL | DT-3 |  | 2 |  | DTIB/DTIB2 |  |
| 10 |  |  | 5 | SL/WH | DR-3 |  |  |  |  |  |
| 11 | 6 |  | 31 | RD/BL | DT-15 |  | 14 |  | DTIB2 only |  |
| 12 |  |  | 6 | BL/RD | DR-15 |  |  |  | (for DTIBE2) |  |
| 13 | 7 |  | 32 | RD/OR | DT-4 |  | 3 |  | DTIB/DTIB2 |  |
| 14 |  |  | 7 | OR/RD | DR-4 |  |  |  |  |  |
| 15 | 8 |  | 33 | RD/GN | DT-16 |  | 15 |  | DTIB2 only |  |
| 16 |  |  | 8 | GN/RD | DR-16 |  |  |  | (for DTIBE2) |  |
| 17 | 9 |  | 34 | RD/BN | DT-5 |  | 4 |  | DTIB/DTIB2 |  |
| 18 |  |  | 9 | BN/RD | DR-5 |  |  |  |  |  |
| 19 | 10 |  | 35 | RD/SL | DT-17 |  | 16 |  | DTIB2 only |  |
| 20 |  |  | 10 | SL/RD | DR-17 |  |  |  | (for DTIBE2) |  |
| 21 | 11 |  | 36 | BK/BL | DT-6 |  | 5 |  | DTIB/DTIB2 |  |
| 22 |  |  | 11 | BL/BK | DR-6 |  |  |  |  |  |
| 23 | 12 |  | 37 | BK/OR | DT-18 |  | 17 |  | DTIB2 only |  |
| 24 |  |  | 12 | OR/BK | DR-18 |  |  |  | (for DTIBE2) |  |
| 25 | 13 |  | 38 | BK/GN | DT-7 |  | 6 |  | DTIB/DTIB2 |  |
| 26 |  |  | 13 | GN/BK | DR-7 |  |  |  |  |  |
| 27 | 14 |  | 39 | BK/BN | DT-19 |  | 18 |  | DTIB2 only |  |
| 28 |  |  | 14 | BN/BK | DR-19 |  |  |  | (for DTIBE2) |  |
| 29 | 15 |  | 40 | BK/SL | DT-8 |  | 7 |  | DTIB/DTIB2 |  |
| 30 |  |  | 15 | SL/BK | DR-8 |  |  |  |  |  |
| 31 | 16 |  | 41 | YL/BL | DT-20 |  | 19 |  | DTIB2 only |  |
| 32 |  |  | 16 | BL/YL | DR-20 |  |  |  | (for DTIBE2) |  |
| 33 | 17 |  | 42 | YL/OR | DT-9 |  | 8 |  | DTIB/DTIB2 |  |
| 34 |  |  | 17 | OR/YL | DR-9 |  |  |  |  |  |
| 35 | 18 |  | 43 | YL/GN | DT-21 |  | 20 |  | DTIB2 only |  |
| 36 |  |  | 18 | GN/YL | DR-21 |  |  |  | (for DTIBE2) |  |
| 37 | 19 |  | 44 | YL/BN | DT-10 |  | 9 |  | DTIB/DTIB2 |  |
| 38 |  |  | 19 | BN/YL | DR-10 |  |  |  |  |  |
| 39 | 20 |  | 45 | YL/SL | DT-22 |  | 21 |  | DTIB2 only |  |
| 40 |  |  | 20 | SL/YL | DR-22 |  |  |  | (for DTIBE2) |  |
| 41 | 21 |  | 46 | VI/BL | DT-11 |  | 10 |  | DTIB/DTIB2 |  |
| 42 |  |  | 21 | BL/VI | DR-11 |  |  |  |  |  |
| 43 | 22 |  | 47 | VI/OR | DT-23 |  | 22 |  | DTIB2 only |  |
| 44 |  |  | 22 | OR/VI | DR-23 |  |  |  | (for DTIBE2) |  |
| 45 | 23 |  | 48 | VI/GN | DT-12 |  | 11 |  | DTIB/DTIB2 |  |
| 46 |  |  | 23 | GN/VI | DR-12 |  |  |  |  |  |
| 47 | 24 |  | 49 | VI/BN | DT-24 |  | 23 |  | DTIB2 only |  |
| 48 |  |  | 24 | BN/VI | DR-24 |  |  |  | (for DTIBE2) |  |
| 49 | 25 |  | 50 | VI/SL | - |  | - |  |  |  |
| 50 |  |  | 25 | SL/VI | - |  |  |  |  |  |

Table 3.5.4.6.a	DTIB/DTIB2 Wiring

Table 3.5.4.6.b. shows the contact assignments of RJ21 type connector on ETIB.

| Punch-Down |  | ETIB connector |  |  |  | ETIB |  | Port |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Block Pin | Pair |  | Pin | Color-Code | Designation |  | No |  |  |  |
| 1 | 1 |  | 26 | WH/BL | VT-1 |  | 0 |  |  |  |
| 2 |  |  | 1 | BL/WH | VR-1 |  |  |  |  |  |
| 3 | 2 |  | 27 | WH/OR | DT-1 |  |  |  |  |  |
| 4 |  |  | 2 | OR/WH | DR-1 |  |  |  |  |  |
| 5 | 3 |  | 28 | WH/GN | VT-2 |  | 1 |  |  |  |
| 6 |  |  | 3 | GN/WH | VR-2 |  |  |  |  |  |
| 7 | 4 |  | 29 | WH/BN | DT-2 |  |  |  |  |  |
| 8 |  |  | 4 | BN/WH | DR-2 |  |  |  |  |  |
| 9 | 5 |  | 30 | WH/SL | VT-3 |  | 2 |  |  |  |
| 10 |  |  | 5 | SL/WH | VR-3 |  |  |  |  |  |
| 11 | 6 |  | 31 | RD/BL | DT-3 |  |  |  |  |  |
| 12 |  |  | 6 | BL/RD | DR-3 |  |  |  |  |  |
| 13 | 7 |  | 32 | RD/OR | VT-4 |  | 3 |  |  |  |
| 14 |  |  | 7 | OR/RD | VR-4 |  |  |  |  |  |
| 15 | 8 |  | 33 | RD/GN | DT-4 |  |  |  |  |  |
| 16 |  |  | 8 | GN/RD | DR-4 |  |  |  |  |  |
| 17 | 9 |  | 34 | RD/BN | VT-5 |  | 4 |  |  |  |
| 18 |  |  | 9 | BN/RD | VR-5 |  |  |  |  |  |
| 19 | 10 |  | 35 | RD/SL | DT-5 |  |  |  |  |  |
| 20 |  |  | 10 | SL/RD | DR-5 |  |  |  |  |  |
| 21 | 11 |  | 36 | BK/BL | VT-6 |  | 5 |  |  |  |
| 22 |  |  | 11 | BL/BK | VR-6 |  |  |  |  |  |
| 23 | 12 |  | 37 | BK/OR | DT-6 |  |  |  |  |  |
| 24 |  |  | 12 | OR/BK | DR-6 |  |  |  |  |  |
| 25 | 13 |  | 38 | BK/GN | VT-7 |  | 6 |  |  |  |
| 26 |  |  | 13 | GN/BK | VR-7 |  |  |  |  |  |
| 27 | 14 |  | 39 | BK/BN | DT-7 |  |  |  |  |  |
| 28 |  |  | 14 | BN/BK | DR-7 |  |  |  |  |  |
| 29 | 15 |  | 40 | BK/SL | VT-8 |  | 7 |  |  |  |
| 30 |  |  | 15 | SL/BK | VR-8 |  |  |  |  |  |
| 31 | 16 |  | 41 | YL/BL | DT-8 |  |  |  |  |  |
| 32 |  |  | 16 | BL/YL | DR-8 |  |  |  |  |  |
| 33 | 17 |  | 42 | YL/OR | VT-9 |  | 8 |  |  |  |
| 34 |  |  | 17 | OR/YL | VR-9 |  |  |  |  |  |
| 35 | 18 |  | 43 | YL/GN | DT-9 |  |  |  |  |  |
| 36 |  |  | 18 | GN/YL | DR-9 |  |  |  |  |  |
| 37 | 19 |  | 44 | YL/BN | VT-10 |  | 9 |  |  |  |
| 38 |  |  | 19 | BN/YL | VR-10 |  |  |  |  |  |
| 39 | 20 |  | 45 | YL/SL | DT-10 |  |  |  |  |  |
| 40 |  |  | 20 | SL/YL | DR-10 |  |  |  | BL:BLUE |  |
| 41 | 21 |  | 46 | VI/BL | VT-11 |  | 10 |  | BK:BLACK |  |
| 42 |  |  | 21 | BL/VI | VR-11 |  |  |  | BN:BROWN |  |
| 43 | 22 |  | 47 | VI/OR | DT-11 |  |  |  | OR:ORANGE |  |
| 44 |  |  | 22 | OR/VI | DR-11 |  |  |  | WH:WHITE |  |
| 45 | 23 |  | 48 | VI/GN | VT-12 |  | 11 |  | GN:GREEN |  |
| 46 |  |  | 23 | GN/VI | VR-12 |  |  |  | SL: SLIVER |  |
| 47 | 24 |  | 49 | VI/BN | DT-12 |  |  |  | VI:VIOLET |  |
| 48 |  |  | 24 | BN/VI | DR-12 |  |  |  | RD:RED |  |
| 49 | 25 |  | 50 | VI/SL | - |  | - |  | YL:YELLOW |  |
| 50 |  |  | 25 | SL/VI | - |  |  |  |  |  |

Table 3.5.4.6.b	ETIB Wiring

Table 3.5.4.6.c. shows the contact assignments of RJ21 type connector on SLIB24/SLIB24M, SLIB48 /SLIB48M, SLIB2E, and SLIB4E.

| Punch-Down |  | SLIB connector |  |  |  | SLIB |  | Port |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Block Pin | Pair |  | Pin | Color-Code | Designation |  | No |  |  |  |
| 1 | 1 |  | 26 | WH/BL | VT-1 |  | 0 |  |  |  |
| 2 |  |  | 1 | BL/WH | VR-1 |  |  |  |  |  |
| 3 | 2 |  | 27 | WH/OR | - |  | - |  |  |  |
| 4 |  |  | 2 | OR/WH | - |  |  |  |  |  |
| 5 | 3 |  | 28 | WH/GN | VT-2 |  | 1 |  |  |  |
| 6 |  |  | 3 | GN/WH | VR-2 |  |  |  |  |  |
| 7 | 4 |  | 29 | WH/BN | - |  | - |  |  |  |
| 8 |  |  | 4 | BN/WH | - |  |  |  |  |  |
| 9 | 5 |  | 30 | WH/SL | VT-3 |  | 2 |  |  |  |
| 10 |  |  | 5 | SL/WH | VR-3 |  |  |  |  |  |
| 11 | 6 |  | 31 | RD/BL | - |  | - |  |  |  |
| 12 |  |  | 6 | BL/RD | - |  |  |  |  |  |
| 13 | 7 |  | 32 | RD/OR | VT-4 |  | 3 |  |  |  |
| 14 |  |  | 7 | OR/RD | VR-4 |  |  |  |  |  |
| 15 | 8 |  | 33 | RD/GN | - |  | - |  |  |  |
| 16 |  |  | 8 | GN/RD | - |  |  |  |  |  |
| 17 | 9 |  | 34 | RD/BN | VT-5 |  | 4 |  |  |  |
| 18 |  |  | 9 | BN/RD | VR-5 |  |  |  |  |  |
| 19 | 10 |  | 35 | RD/SL | - |  | - |  |  |  |
| 20 |  |  | 10 | SL/RD | - |  |  |  |  |  |
| 21 | 11 |  | 36 | BK/BL | VT-6 |  | 5 |  |  |  |
| 22 |  |  | 11 | BL/BK | VR-6 |  |  |  |  |  |
| 23 | 12 |  | 37 | BK/OR | - |  | - |  |  |  |
| 24 |  |  | 12 | OR/BK | - |  |  |  |  |  |
| 25 | 13 |  | 38 | BK/GN | VT-7 |  | 6 |  |  |  |
| 26 |  |  | 13 | GN/BK | VR-7 |  |  |  |  |  |
| 27 | 14 |  | 39 | BK/BN | - |  | - |  |  |  |
| 28 |  |  | 14 | BN/BK | - |  |  |  |  |  |
| 29 | 15 |  | 40 | BK/SL | VT-8 |  | 7 |  |  |  |
| 30 |  |  | 15 | SL/BK | VR-8 |  |  |  |  |  |
| 31 | 16 |  | 41 | YL/BL | - |  | - |  |  |  |
| 32 |  |  | 16 | BL/YL | - |  |  |  |  |  |
| 33 | 17 |  | 42 | YL/OR | VT-9 |  | 8 |  |  |  |
| 34 |  |  | 17 | OR/YL | VR-9 |  |  |  |  |  |
| 35 | 18 |  | 43 | YL/GN | - |  | - |  |  |  |
| 36 |  |  | 18 | GN/YL | - |  |  |  |  |  |
| 37 | 19 |  | 44 | YL/BN | VT-10 |  | 9 |  |  |  |
| 38 |  |  | 19 | BN/YL | VR-10 |  |  |  |  |  |
| 39 | 20 |  | 45 | YL/SL | - |  | - |  |  |  |
| 40 |  |  | 20 | SL/YL | - |  |  |  | BL:BLUE |  |
| 41 | 21 |  | 46 | VI/BL | VT-11 |  | 10 |  | BK:BLACK |  |
| 42 |  |  | 21 | BL/VI | VR-11 |  |  |  | BN:BROWN |  |
| 43 | 22 |  | 47 | VI/OR | - |  | - |  | OR:ORANGE |  |
| 44 |  |  | 22 | OR/VI | - |  |  |  | WH:WHITE |  |
| 45 | 23 |  | 48 | VI/GN | VT-12 |  | 11 |  | GN:GREEN |  |
| 46 |  |  | 23 | GN/VI | VR-12 |  |  |  | SL: SLIVER |  |
| 47 | 24 |  | 49 | VI/BN | - |  | - |  | VI:VIOLET |  |
| 48 |  |  | 24 | BN/VI | - |  |  |  | RD:RED |  |
| 49 | 25 |  | 50 | VI/SL | - |  | - |  | YL:YELLOW |  |
| 50 |  |  | 25 | SL/VI | - |  |  |  |  |  |

Table 3.5.4.6.c	SLIB24/SLIB48/LSIB2E/SLIB4E Wiring

### PFTU Wiring

The PFTU is wired from the RJ21 type male connector on the KSU CO Line connector bracket to the MDF, cross connected to the CO Line inputs from the telephone company, to the CO Line inputs of the system, and to Power Failure SLTs. The basic connections are shown in Figure 3.5.5 and the 25 pair connector configuration is given in Table 3.5.5. Note that the SLT may also be connected to a SLIB in the system as shown in the figure.

The cable with RJ21 type female connector is required between PFTU connector and MDF. It is possible to get the cable locally, or use GDK-162 5MC1 supplied by LGIC on the order if necessary.

The MDF that is in wide use in your country can also be locally used according to the requirements of local regulation.

MDF

1

26

4

29

25

50

2

27

3

28

SLIB1-T

SLIB1-R

PFTU on the right side of

BKSU/EKSU

COT1-IN

COR1-IN

COT1-OUT

COR1-OUT

TEL1-T

TEL1-R

SLIB interface

CO line

LCOB interface

SLT

RJ21 type connector

on the right side of BKSU/EKSU

Figure 3.5.5	PFTU wiring

Table 3.5.5. shows the contact assignments of RJ21 type connector on PFTU.

| Punch-Down |  | PFTU connector |  |  |  | PFTU |  | PFT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Block Pin | Pair |  | Pin | Color-Code | Designation |  | Circuit |  |  |  |
| 1 | 1 |  | 26 | WH/BL | COT1-IN |  | 1st Circuit |  |  |  |
| 2 |  |  | 1 | BL/WH | COR1-IN |  |  |  |  |  |
| 3 | 2 |  | 27 | WH/OR | COT1-OUT |  |  |  |  |  |
| 4 |  |  | 2 | OR/WH | COR1-OUT |  |  |  |  |  |
| 5 | 3 |  | 28 | WH/GN | TEL1-T |  |  |  |  |  |
| 6 |  |  | 3 | GN/WH | TEL1-R |  |  |  |  |  |
| 7 | 4 |  | 29 | WH/BN | SLIB1-T |  |  |  |  |  |
| 8 |  |  | 4 | BN/WH | SLIB1-R |  |  |  |  |  |
| 9 | 5 |  | 30 | WH/SL | COT2-IN |  | 2nd Circuit |  |  |  |
| 10 |  |  | 5 | SL/WH | COR2-IN |  |  |  |  |  |
| 11 | 6 |  | 31 | RD/BL | COT2-OUT |  |  |  |  |  |
| 12 |  |  | 6 | BL/RD | COR2-OUT |  |  |  |  |  |
| 13 | 7 |  | 32 | RD/OR | TEL2-T |  |  |  |  |  |
| 14 |  |  | 7 | OR/RD | TEL2-R |  |  |  |  |  |
| 15 | 8 |  | 33 | RD/GN | SLIB2-T |  |  |  |  |  |
| 16 |  |  | 8 | GN/RD | SLIB2-R |  |  |  |  |  |
| 17 | 9 |  | 34 | RD/BN | COT3-IN |  | 3rd Circuit |  |  |  |
| 18 |  |  | 9 | BN/RD | COR3-IN |  |  |  |  |  |
| 19 | 10 |  | 35 | RD/SL | COT3-OUT |  |  |  |  |  |
| 20 |  |  | 10 | SL/RD | COR3-OUT |  |  |  |  |  |
| 21 | 11 |  | 36 | BK/BL | TEL3-T |  |  |  |  |  |
| 22 |  |  | 11 | BL/BK | TEL3-R |  |  |  |  |  |
| 23 | 12 |  | 37 | BK/OR | SLIB3-T |  |  |  |  |  |
| 24 |  |  | 12 | OR/BK | SLIB3-R |  |  |  |  |  |
| 25 | 13 |  | 38 | BK/GN | COT4-IN |  | 4th Circuit |  |  |  |
| 26 |  |  | 13 | GN/BK | COR4-IN |  |  |  |  |  |
| 27 | 14 |  | 39 | BK/BN | COT4-OUT |  |  |  |  |  |
| 28 |  |  | 14 | BN/BK | COR4-OUT |  |  |  |  |  |
| 29 | 15 |  | 40 | BK/SL | TEL4-T |  |  |  |  |  |
| 30 |  |  | 15 | SL/BK | TEL4-R |  |  |  |  |  |
| 31 | 16 |  | 41 | YL/BL | SLIB4-T |  |  |  |  |  |
| 32 |  |  | 16 | BL/YL | SLIB4-R |  |  |  |  |  |
| 33 | 17 |  | 42 | YL/OR | COT5-IN |  | 5th Circuit |  |  |  |
| 34 |  |  | 17 | OR/YL | COR5-IN |  |  |  |  |  |
| 35 | 18 |  | 43 | YL/GN | COT5-OUT |  |  |  |  |  |
| 36 |  |  | 18 | GN/YL | COR5-OUT |  |  |  |  |  |
| 37 | 19 |  | 44 | YL/BN | TEL5-T |  |  |  |  |  |
| 38 |  |  | 19 | BN/YL | TEL5-R |  |  |  |  |  |
| 39 | 20 |  | 45 | YL/SL | SLIB5-T |  |  |  |  |  |
| 40 |  |  | 20 | SL/YL | SLIB5-R |  |  |  | BL:BLUE |  |
| 41 | 21 |  | 46 | VI/BL | COT6-IN |  | 6th Circuit |  | BK:BLACK |  |
| 42 |  |  | 21 | BL/VI | COR6-IN |  |  |  | BN:BROWN |  |
| 43 | 22 |  | 47 | VI/OR | COT6-OUT |  |  |  | OR:ORANGE |  |
| 44 |  |  | 22 | OR/VI | COR6-OUT |  |  |  | WH:WHITE |  |
| 45 | 23 |  | 48 | VI/GN | TEL6-T |  |  |  | GN:GREEN |  |
| 46 |  |  | 23 | GN/VI | TEL6-R |  |  |  | SL: SLIVER |  |
| 47 | 24 |  | 49 | VI/BN | SLIB6-T |  |  |  | VI:VIOLET |  |
| 48 |  |  | 24 | BN/VI | SLIB6-R |  |  |  | RD:RED |  |
| 49 | 25 |  | 50 | VI/SL | - |  | - |  | YL:YELLOW |  |
| 50 |  |  | 25 | SL/VI | - |  |  |  |  |  |

Table 3.5.5.5	PFTU Wiring

### Analog CO line wiring

The CO Lines are terminated to the individual card slots in the KSU through the CO Line connector bracket on the right side of the KSU. Each card slot has 6 dedicated pairs on the appropriate RJ21 type connector (P1 or P2). Thus, the card slot of the CO interface board determines which pair and CO connector are used to wire a CO Line. Tables 3.5.6a through 3.5.6d show the contact assignments of RJ21 type male connectors in relation to the slots for CO Line wiring.

The cable with RJ21 type female connector is required between CO line connectors (P1 and P2) and MDF. It is possible to get the cable locally, or use GDK-162 5MC1 supplied by LGIC on the order if necessary.

The MDF that is in wide use in each country can be locally used according to the requirements of local regulation.

The following provides details on the connection of each type of analog CO line Board.

### LCOBE wiring

LCOB is capable of connecting 6 loop start CO lines through the CO Line connector bracket on the right side of the KSU.

The following Figure 3.5.6.1 is an example connection in case that LCOB is installed in slot 8. The relation between the contact assignments of P1/P2 and slots is given in Tables 3.5.6.6.a through 3.5.6.6.d.

MDF

1

26

4

29

25

50

2

27

3

28

4R

4T

1R

1T

2R

2T

3R

3T

P1 on the right side of the KSU

Network

Note:

- Above figure is an example connection in case that LCOB is installed in slot # 8.

- CO lines of LCOBE can operate regardless of line polarity.

- LCOBE is capable of connecting to up to 4 lines.

Figure 3.5.6.1	loop start CO line wiring

### AC15 Board wiring -UK

AC15 is capable of connecting 3 Private lines through the CO Line connector bracket on the right side of the KSU.

The following Figure 3.5.6.2 is an example connection in case that the AC15 board is installed in slot 8. The relation between the contact assignments of P1/P2 and slots is given in Tables 3.5.6.6.a through 3.5.6.6.d.

Note:

Above figure is an example connection in case that AC15 board is installed in slot # 8.

A pair of a line is for RX signal, and the other is for TX signal. So, 2 pairs of wires are required for the connection of 1 line.

Each pair can be connected regardless of the polarity of Tip and Ring.

AC15 board is capable of connecting to up to 3 lines.

Figure 3.5.6.2AC15 Private line wiring

MDF

1

26

4

29

4R (TXR2)

4T (TXT2)

P1 on the right side of the KSU

25

50

1R (RXR1)

1T (RXT1)

2

27

2R (TXR1)

2T (TXT1)

3

28

3R (RXR2)

3T (RXT2)

Network

TXR

TXT

RXR

RXT

### CTIB wiring - Korea

CTIB is capable of connecting to four (4) loop start CO lines and two 2-wire E&M tie lines with type V.

The fore 4 interfaces of CTIB are connected to 4 loop start CO lines through the CO Line connector bracket on the right side of the KSU. The connection method of Loop start CO line is the same as shown in Figure 3.5.6.1 LCOB wiring.

The rear 2 interfaces of CTIB are directly connected to the tie lines through the connector with screws on the front of the board as Figure 3.5.6.3.

MDF

CTIB

Network

E

M

R

T

1T

1R

1E

1M

2T

2R

2E

2M

E

M

R

T

Figure 3.5.6.3	E&M tie line wiring

The connecting block with screws on board

Figure 3.5.6.3	E&M tie line wiring

Note:

- A pair of wires is for audio signal, and it can be connected regardless of the polarity of Tip and Ring.

- The other is for signaling, and it is polarity-sensitive. EAR wire of EMIB is connected to MOUTH wire of Network/PBX and vice versa.

It is important to check EAR and MOUTH of Network before wiring, because some network equipment may describe ‘ opposite marks ‘.

3.	The cable for E&M wiring is not supplied, but the cable which confirms to the appropriate national or local electrical codes can be used. AWG #22 or #24 wire is recommended.

### DIDB wiring - Korea

DIDB is capable of connecting six (6) analog DID lines through the CO Line connector bracket on the right side of the KSU.

The following Figure 3.5.6.4 is an example connection in case that the DIDB is installed in slot 8. The relation between the contact assignments of P1/P2 and slots is given in Tables 3.5.6.6.a through 3.5.6.6.d.

Note:

Above figure is an example connection in case that DIDB is installed in slot # 8.

The DIDB it is polarity-sensitive, so it is important to check the polarity of wires when wiring. In idle state, –48V is fed on Ring wire, and Reference is on Tip wire in GDK-162 DIDB.

But some network equipment may operate with both polarities.

Figure 3.5.6.4DID line wiring

MDF

1

26

4

29

P1 on the right side of the KSU

25

50

2

27

3

28

4R

4T

1R

1T

2R

2T

3R

3T

Network

T

R

### BWDIDB wiring - New Zealand

BWDIDB is capable of connecting 6 both-way DID lines through the CO Line connector bracket on the right side of the KSU.

The following Figure 3.5.6.5 is an example connection in case that the BWDIDB is installed in slot 8. The relation between the contact assignments of P1/P2 and slots is given in Tables 3.5.6.6.a through 3.5.6.6.d.

MDF

1

26

4

29

R4

T4

P1 on the right side of the KSU

25

50

R1

T1

2

27

R2

T2

3

28

R3

T3

Network

T

R

Figure 3.5.6.5	Both-way DID line wiring

Note:

- Above figure is an example connection in case that BWDIDB is installed in slot # 8.

- BWDIDB operates with one polarity, so it is important to check the polarity of wires when wiring. In idle state, –48V is fed on Ring wire, and Reference is on Tip wire in GDK-162 system.

- BWDIDB board is capable of connecting with up to 4 lines in turn.

### Contact Assignments of RJ21 type connectors(P1/P2) on BKSU and EKSU

Table 3.5.6.6.a. shows the contact assignments of P1 connector on BKSU.

| Punch-Down |  | P1 connector |  |  |  | Hardware |  | Card |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Block Pin | Pair |  | Pin | Color-Code | Designation |  | Slot No. |  |  |  |
| 1 | 1 |  | 26 | WH/BL | CO-1R |  | 08 |  |  |  |
| 2 |  |  | 1 | BL/WH | CO-1T |  |  |  |  |  |
| 3 | 2 |  | 27 | WH/OR | CO-2R |  |  |  |  |  |
| 4 |  |  | 2 | OR/WH | CO-2T |  |  |  |  |  |
| 5 | 3 |  | 28 | WH/GN | CO-3R |  |  |  |  |  |
| 6 |  |  | 3 | GN/WH | CO-3T |  |  |  |  |  |
| 7 | 4 |  | 29 | WH/BN | CO-4R |  |  |  |  |  |
| 8 |  |  | 4 | BN/WH | CO-4T |  |  |  |  |  |
| 9 | 5 |  | 30 | WH/SL | CO-5R |  |  |  |  |  |
| 10 |  |  | 5 | SL/WH | CO-5T |  |  |  |  |  |
| 11 | 6 |  | 31 | RD/BL | CO-6R |  |  |  |  |  |
| 12 |  |  | 6 | BL/RD | CO-6T |  |  |  |  |  |
| 13 | 7 |  | 32 | RD/OR | CO-7R |  | 07 |  |  |  |
| 14 |  |  | 7 | OR/RD | CO-7T |  |  |  |  |  |
| 15 | 8 |  | 33 | RD/GN | CO-8R |  |  |  |  |  |
| 16 |  |  | 8 | GN/RD | CO-8T |  |  |  |  |  |
| 17 | 9 |  | 34 | RD/BN | CO-9R |  |  |  |  |  |
| 18 |  |  | 9 | BN/RD | CO-9T |  |  |  |  |  |
| 19 | 10 |  | 35 | RD/SL | CO-10R |  |  |  |  |  |
| 20 |  |  | 10 | SL/RD | CO-10T |  |  |  |  |  |
| 21 | 11 |  | 36 | BK/BL | CO-11R |  |  |  |  |  |
| 22 |  |  | 11 | BL/BK | CO-11T |  |  |  |  |  |
| 23 | 12 |  | 37 | BK/OR | CO-12R |  |  |  |  |  |
| 24 |  |  | 12 | OR/BK | CO-12T |  |  |  |  |  |
| 25 | 13 |  | 38 | BK/GN | CO-13R |  | 06 |  |  |  |
| 26 |  |  | 13 | GN/BK | CO-13T |  |  |  |  |  |
| 27 | 14 |  | 39 | BK/BN | CO-14R |  |  |  |  |  |
| 28 |  |  | 14 | BN/BK | CO-14T |  |  |  |  |  |
| 29 | 15 |  | 40 | BK/SL | CO-15R |  |  |  |  |  |
| 30 |  |  | 15 | SL/BK | CO-15T |  |  |  |  |  |
| 31 | 16 |  | 41 | YL/BL | CO-16R |  |  |  |  |  |
| 32 |  |  | 16 | BL/YL | CO-16T |  |  |  |  |  |
| 33 | 17 |  | 42 | YL/OR | CO-17R |  |  |  |  |  |
| 34 |  |  | 17 | OR/YL | CO-17T |  |  |  |  |  |
| 35 | 18 |  | 43 | YL/GN | CO-18R |  |  |  |  |  |
| 36 |  |  | 18 | GN/YL | CO-18T |  |  |  |  |  |
| 37 | 19 |  | 44 | YL/BN | CO-19R |  | 05 |  |  |  |
| 38 |  |  | 19 | BN/YL | CO-19T |  |  |  |  |  |
| 39 | 20 |  | 45 | YL/SL | CO-20R |  |  |  |  |  |
| 40 |  |  | 20 | SL/YL | CO-20T |  |  |  | BL:BLUE |  |
| 41 | 21 |  | 46 | VI/BL | CO-21R |  |  |  | BK:BLACK |  |
| 42 |  |  | 21 | BL/VI | CO-21T |  |  |  | BN:BROWN |  |
| 43 | 22 |  | 47 | VI/OR | CO-22R |  |  |  | OR:ORANGE |  |
| 44 |  |  | 22 | OR/VI | CO-22T |  |  |  | WH:WHITE |  |
| 45 | 23 |  | 48 | VI/GN | CO-23R |  |  |  | GN:GREEN |  |
| 46 |  |  | 23 | GN/VI | CO-23T |  |  |  | SL: SLIVER |  |
| 47 | 24 |  | 49 | VI/BN | CO-24R |  |  |  | VI:VIOLET |  |
| 48 |  |  | 24 | BN/VI | CO-24T |  |  |  | RD:RED |  |
| 49 | 25 |  | 50 | VI/SL | - |  | - |  | YL:YELLOW |  |
| 50 |  |  | 25 | SL/VI |  |  |  |  |  |  |

Table 3.5.6.6.a	BKSU P1 connector

Table 3.5.6.6.b. shows the contact assignments of P2 connector on BKSU.

| Punch-Down |  | P1 connector |  |  |  | Hardware |  | Card |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Block Pin | Pair |  | Pin | Color-Code | Designation |  | Slot No. |  |  |  |
| 1 | 1 |  | 26 | WH/BL | CO-25R |  | 04 |  |  |  |
| 2 |  |  | 1 | BL/WH | CO-25T |  |  |  |  |  |
| 3 | 2 |  | 27 | WH/OR | CO-26R |  |  |  |  |  |
| 4 |  |  | 2 | OR/WH | CO-26T |  |  |  |  |  |
| 5 | 3 |  | 28 | WH/GN | CO-27R |  |  |  |  |  |
| 6 |  |  | 3 | GN/WH | CO-27T |  |  |  |  |  |
| 7 | 4 |  | 29 | WH/BN | CO-28R |  |  |  |  |  |
| 8 |  |  | 4 | BN/WH | CO-28T |  |  |  |  |  |
| 9 | 5 |  | 30 | WH/SL | CO-29R |  |  |  |  |  |
| 10 |  |  | 5 | SL/WH | CO-29T |  |  |  |  |  |
| 11 | 6 |  | 31 | RD/BL | CO-30R |  |  |  |  |  |
| 12 |  |  | 6 | BL/RD | CO-30T |  |  |  |  |  |
| 13 | 7 |  | 32 | RD/OR | CO-31R |  | 03 |  |  |  |
| 14 |  |  | 7 | OR/RD | CO-31T |  |  |  |  |  |
| 15 | 8 |  | 33 | RD/GN | CO-32R |  |  |  |  |  |
| 16 |  |  | 8 | GN/RD | CO-32T |  |  |  |  |  |
| 17 | 9 |  | 34 | RD/BN | CO-33R |  |  |  |  |  |
| 18 |  |  | 9 | BN/RD | CO-33T |  |  |  |  |  |
| 19 | 10 |  | 35 | RD/SL | CO-34R |  |  |  |  |  |
| 20 |  |  | 10 | SL/RD | CO-34T |  |  |  |  |  |
| 21 | 11 |  | 36 | BK/BL | CO-35R |  |  |  |  |  |
| 22 |  |  | 11 | BL/BK | CO-35T |  |  |  |  |  |
| 23 | 12 |  | 37 | BK/OR | CO-36R |  |  |  |  |  |
| 24 |  |  | 12 | OR/BK | CO-36T |  |  |  |  |  |
| 25 | 13 |  | 38 | BK/GN | CO-37R |  | 02 |  |  |  |
| 26 |  |  | 13 | GN/BK | CO-37T |  |  |  |  |  |
| 27 | 14 |  | 39 | BK/BN | CO-38R |  |  |  |  |  |
| 28 |  |  | 14 | BN/BK | CO-38T |  |  |  |  |  |
| 29 | 15 |  | 40 | BK/SL | CO-39R |  |  |  |  |  |
| 30 |  |  | 15 | SL/BK | CO-39T |  |  |  |  |  |
| 31 | 16 |  | 41 | YL/BL | CO-40R |  |  |  |  |  |
| 32 |  |  | 16 | BL/YL | CO-40T |  |  |  |  |  |
| 33 | 17 |  | 42 | YL/OR | CO-41R |  |  |  |  |  |
| 34 |  |  | 17 | OR/YL | CO-41T |  |  |  |  |  |
| 35 | 18 |  | 43 | YL/GN | CO-42R |  |  |  |  |  |
| 36 |  |  | 18 | GN/YL | CO-42T |  |  |  |  |  |
| 37 | 19 |  | 44 | YL/BN | CO-43R |  | 01 |  |  |  |
| 38 |  |  | 19 | BN/YL | CO-43T |  |  |  |  |  |
| 39 | 20 |  | 45 | YL/SL | CO-44R |  |  |  |  |  |
| 40 |  |  | 20 | SL/YL | CO-44T |  |  |  | BL:BLUE |  |
| 41 | 21 |  | 46 | VI/BL | CO-45R |  |  |  | BK:BLACK |  |
| 42 |  |  | 21 | BL/VI | CO-45T |  |  |  | BN:BROWN |  |
| 43 | 22 |  | 47 | VI/OR | CO-46R |  |  |  | OR:ORANGE |  |
| 44 |  |  | 22 | OR/VI | CO-46T |  |  |  | WH:WHITE |  |
| 45 | 23 |  | 48 | VI/GN | CO-47R |  |  |  | GN:GREEN |  |
| 46 |  |  | 23 | GN/VI | CO-47T |  |  |  | SL: SLIVER |  |
| 47 | 24 |  | 49 | VI/BN | CO-48R |  |  |  | VI:VIOLET |  |
| 48 |  |  | 24 | BN/VI | CO-48T |  |  |  | RD:RED |  |
| 49 | 25 |  | 50 | VI/SL | - |  | - |  | YL:YELLOW |  |
| 50 |  |  | 25 | SL/VI |  |  |  |  |  |  |

Table 3.5.6.6.b	BKSU P2 connector

Table 3.5.6.6.c. shows the contact assignments of P1 connector on EKSU.

| Punch-Down |  | P1 connector |  |  |  | Hardware |  | Card |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Block Pin | Pair |  | Pin | Color-Code | Designation |  | Slot No. |  |  |  |
| 1 | 1 |  | 26 | WH/BL | CO-49R |  | 17 |  |  |  |
| 2 |  |  | 1 | BL/WH | CO-49T |  |  |  |  |  |
| 3 | 2 |  | 27 | WH/OR | CO-50R |  |  |  |  |  |
| 4 |  |  | 2 | OR/WH | CO-50T |  |  |  |  |  |
| 5 | 3 |  | 28 | WH/GN | CO-51R |  |  |  |  |  |
| 6 |  |  | 3 | GN/WH | CO-51T |  |  |  |  |  |
| 7 | 4 |  | 29 | WH/BN | CO-52R |  |  |  |  |  |
| 8 |  |  | 4 | BN/WH | CO-52T |  |  |  |  |  |
| 9 | 5 |  | 30 | WH/SL | CO-53R |  |  |  |  |  |
| 10 |  |  | 5 | SL/WH | CO-53T |  |  |  |  |  |
| 11 | 6 |  | 31 | RD/BL | CO-54R |  |  |  |  |  |
| 12 |  |  | 6 | BL/RD | CO-54T |  |  |  |  |  |
| 13 | 7 |  | 32 | RD/OR | CO-55R |  | 16 |  |  |  |
| 14 |  |  | 7 | OR/RD | CO-55T |  |  |  |  |  |
| 15 | 8 |  | 33 | RD/GN | CO-56R |  |  |  |  |  |
| 16 |  |  | 8 | GN/RD | CO-56T |  |  |  |  |  |
| 17 | 9 |  | 34 | RD/BN | CO-57R |  |  |  |  |  |
| 18 |  |  | 9 | BN/RD | CO-57T |  |  |  |  |  |
| 19 | 10 |  | 35 | RD/SL | CO-58R |  |  |  |  |  |
| 20 |  |  | 10 | SL/RD | CO-58T |  |  |  |  |  |
| 21 | 11 |  | 36 | BK/BL | CO-59R |  |  |  |  |  |
| 22 |  |  | 11 | BL/BK | CO-59T |  |  |  |  |  |
| 23 | 12 |  | 37 | BK/OR | CO-60R |  |  |  |  |  |
| 24 |  |  | 12 | OR/BK | CO-60T |  |  |  |  |  |
| 25 | 13 |  | 38 | BK/GN | CO-61R |  | 15 |  |  |  |
| 26 |  |  | 13 | GN/BK | CO-61T |  |  |  |  |  |
| 27 | 14 |  | 39 | BK/BN | CO-62R |  |  |  |  |  |
| 28 |  |  | 14 | BN/BK | CO-62T |  |  |  |  |  |
| 29 | 15 |  | 40 | BK/SL | CO-63R |  |  |  |  |  |
| 30 |  |  | 15 | SL/BK | CO-63T |  |  |  |  |  |
| 31 | 16 |  | 41 | YL/BL | CO-64R |  |  |  |  |  |
| 32 |  |  | 16 | BL/YL | CO-64T |  |  |  |  |  |
| 33 | 17 |  | 42 | YL/OR | CO-65R |  |  |  |  |  |
| 34 |  |  | 17 | OR/YL | CO-65T |  |  |  |  |  |
| 35 | 18 |  | 43 | YL/GN | CO-66R |  |  |  |  |  |
| 36 |  |  | 18 | GN/YL | CO-66T |  |  |  |  |  |
| 37 | 19 |  | 44 | YL/BN | CO-67R |  | 14 |  |  |  |
| 38 |  |  | 19 | BN/YL | CO-67T |  |  |  |  |  |
| 39 | 20 |  | 45 | YL/SL | CO-68R |  |  |  |  |  |
| 40 |  |  | 20 | SL/YL | CO-68T |  |  |  | BL:BLUE |  |
| 41 | 21 |  | 46 | VI/BL | CO-69R |  |  |  | BK:BLACK |  |
| 42 |  |  | 21 | BL/VI | CO-69T |  |  |  | BN:BROWN |  |
| 43 | 22 |  | 47 | VI/OR | CO-70R |  |  |  | OR:ORANGE |  |
| 44 |  |  | 22 | OR/VI | CO-70T |  |  |  | WH:WHITE |  |
| 45 | 23 |  | 48 | VI/GN | CO-71R |  |  |  | GN:GREEN |  |
| 46 |  |  | 23 | GN/VI | CO-71T |  |  |  | SL: SLIVER |  |
| 47 | 24 |  | 49 | VI/BN | CO-72R |  |  |  | VI:VIOLET |  |
| 48 |  |  | 24 | BN/VI | CO-72T |  |  |  | RD:RED |  |
| 49 | 25 |  | 50 | VI/SL | - |  | - |  | YL:YELLOW |  |
| 50 |  |  | 25 | SL/VI |  |  |  |  |  |  |

Table 3.5.6.6.c	EKSU P1 connector

Table 3.5.6.6.d. shows the contact assignments of P2 connector on EKSU.

| Punch-Down |  | P1 connector |  |  |  | Hardware |  | Card |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Block Pin | Pair |  | Pin | Color-Code | Designation |  | Slot No. |  |  |  |
| 1 | 1 |  | 26 | WH/BL | CO-73R |  | 13 |  |  |  |
| 2 |  |  | 1 | BL/WH | CO-73T |  |  |  |  |  |
| 3 | 2 |  | 27 | WH/OR | CO-74R |  |  |  |  |  |
| 4 |  |  | 2 | OR/WH | CO-74T |  |  |  |  |  |
| 5 | 3 |  | 28 | WH/GN | CO-75R |  |  |  |  |  |
| 6 |  |  | 3 | GN/WH | CO-75T |  |  |  |  |  |
| 7 | 4 |  | 29 | WH/BN | CO-76R |  |  |  |  |  |
| 8 |  |  | 4 | BN/WH | CO-76T |  |  |  |  |  |
| 9 | 5 |  | 30 | WH/SL | CO-77R |  |  |  |  |  |
| 10 |  |  | 5 | SL/WH | CO-77T |  |  |  |  |  |
| 11 | 6 |  | 31 | RD/BL | CO-78R |  |  |  |  |  |
| 12 |  |  | 6 | BL/RD | CO-78T |  |  |  |  |  |
| 13 | 7 |  | 32 | RD/OR | CO-79R |  | 12 |  |  |  |
| 14 |  |  | 7 | OR/RD | CO-79T |  |  |  |  |  |
| 15 | 8 |  | 33 | RD/GN | CO-80R |  |  |  |  |  |
| 16 |  |  | 8 | GN/RD | CO-80T |  |  |  |  |  |
| 17 | 9 |  | 34 | RD/BN | CO-81R |  |  |  |  |  |
| 18 |  |  | 9 | BN/RD | CO-81T |  |  |  |  |  |
| 19 | 10 |  | 35 | RD/SL | CO-82R |  |  |  |  |  |
| 20 |  |  | 10 | SL/RD | CO-82T |  |  |  |  |  |
| 21 | 11 |  | 36 | BK/BL | CO-83R |  |  |  |  |  |
| 22 |  |  | 11 | BL/BK | CO-83T |  |  |  |  |  |
| 23 | 12 |  | 37 | BK/OR | CO-84R |  |  |  |  |  |
| 24 |  |  | 12 | OR/BK | CO-84T |  |  |  |  |  |
| 25 | 13 |  | 38 | BK/GN | CO-85R |  | 11 |  |  |  |
| 26 |  |  | 13 | GN/BK | CO-85T |  |  |  |  |  |
| 27 | 14 |  | 39 | BK/BN | CO-86R |  |  |  |  |  |
| 28 |  |  | 14 | BN/BK | CO-86T |  |  |  |  |  |
| 29 | 15 |  | 40 | BK/SL | CO-87R |  |  |  |  |  |
| 30 |  |  | 15 | SL/BK | CO-87T |  |  |  |  |  |
| 31 | 16 |  | 41 | YL/BL | CO-88R |  |  |  |  |  |
| 32 |  |  | 16 | BL/YL | CO-88T |  |  |  |  |  |
| 33 | 17 |  | 42 | YL/OR | CO-89R |  |  |  |  |  |
| 34 |  |  | 17 | OR/YL | CO-89T |  |  |  |  |  |
| 35 | 18 |  | 43 | YL/GN | CO-90R |  |  |  |  |  |
| 36 |  |  | 18 | GN/YL | CO-90T |  |  |  |  |  |
| 37 | 19 |  | 44 | YL/BN | CO-91R |  | 10 |  |  |  |
| 38 |  |  | 19 | BN/YL | CO-91T |  |  |  |  |  |
| 39 | 20 |  | 45 | YL/SL | CO-92R |  |  |  |  |  |
| 40 |  |  | 20 | SL/YL | CO-92T |  |  |  | BL:BLUE |  |
| 41 | 21 |  | 46 | VI/BL | CO-93R |  |  |  | BK:BLACK |  |
| 42 |  |  | 21 | BL/VI | CO-93T |  |  |  | BN:BROWN |  |
| 43 | 22 |  | 47 | VI/OR | CO-94R |  |  |  | OR:ORANGE |  |
| 44 |  |  | 22 | OR/VI | CO-94T |  |  |  | WH:WHITE |  |
| 45 | 23 |  | 48 | VI/GN | CO-95R |  |  |  | GN:GREEN |  |
| 46 |  |  | 23 | GN/VI | CO-95T |  |  |  | SL: SLIVER |  |
| 47 | 24 |  | 49 | VI/BN | CO-96R |  |  |  | VI:VIOLET |  |
| 48 |  |  | 24 | BN/VI | CO-96T |  |  |  | RD:RED |  |
| 49 | 25 |  | 50 | VI/SL | - |  | - |  | YL:YELLOW |  |
| 50 |  |  | 25 | SL/VI |  |  |  |  |  |  |

Table 3.5.6.6.d	BKSU P2 connector

### ISDN wiring

ISDN boards such as BRIB/BRIU, STIB/STIU, and PRIB/PRIU contain RJ45 type jacks on each board, to connect to ISDN.

Following figure shows ISND reference point and Functional group,

ISDN

PX

(LT)

ISDN Network

or

PC

G4-Fax

S

V,X

Terminal

SLT

NT2

TA

ISDN-TEL

NT1

NT1

U

U

T

S

R

V

The position of GDK-System

Figure 3.5.7.aISDN Reference Points and Functional Group

Reference point: Theoretical point to classify the functional unit

- V-point: PX line

- U-point: Subscriber line

Subscriber line between public exchange and NT1 (Network termination), 2wire and 4-5Km distance

- T-point: ISDN Trunk line

Trunk line for PBX between NT1 and PBX system, 4wire and 1Km max distance.

- S-point: ISDN standard user interface

Standard point for ISDN terminal connection, physically same as T-point, multiple connection possible.

- R-point: X or V series terminal interface.

Reference point between Terminal adapter and non-ISDN terminal connection

Functional Units

- NT1: Network Termination

- NT2: Protocol Handle, PBX

- TE1: Standard ISDN interface TE

- TE2: Non-Standard ISDN interface TE

- TA:  Adapter for non-standard ISDN interface

ISDN trunk connection (Basic Rate)

Max. 1000 meters

GDK System

ISDN Board

BRIB

STIB(T)

NT1

ISDN

TR

TR

Figure 3.5.7.bISDN Trunk connection (Basic Rate Interface: T)

1. Point to Point connection

2. TR: Termination resistor, 100 ohm

ISDN Terminal Connection (Basic Rate)

P to P Connection

TR

TE

TR

GDK-SYSTEM

TR

STIB(S)

BRIB

STIB(T)

TR

TR

Max. 1000 meters

TE1

TE2

TE8

Max. 200 meters

TR

TR

TE1

TE2

TE8

Max. 500 meters

Max. 50 meters

P to MP Connection

- Short Passive

P to MP Connection

- Extended Passive

Figure 3.5.7.cISDN Terminal connection (Basic Rate Interface: S)

Connection Type

P to P: Point to Point connection

P to MP: Point to Multi-Point connection

TR: Termination resistor, 100 ohm

This may be contained in a TE among multi-TEs.

ISDN Trunk connection (Primary Rate)

Max. 1000 meters

GDK System

ISDN Board

PRIB

DSU

ISDN

Figure 3.5.7.dISDN Trunk connection (Primary Rate Interface)

Point to Point connection only

Without power feed.

Line Impedance: 120 ohm

Repeater

Physical Connectors

Basic Rate User network Interface

Figure 3.5.7.eRJ-45 Type Jack

1

2

3

4

5

6

7

8

ISO 8877

ITU-T Rec. I.430

Contact

1

2

3

4

5

6

7

8

TE

Power Source 3

Power Source 3

Transmit

Receive

Receive

Transmit

Power Source 2

Power Source 2

NT

Power Source 3

Power Source 3

Receive

Transmit

Transmit

Receive

Power Source 2

Power Source 2

Polarity

+

-

+

+

-

-

-

+

Table 3.5.7.aContact assignments

Primary Rate User network Interface (E1/T1)

Figure 3.5.7.fRJ-45 Type Jack

1

2

3

4

5

6

7

8

ISO 10173

ANSI T1.408

Contact

1

2

3

4

5

6

7

8

TE

Receive

Receive

Shield(R) *

Transmit

Transmit

Shield(T) *

Power Source *

Power Source *

NT

Transmit

Transmit

Shield(T) *

Receive

Receive

Shield(R) *

Power Source *

Power Source *

Polarity

-

+

Table 3.5.7.bContact assignments

* Optional

### BRIB/BRIU wiring

BRIB is capable of connecting to 4 Basic Rate Trunk lines through RJ45 type jacks. BRIU provides further 4 Basic Rate Trunk interfaces.

The contact assignments of RJ45 type jack on BRIB/BRIU show in figure 3.5.7.1.a.

Figure 3.5.7.1.aContact assignments of RJ-45 type jack on BRIB

1

2

3

4

5

6

7

8

RX+

RX-

TX-

TX+

The TX pair of RJ45 type jack on BRIB/BRIU is connected to the RX pair of NT1, and the RX one on BRIB/BRIU is to the TX one of NT1 as Figure 3.5.7.1.b.

Terminating resistors should be required on each line to match line impedance. See clause 3.5.7.4.

NT1 (NT-Box)

BRIB/BRIU

RJ45 type jack

TX+

TX-

RX+

RX-

3

6

4

5

3

6

4

5

RX+

RX-

TX+

TX-

From/to PX

Note:

Connect the TX pair of RJ45 type jack on BRIB/BRIU to the RX pair of NT1 and vice versa.

Check terminating resistors. See clause 3.5.7.4.

BRIB/BRIU are capable of connecting to up to 8 lines (4 lines /BRIB and 4 lines /BRIU).

Figure 3.5.7.1.bBRI trunk line wiring

### STIB/STIU wiring

STIB is capable of connecting to 4 Basic Rate Trunk lines or Terminals through RJ45 type jacks. And STIU provides further 4 Basic Rate Trunk/Terminal interfaces. Each line is switchable by hardware and software setting. See the clause 3.4.6.2.

The contact assignments of RJ45 type jack on STIB/STIU show in figure 3.5.7.2.a.

Figure 3.5.7.2.aContact assignments of RJ-45 type jack on STIB/STIU

1

2

3

4

5

6

7

8

TX+

TX-

RX-

RX+

1

2

3

4

5

6

7

8

RX+

RX-

TX-

TX+

a. When a line is set to T interface

b. When a line is set to S interface

The TX pair of RJ45 type jack on STIB/STIU is connected to the RX pair of NT1 or Terminal, and the RX one on STIB/STIU is to the TX one of NT1 or terminal as Figure 3.5.7.2.b.

Terminating resistors should be required on each line to match line impedance. See clause 3.5.7.4.

Note:

Connect the TX pair of RJ45 type jack on STIB/STIU to the RX pair of NT1 and vice versa. And connect the TX pair of RJ45 type jack on STIB/STIU to the RX pair of ISDN terminal and vice versa.

Check terminating resistors. See clause 3.5.7.4.

SWx means selectable switches of S or T interface on each interface of STIB/STIU, see clause 3.4.6.2.

STIB/STIU are capable of connecting to up to 8 lines/terminals. ( 4 /STIB, 4 /STIU)

Figure 3.5.7.2.bBRI trunk line (T) or terminal connection (S) wiring

NT1 (NT-Box)

STIB/STIU

RJ45 type jack

TX+

TX-

RX+

RX-

3

6

4

5

3

6

4

5

RX+

RX-

TX+

TX-

From/to PX

ISDN Terminal

RX+

RX-

TX+

TX-

3

6

4

5

3

6

4

5

TX+

TX-

RX+

RX-

S

T

SWx

S

T

SWx

OR

T interface

S interface

### PRIB/PRIU wiring

PRIB is capable of connecting 1 Primary Rate Trunk lines through an RJ45 type jack. And PRIU provides further 1 Primary Rate Trunk interface.

The contact assignments of RJ45 type jack on PRIB/PRIU show in figure 3.5.7.3.a.

Figure 3.5.7.3.aContact assignments of RJ-45 type jack on PRIB/PRIU

1

2

3

4

5

6

7

8

RX

TX

GND (T)

GND (R)

RX

TX

Note: Only two pairs of RX and TX are typically connected with RRI line.

The TX pair of RJ45 type jack on PRIB/PRIU is connected to the RX pair of DSU, and the RX one on PRIB is to the TX one of DSU as Figure 3.5.7.3.b.

DSU (NT-Box)

PRIB/PRIU (TE mode)

RJ45 type jack

RX

RX

TX

TX

1

2

4

5

1

2

4

5

TX

TX

RX

RX

From/to PX

Note:

- Connect the TX pair of RJ45 type jack on PRIB/PRIU to the RX pair of DSU and vice versa.

- PRIB/PRIU are capable of connecting to up to 2 PRI lines (1 line /PRIB, 1 line /PRIU).

Figure 3.5.7.3.b	PRI trunk line (TE connection) wiring

GDK-162 PRIB/PRIU is typically used as TE connecting to ISDN PX through DSU. But the PRIB/PRIU can be used as NT in special purpose such as QSIG function with another GDK PRIB/PRIU directly. However a type of contact assignments of RJ45 type jack is provided for TE connection on GDK-162 PRIB/PRIU, that is, the contact assignments of RJ45 type jack on PRIB or PRIU are not changed even though the board is set to NT mode. See Table 3.5.7.b and Figure 3.5.7.3. So it is important that the wiring with contact assignments of RJ45 type connector should be checked, when PRIB or PRIU is set to master mode (NT) during installation. The TX pair of RJ45 connector on PRIB or PRIU in master mode is connected to the RX pair of PRIB of another system and vice versa as following figure 3.5.7.3.c.

Another GDK system

PRIB/PRIU (TE mode)

GDK-162 PRIB or PRIU

(NT mode)

RJ45 type jack

RX

RX

TX

TX

1

2

4

5

1

2

4

5

RX

RX

TX

TX

Figure 3.5.7.3.c	PRI line (NT connection) wiring with GDK system PRIBs

Note:

- When PRIB and/or PRIU is used in NT mode, the hardware settings are required.

See the clause 3.4.6.3 PRIB/PRIU Installation; Function of Connectors.

### Terminating Resistors on Basic Rate interface(BRIB/STIB)

Terminating resistors are required on each line of BRIB/STIB for the proper operation. Switchable terminating resistors are introduced on BRIB/BRIU issue 2 or later version and STIB/STIU issue 2 or later version. So switches on each line of the board should be ON to terminate them when installing the line.

Note:

R is 100 ohm terminating resistor, and the line is terminated at 100 ohm when switches are ON.

SWx means terminating switches on each interface of BRIB/STIB. See clause 3.4.6.1 and 3.4.6.2.

Figure 3.5.7.4.aTerminating method of BRIB/STIB with switchable terminating resistors

BRIB/U issue 2 or later

STIB/U issue 2 or later

RJ45 type jack

TX+

TX-

RX+

RX-

3

6

4

5

RX+

RX-

TX+

TX-

3

6

4

5

OR

T interface

: BRIB and

STIB (T)

S interface

: STIB (S)

SWx

R

R

ON

SWx

R

R

ON

From/To

BRI Line

From/To

ISDN Terminal

But terminating resistors are not introduced on BRIB/BRIU issue 1 and STIB/STIU issue 1, which versions are officially production stopped. So the external terminating unit should be required to terminate them BRIB/BRIU issue 1 and STIB/STIU issue 1. The external terminating unit can be locally got, otherwise 2923NP1656, which is the terminating unit provided by LGIC, can be used.

Figure 3.5.7.4.bTerminating method of BRIB/STIB without switchable terminating resistors

Note: R is 100 ohm terminating resistors.

BRIB/BRIU issue 1

STIB/STIU issue 1

RJ45 type jack

TX+

TX-

RX+

RX-

3

6

4

5

RX+

RX-

TX+

TX-

3

6

4

5

OR

T interface

: BRIB and

STIB (T)

S interface

: STIB (S)

From/To

BRI Line

From/To

ISDN Terminal

R

R

R

R

External terminating Unit

### Clock Control Connection- Multi ISDN board installation

GDK-162 system allows Multi-ISDN boards to be installed in Basic KSU. When Multi-ISDN board installation, the coaxial cable should be connected and the shorting trap (short pin) be set to proper position according to the priority of the cards. Then, in case that the ISDN line of the board with the higher priority is in the state of Loss of signal or clock failed, the synchronized reference clock for system will be supplied from next daisy chained ISDN board. Every ISDN card in TE mode generates the reference clock synchronized with ISDN line, and the card with the highest priority generates the system clock and transmits it to MPB.

Reference clock:

The reference clock is the clock source of PLL circuit to generate the system clock. There are three kinds of reference clocks on each ISDN board.

One is the extracted clock synchronized with ISDN line, which is generated when ISDN line is in normal operation.

Another is the internal oscillation clock, which is used as a clock source of PLL circuit when all the ISDN lines are in the state of Loss OF Signal or clock failed.

The third is the reference clock from next daisy chained ISDN board. This is used as a clock source of PLL circuit when all the ISDN lines of board itself are in the state of Loss OF Signal or clock failed, but it is possible to receive the reference clock from the next ISDN board with lower priority.

PLL circuit:

This circuit generates the system clock and transmits it to MPB for system operation. It is located on each ISDN board.

INT/EXT selection:

This option is used to select the reference clock of PLL circuit in the state of Loss of signal or clock failed.

If INT(FREE) position is selected by setting a shorting trap (short pin), the internal oscillation clock will be supplied to the input of PLL circuit. This position is selected on the board when an ISDN board is installed in a system, or on the ISDN board with the lowest priority when multi-ISDN boards are installed in a system.

If EXT(FREE) position is selected by setting a shorting trap (short pin), the reference clock from the next daisy chained ISDN board will be supplied to the input of PLL circuit. This position is selected on all the ISDN boards except the board with the lowest priority when multi-ISDN boards are installed in a system.

Coaxial cable connection

Coaxial cables are used to transmit/receive the system clock between an ISDN board and MPB, and the reference clock between ISDN boards when Multi-ISDN boards are installed in a system.

The system clock connection: 32MHz

The coaxial cable is connected between MPB and the ISDN board with the highest priority. If an ISDN board is only installed, it is connected between MPB and the board.

The reference clock connection:

The coaxial cables are connected between ISDN boards. The cable should be the coaxial connector, named IN, of the ISDN board with the higher priority and the coaxial connector, named OUT, of another ISDN board with the lower priority.

|  | BRIB | STIB | PRIB | DCOB |
| --- | --- | --- | --- | --- |
| The connector for the system clock output | CON12 | CN8 | CONN8 | CON3 |
| The connector for the reference clock input | CON10 | CN7 | - |  |
| The connector for the reference clock output | CON11 | CN6 | - |  |

- Note:

- In GDK-162 system, the multi-ISDN boards including PRIB can be installed in a system, but the daisy chained connection between PRIB and BRIB/STIB is impossible in this case because of the difference of the reference clock frequencies.

BRIB,STIB – 2MHz,  PRIB,DCOB –1.5MHz

MPB

BRIB

(the highest priority)

IN

CONN7

(CON7)

BRI line

EX

CONN6

(CON6)

INT

EXT

CON9

CON10

CON11

CON12

BRIB

INT

EXT

CON9

STIB

(T interface(s))

(the lowest priority)

INT

EXT

CN5

CON10

CON11

CON12

CN7

CN6

CN8

1

2

2

TE

S

T

Examples of Multi-ISDN board installation

Figure 3.5.7.5.a	Example of multi-ISDN board installation when a BRIB has the highest priority.

Figure 3.5.7.5.a	Example of multi-ISDN board installation when a BRIB has the highest priority.

Note:

- The cable connection to feed the system clock into MPB.

- The cable connection to receive the reference clock from the ISDN board with the lower priority when all the lines of the ISDN board are in the state of Loss of Signal or clock failed.

3.	The coaxial cables for the clock connections are supplied with each ISDN board.

STIB

(T interface(s))

(the lowest priority)

INT

EXT

CN5

CN7

CN6

CN8

S

T

STIB

(T interfaces)

(the highest priority)

INT

EXT

CN5

CN7

CN6

CN8

T

MPB

IN

CONN7

(CON7)

BRI line

EX

CONN6

(CON6)

1

2

TE

STIB

(All S interfaces)

INT

EXT

CN5

CN7

CN6

CN8

S

S

TE

TE

Note:

- The cable connection to feed the system clock into MPB.

- The cable connection to receive the reference clock from the ISDN board with the lower priority when all the lines of the ISDN board are in the state of Loss Of Signal or clock failed.

- The coaxial cables for the clock connections are supplied with each ISDN board.

Figure 3.5.7.5.b	Example of multi-ISDN board installation when a STIB has the highest priority  and all the interfaces of another STIB are set to S interfaces.

BRIB

INT

EXT

CN5

CN7

CN6

CN8

PRIB

(TE mode)

(the highest priority)

EXT

INT

CONN5

CN7

CN6

CN8

MPB

IN

CONN7

(CON7)

BRI line

EX

CONN6

(CON6)

1

TE

STIB

( T interface(s))

INT

EXT

CN5

CN7

CN6

CN8

S

T

PRI line

Figure 3.5.7.5.a	Example of multi-ISDN board installation when PRIB is the highest priority.

Figure 3.5.7.5.c	Example of multi-ISDN board installation when a PRIB has the highest priority.

Note:

- The cable connection to feed the system clock into MPB.

- The cable connection to receive the reference clock from the ISDN board with the lower priority is not required because of the difference of the reference clock frequency of PRIB.

3.	It is recommended to set PRIB(TE mode) to the highest priority in case of the multi ISDN boards installation including PRIB.

### QSIG connection

GDK-162 system supports QSIG function using ISDN boards. There are two kinds of connection for supporting QSIG function. One is the direct connection between GDK systems with PRI line, the other is the connection through Public Network with PRI or BRI lines.

For QSIG function, the SMEMU should be required.

Direct connection

Only GDK PRI boards can be used for the direct connection.

For more details, see the manual for GDK186/100/FPII Q-signaling Interface.

GDK system

GDK system

PRI (NT)

PRI (TE)

Public Network

Note:

- The wiring of RJ45 type jack, the mode selection switches, and the clock control connectors on GDK-162 PRIB should be checked if GDK-162 PRIB/PRIU is set to NT/TE mode respectively.

See clause 3.5.7.3 PRIB wiring and Figure 3.5.7.3.c.

Figure 3.5.7.6.a		Direct connection for QSIG feature

GDK system

GDK -162

PRI (NT)

PRI (TE)

Public Network

GDK system

PRI (NT)

PRI (TE)

Figure 3.5.7.6.b		Direct connection of GDK-162 PRIB/PRIU for QSIG feature

Note:

- GDK-162 PRIB/PRIU can be set to NT/TE mode respectively. So the cascade connection using GDK-162 PRIB/PRIU is possible for QSIG feature.

- The wiring of RJ45 type jack, the mode selection switches, and the clock control connectors on GDK-162 PRIB should be checked if GDK-162 PRIB/PRIU is set to NT/TE mode respectively. See clause 3.5.7.3 PRIB wiring and Figure 3.5.7.3.c.

The connection through Public Network

GDK PRIB/BRIB/STIB can be used for this connection.

For more details, see the manual for GDK186/100/FPII Q-signaling Interface.

GDK system

GDK system

Public ISDN

GDK system

PRI /BR I(TE)

PRI /BRI(TE)

PRI /BRI(TE)

Figure 3.5.7.6.c  	The connection through the Public Network for QSIG feature

### EMI Suppression

Installation of EMI core on line(s) of STIB is required to suppress EMI.

Install two EMI cores supplied with STIB/STIU and BRIB/BRIU as following figure 3.5.7.7.a.

When the core is installed on 1 or 2 lines, it may be slip down. In that case, winding wire with a string such as cable tie is recommended.

GDK-162 BKSU

Line x 4

STIB/STIU

BRIB/BRIU

within 30 Cm

within 30 Cm

Figure 3.5.7.7.a	Ferrite Core installation to suppress EMI

Install a EMI core supplied with PRIB/PRIU as following figure 3.5.7.7.b.

GDK-162 BKSU

PRI Line

Figure 3.5.7.7.bFerrite Core installation to suppress EMI

PRIB/PRIU

within 30 Cm

1.5 turns winding

### Contact Assignments of various types of connectors

The contact assignments of various types of connectors are described in this clause.

### RJ21 type connectors

The contact assignments of RJ21 type (25-pair Amphenol) connectors

25

50

1

26

2

27

3

28

Figure 3.5.8.1Contact assignments of RJ21 type connectors

a. Female connector

b. male connector

25

50

3

28

2

27

1

26

### Miniature 6-position jack

The contact assignments of miniature 6-position jack registered in FCC.

Figure 3.5.8.2Contact assignments of miniature 6-position jack

T1

R2

R1

T2

1

2

3

4

5

6

### Miniature 8-position jack for ISDN connection

The contact assignments of miniature 8-position jack used for ISDN connection.

a. Basic Rate User network Interface

Figure 3.5.8.3.aRJ-45 type jack

1

2

3

4

5

6

7

8

ISO 8877

ITU-T Rec. I.430

Contact

1

2

3

4

5

6

7

8

TE

Power Source 3

Power Source 3

Transmit

Receive

Receive

Transmit

Power Source 2

Power Source 2

NT

Power Source 3

Power Source 3

Receive

Transmit

Transmit

Receive

Power Source 2

Power Source 2

Polarity

+

-

+

+

-

-

-

+

Table 3.5.8.3.aContact assignments

b. Primary Rate User network Interface (E1/T1)

Figure 3.5.8.3.bRJ-45 type jack

1

2

3

4

5

6

7

8

ISO 10173

ANSI T1.408

Contact

1

2

3

4

5

6

7

8

TE

Receive

Receive

Shield(R) *

Transmit

Transmit

Shield(T) *

Power Source *

Power Source *

NT

Transmit

Transmit

Shield(T) *

Receive

Receive

Shield(R) *

Power Source *

Power Source *

Polarity

-

+

Table 3.5.8.3.bContact assignments

* Optional