### SYSTEM WIRING

### Battery Back-Up Wiring

The system can be equipped with external batteries for proper operation when commercial AC power is failed. The back-up batteries are connected to the connector on the front of the PSU as shown in Figure 3.5.1.

Cable ass’y Part No. : 6851NX0145A

Insert Figure 3.5.1 in the existing manual here

Figure 3.5.1  Battery Back-up Wiring

The external batteries should provide 24 Volts DC. This is generally accomplished by connecting two 12volts batteries in a series arrangement. Battery operation is controlled by the PSU. The PSU will provide charging current(maximum 0.5 amp) to the batteries during normal AC power operation. The PSU will automatically stop the battery operation when AC power is re-applied or low battery voltage is detected.

Carefully check the battery polarity with cable colors(RED and BLUE) when connecting the battery to system. It is recommended to use a fuse(20A @250V) between battery and system.

The system operating time by external batteries is depending on several elements, battery charge state, condition of the batteries, capacity of the batteries, and the system configuration (number of station ports).

Recommended battery capacity is 24V/20AH MF. The system will operate more than 5 hours with this battery.

### RS 232C WIRING on SIU

The SIU has one industry standard 9-pin RS-232C connector. The RS-232C ports are connected by 9-pin “D” connectors as shown 3.5.2. Note that the communication settings are 9600bps, 8 bits data, no parity bit and 1 stop bit.

The SIU(1) position of the MBU provides all the serial communication control signals such as TXD, RXD, CTS, RTS, DSR, DTR and ground. But the SIU(2) position does not provide all these signals and only provides three(3) control signals such as TXD, RXD and ground. For the reason, in case of using two SIUs, you have to consider this situation and connect carefully for proper operation.

2

3

4

6

7

8

5

TD

RD

DSR

DTR

CTS

RTS

SG

RD

TD

DTR

DSR

RTS

CTS

SG

2

3

4

6

7

8

5

3

2

20

6

4

5

7

TD

RD

DSR

DTR

CTS

RTS

SG

RD

TD

DTR

DSR

RTS

CTS

SG

2

3

4

6

7

8

5

DB-9 connector

for RS-232C

on Terminal

DB-9 connector

for the RS-232C

of SIU on SIU(1)

DB-9 connector

for the RS-232C

of SIU on SIU(2)

DB-25 connector

for RS-232C

on Terminal

a)	SIU wiring on the SIU(1) position of MBU

b)	SIU wiring on the SIU(2) position of MBU

2

2

TD

RD

2

3

TD

RD

3

3

RD

TD

3

2

RD

TD

4

5

5

SG

SG

6

6

SG

SG

DB-9 connector

for RS-232C

on Terminal

DB-9 connector

for the RS-232C

of SIU on SIU(1)

DB-9 connector

for the RS-232C

of SIU on SIU(2)

DB-25 connector

for RS-232C

on Terminal

Figure 3.5.2	Pin assignments and wiring of DB25 and DB9 for RS-232C

Name	Function

TD	Transmitted Data

RD	Received Data

RTS	Request To Send

CTS	Clear To Send

DTR	Data Terminal Ready

DSR	Data Set Ready

SG	Signal Ground

- Note:

- Protective Ground (Frame Ground) may be used between DTE and DCE(GDK system) for more stable communication. And it is recommended.

In DTEs and DCEs, protective ground (frame ground) is a point which is electrically bonded to the equipment frame. Protective ground (frame ground) is not an interchange circuit in RS232 standard.(EIA/TIA-232-F) If bonding of the equipment frames of the DCE and the DTE is necessary, a separate conductor should be used which confirms to the appropriate national or local electrical codes.

### MBU wiring

There are three(3) RJ21 type connectors(CN14 ~ CN16) and 2 RCA jacks on MBU for the external line or devices. There are three(3) RJ21 type connectors(CN14 ~ CN16) in the left most side of the MBU. These connectors are used to connect all ports of the system except ETIB and ISDN line/terminal, such as CO lines, electronic keysets, digital keysets and single line telephones. So, station wiring is performed by these connectors. The method of RJ21 type cable connection is shown in Figure 3.5.3.

The middle one (CN15) is assigned to CO line connection only, and signal lines of this connector are distributed to slot #3 to slot #7. Refer to [CO Wiring] section.

Insert Figure 3.5.3 in the existing manual here

The cable with RJ21 type male connector is required to connect with terminal or external devices. It is possible to get the cable locally, or use GDK-162 5MC2 supplied by LGIC on the order if necessary.

Figure 3.5.3  Connection Method of RJ21 type connector

And 2 RCA jacks are used to connect the external music source and the external paging device.

You should install a Ferrite Core on the Audio Cable with 2.5 turns in KSU to minimize EMI, when you connect the Audio Cables for MOH(Music On Hold) and/or EXT. PAGE to audio jack(s) on the MBU.

### External Music Source wiring

The GDK-FPII system sends internal or external music signal to CO lines during hold state. The default music source input is assigned to internal source.

And the external music source can be used by changing the system database and connect a external music source to the blue colored audio jack(PJ2) of the MBU.

The level can be adjusted by the administration programming (Program 66) in 0.5 dB steps to ±20 dB of the input. Note, if the output of the source is too high or low, it will be necessary to lower or raise the output level control at the source.

Above External Music sources including a Internal Music Source on MPB are selectable by Admin Programming. See Programming Manual.

### External Paging wiring

One external paging output port is provided from the red colored audio jack(PJ1) of the MBU. The port is used to support the external paging equipment. See Figure 3.5.3.2.

Insert Figure 3.5.10 in the existing manual here

Figure 3.5.3.2	External paging wiring

### External Paging Port and General Purpose Relays wiring

One external paging output port is provided from the red colored audio jack(PJ1) of the MBU. The port is used to page outside through external amplifier as a system extension.

And there are two general purpose relays in the MBU and these contacts are connected through CHAMP connector(CN14) of the MBU. Detailed pin assignment of these contacts are shown in Table 3.5.3.6. The dry relay contact can be used to power on/off function for external amplifier. The control of these relays are done by system programming.

For example, the following Figure 3.5.3.3 shows how to connect external amplifier with relay and external paging port.

External

Amplifier

Ext. paging output

(PJ1 of the MBU)

Pin #1 of CN14

Pin #26 of CN14

RJ21 type connector (CN14)

Figure 3.5.3.3	Example of External Paging Wiring

### Relay contacts for general purpose

GDK-FPII supports up to 2 Relay contacts, which can be used for the functions of Loud Control, Door Open and so on under the software control. These relay contacts are respectively connected with external equipment through 2 pairs of a RJ21 type connector, CN14 on the board.

RLY1: Pin No. 1 and Pin No. 26

RLY2: Pin No. 3 and Pin No. 28

External DC

Power Supply

MDF

GDK-FPII MBU

RJ21 type connector, CN14

1

26

Figure 3.5.3.4An example of Loud Bell wiring

### Alarm Detection wiring

GDK-FPII MBU provides an external alarm detection input, which can be used to notify to extensions when the external switch is on/off. This alarm detection input is provided through a pair of a RJ21 type connector, CN14 on the board.

ALARM: Pin No. 12 and Pin No. 37

Close or open detection is programmable by Admin. Programming.

MDF

GDK-FPII MBU

RJ21 type connector, CN14

12

37

Figure 3.5.3.5An example of External Switch wiring

External Switch

### Contact Assignments of RJ21 type connector for Basic Connection

The upper one(CN14) and lower one(CN16) are assigned to system’s extension line such as digital keysets and single line telephones. Basically the upper most connector(CN14) is used to connect basic system resources and basic digital extensions. The upper one’s usage is listed as follows and detailed pin mapping table is shown in Table 3.5.3.6.

The usage of CN14 in the left upper side of MBU is as followings,

- 6 ports of digital terminals on the MBU: See the Digital Keyset and Terminal wiring in this section.

- 4 ports of SLTs of the 4SLI board on the slot #2: See the Single Line Telephone wiring in this section.

- 2 ports of general purpose relay dry contacts

- 1 alarm detect port

- 1 external PFTU control port; See the PFTU wiring in this section.

Table 3.5.3.6 shows the contact assignments of RJ21 type female connector (CN14) on MBU for Basic Connection.

| Punch-Down | MBU Connector CN14 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Block Pin | Pair | Pin | Color-Code | Designation |  |  |
| 1 | 1 | 26 | WH/BL | RELAY-R1 |  |  |
| 2 |  | 1 | BL/WH | RELAY-T1 |  |  |
| 3 | 2 | 27 | WH/OR |  |  |  |
| 4 |  | 2 | OR/WH |  |  |  |
| 5 | 3 | 28 | WH/GN | RELAY-R2 |  |  |
| 6 |  | 3 | GN/WH | RELAY-T2 |  |  |
| 7 | 4 | 29 | WH/BN |  |  |  |
| 8 |  | 4 | BN/WH |  |  |  |
| 9 | 5 | 30 | WH/SL |  |  |  |
| 10 |  | 5 | SL/WH |  |  |  |
| 11 | 6 | 31 | RD/BL |  |  |  |
| 12 |  | 6 | BL/RD |  |  |  |
| 13 | 7 | 32 | RD/OR | PFT-VO |  |  |
| 14 |  | 7 | OR/RD | PFT-CTL |  |  |
| 15 | 8 | 33 | RD/GN |  |  |  |
| 16 |  | 8 | GN/RD |  |  |  |
| 17 | 9 | 34 | RD/BN |  |  |  |
| 18 |  | 9 | BN/RD |  |  |  |
| 19 | 10 | 35 | RD/SL |  |  |  |
| 20 |  | 10 | SL/RD |  |  |  |
| 21 | 11 | 36 | BK/BL |  |  |  |
| 22 |  | 11 | BL/BK |  |  |  |
| 23 | 12 | 37 | BK/OR | ALARM-T |  |  |
| 24 |  | 12 | OR/BK | ALARM-R |  |  |
| 25 | 13 | 38 | BK/GN |  |  |  |
| 26 |  | 13 | GN/BK |  |  |  |
| 27 | 14 | 39 | BK/BN |  |  |  |
| 28 |  | 14 | BN/BK |  |  |  |
| 29 | 15 | 40 | BK/SL |  |  |  |
| 30 |  | 15 | SL/BK |  |  |  |
| 31 | 16 | 41 | YL/BL | VR-4 |  |  |
| 32 |  | 16 | BL/YL | VT-4 |  |  |
| 33 | 17 | 42 | YL/OR | VR-3 |  |  |
| 34 |  | 17 | OR/YL | VT-3 | SLT |  |
| 35 | 18 | 43 | YL/GN | VR-2 | Port |  |
| 36 |  | 18 | GN/YL | VT-2 |  |  |
| 37 | 19 | 44 | YL/BN | VR-1 |  |  |
| 38 |  | 19 | BN/YL | VT-1 |  |  |
| 39 | 20 | 45 | YL/SL | DR-6 |  |  |
| 40 |  | 20 | SL/YL | DT-6 |  | BL:BLUE |
| 41 | 21 | 46 | VI/BL | DR-5 |  | BK:BLACK |
| 42 |  | 21 | BL/VI | DT-5 |  | BN:BROWN |
| 43 | 22 | 47 | VI/OR | DR-4 |  | OR:ORANGE |
| 44 |  | 22 | OR/VI | DT-4 | Digital | WH:WHITE |
| 45 | 23 | 48 | VI/GN | DR-3 | Port | GN:GREEN |
| 46 |  | 23 | GN/VI | DT-3 |  | SL:SLIVER |
| 47 | 24 | 49 | VI/BN | DR-2 |  | VI:VIOLET |
| 48 |  | 24 | BN/VI | DT-2 |  | RD:RED |
| 49 | 25 | 50 | VI/SL | DR-1 |  | YL:YELLOW |
| 50 |  | 25 | SL/VI | DT-1 |  |  |

Table 3.5.3.6  Basic Connector (CN14) Wiring

### Extension Board wiring

Extension boards consists of DTIB, ETIB, 4SLI, SLIB, and WTIB. DTIB and SLIB are wired to the terminals through CN16, which is the RJ21 type connector on MBU for wiring the extensions to the extension boards installed in the universal slots(Slot No. 4~7). ETIB contains RJ11 type jack for wiring the electronic terminal to each interface circuit on the board itself. But WTIB and WTIU, Wireless Terminal Interface Board/Unit for DECT contains RJ11 type jacks.

But 4SLI is wired to the SLT device through CN14, which is the RJ21 type connector on MBU for Basic Connection. See the Table 3.5.3.6.

The following provides details on the interconnection between each type of Extension Board and the jack on the station.

### Digital Keyset and Terminal wiring

Wiring from digital terminal interface board such as DTIB and Digital Interfaces on MBU to station jack requires one pair of wire. The power for a station, PCM data and signaling between board and station are transmitted over this pair.

Contact assignments of RJ11 type jack in Digital Terminals

Figure 3.5.4.1.a shows the contact assignments of the miniature 6-position jack (RJ11 type jack) in LG proprietary digital terminals. Two wires from the digital terminal interface of the system are basically connected to the contact numbers 3 (R) and 4(T).

But two additional contacts, R1 and T1 are provided in the digital terminal as shown in Figure 3.5.4.1. The contact number 3 (R) and 2 (R1) are connected together in the digital terminals, and also the contact number 4 (T) and 5 (T1) are connected together. Because there is a country in which the contact number 2 and 5 are used for the 2-wire connection method according to his local regulation. So LGIC provide two more contacts, R1 and T1 in the digital terminals so that the special customer can use the line cord, which is in wide use in his country.

But only 2-wire connection should be recommended to get better transmission characteristics between digital terminal interface of the system and digital terminals.

Figure 3.5.4.1.acontact assignments of Digital Terminals

T

T1

R

R1

1

2

3

4

5

6

Connection method between the digital interfaces and the digital terminals

The cable with 25-pair male connector is required between Digital Terminal Interface board and MDF. It is possible to get the cable locally, or use GDK-162 5MC2 supplied by LGIC on the order if necessary.

The MDF that is in wide use in the country can be locally used according to the requirements of local regulation.

Note:

Digital terminals operate regardless of line polarity, not with a fixed polarity.

Each area means the followings,

A: GDK system, B: between the GDK system and MDF, C: cable in building,

D: between Terminal connection point, e.g. terminating adapter and Terminal,

E: digital terminals

DTIB is capable of connecting to up to 6 digital terminals through CN14 on MBU.

MBU is capable of connecting to up to 6 digital terminals through CN14 on MBU..

Figure 3.5.4.1.bBasic connection method

between digital terminal interface of the system and digital terminal

MDF

1

2

3

4

5

6

GDK-FPII DTIB/MBU(DTI)

RJ21 type

connector

on the MBU

25

50

24

49

23

48

22

47

DT1

DR1

DT2

DR2

DT3

DR3

Area A

Area B

Area C

Area D

Area E

Connection method with special condition between the digital interfaces and the digital terminals

Only two-wire connection between the digital terminal interface and digital terminals is recommended to reduce the line noise exited from useless wires. Because R and R1/T and T1 are connected together in the terminals for various national conditions, the useless wires can cause the environment noise, which may affect the data transmission, to flow in the digital terminal the 4-wire line cord is used.

In this case, cut or remove the useless wires connected the contact number R1 and T1 (or, R and T) of the terminals at the Terminal connection point, e.g. terminating adapter, so that those are not drawn out any more.

Area A

Area B

Area C

Area D

Area E

Note:

Each area means the followings,

A: GDK system, B: between the GDK system and MDF, C: cable in building,

D: between Terminal connection point, e.g. terminating adapter and Terminal,

E: digital terminals

Only two wires should be connected to digital terminals and Interface Board, so the rest should be removed at MDF or Terminal Connection Point, as X points shown in above figure.

Figure 2.5.4.1.cConnection method when the 4-wire line cord, or

special connection method with with contact number 2 and 5.

MDF

1

2

3

4

5

6

GDK-FPII DTIB/MBU(DTI)

RJ21 type

connector

on the MBU

25

50

24

49

23

48

22

47

DT1

DR1

DT2

DR2

DT3

DR3

1

2

3

4

5

6

Pin Assignments of RJ21 type jack on DTIB, MBU(DTI)

See the clause 3.5.3.6 Contact Assignments of RJ21 type connector(CN14) on MBU for Basic Connector wiring.

See the clause 3.5.4.6 Contact Assignments of RJ21 type connector (CN16) on MBU for DTIB wiring

### Electronic Keyset and Terminal wiring

Contact assignments of RJ11 type jack in ETIB and Eletronic Terminals

Wiring from electronic terminal interface board to station jack requires two pairs of wires. The first pair is for the audible analog signal including voice, and the second one is for signaling and data communication. The pair for data and signaling is polarity sensitive, so electronic terminal will not properly operate when reversed. However the reversal will not harm terminal or system.

Power is delivered by applying DC voltage to the center tap of the coupling transformers of each of 2 pairs.

Figure 3.5.4.2.a shows contact assignments of RJ11 type jack on ETIB and LG proprietary electronic terminals.

Figure 3.5.4.2.aContact assignments of Electronic Terminals

VT

DT

VR

DR

1

2

3

4

5

6

VR

DR

VT

DT

1

2

3

4

5

6

ETIB

Electronic Terminal

Connection method between the ETIB and the Electronic terminals

The cable with 25-pair male connector is required between Electronic Terminal Interface board and MDF. It is possible to get the cable locally, or use GDK-162 5MC2 supplied by LGIC on the order if necessary.

Area A

Area B

Area C

Area D

Area E

1

2

3

4

5

6

GDK-FPII ETIB

RJ11 type jack

on the ETIB

RJ11 type jack

on terminal

VT1

6

5

4

3

2

1

VR1

DR1

DT1

The MDF that is in wide use in the country can be locally used according to the requirements of local regulation.

Figure 3.5.4.2.bwiring between board and electronic terminal

Note:

Each area means the followings,

A: GDK system, B: between the GDK system and MDF, C: cable in building,

D: between Terminal connection point, e.g. terminating adapter and Terminal,

E: Electronic terminals

A pair for voice signal can be connected regardless of the polarity of its Tip and Ring. But the other for signaling and data transmission should be connected to be suitable for their polarity as above figure.

ETIB is capable of connecting with up to 6 electronic terminals.

### Single line Telephone wiring

SLT interface boards such as SLIB24, SLIB48, SLIB II and DSIB are wired to SLT or standard analog device with a single pair of wire that provides battery feed, voice and signaling from/to the connected SLT. Typical contact assignments of RJ11 modular connector, a miniature 6-position jack as following figure 3.5.4.3.a. But the contact assignments and connector type may be different from following figure in special countries according to the special national condition. Before wiring, it is important to check them.

R

T

Figure 3.5.4.3.acontact assignments of SLT

1

2

3

4

5

6

Registered Jack

specified in FCC, CFR

The cable with 25-pair male connector is required between SLT Interface board and MDF. It is possible to get the cable locally, or use GDK-162 5MC2 supplied by LGIC on the order if necessary.

MDF can be locally used according to the requirements of local regulation and/or an usual usage in the field.

MDF

1

2

3

4

5

6

25

50

24

49

23

48

22

47

T1

R1

T2

R2

T3

R3

4SLI, SLIB

Amphenol connector on board

RJ11 socket

on SLT

Figure 3.5.4.3.b	wiring between board and analog TE including SLT

Note:

- SLT (TE) typically operates with both polarities, since a fixed polarity is not guaranteed according to most of specification.

- 4SLI is capable of connecting to up to 6 SLT through CN14 on MBU.

SLIB is capable of connecting to up to 12 SLT through CN16 on MBU.

### Intercom/Door Phone Box installation

The Intercom/Door Phone Box is intended to be wall mounted. There are two kinds of Intercom/Door Phone Boxes in GDK-FPII system available. KD-Digital Phone Box is can be connected to the digital terminal interface, and KD-ICM Box is can be connected to the electronic terminal interface (ETIB). But another GK or GSX phone box connected to LG Analog Key Telephone System can not be connected to ETIB.

At first, separate the ICM/Door Box from the base plate by inserting a flat thin edged screw driver in the bottom of the product. Press the screw driver inward which will release the retaining tan and twist the screw driver to separate the base plate. Fasten the base plate of the ICM/Door Box to the wall with # 8 screws. Route the cable through the cable entry hole in the base plate and connect the wires to the four screw terminals as shown in the figures 3.5.4.4.a and 3.5.4.4.b. When the wiring is complete, pull any slack wire back through the base plate and push the housing back together.

Connection method between the Digital Interface Board and KD-Digital Phone Box

KD-Digital Phone Box can be only connected to the Digital Interface Board. Wiring from digital terminal interface board to KD-Digital Phone Box requires one pair of wire.

Note:  KD-Digital Phone Box operates regardless of line polarity, not with a fixed polarity

MDF

GDK-FPII DTIB/MBU(DTI)

RJ21 type connector

on the MBU

25

50

24

49

23

48

22

47

DT1

DR1

DT2

DR2

DT3

DR3

The soldering

side of

KD-Digital Phone Box

GR

RD

Figure 3.5.4.4.awiring between the board and KD-Digital Phone Box

Connection method between the ETIB and KD-ICM Box

KD-ICM Box can be only connected to ETIB. Wiring from electronic terminal interface board to KD-ICM Box requires two pairs of wires.

The soldering

side of

KD-ICM Box

GR

BK

YL

RD

GDK-FPII ETIB

RJ11 type jack

on the ETIB

VT1

6

5

4

3

2

1

VR1

DR1

DT1

Figure 3.5.4.4.b	wiring between the board and KD-ICM Box

Note:

- A pair for voice signal can be connected regardless of the polarity of Tip and Ring. But the other for signaling and data transmission should be connected with a fixed polarity as above figure.

- The GSX or GK Phone Box that can be connected to the LG Analog Key Telephone System, should not be connected to the GDK-FPII ETIB.

### Contact Assignments of RJ21 Type Connectors for Expansion Connector

Basically the lower most connector (CN16) is used to connect extensions with option board which is installed in universal slots(slot #4~7). So, any kind of extensions(digital keysets or SLTs) can be connected to this connect. It depends on what kind of interface boards are installed.

The lower one’s detailed pin mapping is shown in Table 3.5.4.5.

| Punch-Down | MBU connector CN16 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Block Pin | Pair | Pin | Color-Code | Designation |  |
| 1 | 1 | 26 | WH/BL |  |  |
| 2 |  | 1 | BL/WH |  |  |
| 3 | 2 | 27 | WH/OR | slot #7 - R6 |  |
| 4 |  | 2 | OR/WH | slot #7 - T6 |  |
| 5 | 3 | 28 | WH/GN | slot #7 - R5 |  |
| 6 |  | 3 | GN/WH | slot #7 - T5 |  |
| 7 | 4 | 29 | WH/BN | slot #7 - R4 |  |
| 8 |  | 4 | BN/WH | slot #7 - T4 |  |
| 9 | 5 | 30 | WH/SL | slot #7 - R3 |  |
| 10 |  | 5 | SL/WH | slot #7 - T3 |  |
| 11 | 6 | 31 | RD/BL | slot #7 - R2 |  |
| 12 |  | 6 | BL/RD | slot #7 - T2 |  |
| 13 | 7 | 32 | RD/OR | slot #7 - R1 |  |
| 14 |  | 7 | OR/RD | slot #7 - T1 |  |
| 15 | 8 | 33 | RD/GN | slot #6 - R6 |  |
| 16 |  | 8 | GN/RD | slot #6 - T6 |  |
| 17 | 9 | 34 | RD/BN | slot #6 - R5 |  |
| 18 |  | 9 | BN/RD | slot #6 - T5 |  |
| 19 | 10 | 35 | RD/SL | slot #6 - R4 |  |
| 20 |  | 10 | SL/RD | slot #6 - T4 |  |
| 21 | 11 | 36 | BK/BL | slot #6 - R3 |  |
| 22 |  | 11 | BL/BK | slot #6 - T3 |  |
| 23 | 12 | 37 | BK/OR | slot #6 - R2 |  |
| 24 |  | 12 | OR/BK | slot #6 - T2 |  |
| 25 | 13 | 38 | BK/GN | slot #6 - R1 |  |
| 26 |  | 13 | GN/BK | slot #6 - T1 |  |
| 27 | 14 | 39 | BK/BN | slot #5 - R6 |  |
| 28 |  | 14 | BN/BK | slot #5 - T6 |  |
| 29 | 15 | 40 | BK/SL | slot #5 - R5 |  |
| 30 |  | 15 | SL/BK | slot #5 - T5 |  |
| 31 | 16 | 41 | YL/BL | slot #5 - R4 |  |
| 32 |  | 16 | BL/YL | slot #5 - T4 |  |
| 33 | 17 | 42 | YL/OR | slot #5 - R3 |  |
| 34 |  | 17 | OR/YL | slot #5 - T3 |  |
| 35 | 18 | 43 | YL/GN | slot #5 - R2 |  |
| 36 |  | 18 | GN/YL | slot #5 - T2 |  |
| 37 | 19 | 44 | YL/BN | slot #5 - R1 |  |
| 38 |  | 19 | BN/YL | slot #5 - T1 |  |
| 39 | 20 | 45 | YL/SL | slot #4 - R6 |  |
| 40 |  | 20 | SL/YL | slot #4 - T6 | BL:BLUE |
| 41 | 21 | 46 | VI/BL | slot #4 - R5 | BK:BLACK |
| 42 |  | 21 | BL/VI | slot #4 - T5 | BN:BROWN |
| 43 | 22 | 47 | VI/OR | slot #4 - R4 | OR:ORANGE |
| 44 |  | 22 | OR/VI | slot #4 - T4 | WH:WHITE |
| 45 | 23 | 48 | VI/GN | slot #4 - R3 | GN:GREEN |
| 46 |  | 23 | GN/VI | slot #4 - T3 | SL:SLIVER |
| 47 | 24 | 49 | VI/BN | slot #4 - R2 | VI:VIOLET |
| 48 |  | 24 | BN/VI | slot #4 - T2 | RD:RED |
| 49 | 25 | 50 | VI/SL | slot #4 - R1 | YL:YELLOW |
| 50 |  | 25 | SL/VI | slot #4 - T1 |  |

Table 3.5.4.5  EXP. Connector(CN16) Wiring

### PFTU Wiring

The PFTU is wired from the RJ21 type male connector on the modue itself to the MDF, cross connected to the CO Line inputs from the telephone company, to the CO Line inputs of the system, and to Power Failure SLTs. The CO lines and SLTs are used as CO line inputs and extensions of GDK-FPII System in normal operation, but each CO line and SLT is directly connected in power failure mode by connecting as shown in figure 3.5.5

The cable with RJ21 type female connector is required between PFTU connector and MDF. It is possible to get the cable locally, or use GDK-162 5MC1 supplied by LGIC on the order if necessary.

The MDF that is in wide use in your country can also be locally used according to the requirements of local regulation.

MDF

1

26

4

29

SLIB1-T

SLIB1-R

PFTU

25

50

COT1-IN

COR1-IN

2

27

COT1-OUT

COR1-OUT

3

28

TEL1-T

TEL1-R

SLIB interface

CO line

LCOB interface

SLT

RJ21 type connector

in PFTU

Connected to

another PFTU

Note:

- A connector block with screws, which is connected to MBU, is supplied in the PFTU package, but control wires are not supplied. It is recommended that 2 wires with AWG22, 24 or 26 be used for the connection of control signal.

- The control wires can be connected regardless of their polarities .

- The control wires can be connected to CN41 on MBU with a connector block or the contact No. 7 and 37 of CN14 on MBU without a connector block. See PFTU installation in section 2.

Figure 3.5.5	PFTU wiring

Connected to MBU

(Note 3)

Table 3.5.5. shows the contact assignments of RJ21 type connector on PFTU.

| Punch-Down | PFTU connector |  |  | PFTU | PFT |  |
| --- | --- | --- | --- | --- | --- | --- |
| Block Pin | Pair | Pin | Color-Code | Designation | Circuit |  |
| 1 | 1 | 26 | WH/BL | COT1-IN | 1st Circuit |  |
| 2 |  | 1 | BL/WH | COR1-IN |  |  |
| 3 | 2 | 27 | WH/OR | COT1-OUT |  |  |
| 4 |  | 2 | OR/WH | COR1-OUT |  |  |
| 5 | 3 | 28 | WH/GN | TEL1-T |  |  |
| 6 |  | 3 | GN/WH | TEL1-R |  |  |
| 7 | 4 | 29 | WH/BN | SLIB1-T |  |  |
| 8 |  | 4 | BN/WH | SLIB1-R |  |  |
| 9 | 5 | 30 | WH/SL | COT2-IN | 2nd Circuit |  |
| 10 |  | 5 | SL/WH | COR2-IN |  |  |
| 11 | 6 | 31 | RD/BL | COT2-OUT |  |  |
| 12 |  | 6 | BL/RD | COR2-OUT |  |  |
| 13 | 7 | 32 | RD/OR | TEL2-T |  |  |
| 14 |  | 7 | OR/RD | TEL2-R |  |  |
| 15 | 8 | 33 | RD/GN | SLIB2-T |  |  |
| 16 |  | 8 | GN/RD | SLIB2-R |  |  |
| 17 | 9 | 34 | RD/BN | COT3-IN | 3rd Circuit |  |
| 18 |  | 9 | BN/RD | COR3-IN |  |  |
| 19 | 10 | 35 | RD/SL | COT3-OUT |  |  |
| 20 |  | 10 | SL/RD | COR3-OUT |  |  |
| 21 | 11 | 36 | BK/BL | TEL3-T |  |  |
| 22 |  | 11 | BL/BK | TEL3-R |  |  |
| 23 | 12 | 37 | BK/OR | SLIB3-T |  |  |
| 24 |  | 12 | OR/BK | SLIB3-R |  |  |
| 25 | 13 | 38 | BK/GN | COT4-IN | 4th Circuit |  |
| 26 |  | 13 | GN/BK | COR4-IN |  |  |
| 27 | 14 | 39 | BK/BN | COT4-OUT |  |  |
| 28 |  | 14 | BN/BK | COR4-OUT |  |  |
| 29 | 15 | 40 | BK/SL | TEL4-T |  |  |
| 30 |  | 15 | SL/BK | TEL4-R |  |  |
| 31 | 16 | 41 | YL/BL | SLIB4-T |  |  |
| 32 |  | 16 | BL/YL | SLIB4-R |  |  |
| 33 | 17 | 42 | YL/OR | COT5-IN | 5th Circuit |  |
| 34 |  | 17 | OR/YL | COR5-IN |  |  |
| 35 | 18 | 43 | YL/GN | COT5-OUT |  |  |
| 36 |  | 18 | GN/YL | COR5-OUT |  |  |
| 37 | 19 | 44 | YL/BN | TEL5-T |  |  |
| 38 |  | 19 | BN/YL | TEL5-R |  |  |
| 39 | 20 | 45 | YL/SL | SLIB5-T |  |  |
| 40 |  | 20 | SL/YL | SLIB5-R |  | BL:BLUE |
| 41 | 21 | 46 | VI/BL | COT6-IN | 6th Circuit | BK:BLACK |
| 42 |  | 21 | BL/VI | COR6-IN |  | BN:BROWN |
| 43 | 22 | 47 | VI/OR | COT6-OUT |  | OR:ORANGE |
| 44 |  | 22 | OR/VI | COR6-OUT |  | WH:WHITE |
| 45 | 23 | 48 | VI/GN | TEL6-T |  | GN:GREEN |
| 46 |  | 23 | GN/VI | TEL6-R |  | SL: SLIVER |
| 47 | 24 | 49 | VI/BN | SLIB6-T |  | VI:VIOLET |
| 48 |  | 24 | BN/VI | SLIB6-R |  | RD:RED |
| 49 | 25 | 50 | VI/SL | - | - | YL:YELLOW |
| 50 |  | 25 | SL/VI | - |  |  |

Table 3.5.5	PFTU Wiring

### Analog CO Line wiring

Analog CO boards such as 4LCO, LCOB, AC15, BWDID, TLIB are wired to the CO lines from the telephone company through CN15, which is a RJ21 type male connector on MBU. Table 3.5.6.5 gives the contact assignments of an RJ21 type male connector, CN15. If using the AC15 or TLIB, the contact assignments of this connector are different to normal CO lines. See AC15 wiring and TLIB wiring in this section; sub-clauses.

The following provides details on the connection of each type of analog CO line Board.

The cable with RJ21 type female connector is required between CN15 connector and MDF. It is possible to get the cable locally, or use GDK-162 5MC1 supplied by LGIC on the order if necessary.

The MDF or terminating points that is in wide use in your country can be locally used according to the requirements of local regulation.

### LCOB/4LCO wiring

4LCO, LCOB are respectively capable of connecting 4 loop start CO lines through an RJ21 type male connector on the MBU.

The following Figure 3.5.6.1 is an example connection in case that 4LCO is installed in slot 3. The relation between the contact assignments of CN15 on MBU and the slots is given in Tables 3.5.6.5.

MDF

1

26

4

29

R4

T4

CN15 on MBU

RJ21 type connector on the MBU

25

50

R1

T1

2

27

R2

T2

3

28

R3

T3

Network

Note:

CO lines of 4LCO/LCOB can operate regardless of line polarity.

Connect TRC (Telecommunication reference cable) to 25th pair of RJ21 type connector if it is supplied from Network.

Both 4LCO/LCOB are respectively capable of connecting to up to 4 lines.

Figure 3.5.6.1loop start CO line wiring

### AC15 Board wiring -UK

AC15 is capable of connecting 2 Private lines through an RJ21 type male connector on the MBU..

The following Figure 3.5.6.2 is an example connection in case that the AC15 board is installed in slot 4. The relation between the contact assignments of CN15 on MBU and the slots is given in Tables 3.5.6.5.

MDF

5

30

8

33

4R (TXR2)

4T (TXT2)

CN15 on MBU

25

50

1R (RXR1)

1T (RXT1)

6

31

2R (TXR1)

2T (TXT1)

7

32

3R (RXR2)

3T (RXT2)

Network

TXR

TXT

RXR

RXT

Note:

- Above figure is an example connection in case that AC15 board is installed in slot No. 4.

- A pair of a line is for RX signal, and the other is for TX signal. So, 2 pairs of wires are required for the connection of 1 line.

- Each pair can be connected regardless of the polarity of Tip and Ring.

- AC15 board is capable of connecting to up to 2 lines.

Figure 3.5.6.2	AC15 Private line wiring

| MBU connector CN15 (for slot #4~7) |  | Direct mapping to AC15 line | Remark |
| --- | --- | --- | --- |
| SLOT #N - CO1-T |  | RX-T1 |  |
| SLOT #N - CO1-R |  | RX-R1 |  |
| SLOT #N - CO2-T |  | TX-T1 | Port 1 |
| SLOT #N - CO2-R |  | TX-R1 |  |
| SLOT #N - CO3-T |  | RX-T2 |  |
| SLOT #N - CO3-R |  | RX-R2 |  |
| SLOT #N - CO4-T |  | TX-T2 | Port 2 |
| SLOT #N - CO4-R |  | TX-R2 |  |

Table 3.5.6.2  Direct Mapping Table for AC15

### BWDIDB wiring -New Zealand

BWDIDB is capable of connecting 4 both-way DID lines through an RJ21 type male connector on the MBU.

The following Figure 3.5.6.3 is an example connection in case that the BWDIDB is installed in slot 4. The relation between the contact assignments of CN15 on MBU and the slots is given in Tables 3.5.6.5.

MDF

5

30

8

33

R4

T4

CN15 on MBU

25

50

R1

T1

6

31

R2

T2

7

32

R3

T3

Network

T

R

Note:

Above figure is an example connection in case that BWDIDB is installed in slot # 8.

BWDIDB is polarity-sensitive, so it is important to check the polarity of wires when wiring. In idle state, –48V is fed on Ring wire, and Reference is on Tip wire in GDK-FPII system.

BWDIDB board is capable of connecting with up to 4 lines.

Figure 3.5.6.3Both-way DID line wiring

| MBU connector CN15 (for slot #4~7) |  | Direct mapping to both-way DID line | Remark |
| --- | --- | --- | --- |
| SLOT #N - CO1-T |  | T1 | Port 1 |
| SLOT #N - CO1-R |  | R1 |  |
| SLOT #N - CO2-T |  | T2 | Port 2 |
| SLOT #N - CO2-R |  | R2 |  |
| SLOT #N - CO3-T |  | T3 | Port 3 |
| SLOT #N - CO3-R |  | R3 |  |
| SLOT #N - CO4-T |  | T4 | Port 4 |
| SLOT #N - CO4-R |  | R4 |  |

Table 3.5.6.3  Direct Mapping Table for BWDIDB

### TLIB wiring - Korea

TLIB is capable of connecting 2 E&M lines through a RJ21 type male connector on the MBU.

The following Figure 3.5.6.4 is an example connection in case that the TLIB is installed in slot 4. The relation between the contact assignments of CN15 on MBU and the slots is given in Tables 3.5.6.5.

MDF

5

30

8

33

R4 (M2)

T4 (E2)

CN15 on MBU

25

50

R1 (R1)

T1 (T1)

6

31

R2 (M1)

T2 (E1)

7

32

R3 (R2)

T3 (T2)

Network

R

T

M

E

Note:

A pair of wire is for audio signal, and this can be connected regardless of line polarity of Tip and Ring.

The other is for signaling, and this is polarity-sensitive. EAR wire of EMIB is connected to MOUTH wire of Network/PBX and vice versa.

It is important to check EAR and MOUTH of Network before wiring, because some network equipment may describe ‘ opposite marks ‘.

TLIB board is capable of connecting with up to 4 lines.

Figure 3.5.6.4wiring E&M tie line

| MBU connector CN15 (for slot #4~7) |  | Direct mapping to TIE line | Remark |
| --- | --- | --- | --- |
| SLOT #N - CO1-T |  | VT - 1 |  |
| SLOT #N - CO1-R |  | VR - 1 |  |
| SLOT #N - CO2-T |  | E-1 | Port 1 |
| SLOT #N - CO2-R |  | M-1 |  |
| SLOT #N - CO3-T |  | VT - 2 |  |
| SLOT #N - CO3-R |  | VR - 2 |  |
| SLOT #N - CO4-T |  | E-2 | Port 2 |
| SLOT #N - CO4-R |  | M-2 |  |

Table 3.5.6.4 Direct Mapping Table for TLIB

### Contact Assignments of RJ21 type Connectors on Analog CO Boards.

Table 3.5.6.5 shows the contact assignments of RJ21 type connector(CN15) on MBU.

| Punch-Down | MBU Connector CN15 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Block Pin | Pair | Pin | Color-Code | Designation |  |
| 1 | 1 | 26 | WH/BL | CO1-R |  |
| 2 |  | 1 | BL/WH | CO1-T |  |
| 3 | 2 | 27 | WH/OR | CO2-R |  |
| 4 |  | 2 | OR/WH | CO2-T | Slot #3 |
| 5 | 3 | 28 | WH/GN | CO3-R |  |
| 6 |  | 3 | GN/WH | CO3-T |  |
| 7 | 4 | 29 | WH/BN | CO4-R |  |
| 8 |  | 4 | BN/WH | CO4-T |  |
| 9 | 5 | 30 | WH/SL | CO5-R |  |
| 10 |  | 5 | SL/WH | CO5-T |  |
| 11 | 6 | 31 | RD/BL | CO6-R |  |
| 12 |  | 6 | BL/RD | CO6-T | Slot #4 |
| 13 | 7 | 32 | RD/OR | CO7-R |  |
| 14 |  | 7 | OR/RD | CO7-T |  |
| 15 | 8 | 33 | RD/GN | CO8-R |  |
| 16 |  | 8 | GN/RD | CO8-T |  |
| 17 | 9 | 34 | RD/BN | CO9-R |  |
| 18 |  | 9 | BN/RD | CO9-T |  |
| 19 | 10 | 35 | RD/SL | CO10-R |  |
| 20 |  | 10 | SL/RD | CO10-T | Slot #5 |
| 21 | 11 | 36 | BK/BL | CO11-R |  |
| 22 |  | 11 | BL/BK | CO11-T |  |
| 23 | 12 | 37 | BK/OR | CO12-R |  |
| 24 |  | 12 | OR/BK | CO12-T |  |
| 25 | 13 | 38 | BK/GN | CO13-R |  |
| 26 |  | 13 | GN/BK | CO13-T |  |
| 27 | 14 | 39 | BK/BN | CO14-R |  |
| 28 |  | 14 | BN/BK | CO14-T | Slot #6 |
| 29 | 15 | 40 | BK/SL | CO15-R |  |
| 30 |  | 15 | SL/BK | CO15-T |  |
| 31 | 16 | 41 | YL/BL | CO16-R |  |
| 32 |  | 16 | BL/YL | CO16-T |  |
| 33 | 17 | 42 | YL/OR | CO17-R |  |
| 34 |  | 17 | OR/YL | CO17-T |  |
| 35 | 18 | 43 | YL/GN | CO18-R |  |
| 36 |  | 18 | GN/YL | CO18-T | Slot #7 |
| 37 | 19 | 44 | YL/BN | CO19-R |  |
| 38 |  | 19 | BN/YL | CO19-T |  |
| 39 | 20 | 45 | YL/SL | CO20-R |  |
| 40 |  | 20 | SL/YL | CO20-T |  |
| 41 | 21 | 46 | VI/BL | - | BL:BLUE |
| 42 |  | 21 | BL/VI | - | BK:BLACK |
| 43 | 22 | 47 | VI/OR | - | BN:BROWN |
| 44 |  | 22 | OR/VI | - | OR:ORANGE |
| 45 | 23 | 48 | VI/GN | - | WH:WHITE |
| 46 |  | 23 | GN/VI | - | GN:GREEN |
| 47 | 24 | 49 | VI/BN | - | SL:SLIVER |
| 48 |  | 24 | BN/VI | - | VI:VIOLET |
| 49 | 25 | 50 | VI/SL | - | RD:RED |
| 50 |  | 25 | SL/VI | - | YL:YELLOW |

Table 3.5.6.5  CO Connector (CN15) Wiring

### ISDN wiring

ISDN boards such as 2BRI, STIB and PRI contain RJ45 type jacks on each board, to connect to ISDN.

Following figure shows ISND reference point and Functional group,

ISDN

PX

(LT)

ISDN Network

or

PC

G4-Fax

S

V,X

Terminal

SLT

NT2

TA

ISDN-TEL

NT1

NT1

U

U

T

S

R

V

The position of GDK-System

Figure 3.5.7.aISDN Reference Points and Functional Group

Reference point: Theoretical point to classify the functional unit

- V-point: PX line

- U-point: Subscriber line

Subscriber line between public exchange and NT1 (Network termination), 2wire and 4-5Km distance

- T-point: ISDN Trunk line

Trunk line for PBX between NT1 and PBX system, 4wire and 1Km max distance.

- S-point: ISDN standard user interface

Standard point for ISDN terminal connection, physically same as T-point, multiple connection possible.

- R-point: X or V series terminal interface.

Reference point between Terminal adapter and non-ISDN terminal connection

Functional Units

- NT1: Network Termination

- NT2: Protocol Handle, PBX

- TE1: Standard ISDN interface TE

- TE2: Non-Standard ISDN interface TE

- TA:  Adapter for non-standard ISDN interface

ISDN trunk connection (Basic Rate)

Max. 1000 meters

GDK- FPII

ISDN Board

2BRI

STIB(T)

NT1

ISDN

TR

TR

Figure 3.5.7.bISDN Trunk connection (Basic Rate Interface: T)

1. Point to Point connection

2. TR: Termination resistor, 100 ohm

ISDN Terminal Connection(Basic Rate)

TR

TE

TR

GDK-SYSTEM

TR

STIB(S)

2BRI

STIB(T)

TR

TR

Max. 1000 meters

TE1

TE2

TE8

Max. 200 meters

TR

TR

TE1

TE2

TE8

Max. 500 meters

Max. 50 meters

P to P Connection

P to MP Connection

- Short Passive

P to MP Connection

- Extended Passive

Figure 3.5.7.cISDN Terminal connection (Basic Rate Interface: S)

Connection Type

P to P: Point to Point connection

P to MP: Point to Multi-Point connection

TR: Termination resistor, 100 ohm

This may be contained in a TE among multi-TEs.

ISDN Trunk connection (Primary Rate)

Max. 1000 meters

GDK System

ISDN Board

PRI

DSU

ISDN

Figure 3.5.7.dISDN Trunk connection (Primary Rate Interface)

Point to Point connection only

Without power feed.

Line Impedance: 120 ohm

Repeater

Physical Connectors

Basic Rate User network Interface

Figure 3.5.7.eRJ-45 Type Jack

1

2

3

4

5

6

7

8

ISO 8877

ITU-T Rec. I.430

Contact

1

2

3

4

5

6

7

8

TE

Power Source 3

Power Source 3

Transmit

Receive

Receive

Transmit

Power Source 2

Power Source 2

NT

Power Source 3

Power Source 3

Receive

Transmit

Transmit

Receive

Power Source 2

Power Source 2

Polarity

+

-

+

+

-

-

-

+

Table 3.5.7.a	Contact assignments

Primary Rate User network Interface (E1/T1)

Figure 3.5.7.fRJ-45 Type Jack

1

2

3

4

5

6

7

8

ISO 10173

ANSI T1.408

Contact

1

2

3

4

5

6

7

8

TE

Receive

Receive

Shield(R) *

Transmit

Transmit

Shield(T) *

Power Source *

Power Source *

NT

Transmit

Transmit

Shield(T) *

Receive

Receive

Shield(R) *

Power Source *

Power Source *

Polarity

-

+

* Optional

Table 3.5.7.b	Contact assignments

### ISDN 2BRI wiring

BRIB is capable of connecting 2 Basic Rate Trunk lines through RJ45 type jacks.

The contact assignments of RJ45 type jack on BRIB show in figure 3.5.7.1.a.

1

2

3

4

5

6

7

8

RX+

RX-

TX-

TX+

Figure 3.5.7.1.a	contact assignments of RJ-45 type jack of 2BRI board

The TX pair of RJ45 type jack on BRIB is connected to the RX pair of NT1, and the RX one on BRIB is to the TX one of NT1 as Figure 3.5.7.1.b.

Terminating resistors should be required on each line to match line impedance. See clause 3.5.7.4.

NT1 (NT-Box)

2BRI board

RJ45 type jack

TX+

TX-

RX+

RX-

3

6

4

5

3

6

4

5

RX+

RX-

TX+

TX-

From/to PX

Note:

Connect the TX pair of an RJ45 type jack on BRIB to the RX pair of NT1 and vice versa.

Check terminating resistors. See clause 3.5.7.4.

2BRI is capable of connecting to up to 2 lines.

Figure 3.5.7.1.bBRI trunk line wiring

### ISDN STIB wiring

STIB is capable of connecting to 2 Basic Rate Trunk lines or Terminals through RJ45 type jacks. Each line is switchable by hardware and software setting. (see clause 3.4.6.2)

The contact assignments of RJ45 type jack on STIB show in figure 3.5.7.2.a.

Figure 3.5.7.2.acontact assignments of RJ-45 Connectors of STIB

1

2

3

4

5

6

7

8

TX+

TX-

RX-

RX+

1

2

3

4

5

6

7

8

RX+

RX-

TX-

TX+

a. When a line is set to T interface

a. When a line is set to S interface

The TX pair of RJ45 type jack on STIB/STIU is connected to the RX pair of NT1 or Terminal, and the RX one on STIB/STIU is to the TX one of NT1 or terminal as Figure 3.5.7.2.b.

Terminating resistors should be required on each line to match line impedance. See clause 3.5.7.4.

NT1 (NT-Box)

STIB

RJ45 type jack

TX+

TX-

RX+

RX-

3

6

4

5

3

6

4

5

RX+

RX-

TX+

TX-

From/to PX

ISDN Terminal

RX+

RX-

TX+

TX-

3

6

4

5

3

6

4

5

TX+

TX-

RX+

RX-

S

T

S

T

OR

T interface

S interface

Figure 3.5.7.2.b	BRI trunk line (T) or terminal connection (S) wiring

Note:

- Connect the TX pair of RJ45 type jack on STIB to the RX pair of NT1 and vice versa. And

connect the TX pair of RJ45 type jack on STIB to an RX pair of ISDN terminal and vice versa.

- Check terminating resistors. See clause 3.5.7.4.

- STIB is capable of connecting to up to 2 lines/terminals.

### ISDN PRI wiring

PRI board is capable of connecting to 1 Primary Rate Trunk lines through an RJ45 type jack.

The contact assignments of RJ45 type jack on PRI board show in figure 3.5.7.3.a.

Figure 3.5.7.3.aContact assignments of RJ-45 type jack of PRI board

1

2

3

4

5

6

7

8

RX

TX

GND (T)

GND (R)

RX

TX

Note: Only two pairs of RX and TX are typically connected with RRI line.

The TX pair of RJ45 type jack on PRI board is connected to the RX pair of DSU, and the RX one on PRI board is to the TX one of DSU as Figure 3.5.7.3.b.

Figure 3.5.7.3.b	PRI trunk line (TE connection) wiring

DSU (NT-Box)

PRI board

RJ45 type jack

RX

RX

TX

TX

1

2

4

5

1

2

4

5

TX

TX

RX

RX

From/to PX

Note:

- Connect the TX pair of RJ45 type jack on PRI board to the RX pair of DSU and vice versa.

- PRI board is capable of connecting with 1 PRI line.

GDK-FPII PRI board is only used as TE. So the PRI board can not be used as TE even though used for QSIG function with another GDK PRIBs directly.

The contact assignments of RJ45 type jack on GDK-162/100 PRIB are not changed even though the board is set to NT mode. See Table 3.5.7.b and Figure 3.5.7.3. So it is important that the wiring with contact assignments of RJ45 type connector should be checked, when PRIB of another system is set to master mode (NT) during installation. The TX pair of RJ45 connector on PRIB in master mode is connected to the RX pair of PRIB of another system and vice versa as following figure 3.5.7.3.c.

GDK-FPII

PRI (TE mode only)

GDK-162/100 PRIB/PRIU

(NT mode)

RJ45 connector

RX

RX-

TX

TX

1

2

4

5

1

2

4

5

RX

RX

TX

TX

Figure 3.5.7.3.c	PRI line wiring with GDK system PRIBs for QSIG function

### Terminating Resistors on Basic Rate interface(2BRI/STIB)

Terminating resistors are required on each line of BRIB/STIB for the proper operation. Switchable terminating resistors are introduced on 2BRIB issue 2 or later version and all STIB version. So the contacts of connectors on each line of the board should be short at ON position to terminate them when installing line.

2BRI/STIB

RJ45 type jack

TX+

TX-

RX+

RX-

3

6

4

5

RX+

RX-

TX+

TX-

3

6

4

5

OR

T interface

: BRIB and

STIB (T)

S interface

: STIB (S)

R

R

R

R

From/To

BRI Line

From/To

ISDN Terminal

TER. RES

ON

OFF

TER. RES

ON

OFF

Note:

R is 100 ohm terminating resistor, and the line is terminated at 100 ohm when switches are ON.

Terminating connectors and shorting traps on each interface of 2BRI/STIB. See clause 3.4.5.1 and 3.4.5.2.

Figure 3.5.7.4.aTerminating method of BRIB/STIB with switchable terminating resistors

But terminating resistors are not introduced on 2BRI board issue 1, which is very old version and not supplied now. So the external terminating unit should be required to terminate if 2BRI issue1. The external terminating unit can be locally got, otherwise 2923NP1656, which is the terminating unit provided by LGIC, can be used.

Note:

R is 100 ohm terminating resistors.

2BRI issue 1 is very old version of 2BRI. Now LGIC stopped to produce 2BRI issue1.

2BRI issue 1

RJ45 type jack

TX+

TX-

RX+

RX-

3

6

4

5

T interface

From/To

BRI Line

R

R

External terminating Unit

Figure 3.5.7.4.b	Terminating method of BRIB/STIB without switchable terminating resistors

### Clock Control Cable wiring- Multi ISDN board installation

GDK-FPII system allows Multi-ISDN boards to be installed. When Multi-ISDN board installation, clock control cable should be connected according to the priority of the cards. Then, in case that the ISDN line of the board with the higher priority is in the state of Loss of signal or clock failed, the synchronized clock for system will be supplied from next daisy chained ISDN board. Every ISDN card in TE mode generates the reference clock synchronized with ISDN line and transmits the clock to PLLU2 on MBU according to the clock control signal.

|  | High(1) state | Low(0) state |
| --- | --- | --- |
| IN | Enable the reference clock circuit | Disable the reference clock out circuit |
| OUT | Enable next clock circuit | Disable next clock circuit |

MBU

PRI

(Slot No.3 only)

STIB

(T interface)

STIB

(T interface(s))

PLLU2

OUT

IN

<

<

OUT

IN

OUT

IN

BRI Line

TE

S

T

PRI Line

1

Priority

of Boards

Figure 3.5.7.5.a	Example of multi-ISDN board installation when PRI is the highest priority.

● Note:

- The Clock control cables in the STIB package should be connected between ISDN boards according to the priority of them when multi-ISDN board installation.

Note:

1. The Clock control cables in the STIB package should be connected between ISDN boards according to the priority of them when multi-ISDN board installation.

MBU

2BRI

(slot No.3 only)

(the highest priority)

STIB

(T interface)

STIB

(T interface(s))

PLLU2

OUT

IN

<

<

OUT

IN

OUT

IN

BRI Line

TE

S

T

1

Priority

of Boards

Figure 3.5.7.5.b	Example of multi-ISDN board installation when a 2BRI has the highest priority.

1

Note:

- The Clock control cables in the STIB package should be connected between ISDN boards according to the priority of them when multi-ISDN board installation.

- If all the lines of ISDN board are set to NT mode (S interface), clock control cable should be not connected to the board.

MBU

STIB

(T interface(s))

(the highest priority)

STIB

(T interface(s))

STIB

(all S interfaces)

OUT

IN

<

OUT

IN

OUT

IN

BRI Line

TE

S

TE

S

<

PLLU2

Priority

of Boards

Figure 3.5.7.5.c	Example of multi-ISDN board installation when a STIB is the highest priority and all the interface of another STIB are set to S interfaces.

Cabling Examples

For ISDN line connection, it is done by using the RJ45 type connector on the ISDN interface cards (2BRI, PRI, STIB), not the CHAMP connector on the MBU. So Figure 3.5.7.5 gives an example of the arranging the cables from ISDN cards.

Insert Figure 3.5.8 in the existing manual here

Figure 3.5.7.5.d  the arrangement of ISDN cables

Various Examples of Clock control cable wiring

Insert Figure 3.5.9-a) in the existing manual here

Insert Figure 3.5.9-b) in the existing manual here

a)  2BRI or PRI card is installed on slot 3 and one or more STIB is(are) installed on universal slot(s).

- 2BRI or PRI card is installed on slot 3, and one or more STIB is(are) installed

on slot 1 and universal slot(s).

Insert Figure 3.5.9 -c) in the existing manual here

c)  STIBs are installed on universal slots.

Figure 3.5.7.5.e  Clock Control Cable Wiring

### QSIG connection

GDK-FPII system supports QSIG function using ISDN boards. There are two kinds of connection for supporting QSIG function. One is the direct connection between GDK systems with PRI line, the other is the connection through Public Network with PRI or BRI lines.

For QSIG function, the SMEMU should be required.

Direct connection

Only GDK PRI boards can be used for the direct connection.

For more details, see the manual for GDK186/100/FPII Q-signaling Interface.

GDK system

GDK system

PRI (NT)

PRI (TE)

Public Network

Note:

- GDK-FPII PRI board could not be set to master (NT) mode.

Figure 3.5.7.6.a		The direct connection for QSIG feature

The connection through Public Network

GDK-PRIB/BRIB/STIB can be used for this connection.

For more details, see the manual for GDK186/100/FPII Q-signaling Interface.

GDK system

GDK system

Public Network

GDK system

PRI /BRI(TE)

PRI /BRI(TE)

PRI /BRI(TE)

Figure 3.5.7.6.b 	The connection through the Public Network for QSIG feature

### Contact Assignments of various types of connectors

The contact assignments of various types of connectors are described in this clause.

### RJ21 type connectors

The contact assignments of RJ21 type (25-pair Amphenol) connectors on MBU

Figure 3.5.8.1Contact assignments of RJ21 type connectors

25

50

1

26

2

27

3

28

a. Female connector

b. male connector

25

50

3

28

2

27

1

26

### Miniature 6-position jack

The contact assignments of miniature 6-position jack registered in FCC.

Figure 3.5.8.2Contact assignments of miniature 6-position jack

T1

R2

R1

T2

1

2

3

4

5

6

### Miniature 8-position jack for ISDN connection

The contact assignments of miniature 8-position jack used for ISDN connection.

a. Basic Rate User network Interface

Figure 3.5.8.3.aRJ-45 Connector

1

2

3

4

5

6

7

8

ISO 8877

ITU-T Rec. I.430

Contact

1

2

3

4

5

6

7

8

TE

Power Source 3

Power Source 3

Transmit

Receive

Receive

Transmit

Power Source 2

Power Source 2

NT

Power Source 3

Power Source 3

Receive

Transmit

Transmit

Receive

Power Source 2

Power Source 2

Polarity

+

-

+

+

-

-

-

+

Table 3.5.8.3.a	Contact assignments

b. Primary Rate User network Interface (E1/T1)

Figure 3.5.8.3.bRJ-45 Connector

1

2

3

4

5

6

7

8

ISO 10173

ANSI T1.408

Contact

1

2

3

4

5

6

7

8

TE

Receive

Receive

Shield(R) *

Transmit

Transmit

Shield(T) *

Power Source *

Power Source *

NT

Transmit

Transmit

Shield(T) *

Receive

Receive

Shield(R) *

Power Source *

Power Source *

Polarity

-

+

Table 3.5.8.3.bContact assignments

* Optional