# APPENDIX A.  The Maximum Number of Boards in a system

The maximum number of boards is described in this section. Some boards can be limited by the capacity of system power supply, PSU linear.

A.1  The Maximum Number of Cards in a system..

The maximum capacity of boards in a system is given in table A.1 Not all maximum capacities may be achieved simultaneously. The complex configuration can be limited by the system power supply, e.g. PSU(linear). Especially if you want to make the complex configuration with various extension boards including WTIB and 2B-modules, consult it with its supplier or authorized agent.

Table 1.1  The maximum number of cards per a cabinet

| Board Name | The Maximum Number of Cards per a cabinet | Slot Position | Remark |
| --- | --- | --- | --- |
| 2BRI | 1 | 3 | Basic slot |
| 4LCO | 1 | 3 | Basic slot |
| 4SLI | 1 | 2 | Basic slot |
| AC15(UK) | 4 | 4~7 |  |
| BWDIDB(NZ) | 4 | 4~7 |  |
| DTIB | 4 | 4~7 |  |
| DVIB | 2 | 1,4~7 |  |
| ETIB | 4 | 4~7 |  |
| LCOB | 4 | 4~7 |  |
| MBU | 1 | KSU |  |
| PRI | 1 | 3 | Basic slot |
| SLIB | 4 | 4~7 |  |
| STIB | 5 | 1,4~7 |  |
| TLIB(KOREA) | 4 | 4~7 |  |
| WTIB | 1 | 1,4~7 |  |
| WTIU | 1 | on WTIB |  |
| 2B-SLIU30 and SLT | 24 | on LKD30 | * Note 1 |

- Note;

- The 2B-modules are limited by the power capacity.

The maximum terminals with PSU(Linear) are limited by 34 without WTIB and 48 with WTIB, because of its power capacity.

The maximum terminals with PSU2(SMPS) are limited by 48 without WTIB and 64 with WTIB, because of its power capacity.

A.2  The Capacity of GDK-FPII Power Supply

The capacity of GDK-FPII Power Supplies, PSU(linear) and PSU2(SMPS), is given in table A.2. The capacity of +30Vdc in table A.2 includes the capacity of +5Vdc,-5Vdc. -48Vdc is locally generated on the card that it is required.

Table A.2  Power Capacity of GDK-FPII PSU and PSU2

| PSU | +5Vdc | -5Vdc | +30Vdc | -48Vdc |
| --- | --- | --- | --- | --- |
| GDK-FPII PSU (Linear) | 2000 | 500 | 3000 | None |
| GDK-FPII PSU2(SMPS) | 3500 | 800 | 4000 | None |

- Note:

Manufacturing GDK-FPII PSU(linear) will be officially stopped after launching PSU2(SMPS) from the first half of 1999.

# APPENDIX B. Power Consumption

| Board Name | The Power Consumption/a Card |  |  |  |
| --- | --- | --- | --- | --- |
| (all boards) | +5V | -5V | +30V | -48V |
| 2BRI | 55 | 0 | 0 | 0 |
| 4LCO | 85 | 65 | 80 | 0 |
| 4SLI | 55 | 50 | 135 | 0 |
| AC15(UK) | 65 | 55 | 0 | 0 |
| BWDIDB(NZ) | 100 | 60 | 490 | 0 |
| DTIB | 50 | 0 | 360 | 0 |
| DVIB | 475 | 5 | 0 | 0 |
| ETIB | 65 | 55 | 410 | 0 |
| LCOB | 100 | 70 | 80 | 0 |
| MBU | 360 | 15 | 370 | 0 |
| PRI | 125 | 0 | 0 | 0 |
| SLIB | 80 | 65 | 195 | 0 |
| STIB | 120 | 0 | 150 | 0 |
| TLIB(KOREA) | 60 | 35 | 280 | 0 |
| WTIB | 680 | 0 | 320 | 0 |
| WTIU | 200 | 0 | 320 | 0 |
| 2B-SLIU30 and SLT | 0 | 0 | 50 | 0 |
| MEMU | 3 | 0 | 0 | 0 |
| SMEMU | 5 | 0 | 0 | 0 |
| PLLU2(issue 1,2) | 20 | 0 | 0 | 0 |
| PLLU2(issue 3) | 30 | 0 | 0 | 0 |
| MODU | 15 | 7 | 0 | 0 |
| SIU(UK) | 58 | 0 | 0 | 0 |
| SIU(ST) | 15 | 0 | 0 | 0 |
| CMU | 8 | 3 | 0 | 0 |
| PFTU(External) | 0 | 0 | 55 | 0 |
| PFTU-I(Internal) | 0 | 0 | 25 | 0 |
| RGU | 0 | 0 | 110 | 0 |
| Charging Battery | 0 | 0 | 500 | 0 |

# APPENDIX C.  PSU installation

This is the description of PSU(linear type) for the user who uses it.

Note: There are 2 kinds of PSUs, PSU which is a linear type of Power supply Unit, and PSU2 which is a SMPS. The production of PSU (linear) has been officially stopped after replacing with PSU2(SMPS) in the 1st half of 1999.

The PSU is basically installed when delivering the system. But if PSU is replaced with new one, assure that the AC power cord is NOT plugged into the AC outlet before installing new one.

The system contains any one of two linear PSU assemblies; one is PSU with transformer 6170NI0008A, the other is PSU with transformer 6170NI0007A

PSU with Main Transformer 6170NI0008A: for European and Oceanic countries

The linear type PSU can operate from 220VAC~240VAC with transformer 6170NI0008A based on the connection of the cable(2-wired) from transformer to the connector CN1 of the PSU PCB. This type only operate from 220~240VAC, not 110VAC. Fuse: T1.6AH250V

PSU with Main Transformer 6170NI0007A: for the countries except European and Oceanic countries.

The linear type PSU can be operate from ether 110VAC or 220VAC with transformer 6170NI0007A based on the connection of the cable(5-wired) from transformer to the connector CN1 or CN2 of the PSU PCB. Generally shipped the PSU with 220VAC. If local AC voltage 110~127VAC volts, move the cable to the CN2 shown as Figure C.1

| Input Voltage | Connect to | Fuse |
| --- | --- | --- |
| 110VAC | CN2 | T3.15AL250V |
| 220VAC | CN1 | T1.6AH250V |

Insert Figure 3.3.3 in the existing manual issue 2 here

Figure C.1  Connection of PSU linear

After the PSU installation, you have to assemble the cover with three(3) screws. The cover assembling method is shown in Figure C.2

Insert Figure 3.3.4 in the existing manual issue 2 here

Figure C.2  Power Supply Unit Cover Assembly