##### SECTION 3. INSTALLATION

### INTRODUCTION

As with any sophisticated communications device, installation of the GDK-FPII System requires the care and fore-thought of a competent technician. Installation proceeds in 5 major steps;

- Site Preparation

- KSU and PSU Installation

- PCB Installation

- System Wiring

- System Programming and Verification

By utilizing the instructions that follow, the installation is quick and efficient. Directions for System Programming are briefly given in SECTION 5. CUSTOMER DATABASE PROGRAMMING

### SITE PREPARATION

### General Site Consideration

The first step is to locate an acceptable site for the common equipment. (KSUs, boards, etc.) When locating a mounting site for the KSUs, the following points should be considered:

- The KSUs are designed for wall-mounting and should not be mounted directly to a masonry or plaster-board wall. It is recommended that a ½ inch thickness plywood back-board be firmly mounted to the wall at first and then the KSU.

- The location must have access to a dedicated 110~127 Volt AC or 220~240 Volt AC, 50-60Hz with a circuit breaker or fuse rated at 15amps. A 3-wire parallel blade grounded outlet should be within approximately 2 meters (6 feets).

- The location must have access to a good earth ground, such as a metallic cold water pipe with no non-metallic joints. The ground source should be located as close as possible to the system.

- The system should be located in an area which is well-ventilated with a recommended temperature of 20~26 degrees C (68~78.8 degrees F) and a relative humidity of 5~90 % (non-condensing).

- The system should be located within 8 meters (25 feets) of the telephone company's termination point. Also, the location should be within the prescribed station loop lengths for all keysets and terminals. If existing cabling is to be employed, the location of existing cabling and conduits should be considered.

- The location should have adequate accessibility, space and lighting for future servicing and should consider the need for future expansion.

- The site should be away from radio transmitting equipment, arc-welding devices, copy machines, and other electrical equipment capable of generating electrical interference. The system should be protected from flooding and heavy machinery as well as excessive dust and vibration.

### Back-Board Installation

A wooden back-board is recommended for all installations and should be installed when the location has masonry or plaster-board walls. A ½ inch plywood material is sufficient for most installations. The back-board should be mounted at a convenient height, about 1 meter above the floor, and be bolted in a number of places to distribute the weight of the system..

Space should be available on the bottom and left side of the back-board for another MDF and for optional equipment such as a music source and PFTU, etc.

### Verify On-Site Equipment

Once the equipment installation site has been identified and a dedicated AC outlet, earth ground, and lighting and ventilation are available, verify that all required equipment are on-site and have not been damaged during shipment. Unpack the KSU and assure there is no shipping damage. Note that a mounting template is packed with KSU and this template will be required later in the installation. Check that the type and quantity of received boards are correct and optional equipment are on-site. Note that the individual boards should not be unpacked at this time.

If any equipment is damaged or missing, notify appropriate personnel to correct the situation.

### KSU INSTALLATION

GDK-FPII system consists of KSU and some optional boards. The exterior view of the GDK-FPII is shown in Figure 3.3.1.

Insert Figure 3.3.1 in the existing Manual here

Figure 3.3	Exterior View of GDK-FPII

### Mounting KSU

The KSU is a metal frame cabinet designed for wall-mounting. Employing the KSU mounting template provided with the KSU, mark the location of the four screws to mount the KSU. Again, the KSU must not be mounted on a masonry or dry-wall surface, in this case a wooden back-board is required, refer to paragraph [Back-Board Installation]. The distance between mounting holes and the position of mounting holes is shown in Figure 3.3.1.

The KSU is mounted with four #10 or larger, 1.5 inch or longer screws. At first, drill pilot holes in the four locations marked, insert the screws and tighten leaving about 0.5 inch exposed. Mount the KSU on the screws and tighten the screws securely.

Insert Figure 3.3.2 in the existing Manual here

Figure 3.3.1	KSU Mounting Holes & Installation Layout

### RGU Installation

The Ring Generator Unit is needed in KSU if 4SLI or SLIB is installed for using single line telephone, to provide ringing power and Message Wait Source power. The RGU-EX is mounted on the right bottom side of the KSU with two screws and is connected to the Main Board Unit(MBU) with a CN5(PCB lettering) connector. (See Figure 3.3.2)

A cable for connecting between RGU and MBU is provided with RGU in the same package.

Insert Figure 3.3.5 in the existing Manual here

Figure 3.3.2 	Ring Generator Unit Installation

### PFTU Installation

The major function of PFTU is to achieve an alternate connection to the system’s CO line ports when the AC power is failed. There are two different types of PFTU. One (PFTU-I) is installed on the 4LCO board and two CO lines on this board can be switched. The other PFTU is connected to MBU to select any CO lines that user requires and the PFTU is installed outside of the system.

PFTU-I

The optional board is installed on the 4LCO board and switches the first two CO lines to single line telephones directly during AC power failure. Single line telephones are connected to the connector CN2 and CN4 on this board. If a SLT is connected to CN2, the SLT can be used to one of the system’s extensions during normal state(means AC power on state) by connecting the connector CN1 to any ports of the 4SLI or SLIB. This situation is also true for the connector CN3 and CN4.

Refer to Figure 3.3.3.a

Insert Figure 3.4.15 in the existing Manual here

Figure 3.3.3.a 	PFTU-I Installation

PFTU

Figure 3.3.3.b shows the exterior of the PFTU module and how to connect this optional module to the MBU. The module switches six CO lines to single line telephones directly via CHAMP connector during AC power failure. The test/normal selection slide switch should be on the ‘Normal’ position for proper operation. If it is switched to ‘Test’ position, the PFTU module works as AC power failure situation. Up to three(3) PFTUs can be connected to the MBU. The detailed connection method is described in [Power Failure Transfer Unit Wiring] of section 3.5.

To connect PFTU control signal,

- Connect a pair of wires between connector(U41) of the MBU and terminal of PFTU. Also it is possible to connect a pair of wires from pin#7 and #32 of the basic CHAMP connector(CN14), because these two signals are tied together in the MBU.

- PFTU is not sensitive to polarity of control signal.

Connecting Wire;

- AWG #22 ~ AWG #26 (diameter : about 0.6mm ~ 0.4mm)

- Length : Max. 10 meters ~ 1 m ~ 1.5 m (recommended)

Note :  The installer can adjust the diameter and length of above wires depending on the site condition.

a)  Exterior of the PFTU Module

CN14

PFTU

U41

CN15

MBU

NORMAL

TEST

pin#7 & #32

OR

AWG 24

b) PFTU Installation

Figure 3.3.3.b  PFTU Installation

### KSU Grounding

Before connection to the mains supply, the GDK system must be earthed in conformance with the appropriate IEE requirements. Earthing is required for user safety and also to minimize EMC interference.

To ensure proper system operation and for safety purpose, a good earth ground is required. A metallic COLD water pipe usually provides a reliable ground. Carefully check that the pipe does not contain insulated joints that could isolate the ground path. If insulated joints exist, another earth ground source should be used or, if allowed, the joints may be bridged.

A separate conductor should be used between the ground source and the KSU, which confirms to the appropriate national or local electrical codes. AGW #14 or larger copper wire is recommended. The wire should be kept as short as possible, it is recommended that the wire be no longer than 8 meters (about 25 feet).

### PSU Installation

The Power Supply Unit(PSU) provides the power for the system boards and telephones, converting the AC voltage input to appropriate DC voltages.

The PSU is basically installed when delivering the system. But if PSU is replaced with new one, assure that the AC power cord is NOT plugged into the AC outlet before installing new one.

Note:

- There are 2 kinds of PSUs, PSU which is a linear type of Power supply Unit, and PSU2 which is a SMPS. The production of PSU (linear) has been officially stopped after replacing with PSU2(SMPS) in the 1st half of 1999.

- Following Figure 3.3.3.a shows PSU2 (SMPS).

- Refer to the APPEDIX C for the installation of PSU (linear).

Insert Figure 3.3.3 in the existing Manual (Manual issue 3) here

- In figure 3.3.3, remove the screws for RGU connection in the right side of figure.

Figure 3.3.5.a	PSU2 without PSU cover

After the PSU installation, assemble the cover with three(3) screws. The cover assembling method is shown in Figure 3.3.5.b.

Insert Figure 3.3.4 in the existing Manual (Manual issue 3) here

Figure 3.3.5.b	PSU2 Cover Assembly

### PCB INSTALLATION

### PCB Handling & General Installation

All boards should not be installed or removed with power applied.

Power must be turned off prior to insertion or removal of the PCB.

The system PCBs contain digital circuitry which, while extremely reliable, can be damaged by exposure to excessive static electricity. When handling PCBs, a grounded wrist strap should be used to protect the boards from static discharges. Also, use common sense when handling the PCBs. For example, do not place a PCB in locations where heavy objects might fall on the PCB and damage components.

To insert a PCB, hold the PCB on the mounting knobs and, with components side facing up, insert the card into the system’s card edge connector firmly and tighten with screw(s). To remove a PCB, reverse the procedure. Installation method of PCB is shown in Figure 3.4.1.a.

Insert Figure 3.4.1 in the existing Manual here

Figure 3.4.1.a	 PCB Installation

There are 7 slots in the GDK-FPII system to extend or modify the system configuration. But, there are some restrictions for optional board installation. Note that the system can not operate properly with wrong installation. The most upper slot (slot #1) is reserved for DVIB, STIB or WTIB, the next slot(slot #2) is reserved only for 4SLI board, the next slot(slot #3) is reserved for one of 4SLI, 2BRI, or PRI boards and the next four(4) slots(slot #4~7) are universal slots.

To summarize, see the following Table 3.4.1.a and Figure 3.4.1.b.

| Slot No. | Cards | Remarks |
| --- | --- | --- |
| SLOT 1 | DVIB, STIB, WTIB | Option cards with own CPU |
| SLOT 2 | 4SLI | Fixed slot for 4SLI board |
| SLOT 3 | 4LCO, 2BRI, PRI | Fixed slot for 4LCO or 2BRI or PRI |
| SLOT 4 SLOT 5 SLOT 6 SLOT 7 | LCOB, AC15, BWDIDB, STIB, SLIB, DTIB, ETIB, DVIB, WTIB, TLIB | Universal Slot *Not allow PRI, 2BRI, 4LCO, 4SLI |

Table 3.4.1	Table for Option Cards Installation

SLOT 1 (DVIB/STIB/WTIB)

SLOT 2 (4SLI)

SLOT 3 (4LCO/2BRI/PRI)

SLOT 5 (Universal Slot)

SLOT 4 (Universal Slot)

SLOT 7 (Universal Slot)

SLOT 6 (Universal Slot)

PSU

RGU

-EX

MBU

CN16

CN15

CN14

MEMU

or

SMEMU

PLLU2

MODU

SIU(1)

SIU(2)

U41

CN18

CN21

For external PFTU

Insert a short pin when PLLU2 is not installed.

Insert a short pin when SIU(2) is installed.

Figure 3.4.1.b Slot Assignments

When the system power is on, the default slot assignment is automatically redefined according to board installation status. (For this, the switch pin #1 and #4 of the SW1 in MBU should be set to ON position.) And it can be modified by user’s needs.

### MBU Installation

The MBU is installed in the KSU in default. It is comprised of main processor, six(6) digital terminal interface circuits, seven(7) card slots for optional boards, memory, real-time clock circuit and data communication circuit which transmits and receives to/from other cards. The MBU may be equipped with four daughter boards; a MEMU for RAM expansion, a SMEMU for the function of QSIG, TAPI and/or PC-Attendant, a MODU for remote access to the system, a PLLU2 for system synchronization and a SIU for serial data transmission. Refer to Figure 3.4.2.a

It should be noted that two SIU boards can be installed when the MODU is NOT installed, and to install a SIU in the position of SIU(2), an option jumper pin contained in the SIU package should be inserted between pin #1 and pin #2 of MODU connector (CN21) as shown in Figure 3.4.1.b and 3.4.2.5.

Insert Figure 3.4.3 in the existing Manual here

Figure 3.4.2.a 	MBU, MODU, MEMU, SIU, and PLLU2 installation

Lithium Battery

The MBU contains a lithium dry-cell to keep the memory contents and real-time clock functions during power failure. The battery is soldered to the MBU, and connected circuitry by an on/off DIP switch (SW3). So the DIP switch knob must be ON position before installing MBU. Refer to Figure 3.4.2.b.

“ CAUTION : Danger of explosion if battery is incorrectly replaced. Replace only with the same or equivalent type recommended by the manufacturer. Dispose of used batteries

according to the manufacturers instruction.”

Insert Figure 3.4.4 in the existing Manual here

Insert Figure 3.4.5 in the existing Manual here

- remove the circles related to SW1 and the extra drawing of SW1 in this figure

Figure 3.4.2.b 	Memory Back-Up switch setting

System Clock Connection without PLLU2

It is important that a shorting trap (option pin) should be inserted to connect the pin #12 and #13 of the connector CN18(See Figure 3.4.1.b and 3.4.2.c) in the MBU for proper operation when the PLLU2 is NOT installed. Refer to the clause of PLLU2 Installation.

Figure 3.4.2.c 	System Clock connection without PLLU2

Function of SW1

The MBU also has an four-position DIP switch, SW1. The function of each switch position is as followings:

Default setting: All ON position

| Switch | Function | ON | OFF |
| --- | --- | --- | --- |
| 1 | Administration Programming Access | Enable | Disable |
| 2 | Trace Control (The purpose of testing software) | Disable | Enable |
| 3 | SMDI (Simplified Message Desk Control –Voice Mail ) | SMDI OFF | SMDI ON |
| 4 | Database default on power up | Enable | Disable |

Table 3.4.2 	The function of each pole of SW2

Once the database has been initialized, switch 4 should be placed in the OFF position to protect the database in the memory. Before programming the system, switch 4 should be placed in ON position. Then, the system program initializes the system database to default value when the system power is on. Refer to Figure 3.4.2.b.

Function of various switches and connectors

| Switch/Connector | Functions | Remark |
| --- | --- | --- |
| SW2 | CPU Reset |  |
| SW3 | Lithium Battery ON/OFF switch |  |
| CN5 | RGU connection |  |
| CN6 | DC Power connection |  |
| CN7~CN13 | Function Board Connection for slots 1~7 |  |
| CN14 | Basic Port Connection including Relay contacts | For MBU, Slot No. 2 |
| CN15 | Analog CO line Connection |  |
| CN16 | Expansion Port Connection | For Slot No. 3~7 |
| CN17,CN18 | PLLU2 connection | ISDN 1) |
| CN19 | SIU(2) connection | System Access 2) |
| CN24 | SIU(1) connection | System Access |
| CN20, CN21 | MODU connection | 2) |
| CN22,23 | MEMU or SMEMU connection |  |
| CN41 | PFTU control signal connection |  |
| PJ1 | RCA Jack for the external paging |  |
| PJ2 | RCA Jack for the external music source |  |

- Note

- When ISDN board is installed, PLLU2 should be installed. Otherwise there is no necessity for installing PLLU. If PLLU is not installed, the shorting trap (short pin) should be located between contacts 12 and 13 of CN18.

The shorting trap (short pin) is installed on above position when the board delivered.

- It is possible to use one of both MODU and SIU(2). If the SIU is installed on SIU(2) position, contacts 1 and 2 of CN21(connector for MODU) should be short.

The shorting trap (short pin) is installed on above position when the board delivered.

Function of LED

| LED | Function | Status | Remark |
| --- | --- | --- | --- |
| LED1 | Display system software operation | Turn ON/OFF at every 100 msec |  |
| LED2 | Display event processing | Turn ON during event processing |  |
| LED3 | Display task changing (software testing purpose) | Turn ON/OFF in toggle when the software task is chaging. |  |
| LED4 | Basic Digital Keyset In-use LED | ON: At least one port is busy. OFF: all ports are idle |  |

Check Points

- The DIP switch, SW3 should be turned ON position to protect system data in the case of a power fail.

- The 4th pole of SW1 should be OFF to protect the variety features by Admin programming after the system power up and initialization. See Table 3.4.2

When the system power is on, the boards in slots will be automatically detected if the 4th pole of SW1 is set to ON position.

- The pin #12 and #13 of the connector CN18 in the MBU should be shorted with a shorting trap (short pin) for proper operation when the PLLU2 is NOT installed.  Refer to the clause 3.4.2.3 PLLU Installation.

- Add-on boards: MEMU, SMEMU, PLLU, MODU. SIU

### MEMU Installation

The MBU may be equipped with MEMU for memory expansion. Before power on, install MEMU as required, refer to Figure 3.4.2.a. The MEMU has a lithium dry-cell and SW1 must be set to ON position to protect the database in the memory, before power on.

- Note:

- If MEMU is newly installed during operation, then the SMDR recorded in the basic memory of MBU before MEMU installation will be lost.

- If MEMU is replaced with SMEMU during operation, the database will be lost. So it is recommended that the database is downloaded to PC before replacement, and unloaded after replacement.

The MEMU should be installed on MPB through the dual line connectors, CN22 and CN23 of MBU, so recommend visual inspection of the connecting state between MBU and MEMU after installing it on MBU.

### SMEMU Installation

The MBU may be equipped with SMEMU for the memory expansion and the QSIG, CTI, PC-ATTN features. Before installing the GDK-FPII, install SMEMU if required, refer to Figure 3.4.2.a. The SMEMU has a lithium dry-cell and SW1 must be set to ON position to protect the database in the memory, before power on.

The SMEMU should be installed on the position of MEMU on MBU.

- Note:

- If SMEMU is newly installed during operation, then the SMDR recorded in the basic memory of MBU before SMEMU installation will be lost.

- If MEMU is replaced with SMEMU during operation, some database will be lost. So it is recommended that the database is downloaded to PC before replacement and unloaded after replacement.

- If you want to use the QSIG, CTI, PC-ATTN features, the software package 99A version should be required.(See the SMEMU in section 2)

The SMEMU should be installed on MBU through the dual line connectors, CN22 and CN23 of MBU, so recommend visual inspection of the connecting state between MBU and MEMU after installing it on MBU.

### PLLU2 Installation

The PLLU2 board should be installed on the MBU to use the ISDN interface board such as 2BRI, PRI and STIB. If PLLU2 is not installed, a short pin should be used to connect the pin #12 and #13 of the connector CN18 (See Figure 3.4.2.c) in the MBU for proper operation.

### MODU Installation

The MODU is used for remote access the system for system maintenance and PC based administration function. Refer to Figure 3.4.2.a.

If MODU is installed, SIU(2) should be removed from the system.

### SIU Installation

The SIU board is used for system maintenance, PC based Admin, SMDR print and SMDI. The system supports up to two SIU boards when the MODU is not installed. The installation method is shown in Figure 3.4.2.5.

It should be noted that two SIU boards can be installed when the MODU is NOT installed, and an option jumper pin packed with the SIU should be inserted between pin#1 and pin#2 of MODU connector(CN21) of the MBU.

Insert Figure 3.4.6-a) in the existing Manual here

a) When 1 SIU is installed

Insert Figure 3.4.6-b) in the existing Manual here

b) When 2 SIUs are installed

Figure 3.4.2.5 	SIU Installation

### Extension Board Installation

The system can be equipped with any combination of the extension boards; DTIB, ETIB, 4SLI, SLIB, and WTIB. The WTIB can be installed in the most upper slot (slot #1) and the universal slots(slot #4~7), the 4SLI board can be only installed in the next slot(slot #2), DTIB, ETIB, SLIB, and WTIB can be in the universal slots (slot #4~7). See the clause 3.4.1.

### DTIB Installation

The DTIB provides 6 interface circuits for normal type or 2B type digital terminals. All the interface circuits can support 2B format of digitized voice data. The connection between DTIB and digital terminals are performed through RJ21 type connector (CN16) on the MBU. The exterior of the board is shown in Figure 3.4.3.1.

Function of LEDs

| LED | Function | Remark |
| --- | --- | --- |
| LD1 | The status of ports. (ON: in use, OFF: idle) |  |

In GDK-FPII system, all Digital Terminal Interfaces are capable of accommodating 2B-channel operation. LDK-30DS with 2B-module and secondary terminal can be connected to every port of DTIB or MBU. See clause 2.4.13.3 2B-module and clause 2.4.13.4 GDK-PC PHONE in section 2 for more details regarding 2B-module and CTI module.

Insert Figure 3.4.12 in the existing Manual here

Figure 3.4.3.1  DTIB layout

The DTIB board can be installed on any slot of universal slots (slot #4~7).

The Maximum 4 boards can be installed in a system.

### ETIB Installation

The ETIB provides 6 interface circuits for analog type electronic key telephones. The connection between ETIB and electronic terminals are performed through RJ11 type jacks on the board. The exterior of the board is shown in Figure 3.4.3.1.

Function of LED

| LED | Function | Remark |
| --- | --- | --- |
| LED1 | The status of ports. (ON: in use, OFF: idle) |  |

Function connector

| Connector | Function | Remark |
| --- | --- | --- |
| MJ1~MJ6 | RJ11 type jack for terminal connection. | See clause 3.4.5 |

Insert Figure 3.4.11 in the existing Manual here

Figure 3.4.11  ETIB layout

The ETIB board can be installed on any slot of universal slots (slot #4~7).

The Maximum 4 boards can be installed in a system.

### SLIB/4SLI Installation

There are two kinds of single line telephone interface boards, one is 4SLI and the other is SLIB. The 4SLI card provides four(4) interface circuits for SLT and the SLIB card provides six(6) interface circuits for SLT. The 4SLI/SLIB can be optionally equipped with a DTRU for DTMF detect and a MSGU for control the message wait lamp of the SLT. And an RGU is required for ringing SLTs.

The exterior of them are shown in Figure 3.4.3.3.

The major difference is shown in below Table 3.4.3.3.

The 4SLI board can be installed on slot #2 only, but the SLIB can be installed any one of universal slots (slot #4~7).

| Board | Slot | Interfaces circuits | Add-on Boards |
| --- | --- | --- | --- |
| 4SLI | Slot # 2 | 4 | DTRU, MSGU |
| SLIB | Slot # 4~7 | 6 | DTRU, MSGU |

Table 3.4.3.3  Difference between 4SLI and SLIB

Function of LED

| LED | Function | Remark |
| --- | --- | --- |
| LED1 | The status of ports. (ON: in use, OFF: idle) |  |

Function of various connectors

| Connector | Function | Remark |
| --- | --- | --- |
| CN1 and CN2 | DTRU connection | 1) |
| CN3 and CN4 | MSGU connection | 1) |

- Note:

- If you want Message Waiting function on 4SLIB or SLIB, GDK-100 MSGU should be installed on 4SLI and SLIB.

- The distance between SLIB and SLT: see Table 2.3.4

Insert Figure 3.4.13-a) in the existing Manual here

a) 4SLI Card

Insert Figure 3.4.13-b) in the existing Manual here

b) SLIB Card

Figure 3.4.3.3  Exterior of the 4SLI/SLIB

### DTRU Installation

A DTRU contains 2 DTMF receiver circuits. Each receiver of DTRU is time-shared under the control of the system software, as a system resource regardless its position.

| Board | The position of Installation | The quantity of receivers | Marking on PCB |
| --- | --- | --- | --- |
| DTRU | 4SLI, SLIB, 4LCO, LCOB, TLIB, BWDIDB | 2 DTMF receivers | DTRU of GDK-34/36 |

The System is capable of supporting DTMF receivers up to 14.

2 DTMF receivers are basically laid out on MBU.

Take a cautious attitude in installing DTRU on the boards because DTRU contains the connectors without the connection guide, otherwise DTRU may be damaged by wrong connection.

### MSGU Installation

A message waiting unit can be optionally installed on a SLT interface board to light a message waiting lamp in the SLT connected.

| Board | The position of Installation | The quantity of circuits | The line voltage |
| --- | --- | --- | --- |
| MSGU | 4SLI, SLIB | 6 circuits | +24Vdc |

- Signal voltage: 90~100Vdc

### Analog CO Line Board Installation

There are several types of analog CO line boards available. These boards include the 4LCO and LCOB. Additional CO interface boards such as AC15, TLIB and BWDIDB may be available for support of local market conditions and regulatory requirements.

The slot No.3 to No. 7 in the system, which are available for use as a CO Line board, are wired to 4 or 2 CO Lines at the CN15 connector through the MBU. The connection matrix is shown in section 3.5. Analog CO line wiring; contact assignments of RJ21 type male connector (CN15).

### LCOB/4LCO Installation

There are two kinds of available analog CO line interface (Loop-start technology) boards. One is 4LCO and the other is LCOB. The exteriors of them are shown in Figure 3.4.4.1.

The major difference is shown in below Table 3.4.2. The 4LCO board can be installed on slot #3 only and the LCOB can be installed on slot #4~7(universal slot).

All these two boards provide four(4) Loop Start lines, can be optionally equipped with Call Metering Unit(CMU) for monitoring call charge and a Call Progress Tone Unit(CPTU) for monitoring call progress signals and a DTMF Receiver Unit(DTRU) for detecting DTMF for single line devices. A CMU should be provided for each Loop Start CO line which requires call metering function. In addition, the appropriate type of CMU should be employed. Each CPTU contains two tone detection circuits that are time shared under software control. These detectors are used to support several features including ACNR. If such features are to be used, it is recommended that CPTU be installed for every 4LCO/LCOB. According to sensitivity of tone detector, two types of CPTUs(CPTU/A, CPTU/B) are available. CPTU/A has wide tone detection range(305Hz~640Hz) and CPTU/B has narrow tone detection range (350Hz, 620Hz, 440Hz, 480Hz±1%). If required, install the CMUs, a CPTU and a DTRU, on the 4LCO/LCOB.

And an optional PFTU-I can be installed on the 4LCO only. This module supports power failure transfer function of the first 2 CO lines of the 4LCO board.

| Board | Slot | Interface Circuits. | Option Boards |
| --- | --- | --- | --- |
| 4LCO | SLOT #3 | 4 | CPTU, CMU, DTRU, PFTU-I |
| LCOB | SLOT #4~7 | 4 | CPTU, CMU, DTRU |

Table 3.4.2  Differences between 4LCO and LCOB

Function of LED

| LED | Function | Remark |
| --- | --- | --- |
| LD1 | The status of line 1, ON: in use, Green |  |
| LD2 | The status of line 2, ON: in use, Green |  |
| LD3 | The status of line 3, ON: in use, Green |  |
| LD4 | The status of line 4, ON: in use, Green |  |

Function of various connectors

| Connector | Function | Remark |
| --- | --- | --- |
| CN1 and CN2 | DTRU connection |  |
| CN3 and CN4 | CPTU connection |  |
| CN5 and CN6 | PFTU-I connection | 4LCO only |
| CN100 and CN104 | The connectors for a CMU on line 1 | See Note |
| CN101 and CN105 | The connectors for a CMU on line 2 | See Note |
| CN102 and CN106 | The connectors for a CMU on line 3 | See Note |
| CN103 and CN107 | The connectors for a CMU on line 4 | See Note |

- Note:

- The pin #6 and #7 of CN104, CN105, CN106 and/or CN107 should be shorted by shorting traps (short pin) when CMU12(12 KHz meter pulse) including CMU12PR or CMU16(16 KHz meter pulse) is not installed on the interface circuit(s). Otherwise the receiving voice path of the related line will be open.

- The 0 ohm resistors are usually mounted on R168-R171 of LCOB for the countries using only CMU PR(Polarity Reversal) and/or 50L(50 Hz longitudinal). If these CMUPR, CMU50L or CMU50PR are not installed on any CO interface circuit neither R168-R171 are mounted on LCOB, the pin #6 and #7 of CN104, CN105, CN106 and/or CN107 should be also shorted by shorting traps (short pin) on the related interface circuit.

- CN104/R168 are on the 1st interface circuit, CN105/R169 on the 2nd interface circuit, CN106/R170 on the 3rd interface circuit and CN107/R171 on 4th interface circuit.

4LCO can be only installed in slot No. 3.

LCOB can be only installed in the universal slots (No. 4 to 7), and the maximum four LCOBs can be installed in a system.

Add-on boards: DTRU, CPTU/A, CPTU/B, CMU, PFTU-I (4CLO only).

Insert Figure 3.4.7-a) in the existing Manual here

a) 4LCO Card

Insert Figure 3.4.7-b) in the existing Manual here

b) LCOB Card

Figure 3.4.4.1  4LCO/LCOB Board

### AC15 Board Installation - UK

The AC15 board provides 2 Tie interface circuits, each line supports AC15a signaling.

It is designed to comply with the UK specification and application, so it may not suitable for the other national specification and/or application.

Function of LED

| LED | Function | Remark |
| --- | --- | --- |
| LD1 | The status of line 1, ON: in use |  |
| LD2 | The status of line 2, ON: in use |  |

The AC15 board can be installed in any of slot No. 4~7.

The maximum four AC15 boards can be installed in a system.

### TLIB Installation - Korea

The TLIB provides 2 analog E&M interfaces. It can support type I and V of 2-wire E&M.

It is designed to comply with the Korean specification and application, so it may not suitable for the other national specification and/or application.

Function of LED

| LED | Function | Remark |
| --- | --- | --- |
| LD1 | The status of line 1, ON: in use |  |
| LD2 | The status of line 2, ON: in use |  |

Function of various connectors

| Connector | Function | Remark |
| --- | --- | --- |
| CN1 and CN2 | DTRU connection |  |

Line mode selection

| connectors | E&M |  | Ring Down |
| --- | --- | --- | --- |
|  | Type V | Type I |  |
| JP9,CN12 | short pin no. 1-2 | short pin no. 1-2 | short pin no. 2-3 |
| JP10,CN11 | short pin no. 2-3 | short pin no. 1-2 | Don’t care |

- Note:

TLIB mainly supports 2-wire E&M type V. It is recommended to set 2 wire E&M Type V.

Check point

TLIB can be installed in any of slot No. 4~7.

The maximum four TLIBs can be installed in a system.

When installing E&M line, It is important to check the type of the opposite line and jumper position.

Add-on board: DTRU

### BWDIDB Installation – New Zealand

The BWDIDB provides 4 analog both-way DID interfaces. Each line supports Outgoing/Incoming call.

It is designed to comply with New Zealand specification and application, so it may not suitable for the other national specification and/or application.

Function of LED

| LED | Function | Remark |
| --- | --- | --- |
| LD1 | The status of line 1, ON: in use |  |
| LD2 | The status of line 2, ON: in use |  |
| LD3 | The status of line 3, ON: in use |  |
| LD4 | The status of line 4, ON: in use |  |

Function of various connectors

| Connector/Switch | Function | Remark |
| --- | --- | --- |
| CN1 and CN2 | DTRU connection |  |
| CN3 and CN4 | CPTU/A or CPTU/B connection |  |

BWDIDB can be installed in any of slot No. 4~7.

The maximum four BWDIDBs can be installed in a system.

Add-on boards: DTRU, CPTU/A, CPTU/B

### ISDN Board Installation

There are 3 kinds of ISDN boards such as PRI, 2BRI, and STIB are available. PRI and 2BRI can be only installed in slot No.3, and STIB in slot No.1, 4~7.

Each ISDN board contains RJ45 type jack on boards themselves.

### ISDN 2BRI (Basic Rate T interface) Installation

The 2BRI card provides 2 interface circuits of BRI(Basic Rate Interface) format digital CO interface. For using this board, the PLLU2 board should be installed on the MBU. If more than one ISDN board is required, some special cable should be used for clock signal control. CN1 and CN2 are used to control the clock signal when multi-ISDN boards are installed. Refer to [Clock Control Cable Wiring] of section 3.5.

Function of LED

| LED | Function | Status | Remark |
| --- | --- | --- | --- |
| LD1 | The status of Line 1 | Red: Error, Green: Synchronized |  |
| LD2 | The status of Line 2 | Red: Error, Green: Synchronized |  |

Function of SW1; default -all OFF when delivering the board

| Pole | Function | ON | OFF | Remark |
| --- | --- | --- | --- | --- |
| 1 | Loop Back Mode (for testing purpose) | Enable | Disable |  |
| 2 | Not used | - | - |  |
| 3 | Trace | Enable | Disable | Note 1 |
| 4 | Trace: for the purpose of Lab. test | Enable | Disable | Note 1 |

- Note:

- These two poles are for the purpose of developing, testing and maintaining 2BRI.

- The TEI mode of 2BRI is set to Fixed TEI as default.

- All poles should be OFF during normal operation. Especially the pole 1 should be OFF.

Function of various connectors

| Connector | Function | Remark |
| --- | --- | --- |
| CN1 | Clock control OUT | Note 1 |
| CN2 | Clock control IN | Note 1 |

- Note:

- CN1, CN2: They are used to control the reference clock signal when multi-ISDN boards are installed. See [Clock Control Cable Wiring] in section 3.5.

The line connector and terminating resistors

| Line No | RJ45 type jack | Terminating Resistors |  |  |
| --- | --- | --- | --- | --- |
|  |  | Switch | All ON | All OFF |
| Line 1 | MJ1 | CN3 | terminating | open |
| Line 2 | MJ2 | CN4 | terminating | open |

- Note

- Two terminating resistors are required on a line, one for TX pair and the other for RX pair, for proper operation. Two connectors of CN3 and CN4 should be set to the ON position to terminate two terminating resistors on a line.

Figure 3.4.5.1.aThe connection of Terminating Resistors on 2BRI issue 2

Ter. Res.

LINE 1

CN3

OFF

ON

OFF

Ter. Res.

LINE 2

CN4

MJ1

MJ2

RJ45 type Jack

ON

Figure 3.4.5.1.b	2BRI board

- Terminating resistors and selectable switches are introduced only on PCB issue 2 or later, but not on PCB issue 1. So if you use 2BRI issue 1, the external terminating resistors are required. For the external termination, you can get the terminating unit locally or use GDK-162/100 TERM, which is the terminating unit with 4 P-to-P connection provided by LGIC.

Insert Figure 3.4.8 in the existing Manual here

Figure 3.4.5.1.b	 2BRI Board

Check Points

- 2BRI boards can be installed only in the slot No. 3.

- The PLLU2 MUST be installed on MBU.

- The 2BRI supports Fixed TEI.

- The RX and TX pairs of line should be correctly connected to the TX and RX pairs of NT1.

- The clock control cable should be connected to the proper position when multi-ISDN cards are installed in a system. See section 3.5.

- Terminating resistors on the line. (CN3 and CN4)

- SMEMU MUST be required on MBU for QSIG operation. See SMEMU in section 2.

### ISDN STIB (Basic Rate S/T Interface) Installation

The STIB provides 2 interface circuits of BRI(Basic Rate Interface) format ‘S’ and ‘T’ mode interface. The operating mode selection is performed by setting the short pins and DIP switch to associate position. The ‘S’ mode is used for connecting the ISDN terminal and the ‘T’ mode is used for connecting the ISDN CO line. For using this board, the PLLU2 board should be installed on the MBU. If more than one ISDN board is required, some special cable should be used for clock signal control. CN1 and CN2 are used to control the clock signal when multi-ISDN boards are installed. Refer to [Clock Control Cable Wiring] of section 3.5.

Function of LED

| No | Function | Status | Remark |
| --- | --- | --- | --- |
| LD1 | The status of Line 1 | Red: Error, Green: Synchronized |  |
| LD2 | The status of Line 2 | Red: Error, Green: Synchronized |  |

Function of SW1: default - all OFF when delivering the board

| Pole | Function | ON | OFF | Remark |
| --- | --- | --- | --- | --- |
| 1 | Not used | - | - |  |
| 2 | TEI | Fixed TEI | Automatic TEI | Note 1, 2 |
| 3 | Layer 2 trace | Enable | Disable | Note 3 |
| 4 | Layer 3 trace | Enable | Disable | Note 3 |

- Note:

- The function of 2nd pole has been implemented in firmware version 1.1 or later.

- This is set to OFF position as default, therefore it is important to note that 2nd pole is set to the proper position after checking the type of TEI in your country.

- These two poles are for the purpose of developing and maintaining STIB.

Function of various connectors

| Connector | Function | Remark |
| --- | --- | --- |
| CN1 | Clock control OUT | Note 1 |
| CN2 | Clock control IN | Note 1 |
| CN104 | Serial interface connector | Note2 |

- Note

- CN1, CN2: They are used to control the reference clock signal when multi-ISDN boards are installed.

See [Clock Control Cable Wiring] on section 3.5.

- CN104: Trace for the purpose of development and maintenance.

The external module for RS 232C serial interface should be required for trace.

See GDK-TRC1 in section 2.

The line mode selection (S or T interface setting)

| Line No | RJ45 type Jack | S/T mode selection |  |  | Changing the contact assignments |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Switch | All to “T “ | All to “S” | Connector | All to “T “ | All to “S” |
| Line 1 | MJ1 | SW2 | T | S | CONN1 | T | S |
| Line 2 | MJ2 | SW3 | T | S | CONN2 | T | S |

- Note:

- The setting method of S/T mode selection switches and connectors: default -all T position when delivering the board.

Each line interface can be used for T interface or S interface with different operation interface. If necessary, the S/T selection switch/connector of SW2/CONN1 or SW3/CONN2 can be respectively set to the T or S position.

- Each line can be selected to T or S interface according to the needs.

- After setting all lines respectively, the slot(s) of STIB must be assigned as CO slot(s) and STA slot(s) together in Admin field 02 for the proper operation of STIB. See the clause 4.2 and the Programming Manual.

The line termination / the power feed to TE

| Line No | Terminating Resistors (Two poles in the right side ) |  |  | Power Feed to TE (Two poles in the left side ) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Connector | Set to ON | Set to OFF | Connector | Set to S | Set to T |
| Line 1 | CONN3 | terminating | open | CONN3 | Feed | open |
| Line 2 | CONN4 | terminating | open | CONN4 | Feed | open |

- Note:

- Two poles in the right side of CONN3/CONN4 should set to the ON position to terminate two terminating resistors on a line.

- Two poles in the left side of CONN3/CONN4 should set to the S position to the power into ISDN terminal.

If the user want to use on-board terminating resistors, the right two short pins of CONN3(for Line1)/CONN4(for Line 2) should be placed on the ‘ ON ‘ position. And If the interface is in “S” mode operation and power feed to external ISDN terminal is not required, the left two short pins of CONN3(or CONN4) should be placed on the “T” position.

On PCB issue 1(6870N01591), the Markings, POWER and TER.RES. may different from above. But the physical tracks are the same as above.

Power is not fed

Power is fed

POWER

TER.RES

T

S

Termination resistor

is not connected.

Termination is

connected.

CONN3/CONN4

OFF

ON

Figure 3.4.5.2.a	The setting method for termination and Power feeding

Check Points

- STIB can be installed in any of the slot No. 1, 4~7.

- PLLU2 MUST be installed on MBU.

- Each interface should be set to the correct type, T or S with S/T selection connectors, according to the needs of user. And the RX and TX pairs of line should be correctly connected to the TX and RX pairs of NT1 or TE according to the interface type of each line.

- Terminating resistors on the line.

- The 2nd pole of SW1 should be set to the proper position according to the type of TEI.

- The clock control cable should be connected to the proper position when multi-ISDN cards are installed in a system. See section 3.5.

- SMEMU MUST be required on MBU for QSIG operation. See SMEMU in section 2.

Insert Figure 3.4.10 in the existing Manual here

Figure 3.4.5.2.b   STIB Installation

Figure 3.4.5.2.b	STIB board

### ISDN PRI ( Primary Rate Interface) Installation

The PRI card provides an interface circuit of PRI(Primary Rate Interface) format digital CO interface. For using this board, the PLLU2 board and the MEMU board should be installed on the MBU. If more than one ISDN board is required, some special cables should be used for clock signal control. CN1 is used to control the clock signal when multi-ISDN boards are installed. Refer to [Clock Control Cable Wiring] of section 3.5.

Function of LED

| LED | Function | Status |  | Remark |
| --- | --- | --- | --- | --- |
|  |  | ON | OFF |  |
| LD1 | Loss Of Signal from the Line | LOS | Normal |  |
| LD2 | AIS | Alarm error | Normal |  |
| LD3 | Frame Alignment Error | Error | Normal |  |
| LD4 | Multi-frame Error | Error | Normal |  |
| LD5 | CRC Error | Error | Normal |  |
| LD6 | The status of the line | In use | All channel idle |  |

- Note

- All LEDs will be the status of OFF when normal operation, but LD6(green LED) will be lit in use.

- When PRI line is plugged out, LD 1 and 3 will be ON.

Function of SW1: default - all OFF when delivering the board

| Pole | Function | ON | OFF | Remark |
| --- | --- | --- | --- | --- |
| 1 | Test Purpose | - | - | Note 1 |
| 2 | CRC | Disable | Enable |  |
| 3 | Not used | - | - |  |
| 4 | Trace | Enable | Disable | Note 2 |

- Note

- This pole is for the purpose of testing PRI, should be OFF during normal operation.

- This pole are for the purpose of developing and maintaining PRIB.

Function of various connectors

| Connector/Switch | Function | Remark |
| --- | --- | --- |
| CN1 | Clock control OUT | Note 1 |
| MJ1 | RJ45 type jack for PRI Line connection | Clause 3.5 |

- Note

- CN1: They are used to control the clock signal when multi-ISDN boards are installed.

See [Clock Control Cable Wiring] on section 3.5.

Insert Figure 3.4.9 in the existing Manual here

Figure 3.4.5.3	PRI Board

Check Points

- One PRI board can be installed only in the slot No.3.

- PLLU2 MUST be installed on MBU.

- For QSIG operation, SMEMU MUST be required on MBU for QSIG operation. See SMEMU in section 2.

- The clock control cable should be connected to the proper position when multi-ISDN cards are installed in a system. See section 3.5.

### DVIB (Digitized Voice Interface Board) Installation

DVIB provides system announcement, ACD/UCD announcement, and User Greeting. If necessary, the DMEU(DVU Memory Expansion Unit) or DMEU4 can be installed on DVIB for the expansion of User Greetings. And, if necessary, the ADPU(ADPCM Unit) can be installed on DVIB for the expansion of user access channels.

Function of LED

| No | Description | Status |  | Remark |
| --- | --- | --- | --- | --- |
|  |  | ON | OFF |  |
| LED1 | The status of channel 1 | In use | Idle |  |
| LED2 | The status of channel 2 | In use | Idle |  |
| LED3 | The status of channel 3 | In use | Idle |  |
| LED4 | The status of channel 4 | In use | Idle |  |
| LED5 | The status of channel 5 | In use | Idle |  |
| LED6 | The status of channel 6 | In use | Idle |  |
| LED7 | The status of channel 7 | In use | Idle |  |
| LED8 | The status of channel 8 | In use | Idle |  |

Function of various connectors

| Connector | Function | Remark |
| --- | --- | --- |
| CN1 | Dummy connector |  |
| CN2 | Serial interface connector( 4pin) for trace | Note 1 |
| CN3 and CN4 | ADPU connection |  |
| CN5 and CN6 | DMEU or DMEU4 connection |  |
| CN13 | For the purpose of development |  |
| PJ1 | RCA jack for Audio Input, DVU MOH input | Blue |
| PJ2 | RCA jack for Audio Output. | Red |

- Note

- CN2: Trace for the purpose of development and maintenance.

The external module for RS 232C serial interface should be required for trace using this connector. See GDK-TRC1 in section 2.

- CN13, PJ2: It is impossible to use them for user access.

These connectors are usually used for the purpose of development and production test. Especially, all the pins of CN13 should be open for the normal operation.

Function of various switches

| Switch | Function | Remark |
| --- | --- | --- |
| SW2 | SRAM back-up when power interruption or failure | ON |
| SW5 | For the purpose of development | OFF |
| SW6 | Not used |  |

- Note

- SW2: This is for the purpose of SRAM back-up for ACD/UCD announcement.

It should be turned ON during installing the DVU/EDVU, to protect announcements in the case of a power fail.

“ Caution :

Danger of explosion if battery is incorrectly replaced. Replace only with the same or equivalent type recommended by the manufacturer. Dispose of used batteries according to the manufacturers instruction.”

Add-on boards: DMEU, DMEU4, ADPU

Figure 3.4.6	Board Layout and ADPU, DMEU/DMEU4 installation

Insert Figure 3.4.14 in the existing Manual here