SECTION 4. ISDN BOARDS

This section briefly introduces ISDN boards, Number Assignment, and the other information.

For more details for installation, see the ISDN part of clauses 2.4.9 System components, 3.4.6 PCB Installation, and 3.5.7 ISDN wiring.

Otherwise, for more details for the features and Admin programming related to ISDN, see Programming Manual.

### Basic Information

- There are three different ISDN cards on GDK-FPII system as follows;

| Board Type | 2BRI | PRI | STIB |
| --- | --- | --- | --- |
| Channels | 4B Ch. (2 BRI) | 30B Ch. (1 PRI) | 4B Ch. (2 BRI) |
| Interface | T-Interface | T-Interface | S/T-Interface |
| Max No. of Cards on the system | 1 Card | 1 Card | 5 Cards |

- *) In case of 2BRI & STIB, 1 port is configured with 2B+D structure, so each board can support 4B channel connection.

- *) In case of STIB, it can be operated with S or T interface mode according to DIP switch position on it and max. 5 STIB cards can be installed in GDK-FP II.

- *) In case of 2BRI & PRI, it can be installed only in the Slot No. 3 of GDK-FP II. So, both of them cannot be installed at the same time to the system.

- *) In case of PRI, MEMU or SMEMU should be installed in the system.

2) 	GDK-FP II system, you can install 2BRI/PRI/STIB within the station and CO line capacity of the system.

| No. of Max. Stations | No. of Max. CO Lines |
| --- | --- |
| 34 | 34 |

- Not achieved the simultaneously. See the system capacity.

3)	Max. 8 ISDN TEs (Terminal Equipments) can be connected to the single S-Interface port (But, only when STIB can support enough power for all 8 ISDN terminals within its capacity or when some or all of them have self-power within STIB’s power capacity), and any kind of ISDN TEs, which are compatible with ETSI ISDN specification, can be connected to the S-Interface. For example, ISDN Phone, G4 Fax, Video Terminal, ISDN PC Card, etc. can be connected.

### Station/CO Line Number Assignments

- As same as any other station/CO cards, the station/CO line number on ISDN card is assigned by PGM 02 in Admin. Programming.

- In PRI board, the 30 continuous CO line numbers are assigned by PGM 02.

- In 2BRI board, the 4 continuous CO line numbers are assigned by PGM 02. Two continuous CO line numbers are assigned per each port (2B+D).

- In STIB with all T-Interface, as same as 2BRI, the 4 continuous CO line numbers are assigned by PGM 02. Two continuous CO line numbers are assigned per each port (2B+D).

- In STIB with all S-Interface, the 4 continuous station numbers are assigned by PGM 02. Two continuous station numbers are assigned per each port (2B+D).

- If STIB is used for S-Interface and T-interface, then system assigns the continuous CO line No. on each T-Interface, and the continuous station No. on each S-Interface as follows;

(If the first Sta. No. is 120 and the first CO No. is 24 )

| STIB Board | Port 1 | S-Interface | Station 120~121 |
| --- | --- | --- | --- |
|  | Port 2 | T-Interface | CO Line 24 ~25 |

- Two continuous station numbers assigned to a single S-Interface act as the same station No. For example, calling to station 120 is the same as calling to station 121.

- If you want to change S-Interface to T-Interface, or vise versa, on STIB board, you have to re-program Slot Assignment (PGM 02) even if the slot assignment is not changed and reset the GDK system.  After that, system works as required.

- In STIB,

To use 'T' interface only - Slot Assignment (PGM 02) at COL only

To use 'T' & 'S' interface - Slot Assignment (PGM 02) at COL & STA

To use 'S' interface only - Slot Assignment (PGM 02) at COL & STA

### Other Information

- ISDN TEs (Terminal Equipment)

- There are many ISDN TEs(ex. ISDN Phone, Video-Phone, G4 Fax, PC-Card, etc.). The GDK-FP II supports all ISDN TEs, which are compatible with Euro-ISDN (ETSI ISDN Specification) till now on.

- ISDN phone can call another station (DKTU,EKTU,SLT) connected to the GDK-FPII and access analog/ISDN CO lines.

- The LCD display on ISDN TE is depending on its own feature, not GDK-FP II system’s feature.

- Etc.

- ISDN station cannot be the master of conference and a member of conference.

- ISDN station cannot call station which is activated to Off-net Call Forward.

- ISDN station cannot be an attendant station.

- ISDN station cannot belong to Hunt Group.