### Cables (5MC1, 5MC2)

There are two kinds of line connection cables, one (5MC1: 6851NF0023A) is for the connection to analog CO lines or PFTU, and the other (5MC2: 6851NF0028A) is for the connection to the terminals or miscellaneous devices. 5MC1 consists of the 25-pair cable with 5 meter line length and a RJ21 type female connector, and 5MC2 consists of the 25-pair cable with 5 meter line length and a RJ21 type male connector. One end of the cable is connected to a RJ21 type connector, and the other is open.

So the RJ21 type connector of the cable can be connected to another RJ21 type connector on the function boards according to the type of connector described above. And open wires of the cable can be connected to a user connection point, e.g. the MDF which a supplier provides.

MDF, which is in wide use in the countries, can be locally used according to the requirements of local regulation.

If necessary, above cables can be supplied from a supplier on the order.

### 2.4.13   BRI Line Terminating Board (GDK-162/100 TERM: 2923NP1656A)

GDK-162/100 TERM unit provides four pairs of 100 ohm terminating resistors. The unit is capable of accommodating 4 BRA lines with RJ45 type jacks. It has metal case.

This can be connected at outside of the system. If a customer use Basic Rate (S or T) interface but the 100 ohm terminating resistors are not introduced on the interface circuit, this board will be useful.

See the parts of 2BRIB Installation and Terminating Resistors on Basic Rate Interfaces in section 3.

### 2.4.14   Trace Tool (GDK-TRC1)

GDK-TRC1, Figure 2.4.7, provides a RS-232 Serial Interface for the maintenance of special cards of GDK-FPII system. It provides 4-wire interface to connect to GDK-FPII boards and RS-232 interface to connect to the serial port of PC. All two cables are offered in the GDK-TRC1 package.

GDK-TRC1 can be connected to the following board for access..

| System Name | Board Name |
| --- | --- |
| GDK-FPII | STIB, WTIB, DVIB |

Caution: This tool should be only used by LG Information & Communications, Ltd. and its agent for the purpose of the maintenance of GDK-FPII system, but not by an end user.

### 2.4.15   Accessories For The Purpose Of Test.

These boards are useful to test GDK-FPII system and functional boards for the purpose of investigation. These are not required for an end customer.

Extension Board

These are dummy boards so that the required function board may be positioned at outside of the system cabinet. They are directly installed on the card slot, and then the function cards are connected to the other side of them.

These boards are used when an engineer or a service man wants to investigate the function boards in state of operation.

| Board Name | Function Board Name | Remark |
| --- | --- | --- |
| BEXT of GDK-FPII | GDK-FPII 4SLI, 4LCO, 2BRI, PRI (slot2,3) | 2923NP1431A |
| UEXT of GDK-FPII | All GDK-FPII boards except above | 2923NP1432A |

Modular connector board

These are the boards for line connection, which contain a RJ21 type connector and RJ-11 type jacks. A RJ21 type connector of the board is connected to the opposite type of RJ21 type connector on the function boards or the system, and RJ-11 type jacks of the board are connected to CO lines or terminals through line cord. It is easy to connect CO line and terminals for the purpose of testing system and terminals.

These boards are used when an engineer or a service man wants to investigate the operation of the system and terminals.

| Board Name | Function Board Name | Remark |
| --- | --- | --- |
| EXT-BLOCK of GDK-FPII | All GDK-FPII extension board except ETIB,WTIB | 2923NP2168A |
| CO-BLOCK of GDK-FPII | GDK-FPII Analog CO boards | 2923NP2169A |

KD-36EXE

KD-33LD

Figure 2.4.1KD Digital Keysets

KD/E-8BTN

KD/E-36EXE

KD/E-36LD

Figure 2.4.2KD/E Digital Keysets

LKD-2NS

LKD-8DS

Figure 2.4.3	LKD Digital Keysets

LKD-30DS

LKD-30LD

GSX-8BTN

GSX-33 EXE

Figure 2.4.4	GSX Electronic Keysets

Note:

For the connection to any digital keyset behind LKD-30DS, 2B-DTIU30 should be inserted in the bottom side of LKD-30DS.

In case of using 2B-DTIU30 and Secondary digital Keyset, It is recommended to use optional mains adapter, which confirms to the appropriate national or local electrical adapter.

2B-DTIU30 Connection

Primary Station

( LKD-30DS)

Secondary Station

(Digital Keyset)

GDK-FPII system

MBU(DTI), DTIB

2 wire

2 wire

Optional

Mains Adapter

(AC/DC)

Figure 2.4.52B-Modules

Note:

For the connection to SLT device (SLT, FAX, Modem etc) behind LKD-30DS, 2B-SLIU30 should be inserted in the bottom side of LKD-30DS.

2B-SLIU30 Connection

Primary Station

( LKD-30DS)

Secondary Station

(SLT)

GDK-FPII system

MBU(DTI), DTIB

2 wire

2 wire

Optional

Mains Adapter

(AC/DC)

Note:

For this type of the connection using the external CTI module (GDK-PC PHONE unit), KD/C or KD/EC keyset should be required.

All the line cords and RS232C cable are provided with GDK-PC PHONE unit in the same package.

The connection method of the external CTI module

GKD-PC PHONE

GDK-PC PHONE

(CTI Module)

KD/C,KD/EC Keyset

(CTI Keyset)

PC

-PC application

-Window 3.1 or 95

environment

2 wire connection

6 wire

line cord

RS-232C

(9 pin)

GDK-FPII system

-MBU(DTI), DTIB

-MP Call Software

(99A or later version)

Note:

For this type of connection using the internal CTI module (CTIU30/CTIU8), LKD Keyset should be required.

The cable with RS232C and RJ11 type plug is provided with CTIU30/CTIU8 in the same package.

The connection method of the internal CTI module

Figure 2.4.6CTI modules

PC

-PC application

-Window 3.1 or 95

environment

The cable with RS-232C(9 pin) and RJ11 type plug

LKD Keyset

with CTI module

-LKD-30DS and CTIU30

-LKD-8DS/2NS and CTIU8

2 wire connection

GDK-FPII system

-MBU(DTI), DTIB

-MP Call Software

(99A or later version)

Note:

A 4-wire and a RS232C cables are provided with GDK-TRC1 unit in the same package.

Caution: This tool should be only used by LG Information & Communications, Ltd. and its agent for the purpose of the maintenance of GDK-FPII system, but not by a end user.

Figure 2.4.7The connection method of the GDK-TRC1

GKD-TRC1

GDK-TRC1

(External trace Tool)

PC

4 wire cable

RS-232C

(9 pin)

GDK-FPII system

- STIB/DVIB/ WTIB