### REVISION HISTORY

| ISSUE | Date | ###### REVISIONS |
| --- | --- | --- |
| ISSUE 1.0 ISSUE 1.1 ISSUE 1.2 ISSUE 1.3 | 1997.05 1997.09 1998.03 1998.05 | - Initial Release - Add the description of ISDN 2BRI installation. - Table 2.3.3 Electrical Specifications. - Power Installation. - Add the description of Ferrite core installation on audio cables. |
| ISSUE 2.0 ISSUE 2.1 | 1998.09 1998.09 | - The manufacturer is changed to LGIC. - Separate from the software programming manual. 1. The design of KSU and board is changed. |
| ISSUE 3.0 | 1998.12 | - The PSU is changed from Linear type to SMPS type. |
| ISSUE 4.0 | 1999.10 | - Update ISDN boards and their installation - Update the cabling method of the whole system - Add SMEMU, 2B-modules, CTI module, GDK-TRC1 module, BWDIDB. - Update the Customer Database Programming (99A version). - Add Alpha-numeric INDEX. |

Blank sheet

### Table of Contents

###### SECTION 1. INTRODUCTION	9

1.1	PURPOSE	9

1.2	REGULATORY INFORMATION	9

1.2.1	Telephone Company Notification	9

1.2.2	Incidence Of Harm	9

1.2.3	Changes In Service	9

1.2.4	Maintenance Limitations	10

1.2.5	Notice Of Radiated Emissions	10

1.2.6	Hearing Aid Compatibility	10

1.2.7	Notice Of Replacement with Lithium Battery	10

###### SECTION 2. GENERAL DESCRIPTION	11

2.1	INTRODUCTION	11

2.2	SYSTEM CAPACITY	12

2.2.1	System Capacity	12

2.2.2	System Configuration Chart	13

2.2.2.1	Overview	13

2.2.2.2	Typical System Configuration Chart	14

2.2.2.3	System Configuration Chart with PRI Line	15

2.2.2.4	System Configuration Chart without PRI Line	16

2.3	SYSTEM SPECIFICATIONS	17

2.4	SYSTEM COMPONENTS	20

2.4.1	KSU (Key Service Unit)	20

2.4.2	PSU (Power Supply Unit)	20

2.4.3	MBU (Main Board Unit)	20

2.4.4	RGU (Ring Generator Unit)	21

2.4.5	PFTU/PFTU-I (Power Failure Transfer Unit)	21

2.4.6	DVIB (Digitized Voice Interface Board)	22

2.4.7	Extension Boards	23

2.4.7.1	DTIB (Digital Terminal Interface Board)	23

2.4.7.2	ETIB (Electronic Terminal Interface Board)	23

2.4.7.3	SLIB/4SLI (Single Line Interface Boards)	24

2.4.7.4	WTIB /WTIU (Wireless Terminal Interface Board / Unit)	25

2.4.8	Analog CO line Boards	25

2.4.8.1	LCOB/4LCO (Loop Start CO Line Interface Board)	26

2.4.8.2	AC15 Board(AC15 private Line Interface Board) - UK	26

2.4.8.3	TLIB (Tie Line Board) -Korea	26

2.4.8.4	BWDIDB (Both-way Direct Inward Dialing Board) - New Zealand	27

2.4.9	ISDN Boards	27

2.4.9.1	ISDN PRI (Primary Rate Interface Board)	27

2.4.9.2	ISDN 2BRI (Basic Rate Interface Board: T interface only)	28

2.4.9.3	ISDN STIB (S/T Basic Rate Interface Board: switchable S/T interface)	28

2.4.10	Add-on boards	29

2.4.10.1	MEMU (Memory Expansion Unit)	29

2.4.10.2	SMEMU (Security Memory Expansion Unit)	30

2.4.10.3	MODU (Modem Unit)	30

2.4.10.4	PLLU2 (Phase Locked Loop Unit2)	31

2.4.10.5	SIU (Serial Interface Unit)	31

2.4.10.6	DTRU (DTMF Receiver Unit)	31

2.4.10.7	MSGU (Message Wait Unit)	31

2.4.10.8	CPTU (Call Progress Tone Detection Unit: CPTU/A, CPTU/B)	32

2.4.10.9	CMU (Call Metering Unit)	32

2.4.10.10	DMEU/DMEU4 (DRAM Memory Expansion Units)	32

2.4.10.11	ADPU (ADPCM unit)	32

2.4.11	Keyset & Terminals	33

2.4.11.1	Digital Keysets and Terminals	33

2.4.11.2	Electronic Keysets and Terminals	34

2.4.11.3	2B-module	36

2.4.11.4	GDK-PC PHONE (CTI)	37

2.4.12	Cables (5MC1, 5MC2)	38

2.4.13	BRI Line Termination Board (GDK-162/100 TERM: 2923NP1656A)	38

2.4.14	Trace Tool (GDK-TRC1)	38

2.4.15	Test Accessories for the Purpose of Test.	38

###### SECTION 3. INSTALLATION	45

3.1	INTRODUCTION		45

3.2	SITE PREPARATION	45

3.2.1	General Site Consideration	45

3.2.2	Back-Board Installation	46

3.2.3	Verify On-Site Equipment	46

3.3	KSU INSTALLATION	47

3.3.1	Mounting KSU	48

3.3.2	RGU Installation	49

3.3.3	PFTU Installation	50

3.3.4	KSU Grounding	51

3.3.5	PSU Installation	52

3.4	PCB INSTALLATION	54

3.4.1	PCB Handling & General Installation	54

3.4.2	MBU Installation	56

3.4.2.1	MEMU Installation	59

3.4.2.2	SMEMU Installation	59

3.4.2.3	PLLU2 Installation	60

3.4.2.4	MODU Installation	60

3.4.2.5	SIU Installation	60

3.4.3	Extension Board Installation	61

3.4.3.1	DTIB Installation	61

3.4.3.2	ETIB Installation	62

3.4.3.3	SLIB/4SLI Installation	63

3.4.3.4	DTRU Installation	64

3.4.3.5	MSGU Installation	64

3.4.4	Analog CO Line Board Installation	64

3.4.4.1	LCOB/4LCO Installation	65

3.4.4.2	AC15 Board Installation - UK	67

3.4.4.3	TLIB Installation - Korea	67

3.4.4.4	BWDIDB Installation - New Zealand	68

3.4.5	ISDN Board Installation	68

3.4.5.1	ISDN 2BRI ( Basic Rate T Interface) Installation	68

3.4.5.2	ISDN STIB ( Basic Rate S/T Interface) Installation	70

3.4.5.3	ISDN PRI ( Primary Rate Interface) Installation	73

3.4.6	DVIB Installation	74

3.5	SYSTEM WIRING	76

3.5.1	Battery Back-Up Wiring	76

3.5.2	RS-32C WIRING on SIU	77

3.5.3	MBU wiring		78

3.5.3.1	External Music Source wiring	79

3.5.3.2	External Paging wiring	79

3.5.3.3	External Paging Port and General Purpose Relays wiring	80

3.5.3.4	Relay Contacts for General Purpose	80

3.5.3.5	Alarm Detection wiring	81

3.5.3.6	Contact Assignments of RJ21 Type Connector for Basic Connection	81

3.5.4	Extension Board wiring	83

3.5.4.1	Digital Keyset and Terminal wiring	83

3.5.4.2	Electronic Keyset and Terminal wiring	85

3.5.4.3	Single line Telephone wiring	87

3.5.4.4	Intercom/Door Phone Box installation	88

3.5.4.5	Contact Assignments of RJ21 Type Connectors for Expansion connection.	90

3.5.5	PFTU Wiring	91

3.5.6	Analog CO line wiring	93

3.5.6.1	LCOB/4LCO wiring	93

3.5.6.2	AC15 Board wiring - UK	94

3.5.6.3	BWDIDB wiring - New Zealand	95

3.5.6.4	TLIB wiring - Korea	96

3.5.6.5	Contact Assignments of RJ21 type connectors on Analog CO line board	97

3.5.7	ISDN wiring		98

3.5.7.1	ISDN 2BRI wiring	101

3.5.7.2	ISDN STIB wiring	102

3.5.7.3	ISDN PRI wiring	103

3.5.7.4	Terminating Resistors on Basic Rate interface(2BRI/STIB)	104

3.5.7.5	Clock Control Connection- Multi ISDN board installation	105

3.5.7.6	QSIG connection	109

3.5.8	Contact Assignments of various types of connectors	110

3.5.8.1	RJ21 type connectors	110

3.5.8.2	Miniature 6-position jack	110

3.5.8.3	Miniature 8-position jack for ISDN connection	111

###### SECTION 4. ISDN BOARDS	112

4.1	Basic Information	112

4.2	Station/CO Line Number Assignments	113

4.3	Other Information	113

###### SECTION 5. CUSTOMER DATABASE PROGRAMMING	114

5.1	INTRODUCTION		114

5.2	TO ENTER THE PROGRAMMING MODE	114

5.3	PERMANENT UPDATE PROCEDURE	115

5.4	NUMBERING PLAN	115

5.5	ADMIN PROGRAMMING INDEX	119

5.6	DEFAULT VALUES	121

TABLE 5.6.1   INITIALIZATION	121

TABLE 5.6.2   FLEXIBLE NUMBERING PLAN – GDK-162	121

TABLE 5.6.3   FLEXIBLE NUMBERING PLAN – GDK-100	122

TABLE 5.6.4   FLEXIBLE NUMBERING PLAN – GDK-FPII	124

TABLE 5.6.5   SLOT ASSIGNMENT	125

TABLE 5.6.6   MSN/SUB-ADDRESS ASSIGNMENT	126

TABLE 5.6.7   FLEXIBLE DID TABLE PROGRAM	126

TABLE 5.6.8   EMERGENCY SERVICE CALL	126

TABLE 5.6.9   LCR TABLE ASSIGNMENT	126

TABLE 5.6.10  SYSTEM ATTRIBUTE - IV	127

TABLE 5.6.11  ISDN COLP TABLE ASSIGNMENT	127

TABLE 5.6.12  STATION BASE PROGRAM 	127

TABLE 5.6.13  STATION LINK PROGRAM	129

TABLE 5.6.14  FLEXIBLE BUTTONS PROGRAM	129

TABLE 5.6.15  CO LINE BASE PROGRAM	130

TABLE 5.6.16  SYSTEM BASE PROGRAM	131

TABLE 5.6.17  TOLL TABLE	136

TABLE 5.6.18  PRINT DATABASE	137

###### APPENDIX A.  Maximum Number of Boards in a System	138

###### APPENDIX B.  Power Consumption	139

APPENDIX C.  PSU Installation	140

###### Alpha-Numeric INDEX	142