SECTION 2. GENERAL DESCRIPTION

### INTRODUCTION

The GDK-FPII Digital Key Telephone System is a fully digital hybrid Key Telephone System, designed to meet the telecommunication needs of small/medium sized business offices.

The GDK-FPII system incorporates state of the art digital technology for command processing and voice switching, utilizing a Pulse Code Modulation/Time Division Multiplexing (PCM/TDM, "A" law) distributed switching matrix.

The GDK-FPII system achieves a high level of flexibility by 1) employing a Universal Card Slot architecture with a cabinet to house plug-in Printed Circuit Boards, and 2) providing support for different types of instrumentation.

The KSU of the GDK-FPII is a wall-mounted cabinet that houses the MBU (Main Board Unit) and contains card slots for the CO line/Key Station/SLT/ISDN/DECT interface boards, and other useful PCBs. There are 7 slots in the system, four (slot #4~7) of them are universal slots and others(slot #1~3) are fixed slots for specific cards. The 1st fixed slot (slot #1) is used to install only one of the DVIB, STIB or WTIB, the 2nd fixed slot(slot #2) is used to install the 4SLI(SLT interface card) card only and the 3rd fixed slot(slot #3) is used to install one of the 4LCO, 2BRI or PRI card.

There are four universal slots (slot #4~7) in the system. Station, CO line, SLT, and another option board can be installed to the system universal card slots up to the system’s maximum configuration. Thus, the system capacity is established by the number of installed interface boards and is not limited by an artificially fixed station to CO line ratio.

The system architecture has been designed to allow a high level of software control over the system's hardware. The software incorporates a vast array of features and capabilities including PC Database Administration, Auto Route Selection, ACD, etc.

The GDK-FPII system supports a combination of Digital Keysets (KD, KD/E, KD/S, KD/C, LKD series), Electronic Keysets(GSX, GSX/E series), various kinds of ISDN terminals, and wireless terminals as well as analogue single line devices. With the keysets, commonly used features are activated by direct button selection. Additionally, many functions may be accessed by dialing specific codes or optionally, by assigning these dial codes to flexible buttons on the keyset. In addition to key telephones, an array of optional terminals is available including DSS/DLS Console, Intercom/Door Box.

With the flexibility of the GDK-FPII extensive feature content, and the capability to use an array of instruments, the GDK-FPII can be tailored to meet the short and long term needs of the most demanding customer requirements.

### SYSTEM CAPACITY

The following Table and Chart provide system capacities and displays the configuration flexibility of the system.

### System Capacity

Table 2.2.1   System Capacities

| Description | Basic & Expansion |  | Total |
| --- | --- | --- | --- |
| Time Slots |  |  | 96 |
| CO Line Ports |  |  | Max. 20 (with analog CO ) 1) Max. 24 (with ISDN BRI) 1) Max. 34 (with ISDN PRI) 1) |
| Station Ports (Digital/Electronic/SLT/DECT) |  |  | Max. 48 ( without GAP-DECT) 1) Max. 64 ( with GAP-DECT) 1) |
| DSS/DLS Consoles |  |  | 3/Keyset |
| Attendant Positions | 1 System, 5 Main, 5Group Attendants |  |  |
| Intercom Links |  | Non-blocking |  |
| Paging All Call External Internal |  | 1 zone 1 zone 5 zones |  |
| Station Speed Dial | 20/station, 24 digits each | 200 |  |
| System Speed Dial | 24 digits each | 200 |  |
| Last Number Redial |  | 48 digits |  |
| Save Number Redial |  | 48 digits |  |
| Memo Dial |  | 48 digits |  |
| SMDR | 100 (without MEME/SMEMU) 2,000 (with MEMU/SMEMU) | 2,000 |  |
| Music Source Inputs | 1 internal / MBU 1 external / MBU | 2 |  |
| External Control Contacts | Flexibly assigned | 2 / MBU |  |
| Alarm/Door Bell Input |  | 1 / MBU |  |
| Power Fail Circuits | 6 lines / PFTU 2 lines / PFTU-I | 20 |  |
| RS 232C Ports | 1 / SIU 2) | 2 |  |
| DTMF Receivers | 2 / MBU 3) 2 / DTRU | 14 |  |
| CO Line Groups |  | 9 |  |
| Intercom Groups |  | 5 |  |
| Hunt/UCD Groups |  | 8 |  |
| DISA Lines |  | All CO ports |  |
| Conference | 3-party 4-party 5-party | no limit 13 5 |  |

- Note:

- Not all maximum capacities may be achieved simultaneously.

For the maximum number of boards in a system, see APPEDIX A.

The 2B-modules are limited by the power capacity.

The maximum extensions with PSU(Linear) are limited by 34 without WTIB and 48 with WTIB, because of its power capacity.

The maximum extensions with PSU2(SMPS) are limited by 48 without WTIB and 64 with WTIB.

- The SIU has one channel of RS-232. Normally, one SIU can be installed in the position of SIU(1) on MBU. However, in case that MODU is not installed, one more SIU can be installed in the position of SIU(2) on MBU. The purpose of each SIU can be programmable in Admin Programming.

- A DTRU contains two DTMF receiver circuits, and can be installed on 4LCO, LCOB, 4SLI and SLIB as a optional.

MBU contains two DTMF receiver circuits.

### System Configuration Chart

### Overview

This typical Configuration Chart is made up according to the physical capacity of system, without 2B-module and Wireless Terminal(GAP-DECT).

Maximum capacity can be limited by software version and the type of PSU.

- With relation to the type of PSU, refer to note 1 on table 2.2.1.

Manufacturing GDK-FPII PSU(linear) has been officially stopped after launching PSU2(SMPS) in 1st half of 1999.

- At the main program version 1.2Xx or earlier,

The number of stations should not be over 34 even though 2B-modules are connected.

- At the main program version 1.4Xx or later,

- If the WTIB is not installed, the number of stations should not be over 48.

- If the WTIB is installed, the number of stations except Wireless Terminal should not be over 32 and the number of stations including Wireless Terminal should not be over 64.

The following configuration charts, figures 2.2.2 to 2.2.5 can be made up in the system installed the software version 1.4Xx or later, and PSU2(SMPS).

### Typical System Configuration Chart

Basic DKT

(6 ports in MBU)

SLT(with 4SLI)

with PRI

with BRI including STIB,

and Analog CO

with Analog CO

0    6   10    16   22    28   34

##### Stations

34

30

(PRI)

24

20

16

12

8

4

##### CO

FIGURE 2.2.2.2	GDK-FPII System Configuration Chart

- Note

- This typical Configuration Chart is made up according to the physical capacity of system, without 2B-module and Wireless Terminal (GAP-DECT).

### System Configuration Chart without PRI Line

0    6   10    16   22    28   34     Stations

0    6    6    12   18    20   14     the number of 2B stations

24

20

16

12

8

4

CO

with BRI including STIB,

and Analog CO

with Analog CO

FIGURE 2.2.2.3.aGDK-FPII System Configuration Chart without WTIB

24

20

16

12

8

4

CO

with BRI including STIB,

and Analog CO

with Analog CO

FIGURE 2.2.2.3.bGDK-FPII System Configuration Chart with WTIB

0    6   10          42    48    54    60     Stations

0    6    6           6    12    10     4     the number of 2B stations

- Note

- Figure 2.2.2.3.b is only the configuration chart when a customer uses the maximum wireless terminals with RMWT 32. If RMWT is set by another value, above will be different configuration chart according to the RMWT.

RMWT can set by multiple of 8 in the range of 8 to 32. A customer can program this RMWT (default 8) in Admin field 02, Flexible button 3.

- RMWT: Registrable Maximum number of Wireless Terminal. For more details, refer to Installation and Maintenance Manual of GDK-162/100/FPII WOTS (Wireless Office Telephone System)

- In above figures, the total extensions are the sum of the quantity of Stations and the number of 2B stations in station axis. With respect to 2B module, see the clause 2B module in Section 2.

### System Configuration Chart with PRI Line

FIGURE 2.2.2.4.aGDK-FPII System Configuration Chart without WTIB

0    6   10    16   22    28   34    38   Stations

0    6    6    12   18    20   14    10   the number of 2B stations

34

30

CO

0    6   10       42    48    54   60    64   Stations

0    6    6        6    12     8    4     0   the number of 2B stations

34

30

CO

FIGURE 2.2.2.4.bGDK-FPII System Configuration Chart with WTIB

- Note

- Figure 2.2.2.4.b is only the configuration chart when a customer uses the maximum wireless terminals with RMWT 32. If RMWT is set by another value, above will be different configuration chart according to the RMWT.

RMWT can set by multiple of 8 in the range of 8 to 32. A customer can program this RMWT (default 8) in Admin field 02, Flexible button 3.

- RMWT: Registrable Maximum number of Wireless Terminal. For more details, refer to Installation and Maintenance Manual of GDK-162/100/FPII WOTS (Wireless Office Telephone System)

- In above figures, the total extensions are the sum of the quantity of Stations and the number of 2B stations in station axis. With respect to 2B module, see the clause 2B module in Section 2.

### SYSTEM SPECIFICATIONS

The following Tables provide general system specifications.

Table 2.3.1   Dimensions & Weights

| Items | Height | Width | Depth | Weight |
| --- | --- | --- | --- | --- |
| Key Service Unit | 463mm 18.2in | 403mm 15.9in | 160mm 6.3in | 15 Kg 31.1 lbs |
| Digital Keyset | 236mm 9.3in | 192mm 7.6in | 84mm 3.3in | 1.5 Kg 3.3 lbs |
| Electronic Keyset | 220mm 8.6in | 245mm 9.6in | 70mm 2.8in | 1.6 Kg 3.5 lbs |
| Digital DSS/DLS Console | 236mm 9.3in | 125mm 4.9in | 62mm 2.4in | 0.9 Kg 2.0 1bs |
| Electronic DSS/DLS Console | 236mm 9.3in | 125mm 4.9in | 62mm 2.4in | 0.9 Kg 2.0 1bs |
| Digital ICM/Door Box | 45mm 1.8in | 140mm 5.5in | 100mm 3.9in | 0.5 Kg 1.1 1bs |
| Electronic ICM/Door Box | 45mm 1.8in | 140mm 5.5in | 100mm 3.9in | 0.5 Kg 1.1 1bs |

Table 2.3.2   Environmental Specifications

|  | degrees C | degrees F |
| --- | --- | --- |
| Operating Temperature | 0-50 | 32-122 |
| Optimum Operating Temperature | 20-26 | 68-78 |
| Storage Temperature | 0-70 | 32-158 |
| Relative Humidity | 0-85% non-condensing |  |

Table 2.3.3    Electrical Specifications

| Power Supply (PSU – linear): 1) AC Voltage Input AC Power AC Input Fuse DC Output Voltages | AC 110-127V / AC 220-240V @ 47~63Hz 100W T 1.6A H250V (220-240V) T 3.15A L250V (110-127V) +5, -5, +28Volts |
| --- | --- |
| Power Supply (PSU2 – SMPS): AC Voltage Input AC Power AC Input Fuse DC Output Voltages | AC 88V~132V, AC 176V~264V @ 47~63Hz 170W T 3.15A H250V +5, -5, +28Volts |
| Back-up Battery : PSU Input Voltage PSU Battery Fuse Charging Current | 24 Volt DC 250 Volt/5A Max. 500mA |
| Ext. Relay Contacts | 1 amp @ 24 Volt DC |
| Music Source Input | Max. 0 dBm @ 600 ohms |
| External Page Port | Max. 0 dBm @ 600 ohms 1 milli-watt max. input |

- Note:

1. Manufacturing GDK-FPII PSU(linear) has been officially stopped after launching PSU2(SMPS) in 1st half of 1999.

Table 2.3.4	Electrical Specifications

|  | 22 AWG Wire | 24 AWG Wire |
| --- | --- | --- |
| Digital Keyset 2-wire loop | 500 m (1.6 Kft) | 330 m (1 Kft) |
| 2B modules( with Power Adapter, 24Vdc/200 mA) System – Primary Keyset Primary Keyset – Secondary Keyset Primary Keyset – Secondary SLT |  | 300 m 300 m 1000 m |
| 2B modules( without Power Adapter) System – Secondary Keyset System – Secondary SLT |  | 10 m 100 m |
| Electronic Keyset 4-wire loop | 500 m (1.6 Kft) | 330 m (1 Kft) |
| Single Line Telephone 2-wire loop | 2,500 m (8.2 Kft) | 1,600 m (5.2 Kft: 490 ohm) |

- Note:  The distance between KSU and SLT may differ from above in some countries, because of the relevant special national specification.

Table 2.3.5	CO Loop Specifications

| Ring Detect Sensitivity | 16-30 Hz, 40 Vrms 30-60 Hz, 30 Vrms |
| --- | --- |
| DTMF Dialing Frequency Deviation Signal Rise time Tone Duration, on-time Inter digit time | less than 1% 3 msec, maximum 75 mesc, minimum 75 mesc, minimum |
| Pulse Dialing Pulse Rate Break/Make Ratio | 10 pps or 20 pps 60/40% or 67/33% |

- Note:  Ring Detection sensitivity and Pulse Ratio may differ from above in some countries, because of the relevant special national specification.

Table 2.3.6	Miscellaneous Specifications

| Main System Memory | Memory size |
| --- | --- |
| Read-Only-Memory, EPROM Random Access Memory, RAM | basic = 2 Megabytes basic = 256 Kbytes expansion = 256 Kbytes (MEMU / SMEMU) |
| Account Codes | Number |
| Number of codes Digits per code | no limit 12 |
| Authorization of codes | Number |
| Number of codes Digits per code | 110 5 |

### SYSTEM COMPONENTS

### KSU (Key Service Unit)

The KSU is a metal frame cabinet designed for wall mounting. It contains a MBU(Main Board Unit), a PSU(Power Supply Unit) and an optional RGU(Ring Generator Unit).

The connecting cables for the extensions and CO lines exit through the outlet in the left bottom side of the KSU using the CHAMP cable. And this cable can be connected to a user installed termination point.

### PSU (Power Supply Unit)

A Power Supply Unit, PSU, is fitted in the Key Service Units when delivering the system. The PSU converts commercial AC power (PSU-linear: 110 ~127 / 220 ~ 240 VAC @ 50-60 Hz, PSU2-SMPS: 110 ~ 240 VAC @ 50-60 Hz) to DC voltages, regulates the voltages, and provides the appropriate DC voltages to the back plane for distribution to other system components. Three DC outputs are provided : ±5 VDC, +28 VDC. LEDs in the PSU front panel indicate valid outputs as well as the presence of AC input power.

The PSU includes circuitry to float charge externally connected 24 volt batteries and control operation of the battery back-up circuits. And the PSU will provide system operating voltages from the batteries if commercial AC power fails. In addition, battery back-up control circuitry is incorporated in the PSU to disconnect the batteries prior to a deep discharge and over charged.

The PSU is fitted in the Key Service Units when delivering the system.

There are two kinds of PSUs; PSU and PSU2. See the clause; table 2.3.3 and the clause 3.3.5.

### MBU (Main Board Unit)

The MBU incorporates the system's memory, master clock, one internal and one external MOH ports, two DTMF receivers, one alarm detection circuit, one external paging port, two general purpose relays, watchdog circuitry and 6 ports digital terminal interface circuits as well as the system's PCM voice processing and main micro-processor. The micro-processor is a 16-bit high speed CPU that receives and transmits signaling information from/to other PCBs, controls feature activation, and PCM time-slot interchange. The MBU contains 256 Kbytes of RAM with battery back-up and can be expanded up to 512 Kbytes with an optional MEMU or SMEMU. RAM is associated with the system database and the real-time-clock is protected from power loss by a long life high energy lithium battery. For digital voice processing, two ROMs contain PCM tone, gain table, etc.

Four(4) EPROMs(2 Mbytes : U1-U4, 27C040) contain the basic system operating software.

There are three fixed slots and four universal slots for extending the system capacity. And four different types of option boards can be installed on the MBU such as PLLU2 for ISDN clock recovery, MEMU for RAM expansion, SMEMU for the functions of QSIG, TAPI and/or PC-ATTENDANT, MODU for modem interface and SIU for serial interface.

There are 3 CHAMP connectors on the MBU’s left most side. The upper one is used to connect 6  digital terminal interface circuits of the MBU, 4 single line interface circuits with 4SLI board that installed on the slot #2, and two dry contacts of general purpose relays. The middle one is used to connect CO lines with CO interface board(s) that installed on slot #3~7. The last one is used to connect extension lines (digital terminals or SLTs) with DTIB or SLIB that installed on universal slots (slot #4~7).

It should be noted that the analog keysets can be connected to the ETIB directly through modular type connectors –RJ11 on the ETIB.

The PSU is fitted in the KSU when delivering the system.

MBU contains 1 internal and 1 external MOH sources, one external paging port, one alarm detection circuit, 2 general purpose relays, 2 DTMF receivers, and 6 digital terminal interface ports on itself.

Add-on boards: MEMU, SMEMU, PLLU2, MODU, SIU

### RGU (Ring Generator Unit)

The Ring Generator Unit provides the 25Hz sinusoidal ring voltage to the 4SLI/SLIB circuits for ringing the SLT. The output of the RGU is 90Vac, 25Hz with RGU-EX and can support simultaneous ringing for five(5) SLTs at a time, so up to 25 SLTs can be ringing by using time sharing technique.

The RGU is installed on the right bottom side of the KSU. And the RGU is electrically connected to the MBU.

### PFTU/PFTU-I (Power Failure Transfer Unit)

PFTU

The PFTU provides the relay contacts for transfer of 6 CO Lines to 6 SLTs in the event of a power or processor failure. The PFTU is installed outside of the KSU and up to 3 PFTU modules can be connected to the KSU with the cascade connection of control wire. The PFTU is equipped with a manual switch that activates the Power Failure Transfer mode for testing purpose.

A 2-pin connector with screws is fitted for the control signal. And a RJ21 type male connector is fitted for the connection between CO line, CO interface circuit, SLT interface circuit and SLT.

PFTU-I

The PFTU-I provides relay contacts for transferring the first 2 CO lines of the 4LCO boards to SLT in the event of a power or processor failure. The PFTU-I can be installed on the 4LCO board only.

### DVIB (Digitized Voice Interface Board)

The Digitized Voice Interface Board (DVIB) provides announcements for the system's ACD/UCD features as well as the System Voice Prompts and Recorded User Greetings. Following table shows the capacity of memory, ADPCM channel of DVIB, and its optional board.

Table 2.4.6	The capacity of DVIB

| Item | Basic | Option(for expansion) | Maximum Capacity |  |
| --- | --- | --- | --- | --- |
|  |  |  | a board | a system |
| ADPCM channel | 4 channels | ADPU; 4channels | 8 channels | 16 channels |
| System Voice Prompt | 2 EPROMs 1 Mbytes 240 seconds | none | 1 Mbyte 4 min. | 1 Mbyte 4 min. |
| ACD/UCD Announcement | 6 SRAMs 768 Kbytes 180 seconds | none | 768 Kbytes 180 sec. (3 min) | 768 Kbytes 180 sec. (3 min) |
| User Greeting | 8 DRAMs 4 MBytes 1024seconds | DMEU:2 Mbytes(512 seconds) or DMEU4:4 Mbytes(1024 seconds) | 8Mbytes 2048 sec. (34 min.) | 16 Mbytes 4096 sec. (68 min.) |

- DVIB allows a DMEU or a DMEU4 to be expanded.

- GDK-FPII allows up to 2 DVIB board to be installed.

- ACD/UCD Announcements are protected from power loss by a long life lithium battery, but User Greetings are not.

- The capacity of ACD/UCD announcements will not be expanded even though 2 boards are installed in a system.

DVIB employs ADPCM channel to store and play System Voice Prompts, ACD/UCD announcements, and Recorded User Greetings. All voice data will be stored and reproduced by using ADPCM technique that compresses the 64Kbps to the 32Kbps in record mode and expands the 32Kbps to the 64Kbps in play mode.

Note that each channel can operate as either record or play mode at a time.

The System Voice Prompts for the standard version of the DVIB are recorded in English. However, customized recordings can be made available for local market requirements.

There are three option board, one is ADPU for ADPCM channel expansion and the others are DMEU and DMEU4 for DRAM expansion.

DVIB can be installed in any of slot No. 1 or slot No. 4~7.

And maximum two boards can be installed in a system.

Add-on boards: ADPU, DMEU, DMEU4

### Extension Boards

The various types of GDK-FPII Extension Boards have capable of supporting various types of terminals as followings,

| Board Name | Function | Remark |
| --- | --- | --- |
| DTIB | Provides 6 Digital Terminal interfaces | 2 wire |
| ETIB | Provides 6 Electronic Terminal interfaces | 4 wire |
| 4SLI | Provides 4 SLT interfaces with a line voltage of 24V. | 2 wire |
| SLIB | Provides 6 SLT interfaces with a line voltage of 24V. | 2 wire |
| WTIB/WTIU | Be capable of accommodating up to 8 Base Stations. (up to 64 wireless terminals) | 4 wire |
| STIB | Be capable of accommodating up to 2 ISDB BRI S interfaces, 4 B channels. (Each interface is T/S switchable) | 4 wire |

- Note:

- 4SLI can be only installed in slot No. 2. SLIB can be installed in the universal slots (slot No. 4~7).

### DTIB (Digital Terminal Interface Board)

The DTIB provides 6 circuits of the LGE proprietary digital interface for the system. It provides 2-wire interface to digital terminals. The DTIB functions as a MUX/DEMUX for the digitized voice and data signals to and from the digital keysets by using the CHAMP connector (CN16) on the MBU. In addition, one LED is mounted on the PCB to indicate the state of connected keysets. LED will turn on when one or more ports are busy. The DTIB allows for either 1 or 2 bearer voice channels from a single hardware port under the control of system software. Note that each keyset requires only a single channel however, 2-channel operation can be provided for simultaneous voice and data applications or future feature as 2B phone.

DTIB can be installed in any of four universal slots. (slot #4~7)

The maximum 4 boards can be installed in a system. See the clause 2.2 SYSTEM CAPACITY.

### ETIB (Electronic Terminal Interface Board)

The ETIB allows GSX series of Electronic Keysets to be connected to the system. The PCB provides for 6 interface circuits and incorporates circuitry for A/D and D/A conversion of voice signals, and interprets and sends signaling data to and from the keysets. The ETIB has an industry standard modular jack connector per interface and mounted on the front edge for connecting to the station termination port. In addition, a LED is mounted on the PCB to indicate the status of ports connected to the ETIB, LED will turn on when one or more ports are busy.

This board can be installed in any one of four universal slots. (slot #4~7)

The maximum 4 boards can be installed in a system. See the clause 2.2 SYSTEM CAPACITY

### SLIB/4SLI (Single Line Interface Board)

SLIB allows single line analog devices to be connected.

Two kinds of SLIB boards are available in GDK-FPII system;

| Board Name | Interface circuit | Feed voltage | Remark |
| --- | --- | --- | --- |
| 4 SLI | 4 cct | 24VDC battery feed | In slot No. 2 |
| SLIB | 6 cct | 24VDC battery feed | In universal slots |

There are two kinds of Single Line Interface boards, one is 4SLI and the other is SLIB. The major difference is the 4SLI board should be installed on slot #2 only and the SLIB can be installed on any one of four universal slots (slot #4~7). The 4SLI provides the capability to connect 4 SLTs to the system, but the SLIB provides the capability to connect 6 SLTs to the system with appropriate A/D and D/A conversion. The 4SLI/SLIB allow SLT access to CO lines connected to the system, other stations, and most features of the system through the use of dial codes. Connections to the single line telephones are made via a 25-pair CHAMP connectors on the MBU. To connect SLTs to the GDK-FPII system, the 4SLI board on slot #2 uses upper most CHAMP connector(CN14) and the SLIB(s) on the universal slot(s) uses lower most CHAMP connector(CN16) of the MBU. The 4SLI/SLIB can be equipped with a Message Wait source(optional MSGU), which activates a lamp in the SLT, if so equipped. The Message Wait source sends a 90Vdc signal to the SLT illuminating the message waiting lamp. And an optional DTMF receiver unit(DTRU) can be installed on the 4SLI/SLIB. A Ring Generator Unit is required in cabinet if a 4SLI/SLIB is installed. The 4SLI/SLIB allows a single line telephone to be connected up to 2.5 Kilometers (8,200 feet, loop resistance: 490 ohm) from the system by using 22 AWG wire. In addition, a LED is mounted on the PCB to indicate the status of interface circuits connected to 4LSI/SLIB, LED will turn on when one or more ports are busy.

4SLI can be only installed in slot No. 2.

SLIB can be installed in the universal slots (slot No. 4~7)

The maximum 4 SLIBs can be installed in a system. See the clause 2.2 SYSTEM CAPACITY

Add-on boards: DTRU, MSGU

### WTIB /WTIU (Wireless Terminal Interface Board / Unit)

The WTIB provides standard interface between the GDK-FPII digital Key Telephone system and DECT (Digital Enhanced Cordless Telecommunications) Network. The system can accommodate one WTIB, which is capable of supporting up to 4 Base Stations. The WTIB can be optionally equipped with WTIU to expand Base Stations, which can support up to additional 4 base stations. So the system allows maximum 8 Base Stations to be connected. Following table shows the capacity of WTIB and its expansion board.

| Items | WTIB | Max. Capacity with WTIU(expansion) |
| --- | --- | --- |
| Maximum Cell Number (Base Station Number) | 4 | 8 |
| Voice channels / Cell | 5 | 5 |
| Registrable Maximum Terminal Number | 8 – 32 by step 8 | 8 – 32 by step 8 |
| Max. Simultaneous Wireless calls | 16 | 16 |

- Note

- Registrable Maximum Wireless Terminal numbers are programmable in Admin field 02.

- WTIB/WTIU contains RJ11 type connectors for the connection to Base Stations.

A WTIB should be installed in any of slot No. 1 or the universal slots (slot No. 4~7).

MEMU( or SMEMU ) should be installed on the MBU for proper operation.

For more details, see INSTALLATION and MAINTENANCE MANUAL (GAP) of GDK-162/100/FII WOTS(Wireless Office Telephone System).

### Analog CO Line Boards

GDK-FPII Analog CO line Boards have capable of supporting various types of analog CO line as followings,

| Board | Function | connection |
| --- | --- | --- |
| LCOB | Provides Loop Start CO interfaces, 4 interfaces | 2 wire |
| AC15 | Provides AC15a signaling interfaces, 2 interfaces; UK | 4 wire |
| TLIB | Provides 2-wire E&M interfaces, 2 interfaces; Korea | 4 wire |
| BWDIDB | Provides Both-way DID interfaces, 4 interfaces; New Zealand | 2 wire |

- Note

Analog CO line Boards except LCOB are designed to comply with special national specification and application, so it may not suitable for the other national specification and/or application.

### LCOB/4LCO (Loop Start CO Line Interface Board)

There are two kinds of Loop-start CO line boards. One is 4LCO and the other is LCOB. The major difference is the 4LCO board should be installed on slot #3 only, but the LCOB can be installed on any one of four universal slots.(slot #4~7)

All these two boards have 4 Loop Start CO lines with ring and loop current detection, A/D and D/A conversions, and pulse signaling. An optional Call Metering Unit(CMU) and a Call Progress Tone detection Unit(CPTU) and a DTMF Receiver Unit(DTRU) can be installed on the 4LCO/LCOB.

An optional Power Failure Transfer Unit-I (PFTU-I) board can be installed on the 4LCO only. This PFTU-I supports power failure transfer function for the first two ports (CO1 & CO2) of the 4LCO boards.

For power failure transfer from LCOB, you have to use external PFTU module.

4LCO and LCOB contain four LEDs, one for each port, to indicate the status of each CO line.

4LCO can be only installed in slot No. 4.

LCOB can be installed in any of universal slots (slot No.4~7).

The maximum four LCOB can be installed in a system. See the clause 2.2 SYSTEM CAPACITY.

Add-on boards: DTRU, CPTU/A, CPTU/B, CMUPR/CMU50/CMU12/CMU16/CMU12PR/CMU50PR

### AC15 Board (AC15 Private Line Interface Board) - UK

The AC15 board provides an analog interface between outside tone controlled private lines and the system. An AC15 board has 2 interface circuits and each AC15 private line interface circuit provides two-pair(transmitting and receiving) audio interface circuit and is designed to comply with the tone signaling requirements of UK. There are two private line interface circuits per one board and each AC15 private line interface circuit provides the A/D and D/A conversions for audio and signaling to and from the system and AC15 private line. The AC15 board contains two LEDs to indicate the status of each line.

It is designed to comply with the UK specification and application, so it may not suitable for the other national specification and/or application.

This board can be installed in any one of four universal slots.(slot #4~7).

The maximum four AC15 boards can be installed in a system. See the clause 2.2 SYSTEM CAPACITY in section 2.

### TLIB (Tie Line Board) -Korea

TLIB provides 2 interface circuits that can support 2-wire E&M or Ring Down according to the administration. Each line supports E&M type V of E&M line.

The TLIB board contains two LEDs to indicate the status of each line.

It is designed to comply with the Korean specification and application, so it may not suitable for the other national specification and/or application.

This board can be installed in any one of four universal slots.(slot #4~7).

The maximum four TLIBs can be installed in a system. See the clause 2.2 SYSTEM CAPACITY.

Add-on board: DTRU

### BWDIDB (Both-way Direct Inward Dialing Board) - New Zealand

BWDIDB provides 4 both-way DID interfaces, which support DTMF or Pulse dialing. It supports both Incoming and Outgoing calls with pulse or DTMF.

Each interface consists of polarity reversal, pulse dialing, call direction control, open loop control, loop detection and polarity detection circuit.

The BWDIDB contains four LEDs to indicate the status of each line. DTMF Receiver Unit (DTRU), Call Progress Tone detection units (CPTU/A) can be installed optionally on the BWDIDB.

It is designed to comply with the New Zealand specification and application, so it may not suitable for the other national specification and/or application.

This board can be installed in any one of four universal slots.(slot #4~7).

The maximum four BWDIDBs can be installed in a system. See the clause 2.2 SYSTEM CAPACITY.

Add-on boards: DTRU, CPTU/A

### ISDN Boards

GDK-FPII ISDN Boards are capable of supporting various types of ISDN as followings,

| Board | Function | connection |
| --- | --- | --- |
| PRI | Provides ISDN Primary Rate interfaces, 1 interface /30 B channels | 4 wire |
| 2BRI | Provides ISDN Basic Rate interfaces(T), 2 interfaces /4 B channels | 4 wire |
| STIB | Provides ISDN Basic Rate interfaces(T/S switchable), 2 interfaces /4 B channels | 4 wire |

### ISDN PRI (Primary Rate Interface Board)

PRI (Primary Rate Interface) board has capability of one(1) PRI line, which has a interface circuit for 30 channel time slots with 2.048Mbps data rate speed. PRI will be positioned at reference point Ia(T) on ETS 300. 001, that is TE slave without power feeding. And, Data & bit clock are transmitted by the NT master and extracted by the TE slave.

PRI contains reset circuit, address decoder and DBID, and the board line’s specific circuitry with PEB2254. The PRI has no local micro-processor and the micro-processor on the MBU controls PRI card directly.

The PRI board can be installed only in the slot No. 3.

The PLLU2 MUST be installed for proper operation.

The MEMU( or SMEMU ) should be installed on the MBU for proper operation.

Only 1 PRI board can be installed in a system, and total CO lines should not be more than 34 lines(channels) including PRI and Analog lines..

QSIG function can be supported through PRI (software version 99A), and SMEMU MUST be used for this function.

### ISDN 2BRI (Basic Rate Interface Board: T interface only)

2BRI has specified to support two interface circuits that allow interfacing the ISDN BRI. Data and bit clock are transmitted by the NT master and extracted by this board. The GDK-FPII system will be positioned at reference point Ia on ETS 300. 012, that is TE slave without power feeding.

2BRI contains reset circuit, address decoder, the circuitry to control the highway interface (DBID), and the board line’s specific circuitry with PEB2086 for physical layer and data link layer. The 2BRI has no local micro-processor and the micro-processor on the MBU controls 2BRI card directly.

The 2BRI board can be installed only in the slot No. 3.

The PLLU2 MUST be installed for proper operation.

QSIG function can be supported through 2BRI (software version 99A), and SMEMU MUST be used for this function.

### ISDN STIB (Basic Rate Interface Board: switchable S/T interface)

The S,T interface is based on the existing interface described in ETSI 300. 012 which is based upon ITU_T Recommendations I.430 and provides modifications and further requirements.

This is applied at the S or T reference points for the basic interface structure defined in ITU_T I.412. Layer 1 interfacing requires a balanced metallic transmission medium, for each direction of transmitting capability to support 192Kbps(2B+D). Data & bit clock are transmitted by the NT master and extracted by TE slave. The GDK-FPII system can be positioned at reference point Ia(T) or Ib(S) on ETS 300. 012, that is TE-slave without power feeding or NT-master with power feeding. By selecting the jumper position and DIP switch on each line interface, the STIB can support either S-interface (Line card function) or T-interface (Trunk function).

SCC1 of CPU(MC68LC302) is assigned as 512Kbps HDLC communication to main processor on MBU. SCC2 of CPU is assigned as 38400bps RS-232C channel. This RS-232C channel will be used only in development stage for software and hardware debugging.

The board line’s specific circuitry contains PEB2086 for physical layer and data link layer.

STIB should be installed in the slot #1 and/or in any one of universal slots (slot #4~7).

The PLLU2 MUST be installed for proper operation.

The maximum 5 STIB boards can be installed in a system. But with respect to total CO lines and extensions, See the clause 2.2 SYSTEM CAPACITY.

QSIG function can be supported through STIB T-interface only (software version 99A), and SMEMU MUST be used for this function.

### Add-on boards

This chapter describes add-on boards, which can be installed on various types of boards to support the proper function

| Board Name | Function | position |
| --- | --- | --- |
| MEMU | Expands SRAM size. | MBU |
| SMEMU | Expands SRAM size and provides QSIG,CTI, and PCATTN. | MBU |
| MODU | Provides a 1200/2400 baud modem for local access | MBU |
| PLLU2 | Provides the synchronized system clock when ISDN installed. | MBU |
| SIU | Provides additional an RS232 serial interface. | MBU |
| DTRU | Provides further 2 DTMF receivers | 1) |
| MSGU | Provides Message Waiting signals to 6 SLT ports | 2) |
| CPTU/A,CPTU/B | Provides Call progress detection function | 3) |
| CMU | Provides Call Metering signal detection | 4) |
| DMEU/DMEU4 | Expands DRAM size for User Greetings(2M/4M) | DVIB |
| ADPU | Expands 4 ADPCM channels | DVIB |

- Note:

- DTRU can be installed on 4SLI, SLIB, LCOB, BWDIDB, and TLIB.

- MSGU can be installed on 4SLI and SLIB.

- CPTU/A, CPTU/B can be installed on LCOB, BWDIDB.

- Each type of CMU can be installed on the LCOB according to the local market condition and regulatory requirements.

- MODU, PLLU2 (issue3 or later), DTRU, MSGU, CPTU/A, CPTU/B and ADPU can be commonly used in both GDK-100 and GDK-FPII systems. But PLLU2 issue 2 or earlier can be only used in GDK-FPII system.

- CMU and DMEU/DMEU4 can be commonly used in GDK-162, GDK-100, and GDK-FPII systems.

### MEMU (Memory Expansion Module)

The MBU contains the basic system RAM memory. The RAM provides the memory for the system database, speed dial numbers, SMDR buffer, etc. The basic RAM is 256 Kbytes and can be expanded up to 512 Kbytes with the MEMU.

MEMU is required for following function.

- Increase the SMDR record buffer from 100 to 2,000

- On-demand Ring

- ISDN CLIP and CO Message Wait: up to 1000

- ISDN Flexible DID table: 000~999 (* User programmable bins: 000~299)

- MSN

- ACD Statistics

- Expanded LCR Table

- WTIB (DECT GAP)

- Enhanced CCR  1)

- Note:

- Above features are supported by the software 98FEB or later version. But the Enhanced CCR is only supported by the software 99A or later version.

MEMU is optionally installed on the MBU.

MEMU or SMEMU MUST be installed for the operation of PRI and/or WTIB.

### SMEMU (Security Memory Expansion Unit)

The SMEMU is an enhanced version of existing MEMU. So it provides all the function of MEMU. In addition, the SMEMU allows QSIG, TAPI and/or PC-ATTENDANT to be operated in GDK-FPII system. The memory size is 256 Kbytes.

SMEMU is required for following features.

- Increase the SMDR record buffer from 100 to 2,000

- On-demand Ring

- ISDN CLIP and CO Message Wait: up to 1000

- ISDN Flexible DID table: 000~999 (* User programmable bins: 000~299)

- MSN

- ACD Statistics

- Expanded LCR Table

- WTIB (DECT GAP)

- Enhanced CCR			1)

- QSIG				1), 2)

- PC-Attendant			1), 2)

- GDKSP based on TAPI 2.1		1), 2)

- Note:

- Above features are supported by the software 98FEB or later version. But the features marked 1) are only supported by the software 99A or later version.

- SMEMU should be required for the features marked 2).

The SMEMU can be optionally installed on the position of MEMU of MBU.

SMEMU MUST be installed on the MBU for the proper operation of QSIG, TAPI and/or PC-ATTENDANT in the system.

SMEMU or MEMU MUST be installed for the operation of PRI and/or WTIB.

### MODU (Modem Unit)

The Modem Unit provides an asynchronous modem for access to the system database and fault reporting features from a remote site. The module is optionally installed on the MBU and incorporates a 1200/2400 baud modem. The modem may be connected to a pre-selected CO line through the system switching matrix. The remote side which contains an external modem port, may be connected to the system through this MODU with pre-selected CO line. The MODU port is independent with the SIU standard RS-232C port, allowing system database access, etc. without the need to interrupt the SMDR output.

Basically, the system can support two(2) serial channels at a time. One is SIU(1) for serial RS-232C interface and the other is one of SIUs(2) for serial RS-232C interface or MODU for modem function.

So, it should be noted that if the MODU is installed, the system supports one SIU(1) module.

### PLLU2 (Phase Locked Loop Unit)

PLLU2(PLL Unit 2) is an option board of MBU and generates 32.768MHz clock which is synchronized to ISDN line. The 32.768MHz clock is provided to GSXD on the MBU. It consists PLL circuit and clock (from Digital Trunk interface board) monitoring circuit.

For the system synchronization, the clock should be extracted from 2BRI, PRI or STIB. But, if clock is not generated from 2BRI, PRI or STIB(clock from ISDN card is not present), this unit generates master clock(32.768MHz) when ISDN card is absent, this unit generates master clock(32.768MHz) using internal clock.

The PLLU should be installed on MBU when any ISDN card is installed in the system.

### SIU (Serial Interface Unit)

The Serial Interface Unit provides a RS-232C interface port with 9-pin connector. The maximum baud rate is 9600bps(default). The SIU is useful for system maintenance, PC based admin., SMDR print out and SMDI for PC based voice mail system. And two SIU boards can be installed on the position of SIU(1) and SIU(2) of the MBU when the MODU is not installed. Followings are the list of the system’s output;

- Administration database

- Off-line SMDR (on-demand)

- On-line SMDR

- Statistical Information

- SMDI (voice mail)

- System trace data

The purpose of each SIU(1 & 2) can be defined by admin. programming.

### DTRU (DTMF Receiver Unit)

One DTMF Receiver Unit contains 2 DTMF receiver circuits. The DTRU can be optionally installed on the 4LCO/LCOB and 4SLI/SLIB and each receiver is time-shared under the control of the system software.

The DTRU can be optionally installed on 4SLI, SLIB, LCOB, BWDIDB, and TLIB.

The maximum DTMF receivers including DTMF receivers on MBU are 14 in a system.

### MSGU (Message Wait Unit)

The Message Wait Unit sends a 90Vdc signal to the SLT for illuminating the Message Wait lamp. The Message Wait Unit contains 6 circuits to support 6 SLTs.

The MSGU can be installed on 4SLI/SLIB.

### CPTU (Call Progress Tone Detection Unit; CPTU/A, CPTU/B)

The Call Progress Tone Detection Unit is used to detect specific call progress tones which are supplied from the CO or PABX. Tone detection is employed to support features such as ACNR(Automatic Called Number Redial).

CPTU provides 2 tone detection circuits that are time-shared under the control of the system software, as a system resource regardless its position.

CPTU have two versions; CPTU/A and CPTU/B. CPTU/A has wide tone detection range (305Hz - 640Hz), and CPTU/B has narrow tone detection range (350Hz, 620Hz, 440Hz, 480Hz  1% ).

Any Kind of CPTU can be optionally installed on the LCOB and BWDIDB.

It is important to check the local specification regarding call progress tone to use CPTU/B.

### CMU (Call Metering Unit)

The Call Metering Unit detects call metering signals from the CO or PABX to monitor call duration/charges. There are six kinds of CMUs as followings,

- 12KHz

- 12KHz and Polarity Reversal

- 16KHz

- 50Hz Longitudinal

- 50Hz Longitudinal and Polarity Reversal

- Polarity Reversal

The CMU can be optionally installed on the 4LCO/LCOB and each CMU supports one CO line.

Each type of CMU can be installed on the LCOB according to the local market condition and national regulatory requirements.

### DMEU/DMEU4 (DRAM Memory Expansion Unit)

The DMEU or DMEU4 allows for expansion of the DRAM memory on the DVIB. The DMEU expands to 512 seconds for Recorded User Greetings time. And the DMEU4 expands to 1024 seconds for Recorded User Greetings time.

DMEU or DMEU4 can be optionally installed on the DVIB. See clause 2.4.8 DVIB.

### ADPU (ADPCM Unit)

The ADPU allows for expansion of ADPCM channels on DVIB. The ADPU expands to 4 channels for ADPCM.

ADPU can be optionally installed on DVIB,

### Keyset& Terminals

In addition to supporting analogue Single Line devices, by selecting appropriate interface boards, the system will support either or both the LG Digital Keysets /Terminals or the LG Electronic Keysets /Terminals.

### Digital Keysets and Terminals

Various types of digital terminals are used with GDK-FPII MBU,DTIB as below:

| Model | Description | Model | Description |
| --- | --- | --- | --- |
| KD-36EXE | 24 Flexible Button Display | KD/E-36EXE | 24 Flexible Button Display |
| KD-36ENH | 24 Flexible Button Normal | KD/E-36ENH | 24 Flexible Button Normal |
| KD-24EXE | 12 Flexible Button Display | KD/E-24EXE | 12 Flexible Button Display |
| KD-24ENH | 12 Flexible Button Normal | KD/E-24ENH | 12 Flexible Button Normal |
| KD-33LD | 8 Flexible Button Large Display | KD/E-8BTN | 8 Flexible Button Normal |
| KD-DSS | 48 Button DSS/DLS Console | KD/E-36LD | 24 Flexible Button Large Display |
|  |  | KD/E DSS | 48 Button DSS/DLS Console |
| LKD-30DS | 30 Flexible Button Display | KD-Digital Phone Box | Digital Intercom Box |
| LKD-8DS | 8 Flexible Button Display | CTI module 1) | CTI module |
| LKD-2NS | 2 Flexible Button Normal |  |  |
| LKD-30LD | 30 Flexible Button Large Display |  |  |
| LKD-48DSS | 48 Button DSS/DLS Console |  |  |

- Note:

- This type of terminal and KD/C and KD/EC series Keysets will not be supplied any more, but CTIU8/CTIU30 which are inserted on the bottom side of LKD Keysets, are supplied for CTI operation. See GDK-PC PHONE (CTI) in this section.

- Minimum software requirements for LKD operation: 1.2 or later version (except 1.4 version)

Digital Keysets

General Description

Each Digital Keyset has a standard 12 button dial-pad, color coordinated handset, an integral Wall Mount kit, a slide-out Directory Tray, an array of  “Flexible Buttons”, and 12 Fixed Feature Access keys. All of the Flexible and Fixed Buttons, except Volume, incorporate a long-life, super-bright LED to indicate the feature or circuit status. The number of “Flexible Buttons” differ from mdes in above table . The fixed feature access keys are the same for each keyset 8/2 button Keyset. Each Digital Keyset includes a RJ-11 type jack for connection to the system. See Figures 2.4.1 to 2.4.3.

Speaker Phone

Each Digital Keyset except LDK-2NS is equipped with circuitry that enables the telephone to be used hands-free in two way conversations. This circuitry provides voice-switched speaker phone operation which gives channel control to the party with the highest energy level. The speaker phone circuit is controlled by the user with the MON button at KD, KD/E and the ON/OFF button at LKD Keyset.

Volume Control

The volume control is a rocker arm button. Pressing the right side increases volume, the left decreases volume. The volume button controls the volume level of voice and ring signals received at the speaker in the keyset. Also, in the Dial-By-Name (Directory Dial) mode, the volume button scrolls the name display up and down, scrolling display information to find a telephone number in the directory.

LCD Display

The Display Keysets with EXE suffix in model name incorporate a 2-line, 48-character Liquid Crystal Display (LCD). The LCD provides alpha-numeric display of various information to assist the user in operation of features. In the idle mode, the display will show the station name or number on the top line and the time and date on the second line.

And The Large Display Keysets with LD suffix also incorporate a 7-line, 112-character large Liquid Crystal Display (LCD)

Digital DSS

The Digital Direct Station Selection/Direct Line Selection (DSS/DLS) Consoles can be connected to any port of a DTIB in place of a Keyset. Up to 6 DSS/DLS Consoles can be associated with a Keyset in the system database. Each console incorporates a 4 by 12 matrix of “Flexible Buttons”, a total of 48 Buttons, each with an associated LED for status indications. The DSS/DLS Console includes a RJ-11 type jack for connection to the system.

Digital Phone Box

The Digital Phone Box provides hands-free intercom conversation from any location that generally does not require a fully functional Key Telephone. The Digital Phone Box is connected to any port of a DTIB in place of a Keyset. The Digital Phone Box includes a volume control slide switch and has 2 feature buttons with LEDs; one to call preprogrammed stations CALL and the second to block incoming calls DND.

### Electronic Keysets and Terminals

Electronic terminal means the Keysets that can be connected to the LG Analog Key Telephone System such as GHX system.

Various types of electronic terminals are used with GDK-FPII ETIB as below:

| Model | Description | Model | Description |
| --- | --- | --- | --- |
| GSX 8 | 8 Button Keyset | GSX/E 8 | 8 Button Keyset |
| GSX 21 ENH | 21 Button Enhanced Keyset | GSX/E 21 ENH | 21 Button Enhanced Keyset |
| GSX 21 EXE | 21 Button Executive Keyset | GSX/E 21 EXE | 21 Button Executive Keyset |
| GSX 33 ENH | 33 Button Enhanced Keyset | GSX/E 33 ENH | 33 Button Enhanced Keyset |
| GSX 33 EXE | 33 Button Executive Keyset | GSX/E 33 EXE | 33 Button Executive Keyset |
| GSX 48DSS | 48 Button DSS/DLS Console |  |  |
| KD-ICMB | Electronic Intercom/Door Box 1) |  |  |

- Note:

All the types of electronic terminals can be connected to the GDK-FPII ETIB. Almost every terminal can be also connected to the LG analog KTS, but KD-ICMB can be connected only to the LG Digital KTS, not to the LG analog KTS. On the contrary, GSX Phone Box can be connected only to the LG Analog KTS, not to the LG Digital KTS

Following table shows three types of Phone Boxes and the system to which they can be connected.

| Model | Digital KTS |  | Analog KTS |
| --- | --- | --- | --- |
|  | The DTIB of Digital KTS | The ETIB of Digital KTS |  |
| KD-Digital Phone Box | Yes | X | X |
| KD-ICM Box | X | Yes | X |
| GSX Phone Box | X | X | Yes |

Electronic Keysets

General Description

Each Electronic Keyset has a standard 12 button dial-pad, color coordinated handset, an integral Wall Mount kit, a slide-out Directory Tray, volume control slide switches, an ICM control switch, an array of “Flexible Buttons”, and 5 or 11 Fixed Feature Access keys. All of the Flexible and Fixed Buttons, incorporate a long-life LED, indicating the feature or circuit status. Each Electronic Keyset includes a RJ-11 type jack for connection to the system. See Figure 2.4.4.

Speaker Phone

Each Electronic Keyset is equipped with circuitry that enables the telephone to be used hands-free in two way conversations. This circuitry provides voice-switched speaker phone operation which gives channel control to the party with the highest energy level. The speaker phone circuit is controlled by the user with the MON button.

Volume Control

The Electronic Keysets have a volume control slide switch RING VOL, which controls the volume of all tone signals sent to the speaker of the Keyset including muted ring. In addition, each 21 and 33 Button Electronic Keyset includes a second volume control slid switch SPKR VOL to control the level of all voice signals sent to the speaker of the Keyset. Moving the control to the right increase volume, to the left decrease volume.

H-T-P switch

The Electronic Keysets are equipped with a three position slide switch which determines the signaling mode for incoming intercom calls.

| Position | Function |
| --- | --- |
| “HF” position | Intercom call announce w/hands free reply |
| “TN” position | Intercom tone ring |
| “PV” position | Intercom call announce w/Privacy |

LCD Display

The 21 33 Button Executive Keysets incorporate a 2-line, 48-character Liquid Crystal Display (LCD). The LCD provides alpha-numeric display of various information to assist the user in operation of features. In the idle mode, the display will show the station name or number on the top line and the time and date on the second line.

Electronic DSS

The Electronic Direct Station Selection/Direct Line Selection(DSS/DLS) Consoles can be connected to any port of an ETIB in place of a Keyset. Up to 6 DSS/DLS Consoles can be associated with a Keyset in the system database. Each console incorporates a 4 by 12 matrix of “Flexible Buttons”, a total of 48 Buttons, each with an associated LED for status indications. The DSS/DLS Console includes a RJ-11 type jack for connection to the system.

Electronic Phone Box

The Intercom/Door Box, KD-ICM Box, provides hands-free intercom conversation from any location that generally does not require a fully functional Key Telephone. The ICM/Door Box is also used to monitor areas such as a door or entry-way. The ICM/Door Box is connected to any port of an ETIB in place of a Keyset. The ICM/Door Box incorporates a volume control slide switch and has 2 feature buttons with LEDs; one for calling preprogrammed stations CALL and The second to block incoming calls DND.

### 2B-module

The 2B-module allows another terminal, DKTU (Digital Key Telephone Unit) or SLT (Single line Telephone) behind LKD-30DS to be connected. There are 2 kinds of 2B-modules; 2B-DTIU30 for the connection to another DKTU behind LKD-30DS, and 2B-SLIU30 for the connection to another SLT behind LKD-30DS. These 2B-modules can be only installed in LKD-30DS. See Figure 2.4.5 2B modules.

In GDK-FPII system, LDK-30DS with 2B-module can be connected to any port of Digital terminal Interfaces.

The advantage of 2B module in GDK-FPII system;

- Increase the system capacity. See the SYSTEM CAPACITY in section 2.

- Simple wiring with two terminals.

- DTIB is capable of connecting to single line device such as Fax/Modem without any SLIB board.

2B modules are supported in the following condition

- Software: 1.2 or later version (except 1.4 version)

- Hardware: Digital Terminal Interface only, LKD-30DS, 2B-DTIU30 or 2B-SLIU30

- Line length. See Table 2.3.4 Electrical Specifications

- Note:

- In case of using 2B-DTIU30 and the secondary digital keyset, it is recommended to use the separate mains adapter, which confirms to the appropriate national or local electrical adapter for the secondary DKTU.

- The separate mains adapter can be get in local market, the recommended specification is;

- Output capacity: 24 to 30Vdc, more than 200 mA

- Size of output plug: the inside diameter - 2.1mm, the outside diameter: 5.5mm

The following table shows LKD series which the 2B module can be installed.

| DKTU | 2B-DTIU30 | 2B-DTIU30 |
| --- | --- | --- |
| LKD 30DS | Yes | Yes |
| LKD 30LDS | X | X |
| LKD 8DS | X | X |
| LKD 2NS | X | X |

- Note: 2B-modules and extensions are limited as followings,

| Classification | Capacity |  |
| --- | --- | --- |
|  | 1.2 or earlier S/W version | 1.4 or later S/W version |
| CO lines | 34 | 34 |
| Max. Extensions | 34 | 48 without WTIB 64 with WTIB |
| Max. 2B-modules | 16 | 24 |
| 2B module numbering | 38 | 48 or (the last number of 1B + 1) |

- At the program 1.2 or earlier version,

- the number of 2B modules should not be over 16,

- the number of stations should not be over 34 even though the 2B-modules are connected.

- At the program version 1.4 or later version,

- the number of 2B modules should not be over 16,

- if the WTIB is not installed, the number of stations should not be over 48 even though the 2B-modules are connected,

- if the WTIB is installed, the number of stations except wireless terminals should not be over 32 even though the 2B-modules are connected,

- Above table is subject to use PSU2 extended capacity.

- The maximum extensions with PSU(Linear) are limited by 34 without WTIB/ 48 with WTIB, because of its power capacity.

- The maximum extensions with PSU2(SMPS) are limited by 48 without WTIB/ 64 with WTIB.

### GDK-PC PHONE (CTI)

GDK-PC PHONE is based on Microsoft TAPI (Telephony Application Programming Interface) Standard Specification and Windows 3.1 or Windows 95 environment. With Computer Telephony, a customer can use computer to take an advantage of a wide range of sophisticated communication features and services over a telephone line. GDK-PC PHONE includes the various value added features to integrate Computer and Telephone, to manage all the details associated with telephone business.

There are 2 kinds of hardware configurations, one is an external CTI module with KD/C or KD/EC series DKTU, the other is an internal CTI module with LKD series DKTU. See Figure 2.4.6 CTI module.

External CTI module package

An external CTI module contains three connectors, 2 RJ-11 type jacks and a RS-232C. One of RJ-11 type jack is used for the connection to Digital Terminal Interface port of the system through 2-wire telephone line, and the other is for the connection to KD/C or KD/EC series through 6-wire telephone line. And RS-232C, comprised of 9-pin connector, is for the connection to a computer. All the line cords and RS-232 cable are provided in the same package.

External CTI module allows only KD/C or KD/EC series to be connected, but not another type of DKTU.

Internal CTI module package

An internal CTI module can be installed on the bottom side of LKD series DKTU. And the cable with RS-232C 9-pin connector and RJ11 type plug is provided for the connection to a computer in the package.

There are two kinds of internal CTI modules, CTIU30 and CTIU8. CTIU30 can be installed in LKD-30DS, and CTIU8 in LKD-8DS and LKD-2NS, but not possible in LKD-30LDS with Large Display.

| DKTU | CTIU30 | CTIU8 |
| --- | --- | --- |
| LKD 30DS | Yes | X |
| LKD 30LDS | X | X |
| LKD 8DS | X | Yes |
| LKD 2NS | X | Yes |

For more details, see “ Installation And User’s Guide for GDK-PC PHONE”.