# SECTION 6.  FEATURE DESCRIPTION

6.1  NUMBERING PLAN

6.1.1  Numbering Plan for GHX-308A

| NUMBER (GHX-308A) | FLEX BUTTON |  |  |  | STATION |  |  |  |  | FEATURE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | K | 8 | B | A |  | K | S | 8 | 4 |  |
| 0 | Y | Y | . | . |  | Y | Y | Y | Y | Call to Attendant |
| 100-699 | Y | Y | . | Y |  | Y | Y | Y | Y | Intercom Call |
| 700-705 | Y | Y | . | Y |  | Y | Y | Y | Y | Call Park |
| * 700-705 | . | . | . | Y |  | Y | Y | Y | Y | Call Park Retrieve |
| 710-717 | Y | Y | . | Y |  | Y | Y | Y | Y | Hunt Group |
| 731 | . | . | . | Y |  | . | . | . | . | Wake-up Registration -Effective until Cancel |
| 732 | . | . | . | Y |  | . | . | . | . | Wake-up Registration - One time Only |
| 733 | . | . | . | Y |  | . | . | . | . | Wake-up Cancel |
| 734 | . | . | . | Y |  | . | . | . | . | Erase Password |
| 735 | . | . | . | Y |  | . | . | . | . | Change System Date Format (MMDDYY or DDMMYY) |
| 746 | . | . | . | Y |  | . | . | . | . | Change System Time Format (am/pm or 24 hour) |
| 785 | . | . | . | Y |  | . | . | . | . | Station COS Change |
| 736 | . | . | . | Y |  | Y | . | . | . | Software Version Display |
| 737 | . | . | . | Y |  | . | . | . | . | Custom Message Registration |
| 740 | . | . | . | Y |  | . | . | . | . | Disable Outgoing CO Line Access |
| 739 | . | . | . | Y |  | . | . | . | . | Setting Time and Date |
| 774 | Y | Y | Y | Y |  | Y | . | Y | . | Dial Memo Button Assignment |
| 3 | . | . | . | Y |  | Y | Y | Y | Y | Camp-on |
| 741 | Y | Y | . | Y |  | Y | Y | Y | Y | Wake-up Registration - Effective until cancel |
| 742 | Y | Y | . | Y |  | Y | Y | Y | Y | Wake-up Registration - One time only |
| 743 | Y | Y | . | Y |  | Y | Y | Y | Y | Wake-up Cancel |
| 744 | Y | Y | . | Y |  | Y | Y | Y | Y | Access Authorization Code Registration |
| 745 | Y | Y | . | Y |  | Y | Y | Y | Y | Access Authorization Code Change |
| 747 | Y | Y | . | Y |  | Y | Y | Y | Y | Station COS Change (to COS 5) |
| 748 | Y | Y | . | Y |  | Y | Y | Y | Y | Original COS Restoration |
| 750 | . | . | . | Y |  | Y | Y | Y | Y | Last Number Redial |
| 753 | . | . | . | Y |  | Y | Y | Y | Y | DND |
| 754 | . | . | . | Y |  | Y | Y | Y | Y | Station Call Forward |
| 755 | . | . | . | . |  | . | Y | . | . | Station Speed Dial Programming |
| 755* | . | . | . | . |  | . | Y | . | . | Name Display Programming |
| 756 | . | . | . | . |  | . | Y | . | Y | Message Wait/Call Back Enable, CO Queuing |
| 756 | . | . | . | . |  | . |  | Y | Y | Message Wait Return |
| 758, 75* | . | . | . | . |  | . | Y | . | . | Speed Dial Access |
| 760 | . | Y | Y | . |  | . | . | Y | . | “SPEED” Button Assignment |
| 761 | . | Y | Y | . |  | . | . | Y | . | “CONF” Button Assignment |
| 762 | . | Y | Y | . |  | . | . | Y | . | “CALL BACK” Button Assignment |
| 763 | . | Y | Y | . |  | . | . | Y | . | “DND/FOR” Button Assignment |
| 764 | . | Y | Y | . |  | . | . | Y | . | “FLASH” Button Assignment |
| 765 | . | Y | Y | . |  | . | . | Y | . | “MUTE” Button Assignment |
| 766 | . | Y | Y | . |  | . | . | Y | . | “MON” Button Assignment |
| 769 | . | Y | Y | . |  | . | . | Y | . | ICM Call Answer Mode (TN or PV, toggle) |
| 76* | . | . | . | . |  | . | . | Y | . | Station Flexible Button Programming |
| NUMBER (GHX-308A) | FLEX BUTTON |  |  |  | STATION |  |  |  |  | FEATURE |
|  | K | 8 | B | A |  | K | S | 8 | 4 |  |
| 7200-7220 | Y | Y | . | Y |  | Y | Y | Y | Y | Pre-selected / Custom Message |
| *73 | Y | Y | . | Y |  | Y | . | Y | Y | Background Music |
| 776 | Y | Y | Y | Y |  | Y | . | Y | . | ICM Key Assignment |
| 777 | Y | Y | . | Y |  | Y | Y | Y | Y | UNA Answer |
| 79 | Y | Y | Y | Y |  | Y | . | Y | . | Loop Button Assignment |
| 89xx, 8#01-8#03 | . | . | . | . |  | . | Y | Y | Y | Retrieve CO on Hold |
| 8* | . | . | . | . |  | . | Y | Y | Y | Retrieve CO on Hold (Held by itself) |
| 81-83 | Y | Y | . | Y |  | Y | Y | Y | Y | CO Line Group |
| 8801-8803 | . | . | . | Y |  | Y | Y | Y | Y | Individual CO line Access |
| 9 | Y | Y | , | Y |  | Y | Y | Y | Y | Line Group Access |
| #6 | Y | Y | . | Y |  | Y | Y | Y | Y | External Zone Page |
| ## | Y | Y | . | Y |  | Y | Y | Y | Y | Meet-Me Page |
| #*0,2,5,8 | . | . | . | Y |  | Y | . | . | . | Ring Frequency 1,2,3,4 Selection |
| ** | Y | Y | . | Y |  | Y | Y | Y | Y | Group Call Pickup |
| *100-699 | . | . | . | Y |  | Y | Y | Y | Y | Directed Call Pickup |
| *7764 | . | . | . | . |  | Y | . | . | . | Customer Program Mode (Station Port #1) |
| HF8 | . | . | . | . |  | . | Y | . | . | System Hold |
| HF751 | . | . | . | . |  | . | Y | . | . | Flash to CO Line |
| HF752 | . | . | . | . |  | . | Y | . | . | Conference |
| SPD# | Y | . | . | Y |  | Y | . | Y | Y | Save Number Redial |
| SPD* | Y | . | . | Y |  | Y | . | Y | Y | Last Number Redial |
| TRANS 752 | . | . | . | . |  | . | . | . | Y | Conference |
| TRANS 756 | . | . | . | . |  | . | . | . | Y | Message Wait/Call Back Enable, CO Queuing |
| *#1, *#2 | Y | Y | . | Y |  | Y | Y | Y | Y | Door Open Command |

| Symbol | Description |
| --- | --- |
| A | Attendant |
| B | Button Assignment Only |
| K | Key Station |
| 8 | 8-Button Station |
| 4 | 4-Button Station |
| S | SLT |

6.1.2  Numbering Plan for GHX-616

| NUMBER (GHX-616) | FLEX BUTTON |  |  | STATION |  |  |  |  | FEATURE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | K | 8 | B | A | K | S | 8 | 4 |  |
| 0 | Y | Y | . | . | Y | Y | Y | Y | Call to Attendant |
| 100 -699 | Y | Y | . | Y | Y | Y | Y | Y | Intercom Call |
| 700 -705 | Y | Y | . | Y | Y | Y | Y | Y | Call Park |
| *700-705 | . | . | . | Y | Y | Y | Y | Y | Call Park Retrieve |
| 700 -705 | . | . | . | . | . | Y | . | . | Call Park Retrieve |
| 710 -717 | Y | Y | . | Y | Y | Y | Y | Y | Hunt Group |
| 731 | . | . | . | Y | . | . | . | . | Wake-up Registration-Effective until cancel |
| 732 | . | . | . | Y | . | . | . | . | Wake-up Registration-One time only |
| 733 | . | . | . | Y | . | . | . | . | Wake-up Cancel |
| 734 | . | . | . | Y | . | . | . | . | Erase Password |
| 735 | . | . | . | Y | . | . | . | . | Change System Date Format (MMDDYY or DDMMYY) |
| 746 | . | . | . | Y | . | . | . | . | Change System Time Format (am/pm or 24 hour) |
| 785 | . | . | . | Y | . | . | . | . | Station COS Change |
| 736 | . | . | . | Y | Y | . | . | . | Software Version Display |
| 737 | . | . | . | Y | . | . | . | . | Custom Message Registration |
| 740 | . | . | . | Y | . | . | . | . | Disable Outgoing CO Line Access |
| 739 | . | . | . | Y | . | . | . | . | Setting Time and Date |
| 774 | Y | Y | Y | Y | Y | . | Y | . | Dial Memo Button Assignment |
| 3 | . | . | . | Y | Y | Y | Y | Y | Camp-On |
| 741 | Y | Y | . | Y | Y | Y | Y | Y | Wake-up Registration-Effective until cancel |
| 742 | Y | Y | . | Y | Y | Y | Y | Y | Wake-up Registration-One time only |
| 743 | Y | Y | . | Y | Y | Y | Y | Y | Wake-up Cancel |
| 744 | Y | Y | . | Y | Y | Y | Y | Y | Access Authorization Code Registration |
| 745 | Y | Y | . | Y | Y | Y | Y | Y | Access Authorization Code Change |
| 747 | Y | Y | . | Y | Y | Y | Y | Y | Station COS change (to COS 5) |
| 748 | Y | Y | . | Y | Y | Y | Y | Y | Original COS Restoration |
| 750,## | . | . | . | Y | Y | Y | Y | Y | Last Number Redial (750 : only SLT) |
| 753 | . | . | . | Y | Y | Y | Y | Y | DND |
| 754 | . | . | . | Y | Y | Y | Y | Y | Station Call Forward |
| 755 | . | . | . | . | . | Y | . | . | Station Speed Dial Programming |
| 755* | . | . | . | . | . | Y | . | . | Name display Programming |
| 756 | . | . | . | . | . | Y | . | Y. | Message Wait/Call Back Enable, CO Queuing |
| 756 | . | . | . |  |  |  | Y | Y | Message Wait Return |
| 758, 75* | . | . | . | . | . | Y | . | . | Speed Dial Access |
| 759 | Y | Y | Y | Y | Y | . | Y | . | Account Code Enter Button Assignment |
| 767 | Y | Y | Y | Y | Y | . | Y | . | Automatic Call Number Redial Button Assignment |
| 760 | . | Y | Y | . | . | . | Y | . | "SPEED", Button Assignment |
| 761 | . | Y | Y | . | . | . | Y | . | "CONF", Button Assignment |
| 762 | . | Y | Y | . | . | . | Y | . | "CALL BACK", Button Assignment |
| 763 | . | Y | Y | . | . | . | Y | . | "DND/FOR", Button Assignment |
| 764 | . | Y | Y | . | . | . | Y | . | "FLASH", Button Assignment |
| 765 | . | Y | Y | . | . | . | Y | . | "MUTE", Button Assignment |
| 766 | . | Y | Y | . | . | . | Y | . | "MON", Button Assignment |
| 769 | . | Y | Y | . | . | . | Y | . | ICM Call Answer Mode (TN or PV, toggle) |
| 76* | . | . | . | . | . | . | Y | . | Station Flexible Button Programming |
| 7200-7220 | Y | Y | . | Y | Y | Y | Y | Y | Pre-Selected/Custom Message |
| 770 |  |  | . | Y | Y | . |  |  | Display SMDR Data |
| 773 | Y | Y | . | Y | Y | . | Y | Y | Background Music |
| 775 | Y | Y | . | Y | Y | Y | Y | Y | Alarm Reset |
| 776 | Y | Y | Y | Y | Y | . | Y | . | ICM Key Assignment |
| 777 | Y | Y | . | Y | Y | Y | Y | Y | UNA Answer |
| 778 | . | . | . | Y | . | . | . | . | Print SMDR |
| 779 | . | . | . | Y | . | . | . | . | Erase SMDR |
| 789 | . | . | . | . | Y | . | . | . | Help (for GK-24E, 36E keyset) |
| 79 | Y | Y | Y | Y | Y | . | Y | . | Loop Button Assignment |
| 7*01-12 | . | . | . | . | Y | . | . | . | Record VAU Message |
| 7#01-12 | . | . | . | Y | Y | Y | Y | Y | Play VAU Message |
| 89xx, 8#01-8#06 | . | . | . | . | . | Y | Y | Y | Retrieve CO on HOLD |
| 8* | . | . | . | . | . | Y | Y | Y | Retrieve CO on HOLD (Held by itself) |
| 81-87 | Y | Y | . | Y | Y | Y | Y | Y | CO Line Group |
| 8801-8806 | . | . | . | Y | Y | Y | Y | Y | Individual CO line Access |
| 9 | Y | Y | . | Y | Y | Y | Y | Y | Line Group Access (0: Norway) |
| #0 | Y | Y | . | Y | Y | Y | Y | Y | All Call Page |
| #1 | Y | Y | . | Y | Y | Y | Y | Y | Internal Zone Page 1 |
| #2 | Y | Y | . | Y | Y | Y | Y | Y | Internal Zone Page 2 |
| #5 | Y | Y | . | Y | Y | Y | Y | Y | Internal All Call Page |
| #6 | Y | Y | . | Y | Y | Y | Y | Y | External Zone Page |
| #7 | Y | Y | . | Y | Y | Y | Y | Y | Meet-Me Page |
| #*0,2,5,8 | . | . | . | Y | Y | . | . | . | Ring Frequency 1,2,3,4 Selection |
| ** | Y | Y | . | Y | Y | Y | Y | Y | Group Call Pick-Up |
| *100-699 | . | . | . | Y | Y | Y | Y | Y | Directed Call Pick-Up |
| *7764 | . | . | . | . | Y | . | . | . | Customer Data Program Mode (Station Port # 1) |
| HF 8 | . | . | . | . | . | Y | . | . | System Hold |
| HF 751 | . | . | . | . | . | Y | . | . | Flash to CO Line |
| HF 752 | . | . | . | . | . | Y | . | . | Conference |
| HF 759 | . | . | . | . | . | Y | . | . | Account Code Enter (During CO conversion) |
| SPD # | Y | . | . | Y | Y | . | Y | Y | Save Number Redial |
| SPD * | Y | . | . | Y | Y | . | Y | Y | Last Number Redial |
| TRANS 752 | . | . | . | . | . | . | . | Y | Conference |
| TRANS 759 | . | . | . | . | . | . | . | Y | Account Code Enter (During CO conversation) |
| *#1, *#2 | Y | Y | . | Y | Y | Y | Y | Y | Door Open Command |

6.2  SYSTEM FEATURES

ACCESS AUTHORIZATION CODE (PASSWORD)

Description

Station users can change their class of service to lower class temporarily to prevent unauthorized use of toll dialing on their stations by others.

Operation

To register password;

1. 	Dial Authorization Code Registration code and 4-digit password.

To change password;

1. 	Dial Authorization Code Change code and current password.

2. 	Enter the new password.

To erase password (Attendant);

1. 	On the attendant station, dial Erase Authorization Code.

2. 	Dial 6-digit station range whose passwords are to be erased.

3. 	Press HOLD button.

To degrade station to COS 5;

1. 	Dial COS Change code.

2. 	MUTE button will be flashing.

To restore the original COS;

1. 	Dial COS Restoration code and 4-digit station password.

Feature Code

1. 	734 : (Attendant) Erase Authorization Code

2. 	744 : Authorization Code Registration

3. 	745 : Authorization Code Change

4. 	747 : COS Change (to COS 5)

5. 	748 : Original COS Restoration

Conditions

1. 	Stations on which the password has not been registered are not allowed to degrade their COS to

change their password (Error tone will be returned).

ACCOUNT CODE  (GHX-616)

Description

Users can enter account code(up to 8 digits) during CO line conversation. The account code is included in the SMDR printout. This is not available in GHX-308A.

Operation

1.	Press preprogrammed Account Code Enter button, before seizing a CO line or during CO

conversation.

2.	Dial the account code (up to 8 digits) and the end character "*".

3.	Continue normal CO access.

Feature Code

1.	759 :	(Keyset) Account Code Enter (Button Assignment only)

2.	759 : 	(4/8 Button) Before seizure CO line (Button Assignment only)

3.	TRANS 759 : 	 (4/8 Button) During CO conversation

4.	HF 759 :	 (SLT) During CO conversation

ALARM  (GHX-616)

Description

An external alarm device may be connected to the system to signal an alarm condition to stations programmed to receive the signal. The system detection may be programmed for either an open or closed loop and for either a continuous signal or single tone alarm. This feature is not available in GHX-308A.

Operation

To reset the alarm, while receiving the alarm signal, remove the alarm condition at first and then dial the Alarm Reset code on any station.

Feature Code

1.	775 :	Alarm Reset

Programming

1.	FLASH 40, Page A, Button 8 : Alarm/Phone Box Signal Receive Yes/No

2.	FLASH 21, Button 1 :		Alarm Enable/Disable

3.	FLASH 21, Button 2 :		Signal Repeat/Once

4.	FLASH 21, Button 3 :		Loop Detect Close/Open

Conditions

1.	The continuous alarm signal will continue until the alarm condition is removed. Once the alarm condition is cleared, the alarm signal is reset by dialing the reset code "775".

2.	The single alarm signal will sound only once, it will wait for the alarm condition to be removed.

If not removed within fifteen(15) seconds, the single alarm will be repeated.

3.	The warning signal is presented to the station programmed.

ATTENDANT

Attendant Disable Outgoing CO Line Access

Description

This feature allows the attendant station to dial a code and disable CO lines from dialing to outside. This applies to all stations that have access to the lines. Incoming calls are not affected.

Operation

1. 	At the attendant, dial Disable Outgoing CO Line Access code.

2. 	Press CO buttons to disable, then CO LEDs at attendant will flash.

3. 	Press flashing CO button (LED off) again to restore outgoing access.

4. 	Go on-hook.

Feature Code

1. 	740 : (Attendant) Disable Outgoing CO Line Access

Conditions

1. 	Only CO lines for which the attendant has direct appearances can be disabled for outgoing access.

Attendant Override

Description

The attendant may interrupt stations in the DND mode by dialing the Call Wait code.

Operation

1.	After receiving DND tone, dial Call Wait code "3".

2.	Wait for station to answer.

Feature Code

1.	3 : Call Wait

Attendant Recall

Description

When a CO line has been left on hold for a programmable period of time, the station placing that line on hold will be recalled. If that station fails to answer the recall, the line will be recalled to the attendant for handling.

Programming

1. 	Flash 01 : System Hold Recall Timer

2. 	Flash 02 : Exclusive Hold Recall Timer

3. 	Flash 03 : Transfer Recall Timer

4. 	Flash 33 : Attendant Recall Timer

5. 	Flash 06 : Call Park Recall Timer

Attendant SMDR Operation (GHX-616 only)

Description

The attendant station may have the system LCD display out or the system print out SMDR data on station basis. The attendant can also erase the SMDR data of a station. This feature is not available in GHX-308A.

Operation

To display station SMDR data on LCD;

1.	At the attendant station, go off hook.

2.	Dial the SMDR Display code.

3.	Dial the 3-digit station number.

4.   	On the LCD, you will see this display:

STA: 105 29 5800 USD

To print station SMDR record;

1.	At the attendant station, go off hook.

2.	Dial the SMDR Print code (778).

3.	Dial the 6-digit station range or 3-digit station number.

4.	Press the HOLD button. Followings will be printed through RS-232C port.

STA CO  TOTAL  START  DATE    DIALED   ACNT CODE   CNT  COST

100  01  0:01:25  09:17  01/04/93  O 1061234      123        5    1000

100  02  0:01:03  10:06  01/06/93  O 12345678     123        2     400

100  01  0:00:25  09:57  01/11/93  O 1261234      123        3     600

--------------------------------------------------------------------------------------------------------

TOTAL CALL      TOTAL COUNT      TOTAL COST

3                 10             	2000

If CMU is set and SMDR type is set to long distance call. The call count and cost of local call will be added to total.

To erase station SMDR record;

1.	At the attendant station, go off hook.

2.	Dial the SMDR Erase code.

3.	Dial the 6-digit station range or 3-digit station number.

4.	Press the HOLD button.

Feature Code

1.	770 : 	(Attendant) Display SMDR Data on station basis

2.	778 : 	(Attendant) Print SMDR Record on station basis

3.	779 : 	(Attendant) Erase SMDR Record on station basis

Programming

1.	FLASH 20, Button 06 : SMDR Data Save Yes/No

2.	FLASH 42 : 			 Call Metering Yes/No

Conditions

1.	To save SMDR, the MBU should have an extra 32Kbyte RAM.

2.	System can save maximum 700 call records.

3.	Upon the system detects that memory for SMDR print out is full, the system will display warning message, "SMDR FULL WARNING" in the LCD of attendant station.

If the memory for SMDR print out is full, call record can not be saved but call metering count and cost of this call will be counted into each total value. When call record of a station could not be saved because the memory was not available and "SMDR FULL WARNING" will be also printed to indicate some of these SMDR records could not be saved.

4.	For call count and cost print out, Call Metering Unit (CMU) should be installed.

Attendant Station Assignment

Description

Any key station in the system may be assigned as the attendant station. This station will receive recalls and can place the system in Night Service and has access to various features.

Programming

1. 	Flash 15 :	Attendant Station Assignment

Conditions

1. 	The attendant station is not necessarily the customer data programming station.

AUTOMATIC PAUSE INSERTION WITH SPEED DIAL

Description

If a flash command or PBX access code is programmed into the system and station speed dial numbers, a pause will automatically be inserted after the flash or PBX access code.

Programming

1. 	Flash 05 : Pause Timer

2. 	Flash 17 : PBX Access Codes

AUTOMATIC PRIVACY

Description

Privacy is automatically provided on all calls. Automatic privacy can be disabled and override on CO line can be made. If privacy is disabled, any keyset in GHX-308A an Override on CO line. But only keyset with override privilege can override on CO line in GHX-616 .

Programming

GHX-308A

1.  	Flash 12, Button 1: Automatic Privacy Enable/Disable

2.  	Flash 12, Button 2: Privacy Warning Tone Yes/No

GHX-616

1.  	Flash 52, Button 1 : Override Privilege Yes/No

2.  	Flash 12, Button 1 : Automatic Privacy Enable/Disable

3.  	Flash 12, Button 2 : Privacy Warning Tone Yes/No

Related Feature

1. 	Override

AUTOMATIC SELECTION

Description

The user can select a CO line, an intercom station, or speed dial bin and automatically place the telephone in the dialing mode without lifting handset or pressing MON button.

Operation

1. 	The user presses the DSS button and the station associated is signaled.

2. 	The user presses the CO button and start dialing.

3. 	The user presses a flexible button programmed as a feature and the action takes place.

AUTO FAX TRANSFER

Description

The system can classify incoming call thru preprogrammed CO line is from automatic FAX (facsimile) station or speech terminal by detecting calling (1100Hz, 0.5s ON/3s OFF repeated) tone. When the system detects a tone from incoming line within predetermined time, the system transfers this line to the station assigned for a FAX. If a tone calling FAX is not detected within predetermined time, this system gives ring to stations which are programmed to ring.

Programming

GHX-308A

1. 	Flash 76 : FAX line / FAX station assignment

2. 	Flash 77 : FAX tone detect timer assignment

3. 	Flash 78 : FAX CO call timer assignment

4. 	Flash 79 : SLT ring off time assignment

GHX-616

1. 	Flash 86 : FAX line / FAX station assignment

2. 	Flash 87 : FAX tone detect timer assignment

3. 	Flash 88 : FAX CO call timer assignment

4. 	Flash 89 : SLT ring off time assignment

Condition

1. 	Only one CO line and station can be programmed as a FAX CO line and a FAX station. If both a FAX

CO and a station are not programmed, AUTO FAX transfer is not activated.

2. 	If a FAX tone is not detected within the FAX tone detect timer, the system gives ring to the stations

programmed by CO ring assignment.

3. 	If the FAX CO line is not answered within the FAX CO call timer, the line is released.

4. 	Outside caller connected to the FAX CO line can hear ring back tone while system is detecting a

FAX calling tone.

5. 	To transfer only a call from FAX to the FAX station, do not assign CO ring to the FAX station.

6. 	If FAX and DISA are programmed on the same CO line in GHX-616, auto FAX transfer will not be

activated.

Hardware

To use FAX transfer, the MFU board is required.

BACKGROUND MUSIC

Description

Station users may receive music over their speakers provided by an internal music source, or an optional music source connected to the system externally.

Operation

GHX-308A

1.  	Dial Background Music code "*73", or press the pre-programmed button (Keyset).

2.  	Dial Background code again to remove music.

GHX-616

1.  	Dial Background Music code "773", or press the pre-programmed button (Keyset).

2.  	Dial Background code again to remove music.

Feature Code

1. 	*73 (GHX-308A) : Background Music (Toggle)

2. 	773 (GHX-616) : Background Music (Toggle)

Programming

1. 	Flash 51 (GHX-616 only) : Music source select

3. 	Flash 52, Button 4 (GHX-616 only) : BGM Enable/Disable

CALL BACK

Description

Stations that are busy can be left a CALL BACK request by other stations in the system. When the busy station goes on-hook, the CALL BACK initiating station will be signaled for automatic CALL BACK. When the station answers, the other station is signaled for an intercom call.

Operation

1. 	While receiving intercom busy tone, press CALLBK button and hang up.

2. 	When busy station hangs up, you will be signaled and your CALLBK button is twinkling.

3. 	Answer the signal by lifting handset or pressing MON button or flashing CALLBK button. The station

you called will then signaled.

Feature Code

1. 	TRANS 756 : (4 Button) Message Wait/Call Back Enable

2. 	756 :        (SLT) Call Back Enable

CALL FORWARD

Description

Station Call Forward

Each key station user may direct intercom calls and incoming CO lines to be forwarded to another station in the system.

Preset Call Forward

The system may be programmed so that incoming CO lines, which are programmed to ring a particular station, can be forwarded to another station predetermined in programming if the original station does not answer. Each station in the system can have one preset forward station.

Operation

To active Station Call Forward for GHX-308A;

1. 	Lift handset or press MON button.

2. 	Press DND/FWD button.

3. 	Dial the station number or press the DSS of the station where calls are to be forwarded. Then you

will hear confirmation tone. The DND/FWD button will flash.

To active Station Call Forward for GHX-616;

1.	Lift handset or press MON button.

2.	Press DND/FWD button.

3.	Dial the Call Forward type number (0-4) and the station number or press the DSS of the station where calls are to be forwarded. Then you will hear confirmation tone. The DND/FWD button will flash.

| Type Number | Description of Call Forward |
| --- | --- |
| 0 | Unconditional |
| 1 | No answer |
| 2 | Busy |
| 3 | Busy/No answer |
| 4 | Follow me |
| 5 | Off-net |

※ Call Forward type number (5)

This feature allows Call Forward to outside line. To activate this feature, station should have privilege.

To activate Off-net Call Forward (Key Telephone);

1.	Press SPEED button and Speed dial bin number.

2.	Press HOLD button.

or,

1.	Press CO button and dial telephone number.

2.	Press HOLD button.

To activate Off-net Call Forward (SLT);

1. 	Dial “754” and “5”.

2. 	Press the telephone number.

3. 	Hook Flash.

To cancel Station Call Forward;

1.	Lift handset or press MON button.

2.	(key station) Press DND/FWD button and go on-hook.

(4/8/SLT)   Dial '754' and go on-hook.

To program Preset Forward Station;

- 	Refer to programming.

Feature Code

- 754xxx (GHX-308A) : (4/8 Button, SLT, xxx= Destination Station Number)

- 754+Call forward type(0-4)+xxx (GHX-616) : (4/8 Button, SLT, xxx= Destination Station Number)

Programming

1. 	Flash 04: Preset Forward Timer

2. 	Flash 40, Page B, Button 6: Preset Forward Station (Destination)

3. 	Flash 40, Page A, Button 7: Station Call Forward Access Yes/No

4. 	Flash 52, Button 5 (GHX-616 Only): Off-net Call Forward Yes/No

5. 	Flash 41, Button 1(GHX-616 Only) : Off-net Call Forward CO to CO Yes/No

Conditions

1. 	The Station Call Forward takes precedence over the Preset Call Forward.

2. 	More than one station can be forwarded to the same destination.

3. 	In Preset Forward, when the ring is forwarded to the preset forward station, the ring stops on the

initiating station.

- Stations already in DND will be released from the DND status and enter into call forward status

- when call forward is activated.

5. 	In GHX-616, incoming CO line forwarded to outside can converse within Conference Time.

(Flash 07)

CALL PARK

Description

A CO line can be placed into one of 6 parking locations and can be retrieved by any station with direct appearance or loop button(except station of COS 6) in the system. Parked calls have their own recall timers and will recall the originating station and, if unanswered, the attendant. If still not answered, all available stations will receive recall for the duration of system hold recall timer until answered or the line is dropped.

Operation

To park CO line;

1. 	While connected to an CO line, press TRANS button.

2. 	Dial 3-digit parking location.

To retrieve parked CO line;

- 	Keyset: Dial Retrieve code "*" and parking location.

- 	SLT:   Dial the parking location.

Feature Code

|  | Park | Unpark |
| --- | --- | --- |
| Keyset | TRANS 700-705 | * 700-705 |
| SLT | HOOK FLASH 700-705 | * 700-705 |

Programming

1. 	Flash 06:	Call Park Recall Timer

2. 	Flash 33:	Attendant Recall Timer

Conditions

1. 	When dialing a parking location to park a CO line and the location is already occupied, the initiating

party receives busy tone.

CALL TRANSFER

Description

A CO line can be transferred from one to another. Screened transfers (with announcement) or unscreened transfers (without announcement) can be made. Several attempts can be made to find someone at different keysets without losing the call. If a line is transferred to a busy station, that station will receive muted ring.

Operation

While connected to a CO line,

1. 	Press TRANS button and dial 3 digits station number, or press appropriate DSS button. CO line is

put on an exclusive hold and music is provided to the held party.

2. 	At this point, you can make a screened or unscreened transfer.

Screened Transfer

1) 	When the called extension answers, announce transfer and hang up.

2) 	The transferred party will be automatically connected to outside party.

Unscreened Transfer

1) 	Once the station you called begins to ring, hang up.

Programming

1. 	Flash 03 : Transfer Recall Timer

2. 	Flash 33 : Attendant Recall Timer

Conditions

1. 	Only the secretary assigned station can transfer a CO line to a busy executive.

2. 	If the transfer recall timer expires when station A transfers a call to station B, the recall will ring both

station A and B. If not answered, then the recall will ring station A, B and the attendant.

3. 	If still not answered, all available stations will receive recall for the duration of system hold recall

timer until answered or the line is dropped.

CALL WAIT

Description

A station user may alert a busy party that a CO lines on hold and waiting for them. The busy party will receive a muted ring and flashing HOLD button to notify them a call is waiting for them.

On keyset, call wait can be answered by pressing the HOLD button. If the station was in CO conversation, the CO line is put on Exclusive Hold, if intercom, then the intercom is terminated.

On SLT, call wait can be answered by answering the ringing signal after placing the CO line on hold or terminating the current call.

Operation

1.   After receiving intercom busy tone, dial "3".

2.   Called station will receive one burst Call Wait tone.

- When called station answers by pressing HOLD button, converse with them or hang up to transfer

call.

Feature Code

1. 	3:  Call Wait

Conditions

1. 	Only the secretary assigned station can place a Call Wait to the busy executive.

CO LINE ACCESS

Description

Through programming, stations are allowed or denied access to particular CO line groups. This access is programmable per station basis.

Operation

1. 	Press CO line button or pool button, or dial CO line access code.

2. 	Dial the phone number of the desired party.

Feature Code

1. 	9 : Line Group Access (the first access allowed group)

- 81-83 (GHX-308A) : Line Group Access

- 81-87 (GHX-616) : Line Group Access

3. 	88xx:(xx = CO line number, 01-03 for GHX-308A, 01-06 for GHX-616 ) Individual CO Line Access

Programming

1. 	Flash 30, Page B, Button 2: CO Line Group Assignment

2. 	Flash 40, Page B, Button 7: CO Group Access

CO RING ASSIGNMENT

Description

CO lines are assigned to ring at individual stations by programming. Any station may be programmed to ring for any CO lines during the Day and/or Night mode, both or neither.

Programming

1. 	Flash 40, Page B, Button 8: CO Button/Ring Assignment

2. 	Flash 40, Page B, Button 9: CO Button/Ring Assignment Display

CO RING DETECT

Description

The duration of the ringing signal from the CO or the PBX is matched with ringing detection circuitry in the KSU. The RING ON detect window can range from 200 to 900 msec, programmed in 100msec increments.

Programming

1. 	Flash 10:	CO Ring ON Detect Timer (2-9 100msec).

Conditions

1. 	The RING ON detect window is from 200 msec to 900 msec, divided into 100 msec increments.

CONFERENCE

Description

The system allows the following conferences:

1) 	Internal Conference	 	- three internals

2) 	Add On Conference	 	- two internals and one external

3) 	Multi-Line Conference 		- one internal and two externals

4) 	Unsupervised Conference

The conference initiator can exit a multi-line conference and leave them in an unsupervised conference. The initiator can re-enter the conference at any time. A programmable conference timer will disconnect the unsupervised multi-line conference if the initiator does not re-enter.

Operation

Establishing a conference;

1. 	Call desired party, outside or internal.

2. 	Press CONF button. The outside party is placed on exclusive hold and hear the music sound.

3. 	Call next party and press CONF button.

4. 	Conference is established.

Exiting a conference (Unsupervised Conference);

- 	During conference, press CONF button. Allowed only if you are the initiator of the conference.

Re-enter a conference;

- 	Press CONF button or go off-hook. When initiator reenter the conference, confirmation tone is heard

to all stations in the conference.

Terminating conference;

- 	Hang up during conference.

Feature Code

1. 	TRANS 752		: (4 Button) Conference

2. 	HOOK FLASH 752 	: (SLT) Conference

Programming

1. 	Flash 07 : Unsupervised Conference Timer

2. 	Flash 39 : Multi-line Conference Enable/Disable

Conditions

1. 	The conference sequence should be completed prior to the expiration of the recall timer.

2.	If you want to make a multi-line conference, multi-line conference should be enabled.

(Program Code: Flash 39)

3.	When the conference initiator exits the multi-line conference, an unsupervised conference timer will

be activated. (Program Code: Flash 07)

4. 	Unsupervised multi-line conference will be dropped if the initiator does not re-enter within

unsupervised conference time.

DIAL SENDING

Description

DTMF/Pulse Sending

CO lines can be programmed to send dial pulse or DTMF(tone) signal. In case of Pulse CO line, Dialing Speed and Make/Break Ratio are programmable per each CO line.

Pulse-to-Tone Switch-over

When commanded, the system will change the signaling on a CO line from dial pulse to DTMF (tone), allowing the use of common carriers behind a dial pulse CO line. This can be done manually when dialing, or can be stored within a speed dial number.

Operation

1. 	When using speed dial numbers, the TRANS button is used to store a switch-over command. When

activated, it will automatically insert a pause and switch over to DTMF sending for a remaining

digits.

2. 	The switch-over is activated manually by dialing the "*" (asterisk) when dialing.

Programming

1. 	Flash 05:  Pause Timer

2. 	Flash 30, Page A, Button 1:  Dial Signaling DTMF/PULSE

3. 	Flash 30, Page A, Button 5:  Pulse Ratio 60-40/66-33

4. 	Flash 30, Page A, Button 6:  Pulse Speed 10 PPS/20 PPS

Related Feature

1. 	Automatic Pause Insertion with Speed Dial

2. 	Station/System Speed Dial

Conditions

1.	The pause length is determined by the System Pause Timer.

DIALING PRIVILEGES (Class of Service)

Description

The system provides a flexible means of providing toll or dialing restriction. Through the assignment of Class of Service(COS) many combinations of allow and deny numbers can be set.  Each station is assigned one of the six(6) Classes of Service (Day and Night).

The 6 classes are,

| COS | Description |
| --- | --- |
| 1 | Unrestricted. |
| 2 | Governed by Toll Allow/Deny Table A. |
| 3 | Governed by Toll Allow/Deny Table B. |
| 4 | GHX-308A : No '0' as the first digit. GHX-616: No '0' or '1' as the first digit, maximum 7 digits. |
| 5 | Intercom calls and Paging only. |
| 6 | Receiving calls and Paging only. |

Programming

1. 	Flash 40, Page B, Button 2: Station COS

Two digits are entered via dial keypad. The first digit is for day COS and the second for night COS.

Default gives all stations COS 11.

2. 	Flash 50, Button 1:	Allow Table A

3. 	Flash 50, Button 2:	Deny Table A

4. 	Flash 50, Button 3:	Allow Table B

5. 	Flash 50, Button 4:	Deny Table B

Related Feature

1. 	Toll Override

DIFFERENTIAL RING FREQUENCY SELECTION

Description

The user of keyset can program the frequency of the ringing signal sent to speaker of keyset. For frequency settings are available to allow users to distinguish which telephone in a given area is ringing. The setting will only affect ringing signals (Intercom, Incoming CO) sent to the speaker of the keyset while idle.

Operation

1. 	Dial Frequency Selection Code and one data digit (2,5,8,0) to select frequency.

Feature Code

1. 	#*2, #*5, #*8, #*0 :  Differential Ring Frequency Selection

Conditions

1.   	No LCD service for selection.

2.   	After dialing a code, user can continue to enter determined preferable ringing frequency.

- Upon entering a digit, system will present selected ringing to the user and data is automatically

saved	 in battery-backed up memory.

- The keyset which is unavailable for differential ringing may not receive any system response after

- entering data, 2/5/8. In this case, user must finish with entering 0 (ordinary ringing).

5.   	This feature is not applicable to SLT (error tone).

6.   	Built-in menu:

| DATA | FREQUENCY | TIME BY TURNS |
| --- | --- | --- |
| 2 | 701/857 Hz | 50 msec |
| 5 | 771/701 Hz | 100 msec |
| 8 | 1471/1215/1336Hz | 50 msec |
| 0 | 1215/1471 Hz | 50 msec |

DISA (Direct Inward System Access) (GHX-616)

Description

An outside caller can access a CO line which is preprogrammed as DISA line and call an internal station directly without through the attendant or any other station. The DISA line can be set either 24-hour or for Night only.

Operation

An outside caller accesses a CO line marked as DISA. Then dial tone will be provided and the caller can dial internal station number after entering the 3-digit DISA access code, if DISA access code has been programmed.

Programming

1.	FLASH 30, Page B, Button 4, Dial '0' :	No DISA (normal CO line).

'1' : DISA - 24 Hours

'2' : DISA - Day only

'3' : DISA - Night only

2.	FLASH 55  : VAU mode program

3.	FLASH 58  : VAU segment usage program

4.	FLASH 59  : VAU channel usage program

5.	FLASH 66  : DISA no answer timer

6.	FLASH 67  : DISA access code

7.	FLASH 68  : DISA voice guide

8.	FLASH 69  : DISA CO line access

9.	FLASH 77  : DISA with hunt group

10.	FLASH 78 	: DISA night action

(Same as day DISA/ Give error tone & drop/ Call predetermined STA)

11.	FLASH 79 	: DISA error treatment

- FLASH 83 	: VAU tone cadence program

- FLASH 84 	: VAU tone time

- FLASH 85 	: VAU DISA day/night message replay

Conditions

1.	If the DISA code is not programmed, then the outside caller can dial the internal station numbers.

2.	Internal key stations equipped with direct or loop button for the associated DISA line can answer the DISA call.

3.	The DISA line can be programmed for Day or Night mode and different announcement is provided in each mode.

4.	If an outside caller accesses a CO line marked as a DISA in night, night announcement is provided and the call is processed on below states by Admin. programming.

-.  Call internal STA number (same as Day mode)

-.  Call pre-selected station (by Admin. programming)

-.  Send error tone and drop the call

5.	Three channels of VAU can be controlled for DISA.

If all the channels are busy, ring back tone is heard and the call is being held until an idle channel is made.

6.	In the following cases, a proper announcement is provided individually.

-.  Invalid digit is dialed.

-.  The STA or CO line is busy.

-.  Dial is late.

7.	In the above cases, (No. 6) DISA call can be differently processed in each state.

-.  Send error tone and drop.

-.  Transfer to Attendant.

-.  Retry (3)

8.	If an outside caller accesses a CO line marked as a DISA, a Hunt Group as well as internal STA number can be called.

9.	To access a CO line, the outside caller dial CO line access code and enter four digits of DISA access code. (The default code is 000). After this procedure, outside caller can dial telephone number.

10.	An outside caller can call a Hunt Group with dialing 1 digit instead of 3-digit internal STA number.

(Dial 0 - 7 to call Hunt Group 710 - 717)

11.  	One path is provided for CO to CO DISA.

12.  	Outside caller can dial with DTMF only for DISA call .

Hardware

1.	VAU for DISA voice announcement (if equipped)

DO NOT DISTURB

Description

Placing a keyset in DND will eliminate incoming CO line ringing, intercom calls, transferred calls and paging announcements. The station in DND can use the telephone to make normal outgoing calls. A station can be denied this feature through programming.

Operation

To activate the DND;

1. 	Press DND/FWD button while on-hook.

2. 	DND/FWD button lights and your phone is in DND.

To deactivate the DND;

1. 	Press DND/FWD button while on-hook (toggle).

2. 	DND/FWD button extinguishes and DND in your phone is removed.

Feature Code

1. 	753: (4/8 Button, SLT) Do Not Disturb

Programming

1. 	Flash 40, Page A, Button 2: DND Access Yes/No

DOOR OPEN CONTROL

Description

This feature allows privileged stations to activate door open feature. The relay is assigned as DOOR OPEN RELAY by system programming. The relay will be activated for a predetermined period.

Operation

To activate the Door Open Relay, dial '*#1' (or *#2) for the first(second) door relay.

Feature Code

1. 	*#1, *#2 : Door Open Control Code

Programming

1.  	Flash 54 : Door Open Timer (0.5 - 9.9 seconds)

2.  	Flash 52, Button 3 (for GHX-308A) : Door Open Privilege

Flash 52, Button 2(for GHX-616) : Door Open Privilege

3.  	Flash 23, Button 1,2 : Program Control Relay

Condition

1. 	The second door relay is available when the MFU is installed in GHX-308A.

2.	Two relay contacts are provided when the MFU board is installed in GHX-616.

DSS/DLS BOX  (GHX-616)

Description

4 DSS/DLS Boxes can be installed in GHX-616.

Programming

1. 	FLASH 40, Page B, Button 1 : Station ID

Default Button MA

01               02            03              04

| All Call Page | External Page | Internal Page 1 | Internal Page 2 |
| --- | --- | --- | --- |

05               06            07              08

| Hunt group 1 | Hunt Group 2 | Hunt Group 3 | Hunt Group 4 |
| --- | --- | --- | --- |

09               10            11              12

| STA 100 | STA 101 | STA 102 | STA 103 |
| --- | --- | --- | --- |

13               14            15              16

| STA 104 | STA 105 | STA 106 | STA 107 |
| --- | --- | --- | --- |

17              18             19              20

| STA 108 | STA 109 | STA 110 | STA 111 |
| --- | --- | --- | --- |

21              22             23              24

| STA 112 | STA 113 | STA 114 | STA 115 |
| --- | --- | --- | --- |

25              26             27              28

.....................

45              46             47              48

EXECUTIVE/SECRETARY TRANSFER

Description

There are 4 sets of Executive/Secretary pairings available. When the executive station is busy or in DND, the secretary station will receive intercom calls and transfers. The secretary station can signal the executive in DND by using the CALL WAIT feature.

Operation

1.	If the executive station is busy or in DND, all calls to the executive will forward to the secretary

station.

2. 	A secretary station can signal an executive busy or in DND via CALL WAIT.

Feature Code

1. 	3 : CALL WAIT

Programming

1. 	Flash 18 :  Executive/Secretary Assignment

Related Feature

1. 	DND

2. 	Preset Call Forward

3. 	Station Call Forward

Conditions

1. 	Intercom calls routed to the secretary station will always signal tone ringing.

2. 	The secretary can signal the executive in DND via Call Wait.

3. 	Chaining of the transfer is possible. If the first secretary is busy, the call can be transferred to the

next programmed secretary.

4. 	If the executive of an executive/secretary pair station forwards his phone, calls to the executive will

follow his station forward.

- If both the executive and the secretary are busy or in DND and the calling party leaves a message

- or call back, the message or call back will be left at the secretary station.

FLASH

Description

Flash provides users with the ability to terminate an outside call or transfer a call behind PBX and restore dial tone without hanging up the handset.

Operation

While connected to a CO line,

Key station : 	Press the FLASH button.

SLT :      		Dial Flash Code after Hook Flash.

Feature Code

1. 	HOOK FLASH 751:  (SLT) Flash command to CO Line

Programming

1. 	Flash 30, Page B, Button 1:  Flash Timer

Related Feature

1. 	Flash with Speed Dial

FLASH WITH SPEED DIAL

Description

A flash can be programmed within a speed dial number. When this is done, a pause will automatically be inserted before the remaining speed dial digits are sent.

Operation

While storing speed numbers, press FLASH button as a digit.

Related Feature

1.	Flash Key

2.	Station/System Speed Dial

FLEXIBLE BUTTON ASSIGNMENT

Description

The buttons on each key station which are not assigned to CO line in system programming can be individually programmed. One of 5 operations can be selected for each button.

1. 	CO line group:

automatically access assigned CO line group.

2. 	Direct Station Select:

automatically signal assigned station provide BLF for off-hook and DND.

3. 	Speed dial:

automatically dial a speed number. (system or station)

4. 	Loop:

used to answer a transferred CO line or to retrieve a parked CO line for which a user does not have

a direct button assignment.

5. 	Features:

Feature codes in the numbering plan can be programmed under the flexible buttons.

Operation

To program flexible buttons;

1. 	Press SPEED button twice and flexible button you wish to program.

2. 	Enter the code for the desired feature.

3. 	You will hear confirmation tone. By programming flexible button, the previously assigned code is

substituted by new one.

To erase a flexible button;

1. 	Press SPEED button twice and flexible button to be erased.

2. 	Press FLASH button.

Feature Code

1. 	76*: (8 Button) Station Flexible Button Programming

Conditions

1. 	Speed bin cannot be programmed under a flexible button on 8-button station.

2. 	Flexible button in 8-button can not be erased, program again to change flexible button.

FLEXIBLE STATION NUMBER

Description

Station intercom numbers in the system can be newly allocated through programming into a range from 100 to 699. Flexible station numbers should be used for call procedures and admin. programming.

Programming

Flash 36, 	Button 1-2 (GHX-308A), Button1-4 (GHX-616): Flexible Station Numbers

Button 1  : Port 1 - 4 (Default : 100-103)

Button 2  : Port 5 - 8 (Default : 104-107)

Button 3  : Port 9 - 12 (Default : 108-111) (GHX-616)

Button 4  : Port 13-16 (Default : 112-115) (GHX-616)

Dial the 4 station numbers(each 3 digit) after pressing the button 1-4.

HOLD

Description

Hold Preference

The system allows either exclusive or system hold as the primary hold on the first depression of the HOLD button depending on programming.

System Hold

When a line is placed on system hold, any station in the system with access to that line can retrieve the call.

Exclusive Hold

When a line is placed on exclusive hold, no other station in the system can retrieve this call.

Hold Recall

If an held line is not re-seized within a programmable length of time (System /Exclusive /Transfer /Call Park Recall Timer), recall ring (for the duration of the associated Hold Recall Timer) is sent to the station placing the call on hold. If this recall is not answered, a recall ring is sent to the attendant (for the duration of the Attendant Recall Timer), and if the attendant doesn't answer, a recall is sent to all phones in the system.

Operation

To place a call on hold;

- If the system is programmed to have system hold preferred (Program Code: Flash 10), press HOLD

button once for System Hold, twice for Exclusive Hold.

Hold Retrieve (To retrieve a call on hold)

- Press flashing CO line button on hold or dial Retrieve code.

Feature Code

1. 	HF 8: (SLT) System Hold Code

2. 	89xx, 8#xx : (xx = CO line number, 4/8 Button, SLT) Retrieve CO on hold

3. 	8* : (4/8 Button, SLT) Retrieve CO on hold (Held by itself)

Programming

1. 	Flash 01 :  System Hold Recall Timer

2. 	Flash 02 :  Exclusive Hold Recall Timer

3. 	Flash 03 :  Transfer Recall Timer

4. 	Flash 33 :  Attendant Recall Timer

5. 	Flash 06 :  Call Park Recall Timer

6. 	Flash 11 :  Hold Preference

Related Feature

1. 	Attendant Recall

2. 	Call Park

3. 	Call Transfer

Conditions

- When an exclusive hold line recalls the attendant, the status of the hold line changes from

- exclusive to system hold.

- A line that recalls the attendant will ring until answered or disconnected, or the attendant recall

timer 	expires. (Program Code: Flash 33)

- If still not answered, all available stations will receive recall for the duration of system hold recall

- timer until answered or the line is dropped.

- When connected to a CO line and pressing a DSS button in an attempt to transfer the CO line, the

- CO line is placed on exclusive hold regardless of Hold Preference.

5.   	A CO line put on hold cannot be retrieved with loop buttons.

HOT LINE

Description

This feature lets a station perform a pre-assigned feature when a station takes no action for Hot Line after going off-hook. Hot Line feature can be programmed as one of the followings;

Activation time for Hot Line feature is determined by Hot Line Timer.

1) 	To activate a feature on flexible button as if pressed.

2) 	To seize a CO Line

3) 	To seize a CO Line Group

4) 	To call an another station

5) 	To dial a SPEED bin number

Operation

Lift handset or press MON button, and wait for a predetermined period while hearing ICM dial tone. If the Hot Line Timer is set to zero, Hot Line feature is activated.

Conditions

1. 	Any flexible button may be assigned as Hot Line feature. When lifting handset or pressing MON

button, system will act as if user presses the pre-selected button.

2. 	If the user selects an another feature prior to going off-hook, Hot Line feature is not activated.

3. 	SLT allows Hot Line feature except flexible button feature.

4. 	On going off-hook, both Dial Tone Timer and Hot Line Timer start. After the Hot Line Timer 	expires (no user action), the Dial Tone Timer is continuous.

Programming

1. 	Hot Line Selection : Flash 53, Button 1-6.

1)  Enter the station range

2)  Select a button (one of button1-6 : hot line operation)

Button 1 : flexible button

Button 2 : CO line

Button 3 : CO group

Button 4 : sta. number

Button 5 : speed dial

Button 6 : delete hot line operation

2. 	Hot Line Timer : FLASH 57

HUNT GROUPS

Description

The system can be arranged for up to eight hunt groups. Each hunt group can contain up to eight stations. When an intercom or transferred CO line is directed to a pilot hunt group, the system will search sequentially for an idle station. Upon finding an idle station within the group, it will ring at the idle station.

Feature Code

1. 	710-717 : Hunt Group Number

Programming

1. 	Flash 19 : Hunt Group

Conditions

1. 	A station placed in DND will be considered busy for hunting purpose.

2. 	A station placed in Call Forward mode will be considered busy. A direct call to that station will  	forwarded.

3. 	Hunt groups can be joined together.

Incoming CO Ring Assignment to Hunt Group  (GHX-616)

Incoming CO ring can be assigned to Hunt Group so that incoming CO call will search for an idle station in the group. If the station does not answer the CO ring within preset forward time, the system continues to search for an available station and give CO ring to it.

Programming

1.   	FLASH 19 : Hunt Group

2.   	FLASH 34 : Incoming CO ring assignment to Hunt Group

- FLASH 04 : Preset Forward Timer

- FLASH 58 : VAU Segment Control

- FLASH 59 : VAU Channel Control

Condition

1.	The system does not give ring to the station in DND, Call Forward or Pre-selected/Custom Message.

2.	The call will continue to route until each station in the group has been tried. The call will remain at the last station.

3.	All the stations in the Hunt Group are busy, busy tone is provided to the CO line.

4.	If VAU is equipped, voice announcement is available.

5.	If CO ring is incoming to Hunt Group and all stations in Hunt Group are busy, UCD 1 message of VAU3 is played .

6.	If any station in Hunt Group doesn't answer the call during playing UCD 1 message, CO line is dropped after playing UCD 2.

7.    To use UCD1, 2 message of VAU3, VAU3 should be installed .

INTERCOM SIGNALING SELECT

Description

Users can control the method by which they receive intercom calls and signals. A convenient slide switch is located on each key station for easy selection. The choices are;

1. 	Tone ringing (TN)

A standard tone ring notifies the party of an incoming intercom call. The called party answers by lifting the handset or sliding switch to HF or pressing MON button.

2. 	Privacy (PV)

The station user receives a burst of tone and a voice announcement over his/her speaker.  The microphone is deactivated for privacy. The called party should lift the handset to answer the call. (Or switch the selector to hands-free(HF))

3. 	Hands-free (HF)

The station user, upon hearing a tone burst and voice announcement over the speaker, can reply hands-free.

Feature code

1. 	769 : (8 Button) ICM Call Answer Mode (TN or PV, Toggle)

INTERCOM SIGNALING CHANGE  (GHX-616)

Description

A keyset user calling internal keyset can change calling signal of called keyset.

Operation

To change intercom calling signal while calling internal station, dial *.

1.	If a called station is in TN mode, the calling signal is changed from TN mode to HF mode.

2.	If a called station is in HF or PV mode, the calling signal is changed to TN mode.

Condition

This can be used once during calling internal station.

LOOP BUTTON

Description

The loop is used primarily to receive transferred CO lines that do not appear(no direct) on a station.

Feature Code

1. 	79 : Loop Key Button Assignment (at the station)

Programming

1. 	Flash 40, Page B, Button 8 : CO Button/Ring Assignment

LOUD BELL CONTROL

Description

Relay-controlled contacts can be connected to a loud ringing device to provide ringing signal of specified station. (2 stations)

Programming

1. 	Flash 16: Loud Bell Station Assignment

2. 	Flash 23, Button1, 2 : Program Control Relay

Conditions

1. 	Intercom calls in the PV and HF mode are not presented via LBC.

2.	Two LBC contacts are available.

Hardware

1.   	External ringing device, using contact closure

- In GHX-308A, the second relay contact is available when MFU is installed.

- In GHX-616, two relay contacts are available when MFU is installed.

MESSAGE DISPLAY

Description

Custom Messages

The system can be programmed with 10 custom messages so that the each station can select the message to be displayed on the LCD of the calling station.

Pre-selected Messages

Each key station can select a pre-assigned message to be displayed on the LCD of the telephone calling that station. GHX-616 provides two kinds of messages and each has 10 messages. Pre-selected message type can be selected by system programming. Some of Type II messages need appropriate digit data.

In GHX-308A, only Message Type I is provided.

| Code | Message Type I |  | Message Type II (GHX-616 only) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Message | Data |
| 00 |  | message cancel | message cancel |  |
| 01 |  | OUT TO LUNCH | LUNCH RETURN AT 00:00 | 4 digits |
| 02 |  | ON VACATION | VACATION, RETURN ON 00:00 | 4 digits |
| 03 |  | OUT OF OFFICE-BACK AM | ABSENT, RTN AT CLK 00:00 | 4 digits |
| 04 |  | OUT OF OFFICE-BACK PM | ABSENT, RTN AT DATE 00:00 | 4 digits |
| 05 |  | OUT OF OFFICE-BACK TOM | OUT OF OFFICE-BACK TOM |  |
| 06 |  | OUT OF OFFICE-UNKN | CALL TO 000000000000 | maximum 12 digits and '*' |
| 07 |  | OUT OF TOWN | IN OFFICE 000 | 3-digit internal party number |
| 08 |  | IN A MEETING | MEETING CLK 00:00 | 4 digits |
| 09 |  | AT HOME | AT HOME |  |
| 10 |  | AT BRANCH OFFICE | AT BRANCH OFFICE |  |

Operation

To program the customized messages (Attendant);

1. 	At the attendant station, dial the Custom Message Registration code "737" and custom message bin

number (11-20).

2. 	Enter messages using the dial pad in the same manner to that used to display names. The

messages are limited to 24 characters including spaces.

3. 	Press HOLD button.

To activate pre-selected/customized messages;

1. 	Dial Pre-selected/Custom Message code "72" and message number (00-20).

2. 	Dial appropriate dial data if you select the new message.

3. 	Appropriate message will be displayed on the LCD of the station calling that station.

Feature Code

1. 	72xx  : (xx=00-20) Pre-selected/Custom Message Code

2. 	737xx : (Attendant, xx=11-20) Custom Message Registration

Related Feature

1. 	DND (Do Not Disturb)

2. 	Pre-selected Messages

3. 	Station Call Forward

Programming

1. 	Flash 43, Button 1 (GHX-616 only) : Pre-selected Message type select

MESSAGE WAIT

Description

Stations that are unattended or in DND can be left a message indication by other stations in the system. Up to 5 messages can be left at a keyset. A station with a message waiting can be reminded at a timed interval with a tone.

Operation

To leave Message Wait indication;

1. 	After receiving DND tone or no answer, press CALLBK button. Confirmation tone will be heard.

2. 	At called party, the Message Wait indication will be displayed and CALLBK button will be twinkling.

To answer the Message Wait indication;

- 	Press flashing CALLBK button. Then station that left message will be signaled with tone ringing.

Programming

1. 	Flash 08 : Message Waiting Remind Tone Timer

Feature Code

1. 	756 : (4 Button /SLT) Call Back Enable

56:  SLT message wait is not implemented.  92/6/122. 	756 : (4/8 Button) Message Wait Return

Conditions

1. 	Message Wait Reminder Tone is provided for key stations only.

2. 	The reminder tone will continue until all messages have been retrieved.

3. 	SLT cannot be left with a message wait.

MONITOR MODE (GROUP LISTENING)

Description

While you are talking on handset, by pressing the MON button, other people around you may hear the conversation through the speaker of keyset.

Programming

1. 	Flash 29 : Monitor Mode Enable/Disable

Conditions

1. 	While in monitor mode, microphone is muted.

MUSIC ON HOLD

Description

The system is equipped with an internal music source to provide music to all lines on hold and background music to key station users.

In GHX-616, voice message can be provided to lines on hold if the system is equipped with VAU3 (Voice

Announcement Unit).

Related Feature

1. 	Exclusive/System Hold

2. 	CO Call Transfer

Hardware

1. 	GHX-308A : To use external music source, set jumper option for music source on the MBU to EXT.

2. 	GHX-616 :  To use external music source, the MFU should be installed and program is required.

Programming

1. 	Program Control Relay : Flash 23, Button 1, 2

2. 	Music source select (GHX-616 only) : Flash 51

MUTE

Description

While on a conversation, the voice transmission can be blocked by pressing MUTE button. The transmission is enabled again by pressing MUTE button again.

NAME DISPLAY

Description

You can program your phone so display phone user will see your name instead of your station number.

Operation

To program your name;

(Keyset)

1. 	Press SPEED button twice and dial "00".

2. 	Press SPEED button and enter your name (up to 7 character) using dial pad.

3. 	Press SPEED button.

(SLT)

1. 	Go off hook and dial "755*".

2. 	Enter the name as in keyset.

3. 	Perform Hook Flash to complete.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | . | A | D | G | J | M | P | T | W |
| * | Q | B | E | H | K | N | R | U | X |
| # | Z | C | F | I | L | O | S | V | Y |

'##': Space

To erase your name;

(Keyset)

1. 	Press SPEED button twice and dial "00".

2. 	Press SPEED button twice.

(SLT)

1. 	Go off hook and dial "755*" and perform Hook Flash.

NIGHT TRANSFER

Description

The system can be placed into night service. This allows designated stations to ring at night that may or may not ring on those lines during the day.

Operation

1. 	Attendant presses DND button to activate or deactivate.

2. 	Station can answer CO line ringing at station to ring at night mode by dialing "777".

Feature Code

1. 	777 : Universal Night Answer

Programming

1. 	Flash 40, Page B, Button 8 : CO Button/Ring Assignment

Conditions

1. 	If attendant placed the system into night mode, other stations can not change to day mode.

ON-HOOK DIALING

Description

The key station user can place calls without lifting the handset.

OVERRIDE

Description

This feature enables keyset user to override CO line in conversation with another internal party.

Operation

GHX-308A

If the system is privileged to override (Program code 12, Button 1), press the CO line button to be overridden. Keyset user will join the conversation immediately.

GHX-616

To activate override, the system is privileged to override (Program code 12, Button 1)and keyset user is  privileged to override (Program code 52, Button 1).

If the station which converses on CO line is not privileged to override(Program code 52, Button 1), press

the CO line button to be overridden.

Programming

1. 	Flash 12, Button 1 :  Automatic Privacy Enable/Disable

2. 	Flash 12, Button 2 :  Privacy Warning Tone Yes/No

3. 	Flash 52, Button 1 (GHX-616 only) : Station override privilege

PAGE

Description

Paging

GHX-308A

There is only external paging.

GHX-616

There are 4 types of paging available: External, Internal, Internal All Call, and All Call. There are 1 external zone and 2 internal zones available. Paging requires a 2-digit dialing code.

Page Access Restriction

A station can be individually allowed or denied the ability to make page.

Meet Me Page

Users may answer a page call from any phone in the system by dialing a special code. The party who initiated the page must remain off-hook.

Operation

1. 	Dial Page code. Warning tone will be heard. When the tone ends speak in a normal tone of voice and 	ask for Meet Me Page.

2. 	After hearing the page announcement, go to the nearest telephone and dial Meet Me Page

code "##" ( #7 in GHX-616), then you will be connected to paging party.

Feature Code

| Page | Page Code (GHX-308A) | Page Code(GHX-616) |
| --- | --- | --- |
| All Call Page | Not used | #0 |
| Internal Zone 1 Page | Not used | #1 |
| Internal Zone 2 Page | Not used | #2 |
| Internal All Call Page | Not used | #5 |
| External Zone Page | #6 | #6 |
| Meet Me | ## | #7 |

Programming

1.  	Flash 40, Page A, Button 1 :  Page Access Yes/No

2.  	Flash 40, Page B, Button 5 (GHX-616 only) : Page Zone Assignment

3.  	Flash 22, Button 1 :  Page Warning Tone Yes/No

4.  	Flash 23, Button1, 2 : Program Relay Control

Conditions

1. 	Meet Me Page does not concern the Page Access Restriction.

2. 	To activate External Page, the MFU should be installed.

PAUSE TIMER

Description

When dialing a speed number, a timed pause in digit sending can be placed in the number. The length of this pause can be programmed in the system database.

Programming

1. 	Flash 05 :  Pause Timer

Related Feature

1. 	Station/System Speed Dial

Conditions

1. 	When pressing HOLD button as an entry into a speed dial bin, it serves as a command to the

system to provide a timed pause before resuming digit sending.

2. 	Successive entries of the HOLD will provide successive timed pause.

3. 	Each programmed pause utilizes one of the spaces for speed dialing digit.

PBX DIAL CODES

Description

The system will allow 5 one- or two-digit access codes to be entered into memory. When one of these codes is dialed, the signals the KSU that toll restriction is to be applied at the next dialed digits after the code. If one of these codes is not dialed, toll restriction does not apply. This allows the dialing of PBX extensions 100. 101, 107, etc. The line must be marked as PBX type and toll restriction must be enabled.

Programming

1. 	Flash 30, Page A, Button 7 : CO/PBX

2. 	Flash 17 : PBX Access Codes

Conditions

1. 	In CO line programming, the line must be marked as PBX type and toll restriction must be enabled.

PHONE BOX

Description

A convenient phone box for receiving announcements, intercom call and signaling can be connected the system. Stations programmed to receive alarm signal (Program Code: Flash 40) will receive a call from a phone box, and can answer the call by dialing phone box number.

Stations with a flexible button programmed as DSS of a phone box can answer a call from the phone box by pressing the flashing DSS button.

To call a phone box, stations can use either the dial pad or the DSS button.

Programming

1. 	Flash 21 : Phone box call signal select

2. 	Flash 25 : Phone box Call Timer

3. 	Flash 40, Page A, Button 8 : Phone box Call Signal Receive Yes/No

4. 	Flash 40, Page B, Button 1 : Station ID

Conditions

- If a phone box originated call is not answered within programmed period, the associated DSS

- button at all key stations will extinguish and the LED of the call button will extinguish, too.

2. 	A phone box can be a part of a conference.

3. 	For proper operation, the station ID should be programmed as a phone box. (Flash 40, Button 1)

PICKUP

Description

Directed Call Pickup

A station can pick up an intercom or CO line call ringing on a specific unattended station. The call must be a tone ringing call.

Group Call Pickup

Stations can be placed in one or more of 8 pickup groups. Stations within the same group can pick up tone ringing intercom calls and recalling or CO line calls for another station.

Branch Group

Stations belonging to Pick-up group 7 or 8 can be used as Branch Group. By lifting handset (or pressing MON button), you can answer the incoming call on another station in the same Branch Group.

Operation

After hearing an unattended ringing (incoming, transferred, intercom, recall rings) at a particular station, dial Pickup code, then you will be connected to the calling party.

Feature Code

(Keyset)

'*' + station number :  Direct Call Pickup

'**' :  Group Call Pickup

(SLT)

'*' + station number :  Direct Call Pickup

'**' :  Group Call Pickup

Programming

1. 	Flash 40, Page B, Button 4 :  Pickup Group

Conditions

1.   	Directed call pickup is allowed regardless of pickup group assignment.

2.   	Group call pickup is allowed to the two stations in the same pickup group.

- Direct button appearance of the CO line is required for pickup.

- If you are belongs to pickup group 7 or 8 (Intercom Branch Group), Group Pickup is available only

- by lifting handset without dialing "**".

POOL BUTTON (CO GROUP BUTTON)

Description

The pool key is used to access CO line that doesn't appear on a station so that outgoing call may be made. When dialing '9', an idle CO in the first access allowed line group will be seized.

Feature Code

1. 	9 : Line Group Access (The first access allowed group)

- 81-83 (GHX-308A), 81-87(GHX-616) : Line Group Access

Programming

1. 	Flash 30, Page B, Button 2 : CO line group Assignment

2. 	Flash 40, Page B, Button 7 : CO Group Access

QUEUING

Description

When all the CO lines allowed to access are busy, stations can be placed in queue awaiting a line to become available. Users are signaled when the line is available. If a station doesn't answer the queue signal within 15 seconds, that station is dropped from the queue.

Operation

Queue Request

- 	After receiving busy tone, press CALLBK button. You will hear confirmation tone. Then you can

see a special display in your LCD and your CALLBK button will be lighted steady.

Queue Recall

When a CO line of the group that you queued is idle, you hear queue recall ring. The queue recall can be answered;

- 	by pressing the line button.

- 	by going off-hook with Ringing Line Preference answer.

Queue Cancel

1. 	Press MON button.

2. 	Press CALLBK button and CALLBK button is extinguished.

Feature Code

1. 	TRANS 756 : 	(4 Button) CO Line Queuing

2. 	756 :	 (SLT) CO Line Queuing

REDIAL

Description

The system allows following 3 (4 in GHX-616) kinds of Redial. Up to 32 digits can be stored.

1) 	Dial Memo

2) 		Last Number Redial

3) 		Save Number Redial

4) 	Automatic Call Number Redial (GHX-616 only)

Operation

To redial the number;

- 	To dial the last number dialed lately on a CO line, dial Last Number Redial code.

- 	To dial the save number on a CO line, dial Save(Memo) Number Redial code.

Last Number Redial

Redial the last telephone number dialed on a CO line. Up to 32 digits can be stored. CO line selection is automatic.

Save Number Redial

Any number dialed on a CO line can be saved permanently to be used at any time by pressing the SPEED button twice before terminating the call. To dial the saved number, press “SPEED + #”.

To save the number;

If you wish to save a number you dialed:

- 	Before terminating the call, press SPEED button twice.

If you wish to save a number during CO conversation:

1. 	During CO conversation, press pre-programmed Dial Memo button.

2. 	Dial the number you wish to save with dial pad.

3. 	Press pre-programmed Dial Memo button again.

4. 	This feature does not have any influence on CO conversation.

To display a saved number;

- 	Press pre-programmed Dial Memo button while on-hook.

Automatic Call Number Redial (GHX-616 only)

This feature allows a station user put himself into an Automatic Redial Mode to automatically call back a busy number or no answer number on outgoing CO/PBX lines by pressing a flexible button programmed as ACNR feature (REDIAL button for GK-24/36 S/E keyset user). The maximum number of automatic redial retry is 3 times with time interval 030-255 sec.

To activate Automatic Call Number Redial;

While hearing busy tone after placing a CO call, press the ACNR button (REDIAL button for GK-24/36 keyset user) and go on-hook.

To deactivate Automatic Call Number Redial;

Press flashing ACNR button. (REDIAL button for GK-24/36 keyset user)

Programming

1.  FLASH 38 (GHX-616) : Auto Redial Timer (30-255 seconds)

Feature Code

1.  	SPD* : Last Number Redial

2.  	750, 75# (SLT in GHX-308A) : Last Number Redial

3.  	750, ## (SLT in GHX-616) : Last Number Redial

4.  	SPD # : Save Number (Memo) Redial

5.  	774 : Dial Memo (Button assignment only)

6.  	767 (GHX-616 only) : Automatic Call Number Redial (Button assignment only)

RINGING LINE PREFERENCE

Description

A station with ringing line preference can answer any assigned ringing CO line by simply lifting the handset or pressing MON button.

Programming

1.	Flash 40, Page A, Button 5 : Preferred Line Answer Enable/Disable

SPEED DIAL

Description

The system allows the following two kinds of Speed Dial;

1) 	Station Speed Dial

Up to 20 station speed numbers are programmed by individual station.

2) 	System Speed Dial

Up to 80 commonly dialed system speed numbers are programmed by system attendant for use by stations allowed this feature. The last 40 numbers(bin 60-99) will not be monitored by toll restriction.

Numbers are dialed by use of the Speed Dial code.

Speed bin 00 can be used to program user's name so that it appears on calling/called display phones instead of station number.

Operation

(Keyset)

To program special commands;

- 	for Pause, press HOLD button.

- 	for Flash, press FLASH button.

- 	for Pulse-to-Tone switch-over, press TRANS button.

To store station speed numbers (at Station) or

To store system speed numbers (at Attendant);

1. 	Press SPEED button once.

2. 	Press SPEED button or, a CO line button or CO group button.

3. 	Dial Speed bin number. (00-19 for station speed or 20-99 for system speed numbers)

4. 	Dial telephone number to be stored.

5. 	Press SPEED button.

To use Speed Numbers;

- 	Press SPEED button and dial speed bin number (00-99).

To erase Speed Number (at Station or Attendant);

1. 	Press SPEED button twice.

2. 	Dial Speed bin number (00-19 or 20-99).

3. 	Press SPEED button twice.

(SLT)

To store station speed numbers;

- 	Lift the handset and dial "755".

- 	Dial the speed bin number (00-19).

- 	Enter the telephone number and perform the Hook Flash.

To use Speed Numbers;

- 	Lift the handset and dial "758" or "75*".

- 	Dial the speed bin number.

To erase Speed Number (at Station or Attendant);

- 	Lift the handset and dial "755".

- 	Dial the speed bin number (00-19) and perform the Hook Flash.

Feature Code

1. 	SPD xx : (xx= 00-99, Keyset) Speed Dial

2. 	75*xx, 758 xx : (SLT) Speed Dial

Programming

1. 	Flash 40, Page A, Button 3 : System Speed Dial Access Yes/No

Related Feature

1. 	Name Display

TIME DISPLAY

Description

The system provides time and date for display key telephone. Attendant or the programming station can enter the system date and time. The time and date in GHX-308A can’t be kept during commercial power failure because it is provided by software.

Operation

1. 	Dial Setting Time and Date code “739”.

2. 	Dial 10 digits data for YYMMDDHHmm.(YY-Year, MM-Month, DD-Day, HH-Hour, mm-minute)

3. 	You will hear confirmation tone and see updated time and date.

Or,

1. 	Enter Admin. Program mode Flash 28.

2. 	Dial 10 digits data for YYMMDDHHmm. (YY-Year, MM-Month, DD-Day, HH-Hour, mm-minute)

3. 	Press HOLD button.

Feature Code

1. 	735 (Attendant) : 	Change System Date Format

2. 	736 (Attendant) :	Change System Time Format

3. 	739 (Attendant) : 	Setting Time and Date

Condition

The system time and date can’t be kept during commercial power failure in GHX-308A.

Programming

1. 	Flash 28 :	 Setting Time and Date

UNIVERSAL NIGHT ANSWER (UNA)

Description

When CO lines are marked UNA and the system is placed into night service, a SLT or Keyset (with a direct or loop button) can answer incoming calls on lines, it does not normally have access to by dialing "777".

Operation

To answer incoming calls, lift handset and dial UNA code "777".

Feature Code

1. 	777 : UNA code

Programming

1. 	Flash 30, Page A, Button 2 : UNA Line Yes/No

Related Feature

1. 	Night Transfer

VERSION DISPLAY

Description

Software version can be displayed by dialing “736” at the station with LCD.

Operation

Dial S/W Version Display code "736".

Feature Code

1. 	736 : S/W Version Display

3 MINUTE WARNING

Description

A station user on a CO line conversation can be given a periodic warning signal every 3 minute.

Programming

1. 	Flash 40, Page A, Button 6 : 3 minute warning Enable/Disable

SECTION 7.  SYSTEM PROGRAMMING

7.1  PROGRAM CODES LIST

7.1.1  Program Code List for GHX-308A

| PROGRAM CODE | BUTTON ENTRY | FUNCTION | RANGE OF PARAMETER | DEFAULT VALUE |
| --- | --- | --- | --- | --- |
| FLASH 01 |  | SYSTEM HOLD RECALL TIMER | 000-255 sec | 060 sec |
| FLASH 02 |  | EXCLUSIVE HOLD RECALL TIMER | 000-255 sec | 180 sec |
| FLASH 03 |  | TRANSFER RECALL TIMER | 000-255 sec | 045 sec |
| FLASH 04 |  | PRESET FORWARD TIMER | 00-99 sec | 10 sec |
| FLASH 05 |  | PAUSE TIMER | 1-9 sec | 2 sec |
| FLASH 06 |  | CALL PARK RECALL TIMER | 000-255 sec | 180 sec |
| FLASH 07 |  | CONFERENCE TIMER | 00-99 min | 10 min |
| FLASH 08 |  | MESSAGE WAIT REMINDER TONE TIMER | 000-104 min | 000 min |
| FLASH 09 |  | NOT USED |  |  |
| FLASH 10 |  | CO RING DETECT TIMER | 2-9 (x100) msec | 3(x100)msec |
| FLASH 11 | 1 | HOLD PREFERENCE | System/Exclusive | System |
| FLASH 12 | 1 | AUTOMATIC PRIVACY | Yes /No | Yes |
|  | 2 | PRIVACY WARNING TONE | Yes/No | No |
| FLASH 13 | 1 | ATTENDANT CALL CODE (0) | Yes/No | Yes |
| FLASH 14 | 1 | INCOMING CO TOLL | Yes/No | No |
| FLASH 15 |  | ATTENDANT STATION ASSIGNMENT | Sta No. | 100 |
| FLASH 16 |  | LOUD BELL CONTROL | Sta No. | None |
| FLASH 17 |  | PBX DIALING CODES | 5 codes upto 2 digits | None |
| FLASH 18 | 1 - 4 | EXECUTIVE/SECRETARY TRANSFER | 4 pairs | None |
| FLASH 19 | 1 - 8 | HUNT GROUP | up to 8 stations/groups |  |
| FLASH 20 |  | NOT USED |  |  |
| FLASH 21 |  | PHONE BOX SIGNAL | Once/Repeat | Once |
| FLASH 22 |  | PAGE WARNING TONE | Yes/No | Yes |
| FLASH 23 | 1,2 | PROGRAM CONTROL RELAY | Loud Bell/Door Open | Loud Bell |
| FLASH 24 |  | NOT USED |  |  |
| FLASH 25 |  | PHONE BOX CALL TIMER | 00-60 sec | 20 sec |
| FLASH26- 27 |  | NOT USED |  |  |
| FLASH 28 |  | SETTING TIME AND DATE |  |  |
| FLASH 29 | 1 | GROUP LISTENING | Yes/No | No |
| FLASH 30 |  | CO LINE ATTRIBUTES |  |  |
|  | 1A | DIAL SIGNALING | DTMF/PULSE | DTMF |
|  | 2A | UNA | Yes/No | Yes |
|  | 3A | TOLL OVERRIDE | Yes/No | No |
|  | 4A | GROUND FLASH | Yes/No | No |
|  | 5A | PULSE RATIO | 60/40 or 66/33 | 66/33 |
|  | 6A | PULSE SPEED | 10 pps or 20 pps | 10 pps |
|  | 7A | CO/PBX | 0(CO) / 1(PBX) | 0(CO) |
|  | 8A | LOOP SUPERVISION | 0-9 min | 0 |
|  | 9 | SELECT PAGE A |  |  |
|  | 1B | FLASH TIMER | 01-20(x100) msec | 10(x100) msec |
|  | 1B | FLASH TIME | 01-20 (x 100ms) | 10x100ms |
|  | 2B | CO LINE GROUP | 0 - 4 (7) | 1 |
|  | 3B | NOT USED |  |  |
|  | 10 | SELECT PAGE B |  |  |
| PROGRAM CODE | BUTTON ENTRY | FUNCTION | RANGE OF PARAMETER | DEFAULT VALUE |
|  | 11 | NEXT CO NO. |  |  |
|  | 12 | PREVIOUS CO NO. |  |  |
| FLASH 31 |  | HOOK FLASH TIME | 01-20(x100)msec | 05(x100)ms |
| FLASH 32 |  | NOT USED |  |  |
| FLASH 33 |  | ATTENDANT RECALL TIMER | 000-255 sec | 030sec |
| FLASH 34 |  | NOT USED |  |  |
| FLASH 35 | 1 | RING TYPE SELECT | LED ON /OFF KST: 0.4/0.4/0.4/2 SLT: 0.4/0.2/0.4/4 or KST: 0.8/2.4 SLT: 1/4 | LED ON INTERCOM RING KST: 0.4/0.4/0.4/2 SLT: 0.4/0.2/0.4/4 CO RING: KST: 0.8/2.4 SLT: 1/4 |
| FLASH 36 | 1-2 | STA NUMBER CHANGE | 100-699 | 100-107 |
| FLASH 37 | 1 | METHOD OF SELECTING CO LINE | CIRCULE (Yes/No) | Yes |
| FLASH 38 |  | NOT USED |  |  |
| FLASH 39 |  | MULTI LINE CONFERENCE | Yes/No | No |
| FLASH 40 |  | STATION ATTRIBUTES |  |  |
|  | 1A | PAGE | Yes/No | No |
|  | 2A | DND | Yes/No | No |
|  | 3A | SYSTEM SPEED DIAL | Yes/No | Yes |
|  | 4A | HEADSET | Yes/No | No |
|  | 5A | PREFERRED LINE ANSWER | Yes/No | Yes |
|  | 6A | 3 MINUTE WARNING | Yes/No | No |
|  | 7A | CALL FORWARD | Yes/No | No |
|  | 8A | PHONE-BOX CALL | Yes/No | No |
|  | 1B | STATION ID | 1 - 6 |  |
|  | 2B | CLASS OF SERVICE | 11 - 66 | 11 |
|  | 3B | NOT USED |  |  |
|  | 4B | PICK-UP GROUP | 0 - 8 | 1 |
|  | 5B | NOT USED |  |  |
|  | 6B | PRESET FORWARD STATION | Sta. No. | None |
|  | 7B | CO GROUP ACCESS | dial 1 - 5 | All (123) |
|  | 8B | CO BUTTON ASSIGNMENT | BBLLR | Square |
|  | 9B | CO BUTTON DISPLAY | BBLLR | Square |
|  | 10 | SELECT PAGE A |  |  |
|  | 11 | SELECT PAGE B |  |  |
|  | 12 | SELECT STATION RANGE |  |  |
| FLASH 41- 44 |  | NOT USED |  |  |
| FLASH 45 | 1 | PREVENT ILLEGAL DIAL | Yes/No | No |
| FLASH 46 |  | CO DIAL WAIT TIMER | 000-255 | 030 |
| FLASH 47- 49 |  | NOT USED |  |  |
| FLASH 50 |  | EXCEPTION TABLES |  |  |
|  | 1 | ALLOW TABLE A | 10 digits (30 bins) 1) | None |
|  | 2 | DENY TABLE A | 10 digits (30 bins) 1) | None |
|  | 3 | ALLOW TABLE B | 10 digits (30 bins) 1) | None |
|  | 4 | DENY TABLE B | 10 digits (30 bins) 1) | None |
| FLASH 52 |  | STATION ATTRIBUTE-2 |  |  |
| * 1) GHX-308A : over S/W Ver. 3.0A |  |  |  |  |
| PROGRAM CODE | BUTTON ENTRY | FUNCTION | RANGE OF PARAMETER | DEFAULT VALUE |
|  | 1 | LCD DISPLAY | KOR/ENG | ENG |
|  | 2 | ICM TRANSFER | Yes/No | No |
|  | 4 | DOOR OPEN | Yes/No | No |
| FLASH 53 | 1-6 | HOT LINE SELECT |  | None |
| FLASH 54 |  | DOOR OPEN TIMER | 05-99 (100msec) | 20 (x100msec) |
| FLASH 55 |  | NOT USED |  |  |
| FLASH 56 |  | CO DIAL DELAY TIME | 00-99 (x100msec) | 05 (x100msec) |
| FLASH 57 |  | HOT LINE TIMER | 0-9 sec | 0 sec |
| FLASH 58- |  | NOT USED |  |  |
| 59 |  |  |  |  |
| FLASH 60 |  | INITIALIZE SYSTEM PARAMETERS |  |  |
| FLASH 61 |  | INITIALIZE CO LINE ATTRIBUTES |  |  |
| FLASH 62 |  | INITIALIZE STATION ATTRIBUTES |  |  |
| FLASH 63 |  | INITIALIZE EXCEPTION TABLES |  |  |
| FLASH 64 |  | INITIALIZE SYSTEM SPEED NUMBERS |  |  |
| FLASH 65 |  | INITIALIZE ALL DATABASE |  |  |
| FLASH 66- 75 |  | NOT USED |  |  |
| FLASH 76 | 1-2 | FAX CO, STA ASSIGNMENT | CO:01-03, STA:100-699 | None |
| FLASH 77 |  | FAX TONE DETECT TIMER | 05-30 sec | 05sec |
| FLASH 78 |  | FAX CO CALL TIMER | 1-3 min | 1 min |
| FLASH 79 | 1 | SLT RING OFF TIME | 3-4 sec | 4 sec |

7.1.2  Program Code List for GHX-616

| PROGRAM CODE | BUTTON ENTRY |  | FUNCTION | RANGE OF PARAMETER | DEFAULT VALUE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FLASH 01 |  |  | SYSTEM HOLD RECALL TIMER | 000-255 sec | 060 sec |  |  |
| FLASH 02 |  |  | EXCLUSIVE HOLD RECALL TIMER | 000-255 sec | 180 sec |  |  |
| FLASH 03 |  |  | TRANSFER RECALL TIMER | 000-255 sec | 045 sec |  |  |
| FLASH 04 |  |  | PRESET FORWARD TIMER | 00-99 sec | 10 sec |  |  |
| FLASH 05 |  |  | PAUSE TIMER | 1-9 sec | 2 sec |  |  |
| FLASH 06 |  |  | CALL PARK RECALL TIMER | 000-255 sec | 180 sec |  |  |
| FLASH 07 |  |  | CONFERENCE TIMER | 00-99 min | 10 min |  |  |
| FLASH 08 |  |  | MESSAGE WAIT REMINDER TONE TIMER | 000-104 min | 000 min |  |  |
| FLASH 09 |  |  | NOT USED |  |  |  |  |
| FLASH 10 |  |  | CO RING DETECT TIMER | 2-9 (x100) msec | 3(x100) msec |  |  |
| FLASH 11 | 1 |  | HOLD PREFERENCE | System/Exclusive | System |  |  |
| FLASH 12 |  |  | AUTOMATIC PRIVACY |  |  |  |  |
|  | 1 |  | PRIVACY | Yes/No | Yes |  |  |
|  | 2 |  | WARNING | Yes/No | No |  |  |
| FLASH 13 | 1 |  | NOT USED |  |  |  |  |
| FLASH 14 | 1 |  | INCOMING TOLL RESTRICTION | Yes/No (STANDARD) | No |  |  |
| FLASH 15 |  |  | ATTENDANT STATION ASSIGNMENT | Sta No. | 100 |  |  |
| FLASH 16 |  |  | LOUD BELL CONTROL | Sta No. | None |  |  |
| FLASH 17 |  |  | PBX DIALING CODES | 5 codes up to 2 digits | None |  |  |
| FLASH 18 | 1 - 4 |  | EXECUTIVE/SECRETARY TRANSFER | 4 pairs | None |  |  |
| FLASH 19 | 1 - 8 |  | HUNT GROUP | upto 8 stations/group |  |  |  |
| FLASH 20 |  |  | SMDR PARAMETERS |  |  |  |  |
|  | 1A |  | SMDR | Yes/No | No |  |  |
|  | 2A |  | CALL TYPE | All Call/LD | LD |  |  |
|  | 3A |  | PRINT COLUMNS | 80/29 | 80 |  |  |
|  | 4A |  | BAUD RATE | 300/1200/2400/4800/ 9600 | 1200 |  |  |
|  | 5A |  | PRINT INCOMING CALL | Yes/No | No |  |  |
|  | 6A |  | SAVE CALLS | Yes/No | No |  |  |
|  | 9 |  | SELECT PAGE A |  |  |  |  |
|  | 10 |  | SELECT PAGE B |  |  |  |  |
|  | 1B |  | CURRENT UNIT | 3 characters | DOL |  |  |
|  | 2B |  | COST UNIT | 4 digits | 0025 |  |  |
|  | 3B |  | FRACTION | 1 digit | 0-4 |  |  |
|  | 4B |  | NOT USED |  |  |  |  |
| FLASH 21 |  |  | ALARM PARAMETERS |  |  |  |  |
|  | 1 |  | ALARM | Yes/No | No |  |  |
|  | 2 |  | SIGNAL | Repeat/Once | Once |  |  |
|  | 3 |  | LOOP DETECT | Close/Open | Close |  |  |
| FLASH 22 |  |  | PAGE WARNING TONE | Yes/No | Yes |  |  |
| FLASH 23 | 1-2 |  | PROGRAM CONTROL RELAY | LBC/DOOR/EPAGE /EMOH | LBC |  |  |
| FLASH 24 |  |  | NOT USED |  |  |  |  |
| FLASH 25 |  |  | PHONE BOX CALL TIMER | 00-60 sec | 20 sec |  |  |
| FLASH 26 |  |  | NOT USED |  |  |  |  |
| FLASH 27 |  |  | RECALL TO ALL STATION | Yes/No |  | No |  |
| FLASH 28 |  |  | SETTING TIME AND DATE |  |  |  |  |
| FLASH 29 | 1 |  | GROUP LISTENING (MONITOR MODE) | Yes/No | No |  |  |
| FLASH 30 |  |  | CO LINE ATTRIBUTES |  |  |  |  |
|  | 1A |  | DIAL SIGNALING | DTMF/PULSE | DTMF |  |  |
|  | 2A 3A |  | UNA TOLL OVERRIDE | Yes/No Yes/No | Yes No |  |  |
|  | 4A |  | GROUND FLASH | Yes/No | No |  |  |
|  | 5A |  | PULSE RATIO | 60/40 or 66/33 | 66/33 |  |  |
|  | 6A |  | PULSE SPEED | 10 pps or 20 pps | 10 pps |  |  |
|  | 7A |  | CO/PBX | 0(CO) / 1(PBX) | 0(CO) |  |  |
|  | 8A |  | NOT USED |  |  |  |  |
|  | 9 |  | SELECT PAGE A |  |  |  |  |
|  | 1B |  | FLASH TIMER | 01-20(x100) msec | 10(x100) msec |  |  |
|  | 2B |  | CO LINE GROUP | 0 - 7 | 1 |  |  |
|  | 4B |  | DISA MODE | 0:No, 1:24Hour, 2:Day 3:Nite | No DISA |  |  |
|  | 10 |  | SELECT PAGE B |  |  |  |  |
|  | 11 |  | NEXT CO NO. |  |  |  |  |
|  | 12 |  | PREVIOUS CO NO. |  |  |  |  |
| FLASH 31 |  |  | HOOK SWITCH TIME | 01-20(x100)msec | 05(x100)msec |  |  |
| FLASH 32 |  |  | CO RELEASE GUARD TIME | 00-20 | 00sec |  |  |
| FLASH 33 |  |  | ATTENDANT RECALL TIMER | 000-255 sec | 030sec |  |  |
| FLASH 34 | 1-8 |  | UNIVERSAL CALL DISTRIBUTION |  |  |  |  |
| FLASH 35 | 1 |  | RING TYPE SELECT | LED ON /OFF KST: 0.4/0.4/0.4/2 SLT: 0.4/0.2/0.4/4 or KST: 0.8/2.4 SLT: 1/4 | LED OFF CO Ring KST: 0.4/0.4/0.4/2 SLT:0.4/0.2/0.4/4 Intercom Ring: KST: 0.8/2.4 SLT: 1/4 |  |  |
| FLASH 36 |  |  | FLEXIBLE NUMBERING | 100 - 699 | 100-115 |  |  |
| FLASH 37 | 1 |  | METHOD OF SELECTING CO LINE | CIRCULE (Yes/No) | Yes |  |  |
| FLASH 38 |  |  | AUTO REDIAL TIMER | 030-255 sec | 060 sec |  |  |
| FLASH 39 |  |  | MULTI LINE CONFERENCE | Yes/No | No |  |  |
| FLASH 40 |  |  | STATION ATTRIBUTES |  |  |  |  |
|  | 1A |  | PAGE | Yes/No | Yes |  |  |
|  | 2A |  | DND | Yes/No | Yes |  |  |
|  | 3A |  | SYSTEM SPEED DIAL | Yes/No | Yes |  |  |
|  | 4A |  | HEADSET | Yes/No | No |  |  |
|  | 5A |  | PREFERRED LINE ANSWER | Yes/No | Yes |  |  |
|  | 6A |  | 3 MINUTE WARNING | Yes/No | No |  |  |
|  | 7A |  | CALL FORWARD | Yes/No | Yes |  |  |
|  | 8A |  | ALARM SIGNAL | Yes/No | No |  |  |
|  | 1B |  | STATION ID | 1 - 7 | 1 |  |  |
|  | 2B |  | CLASS OF SERVICE | 11 - 66 | 11 |  |  |
|  | 3B |  | NOT USED |  |  |  |  |
|  | 4B |  | PICK-UP GROUP | 0 - 8 | 1 |  |  |
|  | 5B |  | PAGE ZONE | 0 - 2 | 1 |  |  |
|  | 6B |  | PRESET FORWARD STATION | Sta. No. | None |  |  |
|  | 7B |  | CO GROUP ACCESS | dial 1 - 7 | All |  |  |
|  | 8B |  | CO BUTTON ASSIGNMENT | BBLLR | Square |  |  |
|  | 9B |  | CO BUTTON DISPLAY | BBLLR | Square |  |  |
|  | 10 |  | SELECT PAGE A |  |  |  |  |
|  | 11 |  | SELECT PAGE B |  |  |  |  |
|  | 12 |  | SELECT STATION RANGE |  |  |  |  |
| FLASH 41 | 1 |  | CO TO CO OFF-NET FORWAED | Yes/No | No |  |  |
| FLASH 42 | 1 |  | CALL METERING UNIT | Yes/No | No |  |  |
| FLASH 43 | 1 |  | PRESELECTED MESSAGE SELECTION | Type I / Type II | Type I |  |  |
| FLASH 44 |  |  | CALL FORWARD NO ANSWER TIMER | 000 - 255 | 015 sec |  |  |
| FLASH 45 |  |  | PREVENT ILLEGAL DIAL | Yes/No | No |  |  |
| FLASH 46 |  |  | CO DIAL WAITING TIMER | 00-99(100m sec) | 05 (100msec) |  |  |
| FLASH 47 | 1 |  | ATTENDANT CALL CODE(0) SELECTION | Yes/No | Yes |  |  |
| FLASH 48 |  |  | SMDR TIMER | 000-255 | 000 |  |  |
| FLASH 49 |  |  | HOOK FLASH DEBOUNCE TIME | 01-99 (100 msec) | 10 (100 msec) |  |  |
| FLASH 50 |  |  | EXCEPTION TABLES |  |  |  |  |
|  | 1 |  | ALLOW TABLE A | 10 digits (16 bins) | None |  |  |
|  | 2 |  | DENY TABLE A | 10 digits (16 bins) | None |  |  |
|  | 3 |  | ALLOW TABLE B | 10 digits (16 bins) | None |  |  |
|  | 4 |  | DENY TABLE B | 10 digits (16 bins) | None |  |  |
| FLASH 51 |  |  | MUSIC SOURCE CONTROL | 0-2 (INT,EXT, VAU) | 0 (INTERNAL) |  |  |
| FLASH 52 |  |  | STATION ATTRIBUTE II |  |  |  |  |
|  | 1 |  | OVERRIDE | Yes/No | No |  |  |
|  | 2 |  | DOOR OPEN | Yes/No | No |  |  |
|  | 3 |  | ICM TRANSFER | Yes/No | No |  |  |
|  | 4 |  | BGM | Yes/No | No |  |  |
|  | 5 |  | OFF-NET | Yes/No | No |  |  |
| FLASH 53 | 1-6 |  | HOT LINE SELECT | FLX/COL/GRP/STA /SPD/NONE | None |  |  |
| FLASH 54 |  |  | DOOR OPEN TIMER | 05-99 (100msec) | 20 (100msec) |  |  |
| FLASH 55 |  |  | VAU MODE PROGRAM | 1-6 | 3 |  |  |
| FLASH 56 |  |  | CO DIAL DELAY TIME | 00-99(100m sec) | 05 (100msec) |  |  |
| FLASH 57 |  |  | HOT LINE TIMER | 0-9 sec | 0 sec |  |  |
| FLASH 58 |  |  | VAU SEGMENT CONTROL | 1: DISA day msg | SEG1 -1 |  |  |
|  |  |  |  | 2: DISA night msg | SEG2 - 3 |  |  |
|  |  |  |  | 3: DISA busy msg | SEG3 - 4 |  |  |
|  |  |  |  | 4: DISA error dial msg | SEG4 - 5 |  |  |
|  |  |  |  | 5: DISA no dial msg |  |  |  |
|  |  |  |  | 6: MOH |  |  |  |
|  |  |  |  | 7: Not used |  |  |  |
|  |  |  |  | 8: UCD1 |  |  |  |
|  |  |  |  | 9: UCD2 |  |  |  |
| FLASH 59 |  |  | VAU CHANNEL CONTROL | MOH, DISA, UCD | CHANNEL 1-3 ALL DISA |  |  |
| FLASH 60 |  |  | INITIALIZE SYSTEM PARAMETERS |  |  |  |  |
| FLASH 61 |  |  | INITIALIZE CO LINE ATTRIBUTES |  |  |  |  |
| FLASH 62 |  |  | INITIALIZE STATION ATTRIBUTES |  |  |  |  |
| FLASH 63 |  |  | INITIALIZE EXCEPTION TABLES |  |  |  |  |
| FLASH 64 |  |  | INITIALIZE SYSTEM SPEED NUMBERS |  |  |  |  |
| FLASH 65 |  |  | INITIALIZE ALL DATABASE |  |  |  |  |
| FLASH 66 |  |  | DISA NO ANSWER TIMER | 000-255 sec | 060 sec |  |  |
| FLASH 67 |  |  | DISA ACCESS CODE | XXX (3 digits) | 000 (no DISA access code) |  |  |
| FLASH 68 |  |  | DISA VOICE GUIDE SELECT | Yes/No | No |  |  |
| FLASH 69 |  |  | CO ACCESS OF DISA | Yes/No | No |  |  |
| FLASH 70 |  |  | PRINT SYSTEM DATA |  |  |  |  |
| FLASH 71 |  |  | PRINT SYSTEM PARAMETERS |  |  |  |  |
| FLASH 72 |  |  | PRINT COLINE ATTRIBUTES | 4 digit CO Line Range |  |  |  |
| FLASH 73 |  |  | PRINT STATION ATTRIBUTES | 6 digit Station Range |  |  |  |
| FLASH 74 | 1-4 |  | PRINT EXCEPTION TABLES | 2 digit Bin Number |  |  |  |
| FLASH 75 |  |  | PRINT SYSTEM SPEED NUMBERS | 4 digit Bin Range |  |  |  |
| FLASH 76 |  |  | UCD MEMBER FIRST CHOICE | Yes/No | No |  |  |
| FLASH 77 | 1 |  | DISA, DIAL ONLY HUNT GROUP | Yes/No | No |  |  |
|  | 2 |  | DISA, CONVERT 1 DIAL TO HUNT | Yes/No | No |  |  |
|  | 3 |  | DISA, BUSY CALLING HUNT | Yes/No | No |  |  |
| FLASH 78 |  |  | DISA NIGHT ACTION | 1: same as DISA-Day 2: Tone 3: Pre-select num | 1: Same as DISA Day |  |  |
| FLASH 79 | 1-5 |  | DISA TREATMENT |  |  |  |  |
|  | 1 |  | DISA BUSY | 1 : TONE | RETRY |  |  |
|  | 2 |  | DIAL ERROR | 2 : ATTD | RETRY |  |  |
|  | 3 |  | NO ANSWER | 3 : RETRY | ATTD |  |  |
|  | 4 |  | INTER DIGIT TIME OUT |  | TONE |  |  |
|  | 5 |  | DISCON TONE DETECT |  | TONE |  |  |
| FLASH 80- 82 |  |  | NOT USED |  |  |  |  |
| FLASH 83 | 1,2 |  | VAU TONE CADENCE | 000-255 (50msec) | ON:005, OFF:005 |  |  |
| FLASH 84 |  |  | VAU TONE TIME | 3-9 sec | 5 |  |  |
| FLASH 85 |  |  | DISA MESSAGE REPLAY | YES/NO | NO |  |  |
| FLASH 86 |  | 1-2 | FAX CO, STA ASSIGNMENT | CO:01-06, STA:101-699(SLT) |  |  | None |
| FLASH 87 |  |  | FAX TONE DETECT TIMER | 05-30 sec |  |  | 05sec |
| FLASH 88 |  |  | FAX CO CALL TIMER | 1-3 min |  |  | 1 min |
| FLASH 89 |  | 1 | SLT RING OFF TIME | 3-4 sec |  |  | 4 sec |

7.2  INTRODUCTION

The GHX-308A/616 Key Telephone System can be programmed to meet each customer's individual needs. All programming are done at station 100(Extension #1) using a key telephone. It can be done with either a 21-button or 33-button in display telephone. However, using a display phone will make programming easier. This display will indicate what features are being programmed and the current programming of these features. When data is changed, it will be displayed so you can see what has been entered.

Upon entering the program mode, the key telephone at station 100 no longer operates as a telephone but as a programming instrument with all of the buttons redefined. These keys of the dial pad are used to enter the various data fields and to enter numerical information such as timers. The 12 buttons are located at the top of program features such as class of service, line groups, station features, toll restriction table, etc.

At the time system is installed, upon entering the program mode, the system MUST be initialized to load default data into memory. If this pre-programming suits the customer, initialization is all that is needed.

Anytime data is to be changed, the program mode must be entered and then the individual program code. Use program codes to determine current programming or to change a specific feature.

During programming, other keysets are operated normally. If a program code is used to determine programming but nothing is changed (or changed but not entered into memory by pressing HOLD), the previous data will remain intact upon leaving the field. Program codes do not have to be used in any particular order.

When programming, tones are provided to help the programmer determines correct or incorrect entry of data. A solid tone indicates that the data was accepted. An interrupted tone means an error was made. When this occurs, re-enter the data. Until new data is entered and accepted, the system will continue to operate under default or previously programmed values.

Data is entered permanently into the database when pressing HOLD button

7.3  PROGRAM MODE ENTRY

Programming is done at station 100 using preferably a 21-or 33-button display telephone. It is always done at this station regardless of attendant assignment or class of service.

To enter the program mode, the programmer must verify that the keyset is properly connected to station 100 (Extension #1) at first.

1)	Go off-hook or press MON button to ON.

2)	Dial an asterisk (*).

3)	Dial 7-7-6-4 (PROG) and confirmation tone is heard. (System database is initialized at this point.)

4)	Press FLASH button.

5)	Dial 2-digit program code.

6)	Enter the data to be programmed or press program buttons to be changed.

7)	Press HOLD button. (Confirmation tone is heard.)

8)	Go On-hook or press MON button to exit.

All data is now stored in permanent memory and will affect system operation.

7.4  INITIALIZATION

The system has been pre-programmed with certain features which are called default data, these features are loaded into memory when the system is initialized. The system should ALWAYS be initialized when installed or at any time the database has been corrupted.

To initialize the system to default values, repeat the procedures explained below for each of the database area.

1)	Enter the programming mode.

2)	Press FLASH button.

3)	Dial 60 to initialize system parameters.

INITIALIZE SYS PARAM PRESS HOLD

4)	Press HOLD button.

5)	Repeat from step 2 for the other areas. In step 3, use to the following program codes.

60  for system parameters

61  for CO lines

62  for station parameters

63  for exception tables

64  for system speed numbers

- for all system data

7.5  SYSTEM PARAMETER PROGRAMMING

SYSTEM HOLD RECALL TIMER  (FLASH 01)

A CO line placed on system hold and left unattended will remain in a hold condition until the expiration of the timer. Once the timer expires, the line will recall the originator of the hold for the duration of the recall timer. (If not answered within the recall time, the recall will go to the attendant for the duration of Attendant Recall Timer.)

Parameters:

Determines the amount of time before a call placed on system hold will recall the station placing the hold.

Range:	000-255 seconds (000: No recall)

Default:	060 seconds

Procedure:

1. 	Press FLASH button and dial 01.

2. 	Dial field # 01.

3. 	If you have a display phone, you will see this display:

SYS HOLD RECALL 000-255 060

4. 	Enter 3 digits on the numeric keypad.

5. 	Press HOLD button.

EXCLUSIVE HOLD RECALL TIMER  (FLASH 02)

A CO line placed on exclusive hold and left unattended will remain in an exclusive hold condition until the timer expires. Once the timer expires, the line will recall the originator of the hold for the duration of the recall timer. (If not answered within the recall time, the recall will go to the attendant for the duration of Attendant Recall Timer.)

Parameters:

Determines amount of time before a call placed on exclusive hold will recall the station placing the hold.

Range:	000-255 seconds (000: No recall)

Default:	180 seconds

Procedure:

1.	Press FLASH button and dial 01.

2.	Dial field # 02.

3.	On a display phone, you will see this display:

EXC HOLD RECALL 000-255 180

4.	Enter 3 digits on the numeric keypad.

5.	Press HOLD button.

TRANSFER RECALL TIMER  (FLASH 03)

A CO line transferred to unattended or unanswered station will recall the station transferring the call upon the expiration of the timer.

Parameters:

Determines the amount of time and a transferred call will ring at the station receiving the transfer before it will recall the station which made the transfer.

Range:	000-255 seconds

Default:	045 seconds

Procedure:

1.	Press FLASH button and dial 01.

2. 	Dial field # 03.

3.	On a display phone, you will see this display:

TRANSFER RECALL 000-255 045

4.	Press HOLD button.

PRESET FORWARD TIMER  (FLASH 04)

Incoming CO lines will ring at the station until the expiration of the timer before being forwarded to a predetermined station.

Parameters:

Determines the amount of time a CO line will ring before being forwarded to a predetermined station.

Range:	00-99 seconds (00:No forward)

Default:	10 seconds

Procedure:

1.	Press FLASH button and dial 01.

2.	Dial field # 05.

3.	On a display phone, you will see this display:

PRESET FWD TIMER 00-99 10

4.	Enter 2 digits on the numeric keypad.

5.	Press HOLD button.

PAUSE TIMER  (FLASH 05)

When dialing a speed number, a timed pause in digit sending can be directed into the number during entry. The length of the pause is controlled by the pause timer.

Parameters:

Determines the length of the pause for use with automatically sent digits or other speed dialing.

Range:	1-9 seconds

Default:	2 seconds

Procedure:

1.	Press FLASH button and dial 01.

2.	Dial field # 06.

3.	On a display phone, you will see this display:

PAUSE TIMER 1 - 9 2

4.	Enter 1 digit on the numeric keypad.

5.	Press HOLD button.

CALL PARK RECALL TIMER  (FLASH 06)

A CO line connected to a station can be placed into one of six parking locations and can be retrieved from the originating or any other station by dialing the Unpark code. A parked call will recall to the station that parked the call when the call park timer expires.

Parameters:

Determines the amount of time before a call placed in a call park location will recall the station placing the park.

Range:	000-255 seconds (000:No Recall)

Default:	180 seconds

Procedure:

1.	Press FLASH button and dial 06.

2.	On a display phone, you will see this display:

CALL PARK TIMER 000-255 180

4.	Enter 3 digits on the numeric keypad.

5.	Press HOLD button.

CONFERENCE TIMER  (FLASH 07)

An unsupervised multi-line conference can continue until the expiration of the timer after the initiator of the conference has exited the conference.

The outside caller can use the CO line he/she has accessed for outgoing call until the expiration of the timer.

Parameters:

Determines the amount of time an unsupervised mode(unsupervised multi-line conference) can be continued.

Range:	00-99 minutes

Default:	10 minutes

Procedure:

1.	Press FLASH button and dial 07.

2.	On a display phone, you will see this display:

CONFERENCE TIMER 00-99 10

3.	Enter 2 digits on the numeric keypad.

4.	Press HOLD button.

MESSAGE WAIT REMINDER TONE TIMER  (FLASH 08)

A station can receive up to five independent messages. A station with a message waiting can be reminded at a timed interval with a tone.

Parameters:

Determines the amount of time between repeated reminder tones to a telephone in a message waiting condition.

Range:	000-104 minutes (000: No reminder tone)

Default:	000 minutes

Procedure:

1.	Press FLASH button and dial 08.

2.	On a display phone, you will see this display:

M/W TONE TIMER 000-104 000

3.	Enter 3 digits on the numeric keypad.

4.	Press HOLD button.

CO RING DETECT TIMER  (FLASH 10)

This parameter can be adjusted to all CO line ringing detection from all types of central offices and PBX's. When the duration of the ringing signal from CO lines is matched with the ringing detection circuitry, the CO lines begin to ring at assigned telephones.

Parameters:

This timer controls the time necessary to detect a CO line as ringing into the system.

Range:	2-9(x 100) msec

Default:	3 (x100) msec

Procedure:

1.	Press FLASH and dial 10.

2.	On a display phone, you will see this display:

CO RING DETECT 2 - 9 3

3.	Enter 1 digit on the number keypad.

4.	Press HOLD button.

HOLD PREFERENCE  (FLASH 11)

Hold preference allows either exclusive hold or system hold as the primary hold on the first depression of HOLD button.

Parameters:

The system may be programmed to have either exclusive or system hold preferred. If exclusive hold is preferred, depress HOLD button once for exclusive hold and twice for system hold when using a key telephone. If system hold is preferred, depress HOLD button once for system hold and twice for exclusive hold.

Default:	System Hold

Procedure:

1.	Press FLASH button and dial 11.

2.	On a display phone, you will see this display:

HOLD PREFERENCE SYS-EXC SYSTEM

3.	To make a change, press the top left program button.

LED off:	Exclusive Hold

LED on:	System Hold

4.	Press HOLD button.

AUTOMATIC PRIVACY  (FLASH 12)

Through the programming, privacy is automatically provided on all calls. If one station is using a CO line or intercom, another station cannot intrude on the call. No one can interrupt or overhear your calls.

Parameters:

If desired, the system can be programmed to eliminate CO line privacy, allowing a maximum of one other station to join in on existing CO line conversations. If privacy is disabled and a station will hear an alert tone.

Default:	Automatic Privacy

Procedure:

1.	Press FLASH button and dial 12.

2.	On a display phone, you will see this display:

PRIVACY WARNING YES NO

3.	To make a change, press the first two program buttons.

LED off:	Automatic privacy disabled, privacy warning tone disabled.

LED on:	Automatic privacy enabled, privacy warning tone enabled.

4.	Press HOLD button.

ATTENDANT CALL CODE (0)  (FLASH 13) (GHX-308A)

This field allows to call Attendant by dialing "0".

Parameters:

Determines whether Attendant Call code is "0" or not.

Range:	Yes/No

Default:	Yes

Procedure:

1. 	Press FLASH button and dial 13.

2. 	On a display phone, you will see this display:

ATTD CALL ENABLE YES

3. 	To make change, press the top left program button.

LED off:	No

LED on:	Yes

4. 	Press HOLD button.

INCOMING CO TOLL RESTRICTION  (FLASH 14)

This field allows to govern station’s toll restriction for incoming CO line. Station’s dialing privilege depends on it’s COS.

Parameters:

Determines whether the system restricts station’s toll thru incoming CO line or not.

Range : Yes/No

Default : No

Procedure :

1. 	Press FLASH button.

2. 	On a display phone, you will see this display;

INCOMING CO TOLL YES/NO NO

3. 	To make change, press the top left program button.

LED Off : No

LED On : Yes

4. 	Press HOLD button.

ATTENDANT STATION ASSIGNMENT  (FLASH 15)

The attendant will receive unattended recalls, i.e., transfers and holds after the initial recall attempt has been made. The attendant places the system into the night service by pressing DND button.

Parameters:

The system identifies a station as the attendant for line recall and attendant features. There can be a maximum of one(1) attendant station.

Default:	Station 100

Procedure:

1. 	Press FLASH button and dial 15.

2. 	On a display phone, you will see this display:

ATTN STA ASSIGNMENT 100

3.	Enter the 3-digit station number on the numeric keypad.

4.	Press HOLD button.

Conditions:

1. 	The attendant is by default assigned to station 100. When assigned to any other station, the

attendant cannot be the programming entry station.

2.	Recalls to the attendant will continue to ring until answered or disconnected, or the attendant recall

timer expires.

LOUD BELL CONTROL  (FLASH 16)

Intercom calls, CO line transfers and CO ringing will be presented to an externally connected loud ringing device via contact closure.

Two sets of control are provided to allow external ringing. Both contacts may be assigned to stations and the contacts will operate when station is ringing.

Parameters:

Two stations can be assigned to Loud Bell Control. The LBC contacts on MDF inside the KSU will allow the ringing assignments of those stations.

Default:	None

Procedure:

1. 	Press FLASH and dial 16.

2. 	On a display phone, you will see this display:

LOUD BELL CONTROL ASSGN ### , ###

3. 	Enter the 6-digit station number.

4. 	Press HOLD button.

Condition

- In GHX-616, the second relay is available when the MFU is installed.

- To use relay contact for LBC in GHX-616, the MFU should be installed.

PBX ACCESS CODES  (FLASH 17)

The system will allow five 1-or 2-digit PBX access codes to be entered into memory. When one of these codes is dialed, this signals the KSU that toll restriction is to be applied at the next dialed digits after the code. If one of these codes is not dialed, toll restriction is not applied.

Parameters:

Five 1-or 2-digit PBX access codes can be programmed into memory. When a single-digit access code(9) is required, the pound(#) key must be entered as the second digit (9#).

Default:	None

Procedure:

1.	Press FLASH button and dial 17.

2. 	On a display phone, you will see this display:

PBX DIAL CODES ## ## ## ## ##

3. 	Enter 2-digit code number one right after the other up to maximum of 10 digits on the numeric

keypad. (To delete an entry, enter two pound(# #) signs.)

4. 	Press HOLD button.

EXECUTIVE/SECRETARY TRANSFER  (FLASH 18)

There are four sets of Executive/Secretary pairs available for assignment so that when the executive designated telephone is busy or in DND, intercom calls and transfers will be automatically routed to the designated secretary station.

Parameters:

There are four pairs available with numerous combinations, i.e., 1 executive and 4 secretaries or 4 executives and 1 secretary. You cannot pair EXECUTIVE to SECRETARY(100-101) and then pair SECRETARY to EXECUTIVE(100-101). You can have the same secretary station for more than one executive station(101-100, 102-100).

Range:	Program button 1-4 (pair 1-4)

Default:	None

Procedure:

1.	Press FLASH button and dial 18.

2.	On a display phone, you will see this display:

EXEC SECY PAIRING ### / ### PAIR 1

3.	The top left program button will be lighted indicating the first pair may be entered.

Enter 3-digit executive station number.

Enter 3-digit secretary station number.

4.	Press HOLD button.

5.	To enter a second/third/fourth pair, press corresponding program button (Program button 2-4).

Repeat procedure 3 and 4. (An entry of pounds (#) will remove the assignments)

HUNT GROUP  (FLASH 19)

The system can be arranged for up to eight(8) hunt groups. Each hunt group can contain up to eight stations each. The hunt group uses a pilot hunting technique. When an intercom or transferred CO line is directed to a pilot hunt group, the system will search sequentially for an idle station. Upon finding an idle station within the group, it will ring at the idle station.

Parameters:

Can arrange up to eight(8) hunt groups of no more than eight(8) station each.

| Button No. | Group No. | Pilot Number | Default Stations (GHX-308A) | Default Stations (GHX-616) |
| --- | --- | --- | --- | --- |
| 1 | 1 | 710 | Station 100-103 | Station 100-103 |
| 2 | 2 | 711 | Station 104-107 | Station 104-107 |
| 3 | 3 | 712 | ... | Station 108-111 |
| 4 | 4 | 713 | ... | Station 112-115 |
| 5 | 5 | 714 | ... | ... |
| 6 | 6 | 715 | ... | ... |
| 7 | 7 | 716 | ... | ... |
| 8 | 8 | 717 | ... | ... |

Procedure:

1.	Press FLASH button and dial 19.

2.	On a display phone, you will see this display:

HUNT GROUP 710 100,101 102,103,___,___,___,___

The top left program button will be lighted for programming the first hunt group.

3. 	Enter the 3-digit station number up to a maximum of 24 digits (8 stations). The station numbers are

entered in order of the hunting desires.

4. 	Press HOLD button.

5. 	To enter further hunt group, press program buttons as follows;

| 710 |  | 711 |  | 712 |  | 713 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 714 |  | 715 |  | 716 |  | 717 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

6. 	Enter station numbers.

7. 	Press HOLD button.

SMDR (FLASH 20)  (GHX-616)

SMDR(Station Message Detail Recording) will provide details on both incoming and outgoing calls. When a printing device is connected to the system RS-232C port, detailed call information provided by the system will be printed according to system option selection.

Parameters:

The data fields are assigned to 5 program buttons in the program PAGE A as follows. These buttons are toggled to set desired data.

SMDR

This button allows the SMDR to be turned on or off in the system. When set to ON the SMDR data will be presented to the RS-232C port for printing.

LD/ALL CALL

Allows selection for the recording of all outgoing calls or just long distance calls. Long distance calls are classified as beginning with a 0 or 1, or exceeding 7 digits.

PRINTER

can be programmed to print in either 80-character format or 29-character format.

BAUD RATE

is to select Baud Rate of output data to the RS-232C port. The rate can be 300/1200/2400/4800/9600 Baud.

To select baud rate, press button 4 and dial 1 digit of baud rate type (0-4).

(0: 9600, 1:4800, 2:2400, 3:1200 , 4:300 Baud)

PRINT INCOM. CALL

allows to print incoming calls. When selected ON, both outgoing calls and incoming calls may be printed.

SAVE

allows to save SMDR data to extended memory (32Kbyte).

The features of the program of the program PAGE B are described as follows;

CURR UNIT

For easy identification of call cost, the currency unit can be input with 3 alphabet characters to be printed in front of call charge amount.

Default is USD. To enter 3 alphabet characters, dial 6 digits according to the manner of name

programming.

COST UNIT

It determines the cost per unit. It can be input with 4 digits. Default is 0025.

FRACTION

The 5 kinds of formula are available to calculate call charge:

For printout of 4-digit fraction part, [(No. of metering pulse) x (cost per unit)] / 10000.

For printout of 2-digit fraction part, [(No. of metering pulse) x (cost per unit)] / 100.

Ex) No. of metering pulse = 2

Cost per pulse      = 000156

1. 	In case of 4 fractional parts:      [2 x 156] / 10000 = 0.0312.

2. 	In case of 2 fractional parts:      [2 x 156] / 100 = 3.12.

Only 12/16K call metering type is available to calculate call charge:

Range & Default:

| Button | Field | Range | Default |
| --- | --- | --- | --- |
| A1.2 | SMDR | Yes/No | No |
| A2 | TYPE | All Call/Long Distance Call (On/Off) | Long Distance Call |
| A3 | PRINTER | 80/29 Columns (On/Off) | 80 |
| A4 | BAUD RATE | 300/1200/2400/4800/9600(4/3/2/1/0) | 1200 |
| A5 | PRINT INCOMING | Yes/No | No |
| A6 | CALL SAVE CALLS | Yes/No | No |
| B1 | CURR UNIT | 3 Characters | USD |
| B2 | COST UNIT | 4 Digits | 0025 |
| B3 | FRACTION | 0-4 | 0 |
| B4 | Not Used |  |  |

Procedure:

1.	Press FLASH button and dial 20.

2.	During selection button 1 to 5, you will see this display on display phone.

SDR TPE PNT BDR PIC SAV NO LD 80 1200 NO NO

| SDR |  | TYPE |  | PRINTER |  | BAUD RATE |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PIC |  | SAVE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PAGE A |  | PAGE B |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

LED off: SMDR disabled, Long Distance Call, 29 columns, Don't print incoming calls, Don't save.

LED on: SMDR enabled, All Call, 80 columns, Print Incoming call, Save.

To change Baud Rate, press Baud Rate button and dial 1 digit of Baud type. When the features of Page A have been selected, press HOLD button. The display will now update to the new data.

4.	The remaining features are located and programmed on Page B. Press PAGE B select button. The display will appear as follows;

CURR COST FRAC DOL 0025 0

5.	To program PAGE B features, use the button as follows:

| CURR UNIT |  | COST UNIT |  | FRACTION |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PAGE A |  | PAGE B |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

Standard Format:

The "Standard Format" output constitutes a maximum 70-character ASCII stream, illustrated below:

STA CO  TOTAL  START DATE       DIALED DIGITS   ACCOUNT CODE

AAA BB D:DD:DD EE:EE FF/FF/FF H CCCCCCCCCCCCCCC GGGGGGGG    (CR)(CR)(LF)

or,

STA CO  TOTAL  START DATE      DIALED DIGITS     COUNT   COST

AAA BB D:DD:DD EE:EE FF/FF/FF H CCCCCCCCCCCCCCC  IIIII     JJJ..JJJ  (CR)(CR)(LF)

29 Character Printer Format:

The 29-character format constitutes a maximum 76-character ASCII stream illustrated below.

STA CO  TOTAL  START DATE

AAA BB D:DD:DD EE:EE FF/FF/FF (CR) (CR) (LF)

H CCC...CCC (CR)(CR)(LF)

GGGGGGGG (CR)(CR)(LF)

or,

STA CO  TOTAL  START DATE

AAA BB D:DD:DD EE:EE FF/FF/FF (CR) (CR) (LF)

H CCC...CCC (CR)(CR)(LF)

IIIII JJJ...JJJ (CR)(CR)(LF)

Where:

AAA = A 3-digit station call originator field

BB = A 2-digit CO line accessed field

D:DD:DD = A 7-digit call duration field (H:MM:SS)

EE:EE = A 5-digit time of day call origination (HH : MM)

CCC...CCC = A 24-digit dial digit field

FF/FF/FF = An 8-digit month, day, and year of call origination (MM/DD/YY)

GG = An 8-digit account code field

H = An 1 character call type indicator

(I = incoming, O = outgoing, T = transferred)

IIIII = A 5-digit metering pulse count

JJJ...JJJ = A 10-digit calculated cost

(CR) = Carriage Return

(LF) = Line Feed

Printout Format on Station Basis by Attendant:

80 column format

STA  CO TOTAL  START    DATE     DIALED    ACNT CODE  CNT    COST

AAA BB D:DD:DD  EE:EE  FF/FF/FF H  1234567                   C     DDDD

AAA BB D:DD:DD  EE:EE  FF/FF/FF H  1234567                   C     DDDD

AAA BB D:DD:DD  EE:EE  FF/FF/FF H  1234567                   C     DDDD

-----------------------------------------------------------------------------------------------------------------

TOTAL CALL      TOTAL COUNT      TOTAL COST

BBBB              CCCC               DDDD

29 column format

TCALL   TCOUNT   TCOST

BBBBB   CCCCC    DDDDDDDD.DD

Where:

BB  = A 5-digit total call number

CC  = A 5-digit total metering count number

DD  = A 10-digit total cost

4.	For SMDR, a header will be printed, the header is enabled at all times. The header will be preceded by two blank lines and will be printed every 62nd line. There will be no form feeds. On the 29 character printout the dial digit field and account code field will have no header. The header has fixed field as shown in the example. The protocol for the out-put is a 8 bit data, 2 stop bits, and no parity.

5.	If the SMDR feature is enable, information acquisition begins at call start time and terminates at call completion. At this point, the acquired information is queued for output format processing.

6.	The account code feature will be accessed by key stations via the DND/FOR button or pre-programmed account code enter button on a key telephone. The "*" asterisk key should close the account code.

Operation:

1.	Select a CO line via appearance button or direct group access button.

2.	Press DND/FOR button or preprogrammed Account Code Enter button.

3.	Dial account code (MAX 8 Digits) and *.

4.	Dial the desired telephone number.

PBX CALLING SIGNAL (FLASH 21) (GHX-308A)

This field allows to select the ring type of extensions called by PBX extension.

Parameters:

Determines whether the system provides single or continuous ring to the called extensions.

Range:	Once/Repeat

Default: 	Once

Procedure:

1. 	Press FLASH button and dial 21.

2. 	On a display phone, you will see this display:

SIGNAL ONCE

3. 	To make change, press the top left program button.

LED off:	Once

LED on:	Repeat

4. 	Press HOLD button.

ALARM DETECTION & SIGNALING (FLASH 21) (GHX-616)

External alarm devices can be connected whereby an alarm signal may be received and upon receipt the system will generate an alarm to station programmed to transmit the signal.

Parameters:

The external alarm device signals the system of an alarm condition via an open or closed loop condition. Upon receipt of the alarm condition, the system will signal selected station with a continuous or single tone.

Range & Default:

| Field | Range | Default |
| --- | --- | --- |
| Alarm Signaling Detection | Yes/No Repeated/Once Close/Open | No Once Closed |

Procedure:

1.	Press FLASH button and dial 21.

2.	On a display phone, you will see this display:

ALARM SIGNAL DETECT NO ONCE CLOSED

3.	To program alarm feature, use the program buttons as follows:

| ALARM |  | SIGNAL |  | DETECT |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

LED off: Alarm disabled, Single tone, Open loop.

LED on: Alarm enabled, Repeated tone, Closed loop.

4.	Press HOLD button.

PAGE WARNING TONE  (FLASH 22)

This field allows to select page warning tone.

Parameters:

Determines whether the system provides page warning tone after user dials page code or not.

Range:	Yes/No

Default:	Yes

Procedure:

1. 	Press FLASH button and dial 22.

2. 	On a display phone, you will see this display:

PAGE WARNING TONE YES/NO YES

3. 	To make change, press the top left program button.

LED off:	No

LED on:	Yes

4. 	Press HOLD button.

PROGRAM CONTROL RELAY (FLASH 23)

This field allows to select the usage of control relays. The relays can be used Loud Bell, Door Open, External Page or External MOH.

Parameters:

Determines the control relays is for Loud Bell, Door Open, External Page or External MOH.

Range:	Loud Bell / Door Open /External Page /External MOH

Default:	Loud Bell

Procedure:

1. 	Press FLASH button and dial 23.

2. 	On a display phone, you will see this display:

PROGRAM CONTROL RELAY

3. 	To make change, press the button 1 (for relay 1) or button 2 (for relay 2).

01 RELAY: LOUD BELL 1LBC 2DOOR 3EPAGE 4EMOH

Dial 1 digit (1-4) for your desired control.

4.  	Press HOLD button.

PHONE BOX CALL TIMER   (FLASH 25)

A convenient phone box for receiving announcements, intercom calls, and signaling can be connected to the system. Any condition of key telephones or phone boxes can be arranged in the system. The positions for key stations are completely interchangeable with phone boxes. (Station ID programming as phone box is required.)

Parameters:

Determines the amount of the ringing. The assigned stations (Program Code: Flash 40) will receive ringing when a phone box user presses the CALL button.

Range:    00-60 seconds

Default:    20 seconds

Procedure:

1.   	Press FLASH button and dial 25.

2.   	On a display phone, you will see this display:

ICM BOX TIMER 00-60 20

3.   	Enter 2 digits on the numeric keypad.

4.   	Press HOLD button.

RECALL TO ALL STATION (FLASH 27) (GHX-616)

This allows to be received recall rings for all stations.

Parameters:

Determine whether all stations are received recall rings or not.

Range : 	Yes/No

Default :	No

Procedure:

1.	Press FLASH and dial 27.

2.	On a display phone, you will see this display:

RECALL TO ALL STA YES/NO NO

3.	To make a change, press the top left program button.

LED off :	Disables monitor mode

LED on :	Enables monitor mode

4.	Press HOLD button.

SETTING TIME AND DATE  (FLASH 28)

This allows the user to program the system date and time.

Parameters:

Range & Default:

| Field | Range | Default |
| --- | --- | --- |
| Year (YY) Month (MM) Day (DD) Hour (HH) Minute (mm) Second (SS) | 00-99 01-12 01-31 00-23 00-59 00-59 | 99 01 01 12 01 00 |

Procedure:

1.	 Press FLASH and dial 28.

2.  	On a display phone, you will see this display:

DATE & TIME MM DD YY HH:mm:am

3.	Enter time and date in form of YYMMDDHHmmSS on the numeric keypad.

4.	Press HOLD button.

GROUP LISTENING  (FLASH 29)

A station user engaged in a conversation with the handset can allow people around to listen to the conversation by pressing MON button.

Parameters:

Determine if this feature is used.

Range:	Yes/No

Default:	No

Procedure:

1.	Press FLASH button and dial 29.

2.	On a display phone, you will see this display:

MONITOR MODE YES/NO NO

3.	To make a change, press the top left program button.

LED off:	Disables monitor mode

LED on:	Enables monitor mode

4.	Press HOLD button

7.6  CO LINE PROGRAMMING  (FLASH 30)

If you are in the program mode, continue using the program codes. If you are starting to program at this point, enter the program mode at first.

This allows the features and services of the CO lines to be entered on each CO line in the system individually.

Parameters:

The data fields are assigned to 8 program buttons as follows;

| Button | Meaning | Range | Default |
| --- | --- | --- | --- |
| 1A | DTMF/PULSE | DTMF/PULSE (On/Off) | DTMF |
| 2A | UNA | Yes/No (On/Off) | Yes |
| 3A | TOLL OVERRIDE | Yes/No (On/Off) | No |
| 4A | GROUND / LOOP | Ground/Loop (On/Off) | Loop |
| 5B | PULSE RATIO | 60:40 / 66:33 (On/Off) | 66:33 |
| 6A | PULSE SPEED | 10 pps / 20 pps (On/Off) | 10 pps |
| 7A | CO/PBX | 0(CO) /1(PBX) | CO |
| 8A | Not used |  |  |
| 1B | FLASH TIMER | 01-20 (x100) msec | 10 (x100) msec |
| 2B | CO LINE GROUP | 0-3 (GHX-308A) 0-7 (GHX-616) | 1 1 |
| 3B | Not used |  |  |
| 4B | Not used (GHX-308A) DISA MODE (GHX-616) | No, 24Hour, Nite DISA | No DISA |

PAGE A

DTMF/PULSE

Each individual CO line can be programmed to be either DTMF(tone) or dial pulse. By default all lines are set for DTMF.

UNA

Incoming CO/PBX lines marked as UNA can be answered by dialing the UNA access code by stations not normally assigned access to the line(s). The stations must have a direct line button or loop key assigned to answer.

Default is Yes (UNA).

TOLL OVR

A CO line may be marked for Toll Override which allows to dial long distance call on this line for toll restricted stations.

Default is No.

GROUND/LOOP

The flash type of each CO line can be programmed to be either Loop flash or Ground flash.

Default is Loop flash.

PULSE RATIO

If out-pulsing is required, the individual CO line must be programmed for dial pulse in program button 1. The Make/Break ratio and the dial speed can be programmed for either 66/33 or 60/40.

Default is set to 66/33.

PULSE SPEED

Dialing speed is set to 10 pps (Pulses per second) and is able to change to 20 pps.

CO/PBX

CO line can be marked either CO or behind PBX. When marked PBX, toll restriction applies after the PBX dial codes.

0=CO line, 1=PBX line.

By default, all lines are CO.

PAGE B

FLASH TIMER

When using an outside, flash allows a user

1) 	to obtain new dial tone without losing the line if the line is CO or

2) 	to transfer the line to another extension if the line is behind PBX.

Default is set to 1 second and is variable from 01-20 (x100 msec).

CO LINE GRP

Three(3)/seven(7) CO line groups in GHX-308A/616 are available for CO assignment. Groups should be assigned according to Trunk Type. All lines are assigned to line group "1" by default.

DISA MODE

This feature is not supported in GHX-308A.

The DISA type of each CO line can be programmed to be no DISA, 24-hour DISA or night DISA.

Enter 1 digit to select type of DISA.

0 = No DISA

1 = 24 Hour DISA

2 = Day DISA

3 = Night DISA

Procedure:

1. 	Press FLASH button and dial 30.

2. 	On a display phone, You'll see this display:

CO LINE ATTRIBUTE SELECT A CO LINE RANGE

The program button 12 will be lighted steady so that you can input the range of CO line to be

programmed.

3. 	From the dial pad, enter 4 digits for the CO line range to be programmed. (For an example, if you

want to program CO 1 to CO 3 enter 0103. If you want to program CO 3, only enter 0303.)

4. 	When the CO line(s) is accessed the display will appear as:

The alternatives for the display PAGE A are:

CO x-y DTMF UNA OVN GN R66 SP1 CO

CO XX-YY (X, Y=01-03),

DTMF(DP),

UNA,

OV# (#: Y=Yes, N=No),

G# (#: Y=Ground, N=Loop),

RXX (XX: 66=66/33, 60=60/40),

SPX (X: 1=10pps, 2=20pps),

CO(PBX),

The alternatives for the display PAGE B are:

GHX-308A

CO xx-yy F10 GRP1

CO xx-yy (xx, y=01-03),

Fxx (xx=01-20),

GRPx (x=0-3),

GHX-616

CO xx-yy F10 GRP1 D0

CO xx-yy (xx, yy=01-06),

Fxx (xx=01-20),

GRPx (x=0-7),

Dx (x=0-3)

5. 	To program PAGE A features, use the button as follows:

| DTMF PULSE |  | UNA |  | TOLL PVERRIDE |  | GROUND LOOP |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PULSE RATIO |  | PULSE SPEED |  | CO PBX |  | Not used |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PAGE A |  | PAGE B |  | NEXT FORWARD |  | NEXT BACKWARD |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

LED off : Dial pulse, Disable UNA, Disable Toll Override, Loop Flash, Pulse Ratio (66/33),

Pulse speed (20 pps).

LED on : DTMF, UNA, Toll override, Ground Flash, Pulse Ratio (60/40), Pulse Speed (10 pps)

- When the attributes are being programmed, button 1 through 6 can be toggled On or Off. Buttons 7

through 8 require numeric entries from the dial pad. The toggled buttons can all be set and then

entered as a block with HOLD button. Each button with a numeric entry requires a pressing of

HOLD button to store the data. Each time pressing HOLD button, the display will refresh with the

new data.

7.	To program PAGE B features, use the button as follows:

| FLASH TIME |  | LINE GROUP |  |  |  | DISA (GHX-616 only) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PAGE A |  | PAGE B |  | NEXT FORWARD |  | NEXT BACKWARD |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

8.  	Button 1 and 2 require numeric entries from the dial pad. The toggled buttons can all be set

and then entered as a block with HOLD button. Each button with a numeric entry requires a

pressing of HOLD button to store the data. Each time pressing HOLD button, the display will refresh

with the new data.

9.	Pressing button 11(Next Forward) steps to the next CO line higher than the upper limit of

the present CO range. Pressing button 12(Next Backward) steps to the lower CO range than the

present range. Subsequent pressing of button 11 and 12 will continue to step to the next CO range.

HOOK FLASH TIMER  (FLASH 31)

When single line telephones are connected to the system, this time determines how long the user should depress the hook switch in order for it to be considered a Flash.

Parameters:

Determines the time range to be considered as hook flash signal. The time variable from 100 msec to 2 seconds.

Range:    	01-20 (x100 msec)

Default:   	05 (x100 msec)

Procedure:

1.  	Press FLASH button dial 31.

2.  	On a display phone, you will see this display:

HOOK FLASH TIME 01-20 05

3.	Enter 2 digits on the numeric keypad.

4.	Press HOLD button.

ATTENDANT RECALL TIMER  (FLASH 33)

A CO line placed on hold, transferred or parked will initiate the recall timer if the associated timer has been enabled in programming. When the timer expires, the CO line will ring the station that placed it on hold. If not answered, the associated timer continues and upon expiration again the attendant station will ring.

Parameters:

Determines the amount of time the attendant will receive the recall.

Range:	000-255 seconds

Default:	30 seconds

Procedure:

1.	Press FLASH button and dial 33.

2.	On a display phone, you will see this display:

ATTENDANT RECALL 000-255 030

3.	Enter 3 digits on the numeric keypad.

4.	Press HOLD button.

INCOMING CO RING ASSIGNMENT TO HUNT GROUP (FLASH 34) (GHX-616)

Hunt Group is assigned to provide ring for incoming CO line calls. The assignment permits Hunt Group to provide ring signal while the system is in day mode, night mode or in either mode. This feature is not supported in GHX-308A.

Procedure:

1.	Press FLASH button and dial 34.

On a display phone, you will see this display:

CO RING ASSIGN TO HUNT PRESS BTN 1-8(GRP:1-8)

2.	Press program button 1-8 to select Hunt Group.

CO RING ASSIGN TO HUNT SELECT A CO LINE RANGE

3.	Dial 4 digits of CO line (01-06) range to assign CO lines to the Hunt Group. A single CO line may be programmed by entering the CO line's number twice.

CO XX-YY SEL DAY/NIGHT(DIAL:0-3)

XX-YY is CO line range assigned to the Hunt Group.

4.	From the dial pad, enter the type code.

Type Code 		Ring Type

0		Unassigned

1		Day

2  		Night

3		Both Day & Night

5. 	Press HOLD button.

RING TYPE SELECTION  (FLASH 35)

This field allows to select the type of rings (intercom and CO rings) for stations.

Parameters:

Determines the ring types.

|  | LED OFF (DEFAULT) |  | LED ON |  |
| --- | --- | --- | --- | --- |
|  | CO RING | INTERCOM RING | CO RING | INTERCOM RING |
| KEY STATION | 0.4/0.4/0.4/2 | 0.8/2.4 | 0.8/2.4 | 0.4/0.4/0.4/2 |
| SLT | 0.4/0.2/0.4/4 | 1/4 | 1/4 | 0.4/0.2/0.4/4 |

Default:	LED OFF

Procedure:

1.	Press FLASH button and dial 35.

2.	On a display phone, you will see this display:

SELECT RING TYPE CO .4/.2/.4/4 INT 1/4

3.	To make change, press button 1.

4.	Press HOLD button.

FLEXIBLE STATION NUMBERING  (FLASH 36)

This field allows to change intercom number in the range from 100 to 699.

Parameters:

Determines intercom number.

Range:	100-699

Default:	100-107 (GHX-308A)

100-115 (GHX-616)

Procedure:

- Press FLASH button and dial 36.

- button 1 : select port 01-04 (default: 100-103)

01 02 03 04 100 101 102 103

- Press button 1-2 (1-4 in GHX-616) .

| Program Button | Ports | Default Number |
| --- | --- | --- |
| Flex. Button 1 | Port 01-04 | 100-103 |
| Flex. Button 2 | Port 05-08 | 104-107 |
| Flex. Button 3 (GHX-616) | Port 09-12 | 108-111 |
| Flex. Button 4 (GHX-616) | Port 13-16 | 112-115 |

3.	Dial 12 digits station numbers.

4.	Press HOLD button.

METHOD OF SEIZING CO LINE  (FLASH 37)

When you dial CO line access code, there are two ways of selecting CO line.

Last CO choice

You can select last CO line available in that CO group.

Circle CO choice

You can select sequentially CO line in that CO group whenever you access CO group.

Parameters:

Determines intercom number.

Range:	Last/Circle

Default:	Circle

Procedure:

1. 	Press FLASH button and dial 37.

2. 	Press button 1.

LED ON  :  Circle Choice

LED OFF :   Last Choice

3. 	Press HOLD button.

AUTO-REDIAL TIMER  (FLASH 38) (GHX-616)

Upon activation by station user, the system automatically makes attempts to redial a connection to a busy number or no answered call on outgoing line.

Parameters:

Determines the time interval between attempts.

Range:     030-255 seconds

Default:     060 seconds

Procedure:

1.	Press FLASH button dial 38.

2.	On a display phone, you will see this display:

AUTO REDIAL 030-255 060

3.	Enter 3 digits on the numeric keypad.

4.	Press HOLD button.

MULTI-LINE CONFERENCE  (FLASH 39)

The system will allow conference with two(2) CO lines and one(1) internal party. The internal party can exit the conference and leave the conference unsupervised.

Parameters:

Determines to set the system parameter to allow multi-line conference.

Range:	Yes/No

Default:	No

Procedure:

1.	Press FLASH button and dial 39.

2.	On a display phone, you will see this display:

MULTI LINE CONF YES/NO NO

3.	To make a change, press the top left program button.

LED off:	No

LED on:	Yes

4.	Press HOLD button.

7.7  STATION PROGRAMMING - (I)  (FLASH 40)

If you are in the program mode, use the program code continuously. If you are starting to program at this point, enter the program mode at first.

It allows the features and services of the station to be entered on each station in the system individually.

Parameters:

PAGE A

PAGE

Station can individually be allowed or denied the ability to make pages. It disallows paging access by default.

DND

Stations can be individually allowed or denied the ability to place their telephones in Do Not Disturb. It disallows DND by default.

SYS SP DIAL

Stations can be individually allowed or denied the ability to use system speed dial numbers. It allows system speed by default.

HEADSET

Stations can be allowed or denied to use industrial standard headset for convenient operation. It is not allowed headset operation by default.

PLA

Stations can be given the ability to answer incoming CO line calls by simply going off-hook (ringing line preference). Allowed by default.

3 MIN WARN

Stations can be allowed or denied to receive alert tone when a CO line is occupied for 3 minutes. The alert tone is heard every 3 minute. Disallowed by default.

CALL FWD

Stations can be disallowed or denied the ability to have intercom and CO lines forwarded to another station. It allows Call Forward by default.

ALARM SIGNAL

Stations can be designated to receive phone box call through the telephone speaker. It gives no stations the feature by default.

PAGE B

STA ID

Each station must be identified as one of the followings;

| ID | Description (GHX-308A) | Description (GHX-616) |
| --- | --- | --- |
| 1 | 21, 33 Button key station (default) | 21, 33 Button key station (default) |
| 2 | Phone Box | Phone Box |
| 3xxx | Not used | DSS/BLF Box (xxx = associated station number) applicable |
| 4 | Dial Pulse SLT | Dial Pulse SLT |
| 5 | DTMF SLT | DTMF SLT |
| 6 | 8 Button station | 8 Button station |

COS

Each station must be assigned a class of service which governs that station's toll restriction. The 6 classes are;

Two digits are entered via dial keypad. The first digit is for day COS and the second for night COS. Default gives all stations COS 11.

| COS | Description |
| --- | --- |
| 1 | No restriction |
| 2 | Governed by Allow/Deny Table A |
| 3 | Governed by Allow/Deny Table B |
| 4 | GHX-308A : no '0' as the first digit GHX-616 : no '0', '1' as the first digit, 7 digits maximum |
| 5 | intercom calls and paging only |
| 6 | receive calls and paging only |

GRP PICK-UP

Each station is assigned into pickup groups. Stations can be in any combination of the 8 groups or in no group at all.

| Group No. | Access allowed to Group |
| --- | --- |
| 0 | No group |
| 1 | Group 1 (Default) |
| 2 | Group 2 |
| 3 | Group 3 |
| 4 | Group 4 |
| 5 | Group 5 |
| 6 | Group 6 |
| 7 | Group 7 (Branch Group) |
| 8 | Group 8 (Branch Group) |

Note:	The group 7 and 8 are used for automatic group pick-up feature, i.e. by simply going off-hook, he can pick the call in the group.

PAGE ZONE

Page Zone program is not allowed in GHX-308A.

Each station can be in any or all zones or in no zone at all.

0 = no zone (will receive no pages)

1 = zone 1

2 = zone 2

12 = zone 1 and 2 (LCD shows '3')

Default assigns all to zone 1.

Internal All Call is defined as the sum of all zones. If a station is not in any internal zone, it will not receive any all call pages either.

PRESET FWD

A station can be programmed so that incoming ringing CO lines can be forwarded to a preset station if the station does not answer after a programmable period of time (Program Code: Flash 04). It assigns no stations by default.

CO GRP ACCESS

A station is allowed to access to any combination of CO line groups. Or a station may not be allowed any access to CO line groups. The followings are the group numbers and their access buttons:

| Data | Access allowed CO line groups (GHX-308A) | Access allowed CO line groups (GHX-616) |
| --- | --- | --- |
| 1 | GROUP 1 | GROUP 1 |
| 2 | GROUP 2 | GROUP 2 |
| 3 | GROUP 3 | GROUP 3 |
| 4 | TO ALL GROUPS | GROUP 4 |
| 5 | NONE | GROUP 5 |
| 6 | No meaning | GROUP 6 |
| 7 | No meaning | GROUP 7 |
| 8 | No meaning | NONE |

CO lines are accessed by the station user dialing the appropriate access code number. Dial tone will be received and they can dial the desired telephone number. A station user can also program one of the flexible buttons as CO Group Access Code and then can access that CO line by simply pressing the button.

When using a code "9" to access a CO line, an idle CO line in the first access allowed CO line group is seized. By default, all stations will be allowed access to all CO line groups.

To assign the CO line access map, press program button 7 on PAGE B. If you have a display phone, you will see this display:

CO GRP ACCESS MAP ENTER CO LINE DATA

Then, the user can enter 1 to 3 (7 in GHX-616) to assign the CO line group 1 to 3 (7 in GHX-616) that can be accessed by this station. An entry of 4 (8 in GHX-616) will make the station to be able to access all CO

line groups and an entry of program button 5 (9 in GHX-616) will make the station not to be allowed to access any CO line groups in the system. Press HOLD button to enter the data.

CO BTN ASSIGN

To assign which buttons are to be CO lines, press CO line button on page B (Button 8). If you have a display telephone, you will see this display:

CO LINE TO BUTTON PROG ENTER BUTTON DATA

Each entry is 5 digits in length and is entered on the dial pad. Press HOLD button after each 5 digit entry.

Button assignments are made in the following format:

BBLLR

BB  =	flexible button number (01-24)

LL  	=	line assigned to that button (01-03 in GHX-308A, 01-06 in GHX-616), 00 for Loop Button

R   	=	ringing status

0 = no ring (display shows '0')

1 = day ring ('1')

2 = night ring ('2')

3 = day and night ring ('3')

By default, the button assignment is square, which means that each button is assigned to the

corresponding line number (button 1 is line 1, button 2 is line 2, etc.). The remaining buttons on the 33-button are initialized by default. To see what assignments were made previously, press "DSP" button on PAGE B. Each depression of this button will display 4-button assignments (up to a maximum of 24 buttons)

BUTTONS XXX-XXX BBLLR BBLLR BBLLR BBLLR

If a button is not to be assigned as a CO line, the entry should be BB.(BB = button number) Any button not assigned as a CO line or as a loop button can be programmed by each station user for one of several functions. Instructions for this programming are in the station user's guide.

Range & Default (GHX-308A) :

| Button | Field | Range | Default |
| --- | --- | --- | --- |
| 1A | PAGE | Yes/No (On/Off) | No (Off) |
| 2A | DND | Yes/No (On/Off) | No (Off) |
| 3A | SYSTEM SPEED DIAL | Yes/No (On/Off) | Yes (On) |
| 4A | HEADSET | Yes/No (On/Off) | No (Off) |
| 5A | PREFERRED LINE ANSWER | Yes/No (On/Off) | Yes (On) |
| 6A | 3 MINUTE WARNING | Yes/No (On/Off) | No (Off) |
| 7A | CALL FORWARD | Yes/No (On/Off) | No (Off) |
| 8A | ALARM SIGNAL | Yes/No (On/Off) | No (Off) |
| 1B | STATION ID | 1-6 |  |
| 2B | COS | XY (X=1-6, Day. Y=1-6, Night) | 11 |
| 3B | Not Used |  |  |
| 4B | GROUP PICK-UP | 0-8 | 1 |
| 5B | Not Used |  |  |
| 6B | PRESET FORWARD | Station Number | None |
| 7B | CO GROUP ACCESS | 1-3 | All |
| 8B | CO LINE BUTTON | BBLLR | Square |

Range & Default (GHX-616):

| Button | Field | Range | Default |
| --- | --- | --- | --- |
| 1A | PAGE | Yes/No (On/Off) | Yes (On) |
| 2A | DND | Yes/No (On/Off) | Yes (On) |
| 3A | SYSTEM SPEED DIAL | Yes/No (On/Off) | Yes (On) |
| 4A | HEADSET | Yes/No (On/Off) | No (Off) |
| 5A | PREFERRED LINE ANSWER | Yes/No (On/Off) | Yes (On) |
| 6A | 3 MINUTE WARNING | Yes/No (On/Off) | No (Off) |
| 7A | CALL FORWARD | Yes/No (On/Off) | Yes (On) |
| 8A | ALARM SIGNAL | Yes/No (On/Off) | No (Off) |
| 1B | STATION ID | 1-6 |  |
| 2B | COS | XY (X=1-6, Day. Y=1-6, Night) | 11 |
| 3B | Not used |  |  |
| 4B | GROUP PICK-UP | 0-8 | 1 |
| 5B | PAGE ZONE | 0-2 | 1 |
| 6B | PRESET FORWARD | Station Number | None |
| 7B | CO GROUP ACCESS | Program Button (1-9) | All |
| 8B | CO LINE BUTTON | BBLLR | Square |

Procedure:

1.	Press FLASH button and dial 40.

2.	On a display phone, you'll see this display:

STATION ATTRIBUTES SELECT A STA RANGE

The program button 12 will be lighted steady so that you can input the range of station to be programmed.

- Enter a 6-digit number for station range being programmed. (For an example, if you want to

program station 100 to 105 with same attributes, enter 100105. If you want to program only one(1)

station, enter that number twice (i.e.101101).

4. 	The display will appear like this while programming Page A for that station or for group of station.

XXX-XXX A PAGE DND SPD HEAD PLA WARN FWD ALM

5.	To program PAGE A features, use the button as follows:

| PAGE |  | DND |  | SYS SPD |  | HEADSET |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PREF LINE |  | 3 MIN WARNING |  | CALL FWD |  | ALARM |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PAGE A |  | PAGE B |  | SELECT STA # |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

These buttons toggle on and off.

LED off:	Paging restricted, no DND, no system speed dial, no headset, no preferred line

answer, no 3 min. warning tone, no call forward, no alarm.

LED on:	Paging access, DND, system speed dial, headset, preferred line answer,

3 min warning tone, call forward, alarm receive.

When the features of Page A have been selected, press HOLD button. The display will be updated

to the new data.

6.	The remaining features of the station are located and programmed on Page B. Press PAGE

B SELECT button. This will bring up additional station features to be programmed. The display of

current programming for those features will appear as follows:

XXX-XXX B ID1 COS11 AAAAAAAA DDD EEE

XXX 	= station range

B   	= page B

ID  	= station ID

COS  	= class of service

A   	= pickup group

D   	= preset forward station

E   	= CO line group access

7.	To program Page B features, use the program buttons as follows:

| STA ID |  | COS |  | Not used |  | PICKUP |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PAGE ZONE (GHX-616) |  | PRESET FWD |  | CO ACCESS |  | CO LINE |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

| DSP |  | PAGE A |  | PAGE B |  | SELECT STA |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

These features except CO LINE ACCESS button (Button 7) require a numeric entry. Press the program button and then enter the number via the dial pad. Press HOLD button after each numeric entry. (Display will be now updated.)

OFF-NET FORWARD CO TO CO  (FLAHS 41) (GHX-616)

This field allows to forward incoming CO line to outside.

Parameters:

Determines whether the off-net forward CO calls to CO.

Range:	Yes/No

Default:	No (LED OFF)

Procedure:

1.	Press FLASH button and dial 41.

2.	On a display phone, you will see this display:

CO TO CO OFF-NET FWD Y/N NO

3.	To make change, press button 1.

4.	Press HOLD button.

CALL METERING UNIT  (FLASH 42) (GHX-616)

This field allows to select the SMDR printout format (CMU or NOT). To use call metering unit(CMU), it should be selected. If not selected, account code will be printed instead of call count and cost.

Parameters:

Determines whether the system select CMU or not.

Range:	Yes/No

Default:	No

Procedure:

1.	Press FLASH button and dial 42.

2.	On a display phone, you will see this display:

CALL METERING YES/NO NO

3.	To make change, press the top left program button.

LED off:	CMU Format disabled

LED on:	CMU Format enabled

4.	Press HOLD button.

PRESELECTED MESSAGE SELECTION  (FLASH 43) (GHX-616)

This field allows to select pre-selected message type in GHX-616.

Parameters:

Determines whether the system select new pre-selected message or not. (See 'Message Display' on Feature Description.)

Range:	Type I/Type II

Default:	Type II

Procedure:

1.	Press FLASH button and dial 43.

2.	On a display phone, you will see this display:

PRE-SEL MSG TYPE1/TYPE2 1

3.	To make change, press the top left program button.

LED off: Type II

LED on: Type I

4.    Press HOLD button.

CALL FORWARD NO ANSWER TIMER  (FLASH 44) (GHX-616)

If a station placed in no answer call forward does not answer the call within this time, the call is transferred to predetermined station.

Parameters:

Determines the amount of time a call will ring before being forward to predetermined station.

Range : 000-255

Default : 015 sec

Procedure:

1.	Press FLASH button and dial 44.

2.	You will see this display;

CALL FWD NO ANS 000-255 015

3.	Enter 3 digits on the numeric keypad.

4.	Press HOLD button.

PREVENT ILLEGAL DIAL  (FLASH 45)

This field allows to prevent illegal dial. When this field is set to “YES”, dial through CO is stored and forwarded.

Parameters:

Determines whether illegal dial is to be prevented or not.

Range:	Yes/No

Default:	No

Procedure:

1.  	Press FLASH button and dial 45.

2.  	On a display phone, you will see this display:

PREVENT ILLEGAL DIAL Y/N NO

3.  	To make a change, press the top left program button.

LED off: 	No

LED on: 	Yes

4.  	Press HOLD button.

CO DIAL WAITING TIMER  (FLASH 46)

When SLT user seizes a CO line, system assigns DTMF Receiver to the SLT for checking disallowed dial. The field determines the time to assign DTMF Receiver to SLT from seizing a CO line to the first digit. When this timer expires, system releases DTMF Receiver from SLT.

1.  	Press FLASH button and dial 46.

2.  	On a display phone, you will see this display:

CO DIAL WAIT TMR 000-255 030

3.  	Enter 3 digits waiting time.  (000-255)

4.  	Press HOLD button.

ATTENDANT CALL CODE(0) SELECTION  (FLASH 47) (GHX-616)

This field allows to determine Attendant Call code to be "0" or "9".

Parameters:

Determines whether Attendant Call code is "0" or not.

Range:	Yes/No

Default:	No

Procedure:

1.	Press FLASH button and dial 47.

2.	On a display phone, you will see this display:

ATTD CALL : 0 CO ACCESS : 9 YES

3.	To make change, press the top left program button.

LED off:	No

LED on:	Yes

4.	Press HOLD button.

SMDR TIMER (FLASH 48) (GHX-616)

SMDR prints for each CO call. If you use CO while SMDR timer, SMDR does not print about that and does toll free.

Parameters:

Determines the amount of toll free time for CO call.

Range:	000-255 sec

Default:	000 sec

Procedure:

1.	Press FLASH button and dial 48.

2.	On a display phone, you will see this display:

SMDR TIMER 000-255 000

3.	To make change, press 3 digits.

4.	Press HOLD button.

HOOK FLASH DEBOUNCE TIME (FLASH 49)

Hook flash debounce time for SLT can be set.

Parameters:

Determines the amount of on-hook time for hook flash.

Range:	01-99 (100 msec)

Default:	10 (100 msec)

Procedure:

1.	Press FLASH button and dial 49.

2.	On a display phone, you will see this display:

HOOK SWT DEBOUNCE 01-99 10

3.	To make change, press 2 digits.

4.	Press HOLD button.

7.8  EXCEPTION TABLES PROGRAMMING  (FLASH 50)

If you are in the programming mode, use the program codes continuously. If you are starting programming here, enter the programming mode at first. Press FLASH button and dial 50. If you have a display telephone, you will see this display:

EXCEPTION TABLES SELECT A TABLE

To program exception tables, use the program button as follows;

| ALLOW A |  | DENY A |  | ALLOW B |  | DENY B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

| DSP |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

Press a button to program a table. You will see a display such as this.

ALLOW TABLE A 01 02

The first 2 bin locations are displayed. Refer to work sheet and enter data via the dial pad.

Digits 0-9            	= numbers

DND/FOR button  	= don't care (D)

HOLD button           = End/Enter Data (E)

Each entry can be up to 10 digits. The entry format is : BBXXXXXXXXXX

BB = bin number

XX = exception number

Press HOLD button after each entry. (Display will be now updated.) When all entries for one table are complete, press program button for the next table to be programmed and follow the same procedure.

To erase an entry, enter that bin number followed by pressing HOLD button. (Display will be updated with that number removed.) To see what has been programmed into a particular table, press the "DSP" button. The display will scroll through the bins which have been programmed. The following characters can be displayed with a code.

D = don't care

E = end/enter

The Allow/Deny Tables are organized into 2 sets of tables to support 2 different toll plans at one installed site. Each allow table may contain up to 16 number strings and each deny table 16 number strings.

Each number string can contain up to 10 digits including any number 0-9, "Don't care". The following rules should be remembered when setting up the Allow/Deny Tables:

1. 	If the tables have no entries, no restriction is applied.

2. 	If entries are made in the allow table and only there, then only those numbers are allowed.

3. 	If entries are made in the deny table and only there, then only those numbers are denied.

4. 	If there are entries in both tables, the allow table is searched at first and if number is found, it

is allowed. If not found, the deny table is searched and if number is found, it is denied. If it is not 	found in either table, it is allowed.

[ALLOW/DENY RULES]

|  | Entries |  | Condition & Result |  |
| --- | --- | --- | --- | --- |
|  | Allow Tables | Deny Tables | Allow | Deny |
| Rule 1 | No | No | Allowed | Allowed |
| Rule 2 | Yes | No | Found - Allowed Not Found - Denied |  |
| Rule 3 | No | Yes |  | Found - Denied Not Found - Allowed |
| Rule 4 | Yes | Yes | Found - Allowed Not Found | Found - Denied Not Found - Allowed |

MUSIC SOURCE CONTROL (FLASH 51) (GHX-616)

This field allows to determine music source for line on hold.

Parameters:

There are three music sources; Internal, external music source and voice announce unit. Default setting is Internal music source.

Procedure:

1.	Press FLASH button and dial 51.

2.	On a display phone, you will see this display:

EXTERNAL MUSIC CONTROL 0

3.	To change music source, dial 1 digit (0-2) of music source type.

0 : Internal music source

1 : External music source

2 : Voice Announcement Unit (VAU3) for MOH.

4.	Press HOLD button.

7.9  STATION PROGRAMMING - (II)  (FLASH 52)

If you are in the programming mode, use the program codes continuously. If you are starting programming here, enter the program mode at first. If station features are to be changed, press FLASH button and dial 52. If you have a display telephone, you will see this display:

STATION ATTRIBUTES SELECT A STA RANGE

Program button #12 (SELECT STA #) will be lighted. Enter a 6-digit number for station range being programmed. If only 1 station, enter that number twice (i.e. 101101).

The display will appear as follows:

GHX-308A

XXX-XXX ENG ICM_TRF DOOR

To program features in GHX-308A , use the buttons as follows;

| DISPLAY |  | ICM TRANSFER |  | DOOR OPEN |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  |  |  |  |  |  | SELECT STA # |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

Buttons 1-3 toggle on and off.

LED off:  Displayed in Korean, No ICM Transfer privileged, No Door Open privileged.

LED on:  Displayed in English, ICM Transfer privileged, Door Open privileged.

The features are described as follows:

DISPLAY

This features selects language on LCD display. (English or Korean)

By default, English.

ICM TRANSFER

If ICM Transfer Privilege is allowed, you can transfer Intercom call to internal party.

By default, no privileged.

DOOR OPEN

If Door Open Privilege is allowed, stations can open the door(relay is controlled ON) for the Door Open timer period by dialing '*#1' (1st door) and '*#2' (2nd door).

By default, no privileged.

GHX-616

XXX-XXX OVR DOR ITRF BGM OFF-NET

To program features in GHX-616, use the buttons as follows:

| OVERRIDE |  | DOOR OPEN |  | ITRF |  | BGM |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

| OFF-NET |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  |  |  |  |  |  | SELECT STA # |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

Buttons 1, 2 toggle on and off.

LED off: No Override privileged, No Door Open privileged, No ICM Transfer privileged, No BGM privileged.

LED on: Override privileged, Door Open privileged, ICM Transfer privileged, BGM privileged.

The features are described as follows:

OVERRIDE

This feature enables privileged stations to override CO line or intercom to gain access to the conversation.

DOOR OPEN

If Door Open Privilege is allowed, stations can open the door(relay is controlled ON) for the Door Open timer period by dialing '*#1'(1st door) or '*#2' (2nd door). By default no privileged.

ICM TRANSFER

If ICM Transfer Privilege is allowed, you can transfer intercom call to internal party. By default no privileged.

BGM

If BGM Privilege is allowed, you can hear BGM by dialing ‘'773'. By default no privileged.

BGM source is determined by music source control program (FLASH 51). If music source is set to VAU, internal music source is provided for BGM.

OFF-NET

If OFF-NET Privilege is allowed, you can transfer calls to CO line. By default no privileged.

HOT LINE  (FLASH 53)

This feature lets a station perform a pre-assigned feature when a station takes no action for Hot Line after going off-hook. Hot Line feature can be programmed as one of the followings:

Activation time for Hot Line feature is determined by Hot Line Timer.

1) 	To activate a feature on flexible button as if pressed

2) 	To seize a CO Line

3) 	To seize a CO Line Group

4) 	To call an another station

5) 	To dial a SPEED BIN number

Procedure:

1. 	Press FLASH button and dial 53.

2. 	On a display phone, you will see this display:

HOT LINE SELECT A STA RANGE

3. 	Dial 6 digits to specify your desired stations. To program Hot Line feature for station 105, dial

105105. After dialing station range, you will see this display:

105-105 SELECT FEATURE FLX COL GRP STA SPD NONE

4. 	To program feature, use the button as follows:

| FLEXIBLE BTN |  | CO LINE |  | CO GROUP |  | STATION |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

| SPD DIAL BIN |  | CANCEL |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

1) 	To program a feature on flexible button as Hot Line feature, press button 1.

You will see following this display:

105-105 SELECT FEATURE FLEX BTN:

Dial flexible button number as 2 digits (01-24) and press HOLD button to save it.

Flexible feature can't be assigned as Hot Line feature on SLT.

2) 	To program a CO line as Hot Line feature, press button 2.

You will see the following display;

105-105 SELECT FEATURE CO LINE:

Dial CO line number as 2 digits (01-03) and press HOLD button to save it.

3) 	To program a CO line group as Hot Line feature, press button 3.

You will see following display;

105-105 SELECT FEATURE CO GROUP:

Dial CO line Group as 1 digit (1-3) and press HOLD button to save it.

4) 	To program a station number as Hot Line feature, press button 4.

You will see following display;

105-105 SELECT FEATURE STA NO:

Dial a station number as 3 digits(100-699) and press HOLD button to save it.

5) 	To program a speed bin number as Hot Line feature, press button 5.

You will see the following display:

105-105 SELECT FEATURE SPD NO:

Dial a speed bin number as 2 digits (00-99) and press HOLD button to save it.

6) 	To erase your programmed Hot Line feature, press button 6 and HOLD button.

VAU MODE PROGRAM  (FLASH 55) (GHX-616)

The user can divide VAU recording area (segment). Total recording time is 120 seconds. It can be divided into 10,20,30,40,60 or 120sec.

Parameters:

There are six modes;

| MODE | TIME OF SEG. | TOTAL SEG. NO |
| --- | --- | --- |
| 1 | 10 sec | 12 |
| 2 | 20 sec | 6 |
| 3 | 30 sec | 4 |
| 4 | 40 sec | 3 |
| 5 | 60 sec | 2 |
| 6 | 120 sec | 1 |

Default value is 3. (30 sec, 4 segments)

Procedure:

1.	Press FLASH button and dial 55.

2.	On a display phone, you will see this display:

VAU MODE 1-6 : 10s 20s 30s 40s 60s 120s

3.	To make change, dial 1 digit (1-6).

4.	Press HOLD button.

CO DIAL DELAY TIMER  (FLASH 56)

The system will give a user CO dial tone after the pre-determined time in this field to prevent a user from dialing a restricted number.

Parameters:

Determines the amount of time before giving a CO dial tone.

Range:	00-99 (x100) msec

Default:	05

Procedure:

1.	Press FLASH button and dial 56.

2.	On a display phone, you will see this display:

CO DIAL DELAY TMR 00-99 00

4.	Enter 2 digits on the numeric keypad.

5.	Press HOLD button.

HOT LINE TIMER  (FLASH 57)

Activation time of Hot Line feature can be programmed. On going off-hook, Hot Line timer starts. When Hot Line timer expires, Hot Line feature is activated.

Range:	0-9 sec

Default:	0

1.	Press FLASH button and dial 57.

2.	On a display phone, you will see this display:

HOT LINE TIMER 0-9 0

3.	Enter 1 digit on the numeric keypad.

4. 		Press HOLD button.

VAU SEGMENT CONTROL (FLASH 58)  (GHX-616)

Segments of VAU can be controlled for DISA, MOH and UCD individually. The system provides 8 message types of segments. There are 5 types of DISA messages, MOH and 2 types of UCD messages.

Range : Each segment can be set to one of 8 message types.

Default : By default, the system provides 4 message segments for DISA.

Message of Segment 1 is used for DISA DAY

Message of Segment 2 is used for DISA BUSY

Message of Segment 3 is used for DISA ERROR

Message of Segment 4 is used for DISA NO ANSWER

Procedure:

1.  	Press FLASH button and dial 58.

2.  	On a display phone, you will see this display;

VAU SEGMENT PROGRAM SELECT FLX BTN 1-12

- Determine message segment to change control, press the program button 1-12 to select message 	segment. If you press button 1 to select message segment 1, you will see the setting of message 	segment 1.

01 SEG : 1 DISA DAY 1DAY 2NIG 3BUSY 4ERR 5NO

4.  	Enter 1 digit (1-9) to change the usage of message segment.

Message Type

1 : Voice announcement for DISA in day mode

2 : Voice announcement for DISA in night mode

3 : Voice announcement for DISA to call busy station or CO

4 : Voice announcement for DISA to dial incorrect number

5 : Voice announcement for DISA to dial in time out

6 : Voice announcement for MOH

7 : Not used

8 : Voice announcement for UCD1 (No idle station in UCD group.)

9 : Voice announcement for UCD2

(No idle station in UCD group during UCD 1 message being played.)

5.  	Press HOLD button.

VAU CHANNEL CONTROL (FLASH 59) (GHX-616)

Four message segments of VAU can be controlled for MOH or DISA individually.

Range:	Program button 1-3(Channel 1-3)

Default:	Channel 1-3 are all set for DISA

Procedure:

1.	Press FLASH button and dial 59.

2.	On a display phone, you will see this display:

SELECT VAU CHANNEL 1-3BTN OR 01-03 DIAL

3.	Determine channel to change control, press the program button 1-3 to select channel.

4.	If you press button 1 to select channel 1, you will see the setting of channel 1.

01 CHANNEL : 2(DISA) 1:MOH 2:DISA 3:UCD

Enter 1 digit (1 or 2) to change control of channel usage.

1 : channel use for MOH

2 : channel use for DISA

3 : channel use for UCD

- Press HOLD button.

INITIALIZATION  (FLASH 60-65)

The system has been preprogrammed with certain features which are called default data, these features are loaded into memory when the system is initialized. The system should ALWAYS be initialized when installed or at any time the database has been corrupted.

To initialize the system to default values, repeat the procedures explained below for each of the database area.

1.  	Enter the programming mode.

2.  	Press FLASH button.

3.  	Dial the 2 digits initialization code.

INITIALIZE SYS PARAM PRESS HOLD

4.  	Press HOLD button.

Initialization Code

60:	System parameters

61:	CO lines

62:	Station parameters

63:	Exception tables

64:	System speed numbers

65:	All data

DISA NO ANSWER TIMER  (FLASH 66) (GHX-616)

If the station called by DISA/DID line does not answer within this timer, system regards the station does not answer the call.

Parameters:

Determines DISA no answer time as 3 digits.

Range:	000-255 sec

Default:	060 sec

Procedure:

1.	Press FLASH button and dial 66.

2.	On a display phone, you will see this display:

DISA NO ANS 000-255 060

3.	Enter the amount of time as 3 digits.

4.	Press HOLD button.

DISA ACCESS CODE  (FLASH 67) (GHX-616)

If a DISA access code is programmed, outside caller can't access system without entering this access code.

Parameters:

Enters DISA password as 4 digits.

Range:	3 digits

Default:	None

Procedure:

1.	Press FLASH button and dial 67.

2.	On a display phone, you will see this display:

DISA ACCESS CODE 000

3.	Enter access code as 3 digits.

4.	Press HOLD button.

DISA VOICE GUIDE (FLASH 68)  (GHX-616)

If a DISA voice guide is selected, system provides outside caller voice message recorded in voice announcement unit.

Parameters:

Determines weather the system select DISA voice guide.

Range:	Yes/No

Default:	No

Procedure:

1.	Press FLASH button and dial 68.

2.	On a display phone, you will see this display:

DISA VOICE GUIDE YES/NO NO

3.	To change, press the top left program button.

LED off = DISA voice guide is disabled.

LED on = DISA voice guide is enabled.

4.	Press HOLD button.

CO ACCESS OF DISA  (FLASH 69) (GHX-616)

The system will allow DISA line to access the CO line of the system.

Parameters:

Determines whether the system allow DISA line to access CO line.

Range:	Yes/No

Default:	No

Procedure:

1.	Press FLASH button and dial 69.

2.	On a display phone, you will see this display:

CO ACCESS OF DISA YES/NO NO

3.	To change, press the top left program button.

LED off = CO access of DISA is disabled.

LED on = CO access of DISA is enabled.

4.	Press HOLD button.

7.10  CUSTOMER DATABASE PRINTOUT

PRINT ALL DATABASE (FLASH 70) (GHX-616)

1.	Press FLASH button and dial 70.

If you have a display telephone, you will see this display:

PRINT SYSTEM DATA PRESS HOLD

2.	Press HOLD button. You will see this display;

PRINTING SYSTEM DATA

PRINT SYSTEM PARAMETERS (FLASH 71) (GHX-616)

1.	Press FLASH and dial 71. You will see this display:

PRINT SYS PARAM PRESS HOLD

2.	Press HOLD button.

PRINTING SYS PARAM

PRINT CO LINE ATTRIBUTES  (FLASH 72) (GHX-616)

1.	Press FLASH and dial 72.

PRINT CO LINES ENTER RANGE PRESS HOLD

2.	Program button 12 will be lighted.

3.	Enter range of lines (i.e. to print first line, enter 0101 ; to print first 5 lines, enter 0105).

4.	Press HOLD button and you will see this display:

PRINTING CO LINES

PRINT STATION ATTRIBUTES  (FLASH 73) (GHX-616)

1.	Press FLASH and dial 73.

PRINT STATIONS ENTER RANGE PRESS HOLD

2.	Program button 12 will be lighted.

3.	Enter station range (i.e. to print station 100, enter 100100: to print stations 100-110, enter 100110).

4.	Press HOLD button. You will see this display:

PRINTING STATIONS

PRINT EXCEPTION TABLES  (FLASH 74) (GHX-616)

1.	Press FLASH and dial 74.

PRINT EX TABLES ENTER BUTTON & BIN NO

2.	Press program button for desired individual table.

3.	Dial bin number.

4.	Press HOLD button.

| ALLOW A |  | DENY A |  | ALLOW B |  | DENY B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

5.	You will see this display:

PRINTING EX TABLES

PRINT SYSTEM SPEED NUMBERS  (FLASH 75) (GHX-616)

1.	Press FLASH and dial 75.

PRINT SYSTEM SPEED NO PRESS HOLD

2.	Program button 12 will be lighted.

3.	Dial the range. (for bin 20, enter 2020; for bins 25-40, enter 2540, etc.)

4.	Press HOLD button.

5.	You will see this display:

PRINTING SYS SPEED NO

FAX CO, STATION ASSIGNMENT  (FLASH 76) (GHX-308A)

This field allows to assign CO line and station for FAX transfer feature. When both a CO line and a station are assigned for FAX transfer, incoming thru CO line is answered by system and the call is checked from FAX machine or not by detecting FAX calling tone. If the incoming CO line is from FAX machine, this call is transferred to the FAX machine connected to SLT port.

Parameters:

Determines the CO line for receiving FAX signal and the station for FAX machine.

Range:	CO range : 01-03,  Station range : 101-107

Default:	None

Procedure:

1. 	Press FLASH button and dial 76.

2. 	On a display phone, you will see this display:

FAX CO, STA ASSGN CO:## STA:###

3. 	To program FAX CO line, press Button 1.

- Enter CO line number(01-03) as 2 digits and press HOLD button.

- To delete an entry, enter pound (# #) signs twice.

4. 	To program station number, press Button 2.

- Enter station number(101-107) as 3 digits and press HOLD button.

- To delete an entry, enter pound (# # #) signs three times.

UCD MEMBER FIRST CHOICE  (FLASH 76) (GHX-616)

When CO ring is assigned to Hunt Group (UCD Group), there are two ways of selecting STA in UCD group.

First STA choice

You can select the first STA available in that UCD group.

Round Robin STA choice

You can select sequentially STA in that UCD group whenever access UCD CO.

Parameters:

Range:	Yes/No

Default:	Round robin

Procedure:

1.	Press FLASH button and dial 76.

2.	On a display phone, you will see this display:

UCD MEMBER FIRST CHOICE NO

3.	Press button 1.

LED ON : First Choice

LED OFF : Round Robin Choice

4.	Press HOLD button.

FAX TONE DETECT TIMER  (FLASH 77) (GHX-308A)

The system answers the incoming CO line assigned to FAX transfer and checks the call comes from FAX by detecting FAX calling tone. This field determines the duration to detect FAX calling tone. If FAX calling

tone is not detected during this time, the CO line calls stations assigned to ring. When FAX calling tone is detected, the system gives ring to the station assigned for FAX.

Range:	05-30 sec

Default:	05 sec

Procedure:

1.	Press FLASH button and dial 77.

2. 	On a display phone, you will see this display:

FAX TONE DTCT TMR 05-30 05

3. 	Enter the time as 2 digits and press HOLD button.

DISA, with HUNT GROUP  (FLASH 77) (GHX-616)

This parameter determines the type of calling hunt group. If the first parameter is set then inputted DISA dial digit is only used to call hunt group. If the second parameter is set then inputted the first digit can be used to call hunt group when the second is not entered within predetermined interval. When 3 digits dial is inputted within predetermined interval, you can call specified extension. If the last parameter is set, Hunt Group is called when the called extension is busy. To call Hunt Group 710-717, dial 0-7.

Parameters:

Range:	Yes/No

Default:	No

Procedure:

1.	Press FLASH button and dial 77.

2.	On a display phone, you will see this display:

ONLY/CNV/BUSY HUNT NO NO NO

3.	Press your desired button (1-3).

LED ON : Yes

LED OFF : No

4.	Press HOLD button.

FAX CO CALL TIMER  (FLASH 78) (GHX-308A)

The incoming line thru FAX CO gives ring to the FAX station or all the stations assigned to ring on this line according to FAX calling tone detection. If this call is not answered till this timer expires, the system drops this CO line.

Range:	1-3 min.

Default:	1 min.

Procedure:

1.   	Press FLASH button and dial 78.

2.   	On a display phone, you will see this display:

FAX CO CALL TMR 1-3 1

3.   	Enter the time as 1 digit and press HOLD button.

DISA NIGHT ACT (FLASH 78) (GHX-616)

To handle DISA call in night mode, you can select one of three methods by programming.

Parameters:

Process :

- Same as day DISA mode.

- Send error tone and drop the line.

- To transfer pre-selected station or hunt group.

Default: Same as day DISA mode

Procedure:

1.	Press FLASH button and dial 78.

2.	On a display phone, you will see this display:

DISA NIGHT ACT : DIAL 1DIAL-NUM 2TONE 3PRE-SEL

3.	To change night mode DISA process, dial 1 digit (1-3).

1 : Same process for day DISA mode. (DISA line can dial station number)

2 : Send night DISA message and error tone then drop the line.

3 : Send night DISA message and transfer to pre-selected station.

4.	Press HOLD button.

SLT RING OFF TIME  (FLASH 79) (GHX-308A)

This field determines SLT ring off time.

Range :  3-4 sec

Default :  4 sec

Procedure :

1.   	Press FLASH button and dial 79.

2.   	On a display phone, you will see this display ;

SLT RING OFF TIME 4

3.   	To change SLT ring off time, press top left button and HOLD button.

DISA BUSY/NO/ERROR DEST  (FLASH 79) (GHX-616)

DISA call is differently processed on the below state. The process of each state is changed by Admin. program.

Parameters:

DISA line can be one of following states.

1.  	Called station is in busy.

2.  	DISA line dialed invalid digits.

3.  	Called station does not answer.

4.  	DISA line dial is late.

- DISA line drop the line without dialing while hearing DISA-Day or Night Message. And CPT detects

- error tone from the PX line.

Each DISA state can be handled by one of three methods.

1. 	Send error tone to DISA CO and drop.

2. 	Transfer to attendant.

3. 	Retry.

Default:

Call to busy STA or CO.	   	- Retry

Input digit is invalid.	   	- Retry

Called party does not answer.  - ATTD : Transfer to attendant

Dial is late.			   	- Give error tone to CO line and drop the line.

Error tone detect		    	- Give error tone to CO line and drop the line.

Procedure:

1. 	Press FLASH button and dial 79.

2. 	On a display phone, you will see this display.

DISA BUSY/NO/ERROR DEST PRESS FLX KEY 1-5

3.	To select DISA state, press flexible button 1-5.

Flex. button 1 - Busy STA

Flex. button 2 - Invalid digit dial

Flex. button 3 - Not answer

Flex. button 4 - Dial is late

Flex. button 5 - Error tone detect from CO line

If press Flex. button 1, then on a display phone, you will see this display :

DISA BUSY : RETRY 1:TONE 2:ATTD 3:RETRY

4.	To select DISA process, dial one digit (1-3).

1 :  Send error tone to DISA CO and drop.

2 :  Transfer to attendant.

3 :  Retry.

5.	Press HOLD button.

VAU TONE CADENCE (FLASH 83) (GHX-616)

When outside caller through DISA line drops the line during VAU being played, CPT on the VAU(voice announcement unit) detect tone from the PX.  If tone cadence specified by this field is detected, system drop this line.

Parameters:

The tone cadence to detect

Range:	000-255 (* 50msec)

Default:	ON time : 010(*50msec) ,  OFF time : 010 (*50msec)

Procedure:

1.	Press FLASH button and dial 83.

2.	On a display phone, you will see this display:

VAU TONE CADENCE (50MS) ON:010 OFF:010

- To enter ON time, press button 1 and enter 3 digits on the numeric keypad.

- (To enter OFF time, press button 1 and enter 3 digits on the numeric keypad.)

4.	Press HOLD button.

VAU TONE TIME (FLASH 84)  (GHX-616)

If the tone specified by FLASH 83 is detected more than this time during DISA message being played,  system regards outside caller disconnects the line and drops the CO line.

Parameters:

The tone cadence to detect

Range:	3-9 (sec)

Default:	5 (sec)

Procedure:

1.	Press FLASH button and dial 84.

2.	On a display phone, you will see this display:

VAU TONE TIME 3-9 5

- To enter time, dial 1 digit on the numeric keypad.

4.	Press HOLD button.

DISA MESSAGE REPLAY (FLASH 85) (GHX-616)

DISA message of VAU can be played once or twice.

Parameters:

Once or Twice

Range:	YES/NO

Default:	NO (Once)

Procedure:

1.	Press FLASH button and dial 85.

2.	On a display phone, you will see this display:

DISA MESSAGE REPLAY NO

- To replay VAU DISA message, press button 1.

4.	Press HOLD button.

FAX CO, STATION ASSIGNMENT (FLASH 86) (GHX-616)

This field allows to assign CO line and station for FAX transfer feature. When both a CO line and a station are assigned for FAX transfer, incoming through CO line is answered by system and the call is checked from FAX machine or not by detecting FAX calling tone. If the incoming CO line is from FAX machine, this call is transferred to the FAX machine connected to SLT port.

Parameters:

Determines the CO line for receiving FAX signal and the station for FAX machine.

Range:	CO range : 01-06,  Station range : SLT port only

Default:	None

Procedure:

1. 	Press FLASH button and dial 86.

2. 	On a display phone, you will see this display:

FAX CO, STA ASSGN CO:## STA:###

3. 	To program FAX CO line, press Button 1.

- Enter CO line number(01-06) as 2 digits and press HOLD button.

- To delete an entry, enter pound(# #) signs twice.

4. 	To program station number, press Button 2.

- Enter station number as 3 digits and press HOLD button.

- To delete an entry, enter pound(# # #) signs three times.

FAX TONE DETECT TIMER  (FLASH 87) (GHX-616)

The system answers the incoming CO line assigned to FAX transfer and checks the call comes from FAX by detecting FAX calling tone. This field determines the duration to detect FAX calling tone. If FAX calling tone is not detected during this time, the CO line calls stations assigned to ring. When FAX calling tone is detected, the system gives ring to the station assigned for FAX.

Range:	05-30 sec

Default:	05 sec

Procedure:

1.	Press FLASH button and dial 87.

2. 	On a display phone, you will see this display:

FAX TONE DTCT TMR 05-30 05

3. 	Enter the time as 2 digits and press HOLD button.

FAX CO CALL TIMER  (FLASH 88) (GHX-616)

The incoming line thru FAX CO gives ring to the FAX station or all the station assigned to ring on this line according to FAX calling tone detection. If this call is not answered till this timer expires, the system drops this CO line.

Range:	1-3 min.

Default:	1 min.

Procedure:

1.   	Press FLASH button and dial 88.

2.   	On a display phone, you will see this display:

FAX CO CALL TMR 1-3 1

3.   	Enter the time as 1 digit and press HOLD button.

SLT RING OFF TIME  (FLASH 89) (GHX-616)

This field determines SLT ring off time.

Range :  3-4 sec

Default :  4 sec

Procedure :

1.   	Press FLASH button and dial 89.

2.   	On a display phone, you wil see this display ;

SLT RING OFF TIME 4

3.   	To change SLT ring off time, pres top left button and HOLD button.

PRINTOUT FORMAT

SYSTEM PARAMETERS

.....................

SHR   ENR   XFR   PFT   PT   CPT

060    180    045    10     2    180

CFT   MWT  CDT   HPR   PRI   ICM

10     000     3     SYS  YES   20

MON  HFT   ART    ADT  DDT

NO    05     030    060   00

ATTENDANT STATIONS

100

LOUD BELL ASSIGNMENTS

###  ###

PBX DIALING CODES

##  ##  ##  ##  ##

EXECUTIVE/SECRETARY PAIRINGS

1 = ###  ###

2 = ###  ###

3 = ###  ###

4 = ###  ###

HUNT GROUPS

HG0 710  100 101 102 103

... ... ... ...

HG1 711  104 105 106 107

... ... ... ...

HG2 712  108 109 110 111

... ... ... ...

HG3 713  112 113 114 115

... ... ... ...

.......

HG7 717  128 129 130 131

SMDR TYPE  PRNT  BAUD  PIC

YES   ALL   80     1200   YES

ALARM   SIGNAL   DETECT

NO       ONCE    CLOSED

CO LINE ATTRIBUTES

CO 01  SIGNAL  TYPE UNA

DIMF    CO  YES

TRO SUPV FLTM  GRP  GF

NO    0    10     1   NO

DIAL PULSE

RATIO    SPEED

6633      10PPS

CO 02  SIGNAL  TYPE  UNA

DIMF    CO   YES

TRO SUPV FLTM  GRP  GF

NO   0     10     1   NO

DIAL PULSE

RATIO     SPEED

6633      10PPS

Continues through the rest of CO lines.

STATION ATTRIBUTES

STA 100  PAGE DND SPD HDT PLA

YES  YES  NO  YES

WARN    FWD  ALM SID  COS SP

NO       YES  NO   1    11  0

PICKUP 1         PAGE 1

PREFWD ### CO ACCESS 1234567

BUTTONS

0101B 0202B 0303B 0404B

0505B 0606B 0707B 0808B

13     14    15     16

17     18    19     20

21     22    23     24

STA 101  PAGE  DND  SPD  HDT  PLA

YES   YES   YES  NO   YES

WARN    FWD  ALM   SID   COS  SP

NO        YES   NO   1     11    0

PICKUP 1         PAGE 1

PREFWD ### CO ACCESS 1234567

BUTTONS

0101B 0202B 0303B 0404B

0505B 0606B 0707B 0808B

13     14    15     16

17     18    19     20

21     22    23     24

Continues through the rest of the stations.

EXCEPTION TABLES

ALLOW TABLE A

01				09

02				10

03				11

04				12

05				13

06				14

07				15

08				16

DENY TABLE A

01				09

02				10

03				11

04				12

05				13

06				14

07				15

08				16

ALLOW TABLE B

01				09

02				10

03				11

04				12

05				13

06				14

07				15

08				16

DENY TABLE B

01				09

02				10

03				11

04				12

05				13

06				14

07				15

08				16

SYSTEM SPEED NUMBERS

20

21

22

..

..

..

99

FLEXIBLE STATION NUMBERING

INTERCOM NUMBER PROGRAM

PORT NUMBER

01   100

02   101

..   ...