iPECS ES-2000 Advanced Smart Switches

Software Version 1.2.20.0

INTRODUCTION

iPECS ES-2000 Advanced Smart Switches consist of 4 models: ES-2026, ES-2026P, ES-2024G, and ES-2024GP.

ES-2026 is a cost effective managed L2/L4 Fast Ethernet Standalone Switch. ES-2026 contains 24 Ethernet/Fast Ethernet ports (1-24) and 2 Combo-Giga ports* (25-26).

ES-2026P is a cost effective managed L2/L4 Fast Ethernet PoE Standalone Switch. ES-2026P contains 24 Ethernet/Fast Ethernet PoE ports** (1-24) and 2 Combo-Giga ports* (25-26).

ES-2024G is a cost effective managed L2/L4 Gigabit Ethernet Standalone Switch. ES-2024G contains 24 Gigabit Ethernet ports (1-24) with the last 4 Combo-Giga ports* (21-24).

ES-2024GP is a cost effective managed L2/L4 Gigabit Ethernet PoE Standalone Switch. ES-2024GP contains 24 Gigabit Ethernet PoE** ports (1-24) with the last 4 Combo-Giga ports* (21-24).

* 10/100/1000 CU built-in associated Mini-GBIC slots

** Any port can support a maximum of 32.4W (high power) per port.

The base hardware is a 1U height, 19” rack mountable metal enclosure. The switch can be managed either by in-band management via the network station remotely or out-of-band management via the console port (RS232) locally. An imbedded Web agent also provides management capability to any computer on the network via common HTTP browsers such as Netscape Navigator (Version 6.2 or above) or Microsoft’s Internet Explorer (Version 5.0 or above).

Management of the switch is password protected; the password is used for the Web browser interface. In order to access the Management Module via a network connection, at factory defaults the switch has already been assigned an IP address: 192.168.1.10 and subnet mask: 255.255.255.0. The management option provides SNMP, RMON (4 groups: 1,2,3,9), and Web management for system control and statistical monitoring.

It is recommended that one thoroughly review this release notes prior to the installation or upgrade of this product.

Firmware Specification

| Firmware | Version | File Name | File Size | Created Date |
| --- | --- | --- | --- | --- |
| Loader | 1.0.1.8 | iPECS_ES-2000_Ldr_V1.0.1.8.bix | 394,440 | 2010/12/16 |
| Software | 1.2.20.0 | iPECS_ES-2000_Op_V1.2.20.0.bix | 9,947,260 | 2013/6/4 |
| Private MIB | - | iPECS_ES-2000.mib | 406,757 | 2012/10/23 |

| Status | Version. | Type | Release Date |
| --- | --- | --- | --- |
| Phase 2 |  |  |  |
| Current Release | 1.2.20.0 | Patch | 2013/6/4 |
| Previous Release | 1.2.18.0 | Patch | 2013/4/11 |
| Previous Release | 1.2.16.0 | Patch | 2012/11/29 |
| Previous Release | 1.2.15.0 | Patch | 2012/10/4 |
| Previous Release | 1.2.14.0 | Patch | 2012/8/6 |
| Previous Release | 1.2.13.0 | Patch | 2012/6/26 |
| Previous Release | 1.2.12.0 | Patch | 2012/6/1 |
| Previous Release | 1.2.11.0 | Patch | 2012/4/6 |
| Previous Release | 1.2.10.0 | Patch | 2012/4/6 |
| Previous Release | 1.2.9.0 | Patch | 2012/3/15 |
| Previous Release | 1.2.8.0 | Patch | 2012/2/20 |
| Previous Release | 1.2.7.1 | Patch | 2012/1/20 |
| Previous Release | 1.2.7.0 | Patch | 2012/1/6 |

Network Management Software Support

| NMS Platform | Version No. | Module No. |
| --- | --- | --- |
| TBD |  |  |

If you install this image, you may not have control of all of the latest features of this product until the next version(s) of network management software. Please review the software release notes for your specific network management platform for details.

SUPPORTED FUNCTIONALITY

(TBD)

Installation and Configuration Notes

In general, ES-2000 Switch will be shipped to you pre-configured with this version of firmware. If you would like to upgrade an existing ES-2000 Switch, please follow the HTTP Upgrade instructions that are included with your firmware image upgrade kit.

Firmware Fixes, Changes, and Enhancements

The following issues have been fixed in this software release. Unless specified, they are common to all 4 models of ES-2000 switches.

Opcode V1.2.20.0; Loader V1.0.2.0

| Ticket No | Headline | Role |
| --- | --- | --- |
| ES-3024GP-01279 | New feature request for ES-2K/3K/4K for Korean government customers => New behavior: System-boot-up only auto-generates the default "admin" user account when no account exists after startup provision is completed. | Customer |
| ES-3024GP-01280 | Ping invalid IP address (0.0.0.0, 224.1.1.1, 255.255.255.255, 255.255.255.0), but get response (time: 0 ms) on WebGUI. | Tech-support |

Opcode V1.2.19.0; Loader V1.0.2.0

| Ticket No | Headline | Role |
| --- | --- | --- |
| ES-3024GP-01267 | DHCP Client: The "DHCP Class-id" does not support on web and help page. | NTC |
| ES-3024GP-01269 | Can't input port description with space via SNMP | NTC |
| ES-3024GP-01270 | IP-AddrConfig: DUT does not reject this invalid gateway address setting. | NTC |
| ES-2026P-00230 | CableDiag: DUT cannot create new session after the last test is failed. | NTC |

Opcode V1.2.18.0; Loader V1.0.2.0

| Ticket No | Headline | Role |
| --- | --- | --- |
| ES-3024GP-01247 | CLI “show interfaces counters” on trunk port have problem(s) – Port Util counters in incredible values, RMON counters in 0 value. | RD |
| ES-3024GP-01248 | FXC5352(ES4552MA)Buglist#34 web-auth Remaining Session Time should be same between CLI(0) and WebGUI(Disabled). | RD |
| ES-3024GP-01251 | Web: Security > ACL: DUT displays incorrect configuration Source-MAC and Sub-Mask. | NTC |
| ES-3024GP-01252 | Web: Traffic > Priority > {DSCP to DSCP ¦ CoS to DSCP ¦ PHB to Queue}: After applying "Delete" to any value that has been modified, it is reset to the default value on "CoS to DSCP Mapping List". | NTC |
| ES-3024GP-01253 | Web: Traffic > Diffesrv: "Action" item does not have its own checkbox, which is separate from “Meter”. But CLI does. | NTC |
| ES-3024GP-01254 | The device’s memory will be eventually exhausted if it receives the specific LLDP packet with TIA. | Tech-support |
| ES-3024GP-01255, 1258, 1259 | Update Web help pages – security_acl, system_reload, system_time | RD, NTC |

Opcode V1.2.17.0; Loader V1.0.1.8

| Ticket No | Headline | Role |
| --- | --- | --- |
| ES-3024GP-01239 ES-3024GP-01244 | Add Web help pages – ip_arp, traffic_packet_flow | NTC RD |
| ES-3024GP-01238, 1240, 1241, 1242, 1243 | Update Web help pages – system_switch, traffic_priority_trust_mode system_time, sitemap traffic_diffserv | NTC RD |

Opcode V1.2.16.0; Loader V1.0.2.0

| Ticket No | Headline | Role |
| --- | --- | --- |
| ES-3024GP-01229 | [ES3510MA_V1.3.4.3] High CPU utilization after enabling switchport ingress-filtering function. | RD |
| ES-3024GP-01231 | ES-3024GP : wrong https description in "Site Map > Secuiity > HTTPS" | Customer |
| ES-3024GP-01232 | SNMP inefficiently processes variable bindings that causes it do jobs repeatedly. | RD |
| ES-3024GP-01233 | Request to change Company name and logo to Ericsson-LG (original: LG-Ericsson) | Customer |
| ES-3024GP-01235 | ACL: The VID range of MAC-ACL not matched to static VLAN setting. | RD |

Opcode V1.2.15.0; Loader V1.0.1.8

| Ticket No | Headline | Role |
| --- | --- | --- |
| ES-3024GP-01209 | ES2026P txt file extension is not able to be download via http | Customer |
| ES-3024GP-01213 | Security: Unexpected debug message would be generated when doing Nessus test. | RD |
| ES-3024GP-01215 | FXC5352(ES4552MA)BugList#19 Reload setting problem if hours set to "0" | RD |
| ES-3024GP-01216 | FXC5352(ES4552MA)BugList#20 System reload setting does not work about regularity time | RD |
| ES-3024GP-01217 | [XC] CLI crash if using Dynamic-qos to set ACL rule binding | RD |
| ES-3024GP-01218 | FXC5352(ES4552MA)BugList#30 Strange loopback syslog is detected in ATC multicast testing | RD |
| ES-3024GP-01219 | FXC5352(ES4552MA)Buglist#37 Error Link Detection trap about action:trap | RD |
| ES-3024GP-01220 | Help Page:System reload help page can not display correct. | RD |
| ES-2024GP-00010 | ES2000 missing Poemgt Object | Customer |
| ES-3024GP-01223 | The state of RSPAN vlan would go back to Active when Suspend RSPAN vlan then reload | RD |
| ES-3024GP-01226 | ES-3024GP : Wrong information in WEB help page and user manual about the https feature | Customer |
| ES-3024GP-01227 | CLI: MAC ACE with Ethertype parameter setting and Time Range name leading with "c" will cause DUT display Exception and Hang | RD |

Opcode V1.2.14.0; Loader V1.0.1.8

| Ticket No | Headline | Role |
| --- | --- | --- |
| ES-3024GP-01198 | wrong operation MAU type in ES30xx | Customer |
| ES-3024GP-01201 | LGE1 : Invalid IPv6 address can be executed | Tech-support |
| ES-3024GP-01203 | LGE1 : arp timeout setting no effect in webgui | Tech-support |
| ES-3024GP-01204 | LGE1 : Strange information about ip source guard on port channel in web and CLI mode | Tech-support |
| ES-3024GP-01205 | LGE1 : We cannot set IP source guard in webui while creating port-channel | Tech-support |
| ES-3024GP-01206 | LGE1 : Traffic-segmentation can't assign port-channel port in CLI | Tech-support |
| ES-3024GP-01207 | ES-3052GP : switch will reboot while running tcp ping | Tech-support |
| ES-3024GP-01210 | [diffserv] can't remove meter from policy-class-map | RD |
| ES-3024GP-01211 | FXC5352(ES4552MA)BugList#23 Mirror port-channel does not work after rebooting | RD |

Opcode V1.2.13.0; Loader V1.0.1.8

| Customer |  |
| --- | --- |
| ES-3024GP-01188 | ES-3024GP : Can't set domain name |
| ES-3024GP-01191 | ES-3024GP : Max sessions of Port mirror on ES-2K/ES3K |
| RD |  |
| ES-3024GP-01192 | DNS: TTL sometimes shows up as a large positive number |
| ES-3024GP-01193 | Web UI: When DUT set IPv6 link-local address via Web UI. Web UI will display incorrect address. |
| ES-3024GP-01194 | Web: Wrong Spelling "Minitue" on Summer Time Date Mode Configuration page. |

Opcode V1.2.12.0; Loader V1.0.1.8

| Customer |  |
| --- | --- |
| ES-3024GP-01181 | ES-3024GP : ES30/20 xx cam't forward the UDP packet dst port = 0 but ES-4k can |
| ES-3024GP-01183 | ES-3024GP : Web page link error |
| RD |  |
| ES-3024GP-01184 | The linux script file (convertWebFiles) did not check if the converting file is an image file or not. |
| ES-3024GP-01186 | Modify information of access-list tcam-utilization |
| ES-3024GP-01187 | Diffserv: when the rule resource is not enough, binding policy map to interface shall be fail. |
| ES-3024GP-01189 | A next client is not able to pass do1x authentication when a preceding client got failure before. |
| ES-2026P-00228 | Port Mirror :DUT can not be set the port mirror function in the Trunk mode ( WEB UI Show Data Invalid ) |

Opcode V1.2.11.0; Loader V1.0.1.8

| RD |  |
| --- | --- |
| ES-3024GP-01182 | FW: 1.2.10.0 Can't save configs after loading factory default config file |

Opcode V1.2.10.0; Loader V1.0.1.8

| Customer |  |
| --- | --- |
| ES-3024GP-01175 | ES-3052GP : Daylight saving enhancement |
| ES-3024GP-01176 | Loopback detection function fails to work on ES-3024GP |
| ES-3024GP-01177 | Change default value of management IP to DHCP for MA model of USA sku |
| ES-3024GP-01178 | Factory default config cannot be produced correctly when it's mismatch with runtime or missed (or deleted). |
| ES-3024GP-01180 | first lldp packet option TIA-Network policy from switch will be untag ES2026P |

Opcode V1.2.9.0; Loader V1.0.1.8

| Customer |  |
| --- | --- |
| ES-3024GP-01171 | error message display after receive adtran lldp packet on ES-3026 |
| ES-3024GP-01173 | ES-3024GP : Add the hostname feature of SNTP server |

Opcode V1.2.8.0; Loader V1.0.1.8

| Customer |  |
| --- | --- |
| ES-3024GP-01167 | ES-3052G : Link Auto-negotiation problem at the Korean National Police Agency sites |
| ES-3024GP-01160 | ES-3024G : Difference behavior for copy command |
| ES-2024GP-00227 | ES-2024GP : switch should stop upgrading if the opcode is wrong |

Opcode V1.2.7.1; Loader V1.0.1.8

| Customer |  |
| --- | --- |
| ES-3024GP-01166 | Telnet: After transfer file completely by telnet session, another session(CLI) sometimes can't ping PC successfully. |

Opcode V1.2.7.0; Loader V1.0.1.8

| Customer |  |
| --- | --- |
| ES-3024GP-01140 | ES-3024GP : 5 switches were down |

Opcode V1.2.6.0; Loader V1.0.1.8

| Customer |  |
| --- | --- |
| ES-3024GP-01124 | Increase the length of the password from 8 characters to 32 characters |
| ES-3024GP-01123 | ES-3026 : TC BPDU will be sent if we plug on/off the PC on the edge port of the switch |

Opcode V1.2.5.0; Loader V1.0.1.8

| Customer |  |
| --- | --- |
| ES-3024GP-01139 | ES-3024GP : CPU load is always 100% for v1.2.4.0 |

Opcode V1.2.4.0; Loader V1.0.1.8

| Customer |  |
| --- | --- |
| ES-3024GP-01105 | ES-3026 : ‘invalid data’ pops-up continuously while downloading wrong configuration file |
| RD |  |
| ES-3024GP-01106 | Implement SW watchdog |

Opcode V1.2.3.1; Loader V1.0.1.8

| Customer |  |
| --- | --- |
| ES-3024GP-01059 | ES-3000 does not report system name(hostname) in the sh lldp info remote function |
| ES-3024GP-01074 | Leverage Service tag in UC to differentiate models of Korea and USA |
| ES-3024GP-01095 | ES-3026P: Heavy traffic load |
| RD |  |
| ES-3024GP-01075 | WEB information: Operation Code "Undefined" information |

Opcode V1.2.3.0; Loader V1.0.1.8

| Customer |  |
| --- | --- |
| ES-3024GP-01017 | Manual revision for VOIP function in LGE 24/26 models |
| ES-3024GP-01023 | [LGE]Show Rule of Security > ACL looks different with ES4524MA |
| ES-3024GP-01047 | ES-3026 : RSPAN setting can't be set via WEB |
| NTC |  |
| ES-3024GP-00412 | RSPAN: It should not display error message in CLI/web when setting RSPAN destination up link port |
| ES-3024GP-00484 | RSPAN: It should display warning message or fail to set because the port is access mode when setting RSPAN VLAN |
| ES-3024GP-00897 | LACP: delete dynamic port-channel will crash |
| ES-3024GP-01063 | DHCPDynamicProvision: After setting class id to maximum length and show running, DUT will crash. |
| RD |  |
| ES-3024GP-00966 | LLDP: Memory leaks occur when receiving a PDU which has many Port and Proto VLAN ID TLV of VLAN id 0. |
| ES-3024GP-00972 | Https: After upgrade the OpCode, the Web UI could not display web information correctly. |
| ES-3024GP-01037 | PreST-009: [CLI][acceptable character] the error message of snmp-server host is strange - Invalid source IP address mp. |
| ES-3024GP-01041 | SNMPv3: DUT auto add non-existent group to SNMP group entry after modify SNMP host entry |
| ES-3024GP-01044 | mib:Creat new mib->Rmon->statistics->etherStatsTable->etherStatsEntry, all the variable should be zero |
| ES-3024GP-01051 | Accton L2 switch : the switch cpu utilization is stuck at 100%, telnet/gui response are slow |
| ES-3024GP-01052 | GVRP: DUT can create vlan 4093 by gvrp, but DUT port can't join vlan 4093 by gvrp. |
| ES-3024GP-01054 | L2-L4-Core: Multicast client can't receive any multicast data aftr saving configuration and rebooting. |
| ES-3024GP-01055 | GVRP: DUT can send LeaveIn message. |
| ES-3024GP-01056 | Checked web help file for runtime version 1.2.2.1 |

Opcode V1.2.2.1; Loader V1.0.1.8

| Customer |  |
| --- | --- |
| ES-3024GP-01029 | ES-3024GP : After sending trap, Security Status is checked. |
| ES-3024GP-01038 | ES-3024G : Web Connection problem via HTTPS for ES-2024GP/ES-3024G |
| ES-3024GP-01039 | [LNA] [enhance] Display 64MB full flash memory size |
| ES-3024GP-01043 | Flash memory : The Flash memory display incorrect on Web UI |
| ES-2024GP-00214 | [LGE]Change Maximum Power consumption from 30W to 34.2W |
| RD |  |
| ES-3024GP-01040 | SNMP/RADIUS: if RadiusServerSataus is vaild(1) we sould allow setting. |

Opcode V1.2.2.0; Loader V1.0.1.8

| Customer |  |
| --- | --- |
| ES-3024GP-01033 | Support POE+ PD on port 7-24 |

Opcode V1.2.1.0; Loader V1.0.1.8

| Customer |  |
| --- | --- |
| ES-3024GP-01027 | [POE] Enhance MAX POWER ALLOCATION function of PoE... |
| ES-3024GP-01033 | Support POE+ PD on port 7-24 |
| RD |  |
| ES-3024GP-00971 | corrections to spelling/grammar - show lldp info remote-device detail |
| ES-3024GP-00983 | CLI/LLPD: "show lldp config detail ..." - MORE - Q (quit) should not print an extra blank line |
| ES-3024GP-00997 | console&telnet: the CLI shows different words to WEB |
| ES-3024GP-01022 | Remove Zebos from platform |
| ES-3024GP-01030 | InBandMGMT: "Invalid netmask" message is undefined error message. |
| ES-3024GP-01032 | IGMPfilter: If DUT has created port trunk, DUT can't set max multicast group number. |

Opcode V1.2.0.13; Loader V1.0.1.8

| Customer |  |
| --- | --- |
| ES-3024GP-01013 | ES-3026P:(Voice VLAN) “voice vlan mac-address” item was gone after rebooting the switch. |
| NTC |  |
| ES-3024GP-00964 | P1-2011-05-02 LLDP-Med issue in hybrid mode: It shows tagged "NO" but the correct value should be "YES". |

Opcode V1.2.0.12; Loader V1.0.1.8

| Customer |  |
| --- | --- |
| ES-3024GP-00965 | ARP Inspection setting question |
| ES-3024GP-00976 | Exact jumbo frame size |
| ES-3024GP-00955 | [LG-E] Port security issue |
| ES-3024GP-00996 | Detecting TS continuity counter skips when send multicast stream across to 2nd ES3024G |
| ES-3024GP-01005 | Memory dump when I test ARP inspection |
| ES-2024GP-00211 | [FR] Enable IGMP proxy in MAW model |
| ES-3024GP-00977 | We want to include the violating MAC address information in trap message for port security |
| NTC |  |
| ES-3024GP-01011 | LACP: Descriptions for the same command parameter in #show LACP and #show LACP 1(channel group) are not consistent. |

Opcode V1.2.0.11; Loader V1.0.1.8

| Customer |  |
| --- | --- |
| ES-3024GP-00855 | SMTP issue : When I try to put “user name”, I can put only up to 20 characters. |
| ES-3024GP-00882 | Issue1-- change a speed for GE port of Smart switch, I see "data is invalid" |
| ES-3024GP-00955 | A CLI command is gone for ES-3000 series. |

Opcode V1.2.0.10; Loader V1.0.1.8

| Customer |  |
| --- | --- |
| ES-3024GP-00841 | The value should be modified when show interface transceiver information. |
| ES-3024GP-00846 | We can't see the current setting after setting port configuration via web. |
| ES-3024GP-00861 | Trust mode for trunk port |
| ES-3024GP-00880 | Missed <CR> when executing qos map command |
| ES-3024GP-00881 | Data is invalid popup when use traffic-> priority menu after setting trunk mode |

Opcode V1.2.0.9; Loader V1.0.1.8

| Customer |  |
| --- | --- |
| ES-3024GP-00766 | I never succeeded upgrade in one time.Once we fail, we can't proceed upgrading without rebooting switch. |
| ES-3024GP-00786 | On the latest release 1.2.0.5 regarding IGMPv3 issue |
| ES-3024GP-00789 | remove clear button for Utilization page and revise WEB for showing utilization statistics(WEB and CLI mismatch) |
| ES-3024GP-00798 | I deleted ACL rule from table. And I tried to delete time table but I couldn’t. |
| ES-3024GP-00800 | ACL:Time range of access group is display incorrect. |
| ES-3024GP-00844 | When I click port 24 of web GUI, I can't see the exact port configuration information since I set truck for some ports. |
| ES-3024GP-00846 | We can't see the current setting after setting port configuration via web. |
| ES-3024GP-00856 | Doesn't remove IGMP membership information after receiving leave |

Opcode V1.2.0.8; Loader V1.0.1.8

| Customer |  |
| --- | --- |
| ES-3024GP-00796 | When we add 8th time range rule with weekdays, we see “data is invalid” pop-up window |
| ES-3024GP-00797 | When I set a periodic time_range (Not absolute) for PoE, it’s not work when we apply this to PoE port. |
| ES-3024GP-00794 | When I enable storm control as attached, it’s not saved correctly and we can’t see unicast storm control item after reboot. |

Opcode V1.2.0.7; Loader V1.0.1.8

| Customer |  |
| --- | --- |
| ES-2024GP-00204 | Support IGMP fast leave and IGMP filter in MAW model |
| ES-3024GP-00772 | LLDP-MED issue “The length of country is 32-32” |
| ES-3024GP-00773 | Add menu “Edit Member by Interface” back to VLAN GUI |

Opcode V1.2.0.6; Loader V1.0.1.8

| Customer |  |
| --- | --- |
| ES-3024GP-00700 | DHCP operation The option 60 field of DHCP request packet uses “LG-Nortel” class. Please changed it to “LG-Ericsson”. |
| ES-3024GP-00705 | DHCP operation When I press “Restart DHCP” at Web, there is no action. |
| ES-3024GP-00732 | [LED]HW cross talk issue - need SW workaround |
| ES-3024GP-00655 | Enhancement:DUT will delay to show "Username:" when booting finished and press "Enter" |
| ES-3024GP-00747 | IGMP issue with Broadcom chipset(Some multicast groups were missing during test. ) |
| NTC |  |
| ES-3024GP-00608 | CLI: in "show version", loader version is misaligned (upgrade loader to 1.0.1.8 in order to fix this misalignment issue) |
| ES-4024GP-00697 | Port Mirror: The mirrored traffic direction mode has the "<cr>" choice. |

Opcode V1.2.0.5; Loader V1.0.1.7

| Customer |  |
| --- | --- |
| ES-3024GP-00740 | Upgrade opcode from v1.1.2.0 (current MP version) to v1.2.0.4 for smart switch. After uploading, we couldn’t access switch. |

Opcode V1.2.0.4; Loader V1.0.1.7

| Customer |  |
| --- | --- |
| ES-3024GP-00673 | WEB: Combine reminding messages of upgrade |
| NTC |  |
| ES-3024GP-00660 | We can't set VLAN membership Mode as Access mode on CLI. |
| ES-3024GP-00634 | Web_UI:IPv6 Configuration max count is incorrect |
| RD |  |
| ES-3024GP-00672 | Some commands have additional option <cr> |
| ES-3024GP-00610 | CLI: at end of "show history", "MORE" is not aligned |

Opcode V1.2.0.3; Loader V1.0.1.7

| Customer |  |
| --- | --- |
| ES-3024GP-00627 | IGMP snooping (Can the switch read a mult-join MC packets). |
| ES-3024GP-00586 | Switches drop normal packets also if the broadcast packets flow over 64kbps since broadcast storm control. |
| ES-3024GP-00597 | EgressRateLimit: output rate of two ports might be different when they have identical egress rate limit config. |
| ES-3024GP-00591 | WEB UI is not reachable after scanning by nessus |
| ES-3024GP-00636 | corrections to grammar/spelling in web ui |

Opcode V1.2.0.2; Loader V1.0.1.7

| Customer |  |
| --- | --- |
| ES-3024GP-00535 | Provide intuitive web GUI (VLAN) |
| ES-3024GP-00511 | Shorten the time of checking if same version upgraded and overwrite |
| ES-3024GP-00513 | Add port 24 as download port |
| ES-3024GP-00574 | Add clear log button on web |
| ES-3024GP-00594 | Add clear button for statistics WEB GUI screen |
| ES-3024GP-00332 | LGN: FR: Fan failure detection and action on PoE switches. |
| ES-3024GP-00333 | LGN: FR: Time-based PoE for power saving. |
| ES-3024GP-00334 | LGN: FR: Port mirroring on the port channel, which acts the monitored or source only. |
| ES-3024GP-00335 | LGN: FR: Support 1000full forced mode on 1000T gigabit copper ports. |
| ES-3024GP-00361 | (DCR) Merge the pop-up window for HTTP/TFTP upgrade progress into the main window. |
| ES-3024GP-00534 | (LGN Enhance) Provide intuitive web GUI (Port Information, Trunk, Reboot/Save Config/Set Factory Default) |
| ES-3024GP-00509 | (LGN Enhance) Change configuration screen of time zone |
| ES-3024GP-00512 | (LGN Enhance) Remove mechanism of button to distinguish host ping from IP ping |
| ES-3024GP-00423 | (LGN Enhance) SMTP logging supports username/password for logon the mail server. |
| ES-3024GP-00514 | (LGN Enhance) Change certificate of HTTPS to LG-Nortel name and change valid date |
| ES-3024GP-00515 | (LGN Enhance) Web log off automatically if someone doesn't use web for 600 seconds |
| ES-3024GP-00548 | Web : Change description of STP from help menu |
| ES-3024GP-00422 | LGN: (DCR) STP: Admin Edge Port: Auto for all ports at default. |
| ES-3024GP-00517 | (LGN Enhance) Change content of showing SFP module information |
| ES-3024GP-00426 | (LGN Enhance) Change default value for ping request at web |
| ES-3024GP-00502 | The max value of broadcast control for FE switch is not proper. |

Opcode V1.1.4.0; Loader V1.0.1.5

| Customer |  |
| --- | --- |
| ES-3024GP-00472 | Web_login: Web session log out auotmatically when another web session manual logouts out. |
| ES-3024GP-00532 | CLI/Web: It could set 127 chars in CLI/web when setting rmon event community, but the description of help page is 32 chars. |
| ES-3024GP-00590 | When System boot (A2 chip), the error message of DEV_SWDRV_CreateTrunk is occurred. |
| ES-3024GP-00595 | (LGE Enhance) Disable broadcast storm control as default configuration. |
| ES-3024GP-00662 | WEB: Modify wording of remote link aggregation capable of LLDP. |

Opcode V1.1.3.0; Loader V1.0.1.5

| Customer |  |
| --- | --- |
| ES-3024GP-00502 | The max value of broadcast control for FE switch is not proper. |
| ES-3024GP-00526 | Authentication Key in AAA Server item is empty even we set the value. |
| ES-3024GP-00543 | System hang when I delete authorization table |
| ES-3024GP-00568 | Web: Change URL of login page from "login_lgn.htm" to "login_ipecs.htm" |

Opcode V1.1.2.0; Loader V1.0.1.5

| Customer |  |
| --- | --- |
| ES-3024GP-00560 | Web: When cursor moved to web site icon, it shows “LG-Nortel”. |

Opcode V1.1.1.0; Loader V1.0.1.5

| Customer |  |
| --- | --- |
| ES-3024GP-00555 | Web: Change logo from LG-Nortel to LG-Ericsson. |
| ES-3024GP-00430 | When you see the first page of sitemap at Web, there is a typo on time item. |
| ES-3024GP-00453 | The error message be printed when show multicast mrouter information. |

Opcode V1.0.1.6; Loader V1.0.1.5

| Customer |  |
| --- | --- |
| ES-3024GP-00449 | Client can’t get multicast packet from certain multicast group address |
| ES-3024GP-00425 | Web : Misspelling in description of Site Map -> Time of web GUI |
| ES-3024GP-00419 | Can't set exec time zero for telnet at Web. |
| ES-3024GP-00418 | If we reboot the commander switch after saving configuration file, we can’t access candidate switch. |
| ES-3024GP-00417 | Show current multicast router’ several times, we see log-out screen |
| ES-3024GP-00413 | When we use clustering, we can’t access some menus of candidate switch. |
| ES-3024GP-00403 | FTP: Fails to do opcode upgrade using ALftp. |
| ES-3024GP-00421 extension | Enhance: Use default runtime file to recover file information for preventing filemapping fail (enhancement for file system, need upgrade loader for all models). Also add “flashtest” command for developer to test flash, need upgrade loader for all models as well. |

Opcode V1.0.1.5; Loader V1.0.1.3

| Customer |  |
| --- | --- |
| ES-3024GP-00402 | FTP: The default "anonymous" username should not be leading in an upper letter in 'A'. ("Anonymous" -> "anonymous") |
| ES-3024GP-00396 | Web “Traffic > Priority > Queue, Cos to DSCP, DSCP to DSCP”, undefined queue weight when a trunk is set for port 1. |

(Deposed) Opcode V1.0.1.4; Loader V1.0.1.3

| Customer |  |
| --- | --- |
| ES-3024GP-00401 | (Enhance) Web “System > File, Auto Upgrade”, add ‘Note: The last character of this URL must be a forward slash (“/”).’. |
| ES-3024GP-00400 | Auto upgrade fails for opcode upgrade while interoperating with some TFTP servers, e.g. Cisco TFTP server v1.1 and 3Com TFTP server. |
| ES-3024GP-00399 | STP: “spanning-tree edge-port auto” is missing in running-config, hence it can't be saved to a configuration file. |
| ES-3024GP-00398 | Web “System > File, Copy”, FTP Upgade does not display the progress status, but TFTP does. However, it functions well. |
| ES-3024GP-00396 | Web “Traffic > Priority > Queue, DSCP to DSCP”, undefined queue weight when a trunk is set for port 1. |
| ES-3024GP-00389 | (DCR) Change the default value to DSCP, instead of CoS, in DSCP trust mode. (Need to boot with “Factory_Default_Config.cfg”) |
| ES-3024GP-00356 | Neither CLI nor Web is able to disable mac-learning status on any ports. (Removed commands - CLI: “(config-if)#[no] mac-learning”, Web: “MAC Address > Learning Status” menu, help as well) |
| ES-3024GP-00353 | (DCR) HTTP/HTTPS mutual exclusion - HTTP enabled and HTTPS disabled at default, and one single is enabled at a same time. |
| ES-3024GP-00347 | (Enhance) Web "System > File, Copy", check option for auto reboot after opcode upgrade completed and automatically set to Startup. |
| ES-2026P-00155 extension | (Enhance) U-Boot supports ‘AUTO’ TFTP download with fixed settings when no bootable runtime image is checked. (Need loader upgrade, Smart only) For smart switches, the followings is fixed… Switch IP address: 192.168.1.10, TFTP server address: 192.168.1.99, Opcode File Name: iPECS_ES-2000_Op.bix, Cable connection port# 1 |

(Deposed) Opcode V1.0.1.3; Loader V1.0.1.2

| Customer |  |
| --- | --- |
| ES-3024GP-00393 | Auto upgrade, on the CLI and Web, does not accept FTP URL, e.g. “ftp://192.168.1.254/”. But TFTP URL is OK. (Managed only) |
| ES-3024GP-00392 | (DCR) Web: Put one-line software (Op/Ldr) version number beneath "iPECS" logo. (ES-3024GP-00346 extension) |
| ES-3024GP-00391 | Web "IP > IPv6 Configuration, Show IPv6 Address", the "Apply" button is confusing and the label should be changed to "Delete" for consistency. (Managed only) |
| ES-3024GP-00389 | (DCR) Change the default value to DSCP, instead of CoS, in DSCP trust mode. (Need to boot with “Factory_Default_Config.cfg”) |
| ES-3024GP-00388 | HTTPS connection will cause memory dump (HTTP_TD/web_proc) on the console, and Web management is lost afterward. |
| ES-3024GP-00387 | Time-based ACL: Absolute time doesn't properly work in some configuration. |
| ES-3024GP-00378 | Web "VLAN > Static, Edit Member by Interface Range, Membership Type: None", fails to remove VLAN membership for ES-3000 switches. (Managed running v1.0.1.2 only) |
| ES-3024GP-00375 | Web “IP > General > Ping”, pinging with the IP address, e.g. 72.14.213.104 of www.google.com, will cause memory dump (HTTP_TD/web_proc) on the console, and Web management is lost afterward. |
| ES-3024GP-00374 | Web “IP > General > Ping”, the ping response time is always 0 ms for a defined host. |
| ES-3024GP-00373 | Web “IP > General > Ping”, still gets response in 0 ms while pinging a defined host being blocked for ping response, e.g. www.g.naver.com (222.122.195.6). But pinging its IP address correctly sees timeout. |
| ES-3024GP-00371 | Web “IP > General > Ping”, pinging a host gets no warning messages for “unknown host” when no DNS servers at all. |
| ES-3024GP-00356 | Neither CLI nor Web is able to disable mac-learning status on any ports. (Removed commands - CLI: “(config-if)#[no] mac-learning”, Web: “MAC Address > Learning Status” menu) |
| NTC |  |
| ES-3024GP-00385 | At an attempt to set static IP address with invalid netmask 0.0.0.0, it gets failure message, but this invalid address causes ping loss. |
| ES-3024GP-00384 | CLI: Looks successful to set static IP address with invalid netmask 0.0.255.255, and it is converted to netmask 255.255.0.0 automatically. |
| ES-3024GP-00383 | Ping loss on changing the switch’s IP address via SNMP. And 2 IP addresses are seen if a new one is set via console. |
| ES-3024GP-00382 | IGMPSNP: When a client sends Leave Report, multicast data will be flooding to all ports. |
| RD |  |
| ES-3024GP-00386 | When switch’s IP address is changed, existing Telnet sessions remain and new connections can’t be established. |
| ES-3024GP-00376 | Same issue as ES-3024GP-00388. |

Note: Code upgrade to (Loader/Opcode V1.0.1.2) at a run-time

a. First of all, please consider to backup the startup configuration file by admin user (CLI "copy file tftp"). Or it will be gone at the 1st reload followingly. b. Use TFTP or HTTP to do code upgrade to (Loader V1.0.1.2 and Opcode V1.0.1.2) at runtime in old version, set startup to Opcode V1.0.1.2, delete the old one, and then reload the switch system. c. Required for the 2nd download Opcode V1.0.1.2. E.X. Managed switches support dual runtime images. There may be following 3 cases happening at the 1st reload in Loader V1.0.1.2. (It depends on which partition, 7 'runtime1' or 8 'runtime2', the startup runtime is saved.) c.1. Boot with the new Opcode V1.0.1.2. In this case, after it successfully boots, none of the runtime image files can be seen in the file system (CLI "dir"). Therefore, it is required to do the 2nd TFTP or HTTP download Opcode V1.0.1.2 and set it startup. And, in order to get Opcode V1.0.1.2 in operation, it may be necessary to have the 2nd reload. c.2. Can't boot, and display error messages on the console... [Error] No valid runtime image(s) to boot. Trying to download the runtime image from a TFTP server or Modem... Needs to TFTP download Opcode V1.0.1.2 again on Loader V1.0.1.2's backdoor via console. (Required to save it to partition 7 'runtime1'.) After that, quit the loader backdoor and boot in Opcode V1.0.1.2.

Opcode V1.0.1.2; Loader V1.0.1.2

| Customer |  |
| --- | --- |
| ES-3024GP-00360 | Web "Administration > PoE, Power Allocation", help messages do not provide the default value of high-power ports 1-6. (Default: 30000 milliwatts) (PoE Only) |
| ES-3024GP-00359 | Web improperly says port type in "1000Base-TX". (Should be "1000Base-T") |
| ES-3024GP-00358 | Web "MAC Address > Dynamic, Aging Time", help messages provide an incorrect range. (Correct range in 10-844 seconds) |
| ES-3024GP-00352 ES-3024GP-00351 | When a port is startup configured: Hybrid mode and VLAN 1 & 2(Natvie), it will not join VLAN 1 after booting. |
| ES-3024GP-00350 | GE link failure, e.g. Nortel WSS, when power saving is enabled (default) (GE port only) |
| ES-3024GP-00349 | (Enhance) Web "VLAN > Static, Edit Member by Interface Range": Insert PVID column when Hybrid mode and Untagged is selected. |
| ES-3024GP-00348 | (Enhance) Web "Spanning Tree > STA, Configure Interface": Check option to enable Admin Edge on all ports. |
| ES-3024GP-00347 | (Enhance) Web "System > File, Copy": Check option for auto reboot after opcode upgrade completed. |
| ES-3024GP-00346 | (DCR) Web: Put software (Op/Ldr) version number beneath “iPECS” logo. |
| ES-3024GP-00344 | Web alerts “invalid data” while adding IPv6 address without specifying the prefix length. |
| ES-2026P-00156 | Web "VLAN > Static, Edit Member by Interface Range, Membership Type: None", IE says page error, and the settings are not applied to ES-2000. (Smart only) |
| RD |  |
| ES-3024GP-00354 | Web access would probably cause exception messages (task SNMP_TD) on the console if "snmp-server host" was ever configured. |
| ES-3024GP-00345 | Web/HTTP connection over IPv6 Link-local address fails. (Managed only) |
| ES-3024GP-00343 | Flash access protection - unlock while programming in progress, and lock while programming finished. (Also need loader upgrade) |
| ES-3024GP-00342 | (Enhance) If the startup runtime image is corrupted, the switch will automatically boot with the other. (Need loader upgrade) |
| ES-3024GP-00341 | (Enhance) Backup "filemapping" table (partition 3 in the boot flash). (Also need loader upgrade) |
| ES-3024GP-00340 | Web: "Spanning Tree > MSTP, Add Member" will cause memory dump (web_proc) on the console. |
| ES-3026P-00100 | CLI "clear arp-cache" causes error messages, but the dynamic ARP cache is actually cleared. |
| ES-3026P-00097 | (Enhance) WebAuth: Can disable session timeout. (Add an option in "0") |
| ES-2026P-00155 | (Enhance) U-Boot supports TFTP download on the Web when no bootable runtime image is checked. (Need loader upgrade, Smart only) |
| ES-2026P-00154 | ES-2000 Smart switches shall not support DNS. (ES-2026P-00149 extension) |

Opcode V1.0.0.1; Loader V1.0.0.8

| Customer |  |
| --- | --- |
| ES-3024GP-00331 | (Enhance) PoE: Replace "Mainpower” with “PoE”, CLI command as well as the output. (PoE only) |
| ES-3024GP-00330 | (Enhance) Web/CLI: Make destination file name identical to the source at default while upgrading a new code. |
| ES-3024GP-00327 | Web: Site Map outlines PoE in Administration category for non-PoE switches. (non-PoE only) |
| ES-3024GP-00326 | (Enhance) Web: to stay on the same port settings page at an auto refresh following the Apply action. |
| ES-3024GP-00324 | Web "Administration > Log > System" has no way to display logging entries in flash. |
| ES-3024GP-00322 | SNTP: The timestamp of logging messages is twice calculated for the time zone setting. |
| ES-3024GP-00309 | DiffServ: policy-map “set ip dscp“ does not work when there is also police metering. |
| ES-2026P-00150 | Web: Need a bigger font size(11->14 px) for those messages on the popup window at code upgrade. |
| ES-2026P-00149 | Web "IP > General > Ping" does not accept pinging with a hostname. |
| NTC |  |
| ES-3024GP-00328 | Cable test inaccuracy. E.X. actual cable length: 50m, test result: 90m. |
| ES-3024GP-00321 | IPSourceGuard: Console displays error messages for rule control failure when entries are binding and filter type {sip ¦ sip-mac} is changed. |
| ES-3024GP-00320 | CLI: Missing “ip source-guard” settings in running-config. |
| ES-3024GP-00315 | ARP inspection may cause console no response. |
| ES-3024GP-00314 ES-2026P-00143 | SNMP: Setting a new IP address will cause console freeze (with “NETCFG_PROC” task exception). |
| ES-3024GP-00313 | Empty configuration file is not blocked prior to file transfer. |
| ES-2026P-00147 | Same issue as ES-3024GP-00322. |
| ES-2026P-00146 | TACACS+: User privilege is always authorized in Guest level when it succeeds in user authentication. |
| ES-2026P-00144 | When an output rate-limit value is set without checking Enabled) via Web, its status will be enabled automatically. However, it does not occur in input rate-limit. |
| RD |  |
| ES-3024GP-00318 | DiffServ: After the action of a policy_map is changed, the “permit” action of an ACE will not work if it follows the one with action – “deny any any”. |
| ES-3024GP-00312 | Web: The port status in Flow Control (on right-top panel) does not fit its operational mode when auto-negotication is enabled. |
| ES-3024GP-00271 | CLI: Many times of <Enter> keystrokes will extend the silent time on login failure (30 seconds and 3 times at default). |
| ES-3026P-00095 | PoE: Software Version is not correct. (Currently “Microsemi SDK V1.0.4”) |
| ES-3026P-00091 | MVR: When join reports are received on Reciever port, they are not forwarded to Source port. |
| ES-3026P-00090 | LLDP PDU carries incorrect IEEE 802.1 organizationally TLV, the port and protocol vlan support flag, in value 1. |
| ES-3026P-00085 | IGMPSNP: Console displays error messages (DEV_AMTRDRV_...) when join reports are received but the groups are not configured in profile. |
| ES-3026P-00078 | At a reload(warmStart), syslog entries in RAM are purged and "coldStart" is logged from the beginning. (Also need loader upgrade) |
| ES-3026P-00076 | RemoteLog: Logging level type has no effect at a reload after it is saved in the startup configuration file. |
| ES-2026P-00152 | Rate-limit /input: The measure value is almost ten times larger than the setting on comboG ports 25&26. (FE only) |

Opcode V1.0.0.0 (Simple reconversion of V0.0.2.5 for Formal Release and Mass Production)

Opcode V0.0.2.5; Loader V1.0.0.7

| Customer |  |
| --- | --- |
| ES-3024GP-00311 | (Enhance) CLI: Remove "rate-limit {input ¦ output}" commands for port channels. |
| ES-3024GP-00310 | (Enhance) Cable Test result: change "Test failed" to "Not supported" for the case that port gets to linkup at speed 100 or 10 Mbps. |
| ES-3024GP-00308 | CLI "show power mainpower" gives a confusing term - "Main Power Consumption". (Managed only) |
| RD |  |
| ES-3026P-00073 | (Enhance) MIB: Update ‘networkAccessPortTable’. (Smart should only have ‘networkAccessPortDynamicVlan’, and Managed should have it additionally.) |
| ES-3026P-00072 | (Enhance) MIB: Remove 'loopbackTestMgt' object group because it is not supported. (Switch Internal Loopback test at runtime) |
| ES-3026P-00071 | (Enhance) Up-to-date Web help. |
| ES-3026P-00070 | SNMP: MIB undefined trap index 96 for network-access link-detection action (missing ‘networkAccessPortLinkDetectionTrap’). (Managed only) |
| ES-3026P-00069 | SNMP: No MIB object to set MAC authentication status per port (missing ‘networkAccessPortMode’). |
| ES-3026P-00068 | Web "Show IPv6 Neighbor Cache" gives a typo in "Stable" for the "Stale" state. |
| ES-2026P-00141 | (Enhance) Eliminate "igmp snooping unsolicited-report-interval" commands on the CLI/Web for the Smart, in which proxy reporting is not supported. (Smart only) |
| Known issues (to be fixed in a next release) |  |
| ES-3024GP-00309 | DiffServ: policy-map “set ip dscp“ does not work when there is also police metering. E.X. policy-map POLICY class CLASS set ip dscp 46 police flow 10000 4000 conform-action transmit violate-action drop |

Opcode V0.0.2.4; Loader V1.0.0.7

| Customer |  |
| --- | --- |
| ES-3024GP-00307 | It fails to delete the default gateway specified by switch users. |
| RD |  |
| ES-3026P-00067 | Cable Test gets incorrect results - "No Cable" for an active link in 100Mbps (shall be "Test Failed"). (V0.0.2.3 only) |

(Deposed) Opcode V0.0.2.3; Loader V1.0.0.7

| Customer |  |
| --- | --- |
| ES-3024GP-00306 | LGN: Web "Interface > Port > Cable Test", add a note that reminds users to wait several seconds and then manually refresh the page to display test results. |
| ES-3024GP-00302 | ARP Cache: The actual timeout is measured around 2 minutes, however, it shall be 1200 seconds at default. |
| ES-3024GP-00301 | Port linkdown is possibly caused by cable dis-/re-connections when power-saving is enabled, no matter if port LED is lighting or not. (GE only) |
| ES-3024GP-00299 | Web "VLAN > Static, Edit Member by Interface" fails to change VLAN membership on Access mode. |
| ES-3024GP-00298 | (DCR) Web "VLAN > Static, Edit Member by Interface Range" default values - Mode: Access, Membership Type: Untagged. |
| ES-3024GP-00295 | Web "IP > General > Ping" to a non-existing host causes Web management loss. |
| ES-2026P-00138 | Web HTTP Upgrade: The file transfer of a runtime image takes a very long time. |
| RD |  |
| ES-3024GP-00300 | (Enhance) Web "Security > Web Authentication, Configure interface", separate "Apply"(Status) and "Re-authenticate"(Host) buttons. |
| ES-3024GP-00296 | (Enhance) CLI "show ip interface" provides an extra Loopback interface. Actually its enough for the management VLAN interface. |
| ES-3026P-00063 | (Enhance) No need for the "POST Result" caption without items on the CLI for Diagnostics Fast mode. (Managed only) |
| ES-3026P-00062 | (Enhance) In order to have better EMI margin for FE models, it is to update MAC Register 0x48 value. (FE only) |
| ES-3026P-00061 | (Enhance) CLI "show interfaces brief" - brief in one line per interface. (Managed only) |
| ES-3026P-00059 | (Enhance) Web "IP > ARP" for ARP Cache timeout and entries. |
| ES-3026P-00058 | SNMP: Some access issues in mcastStormMgt(1), bcastStormMgt(3), and unknownUcastStormMgt(4) of stormMgt(privateMibRoot.1.33) |
| ES-2026P-00140 | (Enhance) Eliminate those remaining GVRP commands on the CLI/Web for the Smart, in which GVRP is not supported. (Smart only) |

Opcode V0.0.2.2; Loader V1.0.0.7

| Customer |  |
| --- | --- |
| ES-3024GP-00293 | (Enhance) Web "System > Reset" provides the time to wait for system reboot. |
| ES-3024GP-00290 | Remove the forced "speed-duplex 1000full" option on the CLI and Web. |
| ES-3024GP-00289 | (DCR) QoS: DSCP 46 maps to (PHB:6, Precedence:0) at factory defaults |
| ES-3024GP-00288 | (DCR) QoS: Queue 0,1,2,3 maps to weight 1,2,4,SPQ at factory defaults |
| ES-3024GP-00287 | Receiving VRRP packets causes ping loss. |
| ES-3024GP-00284 | PoE: Port7-24 maximum allocation power range description is wrong. |
| ES-3024GP-00283 | (Enhance) VLAN: switchport access mode - one single untagged, and “switchport mode access” at factory defaults. |
| ES-3024GP-00282 | (DCR) IP Clustering: disabled, and VLAN: switchport access mode, at factory defaults. |
| ES-3024GP-00263 | PoE port maximum allocation can be configured more than 15400 mW on port# else 1-6. |
| ES-2026P-00139 | Remove unsupported items out of Web Site Map. (Smart only) |
| ES-2026P-00137 | (Enhance) Progress for HTTP/TFTP Upgrade on the Web |
| ES-2026P-00135 | (UpdatedSpec) Support Voice VLAN in Smart switches. (Smart only) |
| RD |  |
| ES-3024GP-00292 | Loader should alert errors, instead of warnings, on detecting incorrect HWINFO checksum. (Loader issue) |
| ES-3026P-00052 | Remove the obsolete CLI command: "...(config-if)#spanning-tree portfast" (Managed only) |
| ES-3026P-00051 | LLDP: System memory corruption messages on the console while LLDP interop with a 3rd party device(Netgear). |
| ES-2024GP-00002 | Loader provides incorrect Product description for the Smart on the console. (Smart only)(Loader issue) |
| Known issues (to be fixed in a next release) |  |
| ES-2026P-00138 | Web HTTP Upgrade: The file transfer of a runtime image takes a very long time. |

Opcode V0.0.2.1; Loader V1.0.0.6

| RD |  |
| --- | --- |
| ES-3026P-00050 | (Enhance) Remove booting messages for "insmod module" and "CPSS". |
| ES-3026P-00046 | (Enhance) Dump stack frame and log messages on task exception |

(Deposed) Opcode V0.0.2.0; Loader V1.0.0.6

| Customer |  |
| --- | --- |
| ES-3024GP-00280 | QoS: PHB-to-Queue mapping incorrectly get Queue 0 for PHB 4-7. |
| ES-3024GP-00278 | (Enhance) “show memory” - Used plus Free will be equal to the Total DRAM size on the board (128MB, i.e. 134217728 Bytes) |
| ES-3024GP-00276 | CLI "show lldp info remote detail port-channel " for a non-exiting trunk_id causes GVRP exception. |
| ES-3024GP-00273 | (Enhance) TCAM utilization - additional Entries Used by System and User. (ES-3024GP-00201 extension) |
| ES-3024GP-00269 | MAC-based VLAN::host#1 can't ping to Port-based VLAN::host#2 - ARP forwarded incorrectly. |
| ES-3024GP-00268 | May not get ping responses when ARP inspection is enabled globally. |
| ES-3024GP-00267 | CLI “running-config” misses "no power-save"(disabled), on which its enabled at default. |
| ES-3024GP-00266 | (DCR) IGMP snooping is disabled at factory defaults. |
| ES-3024GP-00265 | (DCR) WebGUI: Move "Contact Us" button to the right-most among the four and make it grayed. |
| ES-3024GP-00264 | (Enhance) Telnet/SSH: Wait for 30s silent time before it prompts user login in the next iteration. |
| ES-3024GP-00263 | PoE port maximum allocation can be configured more than 15400 mW on port# else 1-6. |
| ES-3024GP-00246 | (UpdatedSpec) When PoE button is pressed, System DIAG/POE LED is lighting in amber. (Smart PoE only) |
| ES-2026P-00133 | Web takes around 5 minutes to come out "Cluster Operate : Member <member_id>" page. (Pulled in 20-25 seconds.) |
| ES-2026P-00131 | Press PoE button has no effect on FE PoE model - System DIAG/POE LED still lighting in green. (Smart PoE only) |
| ES-2026P-00130 | LGN: RSTP: Admin Edge Port is actually disabled, but the Oper is found enabled, on linkdown ports. (Smart only) |
| ES-2026P-00128 | (Enhance) Up-to-date Web help, Site Map also. (Smart only) |
| RD |  |
| ES-3026P-00048 | VLAN trunking can't be configured on FE ports. |
| ES-3026P-00046 | (Enhance) Dump stack frame and log messages on task exception |
| ES-3026P-00043 | (Enhance) Support RFC3621::POWER-ETHERNET-MIB, PoE traps also. |
| ES-3026P-00038 | CLI: "dir" output clarified - "Free space for compressed user config files". (Original: "Total free space") (Managed only) |

Opcode V0.0.1.5; Loader V1.0.0.6

| Customer |  |
| --- | --- |
| ES-3024GP-00262 | LGN: PoE does not afford power to any PDs. (V0.0.1.4 only) |
| ES-3024GP-00246 | (UpdatedSpec) When PoE button is pressed, System DIAG/POE LED is lighting in amber. (PoE only) |

(Deposed) Opcode V0.0.1.4; Loader V1.0.0.6

| Customer |  |
| --- | --- |
| ES-3024GP-00258 | (Enhance) Provide super user for emergent login in case of forgetting password. |
| ES-3024GP-00257 | Voice VLAN remains on the port when it is dynamically joined then cable disconnected. (Managed only) |
| ES-3024GP-00252 | (Enhance) QoS on Voice VLAN - Remarking DSCP 46 and L2 Priority 6 by default. (Managed only) |
| ES-3024GP-00248 | It fails to set output rate-limiting less than 1000 Kbps on the CLI and Web. |
| ES-3024GP-00246 | (UpdatedSpec) When PoE button is pressed, System DIAG/POE LED is lighting in amber. (PoE only) (24GE-PoE NG) |
| ES-3024GP-00238 | It will cause ping loss while performing cable diagnostics on a comboG port that is not ever cable connected. (Console -> freeze, Web/Telnet -> mgmt loss) |
| ES-3024GP-00238 | Console suddenly displays memory dump when igmp snooping querier is enabled. (ports snake connected and traffic forwarding thru all ports within vlan domains) |
| ES-3024GP-00235 | (Enhance) DSCP marking/remarking |
| ES-3024GP-00234 | WebGUI does show memory utilization, like CLI "show memory" |
| ES-3024GP-00219 | (Remove the CLI) Failed to do internal loopback test on ethernet ports. |
| ES-2026P-00125 | Add syslog Web pages for Smart models. (Smart only) |
| RD |  |
| ES-3024GP-00260 | (Enhance) Voice VLAN: Show “Remaining Age (minutes)” per port on the UI. |
| ES-3024GP-00259 | (DesignChange) Need "show cable-diagnostics" to see the test results of "test cable-diagnostics", and to remove the following "tdr". |
| ES-3024GP-00234 | (Enhance) Up-to-date Web help, Site Map also. (Managed only) |
| Known issues (to be fixed in a next release) |  |
| ES-2026P-00128 | (Enhance) Up-to-date Web help, Site Map also. (Smart only) |

Opcode V0.0.1.3; Loader V1.0.0.6

| Customer |  |
| --- | --- |
| ES-3024GP-00232 | WebGUI will see garbage characters shown in the User Name column when 'login' button is clicked. |
| ES-3024GP-00231 | LGN: WebGUI user login needs to wait a bit long time for the whole web screen to come out. |
| ES-3024GP-00230 | (DCR) Fixed "PoE Maximum Available Power" in 195W. (PoE only) |
| ES-3024GP-00225 | (DCR) Change the CLI headline to prompt user login at the first time - “iPECS Ethernet Switch Administration". (Managed only) |
| ES-3024GP-00224 | (DCR) CLI: Wait for 30s silent time before it prompts user login in the next iteration. (Managed only) |
| ES-3024GP-00223 | When upgrade firmware from http upgrade, the web page can not be shown or show failed message. |
| ES-3024GP-00222 | QoS: Failed to get back to normal weights after setting strick queue(s). |
| ES-3024GP-00206 | LLDP: System capabilities (SystemCapSupported, SystemCapEnabled) are incorrect for “Router”. |
| ES-3024GP-00202 | SNMP: Can't get RFC1213-MIB::ifDescr(1.3.6.1.2.1.2.2.1.2) value after interface description is configured on the CLI/Web. (Actually, since RFC2863::IF-MIB::ifDescr(1.3.6.1.2.1.2.2.1.2) is a read-only object, ethernet interface's description is set to 'portName'(privateMibRoot.1.2.1.1.2).) |
| ES-3024GP-00201 | (Enhance) Show total/free Policy Control Entries and TCAM utilization of Marvell xCat on the CLI/Web/SNMP. |
| ES-3024GP-00200 | (DCR) Set default IP to 192.168.1.10/24 for managed switch also. (Managed only) |
| ES-3024GP-00173 | LGN: Poor sw system performance - long response time for continual ping. (Reduced to ~4.5ms average in v0.0.1.2) (Further reduced to ~4.0ms average in v0.0.1.3) |
| ES-3024GP-00172 | High CPU utilization, especially seen up to 90% on PoE devices in v0.0.1.0. (Reduced to ~40% in v0.0.1.2)(Further reduced to ~18% in v0.0.1.3) |
| ES-3026P-00037 | FE ports look configurable status for power saving on the Web and CLI. (26FE[-PoE] only) |
| ES-3026P-00035 | PoE status is reversed between upper and lower ports, e.g. PD connected to port 1(2) but its status shown on port 2(1). (26FE-PoE only) |
| ES-3026P-00034 | When PoE button is pressed, all port LEDs are lighting in amber even though no PDs are connected. (26FE-PoE only) |
| ES-2026P-00123 | (Enhance) Refine Web panel drawing to look like real devices of all 8 models. |
| ES-2026P-00122 | While the Smart is booting, it can see "sflash_init ERROR: SFlash init falied! Flash: 16 MB" (on the console). (Loader issue)(Smart only) |
| ES-2026P-00120 | (Enhance) Support Loader upgrade at runtime, Smart as well as Managed. |
| ES-2026P-00118 | WebGUI "Interface > Port > General", "Configure by Port List" action will get to Page Not Found on the browser window. |
| ES-2026P-00117 | (UpdatedSpec) Support IP clustering in Smart switches (Smart only) |
| Known issues (to be fixed in a next release) |  |
| ES-3024GP-00235 | (Enhance) DSCP marking/remarking. |
| ES-3024GP-00234 | WebGUI does show memory utilization, like CLI "show memory". |

Opcode V0.0.1.2; Loader V1.0.0.5

| Customer |  |
| --- | --- |
| ES-3024GP-00194 | IP clustering shall have up to 36 members. (Managed only) |
| ES-3024GP-00193 | Traffic segmentation can't be disabled for uplink/downlink binding. (Managed only) |
| ES-3024GP-00191 | Remove 'Accton' wording on booting messages. |
| ES-3024GP-00189 | CLI takes up to 30sec to see "show running-config" output on PoE switches. (Managed PoE only) |
| ES-3024GP-00188 | (DCR) CLI's default prompt wishes the actual product name, instead of "Console" at present. (Managed only) |
| ES-3024GP-00176 | "iPECS', the leading letter 'i' shall be in lowercase. |
| ES-3024GP-00175 | Speed booting process - currently measured around 3min, and expected within 1min. (Pulled in around 1 minute, 55s to 1m15s, in v0.0.1.2) |
| ES-3024GP-00174 | Poor sw system performance - very long time for TFTP/HTTP upgrade runtime image. (V0.0.1.0 and lower) |
| ES-3024GP-00173 | LGN: Poor sw system performance - long response time for continual ping. (Reduced to average ~4.5ms in v0.0.1.2) |
| ES-3024GP-00172 | High CPU utilization, especially seen up to 90% on PoE devices in v0.0.1.0. (Reduced to ~40% in v0.0.1.2) |

Known issueS AND Limitations

| ES-3024GP-00325 | <Syslog in flash> Software limitation: In either one of following three conditions, syslog entries will be written to flash. 1) update in one hour period when flash db is not full (Maximum Syslog Entries: 512) 2) update each new syslog entry to flash db when it is full (Least Recently First Overwritten) 3) switch reload, or system warm start (first need to save “logging history flash <level>” in running-config to the startup configuration file) |
| --- | --- |
| ES-3024GP-00328 | <Cable Test on Power Saving> Chip limitation: Power saving will result in highly inaccurate cable length at test/measure. Recommended to disable power saving on the link partner then perform cable test. |
| ES-3024GP-00316 | <RSPAN> Software limitation: In order to set RSPAN a destination port and destination/source uplink ports, first set the switchport mode to Hybrid (Default: Access). |
|  | <Voice VLAN> Software limitation: In order to set Voice VLAN mode {auto ¦ manual} on ethernet ports, first set the switchport mode to Hybrid (Default: Access). |
|  | <Port Mirroring> Software limitation: Port mirroring is only supported on ethernet ports, but not on port channels. |
| ES-3024GP-00088 | <1000Base-T 1000full.> Software limitation: 1000T copper ports do NOT support forced speed-duplex 1000full. 1000full is only operational in auto-negotiation mode on either 1000T copper or 1000SFP fiber ports. |
| ES-3024GP-00255 | <PoE port priority at exceeding Power Budget> Hardware limitation: For ports in a same priority, the actual priority is checked by physical PSE port#, and no changes based on front port# can be made by software. And the powering order of PSE port#: 0 > 1 > ... > 12 > 13 > ... > 21 > 22 > 23. Rule 1. Ports on the 1st PD69012 chip > Ports on the 2nd PD69012 chip Rule 2. Ports on an individual PD69012 are ordered from right to left and then from top to down. Currently, in a same priority, the powering order of all PoE ports is as follows. FE: 11 > 12 > 9 > 10 > 7 > 8 > 5 > 6 > 3 > 4 > 1 > 2 > 23 > 24 > 21 > 22 > 19 > 20 > 17 > 18 > 15 > 16 > 13 > 14 GE: 12 > 11 > 10 > 9 > 8 > 7 > 6 > 5 > 4 > 3 > 2 > 1 > 24 > 23 > 22 > 21 > 20 > 19 > 18 > 17 > 16 > 15 > 14 > 13 |
| ES-3024GP-00215 | <ComboG Port Type> Software limitation: ComboG's Port Type (FE: 25,26, GE: 21-24) is set to 1000T by default. And then, on module insertion, it is determined ‘SFP' (1000Base-SX,LX,LH, 100base-FX). |
| ES-3024GP-00209 | <Jumbo Frame> Chip limitation: When jumbo frame function is enabled, RMON-MIB:: etherStatsPkts1024to1518Octets will also count jumbo frames of which size up to 10K bytes. (MVL xCat "Frames1024toMaxOctects" counter) |
|  | <ACL> Chip: 512 Policy Control Entries (256 avail in extended mode that is utilized by sw) SwSystem: 32 ACEs per ACL, 64 ACLs per system *When binding an ACL to a port, each ACE rule in an ACL will use two PCE entries. **ACL is only supported on ethernet ports, but not on port channels. |
|  | <VLAN> Chip behavior: VLAN Priority: MAC-based > IP Subnet-based > Protocol-based > Port-based, (Voice VLAN). *Traffic segmentation cares nothing about VLAN domains, and all traffic among downlink ports will be blocked. |
|  | <Loopback Detection w/o STP> Chip limitation: Alleycat does not support hardware loop detection. |
| ES-3024GP-00148 | <Traffic Segmentation> Chip limitation: Marvell chipsets cannot support multiple uplink ports on a session. Software limitation: Btw, though the chip can support multiple oneUplink-to-manyDownlink sessions, one single session is implemented so far. |
| ES-3024GP-00072 | <IGMPSNPv3 ASM> Chip limitation: Alleycat and hence CPU only recognizes IGMP V3 report without src_ip_list. |
|  | <Green Ethernet, or Power Saving> Chip limitation: Linkup: GE port only, speed 1000Mbps, limited line length < 60m |
|  | <Cable Diagnostics> Chip limitation: RJ-45 copper connections only, Linkdown: FE & GE ports without link partners, deviation in +/-2m. Linkup: GE port only, speed 1000Mbps, deviation in +/-10m or more. (The cable test is not supported for port linkup at speed 100 or 10 Mbps.) |
|  | <Rate Control> Chip limitation: One single rate-limiting value for ingress rate limit and storm controls, including broadcast, multicast, and unknown unicast. However, individual status. Software limitation: Input/Output rate-limitings are only supported on ethernet ports, but not on port channels. |

Any other problems than those listed above should be reported to our Technical Support Staff.

IEEE Standards Support

| Standard | Title |
| --- | --- |
| 802.1D | Transparent Bridging Specifications (ISO/IEC 10038) |
| 802.1p | Traffic Class Expediting and Dynamic Multicast Filtering |
| 802.1Q | Virtual Bridged Local Area Networks |
| 802.1w | Rapid Spanning Tree |
| 802.1X | Network Access Control |
| 802.2 | Local Area Networks, Logical Link Control (LLC) |
| 802.3 | Ethernet CSMA/CD 9 (ISO/IEC 8802-3) |
| 802.3ab | 1000Base-TX |
| 802.3ad | Link Aggregation |
| 802.3af | Power over Ethernet |
| 802.3i | 10Base-T (ISO/IEC 8802-3, clause 14) |
| 802.3u | 100Base-TX (ISO/IEC 8802-3, clause 25) |
| 802.3x | Flow Control |
| 802.3z | 1000Base-SX, 1000Base-LX |

IETF Standards MIB Support

| RFC No. | # Title | Groups Supported |
| --- | --- | --- |

Private Enterprise MIB Support

| Title | #### Last Updated | #### File Size |
| --- | --- | --- |
| iPECS_ES-2000.mib | 2010/01/13 | 382,756 |

SNMP Trap Support

| RFC No. | Title |
| --- | --- |
| 1215 | coldStart (v1) warmStart (v1) linkDown (v1) linkUp (v1) authenticationFailure (v1) |
| 1907 | coldStart (v2c) warmStart (v2c) authenticationFailure (v2c) |
| 2863 | linkDown (v2c) linkUp (v2c) |
| 1493 | newRoot (v1) topologyChange (v1) |
| 1757 | risingAlarm (v1) fallingAlarm (v1) |
| 2819 | risingAlarm (v2c) fallingAlarm (v2c) |

Open Source Disclosure

Following open source software codes developed by third parties are subject to GNU General Public License version 2 (GPL v2) or GNU Lesser General Public License (LGPL). And they are accessible on Web site: http://www.lg-nortel.com:

- U-Boot 1.3.2	(GPL v2)

- Linux kernel 2.6.22.18	(GPL v2)

- busybox	(GPL v2)

- glibc v2.5	(LGPL)

Other open source software codes include

- DHCP incorporates the library from the Internet Software Consortium.

- SNMP incorporates the library netsnmp

- SSH incorporate the library openssh

- TACACS+ incorporates the library libtacplus

- RADIUS incorporates the library radiusclient

- TFTP incorporates the library tftp-hpa

Customer Support

By Phone:

By Email:

By Web:

By Fax: