### SYSTEM WIRING

### Battery Back-Up Wiring

The system can be equipped with to provide operation from external batteries if commercial AC power fails. The Back-Up batteries are connected to the connector on the front of the PSU through the battery cable, which contains two ring terminals and a 4-pin female connector.

LDK-300

PSU

FUSE

20A

RED

BLUE

LDK-300 PSU

Front Panel

Figure 3.5.1	Battery Back-up wiring

* Note: The cable for the battery connection is supplied with KSU in the same package.

The external batteries must provide 24 Volts DC. This is generally accomplished by connecting two 12 volt batteries in a series arrangement. Operation on batteries is controlled by the PSU. This PSU will provide charging current to the batteries during normal AC power operation at a maximum of about 500 mA. During battery operation, the battery operation of PSU will be stop if the AC power re-applied or the battery voltage is too low to maintain proper system operation.

If a low-battery cut-off occurs and a new battery is installed, it is necessary to reset the Battery Back-Up circuit in PSU manually by momentarily pressing the switch marked as BATT. RESTART located on the upper side of PSU (in only PSU3).

The length of time that the system will operate on the batteries is dependent on several elements including, battery charging state, condition of the batteries, capacity of the batteries, and the size of the system (number of station ports). The chart below gives the approximate back-up time for several system sizes and different battery capacities in ampere-hours.

| Battery Capacity | DKTU 24 Ports | DKTU 72 ports | DKTU 120 ports |
| --- | --- | --- | --- |
| 20AH | 6 hours | 3 hours | 1.5 Hours |
| 40AH | 12 hours | 6 hours | 3 hours |

Table 3.5.1 Battery capacity

### RS-232C WIRING on MPB and SIU

MPB contains two standard RS-232C interface and SIU on MISB contains two standard RS-232C interfaces. RS- 232C is connected by 9-pin “D” connector as shown in Figure 3.5.2. Wiring connector for RS-232C must include the Clear to Send(CTS) lead if it is enabled by Switch position 2 of the MPB. It is recommended that the communication setting is 19200 BPS, 8 bits, no parity and 1 stop bit. But cn4 for smc1 port dose not support the RTS,CTS signal.

Cable for the connection with Terminal(DTE) is not supplied by LGIC because of getting it easily.

For various application, Administration Programming is required according to the desired application.

2

3

4

6

7

8

5

TD

RD

DSR

DTR

CTS

RTS

SG

RD

TD

DTR

DSR

RTS

CTS

SG

2

3

4

6

7

8

5

3

2

20

6

4

5

7

TD

RD

DSR

DTR

CTS

RTS

SG

RD

TD

DTR

DSR

RTS

CTS

SG

2

3

4

6

7

8

5

DB-9 connector

for RS-232C

on Terminal

DB-25

for RS-232C

on Terminal

Name	Function

TD	Transmitted Data

RD	Received Data

RTS	Request To Send

CTS	Clear To Send

DTR	Data Terminal Ready

DSR	Data Set Ready

SG	Signal Ground

DB-9 connector

for RS-232C of

CN3 on MPB

and SIU on MISB

DB-9 connector

for RS-232C of

CN3 on MPB

and SIU on MISB

Figure 3.5.2	Pin assignments and wiring of DB25 and DB9 for RS-232C

- Note:

- Protective Ground (Frame Ground) may be used between DTE and DCE(GDK system) for more stable communication. And it is recommended.

In DTEs and DCEs, protective ground (frame ground) is a point which is electrically bonded to the equipment frame. Protective ground (frame ground) is not an interchange circuit in RS232 standard.(EIA/TIA-232-F) If bonding of the equipment frames of the DCE and the DTE is necessary, a separate conductor should be used which confirms to the appropriate national or local electrical codes.

### MISB wiring

MISB provides connections for 2 music sources, 2 external page ports, 4 relay contact controls, and an alarm/door-bell input monitor through RJ21 type female connector. Table 3.5.2 shows the contact assignments of RJ21 type connector on MISB.

The cable with RJ21 type male connector is required to connect with terminal or external devices. It is possible to get the cable locally, or use GDK-162 5MC2 supplied by LGIC on the order if necessary.

External Music Source wiring

LDK-300 MISB is capable of accommodating up to 2 ports of External Music Sources through 2 pairs of a RJ21 type connector, CHAMP1 on the board.

MOH1: Pin No. 11 and Pin No. 36

MOH2: Pin No. 12 and Pin No. 37

The level can be adjusted by the administration programming (Program 66) in 0.5 dB steps to ±20 dB of the input. Note, if the output of the source is too high or low, it will be necessary to lower or raise the output level control at the source.

The above External Music sources including an Internal Music Source on MPB are selectable by Admin Programming. See Programming Manual.

External Paging wiring

LDK-300 MISB supports up to 2 External Paging ports through 2 pairs of a RJ21 type connector, CHAMP1 on the board.

MOH1: Pin No. 13 and Pin No. 38

MOH1: Pin No. 14 and Pin No. 39

The above External Music sources are selectable by Admin Programming.

External DC

Power Supply

External

Amplifier

Ext. paging

output

MDF

LDK-300 MISB

RJ21 type connector, CHAMP1

1

26

13

38

Figure 3.5.3.aAn example of External paging wiring

Relay contacts for general purpose

LDK-300 MISB supports up to 4 Relay contacts, which can be used for the functions of Loud Control, Door Open and so on under the software control. These relay contacts are respectively connected with external equipment through 4 pairs of a RJ21 type connector, CHAMP1 on the board.

RLY1: Pin No. 1 and Pin No. 26

RLY2: Pin No. 2 and Pin No. 27

RLY3: Pin No. 3 and Pin No. 28

RLY4: Pin No. 4 and Pin No. 29

External DC

Power Supply

MDF

LDK-300 MISB

RJ21 type connector, CHAMP1

2

27

Figure 3.5.3.bAn example of Loud Bell wiring

Alarm Detection wiring

LDK-300 MISB provides an external alarm detection input, which can be used to notify to extensions when the external switch is on/off. This alarm detection input is provided through a pair of a RJ21 type connector, CHAMP1 on the board.

ALARM: Pin No. 18 and Pin No. 48

Close or open detection is programmable by Admin. Programming.

MDF

LDK-300 MISB

RJ21 type connector, CHAMP1

18

43

External Switch

Figure 3.5.3.c	An example of External Switch wiring

Table 3.5.2 shows the contact assignments of RJ21 type female connector on MISB.

| Punch-Down | MISB connector |  |  | MISB |  |
| --- | --- | --- | --- | --- | --- |
| Block Pin | Pair | Pin | Color-Code | Designation |  |
| 1 | 1 | 26 | WH/BL | RLY1-R |  |
| 2 |  | 1 | BL/WH | RLY1-T |  |
| 3 | 2 | 27 | WH/OR | RLY2-R |  |
| 4 |  | 2 | OR/WH | RLY2-T |  |
| 5 | 3 | 28 | WH/GN | RLY3-R |  |
| 6 |  | 3 | GN/WH | RLY3-T |  |
| 7 | 4 | 29 | WH/BN | RLY4-R |  |
| 8 |  | 4 | BN/WH | RLY4-T |  |
| 9 | 5 | 30 | WH/SL |  |  |
| 10 |  | 5 | SL/WH |  |  |
| 11 | 6 | 31 | RD/BL |  |  |
| 12 |  | 6 | BL/RD |  |  |
| 13 | 7 | 32 | RD/OR |  |  |
| 14 |  | 7 | OR/RD |  |  |
| 15 | 8 | 33 | RD/GN |  |  |
| 16 |  | 8 | GN/RD |  |  |
| 17 | 9 | 34 | RD/BN |  |  |
| 18 |  | 9 | BN/RD |  |  |
| 19 | 10 | 35 | RD/SL |  |  |
| 20 |  | 10 | SL/RD |  |  |
| 21 | 11 | 36 | BK/BL | MOH1-R |  |
| 22 |  | 11 | BL/BK | MOH1-T |  |
| 23 | 12 | 37 | BK/OR | MOH2-R |  |
| 24 |  | 12 | OR/BK | MOH2-T |  |
| 25 | 13 | 38 | BK/GN | EXP1-R |  |
| 26 |  | 13 | GN/BK | EXP1-T |  |
| 27 | 14 | 39 | BK/BN | EXP2-R |  |
| 28 |  | 14 | BN/BK | EXP2-T |  |
| 29 | 15 | 40 | BK/SL |  |  |
| 30 |  | 15 | SL/BK |  |  |
| 31 | 16 | 41 | YL/BL |  |  |
| 32 |  | 16 | BL/YL |  |  |
| 33 | 17 | 42 | YL/OR |  |  |
| 34 |  | 17 | OR/YL |  |  |
| 35 | 18 | 43 | YL/GN | ALARM-R |  |
| 36 |  | 18 | GN/YL | ALARM-T |  |
| 37 | 19 | 44 | YL/BN |  |  |
| 38 |  | 19 | BN/YL |  |  |
| 39 | 20 | 45 | YL/SL |  |  |
| 40 |  | 20 | SL/YL |  | BL:BLUE |
| 41 | 21 | 46 | VI/BL |  | BK:BLACK |
| 42 |  | 21 | BL/VI |  | BN:BROWN |
| 43 | 22 | 47 | VI/OR |  | OR:ORANGE |
| 44 |  | 22 | OR/VI |  | WH:WHITE |
| 45 | 23 | 48 | VI/GN |  | GN:GREEN |
| 46 |  | 23 | GN/VI |  | SL: SLIVER |
| 47 | 24 | 49 | VI/BN |  | VI:VIOLET |
| 48 |  | 24 | BN/VI |  | RD:RED |
| 49 | 25 | 50 | VI/SL |  | YL:YELLOW |
| 50 |  | 25 | SL/VI |  |  |

Table 3.5.2 MISB Wiring

### Extension Board wiring

Extension boards such as DTIB12, DTIB24, ETIB, DSIB, SLIB24, SLIB48 and SLIB II contains a RJ21 type female connector for station wiring to each interface circuit on each board.

But WTIB and WTIU, Wireless Terminal Interface Board/Unit for DECT contains RJ11 type jacks. (refer to the section 5)

The following provides details on the interconnection between each type of Extension Board and the jack on the station.

### Digital Keyset and Terminal wiring

Wiring from digital terminal interface board such as DTIB12, DTIB24 and DSIB to station jack requires one pair of wire. The power for a station, PCM data and signaling between board and station are transmitted over this pair.

Contact assignments of RJ11 type jack in Digital Terminals

Figure 3.5.4.a shows the contact assignments of the miniature 6-position jack (RJ11 type jack) in LG proprietary digital terminals. Two wires from the digital terminal interface of the system are basically connected to the contact numbers 3 (R) and 4(T).

But two additional contacts, R1 and T1 are provided in the digital terminal as shown in Figure 3.5.4 The contact number 3 (R) and 2 (R1) are connected together in the digital terminals, and also the contact number 4 (T) and 5 (T1) are connected together. Because there is a country in which the contact number 2 and 5 are used for the 2-wire connection method according to his local regulation. So LGIC provide two more contacts, R1 and T1 in the digital terminals so that the special customer can use the line cord, which is in wide use in his country.

But only 2-wire connection should be recommended to get better transmission characteristics between digital terminal interface of the system and digital terminals.

T

T1

R

R1

1

2

3

4

5

6

Figure 3.5.4.a	contact assignments of Digital Terminals

Connection method between the digital interfaces and the digital terminals

The cable with 25-pair male connector is required between Digital Terminal Interface board and MDF. It is possible to get the cable locally, or use GDK-162 5MC2 supplied by LGIC on the order if necessary.

The MDF that is in wide use in the country can be locally used according to the requirements of local regulation.

MDF

1

2

3

4

5

6

LDK-300 DTIB12/DTIB24/DSIB

RJ21 type

connector

on the board

1

26

2

27

3

28

4

29

DT1

DR1

DT2

DR2

DT3

DR3

Area A

Area B

Area C

Area D

Area E

Figure 3.5.4..b	Basic connection method

between digital terminal interface of the system and digital terminal

Note:

- Digital terminals operate regardless of line polarity, not with a fixed polarity.

- Each area means the followings,

A: LDK system, B: between the LDK system and MDF, C: cable in building,

D: between Terminal connection point, e.g. terminating adapter and Terminal,

E: digital terminals

- DTIB12 is capable of connecting to up to 12 digital terminals.

DTIB24 is capable of connecting to up to 24 digital terminals.

DSIB is capable of connecting to up to 6 digital terminals.

Connection method with special condition between the digital interfaces and the digital terminals

Only two-wire connection between the digital terminal interface and digital terminals is recommended to reduce the line noise exited from useless wires. Because R and R1/T and T1 are connected together in the terminals for various national conditions, the useless wires can cause the environment noise, which may affect the data transmission, to flow in the digital terminal the 4-wire line cord is used.

In this case, cut or remove the useless wires connected the contact number R1 and T1 (or, R and T) of the terminals at the Terminal connection point, e.g. terminating adapter, so that those are not drawn out any more.

Area A

Area B

Area C

Area D

Area E

MDF

1

2

3

4

5

6

LDK-300 DTIB12/DTIB24/DSIB

RJ21 type

connector

on the board

1

26

2

27

3

28

4

29

DT1

DR1

DT2

DR2

DT3

DR3

1

2

3

4

5

6

Figure 3.5.4..c	Connection method when the 4-wire line cord, or

special connection method with with contact number 2 and 5.

Note:

- Each area means the followings,

A: LDK system, B: between the LDK system and MDF, C: cable in building,

D: between Terminal connection point, e.g. terminating adapter and Terminal,

E: digital terminals

- Only two wires should be connected to digital terminals and Interface Board, so the rest should be removed at MDF or Terminal Connection Point, as X points shown in above figure.

Pin Assignments of RJ21 type jack on DTIB12/DTIB24/DSIB

See the clause 3.5.4.5 contact assignments of RJ21 type connector on extension board.

### Single line Telephone wiring

SLT interface boards such as SLIB24, SLIB48, SLIB II,SLIB4E and DSIB are wired to SLT or standard analog device with a single pair of wire that provides battery feed, voice and signaling from/to the connected SLT. Typical contact assignments of RJ11 type jack, a miniature 6-position jack as following figure 3.5.5.a. But the contact assignments and connector type may be different from following figure in special countries according to the special national condition. Before wiring, it is important to check them.

1

2

3

4

5

6

R

T

Registered Jack

specified in FCC, CFR

Figure 3.5.5. a	contact assignments of SLT

The cable with 25-pair male connector is required between SLT Interface board and MDF. It is possible to get the cable locally, or use GDK-162 5MC2 supplied by LGIC on the order if necessary.

The MDF that is in wide use in the country can be locally used according to the requirements of local regulation.

MDF

1

2

3

4

5

6

1

26

2

27

3

28

4

29

T1

R1

T2

R2

T3

R3

RJ21 type connector  on board

RJ11 socket

on SLT

SLIB24,SLIB48, SLIB II,

DSIB

Note:

- SLT (TE) typically operates with both polarities, since a fixed polarity is not guaranteed according to most of specification.

- SLIB24 and SLIB48 are respectively capable of connecting with up to 6 SLT.

SLIB II is capable of connecting with up to 12 SLTs.

DSIB is capable of connecting with up to 6 SLTs after 7th pair of RJ21 type connector.

Figure 3.5.5.b	wiring between board and analog TE including SLT

### Intercom/Door Phone Box installation

The Intercom/Door Phone Box is intended to be wall mounted. There are two kinds of Intercom/Door Phone Boxes in LDK-300 system available. KD-Digital Phone Box is can be connected to the digital terminal interface.

First, separate the ICM/Door Box from the base plate by inserting a flat thin edged screw driver in the bottom of the product. Press the screw driver inward which will release the retaining tan and twist the screw driver to separate the base plate. Fasten the base plate of the ICM/Door Box to the wall with # 8 screws. Route the cable through the cable entry hole in the base plate and connect the wires to the four screw terminals as shown in the figures 3.5.6. When the wiring is complete, pull any slack wire back through the base plate and push the housing back together.

Connection method between the Digital Interface Board and KD-Digital Phone Box

KD-Digital Phone Box can be only connected to the Digital Interface Board. Wiring from digital terminal interface board to KD-Digital Phone Box requires one pair of wire.

MDF

LDK-300 Digital Interface Board

RJ21 type connector

on the board

1

26

2

27

3

28

4

29

DT1

DR1

DT2

DR2

DT3

DR3

The soldering

side of

KD-Digital Phone Box

GR

RD

Figure 3.5.6 wiring between the board and KD-Digital Phone Box

Note:  KD-Digital Phone Box operates regardless of line polarity, not with a fixed polarity

### MOHU installation

The MOHU (Music On Hold Unit) provides External Music Source to the system by connecting to any type of Single Line Interface including 2B-SLIU30 of the LDK-300 system. The External Music Source provided by Single Line Interface is used as music source of MOH channel for System Hold, Background Music and ICM Box Music.

The MOHU has two connections for 2 external music sources (MOH) and 2 Single Line Interfaces (PORT).

- The ‘PORT 1’ is connected to the ‘MOH 1’

- The ‘PORT 2’ is connected to the ‘MOH 2’

The maximum distance of the installation is 1 Km (3.2 Kft) from the system.

LDK-300 system is capable of accommodating up to 5 external music sources through 5 single line interfaces, which are limited by the main program. This function has been supported since the main program GS00P-3.2D or later version.

To use the external music sources through the desired SLT ports, assign the desired SLT ports in the Admin program field #1 and 2nd button. For more details, see the Programming Manual.

External

Music

Source 1

GDK-100

Single Line

Interface

(SLT port)

External

Music

Source 2

PORT1  PORT2

MOH2    MOH1

GDK-162/100(MOHU)

MUSIC ON HOLD UNIT

- Note :

It is recommended that the MOHU should not be installed in the humid place and the place that ESD (Electrostatic Discharge) occurs.

Figure 3.5.7    Connection method of MOHU

### Contact Assignments of RJ21 Type Connectors on Extension Boards.

Table 3.5.3.a. shows the contact assignments of RJ21 type connector on DTIB12/DTIB24.

| Punch-Down | DTIB connector |  |  | DTIB | Port |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Block Pin | Pair | Pin | Color-Code | Designation | No |  |  |
| 1 | 1 | 26 | WH/BL | DT-1 | 0 | DTIB12/24 |  |
| 2 |  | 1 | BL/WH | DR-1 |  |  |  |
| 3 | 2 | 27 | WH/OR | DT-2 | 1 |  |  |
| 4 |  | 2 | OR/WH | DR-2 |  |  |  |
| 5 | 3 | 28 | WH/GN | DT-3 | 2 |  |  |
| 6 |  | 3 | GN/WH | DR-3 |  |  |  |
| 7 | 4 | 29 | WH/BN | DT-4 | 3 |  |  |
| 8 |  | 4 | BN/WH | DR-4 |  |  |  |
| 9 | 5 | 30 | WH/SL | DT-5 | 4 |  |  |
| 10 |  | 5 | SL/WH | DR-5 |  |  |  |
| 11 | 6 | 31 | RD/BL | DT-6 | 5 |  |  |
| 12 |  | 6 | BL/RD | DR-6 |  |  |  |
| 13 | 7 | 32 | RD/OR | DT-7 | 6 |  |  |
| 14 |  | 7 | OR/RD | DR-7 |  |  |  |
| 15 | 8 | 33 | RD/GN | DT-8 | 7 |  |  |
| 16 |  | 8 | GN/RD | DR-8 |  |  |  |
| 17 | 9 | 34 | RD/BN | DT-9 | 8 |  |  |
| 18 |  | 9 | BN/RD | DR-9 |  |  |  |
| 19 | 10 | 35 | RD/SL | DT-10 | 9 |  |  |
| 20 |  | 10 | SL/RD | DR-10 |  |  |  |
| 21 | 11 | 36 | BK/BL | DT-11 | 10 |  |  |
| 22 |  | 11 | BL/BK | DR-11 |  |  |  |
| 23 | 12 | 37 | BK/OR | DT-12 | 11 |  |  |
| 24 |  | 12 | OR/BK | DR-12 |  | DTIB12/24 |  |
| 25 | 13 | 38 | BK/GN | DT-13 | 12 | DTIB24 only |  |
| 26 |  | 13 | GN/BK | DR-13 |  |  |  |
| 27 | 14 | 39 | BK/BN | DT-14 | 13 |  |  |
| 28 |  | 14 | BN/BK | DR-14 |  |  |  |
| 29 | 15 | 40 | BK/SL | DT-15 | 14 |  |  |
| 30 |  | 15 | SL/BK | DR-15 |  |  |  |
| 31 | 16 | 41 | YL/BL | DT-16 | 15 |  |  |
| 32 |  | 16 | BL/YL | DR-16 |  |  |  |
| 33 | 17 | 42 | YL/OR | DT-17 | 16 |  |  |
| 34 |  | 17 | OR/YL | DR-17 |  |  |  |
| 35 | 18 | 43 | YL/GN | DT-18 | 17 |  |  |
| 36 |  | 18 | GN/YL | DR-18 |  |  |  |
| 37 | 19 | 44 | YL/BN | DT-19 | 18 |  |  |
| 38 |  | 19 | BN/YL | DR-19 |  |  |  |
| 39 | 20 | 45 | YL/SL | DT-20 | 19 |  |  |
| 40 |  | 20 | SL/YL | DR-20 |  | BL:BLUE |  |
| 41 | 21 | 46 | VI/BL | DT-21 | 20 | BK:BLACK |  |
| 42 |  | 21 | BL/VI | DR-21 |  | BN:BROWN |  |
| 43 | 22 | 47 | VI/OR | DT-22 | 21 | OR:ORANGE |  |
| 44 |  | 22 | OR/VI | DR-22 |  | WH:WHITE |  |
| 45 | 23 | 48 | VI/GN | DT-23 | 22 | GN:GREEN |  |
| 46 |  | 23 | GN/VI | DR-23 |  | SL: SLIVER |  |
| 47 | 24 | 49 | VI/BN | DT-24 | 23 | VI:VIOLET |  |
| 48 |  | 24 | BN/VI | DR-24 |  | RD:RED |  |
| 49 | 25 | 50 | VI/SL | - | - | YL:YELLOW |  |
| 50 |  | 25 | SL/VI | - | - |  |  |

Table 3.5.3.a	DTIB12/DTIB24 Wiring

Table 3.5.3.b. shows the contact assignments of RJ21 type connector on SLIB24/SLIB48 and SLIBII/SLIB4E.

| Punch-Down | SLIB connector (SLIB24/SLIB48/SLIB II) |  |  | SLIB | Port No |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Block Pin | Pair | Pin | Color-Code | Designation |  |  |  |
| 1 | 1 | 26 | WH/BL | VT-1 | 0 | SLIB24/48 |  |
| 2 |  | 1 | BL/WH | VR-1 |  | and SLIBII |  |
| 3 | 2 | 27 | WH/OR | VT-2 | 1 |  |  |
| 4 |  | 2 | OR/WH | VR-2 |  |  |  |
| 5 | 3 | 28 | WH/GN | VT-3 | 2 |  |  |
| 6 |  | 3 | GN/WH | VR-3 |  |  |  |
| 7 | 4 | 29 | WH/BN | VT-4 | 3 |  |  |
| 8 |  | 4 | BN/WH | VR-4 |  |  |  |
| 9 | 5 | 30 | WH/SL | VT-5 | 4 |  |  |
| 10 |  | 5 | SL/WH | VR-5 |  |  |  |
| 11 | 6 | 31 | RD/BL | VT-6 | 5 | SLIB24/48 |  |
| 12 |  | 6 | BL/RD | VR-6 |  | and SLIBII |  |
| 13 | 7 | 32 | RD/OR | VT-7 | 6 | SLIB II only |  |
| 14 |  | 7 | OR/RD | VR-7 |  |  |  |
| 15 | 8 | 33 | RD/GN | VT-8 | 7 |  |  |
| 16 |  | 8 | GN/RD | VR-8 |  |  |  |
| 17 | 9 | 34 | RD/BN | VT-9 | 8 |  |  |
| 18 |  | 9 | BN/RD | VR-9 |  |  |  |
| 19 | 10 | 35 | RD/SL | VT-10 | 9 |  |  |
| 20 |  | 10 | SL/RD | VR-10 |  |  |  |
| 21 | 11 | 36 | BK/BL | VT-11 | 10 |  |  |
| 22 |  | 11 | BL/BK | VR-11 |  |  |  |
| 23 | 12 | 37 | BK/OR | VT-12 | 11 |  |  |
| 24 |  | 12 | OR/BK | VR-12 |  |  |  |
| 25 | 13 | 38 | BK/GN | - | - |  |  |
| 26 |  | 13 | GN/BK | - |  |  |  |
| 27 | 14 | 39 | BK/BN | - | - |  |  |
| 28 |  | 14 | BN/BK | - |  |  |  |
| 29 | 15 | 40 | BK/SL | - | - |  |  |
| 30 |  | 15 | SL/BK | - |  |  |  |
| 31 | 16 | 41 | YL/BL | - | - |  |  |
| 32 |  | 16 | BL/YL | - |  |  |  |
| 33 | 17 | 42 | YL/OR | - | - |  |  |
| 34 |  | 17 | OR/YL | - |  |  |  |
| 35 | 18 | 43 | YL/GN | - | - |  |  |
| 36 |  | 18 | GN/YL | - |  |  |  |
| 37 | 19 | 44 | YL/BN | - | - |  |  |
| 38 |  | 19 | BN/YL | - |  |  |  |
| 39 | 20 | 45 | YL/SL | - | - |  |  |
| 40 |  | 20 | SL/YL | - |  | BL:BLUE |  |
| 41 | 21 | 46 | VI/BL | - | - | BK:BLACK |  |
| 42 |  | 21 | BL/VI | - |  | BN:BROWN |  |
| 43 | 22 | 47 | VI/OR | - | - | OR:ORANGE |  |
| 44 |  | 22 | OR/VI | - |  | WH:WHITE |  |
| 45 | 23 | 48 | VI/GN | - | - | GN:GREEN |  |
| 46 |  | 23 | GN/VI | - |  | SL: SLIVER |  |
| 47 | 24 | 49 | VI/BN | - | - | VI:VIOLET |  |
| 48 |  | 24 | BN/VI | - |  | RD:RED |  |
| 49 | 25 | 50 | VI/SL | - | - | YL:YELLOW |  |
| 50 |  | 25 | SL/VI | - |  |  |  |

Table 3.5.3.b	SLIB24/SLIB48 Wiring

Table 3.5.3.c  shows the contact assignments of RJ21 type connector on DSIB.

| Punch-Down | DSIB connector |  |  | DSIB | Port |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Block Pin | Pair | Pin | Color-Code | Designation | No |  |  |
| 1 | 1 | 26 | WH/BL | DT-1 | 0 (DKT 0) | DKT I/F |  |
| 2 |  | 1 | BL/WH | DR-1 |  |  |  |
| 3 | 2 | 27 | WH/OR | DT-2 | 1 (DKT 1) |  |  |
| 4 |  | 2 | OR/WH | DR-2 |  |  |  |
| 5 | 3 | 28 | WH/GN | DT-3 | 2 (DKT 2) |  |  |
| 6 |  | 3 | GN/WH | DR-3 |  |  |  |
| 7 | 4 | 29 | WH/BN | DT-4 | 3 (DKT 3) |  |  |
| 8 |  | 4 | BN/WH | DR-4 |  |  |  |
| 9 | 5 | 30 | WH/SL | DT-5 | 4 (DKT 4) |  |  |
| 10 |  | 5 | SL/WH | DR-5 |  |  |  |
| 11 | 6 | 31 | RD/BL | DT-6 | 5 (DKT 5) |  |  |
| 12 |  | 6 | BL/RD | DR-6 |  |  |  |
| 13 | 7 | 32 | RD/OR | VT-1 | 6 (SLT 0) | SLT I/F |  |
| 14 |  | 7 | OR/RD | VR-1 |  |  |  |
| 15 | 8 | 33 | RD/GN | VT-2 | 7 (SLT 1) |  |  |
| 16 |  | 8 | GN/RD | VR-2 |  |  |  |
| 17 | 9 | 34 | RD/BN | VT-3 | 8 (SLT 2) |  |  |
| 18 |  | 9 | BN/RD | VR-3 |  |  |  |
| 19 | 10 | 35 | RD/SL | VT-4 | 9 (SLT 3) |  |  |
| 20 |  | 10 | SL/RD | VR-4 |  |  |  |
| 21 | 11 | 36 | BK/BL | VT-5 | 10 (SLT 4) |  |  |
| 22 |  | 11 | BL/BK | VR-5 |  |  |  |
| 23 | 12 | 37 | BK/OR | VT-6 | 11 (SLT 5) |  |  |
| 24 |  | 12 | OR/BK | VR-6 |  |  |  |
| 25 | 13 | 38 | BK/GN |  |  |  |  |
| 26 |  | 13 | GN/BK |  |  |  |  |
| 27 | 14 | 39 | BK/BN |  |  |  |  |
| 28 |  | 14 | BN/BK |  |  |  |  |
| 29 | 15 | 40 | BK/SL |  |  |  |  |
| 30 |  | 15 | SL/BK |  |  |  |  |
| 31 | 16 | 41 | YL/BL |  |  |  |  |
| 32 |  | 16 | BL/YL |  |  |  |  |
| 33 | 17 | 42 | YL/OR |  |  |  |  |
| 34 |  | 17 | OR/YL |  |  |  |  |
| 35 | 18 | 43 | YL/GN |  |  |  |  |
| 36 |  | 18 | GN/YL |  |  |  |  |
| 37 | 19 | 44 | YL/BN |  |  |  |  |
| 38 |  | 19 | BN/YL |  |  |  |  |
| 39 | 20 | 45 | YL/SL |  |  |  |  |
| 40 |  | 20 | SL/YL |  |  | BL:BLUE |  |
| 41 | 21 | 46 | VI/BL |  |  | BK:BLACK |  |
| 42 |  | 21 | BL/VI |  |  | BN:BROWN |  |
| 43 | 22 | 47 | VI/OR |  |  | OR:ORANGE |  |
| 44 |  | 22 | OR/VI |  |  | WH:WHITE |  |
| 45 | 23 | 48 | VI/GN |  |  | GN:GREEN |  |
| 46 |  | 23 | GN/VI |  |  | SL: SLIVER |  |
| 47 | 24 | 49 | VI/BN |  |  | VI:VIOLET |  |
| 48 |  | 24 | BN/VI |  |  | RD:RED |  |
| 49 | 25 | 50 | VI/SL |  |  | YL:YELLOW |  |
| 50 |  | 25 | SL/VI |  |  |  |  |

* I/F: Interface

Table 3.5.3.c	DSIB Wiring

### PFTU Wiring

The PFTU is wired from the RJ21 type male connector on the modue itself to the MDF, cross connected to the CO Line inputs from the telephone company, to the CO Line inputs of the system, and to Power Failure SLTs. The CO lines and SLTs are used as CO line inputs and extensions of LDK-300 System in normal operation, but each CO line and SLT is directly connected in power failure mode by connecting as shown in figure 3.5.8

The cable with RJ21 type female connector is required between PFTU connector and MDF. It is possible to get the cable locally, or use GDK-162 5MC1 supplied by LGIC on the order if necessary.

The MDF that is in wide use in your country can also be locally used according to the requirements of local regulation.

MDF

1

26

4

29

SLIB1-T

SLIB1-R

PFTU

25

50

COT1-IN

COR1-IN

2

27

COT1-OUT

COR1-OUT

3

28

TEL1-T

TEL1-R

SLIB interface

CO line

LCOB interface

SLT

RJ21 type connector in PFTU

Connected to MB Basic

Connected to

another PFTU

Figure 3.5.8	PFTU wiring

Note:

- A connector block with screws, which is connected to MB Basic (Back plane), is supplied in the PFTU package, but control wires are not supplied. It is recommended that 2 wires with AWG22, 24 or 26 be used for the connection of control signal.

- The control wires can be connected regardless of their polarities

Table 3.5.4 shows the contact assignments of RJ21 type connector on PFTU.

| Punch-Down | PFTU connector |  |  | PFTU | PFT |  |
| --- | --- | --- | --- | --- | --- | --- |
| Block Pin | Pair | Pin | Color-Code | Designation | Circuit |  |
| 1 | 1 | 26 | WH/BL | COT1-IN | 1st Circuit |  |
| 2 |  | 1 | BL/WH | COR1-IN |  |  |
| 3 | 2 | 27 | WH/OR | COT1-OUT |  |  |
| 4 |  | 2 | OR/WH | COR1-OUT |  |  |
| 5 | 3 | 28 | WH/GN | TEL1-T |  |  |
| 6 |  | 3 | GN/WH | TEL1-R |  |  |
| 7 | 4 | 29 | WH/BN | SLIB1-T |  |  |
| 8 |  | 4 | BN/WH | SLIB1-R |  |  |
| 9 | 5 | 30 | WH/SL | COT2-IN | 2nd Circuit |  |
| 10 |  | 5 | SL/WH | COR2-IN |  |  |
| 11 | 6 | 31 | RD/BL | COT2-OUT |  |  |
| 12 |  | 6 | BL/RD | COR2-OUT |  |  |
| 13 | 7 | 32 | RD/OR | TEL2-T |  |  |
| 14 |  | 7 | OR/RD | TEL2-R |  |  |
| 15 | 8 | 33 | RD/GN | SLIB2-T |  |  |
| 16 |  | 8 | GN/RD | SLIB2-R |  |  |
| 17 | 9 | 34 | RD/BN | COT3-IN | 3rd Circuit |  |
| 18 |  | 9 | BN/RD | COR3-IN |  |  |
| 19 | 10 | 35 | RD/SL | COT3-OUT |  |  |
| 20 |  | 10 | SL/RD | COR3-OUT |  |  |
| 21 | 11 | 36 | BK/BL | TEL3-T |  |  |
| 22 |  | 11 | BL/BK | TEL3-R |  |  |
| 23 | 12 | 37 | BK/OR | SLIB3-T |  |  |
| 24 |  | 12 | OR/BK | SLIB3-R |  |  |
| 25 | 13 | 38 | BK/GN | COT4-IN | 4th Circuit |  |
| 26 |  | 13 | GN/BK | COR4-IN |  |  |
| 27 | 14 | 39 | BK/BN | COT4-OUT |  |  |
| 28 |  | 14 | BN/BK | COR4-OUT |  |  |
| 29 | 15 | 40 | BK/SL | TEL4-T |  |  |
| 30 |  | 15 | SL/BK | TEL4-R |  |  |
| 31 | 16 | 41 | YL/BL | SLIB4-T |  |  |
| 32 |  | 16 | BL/YL | SLIB4-R |  |  |
| 33 | 17 | 42 | YL/OR | COT5-IN | 5th Circuit |  |
| 34 |  | 17 | OR/YL | COR5-IN |  |  |
| 35 | 18 | 43 | YL/GN | COT5-OUT |  |  |
| 36 |  | 18 | GN/YL | COR5-OUT |  |  |
| 37 | 19 | 44 | YL/BN | TEL5-T |  |  |
| 38 |  | 19 | BN/YL | TEL5-R |  |  |
| 39 | 20 | 45 | YL/SL | SLIB5-T |  |  |
| 40 |  | 20 | SL/YL | SLIB5-R |  | BL:BLUE |
| 41 | 21 | 46 | VI/BL | COT6-IN | 6th Circuit | BK:BLACK |
| 42 |  | 21 | BL/VI | COR6-IN |  | BN:BROWN |
| 43 | 22 | 47 | VI/OR | COT6-OUT |  | OR:ORANGE |
| 44 |  | 22 | OR/VI | COR6-OUT |  | WH:WHITE |
| 45 | 23 | 48 | VI/GN | TEL6-T |  | GN:GREEN |
| 46 |  | 23 | GN/VI | TEL6-R |  | SL: SLIVER |
| 47 | 24 | 49 | VI/BN | SLIB6-T |  | VI:VIOLET |
| 48 |  | 24 | BN/VI | SLIB6-R |  | RD:RED |
| 49 | 25 | 50 | VI/SL | - | - | YL:YELLOW |
| 50 |  | 25 | SL/VI | - |  |  |

Table 3.5.4	PFTU Wiring

### Analog CO Line wiring

Analog CO boards such as LCOB contains a RJ21 type male connector to connect to analog CO line on each board. The following provides details on the connection of each type of analog CO line Board.

### LCOB wiring

LCOB is capable of connecting 4 loop start CO lines through a RJ21 type male connector on the board.

The cable with RJ21 type female connector is required between PFTU connector and MDF. It is possible to get the cable locally, or use GDK-162 5MC1 supplied by LGIC on the order if necessary.

The MDF that is in wide use in the country can be locally used according to the requirements of local regulation.

MDF

1

26

4

29

T4

R4

LCOB

RJ21 type connector on board

25

50

T1

R1

2

27

T2

R2

3

28

T3

R3

Network

Figure 3.5.9  loop start CO line wiring

Note:

- CO lines of LCOB can operate regardless of line polarity.

- Connect TRC (Telecommunication reference cable) to 25th pair of RJ21 type connector if it is supplied from Network.

- LCOB is capable of connecting to up to 4 lines.

### Contact Assignments of RJ21 type Connectors on Analog CO Boards.

Table 3.5.5. shows the contact assignments of RJ21 type connector on LCOB.

| Punch-Down | LCOB connector |  |  | LCOB | Line |  |
| --- | --- | --- | --- | --- | --- | --- |
| Block Pin | Pair | Pin | Color-Code | Designation | No |  |
| 1 | 1 | 26 | WH/BL | CO1-T | 1 |  |
| 2 |  | 1 | BL/WH | CO1-R |  |  |
| 3 | 2 | 27 | WH/OR | CO2-T | 2 |  |
| 4 |  | 2 | OR/WH | CO2-R |  |  |
| 5 | 3 | 28 | WH/GN | CO3-T | 3 |  |
| 6 |  | 3 | GN/WH | CO3-R |  |  |
| 7 | 4 | 29 | WH/BN | CO4-T | 4 |  |
| 8 |  | 4 | BN/WH | CO4-R |  |  |
| 9 | 5 | 30 | WH/SL | - | - |  |
| 10 |  | 5 | SL/WH |  |  |  |
| 11 | 6 | 31 | RD/BL |  |  |  |
| 12 |  | 6 | BL/RD |  |  |  |
| 13 | 7 | 32 | RD/OR |  |  |  |
| 14 |  | 7 | OR/RD |  |  |  |
| 15 | 8 | 33 | RD/GN |  |  |  |
| 16 |  | 8 | GN/RD |  |  |  |
| 17 | 9 | 34 | RD/BN |  |  |  |
| 18 |  | 9 | BN/RD |  |  |  |
| 19 | 10 | 35 | RD/SL |  |  |  |
| 20 |  | 10 | SL/RD |  |  |  |
| 21 | 11 | 36 | BK/BL |  |  |  |
| 22 |  | 11 | BL/BK |  |  |  |
| 23 | 12 | 37 | BK/OR |  |  |  |
| 24 |  | 12 | OR/BK |  |  |  |
| 25 | 13 | 38 | BK/GN |  |  |  |
| 26 |  | 13 | GN/BK |  |  |  |
| 27 | 14 | 39 | BK/BN |  |  |  |
| 28 |  | 14 | BN/BK |  |  |  |
| 29 | 15 | 40 | BK/SL |  |  |  |
| 30 |  | 15 | SL/BK |  |  |  |
| 31 | 16 | 41 | YL/BL |  |  |  |
| 32 |  | 16 | BL/YL |  |  |  |
| 33 | 17 | 42 | YL/OR |  |  |  |
| 34 |  | 17 | OR/YL |  |  |  |
| 35 | 18 | 43 | YL/GN |  |  |  |
| 36 |  | 18 | GN/YL |  |  |  |
| 37 | 19 | 44 | YL/BN |  |  |  |
| 38 |  | 19 | BN/YL |  |  |  |
| 39 | 20 | 45 | YL/SL |  |  |  |
| 40 |  | 20 | SL/YL |  |  | BL:BLUE |
| 41 | 21 | 46 | VI/BL |  |  | BK:BLACK |
| 42 |  | 21 | BL/VI |  |  | BN:BROWN |
| 43 | 22 | 47 | VI/OR |  |  | OR:ORANGE |
| 44 |  | 22 | OR/VI |  |  | WH:WHITE |
| 45 | 23 | 48 | VI/GN |  |  | GN:GREEN |
| 46 |  | 23 | GN/VI |  |  | SL: SLIVER |
| 47 | 24 | 49 | VI/BN |  |  | VI:VIOLET |
| 48 |  | 24 | BN/VI |  |  | RD:RED |
| 49 | 25 | 50 | VI/SL |  |  | YL:YELLOW |
| 50 |  | 25 | SL/VI |  |  |  |

Table 3.5.5  LCOB Wiring

### ISDN wiring

ISDN boards such as BRIB, STIB and PRIB contain RJ45 type jacks on each board, to connect to ISDN.

Following figure shows ISND reference point and Functional group,

NT2

TA

NT1

NT1

The position of GDK-System

ISDN

PX

(LT)

ISDN Network

or

PC

G4-Fax

S

V,X

Terminal

SLT

ISDN-TEL

U

U

T

S

R

V

Figure 3.5.10.a	ISDN Reference Points and Functional Group

Reference point: Theoretical point to classify the functional unit

- V-point: PX line

- U-point: Subscriber line

Subscriber line between public exchange and NT1 (Network termination), 2wire and 4-5Km distance

- T-point: ISDN Trunk line

Trunk line for PBX between NT1 and PBX system, 4wire and 1Km max distance.

- S-point: ISDN standard user interface

Standard point for ISDN terminal connection, physically same as T-point, multiple connection possible.

- R-point: X or V series terminal interface.

Reference point between Terminal adapter and non-ISDN terminal connection

Functional Units

- NT1: Network Termination

- NT2: Protocol Handle, PBX

- TE1: Standard ISDN interface TE

- TE2: Non-Standard ISDN interface TE

- TA:  Adapter for non-standard ISDN interface

ISDN trunk connection (Basic Rate)

ISDN

GDK System

ISDN Board

BRIB

STIB(T)

NT1

TR

TR

Max. 1000 meters

1. Point to Point connection

2. TR: Termination resistor, 100 ohm

Figure 3.5.10.b	ISDN Trunk connection (Basic Rate Interface: T)

ISDN Terminal Connection (Basic Rate)

P to P Connection

TR

TE

TR

GDK-SYSTEM

TR

STIB(S)

BRIB

STIB(T)

TR

TR

Max. 1000 meters

TE1

TE2

TE8

Max. 200 meters

TR

TR

TE1

TE2

TE8

Max. 500 meters

Max. 50 meters

P to MP Connection

- Short Passive

P to MP Connection

- Extended Passive

Figure 3.5.10.cISDN Terminal connection (Basic Rate Interface: S)

Connection Type

P to P: Point to Point connection

P to MP: Point to Multi-Point connection

TR: Termination resistor, 100 ohm

This may be contained in a TE among multi-TEs.

I

SDN Trunk connection (Primary Rate)

Max. 1000 meters

GDK System

ISDN Board

PRIB

DSU

ISDN

Figure 3.5.10.dISDN Trunk connection (Primary Rate Interface)

Point to Point connection only

Without power feed.

Line Impedance: 120 ohm

Repeater

Physical Connectors

Basic Rate User network Interface

Figure 3.5.10.eRJ-45 Type Jack

1

2

3

4

5

6

7

8

ISO 8877

ITU-T Rec. I.430

Contact

1

2

3

4

5

6

7

8

TE

Power Source 3

Power Source 3

Transmit

Receive

Receive

Transmit

Power Source 2

Power Source 2

NT

Power Source 3

Power Source 3

Receive

Transmit

Transmit

Receive

Power Source 2

Power Source 2

Polarity

+

-

+

+

-

-

-

+

Table 3.5.6.aContact assignments

Primary Rate User network Interface (E1/T1)

Figure 3.5.10.fRJ-45 Type Jack

1

2

3

4

5

6

7

8

ISO 10173

ANSI T1.408

Contact

1

2

3

4

5

6

7

8

TE

Receive

Receive

Shield(R) *

Transmit

Transmit

Shield(T) *

Power Source *

Power Source *

NT

Transmit

Transmit

Shield(T) *

Receive

Receive

Shield(R) *

Power Source *

Power Source *

Polarity

-

+

Table 3.5.6.bContact assignments

* Optional

### BRIB wiring

BRIB is capable of connecting 4 Basic Rate Trunk lines through RJ45 type jacks.

The contact assignments of RJ45 type jack on BRIB show in figure 3.5.11.a.

Figure 3.5.11.acontact assignments of RJ-45 type jack of BRIB

1

2

3

4

5

6

7

8

RX+

RX-

TX-

TX+

The TX pair of RJ45 type jack on BRIB is connected to the RX pair of NT1, and the RX one on BRIB is to the TX one of NT1 as Figure 3.5.11.b.

Terminating resistors should be required on each line to match line impedance. See clause 3.5.7.4

NT1 (NT-Box)

BRIB

RJ45 type jack

TX+

TX-

RX+

RX-

3

6

4

5

3

6

4

5

RX+

RX-

TX+

TX-

From/to PX

Note:

Connect the TX pair of an RJ45 type jack on BRIB to the RX pair of NT1 and vice versa.

Check terminating resistors. See clause 3.5.7.4.

BRIB is capable of connecting to up to 4 lines.

Figure 3.5.11.bBRI trunk line wiring

### STIB wiring

STIB is capable of connecting to 4 Basic Rate Trunk lines or Terminals through RJ45 type jacks. Each line is switchable by hardware and software setting. (see clause 3.4.6.2)

The contact assignments of RJ45 type jack on STIB show in figure 3.5.12.a.

Figure 3.5.12.acontact assignments of RJ-45 Connectors of STIB

1

2

3

4

5

6

7

8

TX+

TX-

RX-

RX+

1

2

3

4

5

6

7

8

RX+

RX-

TX-

TX+

a. When a line is set to T interface

a. When a line is set to S interface

The TX pair of RJ45 type jack on STIB/STIU is connected to the RX pair of NT1 or Terminal, and the RX one on STIB/STIU is to the TX one of NT1 or terminal as Figure 3.5.12.b.

Terminating resistors should be required on each line to match line impedance. See clause 3.5.7.4.

Note:

Connect the TX pair of RJ45 type jack on STIB to the RX pair of NT1 and vice versa. And connect the TX pair of RJ45 type jack on STIB to a RX pair of ISDN terminal and vice versa.

Check terminating resistors. See clause 3.5.7.4.

SWx means selectable switches of S or T interface on each interface of STIB, see clause 3.4.6.2

STIB is capable of connecting to up to 4 lines/terminals.

Figure 3.5.12.bBRI trunk line (T) or terminal connection (S) wiring

NT1 (NT-Box)

STIB

RJ45 type jack

TX+

TX-

RX+

RX-

3

6

4

5

3

6

4

5

RX+

RX-

TX+

TX-

From/to PX

ISDN Terminal

RX+

RX-

TX+

TX-

3

6

4

5

3

6

4

5

TX+

TX-

RX+

RX-

S

T

SWx

S

T

SWx

OR

T interface

S interface

### PRIB wiring

PRIB is capable of connecting to 1 Primary Rate Trunk lines through an RJ45 type jack.

The contact assignments of RJ45 type jack on PRIB show in figure 3.5.13.a.

Figure 3.5.13.aContact assignments of RJ-45 type jack of PRIB

1

2

3

4

5

6

7

8

RX

TX

GND (T)

GND (R)

RX

TX

Note: Only two pairs of RX and TX are typically connected with RRI line.

The TX pair of RJ45 type jack on PRIB is connected to the RX pair of DSU, and the RX one on PRIB is to the TX one of DSU as Figure 3.5.13.b.

DSU (NT-Box)

PRIB

RJ45 type jack

RX

RX

TX

TX

1

2

4

5

1

2

4

5

TX

TX

RX

RX

From/to PX

Figure 3.5.13.b	PRI trunk line (TE connection) wiring

Note:

- Connect the TX pair of RJ45 type jack on PRIB to the RX pair of DSU and vice versa.

- PRIB is capable of connecting with 1 PRI line.

LDK-300 PRIB is typically used as TE connecting with ISDN PX and DSU. But the PRIB can be used as NT in special purpose such as QSIG function with another GDK PRIBs directly. However the type of contact assignments of RJ45 type jack is provided for TE connection on LDK-300 PRIB, that is, the contact assignments of RJ45 type jack on PRIB are not changed even though the board is set to NT mode. See Table 3.5.6.b and Figure 3.5.13. So it is important that the wiring with contact assignments of RJ45 type connector should be checked, when PRIB is set to master mode (NT) during installation. The TX pair of RJ45 connector on PRIB in master mode is connected to the RX pair of PRIB of another system and vice versa as following figure 3.5.13.c.

Another GDK system

PRIB/PRIU (TE mode)

LDK-300 PRIB (NT mode)

RJ45 connector

RX

RX-

TX

TX

1

2

4

5

1

2

4

5

RX

RX

TX

TX

Note:

When PRIB is set to NT mode, two pins of CON3 should be shorted by shorting traps, e.g. a  short pin. See the clause 3.4.6.3 PRIB Installation; Function of Connectors.

Figure 3.5.13.cPRI line (NT connection) wiring with GDK system PRIBs

### Terminating Resistors on Basic Rate interface(BRIB/STIB)

Terminating resistors are required on each line of BRIB/STIB for the proper operation. Switchable terminating resistors are introduced on BRIB issue 3 or later version and STIB issue 2 or later version. So switches on each line of the board should be ON to terminate them when installing line.

BRIB (issue 3 or later)

STIB (issue 2 or later)

RJ45 type jack

TX+

TX-

RX+

RX-

3

6

4

5

RX+

RX-

TX+

TX-

3

6

4

5

OR

T interface

: BRIB and

STIB (T)

S interface

: STIB (S)

SWx

R

R

ON

SWx

R

R

ON

From/To

BRI Line

From/To

ISDN Terminal

Figure 3.5.14.a	Terminating method of BRIB/STIB with switchable terminating resistors

Note:

- R is 100 ohm terminating resistor, and the line is terminated at 100 ohm when switches are ON.

- SWx means terminating switches on each interface of BRIB/STIB. See clause 3.4.6.1 and 3.4.6.2.

But terminating resistors are not introduced on BRIB issue 2 and STIB issue 1, which versions are officially production stopped. So the external terminating unit should be required to terminate them if BRIB is issue 2 or earlier, and STIB is issue 1. The external terminating unit can be locally got, otherwise 2923NP1656, which is the terminating unit provided by LGE, can be used.

Figure 3.5.14.bTerminating method of BRIB/STIB without switchable terminating resistors

Note: R is 100 ohm terminating resistors.

BRIB (issue 2 or earlier)

STIB (issue 1)

RJ45 type jack

TX+

TX-

RX+

RX-

3

6

4

5

RX+

RX-

TX+

TX-

3

6

4

5

OR

T interface

: BRIB and

STIB (T)

S interface

: STIB (S)

From/To

BRI Line

From/To

ISDN Terminal

R

R

R

R

External terminating Unit

### Clock Control Cable wiring- Multi ISDN board installation

LDK-300 system allows Multi-ISDN boards to be installed. When Multi-ISDN board installation, clock control cable should be connected according to the priority of the cards. Then, in case that the ISDN line of the board with the higher priority is in the state of Loss of signal or clock failed, the synchronized clock for system will be supplied from next daisy chained ISDN board. Every ISDN card in TE mode generates the reference clock synchronized with ISDN line and transmits the clock to PLLU on MPB according to the clock control signal.

|  | High (1) state | Low (0) state |
| --- | --- | --- |
| IN | Enable the reference clock circuit | Disable the reference clock out circuit |
| OUT | Enable next clock circuit | Disable next clock circuit |

Table 3.5.7 clcok control cable on ISDN board

Figure 3.5.15.aExample of multi-ISDN board installation when PRIB is the highest priority.

MPB

PRIB

(TE mode)

BRIB

STIB

(T interface(s))

PLLU

Back Plane (Basic Mother Board)

OUT

IN

Priority

of Boards

<

<

OUT

IN

OUT

IN

BRI Line

TE

S

T

PRI Line

1

Note:

Clock control cables supplied by LGE should be connected between ISDN boards according to the priority of them when multi-ISDN board installation.

When PRIB is set to master (NT) mode, the clock cable connected with PRIB is not required.

MPB

BRIB

(the highest priority)

BRIB

STIB

(T interface(s))

PLLU

Back Plane (Basic Mother Board)

OUT

IN

Priority

of Boards

<

<

OUT

IN

OUT

IN

BRI Line

TE

S

T

Figure 3.5.15.bExample of multi-ISDN board installation when a BRIB has the highest priority.

1

Note:

1. Clock control cables supplied by LGE should be connected between ISDN boards according to  the priority of them when multi-ISDN board installation

MPB

STIB

(T interface(s))

(the highest priority)

STIB

(T interface(s))

STIB

(all S interfaces)

PLLU

Back Plane (Basic Mother Board)

OUT

IN

Priority

of Boards

<

OUT

IN

OUT

IN

BRI Line

TE

S

Figure 3.5.15.cExample of multi-ISDN board installation when a STIB is the highest  priority and all the interface of another STIB are set to S interfaces.

1

Note:

Clock control cables supplied by LGE should be connected between ISDN boards according to the priority of them when multi-ISDN board installation.

If all the lines of ISDN board are set to NT mode (S interface), clock control cable should be not connected to the board.

TE

S

<

The followings are the clock chain with the other ISDN board in the LDK-300 system. There are six cases.

Figure 3.5.16.a	case 1 of clock chain

Figure 3.5.16.b	case 2 of clock chain

Figure 3.5.16.c	case 3 of clock chain

Figure 3.5.16.d	case 4 of clock chain

Figure 3.5.16.e	case 5 of clock chain

Figure 3.5.16.f	case 6 of clock chain

### QSIG connection

LDK-300 system supports QSIG function using ISDN boards. There are two kinds of connection for supporting QSIG function. One is the direct connection between LDK systems with PRI line, the other is the connection through Public Network with PRI or BRI lines.

Direct connection

Only GDK PRI boards can be used for the direct connection.

For more details, see the manual for GDK186/100/FPII Q-signaling Interface.

LDK system

LDK system

PRI (NT)

PRI (TE)

Public Network

Figure 3.5.17.a	 Direct connection for QSIG feature

Note:

- The wiring of RJ45 type jack and mode selection connector on LDK-300 PRIB should be checked when LDK-300 PRIB is set to master (NT) mode.

See the clause 3.5.7.3 PRIB wiring and figure 3.5.13.c.

- The clock control cable connected to PRIB is not required when GDK-100 PRIB is set to master (NT) mode.

- To set the LDK-300 PRIB to NT(master) mode, CON3 on LDK-300 PRIB should be short. See the clause 3.4.6.3 PRIB installation.

The connection through Public Network

LDK PRIB/BRIB/STIB can be used for this connection.

For more details, see the manual for GDK186/100/FPII Q-signaling Interface.

LDK system

LDK system

Public ISDN

LDK system

PRI /BR I(TE)

PRI /BRI(TE)

PRI /BRI(TE)

Figure 3.5.17.b 	The connection through the Public Network for QSIG feature

### EMI Suppression

Installation of EMI core on line(s) of STIB is required to suppress EMI.

Install two EMI cores supplied in STIB package as following figure 3.5.18.

When the core is installed on 1 or 2 lines, it may be slip down. In that case, winding wire with a string such as cable tie is recommended.

Below 30 Cm

Below 30 Cm

LDK-300 KSU

Line x 4

STIB

Figure 3.5.18	Ferrite core installation to suppress EMI

### Contact Assignments of various types of connectors

The contact assignments of various types of connectors are described in this clause.

### RJ21 type connectors

The contact assignments of RJ21 type (25-pair Amphenol) connectors

Figure 3.5.19Contact assignments of RJ21 type connectors

25

50

1

26

2

27

3

28

a. Female connector

b. male connector

25

50

3

28

2

27

1

26

### Miniature 6-position jack

The contact assignments of miniature 6-position jack registered in FCC.

Figure 3.5.20Contact assignments of miniature 6-position jack

T1

R2

R1

T2

1

2

3

4

5

6

### Miniature 8-position jack for ISDN connection

The contact assignments of miniature 8-position jack used for ISDN connection.

a. Basic Rate User network Interface

Figure 3.5.21.aRJ-45 Connector

1

2

3

4

5

6

7

8

ISO 8877

ITU-T Rec. I.430

Contact

1

2

3

4

5

6

7

8

TE

Power Source 3

Power Source 3

Transmit

Receive

Receive

Transmit

Power Source 2

Power Source 2

NT

Power Source 3

Power Source 3

Receive

Transmit

Transmit

Receive

Power Source 2

Power Source 2

Polarity

+

-

+

+

-

-

-

+

Table 3.5.8.aContact assignments

b. Primary Rate User network Interface (E1/T1)

Figure 3.5.21.bRJ-45 Connector

1

2

3

4

5

6

7

8

ISO 10173

ANSI T1.408

Contact

1

2

3

4

5

6

7

8

TE

Receive

Receive

Shield(R) *

Transmit

Transmit

Shield(T) *

Power Source *

Power Source *

NT

Transmit

Transmit

Shield(T) *

Receive

Receive

Shield(R) *

Power Source *

Power Source *

Polarity

-

+

Table 3.5.8.bContact assignments

* Optional