##### SECTION 3. INSTALLATION

### INTRODUCTION

As with any sophisticated communications device, installation of the LDK-300 System requires the care and fore-thought of a competent technician. Installation proceeds in 5 major steps;

- Site Preparation

- KSU and PSU Installation

- PCB Installation

- System Wiring

- System Programming and Verification

By utilizing the instructions that follow, the installation is quick and efficient. Directions for System Programming are briefly given in SECTION 6. CUSTOMER DATABASE PROGRAMMING

### SITE PREPARATION

### General Site Consideration

The first step is to locate an acceptable site for the common equipment.(KSUs, boards, etc.) When locating a mounting site for the KSUs, the following points must be considered;

- The KSUs are designed for wall-mounting and should not be mounted directly to a masonry or plaster-board wall. It is recommended that a  inch plywood back-board be firmly mounted to the wall, and the KSU and the MDF, if other than the MDF, be mounted to the back-board.

b) The location must have access to a dedicated 110 Volt AC or 220 Volt AC, 50-60 Hz circuit  with a circuit breaker or fuse rated at 15 amps. A 3-wire parallel blade grounded outlet should be within approximately 2 meters(6 feet) of the lower left rear of the KSU mounting.

c) The location must have access to a good earth ground, such as a metallic cold water pipe with no non-metallic  joints. The ground source should be located as close as possible to the system.

d) The system should be located in an area which is well ventilated with a recommended temperature of 20-26 degrees C (68-78.8 degrees F) and a relative humidity of 5-90 % (non-condensing).

e) The system should be located within 8 meters (25 feet) of the telephone company's termination point. Also, the location should be within the prescribed station loop lengths for all keysets and terminals. If existing cabling is to be employed, the location of existing cabling and conduits should be considered.

f) The location should have adequate accessibility, space and lighting for future servicing and should consider the need for future expansion.

g) The site should be away from radio transmitting equipment, arc-welding devices, copy machines, and other electrical equipment capable of generating electrical interference. The system should be protected from flooding and heavy machinery as well as excessive dust and vibration.

### Back-Board Installation

A wooden back-board is recommended for all installations and must be installed when the location has masonry or plaster-board walls. A 0.5 inch plywood material is sufficient for most installations. The back-board should be mounted at a convenient height, about 1 meter above the floor, and be bolted in a number of places to distribute the weight of the system.

Space should be available on the bottom side of the back-board for the MDF and/or other MDF and for optional equipment such as a music source and PFTU, etc.

### Verify On-Site Equipment

Once the equipment installation site has been identified and a dedicated AC outlet, earth ground, and lighting and ventilation are available, verify that all equipment required is on-site and has not been damaged during shipment. Unpack the KSUs assure there is no shipping damage. Note that a mounting template is packed with KSU and this template will be required later in the installation. Check that the type and quantity of boards received is correct and optional equipment and a Power Line Surge Protector are on-site. Note that the individual boards should not be unpacked at this time.

If any equipment is damaged or missing, notify appropriate personnel to correct the situation.

### KSU INSTALLATION

LDK-300 system consists of three KSUs, 1st,2nd,and 3rd KSU. The 2nd and the 3rd  are used for expansion KSU.

Caution:

Installing 1st,2nd,and 3rd KSU together on the brick wall can cause the connection between Basic and Expansion KSU to be unstable. So we ecommend that three KSUs should be installed on the flat wall.

### Mounting 1st  ,KSU2nd and 3rd KSU

The 1st KSU is metal frame cabinet designed for wall mounting. The KSU must not be mounted on a masonry or dry-wall surface, in this case a wooden Back-Board is required, reference paragraph Back-Board Installation. The distance between mounting holes and the position of mounting holes is shown in Figure 3.3.2 ~ 3.3.3.

The KSU is mounted with four #10 or larger, 1.5 inch or longer screws.

First, drill pilot holes in the two locations marked, insert the screws and tighten leaving about 0.5 inch exposed. Mount the KSU on the screws and tighten the screws securely.

Second, remove the front cover by turning the two cover screws 0.5 turn counterclockwise, tilt and lift the cover to remove. And, insert the screws to the mounting holes of KSU and tighten the screws as Figure 3.3.1~ 3.3.4.

The 2nd and 3rd KSU for expansion is a metal housing designed for wall mount installation as the 1st KSU.

The 2’nd KSU is mounted in a manner similar to the 1’st KSU, refer to Figure 3.3.5. Select the positions of two holes to fix 2nd KSU on the wall so that the proper connection between 1st KSU and 2nd KSU will be possible. Mount the 2nd KSU on the screws and tighten the screws securely. And insert the 4 fasteners in the both left and right side. Fasten the fasteners with 8 screws between the 1st KSU and 2nd KSU, refer to Figure 3.3.5.

When the 3rd KSU is installed, the 3rd  KSU is mounted in a manner similar to the 2nd KSU, refer to Figure 3.3.6.

Figure 3.3.1 KSU Exterior

Figure 3.3.2  1st KSU Mounting Holes & Installation Layout

Figure 3.3.3  2nd  and 3RD KSU Mounting Holes & Installation Layout 1

Figure 3.3.4 2ND and 3rd KSU Mounting Holes & Installation Layout 2

Figure 3.3.5	Connection between 1st KSU and 2nd KSU

Figure 3.3.6 Connection between 2nd KSU and 3rd KSU

### Mounting RACK

Figure 3.3.7 Optional rack

### RGU Installation

The Ring Generator Unit is needed in the 1st,2nd,and 3rd KSU if SLIB24, SLIB48,SLIBII, and/or DSIB are to be installed, to provide Ring voltage and Message Wait Source. According to the installation site, two types of RGUs are available; External(RGU5), Internal(RGU6,LDK-300 RGU).

The external RGU is fixed with the two screws provided, on the wall outside the KSU and is connected to CN14 (marking on PCB) connector on the system back-plane of KSU. The internal RGU is fixed on the right bottom side panel with the two screws provided inside the KSU and is connected to CN14 (marking on PCB) connector on the system back-plane of  KSU.

See figure 3.3.8.

CN12 in BKSU

CN9 in SBKSU

###### External

##### RGU

###### KSU

Figure 3.3.8 The installation of LDK-300 RGU

CN14

On MB

A

###### Internal

###### RGU

B

A corresponding cable for connecting between external /internal RGU and KSU is provided with RGU in the same package.

### PFTU Installation

The PFTU provides the relay contacts for the direct connection of 6 Exchange Lines (CO Lines) to 6 Extension Lines (SLT Lines) in event of a power or processor failure. The PFTU can be optionally installed on the right upper side of every KSU.

2 control wires are connected to the CN15 (marking on PCB) on back-plane of the system as fig. 3.3.8 regardless of the polarity of control wires. The connector for CN15 is provided but control wires should be got locally. The specification of control wires is AWG #22, #24 or #26 regardless of their length.

A RJ21 type connector, which consists of 25 pairs, is connected CO line, LCOB, SLIB and SLT as section 3.5 .5 PFTU Wiring.

An earth terminal is provided, and the grounding method of the PFTU is shown in figure 3.3.8 when the PFTU is installed outside KSU. And there is a test switch on the surface of PFTU; TEST position of the switch is used for testing PFTU function, and NORMAL position of the switch for normal operation.

In case of KSU, a PFTU can be installed on the frame of KSU in the system, its position is beside MPB. See Figure 3.3.10 for connection earth cable.

Earthing point

RJ21 type connector

To CN15

on backplane.

Figure 3.3.9	The exterior of PFTU

### KSU Grounding

Before connection to the mains supply, the LDK system must be earthed in conformance with the appropriate IEE requirements. Earthing is required for user safety and also to minimize EMC interference.

To ensure proper system operation and for safety purposes, a good earth ground is required. A metallic COLD water pipe usually provides a reliable ground. Carefully check that the pipe does not contain insulated joints that could isolate the ground path.  If insulated joints exist, another earth ground source must be used or, if allowed, the joints may be bridged.

A separate conductor should be used between the ground source and the KSU, which confirms to the appropriate national or local electrical codes. AGW #14 or larger copper wire is recommended. The wire should be kept as short as possible, it is recommended that the wire be no longer than 8 meters (about 25 feet).

Earth Point

KSU or BKSU

EKSU

Fastener

to fix EKSU to BKSU

PFTU

External

RGU

Backplane

pPSU

AWG#14 or larger

AWG#18 or larger

PSU

earthing Point

of the system

Connecting Wire: 1. UL 1015 AWG#18 or larger than this.

2. Length ≤ 1 meter  * The shorter the length of wire is, the better it is.

3. Color : Green / Yellow

Figure 3.3.10 The Connection Method of Earth Cable between KSU, External RGU and PFTU.

### PSU Installation

The Power Supply Unit (Figure 3.3.11.a) provides power for the system boards and telephones, converting the AC Voltage input to appropriate DC voltages.

Before installation, assure that the AC plug is not plugged into the AC outlet. Place the PSU in the left most slot in the KSU, aligning the card guides with the PSU PCB and PSU frame flanges. Slide the PSU into place using the injectors to seat the PSU fully in the KSU. And then fix the PSU to the KSU with two screws as figure 3.3.11.b.

The PSU can operate from either 110/115 or 220/230 Volts AC based on the setting of the VOLTAGE SELECTOR Switch on the lower front of the PSU. Generally the PSU is shipped with this switch set for 220/230VAC.

If local AC is 110 volts, move the switch to the upper to display “110/115V”. If local AC is 230 volts, move the switch to the lower to display “220/230V”. Default setup of the products: 220/230VAC.

There are two kinds of PSUs. One is PSU3, the other is LDK-300 PSU.

The range of mains input voltage and the fuse ratings of PSU3 and LDK-300 PSU according to the position of VOLTAGE SELECTOR Switch are as follows;

| Position of VOLTAGE SELECTOR | The Range of Mains Input Voltage | Fuse Ratings PSU3/LDK-300PSU |
| --- | --- | --- |
| AC 115V | 93VAC ~ 132VAC | 6A /8A @125V |
| AC 230V | 187VAC ~ 264 VAC | 4A /5A @250V |

Table 3.3.1 Fuse rating of PSU3 and LDK-300 PSU

PSU capacity is as follows;

Unit: mA

|  | +5 Vdc | -5 Vdc | +30 Vdc | Battery |
| --- | --- | --- | --- | --- |
| LDK-300 PSU (SMPS) | 10000 | 1500 | 8400 | 1A |
| GDK-100 PSU3 (SMPS) | 5000 | 1000 | 7000 | 1A |

Table 3.3.2  Power Capacity of GDK-100 PSU1 and PSU3

Figure 3.3.11..a	Power Supply Unit

Figure 3.3.11.b	Installation of PSU

### MDF Installation

When the external MDF is in wide use in local countries according to the local market conditions or regulatory requirements, the connection method of lines between KSU and MDF is as following figure 3.3.12

Backplane

KSU or SBKSU

KSU

PSU

External MDF

Figure 3.3.12 MDF installation with Local MDF

- Note

- The external MDF being in wide use in local country can be used according to the requirements of local regulation.

- In this case, the cables with RJ21 type connector(25-pair Amphenol) will be required. Those can be locally got, otherwise supplied by LGIC, see 5MC1 and 5MC2.

- See Figure 3.3.4 to 3.3.6 for the shape of cable arrangement.

### PCB INSTALLATION

### PCB Handling & General Installation

All boards should not be installed or removed with power applied.

Power must be turned off prior to insertion or removal of the PCB.

The system PCBs contain digital circuitry which, while extremely reliable, can be damaged by exposure to excessive static electricity. When handling PCBs, a grounded wrist strap should be used to protect the boards from static discharges. Also, use common sense when handling the PCBs. For example, do not place a PCB in locations where heavy objects might fall on the PCB and damage components.

To insert a PCB, hold the PCB by the injectors and, with components facing right, align the top and bottom edge of the PCB in the card guides. Slide the card into the system and use the injectors to seat the PCB firmly into the back plane connectors. To remove a PCB, reverse the procedure. Installation method of PCB is shown in Figure 3.4.1.

Insert Figure 3.3 in the existing Manual here

PCB

Figure 3.4.1 PCB Installation

### MPB Installation

The MPB is installed in the MPB PCB card slot (slot MPB) of the only 1st KSU. The MPB may be equipped with 3 add-on boards; PMU for software program, a MODU for modem access to the system, and a SDRAM for software downloding and additional features. See Figure 3.4.2.

Lithium Battery

The MPB contains a lithium dry-cell to maintain memory and real-time clock functions. The battery is soldered on the MPB, and connected circuitry by on-off DIP switch. So the switch is set to ON position before the MPB installation.

“ CAUTION : Danger of explosion if battery is incorrectly replaced. Replace only with the same or

equivalent type recommended by the manufacturer. Dispose of used batteries according to the manufacturers instruction.”

Figure 3.4.2 MPB,PMU and MODU configuration

Function of SW3

The MPB also has an eight-position DIP switch, SW3. The function of each switch position is as followings:

Default setting: All ON position

| Switch | Function | OFF | ON |
| --- | --- | --- | --- |
| 1 | Administration Programming Access | Disable | Enable |
| 2 | CTS (Send to Clear) of RS232 | Enable | Disable |
| 3 | Command/Event Trace (The purpose of testing software) | Enable | Disable |
| 4 | Not used | - | - |
| 5 | SMDI (Simplified Message Desk Control –Voice Mail ) | SMDI ON | SMDI OFF |
| 6 | Not used | - | - |
| 7 | Power up status trace of board | Enable | Disable |
| 8 | Database default on power up | Disable | Enable |

Table 3.4.1	The function of each pole of SW2

Before programming the system, switch 8 should be placed in the on position and power cycled off and on to initialize the system database to default. Once the database has been initialized, switch 8 should be placed in the off position so as to protect the database.

After the lithium batteries(SW2 ON) and Add-on boards installed, insert the MPB in slot MPB of  KSU.

Function of various switches and connectors

| Switch/Connector | Functions | Remark |
| --- | --- | --- |
| SW1 | CPU Reset |  |
| SW2 | Lithium Battery ON/OFF switch |  |
| CN1 and CN2 | MODU connection |  |
| CN3 AND CN4 | RS232C Serial Interface | System Access |
| CN6 | BDM connection |  |
| CN7 | Card Edge connection |  |
| CN8 and CN9 | PMU connection |  |
| PJ1 and PJ2 | External paging and MOH connection |  |
| S1 | SDRAM module connection |  |

Table 3.4.2 Function of various switches and connectors

Note:

- MPB should be installed in MPB slot in the only 1st KSU.

- The DIP switch, SW2 should be turned ON position before the MPB installation to protect system data in the case of a power fail.

- The 8th pole (switch 8) of SW3 should be OFF to protect the variety features by Admin programming after the system power up and initialization. See Table 3.4.2

- When the system power is on, the default slots will be automatically redefined from the most left slot if the 8th pole of SW3  is set to ON position. So It is recommended that a board among DTIB12, DTIB24 and DSIB is installed in the most left slot and a Digital Keyset with display is connected to the first port the board when the first initialization of the system, since the FIRST STATION of the system can only access the system database without PC Administration.

- Add-on boards: PMU, MODU,SDRAM module.

### PMU Installation

The MPB should be equipped with PMU for system operation. Before installing the MPB, install PMU, refer to Figure 3.4.2

The PMU should be installed on MPB through the dual line connectors, CN8 and CN9 of MPB, so recommend visual inspection of the connecting state between MPB and PMU after installing it on MPB.

### SDRAM Module Installation

The MPB may be equipped with 32M bytes SDRAM module for memory expansion and the software downloding, and additional features. Before installing the MPB, install SDRAM module if required, refer to Figure 2.4.6.1.  The SDRAM module should be installed on the position of SDRAM(s1) on MPB.

The SDRAM module should be installed on MPB through 168pinconnectors, so recommend visual inspection of the connecting state between MPB and SDRAM after installing it on MPB.

### MODU Installation

The MODU is used for remote access the system for system maintenance and PC based administration function. MODU can be installed on MPB through CN1 and CN2 of MPB. refer to Figure 3.4.2

### MISB Installation

The MISB contains 2 MOH inputs, 2 external paging ports and four dry contacts, and an alarm detection.

| Connectors/Switches | Function | Remark |
| --- | --- | --- |
| CONN1 | DRU Connection |  |
| CHAMP1 | RJ21 type female connector (25-pair Amphenol) for External Paging, External MOH, Relay contacts, and Alarm Input | See the clause 3.5 |

Table 3.4.3 Function of various connectors on MISB

One MISB can be installed in the slot No. 9 in the only 1st KSU.

SIU can be installed on MISB.

Insert Figure 3.4.3 in the existing Manual here

Figure 3.4.3	SIU Installation

### SIU Installation

The SIU board is used for system maintenance, PC based Admin, SMDR print and SMDI. The system supports up to 3 Serial Interface circuits, 1 circuit on MPB and additional 2 circuits on SIU board. The SIU should be installed if more than 1 serial interface devices are required.

Followings are the list of the system’s output;

- Administration database

- Off-line SMDR (on-demand)

- On-line SMDR

- Statistical Information

- SMDI (voice mail)

- System trace data

Note:

- A SIU board can be installed only on MISB.

- SIU should be fixed on MISB with two plastic supporters and two screws provided.

### Extension Board Installation

The system can be equipped with any combination of the extension boards; DTIB12, DTIB24, SLIB24, SLIB48, SLIBII, DSIB, and WTIB. The station interface boards can be installed in any card slots in KSU. But WTIB can not be installed in slot 9 of every KSU.

It is recommended to install extension boards, e.g. DTIB from the left most slot (slot No.1). Also, since the first port of SLOT # 1 will be the database access port, it is recommended that a DTIB be installed in this slot and a Digital Keyset with display be connected to this port.

### DTIB12/DTIB24 Installation

DTIB12/DTIB24/DSIB can be installed in any card slot in universal slot in KSU.

Since the first port of SLOT 1 will be the database access port, it is recommended that a DTIB12, DTIB24 or DSIB be installed in this slot and a digital keyset with display be connected to this port.

Function of LEDs

| LED | Function | Remark |
| --- | --- | --- |
| LED1 | The status of ports. (ON: in use, OFF: idle) |  |

Table 3.4.4 LEDs on DTIB12/24

Function of connector

| Connector | Function | Remark |
| --- | --- | --- |
| CHAMP1 | RJ21 type female connector for terminal connection. |  |

Table 3.4.5 connector on DTIB12/24

- Note:

- The distance between Digital Terminal Interface Board and Digital Terminal: see Table 2.3.5

In LDK-300 system, LDK-30DS with 2B-module and secondary terminal can be connected to only every even-numbered port (0,2,4,…) of DTIB12/DTIB24/DSIB, then any digital terminal can not be connected to the consecutive odd port(1,3,5,…) in that case. See clause 2.4.12.3 2B-module and clause 2.4.12.4 GDK-PC PHONE in section 2 for more details regarding 2B-module and CTI module.

DTRU

MSGU

or

MSGU48

CHAMP1

Figure 3.4.4 DTIB

DTIB12/DTIB24 can be installed in any of slot No. 1-9 in KSU.

So It is recommended that a board among DTIB12, DTIB24 and DSIB is installed in the most left slot and a Digital Keyset with display is connected to the first port the board when the first initialization of the system, since the FIRST STATION of the system at first can only access the system database without PC Administration. See clause 3.4.2 MPB Installation.

### SLIB24/SLIB48 Installation

SLIB24/SLIB48 can be installed in any universal slot and allows 6 single line analogue devices to be connected. A MSGU/MSGU48 with 6 circuits can be optionally installed on SLIB24/SLIB48 respectively to light a message waiting lamp in the SLT connected to SLIB24/SLIB48, and a DTRU with 2 receivers can be optionally installed on SLIB24/SLIB48 to receive DTMF signals.

Function of LED

| LED | Function | Remark |
| --- | --- | --- |
| LED1 | The status of ports. (ON: in use, OFF: idle) |  |

Table 3.4.6 connector on SLIB24/48

Function of various connectors

| Connector | Function | Remark |
| --- | --- | --- |
| CONN1 and CONN2 | DTRU connection |  |
| CONN3 and CONN4 | MSGU and MSGU48 connection | 1) |
| CHAMP1 | RJ21 type connector (female) for terminal connection |  |

Table 3.4.7 connector on SLIB24/48

- Note:

- If you want Message Waiting function on SLIB24 or SLIB48, MSGU or MSGU48 should be installed on SLIB24 or SLIB48 respectively. MSGU can be installed on SLIB24, and MSGU48 can be installed on SLIB48.

- The distance between SLIB and SLT: see Table 2.3.5

SLIB24/SLIB48 can be installed in any of slot No. 1-9 in KSU .

Add-on boards: DTRU, MSGU(on SLIB24), MSGU48(on SLIB48)

DTRU

MSGU

or

MSGU48

CHAMP1

Figure 3.4.5 MSGU/MSGU48 and DTRU Installation

### SLIBII Installation

SLIBII can be installed in any universal slot and allows 12 single line analogue devices to be connected. A MSGU with 12 circuits can be optionally installed on SLIB II to light a message waiting lamp in the SLT, and a DTRU II with 4 receivers can be optionally installed on SLIB II to receive DTMF signals.

Function of LED

| LED | Function | Remark |
| --- | --- | --- |
| LD1 | The status of ports. (ON: in use, OFF: idle) |  |

Table 3.4.8 LEDs on SLIBII

Function of various connectors

| Connector | Function | Remark |
| --- | --- | --- |
| CN2 and CN4 | DTRUII connection |  |
| CN3 and CN5 | MSGUII connection | 1) |
| CN1 | RJ21 type connector (female) for terminal connection |  |

Table 3.4.9 connector on SLIBII

- Note:

- If you want Message Waiting function on SLIBII, MSGUII should be installed on SLIBII.

- The distance between SLIBII and SLT: see Table 2.3.5

SLIBII can be installed in any of slot No. 1-9 in KSU

Add-on boards: DTRU II, MSGU II

CN1

MSGU II

DTRU II

Figure 3.4.6 MSGU II and DTRU II Installation

### SLIB4E Installation

Function of LED

| LED | Function | Remark |
| --- | --- | --- |

Table 3.4.10 LEDs on SLIB4E

Function of various connectors

| Connector | Function | Remark |
| --- | --- | --- |
|  |  | 1) |

Table 3.4.11 connector on SLIB4E

- Note:

Figure 3.4.7  SLIB4E

### DSIB Installation

DSIB can be installed in any universal slot and allows 6 Digital terminals and 6 single line analogue devices to be connected. A MSGU with 6 circuits can be optionally installed on DSIB to light a message waiting lamp in the SLT connected to DSIB, and a DTRU with 2 receivers can be optionally installed on DSIB to receive DTMF signals using a system resource.

Function of LED

| LED | Function | Remark |
| --- | --- | --- |
| LED1 | The status of ports. (ON: in use, OFF: idle) |  |

Table 3.4.12 LEDs on DSIB

Function of various connectors

| Connector | Function | Remark |
| --- | --- | --- |
| CN1 and CN2 | DTRU connection |  |
| CN3 and CN4 | MSGU connection | 1) |
| CN5 | RJ21 type connector (female) for terminal connection |  |

Table 3.4.13 connector on DSIB

- Note:

- If you want Message Waiting function with SLT, MSGU should be installed on DSIB.

- The distance between DSIB and Digital terminal/SLT: see Table 2.3.5

DSIB can be installed in any of slot No. 1-9 in KSU

Add-on boards: DTRU, MSGU

CN5

MSGU

DTRU

Figure 3.4.8	MSGU and DTRU Installation

### DTRU/DTRUII Installation

A DTRU contains 2 DTMF receiver circuits and a DTRU II has 4 DTMF receiver circuits. Each receiver of DTRU /DTRU II is time-shared under the control of the system software, as a system resource regardless its position.

| BOARD | The position of Installation | The quantity of receivers | Marking on PCB |
| --- | --- | --- | --- |
| DTRU | SLIB24, SLIB48, DSIB, LCOB, AC15, BWDIDB, EMIB, DIDB | 2 DTMF receivers | DTRU of GDK-34/36 |
| DTRU II | SLIB II | 4 DTMF receivers | GDK-100 DTRU II |

Table 3.4.14 DTRU

The System is capable of supporting DTMF receivers up to 22.

At least a DTRU or DTRU II should be installed to receive DTMF signals.

Take a cautious attitude in installing DTRU on the boards because DTRU contains the connectors without the connection guide, otherwise DTRU may be damaged by wrong connection.

### MSGU/MSGU48/MSGUII Installation

A message waiting unit can be optionally installed on a SLT interface board to light a message waiting lamp in the SLT connected. There are 3 kinds of message waiting unit as followings according to the line voltage.

| BOARD | The position of Installation | The quantity of circuits | The line voltage |
| --- | --- | --- | --- |
| MSGU | SLIB24, DSIB | 6 circuits | +24Vdc |
| MSGU48 | SLIB48 | 6 circuits | -48Vdc |
| MSGU II | SLIB II | 12 circuits | -28Vdc |

Table 3.4.15 MSGU

- Signal voltage: 90~100Vdc

### Analog CO Line Board Installation

There are several types of analog CO line boards available. These boards includes the LCOB .

Every CO line board contains several types of connectors for the line connection on the front edge of the boards themselves.

### LCOB Installation

LCOB provides 4 Loop Start CO line interfaces. The board can be optionally equipped with Call Metering Unit (CMU) boards to monitor call duration and a Call Progress Tone Unit (CPTU) daughter board to monitor call progress signals. A CMU must be provided for each Loop Start CO Line which requires call metering. In addition, the appropriate type of CMU must be employed.

If required, install the CMUs and CPTU on the CO interface board as shown in Figure 3.4.9 and insert the boards in the selected card slot.

Function of LED

| LED | Function | Remark |
| --- | --- | --- |
| LED1 | The status of line 1, ON: in use, Green |  |
| LED2 | The status of line 2, ON: in use, Green |  |
| LED3 | The status of line 3, ON: in use, Green |  |
| LED4 | The status of line 4, ON: in use, Green |  |

Table 3.4.16 LED on LCOB

Function of various connectors

| Connector | Function | Remark |
| --- | --- | --- |
| CN1 and CN2 | DTRU connection |  |
| CN3 and CN4 | CPTU/A or CPTU/B connection |  |
| CN100 and CN104 | The connectors for a CMU on line 1 | See Note |
| CN101 and CN105 | The connectors for a CMU on line 2 | See Note |
| CN102 and CN106 | The connectors for a CMU on line 3 | See Note |
| CN103 and CN107 | The connectors for a CMU on line 4 | See Note |
| CN108 | RJ21 type male connector for CO line connection. |  |

Table 3.4.17 connector on LCOB

- Note:

- The pin #6 and #7 of CN104, CN105, CN106 and/or CN107 should be shorted by shorting traps (short pin) when CMU12(12 KHz meter pulse) including CMU12PR or CMU16(16 KHz meter pulse) is not installed on the interface circuit(s). Otherwise the receiving voice path of the related line will be open.

- The 0 ohm resistors are usually mounted on R168-R171 of LCOB for the countries using only CMU PR(Polarity Reversal) and/or 50L(50 Hz longitudinal). If these CMUPR, CMU50L or CMU50PR are not installed on any CO interface circuit neither R168-R171 are mounted on LCOB, the pin #6 and #7 of CN104, CN105, CN106 and/or CN107 should be also shorted by shorting traps (short pin) on the related interface circuit.

- CN104/R168 are on the 1st interface circuit, CN105/R169 on the 2nd interface circuit, CN106/R170 on the 3rd interface circuit and CN107/R171 on 4th interface circuit.

LCOB can be installed in any one of slot No. 1-9 in KSU

Add-on boards: DTRU, CPTU/A, CPTU/B, CMU

Insert Figure 3.4.9 in the existing Manual here

Figure 3.4.9 CMU & CPTU, DTRU Installation

### LCOB8 Installation

Function of LED

| LED | Function | Remark |
| --- | --- | --- |

Table 3.4.18 LED on LCOB8

Function of various connectors

| Connector | Function | Remark |
| --- | --- | --- |

Table 3.4.19 connector on LCOB8

Note:

Insert Figure 3.4.10 in the existing Manual here

Figure 3.4.10 CMU & CPTU, DTRU Installation

### ISDN Board Installation

There are 3 kinds of ISDN boards such as PRIB, BRIB, and STIB are available. Each ISDN board contains RJ45 type jack on boards themselves.

### ISDN BRIB (Basic Rate T interface) Installation

The BRIB card provides 4 interface circuits of BRI (Basic Rate Interface) format digital CO interface.

If more than one ISDN board is required, some special cable should be used for clock signal control. CON3 and CON4 are used to control the clock signal when multi-ISDN boards are installed. Refer to [Clock Control Cable Wiring] of section 3.5.

Function of LED

| LED | Function | Status | Remark |
| --- | --- | --- | --- |
| LED1 | The status of Line 1 | Red: Error, Green: In-use, OFF: Idle |  |
| LED2 | The status of Line 2 | Red: Error, Green: In-use, OFF: Idle |  |
| LED3 | The status of Line 3 | Red: Error, Green: In-use, OFF: Idle |  |
| LED4 | The status of Line 4 | Red: Error, Green: In-use, OFF: Idle |  |

Table 3.4.20 led on BRIB

Function of SW1; default -all OFF when delivering the board

| Pole | Function | ON | OFF | Remark |
| --- | --- | --- | --- | --- |
| 1 | Not used | - | - |  |
| 2 | TEI | Fixed TEI | Automatic TEI | Note 1, 2 |
| 3 | Layer 2 trace | Enable | Disable | Note 3 |
| 4 | Layer 3 trace | Enable | Disable | Note 3 |

Table 3.4.21 switch on BRIB

- Note:

- The function of the 2nd pole has been implemented in firmware version 3.1 or later.

- This is set to OFF position as default, therefore it is important to note that the 2nd pole is set to the proper position after checking the type of TEI in your country.

- These two poles are for the purpose of developing and maintaining BRIB.

Function of various connectors

| Connector | Function | Remark |
| --- | --- | --- |
| CON1 | Serial interface connector | Note1 |
| CON2 | The purpose of development. It is not fixed on the products. |  |
| CON3 | Clock control IN | Note 2 |
| CON4 | Clock control OUT | Note 2 |

Table 3.4.22 connectoron BRIB

- Note

- CON1: Trace for the purpose of development and maintenance.

The external module for RS 232C serial interface should be required for trace.

See GDK-TRC1 in section 2.

- CON3, CON4: They are used to control the reference clock signal when multi-ISDN boards are installed.

The line connector and terminating resistors

| Line No | RJ45 type jack | Terminating Resistors |  |  |
| --- | --- | --- | --- | --- |
|  |  | Switch | All ON | All OFF |
| Line 1 | CON8 | SW5 | terminating | open |
| Line 2 | CON7 | SW4 | terminating | open |
| Line 3 | CON6 | SW3 | terminating | open |
| Line 4 | CON5 | SW2 | terminating | open |

Table 3.4.23 line connector on BRIB

- Note

- Two poles of SW2, SW3, SW4 and SW5 should be set to the ON position to terminate two terminating resistors on a line. (default ON)

- Terminating resistors and selectable switches are introduced only on PCB issue 3 or later, but not on PCB issue 1 and 2. So if you use BRIB issue 1 and 2, the external terminating resistors are required. For the external termination, you can get the terminating unit locally or use GDK-162/100 TERM, which is the terminating unit with 4 P-to-P connections provided by LGE.

RJ45

CON5

CON3(IN)

SW1

1 2 3 4

GDK-100 BRIB

LED1

LED2

LED3

LED4

CON4(OUT)

CON1

U11

(ROM1)

U10

(ROM2)

RJ45

CON6

RJ45

CON7

CON2

RJ45

CON8

1 2 3 4

SW5

1 2

ON

OFF

SW4

1 2

ON

OFF

SW3

1 2

ON

OFF

SW2

1 2

ON

OFF

Figure 3.4.11	BRIB Layout

Check Points

- BRIB boards should be installed in any of the slot No. 1-8 in the 1st KSU, Slot No. 1~9 in the 2nd and 3rd KSU.

- The 2nd pole of SW1 should be set to the proper position according to the type of TEI.

- The RX and TX pairs of line should be correctly connected to the TX and RX pairs of NT1.

- The clock control cable should be connected to the proper position when multi-ISDN cards are installed in a system. See section 3.5.

- Terminating resistors on the line.

### ISDN STIB ( Basic Rate S/T Interface) Installation

STIB card provides 4 interface circuits of Basic Rate format digital CO and/or Extension interface. If more than one ISDN board is required, some special cable should be used for clock signal control. CON3 and CON4 are used to control the clock signal when multi-ISDN boards are installed. Refer to [Clock Control Cable Wiring] of section 3.5.

Function of LED

| No | Function | Status | Remark |
| --- | --- | --- | --- |
| LED1 | The status of Line 1 | Red: Error, Green: In-use, OFF: Idle |  |
| LED2 | The status of Line 2 | Red: Error, Green: In-use, OFF: Idle |  |
| LED3 | The status of Line 3 | Red: Error, Green: In-use, OFF: Idle |  |
| LED4 | The status of Line 4 | Red: Error, Green: In-use, OFF: Idle |  |

Table 3.4.24 led on STIB

Function of SW1: default - all OFF when delivering the board

| Pole | Function | ON | OFF | Remark |
| --- | --- | --- | --- | --- |
| 1 | Not used | - | - |  |
| 2 | TEI | Fixed TEI | Automatic TEI | Note 1, 2 |
| 3 | Layer 2 trace | Enable | Disable | Note 3 |
| 4 | Layer 3 trace | Enable | Disable | Note 3 |

Table 3.4.25 switch on STIB

- Note:

- The function of 2nd pole has been implemented in firmware version 3.1 or later.

- This is set to OFF position as default, therefore it is important to note that 2nd pole is set to the proper position after checking the type of TEI in your country.

- These two poles are for the purpose of developing and maintaining STIB.

Function of various connectors

| Connector | Function | Remark |
| --- | --- | --- |
| CON1 | Serial interface connector | Note1 |
| CON2 | The purpose of development. It is not fixed on the products. |  |
| CON3 | Clock control IN | Note 2 |
| CON4 | Clock control OUT | Note 2 |

Table 3.4.26 connector on STIB

- Note

- CON1: Trace for the purpose of development and maintenance.

The external module for RS 232C serial interface should be required for trace.

See GDK-TRC1 in section 2.

- CON3, CON4: They are used to control the reference clock signal when multi-ISDN boards are installed.  See [Clock Control Cable Wiring] on section 3.5.

The line connector and termination switches

| Line No | RJ45 type Jack | S/T selection |  |  | Terminating Resistors |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Switch | all to “T “ | All to “S” | Switch | all to ON | all to OFF |
| Line 1 | CON8 | SW4 | T | S | SW9 | terminating | open |
| Line 2 | CON7 | SW3 | T | S | SW8 | terminating | open |
| Line 3 | CON6 | SW2 | T | S | SW7 | terminating | open |
| Line 4 | CON5 | SW5 | T | S | SW6 | terminating | open |

Table 3.4.27 line connector on STIB

- Note:

- The setting method of S/T selection switches: default -all T position when delivering the board.

Each line interface can be used for T interface or S interface with different operation interface. If necessary, the S/T selection switches of SW2, SW3, SW4 and/or SW5 can be respectively set to the T or S position by using shorting traps (short pins) supplied on the boards.

- Two poles of SW6, SW7, SW8 and/or SW9 should set to the ON position to terminate two terminating resistors on a line. (default ON)

- Terminating resistors and selectable switches are introduced only on PCB issue 2 or later, but not on PCB issue 1. So if you use STIB issue 1, the external terminating resistors are required. For the external termination, you can get the terminating unit locally or use GDK-162/100 TERM, which is the terminating unit with 4 P-to-P connection provided by LGIC.

Power does not feeding

Power feeding

POWER

T

S

SW2/SW3/SW4/SW5

Shorting trap

(Short pin)

Figure 3.4.12 Configuration of S/T selection connectors

- Note:

- Each line can be selected to T or S interface according to the needs.

2. After setting all lines respectively, the slot(s) of STIB must be assigned as CO slot(s) and STA slot(s) together in Admin field 02 for the proper operation of STIB. See the clause 4.2 and the Programming Manual.

Check Points

- STIB can be installed in any of the slot No. 1-8 in the 1st KSU, and slot No. 1~9 in the 2nd and 3rd KSU.

- Each interface should be set to the correct type, T or S with S/T selection connectors, according to the needs of user. And the RX and TX pairs of line should be correctly connected to the TX and RX pairs of NT1 or TE according to the interface type of each line.

- Terminating resistors on the line.

- The 2nd pole of SW1 should be set to the proper position according to the type of TEI.

- The clock control cable should be connected to the proper position when multi-ISDN cards are installed in a system. See section 3.5.

RJ45

CON5

CON3(IN)

SW1

1 2 3 4

GDK-100 STIB

LED1

LED2

LED3

LED4

CON4(OUT)

CON1

U10

(ROM2)

U11

(ROM1)

RJ45

CON6

RJ45

CON7

CON2

RJ45

CON8

1 2 3 4

SW9

1 2

ON

OFF

SW8

1 2

ON

OFF

SW7

1 2

ON

OFF

SW6

1 2

ON

OFF

CON13

T

S

SW4

T

S

SW3

T

S

SW2

T

S

SW5

Figure 3.4.13	STIB Layout

### ISDN PRIB ( Primary Rate Interface) Installation

The PRIB card provides one interface circuit of PRI (Primary Rate Interface) format digital CO interface.

If more than one ISDN board is required, some special cables should be used for clock signal control. CON6 and CON7 are used to control the clock signal when multi-ISDN boards are installed. Refer to [Clock Control Cable Wiring] of section 3.5.

Function of LED

| LED | Function | Status |  | Remark |
| --- | --- | --- | --- | --- |
|  |  | ON | OFF |  |
| LED1 | PLL Synchronization | Sync. Error | Normal |  |
| LED2 | Loss Of Signal from the Line | LOS | Normal |  |
| LED3 | AIS | Alarm error | Normal |  |
| LED4 | Frame Alignment Error | Error | Normal |  |
| LED5 | Multi-frame Error | Error | Normal |  |
| LED6 | Remote Alarm Error | Error | Normal |  |
| LED7 | CRC Error | Error | Normal |  |
| LED8 | The status of the line | In use | All channel idle |  |

Table 3.4.28 led on PRIB

- Note

- All LEDs will be the status of OFF when normal operation.

- When PRI line is plugged out, LED 2,4 and 5 will be ON.

Function of SW1: default - all OFF when delivering the board

| Pole | Function | ON | OFF | Remark |
| --- | --- | --- | --- | --- |
| 1 | CRC | Disable | Enable |  |
| 2 | Loop back | Enable | Disable | Note 1 |
| 3 | Not used | - | - |  |
| 4 | Trace | Enable | Disable | Note 2 |

Table 3.4.29 lswitch on PRIB

- Note

- This pole is for the purpose of testing PRIB

- This pole are for the purpose of developing and maintaining PRIB.

Function of various connectors

| Connector/Switch | Function | Remark |
| --- | --- | --- |
| CON1 | Serial interface connector( 4pin) | Note 1 |
| CON2 | The purpose of the software development. |  |
| CON3 | NT/TE mode selection for QSIG. ( Default: open - TE) | Note 2 |
| CON5 | Serial interface connector(9-pin RS232C, male) | Note 1 |
| CON6 | Clock control IN | Note 3 |
| CON7 | Clock control OUT | Note 3 |
| CON8 | RJ45 type jack for PRI Line connection | Clause 3.5 |

Table 3.4.30 lconnector on PRIB

- Note

- Access to the board for the purpose of development and maintenance.

Board issue 2 or earlier version:

CON5 is RS 232C, 9 pin male to access the board for the purpose of development and maintenance.. This connector can be directly connected with PC for trace ( PRIB issue 2 or earlier version).

Board issue 2A or later version:

CON5 and application circuits on PRIB are removed on PRIB issue 2A to unify the method of tracing peripheral cards such as PRIB, BRIB, STIB, DVIB and WTIB in all LDK systems.

CON1 is mounted to access the board for the purpose of development and maintenance. The external module(GDK-TRC1) for RS 232C serial interface should be required to access the board through this connector.

See GDK-TRC1 in section 2.

- CON3: NT/TE mode selection

Open circuit between 2 pins: TE mode (normal operation)

Short circuit between 2 pins: NT mode (Special operation, e.g. NT mode in QSIG operation)

- CON6, CON7: They are used to control the clock signal when multi-ISDN boards are installed.

See [Clock Control Cable Wiring] on section 3.5.

Figure 3.4.14PRIB Layout

GDK-100 PRIB

RJ45

CON8

LED8

CON6(IN)

SW1

U200

(ROM2)

1 2 3 4

1   4

LED1

LED2

LED3

LED4

LED5

LED6

LED7

CON7(OUT)

CON3(TEST)

CON2(JT)

CON1

CON5

U201

(ROM1)

Check Points

- One PRIB should be installed on one of the slot No. 1-8 in the 1st KSU, and slot No. 1-9  in the 2nd and 3rd KSU.

- For QSIG operation, check the mode setting method and the contact assignments of RJ45 type connector according to the mode of line , TE or NT. See the clause 3.5.

- The clock control cable should be connected to the proper position when multi-ISDN cards are installed in a system. See section 3.5.

### VMIB (Voice Mail Interface Board) Installation

VMIB provides system announcement, ACD/UCD announcement, and User Greeting. If necessary, the FMEU(Flash Memory Expansion Unit) can be installed on VMIB for the expansion of User Greetings. And, if necessary, the VCEU(ADPCM Unit) can be installed on VMIB for the expansion of user access channels.

Fig 3.4.15 VMIB PCB layout

Function of LED

| No | Description | Status |  | Remark |
| --- | --- | --- | --- | --- |
|  |  | ON | OFF |  |
| LED1 | The status of channel 1 | In use | Idle |  |
| LED2 | The status of channel 2 | In use | Idle |  |
| LED3 | The status of channel 3 | In use | Idle |  |
| LED4 | The status of channel 4 | In use | Idle |  |
| LED5 | The status of channel 5 | In use | Idle |  |
| LED6 | The status of channel 6 | In use | Idle |  |
| LED7 | The status of channel 7 | In use | Idle |  |
| LED8 | The status of channel 8 | In use | Idle |  |
| LED9~11 | Not assemble |  |  |  |
| LED12 | Reserved |  |  |  |

Table 3.4.31 led on VMIB

Function of various connectors

| Connector | Function | Remark |
| --- | --- | --- |
| CN1 | Serial interface connector( 4pin) for trace | Note 1 |
| CN2 | For the purpose of development | Note 2 |
| CN3,CN6ÇN7 | Not used |  |
| CN4~CN5 | VCEU expansion connector |  |
| CN8~CN9 | FMEU expansion connector |  |
| PJ1,PJ2 | Not used |  |

Table 3.4.32 connector on VMIB

- Note

- CN1: Trace for the purpose of development and maintenance.

The external module for RS 232C serial interface should be required for trace using this connector. See GDK-TRC1 in section 2.

- CN2: It is impossible to use them for user access.

These connectors are usually used for the purpose of development and production test.

Function of various switches

| Switch | Function | Remark |
| --- | --- | --- |
| SW1 | SRAM back-up when power interruption or failure | ON |
| SW2 | CPU reset |  |
| SW3,SW5 | Not used | Not Used |
| SW4 | Service/Normal | Normal |

Table 3.4.33 switch on VMIB

- Note

- SW1:This is for the purpose of SRAM back-up for ACD/UCD announcement.

It should be turned ON during installing the VMIB, to protect announcements in the case of a power fail.

“ Caution :

Danger of explosion if battery is incorrectly replaced. Replace only with the same or equivalent type recommended by the manufacturer. Dispose of used batteries according to the manufacturers instruction.”

- SW4 : This is for the purpose of the state of  “in service” or “normal opreration”

Add-on boards: FMEU, VCEU