SECTION 4. LDK-300 EXPANSION(LDK-600)

### 4.1 System Structure

Maximum Capacity in LDK-300 can be expanded using MPBE, LMUE and System Link Cable from 300 ports to 600 ports. MPBE should be installed in the MPB slot of the first KSU in the Master system and LMUE should be installed in the MPB slot of the 4th KSU in the Expansion system as shown in Figure 1-1

LMUE

MPBE

#### Fig 4.1.1 KSU structure

### 4.2 System Feature

The maximum number of boards to be installed is as follows;

- VOIB : 48

- VMIB : 3

- WTIB : 3

- PRIB : 6

### 4.2.1 Maximum Capacity

-. 	Total Port		: 600 ports

-. 	Total Extension Port	: 600 Ports

-. 	Total Trunk Port	: 400 Ports

-. 	Analog CO		: 384 ports (8 ports x 48 cards)

-. 	ISDN BRI		: 152 Chs (8 chs x 19 cards)

-. 	ISDN PRI		: 180 Chs (30 chs x 6 cards)

-. 	DKTU		: 396 ports (12 ports x 33 cards)

-. 	SLT			: 576 ports (12 ports x 48 cards)

-. 	WHTU		: 192 ports

-. 	SIU			: 4 ports ( Basic 2 ports + Option 2 port )

### 4.2.2 System Capacity

| KSU | Max Port (Number of Boards) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Extension |  |  | Network Side |  |  |
|  | So (STIB) | DKTU (DTIB24) | SLT (SLIBII) | PRI (PRIB) | BRI (BRIB) | LCO (LCOB8) |
| 1’st KSU | 56(7) | 96(4) | 96(8) | 120(4) | 56(7) | 64(8) |
| 1’st KSU + 2’nd | 112(14) | 192(8) | 192(16) | 180(6) | 112(14) | 128(16) |
| 1’st KSU + 2’nd + 3’rd | 152(19) | 288(12) | 288(24) | 180(6) | 152(19) | 192(24) |
| 1’st KSU + 2’nd + 3’rd+ 4th |  | 384(16) | 384(32) |  |  | 256(32) |
| 1’st KSU + 2’nd + 3’rd+ 4th + 5th |  | 396(17) | 480(40) |  |  | 320(40) |
| 1’st KSU + 2’nd + 3’rd+ 4th + 5th + 6th |  | 396(17) | 576(48) |  |  | 384(48) |

Table 4.2.1 System Max. call capacity

| Description |  | Capacity | Description |  | Capacity |
| --- | --- | --- | --- | --- | --- |
| Time Slots (note A) | 1’st, 4’th KSU | 144 | Paging Zone | External | 3 (LMUE:1, MISB:2) |
|  | 2’nd, 5’th KSU | 144 |  | Internal | 35 |
|  | 3’rd, 6’th KSU | 96 | System Speed Dial |  | 5000(24digits) |
| Cabinet(KSU) |  | 6 | Station Speed Dial |  | 100(24digits) per station |
| Serial Port(RS-232C) |  | 4(MPBE:2, MISB:2) | Last Number Redial |  | 10 |
| Alarm/Door bell input |  | 3(RAU:2, MISB:1) | Save Number Redial |  | 1 |
| External Control Contact |  | 7(RAU:3, MISB:4) | DSS/DLS Console |  | 7 |
| Music Source Inputs |  | 3(LMUE:1, MISB:2) | SMDR |  | 10000 |
| DTMF Receivers |  | 112(Master : 80, Expansion:32)(note B) | CO Line Group |  | 72 |
| Power Fail Circuit |  | 180 (30/cabinet) | Hunt/UCD Group |  | 48 |
| Intercom Group |  | 15 |  |  |  |

Table 4.2.2 System capacity

* Note A ;

-. 	Time slots are used to allocate flexibly to support each function PSTN, ISDN, VMIB, WTIB, MOH, Paging, Modem, DTMF detection, etc.

-. 	Maximum ports of the 1’st, 2’nd, 4’th and 5’th KSU are 132.

* Note B ;

-. 	1’st KSU : 32 channels including Basic 4 channels in MPBE

-.	2’nd KSU + 3’rd KSU : 48 channels

-. 	4’th KSU + 5’th KSU + 6’th KSU : 32 channels,

-. 	The number of DTMF receivers cannot be shared between master system and expansion system.

-. 	Option : 2chs/DTRU, 4chs/DTRUII, 4chs/DTRU4

### 4.2.3 Board Component

All the boards used in the LDK-300 can be used. There are two kinds of boards which are IPP(Intelligent PeriPheral) including the local CPU and NIPP(Non Intelligent PeriPheral) not including the local CPU. IPP should be installed in only Master system as in the LDK-300 system. NIPP boards can be expanded in the Secondary system.

| No | Board Name | Master System |  |  |  | Expansion System |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1’st KSU | 2’nd & 3’rd |  | 1’st KSU |  | 2’nd & 3’rd |  |
| 1 | MPBE | MPB slot |  |  |  |  |  | Main Process Board for Expansion |
| 2 | LMUE |  |  |  | MPB slot |  |  | Link Module Unit Board for Expansion |
| 3 | IPP (note A) | 1~8 | 1~9 |  |  |  |  | Intelligent PeriPheral Boards |
| 4 | NIPP (note B) | 1~9 |  |  | 1~9 |  |  | Non Intelligent PeriPheral Boards |
| 5 | MISB | 9 only |  |  |  |  |  | Miscellaneous Board |
| 6 | RAU | RAU only |  |  |  |  |  | Relay and Alarm Unit |

Table 4.2.3 System Board Description

* Note A : IPP board is BRIB, STIB, PRIB, VMIB, VMIB2, AAIB, WTIB and VOIB.

* Note B : NIPP board is DTIB12/24, DSIB, LCOB, LCOB8, SLIB24/48, SLIBII and SLIB2E

4.3 Installation

### 4.3.1 MPBE

The MPBE must be used to expand the system beyond the first set of 1’st KSU, 2’nd KSU, and 3’rd KSU. It must be connected with Link Cable to the LMUE in the 4’th KSU in he Expansion system. The MPBE incorporates the system's RAM, master clock, 2 RS-232C ports, and the system's PCM voice processing and main micro-processor. The microprocessor is a 32-bit high speed “RISC” CPU which receives and transmits signaling information from/to other PCBs, controls feature activation, and PCM time-slot interchange. The MPBE has 4 built-in DTMF receivers. LD1 is the PLLU Synchronization Indicator. This LED will be lit when the PLLU on the MPBE has synchronized with an external clock signal provided to the KSU via a BRI, PRI, or T1 circuit. LD2-LD9 indicate the operation of the system. LD10-LD13 are for LAN.

##### Fig 4.3.1 MPBE Configuration

The Real-Time Clock and the 8 MB of SRAM that is associated with the system database are protected in cases of power loss by a long-life high-energy lithium battery. Onboard ROM contains PCM tone, gain table, etc. needed for digital voice processing. The 32MB SDRAM is used for software downloading and additional features. The 8 MB of flash ROM on the Program Module Unit (PMUE) contains operating software. A Modem Unit (MODU) can be installed on the MPBE for remote programming. The MPBE can only be installed in the MPB slot in the 1’st KSU in the Master system.

###### Add-on Boards: MODU and PMUE

## Function of LEDs

The 13 LEDs mounted in the MPBE provide diagnostic information for states of the board. Table 3-1 LEDs on the MPBE shows the meaning of various states of LEDs.

| LED No | Meaning |
| --- | --- |
| LED1 (Red) | ON - PLL circuit activation by external synchronous clock of ISDN board OFF - PLL circuit activation by internal clock |
| LED2 | Not used |
| LED3 | Timer, Flashing every 100msec |
| LED4 | Dial Processing , Flashing every 100msec |
| LED5 | SLT Ring Processing, Flashing every 200msec |
| LED6 | LCD active updating, Flashing every 300msec |
| LED7 | Call Event Processing |
| LED8 | RS-232C Tasking |
| LED9 | Interrupt Processing |
| LED10 | Collision detection for Ethernet port |
| LED11 | Data transmission for Ethernet port |
| LED12 | Data reception for Ethernet port |
| LED13 | Data link for Ethernet port |

Table 4.3.1 LEDs on the MPBE

### Function of SW3

The MPB also has an eight-position DIP switch, SW3. The function of each switch position is as followings;

Default setting: All ON position

| Switch | Function | OFF | ON |
| --- | --- | --- | --- |
| 1 | Administration Programming Access | Disable | Enable |
| 2 | CTS (Clear to Send) of RS-232C | Enable | Disable |
| 3 | Command/Event Trace (The purpose of testing software) | Enable | Disable |
| 4 | Not used | - | - |
| 5 | SMDI (Simplified Message Desk Control –Voice Mail ) | SMDI ON | SMDI OFF |
| 6 | Not used | - | - |
| 7 | Power up status trace of board | Enable | Disable |
| 8 | Database default on power up | Disable | Enable |

Table 4.3.2 The function of each pole of SW3

Before programming the system, switch 8 should be placed in the on position and power cycled off and on to initialize the system database to default. Once the database has been initialized, switch 8 should be placed in the off position so as to protect the database.

After the lithium batteries (SW2 ON) and Add-on boards installed, insert the MPBE in slot MPB of KSU.

Function of various switches and connectors

| Switch/Connector | Functions | Remark |
| --- | --- | --- |
| SW1 | CPU Reset |  |
| SW2 | Lithium Battery ON/OFF switch for Memory and RTC | Database back-up |
| CN1 | Connection between Master system and Expansion system | For expansion |
| CN2 and CN3 | MODU connection |  |
| CN4 and CN5 | RS-232C Serial Interface | System Access |
| CN6 | BDM connection | For Debug Mode |
| CN7 | Card Edge connection |  |
| CN8 | JTAG port for CPLD | For Programming |
| CN9 and CN10 | PMUE connection |  |

Table 4.3.3 Function of various switches and connectors

- * Note:

- MPBE should be installed in MPB slot in the 1st KSU in the Master system only.

- The DIP switch, SW2 should be turned ON position before the MPBE installation to protect system data in the case of a power fail.

- The 8th pole (switch 8) of SW3 should be OFF to protect the variety features by Admin programming after the system power up and initialization. See Table 4.3.2.

- Add-on boards: PMUE, MODU

### 4.3.2 LMUE

The LMUE is used to connect the Master system with the Expansion system. It must be installed in the 4’th KSU (in the slot marked for MPB) and is connected with Link Cable to the MPBE in the 1’st KSU. Signals are transmitted or received via linked cable as shown in Figure 1.1 KSU Structure. The LMUE contains one external page port and one external background music port.

Fig 4.3.2 LMUE Configuration

Function of various switches and connectors

| Switch/Connector | Functions | Remark |
| --- | --- | --- |
| CN1 | Connection between Master system and Expansion system | For expansion |
| PJ1(RED) and PJ2(BLUE) | External paging and MOH connection |  |

Table 4.3.4 Function of various switches and connectors

### 4.4 Wiring

Link cable is used to connect master system with expansion system using ferrite cores against EMI noise in the following figures.

Fig 4.4.1 System Wiring