# SECTION 7.  CUSTOMER DATABASE PROGRAMMING

## 7.1	INTRODUCTION

The LDK Key Telephone System can be programmed to meet each customer's individual need. All programming is done at station 100 (station port # 00) using KD-36D or LKD-30/44 digital key telephone. (You cannot program with Large LCD keyset.) Additional programming stations may be assigned (PGM 113-BTN 1), but only 1 keyset can be active in programming mode at any one time.

Upon entering the program mode, the key telephone at station 100 cannot operate as a normal telephone but as a programming instrument with all of the buttons redefined. The keys of the dial pad are used to enter the various data fields and to enter numerical information. The 24 buttons located at the top of the phone (Flexible Buttons) are used to indicate the specific data field and to enter information. Sometimes the [SPEED] button and ‘*’ of the dial pad is used to delete the data or to indicate end of data input and the [REDIAL] button is used to delete one digit or character from the end of entered digits or characters.

See TABLE 7.4.1 - 7.4.27 for default data. If this pre-programming suits the customer, additional admin program is not necessary. To change admin data, the user enters the admin programming mode and select program code. During admin programming, other keysets operate normally. (While activating LDK Admin, no keyset can enter Admin mode and the reverse also.)

When Admin programming, LCD and LEDs indicate the current programmed data and status. If the programmer enters correct data, then LCD and LEDs show the entered data and the data is stored in the temporary buffer area. Real system database is not changed and has no effect on telephone operation unless permanent updating procedure is executed. Pressing the [HOLD/SAVE] button, all data in the temporary buffer (same as LCD and LEDs show their status) is saved into permanent memory. Tones are provided to let the programmer know data entry is correct (confirmation tone) or not (error tone).

To return the parent state while admin programming, press the [CONF] button. Pressing the [CONF] button, temporary data fields are cleared.

To reset the system, enter PGM 450 – BTN 15 and press [HOLD/SAVE] button. Or, the system will be reset automatically after programming PGM100 – BTN 1(Nation Code Assign).

## TO ENTER THE PROGRAMMING MODE

1. 	Lift handset or press the [MON] button on the admin station, and hear ICM dial tone (optional).

- Press the [TRANS/PGM] button and dial ＊# (Confirmation tone is heard).

- Enter admin password if the password has been set. This places the station into the admin programming mode (Confirmation tone is heard).

4.	Each program is accessed by pressing the [TRANS/PGM] button and dialing the three-digit program number. If an error is made while entering data, the [TRANS/PGM] button can go the previous status. When the [TRANS/PGM] button is pressed, the LCD will display;

ENTER PROGRAM CODE

## PERMANENT UPDATE PROCEDURE

When the data has been entered, the [HOLD/SAVE] button is used to store the data permanently. If all data was entered correctly, confirmation tone is heard when pressing the [HOLD/SAVE] button. If there were any errors in the entry, then an error tone is presented and data is not stored in the permanent memory.

## 7.2	NUMBERING PLAN

The following numbering plan can be changed by Admin Programming 104-107 depending on the user's needs.

| NUMBER |  |  |  |  | ITEM | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
| LDK-300 | LDK-300E |  |  | LDK-100 |  |  |
| 100 – 399 | 1000-1599 |  |  | 100-227 | Intercom Call |  |
| 620 – 667 |  |  |  | 620-634 | Group Pilot Number |  |
| 501 – 535 |  |  |  | 501-515 | Internal Page Zone |  |
| 543 |  |  |  | 543 | Internal All Call Page |  |
| 544 |  |  |  | 544 | Meet Me Page |  |
| 545 |  |  |  | 545 | External Page Zone 1 |  |
| 546 |  |  |  | 546 | External Page Zone 2 |  |
| 547 |  |  |  | 547 | External Page Zone 3 |  |
| 548 |  |  |  | 548 | External All Call Page |  |
| 549 |  |  |  | 549 | All Call Page (Int & Ext) |  |
| 550 |  |  |  | 550 | SMDR Account Code Enter | SLT |
| 551 |  |  |  | 551 | Flash Command to CO Line | SLT |
| 552 |  |  |  | 552 | Last Number Redial | SLT |
| 553 |  |  |  | 553 | DND (Toggle On/Off) | SLT |
| 554 |  |  |  | 554 | Call Forward | SLT |
| 555 |  |  |  | 555 | Speed Dial Programming | SLT |
| 556 |  |  |  | 556 | Message Wait/Callback Enable | SLT |
| 557 |  |  |  | 557 | Message Wait/Callback Return | SLT |
| 558 |  |  |  | 558 | Speed Dial Access | SLT |
| 559 |  |  |  | 559 | Cancel DND/FWD/Pre-MSG | SLT |
| 560 |  |  |  | 560 | System Hold | SLT |
| 561 |  |  |  | 561 | Station Relocation Backup |  |
| 562 |  |  |  | 562 | Station Relocation Retrieve |  |
| 563 |  |  |  | 563 | Programming Mode Enter Code | SLT |
| 564 |  |  |  | 564 | ACD Reroute |  |
| 565 |  |  |  | 565 | Alarm Reset |  |
| 566 |  |  |  | 566 | Group Call Pickup |  |
| 568 |  |  |  | 568 | UCD DND |  |
| 569 |  |  |  | 569 | Night Answer |  |
| 601 - 619 |  |  |  | 601-610 | Call Parking Locations |  |
| 7 |  |  |  | 7 | Direct Call Pickup |  |
| 801-872 |  |  |  | 801-824 | CO Group Access |  |
| 88001-88200 |  |  |  | 8801-8840 | Individual CO Access |  |
| 8901 |  |  |  | 8901 | Tie Routing Access |  |
| 8* |  |  | 8* |  | Retrieve Held CO Line |  |
| 8#xxx |  |  | 8#xx |  | Retrieve Held Individual CO Line |  |
| 9 |  |  | 9 |  | Access CO Line In the 1st available CO Group |  |
| 0 |  |  | 0 |  | Attendant Call |  |
| #*1 |  |  | #*1 |  | 1st Door Open |  |
| #*2 |  |  | #*2 |  | 2nd Door Open |  |
| NUMBER |  |  |  |  | ITEM | REMARK |
| LDK-300/300E |  | LDK-100 |  |  |  |  |
| #*3 |  | #*3 |  |  | 3rd Door Open |  |
| #*4 |  | #*4 |  |  | 4th Door Open |  |
| #*5 |  | #*5 |  |  | 5th Door Open |  |
| #*6 |  | #*6 |  |  | 6th Door Open |  |
| #*7 |  | N/A |  |  | 7th Door Open | LDK-300/300E Only |
| *8 |  | *8 |  |  | VM Message Waiting Enable |  |
| *9 |  | *9 |  |  | VM Message Waiting Disable |  |

- To enter user programming mode, press the [TRANS/PGM] button in a keyset or dial 5 6 3 (Programming enter code) in a SLT.

- The following numbering plan is fixed, so it cannot be changed by Admin Programming.

- For the stations

| NUMBER | ITEM | REMARK |
| --- | --- | --- |
| 11 | Differential Ring | Keyset |
| 12 | Intercom Answer Mode (1 HF / 2 TONE / 3 PV) | Keyset |
| 21 | Station COS Down |  |
| 22 | Station COS Restore |  |
| 23 | Walking COS | Keyset |
| 31 | Authorization Code Registration |  |
| 32 | Authorization Code Change |  |
| 41 | Wake-up Time Registration (One-time/ Continuous) |  |
| 42 | Wake-up Time Cancel |  |
| 51 | Pre-selected MSG Activation |  |
| 52 | Set Custom Message |  |
| 61 | Record VMIB User Greeting |  |
| 62 | Listen VMIB Time & Date |  |
| 63 | Listen VMIB Station Number |  |
| 64 | Listen VMIB Station Status |  |
| 65 | Record VMIB Page Message |  |
| 66 | Erase VMIB User Greeting |  |
| 67 | Erase VMIB Page Message |  |
| 71 | LCD Display Mode (English/Domestic Language) | Keyset |
| 72 | MPB Version Display | Keyset |
| 73 | Background Music | Keyset |
| 74 | Station User Name Registration |  |
| 75 | Headset/Speakerphone Mode | Keyset |
| 76 | Headset Ring Mode | Keyset |
| 77 | WTU Station Number Receive | Keyset |
| 78 | Serial No/SW Packages | Keyset with LCD |

- For the attendant

| NUMBER | ITEM |  | REMARK |
| --- | --- | --- | --- |
| 0111 | Print SMDR (Station Base) |  | System Attendant |
| 0112 | Delete SMDR (Station Base) |  | System Attendant |
| 0113 | Print SMDR (Group Base) |  | System Attendant |
| 0114 | Delete SMDR (Group Base) |  | System Attendant |
| 0115 | Display Call Charge |  | System Attendant |
| 0116 | Abort Printing |  | System Attendant |
| 0117 | Print Lost Call |  | System Attendant |
| 0118 | Delete Lost Call |  | System Attendant |
| 0121 | Print All Summary | System Attendant |  |
| 0122 | Print All Periodically | System Attendant |  |
| 0123 | Abort Periodic Printing | System Attendant |  |
| 0124 | Print ATD Traffic | System Attendant |  |
| 0125 | Print Call Summary | System Attendant |  |
| 0126 | Print All Hourly | System Attendant |  |
| 0127 | Print H/W Usage | System Attendant |  |
| 0128 | Print CO Summary | System Attendant |  |
| 0129 | Print CO Hourly | System Attendant |  |
| 021 | Station COS Down (COS 7) |  | Attendant |
| 022 | Station COS Restore |  | Attendant |
| 031 | Authorization Code Cancel |  | System Attendant |
| 041 | System Date/Time Setting |  | Attendant |
| 042 | Wake-up Time Registration (One-time /Continuous) |  | Attendant |
| 043 | Wake-up Time Cancel |  | System Attendant |
| 044 | LCD Date Mode Change |  | System Attendant |
| 045 | LCD Time Mode Change |  | System Attendant |
| 046 | Use Network Time & Date |  | System Attendant |
| 051 | Pre-select MSG Activation |  | Attendant |
| 052 | Pre-select MSG Deactivation |  | Attendant |
| 053 | Custom Display Message Program (11-20) |  | System Attendant |
| 054 | Erase VM MSG |  | Attendant |
| 06 | Record VMIB System Greeting | System Attendant |  |
| 071 | DND/Call Forward/Pre-selected MSG Cancel |  | Attendant |
| 072 | Register Station Name |  | Attendant |
| 073 | Disable CO Outgoing |  | System Attendant |
| 074 | Automatic Day/Night/Weekend Mode Program |  | Attendant |
| 075 | ICM BOX BGM Channel select |  | Attendant |
| 076 | External Page Music -1 Assignment/Cancel |  | Attendant |
| 077 | External Page Music -2 Assignment/Cancel |  | Attendant |
| 078 | External Page Music -3 Assignment/Cancel |  | Attendant |
| 079 | Prepaid Call |  |  |
| 07* | LCD Display Language |  |  |
| 0# | WHTU Subscription |  |  |

- Flexible Button Programming Code

| NUMBER | ITEM | REMARK |
| --- | --- | --- |
| 11 | Differential Ring |  |
| 21 | Station COS Down |  |
| 22 | Station COS Restore |  |
| 23 | Walking COS |  |
| 31 | Authorization Code Registration |  |
| 32 | Authorization Code Change |  |
| 41 | Wake-up Time Registration (One-time /Continuous) |  |
| 42 | Wake-up Time Cancel |  |
| 51 | Pre-selected MSG Activation |  |
| 52 | Set Custom Message |  |
| 53 | CLIR Key |  |
| 54 | Two Way Recording |  |
| 55 | Attendant DND | Networking Only |
| 56 | Attendant Camp On(Queue) BTN Assignment | Attendant |
| 61 | Record VMIB User Greeting |  |
| 64 | Listen VMIB Station Status |  |
| 66 | Erase VMIB User Greeting |  |
| 71 | LCD Display Mode (English/Domestic Language) |  |
| 73 | Background Music |  |
| 74 | Station User Name Registration |  |
| 75 | Headset/Speakerphone Mode |  |
| 76 | HeadSet Ring Mode |  |
| 80 | Account Code Activation |  |
| 81 | DID Call Wait |  |
| 83 | [ICM Hold] BTN Assignment |  |
| 84 | [LOOP] BTN Assignment |  |
| 85 | [Camp-on] BTN Assignment |  |
| 86 | [INTRUSION] BTN Assignment | System Attendant |
| 87 | [UCD DND] BTN Assignment | + Hunt Grp No. |
| 89 | Keypad Facility Key |  |
| 8* | {ACD STATUS} BTN Assignment |  |
| 91 | [CONF] BTN Assignment | 2/8 BTN Keyset |
| 92 | [CALLBK] BTN Assignment | 2/8 BTN Keyset |
| 93 | [DND/FWD] BTN Assignment | 2/8 BTN Keyset |
| 94 | [FLASH] BTN Assignment | 2/8 BTN Keyset |
| 95 | [MUTE] BTN Assignment | 2/8 BTN Keyset |
| 96 | [MON] BTN Assignment | 2/8 BTN Keyset |
| 97 | [REDIAL] BTN Assignment | 2/8 BTN Keyset |

## 7.3	ADMIN PROGRAMMING INDEX

| MAIN MENU | PGM | ITEM |
| --- | --- | --- |
| PRE-PROGRAMMED DATABASE | 100 | Location Program |
|  | 101 | Rack Slot Assignment |
|  | 102 | WTIB Port number Assignment |
|  | 103 | Logical Slot Assignment |
|  | 104 | Numbering Plan Type |
|  | 105 | Flexible Number Plan – Station Number |
|  | 106 | Flexible Number Plan A |
|  | 107 | Flexible Number Plan B |
|  | 108 | IP Setting |
|  | 109 | Flexible Number Plan C |
| STATION BASE PROGRAM | 110 | Station ID |
|  | 111 | Station Attribute I |
|  | 112 | Station Attribute II |
|  | 113 | Station Attribute III |
|  | 114 | ISDN Station Attribute |
|  | 115 | Flex Button Assignment |
|  | 116 | Station COS |
|  | 117 | CO Line Group Access |
|  | 118 | Internal Page Zone |
|  | 119 | Conference Page Zone |
|  | 120 | ICM Tenancy Group |
|  | 121 | Preset Call Forward |
|  | 122 | Hot/Warm Line Selection |
|  | 123 | CTI Station Attribute |
|  | 124 | SMDR Account Group |
|  | 125 | Copy DSS Button |
|  | 130 | Display Stations by COS |
|  | 131 | Display Stations by CO Group Access |
| CO LINE BASE PROGRAM | 140 | CO Service Type |
|  | 141 | CO Line Attribute I |
|  | 142 | CO Line Attribute II |
|  | 143 | ISDN CO Line Attribute I |
|  | 144 | CO Ring Assignment |
|  | 145 | CO Ring Assignment Display |
|  | 146 | CO Line Attribute III |
|  | 147 | CO MSN Mapping Table |
| SLOT BASE PROGRAM | 155 | Slot Attribute |
| SYSTEM BASE PROGRAM | 160 | System Attribute – I |
|  | 161 | System Attribute – II |
|  | 162 | Admin Password |
|  | 163 | Alarm Attributes |
|  | 164 | Attendant Assignment |
|  | 165 | Auto Attendant VMIB Annc. Assignment |
|  | 166 | CO-to-CO COS |
|  | 167 | DID/DISA Destination |
|  | 168 | External Control Contact |
|  | 169 | LCD Date/Time/Language Display Mode |
| MAIN MENU | PGM | ITEM |
| SYSTEM BASE PROGRAM | 170 | Modem |
|  | 171 | Music |
|  | 172 | PBX Access Code |
|  | 173 | PLA Priority Setting |
|  | 174 | RS-232C Port Setting |
|  | 175 | Print Port Selection |
|  | 176 | Pulse Dial Ratio |
|  | 177 | SMDR Attributes |
|  | 178 | System Date/Time Setting |
| SYSTEM BASE PROGRAM | 179 | Linked Station Pairs Table |
|  | 180 | System Timers – I |
|  | 181 | System Timers – II |
|  | 182 | System Timers – III |
|  | 185 | CIDU Setting |
| DCOB | 186 | DCOB System attribute |
|  | 187 | DCOB CO Line Attribute |
| STATION GROUP | 190 | Station Group Assign |
|  | 191 | Station Group Attribute |
| ISDN SYSTEM BASE PROGRAM | 200 | ISDN Attributes |
|  | 201 | COLP Table |
|  | 202 | MSN Table |
| TABLES | 220 | LCR Attributes |
|  | 221 | LCR – Leading Digit Table |
|  | 222 | LCR – Digit Modification Table |
|  | 223 | LCR Table Initialization |
|  | 224 | Toll Exception Table – Allow A (Entry no:01-30) |
|  |  | Toll Exception Table – Deny A (Entry no:01-30) |
|  |  | Toll Exception Table – Allow B (Entry no:01-30) |
|  |  | Toll Exception Table – Deny B (Entry no:01-30) |
|  | 225 | Canned Toll Table –Allow (Entry no:01-10) |
|  |  | Canned Toll Table –Deny (Entry no:01-10) |
|  | 226 | Emergency Code Table |
|  | 227 | Authorization Code Table |
|  | 228 | Customer Call Routing |
|  | 229 | Executive/Secretary Table |
|  | 231 | Flexible DID Table |
|  | 232 | System Speed Zone |
|  | 233 | Weekly Time Table |
|  | 234 | Voice Mail Dialing Table |
|  | 235 | Tie Routing Table |
| NETWORKING | 320 | Networking Basic Attribute |
|  | 321 | Networking Supplementary Attribute |
|  | 322 | Networking CO Line Attribute |
|  | 323 | Networking Attendant Assignment |
|  | 324 | Networking Routing Table |
| VOIB | 340 | VOIB IP Setting |
| NATION SPECIFIC | 400 | DTIB Rx Gain Control |
|  | 401 | SLIB Rx Gain Control |
|  | 402 | SLIB12 Rx Gain Control |
|  | 403 | WTIB Rx Gain Control |
| MAIN MENU | PGM | ITEM |
| NATION SPECIFIC | 404 | ACOB Rx Gain Control |
|  | 405 | ACOB8 Rx Gain Control |
|  | 406 | DCOB Rx Gain Control |
|  | 407 | VMIB Rx Gain Control |
|  | 408 | DTMF Receiver Rx Gain Control |
|  | 409 | EXT Page Rx Gain Control |
|  | 410 | CPTU Rx Gain Control |
|  | 411 | Modem Rx Gain Control |
|  | 412 | Short SLIB Gain Control |
|  | 413 | Long SLIB Gain Control |
|  | 414 | Far SLIB Gain Control |
|  | 415 | Short ACO Gain Control |
|  | 416 | Long ACO Gain Control |
|  | 420 | System Tone Frequency |
|  | 421 | Differential Ring Frequency |
|  | 422 | Distinct CO Ring Frequency |
|  | 423 | ACNR Tone Cadence |
|  | 425 | Singular Table (Korea Only) |
| INITIALIZATION (DB INIT) | 450 | Initialization |
| PRINT DATABASE | 451 | Print Prot Database |

## 7.4	DEFAULT VALUES

#### TABLE 7.4.1  LOCATION PROGRAM

| PGM | BTN | ITEM | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 100 | 1 | Nation Code | 82 | Max 4 digits |
|  | 2 | Customer Site Name | . | Max 23 digits |

#### TABLE 7.4.2  RACK SLOT ASSIGNMENT

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 101 | - | Slot Assignment | Refer to Note 2 | Refer to Note 1 | In case of PRIB assignment, it is possible to program logical port number. |

Note 1) 	If the DIP switch of the manual board detection (DIP Switch 8) is ON, system will detect the installed board type automatically. If the DIP switch 8 is OFF, the board type code must be entered at each slot. After manual Rack Slot assignment, user should reset the system manually.

Note 2) 	Board Type Code Table:

| STA | CODE | COL | CODE | STA & COL | CODE | Etc | CODE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DTIB12 DTIB24 SLIB6 SLIB12 WTIB SLIB2E DSIB | 11 12 13 14 15 16 18 | PRIB BRIB LCOB4 LCOB8 DIDB BWDIDB TLIB EMIB AC15 DCOB VOIB NPRIB NBRIB(8) NBRIB(4) CLCOB8 RDIB EMIB8 CLCOB4 | 31 32 33 34 35 36 37 38 39 40 41 42(LDK-100) 43(LDK-100) 44(LDK-100) 45 46 47 49 | STIB | 51 | VMIB MISB | 61 71 |

#### TABLE 7.4.3  WTIB PORT NUMBER ASSIGNMENT

| PGM | BTN | ITEM | RANGE |  | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | LDK-300/300E | LDK-100 |  |  |
| 102 | - | WTIB Port Number Assignment | 008 – 192 (Multiple of 8) | 08-80 (Multiple of 8) | 8 | When LDK-100 has one rack : 08-40 |

#### TABLE 7.4.4  LOGICAL SLOT ASSIGNMENT

| PGM | BTN | ITEM | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 103 | 1 | COL Board | Refer to Note |  |
|  | 2 | STA Board | Refer to Note |  |
|  | 3 | VMIB | Not Assigned |  |

Note) 	If the DIP switch of the manual board detection DIP Switch 8) is ON, system will detect the logical slot assign in sequence as increase order automatically. If the DIP switch 8 is OFF, the logical slot assignment must be entered at each board type. After manual logical slot assignment, user should reset the system manually.

#### TABLE 7.4.5  NUMBERING PLAN TYPE

| PGM | ITEM | STA RANGE |  |  | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | LDK-300E | LDK-300 | LDK-100 |  |  |
| 104 | Number Set Type 1 | 1000 – 1599 | 100 – 399 | 100 – 227 | Type 1 | As the basic type, the 1st digit of station number should be 1 – 4. |
|  | Number Set Type 2 | 1000 – 1599 | 100 – 399 | 100 – 227 |  | The station number can be changed within 799. |
|  | Number Set Type 3 | 1000 – 1599 | 100 – 399 | 100 – 227 |  |  |
|  | Number Set Type 4 | 7000 – 7599 | 700 – 999 | 700 – 827 |  |  |
|  | Number Set Type 5 | 2000 – 2599 | 200 – 499 | 200 – 327 |  |  |
|  | Number Set Type 6 | 21 – 79 | 21 – 79 | 21 – 79 |  | Max Station Ports: 60 Stations above Max ports will be displayed “***” |
|  | Number Set Type 7 | 1000 – 1299 | 100 – 299 | 100 – 227 |  | Max Station Ports: 200 Stations above Max ports will be displayed “***” |
|  | Number Set Type 8 | 1000 – 1599 | 100 – 399 | 100 – 227 |  | The station number can be changed within 999. |

#### TABLE 7.4.6  FLEXIBLE NUMBERING PLAN

| PGM | BTN | FIELD | NUMBER SET1 | NUMBER SET2 | NUMBER SET3 | NUMBER SET4 | NUMBER SET5 | NUMBER SET6 | NUMBER SET7 | NUMBER SET8 | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 105 | - | Intercom Call | 1000 - 1599 | 1000 - 1599 | 100 - 1599 | 7000 - 7599 | 2000 - 2599 | 10 - 79 | 1000 - 1299 | 1000 - 1599 | LDK-300E |
|  |  |  | 100 - 399 | 100 - 399 (100 - 799) | 100 - 399 | 700 - 999 | 200 - 499 | 10 - 79 | 100 - 299 | 100 - 399 (100 - 999) | LDK-300 |
|  |  |  | 100 -227 | 100 - 227 (100 - 799) | 100 - 227 | 700 - 827 | 200 - 327 | 10 - 79 | 100 - 227 | 100 - 227 (100 - 999) | LDK-100 |
| 106 | 1 | Group Pilot Number | 620 - 667 | *620 - *667 | 620 - 667 | 620 - 667 | 620 - 667 | *620 - *667 | 620 - 667 | *620 - *667 | LDK-300E |
|  |  |  | 620 - 634 | *620 - *634 | 620 - 634 | 620 - 634 | 620 - 634 | *620 - *634 | 620 - 634 | *620 - *634 | LDK-100 |
|  | 2 | Internal Page Zone | 501 - 535 | *501 - *535 | #01 - #35 | #01 - #35 | #01 - #35 | *501 - *535 | 401 - 419 | *501 - *535 | LDK-300E |
|  |  |  | 501 - 515 | *501 - *515 | #01 - #15 | #01 - #15 | #01 - #15 | *501 - *515 | 401 - 415 | *501 - *515 | LDK-100 |
|  | 3 | Internal All Call Page | 543 | *543 | #5 | #7 | #5 | *543 | 43 | *543 |  |
|  | 4 | Meet Me Page | 544 | *544 | ## | ## | ## | *544 | 44 | *544 |  |
|  | 5 | External Page Zone 1 | 545 | *545 | #6 | #41 | #6 | *545 | 45 | *545 |  |
|  | 6 | External Page Zone 2 | 546 | *546 | #7 | #42 | #7 | *546 | 46 | *546 |  |
|  | 7 | External Page Zone 3 | 547 | *547 | #8 | #43 | #8 | *547 | 47 | *547 |  |
|  | 8 | External All Call Page | 548 | *548 | #9 | #5 | #9 | *548 | 48 | *548 |  |
|  | 9 | All Call Page (Int & Ext) | 549 | *549 | #00 | #6 | #00 | *549 | 49 | *549 |  |
|  | 10 | SMDR Account Code Enter | 550 | *550 | 550 | 550 | 50 | *550 | 50 | *550 | SLT |
|  | 11 | Flash Command to CO Line | 551 | *551 | 551 | 551 | 51 | *551 | 51 | *551 | SLT |
|  | 12 | Last Number Redial | 552 | *552 | 552 | 552 | 52 | *552 | 52 | *552 | SLT |
|  | 13 | DND (Toggle On/Off) | 553 | *553 | 553 | 553 | 53 | *553 | 53 | *553 | SLT |
|  | 14 | Call Forward | 554 | *554 | 554 | 554 | 54 | *554 | 54 | *554 | SLT |
|  | 15 | Speed Dial Programming | 555 | *555 | 555 | *40 | 55 | *555 | 55 | *555 | SLT |
|  | 16 | Message Wait/Callback Enable | 556 | *556 | 556 | *66 | 56 | *556 | 56 | *556 |  |
|  | 17 | Message Wait/Callback Return | 557 | *557 | 557 | *67 | 57 | *557 | 57 | *557 | SLT |
|  | 18 | Speed Dial Access | 558 | *558 | 558 | *7 | 58 | *558 | 58 | *558 | SLT |
|  | 19 | Cancel DND/FWD/Pre-MSG | 559 | *559 | 559 | 559 | 59 | *559 | 59 | *559 | SLT |
|  | 20 | System Hold | 560 | *560 | 560 | 560 | 690 | *560 | 30 | *560 | SLT |
|  | 21 | Station Relocation Backup | 561 | *561 | 561 | 561 | 691 | *561 | … | *561 |  |
|  | 22 | Station Relocation Retrieve | 562 | *562 | 562 | 562 | 692 | *562 | … | *562 |  |
|  | 23 | Programming Mode Enter Code | 563 | *563 | 563 | 563 | 693 | *3 | 33 | *3 | SLT |
|  | 24 | ACD Reroute | 564 | *564 | 564 | 564 | 694 | *4 | 34 | *4 |  |
| PGM | BTN | FIELD | NUMBER SET1 | NUMBER SET2 | NUMBER SET3 | NUMBER SET4 | NUMBER SET5 | NUMBER SET6 | NUMBER SET7 | NUMBER SET8 | REMARK |
| 107 | 1 | Alarm Reset | 565 | *565 | 565 | *565 | 695 | *565 | 35 | *565 |  |
|  | 2 | Group Call Pickup | 566 | *566 | ** | *1 | ** | *566 | 36 | *566 |  |
|  | 3 | UCD DND | 568 | *568 | 568 | 568 | 698 | *568 | 68 | *568 |  |
|  | 4 | Night Answer | 569 | *569 | 577 | 2 | 699 | *569 | 69 | *569 |  |
|  | 5 | Call Parking Locations | 601 - 619 | *601 -* 619 | 601 – 619 | 601-619 | 601 - 619 | *601 -* 619 | 601 - 619 | *601 -* 619 | LDK-300/300E |
|  |  |  | 601 - 610 | *601 -* 610 | 601 – 610 | 601-610 | 601 - 610 | *601 -* 610 | 601 - 610 | *601 -* 610 | LDK-100 |
|  | 6 | Direct Call Pickup | 7 | *7 | *7 | *42 | 7 | *7 | 7 | *7 |  |
|  | 7 | CO Group Access | 801-872 | 801-872 | 801-872 | 401-472 | 801-872 | 801-872 | 801-872 | #801-#872 | LDK-300/300E |
|  |  |  | 801-824 | 801-824 | 801-824 | 401-424 | 801-824 | 801-824 | 801-824 | #801-#824 | LDK-100 |
|  | 8 | Individual CO Access | 88001-88400 | 88001-88400 | 88001-88400 | 48001-48400 | 88001-88400 | 88001-88400 | 88001-88400 | #88001-#88400 | LDK-300E |
|  |  |  | 88001-88200 | 88001-88200 | 88001-88200 | 48001-48200 | 88001-88200 | 88001-88200 | 88001-88200 | #88001-#88200 | LDK-300 |
|  |  |  | 8801-8840 | 8801-8840 | 8801-8840 | 4801-4840 | 8801-8840 | 8801-8840 | 8801-8840 | #8801-#8840 | LDK-100 |
|  | 9 | Tie Routing Access | 8901 | 8901 | 8901 | 4901 | 89 | 8901 | 8901 | #401 |  |
|  | 10 | Retrieve Held CO Line | 8* | 8* | 8* | 4* | 8* | 8* | 8* | #8* |  |
|  | 11 | Retrieve Held Individual CO Line | 8#xxx | 8#xxx | 8#xxx | 4#xxx | 8#xxx | 8#xxx | 8#xxx | #8#xxx | LDK-300/300E |
|  |  |  | 8#xx | 8#xx | 8#xx | 4#xx | 8#xx | 8#xx | 8#xx | #8#xx | LDK-100 |
|  | 12 | Access CO Line In the 1st available CO Group | 9 | 9 | 9 | 1 | 0 | 9 | 9 | 0 |  |
|  | 13 | Attendant Call | 0 | 0 | 0 | 0 | 9 | 0 | 0 | #9 |  |
|  | 14 | 1st Door Open | #*1 | #*1 | #*1 | #*1 | #*1 | #*1 | #*1 | #*1 |  |
|  | 15 | 2nd Door Open | #*2 | #*2 | #*2 | #*2 | #*2 | #*2 | #*2 | #*2 |  |
|  | 16 | 3rd Door Open | #*3 | #*3 | #*3 | #*3 | #*3 | #*3 | #*3 | #*3 |  |
|  | 17 | 4th Door Open | #*4 | #*4 | #*4 | #*4 | #*4 | #*4 | #*4 | #*4 |  |
|  | 18 | 5th Door Open | #*5 | #*5 | #*5 | #*5 | #*5 | #*5 | #*5 | #*5 |  |
|  | 19 | 6th Door Open | #*6 | #*6 | #*6 | #*6 | #*6 | #*6 | #*6 | #*6 |  |
|  | 20 | 7th Door Open | #*7 | #*7 | #*7 | #*7 | #*7 | #*7 | #*7 | #*7 | LDK-300/300E |
|  |  |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | LDK-100 |
|  | 21 | VM Message Waiting Enable | *8 | *8 | *8 | *8 | *8 | *8 | *8 | *8 |  |
|  | 22 | VM Message Waiting Disable | *9 | *9 | *9 | *9 | *9 | *9 | *9 | *9 |  |
| 109 | 1 | MCID Request | *0 | *0 | *0 | *0 | *0 | *0 | *0 | *0 |  |

#### TABLE 7.4.7  IP SETTING

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 108 | 1 | IP Name | Max 16 |  |  |
|  | 2 | Server IP Address | 12 Digits |  | Skip : # |
|  | 3 | CLI IP Address | 12 Digits |  |  |
|  | 4 | Gateway Address | 12 Digits |  |  |
|  | 5 | Subnet Mask | 12 Digits | 255.255.255.0 |  |

#### TABLE 7.4.8  STATION ID ASSIGNMENT

| PGM | BTN | ITEM | RANGE |  | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | LDK-300/300E | LDK-100 |  |  |
| 110 | 1 | ID | 01-16 | 01-11 |  |  |
|  | 2 | DSS/DLS MAP – Associate STA | STA # | STA # |  |  |

#### TABLE 7.4.9  STATION ATTRIBUTE I/II/III

| PGM | BTN | ITEM |  |  |  | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 111 | 1 | Auto Speaker Selection |  |  |  | ON / OFF | ON |  |
|  | 2 | Call Forward |  |  |  | ON / OFF | ON |  |
|  | 3 | DND |  |  |  | ON / OFF | ON |  |
|  | 4 | Data Line Security |  |  |  | ON / OFF | OFF |  |
|  | 5 | Howling Tone to SLT |  |  |  | ON / OFF | ON |  |
|  | 6 | ICM Box Signaling |  |  |  | ON / OFF | OFF |  |
|  | 7 | No Touch Answer |  |  |  | ON / OFF | ON |  |
|  | 8 | Page Access |  |  |  | ON / OFF | OFF |  |
|  | 9 | Ring Type |  |  |  | 1 - 4 | 1 |  |
|  | 10 | Speaker/Headset Ring |  |  |  | SP/HEAD/ BOTH | Speaker-Phone |  |
|  | 11 | Speaker Phone/Headset |  |  |  | ON/OFF | ON |  |
|  | 12 | VMIB Slot |  |  |  | 0 - 2 (LDK-300/300E) 0 - 1 (LDK-100) | 0 |  |
|  | 13 | ICM Group |  |  |  | 01 - 15 (LDK-300/300E) 01 - 05 (LDK-100) | 01 |  |
|  | 14 | Error Tone for TAD |  |  |  | ON / OFF | OFF |  |
|  | 15 | SLT Flash Drop |  |  |  | ON / OFF | OFF |  |
|  | 16 | Loop LCR Account Code |  |  |  | ON / OFF | OFF |  |
|  | 17 | VMIB MSG TYPE |  |  |  | FIFO/LIFO | LIFO |  |
|  | 18 | OFFNET FWD |  |  |  | EN/DIS | ENABLE |  |
| 112 | 1 | CO Warning Tone |  |  |  | ON / OFF | OFF |  |
|  | 2 | Automatic Hold |  |  |  | ON / OFF | OFF | STA2:ON |
|  | 3 | CO Call Time Restriction |  |  |  | ON / OFF | OFF |  |
|  | 4 | Ind CO Line Access |  |  |  | EN/DIS | ENABLE |  |
|  | 5 | CO Line Queuing |  |  |  | EN/DIS | ENABLE |  |
|  | 6 | CO PGM |  |  |  | EN/DIS | DISABLE |  |
|  | 7 | PLA |  |  |  | EN/DIS | ENABLE |  |
|  | 8 | Prepaid Call |  |  |  | ON / OFF | OFF |  |
| PGM | BTN | ITEM |  |  |  | RANGE | DEFAULT | REMARK |
| 112 | 9 | Speed Dial Access |  |  |  | EN/DIS | ENABLE |  |
|  | 10 | Two Way Record |  |  |  | ON / OFF | OFF |  |
|  | 11 | Fax Mode |  |  |  | ON / OFF | OFF |  |
|  | 12 | OFFNET Call Mode |  |  |  | EXT/ALL | ALL |  |
|  | 13 | UCD Grp Service |  |  |  | ON / OFF | OFF |  |
|  | 14 | Ring Grp Service |  |  |  | ON / OFF | OFF |  |
|  | 15 | Stop Camp On Tone |  |  |  | EN/DIS | DISABLE |  |
|  | 16 | Line Length |  |  |  | SHORT / LONG / FAR | SHORT (Short:0km, Long:0~3km, Far:3~7.5km) | SAF only |
|  | 17 | MSG SCROLL SPEED |  |  |  | 0 - 7 | 3 | LDK-30DH |
|  | 18 | BLOCK BACK CALL for SLT |  |  |  | ON/OFF | OFF |  |
| 113 | 1 | Admin |  |  |  | EN/DIS | DISABLE | STA100 Enable |
|  | 2 | VMIB Access |  |  |  | EN/DIS | DISABLE |  |
|  | 3 | Group Listening |  |  |  | EN/DIS | DISABLE |  |
|  | 4 | Override Privilege |  |  |  | EN/DIS | DISABLE |  |
|  | 5 | SMDR Hidden Dialed Digits |  |  |  | EN/DIS | DISABLE |  |
|  | 6 | Voice Over |  |  |  | EN/DIS | DISABLE |  |
|  | 7 | Warm Line | Hot Line / Warm Line |  |  | Warm Line |  |  |
|  | 8 | DVU MSG Retrieve Password |  |  |  | ON/OFF | OFF |  |
|  | 9 | DVU MSG Retrieve Date/Time |  |  |  | ON/OFF | ON |  |
|  | 10 | Alarm Attribute |  | LDK-300/300E | MISB RAU1 RAU2 | (ON/OFF) (ON/OFF) (ON/OFF) | OFF OFF OFF |  |
|  |  |  |  | LDK-100 | MPB MISB | (ON/OFF) (ON/OFF) | OFF OFF |  |

#### TABLE 7.4.10  ISDN STATION ATTRIBUTE

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 114 | 1 | CLIP LCD Display | ON / OFF | ON |  |
|  | 2 | COLP LCD Display | ON / OFF | OFF |  |
|  | 3 | CLI / REDIRECT Display | CLI/ REDIRECT | CLI |  |
|  | 4 | CLI MSG Wait | ON / OFF | OFF |  |
|  | 5 | EXT or CO ATD | ATD/EXT | EXT |  |
|  | 6 | Keypad Facility | KEYPAG/DTMF | DTMF |  |
|  | 7 | Long/Short | LONG/SHORT | SHORT |  |
|  | 8 | CPN Type | 0-2 | 0(Not used) |  |
|  | 9 | S0 Sub-address | 0-2 | 0(Not used) |  |
|  | 10 | Reserved | - | - |  |
|  | 11 | CLI Name Display | ON/OFF | OFF |  |
|  | 12 | ISDN CLI Station Number | Max 4 digits | Logical STA Number |  |
|  | 13 | Progress Indication | ON / OFF | OFF |  |
|  | 14 | ISDN CLIR | ON / OFF | OFF |  |
|  | 15 | ISDN COLR | ON / OFF | OFF |  |
|  | 16 | DID Restriction | ON / OFF | OFF |  |
|  | 17 | DID Call Wait | ON/OFF | OFF |  |
|  | 18 | CLI Type | LONG/SHORT | SHORT |  |
|  | 19 | Long Station CLI | Max 12 digits | Logical STA Number |  |
|  | 20 | MSN Call Wait | ON/OFF | OFF |  |

#### TABLE 7.4.11   FLEXIBLE BUTTON ASSIGNMENT

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 115 | 01-44 | Flex. Buttons Assignment | BTN 01-44 |  |  |
|  |  | 1: User Button | - |  | Not programmed |
|  |  | 2: {CO} Button | 001-400 (LDK-300E) 001-200 (LDK-300) 01-40 (LDK-100) |  |  |
|  |  | 3: {CO Group} Button | 01-72 (LDK-300/300E) 01-24 (LDK-100) |  |  |
|  |  | 4: {LOOP} Button | - |  |  |
|  |  | 5: {STA xxx} Button | STA No. |  |  |
|  |  | 6: STA PGM Button | 11 - 99 |  |  |
|  |  | 7: {STA SPD xxx} Button | STA SPD Bin No. |  |  |
|  |  | 8: {SYS SPD xxxx} Button | SYS SPD Bin No. |  |  |
|  |  | 9: Num Pln Button | Num Plan Code |  |  |
|  |  | 10: Networking DSS Button | Networking No. |  |  |
|  |  | 11: Hunt Group Button | Hunt Group No. |  |  |
|  |  | 12: MSN Button | MSN No. |  |  |

#### TABLE 7.4.12  STATION BASE PROGRAM

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 116 | 1 | Station COS : Day | 1 - 7 | 1 |  |
|  | 2 | Station COS : Night | 1 - 7 | 1 |  |
| 117 |  | CO Group Access |  |  |  |
|  | 1 | CO Line Group 01~24 |  | 01-24 |  |
|  | 2 | CO Line Group 25~48 |  | 25-48 | LDK-300/300E Only |
|  | 3 | CO Line Group 49~72 |  | 49-72 | LDK-300/300E Only |
| 118 |  | Internal Page Zone Access |  | GRP 01 |  |
|  | 1 | Internal Page Zone 01~24 | LDK-100 : 01-10 |  |  |
|  | 2 | Internal Page Zone 25~30 |  |  | LDK-300/300E Only |
| 119 | 1-5 | Conference Page Zone Access | 31 - 35 (LDK-300/300E) 11 - 15 (LDK-100) |  |  |
| 120 |  | ICM Tenancy Group number |  |  |  |
|  | 1 | ICM Tenancy Group Attendant | STA No. | - |  |
|  | 2 | ICM Tenancy Access Group | 01 - 15 (LDK-300/300E) 01 - 05 (LDK-100) | - |  |
| 121 |  | Preset Call Forward |  | - |  |
| 122 |  | Hot Line / Warm Line |  | - |  |
|  |  | 1: Flex Button | 01 – 44 | - |  |
|  |  | 2: CO Line | 001-400 (LDK-300E) 001-200 (LDK-300) 01-40 (LDK-100) | - |  |
|  |  | 3: CO Group | 01-72 (LDK-300/300E) 01-24 (LDK-100) | - |  |
|  |  | 4: Station | STA No. | - |  |
| 123 | 1 | CTI Mode | 0 – 2 | 1 | 0: Inactive 1: CTI mode 2: AT mode |
|  | 2 | CTI Baud Rate | 0 – 2 | 0 | 0: 1200 1: 2400 2: 4800 |
| 124 |  | SMDR Account Group Assign | 00 – 99 (LDK-300/300E) 00 – 23(LDK-100) | 00(Not Assigned) |  |

#### TABLE 7.4.13  CO LINE BASE PROGRAM

| PGM | BTN | ITEM |  | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
| 140 |  | CO Service Type |  |  |  |  |
|  | 1 | CO Type |  | 1-5 | 1(Normal) | 1:Normal, 2:A_DID, 3:ISDN DID/MSN, 4: TIE, 5: DCO DID |
|  | 2 | Detailed Attribute of the type |  |  |  |  |
|  |  | DISA(D/N/W) |  |  |  | Day / Night / Weekend |
|  |  |  | DISA SVC | ON /OFF | OFF |  |
|  |  |  | VMIB ANNC | 00-70 | 00(NOT_ASG) |  |
|  |  | Analog DID |  |  |  |  |
|  |  |  | SIGNAL | 1-3 | 1 | 1: Immediate 2: Wink 3: Delayed Dial |
|  |  |  | INFO NO | 00-70 | 00(NOT_ASG) |  |
|  |  | TIE Attribute |  |  |  |  |
|  |  |  | TIE SIG | 1-5 | 1 | 1:RD 2:LD 3:EM-C 4:EM-D 5:EM-I |
| 141 |  | CO Line Attributes –I |  | 1-9 |  |  |
|  | 1 | CO Line Group Assignment |  | 00-73 (LDK-300/300E) 00-25 (LDK-100) | 01 |  |
|  | 2 | CO Line COS |  | 1-5 | 1 |  |
|  | 3 | DISA Account Code |  | ON / OFF | OFF |  |
|  | 4 | CO Line Assign |  | POL/LOOP | LOOP | Polarity RV, Loop Start |
|  | 5 | CO Line Type |  | PBX/CO | CO |  |
|  | 6 | CO Line Signal Type |  | DTMF/PULSE | DTMF |  |
|  | 7 | Flash Type |  | GROUND/LOOP | LOOP |  |
|  | 8 | UNA |  | ON / OFF | OFF |  |
|  | 9 | CO Line Group Account |  | ON / OFF | OFF |  |
|  | 10 | Tenancy Group |  | 00-15 (LDK-300/300E) 00-05 (LDK-100) | 01 |  |
| 142 |  | CO Line Attributes – II |  | 1-13 |  |  |
|  | 1 | CO Line Name Display |  | ON / OFF | OFF |  |
|  | 2 | CO Line Name Assign |  |  | - | Max 12 characters |
|  | 3 | Metering Unit |  | 00-06 | 0 |  |
|  | 4 | Line Drop using CPT |  | ON / OFF | OFF |  |
|  | 5 | CO Distinct Ring |  | 0-4 | 0 |  |
|  | 6 | CO Line MOH |  | 0-13 (LDK-300/300E) 0-12 (LDK-100) | 1 |  |
|  | 7 | PABX CO Dial Tone |  | YES / NO | YES |  |
|  | 8 | PABX CO Ring Back Tone |  | YES / NO | NO |  |
|  | 9 | PABX CO Error Tone |  | YES / NO | NO |  |
|  | 10 | PABX CO Busy Tone |  | YES / NO | NO |  |
|  | 11 | PABX CO Announce Tone |  | YES / NO | NO |  |
| PGM | BTN | ITEM |  | RANGE | DEFAULT | REMARK |
| 142 | 12 | CO Flash Timer |  | 000 – 300 | 050 | 10 msec base |
|  | 13 | Open Loop Detect Timer |  | 0 – 20 | 0 | 100 msec base |
|  | 14 | Line Length |  | LONG / SHORT | SHORT | SAF only (Short:0~2km, Long:2~9Km) |
| 143 |  | ISDN CO Line Attribute |  | 1-6 |  |  |
|  | 1 | COLP Table Index |  | 00 ~ 50 | None | 00 ~ 49: PGM 201 Bin No. 50: PGM 114-BTN 5 |
|  | 2 | CLIP Table Index |  | 00 ~ 50 | None | 00 ~ 49: PGM 201 Bin No. 50: PGM 114-BTN 5 |
|  | 3 | Type of Calling Number |  | 0-4 | 2 | 0:Unknown number 1:International type 2:National number 3:Not used 4:Subscriber number |
|  | 4 | DID Conversion Type |  | 0 ~ 2 | 0 |  |
|  | 5 | DID Removal Number |  | 00-99 | 00 | 00:Do not ignore 01-99:indicate an ignored called party number (DID_RN : DID Remove number from called party information) |
|  | 6 | ISDN Enblock Sending |  | ON / OFF | OFF | ON:Enblock Sending Mode OFF:Overlap Sending Mode |
|  | 7 | CLI Transit |  | ORI(1)/CFW(0) | CFW(0) | ORI : Send CLI as the originate caller’s CLI. CFW : Send CLI as the call forwarded station’s CLI. |
|  | 8 | Num Plan ID |  | F1: 0-7 F2: 0-7 |  | Calling Party/Called Party Numbering Plan ID setting. |
|  | 9 | ISDN SS CD |  | ON/OFF | OFF | ISDN call deflection usage. Norway only. |
|  | 10 | ISDN 1 Digit Remove |  | ON/OFF | OFF | ISDN incoming CPN is unknown-unknown case, the first 1 digit is removed. |
| 144 |  | CO Ring Assignment |  |  |  | STA Range (Delay : 0 – 9), Hunt Group, VMIB Message |
|  | 1 | Day |  | STA_R/ HUNT/ VMIB |  |  |
|  | 2 | Night |  | STA_R/ HUNT/ VMIB |  |  |
|  | 3 | Weekend |  | STA_R/ HUNT/ VMIB |  |  |
|  | 4 | On-demand |  | STA_R/ HUNT/ VMIB |  |  |
| PGM | BTN | ITEM |  | RANGE | DEFAULT | REMARK |
| 145 |  | CO Ring Assignment Display |  |  |  |  |
|  | 1 | Day |  |  |  |  |
|  | 2 | Night |  |  |  |  |
|  | 3 | Weekend |  |  |  |  |
|  | 4 | On-demand |  |  |  |  |
| 146 | 1 | Incoming Prefix Code Insertion |  | ON / OFF | OFF |  |
|  | 2 | Outgoing Prefix Code Insertion |  | ON / OFF | ON |  |
|  | 3 | A/u-Law Line Installed |  | U/A | A-Law | ON: u-Law OFF: A-Law |
|  | 4 | Calling Sub-address |  | ON/OFF | OFF | If this field is ON, station number will be filled in calling party number sub-address IE in setup. |
|  | 5 | DID DGT Receive Number |  | 2 – 4 | 3 |  |
|  | 6 | DID DGT Conversion |  | Max 5 digits | #*** |  |
| 147 | BIN 01- 10 | CO-MSN Mapping Table |  | PGM202 MSN Table Bin No. 000 – 249 | - | PGM202 MSN table should be programmed previously. |

#### TABLE 7.4.14 SYSTEM BASE PROGRAM

| PGM | BTN | ITEM |  | RANGE | DEFAULT |  | REMARK |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 160 |  | System Attributes-I |  | 1-13 |  |  |  |  |
|  | 1 | Attendant Call Queuing RB |  | RBT/MOH | MOH |  | RBT:Ring Back Tone MOH:PGM171 - 2 |  |
|  | 2 | CAMP RBT/MOH |  | RBT/MOH | MOH |  |  |  |
|  | 3 | CO Line Choice |  | LAST/ROUND | LAST |  | Round-Robin, Last Choice |  |
|  | 4 | DISA Retry Counter |  | 1-9 | 3 |  |  |  |
|  | 5 | ICM Continuous Dial-Tone |  | CONT / DISCONT | CONT |  | Continuous, Discontinuous |  |
|  | 6 | CO Dial-Tone Detect |  | ON / OFF | OFF |  |  |  |
|  | 7 | External Night Ring |  | ON / OFF | OFF |  |  |  |
|  | 8 | Hold Preference |  | SYS/EXEC | SYS |  | System, Exclusive |  |
|  | 9 | Multi-line Conference |  | ON / OFF | ON |  |  |  |
|  | 10 | Print LCR Conversion Digit |  | ON / OFF | OFF |  |  |  |
|  | 11 | Conference Warning Tone |  | ON / OFF | ON |  |  |  |
|  | 12 | Offnet Prompt Usage |  | ON / OFF | ON |  |  |  |
|  | 13 | Offnet DTMF Tone |  | ON / OFF | ON |  |  |  |
| 161 |  | System Attributes-II |  | 1-13 |  |  |  |  |
|  | 1 | Network Time/Date Setting |  | ON / OFF | OFF |  |  |  |
|  | 2 | Off-Hook Ring Signal Type |  | MUTE/BURST | MUTE |  | Mute Ring, One Burst |  |
|  | 3 | Override 1st CO Group |  | ON / OFF | ON |  |  |  |
|  | 4 | Page Warning Tone |  | ON / OFF | ON |  |  |  |
|  | 5 | Privacy |  | ON / OFF | ON |  |  |  |
|  | 6 | Privacy Warning Tone |  | ON / OFF | ON |  |  |  |
|  | 7 | Single Ring for CO Call |  | YES/NO | NO |  |  |  |
|  | 8 | WTU Auto Release |  | ON / OFF | OFF |  |  |  |
|  | 9 | ACD PRNT Enable |  | ON / OFF | OFF |  | ON:10s |  |
|  | 10 | ACD PRNT Timer |  | 001 – 255 | 001 |  | 10 sec or 1 hour base |  |
|  | 11 | ACD Clr After Print |  | ON / OFF | OFF |  |  |  |
|  | 12 | VMIB PROMPT GAIN |  | 00 - 31 | 08 |  |  |  |
|  | 13 | VM with CLI Info |  | ON / OFF | OFF |  |  |  |
|  | 14 | ACD Print Timer Unit |  | HOUR/SEC | SEC |  | SEC: 10 seconds |  |
|  | 15 | Set VM SMDI Type |  | TYPE II/ TYPE I | TYPE I |  |  |  |
|  | 16 | Incoming Call Toll Check |  | ON/OFF | OFF |  |  |  |
|  | 17 | Reserved |  |  |  |  |  |  |
|  | 18 | No DSS Indication |  | ON/OFF | ON |  |  |  |
| 162 | - | Admin Password |  | 4 Digits | - |  |  |  |
| 163 | 1 | Alarm Enable |  | ON / OFF | OFF |  |  |  |
|  | 2 | Alarm Contact Type |  | CLOSE/OPEN | CLOSE |  | Close, Open |  |
|  | 3 | Alarm Mode |  | ALARM / BELL | ALARM |  | Alarm, Door Bell |  |
|  | 4 | Alarm Signal Mode |  | RPT/ONCE | RPT |  | Repeat , Once |  |
| 164 | 1-5 | Attendant Assignment |  | STA No. | 1 : 101 |  | Max 5 Atds |  |
| PGM | BTN | ITEM |  | RANGE | DEFAULT |  | REMARK |  |
| 165 | - | Auto Attendant |  |  |  |  |  |  |
|  | 1 | Auto Attendant Usage |  | ON / OFF | OFF |  |  |  |
|  | 2 | Auto Attendant VMIB Annc.# |  | 00-70 | 00(not_asgn) |  |  |  |
| 166 |  | CO-to-CO COS |  | 1-7 | 1 |  |  |  |
|  | 1 | Day COS |  | 1-7 | 1 |  |  |  |
|  | 2 | Night/Weekend COS |  | 1-7 | 1 |  |  |  |
| PGM | BTN | ITEM |  | RANGE | DEFAULT |  | REMARK |  |
| 167 |  | DID/DISA Destination |  | F1-F4 |  |  |  |  |
|  | 1 | Busy Destination |  | F1-F3 | F1 |  | 1: Tone 2: ATD (Ring Assign) 3: FWD to Hunt Grp. |  |
|  | 2 | Error Destination |  | F1-F3 | F1 |  |  |  |
|  | 3 | No Answer Destination |  | F1-F3 | F1 |  |  |  |
|  | 4 | VMIB PROMPT USAGE |  | F1-F5 |  |  |  |  |
|  |  | 1 | Busy Prompt Usage | ON / OFF | ON |  |  |  |
|  |  | 2 | Error Prompt Usage | ON / OFF | ON |  |  |  |
|  |  | 3 | DND Prompt Usage | ON / OFF | ON |  |  |  |
|  |  | 4 | No Ans Prompt Usage | ON / OFF | ON |  |  |  |
|  |  | 5 | Atd Xfer Prompt Usage | ON / OFF | ON |  |  |  |
| 168 | 1 | First Contact |  | 1-5 | - |  | 1: LBC(STA #) 2: Door 3: Ext. 1 4: Ext. 2 5: Ext. 3 |  |
|  | 2 | Second Contact |  | 1-5 | - |  |  |  |
|  | 3 | Third Contact |  | 1-5 | - |  |  |  |
|  | 4 | Forth Contact |  | 1-5 | - |  |  |  |
|  | 5 | Fifth Contact |  | 1-5 | - |  |  |  |
|  | 6 | Sixth Contact |  | 1-5 | - |  |  |  |
|  | 7 | Seventh Contact |  | 1-5 | - |  | LDK-300/300E Only |  |
| 169 | 1 | Time Display Mode |  | 12H/ 24H | 12H |  |  |  |
|  | 2 | Date Display Mode |  | MMDD / DDMM | DDMMYY |  |  |  |
|  | 3 | Language Display Mode |  | 00-14 | 12(Korean) |  | 0:ENGLISH, 1:ITALIAN, 2:FINNISH, 3:DUTCH, 4:SWEDISH, 5:DANISH, 6:NORWEGIAN, 7:HEBREW, 8:GERMANY, 9:FRENCH, 10:PORTUGUESE, 11:SPANISH, 12:KOREAN, 13:ESTONIA, 14:RUSSIAN, |  |
| PGM | BTN | ITEM |  | RANGE | DEFAULT |  | REMARK |  |
| 170 |  | Modem Associated Device |  |  | STA1599(LDK-300E) STA 399 (LDK-300) STA 227 (LDK-100) |  | Last Station |  |
|  | 1 | Associated Station |  | Station Number |  |  |  |  |
|  | 2 | Associated CO |  | CO Number |  |  |  |  |
| 171 | 1 | BGM Type |  | 0-12 (LDK-300/300E) 0-11 (LDK-100) |  | 1 |  | 0: None 1: Int. Music 2-4: Ext. Music 1-3 5-6(7): VMIB 1-2(3) 7(8)-11(12): SLT MOH |
|  | 2 | MOH Type |  | 0-13 (LDK-300/300E) 0-12 (LDK-100) |  | 1 |  | 0: NOT_ASG 1: Int. Music 2-4: Ext. Music 1-3 5-6(7): VMIB 1-2(3) 7(8)-11(12): SLT MOH 12(13): Hold Tone |
|  | 3 | ICM Box Music Channel |  | 0-12 (LDK-300/300E) 0-11 (LDK-100) |  | 1 |  | The same as above 1 |
|  | 4 | Assign MOH via SLT |  | Flex. 1-5 (+SLT STA No.) |  | - |  | SLT MOH 1-5 |
| 172 | 1 - 4 | PBX Access Code |  | Max. 2 digits |  | - |  | Maximum 4 PBX access code |
| 173 |  | PLA Priority Setting |  |  |  |  |  | PLA priority is set Exclusively |
|  | 1 | Transfer CO |  | 1 – 4 |  | 1 |  |  |
|  | 2 | Recalling CO |  | 1 – 4 |  | 2 |  |  |
|  | 3 | Incoming CO |  | 1 – 4 |  | 3 |  |  |
|  | 4 | Queued CO |  | 1 – 4 |  | 4 |  |  |
| 174 |  | RS-232 PORT Setting |  |  |  |  |  |  |
|  | 1 | Baud Rate Setting |  | 0-8 |  | 19200(6) |  | 0: UNKNOWN 1: UNKNOWN 2: 1200 BAUD 3: 2400 BAUD 4: 4800 BAUD 5: 9600 BAUD 6: 19200 BAUD 7: 38400 BAUD 8: 57600 BAUD |
|  | 2 | CTS |  | ON / OFF |  | OFF |  |  |
|  | 3 | Page Break |  | ON / OFF |  | OFF |  |  |
|  | 4 | Line Page |  | 001-199 |  | 060 |  |  |

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 175 | 1 | Off-line SMDR/Statistics Print | 01-13 (LDK-300/300E) 01-11 (LDK-100) | COM2(02) : LDK-300/300E COM1(01) : LDK-100 | Value : 1-8 : LDK-300/300E Value : 1-6 : LDK-100 |
|  | 2 | Admin Print |  |  |  |
|  | 3 | Traffic Print |  |  |  |
|  | 4 | SMDI Print |  |  |  |
|  | 5 | CALL Info Print |  |  |  |
|  | 6 | On-line SMDR Print |  |  |  |
|  | 7 | Trace Print |  |  |  |
|  | 8 | Debug Print |  |  |  |
|  | 9 | PC_ADM |  | NET_PCADM | LDK-300/300E : 1-5, 9, 10 LDK-100 : 1-3, 7, 8 |
|  | 10 | PC_ATD |  | NET_PCATD | LDK-300/300E : 1, 2, 4, 5, 11 LDK-100 : 1, 2, 4, 9 |
|  | 11 | CTI |  | NET_CTI | LDK-300/300E : 1, 2, 4, 5, 12 LDK-100 : 1, 2, 4, 10 |
|  | 12 | REMOTE_DIAG |  | NET_REMOTE | LDK-300/300E : 1-5, 13 LDK-100 : 1-3, 11 |
| 176 | - | Pulse Dial/Speed Ratio | 66/33 / 60/40 | 66/33 | 66/33 60/40(10 PPS only) |
| 177 |  | SMDR Attributes | 1-14 |  |  |
|  | 1 | SMDR Save Enable | ON / OFF | OFF |  |
|  | 2 | SMDR Print Enable | ON / OFF | OFF |  |
|  | 3 | SMDR Recording Call Type | LD / ALL | LD | LD : Long Distance ALL: All |
|  | 4 | SMDR Long Distance Call Digit Counter | 07-15 | 07 | Considered more than this value as Long Distance Call. |
|  | 5 | Print Incoming Call | ON / OFF | OFF |  |
|  | 6 | Print Lost Call | ON / OFF | OFF |  |
|  | 7 | Records in Detail | ON / OFF | ON |  |
|  | 8 | SMDR Dial Digit Hidden | 0-9 | 0 |  |
|  | 9 | SMDR Currency | 3 English Chars | - |  |
|  | 10 | SMDR Cost Per Unit Pulse | 6 digits | - |  |
|  | 11 | SMDR Fraction | 0-5 | 0 |  |
|  | 12 | SMDR Start Timer | 0 – 250 | 0 | 1 sec base |
|  | 13 | SMDR Hidden Digit | Right/Left | Right |  |
|  | 14 | SMDR Long Distance Codes | MAX 2 digits | 0 | Max. 5 LD codes |
| 178 | 1 | System Time Setting | 4 digits | - | Hour/Min sequence |
|  | 2 | System Date Setting | 6 digits | - | Month/Day/Year sequence. |
| 179 | 1 | View |  |  | Show the linked pairs |
|  | 2 | Linked Pair Input | 2 STA# |  | Max: 64 pairs |

#### TABLE 7.4.15  SYSTEM TIMER PROGRAM

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 180 | 1 | - Attendant Recall Timer | 00 - 60 | 01 | 1 min base |
|  | 2 | - Call Park Recall Timer | 000 - 600 | 120 | 1 sec base |
|  | 3 | Camp-on Recall Timer | 000 - 200 | 030 | 1 sec base |
|  | 4 | - Exclusive Hold Recall Timer | 000 - 300 | 060 | 1 sec base |
|  | 5 | - I-Hold Recall Timer | 000 - 300 | 030 | 1 sec base |
|  | 6 | - Sys Hold Recall Timer | 000 - 300 | 030 | 1 sec base |
|  | 7 | - Transfer Recall Timer | 000 - 300 | 030 | 1 sec base |
|  | 8 | - ACNR Delay Timer | 000 - 300 | 030 | 1 sec base |
|  | 9 | - ACNR No Answer Timer | 10 - 50 | 30 | 1 sec base |
|  | 10 | - ACNR Pause Timer | 005 - 300 | 030 | 1 sec base |
|  | 11 | - ACNR Retry Counter | 01 - 30 | 3 |  |
|  | 12 | - ACNR No Tone Retry Counter | 1 - 9 | 1 |  |
|  | 13 | - ACNR Tone Detect Timer | 001-300 | 030 | 1 sec base |
|  | 14 | Automatic CO Release Timer. | 020 - 300 | 030 | 1 sec base |
|  | 15 | - CCR Inter-Digit Timer | 000 - 255 | 030 | 100 msec base |
|  | 16 | - CO Call Drop Warning Timer | 00 - 99 | 10 | 1 sec base |
|  | 17 | - CO Call Restriction Timer | 00 - 99 | 0 | 1 min base |
|  | 18 | - CO Dial Delay Timer | 00 - 99 | 01 | 100 msec base |
|  | 19 | - CO Release Guard Timer | 001 - 150 | 020 | 100 msec base |
|  | 20 | CO Ring Off Timer | 010 - 150 | 060 | 100 msec base |
|  | 21 | - CO Ring On Timer | 1 - 9 | 2 | 100 msec base |
|  | 22 | - CO Warning Tone Timer | 060 - 900 | 180 | 1 sec base |
| 181 | 1 | Call FWD No Answer Timer | 000 - 255 | 015 | 1 sec base |
|  | 2 | DID/DISA No Answer Timer | 00 - 99 | 20 | 1 sec base |
|  | 3 | VMIB User Record Timer | 010 - 255 | 020 | 1 sec base |
|  | 4 | VMIB Valid User Message Timer | 0-9 | 4 | 1 sec base |
|  | 5 | Door Open Timer | 05 - 99 | 20 | 100msec base |
|  | 6 | ICM Box Timer | 00 - 60 | 30 | 1 sec base |
|  | 7 | ICM Dial Tone Timer | 01 - 20 | 10 | 1 sec base |
|  | 8 | Inter Digit Timer | 01 - 20 | 05 | 1 sec base |
|  | 9 | MSG Wait Reminder Tone Timer | 00 - 60 | 00 | 1 min base |
|  | 10 | Paging Timeout Timer | 000 - 255 | 015 | 1 sec base |
|  | 11 | - Pause Timer | 1 - 9 | 3 | 1 sec base |
|  | 12 | - Preset Call Forward Timer | 00 - 99 | 10 | 1 sec base |
|  | 13 | - SLT DTMF Release Timer | 00 - 20 | 00 | 1 sec base |
| 182 | 1 | SLT Hook Switch Bounce Timer | 01 - 25 | 01 | 100msec base |
|  | 2 | SLT Maximum Hook Flash Timer | 01-25 | 05 | 100msec base |
|  | 3 | SLT Minimum Hook Flash Timer | 000 - 250 | 020 | 10msec base |
|  | 4 | SLT Ring Phase Timer | 2 - 5 | 5 | 1 sec base |
|  | 5 | Station Auto Release Timer | 020 - 300 | 060 | 1 sec base |
|  | 6 | - Unsupervised Conference Timer | 00 - 99 | 10 | 1 min base |
|  | 7 | Wake-Up Fail Ring Timer | 00 - 99 | 20 | 1 sec base |
|  | 8 | - Warm Line Timer | 01 - 20 | 05 | 1 sec base |
|  | 9 | Wink Timer | 010 - 200 | 010 | 10msec base |
|  | 10 | Enblock Digit timer | 01-20 | 15 | 1 sec base |
|  | 11 | CCR Time Out Timer | 000-300 | 015 | 1 sec base |
|  | 12 | DID Inter Digit Timer | 01-20 | 03 | 1 sec base |

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 185 |  | CIDU Setting |  |  | Korea Only |
|  | 1 | CID Usage | ON / OFF | OFF |  |
|  | 2 | CID Name Display | Name(1) / Telephone No.(0) | Telephone No.(0) |  |
|  | 3 | Serial Port Select | 1-4 | - |  |
|  | 4 | CID/CO Line Port Mapping | 000-063 | - |  |
|  | 5 | Initialize CID Data |  |  |  |

#### TABLE 7.4.16  DCOB ATTRIBUTE

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 186 |  | DCOB System Attributes | 1-12 |  |  |
|  | 1 | DCOB CO Type | 0-2 | 2 | 0:Sweden/Cyprus 1:Italy 2:Korea/Australia |
|  | 2 | Metering Type | 0-1 | 0 |  |
|  | 3 | R2 OUT Manage Timer | 01-50 | 14 | 1 sec |
|  | 4 | R2 IN Manage Timer | 01-50 | 14 | 1 sec |
|  | 5 | R2 Disappear Timer | 01-50 | 14 | 1 sec |
|  | 6 | R2 Pulse Timer | 01-30 | 7 | 20 msec |
|  | 7 | R2 Ready Timer | 000-500 | 7 | 20 msec |
|  | 8 | Dial Tone Delay Timer | 01-30 | 20 |  |
|  | 9 | Line Status | 1-9 | 6 | Free Line |
|  | 10 | Calling Category | 1-9 | 1 | User no priority |
|  | 11 | DNIS Service | ON/OFF | OFF |  |
|  | 12 | CLI Digit Num | 01-10 | 4 | Reserved |
| 187 |  | DCOB CO Line Attributes | CO Line range |  |  |
|  | 1 | IN Digit Type | 0-2 | 2 | 0:PULSE 1:DTMF |
|  | 2 | OUT Digit Type | 0-2 | 2 | 2:R2MFC |
|  | 3 | CLI Digit Num | 01-15 | 10 |  |

#### TABLE 7.4.17  STATION GROUP ASSIGNMENT

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 190 |  | Station Group Number | STA Grp # | - |  |
|  | 1 | Group Type | 0-6 | 0 | 0: Not Assign 1: Circular 2: Terminal 3: UCD 4: Ring 5: VM 6: Pick up 7: Net VM |
|  | 2 | Pick-up Attribute | ON/OFF | OFF |  |
|  | 3 | Member Assignment | Not Assigned | - | Group type should be assigned |

#### TABLE 7.4.18  STATION GROUP PROGRAM

| PGM | ITEM |  | BTN |  | SUB ITEM | RANGE |  |  | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 191 | Circular Group |  | 1 |  | VMIB Announce 1 Timer | 000-999 |  |  | 015 | 1 sec base |
|  |  |  | 2 |  | VMIB Announce 2 Timer | 000-999 |  |  | 000 | 1 sec base |
|  |  |  | 3 |  | VMIB Announce Location 1 | 00-70 |  |  | 00(not_asgn) |  |
|  |  |  | 4 |  | VMIB Announce Location 2 | 00-70 |  |  | 00(not_asgn) |  |
|  |  |  | 5 |  | VMIB Announce 2 Repeat | 000-999 |  |  | 000 | 1 sec base |
|  |  |  | 6 |  | VMIB Announce 2 Repeat E/D | ON / OFF |  |  | OFF |  |
|  |  |  | 7 |  | Overflow Destination | STA #/HUNT#/ VMIB #/SYS SPD# |  |  | - |  |
|  |  |  | 8 |  | Overflow Timer | 000-600 |  |  | 180 | 1 sec base |
|  |  |  | 9 |  | Wrap-up Timer | 002-999 |  |  | 002 | 1 sec base |
|  |  |  | 10 |  | No Answer Timer | 00-99 |  |  | 15 | 1 sec base |
|  |  |  | 11 |  | Pilot Hunt | ON / OFF |  |  | ON |  |
|  |  |  | 12 |  | Alt If No Member | ON / OFF |  |  | OFF |  |
|  |  |  | 13 |  | Music Source | 00 - 12 (LDK-300/300E) 00 - 11 (LDK-100) |  |  | 00 |  |
|  | Terminal |  | 1 |  | VMIB Announce 1 Timer | 000-999 |  |  | 015 | 1 sec base |
|  | Group |  | 2 |  | VMIB Announce 2 Timer | 000-999 |  |  | 000 | 1 sec base |
|  |  |  | 3 |  | VMIB Announce Location 1 | 00-70 |  |  | 00(not_asgn) |  |
|  |  |  | 4 |  | VMIB Announce Location 2 | 00-70 |  |  | 00(not_asgn) |  |
|  |  |  | 5 |  | VMIB Announce 2 Repeat | 000-999 |  |  | 000 | 1 sec base |
|  |  |  | 6 |  | VMIB Announce 2 Repeat E/D | ON / OFF |  |  | OFF |  |
|  |  |  | 7 |  | Overflow Destination | STA #/HUNT#/ VMIB #/SYS SPD# |  |  | - |  |
|  |  |  | 8 |  | Overflow Timer | 000-600 |  |  | 180 | 1 sec base |
|  |  |  | 9 |  | Wrap-up Timer | 002-999 |  |  | 002 | 1 sec base |
|  |  |  | 10 |  | No Answer Timer | 00-99 |  |  | 15 | 1 sec base |
|  |  |  | 11 |  | Pilot Hunt | ON / OFF |  |  | ON |  |
|  |  |  | 12 |  | Alt If No Member | ON / OFF |  |  | OFF |  |
|  |  |  | 13 |  | Music Source | 00 - 12 (LDK-300/300E) 00 - 11 (LDK-100) |  |  | 00 |  |
|  | UCD Group | 1 |  | VMIB Announce 1 Timer |  |  | 000-999 | 015 |  | 1 sec base |
|  |  | 2 |  | VMIB Announce 2 Timer |  |  | 000-999 | 000 |  | 1 sec base |
|  |  | 3 |  | VMIB Announce Location 1 |  |  | 00-70 | 00(not_asgn) |  |  |
|  |  | 4 |  | VMIB Announce Location 2 |  |  | 00-70 | 00(not_asgn) |  |  |
|  |  | 5 |  | VMIB Announce 2 Repeat |  |  | 000-999 | 000 |  | 1 sec base |
|  |  | 6 |  | VMIB Announce 2 Repeat E/D |  |  | ON / OFF | OFF |  |  |
|  |  | 7 |  | Overflow Destination |  |  | STA #/HUNT#/ VMIB #/SYS SPD# | - |  |  |
|  |  | 8 |  | Overflow Timer |  |  | 000-600 | 180 |  | 1 sec base |
|  |  | 9 |  | Wrap Up Timer |  |  | 002-999 | 002 |  | 1 sec base |
|  |  | 10 |  | Alt If No Member |  |  | ON / OFF | OFF |  |  |
| PGM | ITEM |  | BTN |  | SUB ITEM | RANGE |  |  | DEFAULT | REMARK |
| 191 | UCD Group | 11 |  | Music Source |  |  | 00 - 12 (LDK-300/300E) 00 - 11 (LDK-100) | 00 |  |  |
|  |  | 12 |  | ACD Warning Tone |  |  | ON / OFF | ON |  |  |
|  |  | 13 |  | Alternate Destination |  |  | STA #/ STA GRP# |  |  |  |
|  |  | 14 |  | Supervisor Timer |  |  | 000-999 | 030 |  | 1 sec base |
|  |  | 15 |  | Supervisor Call Count |  |  | 00-99 | 00 |  |  |
|  |  | 16 |  | ACD Queued Call |  |  | ON / OFF | OFF |  |  |
|  |  | 17 |  | MAX Que Call Count |  |  | 00-99 | 00 |  |  |
|  |  | 18 |  | Supervisor |  |  | STA # | - |  |  |
|  |  | 19 |  | UCD hunt Stations' Priority |  |  | 0-9 | 0 |  | For each member |
|  | Ring Group | 1 |  | VMIB Announce 1 Timer |  |  | 000-999 | 015 |  | 1 sec base |
|  |  | 2 |  | VMIB Announce 2 Timer |  |  | 000-999 | 000 |  | 1 sec base |
|  |  | 3 |  | VMIB Announce Location 1 |  |  | 00-07 | 00(not_asgn) |  |  |
|  |  | 4 |  | VMIB Announce Location 2 |  |  | 00-07 | 00(not_asgn) |  |  |
|  |  | 5 |  | VMIB Announce 2 Repeat |  |  | 000-999 | 000 |  | 1 sec base |
|  |  | 6 |  | VMIB Announce 2 Repeat E/D |  |  | ON / OFF | OFF |  |  |
|  |  | 7 |  | Overflow Destination |  |  | STA #/HUNT#/ VMIB #/SYS SPD# | - |  |  |
|  |  | 8 |  | Overflow Timer |  |  | 000-600 | 180 |  | 1 sec base |
|  |  | 9 |  | Wrap Up Timer |  |  | 002-999 | 002 |  | 1 sec base |
|  |  | 10 |  | Music Source |  |  | 00 - 12 (LDK-300/300E) 00 - 11 (LDK-100) | 00 |  |  |
|  |  | 11 |  | Max. Queued Call Count |  |  | 00-99 | 00 |  |  |
|  | VM Group | 1 |  | Wrap-up Timer |  |  | 002-999 | 002 |  | 1 sec base |
|  |  | 2 |  | Put Mail Index |  |  | 1-4 | 1 |  |  |
|  |  | 3 |  | Get Mail Index |  |  | 1-4 | 2 |  |  |
|  |  | 4 |  | Hunt Type |  |  | Cir/Term | Term |  |  |
|  |  | 5 |  | SMDI Port |  |  | 01-13 (LDK-300/300E) 01-11 (LDK-100) | 02(COM2) 01(COM1) |  |  |
|  |  | 6 |  | Overflow Timer |  |  | 000-600 | 180 |  | 1 sec base |
|  |  | 7 |  | Overflow Destination |  |  | STA #/HUNT#/ VMIB #/SYS SPD# | - |  |  |
|  | Pick-up Group | 1 |  | Auto Pick-up |  |  | ON / OFF | OFF |  |  |
|  |  | 2 |  | All Group Member Ringing |  |  | ON / OFF | OFF |  |  |

#### TABLE 7.4.19  ISDN ATTRIBUTES

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 200 | 1 | Advice of Charge | 0-5 | 0 | 0: Do not service AOC 1: Italy and Spain 2: Finland 3: Australia 4: Belgium 5: Standard |
|  | 2 | Reserved |  |  |  |
|  | 3 | Reserved |  |  |  |
|  | 4 | Reserved |  |  |  |
|  | 5 | Reserved |  |  |  |
|  | 6 | CLI Print | ON/OFF | OFF |  |
|  | 7 | International Access Code | Max 4 digits | - |  |
|  | 8 | Reserved |  |  |  |
|  | 9 | My Area Code | Max 6 digits | - |  |
|  | 10 | My Area Prefix Code | Max 4 digits | - |  |
|  | 11 | Maintain DID Name | ON/OFF | OFF |  |
| 201 | - | COLP Table | Entry No. (00-49) | - | Max 10 digits |
| 202 | - | MSN Table | Entry No. (000-249) | - |  |
|  | 1 | CO Line number | 001-400(LDK-300E) 001-200 (LDK-300) 01-40 (LDK-100) | - |  |
|  | 2 | Index | 000-999 | - |  |
|  | 3 | SUB Number | 0-9 | - |  |
|  | 4 | MSN number | 20 digits | - |  |
|  | 5 | Block Same MSN Incoming | ON/OFF | OFF | Disconnect the duplicated MSN incoming call. |

#### TABLE 7.4.20  LCR TABLE ASSIGNMENT

| PGM | BTN | ITEM |  |  | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 220 | 1 | LCR Access Mode |  |  | M00/M01/M02/M11/M12/M13 | M00 |  |
|  | 2 | Set the Day of week zone |  |  |  | 1234567 |  |
|  |  | 1 |  | MON | 1 – 3 | 1 |  |
|  |  | 2 |  | TUE | 1 – 3 | 1 |  |
|  |  | 3 |  | WED | 1 – 3 | 1 |  |
|  |  | 4 |  | THUR | 1 – 3 | 1 |  |
|  |  | 5 |  | FRI | 1 – 3 | 1 |  |
|  |  | 6 |  | SAT | 1 – 3 | 1 |  |
|  |  | 7 |  | SUN | 1 – 3 | 1 |  |
|  | 3 | Set the Time Zone of Day zone 1 |  |  |  |  |  |
|  |  | 1 | Time Zone1 |  | 00 – 24 | 0024 |  |
|  |  | 2 | Time Zone2 |  | 00 – 24 | - |  |
|  |  | 3 | Time Zone3 |  | 00 – 24 | - |  |
|  | 4 | Set the Time Zone of Day zone 2 |  |  |  |  |  |
|  |  | 1 |  | Time Zone1 | 00 – 24 | 0024 |  |
|  |  | 2 |  | Time Zone2 | 00 – 24 | - |  |
|  |  | 3 |  | Time Zone3 | 00 – 24 | - |  |
|  | 5 | Set the Time Zone of Day zone 3 |  |  |  |  |  |
|  |  | 1 |  | Time Zone1 | 00 – 24 | 0024 |  |
|  |  | 2 |  | Time Zone2 | 00 – 24 | - |  |
|  |  | 3 |  | Time Zone3 | 00 – 24 | - |  |
| 221 |  | Leading Digit Table |  |  | 000-249 |  |  |
|  | 1 | LCR Type |  |  | 1 – 3 | 3 | 1:int 2:col 3: both |
|  | 2 | Code (leading digit) |  |  | Max 12 digits | - |  |
|  | 3 | Day Zone 1 DMT |  |  | 6digits |  | Time1: 2digit |
|  | 4 | Day Zone 2 DMT |  |  | 6digits |  | Time2: 2digit |
|  | 5 | Day Zone 3 DMT |  |  | 6digits |  | Time3: 2digit |
|  | 6 | Check Password |  |  | ON/OFF | OFF |  |
| 222 |  | Digit Modification Table |  |  | 00-99 |  |  |
|  | 1 | Added Digit |  |  | Max 25 digits |  |  |
|  | 2 | Removal Position |  |  | 1 – 12 | 1 |  |
|  | 3 | Number Of Remove |  |  | 1 – 12 | 0 |  |
|  | 4 | Add Position |  |  | 1 – 13 | 1 |  |
|  | 5 | CO Group |  |  | 1 – 72 (LDK-300/300E) 1 – 24 (LDK-100) | 1 |  |
|  | 6 | Alt Index |  |  | 0 – 99 | - |  |
| 223 |  | LCR Table Initialization |  |  |  |  |  |
|  | 1 | DMT Of Day_zone_1 |  |  | 6 digits |  | Time1: 2digit |
|  | 2 | DMT Of Day_zone_2 |  |  | 6 digits |  | Time2: 2digit |
|  | 3 | DMT Of Day_zone_3 |  |  | 6 digits |  | Time3: 2digit |
|  | 4 | CO Grp Init |  |  | 1 – 72 (LDK-300/300E) 1 – 24 (LDK-100) |  |  |
|  | 5 | Alt Index Init |  |  | 0 - 99 |  |  |
|  | 6 | Init All LCR |  |  |  |  |  |

#### TABLE 7.4.21  TOLL TABLE ASSIGNMENT

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 224 |  | Toll Table |  |  |  |
|  | 1 | Allow Table A (01-30) | Max 14 digits | - |  |
|  | 2 | Deny Table A (01-30) | Max 14 digits | - |  |
|  | 3 | Allow Table B (01-30) | Max 14 digits | - |  |
|  | 4 | Deny Table B (01-30) | Max 14 digits | - |  |
| 225 |  | Canned Toll Table |  |  |  |
|  | 1 | Allow Table (01-20) | Max 14 digits | - |  |
|  | 2 | Deny Table (01-20) | Max 14 digits | - |  |
| 226 |  | Emergency Code Table (01-10) | Max 14 digits |  |  |

#### TABLE 7.4.22  OTHER TABLES

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 227 |  | Author Code Table | 001-999 (LDK-300E) 001 - 600 (LDK-300) 001 - 164 (LDK-100) |  |  |
|  |  | Table entry (001-600) | Max 5 digits |  |  |
| 228 |  | CCR Table | 1-70 |  |  |
|  | 1 | Station | STA # | - |  |
|  | 2 | Hunt Group | HUNT # | - |  |
|  | 3 | VMIB | Announce # | - |  |
|  | 4 | VMIB Drop | Announce # |  |  |
|  | 5 | System Speed | 2000-6999 (LDK-300E) 2000-4999 (LDK-300) 2000-3499 (LDK-100) | - |  |
|  | 6 | Internal Page | 1 - 30 (LDK-300/300E) 1 - 10 (LDK-100) | - |  |
|  | 7 | External Page | 1 – 3 | - |  |
|  | 8 | All Call Page | 1 – 3 | - | 1:INT 2:EXT 3:All |
| 229 |  | Exec/Sec Table | 01-36 (LDK-300/300E) 01-12 (LDK-100) |  |  |
|  |  | Table entry(01-36) | STA #/STA # |  |  |
| 231 | - | Flexible DID Table | Entry No.(000-999) | - |  |
|  | 1 | DID Name | Max 11 chars. | - |  |
|  | 2 | Day Destination | 1-9 | - | 1(sta), 2(hunt), 3(VMIB) |
|  | 3 | Night Destination | 1-9 | - | 4(VMIB drop), 5(spd), |
|  | 4 | Weekend Destination | 1-9 | - | 6(int. page), 7(ext. page) |
|  | 5 | Reroute Destination | 1-6 | - | 8(all page), 9(net call) |
| 232 |  | System Speed Zone | 01-10 |  |  |
|  | 1 | Speed Bin Range in Zone | 2200-6999 (LDK-300E) 2200-4999 (LDK-300) 2200-3499 (LDK-100) | - |  |
|  | 2 | Station Range | Station Range | - |  |
|  | 3 | Toll Checking | On/Off | ON |  |
| 233 |  | Weekly Time Table | 1 – 7 | - | Day of Week |
|  | 1 | Day Start Time | 0000 – 2359 | 0900 |  |
|  | 2 | Night Start Time | 0000 – 2359 | 1800 |  |
|  | 3 | Weekend Start Time | 0000 – 2359 |  |  |
| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| 234 |  | Voice Mail Dial-Table | 1 - 9 |  |  |
|  | 1 | Prefix Index | 12 Digits | - |  |
|  | 2 | Suffix Index | 12 Digits | - |  |
| 235 |  | TIE Line Routing Table | CO number | - |  |

#### TABLE 7.4.23  NETWORKING ATTRIBUTE

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 320 |  | Networking Basic Attributes | F1-F7 |  |  |
|  | 1 | Networking Enable | ON / OFF | OFF |  |
|  | 2 | Networking Retry Count | 00 – 99 | 00 |  |
|  | 3 | Networking CNIP Enable | ON / OFF | OFF |  |
|  | 4 | Networking CONP Enable | ON / OFF | OFF |  |
|  | 5 | Networking Signal Method | FAC / UUS | UUS |  |
|  | 6 | Networking CAS Enable | ON / OFF | OFF |  |
|  | 7 | Networking VPN Enable | ON / OFF | OFF |  |
| 321 |  | Supplementary Attributes | F1-F7 |  |  |
|  | 1 | Networking Transfer Mode | RERT / JOIN | JOIN |  |
|  | 2 | TCP port | 4 digits | 9000 |  |
|  | 3 | UDP port | 4 digits | 9001 |  |
|  | 4 | Gatekeeper IP Address | 12 digits | 0.0.0.0 |  |
|  | 5 | Duration of BLF status | 01 ~ 20 sec | 02 |  |
|  | 6 | Multicast IP Address | 12 digits | 0.0.0.0 |  |
|  | 7 | Net Trans Recall Timer | 1 ~ 300 | 10 |  |
| 322 |  | Networking CO Line Attributes | CO Line range |  |  |
|  | 1 | Networking CO Group | 00 – 24 | 00 |  |
|  | 2 | Reserved |  |  |  |
|  | 3 | Reserved |  |  |  |
|  | 4 | Net Co Line Type | QSIG / PSTN | PSTN |  |
| 324 |  | Networking Routing Table | 00-71 |  |  |
|  | 1 | System Usage | NET / PSTN | NET |  |
|  | 2 | Numbering Plan Code | 16 digits | - |  |
|  | 3 | Numbering Plan CO Group | 00-24 | - |  |
|  | 4 | CPN Information | 16 digits at QSIG, 4 IP Address at Voip |  |  |
|  | 5 | Alternate Speed Bin | 2000-4999 | - |  |
|  | 6 | MPB LAN IP Address | IP address | - |  |
|  | 7 | Digit Repeat | ON / OFF | OFF |  |

#### TABLE 7.4.24  VOIB NET ATTRUBUTE

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 340 |  | VOIB IP SETTING |  |  |  |
|  | 1 | IP Addressing(SKIP:#) |  |  |  |
|  | 2 | GATEWAY Addressing (SKIP:#) |  |  |  |
|  | 3 | SUBNET Mask(SKIP:#) |  |  |  |
|  | 4 | DNS Addressing (SKIP:#) |  |  |  |
|  | 5 | TRACE Password |  |  | Max 10 Digits |
|  | 6 | Default CODEC | 1 – 4 | 1 |  |
|  | 7 | Default GAIN | 1 - 62 | 31 |  |
|  | 8 | NO Delay (TOS) | ON / OFF | OFF |  |
|  | 9 | Throughput (TOS) | HIGH / NORMAL | NORMAL |  |
|  | 10 | Reliability (TOS) | HIGH / NORMAL | NORMAL |  |

#### TABLE 7.4.25  NATION SPECIFIC

| PGM | FLEX | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 400 |  | DTIB RX Gain |  |  | Korean version |
|  | 1 | DTIB/DKT | 00 – 63 | 26 |  |
|  | 2 | DTIB/SLT | 00 – 63 | 33 |  |
|  | 3 | DTIB/CTR SL | 00 – 63 | 22 |  |
|  | 4 | DTIB/WTU | 00 – 63 | 26 |  |
|  | 5 | DTIB/ACO | 00 – 63 | 33 |  |
|  | 6 | DTIB/CTR CO | 00 – 63 | 22 |  |
|  | 7 | DTIB/DCO | 00 – 63 | 33 |  |
|  | 8 | DTIB/VMIB | 00 – 63 | 29 |  |
|  | 9 | DTIB/DTMF | 00 – 63 | 8 |  |
|  | 10 | DTIB/TONE | 00 – 63 | 32 |  |
|  | 11 | DTIB/MUSIC1 | 00 – 63 | 29 |  |
|  | 12 | DTIB/MUSIC2 | 00 – 63 | 29 |  |
|  | 13 | DTIB/MUSIC3 | 00 – 63 | 29 |  |
| 401 |  | SLIB RX Gain |  |  |  |
|  | 1 | SLIB/DKT | 00 – 63 | 12 |  |
|  | 2 | SLIB/SLT | 00 – 63 | 23 | LDK-100 : Default 27 |
|  | 3 | SLIB/CTR SL | 00 – 63 | 12 | LDK-100 : Default 16 |
|  | 4 | SLIB/WTU | 00 – 63 | 12 |  |
|  | 5 | SLIB/ACO | 00 – 63 | 21 |  |
|  | 6 | SLIB/CTR CO | 00 – 63 | 12 |  |
|  | 7 | SLIB/DCO | 00 – 63 | 24 |  |
|  | 8 | SLIB/VMIB | 00 – 63 | 20 |  |
|  | 9 | SLIB/DTMF | 00 – 63 | 8 |  |
|  | 10 | SLIB/TONE | 00 – 63 | 18 |  |
|  | 11 | SLIB/MUSIC1 | 00 – 63 | 20 |  |
|  | 12 | SLIB/MUSIC2 | 00 – 63 | 20 |  |
|  | 13 | SLIB/MUSIC3 | 00 – 63 | 20 |  |

| PGM | FLEX | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 402 |  | CTR SLIB RX Gain |  |  |  |
|  | 1 | CTRSL2/DKT | 00 – 63 | 32 |  |
|  | 2 | CTRSL2/SLT | 00 – 63 | 43 | LDK-100 : Default 47 |
|  | 3 | CTRSL2/ CTR SL | 00 – 63 | 32 | LDK-100 : Default 36 |
|  | 4 | CTRSL2/WTU | 00 – 63 | 32 |  |
|  | 5 | CTRSL2/ACO | 00 – 63 | 41 |  |
|  | 6 | CTRSL2/ATR CO | 00 – 63 | 32 |  |
|  | 7 | CTRSL2/DCO | 00 – 63 | 44 |  |
|  | 8 | CTRSL2/VMIB | 00 – 63 | 40 |  |
|  | 9 | CTRSL2/DTMF | 00 – 63 | 28 |  |
|  | 10 | CTRSL2/TONE | 00 – 63 | 38 |  |
|  | 11 | CTRSL2/MUSIC1 | 00 – 63 | 40 |  |
|  | 12 | CTRSL2/MUSIC2 | 00 – 63 | 40 |  |
|  | 13 | CTRSL2/MUSIC3 | 00 – 63 | 40 |  |
| 403 |  | WTIB RX Gain |  |  |  |
|  | 1 | WTIB/DKT | 00 – 63 | 26 |  |
|  | 2 | WTIB/SLT | 00 – 63 | 33 |  |
|  | 3 | WTIB/CTR SL | 00 – 63 | 22 |  |
|  | 4 | WTIB/WTU | 00 – 63 | 26 |  |
|  | 5 | WTIB/ACO | 00 – 63 | 38 |  |
|  | 6 | WTIB/CTR CO | 00 – 63 | 29 |  |
|  | 7 | WTIB/DCO | 00 – 63 | 33 |  |
|  | 8 | WTIB/VMIB | 00 – 63 | 29 |  |
|  | 9 | WTIB/DTMF | 00 – 63 | 8 |  |
|  | 10 | WTIB/TONE | 00 – 63 | 37 |  |
|  | 11 | WTIB/MUSIC1 | 00 – 63 | 29 |  |
|  | 12 | WTIB/MUSIC2 | 00 – 63 | 29 |  |
|  | 13 | WTIB/MUSIC3 | 00 – 63 | 29 |  |
| 404 |  | ACOB RX Gain |  |  |  |
|  | 1 | ACOB/DKT | 00 – 63 | 26 |  |
|  | 2 | ACOB/SLT | 00 – 63 | 37 |  |
|  | 3 | ACOB/CTR SL | 00 – 63 | 27 |  |
|  | 4 | ACOB/WTU | 00 – 63 | 26 |  |
|  | 5 | ACOB/ACO | 00 – 63 | 36 |  |
|  | 6 | ACOB/STR CO | 00 – 63 | 27 |  |
|  | 7 | ACOB/DCO | 00 – 63 | 33 |  |
|  | 8 | ACOB/VMIB | 00 – 63 | 32 |  |
|  | 9 | ACOB/DTMF | 00 – 63 | 32 |  |
|  | 10 | ACOB/TONE | 00 – 63 | 32 |  |
|  | 11 | ACOB/MUSIC1 | 00 – 63 | 32 |  |
|  | 12 | ACOB/MUSIC2 | 00 – 63 | 32 |  |
|  | 13 | ACOB/MUSIC3 | 00 – 63 | 32 |  |
|  | 14 | ACOB/MODEM | 00 – 63 | 37 |  |

| PGM | FLEX | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 405 |  | CTR ACOB RX Gain |  |  |  |
|  | 1 | CTRCO8/DKT | 00 – 63 | 28 |  |
|  | 2 | CTRCO8/SLT | 00 – 63 | 43 |  |
|  | 3 | CTRCO8/CTR SL | 00 – 63 | 32 |  |
|  | 4 | CTRCO8/WTU | 00 – 63 | 31 |  |
|  | 5 | CTRCO8/ACO | 00 – 63 | 41 |  |
|  | 6 | CTRCO8/CTR CO | 00 – 63 | 32 |  |
|  | 7 | CTRCO8/DCO | 00 – 63 | 38 |  |
|  | 8 | CTRCO8/VMIB | 00 – 63 | 37 |  |
|  | 9 | CTRCO8/DTMF | 00 – 63 | 37 |  |
|  | 10 | CTRCO8/TONE | 00 – 63 | 37 |  |
|  | 11 | CTRCO8/MUSIC1 | 00 – 63 | 37 |  |
|  | 12 | CTRCO8/MUSIC2 | 00 – 63 | 37 |  |
|  | 13 | CTRCO8/MUSIC3 | 00 – 63 | 37 |  |
|  | 14 | CTRCO8/MODEM | 00 – 63 | 44 |  |
| 406 |  | DCOB RX Gain |  |  |  |
|  | 1 | DCOB/DKT | 00 – 63 | 26 |  |
|  | 2 | DCOB/SLT | 00 – 63 | 37 |  |
|  | 3 | DCOB/CTR SL | 00 – 63 | 26 |  |
|  | 4 | DCOB/WTU | 00 – 63 | 26 |  |
|  | 5 | DCOB/ACO | 00 – 63 | 24 |  |
|  | 6 | DCOB/CTR CO | 00 – 63 | 15 |  |
|  | 7 | DCOB/DCO | 00 – 63 | 32 |  |
|  | 8 | DCOB/VMIB | 00 – 63 | 32 |  |
|  | 9 | DCOB/DTMF | 00 – 63 | 32 |  |
|  | 10 | DCOB/TONE | 00 – 63 | 32 |  |
|  | 11 | DCOB/MUSIC1 | 00 – 63 | 32 |  |
|  | 12 | DCOB/MUSIC2 | 00 – 63 | 32 |  |
|  | 13 | DCOB/MUSIC3 | 00 – 63 | 32 |  |
|  | 14 | DCOB/MODEM | 00 – 63 | 37 |  |
| 407 |  | VMIB RX Gain |  |  |  |
|  | 1 | VMIB/DKT | 00 – 63 | 21 |  |
|  | 2 | VMIB/SLT | 00 – 63 | 32 |  |
|  | 3 | VMIB/CTR SL | 00 – 63 | 21 |  |
|  | 4 | VMIB/WTU | 00 – 63 | 26 |  |
|  | 5 | VMIB/ACO | 00 – 63 | 32 |  |
|  | 6 | VMIB/CTR CO | 00 – 63 | 23 |  |
|  | 7 | VMIB/DCO | 00 – 63 | 32 |  |
|  | 8 | VMIB/MUSIC1 | 00 – 63 | 32 |  |
|  | 9 | VMIB/MUSIC2 | 00 – 63 | 32 |  |
| 408 |  | DTMF RC Gain |  |  |  |
|  | 1 | DTMF/SLT | 00 – 63 | 28 |  |
|  | 2 | DTMF/CTR SL | 00 – 63 | 17 |  |
|  | 3 | DTMF/ACO | 00 – 63 | 24 |  |
|  | 4 | DTMF/CTR CO | 00 – 63 | 15 |  |
|  | 5 | DTMF/DCO | 00 – 63 | 24 |  |

| PGM | FLEX | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 409 |  | EXT PAGE Gain |  |  |  |
|  | 1 | EXT PAGE/DKT | 00 – 63 | 26 |  |
|  | 2 | EXT PAGE/SLT | 00 – 63 | 37 |  |
|  | 3 | EXT PAGE/CTR SL | 00 – 63 | 26 |  |
|  | 4 | EXT PAGE/WTU | 00 – 63 | 26 |  |
|  | 5 | EXT PAGE/ACO | 00 – 63 | 37 |  |
|  | 6 | EXT PAGE/CTR CO | 00 – 63 | 28 |  |
|  | 7 | EXT PAGE/DCO | 00 – 63 | 37 |  |
|  | 8 | EXT PAGE/VMIB | 00 – 63 | 37 |  |
|  | 9 | EXT PAGE/MUSIC1 | 00 – 63 | 37 |  |
|  | 10 | EXT PAGE/MUSIC2 | 00 – 63 | 37 |  |
|  | 11 | EXT PAGE/MUSIC3 | 00 – 63 | 37 |  |
| 410 |  | CPT Gain |  |  |  |
|  | 1 | CPT/ACO | 00 – 63 | 24 |  |
|  | 2 | CPT/CTR CO | 00 – 63 | 15 |  |
|  | 3 | CPT/DCO | 00 – 63 | 24 |  |
| 411 |  | MODEM Gain |  |  |  |
|  | 1 | MODEM/ACO | 00 – 63 | 24 |  |
|  | 2 | MODEM/CTR CO | 00 – 63 | 20 |  |
|  | 3 | MODEM/DCO | 00 – 63 | 24 |  |
| 412 |  | Short SLIB Gain |  |  | SAF only |
|  | 1 | Shot ACO | 00 – 63 | 31 |  |
|  | 2 | Long ACO | 00 – 63 | 31 |  |
| 413 |  | Long SLIB Gain |  |  | SAF only |
|  | 1 | Shot ACO | 00 – 63 | 37 |  |
|  | 2 | Long ACO | 00 – 63 | 37 |  |
| 414 |  | Far SLIB Gain |  |  | SAF only |
|  | 1 | Shot ACO | 00 – 63 | 45 |  |
|  | 2 | Long ACO | 00 – 63 | 45 |  |
| 415 |  | Short ACO Gain |  |  | SAF only |
|  | 1 | Short SLIB | 00 – 63 | 35 |  |
|  | 2 | Long SLIB | 00 – 63 | 41 |  |
|  | 3 | Far SLIB | 00 – 63 | 47 |  |
| 416 |  | Long ACO Gain |  |  | SAF only |
|  | 1 | Short SLIB | 00 – 63 | 39 |  |
|  | 2 | Long SLIB | 00 – 63 | 45 |  |
|  | 3 | Far SLIB | 00 – 63 | 51 |  |
| 420 |  | System Tone Frequency |  |  |  |
|  | 1 | Dial Tone | 4digits | 0425, 0000 |  |
|  | 2 | Ring Back Tone | 4digits | 0425, 0000 |  |
|  | 3 | Busy Tone | 4digits | 0425, 0000 |  |
|  | 4 | Error Tone | 4digits | 0620, 000 |  |
|  | 5 | Dummy Dial Tone | 4digits | 0350, 440 |  |
| PGM | FLEX | ITEM | RANGE | DEFAULT | REMARK |
| 421 |  | Differential Ring Frequency |  |  |  |
|  | 1 | Ring 1 | 4digits | 1000, 1020 |  |
|  | 2 | Ring 2 | 4digits | 0890, 0910 |  |
|  | 3 | Ring 3 | 4digits | 1260, 1280 |  |
|  | 4 | Ring 4 | 4digits | 0800, 0820 |  |
| 422 |  | Distinct Ring Frequency |  |  |  |
|  | 1 | Ring 1 | 4digits | 0480, 0000 |  |
|  | 2 | Ring 2 | 4digits | 0400, 0000 |  |
|  | 3 | Ring 3 | 4digits | 0620, 0000 |  |
|  | 4 | Ring 4 | 4digits | 0770, 0000 |  |
| 423 |  | ACNR Tone Cadence |  |  |  |
|  | 1 | Ring-Back Tone | 0-255 | 050, 100 | 20msec base |
|  | 2 | Busy Tone | 0-255 | 025, 025 | 20msec base |
|  | 3 | Error Tone | 0-255 | 012, 012 | 20msec base |
|  | 4 | S –Dial Tone | 0-255 | 070, 000 | 20msec base |

#### TABLE 7.4.26  INITIALIZATION

| PGM | FLEX | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 450 |  | Initialization |  |  |  |
|  | 1 | Flexible Numbering Plan Initialization |  |  |  |
|  | 2 | Station Database Initialization |  |  |  |
|  | 3 | CO Line Database Initialization |  |  |  |
|  | 4 | System Feature Database Initialization |  |  |  |
|  | 5 | Station Group Database Initialization |  |  |  |
|  | 6 | ISDN Tables Database Initialization |  |  |  |
|  | 7 | Reserved |  |  | Reserved |
|  | 8 | System Timer Database Initialization |  |  |  |
|  | 9 | Toll Table Database Initialization |  |  |  |
|  | 10 | LCR Database Initialization |  |  |  |
|  | 11 | Tables Initialization |  |  |  |
|  | 12 | Flexible Button Program Initialization |  |  |  |
|  | 13 | Networking Database Initialization |  |  |  |
|  | 14 | All Database Initialization |  |  |  |
|  | 15 | System Reset By Software |  |  |  |

#### TABLE 7.4.27  PRINT PROT DATABASE

| PGM | FLEX | ITEM |  | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
| 451 |  | Print Prot Data |  |  |  |  |
|  | 1 | Flexible Numbering Plan Print |  |  |  |  |
|  | 2 | Station Database Print |  | STN_R |  |  |
|  | 3 | CO Line Database Print |  | CO_R |  |  |
|  | 4 | System Feature Database Print |  |  |  |  |
|  | 5 | Station Group Database Print |  |  |  |  |
|  | 6 | ISDN Tables Database Print |  |  |  |  |
|  | 7 | System Timer Database Print |  |  |  |  |
|  | 8 | Toll Table Database Print |  |  |  |  |
|  | 9 | LCR Database Print |  |  |  |  |
|  | 10 | Other Tables Print |  |  |  |  |
|  | 11 | Nation Specific Database Print |  |  |  |  |
|  | 12 | Flexible Button Program Print |  | STN_R |  |  |
|  | 13 | Networking Data Print |  |  |  |  |
|  | 14 | All Database Print |  |  |  |  |
|  | 15 | LCD Message Print |  |  |  |  |
|  |  | 1 | Language | 00 – 12 | Nation specific | 00:ENG 01:ITA 02:FIN 03:DUT 04:SWE 05:DAN 06:NOR 07:HUN 08:GER 09:FRE 10:POR 11:SPA 12:KOR |
|  |  | 2 | Station Type | 0 – 2 | 0 | 0: NORMAL 1: LG-GAP 2: LARGE |
|  | 16 | Quit Print |  |  |  |  |