# APPENDIX 1. SYSTEM LIMITATION : POWER SUPPLY UNIT

The capacity of PSU limits the number and type of extensions which may be connected to Peripheral Boards. Each peripheral board has its figure of Power Consumption as in below.

# The sum of the Power consumption of all peripheral boards in each KSU should not exceed the maximum limitation.

# 1. LDK-300

| # Board | # +5V | # -5V | # +30V | # Board | # +5V | # -5V | # +30V |
| --- | --- | --- | --- | --- | --- | --- | --- |
| # MPB | # 1,000mA | # 40mA | # 10mA | # EMIB | # 95mA | # 55mA | # 230mA |
| # MISB | # 55mA | # 40mA | # 40mA | # PRIB | # 280mA | # None | # None |
| # SIU | # 180mA | # None | # None | # R2DCOB | # 425mA | # None | # None |
| # LCOB | # 120mA | # 80mA | # 80mA | # STIB | # 200mA | # None | # 150mA |
| # DTIB12 | # 100mA | # None | # 780mA | # BRIB | # 190mA | # None | # None |
| # SLIB24 | # 100mA | # 70mA | # 215mA | # SLIBII | # 240mA | # 160mA | # 490mA |
| # SLIB48 | # 115mA | # 75mA | # 750mA | # WTIB | # 650mA | # None | # 200mA |
| # PFTU | # None | # None | # 55mA | # WTIU | # 400mA | # None | # 200mA |
| # VMIB/AAIB | # 660mA | # None | # None | # RGU | # None | # None | # 60mA |
| # VOIB | # 700mA | # None | # None | # RGU5(Ext) | # None | # None | # 120mA |
| # VOIU | # 100mA | # None | # None | # LMU1 | # 20mA | # None | # 5mA |
| # LCOB8 | # 482mA | # 83mA | # None | # LMU2 | # 20mA | # None | # 5mA |
| # SLIB2E | # 225mA | # 36mA | # 460mA | # DTIB24 | # 165mA | # None | # 1,560mA |
| # RAU | # 10mA | # None | # 10mA | # DSIB | # 120mA | # 45mA | # 595mA |

# 2. LDK-300 EXPANSION(LDK-600)

| # Board | # +5V | # -5V | # +30V | # Board | # +5V | # -5V | # +30V |
| --- | --- | --- | --- | --- | --- | --- | --- |
| # MPBE | # 1,200mA | # 40mA | # 10mA | # LMUE | # 60mA | # 20mA | # 10mA |

# * Note : Above power consumption is measured with the optional boards.

| # TYPE | # +5V | # -5V | # +30V | # BATT. BACK UP | # REMARK |
| --- | --- | --- | --- | --- | --- |
| # PSU | # 10A | # 1.5A | # 8.4A | # 1A / 27.3V | # 350W |
| # PSU3 | # 5A | # 1A | # 7A | # 0.5A / 27.3V | # 250W |

# APPENDIX 2. BATTERY SPECIFICATION FOR INDIA

| # TYPE | # +5V | # -5V | # +30V | # BATT. BACK UP | # REMARK |
| --- | --- | --- | --- | --- | --- |
| # PSU | # 10A | # 1.5A | # 7.9A | # 1.5A / 27.3V | # 350W |
| # PSU3 | # 5A | # 1A | # 6A | # 1.5A / 27.3V | # 250W |

# APPENDIX 3. DECLARATION OF CONFORMITY

# 1. ST

# 2. IT