SECTION 4. ISDN BOARDS

This section briefly introduces ISDN boards, Number Assignment, and the other information.

For more details for installation, see the ISDN part of clauses 2.4 System components, 3.4.6 Board Installation, and 3.5.7 ISDN wiring.

Otherwise, for more details for the features and Admin programming related to ISDN, see Programming Manual.

### Basic Information

There are several kinds of ISDN board on LDK-100 system as follows;

| Board | Channels (Basic) | Channels (Option) | Interface | Max. Board | Remark |
| --- | --- | --- | --- | --- | --- |
| LDK-100 BRIB | 4B Ch. (2BRI) | 4B Channels(2BRI) (LDK-100 BRIU) | T-interface | 2 | No CPU on board |
| LDK-100 PRIB | 30B Ch. (1PRI) | - | T-interface | 2(max. 40 ch.) | No CPU on board |
| LDK-300 BRIB | 8B Ch. (4BRI) | - | T-interface | 5 | CPU on board |
| LDK-300 STIB | 8B Ch. (4BRI) | - | S/T-interface | 5 | CPU on board |
| LDK-300 PRIB | 30B Ch. (1PRI) | - | T-interface | 2(max. 40 ch.) | CPU on board |

Table 4.1.1 ISDN boards in LDK-100

- Note:

- LDK-300 BRIB/STIB/PRIB board can be used in LDK-100 and LDK-300 system.

- You can install ISDN boards within the station and CO line capacity of the system.

|  | Max. Stations | Max. CO Lines |
| --- | --- | --- |
| LDK-100 | 96 | 40 |

Table 4.1.2 ISDN port

* Not achieved the simultaneously. See the system capacity.

- The maximum 8 ISDN TEs (Terminal Equipment) can be connected to the single S-Interface port, and any kind of ISDN TEs, which are compatible with ETSI ISDN specification, can be connected to the S-Interface. For example, ISDN phone, G4 Fax, Video Terminal, ISDN PC Card, etc. can be connected.

4) 	Firmware Marking Table for LDK-300 ISDN boards

| ##### BOARD | Higher byte |  | Lower byte |  |
| --- | --- | --- | --- | --- |
|  | Label on ROM | Reference on PCB | Label on ROM | Reference on PCB |
| BRIB | GS00B-Version ROM1 (Checksum) | ##### U11(ROM1) | GS00B-Version ROM2 (Checksum) | ##### U10(ROM2) |
| STIB | GS00N-Version ROM1(Checksum) | ##### U11(ROM1) | GS00N-Version ROM2(Checksum) | ##### U10(ROM2) |
| PRIB | GS00Q-Version ROM1 (Checksum) | ##### U201(ROM1) | GS00Q-Version ROM2 (Checksum) | ##### U200(ROM2) |

Table 4.1.3 Firmware Marking Table

### Station/CO Line Number Assignments

- As same as any other station/CO cards, the station/CO line number on ISDN card is assigned by ADMIN 101, 103 in Admin. Programming.

- In PRIB, the 30 continuous CO line numbers are assigned by ADMIN 101, 103.

- In BRIB, the 8 continuous CO line numbers are assigned by ADMIN 101, 103. Two continuous CO line numbers are assigned per each port (2B+D).

- In STIB with all T-Interfaces, as same as BRIB, the 8 continuous CO line numbers are assigned by ADMIN 101, 103. Two continuous CO line numbers are assigned per each port (2B+D).

- In STIB with all S-Interfaces, the 8 continuous station numbers are assigned by ADMIN 101, 103. Two continuous station numbers are assigned per each port (2B+D).

- If STIB is used for both S-Interface and T-interface, then system assigns the continuous CO line number on each T-Interface, and the continuous station number on each S-Interface, as follows;

( If the first Sta. No. is 120 and the first CO No. is 24)

|  | Port 1 | S-Interface | Station 120-121 |
| --- | --- | --- | --- |
| STIB Board | Port 2 | T-Interface | CO line 24-25 |
|  | Port 3 | S-Interface | Station 122-123 |
|  | Port 4 | T-Interface | CO line 26-27 |

Table 4.1.4 STIB port

- Two continuous station numbers are assigned to a single S-Interface act as the same station number. For example, calling to Station 120 is the same to calling to Station 121.

- If you want to change S-Interface to T-Interface, or vise versa on the STIB board, you have to re-program Slot Assignment (ADMIN 101,103) even if the Slot Assignment is not changed and reset the GDK system. After that, system works as required.

- In STIB,

To use 'T' interface only - Slot Assignment (PGM 101,103) at COL only

To use 'T' & 'S' interface - Slot Assignment (PGM 101,103) at COL & STA

To use 'S' interface only - Slot Assignment (PGM 101,103) at COL & STA

### Other Information

- ISDN TES (Terminal Equipment)

- There are many ISDN TES. (ex. ISDN Phone, Video-Phone, G4 Fax, PC-Card, etc.) LDK-100 supports all ISDN TES which are compatible with Euro-ISDN (ETSI ISDN Specification) till now on.

- ISDN phone can call another station on the LDK-100 and access analog/ISDN CO lines.

- Etc.

-	ISDN station cannot be the master of conference and a member of conference.

-	ISDN station cannot call station which is activated to Off-net Call Forward.

-	ISDN station cannot be an attendant station.