# APPENDIX 1. SYSTEM LIMITATION : POWER SUPPLY UNIT

The capacity of PSU limits the number and type of extensions which may be connected to Peripheral Boards. Each peripheral board has its figure of Power Consumption as shown below.

# The sum of the Power consumption of all peripheral boards in each KSU should not exceed the maximum limitation.

| # Board | # +5V | # -5V | # +30V |
| --- | --- | --- | --- |
| # MPB | # 868mA | # 85mA | # 10mA |
| # DTIB 12 | # 50mA | # None | # 558mA |
| # DTIB 24 | # 60mA | # None | # 1116mA |
| # SLIB 24 | # 100mA | # 70mA | # 215mA |
| # SLIB 48 | # 115mA | # 75mA | # 750mA |
| # SLIB II | # 240mA | # 160mA | # 490mA |
| # SLIB 2E | # 225mA | # 36mA | # 460mA |
| # DSIB | # 120mA | # 45mA | # 595mA |
| # LCOB | # 120mA | # 80mA | # 80mA |
| # LCOB 8 | # 482mA | # 83mA | # None |
| # VOIB | # 700mA | # None | # None |
| # VOIU | # 100mA | # None | # None |
| # LDK-300 BRIB | # 190mA | # None | # None |
| # LDK-300 PRIB | # 280mA | # None | # None |
| # LDK-100 BRIB | # 190mA | # None | # None |
| # LDK-100 PRIB | # 280mA | # None | # None |
| # STIB | # 200mA | # None | # 150mA |
| # E1IB | # 340mA | # 30mA | # None |
| # WTIB | # 650mA | # None | # 200mA |
| # WTIB + WTIU | # 1180mA | # None | # 400mA |
| # VMIB | # 660mA | # None | # None |
| # CMU | # 8mA | # 3mA | # None |
| # BWDIDB | # 124mA | # 60mA | # 490mA |
| # MISB | # 55mA | # 40mA | # 40mA |
| # RGU6 | # None | # None | # 85mA |

# * Note : Above power consumption is measured with the optional boards.

| # TYPE | # +5V | # -5V | # +30V | # BATT. BACK UP | # REMARK |
| --- | --- | --- | --- | --- | --- |
| # LDK-100 PSU | # 4A | # 0.5A | # 3A | # 0.5A / 27.3V | # 120W |

# APPENDIX 2. DECLARATION OF CONFORMITY

# 1. ST

# 2. IT