CONTENTS

6.  SYSTEM TIMERS	1

6.1	SYSTEM TIMERS - I  (PGM 180)	1

6.2	SYSTEM TIMERS - II  (PGM 181)	4

6.3	SYSTEM TIMERS - III  (PGM 182)	6

7.	STATION GROUP PROGRAMMING (PGM 190, PGM 191)	8

7.1	GROUPS ASSIGN, TYPE AND MEMBER SETTING  (PGM 190)	8

7.2	ASSIGN THE FEATURE OF SELECTED GROUP  (PGM 191)	10

7.2.1  CIRCULAR / TERMINAL GROUPS  (PGM 191)	10

7.2.2  UCD GROUP ATTRIBUTE  (PGM 191)	13

7.2.3  RING GROUP ATTRIBUTE  (PGM 191)	17

7.2.4  VM GROUP ATTRIBUTE  (PGM 191)	19

7.2.5  PICK_UP GROUP ATTRIBUTE  (PGM 191)	21

8.	ISDN PROGRAM	22

8.1	ISDN ATTRIBUTE  (PGM 200)	22

8.2	COLP TABLE  (PGM 201)	24

8.3	MSN TABLE  (PGM 202)	25

# 6.  SYSTEM TIMERS

## 6.1	SYSTEM TIMERS - I  (PGM 180)

PROCEDURE

| SYSTEM TIMER 1 PRESS FLEX KEY (01-22) | - [TRANS/PGM] + 180. |
| --- | --- |
|  | - To program, use the BTNs as Table 9.1.1. Press one of BTN 1-22 and related data. (LED of selected Flex. Button will be lit.) |
| - ATD RECALL TMR(min) - (00 – 60) : 01 | - To program the Atd Recall Timer, press the 1th flexible BTN. The 2nd LCD line message will be displayed and user can input a desired value. Then LCD shows the changed value |
| - SYSTEM TIMER 1 - PRESS FLEX KEY (01-22) | - Press the [HOLD/SAVE] button for updating database permanently. |
| SYSTEM TIMER 1 PRESS FLEX KEY (01-22) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| - BTN | - ITEM | - RANGE | - DEFAULT | - REMARK |
| --- | --- | --- | --- | --- |
| - 1 | - Attendant Recall Timer | - 00 – 60 - (2 Digits) | - 01 | - Determines the amount of time before system disconnects the call. (1 min base) |
| - 2 | - Call Park Recall Timer | - 000 – 600 - (3 Digits) | - 120 | - Determines the amount of time before a call placed in a call park location will recall the station placing the park. (1 sec base) |
| - 3 | Camp-on Recall Timer | 000 – 200 (3 Digits) | 030 | If a station transfers to busy station and hang up, this recall timer is assigned. (1 sec base) |
| - 4 | - Exclusive Hold Recall Timer | - 000 – 300 - (3 Digits) | - 060 | - Determines the amount of time before a call placed on exclusive hold will recall the station placing the hold. (1 sec base) |
| - 5 | - I-Hold Recall Timer | - 000 – 300 - (3 Digits) | - 030 | - Determines the amount of time before a call recalls the attendant. (1 sec base) |
| - 6 | - Sys Hold Recall Timer | - 000 - 300 - (3 Digits) | - 030 | - Determines the amount of time before a call placed on system hold will recall the station placing the hold. (1 sec base) |
| - 7 | - Transfer Recall Timer | - 000 - 300 - (3 Digits) | - 030 | - Determines the amount of time a transferred call will ring at the station receiving the transfer and how long it will recall the station transferring the call. (1 sec base) |
| - 8 | - ACNR Delay Timer | - 000 - 300 - (3 Digits) | - 030 | - When ACNR Pause Timer expires and there is no available CO Line in the group, this timer is invoked. - When ACNR Delay Timer expired, - - Invoke ACNR Pause Timer if is no available CO line Still, ACNR is activated. (1 sec base) |
| - 9 | - ACNR No Answer Timer | - 10 - 50 - (2 Digits) | - 30 | - This Timer is invoked after system detects CO ring back tone or voice from CO party. After this timer, system retries ACNR. (1 sec base) |
| - 10 | - ACNR Pause Timer | - 030 - 300 - (3 Digits) | - 030 | - When expired, ACNR is activated. - (For CIS : 5-300) (1 sec base) |
| - 11 | - ACNR Retry Counter | 1 – 13 - (1 Digit) | - 03 | - This is decreased every time station retries ACNR, ACNR is canceled if set to 0. - (For CIS : 1-9) |
| - 12 | - ACNR Retry No Tone | - 1 – 9 - (5sec) | - 1 | - 1 means 5 seconds, LDK will wait this value to decide NO TONE. - 3 means 15 seconds. (Only for CIS) |
| - 13 | - ACNR Tone Detect Timert | 000 –300 (3 Digits) | - 030 | - This timer is invoked upon completion of dialing and system considers the CO party as busy in the case that CPTU cannot detect the valid tone type until this timer expires. (1 sec base) |
| - 14 | Automatic CO Release Timer. | 020 –300 (3 Digits) | 030 | Uncompleted CO call will be automatically released after this timer. (1 sec base) |
| - 15 | - CCR Inter-Digit Timer | - 000 - 300 - (3 Digits) | - 030 | - This field is used for the CCR inter-digit timer in the DISA/DID CO line. In DID type 2, it is used for DID inter-digit timer (100ms base). |
| - 16 | - CO Call Drop Warning Timer | - 00 - 99 - (2 Digits) | - 10 | - If prepaid money is going to expire during a CO conversation, give warning tone and after this time the call will be disconnected. - This timer also used for Call Restriction, Unsupervised Conference (1 sec base) |
| - 17 | - CO Call Restriction Timer | - 00-99 | 0 | - Outgoing CO call time is allowed for this time. - (1 min base) |
| - 18 | - CO Dial Delay Timer | - 00 - 99 - (2 Digits) | - 01 | - Voice connection to the outside party will be made after this timer. This can be used to prevent illegal dialing in case of slow response from the Central Office Line or PBX.(100ms base) |
| - 19 | - CO Release Guard Timer | - 001 – 150 - (3 Digits) | - 002 | - The CO Release Guard Timer controls the time necessary to guarantee idle loop state when the line is released. (100 ms base) |
| - 20 | CO Ring Off Timer | 010 -150 (3 Digits) | 060 | This timer is to secure time interval between incoming ringing signals so that the active ringing can be lasted in the system until this timer is expired. (100ms base) |
| - 21 | - CO Ring On Timer | - 1 – 9 - (1 Digit) | - 2 | - The CO Ring On Timer controls the time necessary to detect an outside line as ringing into the system. (100 ms base) |
| - 22 | - CO Warning Tone Timer | - 060 – 900 - (3 Digits) | - 180 | - Determines the amount of time before receiving warning tone in order to remind the call elapsed time in case of outgoing CO conversations (Only for Korea ). (1 sec base) |

TABLE 9.1.1   Button Configuration for System Timers - I  (PGM 180)

## 6.2	SYSTEM TIMERS - II  (PGM 181)

PROCEDURE

| SYSTEM TIMER 2 PRESS FLEX KEY (01-12) | - [TRANS/PGM] + 181. |
| --- | --- |
|  | - To program, use the BTNs as Table 9.2.1. Press one of BTN 1-12 and related data. (LED of selected Flex. Button will be lit.) |
| - CFW NO ANS TMR(sec) - (000 – 600) : 015 | - To program the Call Forward No Ans Timer, press the 1th flexible BTN. The 2nd LCD line message will be displayed and user can input a desired value. Then LCD shows the changed value |
| - SYSTEM TIMER 2 - PRESS FLEX KEY (01-12) | - Press the [HOLD/SAVE] button for updating database permanently. |
| SYSTEM TIMER 2 PRESS FLEX KEY (01-12) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| - BTN | - ITEM | - RANGE | - DEFAULT | - REMARK |
| --- | --- | --- | --- | --- |
| - 1 | Call FWD No Answer Timer | 000 -600 (3 Digits) | 015 | The Call forward busy/no answer feature will take place using this timer. If this timer has a non-zero value and a extension is set at busy, no answer forward by station user then the extension will ring for this timer and take place a forward to the next. (1 sec base) |
| - 2 | DID/DISA No Answer Timer | 00 -99 (2 Digits) | 20 | A DID call will be forwarded attendant if the station is busy or does not answer within this time. (1 sec base) |
| - 3 | DVU User Record Timer | 010 – 255 (3 Digits) | 20 | The time duration of DVU user greeting. (1 sec base) |
| - 4 | DVU Valid User Message Timer | 0-9 (1 Digits) | 4 | The time duration of valid DVU user message. (1 sec base) |
| - 5 | Door Open Timer | 05 –99 (2 Digits) | 20 | This timer determines of the length of time that is needed to activate a door open relay for the set time. (100 ms base) |
| - 6 | ICM Box Timer | 00 -60 (2 Digits) | 30 | Determines the amount of time programmed stations will ring when ICM box user presses the [CALL] button. (1 sec base) |
| - 7 | ICM Dial Tone Timer | 01-20 (2 Digits) | 10 | If action is not taken within ICM dial tone timer, user will hear error-tone. (1 sec base) |
| - 8 | Inter Digit Timer | 01-20 (2 Digits) | 05 | The time between digits cannot exceed Inter-digit timer, or error tone is received. (1 sec base) |
| - 9 | MSG Wait Reminder Tone Timer | 00 -60 (2 Digits) | 00 | Determines the amount of time between repeated reminder tones to a key telephone with a message waiting. (1 min base) |
| - 10 | Paging Timeout Timer | 000 –255 (3 Digits) | 15 | Determines the maximum time of a page. The system will automatically disconnect the page at the end of this time unless the caller has hung up earlier. (1 sec base) |
| - 11 | - Pause Timer | - 1 – 9 - (1 Digit) | - 2 | - Determines the length of the pause for use with automatically sent digits or other speed dialing. - (1 sec base) |
| - 12 | - Preset Call Forward Timer | - 00 – 99 - (2 Digits) | - 10 | - Determines the amount of time an outside line will ring before being forwarded to a predetermined station. This entry works with Preset Forward Assignments in station attributes. More than one station can be forwarded to the same destination. - (1 sec base) |

TABLE 9.2.1   Button Configuration for System Timers - II  (PGM 181)

## 6.3	SYSTEM TIMERS - III  (PGM 182)

PROCEDURE

| SYSTEM TIMER 3 PRESS FLEX KEY (01-11) | - [TRANS/PGM] + 182. |
| --- | --- |
|  | - To program, use the BTNs as Table 9.3.1. Press one of BTN 1-11 and related data. (LED of selected Flex. Button will be lit.) |
| - STA AUTO RLS TMR (sec) - (020 – 300) : 060 | - To program the Station Auto Release Timer, press the 5th flexible BTN. The 2nd LCD line message will be displayed and user can input a desired value. Then LCD shows the changed value |
| - SYSTEM TIMER 3 - PRESS FLEX KEY (01-11) | - Press the [HOLD/SAVE] button for updating database permanently. |
| SYSTEM TIMER 3 PRESS FLEX KEY (01-11) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| - BTN | - ITEM | - RANGE | - DEFAULT | - REMARK |
| --- | --- | --- | --- | --- |
| - 1 | SLT Hook Switch Bounce Timer | 01-25 (2 Digits) | 01 | This timer determines the length of timer that is needed to regard as a valid on-hook or off-hook. (For SLT) (100 ms base) |
| - 2 | SLT Maximum Hook Flash Timer | 01-25 (2 Digits) | 05 | This timer determines how long the user could depress the hook switch in order for it to be considered a FLASH (Timed-Break Recall). (For SLT) (100 ms base) |
| - 3 | SLT Minimum Hook Flash Timer | 000 -250 (3 Digits) | 020 | The minimum bound time that system considers as hook flash for SLT. (10 ms base) |
| - 4 | SLT Ring Phase Timer | 2 – 5 (1 Digit) | 5 | Determines the ring phase of SLT . (5 SEC : 1SEC ON / 4SEC OFF) (1 sec base) |
| - 5 | Station Auto Release Timer | 020 – 300 (3 Digits) | 060 | If a station hears ring back tone and no action is taken, this timer is assigned. When this timer is expired the station is released. (1 sec base) |
| - 6 | - Unsupervised Conference Timer | - 00 - 99 - (2 Digits) | - 10 | - Determines the amount of the time an unsupervised conference can continue after the initiator of the conference has exited the conference. (1 min base) |
| - 7 | Wake-Up Fail Ring Timer | 00-99 | 20 | After a Wake-up fail ring invokes on SYSTEM ATD, the alarm ring exists during this timer. Then if this timer expires, the Alarm ring will be disappeared. (1 sec base) |
| - 8 | - Warm Line Timer | 010 - 200 - (2 Digits) | - 05 | - User takes no action after lifting handset or pressing the [MON] button and warm line timer is expired, then idle line selection for warm line is activated. - (1 sec base) |
| - 9 | Wink Timer | 010 -200 (3 Digits) | 010 | The Time Duration of Seize Acknowledge Signal to DID line. (10 ms base) |
| - 10 | Enblock Dgt timer | 01-20 | 15 | After timer is expired, Setup is sent (1 sec base) |
| - 11 | CCR Time Out Timer | 000-300 | 015 | When this timer is expired, CCR is activated (1 sec base) |

- TABLE 9.3.1   Button Configuration for System Timers - III  (PGM 182)

# 7.	STATION GROUP PROGRAMMING (PGM 190, PGM 191)

Stations in the system can be grouped so that incoming calls will search (hunt) for an idle station in the group. Three hunting processes can be assigned; Circular, Terminal, or UCD (Uniform Call Distribution). Each of the system's groups is assigned as a function; Call Pick-Up Group and/or Hunt Group, Voice Mail Group, and Ring Group. The available group number and station number in a group is as follows:

| System | LDK – 300 |
| --- | --- |
| No. of Group | 48 |
| STA No. in a Group | 64 |

- A station can belong to any number of Pickup groups, but can only belong to one Station Hunt group, Voice mail group or Ring group

- When assigning a station group to any type of hunt group or voice mail group, ring, pick up group, the system initializes hunt attributes by default value for it's own function. It can be programmed to meet each customer's individual need.

## 7.1	GROUPS ASSIGN, TYPE AND MEMBER SETTING  (PGM 190)

PROCEDURE

| STATION GRP ASSIGN ENTER GROUP NUMBER |  | - [TRANS/PGM] + 190. |
| --- | --- | --- |
| STATION GRP 620 F1:TYPE F2:PKUP F3:MEM |  | - Enter Station Group Number 620 – 668 (Ex 620). Then user can set the type, pick_up feature and member of the selected group. First, choose flex BTN 1 and set TYPE. (see TABLE 6.1.1) |
| STATION GRP 620 NOT ASSIGNED (0-6) |  | - To select TYPE, Press one digit 0 – 6 (Ex. 1). |
| STATION GRP 620 CIRCULAR GROUP (0-6) |  | - To save, Press the [HOLD/SAVE] button for updating database.. |
| GROUP 620 PICK-UP (1:ON/0:OFF) : OFF |  | - To set pick_up feature of the Circular Group, Press flex BTN 2. And Dial one digit. |
| STATION GRP 620 F1:TYPE F2:PKUP F3:MEM |  | - To assign member of the Circular Group, Press flex BTN 3. |
| CIRCULAR 620 . . . . . . . . . . . . . . . . |  | - Choose one of 4 flex BTN, enter station number step by step..or enter the Sta range( ex: 100120) |
|  | STATION GRP 620 F1:TYPE F2:PKUP F3:MEM | - Press the [HOLD/SAVE] button for updating database permanently. User can set another group by going step (1). (Ex. 620 – 668) Note: For PGM 191, pre-setting of station group is absolutely necessary. |
|  | STATION GRP 620 F1:TYPE F2:PKUP F3:MEM | - Press [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| DGT | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Group Type | 0-6 | 0 | 0:NOT ASGN 1: Circular 2: Terminal 3: UCD 4: Ring 5: VM 6: Pick up |
| 2 | Pick-up Attribute | ON/OFF | OFF | OFF |
| 3 | Member assignment | Not Assigned | - | First, Group Type must be assigned |

TABLE 7.1.1  Configuration for Station Group TYPE (PGM 190)

## 7.2	ASSIGN THE FEATURE OF SELECTED GROUP  (PGM 191)

### 7.2.1  CIRCULAR / TERMINAL GROUPS  (PGM 191)

The features of Terminal Group are same as that of Circular Group.

PROCEDURE

| STATION GRP ASSIGN ENTER GROUP NUMBER |  | - [TRANS/PGM] + 191. - Note : To program this, first, assign Circular or Terminal group to a certain group number . |
| --- | --- | --- |
| CIRC GRP 621 PRESS FLEX_KEY (01-13) |  | - Enter Station Group Number (Ex 621, If user set 621 to circular or terminal group). Choose one of 13 flex BTNs and enter desired value of the group feature. |
| CIRC 621 ANNC 1 TMR (000 - 999) : 015 |  | - To assign DVU Announce 1 Timer, choose flex BTN 1 and enter three digits. To save, go step (14). |
| CIRC 621 ANNC1 LOC VMIB MSG … (00 – 70) |  | - To assign DVU Announce Location 1, Choose flex BTN 3 and enter two digits (DVU Msg Number). To save, go step (14). |
| CIRC 621 ANNC1 LOC VMIB MSG 065 (#) (00 – 70) |  | - To drop the call after DVU announce, dial digit ’#’ To save, go step (14). |
| CIRC 621 ANNC2 RPT TMR (000 - 999) : 000. |  | - To assign DVU Announce 2 Repeat, choose flex BTN 5 and enter three digits. To save, go step (14). |
| CIRC 621 OVFLOW DEST STA/HUNT/VMIB (D1-D3) |  | - To assign Overflow Destination, choose flex BTN 7 and enter one digit. - Dial ‘1’ Enter station number, - Dial ‘2’ Enter Hunt group number, - Dial ‘3’ Enter DVU number - Press the [SPEED] button for deleting. To save, go step (14). |
| CIRC 621 OVERFLOW DEST STA 105 |  | - For example, enter digit 1 , next enter Station Number.(ex 105) To save, go step (14). |
| CIRC 621 OVERFLOW TMR (000 - 600) : 180 |  | - To assign Overflow Timer, choose flex BTN 8 and enter three digits. To save, go step (14). |
| CIRC 621 NO ANS TMR (00 – 99) : 15 |  | - To assign No Answer Timer, choose flex BTN 10 and enter two digits (00-99) To save, go step (14). |
| CIRC 621 PILOT HUNT (1 : ON/ 0: OFF) : OFF |  | - To assign Pilot Hunt, choose flex BTN 11 and enter desired value. To save, go step (14). |
| CIRC 621 REPT NO MEM (1 : ON/ 0: OFF) : OFF |  | - To assign REPT NO MEM, choose flex BTN 12 and enter desired value. To save, go step (14). |
| CIRC 621 MUSIC SRC (00 – 12) : 00 |  | - To assign Music Source, choose flex BTN 13 and enter two digits. To save, go step (14). |
|  | CIRC GRP 621 PRESS FLEX_KEY (01-13) | - Terminal Group Feature is just same as Circular. Press the [HOLD/SAVE] button for updating database permanently. For another group’s setting, first assign type of certain group number in PGM 190 and go step (1) and continue this process. |
|  | CIRC GRP 621 PRESS FLEX_KEY (01-13) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | DVU Announce 1 Timer | 000-999 | 015 | If this timer expires after call come in the group, the system announces the greeting if exists. (1 Sec Base) |
| 2 | DVU Announce 2 Timer | 000-999 | 000 | If this timer expires after call come in the group, the system announces the DVU if assigned. (1 Sec Base) |
| 3 | DVU Announce Location 1 | 00-70 | 00 (not_asgn) | This is used to announce greeting when the DVU announce 1 timer is expired. |
| 4 | DVU Announce Location 2 | 00-70 | 00 (not_asgn) | This is used to announce DVU when the DVU announce 2 timer is expired. |
| 5 | DVU Announce 2 Repeat | 000-999 | 000 | This is used to repeat DVU announce 2 when the timer is expired. (1 Sec Base) |
| 6 | DVU Announce 2 Repeat E/D | ON/OFF | OFF | This is used to enable or disable DVU Announce 2 Repeat. |
| 7 | Overflow Destination | Sta #./ HUNT #./ DVU # |  | The call to a station in the group will continue to route until answered or each station in the group has been tried. The call will remain at the last station in the group or will be passed to this overflow station/group/DVU. |
| 8 | Overflow Timer | 000-600 | 180 | If this timer expires after a call comes in the group, the call is routed to the overflow destination. (1 Sec Base) |
| 9 | Wrap-Up Timer | 002-999 | 002 | A station in a hunt group is maintained in a busy state for a minimum of six seconds after any call and for hunt group calls for the assigned wrap-up time. (1 Sec Base) |
| 10 | No Answer Timer | 00-99 | 15 | In circular hunt, calls to a station in the group will go to the station, if unavailable or unanswered in this no answer time, the call is directed to the next station in the group. (1 Sec Base) |
| 11 | Pilot Hunt | ON/OFF | ON | A circular hunt group can be assigned with a pilot number (the station group) so that only calls to the pilot number will hunt. |
| 12 | REPT NO MEM | ON/OFF | OFF | If there is no member on duty, ICM call will be dropped or Co incoming call will be routed to ATD |
| 13 | Music Source | 00-12 | 00(not_asgn) | If music source is assigned, calling user will be heard music instead of ring back tone. 00: No Asgn 01: Internal Music 02: External Music 1 03: External Music 2 04: External Music 3 05: VMIB BGM 1 06: VMIB BGM 2 07: VMIB BGM 3 08: SLT 1 09: SLT 2 10: SLT 3 11: SLT 4 12: SLT 5 |

TABLE 7.2.1.1  Button Configuration for Circular/Terminal Group Attribute (PGM 191)

### 7.2.2  UCD GROUP ATTRIBUTE  (PGM 191)

When assigning a station group to any type of hunt group, the system initializes hunt attributes by default value for it's own function. It can be programmed to meet each customer's individual need.

PROCEDURE

| STATION GRP ASSIGN ENTER GROUP NUMBER | - [TRANS/PGM] + 191. - Note : To program this, first, assign UCD group to a certain group number . |
| --- | --- |
| UCD GRP 623 PRESS FLEX_KEY (01-18) | - Enter Station Group Number (Ex 623, If user set 623 to UCD group at PGM 190). Choose one of 18 flex BTNs and enter desired value of the group feature. |
| UCD 623 ANNC 1 TMR (000 – 999) : 000 | - To assign DVU Announce 1 Timer, choose flex BTN 1 and enter three digits. To save, go step (10). |
| UCD 623 ANNC1 LOC VMIB MSG . . . (00 – 70) | - To assign DVU Announce Location 1, choose flex BTN 3 and enter two digits(DVU Msg Number). To save, go step (10). |
| UCD 623 ANNC2 RPT TMR (000 – 999) : 000 | - To assign DVU Announce 2 Repeat, choose flex BTN 5 and enter three digits. To save, go step (10). |
| UCD 623 ACD WARN TONE (1 : ON/ 0: OFF) : OFF | - To assign ACD Warning Tone, choose flex BTN 12 and enter desired value. To save, go step (10). |
| UCD 623 ALTER DEST (STA/STA_G) | - To assign Alternate Destination, choose flex BTN 14 and enter one digit. - Dial '1' and enter station number, - Dial ‘2’ Enter Hunt group number, - Press the [SPEED] button for deleting. To save, go step (10). |
| UCD 623 SUPERVISOR STA … | - To assign Supervisor, choose flex BTN 17 and enter Sta #. - Press the [SPEED] button for deleting. To save, go step (10). |
| 100 110 123 124 0 0 0 0 | - To assign UCD priority, press flx BTN 1-4 and enter priority ( 0 – 9 ) |
| UCD GRP 623 PRESS FLEX_KEY (01-18) | - Press the [HOLD/SAVE] button for updating database permanently. For another group’s setting, first assign the type of certain group number in PGM 190 and go step (1) and continue this process. |
| UCD GRP 623 PRESS FLEX_KEY (01-18) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | DVU Announce 1 Timer | 000 - 999 | 015 | If all stations in the group are busy when a call is received for the group, the call may continue to wait (queue) for an available station in the group. If queued, the call may be sent to a UCD announcement when the queue period exceeds the 1st announcement Timer. If the timer is set to 0 the call will receive the full first announcement prior to the hunting process (guaranteed announcement). (1 Sec Base) |
| 2 | DVU Announce 2 Timer | 000 - 999 (3 Digits) | 000 | The second announcement can be provided if the call continues to wait beyond the 2nd announcement timer. (1 Sec Base) |
| 3 | DVU Announce Location 1 | 00-70 | 00 (not_asgn) | Each Station Hunt Group can be assigned an announcement, which is played when the call is first received. The announcement may be assigned as DVU. |
| 4 | DVU Announce Location 2 | 00-70 | 00 (not_asgn) | The second announcement can be provided after DVU Announce 2 Timer. |
| 5 | DVU Announce 2 Repeat | 000-999 | 000 | This is used to announce DVU announce 2 when the timer is expired. (1 Sec Base) |
| 6 | DVU Announce 2 Repeat E/D | ON/OFF | OFF | This is used to enable or disable DVU Announce 2 Repeat. |
| 7 | Overflow Destination | Sta #./ Sta Grp #. DVU# |  | The queued call may be taken out of the group and directed to an overflow station. |
| 8 | Overflow Timer | 000 - 600 (3 Digits) | 180 | If this timer expires after a call comes in the group, the call is routed to the overflow destination. (1 Sec Base) |
| 9 | Wrap Up Timer | 002 - 999 (3 Digits) | 002 | A station in a hunt group is maintained in a busy state for a minimum of six seconds after any call for the assigned wrap-up time. (1 Sec Base) |
| 10 | REPT NO MEM | ON/OFF | OFF | If there is no member on duty, ICM call will be dropped or Co incoming call will be routed to ATD |
| 11 | Music Source | 00-12 | 00 | If music source is assigned, calling user will be heard music instead of ring back tone. 00: No Asgn 01: Internal Music 02: External Music 1 03: External Music 2 04: External Music 3 05: VMIB BGM 1 06: VMIB BGM 2 07: VMIB BGM 3 08: SLT 1 09: SLT 2 10: SLT 3 11: SLT 4 12: SLT 5 |
| 12 | ACD Warning Tone | ON/OFF | OFF | Determines that the ACD supervisor monitors an agent with warning tone or without warning tone |
| 13 | Alternate destination | Sta No/ Sta Grp # | .... | When a call comes into the group and there is no available station in the group, then the call will be routed to this destination if assigned. |
| 14 | Supervisor Timer | 000 – 999 (3 Digits) | 030 | When the queued timer is longer than this timer, the number of queued lines will be displayed onto supervisor's LCD. (1 Sec Base) |
| 15 | Supervisor Call Cnt | 00 - 99 (2 Digits) | 00 | If the number of queued calls is more than this call count, the supervisor timer will be started. |
| 16 | ACD Queued Call | ON/OFF | OFF | ON : display in LCD OFF: no display |
| 17 | Supervisor | Sta# | - | Supervisor Station No. |
| 18 | UCD hunt Stations' Priority | 0 - 9 (1 Digit) | 0 | Ucd group member's Priority |

TABLE 7.2.2.1  Button Configuration for UCD Group Attribute  (PGM 191)

### 7.2.3  RING GROUP ATTRIBUTE  (PGM 191)

| STATION GRP ASSIGN ENTER GROUP NUMBER |  | - TRANS/PGM] + 191. - Note : To program this, first, assign Ring group to a certain group number . |
| --- | --- | --- |
| RING GRP 624 PRESS FLEX_KEY (1-10) |  | - Enter Station Group Number (Ex 624, If user set 624 to Ring group). Choose one of 10 flex BTNs and enter desired value of the group feature. |
| RING 624 ANNC 1 TMR (000 – 999) : 015 |  | - To assign DVU Announce 1 Timer, choose flex BTN 1 and enter three digits. To save, go step (7). |
| RING 624 ANNC1 LOC VMIB MSG . . . ( 00 – 70 ) |  | - To assign DVU Announce Location 1, choose flex BTN 3 and enter Dvu number. To save, go step (7). |
| RING 624 ANNC 2 RPT (1 : ON/ 0: OFF) : OFF |  | - To assign DVU Announce 2 Repeat E/D, choose flex BTN 6 and enter desired value. To save, go step (7). |
| CIRC 624 MUSIC SRC (00 – 12) : 00 |  | - To assign Music Source, choose flex BTN 10 and enter two digits. To save, go step (7). |
| RING GRP 624 PRESS FLEX_KEY (1-10) |  | - Press the [HOLD/SAVE] button for updating database permanently. For another group’s setting, first assign type of certain group number in PGM 190 and go step (1) and continue this process. |
|  | RING GRP 624 PRESS FLEX_KEY (1-10) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | DVU Announce 1 Timer | 000-999 | 015 | If this timer expires after call come in the group, the system announces the greeting if exists. (1 Sec Base) |
| 2 | DVU Announce 2 Timer | 000-999 | 000 | If this timer expires after call come in the group, the system announces the DVU if assigned. (1 Sec Base) |
| 3 | DVU Announce Location 1 | 00-70 | 00(not_asgn) | This is used to announce greeting when the DVU announce 1 timer is expired. |
| 4 | DVU Announce Location 2 | 00-70 | 00(not_asgn) | This is used to announce DVU when the DVU announce 2 timer is expired. |
| 5 | DVU Announce 2 Repeat | 000-999 | 000 | This is used to announce DVU announce 2 when the timer is expired. (1 Sec Base) |
| 6 | DVU Announce 2 Repeat E/D | ON/OFF | OFF | This is used to enable or disable DVU Announce 2 Repeat. |
| 7 | Overflow Destination | Sta#./ HUNT#/ DVU#. |  | The call to a station in the group will continue to route until answered or each station in the group has been tried. The call will remain at the last station in the group or will be passed to this overflow station/group. |
| 8 | Overflow Timer | 000-600 | 180 | If this timer expires after a call comes in the group, the call is routed to the overflow destination. (1 Sec Base) |
| 9 | Wrap Up Timer | 002-999 (3 digits) | 002 | A station in a hunt group is maintained in a busy state for a minimum of six seconds after any call for the assigned wrap-up time. (1 Sec Base) |
| 10 | Music Source | 00-12 | 00 | If music source is assigned, calling user will be heard music instead of ring back tone. 00: No Asgn 01: Internal Music 02: External Music 1 03: External Music 2 04: External Music 3 05: VMIB BGM 1 06: VMIB BGM 2 07: VMIB BGM 3 08: SLT 1 09: SLT 2 10: SLT 3 11: SLT 4 12: SLT 5 |

TABLE 7.2.3.1  Button Configuration for Ring Group Attribute (PGM 191)

### 7.2.4  VM GROUP ATTRIBUTE  (PGM 191)

PROCEDURE

| STATION GRP ASSIGN ENTER GROUP NUMBER | - [TRANS/PGM] + 191. - Note : To program this, first, assign VM group to a certain group number |
| --- | --- |
| VM GRP 626 PRESS FLEX_KEY (1-7) | - Enter Station Group Number (Ex 626, If user set 626 to VM group). Choose one of 7 flex BTNs and enter desired value of the group feature. |
| VM 626 WRAP UP TMR (006 – 999) : 002 | - To assign Wrap-Up Timer, choose flex BTN 1 and enter three digits. To save, go step (8). |
| VM 626 PUT MAIL INDEX (1 – 4) : 1 | - To assign Put Mail Index, choose flex BTN 2 and enter one digit. (1-4) To save, go step (8). |
| VM 626 HUNT TYPE (1 : CIRC/ 0 : TERM) : TERM | - To assign Hunt Type, choose flex BTN 4 and enter desired value. To save, go step (8). |
| VM 626 OVERFLOW DEST STA/HUNT/VMIB (D1-D3) | - To assign Overflow Destination, choose flex BTN 7 and enter one digit. - Dial ‘1’ Enter station number, - Dial ‘2’ Enter Hunt group number, - Dial ‘3’ Enter Dvn number - Press the [SPEED] button for deleting. To save, go step (8). |
| VM 626 OVERFLOW DEST STA 147 | - For example, dial 1 and enter number 147. To save, go step (8). |
| VM GRP 626 PRESS FLEX_KEY (1-7) | - Press the [HOLD/SAVE] button for updating database permanently. For another group’s setting, first assign type of certain group number in PGM 190 and go step (1) and continue this process. |
| VM GRP 626 PRESS FLEX_KEY (1-7) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Wrap-Up Timer | 002-999 (3 Digits) | 002 | A station in a hunt group is maintained in a busy state for a minimum of six seconds after any call and for hunt group calls for the assigned wrap-up time. (1 Sec Base) |
| 2 | Put Mail Index | 1 -4 | 1 | This index is one of the voice mail dialing table |
| 3 | Get Mail Index | 1 -4 | 2 | This index is one of the voice mail dialing table |
| 4 | Hunt Type | CIRC /TERM | TERM | 1: Circular Hunt Group 0: Terminal Hunt Group |
| 5 | SMDI Port | 01-13 | 02(COM2) |  |
| 6 | Overflow Timer | 000 -600 (3 Digits) | 180 | If this timer expires after a call comes in the group, the call is routed to the overflow destination. (1 Sec Base) |
| 7 | Overflow Destination | Sta #/ HUNT #/ DVU# | - | The call to the group will continue to be reroute until reaching the last station in the group where the call will remain or can be sent to this overflow station/group. |

TABLE 7.2.4.1  Button Configuration for Voice Mail Group Attribute (PGM 191)

### 7.2.5  PICK_UP GROUP ATTRIBUTE  (PGM 191)

| STATION GRP ASSIGN ENTER GROUP NUMBER | - [TRANS/PGM] + 191. - Note : To program this, first, assign Pick Up group to a certain group number . |
| --- | --- |
| PICK UP GRP 625 PRESS FLEX_KEY (1-2) | - Enter Station Group Number (Ex 625, If user set 625 to Pick Up group). Choose one of 2 flex BTNs and enter desired value of the group feature. |
| PICK UP 625 AUTO PICK UP (1 : ON/ 0: OFF) : OFF | - To assign Auto Pick up, choose flex BTN 1 and enter desired value. To save, go step (5). |
| PICK UP 625 ALL RING (1 : ON/ 0: OFF) : OFF | - To assign Auto Ring, choose flex BTN 2 and enter desired value. To save, go step (5). |
| PICK UP GRP 625 PRESS FLEX_KEY (1-2) | - Press the [HOLD/SAVE] button for updating database permanently. For another group’s setting, first assign type of certain group number in PGM 190 and go step (1) and continue this process. |
| PICK UP GRP 625 PRESS FLEX_KEY (1-2) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT(LED) | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Auto Pickup | ON/OFF | OFF | If a hunt member is ringing, another hunt member can pickup automatically only press [MON] or off-hook. |
| 2 | All Ring | ON/OFF | OFF | When a hunt member that is TONE mode is ringing, all the other stations are ringing also. Auto Pickup feature must be set before All Ring is set. |

TABLE 7.2.5.1   Button Configuration for Pick Up Group Attribute  (PGM 191)

### 8.	ISDN PROGRAM

If ISDN system parameters are to be changed, press the [TRANS/PGM] button and dial 200-202. When programming, LCD and LEDs indicate current programmed data and programming status. The programmer enters correct data, then LCD and LEDs show the entered data and the data is stored in the temporary buffer area. After all, press the [HOLD/SAVE] button, all data in the temporary buffer (same as LCD/LEDs show their status) are stored into permanent ISDN memory.

## 8.1	ISDN ATTRIBUTE  (PGM 200)

PROCEDURE

| SYSTEM ISDN ATT PRESS FLEX KEY (1-10) | - [TRANS/PGM] + 200. If user wants to program the ISDN system attributes press flexible button between 1-7. It will be presented the status of the related field. |
| --- | --- |
| - ADVICE OF CHARGE (0-5) - FINLAND(2) | - To program AOC type, press the 1st flexible BTN and dial one digit, Then the 2nd LCD line message will be changed to the related field current status, like left LCD message example. |
| - CO ATD CODE (2DGT) - . . | - To program the CO attendant code, press the 2nd flexible BTN and dial maximum 2 digits.. |
| - IN PREFIX CODE INS - (1:ON/0:OFF) : OFF | - To program the incoming prefix code insertion, press the 3rd flexible BTN. Then the 2nd LCD line message will be changed to the related field current status, like left LCD message example. |
| - ISDN LINE TYPE - (1:U/0:A): A_LAW | - To program the outgoing prefix code insertion, press the 5th flexible BTN. And dial 0 or 1. 1: -LAW 0: A-LAW |
| - INTERNATION ACC CODE 001 | - To program the International Access Code, press the 7th flexible BTN. Then the 2nd LCD line message will be changed to the related field current status, like left LCD message example. To change the data dial digits(Max4). |
| SYSTEM ISDN ATT - PRESS FLEX KEY (1-10) | - Press the [HOLD/SAVE] button for updating database permanently. |
| SYSTEM ISDN ATT - PRESS FLEX KEY (1-10) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Advice of Charge | 0-5 | - 0 | 0: Do not service AOC 1: Italy and Spain 2: Finland 3: Australia 4: Belgium 5: Standard |
| 2 | CO ATD Code | MAX 2 Digits | - - | According to PGM114 - Btn5, CO ATD code or Extension number can be contained to CLI, COLP message |
| 3 | Incoming prefix code Insertion | ON/OFF | OFF(NO) | If this field is ON, prefix code at will be attached in front of incoming phone number. |
| 4 | Outgoing prefix code Insertion | ON/OFF | ON(YES) | If this field is ON, prefix code will be attached in front of outgoing phone number. |
| - 5 | - ISDN Line Type | -Law/ A-Law | A-Law (OFF) | Installed ISDN Back bone type |
| 6 | CLI print | ON/OFF | OFF(NO) | If this field is ON, send the CLI to RS-232C port regardless setting the CLIP |
| - 7 | - International Access Code | MAX 4 Digits | - | International Access Code Assign |
| - 8 | - Calling Sub-address | ON/OFF | OFF(NO) |  |
| - 9 | - My Area Code | MAX 6 Digits | - | Local area code. |
| - 10 | - My Area Prefix Code | MAX 4 Digits | - | Prefix code of local area code. |

TABLE 8.1.1  ISDN Attributes (PGM 200)

## 8.2	COLP TABLE  (PGM 201)

PROCEDURE

| COLP TABLE ENTRY ENTER BIN NO ( 00 – 49 ) | - [TRANS/PGM] + 201. |
| --- | --- |
| - COLP TABLE 05 - ………. | - To program COLP table, dial Bin No ( 00 – 49 ). |
| COLP TABLE ENTRY - ENTER BIN NO ( 00 – 49 ) | - Press the [HOLD/SAVE] button for updating database permanently. |
| COLP TABLE ENTRY - ENTER BIN NO ( 00 – 49 ) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | DEFAULT | RANGE | REMARK |
| --- | --- | --- | --- | --- |
|  | COLP | - - | MAX 10 digits | In CO PGM part, PGM 143 BTN1,BTN3 indicate an entry of this COLP table index. |

TABLE 8.2.1  COLP Table (PGM 202)

## 8.3	MSN TABLE  (PGM 202)

PROCEDURE

| MSN TABLE ATT ENTER BIN NO ( 000 – 249 ) | - [TRANS/PGM] + 202 |
| --- | --- |
| - MSN TABLE 125 - PRESS FLEX_KEY ( 1 – 4 ) | - To enter MSN table, dial digits (Ex. 125). |
| - MSN TABLE 125 - COL NO : 001 | - To enter bin CO line number, press Flex. BTN1 and dial digits (Ex. 001). |
| MSN TABLE 125 INDEX : 100 | - To enter index number of the PGM 231, press Flex. BTN2, and dial index number (Ex. 100). |
| - MSN TABLE 125 - SUB NO : 1 | - To enter Sub-address number, press Flex. BTN3, and dial Sub-address number (Ex. 1). |
| TABLE 125 MSN NUMBER …………………….. | - To enter Incoming MSN telephone number, press Flex. BTN4 and dial MSN that is provided by PX. |
| MSN TABLE ATT ENTER BIN NO ( 000 – 249 ) | - Press the [HOLD/SAVE] button for updating database permanently. |
| MSN TABLE ATT ENTER BIN NO ( 000 – 249 ) | - Press [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN |  | ITEM |  | RANGE |  | DEFAULT |  | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - 1 | - CO Line No. |  | - 001-200 |  | - None |  |  |  |
| - 2 | - Index of Flexible - DID Table |  | - 000-999 |  | - None |  | - If Incoming Col no and MSN number or MSN number are matched with Table entry, follow assigned Flex DID Table |  |
| - 3 | - Sub Number |  | - 0-9 |  | - None |  | - MSN Subscriber number |  |
| - 4 | - MSN Number |  | - 20 Digits |  | - None |  | - ISDN Incoming MSN number. |  |

- TABLE 8.3.1  Button Configuration of MSN Table  (PGM 202)