- PROGRAMMING MANUAL

- of LDK-300

Nov. 2000

CONTENTS

1.	CUSTOMER DATABASE PROGRAMMING	1

1.1	INTRODUCTION	1

1.2	TO ENTER THE PROGRAMMING MODE	1

1.3	PERMANENT UPDATE PROCEDURE	3

1.4	NUMBERING PLAN	3

1.5	ADMIN PROGRAMMING INDEX	7

1.6	DEFAULT VALUES	10

TABLE 1.6.1  Location Information	10

TABLE 1.6.2  RACK Slot Assignment	10

TABLE 1.6.3  WTIB Port Number Assignment	10

TABLE 1.6.4  Logical Slot Assignment	10

TABLE 1.6.5  Numbering Plan Type	10

TABLE 1.6.6  FLEXIBLE NUMBERING PLAN	12

TABLE 1.6.7   STATION ID ASSIGNMENT	14

TABLE 1.6.8   STATION ID ASSIGNMENT	14

TABLE 1.6.9   STATION ATTRIBUTE I/II/III	14

TABLE 1.6.10   ISDN STATION ATTRIBUTE	15

TABLE 1.6.11   FLEXIBLE BUTTONS PROGRAM	15

TABLE 1.6.12   STATION BASE PROGRAM	17

TABLE 1.6.13  CO LINE BASE PROGRAM (1/2)	18

TABLE 1.6.14  CO LINE BASE PROGRAM (2/2)	19

TABLE 1.6.15   SYSTEM BASE PROGRAM (1/4)	20

TABLE 1.6.16   SYSTEM BASE PROGRAM (2/4)	21

TABLE 1.6.17   SYSTEM BASE PROGRAM (3/4)	22

TABLE 1.6.18   SYSTEM BASE PROGRAM (4/4)	23

TABLE 1.6.19  SYSTEM TIMER PROGRAM   (1/2)	24

TABLE 1.6.20  SYSTEM TIMER PROGRAM   (2/2)	25

TABLE 1.6.21  STATION GROUP ASSIGN	26

TABLE 1.6.22    STATION GROUP PROGRAM  (1/3)	26

TABLE 1.6.23    STATION GROUP PROGRAM  (2/3)	27

TABLE 1.6.24    STATION GROUP PROGRAM  (3/3)	28

TABLE 1.6.25   ISDN ATTRIBUTES	28

TABLE 1.6.26   LCR TABLE ASSIGNMENT  (1/2)	29

TABLE 1.6.27    LCR TABLE ASSIGNMENT (2/2)	30

TABLE 1.6.28   TOLL TABLE ASSIGNMENT	30

TABLE 1.6.29   OTHER TABLES	31

TABLE 1.6.30    NATION SPECIPIC (1/5)	32

TABLE 1.6.31    NATION SPECIPIC (2/5)	33

TABLE 1.6.32    NATION SPECIPIC (3/5)	34

TABLE 1.6.33    NATION SPECIPIC (4/5)	오류! 책갈피가 정의되어 있지 않습니다.

TABLE 1.6.34    NATION SPECIPIC (5/5)	35

TABLE 1.6.35    INITIALIZATION	35

TABLE 1.6.36    PRINT PROT DATABASE	36

# 1.	CUSTOMER DATABASE PROGRAMMING

## 1.1	INTRODUCTION

The LDK Key Telephone System can be programmed to meet each customer's individual need. All programming is done at station 100 (station port # 00) using KD-36D or LKD-30/44 digital key telephone.

Upon entering the program mode, the key telephone at station 100 cannot operate as a normal telephone but as a programming instrument with all of the buttons redefined. The keys of the dial pad are used to enter the various data fields and to enter numerical information. The 24 buttons located at the top of the phone (Flexible Buttons) are used to indicate the specific data field and to enter information. Sometimes the [SPEED] button and ‘*’ of the dial pad is used to delete the data or to indicate end of data input and [REDIAL] button is used to delete one digit or character from the end of entered digits or chars.

See TABLE 1.6.1 - 1.6.35 for default data. If this pre-programming suits the customer, additional admin program is not necessary. To change admin data, the user enters the admin. programming mode and select program code. During admin programming other keysets operate normally.

When Admin programming, LCD and LEDs indicate the current programmed data and status. If the programmer enters correct data, then LCD and LEDs show the entered data and the data is stored in the temporary buffer area. Real system databases is not changed and has no effect on telephone operation unless permanent updating procedure is executed. Pressing [HOLD/SAVE] button, all data in the temporary buffer (same as LCD and LEDs show their status) is saved into permanent memory. Tones are provided to let the programmer know data entry is correct (confirmation tone) or not (error tone).

If programmer wants to parent state, press [CONF] button. When pressing [CONF] button temporary data fields are cleared.

If programmer wants to reset System, Enter PGM450 - Btn14 and press [HOLD/SAVE] button or System will be reset automatically after PGM100 – Btn1(Nation Code Assign)

## 1.2	TO ENTER THE PROGRAMMING MODE

1. 	Lift handset or press the [MON] button on the admin station, and hear ICM dial tone (optional).

- Press the [TRANS/PGM] button and dial ＊# (Confirmation tone is heard).

- Enter Admin password if the password has been set. This places the station into the admin programming mode (confirmation tone heard).

Each program is accessed by pressing the [TRANS/PGM] button and dialing the three-digit program number. If an error is made while entering data, the [TRANS/PGM] button can go the previous status. When the [TRANS/PGM] button is pressed, the LCD will display;

ENTER PROGRAM CODE

## 1.3	PERMANENT UPDATE PROCEDURE

When the data has been entered, the [HOLD/SAVE] button is used to store the data permanently. If all data was entered correctly, confirmation tone is heard when pressing the [HOLD/SAVE] button. If there were any errors in the entry, then an error tone is presented and data is not stored in the permanent memory.

## NUMBERING PLAN

The following numbering plan can be changed by Admin Programming 104-107 depending on the user's needs.

| NUMBER | ITEM | REMARK |
| --- | --- | --- |
| 100–399 | Station Number |  |
| 620–667 | Station Group Number |  |
| 501-535 | Internal Page Zones |  |
| 543 | Internal All Call Paging |  |
| 544 | Meet Me Page |  |
| 546 | External Page Zone –1 |  |
| 547 | External Page Zone –2 |  |
| 548 | External All Call Paging |  |
| 549 | All Call Paging |  |
| 550 | Enter SMDR Account Code | SLT |
| 551 | Flash Command to CO Line |  |
| 552 | Last Number Redial (LNR) |  |
| 553 | Do Not Disturb |  |
| 554 | Call Forward |  |
| 555 | Station Speed Dial |  |
| 556 | Message Wait Enable |  |
| 557 | Message Wait Answer |  |
| 558 | Speed Dial Access |  |
| 559 | Cancel DND/Call Forward/Pre-selected MSG |  |
| 563 | PGM Enter Code | SLT |
| 565 | Alarm Reset |  |
| 566 | Group Call Pick-up |  |
| 569 | Universal Night Answer (UNA) |  |
| 601-619 | Call Park Location |  |
| 7XXX | Direct Call Pick-up (XXX = Station Number) |  |
| 8XX | CO Group Access |  |
| 88XXX | CO Line Access |  |
| 89 | Tie Routing Access |  |
| 8* | Retrieve Holding CO Group |  |
| 8#XXX | Retrieve Holding CO LIne |  |

| NUMBER | ITEM | REMARK |
| --- | --- | --- |
| 9 | Access the first CO line |  |
| 0 | Attendant Call |  |
| *0 | Print SMDR (Station Base) | Attendant |
| *1 | Print SMDR (Account Group Base) | Attendant |
| *2 | Delete SMDR (Station Base) | Attendant |
| *3 | Delete SMDR (Account Group Base) | Attendant |
| *4 | Display Call Charge on LCD | Attendant |
| *8 | VM Message Wait Enable |  |
| *9 | VM Message Wait Cancel |  |
| ** | Abort SMDR Printing | Attendant |
| #*1 | Door Open 1 |  |
| #*2 | Door Open 2 |  |
| #*3 | Door Open 3 |  |
| #*4 | Door Open 4 |  |
| [SPEED] + * | Last Number Redial |  |
| [SPEED] + # | Save Number Redial | Keyset |
| [SPEED] + 00-19 | Station Speed Dial Access | Keyset |
| [SPEED] + XXXX | System Speed Dial Access | XXXX : 2000 - 4999 (Bin No. of System Speed Dial) |

- To enter user programming mode, press the [TRANS/PGM] button in a keyset or dial 563 (Programming enter code) in a SLT.

- The following numbering plan is fixed, cannot be changed by Admin. Programming.

1) For the stations

| NUMBER | ITEM | REMARK |
| --- | --- | --- |
| 11 | Differential Ring | Keyset |
| 12 | Intercom Answer Mode (1 HF / 2 TONE / 3 PV) | Keyset |
| 21 | Station COS Change |  |
| 22 | Station COS Restore |  |
| 23 | Walking COS | Keyset |
| 31 | Authorization Code Registration |  |
| 32 | Authorization Code Change |  |
| 41 | Wake-up Time Registration (One-time/ Continuous) |  |
| 42 | Wake-up Time Cancel |  |
| 51 | Pre-selected MSG Activation | " |
| 52 | Set Custom Message |  |
| 61 | Record DVU User Greeting |  |
| 62 | Listen DVU Time & Date |  |
| 63 | Listen DVU Station Number |  |
| 64 | Listen DVU Station Status |  |
| 65 | Record DVU Page Message |  |
| 66 | Erase DVU User Greeting |  |
| 67 | Erase DVU Page Message |  |
| 71 | LCD Display Mode (English/Korean) | Keyset |
| 72 | MPB Version Display | Keyset |
| 73 | Background Music | Keyset |
| 74 | Station User Name Registration |  |
| 75 | Headset/Speakerphone Mode | Keyset |
| 78 | Serial No/SW Packages | Keyset w/ LCD |

2) For the attendant

| NUMBER | ITEM |  | REMARK |
| --- | --- | --- | --- |
| 0111 | Print SMDR (Station Base) |  | System Attendant |
| 0112 | Delete SMDR (Station Base) |  | System Attendant |
| 0113 | Print SMDR (Group Base) |  | System Attendant |
| 0114 | Delete SMDR (Group Base) |  | System Attendant |
| 0115 | Abort Printing |  | System Attendant |
| 0121 | Start Traffic Analysis | System Attendant |  |
| 0122 | Disable Traffic Analysis | System Attendant |  |
| 0123 | Print Traffic Analysis | System Attendant |  |
| 0124 | Abort Traffic Analysis | System Attendant |  |
| 021 | Station COS Change (COS 7) |  | Attendant |
| 022 | Station COS Restore |  | Attendant |
| 031 | Authorization Code Cancel |  | System Attendant |
| 041 | System Date/Time Setting |  | Attendant |
| 042 | Wake-up Time Registration (One-time /Continuous) |  | Attendant |
| 043 | Wake-up Time Cancel |  | System Attendant |
| 044 | LCD Date Mode Change |  | System Attendant |
| 045 | LCD Time Mode Change |  | System Attendant |
| 046 | Use PX Time & Date |  | System Attendant |
| 051 | Pre-select MSG Activation |  | Attendant |
| 052 | Pre-select MSG Deactivation |  | Attendant |
| 053 | Custom Display Message Program (11-20) |  | System Attendant |
| 054 | Erase VM MSG |  | Attendant |
| 06 | Record DVU System Greeting | System Attendant |  |
| 071 | DND/Call Forward/Pre-selected MSG Cancel |  | Attendant |
| 072 | Register Station Name |  | Attendant |
| 073 | Disable CO Outgoing |  | System Attendant |
| 074 | Automatic Day/Night Mode Program | Attendant |  |
| 075 | Door Phone Music Selection |  | Attendant |
| 076 | External Page Music -1 Assignment/Cancel | Attendant |  |
| 077 | External Page Music -2 Assignment/Cancel | Attendant |  |

3) Flex button programming codes

| NUMBER | ITEM |  | REMARK |
| --- | --- | --- | --- |
| 54 | [Record Conversation] BTN Assignment – With Voice Mail |  | Keyset |
| 55 | Restrict DID Ring |  | N/A |
| 56 | Restore DID Ring |  | N/A |
| 80 | [Account Code] BTN Assignment |  |  |
| 83 | [ICM Hold] BTN Assignment |  |  |
| 84 | [LOOP] BTN Assignment |  |  |
| 85 | [Camp-on] BTN Assignment |  |  |
| 86 | [ATD Override] BTN Assignment |  | System Attendant |
| 87 | [UCD DND] BTN Assignment |  | Member of UCD Group Only |
| 89 | Keypad Facility |  |  |
| 8* | [ACD] BTN Assignment |  | Keyset (Supervisor of UCD Group Only) |
| 91 | [CONF] BTN Assignment |  | 2/8 BTN Keyset |
| 92 | [CALLBK] BTN Assignment |  | 2/8 BTN Keyset |
| 93 | [DND/FWD] BTN Assignment |  | 2/8 BTN Keyset |
| 94 | [FLASH] BTN Assignment |  | 2/8 BTN Keyset |
| 95 | [MUTE] BTN Assignment |  | 2/8 BTN Keyset |
| 96 | [MON] BTN Assignment | 2/8 BTN Keyset |  |
| 97 | [REDIAL] BTN Assignment | 2/8 BTN Keyset |  |

## 1.5	ADMIN PROGRAMMING INDEX

| MAIN MENU | PGM | ITEM |
| --- | --- | --- |
| PRE-PROGRAMMED | 100 | Location Information |
|  | 101 | Slot Assignment |
|  | 102 | WTIB Port number Assign |
|  | 103 | Logical Slot Assignment |
|  | 104 | Numbering Plan Type |
|  | 105 | Flexible Number Plan – Station Number |
|  | 106 | Flexible Number Plan A |
|  | 107 | Flexible Number Plan B |
|  | 108 | IP Setting |
| STATION BASE PROGRAM | 110 | Station ID Assignment |
|  | 111 | Station Attribute I |
|  | 112 | Station Attribute II |
|  | 113 | Station Attribute III |
|  | 114 | ISDN Station Attribute |
|  | 115 | Flex Buttons Assignment |
|  | 116 | Station COS |
|  | 117 | CO Line Group Access |
|  | 118 | Internal Page Zone Access |
|  | 119 | Conference Page Zone |
|  | 120 | ICM Tenancy Group |
|  | 121 | Preset Call Forward |
|  | 122 | Hot/Warm Line Selection |
|  | 123 | CTI Attribute |
|  | 124 | SMDR Account Group |
| CO LINE BASE PROGRAM | 140 | CO Service Type |
|  | 141 | CO Line Attribute I |
|  | 142 | CO Line Attribute II |
|  | 143 | ISDN CO Line Attribute |
|  | 144 | CO Ring Assignment |
| SYSTEM BASE PROGRAM | 160 | System Attribute I |
|  | 161 | System Attribute II |
|  | 162 | ADMIN Password |
|  | 163 | Alarm Attributes |
|  | 164 | Attendant Assignment |
|  | 165 | Auto Attendant DVU Annc.# |
|  | 166 | CO-to-CO COS |
|  | 167 | DID/DISA Destination |
|  | 168 | External Control Contact |
|  | 169 | LCD Date/Time/Language Display Mode |
|  | 170 | Modem |
|  | 171 | Music |
|  | 172 | PBX Access Code |
|  | 173 | PLA Priority |
|  | 174 | RS-232C Port Setting |
|  | 175 | Print Serial Port Selection |
|  | 176 | Pulse Dial / Speed Ratio |
|  | 177 | SMDR Attributes |
|  | 178 | System Date/Time |
|  | 179 | Linked Station Pairs Table |
|  | 180 | System Timers – I |
|  | 181 | System Timers – II |
|  | 182 | System Timers – III |
| STATION GROUP | 190 | Station Group Assign |
|  | 191 | Station Group Attribute Assign |
| ISDN SYSTEM BASE PROGRAM | 200 | ISDN Attributes 1 |
|  | 201 | COLP Table |
|  | 202 | MSN Table |
| TABLES | 220 | LCR Assignment |
|  | 221 | LCR - LDT Table |
|  | 222 | LCR - DMT Table |
|  | 223 | LCR Table Initialization |
|  | 224 | Toll Exception Table – Allow A (Entry no:01-30) |
|  |  | Toll Exception Table – Deny A (Entry no:01-30) |
|  |  | Toll Exception Table – Allow B (Entry no:01-30) |
|  |  | Toll Exception Table – Deny B (Entry no:01-30) |
|  | 225 | Canned Toll Table –Allow (Entry no:01-10) |
|  |  | Canned Toll Table –Deny (Entry no:01-10) |
|  | 226 | Emergency Code Table (Entry no:01-10) |
|  | 227 | Authorization Code Table (Entry no:001-600) |
|  | 228 | Customer Call Routing |
|  | 229 | Executive/Secretary Table |
|  | 230 | DID Digit Conversion Table |
|  | 231 | Flexible DID Table |
|  | 232 | System Speed Zone |
|  | 233 | Weekly Time Table |
|  | 234 | Voice-Mail Dialing Table |
|  | 235 | Tie Routing Table |
| Hotel | 300 |  |
| Q-SIG | 320 |  |
| VOIP | 340 |  |
| NATION SPECIFIC | 400 | DTIB Rx Gain Control |
|  | 401 | SLIB Rx Gain Control |
|  | 402 | SLIB12 Rx Gain Control |
|  | 403 | WTIB Rx Gain Control |
|  | 404 | ACOB Rx Gain Control |
|  | 405 | ACOB8 Rx Gain Control |
|  | 406 | DCOB Rx Gain Control |
|  | 407 | VMIB Rx Gain Control |
|  | 408 | DTRU Rx Gain Control |
|  | 409 | EXT Pager Rx Gain Control |
|  | 410 | CPTU Rx Gain Control |
|  | 411 | MODU Rx Gain Control |
|  | 418 | PTRP Rx Gain Control |
|  | 419 | PTRP Rx Gain Control |
|  | 420 | SYSTEM Tone Frequency |
|  | 421 | Differential Ring Frequency |
|  | 422 | Distinct CO Ring Frequency |
|  | 423 | ACNR Tone Cadence |
|  | 424 | Tie Routing Table |
|  | 425 | Singular Table(Korea Only) |
| INITIALIZATION (DB INIT) | 450 | Flexible Numbering Plan Initialization |
| PRINT DATABASE | 451 | Flexible Numbering Plan Print |

## 1.6	DEFAULT VALUES

### TABLE 1.6.1  Location Information

| PGM | BTN | ITEM | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 100 | 1 | Nation Code | 82 | Max 4 digits |
|  | 2 | Customer Site Name | . | Max 23 digits |

### TABLE 1.6.2  RACK Slot Assignment

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 101 | - | Slot Assignment | Refer to note.2 | Refer to note.1 | In case of PRIB assignment, programming of logical port no. is possible. |

Note.1) If DIP switch of the manual board detection is OFF, system will detect the installed board type automatically. If DIP switch is ON, the board type code must be entered at each slot. After manual RACK slot assignment, user must reset the system manually.

Note.2) Board Type Code Table:

| STA | CODE | COL | CODE | STA & COL | CODE | Etc | CODE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DTIB12 DTIB24 SLIB6 SLIB12 WTIB DSIB | 11 12 13 14 15 18 | PRIB BRIB LCOB4 LCOB8 TLIB EMIB | 31 32 33 34 37 38 | STIB | 51 | VMIB | 61 |

### TABLE 1.6.3  WTIB Port Number Assignment

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 102 | - | WTIB Port Number Assignment | 008 – 192 (Multiple of 8) | 8 |  |

### TABLE 1.6.4  Logical Slot Assignment

| PGM | BTN | ITEM | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 103 | 1 | COL Board | Refer to note.1 |  |
|  | 2 | STA Board | Refer to note.1 |  |
|  | 3 | VMIB | Not Assigned |  |

N) If DIP switch of the manual board detection is OFF, system will detect the logical slot assign sequence as increase order automatically. If DIP switch is ON, the logical slot assignment must be entered at each board type. After manual logical slot assignment, user must reset the system manually.

### TABLE 1.6.5  Numbering Plan Type

| PGM | BTN | ITEM | INTERCOM RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 104 | 1 | Number Set Type 1 | 100 – 399 | Yes | As the basic type, the 1st digit of station number should be 1 – 4. |
|  | 2 | Number Set Type 2 | 100 – 399 | No | The station number can be changed within 799. |
|  | 3 | Number Set Type 3 | 100 – 399 | No | Australia Default |
|  | 4 | Number Set Type 4 | 7700-7999 | No | New Zealand Default |
|  | 5 | Number Set Type 5 | 200 – 499 | No | Italy Default |
|  | 6 | Number Set Type 6 | 21 – 79 | No | Finland Default Max Station Ports 60 Stations above Max ports will be dispalyed “***” |
|  | 7 | Number Set Type 7 | 10 – 79 | No | Max Station Ports 70 Stations above Max ports will be dispalyed “***” |
|  | 8 | Number Set Type 8 | 100 – 399 | No | The station number can be changed within 999. |

### TABLE 1.6.6  FLEXIBLE NUMBERING PLAN

| PGM | BTN |  | FIELD |  | NUMBER SET1 | NUMBER SET2 | NUMBER SET3 | NUMBER SET4 | NUMBER SET5 | NUMBER SET6 | NUMBER SET7 | NUMBER SET8 | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 105 |  | - |  | Intercom Call | 100 - 399 | 100 – 399 (100 ~ 799) | 100 – 399 | 7700 – 8073 (7000 ~ 8999) | 200 – 499 | 21-79 | 10 - 79 | 100 – 399 (100 ~ 999) |  |
| 106 |  | 1 |  | Group Pilot Number | 620 - 667 | *620 - *667 | 620 – 667 | 9700-9747 | 620 – 667 | *620 - *667 | *620 - *667 | *620 - *667 |  |
|  |  | 2 |  | Internal Page Zone 1-35 | 501 – 535 | *501 – *535 | #01-#35 | #01-#35 | *501 – *535 | *501 – *535 | *501 – *535 | *501 – *535 |  |
|  |  | 3 |  | Internal All Call Page | 543 | *543 | #5 | #3 | 673 | *543 | *543 | *543 |  |
|  |  | 4 |  | Meet Me Page | 544 | *544 | ## | ## | 674 | *544 | *544 | *544 |  |
|  |  | 5 |  | External Page Zone 1 | 545 | *545 | #6 | #41 | 675 | *545 | *545 | *545 |  |
|  |  | 6 |  | External Page Zone 2 | 546 | *546 | #7 | #42 | 676 | *546 | *546 | *546 |  |
|  |  | 7 |  | External Page Zone 3 | 547 | *547 | #8 | #43 | 677 | *547 | *547 | *547 |  |
|  |  | 8 |  | External All Call Page | 548 | *548 | #9 | #5 | 678 | *548 | *548 | *548 |  |
|  |  | 9 |  | All Call Page (Int & Ext) | 549 | *549 | #00 | #6 | 679 | *549 | *549 | *549 |  |
|  |  | 10 |  | SMDR Account Code Enter | 550 | *550 | 550 | 91 | 680 | *550 | *550 | *550 | SLT |
|  |  | 11 |  | Flash Command to CO Line | 551 | *551 | 551 | 551 | 681 | *551 | *551 | *551 | SLT |
|  |  | 12 |  | Last Number Redial | 552 | *552 | 552 | 552 | 682 | *552 | *552 | *552 | SLT |
|  |  | 13 |  | DND (Toggle On/Off) | 553 | *553 | 553 | 553 | 683 | *553 | *553 | *553 | SLT |
|  |  | 14 |  | Call Forward | 554 | *554 | 554 | 554 | 684 | *554 | *554 | *554 | SLT |
|  |  | 15 |  | Speed Dial Programming | 555 | *555 | 555 | *40 | 685 | *555 | *555 | *555 | SLT |
|  |  | 16 |  | Message Wait/Callback Enable | 556 | *556 | 556 | *66 | 686 | *556 | *556 | *556 |  |
|  |  | 17 |  | Message Wait/Callback Return | 557 | *557 | 557 | *67 | 687 | *557 | *557 | *557 | SLT |
|  |  | 18 |  | Speed Dial Access | 558 | *558 | 558 | *9 | 688 | *558 | *558 | *558 | SLT |
|  |  | 19 |  | Cancel DND/FWD/Pre-MSG | 559 | *559 | 559 | 559 | 689 | *559 | *559 | *559 | SLT |
|  |  | 20 |  | System Hold | 560 | *560 | 560 | 560 | 690 | *560 | *560 | *560 | SLT |
|  |  | 21 |  | Station Relocation Backup | 561 | *561 | 561 | 561 | 691 | *561 | *561 | *561 |  |
|  |  | 22 |  | Station Relocation Retrieve | 562 | *562 | 562 | 562 | 692 | *562 | *562 | *562 |  |
|  |  | 23 |  | Programming Mode Enter Code | 563 | *563 | 563 | 563 | 1 | *3 | *3 | *3 | SLT |
|  |  | 24 |  | Alarm Reset | 565 | *565 | 565 | *565 | 695 | *565 | *565 | *565 |  |
| 107 |  | 1 |  | Group Call Pickup | 566 | *566 | ** | *1 | 696 | *566 | *566 | *566 |  |
|  |  | 2 |  | UCD DND | 568 | *568 | 568 | 568 | 698 | *568 | *568 | *568 |  |
|  |  | 3 |  | Night Answer | 569 | *569 | 577 | 2 | 699 | *569 | *569 | *569 |  |
|  |  | 4 |  | Call Parking Locations | 601 - 619 | *601 -* 619 | 601 – 619 | 950 – 968 | 601 - 619 | *601 -* 619 | *601 -* 619 | *601 -* 619 |  |
|  |  | 5 |  | Direct Call Pickup | 7 | *7 | *7 | *42 | 7 | *7 | *7 | *7 |  |
|  |  | 6 |  | CO Group Access (Group 01 - 72) | 8xx | 8xx | 8xx | 4xx | 8xx | 8xx | 8xx | #8xx |  |
|  |  | 7 |  | Individual CO Access (01 - 200) | 88xxx | 88xxx | 88xxx | 48xxx | 88xxx | 88xxx | 88xxx | #88xxx |  |
|  |  | 8 |  | Tie Routing Access | 8901 | 8901 | 8901 | 4901 | 8901 | 8901 | 8901 | *401 |  |
|  |  | 9 |  | Retrieve Held CO Line | 8* | 8* | 8* | 4* | 8* | 8* | 8* | #8* |  |
|  |  | 10 |  | Retrieve Held Individual CO Line | 8#xxx | 8#xxx | 8#xxx | 4#xxx | 8#xxx | 8#xxx | 8#xxx | #8#xxx |  |
|  |  | 11 |  | Access CO Line In the 1st available CO Group | 9 | 9 | 9 | 1 | 0 | 9 | 9 | #9 |  |
|  |  | 12 |  | Attendant Call | 0 | 0 | 0 | 0 | 9 | 0 | 0 | #0 |  |
|  |  | 13 |  | 1st Door Open | #*1 | #*1 | #*1 | #*1 | #*1 | #*1 | #*1 | #*1 |  |
|  |  | 14 |  | 2nd Door Open | #*2 | #*2 | #*2 | #*2 | #*2 | #*2 | #*2 | #*2 |  |
|  |  | 15 |  | 3rd Door Open | #*3 | #*3 | #*3 | #*3 | #*3 | #*3 | #*3 | #*3 |  |
|  |  | 16 |  | 4th Door Open | #*4 | #*4 | #*4 | #*4 | #*4 | #*4 | #*4 | #*4 |  |
|  |  | 17 |  | 5th Door Open | #*5 | #*5 | #*5 | #*5 | #*5 | #*5 | #*5 | #*5 |  |
|  |  | 18 |  | 6th Door Open | #*6 | #*6 | #*6 | #*6 | #*6 | #*6 | #*6 | #*6 |  |
|  |  | 19 |  | 7th Door Open | #*7 | #*7 | #*7 | #*7 | #*7 | #*7 | #*7 | #*7 |  |
|  |  | 20 |  | VM Message Waiting Enable | *8 | *8 | *8 | *8 | *8 | *8 | *8 | *8 |  |
|  |  | 21 |  | VM Message Waiting Disable | *9 | *9 | *9 | *9 | *9 | *9 | *9 | *9 |  |

### TABLE 1.6.7   STATION ID ASSIGNMENT

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 108 | 1 | IP Name | Max 16 |  |  |
|  | 2 | Server Ip Addr | 12 Digits |  | Skip : # |
|  | 3 | CLI IP Addr | 12 Digits |  |  |
|  | 4 | Gateway Addr | 12 Digits |  |  |
|  | 5 | Subnet Mask | 12 Digits | 255.255.255.0 |  |

### TABLE 1.6.8   STATION ID ASSIGNMENT

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 110 | 1 | ID | 01-15 |  |  |
|  | 2 | DSS/DLS MAP – Associate Sta | Sta# |  |  |

### TABLE 1.6.9   STATION ATTRIBUTE I/II/III

| PGM | BTN | ITEM |  | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
| 111 | 1 | Auto Speaker Selection |  | ON / OFF | ON |  |
|  | 2 | Call Forward |  | ON / OFF | ON |  |
|  | 3 | DND |  | ON / OFF | ON |  |
|  | 4 | Data Line Security |  | ON / OFF | OFF |  |
|  | 5 | Howling Tone to SLT |  | ON / OFF | ON |  |
|  | 6 | ICM Box Signaling |  | ON / OFF | ON |  |
|  | 7 | No Touch Answer |  | ON / OFF | OFF |  |
|  | 8 | Page Access |  | ON / OFF | OFF |  |
|  | 9 | Ring Type |  | 0 - 4 | 0 |  |
|  | 10 | Speaker/Headset Ring |  | SP/HEAD/BOTH | Speaker-Phone |  |
|  | 11 | Speaker Phone/Headset |  | ON/OFF | ON |  |
|  | 12 | VMIB Slot |  | 0 - 2 | 0 |  |
|  | 13 | ICM Group Number |  | 1 - 15 | 1 |  |
| 112 | 1 | CO Warning Tone |  | ON / OFF | OFF |  |
|  | 2 | Automatic Hold |  | ON / OFF | OFF | STA2:ON |
|  | 3 | CO Call Time Restriction |  | ON / OFF | OFF |  |
|  | 4 | CO Line Access |  | EN/DIS | ENABLE |  |
|  | 5 | CO Line Queuing |  | EN/DIS | ENABLE |  |
|  | 6 | CO PGM |  | EN/DIS | DISABLE |  |
|  | 7 | PLA |  | EN/DIS | ENABLE |  |
|  | 8 | Prepaid Call |  | ON / OFF | OFF |  |
|  | 9 | Speed Dial Access |  | EN/DIS | ENABLE |  |
|  | 10 | Two Way Record |  | ON / OFF | OFF |  |
|  | 11 | Fax Mode |  | ON / OFF | OFF |  |
|  | 12 | OFFNET Call Mode |  | EXT/ALL | ALL |  |
| 113 | 1 | ADMIN |  | EN/DIS | DISABLE | STA100 Enable |
|  | 2 | VMIB Access |  | EN/DIS | DISABLE |  |
|  | 3 | Group Listening |  | EN/DIS | DISABLE |  |
|  | 4 | Override Privilege |  | EN/DIS | DISABLE |  |
|  | 5 | SMDR Hidden Dialed Digits |  | EN/DIS | DISABLE |  |
|  | 6 | Voice Over |  | EN/DIS | DISABLE |  |
|  | 7 | Warm Line | HotLine / WarmLine | Warm Line |  |  |
|  | 8 | Alarm Attribute | MISB RAU1 RAU2 | (ON/OFF) (ON/OFF) (ON/OFF) | OFF OFF OFF |  |

### TABLE 1.6.10   ISDN STATION ATTRIBUTE

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 114 | 1 | CLIP LCD DISPLAY | ON / OFF | OFF |  |
|  | 2 | COLP LCD DISPLAY | ON / OFF | OFF |  |
|  | 3 | CLI / REDIRECT DISPLAY | CLI /REDIRECT | CLI |  |
|  | 4 | CLI MSG WAIT | ON / OFF | OFF |  |
|  | 5 | EXT or CO ATD | ATD/EXT | EXT |  |
|  | 6 | KEYPAD FACILITY | KEYPAD/DTMF | DTMF |  |
|  | 7 | LONG/SHORT | LONG/SHORT | SHORT |  |
|  | 8 | CPN TYPE | 0-2 | 0(Not used) |  |
|  | 9 | S0 SUB ADDRESS | 0-2 | 0(Not used) |  |
|  | 10 | TEI Type | AUTO / FIXED | FIXED |  |
|  | 11 | CLI NAME DISPLAY | ON/OFF | OFF |  |
|  | 12 | ISDN CLI STA NUM | Max 4 digit | Logical STA Number |  |
|  | 13 | PROGRESS INDICATION | ON / OFF | ON |  |
|  | 14 | ISDN CLIR | ON/OFF | OFF |  |
|  | 15 | ISDN COLR | ON/OFF | OFF |  |

### TABLE 1.6.11   FLEXIBLE BUTTONS PROGRAM

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 115 | 01-24 | Flex. Buttons Assignment | BTN 01-24 |  |  |
|  |  | 1: User Button | - |  | Not programming |
|  |  | 2: {COxx} Button | 01-200 |  |  |
|  |  | 3: {CO Group xx} Button | 01-72 |  |  |
|  |  | 4: {LOOP} Button | - |  |  |
|  |  | 5: {STA xxx} Button | STA No. |  |  |
|  |  | 6: STA PGM Button | 11 - 99 |  | STN_COS_KEY(21,22,23) STN_WAKEUP_KEY(41,42) STN_MUSIC_KEY(73) STN_COLR_KEY(58) STN_ICM_KEY(83) STN_CALL_WAIT_KEY(85) STN_INTRUSION_KEY(86) STN_KEYPAD_FACILITY_KEY(89) STN_ACCT_CODE_KEY(31) STN_TWO_WAY_RECORD_KEY(54) |
|  |  | 7: {STA SPD xxx} Button | STA SPD Bin No |  |  |
|  |  | 8: {SYS SPD xxxx} Button | SYS SPD Bin No. |  |  |

### TABLE 1.6.12   STATION BASE PROGRAM

| PGM | BTN | ITEM | RANGE |  | DEFAULT |  | REMARK |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 116 | 1 | Station COS : Day | 1 - 7 |  | 1 |  |  |  |
|  | 2 | Station COS : Night | 1 - 7 |  | 1 |  |  |  |
| 117 |  | CO Group Access |  |  |  |  |  |  |
|  | 1 | CO Line Group 01~24 |  | 01-24 |  |  |  |  |
|  | 2 | CO Line Group 25~48 |  | 25-48 |  |  |  |  |
|  | 3 | CO Line Group 49~72 |  | 49-72 |  |  |  |  |
| 118 |  | Internal Page Zone Access |  | GRP 01 |  |  |  |  |
|  | 1 | Internal Page Zone 01~24 |  |  |  |  |  |  |
|  | 2 | Internal Page Zone 25~30 |  |  |  |  |  |  |
| 119 | 1-xx | Conference Page Zone Access | 31 - 35 |  |  |  |  |  |
| 120 |  | ICM Tenancy Group number |  |  |  |  |  |  |
|  | 1 | ICM Tenancy Group Attendant | STA No. |  |  | - |  |  |
|  | 2 | ICM Tenancy Access Group | 01 - 15 |  |  | - |  |  |
| 121 |  | Preset Call Forward |  |  |  | - |  |  |
| 122 |  | Hot Line / Warm Line |  |  |  | - |  |  |
|  |  | 1: Flex Button | 01 – 48 |  |  | - |  |  |
|  |  | 2: CO Line | 001-200 | - |  |  |  |  |
|  |  | 3: CO Group | 01-72 | - |  |  |  |  |
|  |  | 4: Station | STA No. |  |  | - |  |  |
| 123 | 1 | CTI Mode | 0 – 2 | 1 |  |  |  | 0:Inactive 1:CTI mode 2:AT mode |
|  | 2 | CTI Baud Rate | 0 – 2 | 0 |  |  |  | 0:1200 1:2400 2:4800 |
| 124 |  | SMDR Account Group Assign | 00 - 99 | 00(NOT_ASG) |  |  |  |  |

### TABLE 1.6.13  CO LINE BASE PROGRAM (1/2)

| PGM | BTN | ITEM |  | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
| 140 |  | CO Service Type |  |  |  |  |
|  | 1 | CO Type |  | 1-4 | 1(Normal) | 1:Normal, 2:A_DID, 3:ISDN DID/MSN, 4: TIE |
|  | 2 | Detailed Attribute of the type |  |  |  |  |
|  |  | DISA(D/N/W) |  |  |  | Day / Night / Weekend |
|  |  |  | DISA SVC | ON /OFF | OFF |  |
|  |  |  | VMIB ANNC | 00-70 | 00(NOT_ASG) |  |
|  |  | Analog DID |  |  |  |  |
|  |  |  | SIGNAL | 1-3 | 1 | 1: Immediate 2: Wink 3: Delayed Dial |
|  |  |  | INFO NO | 00-70 | 00(NOT_ASG) |  |
|  |  | TIE Attribute |  |  |  |  |
|  |  |  | TIE SIG | 1-4 | 1 | 1:RD 2:LD 3:EM-C 4:EM-D |
| 141 |  | CO Line Attribute 1 |  | 1-8 |  |  |
|  | 1 | CO Line Group Assignment |  | 00-73 | 01 |  |
|  | 2 | CO Line COS |  | 1-5 | 1 |  |
|  | 3 | DISA Account Code |  | ON / OFF | OFF |  |
|  | 4 | CO Line Assign |  | POL/LOOP | LOOP | Polarity RV, Loop Start |
|  | 5 | CO Line Type |  | PBX/CO | CO |  |
|  | 6 | CO Line Signal Type |  | DTMF/PULSE | DTMF |  |
|  | 7 | Flash Type |  | GROUND/LOOP | LOOP |  |
|  | 8 | UNA |  | ON / OFF | OFF |  |
|  | 9 | CO Line Group Account |  | ON / OFF | OFF |  |
| 142 |  | CO Line Attribute2 |  | 1-13 |  |  |
|  | 1 | CO Line Name Display |  | ON / OFF | OFF |  |
|  | 2 | CO Line Name Assign |  |  | - | Max 12 character |
|  | 3 | Metering Unit |  | 0-11 | 0 |  |
|  | 4 | Line Drop using CPT |  | ON / OFF | OFF |  |
|  | 5 | CO Distinct Ring |  | 0-4 | 0 |  |
|  | 6 | CO Line MOH |  | 0-13 | 1 |  |
|  | 7 | PABX CO Dial Tone |  | YES / NO | YES |  |
|  | 8 | PABX CO Ring Back Tone |  | YES / NO | NO |  |
|  | 9 | PABX CO Error Tone |  | YES / NO | NO |  |
|  | 10 | PABX CO Busy Tone |  | YES / NO | NO |  |
|  | 11 | PABX CO Announce Tone |  | YES / NO | NO |  |
|  | 12 | CO Flash Timer |  | 000 – 300 | 050 | 10 msec base |
|  | 13 | Open Loop Detect Timer |  | 0 – 20 | 0 | 100 msec base |

### TABLE 1.6.14  CO LINE BASE PROGRAM (2/2)

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 143 |  | ISDN CO Line Attribute | 1-5 |  |  |
|  | 1 | COLP Table Index | 00 ~ 50 | None | 00 ~ 49: PGM 201 Bin No. 50: PGM 114 Btn 5. |
|  | 2 | CLIP Table Index | 00 ~ 50 | None | 00 ~ 49: PGM 201 Bin No. 50: PGM 114 Btn 5. |
|  | 3 | Type of Calling Number | 0-4 | 2 | 0:Unknown number 1:International type 2:National number 3:Not used 4:Subscriber number |
|  | 4 | DID Conversion Type | 0 ~ 2 | 0 |  |
|  | 5 | DID Removal Number | 00-99 | 00 | 00:Do not ignore 01-99:indicate an ignored called party number (DID_RN : DID Remove number from called party information) |
|  | 6 | ISDN ENBLOCK SENDING | ON / OFF | OFF | ON:Enblock Sending Mode OFF:Overlap Sending Mode |
| 144 |  | CO Ring Assignment |  |  |  |
|  | 1 | DAY | STA_R/ HUNT/ DVU |  | STA RANGE(DELAY : 0 – 9), HUNT GRP, VMIB MSG, |
|  | 2 | NIGHT | STA_R/ HUNT/ DVU |  |  |
|  | 3 | WEEKEND | STA_R/ HUNT/ DVU |  |  |
|  | 4 | ON_DEMAND | STA_R/ HUNT/ DVU |  |  |

### TABLE 1.6.15   SYSTEM BASE PROGRAM (1/4)

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 160 |  | System Attributes-I | 1-10 |  |  |
|  | 1 | Attendant Call Queuing RB | ON/OFF | OFF | ON:Ring Back Tone OFF:PGM172 - 2 |
|  | 2 | CAMP RBT/MOH | RBT/MOH | MOH |  |
|  | 3 | CO Line Choice | LAST/ROUND | LAST | Round-Robin, Last Choice |
|  | 4 | DISA Retry Counter | 1-9 | 3 |  |
|  | 5 | ICM Continuous Dial-Tone | CONT / DISCONT | CONT | Continuous, Discontinuous |
|  | 6 | CO Dial-Tone Detect | ON / OFF | OFF |  |
|  | 7 | External Night Ring | ON / OFF | OFF |  |
|  | 8 | Hold Preference | SYS/EXEC | SYS | System, Exclusive |
|  | 9 | Multi-line Conference | ON / OFF | ON |  |
|  | 10 | Prt LCR Conv Dgt | ON / OFF | OFF |  |
| 161 |  | System Attributes-II | 1-11 |  |  |
|  | 1 | Network Time/Date Setting | ON / OFF | OFF |  |
|  | 2 | Off-Hook Ring Signal Type | MUTE/BURST | MUTE | Mute Ring, One Burst |
|  | 3 | Override 1st CO Group | ON / OFF | ON |  |
|  | 4 | Page Warning Tone | ON / OFF | ON |  |
|  | 5 | Privacy | ON / OFF | ON |  |
|  | 6 | Privacy Warning Tone | ON / OFF | ON |  |
|  | 7 | Single Ring for Co Call | YES/NO | NO |  |
|  | 8 | WTU Auto Rls | ON / OFF | OFF |  |
|  | 9 | ACD PRNT Enable | ON / OFF | OFF | ON:10s |
|  | 10 | ACD PRNT TImer | 001 – 255 | 001 | 10 sec base |
|  | 11 | ACD Clr After PRNT | ON / OFF | OFF |  |
| 162 | - | ADMIN Password | 4 Digits | - |  |
| 163 | 1 | Alarm Enable | ON / OFF | OFF |  |
|  | 2 | Alarm Contact Type | CLOSE/OPEN | CLOSE | Close, Open |
|  | 3 | Alarm Mode | ALARM / BELL | ALARM | Alarm, Door Bell |
|  | 4 | Alarm Signal Mode | RPT/ONCE | RPT | Repeat , Once |
| 164 | 1-5 | Attendant Assignment | STA No. | 1 : 101 | Max 5 Atds |

### TABLE 1.6.16   SYSTEM BASE PROGRAM (2/4)

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 165 | - | Auto Attendant DVU Annc.# | 00-70 | 00(not_asgn) |  |
| 166 |  | CO-to-CO COS | 1-7 | 1 |  |
|  | 1 | Day COS | 1-7 | 1 |  |
|  | 2 | Night COS | 1-7 | 1 |  |
| 167 | 1 | Error Destination | ATD / TONE | TONE |  |
|  | 2 | Busy Destination | ATD / TONE | TONE |  |
| 168 | 1 | First Contact | 1-5 | - | 1: LBC(STA #) 2: Door 3: Ext. 1 4: Ext. 2 5: Ext. 3 |
|  | 2 | Second Contact | 1-5 | - |  |
|  | 3 | Third Contact | 1-5 | - |  |
|  | 4 | Forth Contact | 1-5 | - |  |
|  | 5 | Fifth Contact | 1-5 | - |  |
|  | 6 | Sixth Contact | 1-5 | - |  |
|  | 7 | Seventh Contact | 1-5 | - |  |
| 169 | 1 | Time Display Mode | 12H/ 24H | 12H |  |
|  | 2 | Date Display Mode | MMDD / DDMM | DDMMYY |  |
|  | 3 | Language Display Mode | 00-14 | 12(Korean) | 0:ENGLISH, 1:ITALIAN, 2:FINNISH, 3:DUTCH, 4:SWEDISH, 5:DANISH, 6:NORWEGIAN, 7:HUNGARIAN, 8:GERMANY, 9:FRENCH, 10:PORTUGUESE, 11:SPANISH, 12:KOREAN, 13:ESTONIA, 14:RUSSIAN, |

### TABLE 1.6.17   SYSTEM BASE PROGRAM (3/4)

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 170 |  | Modem Associated Device |  | STA 399 | Last Station |
|  | 1 | Associated Station | STA number |  |  |
|  | 2 | Associated CO | CO number |  |  |
| 171 | 1 | BGM Type | 0-12 | 1 | 0: None 1: Int. Music 2-4: Ext. Music 1-3 5-7: VMIB 1-3 8-12: SLT MOH |
|  | 2 | MOH Type | 0-13 | 1 | 0: NOT_ASG 1: Int. Music 2-4: Ext. Music 1-3 5-7: VMIB 1-3 8-12: SLT MOH 13: Hold Tone |
|  | 3 | ICM Box Music Channel | 0-12 | 1 | The same as above 1 |
|  | 4 | Assign MOH via SLT | Flex. 1-5(+SLT STA No.) | - | SLT MOH 1-5 |
| 172 | 1 - 4 | PBX Access Code | MAX 2 digits | - | Maximum 4 PBX access code |
| 173 |  | PLA Priority Setting |  |  | PLA priority is set Exclusively |
|  | 1 | Transfer CO | 1 – 4 | 1 |  |
|  | 2 | Recalling CO | 1 – 4 | 2 |  |
|  | 3 | Incoming CO | 1 – 4 | 3 |  |
|  | 4 | Queued CO | 1 – 4 | 4 |  |
| 174 |  | RS232 PORT Setting |  |  | Flex 1 – 5 |
|  | 1 | Baudrate Setting | 0-8 | 19200(6) | 0: UNKNOWN 1: UNKNOWN 2: 1200 BAUD 3: 2400 BAUD 4: 4800 BAUD 5: 9600 BAUD 6: 19200 BAUD 7: 38400 BAUD 8: 57600 BAUD |
|  | 2 | CTS | ON / OFF | OFF |  |
|  | 3 | Page Break | ON / OFF | OFF |  |
|  | 4 | Line Page | 001-199 | 060 |  |

### TABLE 1.6.18   SYSTEM BASE PROGRAM (4/4)

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 175 | 1 | Off-line SMDR/Statistics Print | 01-13 | COM2(02) | Value : 1-8 |
|  | 2 | ADMIN Print | 01-13 | COM2(02) | Value : 1-8 |
|  | 3 | Traffic Print | 01-13 | COM2(02) | Value : 1-8 |
|  | 4 | SMDI Print | 01-13 | COM2(02) | Value : 1-8 |
|  | 5 | CALL Info Print | 01-13 | COM2(02) | Value : 1-8 |
|  | 6 | On-line SMDR Print | 01-13 | COM2(02) | Value : 1-8 |
|  | 7 | Trace Print | 01-13 | COM2(02) | Value : 1-8 |
|  | 8 | Debug Print | 01-13 | COM2(02) | Value : 1-8 |
|  | 9 | PC_ADM | 01-13 | NET_PCADM | Value : 1-5 , 10 |
|  | 10 | PC_ATD | 01-13 | NET_PCATD | Value : 1,2,4,5,11 |
|  | 11 | CTI | 01-13 | NET_CTI | Value : 1,2,4,5,12 |
|  | 12 | REMOTE_DIAG | 01-13 | NET_REMOTE | Value : 1-5, 13 |
| 176 | - | Pulse Dial/Speed Ratio | 66/33 / 60/40 | 66/33 | 66/33 60/40 (10 PPS only) |
| 177 |  | SMDR Attributes | 1-14 |  |  |
|  | 1 | SMDR Save Enable | ON / OFF | OFF |  |
|  | 2 | SMDR Print Enable | ON / OFF | OFF |  |
|  | 3 | SMDR Recording Call Type | LD / ALL | LD | LD : Long Distance ALL: All |
|  | 4 | SMDR Long Distance Call Digit Counter | 07-15 | 07 | Consider more than this value as Long Distance Call. |
|  | 5 | Print Incoming Call | ON / OFF | OFF |  |
|  | 6 | Print Lost Call | ON / OFF | OFF |  |
|  | 7 | Records in Detail | ON / OFF | ON |  |
|  | 8 | SMDR Dial Digit Hidden | 0-9 | 0 |  |
|  | 9 | SMDR Currency | 3 English Chars | - |  |
|  | 10 | SMDR Cost Per Unit Pulse | 6 digits | - |  |
|  | 11 | SMDR Fraction | 0-5 | 0 |  |
|  | 12 | SMDR Start Timer | 0 – 250 | 0 | 1 sec base |
|  | 13 | SMDR Hidden Dgt | RIGHT/LEFT | RIGHT |  |
|  | 14 | SMDR Long Distance Codes | MAX 2 digits | 0 | Max 5 LD code |
| 178 | 1 | System Time Setting | 4 digits | - | Hour/Min sequence |
|  | 2 | System Date Setting | 6 digits | - | Month/Day/Year sequence. |
| 179 | 1 | VIEW |  |  | Show the linked pairs |
|  | 2 | Linked Pair Input | 2 STA# |  | Max: 64 pairs |

### TABLE 1.6.19  SYSTEM TIMER PROGRAM   (1/2)

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 180 | 1 | - Attendant Recall Timer | 00 - 60 | 01 | 1 min base |
|  | 2 | - Call Park Recall Timer | 000 - 600 | 120 | 1 sec base |
|  | 3 | Camp-on Recall Timer | 000 - 200 | 030 | 1 sec base |
|  | 4 | - Exclusive Hold Recall Timer | 000 - 300 | 060 | 1 sec base |
|  | 5 | - I-Hold Recall Timer | 000 - 300 | 030 | 1 sec base |
|  | 6 | - Sys Hold Recall Timer | 000 - 300 | 030 | 1 sec base |
|  | 7 | - Transfer Recall Timer | 000 - 300 | 030 | 1 sec base |
|  | 8 | - ACNR Delay Timer | 000 - 300 | 030 | 1 sec base |
|  | 9 | - ACNR No Answer Timer | 10 - 50 | 30 | 1 sec base |
|  | 10 | - ACNR Pause Timer | 030 - 300 | 030 | 1 sec base |
|  | 11 | - ACNR Retry Counter | 01 - 13 | 3 |  |
|  | 12 | - ACNR Retry NoTone | 1 - 9 | 1 |  |
|  | 13 | - ACNR Tone Detect Timert | 001-300 | 030 | 1 sec base |
|  | 14 | Automatic CO Release Timer. | 020 - 300 | 030 | 1 sec base |
|  | 15 | - CCR Inter-Digit Timer | 000 - 300 | 030 | 100 msec base |
|  | 16 | - CO Call Drop Warning Timer | 00 - 99 | 10 | 1 sec base |
|  | 17 | - CO Call Restriction Timer | 00 - 99 | 0 | 1 min base |
|  | 18 | - CO Dial Delay Timer | 00 - 99 | 01 | 100 msec base |
|  | 19 | - CO Release Guard Timer | 001 - 150 | 002 | 100 msec base |
|  | 20 | CO Ring Off Timer | 010 - 150 | 060 | 100 msec base |
|  | 21 | - CO Ring On Timer | 1 - 9 | 2 | 100 msec base |
|  | 22 | - CO Warning Tone Timer | 060 - 900 | 180 | 1 sec base |
| 181 | 1 | Call FWD No Answer Timer | 000 - 600 | 015 | 1 sec base |
|  | 2 | DID/DISA No Answer Timer | 00 - 99 | 20 | 1 sec base |
|  | 3 | DVU User Record Timer | 010 - 255 | 020 | 1 sec base |
|  | 4 | DVU Valid User Message Timer | 0-9 | 4 | 1 sec base |
|  | 5 | Door Open Timer | 05 - 99 | 20 | 100msec base |
|  | 6 | ICM Box Timer | 00 - 60 | 30 | 1 sec base |
|  | 7 | ICM Dial Tone Timer | 01 - 20 | 10 | 1 sec base |
|  | 8 | Inter Digit Timer | 01 - 20 | 05 | 1 sec base |
|  | 9 | MSG Wait Reminder Tone Timer | 00 - 60 | 00 | 1 min base |
|  | 10 | Paging Timeout Timer | 000 - 255 | 015 | 1 sec base |
|  | 11 | - Pause Timer | 1 - 9 | 2 | 1 sec base |
|  | 12 | - Preset Call Forward Timer | 00 - 99 | 10 | 1 sec base |

### TABLE 1.6.20  SYSTEM TIMER PROGRAM   (2/2)

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 182 | 1 | SLT Hook Switch Bounce Timer | 01 - 25 | 01 | 100msec base |
|  | 2 | SLT Maximum Hook Flash Timer | 01-25 | 05 | 100msec base |
|  | 3 | SLT Minimum Hook Flash Timer | 000 - 250 | 020 | 10 msec base |
|  | 4 | SLT Ring Phase Timer | 2 - 5 | 5 | 1 sec base |
|  | 5 | Station Auto Release Timer | 020 - 300 | 060 | 1 sec base |
|  | 6 | - Unsupervised Conference Timer | 00 - 99 | 10 | 1 min base |
|  | 7 | Wake-Up Fail Ring Timer | 00 - 99 | 20 | 1 sec base |
|  | 8 | - Warm Line Timer | 01 - 20 | 05 | 1 sec base |
|  | 9 | Wink Timer | 010 - 200 | 010 | 10 msec base |
|  | 10 | Enblock Dgt timer | 01-20 | 15 | 1 sec base |
|  | 11 | CCR Time Out Timer | 000-300 | 015 | 1 sec base |

### TABLE 1.6.21  STATION GROUP ASSIGN

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 190 |  | Station Group number | Sta Grp # | - |  |
|  | 1 | Group Type | 0-6 | 0 | 0:NOT ASGN 1: Circular 2: Terminal 3: UCD 4: Ring 5: VM 6: Pick up |
|  | 2 | Pick-up Attribute | ON/OFF | OFF | OFF |
|  | 3 | Member assignment | Not Assigned | - | First, Group Type must be assigned |

### TABLE 1.6.22    STATION GROUP PROGRAM  (1/3)

| PGM | ITEM | BTN | SUB ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
| 191 | CIRCULAR GROUP | 1 | DVU Announce 1 Timer | 000-999 | 015 | 1 sec base |
|  |  | 2 | DVU Announce 2 Timer | 000-999 | 000 | 1 sec base |
|  |  | 3 | DVU Announce Location 1 | 00-70 | 00(not_asgn) |  |
|  |  | 4 | DVU Announce Location 2 | 00-70 | 00(not_asgn) |  |
|  |  | 5 | DVU Announce 2 Repeat | 000-999 | 000 | 1 sec base |
|  |  | 6 | DVU Announce 2 Repeat E/D | ON / OFF | OFF |  |
|  |  | 7 | Overflow Destination | STA #/HUNT # /DVU # | - |  |
|  |  | 8 | Overflow Timer | 000-600 | 180 | 1 sec base |
|  |  | 9 | Wrap-Up Timer | 002-999 | 002 | 1 sec base |
|  |  | 10 | No Answer Timer | 00-99 | 15 | 1 sec base |
|  |  | 11 | Pilot Hunt | ON / OFF | ON |  |
|  |  | 12 | REPT NO MEM | ON / OFF | OFF |  |
|  |  | 13 | Music Source | 00-12 | 00 |  |
|  | TERMINAL GROUP | 1 | DVU Announce 1 Timer | 000-999 | 015 | 1 sec base |
|  |  | 2 | DVU Announce 2 Timer | 000-999 | 000 | 1 sec base |
|  |  | 3 | DVU Announce Location 1 | 00-70 | 00(not_asgn) |  |
|  |  | 4 | DVU Announce Location 2 | 00-70 | 00(not_asgn) |  |
|  |  | 5 | DVU Announce 2 Repeat | 000-999 | 000 | 1 sec base |
|  |  | 6 | DVU Announce 2 Repeat E/D | ON / OFF | OFF |  |
|  |  | 7 | Overflow Destination | STA #/HUNT #/ DVU # | - |  |
|  |  | 8 | Overflow Timer | 000-600 | 180 | 1 sec base |
|  |  | 9 | Wrap-Up Timer | 002-999 | 002 | 1 sec base |
|  |  | 10 | No Answer Timer | 00-99 | 15 | 1 sec base |
|  |  | 11 | Pilot Hunt | ON / OFF | ON |  |
|  |  | 12 | Report No member | ON / OFF | OFF |  |
|  |  | 13 | Music Source | 00-12 | 00 |  |

### TABLE 1.6.23    STATION GROUP PROGRAM  (2/3)

| PGM | ITEM | BTN | SUB ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
|  | UCD GROUP | 1 | DVU Announce 1 Timer | 000-999 | 015 | 1 sec base |
|  |  | 2 | DVU Announce 2 Timer | 000-999 | 000 | 1 sec base |
|  |  | 3 | DVU Announce Location 1 | 00-70 | 00(not_asgn) |  |
|  |  | 4 | DVU Announce Location 2 | 00-70 | 00(not_asgn) |  |
|  |  | 5 | DVU Announce 2 Repeat | 000-999 | 000 | 1 sec base |
|  |  | 6 | DVU Announce 2 Repeat E/D | ON / OFF | OFF |  |
|  |  | 7 | Overflow Destination | STA #/HUNT#/ DVU # | - |  |
|  |  | 8 | Overflow Timer | 000-600 | 180 | 1 sec base |
|  |  | 9 | Wrap Up Timer | 002-999 | 002 | 1 sec base |
|  |  | 10 | REPT NO MEM | ON / OFF | OFF |  |
|  |  | 11 | Music Source | 00-12 | 00 |  |
|  |  | 12 | ACD Warning Tone | ON / OFF | OFF |  |
|  |  | 13 | Alternate destination | STA #/ STA GRP# |  |  |
|  |  | 14 | Supervisor Timer | 000-999 | 030 | 1 sec base |
|  |  | 15 | Supervisor Call Cnt | 00-99 | 00 |  |
|  |  | 16 | ACD Queued Call | ON / OFF | OFF |  |
|  |  | 17 | Supervisor | STA # | - |  |
|  |  | 18 | UCD hunt Stations' Priority | 0-9 | 0 | For each member |
|  | RING GROUP | 1 | DVU Announce 1 Timer | 000-999 | 015 | 1 sec base |
|  |  | 2 | DVU Announce 2 Timer | 000-999 | 000 | 1 sec base |
|  |  | 3 | DVU Announce Location 1 | 00-07 | 00(not_asgn) |  |
|  |  | 4 | DVU Announce Location 2 | 00-07 | 00(not_asgn) |  |
|  |  | 5 | DVU Announce 2 Repeat | 000-999 | 000 | 1 sec base |
|  |  | 6 | DVU Announce 2 Repeat E/D | ON / OFF | OFF |  |
|  |  | 7 | Overflow Destination | STA #/HUNT# DVU# | - |  |
|  |  | 8 | Overflow Timer | 000-600 | 180 | 1 sec base |
|  |  | 9 | Wrap Up Timer | 002-999 | 002 | 1 sec base |
|  |  | 10 | Music Source | 00-12 | 00 |  |

### TABLE 1.6.24    STATION GROUP PROGRAM  (3/3)

| PGM | ITEM | BTN | SUB ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
|  | VM GROUP | 1 | Wrap-Up Timer | 002-999 | 002 | 1 sec base |
|  |  | 2 | Put Mail Index | 1-4 | 1 |  |
|  |  | 3 | Get Mail Index | 1-4 | 2 |  |
|  |  | 4 | Hunt Type | Cir/Term | Term |  |
|  |  | 5 | SMDI Port | 01-13 | 02(COM2) |  |
|  |  | 6 | Overflow Timer | 000-600 | 180 | 1 sec base |
|  |  | 7 | Overflow Destination | STA #/ HUNT#/ DVU# | - |  |
|  | PICK_UP GROUP | 1 | Auto Pick_Up | ON / OFF | OFF |  |
|  |  | 2 | All Group Member Ringing | ON / OFF | OFF |  |

### TABLE 1.6.25   ISDN ATTRIBUTES

| PGM | BTN | ITEM |  | RANGE |  | DEFAULT |  |  | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 200 | 1 | Advice of Charge | 0-5 |  | 0 |  | 0: Do not service AOC 1: Italy and Spain 2: Finland 3: Australia 4: Belgium 5: Standard |  |  |
|  | 2 | CO ATD Code | MAX 2 digits |  | - |  |  |  |  |
|  | 3 | Incoming Prefix Code Insertion | ON / OFF |  | OFF |  |  |  |  |
|  | 4 | Outgoing Prefix Code Insertion | ON / OFF |  | ON |  |  |  |  |
|  | 5 | A/u-Law Line Installed | U/A |  | A-Law |  | ON: u-Law OFF: A-Law |  |  |
|  | 6 | CLI Print | ON/OFF |  | OFF |  |  |  |  |
|  | 7 | International Access Code | Max 4 digit |  | - |  |  |  |  |
|  | 8 | Calling Sub-address | ON/OFF |  | OFF |  | If this field is ON, station number will be filled in calling party number sub-address IE in setup. |  |  |
|  | 9 | My Area Code | Max 6 digits |  | - |  |  |  |  |
|  | 10 | My Area Prefix Code | Max 4 digits |  | - |  |  |  |  |
| 201 | - | COLP Table | Entry No. (00-49) |  | - |  | MAX 10 digits |  |  |
| 202 | - | MSN Table | Entry No. (000-249) |  | - |  |  |  |  |
|  | 1 | CO Line number | 001-200 |  | - |  |  |  |  |
|  | 2 | Index | 000-999 |  | - |  |  |  |  |
|  | 3 | SUB Number | 0-9 |  | - |  |  |  |  |
|  | 4 | MSN number | 20 digit |  | - |  |  |  |  |

### TABLE 1.6.26   LCR TABLE ASSIGNMENT  (1/2)

| PGM | BTN | ITEM |  |  | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 220 | 1 | LCR Access Mode |  |  | M00/M01/M02/M11/M12 | M00 |  |
|  | 2 | Set the Day of week zone |  |  |  | 1234567 |  |
|  |  | 1 |  | MON | 1~3 | 1 |  |
|  |  | 2 |  | TUE | 1~3 | 1 |  |
|  |  | 3 |  | WED | 1~3 | 1 |  |
|  |  | 4 |  | THUR | 1~3 | 1 |  |
|  |  | 5 |  | FRI | 1~3 | 1 |  |
|  |  | 6 |  | SAT | 1~3 | 1 |  |
|  |  | 7 |  | SUN | 1~3 | 1 |  |
|  | 3 | Set the Time Zone of Day zone 1 |  |  |  |  |  |
|  |  | 1 | Time Zone1 |  | 00~24 | 0024 |  |
|  |  | 2 | Time Zone2 |  | 00~24 | - |  |
|  |  | 3 | Time Zone3 |  | 00~24 | - |  |
|  | 4 | Set the Time Zone of Day zone 2 |  |  |  |  |  |
|  |  | 1 |  | Time Zone1 | 00~24 | 0024 |  |
|  |  | 2 |  | Time Zone2 | 00~24 | - |  |
|  |  | 3 |  | Time Zone3 | 00~24 | - |  |
|  | 5 | Set the Time Zone of Day zone 3 |  |  |  |  |  |
|  |  | 1 |  | Time Zone1 | 00~24 | 0024 |  |
|  |  | 2 |  | Time Zone2 | 00~24 | - |  |
|  |  | 3 |  | Time Zone3 | 00~24 | - |  |

### TABLE 1.6.27    LCR TABLE ASSIGNMENT (2/2)

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 221 |  | LDT Table | 000-249 |  |  |
|  | 1 | LCR Type | 1 - 3 | 3 | 1:int 2:col 3: both |
|  | 2 | CODE(leading digit) | Max 12 digits | - |  |
|  | 3 | Day Zone 1 DMT | 6digits |  | Time1: 2digit |
|  | 4 | Day Zone 2 DMT | 6digits |  | Time2: 2digit |
|  | 5 | Day Zone 3 DMT | 6digits |  | Time3: 2digit |
| 222 |  | DMT Table | 00-99 |  |  |
|  | 1 | Added Dgt | Max 25 digits |  |  |
|  | 2 | Removeal Position | 1 - 12 | 1 |  |
|  | 3 | Num Of Remove | 1 - 12 | 0 |  |
|  | 4 | Add Position | 1 – 13 | 1 |  |
|  | 5 | CO Group | 1 – 72 | 1 |  |
|  | 6 | Alt Index | 0 - 99 | - |  |
| 223 |  | LCR Table Init |  |  |  |
|  | 1 | DMT Of Day_zone_1 | 6digits |  | Time1: 2digit |
|  | 2 | DMT Of Day_zone_2 | 6digits |  | Time2: 2digit |
|  | 3 | DMT Of Day_zone_3 | 6digits |  | Time3: 2digit |
|  | 4 | Co Grp Init | 1 - 72 |  |  |
|  | 5 | Alt Index Init | 0 - 99 |  |  |
|  | 6 | Init All LCR |  |  |  |

### TABLE 1.6.28   TOLL TABLE ASSIGNMENT

| PGM | BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 224 |  | TOLL Table |  |  |  |
|  | 1 | Allow Table A(01-30) | Max 14 digits | - |  |
|  | 2 | Deny Table A(01-30) | Max 14 digits | - |  |
|  | 3 | Allow Table B(01-30) | Max 14 digits | - |  |
|  | 4 | Deny Table B(01-30) | Max 14 digits | - |  |
| 225 |  | Canned TOLL Table |  |  |  |
|  | 1 | Allow Table(01-20) | Max 14 digits | - |  |
|  | 2 | Deny Table(01-20) | Max 14 digits | - |  |
| 226 |  | Emergency Code Table(01-10) | Max 14 digits |  |  |

### TABLE 1.6.29   OTHER TABLES

| PGM | BTN | ITEM |  | RANGE |  | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 227 |  | Author Code Table |  | 001 - 600 |  |  |  |
|  |  | Table entry(01-600) |  | Max 5 digits |  |  |  |
| 228 |  | CCR Table |  | 1-70 |  |  |  |
|  | 1 | Station |  | STA # |  | - |  |
|  | 2 | Hunt Group |  | HUNT # |  | - |  |
|  | 3 | VMIB |  | Annonuce # |  | - |  |
|  | 4 | VMIB DROP |  | Annonuce # |  |  |  |
|  | 5 | System Speed |  | 2000-4999 |  | - |  |
|  | 6 | Internal Page |  | 1 – 35 |  | - |  |
|  | 7 | External Page |  | 1 – 3 |  | - |  |
|  | 8 | All Call Page |  | 1 – 3 |  | - | 1:INT 2:EXT 3:All |
| 229 |  | Exec/Sec Table |  | 01-36 |  |  |  |
|  |  | Table entry(01-36) |  | STA#/STA# |  |  |  |
| 230 |  | Did Dgt Conversion |  |  |  |  |  |
|  | 1 | Receive Number |  | 2 – 4 |  | 3 |  |
|  | 2 | Dgt Mask |  | Max 5digits |  | #*** |  |
| 231 | - | Flexible DID Table | Entry No. (000-999) |  | - |  |  |
|  | 1 | DID Name | Max 11 |  | - |  |  |
|  | 2 | Day Destination | 1-8 |  | - |  | 1(sta), 2(hunt), 3(dvu), |
|  | 3 | Night Destination | 1-8 |  | - |  | 4(dvu drop), 5(spd), |
|  | 4 | Weekend Destination | 1-8 |  | - |  | 6(int. page), 7(ext. page) |
|  |  |  |  |  |  |  | 8(all page) |
| 232 |  | System Speed Zone | 01-10 |  |  |  | % Zone 1 |
|  | 1 | Spd Bin Range in Zone | 2200 – 4999 |  | - |  | 2200 – 4999 |
|  | 2 | Station Range | Sta range |  | - |  | Sta_1 – Sta_300 |
|  | 3 | Toll Checking | On/Off |  | - |  | On |
| 233 |  | Weekly Time Table | 1 – 7 |  | - |  | Day of Week |
|  | 1 | Day Start Time | 0000 – 2359 |  | 0900 |  |  |
|  | 2 | Night Start Time | 0000 – 2359 |  | 1800 |  |  |
|  | 3 | Weekend Start Time | 0000 – 2359 |  |  |  |  |
| 234 |  | Voice Mail Dial-Table | 1 - 9 |  |  |  |  |
|  | 1 | Prefix Index | 12 Digits |  | - |  | Refer Spec |
|  | 2 | Suffix Index | 12 Digits |  | - |  | Refer Spec |
| 235 |  | TIE Line Routing Table | Co number(001 – 200) |  | - |  |  |

### TABLE 1.6.30    NATION SPECIPIC (1/4)

| PGM | FLEX | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 400 |  | DTIB RX GAIN |  |  | Korea version |
|  | 1 | DTIB/DKT | 00 – 63 | 26 |  |
|  | 2 | DTIB/SLT | 00 – 63 | 33 |  |
|  | 3 | DTIB/WTU | 00 – 63 | 26 |  |
|  | 4 | DTIB/ACO | 00 – 63 | 33 |  |
|  | 5 | DTIB/DCO | 00 – 63 | 33 |  |
|  | 6 | DTIB/DVU | 00 – 63 | 29 |  |
|  | 7 | DTIB/DTMF | 00 – 63 | 8 |  |
|  | 8 | DTIB/TONE | 00 – 63 | 32 |  |
|  | 9 | DTIB/MUSIC1 | 00 – 63 | 29 |  |
|  | 10 | DTIB/MUSIC2 | 00 – 63 | 29 |  |
|  | 11 | DTIB/MUSIC3 | 00 – 63 | 29 |  |
| 401 |  | SLIB RX GAIN |  |  |  |
|  | 1 | SLIB/DKT | 00 – 63 | 12 |  |
|  | 2 | SLIB/SLT | 00 – 63 | 26 |  |
|  | 3 | SLIB/WTU | 00 – 63 | 12 |  |
|  | 4 | SLIB/ACO | 00 – 63 | 21 |  |
|  | 5 | SLIB/DCO | 00 – 63 | 24 |  |
|  | 6 | SLIB/DVU | 00 – 63 | 20 |  |
|  | 7 | SLIB/DTMF | 00 – 63 | 8 |  |
|  | 8 | SLIB/TONE | 00 – 63 | 18 |  |
|  | 9 | SLIB/MUSIC1 | 00 – 63 | 20 |  |
|  | 10 | SLIB/MUSIC2 | 00 – 63 | 20 |  |
|  | 11 | SLIB/MUSIC3 | 00 – 63 | 20 |  |
| 402 |  | SLIB12 RX GAIN |  |  |  |
|  | 1 | SLIB12/DKT | 00 – 63 | 12 |  |
|  | 2 | SLIB12/SLT | 00 – 63 | 26 |  |
|  | 3 | SLIB12/WTU | 00 – 63 | 12 |  |
|  | 4 | SLIB12/ACO | 00 – 63 | 21 |  |
|  | 5 | SLIB12/DCO | 00 – 63 | 24 |  |
|  | 6 | SLIB12/DVU | 00 – 63 | 20 |  |
|  | 7 | SLIB12/DTMF | 00 – 63 | 8 |  |
|  | 8 | SLIB12/TONE | 00 – 63 | 18 |  |
|  | 9 | SLIB12/MUSIC1 | 00 – 63 | 20 |  |
|  | 10 | SLIB12/MUSIC2 | 00 – 63 | 20 |  |
|  | 11 | SLIB12/MUSIC3 | 00 – 63 | 20 |  |

### TABLE 1.6.31    NATION SPECIPIC (2/4)

| 403 |  | WTIB RX GAIN |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 | WTIB/DKT | 00 – 63 | 26 |  |
|  | 2 | WTIB/SLT | 00 – 63 | 33 |  |
|  | 3 | WTIB/WTU | 00 – 63 | 26 |  |
|  | 4 | WTIB/ACO | 00 – 63 | 38 |  |
|  | 5 | WTIB/DCO | 00 – 63 | 33 |  |
|  | 6 | WTIB/DVU | 00 – 63 | 29 |  |
|  | 7 | WTIB/DTMF | 00 – 63 | 8 |  |
|  | 8 | WTIB/TONE | 00 – 63 | 37 |  |
|  | 9 | WTIB/MUSIC1 | 00 – 63 | 29 |  |
|  | 10 | WTIB/MUSIC2 | 00 – 63 | 29 |  |
|  | 11 | WTIB/MUSIC3 | 00 – 63 | 29 |  |
| 404 |  | ACOB RX GAIN |  |  |  |
|  | 1 | ACOB/DKT | 00 – 63 | 26 |  |
|  | 2 | ACOB/SLT | 00 – 63 | 37 |  |
|  | 3 | ACOB/WTU | 00 – 63 | 26 |  |
|  | 4 | ACOB/ACO | 00 – 63 | 24 |  |
|  | 5 | ACOB/DCO | 00 – 63 | 33 |  |
|  | 6 | ACOB/DVU | 00 – 63 | 32 |  |
|  | 7 | ACOB/DTMF | 00 – 63 | 32 |  |
|  | 8 | ACOB/TONE | 00 – 63 | 32 |  |
|  | 9 | ACOB/MUSIC1 | 00 – 63 | 32 |  |
|  | 10 | ACOB/MUSIC2 | 00 – 63 | 32 |  |
|  | 11 | ACOB/MUSIC3 | 00 – 63 | 32 |  |
|  | 12 | ACOB/MODEM | 00 – 63 | 37 |  |
| 405 |  | LCOB8 RX GAIN |  |  |  |
|  | 1 | LCOB8/DKT | 00 – 63 | 26 |  |
|  | 2 | LCOB8/SLT | 00 – 63 | 37 |  |
|  | 3 | LCOB8/WTU | 00 – 63 | 26 |  |
|  | 4 | LCOB8/ACO | 00 – 63 | 24 |  |
|  | 5 | LCOB8/DCO | 00 – 63 | 33 |  |
|  | 6 | LCOB8/DVU | 00 – 63 | 32 |  |
|  | 7 | LCOB8/DTMF | 00 – 63 | 32 |  |
|  | 8 | LCOB8/TONE | 00 – 63 | 32 |  |
|  | 9 | LCOB8/MUSIC1 | 00 – 63 | 32 |  |
|  | 10 | LCOB8/MUSIC2 | 00 – 63 | 32 |  |
|  | 11 | LCOB8/MUSIC3 | 00 – 63 | 32 |  |
|  | 12 | LCOB8/MODEM | 00 – 63 | 37 |  |

### TABLE 1.6.32    NATION SPECIPIC (3/4)

| 406 |  | DCOB RX GAIN |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 | DCOB/DKT | 00 – 63 | 26 |  |
|  | 2 | DCOB/SLT | 00 – 63 | 37 |  |
|  | 3 | DCOB/WTU | 00 – 63 | 26 |  |
|  | 4 | DCOB/ACO | 00 – 63 | 24 |  |
|  | 5 | DCOB/DCO | 00 – 63 | 33 |  |
|  | 6 | DCOB/DVU | 00 – 63 | 32 |  |
|  | 7 | DCOB/DTMF | 00 – 63 | 32 |  |
|  | 8 | DCOB/TONE | 00 – 63 | 32 |  |
|  | 9 | DCOB/MUSIC1 | 00 – 63 | 32 |  |
|  | 10 | DCOB/MUSIC2 | 00 – 63 | 32 |  |
|  | 11 | DCOB/MUSIC3 | 00 – 63 | 32 |  |
|  | 12 | DCOB/MODEM | 00 – 63 | 37 |  |
| 407 |  | VMIB RX GAIN |  |  |  |
|  | 1 | VMIB/DKT | 00 – 63 | 21 |  |
|  | 2 | VMIB/SLT | 00 – 63 | 32 |  |
|  | 3 | VMIB/WTU | 00 – 63 | 26 |  |
|  | 4 | VMIB/ACO | 00 – 63 | 32 |  |
|  | 5 | VMIB/DCO | 00 – 63 | 32 |  |
|  | 6 | VMIB/MUSIC1 | 00 – 63 | 32 |  |
|  | 7 | VMIB/MUSIC2 | 00 – 63 | 32 |  |
| 408 |  | DTMF RC GAIN |  |  |  |
|  | 1 | DTMF/SLT | 00 – 63 | 37 |  |
|  | 2 | DTMF/ACO | 00 – 63 | 24 |  |
|  | 3 | DTMF/DCO | 00 – 63 | 24 |  |
| 409 |  | EXT PAGE GAIN |  |  |  |
|  | 1 | EXT PAGE/DKT | 00 – 63 | 26 |  |
|  | 2 | EXT PAGE/SLT | 00 – 63 | 37 |  |
|  | 3 | EXT PAGE/WTU | 00 – 63 | 26 |  |
|  | 4 | EXT PAGE/ACO | 00 – 63 | 37 |  |
|  | 5 | EXT PAGE/DCO | 00 – 63 | 37 |  |
|  | 6 | EXT PAGE/DVU | 00 – 63 | 37 |  |
|  | 7 | EXT PAGE/MUSIC1 | 00 – 63 | 37 |  |
|  | 8 | EXT PAGE/MUSIC2 | 00 – 63 | 37 |  |
|  | 9 | EXT PAGE/MUSIC3 | 00 – 63 | 37 |  |
| 410 |  | CPT GAIN |  |  |  |
|  | 1 | CPT/ACO | 00 – 63 | 24 |  |
|  | 2 | CPT/DCO | 00 – 63 | 24 |  |
| 411 |  | MODEM GAIN |  |  |  |
|  | 1 | MODEM/ACO | 00 – 63 | 24 |  |
|  | 2 | MODEM/DCO | 00 – 63 | 24 |  |

### TABLE 1.6.33    NATION SPECIPIC (4/4)

| PGM | FLEX | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 420 |  | SYSTEM Tone Freq |  |  |  |
|  | 1 | Dial Tone | 4digits | 0425 , 0000 |  |
|  | 2 | Ring Back Tone | 4digits | 0425 , 0000 |  |
|  | 3 | Busy Tone | 4digits | 0425 , 0000 |  |
|  | 4 | Error Tone | 4digits | 0620 , 000 |  |
|  | 5 | Dummy Dial Tone | 4digits | 0350 , 440 |  |
| 421 |  | Diff Ring Freq |  |  |  |
|  | 1 | Ring1 | 4digits | 1000, 1020 |  |
|  | 2 | Ring2 | 4digits | 0890 , 0910 |  |
|  | 3 | Ring3 | 4digits | 1260 , 1280 |  |
|  | 4 | Ring4 | 4digits | 0800 , 0820 |  |
| 422 |  | Distinct Ring Freq |  |  |  |
|  | 1 | Ring1 | 4digits | 0480 , 0000 |  |
|  | 2 | Ring2 | 4digits | 0400 , 0000 |  |
|  | 3 | Ring3 | 4digits | 0620 , 0000 |  |
|  | 4 | Ring4 | 4digits | 0770 , 0000 |  |
| 423 |  | ACNR Tone Cadence |  |  |  |
|  | 1 | Ring-Back Tone | 0-255 | 050 , 100 | 20msec base |
|  | 2 | Busy Tone | 0-255 | 025 , 025 | 20msec base |
|  | 3 | Error Tone | 0-255 | 012 , 012 | 20msec base |
|  | 4 | S –Dial Tone | 0-255 | 070 , 000 | 20msec base |

### TABLE 1.6.34    INITIALIZATION

| PGM | FLEX | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 450 |  | Initialization |  |  |  |
|  | 1 | Flexible Numbering Plan Initialization |  |  |  |
|  | 2 | Station Database Initialization |  |  |  |
|  | 3 | CO Line Database Initialization |  |  |  |
|  | 4 | System Feature Database Initialization |  |  |  |
|  | 5 | Station Group Database Initialization |  |  |  |
|  | 6 | ISDN Tables Database Initialization |  |  |  |
|  | 7 | Wireless Database Initialization |  |  |  |
|  | 8 | System Timer Database Initialization |  |  |  |
|  | 9 | Toll Table Database Initialization |  |  |  |
|  | 10 | LCR Database Initialization |  |  |  |
|  | 11 | Tables Initialization |  |  |  |
|  | 12 | Flexible Button Program Initialization |  |  |  |
|  | 13 | All Database Initialization |  |  |  |
|  | 14 | System Reset By Software |  |  |  |

### TABLE 1.6.35    PRINT PROT DATABASE

| PGM | FLEX | ITEM |  | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
| 451 |  | Print Prot Data |  |  |  |  |
|  | 1 | Flexible Numbering Plan Print |  |  |  |  |
|  | 2 | Station Database Print |  | STN_R |  |  |
|  | 3 | CO Line Database Print |  | CO_R |  |  |
|  | 4 | System Feature Database Print |  |  |  |  |
|  | 5 | Station Group Database Print |  |  |  |  |
|  | 6 | ISDN Tables Database Print |  |  |  |  |
|  | 7 | System Timer Database Print |  |  |  |  |
|  | 8 | Toll Table Database Print |  |  |  |  |
|  | 9 | LCR Database Print |  |  |  |  |
|  | 10 | Other Tables Print |  |  |  |  |
|  | 11 | Nation Specific Database Print |  |  |  |  |
|  | 12 | Flexible Button Program Print |  | STN_R |  |  |
|  | 13 | All Database Print |  |  |  |  |
|  | 14 | LCD Message Print |  |  |  |  |
|  |  | 1 | Language | 00 – 12 | Nation specific | 00:ENG01:KOR02:ITA 03:SWE04:NOR05:FIN 06:DUT07:SPA08:DAN 09:GER10:EST11:RUS12:POR |
|  |  | 2 | Stn Type | 0 – 2 | 0 | 0: NORMAL 1: LG-GAP 2: LARGE |