- FUNCTIONAL SPECIFICATION

- OF

- LDK Hotel

December, 2003

- REVISION HISTORY

| ISSUE | DATE | BY | REMARK |
| --- | --- | --- | --- |
| 0.1A | Jan 31,01 | LDK GRP | Preliminary Release |
| 0.1B | June 19,/02 | LDK GRP | Added LDK-100 |
| 0.1C | Oct 20, 03 | LDK | Added LDK-300 EXP |
| 0.1D | Dec 17. 03 | LDK | Added the nation’s specification |

DISTRIBUTION

| ISSUE | DATE | BY | COUNTRY | COMPANY | ATTN |
| --- | --- | --- | --- | --- | --- |

- TABLE OF CONTENTS

0.	System Capacity & Programming Code	5

0.1	System Capacity	5

0.2	Hotel Programming Code	6

1.	HOTEL FEATURE	7

1.1	ADMIN STATION / ATTENDANTS	8

1.2	DEFAULT BUTTON CONFIGURATION	9

1.3	HOTEL SERVICE STATIONS	10

1.4	REGISTER HOTEL NAME	11

1.5	CHECK-IN / CHANGE ROOM STATUS	12

1.6	CHECK-OUT	30

1.7  ROOM CHARGE DISPLAY & Deleting Service Station’s SMDR Record	32

1.8	PRINT ROOM STATUS through RS232-C	35

1.9	INTERCOM CALL DISABLE / ENABLE	36

1.10	DEFAULT ROOM TO ROOM CALL GROUP	37

1.11	ONE-TIME CO CALL ENABLE	38

1.12	CHANGE LCD DISPLAY LANGUAGE	39

1.13	REGISTER BAR AND MINI-BAR CHARGE (changed)	40

1.14	MAID STATUS	41

1.15	BATH ALARM	43

1.16	ROOM RATE ASSIGN	44

1.17	DEFAULT CALL CHARGE RATE	45

1.18	BABY LISTENING	47

1.19	FEE FOR PART TIME	48

1.20	ADDITIONAL TAX FIELDS	50

1.21	Call Answer Recognition	51

1.22	AUTO INTERNAL SPEED DIRECTORY	53

1.23	MULTI FRONT DESK	55

1.24	DSS/DLS CONSOLES	56

1.25	Set Call Forward	58

1.26	Hotel/Nation Specific Items	59

2. INTERFACE with PROPRIETARY MANAGEMENT SYSTEM	60

3. FLEXIBLE NUMBERING PLAN	74

# 0.	System Capacity & Programming Code

## 0.1	System Capacity

| ITEM | LDK-100 | LDK-300 | LDK-600 |
| --- | --- | --- | --- |
| Hunt Groups | 10 | 24 | 48 |
| Member per each hunt | 32 | 32 | 32 |
| No Of Intercom Group | 100 (00~99) | 100 (00~99) | 100 (00~99) |
| No Of Account Groups | 24 | 50 | 50 |
| No Of Executive/Secretary Pairs | 6 | 18 | 36 |
| No Of Authorization Code | 128 | 300 | 600 |
| No Of System Speed Dial Bins | 100 (200~299) | 600 (200~799) | 600 (200~799) |
| No Of Speed Buffer For Station Speed Dial | 256 | 600 | 1200 |
| No Of Station Speed Dial Bins Per Station | 2 (0,1) | 2 (0,1) | 2 (0,1) |
| No Of System Speed Zones | 10 | 10 | 10 |
| No Of SMDR Records(call, bar record) without MEMU with MEMU | 3000 | 9000 | 20000 |
| No Of Bar Record without MEMU with MEMU | 3000 | 7000 | 15000 |

## 0.2	Hotel Programming Code

1)	Programming Mode entered by pressing [TRANS/PGM] at the Attendant,

2)	Programming Mode Code is not flexible but fixed.

| Entering Hotel PGM | Main Menu | Sub Menu | Sub Menu-2 | Selection | Remark |
| --- | --- | --- | --- | --- | --- |
| [#] | [1] CHECK IN/OUT | [1] CHECK IN |  |  |  |
|  |  | [2] CHECNK OUT |  |  |  |
|  | [2] PRINT CHARGE/STATUS | [1] CHARGE PRINT |  |  |  |
|  |  | [2] STATUS PRINT |  |  |  |
|  | [3] ICM CALL EN/DISABLE | [1] ICM CALL ENABLE |  |  |  |
|  |  | [2] ICM CALL DISABLE |  |  |  |
|  | [4] CALL BLOCK | [1] SET ROOM CALL |  |  |  |
|  |  | [2] DISABLE ROOM CALL |  |  |  |
|  |  | [3] CABINA ENABLE |  |  |  |
|  | [5] ROOM RATE | [1] ROOM RATE REGISTER |  |  |  |
|  |  | [2] ROOM RATE ASSIGN |  |  |  |
|  | [6] CALL RATE | [1] CALL RATE REGISTER |  |  |  |
|  |  | [2] CALL RATE ASSIGN |  |  |  |
|  |  | [3] PART TIME FEE |  |  |  |
|  |  | [4] BAR/MINI-BAR CHARGE |  |  |  |
|  | [7] BATH ALARM | [1] BATH ALARM ENABLE |  |  |  |
|  |  | [2] BATH ALARM DISABLE |  |  |  |
|  | [8] MAID STATUS | [1] MAID STATUS (CLEAN) |  |  |  |
|  |  | [2] MAID STATUS (DIRTY) |  |  |  |
|  | [9] MISCELLANEOUS | [1] HOTEL NAME |  |  |  |
|  |  | [2] SET CALL FORWARD |  |  |  |

# 1.	HOTEL FEATURE

## 1.1	ADMIN STATION / ATTENDANTS

Description

Admin Station (Physical Port 01)

Only Admin Station can program Admin Database. Admin station is a fixed station as Physical Port 01 by default.

Attendants

Digital Hotel System has Maximum 5 main attendants, and there are 2 different type main attendants as follows,

System Attendant - This is the first station of Main Attendant. This station and main attendant can program hotel programming mode features like Check-In, Check-Out, etc. As default, System Attendant is set Station No. 101(Physical Port 02). System Attendant' station No. can be changed by Admin Program ( PGM 164 ).

Main Attendants - There are maximum five Attendant stations by Admin Program (PGM 164), the first station of Main Attendant is System Attendant.

As default, the first Main Attendant (System Attendant) is assigned Station 101, and the others are not assigned.

Condition

- All Main Attendants can access System Attendant Program exclusively.

Programming

System Base	1) Attendant Assignment (PGM164)

2) System Attendant Assignment (PGM 164 – BTN1)

Related Features

Hardware

## 1.2	DEFAULT BUTTON CONFIGURATION

Description

The following features are assigned to the flexible button as default. This can be allowed to only station with 24-button or more. But this feature is allowed to system attendant and multi-front desk (main attendants).

Operation

User can activate these features by just pressing the assigned flexible button.

| Function name | Flexible button number |
| --- | --- |
| Print SMDR with range. | 15th flex button |
| Hotel check in | 16th flex button |
| Hotel Check out | 17th flex button |
| Display Room Charge | 18th flex button |
| Room Status Print Feat | 19th flex button |
| ICM enable Feat | 20th flex button |
| ICM disable Feat | 21st flex button |
| One Time CO Call | 22nd flex button |
| ROOM Type | 23rd flex button |
| ROOM Rate | 24th flex button |

Programming

System Base	1) Attendant Assignment (PGM164)

2) System Attendant Assignment (PGM 164 – BTN1)

Related Features

Hardware

## 1.3	HOTEL SERVICE STATIONS

Description

Any Hotel Service Station can call guest station and other Hotel Service Station, and any Guest Station can call Hotel Service Station, regardless of ICM Call Disable. Even though the COS of Hotel Service Station is 7(For Italy, the Service station’s COS is 2), a guest can make a outgoing CO call at the Hotel Service Station with his/her password or One-Time-CO-Call Enable Feature. Hotel Service Station can register Bar Charges of any guest. In this case bar item each hotel service station manages should be registered at bar terminal’s name.

Operation

To call Hotel Service Station,

Lift handset or press [MON] button and dial Hotel Service Station Number as a normal intercom call.

Condition

- Any Guest Station can call Hotel Service Station, regardless of ICM Call Disable/Room to room call group.

- If Hotel Service Station Name is registered, then Hotel Service station' name will be displayed on LCD instead of Station No. For example, station (A) calls Service Station whose name is "COFFEE SHOP", then LCD message of Station (A) is "CALL TO COFFEE SHOP". If not registered, then LCD display "CALL TO SVC xxxx".

- Any Hotel Service Station can call Guest Station and other Hotel Service Station.

- Every station is room station or service station. There is no station, which is room and service station at the same time.  As default, only Station 101 is registered as hotel service station whose name is "FRONT-DESK" and the other stations are vacant room. To make service station from room station or room station from service station, program ADM PGM 302 (1st button). But system attendant can’t be made room station using ADM PGM 302. But system attendant name is changeable.

- The COS of Hotel Service Station is 7 (Intercom Only) except for Sta 101(Front Desk) whose COS is 1.

- At Hotel Service Station, a guest can make outgoing CO call with password by using ‘WALKING COS’ feature.

Programming

Station Base

1) Hotel Service Station ( PGM 302, Yes for Sys ATD,No for the others)

2) Hotel Service Name ( PGM 302, FRONT-DESK for Sys ATD, NOT_ASG for the others)

3) Bar terminal’s name( PGM 306)

Related Features

Admin/Attendants

Icm Call Disable/Enable

Room to room Call Group

Hardware

## 1.4	REGISTER HOTEL NAME

Description

This feature allows attendant to register or change the hotel’ name.

Operation

To register hotel’s name

| TRANS/ PGM | + #91 + | Hotel Name + | HOLD/ SAVE |
| --- | --- | --- | --- |

- Press [TRANS/PGM] button, and 	Dial "#91" for registering hotel’ name..

- Enter Hotel' Name within 24 characters.

- Press [HOLD/SAVE] button, then system updates database permanently

Condition

- This feature has no default value.

- In check-out or displaying room charge, registered hotel name is printed out in the headings.

- Character Set for hotel name program :

| A | 21 | M | 61 | Y | 93 | . | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B | 22 | N | 62 | Z | 12 | space | *1 |
| C | 23 | O | 63 | 0 | 00 | : | *2 |
| D | 31 | P | 71 | 1 | 10 | , | *3 |
| E | 32 | Q | 11 | 2 | 20 | ( | #1 |
| F | 33 | R | 72 | 3 | 30 | ) | #2 |
| G | 41 | S | 73 | 4 | 40 | - | #3 |
| H | 42 | T | 81 | 5 | 50 |  |  |
| I | 43 | U | 82 | 6 | 60 |  |  |
| J | 51 | V | 83 | 7 | 70 |  |  |
| K | 52 | W | 91 | 8 | 80 |  |  |
| L | 53 | X | 92 | 9 | 90 |  |  |

Programming

System Base

1) Site Name (PGM100 – BTN 2)

Related Features

Admin/Attendants

Icm Call Disable/Enable

Room to room Call Group

Hardware

## 1.5	CHECK-IN / CHANGE ROOM STATUS

Description

This feature allows the System Attendant to check in and change status of rooms.

The room status provides the following information;

1)	Room Number xxxx

- Maid Status ( CLEAN/DIRTY )

3)	Room Occupancy ( If occupied with name, LCD displays Guest Name. if occupied without name LCD displays ".......". If vacant, LCD displays "vacant". )

4)	Station Class Of Service for day and night.

5)	WAKE-UP Time

6)	ICM Enable/Disable

7)	Room To Room Call Group ( If assigned , 00 - 99, otherwise “..” )

- Authorization Code ( If registered, then LCD shows "A", otherwise ".")

- Prepaid Call Charge ( If registered, then LCD shows "P", otherwise ".")

- Call Charge Rate (If registered then registered rate(0-5) is displayed, otherwise “.”)

Operation

| TRANS/PGM | + #11 + | Room No. + ( ....... + ) | SPEED | + Check-In Type + | HOLD/SAVE |
| --- | --- | --- | --- | --- | --- |

1)	Press [TRANS/PGM] button, [TRANS/PGM] button flashes 60 ipm, [MON] LED lights steady.

2)	Dial "#11", then LEDs of all {DSS} button shows the room checked-in status. The room is occupied then the LED of the room is steady ON, and the room is vacant then the LED of the room is OFF.

3)	Dial Room No. or press the associated {DSS} button. Then LED of the associated {DSS} button is ON. For multiple Check-in/Changing Room Status, then Dial Room No. or pressing the associated {DSS} button Successively.

4.1)	Press [HOLD/SAVE] button, then dialed Rooms are checked-in with default value. After that you can start Step 3).  Default values are like followings:

- Guest Name : Not Registered

- Room Day & Night COS :  22

- Wake-Up Time : Not Registered

- ICM Disable 
(For Italy/Denmark/NZ/Spain, the Default is ICM Enable)

- Room To Room Call Group : system attendant assigned No. if assigned  or Not Assigned

- Authorization Code : Not Registered

- Prepaid Money  : 0

- Call Charge Rate :  system attendant assigned No. if assigned  or Not Assigned

4.2) 	To check-in with user defined values, or change Room Status, press [SPEED] button.

LCD shows current status of the (first) entered Room No. In case that a Room No. is selected, LCD displays the status of previous/next Room if pressing [DOWN]/[UP] key. If the (first)  entered Room No. is 100, then LCD shows Room Status as follows;

100 ( CLEAN ) vacant COS22 . . : . . I ( X ) G ( . . ) . . .

5)	Dial Check-In Type ( 1 - 8 ) and enter data successively.

Check-In Types are as follows;

1	Register/Change Guest Name ( within 12 characters )

2	Change Room Day & Night COS (2 digits : range 11-77 )

3	Register/Change Wake-Up Time ( 4 digits :  hh:mm )

4	Set ICM Enable/Disable (Toggle )

5	Set Room To Room Call Group ( 2 digits : range 00-99 )

6	Register/Change Authorization Code ( 5 digits  )

7	Register/Change Prepaid Money ( 6 digits )

8	Register/Change Call Charge Rate(1 digit)

6)	Press [HOLD/SAVE] button. And system updates database permanently and check-in/change status of the rooms and goes to step 3).

Note. If multiple room numbers are entered then the all selected rooms’ status is updated to the currently showing status on LCD  with [HOLD/SAVE] button.

- To Check-In Single(Multiple) Guest(s) with Default;

| - TRANS /PGM | - + #11 + | - Room No. + | - (Room No. + Room No. | - .... + ) | - HOLD/SAVE |
| --- | --- | --- | --- | --- | --- |

- To Check-In Single(Multiple) Guest(s) with User defined Room Status or

- To Change Single(Multiple) Guest(s) with User defined Room Status;

| TRANS/PGM | + #11 + | (Room No.+ Room No. | .... + ) | SPEED | + Check-In + Type | HOLD/SAVE |
| --- | --- | --- | --- | --- | --- | --- |

Condition

1.	When Check in/Change Room Status being processed, all {DSS} button shows Room Status, that is vacant/occupied. If a room is pressed or dialed and then the [SPEED] button is pressed , LCD shows current room status.

2.	If System Attendant presses [HOLD/SAVE] button without any “Check-in type” programming, system checks in  the rooms with default values.

3.	System Attendant can change Guest Room's Status (Guest Name, COS, Wake-Up Time, Authorization Code, ICM Call Status, Room To Room Call Group, Prepaid Money ) by this feature. With Change Room Status processing for the already checked-in rooms, other Databases ( for example, Check-In time, SMDR Record, etc...) are not changed.

4.	When Check-In/Change Guest Status Process, Room Status is printed through RS232-C port, printed format is as follows,

Chk-In

ROOM CHECK_IN GUEST_NAME  COS ICM GRP WAKE AUTH PRE-PAID Call-RT  CHARGE

0100 12/29-18 ............ 11  No  .. ..:..  No        0        .      0

Changed

ROOM CHECK_IN GUEST_NAME  COS ICM GRP WAKE AUTH PRE-PAID Call-RT    CHARGE

0100 12/29-18 C.H. Lee     11 Yes  01 06:00 Yes    20000        1      -20000

Note : Currency Unit depends on Admin Programming. ( PGM 177 )

5.	When Check-In is in progress, SMDR and BAR record of the room is cleared.

6.	If System Attendant check in a room without Guest Authorization Code, then DISA cannot be accessed ( See also Register/Change Authorization Code ).

7.	If a station is registered as Service Station, System Attendant will be received Single Error Tone when checking in the service station.

- The information of Guest Room will be protected.

- The information print of Guest Room can be blocked by ADMIN300, 3rd Button.

Programming

Related Features

Hardware

1.5.1 Register/Change Guest Name

Description

This feature allows the System Attendant to register/change Guest Name. The registered guest name is included in Total Room Charge Bill.

Operation

To Register/Change Guest Name;

| TRANS / PGM | + #11 + | Room No. + ( ..... + ) | SPEED | + 1 + | Guest_Name + | SPEED | + | HOLD / SAVE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

1)	Press [TRANS/PGM] button, and Dial "#11"  for Check-in/Change Room Status.

2)	Dial Room No. or pressing the associated {DSS} button, and press the [SPEED] button.

LCD shows current status of the (first) entered Room No. In case that a Room No. is selected, LCD displays the status of previous/next Room if pressing [DOWN]/[UP] key. If the (first) entered Room number is 100, then LCD shows Room Status as follows;

(ex : not checked in state)

100 ( CLEAN ) VACANT COS22 . . : . . I ( X ) G ( . . ) . . .

(ex : checked in but guest name is not registered)

100 ( CLEAN ) . . . . . . COS22 . . : . . I ( X ) G ( . . ) . . .

3)	Dial '1' for Register/Change Guest Name.

- Enter Guest' Name within 12 characters. If Guest' Name is less than 12 characters, then press the [SPEED] button that means the end of name string. After pressing the [SPEED] button, system updates temporal database and LCD shows temporal updated information of guest station.

(ex : YOUNGHEE )

100 ( CLEAN ) YOUNGHEE COS22 . . : . . I ( X ) G ( . . ) . . .

5)	Press [HOLD/SAVE] button, then system updates database permanently and check-in/change status of the room and goes to step 3) for another check-in/change.

Condition

- Character Set for Guest name program :

| A | 21 | M | 61 | Y | 93 | . | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B | 22 | N | 62 | Z | 12 | space | *1 |
| C | 23 | O | 63 | 0 | 00 | : | *2 |
| D | 31 | P | 71 | 1 | 10 | , | *3 |
| E | 32 | Q | 11 | 2 | 20 | ( | #1 |
| F | 33 | R | 72 | 3 | 30 | ) | #2 |
| G | 41 | S | 73 | 4 | 40 | - | #3 |
| H | 42 | T | 81 | 5 | 50 |  |  |
| I | 43 | U | 82 | 6 | 60 |  |  |
| J | 51 | V | 83 | 7 | 70 |  |  |
| K | 52 | W | 91 | 8 | 80 |  |  |
| L | 53 | X | 92 | 9 | 90 |  |  |

- Guest' name has no default value.

- For this feature, Guest Name Display Bit must be set in ADM PGM 301 / BTN1 as ON.

- At Inter call, registered guest name with station’ number will be displayed at the other party’s LCD. At that time, if guest name is long guest name with its number at LCD may be truncated.

Programming

Hotel Room Attribute (PGM 301, BTN 1)

Related Features

Hardware

1.5.2 Register/Change COS - DIALING PRIVILEGES

Description

This feature allows the application of dialing restrictions through Station Class of Service Assignments. For a particular call, the CO COS is combined with station COS to determine restriction. Reference the Dialing Privileges Feature.

Operation

To Register/Change Guest Station COS;

| TRANS / PGM | + #11 + | Room No. + ( ....... + ) | SPEED | + 2 + | Day & Night COS (1-7, 2digits) | + | HOLD / SAVE |
| --- | --- | --- | --- | --- | --- | --- | --- |

1)	Press [TRANS/PGM] button, and Dial "#11" for Check-in/Change Room Status.

2)	Dial Room No. or pressing the associated {DSS} button, and press the [SPEED] button.

LCD shows current status of the (first) entered Room No. In case that a Room No. is selected, LCD displays the status of previous/next Room if pressing [DOWN]/[UP] key. If the (first) entered Room number is 100, then LCD shows Room Status as follows;

100 ( CLEAN ) . . . . . . COS22 . . : . . I ( X ) G ( . . ) . . .

3)	Dial '2' for Register/Change Room' COS.

- Dial Room's Day and Night COS ( Range 11 - 77 ), LCD shows temporal updated database.(ex : 23)

100 ( CLEAN ) . . . . . . COS23 . . : . . I ( X ) G ( . . ) . . .

5)	Press [HOLD/SAVE] button, And system updates database permanently and check-in/change status of the room, goes to step 3) for another check-in/change.

Condition

1.	As default, In Checking-in Guest Room's day and night COS is 22

2.	If the station's prepaid call flag is set and prepaid money is expired, then the station can't make outgoing CO call even if the station's COS is 1.

Programming

Station Base	   1) Station COS (PGM116, 1 for Sys ATD, 7 for the others)

2) Prepaid Call (PGM112-BTN 8, Yes/No)

3) Class of Room (PGM 304)

4) Room Cost of Rate(PGM 305)

Coline Base		1) Coline COS (PGM141-BTN 2)

System Base		1) Toll Tables (PGM224)

2) Canned Toll Table (PGM225)

3) PABX Access Codes (PGM 172, not assigned)

4) Long Distance Codes (PGM 177-BTN 14, code 1 = 0, 2-5 = not assigned)

Related Features

Prepaid Call

Hardware

1.5.3 Register/Change Wake-Up Time (changed)

Description

This feature allows the System Attendant to register or change the room's wake-up time  on demand of guest.

Operation

To Register/Change Guest Station Wake-Up Time( for one-time);

| TRANS / PGM | + #11 + | Room No. + ( ....... + ) | SPEED | + 3 + | Wake-Up Time + | SPEED | + | HOLD /SAVE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

To Register/Change Guest Station Wake-Up Time(continuously);

| TRANS / PGM | + #11 + | Room No. + ( ....... + ) | SPEED | + 3 + | Wake-Up Time + | # | + | HOLD /SAVE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

1)	Press [TRANS/PGM] button, and Dial "#11" for Check-in/Change Room Status.

2)	Dial Room No. or pressing the associated {DSS} button, and then press the [SPEED] button.

LCD shows current status of the (first) entered Room No. In case that one Room No. is selected, LCD displays the status of previous/next Room if pressing [DOWN]/[UP] key. If the (first)  entered Room No. is 100, then LCD shows Room Status as follows;

100 ( CLEAN ) . . . . . . COS11 . . : . . I ( X ) G ( . . ) . . .

3)	Dial '3' to Register/Change Wake-Up Time.

- Enter Wake-Up Time ( Format HHMM ).  And Speed button for one time wakeup call or # key for permanent wakeup call. And  LCD shows temporal updated database like HH:MM in case of one time wakeup call or HH:MM* in case of permanent wakeup call.

100 ( CLEAN ) . . . . . . COS11 19 : 30 I ( X ) G ( . . ) . . .

100 ( CLEAN ) . . . . . . COS11 19: 30* I ( X ) G ( . . ) . . .

5)	Press [HOLD/SAVE] button. And system permanently updates database of room status changed or registered. For registering or changing another  check-in type, go to step 3)

Condition

- With wake-up signaling and guest lifting handset, the user hear MOH, or VMIB message if VMIB is installed, or system attendant recorded message if VMIB is installed and attendant recorded wake-up message using “ [Trans/PGM] + 06 + 082 “

- 4channels in VMIB or 8 channels in VMIB can be accessed at the same time so in the case more than 4 guests/8 guests try to access VMIB channels, first having tried 4/8 guests hear of VMIB message and the others hear of MOH.

- Ring signal : 30 Secs. On/90 Secs.Off (3 times) After that, the wake up-fail alarm ring will be given System Attendant station. LCD at System Attendant displays wake-up failing station. The wake-up fail alarm can be deleted by dialing the station number or press {DSS} button at system attendant station.

- Time (hh:mm) must be entered Military Format.

- Using “[Trans/PGM] + 41”or [Trans/PGM]+42” , wake-up time can be registered or canceled at each station. In this case, registered form and operation is same as above.

Programming

Related Features

Prepaid Call

Hardware

VMIB for wake-up announcement

1.5.4 Set ICM Call Enable / Disable

Description

This feature disables or enables a guest station to call another  station in the other groups in system.

Operation

To enable/disable ICM Call;

| TRANS / PGM | + #11 + | Room No. + ( ....... + ) | SPEED | + 4 + | HOLD / SAVE |
| --- | --- | --- | --- | --- | --- |

1)	Press [TRANS/PGM] button, and Dial "#11" for Check-in/Change Room Status.

2)	Dial Room No. or pressing the associated {DSS} button, and press the [SPEED] button.

LCD shows current status of the (first) entered Room No. In case that a Room No. is selected, LCD displays the status of previous/next Room if pressing [DOWN]/[UP] key. If the (first)  entered Room No. is 100, then LCD shows Room Status as follows;

100 ( CLEAN ) . . . . . . COS11 . . : . . I ( X ) G ( . . ) . . .

- Dial '4' for Enable/Disable ICM Call, then ICM Call status is changed (toggle setting). If ICM call is enabled then LCD shows like “I(O)”, else ICM call is disabled then LCD shows like “I(X)”.

100 ( CLEAN ) . . . . . . COS11 . . : . . I ( X ) G ( . . ) . . .

100 ( CLEAN ) . . . . . . COS11 . . : . . I ( O ) G ( . . ) . . .

4)	Press [HOLD/SAVE] button, then system updates database permanently and check-in/change status of the room, goes to step 3) for another check-in/change.

Condition

1.	Guest station can call Attendant Station and Service Station even though ICM call is disabled.

2.	After Check-In, ICM Call Status is disabled for Next Guest. As default, Room Station disables to call another station in the system.(For Italy/Denmark/NZ/Spain, the default is ICM call enable)

3.	Guest can call other station in the same Room to room Group even if ICM Call is disabled, that is Room to room Call Group is higher priority than ICM call Enable/Disable.

Programming

Related Features

QSIG Call is also affected by this field.

Hardware

1.5.5 Register/Change Room To Room Call Group (changed)

Description

Stations in the system can be arranged for up to 100 Room To Room Call Groups at System Attendant. Each Room To Room Call Group can contain unlimited number of stations. A guest station can belong only one room to room call group and the station can call the guest station in the same room to room call group.

Operation

To Register/Change Room To Room Call Group;

| TRANS / PGM | + #11 + | Room No. + ( ....... + ) | SPEED | + 5 + | Group No. ( 00 - 99 ) | + | HOLD / SAVE |
| --- | --- | --- | --- | --- | --- | --- | --- |

1)	Press [TRANS/PGM] button, and Dial "#11" for Check-in/Change Room Status.

2)	Dial Room No. or pressing the associated {DSS} button, and press the [SPEED] button.

LCD shows current status of the (first) entered Room No. In case that a Room No. is selected, LCD displays the status of previous/next Room if pressing [DOWN]/[UP] key. If the (first)  entered Room No. is 100, then LCD shows Room Status as follows;

100 ( CLEAN ) . . . . . . COS11 . . : . . I ( X ) G ( . . ) . . .

if default value is assigned for room to room call group for the room 100, default value is displayed (ex: G(03)), otherwise G(..) .

100 ( CLEAN ) . . . . . . COS11 . . : . . I ( X ) G ( 03 ) . . .

3)	Dial '5' for Register/Change Room to Room Call Group.

4)	Enter Room To Room Call Group No ( 00 - 99 ), LCD shows temporal updated database.

5)	Press [HOLD/SAVE] button, And system updates database permanently and check-in/change status of the room and goes to step 3) for another check-in/change.

Condition

- Guest station can call Attendant Station and Service Station even though station is not belonging to the same Room To Room Call Group.

- Default Room to Room Call Group is assigned by Room to Room Call Group ATD PGM feature.

- After Check-Out , Room to room call group defined here is expired and changed with default .

- Guest can call other station in the same Room to room Group even if ICM Call is disabled, that is Room to room Call Group is higher priority than ICM call Enable/Disable.

Programming

Related Features

Room to Room Call Group System ATD PGM Feature.

Hardware

1.5.6 Register/Change Authorization Code

Description

This feature provides a means for verification following three type usage;

1)	To access denied Co Line;

If the Co Line that is set Co Access flag is seized, user should enter his/her Authorization Code for Co Line access verification. If Authorization Code is matched, Co Dial Tone is presented, otherwise he cannot access the Co line and Error-Tone is presented.

2)	To access other Co Line in the system by DISA caller;

If DISA caller seizes other Co Line in the system and DISA access flag is set in accessed DISA line, he/she should enter Authorization Code for access verification.

3)	To access a Co Line in Service Station (Not available for UK);

If user wants to seize a Co Line in service station, he/she should enter Authorization Code for Co Line access verification.

There can be no duplicated Authorization Codes so system attendant can not assign an authorization code to multiple rooms.

Operation

To Register/Change Authorization Code;

| TRANS / PGM | + #11 + | Room No. + ( ** ) | SPEED | + 6 + | Auth. Code ( 5 digits ) | + | HOLD / SAVE |
| --- | --- | --- | --- | --- | --- | --- | --- |

(**)  Can’t do this program for multiple rooms.

1)	Press [TRANS/PGM] button, and Dial "#11" for Check-in/Change Room Status.

2)	Dial Room No. or pressing the associated {DSS} button, LCD shows current status of Room No. In case that a Room No. is selected, LCD displays the status of previous/next Room if pressing [DOWN]/[UP] key. After entering Room No. 100, LCD shows Room Status as follows;

100 ( CLEAN ) . . . . . . COS11 . . : . . I ( X ) G ( . . ) . . .

If Authorization Code is registered, LCD displays "A" like below.

100 ( CLEAN ) . . . . . . COS11 . . : . . I ( X ) G ( . . ) A . .

3)	Dial '6' for Register/Change Authorization Code.

4)	Enter Guest's Authorization Code ( Fixed 5 digits ), LCD shows temporal updated database ("A").

5)	Press [HOLD/SAVE] button, And system updates database permanently and check-in/change status of the room, goes to step 3) for another check-in/change

.

Condition

1.	Co Lines can be marked to deny access until a matched Authorization code is entered. In this case, DND warning tone is provided when the Co Line is accessed. If the dialed Authorization code is verified, he/she will hear CO dial tone. Otherwise, he will hear Error tone and cannot access the CO line.

2.	When DISA is marked for authorization code, if DISA user wants to seize another Co Line in the system, then the caller will hear DND warning tone and he should dial an Authorization Code to access the DISA feature.

3.	When guest wants to seize a Co Line in Service station, then he/she will hear DND warning tone or DISA Announcement and he/she should dial an Authorization Code to access the Co Line. If matches, he can seize the Co Line, otherwise error-tone is heard.

4.	Authorization codes may be entered at any station to access the CO Line.

5.	The total numbers of Authorization Codes in system are 300(LDK-100; 128) entries.

Programming

System Base	        1) Authorization Code (PGM 227)

CoLine Base	        1) DISA Access Flag (PGM141-BTN 3, Yes/No)

2) Co Access Flag (PGM141-BTN 9, Yes/No)

Related Features

Hardware

1.5.7 Register/Change Prepaid Money

Description

This feature allows the guest station to make a phone call in according to his/her prepaid money. If the prepaid money expires during a conversation a warning tone will be given and after Prepaid Call Warning Over Timer the call will be disconnected. The guest room can't make outgoing CO  calls once the prepaid money has expired.

SMDR displays this pre-paid money. The total Room Charge is as follow;

Total Room Charge  =  Total Charge  - Prepaid Money

Operation

To Register/Change Prepaid Money;

| TRANS / PGM | + #11 + | Room No. + ( ....... + ) | SPEED | + 7 + | Prepaid Money | + | HOLD / SAVE |
| --- | --- | --- | --- | --- | --- | --- | --- |

1)	Press [TRANS/PGM] button, and Dial "#11" for Check-in/Change Room Status.

2)	Dial Room No. or pressing the associated {DSS} button, and press the [SPEED] button.

LCD shows current status of the (first) entered Room No. In case that a Room No. is selected, LCD displays the status of previous/next Room if pressing [DOWN]/[UP] key. If the (first)  entered Room No. is 100, then LCD shows Room Status as follows;

100 ( CLEAN ) . . . . . . COS11 . . : . . I ( X ) G ( . . ) . . .

If Prepaid Money is registered, LCD displays "P" for Prepaid Money.

3)	Dial '7' for Register/Change Pre-paid Money.

4)	Enter Prepaid Money ( Fixed 6 digits, or less than 6 digits + [SPEED] key), LCD shows temporal updated database ("P") like this.

100 ( CLEAN ) . . . . . . COS11 . . : . . I ( X ) G ( . . ) .P .

5)	Press [HOLD/SAVE] button, then system updates database permanently and check-in/change status of the room, goes to step 3) for another check-in/change.

Attendant can register or change stations' prepaid money, like followings.

| TRANS / PGM | + 079 + | Room Range + | Prepaid Money + | HOLD / SAVE |
| --- | --- | --- | --- | --- |

Condition

1.	This feature operates with the CO lines that receive call-metering signal.

2.	For correct operation of this feature, following ADMIN must be done.

ADMIN PGM 177, Button 1 (SMDR save) ON or Button 2 (SMDR print) ON

ADMIN PGM 112, Button 8 (Prepaid call flag) ON

ADMIN PGM 142, Button 3 (SMDR metering type of COL is NOT  0)

ADMIN PGM 177 , Button 10 SMDR cost per unit

3.	Prepaid Money is included SMDR information.

4.	Prepaid money' unit and Prepaid money' fraction is programmed by Admin Programming ( PGM 177), that is same as SMDR information.

5.	If the prepaid money has expired , the room extension can't make outgoing CO call.

6.	If the money expires during a conversation a warning tone is given, and after "prepaid call over time" the call will be disconnected automatically.

Programming

Station Base	1) Prepaid Call (PGM112-BTN 8, No/Yes)

Coline Base	1) SMDR metering type (PGM142-BTN 3, 0)

System Base	1) CO Call Drop Warning Timer (PGM 180 – BTN 16 (10/1sec)))

2) SMDR SAVE or SMDR print (PGM177)

3) SMDR cost per unit (PGM 177)

Related Features

Hardware

1.5.8 Assign Call Charge Rate

Description

This feature allows system attendant to assign Call Charge Rate  to rooms in time of check-in.

Operation

To Assign Call Charge Rate

| TRANS / PGM | + #11 + | Room No. + ( ....... + ) | SPEED | + 8 + | Call Charge Rate Bin No + | HOLD / SAVE |
| --- | --- | --- | --- | --- | --- | --- |

1)	Press [TRANS/PGM] button, and Dial "#11" for Check-in/Change Room Status.

2)	Dial Room No. or pressing the associated {DSS} button, and press the [SPEED] button.

LCD shows current status of the (first) entered Room No. In case that a Room No. is selected, LCD displays the status of previous/next Room if pressing [DOWN]/[UP] key. If the (first)  entered Room No. is 100, then LCD shows Room Status as follows;

100 ( CLEAN ) . . . . . . COS11 . . : . . I ( X ) G ( . . ) . . 1

If Call Charge Rate is assigned, LCD displays the assigned rate (ex: 1). Otherwise “.” is displayed

3)	Dial '8' for Assigning/Change Call Charge Rate.

4)	Enter Call Charge Rate Bin No. ( 1 digit ). Or SPD button to erase. And LCD shows temporal updated database .

(ex : SPD button to erase)

100 ( CLEAN ) . . . . . . COS11 . . : . . I ( X ) G ( . . ) . . .

5)	Press [HOLD/SAVE] button. And system updates database permanently. Go to step 3) for another check-in type registering or changing.

Condition

- Supported number of bin is 6 from 0 to 5.

- If Call Charge Rate is not assigned, SMDR call charge is applied.

- Only registered bin no can be used. If not registered bin no is used, error tone is provided.

- Default Call charge rate bin number to room is assigned by Attendant feature code #61.

- After Check-out, call charge rate registered here is expired and changed with default.

Programming

Coline Base	1) SMDR metering type (PGM142-BTN 3, 0)

System Base	1) Register Call Charge Rate (PGM 305)

2) Register SMDR Call Charge Rate ( PGM 177)

Related Features

Hardware

## 1.6	CHECK-OUT

Description

This feature allows the System Attendant to check out guest' room. At this time system attendant can select the method of payment among registered ones according to guest’s payment method.

Operation

To Check-Out Guest Room with pay o f method;

| TRANS/ PGM | + #12 + | Room No. + ( ....... + ) | SPEED | + Bin No + | HOLD/ SAVE |
| --- | --- | --- | --- | --- | --- |

To Check-Out Guest Room without pay of method;

| TRANS/ PGM | + #12 + | Room No. + ( ....... + ) | HOLD/ SAVE |
| --- | --- | --- | --- |

- Press [TRANS/PGM] button. And [TRANS/PGM] button flashes 60 ipm and [MON] LED lights steady. Then Dial Code #12. And confirmation tone is provided. Or if check-out DSS button is programmed, you may press the DSS button instead of [TRRANS/PGM] and Code #12.

- Dial Room No. or press the associated {DSS} button. And LCD shows Room No & the guest name. For multiple Check-out, Dial Room No. or press the associated {DSS} button successively. Then LED of the associated {DSS} button is OFF.

- To select the method of payment press [SPEED] button. Then press bin number of method of payment you want and then press [HOLD/SAVE] button. Then LCD shows the method of payment. To see other registered method of payment, press [DOWN]/[UP] key. Then LCD displays the next registered method.  If  you don’t want to select method of payment you can just press [HOLD/SAVE] button without pressing [SPEED] and bin number.  Then method of payment will not displayed at guest’s bill. After Pressing [HODL/SAVE] button room(s) is(are) checked out and confirmation tone is provided.  Go to step 2) for checking out another room.

Condition

1.	When Check-Out is done by System Attendant, Maid status of the room is changed to "DIRTY ( To be cleaned )". And followings are automatically cleared for the next guest:

Clear Fields

Message Waiting / Guest Name / DND / Wake-Up Time /

Call Forward Status / Authorization Code / Maid Status / Station COS (77) /

Prepaid Money.

(For Australia, don’t cancel CFW No Answer, Busy to VMIB/VM)

Set Fields

ICM Disable / LCD language (go to system attendant's language) /

Room to room Call Group (default Group no defined at ATD PGM #41) /

Call charge rate (default call charge rate assigned at ATD PGM #61)

- When Check-Out Process, Room Total Charge is printed through RS232-C port and printed format is as follows. At this time, if hotel name is registered then hotel name is printed in place of HOTEL NAME. Otherwise, this row is not.  And if you select method of payment it was printed. Otherwise that row is not printed.

| HOTEL NAME |  |  |  |
| --- | --- | --- | --- |
| TOTAL CHARGE IN ROOM 100 ( Guest-Name : ............ ) |  |  |  |
| Check - In : 94/12/26-18 |  |  |  |
| Check - Out : 94/12/31-11 ( 5 days ) |  |  |  |
|  | Start-Time CO Duration Dialed-No. Count Call-Cost Remark |  |  |
|  | 12/27-13:10 01 00:00:32 O00182343507951 0 0 Unanswered |  |  |
|  | 12/27-13:30 01 00:01:23 O00182343507951 3 1500 |  |  |
|  | 12/28-21:22 02 00:10:18 O00182343507953 31 15500 |  |  |
|  | Charge-Time Charged-STA Item Bar-Cost tax |  |  |
|  | 12/28-21:32 COFFEE-SHOP COFFEE 5000 100 |  |  |
|  | 12/29-10:10 FRONT-DESK COKE 3000 30 |  |  |
|  | Item Charge Tax(rate) Sum |  |  |
|  | (1) ROOM CHARGE : 300000 30000(10.00%) 330000 (rate 02 : GOLD) |  |  |
|  | (2) CALL CHARGE : 17000 170(10.00%) 17170 |  |  |
|  | (3) BAR CHARGE : 8000 130 8130 |  |  |
|  | (4) PRE-PAID : -20000 |  |  |
|  | Method of payment : VISA |  |  |
|  | TOTAL : 335300 WON |  |  |

Note : Currency Unit/SMDR Fraction depend on Admin Programming 177

3.    The information print of Guest Room can be blocked by ADMIN 300, 3rd Button.

Programming

System Base		1) SMDR (PGM177)

2) Registering methods of payment (PGM 300-BTN 5)

3) CHK-IN/OUT On-Line Print (PGM 300-BTN 3)

Related Features

Register Hotel Name.

Hardware

## 1.7  ROOM CHARGE DISPLAY & Deleting Service Station’s SMDR Record

Description

System Attendant can see Total Charge of a single room or multiple room on LCD display, and can print SMDR Records of that/those rooms through RS232-C port or delete service station’s SMDR Record.

Operation

To display Room Charge Information (total charge only);

| TRANS/ PGM | + #21 + | Station No. + ( ....... + ) | HOLD/ SAVE |
| --- | --- | --- | --- |

To display Room Charge Information (for detail information);

| TRANS/ PGM | + #21 + | Station No. + ( ....... + ) | # + | HOLD/ SAVE |
| --- | --- | --- | --- | --- |

1)	Press [TRANS/PGM] button. And [TRANS/PGM] button flashes 60 ipm and [MON] LED lights steady.

2)	Dial Code #21. And confirmation tone is provided.

3)	Dial Room No. And LCD shows Total Charge of that room, if entering other Room No, then LCD display Total Charge Summation of those rooms. ( In this case, system cannot support Room Range).

4)	To print total cost, just press [HOLD/SAVE] button. To print detail room charge, dial Code '#' and then press [HOLD/SAVE] button. And system prints detail room charge of rooms through RS232-C port.  Here, Code # is a toggle key between printing total cost and printing detailed room charge and default is a total cost displaying.

To delete Service Station’s SMDR record. :

| TRANS/ PGM | + #21 + | Service Station No. + ( ....... + ) | * + | HOLD/ SAVE |
| --- | --- | --- | --- | --- |

- 2) same as above.

- Dial Service station’s number.  And LCD show Total Charge of that room, if entering other Service station number, then LCD display summation of Total Charge of those stations

4)  Dial code '*' and press [HOLD/SAVE] button. And system deletes detail call charge and go to step 3).

Printing Format

1)	Detailed Information Print Format (ex. room 100 , 102);

| HOTEL NAME |  |  |  |
| --- | --- | --- | --- |
| TOTAL CHARGE IN ROOM 100 ( Guest-Name : ............ ) |  |  |  |
| Check - In : 94/12/26-18 |  |  |  |
| Current-time : 94/12/31-11 ( 5 days ) |  |  |  |
|  | Start-Time CO Duration Dialed-No. Count Call-Cost Remark |  |  |
|  | 12/27-13:10 01 00:00:32 o00182343507951 0 0 Unanswered |  |  |
|  | 12 / 27 - 13: 30 01 00:01:23 O00182343507951 3 1500 |  |  |
|  | 12/28-21:22 02 00:10:18 O00182343507953 31 15500 |  |  |
|  | Charge-Time Charged-STA Item Bar-Cost tax |  |  |
|  | 12/28-21:32 COFFEE-SHOP COFFEE 5000 100 |  |  |
|  | 12/29-10:10 FRONT-DESK COKE 3000 30 |  |  |
|  | Item Charge Tax(rate) Sum |  |  |
|  | (1) ROOM CHARGE : 300000 30000(10.00%) 330000 (rate 02 : GOLD) |  |  |
|  | (2) CALL CHARGE : 17000 170(10.00%) 17170 |  |  |
|  | (3) BAR CHARGE : 8000 130 8130 |  |  |
|  | (4) PRE-PAID : -20000 |  |  |
|  | Method of payment : VISA |  |  |
|  | TOTAL : 335300 WON |  |  |

Note : Currency Unit/SMDR Fraction depend on Admin Programming 177

| HOTEL NAME |  |  |  |
| --- | --- | --- | --- |
| TOTAL CHARGE IN ROOM 102 ( Guest-Name : ............ ) |  |  |  |
| Check - In : 94/12/26-18 |  |  |  |
| Current-time : 94/12/31-11 ( 5 days ) |  |  |  |
|  | Start-Time CO Duration Dialed-No. Count Call-Cost Remark |  |  |
|  | 12/27-13:10 01 00:00:32 o00182343507951 0 0 Unanswered |  |  |
|  | 12/27-13:30 01 00:01:23 O00182343507951 3 1500 |  |  |
|  | 12/28-21:22 02 00:10:18 O00182343507953 31 15500 |  |  |
|  | Charge-Time Charged-STA Item Bar-Cost tax |  |  |
|  | 12/28-21:32 COFFEE-SHOP COFFEE 5000 100 |  |  |
|  | 12/29-10:10 FRONT-DESK COKE 3000 30 |  |  |
|  | Item Charge Tax(rate) Sum |  |  |
|  | (1) ROOM CHARGE : 300000 30000(10.00%) 330000 (rate 02 : GOLD) |  |  |
|  | (2) CALL CHARGE : 17000 170(10.00%) 17170 |  |  |
|  | (3) BAR CHARGE : 8000 130 8130 |  |  |
|  | (4) PRE-PAID : -20000 |  |  |
|  | Method of payment : VISA |  |  |
|  | TOTAL : 35300 WON |  |  |

Note : Currency Unit/SMDR Fraction depend on Admin Programming 177

2)	 Total Charge Print Format ;

- Echo Mode

RM(0100) PRE-PAID (    20000)   CHARGED (     53000) =      330000

RM(0102) PRE-PAID (    20000)   CHARGED (     14600 ) = -     5400

RM_SUMS: PRE-PAID (    40000)   CHARGED (     67600) =       27600

- Non-Echo Mode

- &SM000100   20000     53000     330000&SM000102   20000     14600-      5400

- &SM01   40000     67600     27600

- cf.) In Non-Echo Mode &SM00 means the string is about station and &SM01 means total information.

3)	Detailed Information Print Format for service station (ex. Service station 101);

| HOTEL NAME |  |  |  |
| --- | --- | --- | --- |
| TOTAL CHARGE IN SERVICE STATION 101 (Front-desk ) |  |  |  |
| Check - In : 94/12/26-18 |  |  |  |
| Current-time : 94/12/31-11 ( 5 days ) |  |  |  |
|  | Start-Time CO Duration Dialed-No. Count Call-Cost Remark |  |  |
|  | 12/27-13:10 01 00:00:32 o00182343507951 0 0 Unanswered |  |  |
|  | 12 / 27 - 13: 30 01 00:01:23 O00182343507951 3 1500 |  |  |
|  | 12/28-21:22 02 00:10:18 O00182343507953 31 15500 |  |  |
|  | Item Charge Tax(rate) Sum |  |  |
|  | (1) CALL CHARGE : 17000 170(10.00%) 17170 |  |  |
|  | (2) PRE-PAID : - 0 |  |  |
|  | TOTAL : 17170 WON |  |  |

Note : Currency Unit/SMDR Fraction depend on Admin Programming 177

Condition

- Total number of SMDR Records is 9000(LDK-100; 3000) entries. And total number of Bar Records is 7000 (LDK-100; 3000) entries.

- If Available SMDR records are 200 entries, then System ATD receives SMDR Full Warning Tone, (1st Warning ), if that is 150, 2nd Warning is presented ( 100 - 3rd Warning, 50 - 4th Warning ). Finally there is no available SMDR record, then system cannot record further SMDR information and Co Line access is denied

- If Available BAR records are 200 entries, then System ATD receives BAR Full Warning Tone, (1st Warning ), if that is 150, 2nd Warning is presented ( 100 - 3rd Warning, 50 - 4th Warning ). Finally there is no available BAR record, then system cannot record further BAR information and registering bar item at service station is denied.

- SMDR and BAR Detailed Record and total Room Charge for the room is deleted in check-in of room for the next guest.

- After printing, system sends a Form Feed.

- The information print of Guest Room can be blocked by ADMIN 300, 3rd Button.

Programming

System Base   1) CHK-IN/OUT On-Line Print (PGM300-BTN 3)

Related Features

Hardware

## 1.8	PRINT ROOM STATUS through RS232-C

Description

This feature allows printing room status through RS232-C.

Operation

To print Room Status;

| TRANS/ PGM | + #22 + | Room Range + | HOLD/ SAVE |
| --- | --- | --- | --- |

1)	Press [TRANS/PGM] button. And [TRANS/PGM] LED flashes 60 ipm and [MON] LED lights steady.

2)	Dial Code #22. And confirmation tone is provided,

3)	Dial Room range and press [HOLD/SAVE] button. And confirmation tone is provided,

Condition

1.	Service Stations within Room Ranges are not printed.

2.	If Room is vacant, then only Maid Status is printed for the room.

3.	Room Status is printed through RS232-C port and printed format is as follows,

Status

S  ROOM CHECK_IN GUEST_NAME  COS  ICM GRP WAKE AUTH PRE-PAID  C-RT   CHARGE

C   0100  12/29-23     C.H. Kim       1   Yes  ..    ..:..    No     20000      .       12345

C   0102  12/25-18     C.B. Choi       1   No   ..   07:00  No         0      1      500000

C   0103  12/29-15     I.S. Lee         1   No   ..   08:00  Yes        0      2         500

D   0104  Vacant

C   0105  Vacant

- S means maid status and C clean and D dirty.

- C-RT means call charge rate.

- PRE-PAID and CHAGE use the SMDR fraction which is registered ADM PGM.

4.    The information print of Guest Room can be blocked by ADMIN 300, 3rd Button.

Programming

System Base	1) SMDR (PGM 177)

2) CHK-IN/OUT On-Line Print (PGM 300-BTN 3)

Related Features

Check-In / Check-Out

Hardware

## 1.9	INTERCOM CALL DISABLE / ENABLE

Description

This feature disables or enables a guest station to call the other all stations in the system.

Operation

To enable ICM Call;

| TRANS/PGM | + #31 + | Room Range + | HOLD/SAVE |
| --- | --- | --- | --- |

To disable ICM Call;

| TRANS/PGM | + #32 + | Room Range. + | HOLD/SAVE |
| --- | --- | --- | --- |

Condition

1.	Guest station can call Attendant Station and Service Station even though ICM call is disabled.

2.	After Check-In, ICM Call Status is disabled for Next Guest. As default, Room Station disables to call another station in the system.(For Italy/Denmark/NZ/Spain, the default is ICM call enable)

- Guest can call other station in the same Room to room Group even if ICM Call is disabled, that is Room to room Call Group is higher priority than ICM call Enable/Disable.

Programming

Related Features

Room to Room Call Group

Attendant Station

Hotel Service Station

Hardware

## 1.10	DEFAULT ROOM TO ROOM CALL GROUP

Description

This feature allows system attendant to assign or change Room to Room Call Group among stations. Stations in the system can be arranged for up to 200 Room To Room Call Groups at System Attendant. Each Room to Room Call Group can contains unlimited number of stations. A guest station can belong to only one room to room call group and the station can call the guest station in the same room to room call group.

Operation

To set Room To Room Call Group;

| TRANS/PGM | + #41 + | Grp No. + (00~99) | Room Range + | HOLD/SAVE |
| --- | --- | --- | --- | --- |

To erase Room To Room Call Group;

| TRANS/PGM | + #42 + | Room Range + | HOLD/SAVE |
| --- | --- | --- | --- |

Condition

- You can see group member about the specific group number using setting room to room call group feature. After TRANS/PGM, Code #41 and Grp number, LCD displays members of that group. At this time Volume UP/DOWN key is used to next members of that group.

- Changed Room to Room call group using this feature does not effect on rooms occupied now but after check-out of the rooms.

- Guest station can call Attendant Station and Service Station whether station is belonging to Room To Room Call Group or not.

- After Check-Out , Room to room call group is changed to room to room call group defined in this feature. So, when check-in, Room to Room Call Group of the room is one defined here.

- As default, Room to room call group is not assigned.

- Guest can call other station in the same Room to room Group even if ICM Call is disabled, that is Room to room Call Group is higher priority than ICM call Enable/Disable.

Programming

Related Features

Checck-In

Hardware

## 1.11	ONE-TIME CO CALL ENABLE

Description

By guest’s request, System Attendants can enable one-time CO call to Intercom Only Station. In this time calling station's COS is temporally changed Charged Station's COS. Guest can make only one external call for one request. After the Co Line Call, calling Information is printed through RS-232.

Operation

To enable ICM Only station to dial outside calls.

| TRANS/PGM | + #43 + | Calling Sta. No. + | Charged Sta. No. + | HOLD/SAVE |
| --- | --- | --- | --- | --- |

1)	Press [TRANS/PGM] button. And [TRANS/PGM] button flashes 60 ipm and [MON] LED lights steady.

2)	Dial Code #43. And confirmation tone is provided.

3)	Dial Calling Station No and then Charged Station No.

4)	Press [HOLD/SAVE] button, confirmation tone is provided and one-time Co Line Call is activated at the calling sta.

Condition

- Charged station must be Checked-In Room Station.

- At the end of the call, the station can not access Co Line.

- Calling station's COS is changed Charged Station's COS temporally.

- Charged Sta pays for this call charge. And

- Print Format is as follows. Sta field means charged station and Remark field calling sta.

(ex: calling sta:112,  charged sta:104)

6.	Cost is calculated using charged sta’s call charge rate.

No   STA CO    TIME         START           DIALED       COUNT       COST       Remark

0001  104 01 00:01:23 12/29/94 08:16  O001034507950     3          500        R 0112

Programming

Related Features

Hardware

## 1.12	CHANGE LCD DISPLAY LANGUAGE

Description

This feature enables to change the LCD Language of any key station by attendant. System supports 15 different  LCD Languages.

Operation

To set the LCD Language type ( By Attendant )

| TRANS/PGM | + 07* + | Start Room No. + | End Room No. + | Type + | HOLD/SAVE |
| --- | --- | --- | --- | --- | --- |

Condition

- Default LCD Language depends on Nation Code in Admin Programming.

- Language Type is as follows;

- 00 : English         01 : Italian          02 : Finnish

- 03 : Dutch          04 : Swedish         05 : Danish

- 06 : Norwegian      07 : Hebrew         08 : Germany

- 09 : French         10 : Portuguese       11 : Spanish

- 12 : Korean         13 : Estonia          14 : Russian

- Attendant station is allowed to program variable LCD Display Language. For the Other station, PGM 71 operates LCD Language Toggle Changing Feature (English / Domestic).

- When Check-Out, the room's LCD language goes to the same language as FRONT DESK LCD language.

Programming

Related Features

Hardware

## 1.13	REGISTER BAR AND MINI-BAR CHARGE (changed)

Description

Guest may pay for charge at service station or mini-bar in time of check-out. Bar terminal  operator registers guest’s charge using the following procedure. Mini-bar charge of each room is registered by system attendant (Front desk service station) and service station. Registered Bar or Mini-bar Charge is stored in BAR record in system and will be displayed or printed in time of Hotel Charge Display and Hotel check-out .

Operation

To Register Bar Charge at Service station or Mini-Bar Charge at attendant;

| TRANS /PGM | + #64 + | Room No. + | Product Code + | (Mini) Bar Cost + | HOLD /SAVE |
| --- | --- | --- | --- | --- | --- |

To Register Bar Charge at Service station or Mini-Bar Charge at Service station;

| TRANS /PGM | + 79 + | Room No. + | Product Code + | (Mini) Bar Cost + | HOLD /SAVE |
| --- | --- | --- | --- | --- | --- |

- Press [TRANS/PGM] button. And [TRANS/PGM] button flashes 60 ipm and [MON]LED lights steady.

2)	Dial Code #64 (or 79), confirmation tone is provided,

3)	Dial Room No, Product code(2digits) and (Mini-)Bar Cost ( within 6 digits ). And LCD shows Room No, (Mini-)Bar Cost, Item's Name of Bar Cost. If (Mini-)Bar Cost is less than 6 digits, user must press the [SPEED] button which means the end of (Mini-)Bar Charge.

4)	Press [HOLD/SAVE] button. And  (Mini-)Bar Cost is registered and  SMDR Information for that (Mini-)Bar Cost  is printed through RS-232C.

5)	Before [HOLD/SAVE] button, you can toggle plus cost or minus cost by pressing [SPEED] button.

Condition

1.	System sends the following information to the printer in order;

charging station number, charged date/time,  FRONT-DESK in case of mini-bar or service station’s name for the other charge, product name and cost. Printing format is as follows.

//0100   12/30-22:12    MAIN-BAR	 BEER		50000

- This bar and mini-bar cost will be included in total room charge.

- The unit of cost and fraction are the same as SMDR currency unit and SMDR fraction.

- Each service station and mini-bar’s product codes must be registered by PGM 306 before.

Programming

System Base  1) Bar terminal product Name (PGM 306)

Related Features

Hardware

## 1.14	MAID STATUS

Description

This feature allows Maid or Attendant to update the maid status of room. At this time-updated maid status information is printed through RS-232C. Front-Desk can also see the maid status with room status feature.

Room status will be one of followings;

To Be Cleaned 		( = DIRTY )

Cleaned	  	( = Ready For Guest )

Operation

To register "Cleaned" at a guest station by maid;

| TRANS/PGM | + 7* |
| --- | --- |

- Press [TRANS/PGM] button. And  [TRANS/PGM] button flashes 60 ipm and [MON] LED lights steady.

- Dial Code 7*. And Confirmation Tone is received and changed maid status of the room is printed through RS-232C.

To register "Cleaned" by attendant;

| TRANS/PGM | + #81 + | Room Range + | HOLD/SAVE |
| --- | --- | --- | --- |

1)	Press [TRANS/PGM] button. And  [TRANS/PGM] button flashes 60 ipm and  [MON] LED lights steady.

- Dial Code #81. And Confirmation Tone is received.

- Dial Room range. And Confirmation Tone is received.

- Press [HOLD/SAVE] button. And rooms’ maid status is changed clean and confirmation tone is provided and changed maid status of rooms is printed through RS-232C.

To register "Dirty ( To Be Cleaned )" at a guest station by maid;

| TRANS/PGM | + 7# |
| --- | --- |

1)	Press [TRANS/PGM] button. And [TRANS/PGM] button flashes 60 ipm and [MON] LED lights steady.

- Dial Code 7#. And Confirmation Tone is received and changed maid status of the room is printed through RS-232C.

To register "Dirty ( To Be Cleaned )" by attendant;

| TRANS/PGM | + #82 + | Room Range + | HOLD/SAVE |
| --- | --- | --- | --- |

1)	Press [TRANS/PGM] button. And  [TRANS/PGM] button flashes 60 ipm and [MON] LED lights steady.

- Dial Code #82. And Confirmation Tone is received.

- Dial Room No range. And Confirmation Tone is received.

- Press [HOLD/SAVE] button. And rooms’ maid status is changed dirty and confirmation tone is provided and changed maid status of rooms is printed through RS-232C.

Condition

- Maid status is automatically set "Dirty ( To be cleaned )" after check-out has been done by System Attendant.

- To see specific one room’s maid status, use check-in feature and to print and see rooms’ maid status use printing room status feature.

- Format of printed maid status of rooms through RS-232Cis as follows.

- // 0100  Clean   01/12/29-14:20

- // 0102  Clean   01/12/29-14:20

- // 0103  Clean   01/12/29-14:20

Programming

Related Features

Hardware

## 1.15	BATH ALARM

Description

In case station with bath alarm device lifts handset and no other action is taken for pre-defined time (Dial-Tone Timer + Bath Alarm Timer), Alarm Ring and LCD Indication is presented to System Attendant Station. System Attendant can cancel Bath Alarm Ring by dialing Station No. ({DSS} button), but alarm condition is still active, Bath alarm ring is not removed.

Operation

To set Bath Alarm Device by attendant;

| TRANS/PGM | + #71 + | Room Range + | HOLD/SAVE |
| --- | --- | --- | --- |

To disable Bath Alarm Device by attendant;

| TRANS/PGM | + #72 + | Room Range + | HOLD/SAVE |
| --- | --- | --- | --- |

To reset Bath Alarm Ring;

1)	Lift handset or press [MON] button,

2)	Dial Station No. or press associated {DSS} button at system Attendant station, if Bath Alarm Condition is already cleared, then Bath Alarm ring is removed, otherwise Bath Alarm Ring is still presented.

Condition

- Bath Alarm Call (Ring) is generated at only the station with Bath Alarm Device, which is registered at ADM PGM 301 or by system attendant.

- The Bath Alarm Call (Ring) cannot be forwarded as a normal call.

- System Attendant can cancel Bath Alarm Ring, but if Bath Alarm Condition is still active, Bath Alarm Ring is not removed. And Final Reset is possible only if alarm condition have been cleared.

- When Bath Alarm happens and is erased by ATD, the message with time is printed through RS-232C.

Programming

1)  Bath Alarm Timer (PGM 300-BTN 1, 05 (01-20/Sec)

2)  Bath Alarm Device(PGM 301-BTN 2, ON/OFF)

Related Features

When Warm Line is set, and the warm line destination is Group or Station, the Emergency sign is printed on a terminal through RS-232C.

Caution) PGM 181-BTN 7 (Internal Dial tone Timer) have to be greater than PGM 182-BTN 8 (Warm Line timer), Associate Feature (PGM 113-BTN 7 Warm Line, PGM 122 Idle Line Set).

This function is implemented for ITALY.

Hardware

## 1.16	ROOM RATE ASSIGN

Description

This feature allows the system attendant to assign a room rate among 20 different ones to individual rooms. This program is also possible at the Admin. Program. This information is included in the Total Room Charge Bill that printed out on the SMDR upon request and on Check-Out.

Operation

Room Type and Rate program  at the attendant (all FRONT-DESK);

| TRANS/ PGM | + #51 + | Room Rate Bin No. + | Room cost and Name(with Flex Btn) + | HOLD/ SAVE |
| --- | --- | --- | --- | --- |

1)	Press [TRANS/PGM] button. And [TRANS/PGM] button flashes 60 ipm and [MON] LED lights steady.

2)	Dial Code #51. And confirmation tone is provided and LCD displays the room cost and room type name of the first room rate bin.

RATE (00) 0000000 ......

3)	If you want to program another Room Rate Bin No. not currently displayed , then Dial the Room Rate Bin No.  LCD displays the selected Room Rate bin’s room cost and room type name.

4)   To assign room cost then press the FLEX button 1 and Dial room cost (7 digits).

To assign room type name then press the FLEX button 2 and Enter Room Type name (Max. 6 characters).

5)	Press [HOLD/SAVE] button. And system updates database permanently. Go to step 3) for another Room Rate Bin programming.

To assign Room rate to rooms;

| TRANS/ PGM | + #52 + | Room Range + | Room Rate Bin No. + | HOLD/ SAVE |
| --- | --- | --- | --- | --- |

1)	Press [TRANS/PGM] button. And  [TRANS/PGM] button flashes 60 ipm and [MON] LED lights steady.

2)	Dial Code #52. And confirmation tone is provided.

3)	Enter Room range. LCD displays currently assigned Room Rate Bin No. of the first room of selected range. For example, if selected rooms are 100 - 105, LCD displays like :

100 - 105 00 : 0020000 SINGLE

4)   To assign Room Rate Bin, dial Bin No. (00-19).

5)	Press [HOLD/SAVE] button, then system updates database permanently. Go to step 3) for another Room Rate Assigning.

Condition

Programming

1)  Room Cost and Name for Rate  (PGM 304)

2)  Room Class Assign (PGM 303)

Related Features

Hardware

## 1.17	DEFAULT CALL CHARGE RATE

Description

This feature allows system attendant to assign Call Charge Rate to individual room or service station in order to charge different call charge. To use this feature, at first the system attendant must register Call Charge Rate Bin and then assign Call Charge Rate to individual rooms. Call Charge Rate means percentage (call charge rate) of SMDR call charge

Operation

Register and change Call Charge Rate at the attendant (all FRONT-DESK);

| TRANS / PGM | + #61 + | Call Charge Rate Bin No. | + Call Charge Rate and Type + | HOLD / SAVE |
| --- | --- | --- | --- | --- |

1)	Press [TRANS/PGM] button. And [TRANS/PGM] button flashes 60 ipm and [MON] LED lights steady.

2)	Dial Code #61. And confirmation tone is provided and LCD displays the Call Charge Rate and Call Charge Rate type of the first bin.

CALL CHARGE RATE ( 0 ) 000 % . . . . .

3)	If you want to program another Call Charge Rate Bin No. currently not displayed, then dial the Call Charge Rate Bin No. And LCD displays the selected bin’s Call Charge Rate and its type if registered.

4)   To register Call Charge Rate press the FLEX button 1 and Dial Call Charge Rate (3 digits). To erase, type [SPEED] button instead of dialing.

To register Call Charge Rate type press the FLEX button 2 and Enter type name (Max. 6 characters). To erase, type [SPEED] button instead of dialing.

5)	Press [HOLD/SAVE] button. And system updates database permanently. Go to step 3) for another Call Charge Rate Bin programming.

To assign Call Charge Rate to rooms at the attendant (all FRONT-DESK);

| TRANS / PGM | + #62 + | Room Range | + Call Charge Rate Bin No. + | HOLD / SAVE |
| --- | --- | --- | --- | --- |

1)	Press [TRANS/PGM] button. And [TRANS/PGM] button flashes 60 ipm and [MON] LED lights steady.

2)	Dial Code #62. And confirmation tone is provided.

3)	Enter Room range. LCD displays currently assigned Call Charge Rate Bin No. of first room of selected range. For example, if selected rooms are 100 - 105, LCD displays like :

100 - 105 0 : 090 % HOLIDA

4)   To assign Call Charge Rate Bin, dial Bin No. (0-5). To erase, type [SPEED] button instead of dialing. If dialed bin no is not registered in Call Charge Rate Table yet, error tone is provided.

5)	Press [HOLD/SAVE] button, then system updates database permanently. Go to step 3) for another Call Charge Rate Assigning.

Condition

- Supported Call Charge Rates are up to 6 (from 0 to 5).

- If a room or service station is not assigned Call Charge Rate then 100 percentage of SMDR call charge is charged.

- Registration of Call Charge Rate type is optional.

Programming

PGM 305

PGM 177

Related Features

Hardware

## 1.18	BABY LISTENING

Description

This feature allows a guest to monitor his room at the other place of hotel. In other words, you can hear of room which is set baby listening but the side of the room can not hear of you.

Operation

To activate baby listening feature at guest’s room;

| OFF-HOOK | + Dial guest’s own room No. |
| --- | --- |

- Do off-hook.

- Dial guest’s own room number. And confirmation tone is provided.

To cancel  baby listening feature at guest’s room;

- Goes on-hook.

To operate baby listening feature at the other places;

| OFF-HOOK | + Dial guest’s own room No. | + Dial guest’s own room No. |
| --- | --- | --- |

- Do off-hook .

- Dial guest’s own room #. And DND tone is provided.

- Dial guest’s own room # again. Then monitoring guest’s room allowed.

Condition

- Baby listening feature is set at the only occupied guest room.

- The other features are processed the same as called party is DND state.

- While this feature is activating, calling party to this station hears of DND state tone.

- Baby listening is not provided to ICM box and Gap Terminal.

- Baby listening is available for the only intercom.

Programming

Related Features

Hardware

## 1.19	FEE FOR PART TIME

Description

In case the day of check-in is the same day of check-out, part time fee may be charged according to room type of checked in room or hotel policy. Each room type may have up to 6 fields for different part time range and fee. There are 32 fields to program part time range and fee in entire hotel system.

e.g.  Room type table (ADM PGM 304)

| room type | Charge | part time bins |
| --- | --- | --- |
| . . GOLD . . . | . . 100$ . . . | . . 01, 03 . . . |

Part time table (ADM PGM 308)

| Bin | part time range | rate |
| --- | --- | --- |
| 00 01 | . 00 – 03 hours | . 50 % |
| 02 03 | 04 – 12 hours | 80 % |
| 04 . . | 13 – 15 hours . . | 90 % . . |

If type of room 100 is GOLD and the room is used for 3 hours then $50 will be charged for the room.  But for 13 hours $100 will be charged because of room type GOLD has no part time bin 04.

Operation

Register and change fee for part time table at the attendant (all FRONT-DESK);

| TRANS / PGM | + #63 + | Fee for part time Bin No. | + Time Range and Rate(with Flex Btn) + | HOLD / SAVE |
| --- | --- | --- | --- | --- |

1)	Press [TRANS/PGM] button. And [TRANS/PGM] button flashes 60 ipm and [MON] LED lights steady.

2)	Dial Code #63. And confirmation tone is provided and LCD displays the time range of staying at hotel and fee for that range of first bin.

RATE ( 00 ) 01 - 03 : 050 %

3)	If you want to program another Bin No. not currently displayed, then Dial the Room Rate Bin No.  And LCD displays the selected bin’s  (00 – 31)

4)   To register time range of staying at hotel,  press the FLEX button 1 and Dial 4 digits. To erase, type [SPEED] button instead of dialing.

To assign charge rate for that part time, press the FLEX button 2 and Enter fixed 3 digits. The range of the rate is from 000 to 100.

5)	Press [HOLD/SAVE] button. And system updates database permanently. Go to step 3) for another Fee for Part Time Bin programming.

When check-out if part time fee is applied guest bill is as follows:

| HOTEL NAME |  |  |  |
| --- | --- | --- | --- |
| TOTAL CHARGE IN ROOM 100 ( Guest-Name : ............ ) |  |  |  |
| Check - In : 94/12/26-13 |  |  |  |
| Check - Out : 94/12/26-20 ( 7 hours ) |  |  |  |
|  | Start-Time CO Duration Dialed-No. Count Call-Cost Remark |  |  |
|  | 12/26-13:10 01 00:00:32 O00182343507951 1 0 Unanswered |  |  |
|  | 12/27-13:30 01 00:01:23 O00182343507951 3 150 |  |  |
|  | Charge-Time Charged-STA Item Bar-Cost tax |  |  |
|  | 12/26-19:32 COFFEE-SHOP COFFEE 500 10 |  |  |
|  | Item Charge Tax(rate) Sum |  |  |
|  | (1) ROOM CHARGE : 80 8(10.00%) 88 (rate 02 : GOLD) |  |  |
|  | (2) CALL CHARGE : 150 15(10.00%) 165 |  |  |
|  | (3) BAR CHARGE : 500 10 510 |  |  |
|  | (4) PRE-PAID : -200 |  |  |
|  | Method of payment : VISA |  |  |
|  | TOTAL : $ 475 |  |  |

Note : Currency Unit/SMDR Fraction depend on Admin Programming 177

Condition

- Part time range and fee in the Part time table is also programmed by ADM PGM 308.

- 32 bins in part time table (00 ~ 31) are supported

- Fee of the part time registered is percent of the room cost.

- Part Time table has no default value.

- Each room type may have up to 6 indexes for Part time table bin by ADM PGM 304.

- If a room type has no index for part time table bin or if any bin among registered index of bins is   not matched with staying time at hotel, then room of the type is not applied with part time fee.

- Each room type has no default value for index for part time table bin.

- Lower number is first referenced among registered index of part time table bins. In other word, the different bins include same time; lowest bin will be applied with part time fee.

- Time for staying at hotel is the passed time between check in and check out, when only hour is meaning and minute is ignored

- If any bin for part time fee is not registered then one-day charge is applied.

- If there is related bin for specific staying time range, that bin’s fee is applied for room charge.

- If there is not related bin for specific staying time range, one-day charge is applied.

Programming

PGM 304

PGM 308

Related Features

Hardware

## 1.20	ADDITIONAL TAX FIELDS

Description

Hotel System is able to cater for different tax fields for different rates of TAX

e.g.,     Tax 0 = 10.00 %

Tax 1 = 20.00 %

Tax 2 = 0 %

. . . .

Each bar item that is registered in the bar terminal table (ADM PGM 306) should have a flag for which type of tax will be applied to it (either TAX 0, TAX 1 or TAX 2...)

Tax rates for call charge and room charge is the first tax rate bin, Tax 0.

Condition

- Tax for he telephone and room charge will be charged using TAX 0 rate

- There are 5 programmable tax rates fields from field 0 to field 4 which are registered at the ADM PGM 307.

- Every tax rates has 00.00 % default value and may have the value from 00.00 % to 99.99%

- Each bar item may have only one index of tax rate for the bar item tax charge by registering at ADM PGM 306 and available index is from 0 to 4

- The index of tax rate in bar item is assigned with Tax 0 as default.

- Tax charge of each item is integer part of the result that is calculated by the following equation.

- ( Equation :  charge * tax rate* 100  /10000)

Programming

Related Features

Hardware

## 1.21	Call Answer Recognition

Description

If ISDN line is used for outgoing calls then it is required that a called party answer signal is recognized by the System and flagged on the guests bill and call logging SMDR output

Operation

When a call is made from a guest station and the called party does not answer the call will be displayed on the guest’s bill as follows:

| HOTEL NAME |  |  |  |
| --- | --- | --- | --- |
| TOTAL CHARGE IN ROOM 100 ( Guest-Name : ............ ) |  |  |  |
| Check - In : 94/12/26-18 |  |  |  |
| Check - Out : 94/12/31-11 ( 5 days ) |  |  |  |
|  | Start-Time CO Duration Dialed-No. Count Call-Cost Remark |  |  |
|  | 12/27-13:10 01 00:00:32 O00182343507951 1 0 Unanswered |  |  |
|  | 12/27-13:30 01 00:01:23 O00182343507951 3 1500 |  |  |
|  | 12/28-21:22 02 00:10:18 O00182343507953 31 15500 |  |  |
|  | Charge-Time Charged-STA Item Bar-Cost tax |  |  |
|  | 12/28-21:32 COFFEE-SHOP COFFEE 5000 100 |  |  |
|  | 12/29-10:10 FRONT-DESK COKE (MINI-BAR) 3000 30 |  |  |
|  | Item Charge Tax(rate) Sum |  |  |
|  | (1) ROOM CHARGE : 300000 30000(10.00%) 330000 (rate 02 : GOLD) |  |  |
|  | (2) CALL CHARGE : 17000 170(10.00%) 17170 |  |  |
|  | (3) BAR CHARGE : 8000 130 8130 |  |  |
|  | (4) PRE-PAID : -20000 |  |  |
|  | Method of payment : VISA |  |  |
|  | TOTAL : 335300 WON |  |  |

Note : Currency Unit/SMDR Fraction depend on Admin Programming 177

The SMDR print out will show the following:

| NO | STA CO TIME START DIALED CNT COST REMARK |
| --- | --- |
| 0009 | 101 01 00:04 13/06/95 12:47 O123 1 0.10 |
| 0010 | 101 01 00:15 13/06/95 12:50 o01444872014 0 0.00 UNANSWERED |

Alphabet small o at the DIALED field means unanswered outgoing call using ISDN line.

Condition

- This feature will be applied during from seizing ISDN line to hearing a ring back tone

- The feature will only be available when ISDN is used for outgoing

- There will be no charge to the all stations for the unanswered call.

- This feature is applied to the SMDR record

Note !

- This feature is applied for UK, NZ, Australia.

- In NZ and Australia, this feature is extended for LCOB line.

Programming

Related Features

Hardware

## 1.22	AUTO INTERNAL SPEED DIRECTORY

Description

This feature will automatically build an internal directory of registered Service Stations and Guests staying at the Hotel from the Name information that is entered when they check in. Also, once the name has been added to the list it is possible to choose a name from the list for dialing and transferring of calls.

Operation

#### To INSERT or To DELETE Internal Speed Directory Entry

- Manually

System ATD (Front_desk) can program a name of each station for DIAL BY NAME feature.

To insert;

[TRANS/PGM] + 072 + “station number” + “name” + [HOLD/SAVE]

To delete;

[TRANS/PGM] + 072 + “station number” + [HOLD/SAVE]

Each station can program own name for Dial By Name feature.(see the note below)

To insert;

[TRANS/PGM] + 74 + “name” + [HOLD/SAVE]

To delete;

[TRANS/PGM] + 74 + [HOLD/SAVE]

- 2)  Automatically

- When registering service station and station name by ADM PGM 302, service station name is automatically registered with Internal Speed Directory. At this time if service station name is blank then any action will not happened.

- When registering or changing Guest name by attendant command #11, guest name is automatically registered with Internal Speed Directory. At this time if guest name is blank then that guest is deleted in directory.

- When check-out by attendant command #12, the guest name will be deleted in Internal Speed Directory.

- When service station is changed to normal station, the service station name will be deleted in Internal Speed Directory.

To dial by name using internal speed directory entry

Normal LCD

- Press the [SPEED] button twice.

- Dial the 1 for the internal speed dial directory. And confirmation tone is received.

- 1= Intercom / 2 = Station Speed Dial / 3 = System Speed Dial

- Press the [UP]/[DOWN] button or dial digits. The LCD displays 2 names, one on each LCD line. If user dials a character (2 digits), the LCD shows the first match. Pressing the [UP]/[DOWN] button will scroll a line up or down in the directory.

- When the desired name is shown;

Press [HOLD/SAVE] button, Dial by Name is activated.

Large LCD

- The large display telephone can be set to show Alphabetical listing by pressing ICM button in the default state ( i.e. instead of Main Menu):

- To dial an internal room by name, press appropriate Alpha letter.

e.g. to dial entry “Smith Mr”

Press <RST>Key  (Display changes to RST entries)

Press <Smith Mr>key

Condition

- This feature is available with Digital LCD display keyset.

- Registered name size is up to 12 characters

- If there are more entries than can be displayed in the Key station display then it is possible to scroll onto the screen by pressing [UP]/[DOWN] key.

- The internal directory can be displayed when the station is idle, when it has answered a call, and when it is transferring a call.

Programming

Related Features

Hardware

## 1.23	MULTI FRONT DESK

Description

Main attendants can also manipulate Hotel Programming.

Operation

#### To Make Multiple attendants:

- Just insert another station into main attendants in ADMIN PGM 164.

Condition

1) The added multiple front desk station ‘s COS is also unrestricted.

2) If one multiple stations enter the Hotel feature PGM, the others cannot enter the HOTEL feature PGM.

Programming

1) Assign MAIN Attendant    (PGM 164)

Related Features

Hardware

## 1.24	DSS/DLS CONSOLES

Description

The system can be allowed to assign a station port as a Direct Station Select (DSS) or Direct Line Select(DLS) unit. These units are arranged as flexible mapped units. The DSS/DLS unit is assigned in programming to appear as one of the maps for LDK-300 (3 maps for LDK-100). All buttons of any map are programmable.

Operation

Conditions

- The DSS/DLS is assigned to operate in connection with a station.

- Up to  units can be assigned to a station. (Up to 3 units for LDK-100)

- There is no limit to the number of DSS/DLS units in a system.

- The default button assignment of each map is as follow :

| MAP 1 | Intercom Port 1 - 48 |  |
| --- | --- | --- |
| MAP 2 | Intercom Port 49 - 96 |  |
| MAP 3 | Intercom Port 97 – 144 |  |
| MAP 4 | Intercom Port 145 – 192 |  |
| MAP 5 | Intercom Port 193 – 240 |  |
| MAP 6 | Flex 1 – Check In | Flex 2 – Check Out |
|  | Flex 3 – SMDR | Flex 4 – Room Status Print |
|  | Flex 5 – Intercom Enable | Flex 6 – Intercom Disable |
|  | Flex 7 – Call Park 02 | Flex 8 – Station Group 2 |
|  | Flex 9 – Group Call Pickup | Flex 10 – All Call Page |
|  | Flex 11 – Call Park 03 | Flex 12 – Station Group 3 |
|  | Co Line 01 – 36 |  |
| MAP 7 | Co Line 37 – 84 |  |
| MAP 8 | Co Line 85 – 132 |  |

- Maps for LDK-300

| MAP 1 | Intercom Port 1 – 48 |  |
| --- | --- | --- |
| MAP 2 | Intercom Port 49 – 96 |  |
| MAP 3 | Flex 1 – Check In | Flex 2 – Check Out |
|  | Flex 3 – SMDR | Flex 4 – Room Status Print |
|  | Flex 5 – Intercom Enable | Flex 6 – Intercom Disable |
|  | Flex 7 – Call Park 02 | Flex 8 – Station Group 2 |
|  | Flex 9 – Group Call Pickup | Flex 10 – All Call Page |
|  | Flex 11 – Call Park 03 | Flex 12 – Station Group 3 |
|  | Co Line 01 – 36 |  |

- Maps for LDK-100

Programming

Station Base	1) Station ID (PGM 110 – BTN 1)

2) The associate station of DSS/DLS MAP (PGM 110 – BTN 2)

Related Features

Hardware

## 1.25	Set Call Forward

Description

This feature allows the attendant to set the room's call forward on demand of guest.

Operation

To set the room’s call forward;

| TRANS / PGM | + #92 + | Room No. + ( ....... + ) | + Call Forward Type + (1 ~ 4) | Call Forward Destination + | HOLD /SAVE |
| --- | --- | --- | --- | --- | --- |

To remove the room’s call forward;

| TRANS / PGM | + #92 + | Room No. + ( ....... + ) | + ‘#’ + | HOLD /SAVE |
| --- | --- | --- | --- | --- |

Condition

1.	The attendant can set the room’s call forward with the call forward type (Unconditional, busy, no-answer and busy/no-answer).

Programming

Related Features

Station Base	1) Call Forward (PGM 111 – BTN 2)

2) VMIB access (PGM 113 – BTN 2)

Hardware

## 1.26	Hotel Specific Items

1) PGM 112 – BTN 6

| office version | hotel version |
| --- | --- |
| only blocking CO button programming | blocking all flexible button programming |

## 1.27	Nation Specific Items

The modification for AUS Hotel version

1) Change Numbering Plan (see the Flexible Number Plan)
; The default Numbering Plan for AUS is Type 3.

2) The call forward to Busy(No-Ans, Busy/No-Ans) VMIB or VM group does not cancel even though a guest checkout. (UK also)

3) The call to a room which is activated DND will be routed to VMIB immediately. (UK also)

4) A guest name will be displayed “Number and Name” instead of “Name and Number”

# 2. INTERFACE with PROPRIETARY MANAGEMENT SYSTEM

Description

System provides easy interface between system and PC S/W. So PC S/W supporting PMS feature can be interface to system, with minor change of it.

Operation

For Echo mode, enter ADM PGM 300/BTN 4”. In case user has Commercial Hotel Management S/W, Echo Mode has to be off. Otherwise, user want to MS system without Commercial Hotel Management S/W, then Echo Mode has to on. In Non-Echo Mode, System only waits command from PC through RS-232 without printing-out the command on PC.

To execute any MS command, just type MS command which start  ‘$’ character and press the [ENTER] key when prompt is located in front of a line. If user presses [ENTER] key without any command, then following message is displayed on the PC:

LDK-300 System

Version LGE/DX80P-0.1Da JUN/01

DATE: 12/22/01

TIME: 07:14:36

HOTEL NAME    :

ENTER PASSWORD:

At that time if user presses the ENTER  key, “ Access Canceled”  message is displayed and the system  returns the MS command enter mode in front of line.

If user enters any command and press the [ENTER] key in the PC, then MS System processes the command and send the appropriate message.  With Echo Mode, processing result is given by detailed message. When Non-Echo mode, "ACK" or "NAK xxx" (xxx is error no.) message is given to PC.

NOTE

1.	A command Line is consists of  character string and  new line character. Command line cannot exceed 128 characters.

2.	Spaces and tabs in command line are ignored by MS System.

3.	Comment Line which is responded by MS system starts with "//".

4.	Character Set in MS system is case sensitive.

COMMANDS

CHECK-IN COMMAND

- command format : $00 xxxx

- xxxx : fixed 4 digits logical room number.

- example : Check-In the room no. 103

- $00 0103

- System response :

- //Checked-In Room 103.

- REGISTER or CHANGE GUEST NAME COMMAND (in Check-in mode)

- command format : $01 xxxx cc..c

- xxxx : fixed 4 digits logical room number.

- cc..c : guest name within 12 characters. (for space input, use character '*')

- example : Register guest name "'MR BROWN" to room no.103

- $01 0103 MR*BROWN

- System response :

- //Room 103 ( Name - MR BROWN ).

- CHANGE ROOM COS COMMAND(in Check -in mode)

- command format : $02 xxxx dd

- xxxx : fixed 4 digits logical room number.

- dd : 1st d for day cos (1-7) and 2nd d for night cos (1-7).

- example : Change day & night cos of room 103 to '2' and ‘2’

- $02 0103 22

- System response :

- //Room 103 ( DAY : 2, NIGHT : 2)

- REGISTER or CHANGE ROOM TO ROOM CALL GROUP COMMAND(in Check- in mode)

- command format : $03 xxxx gg

- xxxx : fixed 4 digits logical room number.

- gg : room to room call group number (00-99).

- example : Register room to room call group 01 to room no.103

- $03 0103 01

- System response :

- //Room 103 ( Rm. Grp - 1 ).

- CHANGE ICM CALL TO ENABLE/DISABLE STATUS COMMAND(in Check -in mode)

- command format : $04 xxxx f

- xxxx : fixed 4 digits logical room number.

- f : Enable (1), Disable (0).

- example : Enable ICM call in the room103

- $04 0103 1

- System response :

- //Room 103 ( ICM Call - Enable ).

- REGISTER or CHANGE PREPAID MONEY COMMAND(in Check -in mode)

- command format : $05 xxxx dd..d

- xxxx : fixed 4 digits logical room number.

- dd..d : 6 digits prepaid money (000000-999999).

- example : Register prepaid money 2000 to the room103

- $05 0103 002000

- System response :

- //Room 103 ( Pre-Paid - 2000 ).

- REGISTER or CHANGE AUTHORIZATION CODE COMMAND(in Check- in mode)

- command format : $06 xxxx aa..a

- xxxx : fixed 4 digits logical room number.

- aa..a : 5 digits authorization code (00000-99999).

- example : Register authorization code '12345' to the room103

- $06 0103 12345

- System response :

- //Room 103 ( Password is registered ).

- REGISTER or CHANGE WAKE UP TIME COMMAND(in Check -in mode)

- command format : $07 xxxx hhmm flag

- xxxx : fixed 4 digits logical room number.

- hhmm : hour and minute (military form).

- Flag : # in case of continuous wake-up or null string in case of one-time wake-up.

- example : Register one-time wake up 06:00 a.m. to the room103

- $07 0103 0600

- System response :

- //Room 103 (Wake-Up - 6: 0 ).

- example : Register continuous wake up time 06:00 a.m. to the room103

- $07 0103 0600 #

- System response :

- //Room 103 (Wake-Up - 6: 0 *).

- REGISTER or CHANGE CALL CHARGE RATE COMMAND(in Check in mode)

- command format : $08 xxxx d

- xxxx : fixed 4 digits logical room number.

- d : fixed 1 digit call charge rate bin number or * for delete call charge rate

- example :

- Register call charge rate bin no 1. to the room103

- $08 0103 1

- System response :

- //Room 103 (Call charge rate bin : 1 ).

- example :

- Delete call charge rate to the room103

- $08 0103 *

- System response :

- //Room 103 (Call charge rate Not_Assigned ).

- CHECK- OUT COMMAND without method of payment

- command format : $10 xxxx

- xxxx : fixed 4 digits logical room number.

- example : Check-Out the room no. 103

- $10 0103

- System response :

- Room Charge Printed Out

- CHECK- OUT COMMAND with method of payment

- command format : $10 xxxx d

- xxxx : fixed 4 digits logical room number.

- d : fixed 1 digit bin no of payment method

- example : Check-Out the room no. 103

- $10 0103 1

- System response :

- Room Charge Printed Out

- ROOM CHARGE REPORT COMMAND

- command format : $12 xxxx

- xxxx : fixed 4 digits logical room number.

- example : For CRT display the room charge of the room no. 103

- $12 0103

- System response :

- Room Cost and SMDR Printed out

- ROOMS STATUS REPORT COMMAND

- command format : $13 xxxx yyyy

- xxxx : fixed 4 digits logical room.

- yyyy : fixed 4 digits logical room.

- example : To display rooms' status of the room range 103 - 120

- $13 0103 0120

- System response :

- Room Status Printed out

- ROOMS ICM CALL ENABLE COMMAND

- command format : $14 xxxx yyyy

- xxxx : fixed 4 digits logical room.

- yyyy : fixed 4 digits logical room.

- example : To enable ICM call for the room range 103 - 120

- $14 0103 0120

- System response :

- //Room 103- 120 ICM call Enable.

- ROOMS ICM CALL DISABLE COMMAND

- command format : $15 xxxx yyyy

- xxxx : fixed 4 digits logical room.

- yyyy : fixed 4 digits logical room.

- example : To disable ICM call for the room range 103 - 120

- $15 0103 0120

- System response :

- //Room 103- 120 ICM call Disable.

- REGISTER DEFAULT ROOM TO ROOM CALL GROUP FOR ROOMS COMMAND

- command format : $16 xxxx yyyy gg

- xxxx , yyyy : fixed 4 digits logical room.

- gg : room to room call group number (00-99).

- example : Register room to room call group 01 to room range103 - 120

- $16 0103 0120 01

- System response :

- //Room 103- 120 : Rm. to Rm. Grp 1.

- CLEAR DEFAULT ROOM TO ROOM CALL GROUP FOR ROOMS COMMAND

- command format : $17 xxxx yyyy

- xxxx yyyy : fixed 4 digits logical room

- example :

- Clear room to room call group for the room range103 - 120

- $17 0103 0120

- System response :

- //Room 103- 120 : Rm. to Rm. Grp Deleted.

- ONE TIME CO CALL ENABLE COMMAND

- command format : $18 ssss cccc

- ssss : fixed 4 digits service station number.

- cccc : fixed 4 digits charged room number.

- example : Enable one time CO call at service station 111 for the guest of room 103

- $18 0111 0103

- System response :

- //1-time CO call at SVC 111 Charged 103.

- CHANGE ICM SIGNALING MODE FOR ROOMS COMMAND

- command format : $19 xxxx yyyy d

- xxxx, yyyy : fixed 4 digits logical room number.

- d : 1(H), 2(T), 3(P).

- example : Set ICM signaling mode 'T' to the station range103 - 120

- $19 0103 0120 2

- System response :

- //Room 103- 120 Icm-Ans-Mode T.

- ASSIGN DIFFERENTIAL RING FOR ROOMS COMMAND

- Command format : $20 xxxx yyyy d

- Xxxx, yyyy : fixed 4 digits logical room number.

- d : ring type (1 - 4).

- example : Set Ring type 1 to the station range103 - 120

- $20 0103 0120 1

- System response :

- //Room 103- 120 Ring Type 1.

- CHANGE LCD LANGUAGE FOR ROOMS COMMAND

- Command format : $21 xxxx yyyy dd

- Xxxx yyyy : fixed 4 digits logical room number

- dd : language type (00 - 24)

- 00 : English 01 : Italian 02 : Finnish - 03 : Dutch 04 : Swedish 05 : Danish - 06 : Norwegian 07 : Hungarian 08 : Germany - 09 : French 10 : Portuguese 11 : Spanish - 12 : Korean 13 : Estonia 14 : Russian

- example : Change LCD language to Korean from room 103 to room 120

- $21 0103 0120 12

- System response :

- //Room 103- 120 LCD Language 12.

- CHANGE LCD DATE FORMAT COMMAND

- command format : $22

- toggle setting (MMDDYY or DDMMYY)

- System response :

- //LCD DATE Format : DDMMYY.

- CHANGE LCD TIME FORMAT COMMAND

- Command format : $23

- toggle setting (12 Hour or 24 Hour)

- System response :

- //LCD TIME Format : 24Hour.

- SYSTEM CLOCK SET COMMAND

- Command format : $25 YYMMDDhhmm

- YY : Year, MM : Month, DD : Day, hh : Hour, mm : Minute

- System response :

- //System Clock Changed.

- CONTROL MAID STATUS COMMAND

- command format : $26 xxxx f

- xxxx : fixed 4 digits logical room number.

- f : 0 to make maid status clean for the room

- 1 to make maid status dirty for the room

- 2 to read maid status for the room

- example 1 : To make maid status clean for room 0103

- $26 0103 0

- System response :

- //Room 103 (Set Maid Status - Clean)

- example 2 : To make maid status dirty for room 0103

- $26 0103 1

- System response :

- //Room 103 (Set Maid Status - Dirty)

- example 3 : To read maid status for room 0103

- $26 0103 2

- System response :

- //Room 103 (Current Maid Status - Dirty)

- DISPLAY BAR CHARGE FOR ROOM COMMAND

- command format : $27 xxxx

- xxxx : fixed 4 digits logical room number.

- example : To read bar charge for room 0120

- $27 0120

- System response :

- Bar Charge Printed out.

- DISPLAY MINI-BAR CHARGE FOR ROOM COMMAND

- command format : $28 xxxx

- xxxx : fixed 4 digits logical room number.

- example : To read mini-bar charge for room 0120

- $28 0120

- System response :

- Mini-Bar Charge Printed out.

- SET MESSAGE WAIT COMMAND

- command format : $29 xxxx yyyy

- yyyy : fixed 4 digits logical room number.

- example : To set message wait from room 0100 to room 0105

- $29 0100 0105

- System response :

- //Room 105 : Message from 100

- NOTE !! - LDK system supports up to 5 messages per station. In case PMS command $29 happens on the station with 5 waiting message, the command is canceled. At this time, system responses with no message.

- CANCEL MESSAGE WAIT COMMAND

- command format : $30 xxxx yyyy

- xxxx, yyyy : fixed 4 digits logical room number.

- example : To cancel message wait from room 0100 to room 0105

- $30 0100 0105

- System response :

- //Room 105 : message from 100 canceled.

- ASSIGN CALL CHARGE RATE FOR ROOMS COMMAND

- command format : $31 xxxx yyyy f

- xxxx, yyyy : fixed 4 digits logical room number.

- F : 1 digit call charge rate bin no (0-5) or * to erase.

- example : To assign call charge rate 1 from room 0100 to room 0105

- $31 0100 0105 1

- System response :

- //Room 100- 105 Call Charge Rate Bin No. 1

- example : To delete call charge rate from room 0100 to room 0105

- $31 0100 0105 *

- System response :

- //Room 100- 105 Call Charge Rate Not Assigned.

- REGISTER MINI-BAR CHARGE FOR ROOM COMMAND

- command format : $32 xxxx yy cccccc

- xxxx : fixed 4 digits logical room number.

- yy : fixed 2 digits product code (00-99)

- cccccc : fixed 6 digits bar cost

- example : To register mini-bar charge of room 0100 , - at this time if product code is 05 and mini-bar charge is 000123

- $32 0100 05 000123

- System response :

//Room 100 : Mini-Bar Charge Registered

Mini-Bar Information for the room is printed out

PRINT SMDR with Stations Range

- command format : $33 xxxx yyyy xxxx : the start fixed 4 digits logical room number. yyyy : the end fixed 4 digits logical room number

- Example : To print Call SMDR from room 0100 to room 0102 , $33 0100 0102

- System response : SMDR Information for the rooms are printed out

Cancel Wake-up service

- Command format : $34 xxxx xxxx : the start fixed 4 digits logical room number.

- Example : To cancel wakeup of room 0100 , $34 0100

- System response : //Room 100 (Wake-up Set – Erased).

Print Day / Time

- Command format : $35

- Example : To print Day/ Time, $35

- System response : this print format can be four types by day represent mode or 12/24 hour mode // Date: 31-03-00 Time:12:01

ERROR RESPONSE

| Error Code | In Echo Mode | In Non-Echo Mode |
| --- | --- | --- |
| 000 | // ERR : Invalid Command. | $NAK000 |
| 001 | // ERR : Invalid Room No. | $NAK001 |
| 002 | // ERR : Service Station. | $NAK002 |
| 003 | // ERR : Room is occupied. | $NAK003 |
| 004 | // ERR : Room is not clean. | $NAK004 |
| 005 | // ERR : Room Is Vacant. | $NAK005 |
| 006 | // ERR : Invalid Room COS. | $NAK006 |
| 007 | // ERR : Invalid Rm Grp. | $NAK007 |
| 008 | // ERR : Invalid ICM En/Disable. | $NAK008 |
| 009 | // ERR : Invalid Pre-paid Money. | $NAK009 |
| 010 | // ERR : Invalid Password. | $NAK010 |
| 011 | // ERR : Duplicated Password. | $NAK011 |
| 012 | // ERR : Invalid Time. | $NAK012 |
| 013 | // ERR : Invalid Room Range. | $NAK013 |
| 014 | // ERR : Invalid Input Data. | $NAK014 |
| 015 | // ERR : Invalid Date. | $NAK015 |
| 016 | // ERR : Invalid Maid Status | $NAK016 |
| 017 | // ERR : Station is Busy | $NAK017 |
| 018 | // ERR : No Message Wait | $NAK018 |
| 019 | // ERR : Invalid Call Charge Rate Bin No. | $NAK019 |
| 020 | // ERR : Not Enough Memory | $NAK020 |
| 021 | // ERR : Printer is Busy | $NAK021 |

# 3. FLEXIBLE NUMBERING PLAN

| PGM | BTN | FIELD | NUMBER SET1 | NUMBER SET2 | NUMBER SET3 | NUMBER SET4 | NUMBER SET5 | NUMBER SET6 | NUMBER SET7 | NUMBER SET8 | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 105 | - | Intercom Call | 1000 - 1599 | 1000 - 1599 | 7000 - 7599 | 7000 - 7599 | 2000 - 2599 | 10 - 79 | 1000 - 1299 | 1000 - 1599 | LDK-300E |
|  |  |  | 100 - 399 | 100 - 399 (100 - 799) | 7000 – 7299 | 700 - 999 | 200 - 499 | 10 - 79 | 100 - 299 | 100 - 399 (100 - 999) | LDK-300 |
|  |  |  | 100 -227 | 100 - 227 (100 - 799) | 7000 – 7127 | 700 - 827 | 200 - 327 | 10 - 79 | 100 - 227 | 100 - 227 (100 - 999) | LDK-100 |
| 106 | 1 | Group Pilot Number | 620 - 667 | *620 - *667 | *620 - *667 | 620 - 667 | 620 - 667 | *620 - *667 | 620 - 667 | *620 - *667 | LDK-300E |
|  |  |  | 620 - 634 | *620 - *634 | *620 - *634 | 620 - 634 | 620 - 634 | *620 - *634 | 620 - 634 | *620 - *634 | LDK-100 |
|  | 2 | Internal Page Zone | 501 - 535 | *501 - *535 | #01 - #35 | #01 - #35 | #01 - #35 | *501 - *535 | 401 - 419 | *501 - *535 | LDK-300E |
|  |  |  | 501 - 515 | *501 - *515 | #01 - #15 | #01 - #15 | #01 - #15 | *501 - *515 | 401 - 415 | *501 - *515 | LDK-100 |
|  | 3 | Internal All Call Page | 543 | *543 | ## | #7 | #5 | *543 | 43 | *543 |  |
|  | 4 | Meet Me Page | 544 | *544 | #7 | ## | ## | *544 | 44 | *544 |  |
|  | 5 | External Page Zone 1 | 545 | *545 | #41 | #41 | #6 | *545 | 45 | *545 |  |
|  | 6 | External Page Zone 2 | 546 | *546 | #42 | #42 | #7 | *546 | 46 | *546 |  |
|  | 7 | External Page Zone 3 | 547 | *547 | #43 | #43 | #8 | *547 | 47 | *547 |  |
|  | 8 | External All Call Page | 548 | *548 | #40 | #5 | #9 | *548 | 48 | *548 |  |
|  | 9 | All Call Page (Int & Ext) | 549 | *549 | #6 | #6 | #00 | *549 | 49 | *549 |  |
|  | 10 | SMDR Account Code Enter | 550 | *550 | *80 | 550 | 50 | *550 | 50 | *550 | SLT |
|  | 11 | Flash Command to CO Line | 551 | *551 | *81 | 551 | 51 | *551 | 51 | *551 | SLT |
|  | 12 | Last Number Redial | 552 | *552 | *82 | 552 | 52 | *552 | 52 | *552 | SLT |
|  | 13 | DND (Toggle On/Off) | 553 | *553 | *5 | 553 | 53 | *553 | 53 | *553 | SLT |
|  | 14 | Call Forward | 554 | *554 | *4 | 554 | 54 | *554 | 54 | *554 | SLT |
|  | 15 | Speed Dial Programming | 555 | *555 | *83 | *40 | 55 | *555 | 55 | *555 | SLT |
|  | 16 | Message Wait/Callback Enable | 556 | *556 | *84 | *66 | 56 | *556 | 56 | *556 |  |
|  | 17 | Message Wait/Callback Return | 557 | *557 | 6 | *67 | 57 | *557 | 57 | *557 | SLT |
|  | 18 | Speed Dial Access | 558 | *558 | 8 | *7 | 58 | *558 | 58 | *558 | SLT |
|  | 19 | Cancel DND/FWD/Pre-MSG | 559 | *559 | #5 | 559 | 59 | *559 | 59 | *559 | SLT |
|  | 20 | System Hold | 560 | *560 | *85 | 560 | 690 | *560 | 30 | *560 | SLT |
|  | 21 | Station Relocation Backup | 561 | *561 | *86 | 561 | 691 | *561 | … | *561 |  |
|  | 22 | Station Relocation Retrieve | 562 | *562 | *87 | 562 | 692 | *562 | … | *562 |  |
|  | 23 | Programming Mode Enter Code | 563 | *563 | *1 | 563 | 693 | *3 | 33 | *3 | SLT |
|  | 24 | ACD Reroute | 564 | *564 | *88 | 564 | 694 | *4 | 34 | *4 |  |
| PGM | BTN | FIELD | NUMBER SET1 | NUMBER SET2 | NUMBER SET3 | NUMBER SET4 | NUMBER SET5 | NUMBER SET6 | NUMBER SET7 | NUMBER SET8 | REMARK |
| 107 | 1 | Alarm Reset | 565 | *565 | *90 | *565 | 695 | *565 | 35 | *565 |  |
|  | 2 | Group Call Pickup | 566 | *566 | ** | *1 | ** | *566 | 36 | *566 |  |
|  | 3 | UCD DND | 568 | *568 | *91 | 568 | 698 | *568 | 68 | *568 |  |
|  | 4 | Night Answer | 569 | *569 | *92 | 2 | 699 | *569 | 69 | *569 |  |
|  | 5 | Call Parking Locations | 601 - 619 | *601 -* 619 | *601 – *619 | 601-619 | 601 - 619 | *601 -* 619 | 601 - 619 | *601 -* 619 | LDK-300/300E |
|  |  |  | 601 - 610 | *601 -* 610 | *601 – *610 | 601-610 | 601 - 610 | *601 -* 610 | 601 - 610 | *601 -* 610 | LDK-100 |
|  | 6 | Direct Call Pickup | 7 | *7 | *7 | *42 | 7 | *7 | 7 | *7 |  |
|  | 7 | CO Group Access | 801-872 | 801-872 | #801-#872 | 401-472 | 801-872 | 801-872 | 801-872 | #801-#872 | LDK-300/300E |
|  |  |  | 801-824 | 801-824 | #801-#824 | 401-424 | 801-824 | 801-824 | 801-824 | #801-#824 | LDK-100 |
|  | 8 | Individual CO Access | 88001-88400 | 88001-88400 | #88001-#88400 | 48001-48400 | 88001-88400 | 88001-88400 | 88001-88400 | #88001-#88400 | LDK-300E |
|  |  |  | 88001-88200 | 88001-88200 | #88001-#88200 | 48001-48200 | 88001-88200 | 88001-88200 | 88001-88200 | #88001-#88200 | LDK-300 |
|  |  |  | 8801-8840 | 8801-8840 | #8801-#8840 | 4801-4840 | 8801-8840 | 8801-8840 | 8801-8840 | #8801-#8840 | LDK-100 |
|  | 9 | Tie Routing Access | 8901 | 8901 | #901 | 4901 | 89 | 8901 | 8901 | #401 |  |
|  | 10 | Retrieve Held CO Line | 8* | 8* | #8* | 4* | 8* | 8* | 8* | #8* |  |
|  | 11 | Retrieve Held Individual CO Line | 8#xxx | 8#xxx | #8#xxx | 4#xxx | 8#xxx | 8#xxx | 8#xxx | #8#xxx | LDK-300/300E |
|  |  |  | 8#xx | 8#xx | #8#xx | 4#xx | 8#xx | 8#xx | 8#xx | #8#xx | LDK-100 |
|  | 12 | Access CO Line In the 1st available CO Group | 9 | 9 | 0 | 1 | 0 | 9 | 9 | 0 |  |
|  | 13 | Attendant Call | 0 | 0 | 1 | 0 | 9 | 0 | 0 | #9 |  |
|  | 14 | 1st Door Open | #*1 | #*1 | #*1 | #*1 | #*1 | #*1 | #*1 | #*1 |  |
|  | 15 | 2nd Door Open | #*2 | #*2 | #*2 | #*2 | #*2 | #*2 | #*2 | #*2 |  |
|  | 16 | 3rd Door Open | #*3 | #*3 | #*3 | #*3 | #*3 | #*3 | #*3 | #*3 |  |
|  | 17 | 4th Door Open | #*4 | #*4 | #*4 | #*4 | #*4 | #*4 | #*4 | #*4 |  |
|  | 18 | 5th Door Open | #*5 | #*5 | #*5 | #*5 | #*5 | #*5 | #*5 | #*5 |  |
|  | 19 | 6th Door Open | #*6 | #*6 | #*6 | #*6 | #*6 | #*6 | #*6 | #*6 |  |
|  | 20 | 7th Door Open | #*7 | #*7 | #*7 | #*7 | #*7 | #*7 | #*7 | #*7 | LDK-300/300E |
|  |  |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | LDK-100 |
|  | 21 | VM Message Waiting Enable | *8 | *8 | #*8 | *8 | *8 | *8 | *8 | *8 |  |
|  | 22 | VM Message Waiting Disable | *9 | *9 | #*9 | *9 | *9 | *9 | *9 | *9 |  |