- FUNCTIONAL SPECIFICATION

- OF

- LDK-RSG

July 2003

- REVISION HISTORY

| ISSUE | DATE | BY | REMARK |
| --- | --- | --- | --- |
| 0.1A | 25/July/03 | LDK GRP | Preliminary Release |

DISTRIBUTION

| ISSUE | DATE | BY | COUNTRY | COMPANY | ATTN |
| --- | --- | --- | --- | --- | --- |

- TABLE OF CONTENTS

1.	OVERVIEW	6

1.1	Network Diagram	6

2.	CONFIGURATION	7

2.1	Capacity	7

2.2	Appliance of Security Application	7

2.3	Regulatory Spec. of RSG	7

3.	CALL FEATURES	9

3.1	VOIB Configuration for RSG	9

3.2	RSG Registration	9

3.3	RSG Associated CO Line Usage	9

3.4	RSG Associated Resource Usage	9

4.	RSG SETTING	11

4.1	Hardware/Software Requirements	11

4.2	Hardware Configuration	11

4.3	Operations	11

4.3.1	Set Master’s IP Address	11

4.3.2	Set the WAN and get information	13

4.3.3	Set the PPPoE and get information	14

4.3.4	Select trace of call traffic	15

5.	RSG / IP Phone Admin Programming	16

5.1	VOIB SLOT ASSIGNMENT for RSG/IP Phone (PGM 380)	16

5.2	RSG/IP Phone Port Number ASSIGNMENT (PGM 381)	17

5.3	RSG / IP Phone ATTRIBUTE  (PGM 382)	18

5.4	RSG ATTRIBUTE 1  (PGM 383)	19

5.5	RSG ATTRIBUTE 2  (PGM 384)	20

5.6	RSG ALARM ASSIGNMENT  (PGM 385)	22

5.7	RSG ATTRIBUTE 1  (PGM 386)	23

5.8	EXPANDED FLEXIBLE NUMBERING PLAN  (PGM 109)	24

5.9	RSG DKT RX GAIN CONTROL  (PGM 390)	25

5.10	RSG DKT TX GAIN CONTROL  (PGM 391)	26

5.11	RSG SLT RX GAIN CONTROL  (PGM 392)	27

5.12	RSG SLT TX GAIN CONTROL  (PGM 393)	28

5.13	RSG LCO RX GAIN CONTROL  (PGM 394)	29

5.14	RSG LCO TX GAIN CONTROL  (PGM 395)	30

5.15	IP PHNOE RX GAIN CONTROL  (PGM 396)	31

5.16	IP PHONE TX GAIN CONTROL  (PGM 397)	32

5.17	INIT BY MPB VERSION  (PGM 452)	33

Appendix A. RSG Installation Procedure on MPB	34

Appendix B. RSG Setting Procedure on RSG	35

Appendix C. IP Phone Setting Procedure	36

Appendix D. RSG / IP Phone Default Gain Table	1

# 1.	OVERVIEW

Modern businesses often have remote office locations consisting of a single employee home office to offices with a number of employees. These remote locations are served by independent telecommunications systems, which do not easily provide continuity of service. Services to the corporate telecomm system are typically via a DISA type facility and the remote user is excluded from services to many of the system’s resources and functionality.

While telecomm systems may be networked over PSTN or leased line facilities to provide remote connection, such connections are expensive and do not incorporate protocols to efficiently provide interconnection to a small number of remote users.  Nor do these protocols allow for the simple transfer of the detailed information needed between the two end-points necessary to support today’s complex communication features.

A major advantage of VoIP in the telecomm market is the ability of the protocol to carryout a transparent remote connection efficiently for even a small number of remote users.

## 1.1	Network Diagram

Remote-Service permits remote users and interface services to the LDK System over broadband IP networks. Remote Services are implemented with the Remote Services Gateway and RTP packet is relayed via the system’s VoIB channels to communicate between LDK System and remote terminal. The following diagram is the typical remote LDK network connection.

LDK

w/ VoIB

xDSL, Cable modem or Router

RSG

PC

LKD

SLT,

FAX

LAN

E911

LDK-100

DIGITAL KEY TELEPHONE

SYSTEM

IP Network

Voice & Data

PSTN/

Ethernet

101

102

103

119

125

141

124

IP KTU

# 2.	CONFIGURATION

## 2.1	Capacity

System Capacity for RSG (Remote Services Gateway)

| RSG | RSG numbers can be serviced by LDK | 96 (LDK-300/300E) 32 (LDK-100) |
| --- | --- | --- |
|  | Devices per RSG | 1) 1 Digital Keyset and 1 Single Line Telephone 2) 1 PSTN |
| RTP Relay Channel for RSG |  | Up to channel number of VOIP assigned for RSG |
| IP Phone numbers can be serviced by LDK |  | 96 (LDK-300/300E) 64 (LDK-100) |
| Recommend RSG/IP Phone per VOIB |  | 8 |

- System Capacity for RSG/IP Phone is limited by PGM 381/Btn 1&2.

## 2.2	Appliance of Security Application

Security is needed for the communication between local and remote devices.

Between local device and RSG associated device, the Voice communication channel is established via VOIP gateway because of public routing path.

Because of this, one good point is automatically made. VOIP gives the operation of security not only for a VoIP call but also for the communication to RSG.

## 2.3	Regulatory Spec. of RSG

- Interface:

. ADSL (PPPoE)

. 2 Lan Port (1 uplink, 1 PC)

. 1 Digital Keyset Interface

. 1 Single Line Interface

. 1 PSTN Interface

. 2 External Contact Relays

. 1 Internal MOH, 1 External MOH

. 1 Alarm Port

- Voice Quality:

G.711_ALAW / G.711_ULAW / G.723.1

- IP Address:

Fixed or Dynamic

- Security

IPsec, STUN

- NAT (Network Address Translation)

# 3.	CALL FEATURES

For the most of Call features, stations and co lines on RSG is same as local devices.

(Refer to the RSG Call Feature List).

But there are some different features as follows.

## 3.1	VOIB Configuration for RSG

The RSG is serviced through VOIB. So, the VOIB for RSG should be assigned in PGM 380.

If several VOIBs are assigned in PGM 380, please assign the first VOIB slot on STA/COL Board in PGM 103/BTN 1 & 2. And reset the system for board configuration.

## 3.2	RSG Registration

For registration of RSG to the LDK system, enter the MAC of RSG in PGM 383 after VOIB Configuration.

## 3.3	RSG Associated CO Line Usage

- CO Outgoing.

By Dial ‘9’ the RSG devices can access their local CO Line if ‘First Access RSG CO’ (PGM 382.BTN 6) is set to ON.

- CO Incoming

The RSG devices can receive the CO incoming ring of their RSG CO without CO ring assignment if the ‘RING w/o CO Ring Assign’ is set to ON. But it can be extended to the other stations by ring assignment in PGM-144.

- Emergency Call

Local Emergency Call from RSG devices is done via their RSG CO Line.

## 3.4	RSG Associated Resource Usage

There are the following resources on RSG.

- External Relay Control

  - There are two external contact relays on the RSG.

  - They can be used for LBC or Door Open. It is assigned in PGM 384/Btn 5&6.

  - To assign to the LBC, please enter ‘1’ and the associated station number. Then the LBC is working when the association station receives a ring.

  - To assign to the Door Open, please enter ‘2’. Then it can be activated by dialing RSG Door Open Code (PGM 109/ Btn 2 &3) from RSG devices.

- Local MOH

  - The RSG has one internal and one external MOH source.

  - RSG associated devices use their local MOH source in the RSG.

  - The music will be heard when RSG devices on RSG are held or digital keyset on RSG activates the BGM.

- Alarm Signal

  - When Alarm Signal is detected from RSG, the LDK system gives the Alarm Ring to the Alarm assign station in PGM 385. And the alarm will be stop by Alarm Reset code.

  - To activate an Alarm on RSG, the Alarm should be enabled in PGM 384/Btn 7.

# 4.	RSG SETTING

## 4.1	Hardware/Software Requirements

- RSGM board

- Firmware Version 1.0B or later

- Terminal Emulator Software

- Z-Modem protocol should be supported (e.g. Hyper Terminal, Procomm Plus, etc)

- RS-232C Cable

- RS-232C Type Cable for connecting PC and RSGM board : Two connectors are needed for this connection. One connector should be a 9-pin female connector that is to be connected to RSGM board, and the other one should fit the serial port of the PC. There are three required lines that should be connected for the communication between PC and RSGM board: Ground-Ground, Transmit-Receive, and Receive-Transmit.

- RSGM board

- Z-Modem protocol should be supported (e.g. Hyper Terminal, Procomm Plus, etc)

- Baud Rate

- 38400, should not be changed

## 4.2	Hardware Configuration

In order to use serial connection between PC and RSGM board, the serial port of PC should be connected to the serial port of RSGM board with RS-232C type cable (serial cable).

Firmware upgrade commands can be executed only under "LAN Firmware Upgrade Mode". In order to enter this mode, please follow the procedure below.

- If you did not log into the trace mode yet, log into the trace mode by entering password at "Password" prompt. After entering password, you will see "root" prompt that means that you are now in root mode.

- If you are not in root mode, you are probably in one of the sub-modes. In this case, type 'b' to go back to root mode.

- After you confirm that you are now in root mode, enter 'u' to enter "LAN Firmware Upgrade Mode". Then, you will see "swup" prompt.

## 4.3	Operations

### 4.3.1	Set Master’s IP Address

- If the RSGM wants to operate, the master IP address should be input.

Operations

- Press ‘Enter’ Key, the ‘main:>’ is out after menu list.

Ex)

==========================================================

Command                      Description

==========================================================

wan                    Change to WAN Configuration

lan                    Change to LAN Configuration

system                Change to System Configuration

status                 Change to System Status

trace                  Change to Trace Options

passwd                 Change Password

reset                  Reset RSGM

exit (or x)            Exit Monitor Mode

help (or ?)            Display Help on Menu

main:>

- Type ‘system’ and enter.

Ex)

main:> system

===============================================================

Command                      Description

===============================================================

/                      Change to Root Catalog

..                     Change to Parent Catalog

get all                Read All Parameters

get [arg]              Read [arg] Parameter

{ver|mfim|http|vtag|ptag}

set mfim [ipaddr]      Write MFIM IP Address

set http               Write HTTP Access Mode

{wan|lan} {yes|no}

set vtag [digt]        Write RTP VLAN Tag (0-4094|del)

set ptag [digt]        Write RTP Priority Tag (0-7|del)

- If you want to have the current information for system, type ‘get all’ and enter.

Ex)

get all

Software Version: RSGM1-1.0Ad

Revision Date: 24 Feb 2003

MFIM IP Address: 218.148.14.252

HTTP WAN Access: YES

HTTP LAN Access: YES

RTP VLAN Tag: N/A

RTP Priority Tag: N/A

- If you want to set the Master’s IP address, type ‘set mfim xxx.xxx.xxx.xxx’.

The ‘xxx.xxx.xxx.xxx’ means the IP address number.

After entering the new Master’s IP address, you would check the changed IP address

by entering ‘get all’ command.

Ex) If the IP address of master is 150.150.54.117.

set master 150.150.54.117

MFIM IP Address Changed Successfully: 150.150.54.117

get all

Software Version: RSGM1-1.0Ad

Revision Date: 24 Feb 2003

MFIM IP Address: 150.150.54.117

HTTP WAN Access: YES

HTTP LAN Access: YES

RTP VLAN Tag: N/A

RTP Priority Tag: N/A

- Let’s go to the upper level by typing ‘..’ and enter, then the help menu is displayed again.

Ex)

main:/system> ..

==========================================================

Command                      Description

==========================================================

wan                    Change to WAN Configuration

lan                    Change to LAN Configuration

system                 Change to System Configuration

status                 Change to System Status

trace                  Change to Trace Options

passwd                 Change Password

reset                  Reset RSGM

exit (or x)            Exit Monitor Mode

help (or ?)            Display Help on Menu

main:>

### 4.3.2	Set the WAN and get information

- The RSGM could select WAN set and PPPoE set alternatively.

If the WAN set is selected, the PPPoE set should be disabled. Otherwise, if the PPPoE set is selected, the WAN set should be disabled. The default mode of WAN is ‘fixed’.

Operations

- At ‘main:>’, type ‘wan’ and confirm to display the help menu.

Ex)

main:> wan

=============================================================

Command                      Description

=============================================================

/           		Change to Root Catalog

..                     	Change to Parent Catalog

wanset                	Change to WAN Settings Catalog

pppoe                	Change to PPPoE Catalog

wanvlan              	Change to VLAN Catalog

main:/wan>

- At ‘main:/wan’, type ‘wanset’ and it displays the help menu and the current value of mode.

Ex)

main:/wan> wanset

==============================================================

Command                      Description              Mode

==============================================================

/                      Change to Root Catalog

..                     Change to Parent Catalog

mode                  Toggle Address Config Mode  : FIXED

get all                Read All Parameters

get [arg]              Read [arg] Parameter

{ip|subnet|gateway|dns|host|domain|

mac|blimit|mlimit}

set ip [ipaddr]        Write (Fixed Mode) IP Address

set subnet [ipaddr]    Write (Fixed Mode) Subnet Mask

set gateway [ipaddr]   Write (Fixed Mode) Gateway Address

set dns [ipaddr]       Write (Fixed Mode) DNS Address (..|del)

set host [string]      Write (Fixed Mode) Host Name (..|del)

set domain [domain]    Write (Fixed Mode) Doamin Name (..|del)

set blimit [digt]      Write Broadcast Limit(%) (1-100)

set mlimit [digt]      Write Multicast Limit(%) (1-100)

main:/wan/wanset>

- Confirm the value of mode is ‘fixed’ if you want to set WAN set.

.

- As the help menu says, type the value for each setting.

For example, IP address, subnet mask, gateway address, DNS, etc.

Ex) If you want to set IP address of 150.150.54.107

main:/wan/wanset> set ip 192.168.54.107

IP Address Changed Successfully: 192.168.54.107

main:/wan/wanset>

4. If you want to verify the changed value, please type ‘get all’.

### 4.3.3	Set the PPPoE and get information

The default mode of WAN is FIXED. Therefore, if you want to set PPPoe, you need to change the mode of PPPoe from INACTIVE into ACTIVE and need to change the mode of WAN from FIXED into DYNAMIC. That’s, The mode is toggled whenever type ‘mode’. And then, please input the information for PPPoE at PPPoE menu.

Operations

- Change the mode of WAN from FIXED into DYNAMIC by typing ‘mode’ at ‘main:/wan/wanset’.

Ex)

main:/wan/wanset> mode

Address Config Mode Changed to DYNAMIC

main:/wan/wanset>

- Change the mode of PPPoE from INACTIVE into ACTIVE by typing ‘mode’ at ‘main:/wan/pppoe’.

Ex)

main:/wan/wanset> ..

==============================================================

Command                      Description

==============================================================

/                      Change to Root Catalog

..                     Change to Parent Catalog

wanset                 Change to WAN Settings Catalog

pppoe                  Change to PPPoE Catalog

wanvlan                Change to VLAN Catalog

main:/wan> pppoe

==============================================================

Command                      Description              Mode

==============================================================

/                      Change to Root Catalog

..                     Change to Parent Catalog

mode                   Toggle PPPoE Activation Mode: INACTIVE

get all                Read All Parameters

get [arg]              Read [arg] Parameter

{user|passwd|service|ac}

set user [string]      Write User Name

set passwd [string]    Write Password

set service [string]   Write Service Name (..|del)

set ac [string]        Write AC Name (..|del)

main:/wan/pppoe> mode

PPPoE Activation Mode Changed to ACTIVE

main:/wan/pppoe>

- As the help menu says, type the value for each setting.

For example, ID, password, etc.

Ex)

main:/wan/pppoe> set user lg0000002

User Name Changed Successfully: lg0000002

main:/wan/pppoe>

- If you want to verify the changed value, please type ‘get all’.

### 4.3.4	Select trace of call traffic

While the RSGM is operating, sometimes you need to get some trace data for something.

At that time, type ‘bt’ of ‘trace’ menu at main. The result is displayed with other information.

And then, it starts to display the call traffic trace.

Operations

- The board trace mode is toggled whenever typing ‘bt’.

Ex)

main:/trace> bt

=============================================================

Command                      Description                 Status

=============================================================

/                      Change to Root Catalog

..                     Change to Parent Catalog

bt                     Toggle Board Trace Mode      :  Y

g_bMFIMConnected[TYPE] View TRUE or FALSE        : 01, 01, 01, 01

g_SystemRelayEnable    View LDK(55) or LIK(AA) RSGM : 55

===================================================================

main:/trace>

# RSG / IP Phone Admin Programming

## 5.1	VOIB SLOT ASSIGNMENT for RSG/IP Phone (PGM 380)

The RSG/IP Phone receives call service through VOIB.

Then the VOIB for RSG/IP can be assigned.

If several boards are assigned, please assign the first VOIB slot on STA/COL Board in PGM 103/BTN 1 & 2.

PROCEDURE

| VOIB SLOT FOR RSG/IP F1:SLOT | - [TRANS/PGM] + 380 |
| --- | --- |
| 05 06 .. .. .. .. .. .. .. .. .. .. .. .. .. .. | - For VOIB slot assignment, Press Flex_1. Dial slot numbers. |
| RSG/IP CHANNEL ASSIGN ENTER VOIB SLOT NO: | - For VOIB slot assignment, Press Flex_2. Dial slot numbers. |
| SLOT 05 RSG/IP CHANNEL (00-08) : 00 | - Enter VOIB Channel number for RSG/IP |
| VOIB SLOT FOR RSG/IP PRESS FLEX_BTN (1-2) | - Press the [HOLD/SAVE] button for saving database permanently. |
|  | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| BTN | ITEM | - RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | VOIB SLOT for RSG/IP Phone |  | - | VOIB slot assignment for RSG/IP Phone |
| 2 | VOIB Channel Number for RSG/IP Phone |  | - | VOIB Channel number used for RSG/IP Phone |

TABLE 5.1.1	  VOIB Slot Assignment for RSG/IP Phone (PGM 380)

## 5.2	RSG/IP Phone Port Number ASSIGNMENT (PGM 381)

The port number for RSG /IP Phone can be assigned.

PROCEDURE

| RSG/IP NO ASSIGN F1:RSG F2:IP PHONE | - [TRANS/PGM] + 381 |
| --- | --- |
| RSG NO 008 (00 ~ 96) | - To program the number of to be serviced RSG number, press FLEX BTN 1 and dial RSG number. |
| IP PHONE NO 000 (00 ~ 96) | - To program the number of to be serviced IP Phone number, press FLEX BTN 2 and dial IP Phone number. |
| RSG/IP NO ASSIGN F1:RSG F2:IP PHONE | - Press the [HOLD/SAVE] button for saving database permanently. |

| BTN | ITEM | - RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| F1 | RSG NO | 00~96 (00-32) | 08 (08) | The RSG number to be serviced from system |
| F2 | IP PHONE NO | 00~96 (00-64) | 00 (00) | The IP Phone number to be serviced from system |

TABLE 5.2.1	  Port Number Assignment for RSG/ IP Phone

- Note) The remote station port number (RSG NO * 2 + IP Phone NO) should be assigned less than 255.

## 5.3	RSG / IP Phone ATTRIBUTE  (PGM 382)

The following is the attributes of RSG/IP Phone.

PROCEDURE

| RSG/IP ATTR1 PRESS FLEX KEY (1-5) | - [TRANS/PGM] + 382. |
| --- | --- |
| TRANSFER MODE (1:IP/0:MAC): IP | - To program, press Flex BTN 1-5 for setting each value. After pressing a Flex BTN, the revised value can be set by entered digit. |
| CASTING MODE (1:MULTI/0:UNI): UNI |  |
| TONE SOURCE (1:REMOTE/0:LDK): REMOTE |  |
| PEER TO PEER (1:ON/0:OFF): ON |  |
| IP/RSGM ATTRIBUTE PRESS FLEX KEY (1-5) | - Press the [HOLD/SAVE] button for updating database permanently. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| F1 | Transfer Mode | IP or MAC | IP |  |
| F2 | Casting Mode | Unicast or Multicast | Unicast |  |
| F3 | Tone Generation | LDK or Remote(RSGM/IP Phone) | Remote |  |
| F4 | Peer to Peer | ON/OFF | ON |  |
| F5 | Codec Type | G.711_ALAW(0)/G.711_ULAW(1)/ G.723.1(2) | G.711_ALAW(0) |  |
| F6 | First Access RSG CO | ON/OFF | ON | If the field is set, the station on RSG can access a CO line on his RSG by dialing CO Line access code in the 1st available CO group (ex> 9). |
| F7 | RING w/o CO Ring Assign | ON/OFF | ON | If the field is set, stations on RSG will receive the incoming CO ring even though the CO ring is not assigned. |

TABLE 5.3.1	  RSGM/IP Phone Attributes 1 (PGM 382)

## 5.4	RSG ATTRIBUTE 1  (PGM 383)

The following is the attributes of RSG.

PROCEDURE

| RSG ATTR1 ENTER NO (01-96) | - [TRANS/PGM] + 383. Enter the RSG number |
| --- | --- |
| 01 RSG ATTR1 PRESS FLEX (1-7) |  |
| 01 SET MAC ADDR xx-xx-xx-xx-xx-xx | - To program MAC address, press Flex BTN 1, enter the MAC address and press the [HOLD/SAVE] button for updating database permanently. |
| 01 IP ADDR DISP xx.xxx.xxx.xxx | - Press Flex BTN 2 to check the IP address. Then IP address will be displayed. |
| 01 PORT VIEW D(xxxx) S(xxxx) C(xxxx) | - Press Flex BTN 3 to check Station and CO number. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| F1 | MAC ID |  | 0.0.0.0.0.0- | [*] : A / [#] : B [CB] : C / [MUTE] : D [DND] : E / [FLASH] : F |
| F2 | IP Address View |  |  |  |
| F3 | RSG Port View |  |  | D(xxxx) S(xxxx) C(xxx) |
| F4 | Port Number View |  |  |  |
| F5 | NAT IP Address View |  |  |  |
| F6 | NAT Port Number View |  |  |  |
| F7 | STUN Enabled View |  |  | None, PAT, NAT, NAT/PAT |

TABLE 5.4.1	  RSG Attributes (PGM 383)

## 5.5	RSG ATTRIBUTE 2  (PGM 384)

The following is the attributes of RSG.

PROCEDURE

| RSG ATTR2 ENTER RANGE(01-96) | - [TRANS/PGM] + 384. Enter the RSG range |
| --- | --- |
| 01-01 RSGM ATTR2 PRESS FLEX (1-11) | - To program, press Flex BTN 1-10 for setting each value. After pressing a Flex BTN, the revised value can be set by entered digit. |
| 01-01 I-MOH RTP PORT 8186 |  |
| 01-01 E-MOH RTP PORT 8188 |  |
| 01-01 MOH TYPE (1:MUSIC/0:H-TN):MUSIC |  |
| 01-01 MUSIC SOURCE (1:/EXT1/0:INT): INT |  |
| 01-01 EXT CONTACT 1 …. |  |
| 01-01 EXT CONTACT 2 …. |  |
| 01-01 ALARM ENABLE (1:ON/0:OFF) OFF |  |
| 01-01 ALARM CONTACT (1:CLOSE/0:OPEN): CLOSE |  |
| 01-01 ALARM MODE (1:ALARM/0:BELL): ALARM |  |
| 01-01 ALARM SIGNAL (1:RPT/0:ONCE): RPT |  |
| 01-01 CTI PORT NOT_USED (0-2) |  |
| RSGM ATTR2 ENTER RANGE (01-96) | - Press the [HOLD/SAVE] button for updating database permanently. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| F1 | RTP Port number of Internal MOH |  | 8186 |  |
| F2 | RTP Port number of External MOH |  | 8188 |  |
| F3 | MOH Type | MUSIC/Hold Tone | Hole Tone |  |
| F4 | Music Source | EXT1/INT | INT |  |
| F5 | External Contact 1 | LBC/Door Open | Not Assigned |  |
| F6 | External Contact 2 | LBC/Door Open | Not Assigned |  |
| F7 | Alarm Enable | ON/OFF | OFF |  |
| F8 | Alarm Contact Type | Close/Open | Close |  |
| F9 | Alarm/Door Bell Mode | Alarm/Door Bell | Alarm |  |
| F10 | Alarm Signal | RPT/ONCE | RPT |  |
| F11 | CTI Port | 0-2 | 0 | CTI port to be assigned in RSG ( 0 – Not Used 1 – DKTU in RSG 2 – SLT in RSG) |
| F12 | RSG NATION CODE |  | System Nation |  |

TABLE 5.5.1	  RSG Attributes (PGM 384)

## 5.6	RSG ALARM ASSIGNMENT  (PGM 385)

The station can receive the alarm ring when the alarm on RSG is detected.

PROCEDURE

| RSG ALARM ATT ENTER STA RANGE | - [TRANS/PGM] + 385. Enter the station range |
| --- | --- |
| SELECT RSG ALARM ZONE F1~F4 (4*24) | - Press FLEX btn to select RSG Alarm Zone. Then LEDs of BTNs show currently assigned RSG alarm zone of the first station in range. To assign alarm, press the BTNs for toggle setting. |
| 100-100 (RSG 01-24) PRESS FLEX KEY (01-24) |  |
| RSG ALARM ATT ENTER STA RANGE | - Press the [HOLD/SAVE] button for updating database permanently. |

| BTN | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- |
| F1 | RSG 01~24 | None |  |
| F2 | RSG 25~48 | None |  |
| F3 | RSG 49~72 | None |  |
| F4 | RSG 73~96 | None |  |

TABLE 5.6.1	  RSGM Attributes (PGM 385)

## 5.7	RSG ATTRIBUTE 1  (PGM 386)

The following is the attributes of IP Phone Attribute.

PROCEDURE

| IP PHONE ATTR ENTER NO (01-96) | - [TRANS/PGM] + 383. Enter the RSG number |
| --- | --- |
| 01 IP PHONE ATTR PRESS FLEX (1-8) |  |
| 01 SET MAC ADDR xx-xx-xx-xx-xx-xx | - To program MAC address, press Flex BTN 1, enter the MAC address and press the [HOLD/SAVE] button for updating database permanently. |
| 01 IP ADDR DISP xx.xxx.xxx.xxx | - Press Flex BTN 2 to check the IP address. Then IP address will be displayed. |
| 01 PORT VIEW D(xxxx) S(xxxx) C(xxxx) | - Press Flex BTN 3 to check Station. |

| BTN | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| F1 | MAC ID |  | 0.0.0.0.0.0- | [*] : A / [#] : B [CB] : C / [MUTE] : D [DND] : E / [FLASH] : F |
| F2 | IP Address View |  |  |  |
| F3 | IP Phone Station Number View |  |  | STA (xxxx) |
| F4 | Port Number View |  |  |  |
| F5 | NAT IP Address View |  |  |  |
| F6 | NAT Port Number View |  |  |  |
| F7 | STUN Enabled View |  |  | None, PAT, NAT, NAT/PAT |
| F8 | CTI IP Address |  |  | CTI IP Address to support the first party CTI |

TABLE 5.7.1	  IP Phone Attributes (PGM 386)

## 5.8	EXPANDED FLEXIBLE NUMBERING PLAN  (PGM 109)

To serve the expanded flexible numbering plan from PGM106 & 107, PGM 109 is added.

PROCEDURE

| FLEX NUMBERING PLAN C PRESS FLEX KEY (1-3) | - [TRANS/PGM] + 109. You can program the 1 Flex. Numbers. Table 2.9.1 illustrates the programmable list of the 1 flexible Numbering plan used by PGM 109. |
| --- | --- |
| MCID REQUEST ENTER NEW #(*0) | - To change a numbering plan, press the related flexible button. If you press other Flex. BTN, you can assign other numbering plan. |
| MCID REQUEST ENTER NEW #(*0) | - If you want to save all changed flexible numbers to system memory, press the [HOLD/SAVE] button. There are no errors in the Flexible Numbers, then confirmation tone will be heard. |
|  | - If some errors are detected, then error tone will be heard without updating system memory. |

| BTN | ITEM | DEFAULT | REMARK |
| --- | --- | --- | --- |
| F1 | MCID REQUEST | *0 | ISDN supplementary service -Malicious Caller ID request. |
| F2 | RSG Door Open 1 | *1 | This can be activated from RSG stations on his RSG. |
| F3 | RSG Door Open 2 | *2 |  |

TABLE 5.8.1	  Expanded Flexible Numbering Plan  (PGM 109)

## 5.9	RSG DKT RX GAIN CONTROL  (PGM 390)

The RX gain of DKT on RSG can be adjusted.

PROCEDURE

| RSG_DKT RX GAIN PRESS FLEX KEY (01-16) | - [TRANS/PGM] + 390. |
| --- | --- |
| RSG_DKT RX FROM DKTU (00-63) : 25 | - Press one of Flex. BTNs (1~16) to select a device type to change the gain. (Ex. Press Flex. BTN 1.) The LCD shows RX gain of the device from other devices. |
| RSG_DKT RX FROM DKTU (00-63) : 45 | - To change the gain dial new gain (00-63) and LCD shows the changed value. (Ex. dial 45.) |
| RSG_DKT RX GAIN PRESS FLEX KEY (01-16) | - Press the [HOLD/SAVE] button for updating database permanently. |
| RSG_DKT RX GAIN PRESS FLEX KEY (01-16) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| FLEX | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | RSG_DKT RX from DKTU | 00 – 63 |  |  |
| 2 | RSG_DKT RX from SLT | 00 – 63 |  |  |
| 3 | RSG_DKT RX from CTR_SLT | 00 – 63 |  |  |
| 4 | RSG_DKT RX from WKT | 00 – 63 |  |  |
| 5 | RSG_DKT RX from ACO | 00 – 63 |  |  |
| 6 | RSG_DKT RX from CTR_ACO | 00 – 63 |  |  |
| 7 | RSG_DKT RX from DCO | 00 – 63 |  |  |
| 8 | RSG_DKT RX from VMIB | 00 – 63 |  |  |
| 9 | RSG_DKT RX from DTMF | 00 – 63 |  |  |
| 10 | RSG_DKT RX from TONE | 00 – 63 |  |  |
| 11 | RSG_DKT RX from MUSIC 1 | 00 – 63 |  |  |
| 12 | RSG_DKT RX from MUSIC 2 | 00 – 63 |  |  |
| 13 | RSG_DKT RX from RSG_DKT | 00 – 63 |  |  |
| 14 | RSG_DKT RX from RSG_SLT | 00 – 63 |  |  |
| 15 | RSG_DKT RX from RSG_LCO | 00 – 63 |  |  |
| 16 | RSG_DKT RX from IP Phone | 00 – 63 |  |  |

TABLE 5.9.1	  RSG_DKT RX Gain  (PGM 390)

## 5.10	RSG DKT TX GAIN CONTROL  (PGM 391)

The TX gain of DKT on RSG can be adjusted.

PROCEDURE

| RSG_DKT TX GAIN PRESS FLEX KEY (1-8) | - [TRANS/PGM] + 391. |
| --- | --- |
| RSG_DKT TX TO DKTU (00-63) : 25 | - Press one of Flex. BTNs (1~8) to select a device type to change the gain. (Ex. Press Flex. BTN 1.) The LCD shows TX gain of the device from other devices. |
| RSG_DKT TX TO DKTU (00-63) : 25 | - To change the gain dial new gain (00-63) and LCD shows the changed value. (Ex. dial 45.) |
| RSG_DKT TX GAIN PRESS FLEX KEY (1-8) | - Press the [HOLD/SAVE] button for updating database permanently. |
| RSG_DKT TX GAIN PRESS FLEX KEY (1-8) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| FLEX | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | RSG_DKT TX to DKTU | 00 – 63 |  |  |
| 2 | RSG_DKT TX to SLT | 00 – 63 |  |  |
| 3 | RSG_DKT TX to CTR_SLT | 00 – 63 |  |  |
| 4 | RSG_DKT TX to WKT | 00 – 63 |  |  |
| 5 | RSG_DKT TX to ACO | 00 – 63 |  |  |
| 6 | RSG_DKT TX to CTR_ACO | 00 – 63 |  |  |
| 7 | RSG_DKT TX to DCO | 00 – 63 |  |  |
| 8 | RSG_DKT TX to DVU | 00 – 63 |  |  |

TABLE 5.10.1	  RSG_DKT TX Gain  (PGM 391)

## 5.11	RSG SLT RX GAIN CONTROL  (PGM 392)

The RX gain of SLT on RSG can be adjusted.

PROCEDURE

| RSG_SLT RX GAIN PRESS FLEX KEY (01-16) | - [TRANS/PGM] + 390. |
| --- | --- |
| RSG_SLT RX FROM DKTU (00-63) : 25 | - Press one of Flex. BTNs (1~16) to select a device type to change the gain. (Ex. Press Flex. BTN 1.) The LCD shows RX gain of the device from other devices. |
| RSG_SLT RX FROM DKTU (00-63) : 45 | - To change the gain dial new gain (00-63) and LCD shows the changed value. (Ex. dial 45.) |
| RSG_SLT RX GAIN PRESS FLEX KEY (01-16) | - Press the [HOLD/SAVE] button for updating database permanently. |
| RSG_SLT RX GAIN PRESS FLEX KEY (01-16) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| FLEX | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | RSG_SLT RX from DKTU | 00 – 63 |  |  |
| 2 | RSG_SLT RX from SLT | 00 – 63 |  |  |
| 3 | RSG_SLT RX from CTR_SLT | 00 – 63 |  |  |
| 4 | RSG_SLT RX from WKT | 00 – 63 |  |  |
| 5 | RSG_SLT RX from ACO | 00 – 63 |  |  |
| 6 | RSG_SLT RX from CTR_ACO | 00 – 63 |  |  |
| 7 | RSG_SLT RX from DCO | 00 – 63 |  |  |
| 8 | RSG_SLT RX from VMIB | 00 – 63 |  |  |
| 9 | RSG_SLT RX from DTMF | 00 – 63 |  |  |
| 10 | RSG_SLT RX from TONE | 00 – 63 |  |  |
| 11 | RSG_SLT RX from MUSIC 1 | 00 – 63 |  |  |
| 12 | RSG_SLT RX from MUSIC 2 | 00 – 63 |  |  |
| 13 | RSG_SLT RX from RSG_DKT | 00 – 63 |  |  |
| 14 | RSG_SLT RX from RSG_SLT | 00 – 63 |  |  |
| 15 | RSG_SLT RX from RSG_LCO | 00 – 63 |  |  |
| 16 | RSG_SLT RX from IP Phone | 00 – 63 |  |  |

TABLE 5.11.1	  RSG_DKT RX Gain  (PGM 392)

## 5.12	RSG SLT TX GAIN CONTROL  (PGM 393)

The TX gain of SLT on RSG can be adjusted.

PROCEDURE

| RSG_SLT TX GAIN PRESS FLEX KEY (1-8) | - [TRANS/PGM] + 391. |
| --- | --- |
| RSG_SLT TX TO DKTU (00-63) : 25 | - Press one of Flex. BTNs (1~8) to select a device type to change the gain. (Ex. Press Flex. BTN 1.) The LCD shows TX gain of the device from other devices. |
| RSG_SLT TX TO DKTU (00-63) : 25 | - To change the gain dial new gain (00-63) and LCD shows the changed value. (Ex. dial 45.) |
| RSG_SLT TX GAIN PRESS FLEX KEY (1-8) | - Press the [HOLD/SAVE] button for updating database permanently. |
| RSG_SLT TX GAIN PRESS FLEX KEY (1-8) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| FLEX | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | RSG_SLT TX to DKTU | 00 – 63 |  |  |
| 2 | RSG_SLT TX to SLT | 00 – 63 |  |  |
| 3 | RSG_SLT TX to CTR_SLT | 00 – 63 |  |  |
| 4 | RSG_SLT TX to WKT | 00 – 63 |  |  |
| 5 | RSG_SLT TX to ACO | 00 – 63 |  |  |
| 6 | RSG_SLT TX to CTR_ACO | 00 – 63 |  |  |
| 7 | RSG_SLT TX to DCO | 00 – 63 |  |  |
| 8 | RSG_SLT TX to DVU | 00 – 63 |  |  |

TABLE 5.12.1	  RSG_SLT TX Gain  (PGM 393)

## 5.13	RSG LCO RX GAIN CONTROL  (PGM 394)

The RX gain of LCO on RSG can be adjusted.

PROCEDURE

| RSG_LCO RX GAIN PRESS FLEX KEY (01-16) | - [TRANS/PGM] + 394. |
| --- | --- |
| RSG_LCO RX FROM DKTU (00-63) : 25 | - Press one of Flex. BTNs (1~16) to select a device type to change the gain. (Ex. Press Flex. BTN 1.) The LCD shows RX gain of the device from other devices. |
| RSG_LCO RX FROM DKTU (00-63) : 45 | - To change the gain dial new gain (00-63) and LCD shows the changed value. (Ex. dial 45.) |
| RSG_LCO RX GAIN PRESS FLEX KEY (01-16) | - Press the [HOLD/SAVE] button for updating database permanently. |
| RSG_LCO RX GAIN PRESS FLEX KEY (01-16) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| FLEX | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | RSG_LCO RX from DKTU | 00 – 63 |  |  |
| 2 | RSG_LCO RX from SLT | 00 – 63 |  |  |
| 3 | RSG_LCO RX from CTR_SLT | 00 – 63 |  |  |
| 4 | RSG_LCO RX from WKT | 00 – 63 |  |  |
| 5 | RSG_LCO RX from ACO | 00 – 63 |  |  |
| 6 | RSG_LCO RX from CTR_ACO | 00 – 63 |  |  |
| 7 | RSG_LCO RX from DCO | 00 – 63 |  |  |
| 8 | RSG_LCO RX from VMIB | 00 – 63 |  |  |
| 9 | RSG_LCO RX from DTMF | 00 – 63 |  |  |
| 10 | RSG_LCO RX from TONE | 00 – 63 |  |  |
| 11 | RSG_LCO RX from MUSIC 1 | 00 – 63 |  |  |
| 12 | RSG_LCO RX from MUSIC 2 | 00 – 63 |  |  |
| 13 | RSG_LCO RX from RSG_DKT | 00 – 63 |  |  |
| 14 | RSG_LCO RX from RSG_SLT | 00 – 63 |  |  |
| 15 | RSG_LCO RX from RSG_LCO | 00 – 63 |  |  |
| 16 | RSG_LCO RX from IP Phone | 00 – 63 |  |  |

TABLE 5.9.1	  RSG_LCO RX Gain  (PGM 394)

## 5.14	RSG LCO TX GAIN CONTROL  (PGM 395)

The TX gain of LCO on RSG can be adjusted.

PROCEDURE

| RSG_LCO TX GAIN PRESS FLEX KEY (1-8) | - [TRANS/PGM] + 391. |
| --- | --- |
| RSG_LCO TX TO DKTU (00-63) : 25 | - Press one of Flex. BTNs (1~8) to select a device type to change the gain. (Ex. Press Flex. BTN 1.) The LCD shows TX gain of the device from other devices. |
| RSG_LCO TX TO DKTU (00-63) : 25 | - To change the gain dial new gain (00-63) and LCD shows the changed value. (Ex. dial 45.) |
| RSG_LCO TX GAIN PRESS FLEX KEY (1-8) | - Press the [HOLD/SAVE] button for updating database permanently. |
| RSG_LCO TX GAIN PRESS FLEX KEY (1-8) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| FLEX | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | RSG_LCO TX to DKTU | 00 – 63 |  |  |
| 2 | RSG_LCO TX to SLT | 00 – 63 |  |  |
| 3 | RSG_LCO TX to CTR_SLT | 00 – 63 |  |  |
| 4 | RSG_LCO TX to WKT | 00 – 63 |  |  |
| 5 | RSG_LCO TX to ACO | 00 – 63 |  |  |
| 6 | RSG_LCO TX to CTR_ACO | 00 – 63 |  |  |
| 7 | RSG_LCO TX to DCO | 00 – 63 |  |  |
| 8 | RSG_LCO TX to DVU | 00 – 63 |  |  |

TABLE 5.10.1	  RSG_LCO TX Gain  (PGM 395)

## 5.15	IP PHNOE RX GAIN CONTROL  (PGM 396)

The RX gain of IP PHONE can be adjusted.

PROCEDURE

| IP PHONE RX GAIN PRESS FLEX KEY (01-16) | - [TRANS/PGM] + 390. |
| --- | --- |
| IP PHONE RX FROM DKTU (00-63) : 25 | - Press one of Flex. BTNs (1~16) to select a device type to change the gain. (Ex. Press Flex. BTN 1.) The LCD shows RX gain of the device from other devices. |
| IP PHONE RX FROM DKTU (00-63) : 45 | - To change the gain dial new gain (00-63) and LCD shows the changed value. (Ex. dial 45.) |
| IP PHONE RX GAIN PRESS FLEX KEY (01-16) | - Press the [HOLD/SAVE] button for updating database permanently. |
| IP PHONE RX GAIN PRESS FLEX KEY (01-16) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| FLEX | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | IP PHONE RX from DKTU | 00 – 63 |  |  |
| 2 | IP PHONE RX from SLT | 00 – 63 |  |  |
| 3 | IP PHONE RX from CTR_SLT | 00 – 63 |  |  |
| 4 | IP PHONE RX from WKT | 00 – 63 |  |  |
| 5 | IP PHONE RX from ACO | 00 – 63 |  |  |
| 6 | IP PHONE RX from CTR_ACO | 00 – 63 |  |  |
| 7 | IP PHONE RX from DCO | 00 – 63 |  |  |
| 8 | IP PHONE RX from VMIB | 00 – 63 |  |  |
| 9 | IP PHONE RX from DTMF | 00 – 63 |  |  |
| 10 | IP PHONE RX from TONE | 00 – 63 |  |  |
| 11 | IP PHONE RX from MUSIC 1 | 00 – 63 |  |  |
| 12 | IP PHONE RX from MUSIC 2 | 00 – 63 |  |  |
| 13 | IP PHONE RX from RSG_DKT | 00 – 63 |  |  |
| 14 | IP PHONE RX from RSG_SLT | 00 – 63 |  |  |
| 15 | IP PHONE RX from RSG_LCO | 00 – 63 |  |  |
| 16 | IP PHONE RX from IP Phone | 00 – 63 |  |  |

TABLE 5.15.1	  IP Phone RX Gain  (PGM 396)

## 5.16	IP PHONE TX GAIN CONTROL  (PGM 397)

The TX gain of IP PHONE can be adjusted.

PROCEDURE

| IP PHONE TX GAIN PRESS FLEX KEY (1-8) | - [TRANS/PGM] + 391. |
| --- | --- |
| IP PHONE TX TO DKTU (00-63) : 25 | - Press one of Flex. BTNs (1~8) to select a device type to change the gain. (Ex. Press Flex. BTN 1.) The LCD shows TX gain of the device from other devices. |
| IP PHONE TX TO DKTU (00-63) : 25 | - To change the gain dial new gain (00-63) and LCD shows the changed value. (Ex. dial 45.) |
| IP PHONE TX GAIN PRESS FLEX KEY (1-8) | - Press the [HOLD/SAVE] button for updating database permanently. |
| IP PHONE TX GAIN PRESS FLEX KEY (1-8) | - Press the [CONF] button instead of the [HOLD/SAVE] button, then system goes to step (1) without updating system memory. |

| FLEX | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | IP PHONE RX to DKTU | 00 – 63 |  |  |
| 2 | IP PHONE RX to SLT | 00 – 63 |  |  |
| 3 | IP PHONE RX to CTR_SLT | 00 – 63 |  |  |
| 4 | IP PHONE RX to WKT | 00 – 63 |  |  |
| 5 | IP PHONE RX to ACO | 00 – 63 |  |  |
| 6 | IP PHONE RX to CTR_ACO | 00 – 63 |  |  |
| 7 | IP PHONE RX to DCO | 00 – 63 |  |  |
| 8 | IP PHONE RX to DVU | 00 – 63 |  |  |

TABLE 5.16.1	  IP PHONE TX Gain  (PGM 397)

## 5.17	INIT BY MPB VERSION  (PGM 452)

To initialize database, PGM 452 is added.

PROCEDURE

| INIT BY MPB VERSION PRESS FLEX KEY (1-3) | - [TRANS/PGM] + 452. You can initialize the database. |
| --- | --- |
| INIT BY MPB VERSION VERSION 2.5 | - To initialize database with version2.5, press the third flexible button. |
| MCID REQUEST ENTER NEW #(*0) | - If you want to initialize the database related to the version 2.5, press the [HOLD/SAVE] button. |

| BTN | ITEM | DEFAULT | REMARK |
| --- | --- | --- | --- |
| F1 | INIT VERSION 2.2 |  |  |
| F2 | INIT VERSION 2.3 |  |  |
| F3 | INIT VERSION 2.5 |  |  |

# Appendix A. RSG Installation Procedure on MPB

- Insert the following boards into the LDK-System

  - MPB

  - Station Boards

  - CO Boards

  - VOIB

- Power on the system (All switches of MPB are set to ON.)

- Set the 8th DIP-switch to OFF to preserve the programming data.

- At the admin station (100)

  - Enter the VOI Slot to be used for RSG in PGM 380

  - Enter the RSG No. (or, IP Phone No.) in PGM 381

  - Enter the VOIB slot # on STA/COL Board in PGM 103/BTN 1 & 2.

(If several VOIBs will be used for RSG, enter the 1st VOIB slot # in PGM 380)

- Reset the system for board configuration

- Re-enter the admin programming mode

  - Enter the MAC ID in PGM 383 (or, 386)

  - Enter the IP Address and Gateway of VOIB in PGM 340.

- Reset the RSG (or, IP Phone)

** If the RSG no (or, IP Phone No) is changed, reset the system for board reconfiguration

# Appendix B. RSG Setting Procedure on RSG

1. baud rate : 38400

2. Set Master’s IP Address :

1) Press ‘Enter’ Key, the ‘main:>’ is out after menu list

2) Type ‘system’ and enter.

3) Type ‘set master xxx.xxx.xxx.xxx’. The ‘xxx.xxx.xxx.xxx.’ is IP address.

Ex) main:/system> set master xxx.xxx.xxx.xxx

4) Return to ‘main:>’ mode by typing ‘..’ and entering twice.

3. Set the WAN and get information (When setting Fixed IP address)

1) At ‘main:>’, type ‘wan’ and confirm to display the help menu.

2) At ‘main:/wan’, type ‘wanset’ and it displays the help menu and the current value of mode.

3) As the help menu says, type the value of IP address and Gateway address as well as other setting value.

Ex) main:/wan/wanset> set ip xxx.xxx.xxx.xxx

Ex) main:/wan/wanset> set gateway xxx.xxx.xxx.xxx

4) If you want to verify the changed value, please type ‘get all’.

5) The mode of wan has to be FIXED. Verify the mode by typing ‘mode’ or ‘get all’.

Ex) main:/wan/wanset> get all

4. Set the PPPoE and get information (When setting Dynamic IP address)

1) Change the mode of WAN from FIXED into DYNAMIC by typing ‘mode’ at ‘main:/wan/wanset’.

Ex) main:/wan/wanset> mode

2) Change the mode of PPPoE from INACTIVE into ACTIVE by typing ‘mode’ at ‘main:/wan/pppoe’.

Ex) main:/wan/pppoe> mode

3) As the help menu says, type the value of ID and password.

Ex) main:/wan/pppoe> set user xxxxx

Ex) main:/wan/pppoe> set passwd xxxxx

4) If you want to verify the changed value, please type ‘get all’.

5) The ID and password are dedicated value by ADSL provider.

5. Select trace of call traffic

1) The board trace mode is toggled whenever typing ‘bt’.

# Appendix C. IP Phone Setting Procedure

When power up the IP Phone, you will see the following message in LCD.

NO RESPONSE FROM MFIM[L] SET[*] - RETRY[#]

1. If you want to retry, press [#].

Searching MFIM[2]

2. If you want to change the value related with registration, press [*]

You can move the menu pressing [Volume Up] or [Volume Down].

2.1 You can change the mode (LOCAL/REMOTE).

MODE[R/L] [LOCAL] - CHANGE[#]

LOCAL Mode: Phone is connected with VOIB not passing through Router.

REMOTE Mode: Phone is connected with VOIB  passing through Router. (Recommend)

2.2 You can change the DHCP setting (DISABLED/ENABLED).

DHCP ? [DISABLED] - CHANGE[#]

ENABLED: enable DHCP Mode

DISABLED: Disable DHCP Mode

2.3 You can change the Device number( Not Used)

INPUT DEVICE NUMBER :

2.4 You can enter or change VOIB IP Address.

MFIM IP ADDRESS(DOT:*) 150.150.54.134

2.5 You can enter or change Router IP Address.

ROUTER IP ADDRESS(DOT:*) 150.150.54.254

2.6 You can enter or change Net Mask.

PHONE NET MASK(DOT:*) 255.255.255.0

2.7 You can enter or change Phone IP Address

PHONE IP ADDRESS(DOT:*) 150.150.54.135

# Appendix D. RSG / IP Phone Default Gain Table

/* Default Gain */

| RX From | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU | DTMF | TONE | M1 | M2 | RSG_DKT | RSG_SLT | RSG_LCO | IPKTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 26 | 33 | 22 | 26 | 33 | 22 | 33 | 29 | 8 | 32 | 29 | 29 | 26 | 22 | 22 | 26 |
| RSG_SLT | 32 | 43 | 32 | 32 | 41 | 32 | 44 | 40 | 28 | 38 | 40 | 40 | 32 | 32 | 32 | 32 |
| RSG_LCO | 28 | 43 | 32 | 31 | 41 | 32 | 38 | 37 | 26 | 37 | 37 | 37 | 28 | 32 | 32 | 32 |
| IP Phone | 26 | 33 | 22 | 26 | 33 | 22 | 33 | 29 | 8 | 32 | 29 | 29 | 26 | 22 | 22 | 26 |

/* UK Gain */

| RX From | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU | DTMF | TONE | M1 | M2 | RSG_DKT | RSG_SLT | RSG_LCO | IPKTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 29 | 24 | 25 | 37 | 32 | 26 | 20 | 8 | 32 | 29 | 29 | 25 | 24 | 32 | 25 |
| RSG_SLT | 32 | 37 | 32 | 32 | 37 | 32 | 32 | 32 | 8 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| RSG_LCO | 32 | 33 | 32 | 26 | 33 | 22 | 33 | 29 | 26 | 32 | 29 | 29 | 32 | 32 | 22 | 32 |
| IP Phone | 25 | 29 | 24 | 25 | 37 | 32 | 26 | 20 | 8 | 32 | 29 | 29 | 25 | 24 | 32 | 25 |

| TX To | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 26 | 33 | 22 | 26 | 33 | 22 | 33 | 29 |
| RSG_SLT | 26 | 33 | 22 | 26 | 33 | 22 | 33 | 29 |
| RSG_LCO | 26 | 33 | 22 | 26 | 33 | 22 | 33 | 29 |
| IP Phone | 26 | 33 | 22 | 26 | 33 | 22 | 33 | 29 |

| TX To | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 24 | 32 | 25 | 24 | 32 | 24 | 26 |
| RSG_SLT | 24 | 24 | 32 | 24 | 24 | 32 | 32 | 32 |
| RSG_LCO | 32 | 24 | 32 | 32 | 24 | 32 | 25 | 32 |
| IP Phone | 25 | 24 | 32 | 25 | 24 | 32 | 24 | 26 |

/* Sweden Gain */

| RX From | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU | DTMF | TONE | M1 | M2 | RSG_DKT | RSG_SLT | RSG_LCO | IPKTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 21 | 32 | 25 | 24 | 32 | 26 | 20 | 8 | 32 | 29 | 29 | 25 | 24 | 32 | 25 |
| RSG_SLT | 32 | 33 | 32 | 32 | 24 | 32 | 32 | 32 | 8 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| RSG_LCO | 32 | 33 | 32 | 32 | 24 | 32 | 32 | 32 | 26 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| IP Phone | 25 | 25 | 24 | 25 | 24 | 32 | 26 | 20 | 8 | 32 | 29 | 29 | 25 | 24 | 32 | 25 |

| TX To | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 24 | 32 | 25 | 35 | 32 | 24 | 26 |
| RSG_SLT | 24 | 21 | 32 | 24 | 35 | 32 | 32 | 32 |
| RSG_LCO | 32 | 21 | 32 | 32 | 35 | 32 | 25 | 32 |
| IP Phone | 25 | 24 | 32 | 25 | 35 | 32 | 24 | 26 |

/* Norway Gain */

| RX From | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU | DTMF | TONE | M1 | M2 | RSG_DKT | RSG_SLT | RSG_LCO | IPKTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 30 | 24 | 25 | 25 | 32 | 26 | 20 | 8 | 32 | 29 | 29 | 25 | 24 | 32 | 25 |
| RSG_SLT | 32 | 40 | 32 | 25 | 23 | 32 | 32 | 32 | 8 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| RSG_LCO | 32 | 38 | 32 | 32 | 26 | 32 | 32 | 32 | 26 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| IP Phone | 25 | 30 | 24 | 25 | 25 | 32 | 26 | 20 | 8 | 32 | 29 | 29 | 25 | 24 | 32 | 25 |

| TX To | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 24 | 32 | 25 | 35 | 32 | 24 | 26 |
| RSG_SLT | 24 | 40 | 32 | 24 | 23 | 32 | 32 | 32 |
| RSG_LCO | 32 | 40 | 32 | 32 | 23 | 32 | 25 | 32 |
| IP Phone | 25 | 24 | 32 | 25 | 35 | 32 | 24 | 26 |

/* Italy Gain */

| RX From | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU | DTMF | TONE | M1 | M2 | RSG_DKT | RSG_SLT | RSG_LCO | IPKTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 24 | 24 | 25 | 34 | 32 | 26 | 20 | 8 | 32 | 29 | 29 | 25 | 24 | 32 | 25 |
| RSG_SLT | 32 | 32 | 32 | 32 | 34 | 32 | 32 | 32 | 8 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| RSG_LCO | 26 | 33 | 32 | 26 | 33 | 22 | 33 | 29 | 26 | 32 | 29 | 29 | 32 | 32 | 32 | 32 |
| IP Phone | 25 | 24 | 24 | 25 | 34 | 32 | 26 | 20 | 8 | 32 | 29 | 29 | 25 | 24 | 32 | 25 |

| TX To | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 32 | 32 | 25 | 34 | 32 | 24 | 26 |
| RSG_SLT | 24 | 32 | 32 | 24 | 34 | 32 | 32 | 32 |
| RSG_LCO | 32 | 32 | 32 | 32 | 34 | 32 | 25 | 32 |
| IP Phone | 25 | 32 | 32 | 25 | 34 | 32 | 24 | 26 |

/* Netherlands Gain */

| RX From | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU | DTMF | TONE | M1 | M2 | RSG_DKT | RSG_SLT | RSG_LCO | IPKTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 27 | 24 | 25 | 44 | 32 | 26 | 20 | 8 | 32 | 29 | 29 | 25 | 24 | 32 | 25 |
| RSG_SLT | 32 | 35 | 32 | 32 | 44 | 32 | 32 | 32 | 8 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| RSG_LCO | 32 | 35 | 32 | 32 | 44 | 32 | 32 | 32 | 26 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| IP Phone | 25 | 27 | 24 | 25 | 44 | 32 | 26 | 20 | 8 | 32 | 29 | 29 | 25 | 24 | 32 | 25 |

| TX To | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 12 | 132 | 25 | 26 | 32 | 24 | 32 |
| RSG_SLT | 24 | 12 | 32 | 24 | 26 | 32 | 32 | 32 |
| RSG_LCO | 32 | 12 | 32 | 32 | 34 | 32 | 25 | 32 |
| IP Phone | 25 | 12 | 132 | 25 | 26 | 32 | 24 | 32 |

/* NZ Gain */

| RX From | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU | DTMF | TONE | M1 | M2 | RSG_DKT | RSG_SLT | RSG_LCO | IPKTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 10 | 10 | 8 | 10 | 32 | 15 | 32 | 32 | 8 | 32 | 32 | 32 | 10 | 8 | 15 | 10 |
| RSG_SLT | 31 | 27 | 23 | 31 | 49 | 32 | 50 | 53 | 29 | 53 | 53 | 53 | 31 | 23 | 32 | 31 |
| RSG_LCO | 37 | 33 | 29 | 37 | 53 | 38 | 37 | 37 | 26 | 37 | 53 | 53 | 37 | 29 | 38 | 37 |
| IP Phone | 10 | 10 | 8 | 10 | 32 | 15 | 32 | 32 | 8 | 32 | 32 | 32 | 10 | 8 | 15 | 10 |

| TX To | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 10 | 10 | 31 | 10 | 32 | 37 | 32 | 32 |
| RSG_SLT | 8 | 7 | 23 | 8 | 24 | 29 | 23 | 28 |
| RSG_LCO | 15 | 11 | 32 | 20 | 33 | 38 | 14 | 23 |
| IP Phone | 10 | 10 | 31 | 10 | 32 | 37 | 32 | 32 |

/* Australia Gain */

| RX From | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU | DTMF | TONE | M1 | M2 | RSG_DKT | RSG_SLT | RSG_LCO | IPKTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 22 | 22 | 19 | 22 | 14 | 14 | 30 | 22 | 8 | 32 | 32 | 32 | 22 | 19 | 14 | 22 |
| RSG_SLT | 36 | 35 | 32 | 36 | 32 | 32 | 32 | 36 | 18 | 46 | 36 | 36 | 36 | 32 | 32 | 36 |
| RSG_LCO | 31 | 36 | 33 | 35 | 33 | 33 | 34 | 32 | 26 | 36 | 36 | 36 | 31 | 33 | 33 | 31 |
| IP Phone | 22 | 22 | 19 | 22 | 14 | 14 | 30 | 22 | 8 | 32 | 32 | 32 | 22 | 19 | 14 | 22 |

| TX To | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 22 | 36 | 36 | 22 | 27 | 31 | 31 | 32 |
| RSG_SLT | 19 | 32 | 32 | 19 | 29 | 33 | 29 | 29 |
| RSG_LCO | 14 | 32 | 32 | 19 | 29 | 33 | 32 | 24 |
| IP Phone | 22 | 36 | 36 | 22 | 27 | 31 | 31 | 32 |

/* Belgium Gain */

| RX From | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU | DTMF | TONE | M1 | M2 | RSG_DKT | RSG_SLT | RSG_LCO | IPKTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 29 | 24 | 25 | 41 | 32 | 26 | 26 | 8 | 32 | 29 | 29 | 25 | 24 | 32 | 25 |
| RSG_SLT | 32 | 37 | 32 | 32 | 41 | 32 | 32 | 32 | 8 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| RSG_LCO | 32 | 37 | 32 | 32 | 41 | 32 | 32 | 32 | 26 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| IP Phone | 25 | 29 | 24 | 25 | 41 | 32 | 26 | 26 | 8 | 32 | 29 | 29 | 25 | 24 | 32 | 25 |

| TX To | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 16 | 32 | 25 | 32 | 32 | 24 | 26 |
| RSG_SLT | 24 | 16 | 32 | 24 | 32 | 32 | 32 | 32 |
| RSG_LCO | 32 | 16 | 32 | 32 | 32 | 32 | 25 | 32 |
| IP Phone | 25 | 16 | 32 | 25 | 32 | 32 | 24 | 26 |

/* Germany Gain */

| RX From | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU | DTMF | TONE | M1 | M2 | RSG_DKT | RSG_SLT | RSG_LCO | IPKTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 35 | 35 | 25 | 29 | 29 | 15 | 26 | 8 | 15 | 26 | 26 | 25 | 35 | 29 | 25 |
| RSG_SLT | 36 | 38 | 38 | 36 | 32 | 32 | 23 | 29 | 17 | 27 | 29 | 29 | 36 | 38 | 32 | 36 |
| RSG_LCO | 35 | 31 | 31 | 35 | 32 | 32 | 15 | 29 | 26 | 29 | 29 | 29 | 35 | 31 | 32 | 35 |
| IP Phone | 25 | 35 | 35 | 25 | 29 | 29 | 15 | 26 | 8 | 15 | 26 | 26 | 25 | 35 | 29 | 25 |

| TX To | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 36 | 36 | 35 | 35 | 35 | 30 | 35 |
| RSG_SLT | 35 | 38 | 38 | 31 | 31 | 31 | 32 | 32 |
| RSG_LCO | 29 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| IP Phone | 25 | 36 | 36 | 35 | 35 | 35 | 30 | 35 |

/* Finland Gain */

| RX From | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU | DTMF | TONE | M1 | M2 | RSG_DKT | RSG_SLT | RSG_LCO | IPKTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 24 | 24 | 25 | 35 | 32 | 26 | 20 | 8 | 32 | 29 | 29 | 25 | 24 | 32 | 25 |
| RSG_SLT | 32 | 32 | 32 | 32 | 35 | 32 | 32 | 32 | 8 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| RSG_LCO | 32 | 32 | 32 | 32 | 35 | 32 | 32 | 32 | 26 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| IP Phone | 25 | 24 | 24 | 25 | 35 | 32 | 26 | 20 | 8 | 32 | 29 | 29 | 25 | 24 | 32 | 25 |

| TX To | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 32 | 32 | 25 | 35 | 32 | 24 | 26 |
| RSG_SLT | 24 | 32 | 32 | 24 | 35 | 32 | 32 | 32 |
| RSG_LCO | 32 | 32 | 32 | 32 | 35 | 32 | 25 | 32 |
| IP Phone | 25 | 32 | 32 | 25 | 35 | 32 | 24 | 26 |

/* Israel Gain */

| RX From | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU | DTMF | TONE | M1 | M2 | RSG_DKT | RSG_SLT | RSG_LCO | IPKTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 24 | 24 | 26 | 17 | 22 | 26 | 20 | 8 | 32 | 29 | 29 | 25 | 24 | 22 | 25 |
| RSG_SLT | 32 | 36 | 36 | 12 | 31 | 36 | 32 | 32 | 8 | 18 | 32 | 32 | 32 | 36 | 36 | 32 |
| RSG_LCO | 25 | 32 | 32 | 27 | 27 | 32 | 33 | 33 | 26 | 33 | 33 | 33 | 25 | 32 | 32 | 25 |
| IP Phone | 25 | 24 | 24 | 265 | 17 | 22 | 26 | 20 | 8 | 32 | 29 | 29 | 25 | 24 | 22 | 25 |

| TX To | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 32 | 28 | 26 | 24 | 25 | 24 | 26 |
| RSG_SLT | 24 | 36 | 32 | 33 | 31 | 32 | 32 | 32 |
| RSG_LCO | 22 | 36 | 32 | 43 | 31 | 32 | 32 | 32 |
| IP Phone | 25 | 32 | 28 | 26 | 24 | 25 | 24 | 26 |

/* Poland Gain */

| RX From | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU | DTMF | TONE | M1 | M2 | RSG_DKT | RSG_SLT | RSG_LCO | IPKTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 26 | 33 | 28 | 26 | 33 | 20 | 33 | 29 | 8 | 32 | 29 | 29 | 26 | 28 | 20 | 26 |
| RSG_SLT | 32 | 37 | 32 | 32 | 45 | 32 | 44 | 40 | 28 | 38 | 40 | 40 | 32 | 32 | 32 | 32 |
| RSG_LCO | 31 | 37 | 32 | 31 | 45 | 32 | 38 | 37 | 26 | 37 | 37 | 37 | 31 | 32 | 32 | 31 |
| IP Phone | 26 | 33 | 28 | 26 | 33 | 20 | 33 | 29 | 8 | 32 | 29 | 29 | 26 | 28 | 20 | 26 |

| TX To | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 26 | 12 | 32 | 26 | 26 | 31 | 26 | 21 |
| RSG_SLT | 28 | 12 | 32 | 28 | 27 | 32 | 32 | 27 |
| RSG_LCO | 20 | 12 | 32 | 25 | 27 | 32 | 11 | 19 |
| IP Phone | 26 | 12 | 32 | 26 | 26 | 31 | 26 | 21 |

/* Spain Gain */

| RX From | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU | DTMF | TONE | M1 | M2 | RSG_DKT | RSG_SLT | RSG_LCO | IPKTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 24 | 24 | 25 | 36 | 32 | 26 | 20 | 8 | 32 | 29 | 29 | 25 | 24 | 32 | 25 |
| RSG_SLT | 32 | 32 | 32 | 32 | 36 | 32 | 32 | 32 | 8 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| RSG_LCO | 32 | 32 | 32 | 31 | 36 | 32 | 32 | 32 | 26 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| IP Phone | 25 | 24 | 24 | 25 | 36 | 32 | 26 | 20 | 8 | 32 | 29 | 29 | 25 | 24 | 32 | 25 |

| TX To | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 32 | 32 | 25 | 32 | 32 | 24 | 26 |
| RSG_SLT | 24 | 32 | 32 | 24 | 32 | 32 | 32 | 32 |
| RSG_LCO | 32 | 32 | 32 | 32 | 32 | 32 | 25 | 32 |
| IP Phone | 25 | 32 | 32 | 25 | 32 | 32 | 24 | 26 |

/* Czech Gain */

| RX From | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU | DTMF | TONE | M1 | M2 | RSG_DKT | RSG_SLT | RSG_LCO | IPKTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 26 | 26 | 26 | 26 | 26 | 26 | 33 | 29 | 8 | 36 | 29 | 29 | 26 | 26 | 26 | 26 |
| RSG_SLT | 26 | 32 | 32 | 12 | 32 | 32 | 24 | 32 | 8 | 18 | 32 | 32 | 26 | 32 | 32 | 26 |
| RSG_LCO | 33 | 32 | 32 | 33 | 34 | 34 | 40 | 39 | 26 | 39 | 39 | 39 | 33 | 32 | 34 | 33 |
| IP Phone | 26 | 26 | 26 | 26 | 26 | 26 | 33 | 29 | 8 | 36 | 29 | 29 | 26 | 26 | 26 | 26 |

| TX To | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 26 | 26 | 26 | 26 | 26 | 33 | 26 | 21 |
| RSG_SLT | 26 | 32 | 32 | 33 | 25 | 32 | 37 | 32 |
| RSG_LCO | 26 | 32 | 32 | 38 | 27 | 34 | 24 | 32 |
| IP Phone | 26 | 26 | 26 | 26 | 26 | 33 | 26 | 21 |

/* Ireland Gain */

| RX From | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU | DTMF | TONE | M1 | M2 | RSG_DKT | RSG_SLT | RSG_LCO | IPKTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 43 | 34 | 25 | 41 | 32 | 26 | 20 | 8 | 32 | 29 | 29 | 25 | 34 | 32 | 25 |
| RSG_SLT | 32 | 51 | 32 | 32 | 41 | 32 | 32 | 32 | 8 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| RSG_LCO | 32 | 51 | 32 | 32 | 41 | 32 | 32 | 32 | 26 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| IP Phone | 25 | 43 | 34 | 25 | 41 | 32 | 26 | 20 | 8 | 32 | 29 | 29 | 25 | 34 | 32 | 25 |

| TX To | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 11 | 32 | 25 | 24 | 32 | 24 | 26 |
| RSG_SLT | 24 | 11 | 32 | 24 | 24 | 32 | 32 | 32 |
| RSG_LCO | 32 | 11 | 32 | 32 | 24 | 32 | 25 | 32 |
| IP Phone | 25 | 11 | 32 | 25 | 24 | 32 | 24 | 26 |

/* Malaysia Gain */

| RX From | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU | DTMF | TONE | M1 | M2 | RSG_DKT | RSG_SLT | RSG_LCO | IPKTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 26 | 33 | 22 | 26 | 33 | 24 | 33 | 29 | 8 | 32 | 29 | 29 | 26 | 22 | 24 | 26 |
| RSG_SLT | 32 | 43 | 32 | 32 | 41 | 32 | 44 | 40 | 28 | 38 | 40 | 40 | 32 | 32 | 32 | 32 |
| RSG_LCO | 31 | 43 | 32 | 31 | 41 | 32 | 38 | 37 | 26 | 37 | 37 | 37 | 31 | 32 | 32 | 31 |
| IP Phone | 26 | 33 | 22 | 26 | 33 | 24 | 33 | 29 | 8 | 32 | 29 | 29 | 26 | 22 | 24 | 26 |

| TX To | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 26 | 12 | 32 | 26 | 26 | 31 | 26 | 21 |
| RSG_SLT | 22 | 12 | 32 | 22 | 27 | 32 | 26 | 21 |
| RSG_LCO | 24 | 12 | 32 | 33 | 27 | 32 | 15 | 23 |
| IP Phone | 26 | 12 | 32 | 26 | 26 | 31 | 26 | 21 |

/* South Africa Gain */

| RX From | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU | DTMF | TONE | M1 | M2 | RSG_DKT | RSG_SLT | RSG_LCO | IPKTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 26 | 35 | 30 | 26 | 26 | 17 | 33 | 29 | 8 | 32 | 29 | 29 | 26 | 30 | 17 | 26 |
| RSG_SLT | 42 | 37 | 32 | 22 | 41 | 32 | 34 | 42 | 38 | 48 | 42 | 42 | 42 | 32 | 34 | 42 |
| RSG_LCO | 30 | 40 | 35 | 30 | 41 | 32 | 37 | 36 | 26 | 42 | 36 | 36 | 30 | 35 | 32 | 30 |
| IP Phone | 26 | 35 | 30 | 26 | 26 | 17 | 33 | 29 | 8 | 32 | 29 | 29 | 26 | 30 | 17 | 26 |

| TX To | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 26 | 32 | 42 | 26 | 26 | 30 | 32 | 21 |
| RSG_SLT | 30 | 22 | 32 | 26 | 31 | 35 | 27 | 27 |
| RSG_LCO | 17 | 22 | 32 | 29 | 28 | 32 | 23 | 23 |
| IP Phone | 26 | 32 | 42 | 26 | 26 | 30 | 32 | 21 |

/* India Gain */

| RX From | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU | DTMF | TONE | M1 | M2 | RSG_DKT | RSG_SLT | RSG_LCO | IPKTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 24 | 24 | 26 | 17 | 17 | 26 | 20 | 8 | 32 | 29 | 29 | 25 | 24 | 17 | 25 |
| RSG_SLT | 32 | 32 | 32 | 12 | 32 | 32 | 32 | 22 | 8 | 50 | 32 | 32 | 32 | 32 | 32 | 32 |
| RSG_LCO | 24 | 32 | 32 | 26 | 32 | 32 | 32 | 32 | 26 | 32 | 32 | 32 | 24 | 32 | 32 | 24 |
| IP Phone | 25 | 24 | 24 | 26 | 17 | 17 | 26 | 20 | 8 | 32 | 29 | 29 | 25 | 24 | 17 | 25 |

| TX To | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 32 | 32 | 26 | 24 | 24 | 24 | 26 |
| RSG_SLT | 24 | 32 | 32 | 33 | 32 | 32 | 32 | 32 |
| RSG_LCO | 17 | 32 | 32 | 38 | 32 | 32 | 26 | 32 |
| IP Phone | 25 | 32 | 32 | 26 | 24 | 24 | 24 | 26 |

/* Denmark Gain */

| RX From | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU | DTMF | TONE | M1 | M2 | RSG_DKT | RSG_SLT | RSG_LCO | IPKTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 24 | 24 | 25 | 32 | 32 | 26 | 20 | 8 | 32 | 29 | 29 | 25 | 24 | 32 | 25 |
| RSG_SLT | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 8 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| RSG_LCO | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 26 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| IP Phone | 25 | 24 | 24 | 25 | 32 | 32 | 26 | 20 | 8 | 32 | 29 | 29 | 25 | 24 | 32 | 25 |

| TX To | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 32 | 32 | 25 | 32 | 32 | 24 | 26 |
| RSG_SLT | 24 | 32 | 32 | 24 | 32 | 32 | 32 | 32 |
| RSG_LCO | 32 | 32 | 32 | 32 | 32 | 32 | 25 | 32 |
| IP Phone | 25 | 32 | 32 | 25 | 32 | 32 | 24 | 26 |

/* Greece Gain */

| RX From | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU | DTMF | TONE | M1 | M2 | RSG_DKT | RSG_SLT | RSG_LCO | IPKTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 35 | 24 | 25 | 41 | 32 | 26 | 20 | 8 | 32 | 29 | 29 | 25 | 24 | 32 | 25 |
| RSG_SLT | 32 | 43 | 32 | 32 | 41 | 32 | 32 | 32 | 8 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| RSG_LCO | 32 | 43 | 32 | 32 | 41 | 32 | 32 | 32 | 26 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| IP Phone | 25 | 35 | 24 | 25 | 41 | 32 | 26 | 20 | 8 | 32 | 29 | 29 | 25 | 24 | 32 | 25 |

| TX To | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 12 | 32 | 25 | 27 | 32 | 24 | 26 |
| RSG_SLT | 24 | 12 | 32 | 24 | 27 | 32 | 32 | 32 |
| RSG_LCO | 32 | 12 | 32 | 32 | 27 | 32 | 25 | 32 |
| IP Phone | 25 | 12 | 32 | 25 | 27 | 32 | 24 | 26 |

/* China Gain */

| RX From | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU | DTMF | TONE | M1 | M2 | RSG_DKT | RSG_SLT | RSG_LCO | IPKTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 24 | 24 | 26 | 40 | 40 | 26 | 20 | 8 | 32 | 29 | 29 | 25 | 24 | 40 | 25 |
| RSG_SLT | 32 | 32 | 32 | 12 | 32 | 32 | 32 | 32 | 8 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| RSG_LCO | 26 | 32 | 32 | 26 | 32 | 32 | 32 | 32 | 26 | 32 | 32 | 32 | 26 | 32 | 32 | 26 |
| IP Phone | 25 | 24 | 24 | 26 | 40 | 40 | 26 | 20 | 8 | 32 | 29 | 29 | 25 | 24 | 40 | 25 |

| TX To | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 25 | 32 | 32 | 26 | 26 | 26 | 24 | 26 |
| RSG_SLT | 24 | 32 | 32 | 33 | 32 | 32 | 32 | 32 |
| RSG_LCO | 40 | 32 | 32 | 38 | 32 | 32 | 32 | 32 |
| IP Phone | 25 | 32 | 32 | 26 | 26 | 26 | 24 | 26 |

/* Telkom Gain */

| RX From | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU | DTMF | TONE | M1 | M2 | RSG_DKT | RSG_SLT | RSG_LCO | IPKTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 26 | 30 | 30 | 26 | 26 | 26 | 33 | 29 | 8 | 32 | 29 | 29 | 26 | 30 | 26 | 26 |
| RSG_SLT | 42 | 32 | 32 | 22 | 31 | 31 | 34 | 42 | 38 | 30 | 42 | 42 | 42 | 32 | 31 | 42 |
| RSG_LCO | 38 | 35 | 35 | 30 | 32 | 32 | 37 | 36 | 26 | 42 | 36 | 36 | 38 | 35 | 32 | 38 |
| IP Phone | 26 | 30 | 30 | 26 | 26 | 26 | 33 | 29 | 8 | 32 | 29 | 29 | 26 | 30 | 26 | 26 |

| TX To | DKT | SLT | SL2 | WKT | ACO | AC8 | DCO | DVU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSG_DKT | 26 | 42 | 42 | 26 | 38 | 38 | 32 | 21 |
| RSG_SLT | 30 | 32 | 32 | 28 | 35 | 35 | 27 | 27 |
| RSG_LCO | 26 | 31 | 31 | 29 | 32 | 32 | 23 | 23 |
| IP Phone | 26 | 42 | 42 | 26 | 38 | 38 | 32 | 21 |