- SIP for ipLDK

December 2005

- TABLE OF CONTENTS

1.	OVERVIEW	3

1.1	Network Diagram	3

1.2	SIP Registration	4

1.3	Receive Incoming Call	5

1.4	Make Outgoing Call	5

1.5	100 Rel Support	6

1.6	DTMF Sending	6

2.	SIP Admin Programming	7

2.1	SIP Attribute	7

2.2	Station SIP Attribute	9

Example. SIP Setting	10

# 1.	OVERVIEW

There are many applications of the Internet that require the creation and management of a session, where a session is considered an exchange of data between associations of participants.  The implementation of these applications is complicated by the practices of participants: users may move between endpoints, they may be addressable by multiple names, and they may communicate in several different media - sometimes simultaneously.  Numerous protocols have been authored that carry various forms of real-time multimedia session data such as voice, video, or text messages.  The Session Initiation Protocol (SIP) works in concert with these protocols by enabling Internet endpoints (called user agents) to discover one another and to agree on a characterization of a session they would  like to share.  For locating prospective session participants, and for other functions, SIP enables the creation of an infrastructure of network hosts (called proxy servers) to which user agents can send registrations, invitations to sessions, and other requests.  SIP is an agile, general-purpose tool for creating, modifying, and terminating sessions that works independently of underlying transport protocols and without dependency on the type of session that is being established.

## 1.1	Network Diagram

## 1.2	SIP Registration

LDK-SIP can be registered to a SIP proxy server following standard registration call flow.

LDK-SIP can generate a Registration for each station to a SIP proxy server. This means that each

station has a separate address-of-record so, each station has different user ID and authentication credentials.

But some SIP proxy server allow wild character “*”, then just one registration is enough.

Conditions

- If you set User ID Provision as Register, REGISTER method will be sent after initialization.

- If you set User ID Provision as Provision, the LDK-SIP does not attempt to register.

- In multiple registration, you must set ‘Station Attribute-2 Btn30’ considering ‘SIP Attribute-2’.

Ex) If ‘SIP Attribute-2 Index1’ contains registration information about station 2001, ‘Station Attribute-2 Btn30” of station 2001 must be set ‘1’ – Index number.

Related Admin

Station Attribute – 2

| 30 |  | SIP UID Bin No |  |
| --- | --- | --- | --- |

SIP Attribute -1

| Attribute | Value | Range |
| --- | --- | --- |
| Proxy Server Address |  | Max 32 char |
| Use Outbound Proxy |  |  |
| Primary DNS Address |  | Max 32 |

SIP Attribute -2

| Index | Value |  | Range |
| --- | --- | --- | --- |
| 1 | User ID |  | Include Domain Name (MAX 64 Char) |
|  | Authentication User Name |  | MAX 64 Characters |
|  | Authentication User Password |  | MAX 64 Characters |
|  | Authentication User Password Repeat |  | MAX 64 Characters |
|  | Contact Number |  | DID / Station Number |
|  | User ID Registeration |  | *see 1 |
|  | User ID Usage |  |  |

## 1.3	Receive Incoming Call

LDK-SIP support two kinds of Call mode – Normal Mode, DID Mode.

1. Normal Mode

When system receives INVITE Method from Proxy, Incoming call will be routed following system Ring Assignment.

2. DID Mode

Incoming call will be routed by received number – all stations have their own user ID and incoming call can be routed to each station.

Conditions

- When you use incoming call as Normal mode, you can use DISA feature and it is treated as normal CO line

Related Admin

Co Type(PGM140)

| Feature | Value |
| --- | --- |
| CO Type |  |

## 1.4	Make Outgoing Call

LDK-SIP can place basic call and 2-way audio will be established with called party.

Call Procedure

- Dial Co-Access-Code.

- Dial the number you want to call.

- Dial “#”.

Conditions

- Enblock Dialing is just supported.

Related Admin

PGM143-Btn6

| 6 |  | ISDN Enblock Send |  |
| --- | --- | --- | --- |

## 1.5	100 Rel Support

LDK-SIP supports RFC 3262 Reliability of Provisional Responses in Session Initiation Protocol.

User can insert “Supported:100rel” to indicate that LDK-SIP can support provisional response and can send “PRACK” for the all provisional response.

##### Call Procedure

- Send INVITE with “Supported:100rel”.

- Receive Provisional Response.

- Send PRACK for the Provisional Response.

##### Conditions

- To work with PRACK, 2 parties must support 100REL.

##### Related Adm

SIP Attribute – 1

| 100rel support |  |
| --- | --- |

## 1.6	DTMF Sending

LDK-SIP support the ability to transport DTMF tones in-band when using the G.711 CODEC and support the transportation of DTMF tones using the RTP telephone-event payload format as described in RFC 2833.

User can configure the DTMF transport Type by Admin.

Call Procedure

Conditions

- By user setting, System can only send In-Band DTMF.

- 2 Party must support RFC2833 to send DTMF using RTP telephone payload.

Related Admin

SIP Attribute – 1

| DTMF Mode |  |
| --- | --- |

# 2.	SIP Admin Programming

## 2.1	SIP Attribute

You can set general SIP Attributes in here. And system reset is recommended after changing configuration.

| PGM | FLEX | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 350 |  | SIP SETTING |  |  |  |
|  |  | SIP Proxy Server Address |  | 0.0.0.0 | Set SIP Proxy address |
|  |  | SIP Proxy Server Port |  | 5060 | Set SIP Proxy signaling port |
|  |  | Proxy Registration Timer |  | 1800 | Set Proxy Registration Timer value |
|  |  | Use Outbound Proxy | 0-1 | 1 | Determine outbound proxy usage |
|  |  | Primary DNS Address |  |  | Set primary DNS address |
|  |  | Secondary DNS Address |  |  | Set secondary DNS address which is used when primary DNS is down |
|  |  | Called Party Domain |  |  | Set called party domain name. VOIB makes ”TO” header of “INVITE” message using dialed number and this field. (ex: dialed_no@calledparty.domain ) |
|  |  | Connection Mode | 0-1 | 0 | Set SIP transport protocol 0 : UDP 1 : TCP |
|  |  | 100rel support | 0-1 | 0 | Usage of SIP “100rel” extension (reliable transfer of SIP protocol) |
|  |  | Use rport method | 0-1 | 0 | Usage of SIP “rport” extension (to support NAT) |
|  |  | Use single codec only | 0-1 | 0 | If this value is true, VOIB suggests default codec only through a SDP codec negotiation. Or this value is false, VOIB suggests all available codec through a SDP codec negotiation. |

| 351 |  | SIP UID Table |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | User ID | 64 ASCII character |  | Set SIP user ID which is used “From” Header (ex: caller@caller.domain) |
|  |  | Authentication User Name | 64 ASCII character |  | Set authentication user name if authentication is used. |
|  |  | Authentication Password | 64 ASCII character |  | Set authentication user password if authentication is used. |
|  |  | Contact Number | Maximum 8 digit |  | VOIB use “Contact” header using this field and VOIB IP address. Usually set station number or DID number to route this SIP UID. |
|  |  | User ID Register | 0-1 | 0 | Determine registration of this SIP UID |
|  |  | Associate Station | Station Number |  | To support a SIP supplement service - Click to dial - Click to answer - Voice Mail notify (only for the BroadWorks soft switch) |
|  |  | User ID Usage | 0-1 | 0 | Determine this SIP UID bin is valid or not |

## 2.2	Station SIP Attribute

You can set SIP Station Attributes in PGM 111.

| BTN | ITEM | - RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 23 | User ID bin no | 00 - 32 | 00 | UID table index for SIP outgoing call VOIB make “From” Header if this value is 00 : Use COLP 01~32 : Use SIP UID (PGM 351 – 1) |

# Example. SIP Setting

* SIP Server IP Address : 150.150.131.240

* LDK-300 H/W Configuration : CO Line 1 – 8  SIP Co Line

* LDK-300 System IP Address : 192.168.131.151

* LDK-300 admin :

PGM 105 – Extension Number 100 - 131

PGM 111 – Extension 100, SIP Tbl Bin No : 01

PGM 111 – Extension 101, SIP Tbl Bin No : 02

PGM 140 – CO 1-8, Service Type : DID

PGM 143 – CO 1-8, DID Type 1

PGM 143 – CO 1-8, Enblock Send : On

PGM 340 – VOIP IP Address : 192.168.131.152

PGM 350 – SIP Proxy :150.150.131.240

PGM 350 – DNS : 150.150.131.240

PGM 350 – Domain : 150.150.131.240

PGM 351 – Bin 01

User ID : 100@150.150.131.240

Contact No  l00

User ID Registration : Register

User ID Usage : True

PGM 351 – Bin 02

User ID : 101@150.150.131.240

Contact No  l01

User ID Registration : Register

User ID Usage : True