IP-LDK

Enhanced Trace Guide

(Graphical Trace Mode)

July 6, 2005

LG Electronics, Inc.

CONTENS

1. Profile of Enhanced Trace	2

2. Usage of Enhanced Trace	3

3. Supported Messages of Enhanced Trace	3

3. 1 Digital Line (PRI, VOI, BRI, STI-t, NPRI, NBRI) Type	3

3.2 Analog Line Type (DCO, LCO) Type	4

4. Enhanced Trace Examples	6

4.1 Incoming Call with PRIB. (ISDN Call)	7

4.2 Outgoing Call with VOIB. (NET Call)	10

4.2 Outgoing Call with LCOB. (Analog Call)	12

## 1. Profile of Enhanced Trace

Enhanced Trace is a graphical trace mode of IP-LDK V3.5. Enhanced Trace must need the DRAM. This new feature supports the networking message trace of PRI / VOI / BRI / STI (t-mode) / DCO / LCO / NBIR / NPRI Boards. Enhanced Trace is composed of translated messages of digital and analog lines. In case of digital line, networking messages are analyzed by the standard ETSI/ITU protocol and analyzed data is printed with large arrows. In case of analog line, communicated commands with other systems are analyzed and analyzed data is printed with small arrows. Below picture is example of Enhanced Trace.

Digital CO line Trace (with large arrows)                     Analog CO line Trace (with small arrows)

Toward right arrows mean the outgoing message. Otherwise toward left arrows mean the incoming message

This is outgoing network message for SETUP with CO line 1 (Digital line)

&

This message compounds the information elements of bearer capability, called IP, facility, calling number, and called number.

(Example) Digital CO case

## 2. Usage of Enhanced Trace

Enhanced Trace is one of system monitoring commands.

Board Trace case:		t  b  x  n	(‘x’ means board number)

Device Trace case:		t  n  c#		(device trace option ‘n’ could be used with other options)

t  nct		(‘nct’ means the set all of networking CO graphical trace)

* Board Trace & All of Networking Trace (‘nct’ commnad): Simple Display Type

* Device Trace: Complete Display Type

=> Simple Display Type shows the Networking Connection Message and Information Elements, but all of Information Elements (IE) could not be analyzed. Only including data of Called IP, Calling Number, Called Number and Cause element could be analyzed, and then these data is printed.

=> Complete Display Type shows the Networking Connection Message, Information Elements and all of data contained on Information Elements (IE) data.

## 3. Supported Messages of Enhanced Trace

### 3. 1 Digital Line (PRI, VOI, BRI, STI-t, NPRI, NBRI) Type

In case of digital line, networking messages are analyzed by the standard ETSI/ITU protocol.

Enhanced Trace Supported Messages for Networking Connection

| Call Establishment Message | Call Information Phase Messages |
| --- | --- |
| ALERTING CALL PROCEEDING CONNECT CONNECT ACK PROGRESS SETUP SETUP ACK | RESUME RESUME ACK RESUME REJECT SUSPEND SUSPEND ACK SUSPEND REJECT USER INFORMATION |
| Call Clearing Messages | Miscellaneous Messages |
| DISCONNECT RELEASE RELEASE COMPLETE | SEGMENT CONGESTION CONTROL FACILITY INFORMATION NOTIFY STATUS STATUS REQ |

* Additional Messages on ISDN also could be supported.

Enhanced Trace Supported Message for Information Elements

| Networking Information Element | Decoded & Print / Only Print | Networking Information Element | Decoded & Print / Only Print |
| --- | --- | --- | --- |
| IE_BEARER_CAPABILITY | Decoded & Print | IE_CONNECTED_NO | Decoded & Print |
| IE_CAUSE | Decoded & Print | IE_CONNECTED_SUBADDR | Only Print Data |
| IE_CALL_IDENTITY | Only Print Data | IE_CALLING_NO | Decoded & Print |
| IE_CALL_STATUS | Decoded & Print | IE_CALLING_SUBADDR | Only Print Data |
| IE_CHANNEL_INFO | Decoded & Print | IE_CALLED_NO | Decoded & Print |
| IE_AOC | Only Print Data | IE_CALLED_SUBADDR | Only Print Data |
| IE_FACILITY | Only Print Data | IE_TRANS_NET_SELECT | Decoded & Print |
| IE_PROG_INDICATION | Decoded & Print | IE_LOW_LAYER_COMPATIBLE | Only Print Data |
| IE_NET_SPECIFIC_FACILITY | Only Print Data | IE_HIGH_LAYER_COMPATIBLE | Only Print Data |
| IE_NOTIFY_INDICATION | Decoded & Print | IE_USER_USER | Only Print Data |
| IE_DISPLAY | Only Print Data | IE_CONGEST | Decoded & Print |
| IE_DATE_TIME | Decoded & Print | IE_ISDN_FLASH_REQ | Only Print Data |
| IE_KEYPAD_FACILITY | Only Print Data | IE_ISDN_FLASH_ACK | Only Print Data |
| IE_CALLING_IP | Decoded & Print |  |  |
| IE_CALLED_IP | Decoded & Print |  |  |

*Blue Mark: able to decode IE & print the analyzed results. And the others is printed raw (HEX Value) data.

### 3.2 Analog Line Type (DCO, LCO) Type

In this case Enhanced Trace could show the related CO commands to communicate with other systems. Analog messages dose not include any information elements, so Enhanced Trace show only the sent or received digit.

DCO case

| Supported Messages |  |  |  |
| --- | --- | --- | --- |
| RING_DETECT | RLS_ACK | R2_END_OF_SIGNAL | SND_REGISTER_RCL |
| RING_STOP | FLASH_SENT_ACK | AB_SIG_PCO_RSP | SND_READY |
| METERING | RCV_DGT | OUTPULSE_RESND_REQ | SND_END_OF_DIAL |
| LOOP_IDLE | RCV_ANS | ANI_SVC_START | SND_END_OF_DIAL_BUSY |
| DGT_SENT | ACNR_DETECTED_TONE | ANI_DGT_SENT | MAKE_IDLE |
| SEIZE_ACK | RCV_OFFER |  | SND_BLOCKING |
| SEIZE_NAK_EVT | RCV_RCL | SEIZE |  |
|  | RCV_BLOCK | RLS | OUTDIGITS_TYPE |
|  |  | ANSWER | AB_SIG_REQ_PCO |
|  | RCV_END_OF_DIAL | OP_DGT_SENT | ANI_DGT_OP |
|  | CALLER_ID |  |  |

*Blue Mark: Able to see the related digit.

LCO case

| Supported Messages |  |  |
| --- | --- | --- |
| RING_START | RELEASE_ACK | SEIZE |
| RING_STOP | FLASH_ACK | SEIZE_ACK |
| CALL_METERING | DISA_DGT | SEIZE_NAK |
| LOOP_SUP_DETECT | TONE_DETECTED | RELEASE |
| DIAL_TX_ACK | RELEASED | ANSWER |
|  | CID_DETECT | SEND_DIGIT |

*Blue Mark: Able to see the related digit.

## 4. Enhanced Trace Examples

### 4.1 Incoming Call with PRIB. (ISDN Call)

CO 1 Enhanced Device Trace

Complete Display Type

Receive the SETUP networking message

with bearer cap, channel, calling number

of Information Elements

Printed IE Data

Send the SETUP_ACK networking message

with channel, prog_indicator of Information Elements

Calling Number

‘4501000’

Printed IE Data

Printed IE Data

Receive the INFORMATION networking message

with called number ‘3’

Receive the INFORMATION networking message

with called number ‘0’

Receive the INFORMATION networking message

with called number ‘0’

Receive the INFORMATION networking message

with called number ‘1’

Send the PROCEEDING networking message

Send the ALRTING networking message

Send the CONNECT networking message

with ‘3001’ connected number

Receive the CONNECT ACK networking message

Receive the DISCONNECT networking message

with ‘normal call clearing’ message

Send the RELEASE networking message

Receive the RELEASE Complete networking message

### 4.2 Outgoing Call with VOIB. (NET Call)

Enhanced Board Trace

Simple Display Type

Data of Information Elements have not been printed, except Called IP, Calling No, and Called No.

Send the SETUP networking message

by sending complete with bearer cap, channel, called IP, calling number, called number

Receive the CALL PROCEEDING

Receive the ALERTING with facility

Receive the CONNECT with called number ‘2000’

Send the CONNECTACK

Send the RELEASE COMPLETE

This message means the DISCONNECT.

Receive the RELEASE

Receive the RELEASE COMPLETE

### 4.2 Outgoing Call with LCOB. (Analog Call)

Enhanced all of Networking CO Trace

Simple Display Type

CO 35 Seize Request Message.

CO 35 Seize ACK Message.

Sending Digit

‘2000’

TALKING

CO 35 Release Request

CO 35 Release ACK

Dial Digit ‘2’

Dial Digit ‘0’

Dial Digit ‘0’

Dial Digit ‘0’