LG Key System Software

Technical Information

STI-0046

Sep 21 2001

| Title | #### LDK-300 and LDK-100 Monitor/Maintenance |
| --- | --- |
| System | LDK-300/LDK-100 |
| Abstract |  |

Revision History

| Revision | By | History |
| --- | --- | --- |
| 1.0 | J. Kwon | 2001. 09. 21 |
| 1.1 | J. Kwon | 2001. 11. 06 – System access procedure added. |
| 1.2 | H. Lim | 2002. 04. 23 – LDK-100 Added |
| 1.3 | Cho.W.S | 2005. 06. 16 – Enhanced Trace Added |

Table of Contents

1. System access procedure	2

1.1	Serial Connection	2

1.2	LAN Connection	2

1.3	Modem Connection	2

2. System monitoring commands	3

2.1	DIP Switch setting and meaning	3

2.2 Trace Commands	3

Example	4

2.3	Other Commands	5

3. System Diagnostic/Maintenance commands	6

3.1 Maintenance Commands	6

Example	6

3.2 Diagnostic Commands	8

# 1. System access procedure

## 1.1	Serial Connection

- Be sure PC application port is not assigned to serial port that you want to connect.

- For trace, program print port to the proper serial port(PGM 175 - BTN 7).

- Connect using hyper terminal or other terminal program.

## 1.2	LAN Connection

- Be sure PC application port is not assigned to telnet.

- Program IP address and Gateway of the system(PGM 108 – BTN 2, BTN 4).

- Reset the system with DIP Switch 8 at OFF position.

- For trace, program print port to telnet(PGM 175 - BTN 7).

- Connect via telnet.

## 1.3	Modem Connection

- Be sure PC application port is not assigned to COM3-MODU.

- For trace, program print port to COM3-MODU(PGM 175 - BTN 7).

- Connect via modem.

# 2. System monitoring commands

Password : jennie

mon [t|d|m|s|c|p|?|x] [option|parameters]

## 2.1  DIP Switch setting and meaning

| DIP switch/LED | Setting and Meaning |
| --- | --- |
| DIP3 | Trace Control : - ON-trace off - OFF-trace on |
| DIP8 | Database initialize: - ON - initialize memory - OFF-save database |

## 2.2  Trace Commands

| Monitoring Type | Command | Meaning |
| --- | --- | --- |
| Board Trace (Max 6) | mon t d  | Delete all current trace commands and revert to idle condition. |
|  | mon t b xx n | Set trace for board in slot xx ,where 1 y  29 for LDK-300. (28 is CPTU, and 29 is DTRU) ,where 1 y  14 for LDK-100. (13 is CPTU, and 14 is DTRU) Option ‘n’ is graphical CO simple trace mode (Enhanced Trace). Option ‘n’ support the PRIB, VOIB, DCOB & LCO Type Boards. If PRI or VOI board was set, you can see the net msgs and information elements: IP info, Calling num, Called num and etc… Also in DCO or LCO board case, you can see the Calling num, Called num, and some of circuit commands related communications with outside system. |
|  |  | e.g1) For setting trace for board in slot 7; mon>t b 07 e.g2) enhanced trace for board in slot 8; mon>t b 07 n |
| Device Trace (Max 15) | mon t ceht sxxx  | At the sxxx, xxx should be physical station number. (i.e. port number of station) Command options : c : device command tracing. (MP  PP) e : event tracing. (PP, Internal, Timer Event) h : highway tracing. t : CTI device command / event tracing. (Multi-options can be used; ceh, c, eh, chi .etc.) Example If you want to trace the Station 102, you should enter the physical station number as following; mon>t ceh s003 Even if the station number was changed from 102 to 702, the physical station number is not changed; mon>t ceh s003 |
|  | mon t cehtn cxxx  | Tracing related to CO xxx. Command options : n: Enhanced Tracing(Graphical Mode) (This side option ‘n’ is detailed enhanced trace mode. If you set the option ‘n’ at the particular CO in PIB or VOI boards, you may see all of network information elements.) ## Example For setting the command/event/hiway trace for CO line 07; mon>t ceh c007 For the command/event graphical trace for CO line 08; mon>t cen c008 |
|  | mon t c sxxx a  | Include LED / LCD data. |
|  | mon> t nct | (Network Command Trace) Only printed network text message. |

* Enhanced Trace Option ‘n’ must need the DRAM. And It is executed over IP-LDK V3.5

## 2.3  Other Commands

| Monitoring Type | Command | Meaning |
| --- | --- | --- |
| Dump Memory | mon d xxxxxx yyyyyy | Dump memory between address xxxxxx and yyyyyy. (xxxxxx and yyyyyy : memory address in hexadecimal.) |
| Modify Memory Content | mon m xxxxxx bb | Modify memory of address xxxxxx to bb. (xxxxxx: memory address in hexadecimal, bb: byte value in hexadecimal) |
| Show STA Status | mon s xxx  | Show STA xxx status , where 1  y  300 for LDK-300. , where 1  y  96 for LDK-100. |
|  |  | e.g., For showing status of station 100 mon> s 001 |
| Show CO Status | mon c xxx  | Show CO xxx status , where 1  x  200 for LDK-300. , where 1  x  40 for LDK-100. |
|  |  | e.g., For showing status of CO number 7 mon> c 007 |
| Print Channel Assign | mon p [v¦s] | This command views and sets print channel. Command options are: v: View print channel s: Set print channel |
| Help Menu | mon ?  | This command shows the available commands and their description. |
| Exit | mon x  | Exit monitor. |

# 3. System Diagnostic/Maintenance commands

Password : brandy

## 3.1  Maintenance Commands

|  | Commands | Meaning |
| --- | --- | --- |
| Maintenance Commands | maint> bc | Shows board configuration table. |
|  | maint> bl xxx yyy {r¦t} | Listens B-channel of specified time-slot. ## Example To listen to rx channel of time-slot 2 at station 100(port 1): maint> bl 1 2 r Press any key to finish or after finished. |
|  | maint> bs [xx] | Shows board status. xx is slot number, 00 for MPB. |
|  | maint> hd | Shows HDLC status. |
|  | maint> reboot | This commands reboots the system. It needs admin password,and if it is not specified, ‘147*’ is used temporarily. |
|  | maint> ta [options] | Shows traffic analysis data. Command options: all <timetype>: Print All Traffic Report period <hhmm> <timetype>: Print Traffic Report Periodically period_abort: Cancel Periodic Print atd <timetype>: Print Attendant Traffic Report callsum: Print Call Summary Report callhour: Print Call Hourly Report hw <timetype>: Print H/W Usage Summary Report cosum <timetype>: Print CO Traffic Summary Report cohour <cogrp#>: Print CO Traffic Hourly Report <timetype> can be one of following: tt : Today Total yt : Yesterday Total lh : Last Hour yp : Yesterday Peak tp : Today Peak |
|  | maint> ws xx [options] | xx is slot number. Shows WTIB statistics data. Command options: upload: Upload statistics data from WTIB call: Total number of call & direction of the call subs: Information per subscripted device eoc: End of call cell: Usage of frequency and slot traf: Show holding time acce: Access info.(basic/hand-over) clea: Clear statistics data |

## 3.2  Diagnostic Commands

|  | Commands | Meaning |
| --- | --- | --- |
| Diagnostic Commands | maint> db cc | Shows configuration constraints check result. It gives OK if the system configuration meets the maximum board configuration constraints. |
|  | maint> db pf | Shows preprogrammed faults. It checks if the admin programming configuration is same with the installed boards. |
|  | maint> db ts {sl¦cf} | Shows time slot assignment and conflict status. |
|  | maint> db ht xx yy # | Tests PCM hiway using one DKT and one DTMF receiver.The first DKT of the assigned slot is used for the test. Command options: xx: slot number(DTIB) yy: dtmf duration time(10ms base) #: hiway number (0-7 for LDK-300, 0-2 for LDK-100) Example To test hiway #1 using DTIB installed at 1st slot: maint> db ht 01 01 1 |
|  | maint> db dt | Tests DTMF receiver. This feature tests all DTMF receivers(up to 32) in the system. |
|  | maint> db ct | This command tests CPTU. |
|  | maint> db mt | This command tests DRAM module installed. |
|  | maint> db rt | Tests RTC. You can see current time and modify it. The time does not elapse while you are testing RTC. |
|  | maint> db pc | Shows power capacity of the PSU and current power usage status of the system. |
|  | maint> db rc | Shows RGU capacity. This feature shows RGU capacity, CO incoming ring cadence and ICM call ring cadence. |
|  | maint> de hs | Shows HDLC status. |
|  | maint> dr {is¦ic} | Shows ISDN line status information. Command options: is: Shows ISDN line information. ic: Clears ISDN line information. |
|  | maint> dr {ri¦rc} | Shows reset information. Command options: ri: Shows reset information. rc: Clears reset information. |
|  | maint> dr li {d¦c¦v¦a} | Shows current resource assignment. Command options: d: DTMF receiver assignment c: CPTU assignment v: VMIB channel assignment a: DTMF, CPTU, VMIB channel assignment |