ipLDK SW Upgrade

Installation And User Guide

REVISION HISTORY

| DATE | ISSUE | S/W VER. | CONTENTS |
| --- | --- | --- | --- |
| 2000/11/10 | 1.0 | A.0Aa | Initial Release |
| 2001/2/6 | 1.0a | A.0Ae (1.0Ae) | Insert the recommended modem speed |
| 2001/2/25 | 1.0b | 1.0Af | Insert the wait dialog and fail dialog after finishing the s/w transferring Insert the available modem speeds |
| 2002/2/5 | 1.0c | A.0Ca | Designed for both ipLDK-300 and ipLDK-100 |
| 2002/3/29 | 1.0c | X.0Ca | Designed for both ipLDK-300 and ipLDK-100 |
| 2004/09/22 | 3.0 | 3.0An | V3 Feature is added. The information of ipLDK-20 System is added |
| 2006/05/14 | 3.6 | C.6Ab | USB Connection for ARIA SOHO is added. |

- Contents –

1. Introduction	4

2. PC Requirement	4

3. Connection method with ipLDK	4

4. ipLDK MPB S/W version Compatibility	5

5. Installation	5

6. File Component in your Execution Directory	5

7. Choosing Connection Type	5

8. User Information Dialog	6

9. Starting S/W upgrade	11

10. Transferring Process.	12

11. Finishing S/W Upgrade.	13

# 1. Introduction

ipLDK S/W Upgrade is a PC based Window S/W to update ipLDK MBU S/W by the transferred ROM file instead of replacing the whole ROM. Four connection types are available(ISDN, TCP, Serial or MODEM connection). It is up to the user which connection method he selects. These 4 types slightly differ by configuration, transfer speed and transferring protocol.

# 2. PC Requirement

- - PC OS : Windows 2000 or later

- - PC Memory : 128 MB at least

- - PC Free Disk Space : 10Mbytes or higher (larger than 20 M Bytes recommended)

- - PC Monitor Resolution : VGA or higher

- - PC Modem : General analog dial-up modem (only for upgrading through an analog phone line)

- - PC ISDN Modem : General ISDN modem (only for upgrading through an ISDN phone line)

- - USB Port For USB connection( ARIR SOHO System only)

# 3. Connection method with ipLDK

ipLDK S/W Upgrade is available by the following connection method;

- Serial connection

ipLDK MBU S/W can be updated through RS-232C interface in PC.

- ISDN line connection

ipLDK MBU S/W can be updated through ISDN BRI Line interface at the remote PC. Location can be any assigned number in ipLDK BRI, PRI or STIB T-interface.

- TCP connection

ipLDK MBU S/W can be updated through LAN interface at the remote PC.

- MODEM connection

ipLDK MBU S/W can be updated through MODEM interface at the remote PC.

# 4. ipLDK MPB S/W version Compatibility

The available ipLDK MPB S/W version is 1.0Ap or later.

- Note >

- The ARIA Soho version is C.6Ac or later

# 5. Installation

Upgrade ipLDK is distributed with PC Admin for ipLDK/ARIA SOHO.

- Refer to the installation of PC Admin for ipLDK.

Note : In setup procedure, If your PC is Windows 95 generic or some system DLL Files should be update , setup program request restart your PC

# 6. File Component in your Execution Directory

If you install GDK Admin successfully, the following files reside in execution directory

- Up ipLDK.Exe : Main Execution File

- Capi2032.dll : DLL File for ISDN operations

# 7. Choosing Connection Type

You have to select which system you are connecting to.

ipLDK Upgrade is available through ISDN, TCP, Modem and Serial connection. Please choose the desired connection type. User will see a short dialog showing the requirements for the selected connection type. Please check this basic requirements before getting started.

# 8. User Information Dialog

After selecting the desired connection type user has to decide several configuration requirements including Admin Password, Binary File etc. in the “User Information Dialog”. It slightly differs according to the selected connection type.

###### Upgrade by ISDN Connection

Please enter the following entries for ISDN connection.

ISDN Dial No.: ISDN Telephone number of a T-interface in STIB, BRIB or PRIB in ipLDK system .

Admin Password: If ipLDK requires password to update MBU S/W.

Binary File: The new S/W file data to be updated.

User can also can set the details for ISDN connections.

“Settings” dialog box contains three parts, which are waiting time, retry count and the volume of trace data.

You can change the default values of settings, but it is strongly recommended that the default settings should not be changed. The change of default values can severely affect the normal operation of software upgrade so that the upgrade time can be doubled or the upgrade can be abnormally aborted.

You can discard all the changes you made by selecting Set defaults button.

Press Next button to go to the next step.

###### Upgrade by Serial Connection

Please enter the following entries for Serial connection.

Serial Port.: Serial port No. in PC, which is connected to ipLDK COM port 2.(Unable to use ipLDK COM port 1)

Admin Password: If ipLDK requires password to update MBU S/W.

Binary File: The new S/W file data to be updated.

Baud Rate: Choose the transferring baud rate. 19200, 38400, or 57600 bps are recommended. (Note)

Note: The transferring baud rate in PC and the baud rate of COM port 2 in ipLDK system must be identical. You can program ipLDK system baud rate by PGM174.(For more information please refer to ipLDK Functional Specification.)

Note: If you set PC’s and ipLDK system’s transferring baud rate as 57600 bps, it might happen that some keysets display incorrect characters in their LCD, or that ipLDK system be unstable. But, the rate will be the faster way of upgrading.

Press Next button to go to the next step.

###### Upgrade by TCP Connection

Please enter the following entries for TCP connection.

IP Address.: IP address of the destination ipLDK system .( Note)

Admin Password: If ipLDK requires password to update MBU S/W.

Binary File: The new S/W file data to be updated.

Note: Assign ipLDK system IP address before you attempt to TCP connection. You can program ipLDK system IP address by PGM108.(For more information please refer to ipLDK Functional Specification.)

Note: It might happen that the TCP connection will be disconnected due to the overloaded network.

Press Next button to go to the next step.

###### Upgrade by MODEM Connection

If user selects MODEM configuration the next step is to set the

Please enter the following entries.

Dial No.: Analog telephone number of the destination ipLDK system.(Note)

Admin Password: If ipLDK requires password to update MBU S/W.

Binary File: The new S/W file data to be updated.

Note: Assign The Analog telephone number of ipLDK system to Modem associated device before you attempt to MODEM connection. You can program set the analog line to modem associated device by PGM170.(For more information please refer to ipLDK Functional Specification.)

Note: The transferring baud rate in PC and the baud rate of Modem port (COM Port 3) in ipLDK system must be identical. You can program ipLDK system baud rate by PGM174.(For more information please refer to ipLDK Functional Specification.) Using 9600 bps is recommended. (2400bps is available but slow. And, 19200bps is also available but a little unstable.)

Note: It might happen that the MODEM connection will be lost due to the PSTN network situation.

Press Next button to go to the next step.

###### Upgrade by USB Connection

Please enter the following entries for Serial connection.

Admin Password: If ipLDK requires password to update MBU S/W.

Binary File: The new S/W file data to be updated.

Press Next button to go to the next step.

# 9. Starting S/W upgrade

After setting the detailed configuration for the selected connection type, now user is ready to start ipLDK S/W upgrade. User has the following choice.

Start.:  S/W transfer process will start .

Back:  Go to the previous step.

Monitor:  Monitor Trace window.

Quit: ipLDK S/W upgrade program will be terminated.

If user presses Start button at this time all other button except Monitor button will be deactivated. In Remote upgrade in “Monitor Trace” window the undergoing progress will be displayed. (Note)

Note: In MODEM connection after pressing Start button user has to select the details for the MODEM interface in the PC. The below dialog will be displayed. Press OK to getting started.

# 10. Transferring Process.

When S/W file transfer is undergoing user can monitor the process.

In ISDN connection user can show the transferring process in the Monitor Dialog window.

And hence Serial, Modem and TCP connection is using Z-Modem transfer protocol the sending process will be displayed in Z-Modem window.

Note> In version3, TCP connection isn’t using Z-Modem transfer. So, you can upgrade the SW quickly.

# 11. Finishing S/W Upgrade.

If the transferring process is finished successfully, “Wait Dialog” will be displayed. That means that ipLDK system has just started software upgrading internally. Every keysets stop operating. User must not reset the ipLDK system until it will be automatically reset and run again. This will take a few minutes. (You can check whether the upgrading is in proceeding or not in ipLDK system’s MPB lamp flushing)

After S/W transferring process is failed “Fail Dialog” will be displayed. Check the reason of failure in the monitor dialog or trace file that was automatically generated in the PC directory where the upgrading software is. User can start another upgrade by selecting Yes button of “Again Dialog”.