LG Key System Software

Technical Information

STI-0046

Sep 21 2001

| Title | #### LDK-300 and LDK-100 Monitor/Maintenance |
| --- | --- |
| System | LDK-300/LDK-100 |
| Abstract |  |

Revision History

| Revision | By | History |
| --- | --- | --- |
| 1.0 | J. Kwon | 2001. 09. 21 |
| 1.1 | J. Kwon | 2001. 11. 06 – System access procedure added. |
| 1.2 | H. Lim | 2002. 04. 23 – LDK-100 Added |
| 1.3 | Cho.W.S | 2005. 06. 16 – Enhanced Trace Added |
| 1.4 | SJ Ryu | 2005. 12. 14 – Enhanced Trace Guide Added |

Table of Contents

1. System access procedure	1

1.1	Serial Connection	1

1.2	LAN Connection	1

1.3	Modem Connection	1

2. System monitoring commands	2

2.1  DIP Switch setting and meaning	2

2.2  Trace Commands	2

2.3  Other Commands	4

3. System Diagnostic/Maintenance commands	5

3.1  Maintenance Commands	5

3.2  Diagnostic Commands	7

4.Enhanced Trace Guide(Graphical Trace Mode)	1

4.1  Profile of Enhanced Trace	1

4.2  Usage of Enhanced Trace	1

4.3  Supported Messages of Enhanced Trace	1

4.3.1  Digital Line (PRI, VOI, BRI, STI-t, NPRI, NBRI) Type	1

4.3.2  Analog Line Type (DCO, LCO) Type	2

4.4  Enhanced Trace Examples	4

4.4.1  Incoming Call with PRIB. (ISDN Call)	4

4.4.2  Outgoing Call with VOIB. (NET Call)	1

4.4.3  Outgoing Call with LCOB. (Analog Call)	3

# 1. System access procedure

## 1.1	Serial Connection

- Be sure PC application port is not assigned to serial port that you want to connect.

- For trace, program print port to the proper serial port(PGM 175 - BTN 7).

- Connect using hyper terminal or other terminal program.

## 1.2	LAN Connection

- Be sure PC application port is not assigned to telnet.

- Program IP address and Gateway of the system(PGM 108 – BTN 2, BTN 4).

- Reset the system with DIP Switch 8 at OFF position.

- For trace, program print port to telnet(PGM 175 - BTN 7).

- Connect via telnet.

## 1.3	Modem Connection

- Be sure PC application port is not assigned to COM3-MODU.

- For trace, program print port to COM3-MODU(PGM 175 - BTN 7).

- Connect via modem.

# 2. System monitoring commands

Password : jennie

mon [t|d|m|s|c|p|?|x] [option|parameters]

## 2.1  DIP Switch setting and meaning

| DIP switch/LED | Setting and Meaning |
| --- | --- |
| DIP3 | Trace Control : - ON-trace off - OFF-trace on |
| DIP8 | Database initialize: - ON - initialize memory - OFF-save database |

## 2.2  Trace Commands

| Monitoring Type | Command | Meaning |
| --- | --- | --- |
| Board Trace (Max 6) | mon t d  | Delete all current trace commands and revert to idle condition. |
|  | mon t b xx n | Set trace for board in slot xx ,where 1 y  29 for LDK-300. (28 is CPTU, and 29 is DTRU) ,where 1 y  14 for LDK-100. (13 is CPTU, and 14 is DTRU) Option ‘n’ is graphical CO simple trace mode (Enhanced Trace). Option ‘n’ support the PRIB, VOIB, DCOB & LCO Type Boards. If PRI or VOI board was set, you can see the net msgs and information elements: IP info, Calling num, Called num and etc… Also in DCO or LCO board case, you can see the Calling num, Called num, and some of circuit commands related communications with outside system. |
|  |  | e.g1) For setting trace for board in slot 7; mon>t b 07 e.g2) enhanced trace for board in slot 8; mon>t b 07 n |
| Device Trace (Max 15) | mon t ceht sxxx  | At the sxxx, xxx should be physical station number. (i.e. port number of station) Command options : c : device command tracing. (MP  PP) e : event tracing. (PP, Internal, Timer Event) h : highway tracing. t : CTI device command / event tracing. (Multi-options can be used; ceh, c, eh, chi .etc.) Example If you want to trace the Station 102, you should enter the physical station number as following; mon>t ceh s003 Even if the station number was changed from 102 to 702, the physical station number is not changed; mon>t ceh s003 |
|  | mon t cehtn cxxx  | Tracing related to CO xxx. Command options : n: Enhanced Tracing(Graphical Mode) (This side option ‘n’ is detailed enhanced trace mode. If you set the option ‘n’ at the particular CO in PRI or VOI boards, you may see all of network information elements.) Example For setting the command/event/hiway trace for CO line 07; mon>t ceh c007 For the command/event graphical trace for CO line 08; mon>t cen c008 |
|  | mon t c sxxx a  | Include LED / LCD data. |
|  | mon> t nct | (Network Command Trace) Only printed network text message. |

* Enhanced Trace Option ‘n’ must need the DRAM. And It is executed over IP-LDK V3.5

## 2.3  Other Commands

| Monitoring Type | Command | Meaning |
| --- | --- | --- |
| Dump Memory | mon d xxxxxx yyyyyy | Dump memory between address xxxxxx and yyyyyy. (xxxxxx and yyyyyy : memory address in hexadecimal.) |
| Modify Memory Content | mon m xxxxxx bb | Modify memory of address xxxxxx to bb. (xxxxxx: memory address in hexadecimal, bb: byte value in hexadecimal) |
| Show STA Status | mon s xxx  | Show STA xxx status , where 1  y  300 for LDK-300. , where 1  y  96 for LDK-100. |
|  |  | e.g., For showing status of station 100 mon> s 001 |
| Show CO Status | mon c xxx  | Show CO xxx status , where 1  x  200 for LDK-300. , where 1  x  40 for LDK-100. |
|  |  | e.g., For showing status of CO number 7 mon> c 007 |
| Print Channel Assign | mon p [v¦s] | This command views and sets print channel. Command options are: v: View print channel s: Set print channel |
| Help Menu | mon ?  | This command shows the available commands and their description. |
| Exit | mon x  | Exit monitor. |

# 3. System Diagnostic/Maintenance commands

Password : brandy

## 3.1  Maintenance Commands

|  | Commands | Meaning |
| --- | --- | --- |
| Maintenance Commands | maint> bc | Shows board configuration table. |
|  | maint> bl xxx yyy {r¦t} | Listens B-channel of specified time-slot. Example To listen to rx channel of time-slot 2 at station 100(port 1): maint> bl 1 2 r Press any key to finish or after finished. |
|  | maint> bs [xx] | Shows board status. xx is slot number, 00 for MPB. |
|  | maint> hd | Shows HDLC status. |
|  | maint> reboot | This commands reboots the system. It needs admin password,and if it is not specified, ‘147*’ is used temporarily. |
|  | maint> ta [options] | Shows traffic analysis data. Command options: all <timetype>: Print All Traffic Report period <hhmm> <timetype>: Print Traffic Report Periodically period_abort: Cancel Periodic Print atd <timetype>: Print Attendant Traffic Report callsum: Print Call Summary Report callhour: Print Call Hourly Report hw <timetype>: Print H/W Usage Summary Report cosum <timetype>: Print CO Traffic Summary Report cohour <cogrp#>: Print CO Traffic Hourly Report <timetype> can be one of following: tt : Today Total yt : Yesterday Total lh : Last Hour yp : Yesterday Peak tp : Today Peak |
|  | maint> ws xx [options] | xx is slot number. Shows WTIB statistics data. Command options: upload: Upload statistics data from WTIB call: Total number of call & direction of the call subs: Information per subscripted device eoc: End of call cell: Usage of frequency and slot traf: Show holding time acce: Access info.(basic/hand-over) clea: Clear statistics data |

## 3.2  Diagnostic Commands

|  | Commands | Meaning |
| --- | --- | --- |
| Diagnostic Commands | maint> db cc | Shows configuration constraints check result. It gives OK if the system configuration meets the maximum board configuration constraints. |
|  | maint> db pf | Shows preprogrammed faults. It checks if the admin programming configuration is same with the installed boards. |
|  | maint> db ts {sl¦cf} | Shows time slot assignment and conflict status. |
|  | maint> db ht xx yy # | Tests PCM hiway using one DKT and one DTMF receiver.The first DKT of the assigned slot is used for the test. Command options: xx: slot number(DTIB) yy: dtmf duration time(10ms base) #: hiway number (0-7 for LDK-300, 0-2 for LDK-100) Example To test hiway #1 using DTIB installed at 1st slot: maint> db ht 01 01 1 |
|  | maint> db dt | Tests DTMF receiver. This feature tests all DTMF receivers(up to 32) in the system. |
|  | maint> db ct | This command tests CPTU. |
|  | maint> db mt | This command tests DRAM module installed. |
|  | maint> db rt | Tests RTC. You can see current time and modify it. The time does not elapse while you are testing RTC. |
|  | maint> db pc | Shows power capacity of the PSU and current power usage status of the system. |
|  | maint> db rc | Shows RGU capacity. This feature shows RGU capacity, CO incoming ring cadence and ICM call ring cadence. |
|  | maint> de hs | Shows HDLC status. |
|  | maint> dr {is¦ic} | Shows ISDN line status information. Command options: is: Shows ISDN line information. ic: Clears ISDN line information. |
|  | maint> dr {ri¦rc} | Shows reset information. Command options: ri: Shows reset information. rc: Clears reset information. |
|  | maint> dr li {d¦c¦v¦a} | Shows current resource assignment. Command options: d: DTMF receiver assignment c: CPTU assignment v: VMIB channel assignment a: DTMF, CPTU, VMIB channel assignment |

# 4.Enhanced Trace Guide(Graphical Trace Mode)

## 4.1  Profile of Enhanced Trace

Enhanced Trace is a graphical trace mode of IP-LDK V3.5. This new feature supports the networking message trace of PRI / VOI / BRI / STI (t-mode) / DCO / LCO / NBRI / NPRI Boards. Enhanced Trace is composed of translated messages of digital and analog lines. In case of digital line, networking messages are analyzed by the standard ETSI/ITU protocol and analyzed data is printed with large arrows. In case of analog line, communicated commands with other systems are analyzed and analyzed data is printed with small arrows. Below picture is example of Enhanced Trace.

Digital CO line Trace (with large arrows)                      Analog CO line Trace (with small arrows)

Toward right arrows mean the outgoing message. Otherwise toward left arrows mean the incoming message

(Example) Digital CO case

This is outgoing network message for SETUP with CO line 1 (Digital line)

&

This message compounds the information elements of bearer capability, called IP, facility, calling number, and called number.

* Enhanced Trace must need the DRAM. And It is executed over IP-LDK V3.5

## 4.2  Usage of Enhanced Trace

Enhanced Trace is one of system monitoring commands.

Board Trace:		t  b  x  n	(‘x’ means board number)

All of Networking Trace:	t  nct		(‘nct’ means the set all of networking CO graphical trace)

Specific CO Trace:	t  n  c#		(option ‘n’ could be used with other options (e, h, c, t) )

* Board Trace & All of Networking Trace: Simple Display Type

* Specific CO Trace: Complete Display Type

=> Simple Display Type shows the Networking Connection Message and Information Elements, but all of Information Elements (IE) could not be analyzed. Only including data of Called IP, Calling Number, Called Number and Cause elements could be analyzed, and then these data is printed.

=> Complete Display Type shows the Networking Connection Message, Information Elements and all of data contained on Information Elements (IE) data.

## 4.3  Supported Messages of Enhanced Trace

### 4.3.1  Digital Line (PRI, VOI, BRI, STI-t, NPRI, NBRI) Type

In case of digital line, networking messages are analyzed by the standard ETSI/ITU protocol.

Enhanced Trace Supported Messages for Networking Connection

| Call Establishment Message | Call Information Phase Messages |
| --- | --- |
| ALERTING CALL PROCEEDING CONNECT CONNECT ACK PROGRESS SETUP SETUP ACK | USER INFORMATION |
| Call Clearing Messages | Miscellaneous Messages |
| DISCONNECT RELEASE RELEASE COMPLETE | FACILITY INFORMATION STATUS STATUS REQ |

* Additional Messages on ISDN also could be supported.

Enhanced Trace Supported Message for Information Elements

| Networking Information Element | Decoded & Print / Only Print | Networking Information Element | Decoded & Print / Only Print |
| --- | --- | --- | --- |
| IE_BEARER_CAPABILITY | Decoded & Print | IE_CONNECTED_NO | Decoded & Print |
| IE_CAUSE | Decoded & Print | IE_CONNECTED_SUBADDR | Only Print Data |
| IE_CALL_IDENTITY | Only Print Data | IE_CALLING_NO | Decoded & Print |
| IE_CALL_STATUS | Decoded & Print | IE_CALLING_SUBADDR | Only Print Data |
| IE_CHANNEL_INFO | Decoded & Print | IE_CALLED_NO | Decoded & Print |
| IE_AOC | Only Print Data | IE_CALLED_SUBADDR | Only Print Data |
| IE_FACILITY | Only Print Data | IE_TRANS_NET_SELECT | Decoded & Print |
| IE_PROG_INDICATION | Decoded & Print | IE_LOW_LAYER_COMPATIBLE | Only Print Data |
| IE_NET_SPECIFIC_FACILITY | Only Print Data | IE_HIGH_LAYER_COMPATIBLE | Only Print Data |
| IE_NOTIFY_INDICATION | Decoded & Print | IE_USER_USER | Only Print Data |
| IE_DISPLAY | Only Print Data | IE_CONGEST | Decoded & Print |
| IE_DATE_TIME | Decoded & Print | IE_ISDN_FLASH_REQ | Only Print Data |
| IE_KEYPAD_FACILITY | Only Print Data | IE_ISDN_FLASH_ACK | Only Print Data |
| IE_CALLING_IP | Decoded & Print |  |  |
| IE_CALLED_IP | Decoded & Print |  |  |

*Blue Mark: able to decode IE & print the analyzed results. And the others is printed raw (HEX Value) data.

### 4.3.2  Analog Line Type (DCO, LCO) Type

In this case Enhanced Trace could show the related CO commands to communicate with other systems. Analog messages dose not include any information elements, so Enhanced Trace show only the sent or received digit.

DCO case

| Supported Messages |  |  |  |
| --- | --- | --- | --- |
| Receive Message Part |  | Send Message Part |  |
| Message Type | When the Msg is appeared | Message Type | When the Msg is appeared |
| RING DETECT | Receive The Call | SEIZE | Try to Seize the CO Line |
| RING STOP |  | RLS | CO Release |
| METERING | Detect Call Metering Signal | ANSWER | Answer the Call |
| LOOP IDLE |  | DGT SENT | Digit Sent |
| DGT SENT ACK |  | SND REGISTER RCL |  |
| SEIZE ACK | Receive CO Seize ACK | SND READY |  |
| SEIZE NAK |  | SND END OF DIAL |  |
| RLS ACK | Receive CO Release ACK | SND END OF DIAL BUSY |  |
| RCV DGT | Receive Digit | MAKE IDLE |  |
| RCV ANS | Receive Answer Signal | SND BLOCKING |  |
| RCV BLOCK |  | ANI DGT OP | Send Another Digit |
| RCV END OF DIAL |  |  |  |
| CALLER ID | Receive Caller ID |  |  |
| R2 END OF SIGNAL |  |  |  |
| ANI SVC START | Detect Another Service Start |  |  |
| ANI DGT SENT |  |  |  |

*Blue Mark: Able to see the related digit.

LCO case

| Supported Messages |  |  |  |
| --- | --- | --- | --- |
| Receive Message Part |  | Send Message Part |  |
| Message Type | When the Msg is appeared | Message Type | When the Msg is appeared |
| RING START | Receive The Call | SEIZE | Try to Seize the CO Line |
| RING STOP |  | RELEASE | CO Release |
| CALL METERING | Detect Call Metering Signal | ANSWER | Answer the Call |
| LOOP SUP DETECT |  | SEND_DIGIT | Digit Sent |
| DIAL TX ACK |  |  |  |
| SEIZE ACK | Receive CO Seize ACK |  |  |
| SEIZE NAK |  |  |  |
| RELEASE ACK | Receive CO Release ACK |  |  |
| FLASH ACK |  |  |  |
| DISA DGT |  |  |  |
| TONE DETECTED | Detect the Tone |  |  |
| RELEASED |  |  |  |
| CID DETECT | Receive Caller ID |  |  |

*Blue Mark: Able to see the related digit.

## 4.4  Enhanced Trace Examples

### 4.4.1  Incoming Call with PRIB. (ISDN Call)

CO 1 Enhanced Device Trace

Complete Display Type

Receive the SETUP networking message

with bearer cap, channel, calling number

of Information Elements

Printed IE Data

Send the SETUP_ACK networking message

with channel, prog_indicator of Information Elements

Calling Number

‘4501000’

Printed IE Data

Printed IE Data

Receive the INFORMATION networking message

with called number ‘3’

Receive the INFORMATION networking message

with called number ‘0’

Receive the INFORMATION networking message

with called number ‘0’

Receive the INFORMATION networking message

with called number ‘1’

Send the PROCEEDING networking message

Send the ALRTING networking message

Send the CONNECT networking message

with ‘3001’ connected number

Receive the CONNECT ACK networking message

Receive the DISCONNECT networking message

with ‘normal call clearing’ message

Send the RELEASE networking message

Receive the RELEASE Complete networking message

### 4.4.2  Outgoing Call with VOIB. (NET Call)

Enhanced Board Trace

Simple Display Type

Data of Information Elements have not been printed, except Called IP, Calling No, and Called No.

Send the SETUP networking message

by sending complete with bearer cap, channel, called IP, calling number, called number

Receive the CALL PROCEEDING

Receive the ALERTING with facility

Receive the CONNECT with called number ‘2000’

Send the CONNECTACK

Send the RELEASE COMPLETE

This message means the DISCONNECT in this case.

Receive the RELEASE

Receive the RELEASE COMPLETE

### 4.4.3  Outgoing Call with LCOB. (Analog Call)

Enhanced all of Networking CO Trace

Simple Display Type

CO 35 Seize Request Message.

CO 35 Seize ACK Message.

Sending Digit

‘2000’

TALKING

CO 35 Release Request

CO 35 Release ACK

Dial Digit ‘2’

Dial Digit ‘0’

Dial Digit ‘0’

Dial Digit ‘0’