LG Key System Software

Technical Information

STI-0062

Sept 13th 2004

| Title | #### Introduction of the VMIBE/AAIBE on LDK series |
| --- | --- |
| System | LDK-50/100/300/600 |
| Abstract | This document includes a brief information of the VMIBE/AAIBE software upgrade method |

Revision History

| Revision | By | History |
| --- | --- | --- |
| 1.0A | J. H. Kim | 2004. 09. 13 |

Table of Contents

1. Introduction	3

1.1	General Description	3

1.2	What is improved	3

2.	SOFTWARE uPGRADE 	4

Software upgrade guidance	4

# 1. Introduction

  - General Description

The VMIBE/AAIBE is the Enhanced Voice Mail Interface Board/Enhanced Auto Attendant Interface Board of LDK-50/100/300/600 Digital Key Telephone System. The main application of the VMIBE/AAIBE is to record and play back system announcements and individual messages. Digitized PCM voice data transmitted to the VMIBE/AAIBE through system back-plane highway will be compressed or decompressed by the algorithm (ADPCM; G.726) of the DSP (TMS320C54CST) chip and stored to the Flash memory (K9F2808U0B).

Basically, the VMIBE/AAIBE supports 4 voice channels, and 1 optional Voice Channel Expansion Unit (VCEUE) can be installed on the VMIBE to get the maximum channel capacity up to 8 channels.

Also, the VMIBE allows us to record message for 5 hours. When an optional Flash Memory Expansion Unit (FMEU2) is installed on the VMIBE, recording time will be increased up to 10 hours.

In case of the AAIBE, recording time for individual message is of 3 minutes and for system announcement, 10mimutes. Note that no FMEU will be installed onto the AAIBE.

The VMIBE/AAIBE is made up of the CPU (ARM7-TDMI, S3C45430A), 2Mbytes program memory (NOR Flash), 2Mbytes SRAM, 80Mbytes (Max 160Mbytes) NAND Flash memory for message storage, maximum 2 DSP circuits (supports 4- channels, Max 8 channels), reset & watchdog circuit and miscellaneous board status circuits.

This VMIBE/AAIBE can be served on any version of the released MP software in which the VMIB, VMIB2, or AAIB can be supported.

The block diagram of VMIBE/AAIBE is as shown below.

  - What is improved

The voice quality, specially the musical sound of the VMIBE/AAIBE is improved (99.9% original sound).

Also, the firmware software on the VMIBE/AAIBE can be upgraded by RS-232C. Some new features including message rewinding, message adding, and CID storage can be activated with VMIBE/AAIBE only.

The Multiple Language Unit (MLU) can be used to provide two or more languages in a board.

However, LG has no plan to develop it by for now.

# 2 SOFTWARE UPGRADE

The establishment for VMIBE/AAIBE Monitor Program

-   RS-232C cable (Baud Rate 19200)

-   PC/Notebook

-   VMIBE/AAIBE Board

The one side of RS-232C cable should be connected to a VMIBE/AAIBE, and the other side should

be connected to a PC.

After setting up, turn on the system, the initial screen of VMIBE/AAIBE Monitor Program will be displayed on the hyper terminal.

- VMIBE

- AAIBE

Please type the password (‘millionire’). With correct password, you can see the screen as shown below.

If you type the character “?”, you can see the monitor Command List.

The command for software upgrade is ‘z’. If you type the character ‘z’, you can see the following screen.

Type the character ‘z’ once again, and the monitor program will ask the “yes” or “no” of z-modem execution. If you type the character ‘y(yes)’, it is displayed the comment ‘NAND Flash erasure has been finished..’.

And then, click the menu tab ‘Transfer (T)’ on the hyper terminal screen, Select the menu ‘Transfer files (S)’

Search and enter the path of the upgrading software and click the Send (S).

Transmission will be on via Z-modem as the following screen, and total estimated time of transmission will be of 15 or 16 minutes (Baud Rate 19200).

Type the character ‘y’ after completing download.  The 2nd, 3rd, and 4th LED of VMIBE/AAIBE LED will be on and off for deleting Flash Rom. Subsequently, the whole LED will be on and off for inspecting the Flash Rom. Finally, the 5th, 6th, 7th, and 8th led of VMIBE/AAIBE LED will be on and off for recording the upgrading software to Flash Rom.

As soon as the software upgrading is done, VMIBE/AAIBE will be reset automatically. Then, the initial screen will be displayed again.