LIP-7016D & 7024D

TABLE OF CONTENTS

1.	Introduction	1

1.1	General	1

1.2	Feature Information	1

1.3	Feature Groupings	1

1.4	iPECS LIP-7016D & 7024D Phone Description	2

2.	iPECS Phone LCD, Speakerphone & related features	5

2.1	LCD operation	5

2.2	Using the Speakerphone	5

2.3	Using a Bluetooth Wireless Headset	6

3.	Receiving calls	7

3.1	Answering a call while idle	7

3.2	Responding to a call a while busy	8

3.3	Using Answering Machine Emulation (AME)	9

3.4	Differential Ring Signals	10

3.5	Answering Calls at Night	10

3.6	Answering Calls to other stations	11

3.7	Blocking Incoming Calls with Do-Not-Disturb (DND)	11

3.8	Forwarding Calls	12

3.8.1	Forwarding calls to another station or Voice Mail	12

3.8.2	Forwarding calls to an external number	13

3.8.3	Forwarding callers to a Text Message	14

4.	Placing a call	16

4.1	Internal Calls	16

4.1.1	Placing Intercom calls	16

4.1.2	Calls to a busy station, Camp-On	17

4.1.3	To leave a Message Wait	17

4.2	External Calls	18

4.2.1	Placing external CO/VoIP calls	18

4.2.2	To wait for the next available CO/IP line	19

4.2.3	Tracking External calls for billing, Account Codes	19

4.2.4	To disable dialing from your phone	20

4.2.5	To temporarily override dialing restrictions at a phone	20

4.3	Using Automated Dialing	21

4.3.1	Using Last Number Redial (LNR)	21

4.3.2	Using and Entering Save Number Dial	21

4.3.3	Using System Speed Dial Numbers	22

4.3.4	Using and Entering Station Speed Dial Numbers	22

4.3.5	Using Dial by Name & Entering your Name	24

4.3.6	To retry a busy external number until answer, ACNR	24

5.	Transfer, Conference & other Call Handling	25

5.1	Sending a call to a different destination, Call Transfer	25

5.2	Placing a call in a waiting state, Call Hold	25

5.3	Switching between two calls, Broker Call	26

5.4	To join multiple people in a Conference	26

5.5	Placing a call on hold to Page, Call Park	27

5.6	Making a recording of a call, Two-Way Record	27

6.	Leaving & Responding to Messages	28

6.1	Responding to a Station Message Waiting Indication	28

6.2	Responding to a CLI Message indication	29

6.3	Getting your Voice Mail Messages	30

6.4	Sending and Receiving Short Text Messages	31

7.	Remote System Access	32

7.1	Accessing the System remotely, Direct Inward System Access (DISA)	32

7.2	Mobile Phone Extension	32

8.	Miscellaneous Features	33

8.1	Background Music and Music on Hold	33

8.2	Using Internal, External and Meet-Me Paging	34

8.3	Push-To-Talk (PTT) Page	35

8.4	Wake-Up Alarm	35

8.5	Alarm/Door Bell Operation	36

8.6	Power Fail Transfer Operation	36

8.7	System Voice Memo	36

8.8	Hot Desk	37

9.	Working with PBX, Centrex and ISDN features	38

9.1	Access PBX or Centrex features	38

9.2	Access ISDN network features	38

9.2.1	Using the Keypad facility to access ISDN features	38

9.2.2	Using ISDN Supplementary services	39

9.2.3	Activating ISDN Caller Id restriction	39

10.	Setting Features & Flex buttons	40

10.1	Entering Station Program Data	40

10.2	Assigning features to Flex buttons	40

11.	Appendix A  Flexible Numbering Plan	41

12.	Appendix B  User Program Codes	43

# Introduction

## General

Your telephone is connected to an advanced technology highly versatile fully featured telecom system designed to make office communications easy and productive.  Employing state of the art Voice over Internet Protocol (VoIP) technology, voice and data converge on a single IP packet network.  Because each iPECS Phone is an IP appliance, it can be moved easily throughout the office LAN and maintain normal operation.

## Feature Information

We have taken every effort to make this user guide simple and straightforward.  The guide starts with the simpler operations of the iPECS Phone Speakerphone, moves to receiving and placing calls and then to more advanced features.  Each section includes a brief and basic description of each feature and step-by-step operations

The operations shown in this guide use the system’s base default Numbering Plan.  Your Numbering Plan may be different.  Some features may not be available for you to access or may be subject to certain limitations based on the set-up of the system

## Feature Groupings

Features have been divided into groups, Receiving calls, Placing calls etc.  Within each group, features are arranged according to the difficulty of operation and frequency of use.  Every effort has been taken to divide the features into a logical and consistent sequence so that features can be quickly located.

## iPECS LIP-7016D & 7024D Phone Description

The iPECS LIP-7016D and LIP-7024D Phones incorporate the latest in VoIP technology and user interface to provide you with a cost effective, simple to use, productive communications tool.  It includes a large 3-line 24-character per line LCD.  The bottom line of the display is an interactive menu accessed with the 3 Soft buttons just below the display.  The Navigation button in the lower center of the Phone allows you to move through the menu choices when more than three selections are available.

Features and functions of your iPECS are accessed using the Fixed or Flexible buttons or through selecting a menu item using the Soft buttons.  In some cases a feature may be accessed through the Fixed/Flex buttons or via the Menu.  In the remainder of this User Guide, the Fixed or Flexible buttons are show with a box surrounding the button designation while Soft button menu selections are shown shaded in blue.

iPECS LIP-7016D

iPECS LIP-7024D

Interactive

LCD

Handset

Flex buttons

3 Soft buttons

Navigation

button

Microphone

Speaker

Fixed Feature buttons:

Speaker

Hold/Save

Speed

Trans/Pgm

Dnd

Call Back

Volume

Dial pad

iPECS LIP-7016D/7024D call-outs

iPECS Phone LAN connection

iPECS Phone to PC connection

iPECS Phone Handset jack connection

iPECS Phone headset jack

iPECS LIP-7016D/7024D connections

# iPECS Phone LCD, Speakerphone & related features

## LCD operation

The Liquid Crystal Display (LCD) of the iPECS LIP-7016D and LIP-7024D Phones has three lines of 24 characters each.  The LCD is used to convey information to you.  It provides date, time and station number display while idle.  It will also provide called/calling name/number display, feature status and an interactive menu to guide you through feature access and User Program selections.

## Using the Speakerphone

Except for Paging, you can activate the iPECS Speakerphone at any time in place of lifting the handset to receive or place calls.  Place calls with On-hook Dialing; dialing while using the Speakerphone.

In addition, you may switch between the handset and Speakerphone during a call.  By keeping the handset Off-hook, you activate the Group Listen feature, which provides incoming audio over the speaker with outgoing audio from the handset.  This permits a local ‘group’ to listen to both sides of the conversation without interfering with the conversation.

While in a conversation using the Speakerphone, you can Mute the microphone with the MUTE Soft button.  When Mute is active, the MUTE Soft button changes to SPEAK and is used to turn off Mute.

With Auto Speaker Select, when you activate/access a feature by pressing its button, your Speakerphone is activated automatically.

When using a Headset, the speaker button controls the On/Off hook state for the phone.

operation

To activate the Speakerphone to answer or place a call;

- Press speaker, the phone goes off-hook with audio from the speaker and to microphone.

To control the volume of the Speakerphone or handset;

- Press ▼ volume ▲.

To Mute the microphone;

- Select the MUTE Soft button.

To turn off MUTE (turn the microphone on);

- Select the SPEAK Soft button.

To activate Group Listen while on an active handset call;

- Press speaker.

## Using a Bluetooth Wireless Headset

The iPECS LIP-7024D may be equipped with the option BTU (Bluetooth Unit) that lets you use a Wireless headset such as the Jabra BT-200, BT-250 or Plantronics M3000.  Using the headset permits you to move freely around the phone within approximately 10 meters and maintain a conversation.  Before you can use the headset, the headset must be “paired” with the BTU.  Once paired, the headset must be enabled for use.  Operation of the iPECS LIP-7024D with the headset is the same as with the Speakerphone; merely use the Speaker button to go on and off-hook.  Note during the pairing operation, the headset and Phone should be as close as possible but not more than 1 meter apart.

operation

To pair the wireless headset to the BTU;

- Press trans/pgm,

- Dial 3,

- Press hold/save.

To enable/disable the wireless headset;

- Press trans/pgm,

- Dial 4,

- Press hold/save.

To unpair the headset should it need to be replaced;

- Press trans/pgm,

- Dial 3,

- Press hold/save.

# Receiving calls

## Answering a call while idle

There are 3 basic types of incoming calls; internal/external ringing calls, Intercom Voice Announce calls, and Hold recalls.

Your phone may be assigned Ringing Line Preference; calls that ring are answered by lifting the handset.  Otherwise, you must press the flashing Flex button.

External calls are from CO lines or VoIP channels that are programmed to ring at your phone.  For external calls, Delay Ring may be assigned allowing others to answer the call before the call rings at your phone.

External calls to an iPECS Phone will appear under the CO/IP line button or a LOOP button

You might receive calls forwarded in some manner from another user.  If you provide call coverage for other users, you may also receive Station Call Coverage calls.

Calls you place on Hold, including Transfers, parked calls, etc. will recall if left on hold for too long.

ICM Voice Announce calls, which are only available to the iPECS Phone, are answered based on the ICM Signaling mode at the receiving phone; in HF (Hands-free) mode, after splash tone and announcement, speak normally into the microphone.  In the PV (Privacy) mode, lift the handset to respond.  You can assign the ICM Signaling mode in Station Programming.

The LCD will display the calling number or, if available, the name for the station or CO/VoIP line, CO Line Name Display.

operation

To answer a call ringing at your phone;

- Lift the handset and, if required, press the flashing Flex button, the call is connected.

To respond to an Intercom Voice Announce call;

- In HF mode signaling, simply speak as normal into the microphone or,

- In Privacy Mode signaling, lift the handset to respond.

## Responding to a call a while busy

While you are busy on another call, you may receive Muted Ring, Camp-On tones, or Voice-Over announcements.  Muted Ring is provided over the speaker of your Phone indicating a Call Waiting.  You may receive Camp On tones, a burst of tones on top of the existing call, as an indication of another call waiting.

While busy, certain stations can activate Voice Over to your phone.  In this case, you receive audio from both the active call and the Voice Over announcement at the same time.

You may respond to any of these Off-hook Signals in one of several ways; you may place your existing call on Hold and respond to the new incoming call, you may activate One-time DND, you may send a Silent Text Message or, you may ignore the new call.

Silent Text Message requires a text message button, and cannot be sent to a SLT or other non-display terminal.

The Attendant or Secretary may use Intrusion to announce a call while you are busy.

operation

To answer a Call Waiting;

- Press hold/save and/or,

- Press the flashing hold/save button.

To activate One-time DND;

- Press dnd.

To send a Silent Text Message;

- Press the desired text message button.

## Using Answering Machine Emulation (AME)

When a call is sent to your voice mail-box, you can screen the call as you would with a normal Answering Machine.  The caller’s voice is played over your speaker while the message is recorded.  There are two methods of notification and call screening provided, Ring or Speaker mode.

In the Ring mode, you are notified by flashing of the AME (Answering Machine Emulation) Flex button.  You may press the Flex button to hear the caller as the voice message is stored.  In the Speaker mode, when the call is sent to the Voice Mail-box, the caller’s voice is automatically broadcast over the speaker of your iPECS Phone.

You may terminate the screening leaving the caller in voice mail to record a message, talk with the caller and record the conversation in the mail-box, or answer the call and disconnect the Voice Mail.

operation

To assign an AME button:

Ring Mode	trans/pgm + flex + 564 + 0 + hold/save

Speaker Mode	trans/pgm + flex + 564 + 1 + hold/save

To screen a call in the Ring mode;

- Press the flashing ame button, the caller’s voice is broadcast over the station speaker and stored in the Voice Mail-box.  In the Speaker operation mode, broadcast is automatic.

To stop the voice broadcast and leave the caller in Voice Mail;

- Press the illuminated speaker button.

To talk with the caller and record the conversation in Voice Mail;

- Press the SPEAK Soft button.

To answer the call and cancel the voice message;

- Press the illuminated ame button, the caller is connected and the Voice Mail disconnected.

## Differential Ring Signals

When multiple phones in a small area ring, it can be difficult to tell which are ringing.  The iPECS Phone has 14 Ring Tones available for differentiating ring from one phone to another.  Four of the tones are stored in the phone’s permanent memory; the remaining ten tones are in the system’s memory.  Four of these ten can be downloaded into the phone memory for use as the 5th  to 8th Ring Tone.

operation

To download a Ring Tone from system’s memory;

- Press trans/pgm,

- Dial 1 for Ring Tones,

- Dial 5 for Ring Tone download,

- Dial the memory location to receive the tone (5~8),

- Dial tone number 0~9, tone is heard,

- Press hold/save to download.

To select a Ring Tone from the phone’s memory;

- Press the trans/pgm button,

- Dial 1 for Ring Tones,

- Dial 1 or 2 for Internal or External ring,

- Dial tone number 1~8,

- Press hold/save to make the selection.

## Answering Calls at Night

In the Night mode, Loud Bell Control may be used to send ring signals to external bells.  You may then answer these calls with Universal Night Answer (UNA).

operation

To answer a call ringing over an external bell;

- Lift the handset,

- Dial the UNA code 567.

## Answering Calls to other stations

When nearby stations are ringing, you may pick-up (answer) the call.  You may select to answer a call ringing at a specific station (Directed Call Pick-Up) or you may choose to answer the oldest call ringing to your station group (Group Call Pick-Up).  Assign a Flex button with the Group Call Pick-Up code (566) for one-touch access.

Note that Pick-Up will not answer calls on the ringing station’s Private Line unless you also have an appearance of the Private Line.

operation

To answer a call ringing at another station;

- Lift the handset,

- Dial Directed Call Pick-Up code 7,

- Dial the Intercom Number of the ringing station, call is connected.

To answer a call ringing at a station in your group;

- Lift the handset,

- Dial Group Call Pick-Up code 566, the call is connected.

## Blocking Incoming Calls with Do-Not-Disturb (DND)

When you need quiet, you can activate Do-Not-Disturb and the phone will not receive ring.  Callers will receive a fast busy tone and displays will indicate the DND status.  You can activate Do-Not-Disturb while busy, One-Time DND.  One-Time DND will be active only for the duration of the present ringing call.

When an Executive activates DND, calls are forward to the Secretary of the pair automatically.

The Attendants and Secretary may be able to activate DND Override and Intrude on an active call.  The Attendants may also cancel DND at other stations.

operation

To toggle DND On and Off;

- Press dnd.

## Forwarding Calls

### Forwarding calls to another station or Voice Mail

You may request or, with Preset Call Forward, the system may divert your incoming calls to other resources of the system.  Resources include other stations, VSF Voice Mail and External Voice Mail.  Your LCD and the LCD of the forward receiver will both indicate the forward status.

Call Forward can be activated from your phone or from a different station, Call Forward, Follow-me.  You may define the conditions or ‘type of forward’ as below:

1:	Unconditional, all calls to the station, except recalls, are forwarded internally or externally.

2:	Busy, immediately forwards all calls, except recalls, when station is busy.

3:	No Answer, forwards all calls to the station, except recalls, when the station does not answer within the No Answer timer.

4:	Busy/No Answer, forwards calls if the station is busy or does not answer within the No Answer timer.

When you forward calls from a different station, you may need to enter your Authorization Code.

Calls on a Private Line will not forward except to a station with an appearance of the Private Line or to Voice Mail.

operation

To activate Call Forward from your phone to another phone or internal system resource;

- Lift the handset,

- Press the FWD Soft button,

- Dial the type of forward code 1~4,

- Dial the destination.

To deactivate Call Forward from your phone;

- Press the FWD Soft button.

To activate Call Forward for your phone from a different phone (Call Forward, Remote);

- Lift the handset,

- Press the FWD Soft button,

- Dial the Remote Call Forward code 0,

- Dial your Authorization code (station number & Password),

- Dial the ‘type of forward’ code 1~4,

- Dial the destination.

To deactivate Call Forward, Remote;

- Lift the handset,

- Press the FWD Soft button,

- Dial the Remote Call Forward code 0,

- Dial your Authorization code (station number & Password),

- Dial #.

### Forwarding calls to an external number

You can forward your calls to an external CO/IP destination.  Calls can be forward from an internal station or a remote location.

You may define the conditions or ‘type of forward’ as below:

1:	Unconditional, all calls to the station, except recalls, are forwarded internally or externally.

2:	Busy, immediately forwards all calls to the station, except recalls, when station is busy.

3:	No Answer, forwards all calls to the station, except recalls, when the station does not answer within the No Answer timer.

4:	Busy/No Answer, forwards calls if the station is busy or does not answer within the No Answer timer.

When you forward calls from a different station or a remote location, you may need to enter your Authorization Code.  In addition, from a remote location, you must access the system through a DISA enabled CO line.

operation

To activate Call Forward, Off Premise (forward to an external number) from your phone;

- Lift the handset,

- Press the FWD Soft button,

- Dial the ‘type of forward’ code 1~4,

- Press speed,

- Dial Speed Dial bin number,

- Replace the handset, return to idle.

To activate Call Forward, Off Premise (forward to an external number) from a different phone;

- Lift the handset,

- Press the FWD Soft button,

- Dial the Remote Call Forward code 0,

- Dial your Authorization code (station number & Password),

- Dial the ‘type of forward’ code 1~4,

- Press speed,

- Dial Speed Dial bin number,

- Replace the handset, return to idle.

To activate Call Forward, Off Premise (forward to an external number) from a remote location;

- Lift the handset,

- Dial the telephone number of a DISA assigned CO line,

- While receiving dial-tone, dial the Call Forward feature code 554,

- Dial your Authorization Code (station number & Password)

- Dial the ‘type of forward’ code 6~9,

- Dial Speed Dial bin number,

- Replace the handset, return to idle.

### Forwarding callers to a Text Message

Using an iPECS Phone, you may request the system return a ‘Text Message’ to callers.  The LCD of the caller’s iPECS Phone will show the selected message.  Internal callers receive the selected message or you can send a text message in response to an internal call, Silent Text Message.

There are 11 Custom Display Messages each up to 24 characters.  Ten are assigned by the Attendant and one assigned by you, see Station Speed Dial Character entry chart.

In addition, there are ten fix Pre-Selected Text Messages, some allow auxiliary input for time, date, etc., see bold italics below.

Msg #	Text

01	lunch return at  hh:mm

02	on vacation

return date           mm:dd

03	out of office

return at time      hh:mm

04	out of office

return date           mm:dd

05	out of office

return unknown

06	call ;       up to 17 digits

07	in office : sta xxxx

08	in meeting

return at time      hh:mm

09		at home

10	at branch office

operation

To activate Custom or Pre-Selected Message Forward;

- Press trans/pgm,

- Dial User Program feature code 51,

- Dial the two-digit text Message code (00-20),

- Dial any auxiliary input for messages 01~04 and 06~08,

- Press hold/save, forward confirmed.

To cancel activated Message;

- Press the FWD Soft button.

To program the Station Custom Message (00) at your station;

- Press trans/pgm,

- Dial User Program feature code 52,

- Enter Message contents, up to 24 characters,

- Press hold/save, message saved.

# Placing a call

## Internal Calls

### Placing Intercom calls

You can place calls to other stations in the system over the Intercom.  The call will ring or, for an ICM Voice Announce call, you will hear splash tone.  Ring or Voice Announce is normally determined by settings at the called station but you may change this by dialing ‘#’ after calling the user.  By changing the mode, a Voice Announce call will ring or a ringing call will allow Voice Announce.

You can assign a Flex button for Direct Station Selection/Busy Lamp Field (DSS/BLF).  A dss/blf button lets you call the other station with a single button press.  The button LED displays the other station’s status, On, is busy, Off, is idle.

When you go off-hook, you may connect to another station or system resource automatically.  This Prime Line Preference may be immediate or delayed allowing you to take other actions during the delay.  Otherwise, you may press another Flex button before going off-hook to override Prime Line Preference.

If you go off-hook and take no action, Intercom dial-tone will time-out and after receiving Howler Tone, your phone will be placed out-of-service in Intercom Lock-Out.  You must hang-up to return to a normal state.

operation

To place an Intercom call;

- Lift the handset,

- Dial the Intercom number of the desired station,

- At answer or Splash tone, speak to the called party.

### Calls to a busy station, Camp-On

If your called party is busy, you may activate Camp-On, Callback or Voice Over to the station.  With Camp-On, you wait off-hook for the called station to answer.  With Callback, you wait on-hook for the station to return to idle at which time; the system will recall you and then automatically call the previously busy station.

If you are the Attendant or Secretary, you may activate Intrusion.  You may also use Step Call; dial the next station by dialing just the last digit of the Intercom number.

operation

To wait off-hook for a busy station, Camp-On;

- Dial ‘*’ and await answer.

To wait on-hook for a busy station; Callback;

- Press call back and hang-up.

To make a Voice Over announcement to a busy station;

- Dial #, after splash-tone, speak.

### To leave a Message Wait

If the called user does not answer or is in DND, you may leave a Message Wait.  The called user will receive a Message Wait indication; the LED of the call back button will flash.

operation

To activate Message Waiting,

- Press call back and hang-up.

## External Calls

### Placing external CO/VoIP calls

External calls are placed on CO/IP lines.  These ‘lines’ are accessed either by a button on the iPECS Phone assigned as a CO/IP line or by dial codes.  The CO/IP lines are grouped for different types of calls, local, long distance, etc.  Using the CO/IP line group code, you will be able to access any line from the group.  A Flex button on the iPECS Phone assigned as a LOOP button lets you access a line from a group.  Dial codes for access to CO/IP lines are:

Individual CO/IP line access

88 + Line number (01~42 for iPECS-100 or 001~200 for iPECS-300).

CO/IP Group access

801~820 for iPECS-100 or 801~872 for iPECS-300

Any CO/IP Line access

9

During dialing on a CO line, which uses pulse style signals, you may activate Dial Pulse to Tone Switchover changing to DTMF style dialing for banking services, etc.

When placing an IP call, you must dial an IP address using ‘*’ in place of the dot.

If Least Cost Routing is set-up, the number you dial will be analyzed and the system will place the call on the least expensive route.

You may be assigned Dialing and/or CO/IP Line Access Restrictions, in which case you will receive error tone if you attempt to dial or access a restricted number/CO/IP line.  CO/IP line and Station Class Of Service (COS) may vary during Day and Night operation (Day/Night COS) and set dialing restrictions.  If you dial an Emergency Number, the system will ignore any restrictions and complete the emergency call.

You may need to enter an Authorization code before placing calls, in which case you will receive a second dial tone.

Your external calls may be subject to Call Time Restrictions, you will receive a Call Warning Tone prior to disconnect.

operation

To place an external call;

- Lift the handset,

- Select/Dial the CO/IP line/group,

- Dial the desired number.

### To wait for the next available CO/IP line

operation

To activate a queue if the selected/dialed line is busy;

- Select/Dial a busy CO/IP line button,

- Press call back,

- Hang-up.

### Tracking External calls for billing, Account Codes

In some cases, you or your company may desire to track calls for billing or other purposes.  You may enter an Account Code, which is output from the system to a printer in the SMDR report.  You must have an Account Code Flex button, see Assigning features to Flex buttons.

operation

To enter an Account Code before the call;

- Lift the handset,

- Press the assigned account code button,

- Dial the Account Code (1-12 digits),

- Press *, Intercom dial tone is heard,

- Place the CO/IP call as normal.

To enter an Account Code during a call;

- Press the assigned account code button,

- Dial the Account Code (1-12 digits),

- Dial *.

### To disable dialing from your phone

You can temporarily ‘Lock’ your phone by activating Temporary COS, which will deny all dialing from the phone.  To deactivate Temporary COS you must enter the station’s Authorization code.

operation

To activate Temporary COS;

- Press trans/pgm,

- Dial User Program code 21,

- Press hold/save.

To deactivate Temporary COS;

- Press trans/pgm,

- Dial User Program code 22,

- Dial your Authorization code,

- Press hold/save.

### To temporarily override dialing restrictions at a phone

You may override the dialing restrictions at any phone by activating Walking COS.  The Walking COS dialing restriction levels are applied for only a single call.  Thus for multiple calls, you will need to reactivate Walking COS.  However, you may use Flash to maintain control of the CO/IP line and you will not need to reactivate Walking COS.

operation

To activate Walking COS;

- Press trans/pgm,

- Dial User Program code 23,

- Dial your Authorization Code,

- Place call as normal.

## Using Automated Dialing

### Using Last Number Redial (LNR)

The last number dialed on an external call is automatically saved in the Last Number Redial (LNR) buffer.  For iPECS Phone users, the system will store the numbers dialed on the last ten calls.

operation

To dial the Last Number for an external call;

- Lift the handset,

- Select the REDIAL Soft button,

- Press ▼ volume ▲ to select from the last 10 numbers dialed.

- Press the SEND Soft button or hold/save.

If you select the NEXT Soft button, the LCD will show the previous dialed number.

### Using and Entering Save Number Dial

You can instruct the system to store the last number dialed on an external call to the Save Number Dial buffer.  You can store a Save Number for quick redial at any time.  This number will be stored until a new number is stored and can be easily redialed at any time.

operation

To place a call using Save Number Dial;

- Lift the handset,

- Press speed,

- Dial #.

To store a number in the Save Number Dial;

- While on an outgoing external call, press speed twice.

### Using System Speed Dial Numbers

The iPECS-100 the system has memory allocated for 800 System Speed Dial numbers (bins 200~999).  The iPECS-300 system has memory allocated for 3000 System Speed Dial numbers (bins 2000~4999).  You may, if allowed, use the System Speed Dial to dial frequently called numbers.

In some cases, these numbers are divided into groups called System Speed Zone Groups.  In this case, you may access some numbers and not others and your dialing restrictions may be invoked.

Only an Attendant can enter System Speed Dial numbers.

operation

To place a call using System Speed Dial;

- Lift the handset,

- Press speed,

- Dial the desired System Speed Dial bin number (200~999 for iPECS-100 or 2000~4999 for iPECS-300).

### Using and Entering Station Speed Dial Numbers

Your iPECS Phone has access to 20 or 100 Station Speed Dial numbers.  For users in an iPECS-100, 20 Station Speed Dial numbers (bins 00~19) are available.  The iPECS-300 supports 100 Station Speed Dial numbers (bins (000~099).  You can easily program each speed dial bin with telephone numbers, up to 48 digits, to meet your needs.  The numbers may include several ‘special’ instructions:

PAUSE will momentarily stop dialing.

FLASH as 1st digit – activate dial tone detect.

FLASH not as 1st digit – the system will generate a flash on the CO line.

call back as 1st digit – Send digits as ISDN Keypad Facility message.

‘*’ as 1st digit - Display Security, the stored number will not be displayed when used.

‘*’ not as 1st digit, - Dial Pulse to Tone Switchover, the system will switch from pulse to tone dialing.

When entering a Speed Dial, a 16 character name may be associated with the number for Dial by Name.  Characters are entered with two keystrokes as in the chart below.

| Q – 11 Z – 12 . – 13 1 – 10 | A - 21 B - 22 C - 23 2 - 20 | D - 31 E - 32 F - 33 3 - 30 |
| --- | --- | --- |
| G – 41 H - 42 I - 43 4 - 40 | J - 51 K - 52 L - 53 5 - 50 | M - 61 N - 62 O - 63 6 - 60 |
| P - 71 R - 72 S - 73 Q - 7* 7 - 70 | T - 81 U - 82 V - 83 8 - 80 | W - 91 X - 92 Y - 93 Z - 9# 9 - 90 |
| *1 - Blank *2 - : *3 - , | 0-00 | # |

operation

To place a call using Station Speed Dial;

- Lift the handset,

- Press speed,

- Dial the desired Station Speed Dial bin number (00~19 or 000~099).

To enter a Station Speed Dial number with CO Line and Name;

- Press trans/pgm,

- Press speed,

- Dial the desired Station Speed Dial bin number,

- Select/Dial the CO/IP line/group,

- Dial the desired number,

- Press hold/Save,

- Enter the associated name if desired,

- Press hold/Save.

To assign a Station Speed Dial number directly to a Flex button;

- Press trans/pgm,

- Press the desired Flex button,

- Select the TEL NUM Soft button,

- Select/Dial the CO/IP line/group,

- Dial the desired number,

- Press hold/Save,

- Enter the associated name if desired,

- Press hold/Save.

### Using Dial by Name & Entering your Name

Dial by Name employs three ‘Directories’: Intercom, User (Station) Speed Dial and System Speed Dial.

To allow others to contact you via the Intercom directory, you must enter a name for your station, up to 12 characters.

operation

To place a call using Dial by Name;

- Press speed twice.

- Dial the desired directory, 1: Intercom, 2: User Speed Dial, 3: System Speed Dial, the LCD displays the first two names, in alphabetical order.

- Scroll using ▼ volume ▲.  To enter characters, refer to the Station Speed Dial character entry chart.

- Press hold/save to place the call.

To enter your station user name;

- Press trans/pgm,

- Dial 74,

- Dial name, up to 12 characters (see Station Speed Dial),

- Press hold/save.

### To retry a busy external number until answer, ACNR

You may request and have the system retry a busy external call until the call is connected, the feature is cancelled or the maximum re-try’s are attempted.

For each re-try, the system will activate your speaker then place the call with the microphone muted.  When the remote end answers, you must select the SPEAK Soft button or lift the handset.  Either of these actions will cancel the ACNR request as well as connect you to the remote party.

operation

To have the system periodically re-try a busy number, Automatic Called Number Redial (ACNR);

- Use the Navigation button display next menu and Select the ACNR Soft button,

- Hang-up handset,

To cancel the ACNR request;

- Select the ACNR Soft button.

# Transfer, Conference & other Call Handling

## Sending a call to a different destination, Call Transfer

You can send an active call to another station or other resource of the system, Call Transfer.  You can screen the transfer with the receiving party (Screened Call Transfer) or complete the transfer without screening (Unscreened Call Transfer).

Transferred calls, internal or external, are place in Transfer Hold, will receive MOH and will recall if not answered in a timely manner.

If an Unscreened Transfer call encounters an error or DND, it may immediately recall you.

DSS/BLF buttons may be employed to transfer calls.

operation

To Transfer an active call;

- Press trans/pgm,

- Call receiving party,

- For Unscreened Call Transfer, hang-up,

- For Screened Call Transfer, at answer or Splash tone, announce call.

## Placing a call in a waiting state, Call Hold

You may place an active Internal or External call in one of several holding states.  In System Hold, other non-restricted stations may pick-up the call.  In Exclusive Hold, only the holding station may pick-up the call.

The system is assigned a preferred hold type (System or Exclusive).  You may override this Hold Preference by pressing the hold/save button twice.

The LEDs for buttons assigned CO/IP line flash at distinctive rates for the holding station and other iPECS Phone users.

There are also user operations such as pressing a dss/blf button that will place a call in Automatic Hold.

Calls will remain in the held state for a period and then recall the user.  This Hold Recall is provided with a different ring signal.  If not answered in a timely manner, the Attendant will receive Attendant Hold Recall.

operation

To place a call on Hold;

- Press hold/save.

## Switching between two calls, Broker Call

You may switch between the active call and a call on Hold, placing the active call on Hold acting as a broker between two or more parties, Broker Call.  iPECS Phone users may ‘broker’ for multiple calls up to the number of CO/IP line assigned buttons.

operation

To switch between two calls, Broker Call;

- Press the desired CO/IP line assigned button.

## To join multiple people in a Conference

You can establish a Conference with up to 3 parties.  The other parties in the Conference may be internal or external.

A Conference can be placed on Hold and is subject to Hold Recall.

You may also use the Conference operation to join 2 external parties in a private conversation.  This is an Unsupervised Conference, which is not subject to the normal Hold Recall operation.

operation

To establish a Conference;

- Establish call with one of the desired conference parties,

- Select the CONF Soft button.

- Establish call with the other conference party,

- Select the CONF Soft button.

- Select the CONF Soft button again to establish the conference.

To place a Conference on Hold;

- Press the hold/save button.

To retrieve the Conference from Hold;

- Select the CONF Soft button.

To establish an Unsupervised Conference;

- Establish conference with two external parties,

- Select the CONF Soft button.

To retrieve an Unsupervised Conference;

- Select the CONF Soft button.

## Placing a call on hold to Page, Call Park

A user may transfer an active CO/IP call to a special holding location (Park Orbit), which can be accessed easily from any station in the system.  Typically, this feature is used with Paging to notify the desired user of the Call Park.

Parked calls are subject to Hold Recall after the Call Park timer.

operation

To park an active external call;

- Press trans/pgm,

- Dial the Park Orbit (601-610 for iPECS-100 or 601-619 for iPECS-300),

- Return to idle.

To retrieve a parked call;

- Lift the handset,

- Dial the Park Orbit (601-610 for iPECS-100 or 601 for iPECS-300).

## Making a recording of a call, Two-Way Record

You may record an active external conversation in your Voice Mail Box.

operation

To activate Two-Way Record while on an CO/IP call;

- Use the Navigation button to display next Menu and Select the RECORD Soft button, record warning tone heard and recording starts.

To stop Two-Way Record while on an CO/IP call;

- Use the Navigation button to display next menu and Select the RECORD Soft button or,

- Hang-up, return to idle.

# Leaving & Responding to Messages

Your iPECS Phone allows you to send and receive several types of messages.  These include a Station Message Waiting, Calling Line ID (missed call) Message Waiting, Voice Mail Message Waiting and SMS (Short Text Message) Message Waiting.

Each of the message types is described in the following paragraphs and is accessed via the Message Summary display or using a Flex button as described in the section on each message type.

## Responding to a Station Message Waiting Indication

Another station can leave a Station Message Waiting indication when you do not answer or your phone is in DND.  A flashing call back button on the iPECS Phone indicates a Message Waiting.  As a further notification, a Message Wait Reminder Tone can be provided.  You may review and delete messages as well as respond with a call back.

operation

To review your Station messages;

- Press call back, the LCD shows the Message Summary display,

ST   CL   VS   VM   FS     MS

001  001  005   006   001   004

- Dial 1 to view the Station messages

To return a call for your first message;

- Press call back again to return message.

To delete the Station Message Wait indications;

- Dial ‘*’

- Dial 1.

To delete all Station Message Wait indications;

- Dial ‘#’

- Dial 1.

## Responding to a CLI Message indication

When external calls to your phone are not answered and Caller Identification is provided from the network, a Calling Line Id. (CLI) message is logged.  You may review, delete or return these messages.

operation

To review CLI Messages using a Flex button;

- Press the flashing cli Message button, the first CLI message information is displayed,

- Press ▼ volume ▲ to scroll through CLI message wait information.

To review CLI Messages using the Summary Display;

- Press the flashing call back button, the LCD shows the Message Summary display,

ST   CL   VS   VM   FS     MS

001  001  005   006   001   004

- Dial 2, the first CLI message information is displayed,

- Press ▼ volume ▲ to scroll through CLI message wait information.

To delete the current CLI Message;

- Dial ‘*’ button, the next CLI message is displayed.

To delete all CLI Messages;

- Press speed,

- Press * twice.

To return the current CLI message;

- Press hold/save.

## Getting your Voice Mail Messages

When callers are forwarded or recall to your Voice Mail Box, they can leave a voice message.  Your Voice Mail Box is part of the integrated Voice Store & Forward (VSF) module, the iPECS Feature Server or an external Voice Mail system.

These Voice Mail systems allow access to and management of the received voice messages.  The VSF allows you to listen to, save, and forward messages.  While in your Mail Box, you have control of your password and Mail Box greeting.  In addition, you can control Call Forward for your station remotely from the VSF.  You will need to register an Authorization Code for your station to access the VSF Voice messages.

operation

To retrieve Voice Mail locally using a Flex button;

- Press the VM Flex button,

- Dial your Mail Box number and corresponding password to receive the ‘Number of Messages’ prompt,

- Dial desired option codes,

- At completion of session, hang-up to return to idle.

To retrieve Voice Mail locally using the call back button;

- Press the flashing call back button, the LCD shows the Message Summary display,

ST   CL   VS   VM   FS     MS

001  001  005   006   001   004

- Dial 3 for VSF, 4 for external VM or 4 for Feature Server Voice Mail

- After the prompt enter your station number and password,

- Dial desired option codes,

- At completion of session, hang-up to return to idle.

To access Voice Mail Box from a Remote Location;

- Lift the handset,

- Dial the telephone number of a DISA CO line assigned for answer by a VSF Auto Attendant,

- Upon answer, dial # to receive ‘Mail Box & Password’ prompt,

- Dial the Mail Box and corresponding password to receive the ‘Number of Messages’ prompt,

- Dial desired option codes,

- At completion of session, hang-up.

## Sending and Receiving Short Text Messages

You can exchange short text messages (max. 100 characters) with other iPECS display Phone users.  These messages will activate the call back button LED are displayed in the iPECS Phone LCD.

operation

To send an SMS message to another iPECS display Phone user;

- Press the trans/pgm button,

- Dial 36,

- Dial 1 to resend an existing message, or 2 to send a new or edit an existing message,

- Enter the station range to receive the message, to send to a single station enter the station number twice,

- Dial your message using 2 key strokes for each character,

- Press hold/save to send your message.

To view your received Short text messages;

- Press the flashing call back button, the LCD shows the Message Summary display,

ST   CL   VS   VM   FS     MS

001  001  005   006   001   004

- Dial 6, the first two SMS messages are shown in brief,

- Dial the Message number to view the entire SMS message

To delete a received Short text messages;

- Press #,

- Dial 1 to delete the message, 2 to cancel or 3 to delete all received Short text messages.

# Remote System Access

## Accessing the System remotely, Direct Inward System Access (DISA)

DISA (Direct Inward System Access) lets remote users call and gain access to the system’s resources.  The system will recognize remote user dialed inputs.  The user can call other stations, place calls over CO/IP lines, review voice mails, etc.

The remote user may be required to enter an Authorization Code.

operation

To access system resources remotely;

- Call the system’s DISA facility

- Dial your authorization code (station number and password)

- Dial as needed for the desired system resource.

## Mobile Phone Extension

When away from your desk or office you can place and receive calls on you mobile phone.  Calls to your iPECS Phone will ring your Mobile Phone and you can place calls using the resources of the iPECS.  You may be allowed to activate the Mobile Phone Extension feature and assign the Mobile Phone number.

operation

To register a the mobile phone number;

- Press the trans/pgm button,

- Dial 37.

- Dial the mobile phone number.

- Press the hold/save button.

To activate a registered mobile phone from the user’s station:

- Press the trans/pgm button.

- Dial 38.

- 1 to activate, 0 to deactivate,

- Press the hold/save button.

To place a call from the mobile extension using the iPECS:

- Dial the ISDN DID number of the station, the system will check the Caller Id, answer the call and the user will receive intercom dial tone,

- Place internal or external iPECS call as normal.

To Transfer a call from the mobile extension using the iPECS:

- Dial “*” while on an iPECS call.

- Dial the desired extension, the call is transferred and the mobile phone returns to idle.

Note: the mobile may reconnect by dialing #.

# Miscellaneous Features

## Background Music and Music on Hold

Two audio sources can be connected to the system.  These sources or an internal source provide input for Background Music (BGM) and Music On Hold (MOH).  BGM is played over the speakers of iPECS Phone under user control.  MOH is played to callers placed on Hold.

There are three possible selections for BGM and MOH:

Off

Source BGM1

Source BGM2

operation

To turn on Background Music;

- Press hold/save to cycle through the BGM selections.

## Using Internal, External and Meet-Me Paging

You may broadcast announcements to other stations and/or external speakers.  Stations are assigned to one or more of the Internal Page Zones.  The system has two External Page Zones that are connected to external speakers.

Page Zone codes are:

Internal Page Zones 	iPECS 100 	501~510

iPECS 300 	501~535

Internal All Call Page	543

External Page Zones	545~546

External All Call Page	548

All Call Page		549

You may queue if the desired Page Zone is busy.  You are recalled when the queue can be honored.

You may request a paged party to answer a Page using Meet-Me Paging.  In this case, the paged party may answer your page from any phone in the system by dialing the Meet-Me code.

operation

To make a page;

- Lift the handset,

- Dial the desired Page zone,

- If assigned, after page warning tone, make announcement.

To queue for a page when busy is received;

- Press call back,

- Replace the handset returning to idle.

To answer a Meet-me-Page;

- Lift the handset.

- Dial the Meet-Me-Page code 544.

## Push-To-Talk (PTT) Page

Your iPECS Phone can be assigned as a member of one or more of the system’s nine Push-To-Talk page groups.  You may log-in or log-out of any one or all PTT groups to which you are assigned.  Once logged in, you may place or receive one-way page announcements to/from other users who are logged in to the same PTT group.  To place a PTT page announcement, you must press and hold the ptt Flex button.

operation

To log-in to a PTT group;

- Dial #0, the PTT Log-in/out code.

- Dial the desired PTT group number (1~9 or 0 for all groups).

To log-out of the PTT group(s);

- Dial #0, the PTT Log-in/out code.

- Dial *.

To place a page to the active PTT group;

- Press and hold the ptt Flex button.

- After confirmation tone make page announcement.

## Wake-Up Alarm

You can set the station Wake-Up Alarm clock.  The alarm clock can be set to repeat daily or as a one-time alarm.

When the user responds to the alarm by lifting the handset, BGM is provided.

operation

To set the Wake Up Alarm clock;

- Press trans/pgm,

- Dial the Station User Program code 41,

- Dial 2-digit hour and minute for alerting,

- For a daily (repeating alarm), dial #,

- Press hold/save.

To erase Wake-Up;

- Press trans/pgm,

- Dial the Station User Program code 42,

- Press hold/save.

## Alarm/Door Bell Operation

The system can monitor external Alarm or Door Bell contacts.

As an Alarm monitor, should the external contact activate, you may receive a unique Alarm Ring signal and LCD display.  To restart the monitor circuit, the external contacts must be deactivated and the Alarm Ring signal terminated.

As a Door Bell, you may receive a unique Door Bell Ring (single tone burst) each time the external contact is activated.  You may also control a system contact to act as a Door Open contact.

operation

To terminate the Alarm signal;

- Dial the Alarm Stop code 565.

To activate the Door Open contacts;

- Lift the handset,

- Dial Door Open code #* and contact number 1 to 4.  Note contacts 3 & 4 are only available in the iPECS -300.

## Power Fail Transfer Operation

When AC power to the system fails, back-up operation may be provided with back-up batteries or Power Fail Transfer (PFT).  When battery back-up is provided the station will operate normally.  When PFT is implemented, an SLT is connected to a CO line by the system.  This SLT can be used as any normal SLT, providing service while power is out.

## System Voice Memo

The integrated Voice Store & Forward (VSF) module incorporates several Voice Memos to provide the station users with general information such as station number, date, time, and feature status.  Voice Memos are heard over the speaker of the iPECS Phone and over the handset for SLTs.

For the Station Status Memo, the following items are reported.  Note only activated features are announced.

- ICM Signaling Mode (Handsfree/Tone/Privacy)

- Station IP Address

- Station Mac Address

- Number of messages x (x: number of all message waiting)

- Wake-Up Time (hh:mm)

- Do Not Disturb

- Forwarded to station xxx

- Forwarded to speed bin xxx

- Queued CO/IP line xxx

- Locked (Temporary COS)

- COS x

operation

To hear Date & Time memo

- Dial Voice Memo feature code (661 for iPECS-100 or *661 for iPECS-300), date and time memo is heard, “Date is May 2nd.Time is xx:xx pm”.

To hear Station Number Prompt

- Dial Voice Memo feature code (662 for iPECS-100 or *662 for iPECS-300), the station number memo is heard, “This is station 150”.

To hear Station Settings

- Dial Voice Memo feature code (663 for iPECS-100 or *663 for iPECS-300), Station Status Memo is heard.

## Hot Desk

iPECS Phones in the system may be assigned for Hot Desk operation.  These Hot Desk Phones let you log-on and use the Hot Desk Phone as if it were your station.  Once you log on, the Hot Desk station takes on the attributes assigned to your station and you can make calls as normal.  When you log out, calls to your station are forward to the destination you chose.

To Log on to a Hot Desk station;

- Lift the handset,

- Dial “* 0”.

- Dial your Authorization Code (station number and Password).

To Log out of the Hot Desk station;

- Press the trans/pgm button.

- Dial “* *”.

- Use the ▼ volume ▲ to select a call forward destination for your calls, speed dial, joined mobile phone, VSF, or VM group.

- Press the hold/save button.

# Working with PBX, Centrex and ISDN features

## Access PBX or Centrex features

The iPECS has been designed to operate with a host PBX system or Centrex services from your service provider.  The iPECS connects to such a host with analog CO lines.

When you access such a CO line, your phone will act as an extension of the host, allowing access to the host system features through dial codes of the host.

While on a call, you use the iPECS Flash feature to transfer calls to other host extensions or access features of the host.

The Flash feature may be used on other CO lines to regain dial tone from the CO line without relinquishing control of the CO line to place another call.

operation

To access PBX or Centrex features while idle;

- Lift the handset,

- Select/dial access a PBX/Centrex CO line,

- Dial PBX/Centrex feature code.

To access PBX/Centrex features while on a PBX/Centrex CO line;

- Use the Navigation button to display next menu and Select the FLASH Soft button,

- After receiving new dial tone, dial PBX/Centrex feature code.

## Access ISDN network features

### Using the Keypad facility to access ISDN features

Access to certain features of your ISDN services requires that dialed digits be sent to the ISDN as data and not tones.  In this case, dialed digits are sent as ‘Keypad messages’.

You must have a keypad facility button assigned using the Flex Button program procedures.  Once active, you may dial into the ISDN for services; however, actions other than dialing will deactivate the ‘Keypad message’ operation.

operation

To activate Keypad Facility operation;

- Press the keypad facility assigned button to switch to keypad mode,

- Dial as desired.

To deactivate Keypad Facility operation;

- Press keypad facility,

- or

- Press a Feature or Flex button.

### Using ISDN Supplementary services

ISDN service providers may incorporate features similar to those of Centrex.  These ISDN Supplementary services are provided on a subscription basis and, as with Centrex, are accessed using the Flash feature.

iPECS Phone users notify the ISDN of a feature request using the FLASH Soft button.  An ISDN line can be placed on Hold and a second call placed on the line.  You may then alternate between the two held calls (Broker Call) using the hold/save button.  An ISDN line conference can be established using the FLASH and CONF Soft buttons.  The conference can also be placed on hold.

operation

While on an ISDN CO line, to access ISDN Supplementary services;

- Select the FLASH Soft button,

- Press Hold/save, and Select the CONF Soft button, or CO line assigned button to activate the desired feature.

### Activating ISDN Caller Id restriction

iPECS Phone users can restrict the transmission of Caller Id to the far-end.  This may be a subscription service of your carrier.  Separate Flex buttons are required to activate outgoing (Connected Line) and incoming (Calling Line Id) id restriction.

operation

To restrict Caller Id;

- Lift the handset,

- Press the CLIR/COLR assigned button,

- Place call as usual.

# Setting Features & Flex buttons

## Entering Station Program Data

A special code set is available for iPECS Phone users to assign certain attributes such as Authorization Codes (Passwords).  After entering the User Program mode, you dial the codes to select the attribute and setting.  Once selected, the setting is saved.  A list of the User Program codes, the feature/function and any input required are given in Appendix B.  In some cases, you enter these codes as part of Flex button assignments.

operation

To assign user programmable station attributes;

- Press trans/pgm,

- Dial desired User Program codes shown in Appendix B,

- Press hold/save.

## Assigning features to Flex buttons

You may assign features and functions to the Flex buttons on your iPECS Phone.  If allowed, you may even assign CO/IP lines to the Flex buttons.

Flex buttons may be assigned for most features providing One-Touch feature activation e.g., a Flex button may be assigned to access the Account Code feature and may include the Account code digits, as a one-touch account code button.

Features that may be assigned to a Flex button include:

DSS/BLF – enter station number,

Speed Dial – enter Station/System Speed dial bin number,

Flex Numbering Plan – enter Flex Numbering Plan code, see Appendix A,

User Program Code – press trans/pgm and enter a User Program code, see Appendix B, or

CO/IP line – enter the CO/IP line or group number.

operation

To assign a feature to a Flex button;

- Press trans/pgm,

- Press the desired Flex button,

- Dial desired code and required inputs,

- Press hold/save.

# Appendix A  Flexible Numbering Plan

| Feature | 100 | 300 | Remark |
| --- | --- | --- | --- |
| Intercom Call | 100~169 | 100~399 |  |
| Internal Page Zone | 501~510 | 501~535 |  |
| Internal All Call Page | 543 | 543 |  |
| Meet Me Page | 544 | 544 |  |
| External Page Zone 1~2 | 545~546 | 545~546 |  |
| External All Call Page | 548 | 548 |  |
| All Call Page (Internal/External) | 549 | 549 |  |
| SMDR Account Code Enter | 550 | 550 | SLT use only |
| Flash Command to CO Line | 551 | 551 | SLT use only |
| Last Number Redial | 552 | 552 | SLT use only |
| DND (Toggle On/Off) | 553 | 553 | SLT use only |
| Call Forward | 554 | 554 |  |
| Speed Dial Programming | 555 | 555 | SLT use only |
| Message Wait/Callback Enable | 556 | 556 | SLT use only |
| Message Wait/Callback Return | 557 | 557 | SLT use only |
| Speed Dial Access | 558 | 558 | SLT use only |
| Cancel DND/FWD/Pre MSG | 559 | 559 | SLT use only |
| CO System Hold | 560 | 560 | SLT use only |
| Program Mode Access | 561 | 561 | SLT use only |
| Attendant Unavailable | 562 | 562 |  |
| Alarm Reset | 565 | 565 |  |
| Group Call Pickup | 566 | 566 |  |
| Universal Answer | 567 | 567 |  |
| Account Code with bin | 568 | 568 |  |
| Walking COS Code | 569 | 569 |  |
| ACD Supervisor On/Off Duty | 571 | 571 |  |
| ACD Supervisor Login | 572 | 572 |  |
| ACD Supervisor Logout | 573 | 573 |  |
| ACD Help Code | 574 | 574 |  |
| ACD Calls In Queue Display | 575 | 575 |  |
| ACD Supervisor Status Display | 576 | 576 |  |
| ACD Supervisor Monitor | 577 | 577 |  |
| ACD Reroute Queued Call w/answer | 578 | 578 |  |
| ACD Reroute Queued Call w/o answer | 579 | 579 |  |
| Camp-On Answer | 600 | 600 | SLT use only |
| Call Parking Locations | 601~610 | 601~619 |  |
| Group Pilot Number | 620 – 659 | 620 – 667 |  |
| Station User VSF Features | 66 | *66 |  |
| Call Coverage button | 67 | 67 |  |
| Direct Call Pickup | 7 | 7 |  |
| CO/IP Group Access | 8xx 01~20 | 8xx 01~72 |  |
| Individual CO/IP Line Access | 88xx 01~42 | 88xxx 001~200 |  |
| Retrieve Last Held CO/IP | 8* | 8* |  |
| Retrieve Individual Held CO/IP (01~42) | 8#xx | 8#xxx | xx(x) = CO Line number |
| Access CO Line in the 1st available CO Group | 9 | 9 |  |
| Attendant Call | 0 | 0 |  |
| VM Message Wait Enabled | *8 | *8 |  |
| VM Message Wait Disable | *9 | *9 |  |
| Door Open (1st Door ) | #*1 | #*1 |  |
| Door Open (2nd Door ) | #*2 | #*2 |  |
| Door Open (3rd Door ) | #*3 | #*3 | LiK-300 only |
| Door Open (4th Door ) | #*4 | #*4 | LiK-300 only |
| MCID Request | *0 | *0 |  |
| AME Feature | 564 | 564 |  |
| Unsupervised Conference extend code | ## | ## |  |
| PTT Group Login/Logout | #0 | #0 |  |

# Appendix B  User Program Codes

| Code | Function | Remarks | SLT |
| --- | --- | --- | --- |
| 10 | Ear&Mic Headset Usage | iPECS LIP-7000 only |  |
| 11 x | Differential Ring, Intercom | Select Ring Tone 1~8 |  |
| 12 x | Differential Ring, CO/IP line | Select Ring Tone 1~8 |  |
| 13 x | Intercom Answer Mode | Select ICM Signal mode, 1: HF, 2: TONE, 3: Privacy |  |
| 14 x | Call Coverage Attribute Setting ( X = 1-2 ) | 1+; On/Off, 2+: ring delay (0~9) |  |
| 15 x | Station Ring Download | Select download Ring Tone 0~9 |  |
| 19 | Enblock Mode Dialing | iPECS LIP-7000 only |  |
| 21 | Knock Down Station COS |  | Yes |
| 22 | Restore Station COS | May require Authorization code | Yes |
| 23 | Walking COS | May require Authorization code | Yes |
| 31 | Station Message Wait Retrieve |  | Yes |
| 32 | CLI/IP Message Wait Retrieve |  |  |
| 33 x | Authorization Code (Password) Entry | Station number +up to 12 digits, no Flex button | Yes |
| 34 | Assign DID Call Wait Button |  |  |
| 35 | Message Wait in Executive/Secretary pair |  |  |
| 36 | Send SMS Message |  |  |
| 37 | Register Mobile Extension |  |  |
| 38 | Activate Mobile Extension |  |  |
| 41 xx | Set Wake-Up Time | Input time, 24 hour clock | Yes |
| 42 | Erase Wake-Up Time |  | Yes |
| 51 xx | Custom/Pre-select Message Display | Select Message 00~20 | Yes |
| 52 xx | Register Custom Message (Message 00) | Input up to 24 characters | Yes |
| 61 x | Headset/Speakerphone Mode |  |  |
| 62 x | Headset Ring Mode | 1: Speakerphone, 2: Headset, 3: both |  |
| 71 | LCD Display Mode |  |  |
| 72 | Version Display |  |  |
| 73 | Background Music |  |  |
| 74 x | Station User Name Registration | Input up to 7 characters | Yes |
| 75 | Display Phone IP Address |  |  |
| 76 | Change Phone IP Address |  |  |
| 77 | Display Phone MAC IP Address |  |  |
| 78 | Change Mode |  |  |
| 79 | Display Phone Version |  |  |
| 70 | Display Soft Message |  |  |
| 7* | Display Serial number/Package for SMEMU |  |  |
| 80 | Assign Record Button | Requires VSF/External SMDI VM |  |
| 81 | Assign ISDN CLIR Button |  |  |
| 82 | ISDN COLR Button |  |  |
| 84 | Assign Account Code Button |  |  |
| 85 | Assign LOOP Button |  |  |
| 86 | Assign ATD Intrusion Button |  |  |
| 88 | Assign Camp-on Button |  |  |
| 89 | Assign Keypad Facility Button |  |  |
| 8# | Assign OHVO Button |  |  |
| 90 | Speed Button Assignment | iPECS LIP-7000 only |  |
| 91 | Conf Button Assignment | iPECS LIP-7000 only |  |
| 92 | Call Back Button Assignment | iPECS LIP-7000 only |  |
| 93 | Dnd Button Assignment | iPECS LIP-7000 only |  |
| 94 | Flash Button Assignment | iPECS LIP-7000 only |  |
| 95 | Mute Button Assignment | iPECS LIP-7000 only |  |
| 96 | Monitor Button Assignment | iPECS LIP-7000 only |  |
| 97 | Redial Button Assignment | iPECS LIP-7000 only |  |
| 98 | Fwd Button Assignment | iPECS LIP-7000 only |  |
| 99 | Assign PTT Button |  |  |
| *0 | Hot Desk Login Code |  |  |
| ** | Hot Desk Log out Code |  |  |