| To: Lev Butsev | Doc. No. | SEC-RN-ACD-01122009-10 |
| --- | --- | --- |
| CC: | Country | Russia |
| From : Seegyoung Seol (SEC) | Tel: Fax: E-mail: |  |
| Product: OfficeServ ACD V3.1.2.1 (January 12, 2009) |  |  |

For Lab Testing Only

- Descriptions:

## New Features and Bugs Fixed

- Single setup file

- Exiting Server service without opening Windows Services

- Overflow to Phone reinstated

- CLI/DID to Agent (if not answered or busy failed, queued in alternative split)

- Only one Agent instance can be launched (To allow multiple instances, set SINGLE_INSTANCE in .ini to 0)

- License: 30-day trial license (PBX-based, 1 Supervisor/Reporting/Wallboard, 10 Agents)

- License: No RAD/IVR port license

- Sorry Message (play Sorry to new call before putting into Q if no available agent). To turn it off, set USE_NOT_AVAIL_IVR_MSG in Work.ini to 0

- Ring back tone timer. The default ring tone time is 0 sec (it is heard for 0.5 sec). To change the ring back duration, modify RINGBACK_TM_IN_GW in Work.ini.

## Updated Module

- All

## Known Bugs

- After changing the ACD Server IP, the RAD should be restarted

- Agent: Logging in with the same Agent ID and different station results in wrong statistics

## Remarks

- Overflow to Phone target shouldn’t be Agent Station (ACD Statistics becomes wrong)

- Agent should have only one call button (MMC 722: Remove second call button)

- ACD call is not routed to an agent with ALL FWD or Follow Me setting

- Call Pickup is not supported (resulting in wrong statistics)

- Agent should not belong to UNCONDITIONAL station group (MMC 601)

- Do not run both ACD and UCD simultaneously

- SIP license is required for RAD

- # IVR ports in MMC 601, ACD Supervisor, and IVR Engine should be identical

- Phone-only agent can log-on to one split (no multi-split login)

- 64-bit machine is not supported

- In Exceptions of Windows Firewall, register the ports 18828, 17774, 30000, and 30001

(Or turn off Windows Firewall)

- The ACD Server icon doesn’t display in the System Tray if the user logs in to the XP

SP3 server remotely for the first time.

- Compatibility:

| Model/ Version | OfficeServ 7200 OfficeServ 7400 | MCP V4.30b |
| --- | --- | --- |
|  | Link | V3.0.0.4 (December 02, 2008) |
|  | IVR | V1.5.2.1 (December 22, 2008) |
| OS/DB | Windows XP SP2 Windows Server 2000 Windows Server 2003 Windows Vista (Client only) | SQL Express SQL Server 2000/2005 |