| To : David Nicholls | Doc. No. | SEC-RN-020118-02 |
| --- | --- | --- |
| C.C. : David Ridley | Country | UK, Italy |
| From : Gary Hong | Tel: 82-2-751-2684 Fax:82-2-751-2684 E-mail: hongky@samsung.co.kr |  |
| Subject: DCS / Compact-II S/W V7.01 |  |  |

Overview :

## DCS / Compact-II S/W

[DCS]        MP: V7.01 (01.12.08)

[Compact-II]  MP: V7.01 (01.12.08)

Contents :

- Euro Currency is applied.

- Italy requirements are added.

S/W Compatibility Table :

[DCS]

|  | MP | SP | LP | PRI | BRI | PCMMC |
| --- | --- | --- | --- | --- | --- | --- |
| Version | V7.01 | V7.0 | V7.0 | V3.4 | V3.4 | 7.01 |
| Date | 011208 | 011012 | 011012 | 011106 | 011106 | 011208 |

[Compact-II]

|  | MP | PRI | BRI | PCMMC |
| --- | --- | --- | --- | --- |
| Version | V7.01 | V3.4 | V3.4 | 7.01 |
| Date | 011208 | 011106 | 011106 | 011208 |

Checksum :

[DCS]

| Even 1 | Odd 1 | Even 2 | Odd 2 |
| --- | --- | --- | --- |
| #### 00FE | A94C | 07EC | 759F |

[Compact-II]

| Even 1 | Odd 1 | Even 2 | Odd 2 |
| --- | --- | --- | --- |
| #### FC84 | 8D7D | 0604 | C9CC |

[MP]

From T6.34(010723) to T6.35(010809)

- When using the ‘Call Return’ feature, when LCR is used, not all the CLI digits get sent to line. It appears to be cutting off the first seven digits. (i.e. 0398722900 becoms 900). When four digits are inserted into the LCR Modify Table-Insert digits, uch as 1414, the digits 1414039 gets cut off and only the digits ‘8722900’ get sent to line. Inserting pause before the ‘1414’ will solve this problem.

 Fixed.

- For KOREA, CID name field is increased to 30 digits.

- MMC 110 NOT CONT.CID option is added. (default: ON)

When this option is set to OFF, the LCD shows the CID number during incoming trunk call.

Ex)   CID number: 0312187647

701:0312187647 CONF PAGE MUTE->

- MMC 110 call cost option is opened to all country.

- For Greece, the LCR insert and append digit is increased to 30 digits, but LCR Modification Table Entry is reduced. (Greece Only)

- The Korean Langauge display error is fixed. (Korea Only)

- For Italy, CO busy tone cadence is changed to 500 ms On/Off from 200 ms On/Off. (Italy Only)

- To solve the Australia DCS restart problem, we increase the OS stack size and change the Dummy interrupt processing. (DCS only)

[MP]

From T6.35(010809) to V6.36(010823)

- In case of KOREA, analog CID cause display is changed. (KOREA only)

- The FALCON 8D will not be serviced the KOREAN language display. (KOREA only)

- We changed the MFR parameter to solve the problem that the Keyphone System does not recognize the DTMF signal sent by some single line telephone. (ex: SIEMANS SLT).

- The KOREAN language insert method is changed to “천지인” type. (KOREA only)

[MP]

From V6.36(010823) to V7.00(010925)

- When AA transfer 2 PRI calls to the UCD group simultanously, the system restart problem is occurred. This problme is fixed.

- FALCON korean language service error is fixed. (Korea Only)

- In case of KOREA, the R2 singal is changed to B5 from B4 which is sent to the caller when the user dial the invalid number. (Korea Only)

- When FALCON keyset is connect newly, the DSS key or DTS key of the FALCON keyset does not turn on even though the station or trunk is busy. This problem is fixed.

- MMC 210 E&M DIALTONE option is added. (Korea Only)

- For Israel, MMC 501 system timer default is changed as follows. (Israel Only)

- HOOK FLASH TIME MAX = 800 ms

- HOOK FLASH TIME MIN = 200 ms

- HOOK OFF TIME = 130 ms

- AA TRANS TIME = 2 sec

- The RING LED of DS-24SE keyset service works wrong. This problem only occures after Version 6.30. It is fixed.

- The SVMi-8 card interface is added.

[MP]

From V7.00(010925) to V7.01(011208)

- The Korean display when the caller id is empty is changed. (Korea Only)

- We changed S/W when the FLASH feature id is pressed on SLT, the first alarm will be cleared to provide the method to cancel the alarm on SLT.

- [Italy Only] MMC 430 ISDN Number Type & Plan MMC is added.  This MMC setting only available on the ISDN S/W version V3.4

- [Italy Only] MMC 300 Night Passcode option is added.

- [Italy Only] MSN call wait option operation is changed.

In case of being assigned into “NO”, the call wait on MSN will work same as DDI operation.

6.  [Italy Only] Add LCR/ENBLOCK tone instead of MSGWAT tone.

DND -> DND/MSGWAT

MSGWAT -> LCR/ENBLOCK

DIAL -> DIAL/TRSFER

7.  Italy Only] Changing Italian day name.

- It is fixed that the CTI interface does not deflect the incoming call by LCR feature.

- [DCS/053] Forward no answer timer on DECT extension does not follow MMC setting value.

: Fixed.

- [DCS/054] Display problem showing 20202020 when the DID call is transferred to busy station is fixed. (UK only problem)

- It is fixed when the Forward Delay feature does not work when forward destination is set to station group.

12.  EURO Currency is applied.

- Related MMC

MMC 210  USE EURO (added)

MMC 508  Cost Unit display is changed to “ECNT” when the MMC 210 USE EURO

is set to ON.

- Operation

: When USE EURO option is set ‘ON’, EURO mark will be displayed in front of call  cost on.

: display format

Keyset : E30,40 (‘E’ position is not fixed.)

SMDR: E  30,40(‘E’ position is fixed)