# Field Report for the DCS

| Bug No. |  | DCS/066 |  |  |  | SYSTEM |  |  |  |  |  |  |  |  | VERSION |  |  |  | CLOSED |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date |  | 13/09/02 |  |  |  | DCS |  |  |  |  |  |  |  |  | No. |  |  | Date | Ver |  |  | Date | Type | ## By |  |
| From |  | W. Wood |  |  |  |  |  |  |  |  |  |  |  |  | T7.04 |  |  | 18/07/02 |  |  |  |  | MP |  |  |
|  |  | (NG) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | S/W |  |  |
| Country |  | UK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HW/SW |  |  |  | H/W |  |  |  |  |  | X | S/W |  |  |  |  |  | PCMMC |  |  |  |  |  |  |  |  |
| Feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  | New |  |  |  |  | X |  | Bug/Always |  |  |  |  |  | Others |  |  |  |  |  |  |  |  |
|  |  |  |  | Change |  |  |  |  |  |  | Bug/Intermittent |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Description (Detailed description of a problem or a feature, new or current) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # key does not work with AME feature to send ringing call directly to mailbox on SVMi-8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Operation (Step by step sequence to reproduce a problem) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. Install SVMi-8 (software level V2.1 06.02.02) and program system accordingly |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. Set VMMEMO feature in MMC 724 to #. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. Enable VMAME in MMC 701 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. When a call rings at an extension with AME set, allow call to go to VM, * key drops out of AME and caller leaves a message normally. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5. When a call rings at an extension and before AME starts pressing # should send the caller directly to your mailbox: this feature does not work |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6. This feature does work with T7.02 software |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Configuration (The system environment, installation) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Customer/Site information (Customer/Site where the problem or feature addition of change request is reported) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Version of related software (to be filled in when necessary) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SP : ver 7.1 |  |  |  |  |  |  | AA : 2.04 |  |  |  |  |  |  | PRI ver 3.4 |  |  |  |  |  | BSI : |  |  |  |  |  |
| LP : |  |  |  |  |  |  | VD : |  |  |  |  |  |  | BRI : |  |  |  |  |  | PCMMC : |  |  |  |  |  |
| Programming (programming required to reproduce a problem or required for a new/modified feature) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Remarks (Related problems, history, etc.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Management results |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Duplication |  |  |  |  |  |  | Status |  |  |  |  |  |  | Waiting for more information |  |  |  |  |  |  | Requested By |  |  |  |  |
|  | Success |  |  |  | Failed |  |  | Open |  |  |  |  | Closed |  |  |  |  |  |  |  |  |  |  |  |  |