# Field Report for the DCS

| Bug No. |  | DCS/070 |  |  |  | SYSTEM |  |  |  |  |  |  |  |  | VERSION |  |  |  | CLOSED |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date |  | 17/10/02 |  |  |  | DCS |  |  |  |  |  |  |  |  | No. |  |  | Date | Ver |  |  | Date | Type | ## By |  |
| From |  | W. Wood |  |  |  |  |  |  |  |  |  |  |  |  | T7.06 |  |  | 04/09/02 |  |  |  |  | MP |  |  |
|  |  | (NG) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | S/W |  |  |
| Country |  | UK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HW/SW |  |  |  | H/W |  |  |  |  |  | X | S/W |  |  |  |  |  | PCMMC |  |  |  |  |  |  |  |  |
| Feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  | New |  |  |  |  | X |  | Bug/Always |  |  |  |  |  | Others |  |  |  |  |  |  |  |  |
|  |  |  |  | Change |  |  |  |  |  |  | Bug/Intermittent |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Description (Detailed description of a problem or a feature, new or current) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Calls that reach a UCD group when overflowed from another are not correctly added to the “Calls Answered” statistics |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Operation (Step by step sequence to reproduce a problem) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. Set up normal group 501 with 1 member (any type of phone) and overflow to UCD group 528 after 001 seconds. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. Set up UCD group 528 with 2 members (201 and 202) and set an overflow timer of 010 seconds to the AA group so that queuing message can be played to the call. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. Program a DDI number to ring at 501 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. When the call rings at group 528 it is logged as a “Call Waiting” in the UCD stats |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5. When the call is answered before the UCD group overflows to get it’s first queuing message the call is not listed as answered in the UCD stats. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6. If the call is allowed to get it’s first queuing message and is then answered when the call returns to the UCD group 528 it will then be added to the “Calls Answered” stats |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Configuration (The system environment, installation) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Customer/Site information (Customer/Site where the problem or feature addition of change request is reported) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Version of related software (to be filled in when necessary) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SP : ver 7.1 |  |  |  |  |  |  | AA : 2.04 |  |  |  |  |  |  | PRI ver 3.4 |  |  |  |  |  | BSI : |  |  |  |  |  |
| LP : |  |  |  |  |  |  | VD : |  |  |  |  |  |  | BRI : |  |  |  |  |  | PCMMC : |  |  |  |  |  |
| Programming (programming required to reproduce a problem or required for a new/modified feature) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Remarks (Related problems, history, etc.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Management results |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Duplication |  |  |  |  |  |  | Status |  |  |  |  |  |  | Waiting for more information |  |  |  |  |  |  | Requested By |  |  |  |  |
|  | Success |  |  |  | Failed |  |  | Open |  |  |  |  | Closed |  |  |  |  |  |  |  |  |  |  |  |  |