How to block “SNMP” packets in OS7070, OS7100 MP10a and OS7200 MP20s

- Connect a PC to the OfficeServ system via SIO or Telnet and then Go to Shell below. The red texts are the things or command that you have to do and the green texts are comments.

OS7100]login: (press “Enter” key) admin (Enter “admin” and then press “Enter” key) Password: (Enter “samsung” and then press “Enter” key) Login incorrect [OS7100]login: admin Password: login[690]: root login on 'tts/0' 1. bash shell 2. AFT 3. Voice Mail CLI 4. MGI CLI 5. SP CLI 6. Router/Switch CLI 7. Quit Please select a menu number :1 (select ‘1’ and then press enter) bash-3.00# bash-3.00#

- Type the following command in order to open/edit “snmpd.conf”.

bash-3.00# vi /mnt/nand/snmp/snmpd.conf (Type this command and then press Enter.)

- Remove ‘nine (9) line texts’ which is highlighted in BULE” in the following box.

#ENGINEID engineID OS7100 ####/root/ #SNMPv3 User Configuration createUser root MD5 "officeServ 7100" DES #### # First, map the community name (COMMUNITY) into a security name # (local and mynetwork, depending on where the request is coming # from): Here, place cursor in front of “r” of “recommunity” and type the command “9dd” (the next 9 lines are deleted as soon as type “9dd”). After checking the 9 line texts (blue text below) are removed, press “ESC” key. After that, type “:wq” (write and quit) in order to save above changes and then exit from editing mode. During the editing the following, if you type wrong command accidently, you please press “ESC” key and then “:q” (quit) to exit from editing mode without saving the change. After that, you can try it again from Step2 above. rocommunity public OfficeServ rwcommunity OfficeServ7--0 #COMM sec.name source community com2sec local 0.0.0.0/0 OfficeServ7--0 com2sec anynet 0.0.0.0/0 OfficeServ com2sec local localhost private com2sec anynet 0.0.0.0/0 public

- After exit from the edit mode, please perform the followings (enter “sync” command 3 times).

bash-3.00# sync (enter “sync” command and then press “Enter” key) bash-3.00# bash-3.00# sync (enter “sync” command and then press “Enter” key) bash-3.00# bash-3.00# sync (enter “sync” command and then press “Enter” key) bash-3.00# bash-3.00# reboot (enter “reboot” command and then press “Enter” key) Now system will start to reboot process.

- After rebooting completely, please check if the changes you did in above step3 are correctly saved. You can do it by the following. (this is the same command you did in step2.)

bash-3.00# vi /mnt/nand/snmp/snmpd.conf (Type this command and then then press enter)

Now, you can look at the following if you correctly modified.

#ENGINEID engineID OS7100 ####/root/ #SNMPv3 User Configuration createUser root MD5 "officeServ 7100" DES #### # First, map the community name (COMMUNITY) into a security name # (local and mynetwork, depending on where the request is coming # from):

- If the changes are correctly reflected, press “ESC” key and then “:q” to exit editing mode.