| To : ALL | Doc. No. | SECYYMM-XX-X |
| --- | --- | --- |
| C.C. : | Country | ALL |
| From: Ryan Min | Tel: Fax: E-mail: |  |
| Subject : OAS1 (V2.05) and MGI 16/64 (V1.30) S/W release |  |  |

- Overview:

- OAS1 S/W V2.05 Release (2014.01.16)

- MGI16/64 S/W V1.30 Release (2014.01.16)

Contents:

[OAS1, MGI16/64 S/W Change items]

- OAS1 S/W V2.05, MGI16/64 V1.30 Release (Each items are described by E or P or N )

  - [P] OAS rebooting when receiving ICMP Redirect Packet (only OAS)

- When OAS received ICMP Redirect Packet, OAS Card was rebooted because of a related task’s stack overflow. This problem is fixed.

  - [P] FAX sending failure from IP-UMS to specific FAX machine (MGI16/64, OAS)

- When IP-UMS sent FAX to specific FAX machine, which must receive CNG tone. There was a FAX sending failure because MGI/OAS didn’t forward CNG tone from IP side to TDM side. This problem is fixed.

  - [P] MGI/OAS rebooting during FAX T.38 negotiation (MGI16/64, OAS)

When MGI channel which is VoIP state is changed to FoIP(T.38) state and is destroyed in a short time. (e.g. when receiving 200 OK including T.38 and BYE at the same time) MGI/OAS is stuck and would be rebooted eventually. This problem is fixed.

  - [E] Improved MGI64’s booting flow for intermittent network failure (only MGI64)

Intermittently there was a network failure after MGI64’s boot-up. In this case, MGI64 will be rebooted by itself for recovering network failure..

  - [P] One way voice path after sRTP call’s hold/resume from opposite side (MGI16/64, OAS)

There was a one way voice path problem after sRTP call’s hold/resume from specific opposite SIP Phone/Trunk because of new sRTP stream’s changed sequence number. This problem is fixed by changing sRTP stream’s SSRC.

E is an acronym about Enhancement

P is an acronym about Problem

N is an acronym about New Feature