| Engineering Change Request (Key Telephone System) |  |  |  |
| --- | --- | --- | --- |
| ECR No | SECECR-1093 | Date issued | May 26, 2006 |
| Proposed | Originated by Approved by ___________ Kyungok Ko Honghan Kim |  |  |
| Model | OfficeServ12 | Unit | OS12 Startup S/W |
| Subject | # OS12 Startup S/W Version Upgrade to V1.04 |  |  |
| Summary of Change | - D060602012> System stop problem while decompressing MP ZPG file. ※ The detailed contents refer to the following page 2 |  |  |
| Reason of Change | To fix problems. |  |  |
| Details of Change | 1. System stop problem while decompressing MP ZPG file. - System can stop while decompressing some MP ZPG file compressed PGM above 8.1M because of the lack of space for decompressing |  |  |
| Impact of Change | PTT : Not affected Safety/EMC : Not affected. Classification : On a running change basis |  |  |
| DOC Affected | Circuit Diagrams : Not affected |  |  |
| Proposed Implementation Date |  | June, 2006 |  |
| TO | All |  |  |
| Approval of ALE | Please sign your signature here and return to us by FAX. Signature ALE Name ................................................ Approval Authority .......................................... Approval Number ............................................ |  |  |

|  | Startup.12 |
| --- | --- |
| CHECKSUM | −(PGM type) |
| VERSION | V1.04 |
| DATE | 060526 |

Compress MP file for using current 16M NAND

- Reason of using compressed MP file

Up to now OfficeServ12 system use 16M NAND for saving MP file, Startup file, DB, etc.

Though OfficeServ12 use 32M NAND in the near future, current system will keep in

using 16M NAND for saving program.

But in such cases it has one problem. If we need to change MP file, we have to save two

MP files into 16M NAND. But we can’t save two MP files into 16M NAND because MP file

size increase more than 8M from version 2.63

So we’ll use compressed MP file to solve this problem.

- Difference between current MP file and new compressed MP file

- Change startup program

For using compressed MP file, must change startup program of version 1.03

New startup program(above version 1.03) can load both new compressed MP file

(“zpg” file) and current MP file(“pgm” file).

- Use different file extension

New compressed MP file use “zpg” as file extension instead of  “pgm”.

- Step for using new compressed MP file

[Remark] After change of startup program, must upload new compressed MP file.

While system is running, connect to system through PCMMC

(1) Change Startup program(startup.12) of version 1.03

Erase current startup program and upload new startup program of version 1.03.

(2) Upload new compressed MP file of version 2.63

Check if the file extension is “zpg”

(3) Check if startup.ini file exist

If startup.ini and MP files above two exist in NAND, system loads MP file

designated in startup.ini. In the various case, which MP file is loaded refer to

the following data(OfficeServ System’s MP file loading process).

- Restart system and check if MP is running normally

OfficeServ System’s MP file loading process

[Remark] In the following “file name” exclude file extension.

1. If startup.ini doesn’t exist in NAND

- According to MP file name, system loads the latest version.

But if the latest version files are two, the file name is same and extension is different,

system loads “pgm” file.

2. If startup.ini exists in NAND, find the file designated in startup.ini.

(1) If the designated file exist

- System loads the designated file.

(2) If the designated file doesn’t exist

- According to MP file name, system loads the latest version.

But if the latest version files are two, the file name is same and extension is different,

system loads “pgm” file.