| To : | Doc. No. |  |
| --- | --- | --- |
| C.C. : | Country | ALL |
| From : M.W.Choi | Tel: E-mail: |  |
| Subject: Release OfficeServ 7400 LP40 S/W V2.04 |  |  |

Overview :

Release LP40 SW v2.04 – 141124

Contents :

- LP40 lockup by unhandled interrupt.

- LP40 auto restart feature.

S/W Compatibility Table :

| MODEL | LP40 | MP40 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| VERSION | V2.04 | V4.80 |  |  |  |
| DATE | 14.11.24 | 14.06.05 |  |  |  |

Version Information :

| Board | File Name | Version | Date | Description |
| --- | --- | --- | --- | --- |
| LP40 | SP40V204.PGM | V2.04 | 20141124 | SP S/W for LP40 card |

Version History :

V2.04

- PKG : 2014.11.24

- Change Notes

1. LP40 lockup by unhandled interrupt.

: There were a few unhandled interrupts and these caused a lockup of LP40 card. In that situation, a user was not able to use extensions or PRI line. We modified the LP40 S/W to ignore that unhandled interrupt.

2. LP40 auto restart feature.

: When certain software task did not work because of unsuspected reason, there was no way to recover the LP40. We have developed additional monitoring feature and the LP40 will restart automatically in case of software task lockup.

V2.03

- PKG : 2014.04.01

- Change Notes

1. The SLI3 Long Loop check feature is now able to be turned off.

: The SLI3 card has Long Loop Check and Disconnect feature for preventing overheat. But, in case of OS7200/OS7400, fans of the cabinet are enough to cool the card. So, we provide an option to turn off the feature. Now you can show the option in MMC861.

2. (Argentina) The End of ANI was changed from I-15 to I-12.

: The end of ANI of E1R2 was changed from I-15 to I-12 in Argentina.

V2.02

- PKG : 2013.01.04

- Change Notes

1. SLI3 DTMF detection problem with a phone made by OPHIR TELECOM was fixed.

: Because of SLI3’s DC characteristic, the DTMF signal with a phone(made by OPHIR TELECOM) was distorted. We modified DC characteristic to support this phone.

2. SLI/SLI2 CID problem with common CID Resource on 2nd/3rd cabinet.

: When CID common resource is only exist on 2nd/3rd cabinet and a SLT is connected to a

16SLI2 card on same cabinet, the CID Tx feature does not work. There was a fix related

SLI3 CID Tx problem in V2.01 but that resulted in this problem. We modified the CID port

calculation logic to support both SLI2 and SLI3 cards.

V2.01

- PKG : 2012.07.23

- Change Notes

1. CID feature of SLI3 cards did not work on 2nd/3rd cabinet.

: There was a calculation error of port offset. We modified the calculation logic

V2.00

- PKG : 2011.12.09

- Change Notes

1. SLI3 cards(16SLI3, 16SLI3s, 8SLI3, 8COMBO3) are now available.

: SLI3 cards(16SLI3, 16SLI3s, 8SLI3, 8COMBO3) were developed. We add service routine of SLI3 cards to the system.

2. SVMi-20i card is now available.

: SVMi-20i card was developed. We add the service routine of SVMi-20i card to the system.

3. Auto Upgrade feature of LP40 BOOT ROM.

: The program size of LP40 V2.00 exceeded the limit of current BOOT ROM. We support the BOOT ROM upgrade feature through S/W for your convenience. If you use LP40 under V2.00, LP40 will upgrade the BOOT ROM automatically. The upgrade from under V2.00 to V2.00 or upper version will take about 13 minutes because of BOOT ROM UPGRADE. For this feature, LP40 package will be released with new package name and you can select the package in MMC818. New package has the name format of “SP40xxxx.PGM”

V1.27

- PKG : 2010.08.26

- Change Notes

1. The CNF24 card is available.

: The CNF24 card was developed. We add IPC feature between MP and CNF24.

V1.26

- PKG : 2010.08.18

- Change Notes

1. 16TRK/8TRK2 Trunk Port Lock up.

: There was a port lock up problem. The diagnostic feature did not worked on

16TRK/8TRK2. We fix the TRK diagnostic feature to work on 16TRK/8TRK2.

V1.25

- PKG : 2009.12.28

- Change Notes

1. The IRM option board is now available.

: New option board(IRM) was developed. We add IRM service routine.

2. Version compatibility problem.

: A CRM option board is not working on LOC2 with LP40 V1.24 and MP40 V4.30i. When

the IRM board was developed, some messages were fixed. But those messages are used

in old version of MP. We fix LP40 to work with old version of MP.

V1.24

- PKG : 2009.06.11

- Change Notes

1. The IRM option board is now available.

: New option board(IRM) was developed. We add IRM service routine.

V1.23

- PKG : 2009.06.11

- Change Notes

1. Busy Tone detect Issue on Analog Trunk.

: System often detect busy tone on woman's voice. There is a calculation error when

checking busy tone cadence.

2. Echo issue between SLT and Wi-Fi phone.

: Because of the faulty initialization parameter for 16MWSLI card, the gain level of the SLT is a little bit high(3dB). So, Wi-Fi phone user can hear some echo during conversation.

V1.21

- PKG : 2009.04.27

- Change Notes

1. MPS feature of OAS card is available.

: MPS feature of OAS card is required. We add MPS messages.

2. Changing inter digit time didn't be affected when using dial pulse feature.

: System counts inter digit time by 10ms. But inter digit time is set by 100ms. We modified

inter digit timer to count time by 100ms.

3. Russia busy tone detect problem.

: System often detect busy tone on woman's voice. There is calculation error when busy tone

cadence check. We modify Russia busy tone calculation scheme.

V1.20

- PKG : 2008.12.08

- Change Notes

1. CID feature is available in UK.

: Using of CID in UK is required. And we add UK CID feature.

2. PRS is now working in SWEDEN and SPAIN.

: PRS feature is required in SWEDEN and SPAIN. PRS feature is modified to be worked in

SWEDEN and SPAIN.

3. OAS card is available.

: OAS card is developed. And we add OAS services.

4. DLI port changes to SLI port.

: Digital phone send phone type when digital phone boot up. But the phone type is different

often. We modify system to check the phone type 3 times.

v1.17

-. PKG

. Main PKG : 11/12/2007

-. Change Notes

1. MW feature is available on 16SLI2/8SLI2/8COMBO2.

=> In Italy, Spain and France, MW feature was not working. We add MW feature based on PRS.

2. RCM2 D'bd on LOC 1 was not working.

=> If RCM2 on LOC 1 is initialized with no delay, channels of RCM2 are not working. We add interval time to D'bd initial phase.

3. When using LP40(V1.16) and MP40(V3.34), CID feature is not working

=> Because CID parameters have been downloaded after V1.16, default law setting value according to nation is eliminated. We set A-law, u-law by nation again.

4. 16TRK/8TRK2 Initial problem.

=> When 16TRK/8TRK2 is on initial phase, if trunk gain parameters are downloaded and system reset the slot, then initialization is failed. So we modify TRK initial phase to block slot reset when LP40 download trunk gain parameters.

5. TEPRI2 did not work correctly sometimes.

=> 1st 32 ports and 2nd 32 ports of TEPRI2 can be set separately. But all messages have been sent only to 1st part. In this condition the card do not work correctly. So we modify message service routine to separate message.

6. PRS3 hangs up call on answer.

=> Base polarity value was stored before ringing state. But in PRS3 mode it is necessary the value to be stored after answer. We modify system store the base polarity value after answer.

v1.16

-. PKG

. Main PKG : 08/03/2007

-. Change Notes

1. The feature to set the cid configuration is required.

=> We add the MMC to configure the standard of cid,receive level, transmission level and the coding rule.

2. Four kind of new cards are developed.

=> We developed the new cards and add the interface routine for new cards.

3. We change the engine chip to STL9604 currently used in OS100 and some other system.

=> We change the engine interface routine for new one.

4. The mapping logical port number with physical port number is not correct.

=> We change port mapping initial routine.

5. Dial Pulse feature of TRK cards(16TRK and 8TRK2) is required.

=> We add Dial Pulse feature to 16TRK/8TRK2

6. If user cancel MW when SLT's lamp is on, the lamp is not off.

=> We add the condition to blink control routine.

7. If TEPRIa card is E1/T1 mode, or TEPRI2 card of 2nd/3rd cabinet is E1/T1 mode, system can't display TEPRI version correctly.

=> We modify routine to process version information over 0x20.

8. CID receiver did not work within 5 sec after hook-on in New Zealand.

=> We add non-PRS feature to condition.