# PMS TCP/IP Interface Protocol

User Guide

## TABLE OF CONTENTS

Part A. Introduction	3

1. What's the PMS Interfacer.	3

2. Connection of PMS interface	3

3. PROTOCOL BETWEEN K/P AND PMS SYSTEM	4

4. REAL MESSAGE CONTENT FORMAT	5

Part B. Details of PMS Interface	8

1. start procedure of pms interface	8

2. Message list	9

Part C. Detailed Message Format	13

Part A. Introduction

1. What's the PMS TCP/IP Interface.

Switch suppliers want to connect their PBX or K/P with existing PMS(Property        management system) through Internet.

Now, by using the SEC's PMS TCP/IP Interface Protocol, all information used in hotel can be transmitted and received between switch and PMS.

2. Connection of PMS TCP/IP Interface

OS100/500/7200/7400

LAN

PMS TCP/IP Interface

Socket

### PMS System

- Connect the OS System LAN port to network.

- Connect a PC which has existing PMS package to network.

- Connect from PC to OS System using terminal program(IP:OS System IP, Port : 5107)

3. Protocol between K/P and PMS system

  - Protocol Procedure

K/P System

Message (Note 1)

PMS System

(Note 1) Message Format

| Length | Real Message Contents |
| --- | --- |
| 2 Byte | N Byte |

- Length : The length of Real Message Contents Field.

- Real Message Contents: This is a real message content. This message format will be described in Part A. 4 for details.

Example)

<Real Message Contents>

0x00 0x08 0x03 0xD0 0x00 0x00 0x00 0x00 0x00 0x00

  - Description of Protocol

K/P System

- K/P System creats TCP socket then waits a connection from client(PMS System). IP is set at MMC 830 and Port number is 5107

- If K/P System receives a message, it sends a response message.

- If K/P System can’t receives “Keep Alive” message per 1 minute, it disconnects a connection.

PMS System

- PMS System can connect to K/P System using TCP socket. K/P System IP is set at MMC 830 and Port number is 5107.

- PMS System sends a message then receives a response message.

- PMS System has to send “Keep Alive” message per 1 minute, othrewise K/P System will disconnect a connection.

4. Real Message Content Format

In this Section, we will describe Real Message Contents Format except ENQ/ACK protocol header.

- 4.1 Frame Structure

PMS → K/ P

| MSG TYPE | MSGID | Reserved 1 | SEQ No. | Reserved2 | Reserved 3 | SW PORT No. | Data |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 2 Byte | M Byte |

- MSG TYPE(0x03): Fixed Value(It means HOTEL or Bi-Directional PMS Message)

- MSGID: It may be different according to the requirement of PMS system. For details, refer to the Part B.

- Reserved 1: It is not used. Just set to value ‘0x00’.

- SEQ No.: This is a Sequence number of the Message Frame. The message contents are too much to send within one message frame. The message is splited several message pieces. In this cases, the Sequence Number of the first message piece is ‘0x01’, the Sequence Number of the second message pieces is ‘0x02’ and the Sequence Number of the last message piece is ‘0x00’. Because the message length for PMS interface is not long, the Sequence Number of the message frame is ‘0x00’.

- Reserved 2: It is not used. Just set to value ‘0x00’.

- Reserved 3: It is not used. Just set to value ‘0x00’.

- SW PORT No.: This is the SW port number corresponding the TEL number which the PMS system want to change the status. The SW port number corresponding the Tel number can get from the K/P by DIAL NUMBER REQUEST Message(0xD0). This is the TEL number which the PMS system want to change the status.

- Data: This filed may be different according the MSGID.

K/P → PMS

| MSG TYPE | MSGID | Reserved 1 | SEQ No. | Reserved 2 | Reserved 3 | SW PORT No. | Data |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 Byte | 0xD0, 0xF0, 0xE0 | 1 Byte | 1 Byte | 1 Byte | 1 Byte | 2 Byte | M Byte |

- MSG TYPE(0x03): Fixed Value(It means HOTEL or Bi-Directional PMS Message)

- MSGID: It may be different according to the requirement of PMS system. For details, refer to the Part B.

- Reserved 1: It is not used. Just set to value ‘0x00’.

- SEQ No.: This is a Sequence number of the Message Frame. The message contents are too much to send within one message frame. The message is splited several message pieces. In this cases, the Sequence Number of the first message piece is ‘0x01’, the Sequence Number of the second message pieces is ‘0x02’ and the Sequence Number of the last message piece is ‘0x00’. Because the message length for PMS interface is not long, the Sequence Number of the message frame is ‘0x00’.

- Reserved 2: It is not used. Just set to value ‘0x00’.

- Reserved 3: It is not used. Just set to value ‘0x00’.

- SW PORT No.: This is the SW port number corresponding the TEL number which the PMS system want to change the status. The SW port number corresponding the Tel number can get from the K/P by DIAL NUMBER REQUEST Message(0xD0). This is the TEL number which the PMS system want to change the status.

- Data: This filed may be different according the MSGID. Except DIAL NUMBER RESPONSE Message (0xD0) and KEEP ALIVE RESPONSE message(0xE0), the Message from K/P to PMS is similar to the PMS string output of PMS service type in MMC 804.

See Follow example data.

- The data field structure of Dial Number Response message

Dial_Numer (4 Byte): ex: Extension 201 -> 0x02 0x00 0x01 0x0F

Trunk 701    -> 0x07 0x00 0x01 0x0F

Room Status (1 Byte): In case of Trunk -> 0xFF

In case of Station

0x00 (Available)

0x01 (Occupied)

0x02 (Need Clean)

0x03 (Need Maintenance)

0x04 (Holding)

0x07 (Hold & Need Clean)

0x08 (Hold & Need Maintenance)

Phone Type (1 Byte):  In case of Trunk -> 0xFF

In case of Station

0x00 (Normal Station)

0x01 (Smoking Room)

0x02 (Non Smoking Room)

0x03 (Meeting Room)

0x04 (Administrator)

0x05 (Fax Station)

Line Type (1 Byte):  0x00 (Station)

0x01 (Trunk)

0x02 (AOM)

- The data filed structure of other messages

Space (1 Byte): 0x20

Tel Number (4 Byte): ex “201 “ (0x32 0x30 0x31 0x20)

Space (1 Byte): 0x20

Date (5 Byte): ex “08/28” (0x30 0x38 0x2F 0x32 0x38)

Space (1 Byte): 0x20

Time (5 Byte): ex “10:29” (0x31 0x30 0x3A 0x32 0x39)

Space (1 Byte): 0x20

Item Code (3 Byte): ex “90 ” (0x39 0x30 0x20)

Space (1 Byte): 0x20

Description (18 Byte): ASCII Value

Space (1 Byte): 0x20

Details (8 Byte): ASCII Value

Space (1 Byte): 0x20

Cost(6 Byte): ex “000.10” (0x30 0x30 0x30 0x2E 0x31 0x30)

Example)

| 7224 | 06/11 | 17:50 | 93 | Check In | 1111 | 000.00 |
| --- | --- | --- | --- | --- | --- | --- |
| 7224 | 06/11 | 17:50 | 95 | Occupied |  | 000.00 |

Part B. Details of PMS interface

1. Start Procedure of PMS interface

Because the K/P system only understand the SW port instead of TEL number, the PMS system must get the SW port number corresponding TEL number first.

If the PMS system send the DIAL NUMBER REQUEST message (0xD0), the K/P will send the information of the S/W port number, TEL number and etc as follows.

PMS                                      K/ P

Dial Number Request Message

(0x03 0xD0 0x00 0x00 0x00 0x00 0x0000)

Dial Number Response Message (0xD0)

1st Frame (0x03 0xD0 0x00 0x00 0x00 0x00 0x0000 0x02 0x00 0x01 0x0F …)

2nd Frame (0x03 0xD0 0x00 0x00 0x00 0x00 0x0001 0x02 0x00 0x02 0x0F …)

3rd Frame (0x03 0xD0 0x00 0x00 0x00 0x00 0x0002 0x02 0x00 0x03 0x0F …)

.

.

.

M th Frame (0x03 0xD0 0x00 0x00 0x00 0x00 0x0017 0x07 0x00 0x08 0x0F …)

Last Frame (0x03 0x0D 0x00 0x00 0x00 0x00 0x0017 0xEF)

* 0xEF: It means Last Message Frame.

2. Message List

  - PMS -> K/P Message List

| - MSGID | - Message Name | - Description |
| --- | --- | --- |
| - 0xD0 | - Dial Number Request | - This Message request the SW port number corresponding TEL number and other information (ex: room status, station or trunk, room type and etc) to K/P. |
| - 0x01 | - Inquire of Room Status | - This message allows PMS to inquire of the room status. And then, PMS will get the current room status through the PMS data stream. This command message will be used to know the current room status after something was done on a certain room. |
| - 0x02 | - Check-In | - PMS request the K/P of the guest Check In function by this 'Check In' message. The status of guest room will be changed into occupied and the guest can use the room phone just after check in procedure. If PMS sends the check in command to check-in the room, which is occupied already, the K/P will return the error status to the PMS. |
| - 0x03 | - Modification of Guest Information | - K/P holds the guest information such as Guest Name, deposit value for phone credit and payment methods, which are card or cash. Among these, information of guest name can be modified by this PMS command easily |
| - 0x04 | - Inquiring of room phone using or not | - PMS can inquire of the status of room phone, which is using or not. This message is useful to know the status of room phone before the clerk check out the guest. Normally, the clerk will not check out the room if someone is on the phone of his room because the call will be released just after checking out the room. So, before you check out the room, it would be better to check the status of room phone. |
| - 0x05 | - Check-Out | - PMS can request the check out command to K/P. This make K/P release the line if the room phone is using and change the room status into 'Need Cleaning'. And the room phone which the room was checked out will be no more used until that room is checked in again. |
| - 0x07 | - Inquiring of current deposit value of Phone Credit | - OS SYSTEM has a feature of 'Phone Credit Limitation'. This feature will allow PMS to control the limitation of the call costing by setting the deposits of phone credit when the guest checks in the room. When the call cost reaches the limitation of the phone credit value OS SYSTEM will release the call and the guest can no more use the room phone. |
| - 0x08 | - Add deposit of Phone Credit | - For the credit of phone bill, if the guest phone reaches the cost limit for the phone bill, this limit will only be monitored if the room was flagged as a CASH for the phone bill during the checking procedure. When the credit expiration warning threshold is reached the user will hear beeps in their ear. This warning threshold is determined by the call costing feature and occurs one billing period before the credit limit is reached the call will be drop. To add the credit, PMS sends the "Add Phone Credit" command to OS SYSTEM. |
| - 0x09 | - Inquire Current Wake-up time | - PMS can inquire the currently set wake up time of the guest room. Before the clerk changes the wake up time he would like to know the currently set wake up time or after setting the wake up time he want to know whether the time is set properly. This message will allow the desk clerk to give the information of currently set wake up time. |
| - 0x0A | - Set/Clear Wake-up time | - This message allows PMS to set the wake up time into OS SYSTEM by this message. there are two wake up times per each room such as timer 1 and timer 2. And also PMS set the three kind of timer mode, that is, none, daily, once. |
| - 0x0B | - Change Room Status | - OS SYSTEM has the five kinds of room status which are available, occupied, need cleaning, need maintenance and hold. This message allows PMS to change the currently set room status into other status. For more detailed description, refer to Appendix. |
| - 0x0C | - Answer Available or not | - Some guest wants to answer the phone call from the guest of other room, but the others do not want it. For this reason, PMS wants to set the status of room phone during check in procedure and PMS has to send that of information to OS SYSTEM. If the flag is set to "Answer Available Room". Anyone can make a phone call to this room. But if it is set to "Answer Deniable Room" all internal calls will be blocked |
| - 0x0D | - Message Waiting | - With the guest room phone which has message waiting indicator(lamp), OS SYSTEM can show the status of left messages on MWI . Addition to this , this message allows PMS to send the information to OS SYSTEM about the recall station which is recalled after guest lift up the handset and dial the message alarm code in order to know what kind of messages are left on the front desk. |
| - 0x0E | - Request Current Time | - In order to adjust the system time between OS SYSTEM and PMS, PMS can request the current system time of OS SYSTEM by this message. To set the system time of OS SYSTEM, you can set time by manipulating OS SYSTEM's KeyMMC or PCMMC directly. |
| - 0x0F | - Check the Staff Code | - OS SYSTEM has 4 digit of staff code which is used as a kind of passcode. This staff code should be registered into OS SYSTEM by user key programming and will be compared with the code which is entered by room phone, administrator or PMS. But this is not requisite item for the PMS operation. By this message PMS can check the fact that OS SYSTEM has the staff code or not. |
| - 0x10 | - Initialisze Room Status | - PMS system can initialize the room status into available and release the phone call intentionally if it is being used. Especially when the room status of OS SYSTEM is different from that of PMS it will be necessary to initialize the room status of OS SYSTEM intentionally. But, normally you don't need to use it. This message allows PMS to make a room status into available and release the call if it is being used. |
| - 0x11 | - Request Link Connection | - To communicate information between PMS and OS SYSTEM, the request of 'Link Connection On' by PMS is sent to the OS SYSTEM before the any other messages. After OS SYSTEM receive this message it will send the reply to indicate that connection is ok. |
| - 0x12 | - Request Link Disconnection | - To finish communicating with each other(PMS, OS SYSTEM), PMS will request of "Link Connection Off". Just after OS SYSTEM receives this command from PMS, OS SYSTEM will quit communicating with PMS and PMS also quit corresponding. |
| - 0xE0 | - Keep Alive | - To maintain the connection, PMS have to send keep alive message to OS System per 1 minute. |

  - K/P -> PMS Message List

The message from K/P to PMS has three kinds of MSGID (0xD0, 0xE0, 0xF0) and each message has different message structure.

In case of MSGID ‘0xF0’, the ITEM CODE of data filed have means like as follows.

| MSGID | Item Code | Description | Usage |
| --- | --- | --- | --- |
| 0xD0 | - | - | Get SW port number and corresponding TEL number information and etc |
| 0xE0 | - | - | A response of Keep Alive message |
| 0xF0 | 00 | Not used | It will be used in Hotel Plus |
|  | 01 | Phone deposit | the code used for pre pay room deposit |
|  | 02 - 87 | Not used | It will be used in Hotel Plus |
|  | 89 | W/UP SET | A wake up call was set |
|  | 90 | W/UP ANS | A wake up call was answered |
|  | 91 | W/UP N/ANS | A wake up call was not answered |
|  | 92 | W/UP CANCL | A Wake up call cancelled |
|  | 93 | Check In | A guest has checked into a room |
|  | 94 | Check Out | A guest has checked out of a room |
|  | 95 | Occupied | A room has been flagged as OCCUPIED |
|  | 96 | Available | A room has been flagged as AVAILABLE |
|  | 97 | Clean Room | A room has been flagged as NEEDS CLEANING |
|  | 98 | Fix Room | A room has been flagged as NEEDS MAINTENANCE |
|  | 99 | Hold | A room has been flagged as HOLD |
|  | 100-101 | xxxxxxxxxxx | the code used for call costing of a room phone |
|  | 102 | xxxxxxxxxxx | the code used for call costing of a Fax Phone |
|  | 103 | ChkIn Modified | the code used for indicating Information was changed |
|  | 104 | Phone Idle | the code used for indicating that room phone is idle |
|  | 105 | Phone Busy | the code used for indicating that room phone is busy |
|  | 106 | Reserved | reserved for the future |
|  | 107 | W/UP Time | the code used for answer of requesting current wake up time |
|  | 108 | ANS Available | A room has been flagged as Answer Available |
|  | 109 | DND | A room has been flagged as Do not disturb |
|  | 110 | MW set | A room has been flagged as message wait set |
|  | 111 | MW cleared | A room has been flagged as message wait cancel |
|  | 112 | Time&Date | the code used for Answer of Current time & date |
|  | 113 | Avail Staffcode | the code used for answer of available staff code |
|  | 114 | N/Avail staffcode | the code used for answer of not availabel staff code |
|  | 115 | Room Initialized | the code used for answer of room initializing |
|  | 116 | Error Status | the code used for answer of error status |
|  | 117 | Connection OK | the code used for answer of request of connection |
|  | 118 | PH Deposit View | the code used for answer of viewing current deposit |
|  | 119 | Reserved |  |
|  | 120 | Occupied status | Currently, the room is set to occupied status |
|  | 121 | Available status | Currently, the room is set to available status |
|  | 122 | Need cleaning status | Currently, the room is set to Need cleaning status |
|  | 123 | Need fixing status | Currently, the room is set to Need fixing status |
|  | 124 | Hold status | Currently, the room is set to Hold status |

[K/P->PMS sample message]

For a convenience, following message header did not described and following message are

described for ASCII format.

Omitted Message Header:     “0x03 0xF0 0x00 0x00 0x00 0x00 0x00”

| 7224 | 06/12 | 02:25 | 00 | RM Deposit | 1111 | 000.00 |
| --- | --- | --- | --- | --- | --- | --- |
| 7224 | 06/12 | 02:25 | 01 | PH Deposit | 1111 | 000.00 |
| 7224 | 06/12 | 02:25 | 89 | W/UP SET | 1111 | 000.00 |
| 7224 | 06/12 | 02:25 | 90 | W/UP ANS |  | 000.00 |
| 7224 | 06/12 | 02:25 | 91 | W/UP N/ANS |  | 000.00 |
| 7224 | 06/12 | 02:25 | 92 | W/UP CANCL | 1111 | 000.00 |
| 7224 | 06/12 | 02:25 | 93 | Check In | 1111 | 000.00 |
| 7224 | 06/12 | 02:25 | 94 | Check Out | 1111 | 000.00 |
| 7224 | 06/12 | 02:25 | 95 | Available | 1111 | 000.00 |
| 7224 | 06/12 | 02:25 | 96 | Occupied | 1111 | 000.00 |
| 7224 | 06/12 | 02:25 | 98 | Fix Room | 1111 | 000.00 |
| 7224 | 06/12 | 02:25 | 99 | Hold | 1111 | 000.00 |
| 7224 | 06/12 | 02:25 | TEL | 12345678 | 00:00:05 | 000.00 |
| 7224 | 06/12 | 02:25 | FAX | 12356789 | 00:00:05 | 000.00 |
| 7224 | 06/12 | 02:25 | 103 | CheckIn Modified | 1111 | 000.00 |
| 7224 | 06/12 | 02:25 | 104 | Phone is Idle |  | 000.00 |
| 7224 | 06/12 | 02:25 | 105 | Phone is Busy |  | 000.00 |
| 7224 | 06/12 | 02:25 | 106 | -Reserved |  | 000.00 |
| 7224 | 06/12 | 02:25 | 107 | HH:mm(0),HH:mm(0) |  | 000.00 |
| 7224 | 06/12 | 02:25 | 108 | Answer Available |  | 000.00 |
| 7224 | 06/12 | 02:25 | 109 | Answer Block |  | 000.00 |
| 7224 | 06/12 | 02:25 | 110 | Msg Wait Set | 1111 | 000.00 |
| 7224 | 06/12 | 02:25 |  |  |  | 000.00 |
| 7224 | 06/12 | 02:25 | 111 | Msg Wait Cleared | 1111 | 000.00 |
| 7224 | 06/12 | 02:25 | 112 | Current Time |  | 000.00 |
| 7224 | 06/12 | 02:25 | 113 | Right Staff Code | 1111 | 000.00 |
| 7224 | 06/12 | 02:25 | 114 | Wrong Staff Code | 1111 | 000.00 |
| 7224 | 06/12 | 02:25 | 115 | RM was initialized | 1111 | 000.00 |
| 7224 | 06/12 | 02:25 | 116 | Error | 01 | 000.00 |
| 7224 | 06/12 | 02:25 | 117 | Connected | 1111 | 000.00 |
| 7224 | 06/12 | 02:25 | 118 | PH Deposit View |  | 000.00 |
| 7224 | 06/12 | 02:25 | 119 | -Reserved |  | 000.00 |
| 7224 | 06/12 | 02:25 | 120 | Occupied |  | 000.00 |
| 7224 | 06/12 | 02:25 | 121 | Available |  | 000.00 |
| 7224 | 06/12 | 02:25 | 122 | Need Cleaning |  | 000.00 |
| 7224 | 06/12 | 02:25 | 123 | Need Fixing |  | 000.00 |
| 7224 | 06/12 | 02:25 | 124 | Held Room |  | 000.00 |

Part C. Detailed Message Format

A.0  Dial Number Request

| PMS → K/P (Dial Number Request) |  |  |
| --- | --- | --- |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0xD0 | 1 |
| Reserved 1 SEQ No. Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No | 0x0000 | 2 |
| K/P → PMS (Dial Number Response) |  |  |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0xD0 | 1 |
| Reserved 1 SEQ No Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No. | 0x???? (Variable) | 2 |
| Dial Number | Ex) “201 “ (0x02 0x00 0x01 0x0F) | 4 |
| Room status | Trunk: 0xFF Station: 0x00 (Available) 0x01 (Occupied) 0x02 (Need Clean) 0x03 (Need Maintenance) 0x04 (Holding) 0x07 (Hold & Need Clean) 0x08 (Hold & Need Maintenance) | 1 |
| Phone Type | Trunk: 0xFF Station: 0x00 (Normal Station) 0x01 (Smoking Room) 0x02 (Non Smoking Room) 0x03 (Meeting Room) 0x04 (Administrator) 0x05 (Fax Station) | 1 |
| Line Type | 0x00 (Station) 0x01 (Trunk) 0x02 (AOM) | 1 |
| Reserved | - | 1 |

A.1  Inquire of a room status

| PMS → K/P (Inquire) |  |  |
| --- | --- | --- |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0x01 | 1 |
| Reserved 1 SEQ No. Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No | 0x???? (Variable) | 2 |
| K/P → PMS (Answer the room status) |  |  |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0xF0 | 1 |
| Reserved 1 SEQ No Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No. | 0x0000 | 2 |
| D1- space | 0x20 | 5 |
| D2- Tel No. | ASCII (“3357” → 0x33 0x33 0x35 0x37) | 4 |
| D3- space | 0x20 | 4 |
| D4- date | ASCII (“08/28” → 30 38 2f 32 38) | 5 |
| D5- space | 0x20 | 4 |
| D6- Time | ASCII (ex, “10:29”) | 5 |
| D7- space | 0x20 | 4 |
| D8- Item Code | (“120...124” is ASCII Value) 120: Current Room status is Occupied 121: Current Room status is Available 122: Current Room status is Need Cleaning 123: Current Room status is Need Fixing 124: Current Room status is Held Room | 3 |
| D9- space | 0x20 | 5 |
| D10-description | Blank(0x20) | 18 |
| D11-space | 0x20 | 3 |
| D12-details | Blank(0x20) | 8 |
| D13-space | 0x20 | 2 |
| D14-Cost | ASCII(“000.10”-0x30 0x30 0x30 0x2E 0x31 0x30) | 6 |

A.2 Check In

| PMS → K/P (Request) |  |  |
| --- | --- | --- |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0x02 | 1 |
| Reserved 1 SEQ No. Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No | 0x???? (Variable) | 2 |
| D1-Phone deposit | 4 byte | 4 |
| D2-Pay method | 00-cash, 01-card | 1 |
| D3-Name | ASCII(11 characters) | 11 |
| D4-staff code | 4 nibbles | 2 |
| K/P → PMS (Answer 1-Checked In) |  |  |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0xF0 | 1 |
| Reserved 1 SEQ No Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No. | 0x0000 | 2 |
| D1- space | 0x20 | 5 |
| D2- Tel No. | ASCII(“3357” → 33 33 35 37) | 4 |
| D3- space | 0x20 | 4 |
| D4- date | ASCII(“08/28” → 30382f3238) | 5 |
| D5- space | 0x20 | 4 |
| D6- Time | ASCII(ex, “10:29”) | 5 |
| D7- space | 0x20 | 4 |
| D8- Item Code | Ascii “93 “ : checked in | 3 |
| D9- space | 0x20 | 5 |
| D10-description | ASCII | 18 |
| D11-space | 0x20 | 3 |
| D12-details | staff code: (ex) ”1111 ”(0x31 0x 31 0x31 0x 31 0x20 0x20 0x20 0x20) (same as input staff code) | 8 |
| D13-space | 0x20 | 2 |
| D14-Cost | ASCII(“000.10”-0x30 0x30 0x30 0x2E 0x31 0x30) | 6 |

A.3 Modify Check In Information

| PMS → K/P (Request) |  |  |
| --- | --- | --- |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0x03 | 1 |
| Reserved 1 SEQ No. Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No | 0x???? (Variable) | 2 |
| D1-Name | ASCII(11 characters) | 11 |
| D2-staff code | 4 nibbles | 2 |
| K/P → PMS (Answer -Check In Modified) |  |  |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0xF0 | 1 |
| Reserved 1 SEQ No Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No. | 0x0000 | 2 |
| D1- space | 0x20 | 5 |
| D2- Tel No. | ASCII(“3357” → 33 33 35 37) | 4 |
| D3- space | 0x20 | 4 |
| D4- date | ASCII(“08/28” → 30382f3238) | 5 |
| D5- space | 0x20 | 4 |
| D6- Time | ASCII(ex, “10:29”) | 5 |
| D7- space | 0x20 | 4 |
| D8- Item | ASCII “103” : checked in Modified | 3 |
| D9- space | 0x20 | 5 |
| D10-description | ASCII blank(0x20) | 18 |
| D11-space | 0x20 | 3 |
| D12-details | staff code: (ex) ”1111 “(0x31 0x31 0x31 0x31 0x20 0x20 0x20 0x20) | 8 |
| D13-space | 0x20 | 2 |
| D14-Cost | ASCII(“000.10”-0x30 0x30 0x30 0x2E 0x31 0x30) | 6 |

A.4 Inquire of Room phone  Using or Not

| PMS → K/P (Request) |  |  |
| --- | --- | --- |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0x04 | 1 |
| Reserved 1 SEQ No. Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No | 0x???? (Variable) | 2 |
| K/P → PMS (Answer -Return Room Phone status) |  |  |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0xF0 | 1 |
| Reserved 1 SEQ No Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No. | 0x0000 | 2 |
| D1- space | 0x20 | 5 |
| D2- Tel No. | ASCII(“3357” → 0x33 0x33 0x35 0x37) | 4 |
| D3- space | 0x20 | 4 |
| D4- date | ASCII(“08/28” → 0x30 0x38 0x2f 0x32 0x38) | 5 |
| D5- space | 0x20 | 4 |
| D6- Time | ASCII(ex, “10:29”) | 5 |
| D7- space | 0x20 | 4 |
| D8- Item | ASCII “104” : Room Phone is Idle ASCII “105” : Room Phone is Busy | 3 |
| D9- space | 0x20 | 5 |
| D10-description | ASCII blank(0x20) | 18 |
| D11-space | 0x20 | 3 |
| D12-details | ASCII Blank(0x20) | 8 |
| D13-space | 0x20 | 2 |
| D14-Cost | ASCII(“000.10”-0x30 0x30 0x30 0x2E 0x31 0x30) | 6 |

A.5 Check Out Request

| PMS → K/P (Request-Check Out) |  |  |
| --- | --- | --- |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0x05 | 1 |
| Reserved 1 SEQ No. Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No | 0x???? (Variable) | 2 |
| Data | 2 nibbles(staff code) | 2 |
| K/P → PMS (1st Answer -Return Checked out) (2nd Answer-Return Need Cleaning) |  |  |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0xF0 | 1 |
| Reserved 1 SEQ No Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No. | 0x0000 | 2 |
| D1- space | 0x20 | 5 |
| D2- Tel No. | ASCII(“3357” → 0x33 0x33 0x35 0x37) | 4 |
| D3- space | 0x20 | 4 |
| D4- date | ASCII(“08/28” → 0x30 0x38 0x2f 0x32 0x38) | 5 |
| D5- space | 0x20 | 4 |
| D6- Time | ASCII(ex, “10:29”) | 5 |
| D7- space | 0x20 | 4 |
| D8- Item | (1st) ASCII “94 “ : Room was checked Out (2nd) ASCII “97 “ : Room is need cleaning | 3 |
| D9- space | 0x20 | 5 |
| D10-description | ASCII | 18 |
| D11-space | 0x20 | 3 |
| D12-details | (1st)staff code: (ex) ”1111 “(0x31 0x31 0x31 0x31 0x20 0x20 0x20 0x20) (same as request staff code) (2nd) blank display | 8 |
| D13-space | 0x20 | 2 |
| D14-Cost | ASCII(“000.10”-0x30 0x30 0x30 0x2E 0x31 0x30) | 6 |
| A.6 Inquire Current Phone Credit |  |  |
| PMS → K/P (Inquire Current Credit) |  |  |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0x07 | 1 |
| Reserved 1 SEQ No. Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No | 0x???? (Variable) | 2 |
| K/P → PMS (Answer -Return Current Credit) |  |  |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0xF0 | 1 |
| Reserved 1 SEQ No Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No. | 0x0000 | 2 |
| D1- space | 0x20 | 5 |
| D2- Tel No. | ASCII(“3357” → 0x33 0x33 0x35 0x37) | 4 |
| D3- space | 0x20 | 4 |
| D4- date | ASCII(“08/28” → 0x30 0x38 0x2f 0x32 0x38) | 5 |
| D5- space | 0x20 | 4 |
| D6- Time | ASCII(ex, “10:29”) | 5 |
| D7- space | 0x20 | 4 |
| D8- Item | ASCII “118” : Current Phone Credit View | 3 |
| D9- space | 0x20 | 5 |
| D10-description | ASCII | 18 |
| D11-space | 0x20 | 3 |
| D12-details | ASCII Blank(0x20) | 8 |
| D13-space | 0x20 | 2 |
| D14-Cost | Credit Value(“111.11”-0x31 0x31 0x31 0x2E 0x31 0x31) | 6 |
| A.7 Add Current Deposit |  |  |
| PMS → K/P (Add Credit) |  |  |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0x08 | 1 |
| Reserved 1 SEQ No. Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No | 0x???? (Variable) | 2 |
| Data 1 | Added Value | 4 |
| Data 2 | 4 nibbles(staff code) | 2 |
| K/P → PMS (Answer -Return Current Credit) |  |  |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0xF0 | 1 |
| Reserved 1 SEQ No Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No. | 0x0000 | 2 |
| D1- space | 0x20 | 5 |
| D2- Tel No. | ASCII(“3357” → 0x33 0x33 0x35 0x37) | 4 |
| D3- space | 0x20 | 4 |
| D4- date | ASCII(“08/28” → 0x30 0x38 0x2f 0x32 0x38) | 5 |
| D5- space | 0x20 | 4 |
| D6- Time | ASCII(ex, “10:29”) | 5 |
| D7- space | 0x20 | 4 |
| D8- Item | ASCII “01 ” : Phone Credit | 3 |
| D9- space | 0x20 | 5 |
| D10-description | ASCII | 18 |
| D11-space | 0x20 | 3 |
| D12-details | staff code: (ex) ”1111 “(0x31 0x31 0x31 0x31 0x20 0x20 0x20 0x20) - same as request staff code | 8 |
| D13-space | 0x20 | 2 |
| D14-Cost | Added Value(ex. “000.10”-0x30 0x30 0x30 0x2E 0x30 0x30) | 6 |
| A.8 Inquire Current Wake Up time |  |  |
| PMS → K/P (Inquire Current WakeUp Time) |  |  |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0x09 | 1 |
| Reserved 1 SEQ No. Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No | 0x???? (Variable) | 2 |
| K/P → PMS (Answer -Return Current wakeup Time) |  |  |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0xF0 | 1 |
| Reserved 1 SEQ No Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No. | 0x0000 | 2 |
| D1- space | 0x20 | 5 |
| D2- Tel No. | ASCII(“3357” → 0x33 0x33 0x35 0x37) | 4 |
| D3- space | 0x20 | 4 |
| D4- date | ASCII(“08/28” → 0x30 0x38 0x2f 0x32 0x38) | 5 |
| D5- space | 0x20 | 4 |
| D6- Time | ASCII(ex, “10:29”) | 5 |
| D7- space | 0x20 | 4 |
| D8- Item | ASCII “107” : Current WakeUp Time | 3 |
| D9- space | 0x20 | 5 |
| D10-description | ASCII VALUE HH:mm(0),HH:mm(0) 10:30(1),11:20(1) mode : 0 → None 1 → Today 2 → Daily | 18 |
| D11-space | 0x20 | 3 |
| D12-details | blank(0x20) | 8 |
| D13-space | 0x20 | 2 |
| D14-Cost | ASCII (“000.00”-0x30 0x30 0x30 0x2E 0x30 0x30) | 6 |

A.9 Set/Clear Wake Up Time

| PMS → K/P (Set/Clear Wake Up Time) |  |  |
| --- | --- | --- |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0x0A | 1 |
| Reserved 1 SEQ No. Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No | 0x???? (Variable) | 2 |
| Data1-HH | 2 nibbles(13 hour → 0x13) | 1 |
| Data2-mm | 2 nibbles(12 min → 0x12) | 1 |
| Data3-mode | 1 byte (0x00:none 0x01:today, 0x02:daily) | 1 |
| Data4-reserved | reserved(0x00) | 1 |
| Data5-HH | 2 nibbles | 1 |
| Data6-mm | 2 nibbles | 1 |
| Data7-mode | 1 byte(0:none 1:today, 2:daily) | 1 |
| Data8-reserved | reserved(0x00) | 1 |
| Data9-staff code | Input staff code(4 nibbles) | 2 |
| K/P → PMS (Answer -Return Current wakeup Time) |  |  |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0xF0 | 1 |
| Reserved 1 SEQ No Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No. | 0x0000 | 2 |
| D1- space | 0x20 | 5 |
| D2- Tel No. | ASCII(“3357” → 0x33 0x33 0x35 0x37) | 4 |
| D3- space | 0x20 | 4 |
| D4- date | ASCII(“08/28” → 0x30 0x38 0x2f 0x32 0x38) | 5 |
| D5- space | 0x20 | 4 |
| D6- Time | ASCII(ex, “10:29”) | 5 |
| D7- space | 0x20 | 4 |
| D8- Item | ASCII “89 “ : wakeup set or ASCII “92 “ : wakeup cancel | 3 |
| D9- space | 0x20 | 5 |
| D10-description | ASCII VALUE | 18 |
| D11-space | 0x20 | 3 |
| D12-details | staff code (same as input staff code) | 8 |
| D13-space | 0x20 | 2 |
| D14-Cost | ASCII (“000.00”-0x30 0x30 0x30 0x2E 0x30 0x30) | 6 |
| A.10 Change Room Status |  |  |
| PMS → K/P (Change Room Status) |  |  |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0x0B | 1 |
| Reserved 1 SEQ No. Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No | 0x???? (Variable) | 2 |
| Data1 - room status | 0x01: make room Occupied status 0x02: setting as a Need cleaning status 0x03: setting as a Need Maintenance 0x04: make the room hold status 0x05: resetting Need cleaning status 0x06: resetting Need Maintenance status | 1 |
| Data 2-staff code | Input staff code : 4 nibble | 2 |
| K/P → PMS (Answer -Return Current Room status) |  |  |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0xF0 | 1 |
| Reserved 1 SEQ No Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No. | 0x0000 | 2 |
| D1- space | 0x20 | 5 |
| D2- Tel No. | ASCII(“3357” → 0x33 0x33 0x35 0x37) | 4 |
| D3- space | 0x20 | 4 |
| D4- date | ASCII(“08/28” → 0x30 0x38 0x2f 0x32 0x38) | 5 |
| D5- space | 0x20 | 4 |
| D6- Time | ASCII(ex, “10:29”) | 5 |
| D7- space | 0x20 | 4 |
| D8- Item | ASCII “95 “: Occupied ASCII “96 “: Available ASCII “97 “: Clean Room ASCII “98 “: Fix Room ASCII “99 “: Hold | 3 |
| D9- space | 0x20 | 5 |
| D10-description | ASCII VALUE | 18 |
| D11-space | 0x20 | 3 |
| D12-details | staff code: (ex) ”1111 “(0x31 0x31 0x31 0x31 0x20 0x20 0x20 0x20) (staff code same as input) | 8 |
| D13-space | 0x20 | 2 |
| D14-Cost | ASCII (“000.00”-0x30 0x30 0x30 0x2E 0x30 0x30) | 6 |
| A.11 Call Restriction (Room to Room answer) |  |  |
| PMS → K/P (Room To Room Status) |  |  |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0x0C | 1 |
| Reserved 1 SEQ No. Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No | 0x???? (Variable) | 2 |
| Data - mode | 0x00 : Answer available 0x01 : Answer Block | 1 |
| K/P → PMS (Answer -Return DND status) |  |  |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0xF0 | 1 |
| Reserved 1 SEQ No Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No. | 0x0000 | 2 |
| D1- space | 0x20 | 5 |
| D2- Tel No. | ASCII(“3357” → 0x33 0x33 0x35 0x37) | 4 |
| D3- space | 0x20 | 4 |
| D4- date | ASCII(“08/28” → 0x30 0x38 0x2f 0x32 0x38) | 5 |
| D5- space | 0x20 | 4 |
| D6- Time | ASCII(ex, “10:29”) | 5 |
| D7- space | 0x20 | 4 |
| D8- Item | ASCII “108” : Answer available ASCII “109” : Answer Block | 3 |
| D9- space | 0x20 | 5 |
| D10-description | ASCII blank(0x20) | 18 |
| D11-space | 0x20 | 3 |
| D12-details | ASCII Blank(0x20) | 8 |
| D13-space | 0x20 | 2 |
| D14-Cost | ASCII(“000.00”--0x30 0x30 0x30 0x2E 0x30 0x30) | 6 |

A.12 Message Waiting

| PMS → K/P (Set/Cancel Message Waiting) |  |  |
| --- | --- | --- |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0x0D | 1 |
| Reserved 1 SEQ No. Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No | 0x???? (Variable) | 2 |
| Data1 - Type | 0x00 : Set Message Waiting 0x01 : Cancel Message Waiting | 1 |
| Data2 - reserved | 0xff | 1 |
| Data3- Recall stn | 4 digits (3351: 0x33 0x51) if none, return error status | 2 |
| Data4- staff code | 4 nibbles staff code | 2 |
| K/P → PMS (Answer -Return Message Wait status) |  |  |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0xF0 | 1 |
| Reserved 1 SEQ No Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No. | 0x0000 | 2 |
| D1- space | 0x20 | 5 |
| D2- Tel No. | ASCII(“3357” → 0x33 0x33 0x35 0x37) | 4 |
| D3- space | 0x20 | 4 |
| D4- date | ASCII(“08/28” → 0x30 0x38 0x2f 0x32 0x38) | 5 |
| D5- space | 0x20 | 4 |
| D6- Time | ASCII(ex, “10:29”) | 5 |
| D7- space | 0x20 | 4 |
| D8- Item | ASCII “110” : Message Wait Set ASCII “111” : Message Wait Cleared | 3 |
| D9- space | 0x20 | 5 |
| D10-description | ASCII blank(0x20) | 18 |
| D11-space | 0x20 | 3 |
| D12-details | staff code: (ex) ”1111 “(0x31 0x31 0x31 0x31 0x20 0x20 0x20 0x20) (same as input staff code) | 8 |
| D13-space | 0x20 | 2 |
| D14-Cost | ASCII (“000.00”-0x30 0x30 0x30 0x2E 0x30 0x30) | 6 |
| A.13 Inquire Current Time |  |  |
| PMS → K/P (Inquire Current Time of k/p) |  |  |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0x0E | 1 |
| Reserved 1 SEQ No. Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No | 0x???? (Variable) | 2 |
| K/P → PMS (Answer -Return current Time) |  |  |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0xF0 | 1 |
| Reserved 1 SEQ No Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No. | 0x0000 | 2 |
| D1- space | 0x20 | 5 |
| D2- Tel No. | ASCII(“3357” → 0x33 0x33 0x35 0x37) | 4 |
| D3- space | 0x20 | 4 |
| D4- date | ASCII(“08/28” → 0x30 0x38 0x2f 0x32 0x38) | 5 |
| D5- space | 0x20 | 4 |
| D6- Time | ASCII(ex, “10:29”) | 5 |
| D7- space | 0x20 | 4 |
| D8- Item | ASCII “112” : Current Time & Date (D4 & D6 indicate current Time&Date) | 3 |
| D9- space | 0x20 | 5 |
| D10-description | ASCII Blank(0x20) | 18 |
| D11-space | 0x20 | 3 |
| D12-details | ASCII Blank(0x20) | 8 |
| D13-space | 0x20 | 2 |
| D14-Cost | ASCII (“000.00”-0x30 0x30 0x30 0x2E 0x30 0x30) | 6 |

A.14  Checking Staff Code

| PMS → K/P (Checking the Staffcode) |  |  |
| --- | --- | --- |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0x0F | 1 |
| Reserved 1 SEQ No. Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No | 0x???? (Variable) | 2 |
| Data | 4 nibbles(staff code) (1111 : 0x11 0x11) | 2 |
| K/P → PMS (Answer -Return result of checking the staff Code ) |  |  |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0xF0 | 1 |
| Reserved 1 SEQ No Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No. | 0x0000 | 2 |
| D1- space | 0x20 | 5 |
| D2- Tel No. | ASCII(“3357” → 0x33 0x33 0x35 0x37) | 4 |
| D3- space | 0x20 | 4 |
| D4- date | ASCII(“08/28” → 0x30 0x38 0x2f 0x32 0x38) | 5 |
| D5- space | 0x20 | 4 |
| D6- Time | ASCII(ex, “10:29”) | 5 |
| D7- space | 0x20 | 4 |
| D8- Item | ASCII “113” : Right Staff code ASCII “114” : Wrong Staff Code (referred to staff code in details item) | 3 |
| D9- space | 0x20 | 5 |
| D10-description | ASCII blank(0x20) | 18 |
| D11-space | 0x20 | 3 |
| D12-details | staff code: (ex) ”1111 “(0x31 0x31 0x31 0x31 0x20 0x20 0x20 0x20) (same as input staff code) | 8 |
| D13-space | 0x20 | 2 |
| D14-Cost | ASCII (“000.00”-0x30 0x30 0x30 0x2E 0x30 0x30) | 6 |

A.15  PMS data of call costing

| K/P → PMS (Result of Call Cost) |  |  |
| --- | --- | --- |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0xF0 | 1 |
| Reserved 1 SEQ No Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No. | 0x0000 | 2 |
| D1- space | 0x20 | 5 |
| D2- Tel No. | ASCII(“3357” → 0x33 0x33 0x35 0x37) | 4 |
| D3- space | 0x20 | 4 |
| D4- date | ASCII(“08/28” → 0x30 0x38 0x2f 0x32 0x38) | 5 |
| D5- space | 0x20 | 4 |
| D6- Time | ASCII(ex, “10:29”) | 5 |
| D7- space | 0x20 | 4 |
| D8- Item | ASCII “TEL” : For Room Phone or ASCII “FAX” : For Fax Phone | 3 |
| D9- space | 0x20 | 5 |
| D10-description | ASCII blank(0x20) | 18 |
| D11-space | 0x20 | 3 |
| D12-details | Call duration (“00:00:05”) | 8 |
| D13-space | 0x20 | 2 |
| D14-Cost | Value(“000.00”-0x30 0x30 0x30 0x2E 0x30 0x30) | 6 |

A.16 Room Reset (0x10)

| PMS → K/P (Request Room Resetting) |  |  |
| --- | --- | --- |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0x10 | 1 |
| Reserved 1 SEQ No. Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No | 0x???? (Variable) | 2 |
| Data | 4 nibbles(staff code) | 2 |
| K/P → PMS (Answer the result of Resetting) |  |  |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0xF0 | 1 |
| Reserved 1 SEQ No Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No. | 0x0000 | 2 |
| D1- space | 0x20 | 5 |
| D2- Tel No. | ASCII(“3357” → 0x33 0x33 0x35 0x37) | 4 |
| D3- space | 0x20 | 4 |
| D4- date | ASCII(“08/28” → 0x30 0x38 0x2f 0x32 0x38) | 5 |
| D5- space | 0x20 | 4 |
| D6- Time | ASCII(ex, “10:29”) | 5 |
| D7- space | 0x20 | 4 |
| D8- Item | ASCII “115” : Room was initialized( It means that all status of the room was cleared and initialized. And the result of the room status after this item '”15'”will be sent to PMS.) | 3 |
| D9- space | 0x20 | 5 |
| D10-description | ASCII blank (0x20) | 18 |
| D11-space | 0x20 | 3 |
| D12-details | Staff code: (ex) ”1111 “(0x31 0x31 0x31 0x31 0x31 0x20 0x20 0x20 0x20) (same as input staff code) | 8 |
| D13-space | 0x20 | 2 |
| D14-Cost | ASCII (“000.00”-0x30 0x30 0x30 0x2E 0x30 0x30) | 6 |

A.17 PMS data for error status

| K/P → PMS (Error status Indication) |  |  |
| --- | --- | --- |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0xF0 | 1 |
| Reserved 1 SEQ No Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No. | 0x0000 | 2 |
| D1- space | 0x20 | 5 |
| D2- Tel No. | ASCII(“3357” → 0x33 0x33 0x35 0x37) | 4 |
| D3- space | 0x20 | 4 |
| D4- date | ASCII(“08/28” → 0x30 0x38 0x2f 0x32 0x38) | 5 |
| D5- space | 0x20 | 4 |
| D6- Time | ASCII(ex, “10:29”) | 5 |
| D7- space | 0x20 | 4 |
| D8- Item | ASCII “116” : ERROR STATUS | 3 |
| D9- space | 0x20 | 5 |
| D10-description | ASCII blank(0x20) | 18 |
| D11-space | 0x20 | 3 |
| D12-details | ASCII (ex: ASCII “10“message i.d for this error, such as 0x10 for the room reset message) | 8 |
| D13-space | 0x20 | 2 |
| D14-Cost | ASCII (“000.00”-0x30 0x30 0x30 0x2E 0x30 0x30) | 6 |

A.18 PMS data for result of Alarm Call

| K/P → PMS (Result of Alarm Call) |  |  |
| --- | --- | --- |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0xF0 | 1 |
| Reserved 1 SEQ No Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No. | 0x0000 | 2 |
| D1- space | 0x20 | 5 |
| D2- Tel No. | ASCII(“3357” → 0x33 0x33 0x35 0x37) | 4 |
| D3- space | 0x20 | 4 |
| D4- date | ASCII(“08/28” → 0x30 0x38 0x2f 0x32 0x38) | 5 |
| D5- space | 0x20 | 4 |
| D6- Time | ASCII(ex, “10:29”) | 5 |
| D7- space | 0x20 | 4 |
| D8- Item | ASCII “90 “ : Wake Up Answer ASCII “91 “ : Wake Up No Answer | 3 |
| D9- space | 0x20 | 5 |
| D10-description | ASCII VALUE | 18 |
| D11-space | 0x20 | 3 |
| D12-details | ASCII BLANK(0x20) | 8 |
| D13-space | 0x20 | 2 |
| D14-Cost | ASCII (“000.00”-0x30 0x30 0x30 0x2E 0x30 0x30) | 6 |

A.19 Request PMS Connection On

| PMS → K/P (Request PMS Connection On) |  |  |
| --- | --- | --- |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0x11 | 1 |
| Reserved 1 SEQ No. Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No | 0x???? (Variable) | 2 |
| Data1-staff code | 4 nibbles | 2 |
| K/P → PMS (Answer the result of Connection) |  |  |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0xF0 | 1 |
| Reserved 1 SEQ No Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No. | 0x0000 | 2 |
| D1- space | 0x20 | 5 |
| D2- Tel No. | ASCII(ex: “3357” – 0x33 0x33 0x35 0x37) | 4 |
| D3- space | 0x20 | 4 |
| D4- date | ASCII(ex: “08/28” – 0x30 0x38 0x2f 0x32 0x38) | 5 |
| D5- space | 0x20 | 4 |
| D6- Time | ASCII(ex: “10:29”) | 5 |
| D7- space | 0x20 | 4 |
| D8- Item | ASCII “117” : Connection Ok | 3 |
| D9- space | 0x20 | 5 |
| D10-description | ASCII VALUE | 18 |
| D11-space | 0x20 | 3 |
| D12-details | staff code: (ex) ”1111 “(0x31 0x31 0x31 0x31 0x20 0x20 0x20 0x20) (same as input staff code) | 8 |
| D13-space | 0x20 | 2 |
| D14-Cost | ASCII (“000.00”-0x30 0x30 0x30 0x2E 0x30 0x30) | 6 |

A.20 Request PMS Connection Off

| PMS → K/P (Request PMS Connection Off) |  |  |
| --- | --- | --- |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0x12 | 1 |
| Reserved 1 SEQ No. Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No | 0x???? (Variable) | 2 |

A.21 Keep Alive message per 1 minute

| PMS → K/P (Dial Number Request) |  |  |
| --- | --- | --- |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0xE0 | 1 |
| Reserved 1 SEQ No. Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No | 0x0000 | 2 |
| K/P → PMS (Dial Number Response) |  |  |
| Item | Value | Size(Byte) |
| MSG TYPE | 0x03 | 1 |
| MSGID | 0xE0 | 1 |
| Reserved 1 SEQ No Reserved 2 Reserved 3 | 0x00000000 | 4 |
| SW PORT No. | 0x0000 (Variable) | 2 |