| To : | Doc. No. | SECYYMM-XX-x |
| --- | --- | --- |
| C.C. : | Country | ALL |
| ## From : | E-mail: |  |
| Subject : SVMi-20i Software Release |  |  |

Overview :

SVMi-20i software V6.08 is now available.

- Sandisk 16GB SSD(U100) will be used instead of current 8GB SSD. Kernel is modified to improve compatibility.

- [SEUK] Average call per week in 8.11 Status Screen is always 0. It’s fixed.

Details :

- Sandisk 16GB SSD(U100) will be used instead of current 8GB SSD.

Kernel is modified to improve compatibility.

  - Qnet ID: None

  - Description

Sandisk 16GB SSD(U100) will be used instead of 8GB SSD.

SVMi-20i rarely fails to initialize new SSD device.

  - Cause

Current kernel code is possible to be abnormal when it retries to initialize SSD.

(It retries to init while DMA enabled and the ATA PHY link up/down timing is tight.)

  - Solution

Recent kernel patch applied.

(It retries to init SSD after DMA disabled and ATA PHY link up/down timing is changed.)

  - Consideration

None.

- [SEUK] Average call per week in 8.11 Status Screen is always 0. It’s fixed.

  - Qnet ID: None

  - Description

Average calls per week in DM 8.11 Status Screen is always 0.

  - Cause

The start time for this statistics is fixed to 2006. So total calls divided with a big number.

  - Solution

Start time of reporting will be assigned with current time when SVMi-20i starts if the configuration(block table) is default.

S/W Compatibility Table :

|  | S/W | Previous Version |  | New Version |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Version | Date | Version | Date |  |
|  |  | V6.07 | ‘15.06.24 | V6.08 | ‘16.01.18 |  |