SIO Parameters: 19200, 8, N, 1

The password is "plussamsung"

If you log in the CLI then you will see below screen.

*************************************************************************** TEPRI SIO command help Oct 17 2008 IP-KP Development Group --------------------------------------------------------------------------- LMEM ... List memory (LMEM Address) WMEM ... Revise memory Word (WMEM Address Word) OMEM ... Revise memory Byte (OMEM Address Byte) BMEM ... Revise memory Block (BMEM StartAddr EndAddr Word) STPO ... Set Trace Port (STPO +/-Port Number) MSGT ... Message Trace (MSGT ON/OFF) MOTF ... Modify Trace Flags (MOTF +/-Option) (T1,E1 : I(Ipc), S(State), A(Abcd), T(Traffic)) (ISDN : I(Ipc), L(aLarm), M(Monitor), D(D-ch)), F(Full audit) MACW ... MAC address Write (MACW Mac Address) IPWR ... IP address WRite (IPWR IP Address) GWWR ... GateWay address WRite (GWWR Gateway Address) SMWR ... Subnet Mask write (SMWR Subnet Mask Address) SYSI ... SYStem Information GAIN ... control GAIN of each CH (GAIN 0/1/2/3) (0 = 0dB, 1 = +1.94dB, 2 = - 6dB, 3 = -2.5dB) ALRM ... Alarm message to MP (ALRM 0/1/2/3/4) (0=SYNC, 1=SYNC_Recovery, 2=LOS, 3=LOS_Recovery, 4=Bit Error) FRSB ... FRame Sending Block (FRSB 0/1 (0 = Send, 1 = Block)) HELP ... DCSPLUS TEPRI help command QUIT ... Quit ***************************************************************************

TEPRI> motf +M

TEPRI> msgt on

After setting IPWR, GWWR, SMWR please check SYSI and restart the TEPTIa/2 card

In case of LAN port command and password is same.

Telnet port: 5082