Here are ECR regarding SMT-R2000 new software version, V2.10.00 (2008.10.13)

The following are changes in this version of software.

1) To reduce Flash memory failure,

- The file system type change from JFSS2 to Squashfs.

- Flash writing method change from buffered writing to word writing

2) Dualization of Config. Area

3) Removal of periodical WLAN Chipset reset when the 802.11a is disabled.

4) Adding consol commands for weak signal detection.

Regarding more details, please refer to the attached change notes.

Note) Due to the above changes, you have to notice the followings before the software upgrade

|  | SMT-R200 software versions |  | What happens / What to do |
| --- | --- | --- | --- |
|  | FROM | TO |  |
| 1 | V2.08 or V2.09 | V2.10 | It's okay |
| 2 | V1.00 to V2.07 | V2.10 | The existing DB will be defaulted. To avoid it, you have to follow the below procedure: V1.00 ~V2.07 -> upgrader file (Upgrader080119.upgrade.tar) -> V2.08 or V2.09 -> 2.10 1. Upgrade SMT-R2000 software using "Upgrader080119.upgrade.tar" file first via WEB upgrade method 2. Upgrade SMT-R200o software to V2.08 or 2.09 again. 3. Finally upgrade SMT-R2000 software to V2.10. |
| 3 | V1.00 to V2.07 | V2.10 | The existing DB will be defaulted |

Attachment

|  | Attached file names | Comments |
| --- | --- | --- |
| 1 | v20100_081013.upgrade.tar | SMT-R2000 V2.10.00 (2008.10.13) Package for Web upgrade |
| 2 | SMT-R2000 Release Notes(ENG)_081014.doc | Release notes |
| 3 | [SECECR-1386] SMT-R2000-V2.10.00.doc | Relevant ECR |
| 4 | Upgrader080119.upgrade.tar | upgrader file from old versions(V1.00~V2.07) to V2.08 or 2.09, which we released before. |
| 5 | rootfs.squashfs | SMT-R2000 V2.10.00 (2008.10.13) Package for TFTP upgrade |
| 6 | zImage |  |