| To : | Doc. No. | SECYYMM-XX-X |
| --- | --- | --- |
| C.C. : | Country | ALL |
| ## From: | ## Tel: 82-31-279- Fax: ## E-mail: @samsung.com |  |
| Subject : SMT-i310X S/W Release |  |  |

Overview :

SMT-i310x S/W : V01.64 (2013.03.07)

## New Version Changes

’V01.64(2013.03.07) :  7 March, 2013

* New feature

1. Multicasting packet processing scheme on SMT handset has been improved.

When IP multicasting heavily occurs in the network, it will affect the packet loss or slow operation on the SMT handset.

So, SMT handset has been improved not to respond to a multicast packet coming into the port of SMT handset(especially 8384) except in the following 3 ports.

- 8384 : OS communicator

- 9000,9001 : RTP, RTCP

* Fixed problem

1. When a SMT handset’s DHCP address is updated, the phone has not notified it to OfficeServ system. It has been fixed.

’V01.63(2012.12.14) :  14 December, 2012

* New feature

1. The Polish language has been added.

2. The SMT handset type migration between SCME and OSPP has been restricted. (STA only)

* Fixed problem

1. A character length of IP phone ID/Password extended to 32 bytes.
   - The ID and password length of SCME phone is max. 32 characters and that of OSPP phone is max 4 characters. It doesn’t allow user to insert an id and password prior to the phone type exchange from OSPP to SCME. Bug fixed.

2. When the user uses directory service in idle state in the headset mode, the volume key should operate as name searching. But it operated as volume up/down key. Bug fixed.

3. The software upgrade for the system type migration between SCME and OSPP has been restricted. (Only STA)

4. Some handsets would not boot if ‘Extension Login’ feature was enabled. It has been fixed.

’V01.57(2012.03.08) :  8 Mar, 2012

* New feature

- Additional items for AFT(Automatic funtion test) in the production phase, has been implemented to this S/W version. 
Following items have been added.
- Memory Test / Boot mode(DHCP) Test / Save the test result

’V01.56(2012.01.20) :  20 Jan, 2012

* Fixed Problems

- Enhance ITP's DHCP operation against abnormal DHCP environment.

à When ITP sends DHCP discover, it generates different TCID(Transaction ID) per each ITP

- When ITP operates with OfficeServ 7100 or 7030, it cannot download system phonebook. Bug Fixed.

- Some Italian strings have been replaced to correct ones.

’V01.55(2011.12.29) :  29 Dec, 2011

* New features

- SRTP function is added.

à SRTP is enabled or disabled by the system DB.

- PnP function is enhanced.(PreMAC, Auto PnP, Normal Login Mode)

PnP operation is configured by the system DB. When the IP phone is in the PnP mode, it operates differently according to system configuration.

- System phonebook feature is added.

When the system phonebook is added to system DB, IP phone downloads this phonebook and uses it. When the system phonebook is downloaded to IP phone, system phonebook menu is displayed in the phone’s phonebook menu.

- Multicast paging is supported.

à When the page group is called, multicast RTP is sent to each IP phone.

- Auto upgrade function is added.

* Fixed problem

- When the SMT-i3100 operates with the MCP(V4.46, not MP), it cannot have a normal conversation.

àBecause of inconsistency with the MCP, SMT-i3100 has a call using SRTP.

- During ring-back heard on the set, camp-on tone is generated at the same time because of call waiting from another phone. In the SMT phones, ring-back tone is stopped when a camp-on tone is generated.

à Modified that ring-back tone is not stopped when a camp-on tone is generated.

- When the IP phone is in the PnP mode and user enter wrong ID or password, IP phone has to be rebooted to enter ID or password again.

à When an error response is received, IP phone returns to login window to enter ID or password again.

- When the IP phone uses 8 character passwords, IP phone cannot register to system. (User password is set to use 8 characters on the system DB).

- When the IP phone has a SRTP conversation for about 10 minutes or longer, Rx voice is muted.(One-way)

’V01.45(2011.07.04) :  4 Jul, 2011

* Fixed problem
1. For applying new LCD panel, contrast parameter has been adjusted by main software.

2. In upgrade mode, a terminology has been changed as below.
 - TFTP Server -> Upgrade Server

3. To scroll ‘left & right’ for browsing symbol characters, ‘Left & Right’ soft keys has been implemented.
In the menu ‘Menu -> 7.Settings -> 2 Phone Name’, Symbol character such as “+” (i.e. “-“,”*”, etc.)can’t be used and, in this state, the soft keys are blocked.

* New featureca

1. checksum value added in upgrade mode.
To protect against error-detected PKG upgrade, add check CheckSum Value when processing Upgrade.

’V01.43(2011.05.18) :  18 May, 2011

* Fixed problem
1. In headset mode, No-way speech problem.
If push the volume key then keystroke any digit key while a voice call in headset mode, key tone was heard from the speaker and call session seemed to be disconnected.

* New feature

1. HTTP Upgrade support for SCME & iBG’s auto upgrade feature
   - DHCP Option 43 support for PnP mode.
   - HTTP upgrade support
2. In Easy-Install mode, VLAN setting can be set.

’V01.41(2011.04.08) :  08 Apr, 2011

* Fixed problem

- Qnet [P110215001] : Delayed speech error has been fixed.
Approx 700ms speech delay happened after answering a call and now this delay has been shorten to 200~300ms.

- Add Phone Name menu(Menu -> 7.Settings -> 2 Phone Name)

- DSS/BLF LED couldn’t be turned off intermittently. It has been fixed.

- Key tone was changed(Menu -> 4. Sound -> 3. Keytone( DTMF à BEEP))

- AGC(Automatic Gain Control)is disabled as default.
AGC was ‘Enable’ as default value and it caused speech volume change problem while a conversation.

- Deregistering issue
A phone intermittently had gone to the unregistered state and “Invalid User ID” message displayed on the LCD. It has been fixed.

- Initialize all led and icons at logout state.(Extension login mode\

- OSC login password has been unified same as Security password of IP Phones.

| Phone | Version/Date | Version/Date |
| --- | --- | --- |
| SMT-i3100 | ~ V01.33(2010.07.22) | V1.41 (2011.04.08) ~ |
| SMT-i5210 | ~ V01.11 (2010.07.22) | V1.17 (2011.04.08) ~ |
| SMT-i5230 | ~ V01.11 (2010.07.21) | V1.13 (2011.04.12) ~ |
| Password for OSC Login on IP Phone | System Login password will be used from above versions of package. | Security password (which is used for Phone Menu lock) has been used. |
| SMT-i5220 | Security password (which is used for Phone Menu lock) has always been used for OSC login on IP phones. |  |
| SMT-i5243 |  |  |

- PC Port didn’t work
Only on V1.33, PC port didn’t work due to some s/w bugs and ‘Lan disconnected’ warning message was shown on PC. It has been fixed.

- Call log was missed.
When a call was routed through AA, sometimes call log was missed.
It has been fixed.

’V01.33(2010.07.22) :  22 Jul, 2010

* Fixed problem

- Headset volume issue has been fixed   (Headset volume falls back to previous call's volume level.)

- Warning message about 'Busy/NoAns forward' has been modified to clarify the meaning

- In Call Forward menu, added  input number and P,C,F,[,] code.

- Added French, Turkish language.

’V01.30(2010.05.22) :  22 May, 2010

- Fixed below issues.

- In SMT-i3100/3105 Phone, Finished Phone Initialize with OfficeServ System, Sometime phone does not send Link Test message to OfficeServ, and deregistered with OfficeServ System .

- Error reason : when Phone finished initial flow with System, Phone does not check link test timer in error case.

- Fixed this issue.

’V01.28(2010.04.01) :  01 Apr, 2010

- Fixed below issues.

- If you are on a call and you scroll thru the options either on the Func or AOM menus speech is cutoff i3100 V1.26, OS7400 V4.46a 10.02.26

- Added PageUp and Page Down soft men in AOM or Func. Menu.

- when reviewing a short message the time format is in 12Hr, I think the time format should follow what you have set on your phone. Send short message to ext 3201, review the message on ext 3201 time format is in 12Hr when phone is set to 24Hr. All SMT-I phones.

’V01.27(2010.03.25) :  25 Mar, 2010

- Fixed below issues.

- If you are on a call and you scroll thru the options either on the Func or AOM menus speech is cutoff i3100 V1.26, OS7400 V4.46a 10.02.26

- Added PageUp and Page Down soft men in AOM or Func. Menu.

- when reviewing a short message the time format is in 12Hr, I think the time format should follow what you have set on your phone. Send short message to ext 3201, review the message on ext 3201 time format is in 12Hr when phone is set to 24Hr. All SMT-I phones.

’V01.26(2010.03.11) :  11 Mar, 2010

- Fixed below issue.

- Ring Indicate Led does not Blinking or On in Ring/Ring Back and Conv station .

(This issue is occurred only version 01.25)

’V01.25(2010.03.9) :  9 Mar, 2010

1. In headset Mode, Speaker LED does not On/Off. This issue is fixed.

2 . When phone received and incoming call and answering the call, the ring tone will continue to ring at the answering station. Problem on happens when station is set for USER SYS RING:off in MMC 110.

’V01.23(2010.03.02) :  02 Mar, 2010

1. Changed Phone Menu

- Samsung UI Guide format is adapted in this version.

2. In Call sate, AOM Menu is added in Soft Menu.

- in old version, AOM is displayed in idle state,

but this version AOM is display call state(ringing, ring back, conv state).

3. STA field issue is fixed.

- Need improvement on AOM key function

- MMC 115 (PGMMSG) When using the volume key to scroll through the 2 digit program messages, the display will change from a 2 digits message (00) to a 3 digit message (010.

- MMC 115 (PGMMSG) The ) parenthesis is missing at the far right of the display.

- During the call state, the phone do not have a soft button option to allow access to the AOM. Customers will need this so that they can quickly  and often transfer calls to other internal stations.

- When set to Enblock and LCR, dial 9-2143334444, before hitting the call or send there are no soft button options to backspace or clear the entry. The current IP phone have soft keys to clear, the 5243 has a cancel soft key to delete digits.  The 5230 has a Del soft key to delete digits.  In consistent.  Make the same for all phones.

’V01.09(2009.10.05) :  05 Oct. 2009

1.Changed character in administration (Installation) menu(STA Issue)

4. Idle Mode  è 4. Ext. Login

2. When phone dialing in Enblock mode, send call type 0x02(phonebook call type) in digit message.

If dialed digits are more than 5 digits, then this call is processed prefixed trunk outgoing call in OfficeServ.

This issue is occurred in only Version 01.08.

We fixed this issue. When dialing in Enblock mode, call type change 0x02 to 0xff in digit message.

.

’V01.00(2009.06.17) :  17 June, 2009

- This S/W is fixed Italian language(site requirement).

When hook off the handset in enblock mode, change SMT-i3100 display string

SMT-i3100 v00.95:

|  |  |  |  | I | n | s | e | r | i | r | e |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | n | . |  | t | e | l | e | f | o |

SMT-i3100 v01.00:

|  |  |  |  | C | o | m | p | o | r | r | e |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | p | r | e | g | o |  |  |  |  |  |

’V00.91(2009.05.23) :  23 May, 2009

- This S/W is changed Phone User Menu String and added Menu.

.   Phone Book  è   Phonebook (changed string)

.   Message

- InBox  è Inbox(changed string)

- Msg Send  è New Msg. (changed string)

- OutBox    è Outbox(changed string)

.  Sound

- Volume

- Bell  è Ring Tone(changed string)

- Key tone  è Key Tone(changed string)

. Security (added)

- Change PWD

- Call Lock

- Phone Lock

. Forward  è Call FWD(changed string)

. Setting    è Settings(changed string)

- Version  è Phone Info. (changed string)

- Del P-data  è User Init. (changed string)

’V00.91(2009.05.23) :  23 May, 2009

- This S/W is changed Phone User Menu String and added Menu.

.   Phone Book  è   Phonebook (changed string)

.   Message

- InBox  è Inbox(changed string)

- Msg Send  è New Msg. (changed string)

- OutBox    è Outbox(changed string)

.  Sound

- Volume

- Bell  è Ring Tone(changed string)

- Key tone  è Key Tone(changed string)

. Security (added)

- Change PWD

- Call Lock

- Phone Lock

. Forward  è Call FWD(changed string)

. Setting    è Settings(changed string)

- Version  è Phone Info. (changed string)

- Del P-data  è User Init. (changed string)