| To : | Doc. No. | SECYYMM-XX-X |
| --- | --- | --- |
| C.C. : | Country | ALL |
| ## From: | ## Tel: 82-31-279- Fax: ## E-mail: @samsung.com |  |
| Subject : SMT-i5210 S/W Release |  |  |

Overview :

- SMT-i5210 S/W release: V01.41(2013.03.07)

## New Version Changes

’V01.41(2013.03.07) :  07 March, 2013

* New feature

1. Multicasting packet processing scheme on SMT handset has been improved.

When IP multicasting heavily occurs in the network, it will affect the packet loss or slow operation on the SMT handset.

So, SMT handset has been improved not to respond to a multicast packet coming into the port of SMT handset(especially 8384) except in the following 3 ports.

- 8384 : OS communicator

- 9000,9001 : RTP, RTCP

* Fixed problem

1. When a SMT handset’s DHCP address is updated, the phone has not notified it to OfficeServ system. It has been fixed.

’V01.40(2012.12.13) :  13 December, 2012

* New feature

1. Polish has been added as new language option.

2. The SMT handset type migration (through the SW upgrade) between SCME and OSPP has been restricted. (STA only)

* Fixed problem

1. A character length of IP phone ID/Password extended to 32 bytes.
   - The ID and password length of SCME phone is max. 32 characters and that of OSPP phone is max 4 characters. It doesn’t allow user to insert an id and password prior to the phone type exchange from OSPP to SCME. It has been fixed.

2. Default value changed
* Menu-> 7.Settings-> Application Set.->5.Headset Key

- After the factory reset, [Headset Key] menu’s default value changes from ‘Not Use’ to 'Use’.

And when the headset key has been pressed and the headset menu has set to ‘Not use’, this status notification will be displayed on a pop-up window for reminding a user of it.

3. The SMDR message had been overlapped on the date information on the screen due to a text alignment problem. It has been fixed.

4. The name search result on the system phonebook had shown wrong information when a handset set to Deutsch. It has been fixed.

5. Some handsets would not boot if ‘Extension Login’ feature was enabled. It has been fixed.

Note) The following change has not been applied to V1.40 and will be added next version.

1. The voice quality on the SMT handsets for HF was very poor compared to the older models of ITP or DS.

- Noise suppression value(AEC) has been set from 12 to 5, for improved voice quality on the handset.

’V01.37 (2012.03.08) :  8 March, 2012

* New feature

- Additional items for AFT(Automatic funtion test) in the production phase, has been implemented to this S/W version. 
Following items have been added.
- Memory Test / Boot mode(DHCP) Test / Save the test result

’V01.36 (2012.01.20) :  20 Jan, 2012

* Fixed Problems

- Enhance ITP's DHCP operation against abnormal DHCP environment.

à When ITP sends DHCP discover, it generates different TCID(Transaction ID) per each ITP

- When ITP operates with OfficeServ 7100 or 7030, it cannot download system phonebook. Bug Fixed.

- Some Italian strings have been replaced to correct ones.

’V01.35 (2011.12.16) :  16 Dec, 2011

* New features

For more details, refer to System Administration Guide for new features.

- SRTP function is added.

à SRTP is enabled or disabled by the system DB.

- PnP function is enhanced.(PreMAC, Auto PnP, Normal Login Mode)

PnP operation is configured by the system DB. When the IP phone is in the PnP mode, it operates differently according to system configuration.

- System phonebook feature is added.

When the system phonebook is added to system DB, IP phone downloads this phonebook and uses it. When the system phonebook is downloaded to IP phone, system phonebook menu is displayed in the phone’s phonebook menu.

- Multicast paging is supported.

à When the page group is called, multicast RTP is sent to each IP phone.

- Auto upgrade function is added.

* Fixed problem

- When the SMT-i5210 operates with the MCP(V4.46, not MP), it cannot have a normal conversation.à Bug Fixed. Because of inconsistency with the MCP, SMT-i5210 has a call using SRTP.

- During ringback heard on the set, campon tone is generated at the same time becaue of call waiting from another phone.In the SMT phones, ringback tone is stopped when a campon tone is generated.

à Modified that ringback tone is not stopped when a campon tone is generated.

- When the IP phone is in the PnP mode and user enter wrong ID or password, IP phone has to be rebooted to enter ID or password again.

à When an error response is received, IP phone returns to login window to enter ID or password again.

- When the IP phone uses 8 character password, IP phone cannot register to system. (User password is set to use 8 characters on the system DB). Bug Fixed.

- When the IP phone has a SRTP conversation for about 10 minutes or longer, Rx voice is muted.(Oneway)

’V01.22 (2011.08.13) :  13 Aug, 2011

* Fixed problem
1. (SEI/SEG/SEUK) “De-registration(Connection failure)“ has been fixed.

Memory leakage occurs when SMT-i5210 is received LARGE_PHONE_DB0 message which system send to the phone whenever there happen system DB changes, changes some data on the phone (volume change, name change, setting forward and so on).
This problem only happened to S/W V1.19 and V1.20.

’V01.20 (2011.07.04) :  4 Jul, 2011

* Fixed problem
1. In Italian Language Mode, display error fixed in Scheduler Menu.
In the Menu New Alarm Reminder the translation of ring on time must be changed from ‘Oraaesattaa’ to ‘Ora esatta’  and the minutes from ‘minn’ to ‘min’.

2. One way speech issue
This problem intermittently happened when IP phone user makes a call to digital phone or PSTN.
It has been occurred by V1.17 and V1.19, even though both s/w haven’t been released officially.

* New feature

1. checksum value added in upgrade mode.
To protect against error-detected PKG upgrade, add check CheckSum Value when processing Upgrade .

’V01.19 (2011.05.18) :  18 May, 2011

* Fixed problem
1. In headset mode, No-way speech problem.
If push the volume key then keystroke any digit key while a voice call in headset mode, key tone was heard from the speaker and call session seemed to be disconnected.

2. One way speech issue
This problem intermittently happened when IP phone user makes a call to digital phone or PTSN.

3. Voice mode could not be changed sRTP to RTP.(SCMv2)
If sRTP Call had been finished and then new RTP Call was followed, noise was occurred.

* New feature

1. HTTP Upgrade support for SCME & iBG’s auto upgrade feature
   - DHCP Option 43 support for PnP mode.
   - HTTP upgrade support

’V01.17 (2011.04.08) :  08 Apr, 2011

* Fixed problem

- Qnet [P110215001] : Delayed speech error has been fixed.
Approx 700ms speech delay happened after answering a call and now this delay has been shorten to 200~300ms.

2. DSS/BLF LED couldn’t be turned off intermittently. It has been fixed.

3. Key tone was changed(Menu -> 4. Sound -> 3. Keytone( DTMF à BEEP))

4. Menu display Error in Russia Language Mode was fixed.

5. Key tone was changed(Menu -> 4. Sound -> 3. Keytone( DTMF à BEEP))

6. Initialize all led and icons at logout state.(Extension login mode)

7. OSC login password has been unified same as Security password of IP Phones.

| Phone | Version/Date | Version/Date |
| --- | --- | --- |
| SMT-i3100 | ~ V01.33(2010.07.22) | V1.41 (2011.04.08) ~ |
| SMT-i5210 | ~ V01.11 (2010.07.22) | V1.17 (2011.04.08) ~ |
| SMT-i5230 | ~ V01.11 (2010.07.21) | V1.13 (2011.04.12) ~ |
| Password for OSC Login on IP Phone | System Login password will be used from above versions of package. | Security password (which is used for Phone Menu lock) has been used. |
| SMT-i5220 | Security password (which is used for Phone Menu lock) has always been used for OSC login on IP phones. |  |
| SMT-i5243 |  |  |

’V01.11 (2010.07.22) :  22 Jul, 2010

* Fixed problems

1. Special code has beed added in forward setting mode. Below code is possible to input.

F,f  (0x0c): Flash Code

P ,p (0x0d): Pause Code

C ,c (0x0e): Change PTMFC CODE, Dial way

[, or ] (0x0f): OCC Code, Encryption

2. All of the Muti language String have been included with reference to your requests.

(Italian, Spanish, German, Danish, Dutch, Finish, French, Polish, Swidish, Russian, Greek, Turkish)

3. Headset volume issue has been fixed. (Headset volume falls back to previous call's volume level.)

4. Warning message about 'Busy/NoAns forward' has been modified to clarify the meaning.

5. VLAN setting error in the Easyinstall menu of Engineer mode has been fixed.

6. Phonebook setting error in adding entry has beeb fixed.

7. Fixed Headset key does not operated in BGM Mode.

’V01.07 (2010.04.01) :  1 Apr, 2010

* Fixed problems

- Changed Default  value. (Menu-> 7.Settings-> Application Set.->4.Short Message)

After  Factory Reset,  Changed [ Msg Contents View] menu option(OFF à ON)

’V01.06 (2010.03.25) :  25 Mar, 2010

* Fixed problems

- upgrade i5210 to version 1.05, check MMC 840 either with IT Tool or KMMC, phone version shows as 1.01. Phone Information displays correct version 1.05 on IP Phone.

- If you are on a call and you scroll thru the options either on the Func or AOM menus speech is cutoff i5210 V1.05, OS7400 V4.46a 10.02.26

- Added PageUp and Page Down soft men in AOM or Func. Menu.

- when reviewing a short message the time format is in 12Hr, I think the time format should follow what you have set on your phone. Send short message to ext 3201, review the message on ext 3201 time format is in 12Hr when phone is set to 24Hr. All SMT-I phones.

- Added Headset Key menu.(Menu -> 7 Settings -> 1 Application Setting -> 7 Headset Key)

’V01.05 (2010.03.09) :  09 Mar, 2010

* Fixed problems

1. When phone received and incoming call and answering the call, the ring tone will continue to ring at the answering station. Problem on happens when station is set for USER SYS RING:off in MMC 110.

’V01.03 (2010.03.02) :  02 Mar, 2010

* Fixed problems

1. Changed Phone Menu

- Samsung UI Guide format is adapted in this version.

2. In Call sate, AOM Menu is added in Soft Menu.

- in old version, AOM is displayed in idle state,

but this version AOM is display call state(ringing, ring back, conv state).

3. STA field issue is fixed.

- Need improvement on AOM key function

- MMC 115 (PGMMSG) When using the volume key to scroll through the 2 digit program messages, the display will change from a 2 digits message (00) to a 3 digit message (010.

- MMC 115 (PGMMSG) The ) parenthesis is missing at the far right of the display.

- During the call state, the phone do not have a soft button option to allow access to the AOM. Customers will need this so that they can quickly  and often transfer calls to other internal stations.

- When set to Enblock and LCR, dial 9-2143334444, before hitting the call or send there are no soft button options to backspace or clear the entry. The current IP phone have soft keys to clear, the 5243 has a cancel soft key to delete digits.  The 5230 has a Del soft key to delete digits.  In consistent.  Make the same for all phones.

-. Call LED has wrong flash rate during DND state

- Need to have backlight when user uses the phone even backlight is turned off at the schedule time

- Service key and Menu for service key is showing different options on the 5210 phone.  Both service options menus should show the same.

- Phone locked up while in AOM setting menu

- When set to Enblock and LCR, dial 9-2143334444, before hitting the call or send there are no soft button options to backspace or clear the entry. The current IP phone have soft keys to clear, the 5243 has a cancel soft key to delete digits.  The 5230 has a Del soft key to delete digits.  In consistent.  Make the same for all phones.

- When adding the same name and Number to the first four fields [Mobile, Home office, Extension number] to the phone book  a second time with  same information. When the entry is duplicated, the display will show "Duplicate Extrnsion No, Save Again?" NO soft key to go back or Yes or No.  After about 30 seconds the phone locks (5220) or resets (5330).